IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Channel models for TG8 |
| Date Submitted | September 2012 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Re: | In response to Technical Guidance Document contributions |
| Abstract | Channel models |
| Purpose | Reference channel models for proposals evaluation to TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1	Introduction	3

2	Large-Scale Path Loss	4

2.1	Outdoor Path Loss	4

2.1.1	Propagation situations	4

2.1.2	Line-of-sight (LoS) paths	4

2.1.3	Propagation over rooftops, non-line-of-sight (NLoS1)	5

2.1.4	Propagation along street canyons, non-line-of-sight (NLoS2)	6

2.2	Path Loss for sub-1 GHz, 2.4 GHz and 5 GHz bands	6

2.2.1	LoS in street canyons	6

2.2.2	NoLoS over roof-tops for urban area	7

2.2.3	NoLoS over roof-tops for suburban area	10

2.2.4	NoLoS in street canyons for sub-1 GHz band	11

2.2.5	NoLoS in street canyons for 2.4 GHz and 5 GHz bands	12

2.2.6	Path loss between terminals located below roof-top for 900 MHz and 2.4 GHz bands.	12

2.3	Outdoor Path Loss for UWB band	15

2.4	Indoor Path Loss	15

2.4.1	Attenuation Factor Model for 900 MHz band	15

2.4.2	Indoor Path Loss for 2.4 GHz and 5 GHz bands	18

2.4.3	Indoor Path Loss for UWB band	19

3	Small-Scale Fading	21

3.1	SISO ITU models	21

3.2	MatLab™ code	23

3.3	SISO ITU extended models	24

3.4	MatLab™ code	25

3.5	SISO ETSI BRAN 5 GHz channel model	26

3.6	MatLab™ code	30

3.7	UWB Channel model	31

4	MIMO channel models	32

4.1	Correlated MIMO fading channel	33

4.1.1	Simulation methodology	35

4.2	MatLab™ code	37

4.2.1	Beam steering example	45

5	Bibliography	46

6	Annex A – Matlab code for IEEE802.15.4a UWB channel model	47

# Introduction

There are numerous contributions to path loss propagation in the literature for the target frequency bands of TG8 indicated in the PAR. Namely, sub-1 GHz, 2.4 GHz, 5 GHz and high band of UWB.

We present the most accepted contributions to be used for a fair comparison of PHY and MAC proposals.

# Large-Scale Path Loss

## Outdoor Path Loss

Outdoor path loss is based on ITU-R  P.1411-6 “Propagation data and prediction methods for the planning of short-range outdoor radio communication systems and radio local area networks in the frequency range 300 MHz to 100 GHz”.

It provides an up to date recommendation for propagation over paths of length less than 1 Km, which is affected primarily by buildings and trees. The effect of buildings is predominant, since most short-path radio links are found in urban and suburban areas. The mobile terminal is most likely to be held by a pedestrian or located in a vehicle.

The type of propagation mechanism that dominates depends also on the height of the base station antenna relative to the surrounding buildings. Table 1 lists the typical cell types relevant for outdoor short-path propagation.

Table 1  Definition of cell types.

| Cell type | Cell radius | Typical position of base station antenna |
| --- | --- | --- |
| Micro-cell | 0.05 to 1 km | Outdoor; mounted above average roof-top level, heights of some surrounding buildings may be above base station antenna height |
| Dense urban micro-cell | 0.05 to 0.5 km | Outdoor; mounted below average roof-top level |
| Pico-cell | Up to 50 m | Indoor or outdoor (mounted below roof-top level) |

### Propagation situations

Four situations of base station (BS) and mobile station (MS) geometries are depicted in Figure 1. Base station BS1 is mounted above roof-top level. The corresponding cell is a micro-cell. Propagation from this BS is mainly over the roof-tops. Base station BS2 is mounted below roof-top level and defines a dense urban micro- or pico-cellular environment. In these cell types, propagation is mainly within street canyons. For mobile-to-mobile links, both ends of the link can be assumed to be below roof-top level, and the models relating to BS2 may be used.

### Line-of-sight (LoS) paths

The paths BS1-MS2 and BS2-MS4 illustrated in Figure 1 are examples of LoS situations. The same models can be applied for both types of LoS path.

Figure 1  Typical propagation situation in urban areas.

### Propagation over rooftops, non-line-of-sight (NLoS1)

The typical NLoS case (link BS1-MS1 in Figure 1) is described in Figure 2.

Figure 2 Parameters for the NoLoS1 case.

The relevant parameters are given by

hr :	average height of buildings (m)

w :	street width (m)

b :	average building separation (m)

 :	street orientation with respect to the direct path (degrees)

hb :	BS antenna height (m)

hm :	MS antenna height (m)

l :	length of the path covered by buildings (m)

d :	distance from BS to MS.

### Propagation along street canyons, non-line-of-sight (NLoS2)

Figure 3 shows the situation for a typical dense urban micro-cellular NLoS case, link BS2-MS3 in Figure 1.

Figure 3 Parameters for the NoLoS2 case.

The relevant parameters are given by

w1 :	street width at the position of the BS (m)

w2 :	street width at the position of the MS (m)

x1 :	distance BS to street crossing (m)

x2 :	distance MS to street crossing (m)

 :	is the corner angle (rad).

## Path Loss for sub-1 GHz, 2.4 GHz and 5 GHz bands

### LoS in street canyons

The path loss is characterized by two slopes and a breakpoint. An upper and lower bounds are provided. However, we propose to use the median value that is given by

LLoS,m = Lbp +6+Xσ+ 20 log10dRbpford ≤ Rbp40 log10dRbpford > Rbp                               Equation 1

Lbp = 20 log10λ28 π hbhm ,  Rbp ≈ 4 hbhmλ,  λ=cfc.

where Lbp is transmission loss at break point, Rbp is the breakpoint distance andfcis the central frequency. X represents a normal random variable in dB having a standard deviation of  dB.

### NoLoS over roof-tops for urban area

The recommendation presents values for which the models are valid. However, for the purposes of comparison of proposals, such values can be relaxed a bit.  Models are defined for the two situations described in . The models are valid for:

hb: 4 to 50 m

hm: 1 to 3 m

f: 800 to 5 000 MHz

2 to 16 GHz	 for hb < hr and w2 < 10 m (or sidewalk)

d: 20 to 1000 m.

In the model for transmission loss in NLoS for roof-tops of similar height, the loss between isotropic antennas is expressed as the sum of free-space loss, Lbf, the diffraction loss from roof-top to street Lrts and the reduction due to multiple screen diffraction past rows of buildings, Lmsd. In this model Lbf and Lrts are independent of the BS antenna height, while Lmsd is dependent on whether the base station antenna is at, below or above building heights.

LNLoS1 = Lbf + Lrts + Lmsd+XσforLrts + Lmsd > 0Lbf+XσforLrts + Lmsd ≤ 0                       Equation 2

Lbf = 32.4 + 20 log10 (d  1 000) + 20 log10 (f)

where d  is path length,  f  is frequency (MHz),  Lrts describes the coupling of the wave propagating along the multiple-screen path into the street where the mobile station is located and given by

Lrts = –8.2 – 10 log10 (w) + 10 log10 (f) + 20 log10 (Δhm) + Lori

Lori = –10 + 0.354ϕfor 0° ≤ ϕ < 35°2.5 + 0.075(ϕ – 35)for35° ≤ ϕ < 55°4.0 – 0.114(ϕ – 55)for55° ≤ ϕ ≤ 90°

Δhm = hr – hm

Lori is the street orientation correction factor, which takes into account the effect of roof-top-to-street diffraction into streets that are not perpendicular to the direction of propagation (see Figure 2). X represents a normal random variable in dB having a standard deviation of  dB.

The multiple screen diffraction loss from the BS due to propagation past rows of buildings depends on the BS antenna height relative to the building heights and on the incidence angle. A criterion for grazing incidence is the “settled field distance”, ds:

ds = λd2Δhb2  ,  Δhb = hb – hr

Finally, for the calculation of Lmsd, ds is compared to the distance l over which the buildings extend:

Lmsd=−tanhlogd−logdbpχ⋅L1msdd−Lmid+Lmid                    for  l>ds and dhbp>0tanhlogd−logdbpχ⋅L2msdd−Lmid+Lmid                      for  l≤ds and dhbp>0L2msdd                                                                                      for     dhbp=0L1msdd−tanhlogd−logdbpζ⋅Lupp−Lmid−Lupp+Lmid  for  l>ds and dhbp<0

dhbp=Lupp−Llow

ζ=(Lupp−Llow)⋅υ

Lmid=(Lupp+Llow)2

Lupp=L1msddbp

Llow=L2msddbp

dbp = Δhblλ

 = 0.0417

 = 0.1

#### Calculation of L1msd for l  d

L1msdd = Lbsh + ka + kd log10 (d  1 000) + kf log10 (f) − 9 log10 (b)

where:

Lbsh = –18 log10(1 + Δhb)forhb > hr0forhb ≤ hr

is a loss term that depends on the BS height:

ka=71.4 for hb> hr and f > 2 000 MHz73−0.8Δhb for   hb≤ hr,  f > 2 000 MHz  and d ≥ 500 m73−1.6Δhb d1 000 for   hb≤ hr,  f > 2 000 MHz  and d < 500 m54 for hb> hr and f ≤ 2 000 MHz54−0.8Δhb for   hb≤ hr,  f ≤ 2 000 MHz  and d ≥ 500 m54−1.6Δhb d1 000 for   hb≤ hr,  f ≤ 2 000 MHz  and d < 500 m

kd = 18 for hb > hr18 – 15Δhbhr for hb ≤ hr

kf = −8 for    f>2 000 MHz

#### Calculation of L2msd  for l  ds

L2msdd = –10 log10 QM2

where:

QM = 2.35 Δhbd bλ0.9forhb > hr+δhubdforhb ≤ hr+δhu  and  hb ≥ hr+δhlb2πd λρ 1θ – 12π + θforhb < hr+δhl

and

θ = arctan Δhbb

ρ = Δhb2 + b2

and		δhu = 10−log10bλ − log10d9 + 109log10b2.35

δhl = 0.00023b2−0.1827b−9.4978log10f2.938+0.000781 b+0.06923

### NoLoS over roof-tops for suburban area

The recommendation presents values for which the models are valid. However, for the purposes of comparison of proposals, such values can be relaxed a bit.  Model is defined for the situation of hb > hr described in . The model is valid for

hr: any height m

hb: 1 to 100 m

hm: 4 to 10 (less than hr) m

hb: hr + hb m

hm: hr − hm m

f: 0.8 to 20 GHz

w: 10 to 25 m

d: 10 to 1 000 m

LNLoS1=20⋅log104πdλ+Xσ for d < d0 (Direct wave dominant region) L0n+Xσ for d0 ≤ d<dRD (Reflected wave dominant region)32.1⋅log10ddRD+LdRD+Xσ for d≥dRD  (Diffracted wave dominant region)     Equation 3

where

L0n=Ldk+Ldk+1−Ldkdk+1−dkd−dkwhen dkd<dk+1dRDk=0, 1, 2...Ldk+LdRD−LdkdRD−dkd−dkwhen  dkd<dRDdk+1

dk=1sinϕ⋅Bk2+hb−hm2

Ldk=20⋅log104πdkp0.4kλ

dRDf=0.625⋅d3−d1⋅log10f+0.44⋅d1+0.5⋅d2+0.06⋅d3 0.8 GHz ≤f≤ 20 GHz

LdRD=Ldk+Ldk+1−Ldkdk+1−dkdRD−dkdkdRDdk+1

dkp=1sinϕk⋅Ak2+hb−hm2

Ak=w⋅hb−hm⋅2k+12⋅hr−hm

Bk=w⋅hb−hm⋅2k+12⋅hr−hm−k⋅w

ϕk=tan−1BkAk⋅tanϕ

X represents a normal random variable in dB having a standard deviation of  dB.

### NoLoS in street canyons for sub-1 GHz band

For this NLoS situation both antennas are below roof-top level (see Figure 3 for reference).

LNLoS2=−10log1010−Lr10+10−Ld10+Xσ                 Equation 4

Lr=20 log10 (x1+x2)+x1x2f(α)w1w2+20 log104πλ

f(α) = 3.86α3.5  ,   0.6 <   < .

Ld=10 log10x1x2(x1+x2)+2Da−0.190−α180π+20 log104πλ

Da = 402πarctanx2w2 + arctanx1w1 –π2

X represents a normal random variable in dB having a standard deviation of  dB.

### NoLoS in street canyons for 2.4 GHz and 5 GHz bands

This NLoS propagation model is described in 2.1.4 with the corner angle  = /2. Such model assumes hb < hr and w2 is up to 10 m (or sidewalk).   Using x1, x2, and w1, as shown in Figure 3, the overall path loss (LNLoS2) beyond the corner region (x2 > w1/2 + 1) is given by

LNLoS2=LLos+Lc+Latt+Xσ                 Equation 5

Lc=Lcornerlog101+ d log10x2−w1/2 w1/2+1 <x2≤w1/2+1+ dLcorner x2>w1/2+1+ d

Latt=10βlog10x1+x2x1+w1/2+ d x2>w1/2+1+ d0 x2>w1/2+1+ d

where LLoS is the path loss in the LoS street for x1 (> 20 m) is given in Equation 1.  Lcorner is given as 20 dB in an urban environment and 30 dB in a residential environment. dcorner is 30 m and is in both environments. X represents a normal random variable in dB having a standard deviation of  dB.

### Path loss between terminals located below roof-top for 900 MHz and 2.4 GHz bands.

This model is recommended for propagation between low-height terminals where both terminal antenna heights are near street level well below roof-top height, but are otherwise unspecified. The model is based on measurements made in the UHF band with antenna heights between 1.9 and 3.0 m above ground, and transmitter-receiver distances up to 3,000 m.

#### Location variability

Location variability refers to the spatial statistics of local ground cover variations. This is a useful result over scales substantially larger than the ground cover variations, and over which path variations are insignificant. As location variability is defined to exclude multipath variations, it is not dependent on system bandwidth. Percentage location p can vary between 1 and 99.

Table 2  LoS and NLoS location variability corrections.

| p (%) | LLoS (dB) | LNLoS (dB) | dLoS (m) |
| --- | --- | --- | --- |
| 1 | –11.3 | –16.3 | 976 |
| 10 | –7.9 | –9.0 | 276 |
| 50 | 0.0 | 0.0 | 44 |
| 90 | 10.6 | 9.0 | 16 |
| 99 | 20.3 | 16.3 | 10 |

#### Path loss law

The path loss L at distance d and percentage location p is given by

L(d,p)=LLOS(d,p)+Xσ,LNLOS(d,p)+Xσ,LLOS+Xσ+(LNLOS−LLOS)(d−dLOS)w,    d<dLOSd>dLOSd=dLOS                                Equation 6

where w is introduced to provide a transition region between the LoS and NLoS regions. This transition region is seen in the data and typically has a width of w = 20 m. dLOS is the distance for which the LoS fraction equals the location percentage p and given by

dLOS(p)=212log10(p100)2−64 log10(p100)p<4579.2−70(p100)p≥45                              Equation 7

LLOS and LNLOS are the LoS/NLoS location correction to the median value of LoS/NLoS loss respectively, and given by

LLoS(d,p)=LLoSmedian(d)+ΔLLoS(p)                                                                        Equation 8

LLoSmedian(d)=32.45+20 log10f+20 log10(d1 000)

ΔLLoS(p)=1.5624σ−2ln(1−p100)−1.1774

where the frequency f  is in MHz, the distance between the terminals d is in meters and  = 7 dB.

LNLoS(d,p)=LNLoSmedian(d)+ΔLNLoS(p)                                                                       Equation 9

LNLoSmedian(d)=9.5+45 log10f+40 log10(d1 000)+Lurban

ΔLNLoS(p)=σN−1(p100)

where Lurban depends on the urban category and is 0 dB for suburban, 6.8 dB for urban and 2.3 dB for dense urban/high-rise, and  = 7 dB. X represents a normal random variable in dB having a standard deviation of  dB.

N1(.) is the inverse normal cumulative distribution function. An approximation to this function, when p is between 1 and 99%, is given by the location variability function Qi (x), where Qi (x) is the inverse complementary cumulative normal distribution as a function of probability. Alternatively, values of the NLoS location correction for p = 1, 10, 50, 90 and 99% are given in Table 2.

#### Approximation to the inverse complementary cumulative normal distribution function

The following approximation to the inverse complementary cumulative normal distribution function, Qi (x), is valid for 0.01 ≤ x ≤ 0.99:

Qi(x)=T(x)−ξ(x)x≤0.5−(T(1−x)−ξ(1−x))x>0.5                                                            Equation 10

where

T(x) = – 2 ln(x)

ξ(x) = (C2 ⋅ T(x) + C1) ⋅ T(x) + C0(D3 ⋅ T(x) + D2) ⋅ T(x) + D1 ⋅ T(x) + 1

C0  =  2.515517, C1  =  0.802853, C2  =  0.010328, D1  =  1.432788, D2  =  0.189269, D3  =  0.001308.

Values given by the above equations are given in Table 3.

Table 3  Approximate inverse complementary cumulative normal distribution values.

| q% | Qi (q/100) | q% | Qi (q/100) | q% | Qi (q/100) | q% | Qi (q/100) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2.327 | 26 | 0.643 | 51 | –0.025 | 76 | –0.706 |
| 2 | 2.054 | 27 | 0.612 | 52 | –0.050 | 77 | –0.739 |
| 3 | 1.881 | 28 | 0.582 | 53 | –0.075 | 78 | –0.772 |
| 4 | 1.751 | 29 | 0.553 | 54 | –0.100 | 79 | –0.806 |
| 5 | 1.645 | 30 | 0.524 | 55 | –0.125 | 80 | –0.841 |
| 6 | 1.555 | 31 | 0.495 | 56 | –0.151 | 81 | –0.878 |
| 7 | 1.476 | 32 | 0.467 | 57 | –0.176 | 82 | –0.915 |
| 8 | 1.405 | 33 | 0.439 | 58 | –0.202 | 83 | –0.954 |
| 9 | 1.341 | 34 | 0.412 | 59 | –0.227 | 84 | –0.994 |
| 10 | 1.282 | 35 | 0.385 | 60 | –0.253 | 85 | –1.036 |
| 11 | 1.227 | 36 | 0.358 | 61 | –0.279 | 86 | –1.080 |
| 12 | 1.175 | 37 | 0.331 | 62 | –0.305 | 87 | –1.126 |
| 13 | 1.126 | 38 | 0.305 | 63 | –0.331 | 88 | –1.175 |
| 14 | 1.080 | 39 | 0.279 | 64 | –0.358 | 89 | –1.227 |
| 15 | 1.036 | 40 | 0.253 | 65 | –0.385 | 90 | –1.282 |
| 16 | 0.994 | 41 | 0.227 | 66 | –0.412 | 91 | –1.341 |
| 17 | 0.954 | 42 | 0.202 | 67 | –0.439 | 92 | –1.405 |
| 18 | 0.915 | 43 | 0.176 | 68 | –0.467 | 93 | –1.476 |
| 19 | 0.878 | 44 | 0.151 | 69 | –0.495 | 94 | –1.555 |
| 20 | 0.841 | 45 | 0.125 | 70 | –0.524 | 95 | –1.645 |
| 21 | 0.806 | 46 | 0.100 | 71 | –0.553 | 96 | –1.751 |
| 22 | 0.772 | 47 | 0.075 | 72 | –0.582 | 97 | –1.881 |
| 23 | 0.739 | 48 | 0.050 | 73 | –0.612 | 98 | –2.054 |
| 24 | 0.706 | 49 | 0.025 | 74 | –0.643 | 99 | –2.327 |
| 25 | 0.674 | 50 | 0.000 | 75 | –0.674 |  |  |

## Outdoor Path Loss for UWB band

The outdoor path loss model for UWB band is based in the final report for UWB channel models document of former IEEE802.15.4a TG, as the propagation situations are the same for IEEE802.15.8 TG.

L=L0−10nlog10dd0+Xσ               Equation 11

where d0=1m is the reference distance, L0 is path loss at the reference distance and n is path loss exponent. X represents a normal random variable in dB having a standard deviation of  dB.

L0= -45.6 (LOS) , -73.0 (NLOS).

n= 1.76 (LOS) ,  2.5 (NLOS).

σ=1.9 (LOS), σ=3.9 (NLOS)

d is valid up to 17m.

## Indoor Path Loss

### Attenuation Factor Model for 900 MHz band

An indoor propagation model that includes the effect of the building type and variations caused by obstacles is describe by Sei92b. This model is flexible and more accurate than the conventional log distance path loss model. The path loss is given by

PL(d)=20log10(4πfcd0c)+10nSFlog10dd0+FAF+n=1NBWPAF+Xσ                   Equation 12

where d0 is a reference distance, c is the speed of light, nSF is the exponent of the same floor measurement. FAF is the floor attenuation factor for a specified number of building floors, PAF represents the partition attenuation factor for a specific obstruction encountered by a ray drawn between transmitter and receiver. X represents a normal random variable in dB having a standard deviation of  dB.

Alternatively, FAF may be replaced by nMF, an exponent that considers the effects of multiple floor separation.

PL(d)=20log10(4πfcd0c)+10nMFlog10dd0+n=1NBWPAF+Xσ                               Equation 13

Example: An estimate of the path loss from a distance of 30m, through 3 floors of office building 1. Assume that 2 concrete block walls are between transmitter and receiver on the intermediate floors.

The mean path loss exponent for the same floor measurements in a building is n=3.27 (see Table 5). The mean path loss exponent for the three-floor measurements in a building is n=5.22 (see Table 5). The average floor attenuation factor for 3 floors is FAF=24.4 dB (see Table 4). The average attenuation for concrete block wall is PAF=13 dB (see Table 6) and NBW=2. The reference distance is d0=1m and fc=900 MHz.

The mean path loss (without shadowing) using                    Equation 12 is given by

PL(30m)=31.5266+10*3.27log10(30)+24.4+2*13=130.2 dB

The mean path loss (without shadowing) using                                Equation 13 is given by

PL(30m)=31.5266+10*5.22log10(30)+2*13=134.6 dB

Table 4  Average floor attenuation factor in two office buildings.

| Building | FAF (dB) |  (dB) |
| --- | --- | --- |
| Office building 1 |  |  |
| Through 1 floor | 12.9 | 7.0 |
| Through 2 floor | 18.7 | 2.8 |
| Through 3 floor | 24.4 | 1.7 |
| Through 4 floor | 27.0 | 1.5 |
| Office building 2 |  |  |
| Through 1 floor | 16.2 | 2.9 |
| Through 2 floor | 27.5 | 5.4 |
| Through 3 floor | 31.6 | 7.2 |

Table 5  Path loss exponent for various types of buildings

| Building | n |  (dB) |
| --- | --- | --- |
| All buildings |  |  |
| All locations | 3.14 | 16.3 |
| Same floor | 2.76 | 12.9 |
| Through 1 floor | 4.19 | 5.1 |
| Through 2 floor | 5.04 | 6.5 |
| Through 3 floor | 5.22 | 6.7 |
| Grocery store | 1.81 | 5.2 |
| Retail store | 2.18 | 8.7 |
| Office building 1 |  |  |
| Entire building | 3.54 | 12.8 |
| Same floor | 3.27 | 11.2 |
| West wing 5th floor | 2.68 | 8.1 |
| Central Wing 5th floor | 4.01 | 4.3 |
| West wing 4th floor | 3.18 | 4.4 |
| Office building 2 |  |  |
| Entire building | 4.33 | 13.3 |
| Same floor | 3.25 | 5.2 |

Table 6  Average signal loss measurements for radio paths obstructed by common building materials

| Material type | Loss (dB) | Frequency (MHz) |
| --- | --- | --- |
| All metal | 26 | 815 |
| Aluminum siding | 20.4 | 815 |
| Foil insulation | 3.9 | 815 |
| Concrete block wall | 13 | 1300 |
| Metallic inventory | 4 - 7 | 1300 |
| Loss from 1 floor | 20 - 30 | 1300 |
| Ceiling duct | 1 - 8 | 1300 |

### Indoor Path Loss for 2.4 GHz and 5 GHz bands

The site-specific model of Recommendation ITU-R P.1238-7 propagation and prediction methods for the planning of indoor radio communication systems and radio local area networks in the frequency range 900 MHz to 100 GHz is employed. Such propagation model would have the option of explicitly accounting for the loss due to each wall instead of including it in the distance model.

Ltotal    20 log10 f    n log10 d    Lf  (n)  –  28                                   Equation 14

where:

n :	distance power loss coefficient;

f :	frequency (MHz);

d :	separation distance (m) between the base station and portable terminal (where d  1 m);

Lf  :	floor penetration loss factor (dB);

n :	number of floors between base station and portable terminal (n  1).

Typical parameters are given in Table 7 and Table 8.

Table 7  Power loss coefficient, n.

| Frequency | Residential | Office | Commercial |
| --- | --- | --- | --- |
| 1.8-2 GHz | 28 | 30 | 22 |
| 2.4 GHz | 28 | 30 |  |
| 3.5 GHz |  | 27 |  |
| 4 GHz | – | 28 | 22 |
| 5.2 GHz | 30 (apartment) 28 (house) (2) | 31 | – |
| 5.8 GHz |  | 24 |  |

2Apartment: Single or double storey dwellings for several households. In general most walls separating rooms are concrete walls.

House: Single or double storey dwellings for a household. In general most walls separating rooms are wooden walls.

Table 8  Floor penetration factor, Lf, with n being the number of floors penetrated.

| Frequency | Residential | Office | Commercial |
| --- | --- | --- | --- |
| 1.8-2 GHz | 4 n | 15 + 4 (n – 1) | 6 + 3 (n – 1) |
| 2.4 GHz | 10(1) (apartment) 5 (house) | 14 |  |
| 3.5 GHz |  | 18 (1 floor) 26 (2 floors) |  |
| 5.2 GHz | 13(1) (apartment) 7(2) (house) | 16 (1 floor) | – |
| 5.8 GHz |  | 22 (1 floor) 28 (2 floors) |  |

(1) Per concrete wall.

(2) Wooden mortar.

### Indoor Path Loss for UWB band

The outdoor path loss model for UWB band is based in the final report for UWB channel models document of former IEEE802.15.4a TG, as the propagation situations are the same for IEEE802.15.8 TG.

L=L0−10nlog10dd0+Xσ               Equation 15

where d0=1m is the reference distance, L0 is path loss at the reference distance, n is path loss exponent and Xσis a normal random variable in dB with standard deviation σin dB.

Residential:

L0= -43.9 (LOS) , -48.7 (NLOS).

n= 1.79 (LOS) ,  4.58 (NLOS).

Xσis not specified.

d is valid up to 20m.

Indoor Office:

L0= -35.4 (LOS) , -59.9 (NLOS).

n= 1.63 (LOS) ,  3.07 (NLOS).

σ=1.9 (LOS), σ=3.9 (NLOS)

d is valid up to 28m.

# Small-Scale Fading

## SISO ITU models

For the frequency bands and bandwidths targeted by TG8, extensive measurement datasets for SISO (single-input single-output) channel modeling were performed throughout the 1980s and 1990s, like projects COST207, COST231, COST259, etc. Most of these channel models form a basis for the ITU models, which were largely applied in the development of the third generation wireless communication systems.

The ITU profiles model the temporal dispersion of the time-variant wireless channel h(τ,t), as a discrete tapped-delay-line with K taps given by

h(τ,t)=k=1Kaktδ(τ−τk)          Equation 16

The tapped-delay-line model represents a wide sense stationary uncorrelated scattering (WSSUS) channel as long as the time-variant tap coefficients akt are uncorrelated Gaussian random processes with a Doppler power spectrumSk(f). Such power spectrum could be either the classic (Jakes) Doppler spectrum or flat Doppler spectrum.

The classic Doppler spectrum is given by

S(f)=1πfd1−(ffd)2  ,  |f|≤fd

The flat Doppler spectrum is given by

S(f)=12fd ,  |f|≤fd

where fdis the maximum Doppler shift or Doppler spread:

fd=vfcc

v is the relative velocity, fc is the carrier frequency and c is the speed of light.

Table 9   ITU Indoor office

| Tap | Relative delay (ns) | Average power (dB) | Doppler spectrum |
| --- | --- | --- | --- |
| 1 | 0 | 0 | flat |
| 2 | 50 | -3 | flat |
| 3 | 110 | -10 | flat |
| 4 | 170 | -18 | flat |
| 5 | 290 | -26 | flat |
| 6 | 310 | -32 | flat |

Table 10  ITU outdoor to indoor and pedestrian

| Tap | Relative delay (ns) | Average power (dB) | Doppler spectrum |
| --- | --- | --- | --- |
| 1 | 0 | 0 | classic |
| 2 | 110 | -9.7 | classic |
| 3 | 190 | -19.2 | classic |
| 4 | 410 | -22.8 | classic |

Table 11  ITU vehicular

| Tap | Relative delay (ns) | Average power (dB) | Doppler spectrum |
| --- | --- | --- | --- |
| 1 | 0 | 0 | classic |
| 2 | 310 | -1 | classic |
| 3 | 710 | -9 | classic |
| 4 | 1090 | -10 | classic |
| 5 | 1730 | -15 | classic |
| 6 | 2510 | -20 | classic |

## MatLab™ code

The following scripts were written in MatLabR2011 and the Communications Toolbox.

%                                                                   %

% function [h] = ITUchannel(model,fd,Ts)                            %

%                                                                   %

% Return object for FIR channel taps of ITU channel models          %

%                                                                   %

% INPUT PARAMETERS:                                                 %

%   model       =   ITU channel model                               %

%   fd          =   Maximum Doppler shift                           %

%   Ts          =   Simulation sampling time                        %

%                                                                   %

% OUTPUT VALUE:                                                     %

%   h   =   complex channel taps                                    %

%                                                                   %

% Author: Marco Hernandez, v1.0                                     %

% ***************************************************************** %

function [h] = ITUchannel(model,fd,Ts)

switch model

case 'office'

%  vector of path delays in sec

tau=[0 50 110 170 290 310]*1e-9;

% vector of average path power gains in dB

pdb=[0 -3 -10 -18 -26 -32];

case 'pedestrian'

tau=[0 110 190 410]*1e-9;

pdb=[0 -9.7 -19.2 -22.8];

case 'vehicular'

tau=[0 310 710 1090 1730 2510]*1e-9;

pdb=[0 -1 -9 -10 -15 -20];

otherwise

error('Model not implemented.');

end

% Object for Rayleigh channel with classic Doppler spectrum

h = rayleighchan(Ts,fd,tau,pdb);

% Choose flat Doppler spectrum for office scenario.

if strcmp(model,'office');

h.DopplerSpectrum=doppler.flat;

end

% The state of the channel object is not reset between calls.

% Hence, preserving continuity of the fading process across frames.

h.ResetBeforeFiltering=0;

end

## SISO ITU extended models

The previous channel models were extended to include bandwidths in the order of 20 MHz to reflect the fact that the characteristics of the radio channel frequency response are connected to the delay resolution. Also, such extended channel models are applied with Doppler shift of 5 Hz, 70 Hz and 300 Hz, which at a carrier frequency of 2.5 GHz, correspond to mobile velocities of 2 km/h, 30 km/h and 130 km/h respectively.

Table 12  Extended ITU outdoor to indoor and pedestrian

| Tap | Relative delay (ns) | Average power (dB) | Doppler spectrum |
| --- | --- | --- | --- |
| 1 | 0 | 0 | classic |
| 2 | 30 | -1 | classic |
| 3 | 70 | -2 | classic |
| 4 | 80 | -3 | classic |
| 5 | 110 | -8 | classic |
| 6 | 190 | -17.2 | classic |
| 7 | 410 | -20.8 | classic |

Table 13  Extended ITU vehicular

| Tap | Relative delay (ns) | Average power (dB) | Doppler spectrum |
| --- | --- | --- | --- |
| 1 | 0 | 0 | classic |
| 2 | 30 | -1.5 | classic |
| 3 | 150 | -1.4 | classic |
| 4 | 310 | -3.6 | classic |
| 5 | 370 | -0.6 | classic |
| 6 | 710 | -9.1 | classic |
| 7 | 1090 | -7 | classic |
| 8 | 1730 | -12 | classic |
| 9 | 2510 | -16.9 | classic |

Table 14  Extended ITU typical urban

| Tap | Relative delay (ns) | Average power (dB) | Doppler spectrum |
| --- | --- | --- | --- |
| 1 | 0 | -1 | classic |
| 2 | 50 | -1 | classic |
| 3 | 120 | -1 | classic |
| 4 | 200 | 0 | classic |
| 5 | 230 | 0 | classic |
| 6 | 500 | 0 | classic |
| 7 | 1600 | -3 | classic |
| 8 | 2300 | -5 | classic |
| 9 | 5000 | -7 | classic |

## MatLab™ code

The following scripts were written in MatLabR2011 and the Communications Toolbox.

%                                                                   %

% function [h] = ExtendedITUchannel(model,fd,Ts)                    %

%                                                                   %

% Return object for FIR channel taps of Extended ITU channel models %

%                                                                   %

% INPUT PARAMETERS:                                                 %

%   model       =   Extended ITU channel model                      %

%   fd          =   Maximum Doppler shift                           %

%   Ts          =   Simulation sampling time                        %

%                                                                   %

% OUTPUT VALUE:                                                     %

%   h   =   complex channel taps                                    %

%                                                                   %

% Author: Marco Hernandez, v1.0                                     %

% ***************************************************************** %

function [h] = ExtendedITUchannel(model,fd,Ts)

switch model

case 'pedestrian'

%  vector of path delays in sec

tau = [0 30 70 80 110 190 410]*1e-9;

% vector of average path power gains in dB

pdb = [0 -1 -2 -3 -8 -17.2 -20.8];

case 'vehicular'

tau=[0 30 150 310 370 710 1090 1730 2510]*1e-9;

pdb=[0 -1.5 -1.4 -3.6 -0.6 -9.1 -7 -12 -16.9];

case 'urban'

tau=[0 50 120 200 230 500 1600 2300 5000]*1e-9;

pdb=[-1 -1 -1 0 0 0 -3 -5 -7];

otherwise

error('Model not implemented.');

end

% Object for Rayleigh channel with classic Doppler spectrum

h = rayleighchan(Ts,fd,tau,pdb);

% The state of the channel object is not reset between calls.

% Hence, preserving continuity of the fading process across frames.

h.ResetBeforeFiltering=0;

end

Example of use:

Ts = 1e-6; % simulation sampling time: 1usec

fd = 100;  % maximum Doppler shift: 100 Hz

model='vehicular';

% Create an instance of object ExtendedITUchannel

h=ExtendedITUchannel(model,fd,Ts);

% How to use "h" in Monte Carlo simulation

Nframes = 1000;

Nsamp = 1000;

for i = 1:Nframes

input = ones(1,Nsamp);

y = filter(h,input);

end

## SISO ETSI BRAN 5 GHz channel model

For the development of the standard HiperLAN2, the ETSI BRAN Group worked out channel models in the 5 GHz band. There are five scenarios:

Table 15   5 GHz HiperLAN2 channel models

| Model | RMS delay spread (ns) | Rice factor 1st tap (dB) | Scenario |
| --- | --- | --- | --- |
| A | 50 | 0 | Office |
| B | 100 | 0 | Open space NLOS/ office large delay spread |
| C | 150 | 0 | Large open space indoor/outdoor large delay spread |
| D | 140 | 10 | Large open space indoor LOS/outdoor |
| E | 250 | 0 | Large open space indoor/outdoor large delay spread |

Table 16  Model A – office scenario NLOS and 50ns average rms delay spread.

| Tap | Relative delay (ns) | Average power (dB) | Rice factor | Doppler spectrum |
| --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | classic |
| 2 | 10 | -0.9 | 0 | classic |
| 3 | 20 | -1.7 | 0 | classic |
| 4 | 30 | -2.6 | 0 | classic |
| 5 | 40 | -3.5 | 0 | classic |
| 6 | 50 | -4.3 | 0 | classic |
| 7 | 60 | -5.2 | 0 | classic |
| 8 | 70 | -6.1 | 0 | classic |
| 9 | 80 | -6.9 | 0 | classic |
| 10 | 90 | -7.8 | 0 | classic |
| 11 | 110 | -4.7 | 0 | classic |
| 12 | 140 | -7.3 | 0 | classic |
| 13 | 170 | -9.9 | 0 | classic |
| 14 | 200 | -12.5 | 0 | classic |
| 15 | 240 | -13.7 | 0 | classic |
| 16 | 290 | -18 | 0 | classic |
| 17 | 340 | -22.4 | 0 | classic |
| 18 | 390 | -26.7 | 0 | classic |

Table 17  Model B -large open space and office NLOS and 100ns average rms delay spread

| Tap | Relative delay (ns) | Average power (dB) | Rice factor | Doppler spectrum |
| --- | --- | --- | --- | --- |
| 1 | 0 | -2.6 | 0 | classic |
| 2 | 10 | -3 | 0 | classic |
| 3 | 20 | -3.5 | 0 | classic |
| 4 | 30 | -3.9 | 0 | classic |
| 5 | 50 | 0 | 0 | classic |
| 6 | 80 | -1.3 | 0 | classic |
| 7 | 110 | -2.6 | 0 | classic |
| 8 | 140 | -3.9 | 0 | classic |
| 9 | 180 | -3.4 | 0 | classic |
| 10 | 230 | -5.6 | 0 | classic |
| 11 | 280 | -7.7 | 0 | classic |
| 12 | 330 | -9.9 | 0 | classic |
| 13 | 380 | -12.1 | 0 | classic |
| 14 | 430 | -14.3 | 0 | classic |
| 15 | 490 | -15.4 | 0 | classic |
| 16 | 560 | -18.4 | 0 | classic |
| 17 | 640 | -20.7 | 0 | classic |
| 18 | 730 | -24.6 | 0 | classic |

Table 18  Model C –large open space NLOS and 150 ns rms delay spread

| Tap | Relative delay (ns) | Average power (dB) | Rice factor | Doppler spectrum |
| --- | --- | --- | --- | --- |
| 1 | 0 | -3.3 | 0 | classic |
| 2 | 10 | -3.6 | 0 | classic |
| 3 | 20 | -3.9 | 0 | classic |
| 4 | 30 | -4.2 | 0 | classic |
| 5 | 50 | 0 | 0 | classic |
| 6 | 80 | -0.9 | 0 | classic |
| 7 | 110 | -1.7 | 0 | classic |
| 8 | 140 | -2.6 | 0 | classic |
| 9 | 180 | -1.5 | 0 | classic |
| 10 | 230 | -3 | 0 | classic |
| 11 | 280 | -4.4 | 0 | classic |
| 12 | 330 | -5.9 | 0 | classic |
| 13 | 400 | -5.3 | 0 | classic |
| 14 | 490 | -7.9 | 0 | classic |
| 15 | 600 | -9.4 | 0 | classic |
| 16 | 730 | -13.2 | 0 | classic |
| 17 | 880 | -16.3 | 0 | classic |
| 18 | 1050 | -21.2 | 0 | classic |

Table 19 Model D –same as Model C but LOS

| Tap | Relative delay (ns) | Average power (dB) | Rice factor | Doppler spectrum |
| --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 10 | classic |
| 2 | 10 | -10 | 0 | classic |
| 3 | 20 | -10.3 | 0 | classic |
| 4 | 30 | -10.6 | 0 | classic |
| 5 | 50 | -6.4 | 0 | classic |
| 6 | 80 | -7.2 | 0 | classic |
| 7 | 110 | -8.1 | 0 | classic |
| 8 | 140 | -9 | 0 | classic |
| 9 | 180 | -7.9 | 0 | classic |
| 10 | 230 | -9.4 | 0 | classic |
| 11 | 280 | -10.8 | 0 | classic |
| 12 | 330 | -12.3 | 0 | classic |
| 13 | 400 | -11.7 | 0 | classic |
| 14 | 490 | -14.3 | 0 | classic |
| 15 | 600 | -15.8 | 0 | classic |
| 16 | 730 | -19.6 | 0 | classic |
| 17 | 880 | -22.7 | 0 | classic |
| 18 | 1050 | -27.6 | 0 | classic |

Table 20  Model E –large open space NLOS and 250 ns rms delay spread

| Tap | Relative delay (ns) | Average power (dB) | Rice factor | Doppler spectrum |
| --- | --- | --- | --- | --- |
| 1 | 0 | -4.9 | 0 | classic |
| 2 | 10 | -5.1 | 0 | classic |
| 3 | 20 | -5.2 | 0 | classic |
| 4 | 40 | -0.8 | 0 | classic |
| 5 | 70 | -1.3 | 0 | classic |
| 6 | 100 | -1.9 | 0 | classic |
| 7 | 140 | -0.3 | 0 | classic |
| 8 | 190 | -1.2 | 0 | classic |
| 9 | 240 | -2.1 | 0 | classic |
| 10 | 320 | 0 | 0 | classic |
| 11 | 430 | -1.9 | 0 | classic |
| 12 | 560 | -2.8 | 0 | classic |
| 13 | 710 | -5.4 | 0 | classic |
| 14 | 880 | -7.3 | 0 | classic |
| 15 | 1070 | -10.6 | 0 | classic |
| 16 | 1280 | -13.4 | 0 | classic |
| 17 | 1510 | -17.4 | 0 | classic |
| 18 | 1760 | -20.9 | 0 | classic |

## MatLab™ code

The following scripts are written in MatLabR2011 and the Communications Toolbox.

%                                                                   %

% function [h] = ETSIchannel(model,fd,Ts)                           %

%                                                                   %

% Return object for FIR channel taps of ETSI channel models         %

%                                                                   %

% INPUT PARAMETERS:                                                 %

%   model       =   ETSI channel model                              %

%   fd          =   Maximum Doppler shift                           %

%   Ts          =   Simulation sampling time                        %

%                                                                   %

% OUTPUT VALUE:                                                     %

%   h   =   complex channel taps                                    %

%                                                                   %

% Author: Marco Hernandez, v1.0                                     %

% ***************************************************************** %

function [h] = ETSIchannel(model,fd,Ts)

switch model

case 'A'

%  vector of path delays in sec

tau=[0 10 20 30 40 50 60 70 80 90 110 140 170 200 240 290 340 390]*1e-9;

% vector of average path power gains in dB

pdb=[0 -0.9 -1.7 -2.6 -3.5 -4.3 -5.2 -6.1 -6.9 -7.8 -4.7 -7.3 -9.9 -12.5 -13.7 -18 -22.4 -26.7];

% K factor of Rician model per path

kFactor=zeros(1,18);

case 'B'

tau=[0 10 20 30 50 80 110 140 180 230 280 330 380 430 490 560 640 730]*1e-9;

pdb=[-2.6 -3 -3.5 -3.9 0 -1.3 -2.6 -3.9 -3.4 -5.6 -7.7 -9.9 -12.1 -14.3 -15.4 -18.4 -20.7 -24.6];

kFactor=zeros(1,18);

case 'C'

tau=[0 10 20 30 50 80 110 140 180 230 280 330 400 490 600 730 880 1050]*1e-9;

pdb=[-3.3 -3.6 -3.9 -4.2 0 -0.9 -1.7 -2.6 -1.5 -3 -4.4 -5.9 -5.3 -7.9 -9.4 -13.2 -16.3 -21.2];

kFactor=zeros(1,18);

case 'D'

tau=[0 10 20 30 50 80 110 140 180 230 280 330 400 490 600 730 880 1050]*1e-9;

pdb=[0 -10 -10.3 -10.6 -6.4 -7.2 -8.1 -9 -7.9 -9.4 -10.8 -12.3 -11.7 -14.3 -15.8 -19.6 -22.7 -27.6];

kFactor=zeros(1,18); kFactor(1)=10;

case 'E'

tau=[0 10 20 40 70 100 140 190 240 320 430 560 710 880 1070 1280 1510 1760]*1e-9;

pdb=[-4.9 -5.1 -5.2 -0.8 -1.3 -1.9 -0.3 -1.2 -2.1 0 -1.9 -2.8 -5.4 -7.3 -0.6 -13.4 -17.4 -20.9];

kFactor=zeros(1,18);

otherwise

error('Model not implemented.');

end

% Object for Rician channel with classic Doppler spectrum

h = ricianchan(Ts,fd,kFactor,tau,pdb);

% The state of the channel object is not reset between calls.

% Hence, preserving continuity of the fading process across frames.

h.ResetBeforeFiltering=0;

end

## UWB Channel model

The UWB channel impulse responses are taken from the channel models document of former IEEE802.15.4a TG. Scenarios, bandwidths and central frequencies are similar to the ones targeted for IEEE802.15.8 TG. The MatLab code is attach in Annex A.

# MIMO channel models

Figure 4  Multipath signals in a uniform linear array.

The incredible versatility and performance gains of MIMO systems have produce many MIMO channel models in the last years. Such MIMO channel models can be classified into 2 main groups: physical models and analytical models.

- Physical models characterize the double-directional impulse responses h(t,τ,ΩtΩr), based on physical wave propagation: deterministic (ray-tracing), stochastic, geometry-based, geometry-stochastic based, etc.

- Analytical models describe the MIMO channel matrix H(t,τ) in function of spatial and time correlations properties.

Both approaches have advantages and disadvantages. In one hand physical models have a sophisticated approach to the propagation between multiple antennas and their surroundings. However, they do not have analytical expressions that allow a PHY design (space-time coding, beamforming, signal processing for instance), besides of being very computationally intensive. On the other hand, analytical models are relative simple to use for design and to implement for simulations at the PHY level. However, they are a formalism and they do not give propagation quantitative information.

The ITU guidelines for the evaluation of advanced radio technologies ITU-R-2135-1 uses a physical model: geometry-stochastic channel model (GSCM). It can be used to simulate large scale fading and small scale fading in a network deployment.

Caveat: a typical scenario, say 57 access points with 10 devices per access point requires 57*57*10=32,490 links. An antenna configuration 4x4 means 16 antenna pairs per link. Up to 24 clusters between antenna pairs with 20 rays per cluster results in 32,490*16*24*20=249,423,200 channel values to compute per simulation sampling time (kTs).  We need an efficient implementation.

However, we recommend the possibility to use such GSCM approach for evaluation of typical TG8 scenarios, once TG8 consolidates at least a PHY (including a MAC in an ideal World).

For call for proposal, we recommend to use an analytical model (described below), enabling proposers to design signal processing techniques, including detection/decoding, for diversity schemes such as space-time coding, beamforming, spatial multiplexing, etc., for their proposals.

## Correlated MIMO fading channel

Figure 5  Schematic diagram of the MIMO correlation model.

A typical analytical model describes the MIMO channel matrix as a function of a random Gaussian fading matrix shaped by spatial and temporal correlations. Such correlation values depend on the antenna array structure, environment, power delay profile (PDP), power azimuth spectrum (PAS) and Doppler spectrum. Those values are obtained after intense characterization of measurement campaigns.

The spatial correlation matrix is defined as

R=E [vec(H)Hvec(HH)H]               Equation 17

where R is a NtNr x NtNr positive semi-definite Hermitian matrix that describes the correlation between all pairs of transmit-receive channels, Nt is the number of antennas at transmitter, Nr is the number of antennas at receiver.

Considering the channel between antenna pairs as a sum of a large number of contributions with random and independent phases, directions of departure and directions of arrival, each individual channel is modeled as a zero-mean complex circularly symmetric Gaussian variable.

Consequently, the correlation matrix R constitutes a sufficient description of the stochastic behavior of the MIMO channel. A channel realization is given by

vec(H'H)=Rvec(HwH)              Equation 18

where Hw is one realization of i.i.d complex circularly symmetric Gaussian matrix.

Furthermore,               Equation 18 is simplified by the so-called Kronecker model or correlation separability:R=Rr⊗Rt where Rt=E [HHH]and Rr=E [(HHH)T]are the transmit and receive correlation matrices, respectively. This model is valid when the transmit and receive correlation values are independent of the considered receive and transmit antenna respectively, irrespective of antenna configurations and intra-array spacings and if all DoDs couple with the same profile into all DoAs and vice-versa.

Finally, under the Kronecker model, the MIMO channel matrix can be expressed as

H'=RrHwRt                            Equation 19

This greatly simplifies expressions like mutual information, error probability, etc., besides it allows for separate transmit and receive optimizations.

Correlation matrices Rt and Rr are provided by typical scenarios and antenna configurations targeted for practical applications found in [8] (after many characterizations of measurement campaigns) and the Kroneker model has been implemented in MatLab already.

Table 21   Transmit correlation matrix

|  | One antenna | Two antennas | Four antennas |
| --- | --- | --- | --- |
| Tx correlation | Rt=1 | Rt=1αα1 | Rt=1α19α49αα(19)1α19α49α(49)α(19)1α19αα(49)α(19)1 |

Table 22  Receive correlation matrix

|  | One antenna | Two antennas | Four antennas |
| --- | --- | --- | --- |
| Rx correlation | Rr=1 | Rr=1ββ1 | Rr=1β19β49ββ(19)1β19β49β(49)β(19)1β19ββ(49)β(19)1 |

Table 23   Correlation values

| Low correlation |  | Medium correlation |  | High correlation |  |
| --- | --- | --- | --- | --- | --- |
| α | β | α | β | α | β |
| 0 | 0 | 0.3 | 0.9 | 0.9 | 0.9 |

### Simulation methodology

The power delay profiles are taken from the extended ITU channel models described in clause 3.3 with Doppler spectrum characterized by the Jakes spectrum shape and maximum Doppler frequency shift, as suggested in [8]. The spatial correlation is described in clause 4.1.  Figure 6 shows the schematic flow for simulation of Kronecker model, already implemented in Matlab Simulink. In Figure 7, the correlated MIMO channel simulation is delineated.

Figure 6  Schematic flow of simulation for the Kronecker model.

where σαij2(q)is the power of the channel coefficient of qth tap resulting between the coupling of ith antenna at transmitter to jth antenna at receiver.

Figure 7  MIMO channel simulation methodology.

## MatLab™ code

The following scripts were written in MatLabR2011 and the Communications Toolbox. It exploits the new interface between Matlab and Simulink for optimizing speed.

%                                                                   %

% function [H] = ExtendedITU_MIMOChannel(model,correlation,         %

%                                        configuration,fd,fs)       %

%                                                                   %

% Return object for FIR taps of Extended ITU MIMO channel models    %

%                                                                   %

% INPUT PARAMETERS:                                                 %

%   model         = Extended ITU channel model                      %

%   correlation   = Spatial correlation level                       %

%   configuration = Antenna configuration                           %

%   fd            = Maximum Doppler shift in Hz                     %

%   fs            = Simulation sampling frequency in Hz             %

%                                                                   %

% OUTPUT VALUE:                                                     %

%   H   =   complex channel taps                                    %

%                                                                   %

% Author: Marco Hernandez, v1.0                                     %

% ***************************************************************** %

function [H] = ExtendedITU_MIMOChannel(model,correlation,configuration,fd,fs)

switch model

case 'pedestrian'

%  vector of path delays in sec

tau = [0 30 70 80 110 190 410]*1e-9;

% vector of average path power gains in dB

pdb = [0 -1 -2 -3 -8 -17.2 -20.8];

case 'vehicular'

tau=[0 30 150 310 370 710 1090 1730 2510]*1e-9;

pdb=[0 -1.5 -1.4 -3.6 -0.6 -9.1 -7 -12 -16.9];

case 'urban'

tau=[0 50 120 200 230 500 1600 2300 5000]*1e-9;

pdb=[-1 -1 -1 0 0 0 -3 -5 -7];

otherwise

error('Model not implemented.');

end

switch correlation

case 'low'

a=0; b=0;

case 'medium'

a=0.3; b=0.9;

case 'high'

a=0.9; b=0.9;

otherwise

error('Spatial correlation profile not implemented.');

end

switch configuration

case '1x2'

Nt=1; Nr=2; Rt=1; Rr=[1 b;b 1];

case '2x1'

Nt=2; Nr=1; Rt=[1 a;a 1]; Rr=1;

case '2x2'

Nt=2; Nr=2; Rt=[1 a;a 1]; Rr=[1 b;b 1];

case '4x2'

Nt=4; Nr=2; Rt=[1 a^(1/9) a^(4/9) a;a^(1/9) 1 a^(1/9) a^(4/9);a^(4/9) a^(1/9) 1 a^(1/9);a a^(4/9) a^(1/9) 1];

Rr=[1 b;b 1];

case '2x4'

Nt=2; Nr=4; Rt=[1 a;a 1];

Rr=[1 b^(1/9) b^(4/9) b;b^(1/9) 1 b^(1/9) b^(4/9);b^(4/9) b^(1/9) 1 b^(1/9);b b^(4/9) b^(1/9) 1];

case '4x4'

Nt=4; Nr=4; Rt=[1 a^(1/9) a^(4/9) a;a^(1/9) 1 a^(1/9) a^(4/9);a^(4/9) a^(1/9) 1 a^(1/9);a a^(4/9) a^(1/9) 1];

Rr=[1 b^(1/9) b^(4/9) b;b^(1/9) 1 b^(1/9) b^(4/9);b^(4/9) b^(1/9) 1 b^(1/9);b b^(4/9) b^(1/9) 1];

otherwise

error('Antenna configuration not implemented.');

end

% Object MIMO multipath fading channel

H=comm.MIMOChannel(...

'SampleRate',                   fs,...

'PathDelays',                   tau,...

'AveragePathGains',             pdb,...

'MaximumDopplerShift',          fd,...

'DopplerSpectrum',              doppler.jakes,...

'NumTransmitAntennas',          Nt,...

'NumReceiveAntennas',           Nr,...

'TransmitCorrelationMatrix',    Rt,...

'ReceiveCorrelationMatrix',     Rr,...

'FadingDistribution',           'Rayleigh',...

'RandomStream',                 'mt19937ar with seed',...

'Seed',                         99,...

'NormalizePathGains',           true,...

'NormalizeChannelOutputs',      true,...

'PathGainsOutputPort',          false );

end

The previous MIMO channel model is used intensively by the industry for testing of implementation designs in the 2 GHz band, like LTE Advanced.  Regarding the 900 MHz band and 5 GHz band, we recommend using the same approach. That is, taking the power delay profiles of the ITU channel models in clause 3.1 and the ETSI BRAN channel models in clause 3.5 and to use the same typical spatial correlation values to construct the spatial correlation matrix in the Kronecker method, and so an extension to create a MIMO channel model in those frequency bands.

%                                                                   %

% function [H] = ITU_MIMOChannel(model,correlation,                 %

%                                        configuration,fd,fs)       %

%                                                                   %

% Return object for FIR taps of ITU MIMO channel models             %

%                                                                   %

% INPUT PARAMETERS:                                                 %

%   model         = ITU channel model                               %

%   correlation   = Spatial correlation level                       %

%   configuration = Antenna configuration                           %

%   fd            = Maximum Doppler shift in Hz                     %

%   fs            = Simulation sampling frequency in Hz             %

%                                                                   %

% OUTPUT VALUE:                                                     %

%   H   =   complex channel taps                                    %

%                                                                   %

% Author: Marco Hernandez, v1.0                                     %

% ***************************************************************** %

function [H] = ITU_MIMOChannel(model,correlation,configuration,fd,fs)

switch model

case 'office'

%  vector of path delays in sec

tau=[0 50 110 170 290 310]*1e-9;

% vector of average path power gains in dB

pdb=[0 -3 -10 -18 -26 -32];

case 'pedestrian'

tau=[0 110 190 410]*1e-9;

pdb=[0 -9.7 -19.2 -22.8];

case 'vehicular'

tau=[0 310 710 1090 1730 2510]*1e-9;

pdb=[0 -1 -9 -10 -15 -20];

otherwise

error('Model not implemented.');

end

switch correlation

case 'low'

a=0; b=0;

case 'medium'

a=0.9; b=0.3;

case 'high'

a=0.9; b=0.9;

otherwise

error('Spatial correlation profile not implemented.');

end

switch configuration

case '1x2'

Nt=1; Nr=2; Rt=1; Rr=[1 b;b 1];

case '2x1'

Nt=2; Nr=1; Rt=[1 a;a 1]; Rr=1;

case '2x2'

Nt=2; Nr=2; Rt=[1 a;a 1]; Rr=[1 b;b 1];

case '4x2'

Nt=4; Nr=2; Rt=[1 a^(1/9) a^(4/9) a;a^(1/9) 1 a^(1/9) a^(4/9);a^(4/9) a^(1/9) 1 a^(1/9);a a^(4/9) a^(1/9) 1];

Rr=[1 b;b 1];

case '2x4'

Nt=2; Nr=4; Rt=[1 a;a 1];

Rr=[1 b^(1/9) b^(4/9) b;b^(1/9) 1 b^(1/9) b^(4/9);b^(4/9) b^(1/9) 1 b^(1/9);b b^(4/9) b^(1/9) 1];

case '4x4'

Nt=4; Nr=4; Rt=[1 a^(1/9) a^(4/9) a;a^(1/9) 1 a^(1/9) a^(4/9);a^(4/9) a^(1/9) 1 a^(1/9);a a^(4/9) a^(1/9) 1];

Rr=[1 b^(1/9) b^(4/9) b;b^(1/9) 1 b^(1/9) b^(4/9);b^(4/9) b^(1/9) 1 b^(1/9);b b^(4/9) b^(1/9) 1];

otherwise

error('Antenna configuration not implemented.');

end

% Object MIMO multipath fading channel

H=comm.MIMOChannel(...

'SampleRate',                   fs,...

'PathDelays',                   tau,...

'AveragePathGains',             pdb,...

'MaximumDopplerShift',          fd,...

'DopplerSpectrum',              doppler.jakes,...

'NumTransmitAntennas',          Nt,...

'NumReceiveAntennas',           Nr,...

'TransmitCorrelationMatrix',    Rt,...

'ReceiveCorrelationMatrix',     Rr,...

'FadingDistribution',           'Rayleigh',...

'RandomStream',                 'mt19937ar with seed',...

'Seed',                         99,...

'NormalizePathGains',           true,...

'NormalizeChannelOutputs',      true,...

'PathGainsOutputPort',          false );

end

%                                                                   %

% function [H] = ETSI_MIMOChannel(model,correlation,                %

%                                        configuration,fd,fs)       %

%                                                                   %

% Return object for FIR taps of ETSI MIMO channel models            %

%                                                                   %

% INPUT PARAMETERS:                                                 %

%   model         = ETSI channel model                              %

%   correlation   = Spatial correlation level                       %

%   configuration = Antenna configuration                           %

%   fd            = Maximum Doppler shift in Hz                     %

%   fs            = Simulation sampling frequency in Hz             %

%                                                                   %

% OUTPUT VALUE:                                                     %

%   H   =   complex channel taps                                    %

%                                                                   %

% Author: Marco Hernandez, v1.0                                     %

% ***************************************************************** %

function [H] = ETSI_MIMOChannel(model,correlation,configuration,fd,fs)

% K factor of Rician model per path gain

kFactor=zeros(1,18);

% Direct path Doppler shift

directPathDoppler=zeros(1,18);

% Direct path initial phase

directpathPhase=zeros(1,18);

switch model

case 'A'

%  vector of path delays in sec

tau=[0 10 20 30 40 50 60 70 80 90 110 140 170 200 240 290 340 390]*1e-9;

% vector of average path power gains in dB

pdb=[0 -0.9 -1.7 -2.6 -3.5 -4.3 -5.2 -6.1 -6.9 -7.8 -4.7 -7.3 -9.9 -12.5 -13.7 -18 -22.4 -26.7];

case 'B'

tau=[0 10 20 30 50 80 110 140 180 230 280 330 380 430 490 560 640 730]*1e-9;

pdb=[-2.6 -3 -3.5 -3.9 0 -1.3 -2.6 -3.9 -3.4 -5.6 -7.7 -9.9 -12.1 -14.3 -15.4 -18.4 -20.7 -24.6];

case 'C'

tau=[0 10 20 30 50 80 110 140 180 230 280 330 400 490 600 730 880 1050]*1e-9;

pdb=[-3.3 -3.6 -3.9 -4.2 0 -0.9 -1.7 -2.6 -1.5 -3 -4.4 -5.9 -5.3 -7.9 -9.4 -13.2 -16.3 -21.2];

case 'D'

tau=[0 10 20 30 50 80 110 140 180 230 280 330 400 490 600 730 880 1050]*1e-9;

pdb=[0 -10 -10.3 -10.6 -6.4 -7.2 -8.1 -9 -7.9 -9.4 -10.8 -12.3 -11.7 -14.3 -15.8 -19.6 -22.7 -27.6];

kFactor(1)=10; directPathDoppler(1)=fd;

case 'E'

tau=[0 10 20 40 70 100 140 190 240 320 430 560 710 880 1070 1280 1510 1760]*1e-9;

pdb=[-4.9 -5.1 -5.2 -0.8 -1.3 -1.9 -0.3 -1.2 -2.1 0 -1.9 -2.8 -5.4 -7.3 -0.6 -13.4 -17.4 -20.9];

otherwise

error('Model not implemented.');

end

switch correlation

case 'low'

a=0; b=0;

case 'medium'

a=0.9; b=0.3;

case 'high'

a=0.9; b=0.9;

otherwise

error('Spatial correlation profile not implemented.');

end

switch configuration

case '1x2'

Nt=1; Nr=2; Rt=1; Rr=[1 b;b 1];

case '2x1'

Nt=2; Nr=1; Rt=[1 a;a 1]; Rr=1;

case '2x2'

Nt=2; Nr=2; Rt=[1 a;a 1]; Rr=[1 b;b 1];

case '4x2'

Nt=4; Nr=2; Rt=[1 a^(1/9) a^(4/9) a;a^(1/9) 1 a^(1/9) a^(4/9);a^(4/9) a^(1/9) 1 a^(1/9);a a^(4/9) a^(1/9) 1];

Rr=[1 b;b 1];

case '2x4'

Nt=2; Nr=4; Rt=[1 a;a 1];

Rr=[1 b^(1/9) b^(4/9) b;b^(1/9) 1 b^(1/9) b^(4/9);b^(4/9) b^(1/9) 1 b^(1/9);b b^(4/9) b^(1/9) 1];

case '4x4'

Nt=4; Nr=4; Rt=[1 a^(1/9) a^(4/9) a;a^(1/9) 1 a^(1/9) a^(4/9);a^(4/9) a^(1/9) 1 a^(1/9);a a^(4/9) a^(1/9) 1];

Rr=[1 b^(1/9) b^(4/9) b;b^(1/9) 1 b^(1/9) b^(4/9);b^(4/9) b^(1/9) 1 b^(1/9);b b^(4/9) b^(1/9) 1];

otherwise

error('Antenna configuration not implemented.');

end

% Object MIMO multipath fading channel

H=comm.MIMOChannel(...

'SampleRate',                   fs,...

'PathDelays',                   tau,...

'AveragePathGains',             pdb,...

'MaximumDopplerShift',          fd,...

'DopplerSpectrum',              doppler.jakes,...

'NumTransmitAntennas',          Nt,...

'NumReceiveAntennas',           Nr,...

'TransmitCorrelationMatrix',    Rt,...

'ReceiveCorrelationMatrix',     Rr,...

'FadingDistribution',           'Rician',...

'KFactor',                      kFactor,...

'DirectPathDopplerShift',       directPathDoppler,...

'DirectPathInitialPhase',       directpathPhase,...

'RandomStream',                 'mt19937ar with seed',...

'Seed',                         99,...

'NormalizePathGains',           true,...

'NormalizeChannelOutputs',      true,...

'PathGainsOutputPort',          false );

end

Example of use

clc;

close all;

clear all;

% Simulation sampling rate: 30.72 MHz

fs = 30.72e6;

% Maximum Doppler shift: 100 Hz

fd = 100;

% Power delay profile scenario

model='vehicular';

% Spatial correlation profile

correlation='medium';

% MIMO antenna configuration

configuration='2x2';

% Create an instance of object ExtendedITUchannel_MIMOChannel

H=ExtendedITU_MIMOChannel(model,correlation,configuration,fd,fs);

% 8-PSK modulator object; Gray enconding and integer symbols input

hMod=comm.PSKModulator;

% Random symbols

data=randi([0 hMod.ModulationOrder-1],2e3,1);

% Modulate data

modData=step(hMod,data);

% Format for input data matrix of MIMO channel object:

% rows = No of modulated data symbols per antenna

% cols = No of antennas

% Split modulated data into 2 streams (1000 symbols x 2 antennas)

channelInput=reshape(modData, [2, 1e3]).';

% Filter the modulated data using ExtendedITU_MIMOChannel object: H

ChanOut=step(H,channelInput);

% If you need to reset the states of the MIMOChannel object H (it resets the

% filters and reinitializes the random number stream to the value of the

% ‘Seed’ property.

% reset(H);

% Add AWGN, etc...

### Beam steering example

Once generated a random MIMO channel matrix H with configuration 4x4 or 4x2, a beam steering approach may be generated as

y=HDWx+n

D=10000ejθk0000ej2θk0000ej3θk

where D is the steering matrix, θk=θ0+kΔθ, W is a precoding matrix, y is the received signal, x is the transmitted signal and n is AWGN.

# Bibliography

[1] Recommendation ITU-R P.1411-6 Propagation data and prediction methods for the planning of short-range outdoor radio communication systems and radio local area networks in the frequency range 300 MHz to 100 GHz.

[2] Seidel S., Rappaport T., “914 MHz Path Loss Prediction Models for Indoor Wireless Communications in Multifloor Buildings”, IEEE Transactions on Antennas and Propagation, Vol 40, No 2, February 1992, pp207-217.

[3] Recommendation ITU-R P.1238-7 Propagation data and prediction methods for the planning of indoor radio communication systems and radio local area networks in the frequency range 900 MHz to 100 GHz.

[4]  IEEE802.15-04-0662-04-004a-channel-model-final-report-r1

[5] Recommendation ITU-R M. 1225 Guidelines for evaluation of radio transmission technologies for IMT-2000.

[6]  T.B. Sørensen, P.E. Mogensen, and F. Frederiksen, “Extension of the ITU Channel Models for Wideband (OFDM) Systems”, in Proc. IEEE Vehicular Technology Conf., Dallas, USA, Sept. 2005.

[7] ETSI BRAN, 3ER1085B, Channel models for HiperLAN2

[8] 3GPP TS 36.104, Evolved Universal Terrestrial Radio Access; Base Station radio transmission and reception

[9] 3GPP TS 36.101, Evolved Universal Terrestrial Radio Access; User Equipment radio transmission and reception

[10]  K. Yu, M. Bengtsson, et.al., “A wideband statistical model for nlos indoor mimo channels”, VTC Spring, Birmingham, AL, USA, May 2002.

[11] J. Philippe, L. Schumacher, et.al., “A Stochastic MIMO Radio Channel Model with Experimental Validation”, IEEE Journal on Selected Areas in Communications, Vol 20, No 6, August 2002.

# Annex A – Matlab code for IEEE802.15.4a UWB channel model

% modified S-V channel model evaluation

%%Written by Sun Xu, Kim Chee Wee, B. Kannan & Francois Chin on 22/02/2005

clear;

no_output_files = 1; % non-zero: avoids writing output files of continuous-time responses

num_channels = 100; % number of channel impulse responses to generate

randn(’state’,12); % initialize state of function for repeatability

rand(’state’,12); % initialize state of function for repeatability

cm_num = 6; % channel model number from 1 to 8

% get channel model params based on this channel model number

[Lam,Lmean,lambda_mode,lambda_1,lambda_2,beta,Gam,gamma_0,Kgamma, ...

sigma_cluster,nlos,gamma_rise,gamma_1,chi,m0,Km,sigma_m0,sigma_Km, ...

sfading_mode,m0_sp,std_shdw,kappa,fc,fs] = uwb_sv_params_15_4a( cm_num );

fprintf(1,[’Model Parameters\n’ ...

’ Lam = %.4f, Lmean = %.4f, lambda_mode(FLAG) = %d\n’ ...

’ lambda_1 = %.4f, lambda_2 = %.4f, beta = %.4f\n’ ...

’ Gam = %.4f, gamma0 = %.4f, Kgamma = %.4f, sigma_cluster = %.4f\n’ ...

’ nlos(FLAG) = %d, gamma_rise = %.4f, gamma_1 = %.4f, chi = %.4f\n’ ...

’ m0 = %.4f, Km = %.4f, sigma_m0 = %.4f, sigma_Km = %.4f\n’ ...

’ sfading_mode(FLAG) = %d, m0_sp = %.4f, std_shdw = %.4f\n’, ...

’ kappa = %.4f, fc = %.4fGHz, fs = %.4fGHz\n’], ...

Lam,Lmean,lambda_mode,lambda_1,lambda_2,beta,Gam,gamma_0,Kgamma, ...

sigma_cluster,nlos,gamma_rise,gamma_1,chi,m0,Km,sigma_m0,sigma_Km,...

sfading_mode,m0_sp,std_shdw,kappa,fc,fs);

ts = 1/fs; % sampling frequency

% get a bunch of realizations (impulse responses)

[h_ct,t_ct,t0,np] = uwb_sv_model_ct_15_4a(Lam,Lmean,lambda_mode,lambda_1, ...

lambda_2,beta,Gam,gamma_0,Kgamma,sigma_cluster,nlos,gamma_rise,gamma_1, ...

chi,m0,Km,sigma_m0,sigma_Km,sfading_mode,m0_sp,std_shdw,num_channels,ts);

% change to complex baseband channel

h_ct_len = size(h_ct, 1);

phi = zeros(h_ct_len, 1);

for k = 1:num_channels

phi = rand(h_ct_len, 1).*(2*pi);

h_ct(:,k) = h_ct(:,k) .* exp(phi .* i);

end

% now reduce continuous-time result to a discrete-time result

[hN,N] = uwb_sv_cnvrt_ct_15_4a( h_ct, t_ct, np, num_channels, ts );

if N > 1,

h = resample(hN, 1, N); % decimate the columns of hN by factor N

else

h = hN;

end

% add the frequency dependency

[h]= uwb_sv_freq_depend_ct_15_4a(h,fc,fs,num_channels,kappa);

%********************************************************************

% Testing and ploting

%********************************************************************

% channel energy

channel_energy = sum(abs(h).^2);

h_len = length(h(:,1));

t = [0:(h_len-1)] * ts; % for use in computing excess & RMS delays

excess_delay = zeros(1,num_channels);

RMS_delay = zeros(1,num_channels);

num_sig_paths = zeros(1,num_channels);

num_sig_e_paths = zeros(1,num_channels);

for k=1:num_channels

% determine excess delay and RMS delay

sq_h = abs(h(:,k)).^2 / channel_energy(k);

t_norm = t - t0(k); % remove the randomized arrival time of first cluster

excess_delay(k) = t_norm * sq_h;

RMS_delay(k) = sqrt( ((t_norm-excess_delay(k)).^2) * sq_h );

% determine number of significant paths (paths within 10 dB from peak)

threshold_dB = -10; % dB

temp_h = abs(h(:,k));

temp_thresh = 10^(threshold_dB/20) * max(temp_h);

num_sig_paths(k) = sum(temp_h > temp_thresh);

% determine number of sig. paths (captures x % of energy in channel)

x = 0.85;

temp_sort = sort(temp_h.^2); % sorted in ascending order of energy

cum_energy = cumsum(temp_sort(end:-1:1)); % cumulative energy

index_e = min(find(cum_energy >= x * cum_energy(end)));

num_sig_e_paths(k) = index_e;

end

energy_mean = mean(10*log10(channel_energy));

energy_stddev = std(10*log10(channel_energy));

mean_excess_delay = mean(excess_delay);

mean_RMS_delay = mean(RMS_delay);

mean_sig_paths = mean(num_sig_paths);

mean_sig_e_paths = mean(num_sig_e_paths);

fprintf(1,’Model Characteristics\n’);

fprintf(1,’ Mean delays: excess (tau_m) = %.1f ns, RMS (tau_rms) = %1.f\n’, ...

mean_excess_delay, mean_RMS_delay);

fprintf(1,’ # paths: NP_10dB = %.1f, NP_85%% = %.1f\n’, ...

mean_sig_paths, mean_sig_e_paths);

fprintf(1,’ Channel energy: mean = %.1f dB, std deviation = %.1f dB\n’, ...

energy_mean, energy_stddev);

figure(1); clf; plot(t, abs(h)); grid on

title(’Impulse response realizations’)

xlabel(’Time (nS)’)

figure(2); clf; plot([1:num_channels], excess_delay, ’b-’, ...

[1 num_channels], mean_excess_delay*[1 1], ’r–’ );

grid on

title(’Excess delay (nS)’)

xlabel(’Channel number’)

figure(3); clf; plot([1:num_channels], RMS_delay, ’b-’, ...

[1 num_channels], mean_RMS_delay*[1 1], ’r–’ );

grid on

title(’RMS delay (nS)’)

xlabel(’Channel number’)

figure(4); clf; plot([1:num_channels], num_sig_paths, ’b-’, ...

[1 num_channels], mean_sig_paths*[1 1], ’r–’);

grid on

title(’Number of significant paths within 10 dB of peak’)

xlabel(’Channel number’)

figure(5); clf; plot([1:num_channels], num_sig_e_paths, ’b-’, ...

[1 num_channels], mean_sig_e_paths*[1 1], ’r–’);

grid on

title(’Number of significant paths capturing > 85% energy’)

xlabel(’Channel number’)

temp_average_power = sum((abs(h))’.*(abs(h))’, 1)/num_channels;

temp_average_power = temp_average_power/max(temp_average_power);

average_decay_profile_dB = 10*log10(temp_average_power);

threshold_dB = -40;

above_threshold = find(average_decay_profile_dB > threshold_dB);

ave_t = t(above_threshold);

apdf_dB = average_decay_profile_dB(above_threshold);

figure(6); clf; plot(ave_t, apdf_dB); grid on

title(’Average Power Decay Profile’)

xlabel(’Delay (nsec)’)

ylabel(’Average power (dB)’)

if no_output_files,

return

end

%**************************************************************************

%Savinge the data

%**************************************************************************

%%% save continuous-time (time,value) pairs to files

save_fn = sprintf(’cm%d_imr’, cm_num);

% A complete self-contained file for Matlab users

save([save_fn ’.mat’], ’t’, ’h’,’t_ct’, ’h_ct’, ’t0’, ’np’, ’num_channels’, ’cm_num’);

% Three comma-delimited text files for non-Matlab users:

% File #1: cmX_imr_np.csv lists the number of paths in each realization

dlmwrite([save_fn ’_np.csv’], np, ’,’); % number of paths

% File #2: cmX_imr_ct.csv can open with Excel

% n’th pair of columns contains the (time,value) pairs for the n’th realization

% save continous time data

th_ct = zeros(size(t_ct,1),3*size(t_ct,2));

th_ct(:,1:3:end) = t_ct; % time

th_ct(:,2:3:end) = abs(h_ct); % magnitude

th_ct(:,3:3:end) = angle(h_ct); % phase (radians)

fid = fopen([save_fn ’_ct.csv’], ’w’);

if fid < 0,

error(’unable to write .csv file for impulse response, file may be open in another application’);

end

for k = 1:size(th_ct,1)

fprintf(fid,’%.4f,%.6f,’, th_ct(k,1:end-2));

fprintf(fid,’%.4f,%.6f\r\n’, th_ct(k,end-1:end)); % \r\n for Windoze end-of-line

end

fclose(fid);

% File #3: cmX_imr_dt.csv can open with Excel

% discrete channel impulse response magnitude and phase pair realization.

% the first column is time. phase is in radians

% save discrete time data

th = zeros(size(h,1),2*size(h,2)+1);

th(:,1) = t’; % the first column is time scale

th(:,2:2:end) = abs(h); % even columns are magnitude

th(:,3:2:end) = angle(h); % odd columns are phase

fid = fopen([save_fn ’_dt.csv’], ’w’);

if fid < 0,

error(’unable to write .csv file for impulse response, file may be open in another application’);

end

for k = 1:size(th,1)

fprintf(fid,’%.4f,%.6f,’, th(k,1:end-2));

fprintf(fid,’%.4f,%.6f\r\n’, th(k,end-1:end)); % \r\n for Windoze end-of-line

end

fclose(fid);

return; % end of program

function [Lam,Lmean,lambda_mode,lambda_1,lambda_2,beta,Gam,gamma_0,Kgamma, ...

sigma_cluster,nlos,gamma_rise,gamma_1,chi,m0,Km,sigma_m0,sigma_Km, ...

sfading_mode,m0_sp,std_shdw,kappa,fc,fs] = uwb_sv_params_15_4a( cm_num )

% Written by Sun Xu, Kim Chee Wee, B. Kannan & Francois Chin on 22/02/2004

% Return modified S-V model parameters for standard UWB channel models

%————————————————————————–

% Lam Cluster arrival rate (clusters per nsec)

% Lmean Mean number of Clusters

% lambda_mode Flag for Mixture of poission processes for ray arrival times

% 1 -> Mixture of poission processes for the ray arrival times

% 2 -> tapped delay line model

% lambda_1 Ray arrival rate for Mixture of poisson processes (rays per nsec)

% lambda_2 Ray arrival rate for Mixture of poisson processes (rays per nsec)

% beta Mixture probability

%————————————————————————–

% Gam Cluster decay factor (time constant, nsec)

% gamma0 Ray decay factor (time constant, nsec)

% Kgamma Time dependence of ray decay factor

% sigma_cluster Standard deviation of normally distributed variable for cluster energy

% nlos Flag for non line of sight channel

% 0 -> LOS

% 1 -> NLOS with first arrival path starting at t ~= 0

% 2 -> NLOS with first arrival path starting at t = 0 and diffused first cluster

% gamma_rise Ray decay factor of diffused first cluster (time constant, nsec)

% gamma_1 Ray decay factor of diffused first cluster (time constant, nsec)

% chi Diffuse weight of diffused first cluster

%————————————————————————–

% m0 Mean of log-normal distributed nakagami-m factor

% Km Time dependence of m0

% sigma_m0 Standard deviation of log-normal distributed nakagami-m factor

% sigma_Km Time dependence of sigma_m0

% sfading_mode Flag for small-scale fading

% 0 -> All paths have same m-factor distribution

% 1 -> LOS first path has a deterministic large m-factor

% 2 -> LOS first path of each cluster has a deterministic

% large m-factor

% m0_sp Deterministic large m-factor

%————————————————————————–

% std_shdw Standard deviation of log-normal shadowing of entire impulse response

%————————————————————————–

% kappa Frequency dependency of the channel

%————————————————————————–

% fc Center Frequency

% fs Frequency Range

%%

modified by I2R

if cm_num == 1, % Residential LOS

% MPC arrival

Lam = 0.047; Lmean = 3;

lambda_mode = 1;

lambda_1 = 1.54; lambda_2 = 0.15; beta = 0.095;

%MPC decay

Gam = 22.61; gamma_0 = 12.53; Kgamma = 0; sigma_cluster = 2.75;

nlos = 0;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.67; Km = 0; sigma_m0 = 0.28; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN;

% Large-scale Fading – Shadowing

std_shdw = 2.22;

% Frequency Dependence

kappa = 1.12;

fc = 6; % GHz

fs = 8; % 2 - 10 GHz

elseif cm_num == 2, % Residential NLOS

% MPC arrival

Lam = 0.12; Lmean = 3.5;

lambda_mode = 1;

lambda_1 = 1.77; lambda_2 = 0.15; beta = 0.045;

%MPC decay

Gam = 26.27; gamma_0 = 17.5; Kgamma = 0; sigma_cluster = 2.93;

nlos = 1;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.69; Km = 0; sigma_m0 = 0.32; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN;

% Large-scale Fading – Shadowing

std_shdw = 3.51;

% Frequency Dependence

kappa = 1.53;

fc = 6; % GHz

fs = 8; % 2 - 10 GHz

elseif cm_num == 3, % Office LOS

% MPC arrival

Lam = 0.016; Lmean = 5.4;

lambda_mode = 1;

lambda_1 = 0.19; lambda_2 = 2.97; beta = 0.0184;

%MPC decay

Gam = 14.6; gamma_0 = 6.4; Kgamma = 0; sigma_cluster = 3; % assumption

nlos = 0;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.42; Km = 0; sigma_m0 = 0.31; sigma_Km = 0;

sfading_mode = 2; m0_sp = 3; % assumption

% Large-scale Fading – Shadowing

std_shdw = 0; %1.9;

% Frequency Dependence

kappa = 0.03;

fc = 6; % GHz

fs = 8; % 3 - 6 GHz

elseif cm_num == 4, % Office NLOS

% MPC arrival

Lam = 0.19; Lmean = 3.1;

lambda_mode = 1;

lambda_1 = 0.11; lambda_2 = 2.09; beta = 0.0096;

%MPC decay

Gam = 19.8; gamma_0 = 11.2; Kgamma = 0; sigma_cluster = 3; % assumption

nlos = 2;

gamma_rise = 15.21; gamma_1 = 11.84; chi = 0.78;

% Small-scale Fading

m0 = 0.5; Km = 0; sigma_m0 = 0.25; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN; % assumption

% Large-scale Fading – Shadowing

std_shdw = 3.9;

% Frequency Dependence

kappa =0.71;

fc = 6; % GHz

fs = 8; % 3 - 6 GHz

elseif cm_num == 5, % Outdoor LOS

% MPC arrival

Lam = 0.0448; Lmean = 13.6;

lambda_mode = 1;

lambda_1 = 0.13; lambda_2 = 2.41; beta = 0.0078;

%MPC decay

Gam = 31.7; gamma_0 = 3.7; Kgamma = 0; sigma_cluster = 3; % assumption

nlos = 0;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.77; Km = 0; sigma_m0 = 0.78; sigma_Km = 0;

sfading_mode = 2; m0_sp = 3; % assumption

% Large-scale Fading – Shadowing

std_shdw = 0.83;

% Frequency Dependence

kappa = 0.12;

fc = 6; % GHz

fs = 8; % 3 - 6 GHz

elseif cm_num == 6, % Outdoor NLOS

% MPC arrival

Lam = 0.0243; Lmean = 10.5;

lambda_mode = 1;

lambda_1 = 0.15; lambda_2 = 1.13; beta = 0.062;

%MPC decay

Gam = 104.7; gamma_0 = 9.3; Kgamma = 0; sigma_cluster = 3; % assumption

nlos = 1;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.56; Km = 0; sigma_m0 = 0.25; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN; % assumption

% Large-scale Fading – Shadowing

std_shdw = 2; % assumption

% Frequency Dependence

kappa = 0.13;

fc =6; % GHz

fs = 8; % 3 - 6 GHz

elseif cm_num == 7, % Industrial LOS

% MPC arrival

Lam = 0.0709; Lmean = 4.75;

lambda_mode = 2;

lambda_1 = 1; lambda_2 = 1; beta = 1; % dummy in this scenario

%MPC decay

Gam = 13.47; gamma_0 = 0.615; Kgamma = 0.926; sigma_cluster = 4.32;

nlos = 0;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.36; Km = 0; sigma_m0 = 1.13; sigma_Km = 0;

sfading_mode = 1; m0_sp = 12.99;

% Large-scale Fading – Shadowing

std_shdw = 6;

% Frequency Dependence

kappa = -1.103;

fc = 6; % GHz

fs = 8; % 2 - 8 GHz

elseif cm_num == 8, % Industrial NLOS

% MPC arrival

Lam = 0.089; Lmean = 1;

lambda_mode = 2;

lambda_1 = 1; lambda_2 = 1; beta = 1; % dummy in this scenario

%MPC decay

Gam = 5.83; gamma_0 = 0.3; Kgamma = 0.44; sigma_cluster = 2.88;

nlos = 2;

gamma_rise = 47.23; gamma_1 = 84.15; chi = 0.99;

% Small-scale Fading

m0 = 0.3; Km = 0; sigma_m0 = 1.15; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN; % m0_sp is assumption

% Large-scale Fading – Shadowing

std_shdw = 6;

% Frequency Dependence

kappa = -1.427;

fc = 6; % GHz

fs = 8; % 2 - 8 GHz

elseif cm_num == 9, % Open Outdoor Environment NLOS (Fram, Snow-Covered Open Area)

% MPC arrival

Lam = 0.0305; Lmean = 3.31;

lambda_mode = 1;

lambda_1 = 0.0225; lambda_2 = 1; beta = 1;

%MPC decay

Gam = 56; gamma_0 = 0.92; Kgamma = 0; sigma_cluster = 3; % sigma_cluster is assumption

nlos = 1;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN;

% Small-scale Fading

m0 = 4.1; Km = 0; sigma_m0 = 2.5; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN; % m0_sp is assumption

% Large-scale Fading – Shadowing

std_shdw = 3.96;

% Frequency Dependence

kappa = -1; % Kappa is assumption

fc = 6; % GHz

fs = 8; % 2 - 8 GHz

else

error(’cm_num is wrong!!’)

end

return

function [h]= uwb_sv_freq_depend_ct_15_4a(h,fc,fs,num_channels,kappa)

% This function is used to include the frequency dependency

f0 = 5; % GHz

h_len = length(h(:,1));

f = [fc-fs/2 : fs/h_len/2 : fc+fs/2]./f0;

f = f.^(-2*(kappa));

f = [f(h_len : 2*h_len), f(1 : h_len-1)]’;

i = (-1)^(1/2); % complex i

for c = 1:num_channels

% add the frequency dependency

h2 = zeros(2*h_len, 1);

h2(1 : h_len) = h(:,c); % zero padding

fh2 = fft(h2);

fh2 = fh2 .* f;

h2 = ifft(fh2);

h(:,c) = h2(1:h_len);

% Normalize the channel energy to 1

h(:,c) = h(:,c)/sqrt(h(:,c)’ * h(:,c) );

end

return

function [h,t,t0,np] = uwb_sv_model_ct_15_4a(Lam,Lmean,lambda_mode,lambda_1, ...

lambda_2,beta,Gam,gamma_0,Kgamma,sigma_cluster,nlos,gamma_rise,gamma_1, ...

chi,m0,Km,sigma_m0,sigma_Km,sfading_mode,m0_sp,std_shdw,num_channels,ts)

% Written by Sun Xu, Kim Chee Wee, B. Kannan & Francois Chin on 22/02/2005

% IEEE 802.15.4a UWB channel model for PHY proposal evaluation

% continuous-time realization of modified S-V channel model

% Input parameters:

% detailed introduction of input parameters is at uwb_sv_params.m

% num_channels number of random realizations to generate

% Outputs

% h is returned as a matrix with num_channels columns, each column

% holding a random realization of the channel model (an impulse response)

% t is organized as h, but holds the time instances (in nsec) of the paths whose

% signed amplitudes are stored in h

% t0 is the arrival time of the first cluster for each realization

% np is the number of paths for each realization.

% Thus, the k’th realization of the channel impulse response is the sequence

% of (time,value) pairs given by (t(1:np(k),k), h(1:np(k),k))

%%

modified by I2R

% initialize and precompute some things

std_L = 1/sqrt(2*Lam); % std dev (nsec) of cluster arrival spacing

std_lam_1 = 1/sqrt(2*lambda_1);

std_lam_2 = 1/sqrt(2*lambda_2);

% std_lam = 1/sqrt(2*lambda); % std dev (nsec) of ray arrival spacing

h_len = 1000; % there must be a better estimate of # of paths than this

ngrow = 1000; % amount to grow data structure if more paths are needed

h = zeros(h_len,num_channels);

t = zeros(h_len,num_channels);

t0 = zeros(1,num_channels);

np = zeros(1,num_channels);

for k = 1:num_channels % loop over number of channels

tmp_h = zeros(size(h,1),1);

tmp_t = zeros(size(h,1),1);

if nlos == 1,

Tc = (std_L*randn)^2 + (std_L*randn)^2; % First cluster random arrival

else

Tc = 0; % First cluster arrival occurs at time 0

end

t0(k) = Tc;

if nlos == 2 & lambda_mode == 2

L = 1; % for industrial NLOS environment

else

L = max(1, poissrnd(Lmean)); % number of clusters

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if Kgamma ~= 0 & nlos == 0

Tcval = []; Tc_cluster= [];

Tc_cluster(1,1)=Tc;

for i_Tc=2:L+1

Tc_cluster(1,i_Tc)= Tc_cluster(1,i_Tc-1)+(std_L*randn)^2 + (std_L*randn)^2;

end

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

cluster_index = zeros(1,L);

path_ix = 0;

nak_m = [];

for ncluster = 1:L

% Determine Ray arrivals for each cluster

Tr = 0; % first ray arrival defined to be time 0 relative to cluster

cluster_index(ncluster) = path_ix+1; % remember the cluster location

gamma = Kgamma*Tc + gamma_0; % delay dependent cluster decay time

if nlos == 2 & ncluster == 1

gamma = gamma_1;

end

Mcluster = sigma_cluster*randn;

Pcluster = 10*log10(exp(-1*Tc/Gam))+Mcluster; % total cluster power

Pcluster = 10^(Pcluster*0.1);

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if Kgamma ~= 0 & nlos == 0

Tr_len=Tc_cluster(1,ncluster+1)-Tc_cluster(1,ncluster);

else

Tr_len = 10*gamma;

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

while (Tr < Tr_len),

t_val = (Tc+Tr); % time of arrival of this ray

if nlos == 2 & ncluster == 1

% equation (22)

h_val = Pcluster*(1-chi*exp(-Tr/gamma_rise))*exp(-Tr/gamma_1) ...

*(gamma+gamma_rise)/gamma/(gamma+gamma_rise*(1-chi));

else

% equation (19)

h_val = Pcluster/gamma*exp(-Tr/gamma)/(beta*lambda_1+(1-beta)*lambda_2+1);

end

path_ix = path_ix + 1; % row index of this ray

if path_ix > h_len,

% grow the output structures to handle more paths as needed

tmp_h = [tmp_h; zeros(ngrow,1)];

tmp_t = [tmp_t; zeros(ngrow,1)];

h = [h; zeros(ngrow,num_channels)];

t = [t; zeros(ngrow,num_channels)];

h_len = h_len + ngrow;

end

tmp_h(path_ix) = h_val;

tmp_t(path_ix) = t_val;

% if lambda_mode == 0

% Tr = Tr + (std_lam*randn)^2 + (std_lam*randn)^2;

if lambda_mode == 1

if rand < beta

Tr = Tr + (std_lam_1*randn)^2 + (std_lam_1*randn)^2;

else

Tr = Tr + (std_lam_2*randn)^2 + (std_lam_2*randn)^2;

end

elseif lambda_mode == 2

Tr = Tr + ts;

else

error(’lambda mode is wrong!’)

end

% generate log-normal distributed nakagami m-factor

m_mu = m0 - Km*t_val;

m_std = sigma_m0 - sigma_Km*t_val;

nak_m = [nak_m, lognrnd(m_mu, m_std)];

end

Tc = Tc + (std_L*randn)^2 + (std_L*randn)^2;

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if Kgamma ~= 0 & nlos == 0

Tc = Tc_cluster(1,ncluster+1);

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

end

% change m value of the first multipath to be the deterministic value

if sfading_mode == 1

nak_ms(cluster_index(1)) = m0_sp;

elseif sfading_mode == 2

nak_ms(cluster_index) = m0_sp;

end

% apply nakagami

for path = 1:path_ix

h_val = (gamrnd(nak_m(path), tmp_h(path)/nak_m(path))).^(1/2);

tmp_h(path) = h_val;

end

np(k) = path_ix; % number of rays (or paths) for this realization

[sort_tmp_t,sort_ix] = sort(tmp_t(1:np(k))); % sort in ascending time order

t(1:np(k),k) = sort_tmp_t;

h(1:np(k),k) = tmp_h(sort_ix(1:np(k)));

% now impose a log-normal shadowing on this realization

% fac = 10^(std_shdw*randn/20) / sqrt( h(1:np(k),k)’ * h(1:np(k),k) );

% h(1:np(k),k) = h(1:np(k),k) * fac;

end

return

function [hN,N] = uwb_sv_cnvrt_ct_15_4a( h_ct, t, np, num_channels, ts )

% convert continuous-time channel model h_ct to N-times oversampled discrete-time samples

% h_ct, t, np, and num_channels are as specified in uwb_sv_model

% ts is the desired time resolution

%%hN will be produced with time resolution ts /

N.

% It is up to the user to then apply any filtering and/or complex downconversion and then

% decimate by N to finally obtain an impulse response at time resolution ts.

min_Nfs = 100; % GHz

N = max( 1, ceil(min_Nfs*ts) ); % N*fs = N/ts is the intermediate sampling frequency before decimation

N = 2^nextpow2(N); % make N a power of 2 to facilitate efficient multi-stage decimation

Nfs = N / ts;

t_max = max(t(:)); % maximum time value across all channels

h_len = 1 + floor(t_max * Nfs); % number of time samples at resolution ts / N

hN = zeros(h_len,num_channels);

for k = 1:num_channels

np_k = np(k); % number of paths in this channel

t_Nfs = 1 + floor(t(1:np_k,k) * Nfs); % vector of quantized time indices for this channel

for n = 1:np_k

hN(t_Nfs(n),k) = hN(t_Nfs(n),k) + h_ct(n,k);

end

end

Signal variations will occur due to obstruction by ground cover in the local vicinity, e.g. buildings, trees, etc., over scales of the order of the sizes of such objects. The scale of these variations will normally be significantly larger than multipath variations.