IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TVWS-NB-OFDM PHR description |  |
| Date Submitted | Jan. 14, 2013 |  |
| Source | [Ryuhei Funada, Chin-Sean Sum, Keiichi Mizutani and Hiroshi Harada (NICT)] | Voice: [[+81-46-847-5055] Fax: [[+81-46-847-5440] E-mail: [funada@nict.go.jp] |
| Re: |  |  |
| Abstract | Text update for PHR for TVWS-NB-OFDM according to PHY editor request |  |
| Purpose | Update of clause 20.3.1.3 PHR for TVWS-NB-OFDM |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

20.3.1.3 PHR

Figure 127 shows PHR format, which is composed of 40 bits for controlling the TVWS-NB-OFDM PHY.

| Bit string index | 0-4 | 5 | 6-9 | 10-20 | 21-24 | 25 | 26-33 | 34-39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit mapping | R4-R0 | RNG | M1-M0 | L10-L0 | A3-A0 | R | H7-H0 | T5-T0 |
| Field name | Reserved | Ranging | Rate | Frame Length | Channel aggregation | Reserved | HCS | Tail |

Figure 127 –PHY header fields for TVWS-NB-OFDM

The Ranging (RNG) field is set to “1” to indicate that this particular frame is intended for ranging. If the frame is not intended for ranging the Ranging (RNG) field is set to “0”.

The Rate field (M3- M0) specifies the data rate of the payload and is equal to the numerical value of the

MCS index as described in 20.3.3, expressed in binary format. The list of data rates for TVWS-OFDM

can be found in 20.3.3.

The Frame Length field (L10-L0) specifies the total number of octets contained in the PSDU (prior to FEC encoding).

The Channel aggregation field (A3-A0) specifies sequence number in the use of channel aggregation described in 20.3.4. If channel aggregation is used, the Channel aggregation field is set to “0”.

The Header Check Sequence (HCS) field (H7–H0) is a 8-bit CRC taken over the PHY header (PHR) fields.

The HCS shall be computed using the first 22 bits of the PHR. The HCS shall be calculated using the

polynomial G8(x) = x8 + x2 + x + 1.

The Tail bit field (T5-T0) is for Viterbi decoder flushing. The Tail bit field is shall be set as described in 20.3.3.2.2.