IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | System Level Simualtion |
| Date Submitted | January 2013 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) Seung-Hoon Park (Samsung) Sungrae Cho (Chung-Ang Univ.) |
| Re: | In response to Technical Guidance Document contributions |
| Abstract | Comments to TGD |
| Purpose | Comments and resolutions to TGD |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1	System level simulations	4

1.1	General Simulation Parameters	4

1.2	Discovery Simulation Parameters	4

1.3	Performance metrics	5

1.4	Communication Simulation Parameters	5

1.5	Simulation scenario	6

1.6	Performance metrics	6

1.7	Test Scenarios	7

1.8	Proposed reference system (optional)	7

# System level simulations

## General Simulation Parameters

| Long-term fading | - Pathloss value depends on frequency band, refer to TG8 Channel Model Document - 2.4 GHz band, outdoor (under rooftop) scenario is mandatory for comparison. |
| --- | --- |
| Channel bandwidth | x MHz (up to proposers) (to calculate noise value e.g. -174dBm/Hz x bandwidth) |
| Maximum TX. power | 20 dBm |
| Tx/Rx antenna gain | -2.5 dB |
| Rx noise figure | 7 dB |
| Receiver sensitivity | -76 dBm (for channel sensing) |

## Discovery Simulation Parameters

| PD deployment | - Uniform drop in 500x500 m2 area - The number of PDs : 100, 500, 1000,5000, 10000. |
| --- | --- |
| Simulation time | over 10 sec |
| Iteration | over 1000 rounds |
| PHY Abstraction | BPSK, 1/2 coding rate - Common PHY mode is referred to DCN13-0058 - Additional PHY mode is up to proposers |
| Discovery ID length | 16 bytes |
| Discovery transmission interval | It depends on proposers. |

## Performance metrics for discovery

  - Average number of discovered PDs over the simulation time.

  - CDF of the discovery latency according to the number of PDs

  - Average power consumption

    - *Power consumption for Tx, Rx (active and idle) TBD

## Communication Simulation Parameters

Unicast with 1 hop is mandatory with optional multicast, group-cast, multihop scenarios.

| PD deployment | Link class-based model |
| --- | --- |
| Traffic | Poisson - Inter arrival time : mean 100 msec |
| Simulation time | at least 10 sec |
| Iteration | until getting smooth curve |
| PHY Abstraction | Perfect rate adaptation with target PER 0.1 - Common PHY mode is referred to DCN13-0058 - Additional PHY mode is up to proposers |
| Packet (MPDU) length | - 256 bytes (for low data rate) - 1024 bytes (for high data rate) |

## Simulation scenario for communications

Link class-based model is assumed.

In an area of 500m x 500m a total number of nodes Nt is assumed.

- At first Tx is deployed uniform randomly.

- Peered receivers to this Tx are deployed uniform randomly under a given distance d<100m from Tx.

-The process is repeated until a total number of terminals Nt is reached.

## Performance metrics for communications

    - Areal sum goodput* [bps/km2]

      - In packets, the average amount of received packets in the area

    - Data packet reception efficiency [ratio]

      - The total number of successfully received packet to the total number of transmitted packet including retransmission procedure

    - Jx1,x2,…,xn=xi2n⋅xi2Jain’s fairness index

    - Latency [sec]

      - Time until success per message, is the average time a node needs to transmit successfully a complete message.

*Goodput is the number of bits in the payload delivered by the network to a certain destination per unit of time.

## Test Scenarios

- Density analysis is mandatory

  - The number of nodes is increasing as

    - 1, 2, 4, …, up to 512

- with optional Packet(MPDU) length analysis

  - Packet length is increasing as

    - 256, 512, 1024, …, up to 4096 bytes

- and optional Packet(MPDU) arrival analysis

  - Arrival time is increasing as 100, 200, 400, … up to 800 msec

## Proposed reference system (optional)

For  convenience of comparison, a simplified IEEE802.11g is provided as follows:

|  | Discovery, Multicast, Groupcast | Unicast |
| --- | --- | --- |
| RTS/CTS | Disable | Enable |
| NAV | Disable | Disable |
| ACK/NACK | Disable | Enable |
| CSMA/CA | No increase of CW* | Exponential increase of CW* |
| Carrier Sensing | Enable | Enable |
| CWmin | 24 | 24 |
| CWmax | - | 210 |
| Retry Count | Disable | Disable |

* CW (Contention Window).  The number of OFDM tones is 48.