IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG8 PAC Framework Document (Draft) |  |
| Date Submitted | May 16, 2013 (r0) May 16, 2013 (r2) Nov. 12, 2013 (r4) Nov. 14, 2013 (r7) Nov. 14, 2013 (r8) |  |
| Source | 802.15.8 Technical Editor: Seung-Hoon Park (Samsung) Byung-Jae Kwak (ETRI) | E-Mail: [shannon.park@samsung.com] [bjkwak@etri.re.kr] |
| Re: |  |  |
| Abstract | This is the draft version of 802.15.8 PAC Framework Document. |  |
| Purpose | This document provides the framework from which the draft PAC specification will be developed. The document provides an outline of each the functional blocks that will be a part of the final specification. The document is intended to reflect the working consensus of the group on the broad outline for the draft specification. As such it is expected to begin with minimal detail reflecting agreement on specific techniques and highlighting areas on which agreement is still required. It may also begin with an incomplete feature list with additional features added as they are justified. The document will evolve over time until it includes sufficient detail on all the functional blocks and their inter-dependencies so that work can begin on the draft specification itself. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.15 Working Group or IEEE 802.15.8 Task Group. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table of Contents

1.	Overview	7

2.	Definitions	7

3.	Abbreviations and acronyms	8

4.	General descriptions	10

4.1.	Concepts and architecture	10

4.2.	Topology	15

4.3.	Reference model	17

5.	MAC layer	21

5.0.	Overview	21

5.1.	Synchronization	21

5.1.1.	Distributed time synchronization for PAC	23

5.2.	Discovery	23

5.3.	Peering	25

5.3.1.	Peering phase	27

5.3.2.	Peering procedure	28

5.3.1.	Resource Shuffling	29

I-PD Procedure	30

J-PD Procedure	31

5.4.	Communications	31

5.4.1.	Unicast	31

5.4.2.	Multicast	31

5.4.3.	Broadcast	32

5.10.1 Multicast Group Management	33

5.10.2 Multicast Data Transmission	41

5.10.3	Prevention Loopback Problem	43

5.10.4	Reliable Multicast	43

5.10.5	Multicast Protocol Using Directional Antenna	44

5.5.	MPDU structure	46

5.5.1.	Packet Format	49

Table 1. ACF (Advertisement Command Frame)	49

Table 2. ARCF (Advertisement Reply Command Frame)	49

Table 3. MGNF (Multicast Group Notification Frame)	49

Table 4. Multicast Data Frame	49

PPDU structure	50

MPDU structure	50

5.5.2.	Frame Check Sequence (FCS)	51

5.6.	Multiple access	52

5.6.1.	Basic Operation	53

5.6.2.	Backoff Procedure	53

5.6.3.	Collision Detection	54

5.6.4.	RTS/CTS Frame Exchange Procedure	54

5.6.5.	Unicast Frame Transmission Procedure	54

5.6.6.	PAC Frame Structure	54

5.6.7.	PAC Frame Structure	55

5.7.	Synchronization procedure	56

5.7.1.	PAC synchronization modes and procedure	57

5.7.2.	Synchronization procedure for operations in unlicensed band	59

5.8.	Discovery procedure	60

5.8.1.	Low Energy Service Discovery (LESD) protocol	60

5.8.2.	LESD MAC command frames	65

5.8.3.	LESD MAC Primitives	67

5.8.4.	LESD MAC constants and PIB attributes	73

5.8.5.	Discovery operation	76

5.8.6.	Resource shuffling	78

5.9.	Scheduling	78

5.9.1.	Communication phase	82

5.9.2.	Communication procedure	82

5.9.3.	Hopping Pattern in 2.4 GHz band	83

5.9.4.	Hopping Pattern decision rule	83

5.9.5.	Resource mapping	84

5.9.6.	Resource allocation	85

5.10.	QoS	86

5.11.	Interference management	87

5.12.	Transmit power control	87

5.13.	Multicast	89

5.14.	Broadcast	89

5.15.	Multi-hop operation	89

5.12.1	Multi-hop Unicast Data Transmission	89

5.16.	Relative positioning	91

5.17.	Power management	91

5.18.	Security	91

5.15.1 Security modes	91

5.19.	Coexistence	99

5.19.1.	Interference sensing	99

5.19.2.	Blocking signal	99

5.19.3.	Low power transmission	100

5.20.	Higher layer interaction	100

6.	Physical layer	101

6.1.	Block Diagram	101

6.2.	Channelization	101

6.2.1.	Sub 1GHz band	101

6.2.2.	2.4 GHz band	102

6.2.3.	5 GHz band	102

6.2.4.	6.2.4. UWB band	103

6.3.	Duplex schemes	103

6.3.1.	TDD	103

6.3.2.	FDD	103

6.4.	Multiplex schemes	103

6.5.	Frame structure	105

6.5.1.	Preamble	107

6.5.2.	Beam Jitter field	110

6.5.1.	Discovery frame structure	111

6.5.2.	Data frame structure	112

6.5.3.	Synchronization region	113

6.5.4.	Discovery region	113

6.5.5.	Peering region	114

6.5.6.	Data region	114

6.5.7.	Discovery frame structure	115

6.5.8.	Data frame structure	120

6.6.	Modulation and coding scheme (MCS)	121

6.6.1.	Modulation	121

6.6.2.	Coding Scheme	121

6.6.3.	Modulation	123

6.6.4.	Pulse shape and duration	125

1.1.1.	Packet structure	125

6.6.5.	SHR Structure	125

6.6.6.	Symbol structure	126

6.6.7.	Data rates	126

6.6.8.	Channel coding and data rates	126

6.6.9.	Symmetrical double-sided two-way ranging	127

6.6.10.	Discovery mode	128

6.7.	Multiple antennas	128

6.8.	Bit interleaver	134

6.9.	Scrambling	134

6.10.	Discovery procedure	135

6.11.	Discovery procedure	137

6.12.	Reference signals	139

6.12.1.	Channel time dispersion	139

6.12.2.	Channel frequency dispersion	139

6.12.3.	ZC sequences	140

7.	UWB Physical (PHY) layer specification	140

7.1.	General	140

7.2.	PPDU format	141

7.2.1.	PPDU encoding process	141

7.2.2.	Symbol structure	143

7.2.3.	PSDU timing parameters	144

7.2.4.	Preamble timing parameters	146

7.2.5.	SHR preamble	148

7.2.6.	PHY header (PHR)	151

7.2.7.	Data field	153

7.3.	UWB PHY modulation	153

7.3.1.	Modulation mathematical framework	153

7.3.2.	Spreading	154

7.3.3.	Forward error correction (FEC)	156

7.4.	UWB PHY RF requirements	158

7.4.1.	Operating frequency bands	158

7.4.2.	Channel assignments	158

7.4.3.	Transmitter specification	159

7.5.	Timestamps and time units	161

Appendix A	162

<Tags>

269r2: Chang Sub Shin (ETRI, shincs@etri.re.kr) ppt  708r0

278r2: Billy Verso (DecaWave, billy.verso@decawave.com) ppt  Text of UWB PHY proposed by DecaWave to TG8.docx

<368r1> </368r1>: SS Joo (ETRI, ssjoo@etri.re.kr)

<369r1> </369r1>: Marco (NICT)

<370r0> </370r0>: Marco (NICT)

<373r1> </373r1>: BJ Kwak (ETRI, bjkwak@etri.re.kr)

<377r0> </377r0>: Shannon (Samsung)

<379r0> </379r0>: W Jeong (ETRI, wjeong@etri.re.kr)

380r2: Qing Li (InterDigital, Qing.Li@InterDigital.com) ppt  15-13-0328-06-0008-tg8-pac-framework-document-draft_380r2_QLupdated.docx

<382r0> </382r0>: Igor Dotlic (NICT, dotlic@nict.go.jp) PDF file  Superseded by 686r0

<384r0> </384r0>: TJ Park (ETRI, tjpark@etri.re.kr)

<388r0> </388r0>: Sungrae Cho srcho@cau.ac.kr

<392r1> </392r1>: SK Cho (ETRI; skcho@etri.re.kr)

<395r1> </395r1>: Jinyoung Chun (LG; jiny.chun@lge.com)

<396r1> </396r1>: Huan-Bang Li (NICT; lee@nict.go.jp)

Additional Text:

- <686r0> </686r0>: Igor Dotlic (NICT, dotlic@nict.go.jp)

- <Marco> </Marco>: Marco (NICT) IEEE802.7z: still missing text

- <Billy> </Billy>: Billy Verso (DecaWave, billy.verso@decawave.com) Text of UWB PHY proposed by DecaWave to TG8.docx

- <390r1> </390r1>: SK Cho (ETRI; skcho@etri.re.kr) Text.for.AppendixOfPFD.docx

- <Park> </Park>: TJ Park (ETRI, tjpark@etri.re.kr) PFD_6.2.1.docx

- <707r0> </707r0>: Marco (NICT)

- <708r0> </708r0>: Chang Sub Shin (ETRI, shincs@etri.re.kr)

- <380r2> </380r2>: Qing Li (InterDigital, Qing.Li@InterDigital.com)

- <392r1+> </392r1+>: SK Cho (ETRI; skcho@etri.re.kr) Text.for.PFD.regarding392r1.docx

- <368r2> </368r2>: SS Joo (ETRI, ssjoo@etri.re.kr)

</Tags>

<IMPORTANT>

Editor’s Note:

- Please use Word file COMPATIBLE with PFD draft file. Otherwise, it makes editing unnecessarily difficult.

  - There are sections with serious format incompatibility. Please provide text to replace those sections in a file compatible with PFD draft file. Otherwise, those sections will be IGNORED in future revisions of this file.

  - Any further text submission with format incompatibility will NOT be accepted.

- When you send an updated text, please MARK CLEARLY what is updated and EXPLAIN how to interpret the markings, to make the life of editor easier. Editor needs to know what THE EXACT CHANGES are.

</IMPORTANT>

In this revision

- All change tracking is gone; proved to make reading the document difficult.

- Text that were already there are in black. Black does NOT mean the text is agreed by the TG if the text is enclosed by tags.

- Text newly added is in blue.

- Text to be deleted with the agreement of the TG in in red.

Overview

The 802.15.8 specification shall be developed according to the P802.15.8 Peer Aware Communication (PAC) project authorization request (PAR), document number 15-12-0063r2 and Five Criteria (5c), document number 15-12-0064r1, which were approved by the IEEE-SA in March of 2012.

<380r2>

This standard defines PHY and MAC mechanism for Wireless Personal Area Networks (WPAN) Peer Aware Communications (PAC) optimized for Peer-to-Peer and infrastructure-less communications with fully distributed coordination. A PAC or Peer-to-Peer Network (P2PNW) is formed for a desired service or application within proximity. A peer can participate in multiple services or applications, i.e. multiple P2PNWs. Many P2PNWs may coexist in proximity.

PAC key features may include the following:

- Operational in selected globally available unlicensed/licensed bands below 11 GHz capable of supporting these requirements

- Scalable data transmission rates, typically up to 10 Mbps

- Discovery for peer information without association

- Discovery signaling rate typically greater than 100 kbps

- Discovery of the number of devices in the network

- Group communications with simultaneous membership in multiple groups, typically up to 10

- Multi-hop relay

- Relative positioning

- Security

</380r2>

Definitions

Device ID: Unique PAC device address e.g. MAC address

Discovered PD: PD which transmits the response signal to discovering PD

Discovering PD: PD which transmits the request signal to discover other PD(s)

FFPD: A PD which supports whole specification including routing table and it is always relay-enabled.

I-PD seen by J-PD: TS beacon received

J-PD Joined: Received TB beacon with ID in list

PAC enabled X-network: A X-network of the devices on which the PAC features are equipped

PAC WPAN: The network of which a device can act as a client or a server for the other devices by allowing shared access to various resources such as configuration or control information, location information, sensing data, advertisement, multi-media contents, social contents, etc.

Peer aware communication (PAC) network: A peer-to-peer wireless proximity network of which a device can act as a client or a server for the other devices by allowing shared access to various resources such as configuration or control information, location information, sensing data, advertisement, multi-media contents, social contents, etc.

PAC enabled network: An external network of the devices on which the PAC features are equipped

Peer network initiator: A PAC device which defines a mission, configures the peer group, hosts peers, and authenticates peers

RFPD: It has two types. These are relay-enabled PD and relay-disabled PD. Relay-enabled is a reduced-function PD which supports routing table for specific multicast group and relay-disabled is a reduced-function PD which does not support routing table.

Service group: The set of PDs which became peering each other

Service type ID: Service category for PAC discovery [Appendix A]

Burst: group of ultra wide band (UWB) pulses occurring at consecutive chip periods

Complex channel: combination of a channel [radio frequency (RF) center frequency] and a ternary code sequence

Frame: format of aggregated bits that are transmitted together in time

Mean pulse repetition frequency (PRF): total number of pulses within a symbol divided by the symbol duration

Payload data: contents of a data message that is being transmitted

Peak pulse repetition frequency (PRF): maximum rate at which an ultra wide band (UWB) physical layer (PHY) emits pulses

Ranging frame (RFRAME):  ultra wide band (UWB) frame having the ranging bit set in the physical layer (PHY) header (PHR)

Ranging marker (RMARKER): first ultra wide band (UWB) pulse of the first bit of the physical layer (PHY) header (PHR) of a ranging frame (RFRAME).

Symbol: a period of time and a portion of the transmitted signal that is logically considered to be a unit signaling event conveying some defined number of data bits or repeated portion of the synchronization signal.

Abbreviations and acronyms

AGC		Automatic Gain Control

BCI		Burst Control Indicator

BU		Blocking Unit

CAP		contention access period

CAR		Consecutive Allocation Request

CCA		clear channel assessment

CFP		contention free period

CP		Cyclic Prefix

CS		Channel Sampling

CTS		Clear To Send

CW		Contention Window

DS-REQ		Distributed Scheduling – Request

DS-RSP		Distributed Scheduling – Response

EIED		Exponential Increase Exponential Decrease

FCS		Frame Check Sequence

FFPD

GI		Guard Interval

GSW		Gold Sine Wave

I-PD 		Initiator PD

IS		Interference Sensing

J-PD: 		Joiner PD

LESD		Low Energy Service Discovery

LTF		Long Training Field

MLME		MAC sublayer Management Entity

MLSDE		MAC sublayer Service Discovery Entity

MZC1		Modified Zadoff-Chu sequence 1

MZC2		Modified Zadoff-Chu sequence 2

OFDM		Orthogonal Frequency Division Multiplexing

PAN		Personal Area Network

PD		PAC Device

PDU		Protocol Data Unit

PID    		Peering Identifier

Peering-REQ	Peering-Request

Peering-RSP	Peering-Response

RFPD

PLME		PHY layer Management Entity

RTS		Request To Send

RU		Resource Unit

SC		Single Carrier

SDU		Service Date Unit

SIV		Service Information Version

SP		Scheduling Priority

SRI		Scheduling Request Indicator

SSF		Self Spatial Filtering

STF		Short Training Field

TB		Temporary beacon

TS		Trigger signal

ULA		Uniform Linear Array

ZC		Zadoff-Chu

BPM		burst position modulation

BPSK		binary phase-shift keying

CRC		cyclic redundancy check

DPS		dynamic preamble selection

FCS		frame check sequence

FEC		forward error correction

LFSR		linear feedback shift register

LSB		least significant bit

MAC		medium access control

MSB		most significant bit

PHR		PHY header

PHY		physical layer

PPDU		PHY protocol data unit

PRBS		pseudo-random binary sequence

PRF		pulse repetition frequency

PSD		power spectral density

PSDU		PHY service data unit

RF		radio frequency

RFRAME	ranging frame

RMARKER	ranging marker

SFD		start-of-frame delimiter

SHR		synchronization header

SYNC		synchronization

UWB		ultra wide band

General descriptions

This clause provides the basic framework of PDs. The framework serves as a guideline in developing the functionalities of PDs and their interactions specified in detail in the subsequent clauses.

Concepts and architecture

<368r2>  Supersedes 368r1

The peer aware communication (PAC) network is a peer-to-peer wireless proximity network of which a device can act as a client or a server for the other devices by allowing shared access to the various resources such as configuration or control information, location information, sensing data, advertisement, multi-media contents, social contents, etc.

The PAC enabled network consists of the PAC devices (PDs) which carry one of roles: peer network initiator, proxy peer network initiator, peer network peers, peer network relay, and peer network observer. The initiator defines a mission, configures the peer group, hosts peers, and authenticates peers. The peer network relay provides one-hop frame relaying. The peer network observer is a neighbor of the peer group, but is not a member of the peer group.

Figure 4.1- Components of the PAC enabled networks

The peer aware communications (PAC) features may be equipped on a PAC dedicated device or on external network equipment. The PAC network is composed of the PAC dedicated devices. The PAC enabled network is composed of the PAC enabled external network devices.

The PAC networks and the PAC enabled networks construct a peer group consisted of: single PAC group, multiple PAC groups, single PAC group and single PAC enabled network, single PAC group and multiple PAC enabled networks, multiple PAC groups and single PAC enabled network, and multiple PAC groups and multiple PAC enabled networks.

Figure 4.2- Possible peer groups constructed with the PAC equipped devices

</368r2>

<380r2>

A PAC system may contain the following functions at MAC and/or PHY.

Higher Layer: the layer above PHY/MAC, such as service or application layer for an infrastructure-less P2P wireless system.

PD Management Entity: manages PD information across PHY/MAC and higher layer for P2P communications.

Synchronization Function: performs initial and/or periodic time boundary synchronization with a peer at MAC; maintains frequency and/or clock phase synchronization at PHY.

Discovery Function: discovers peer(s) in proximity by using  IDs such as Device ID, Device Group ID, Application type ID, Application-specific ID, Application-specific user ID, and Application-specific group ID, and/or peer context information; sends discovery request messages with  IDs such as Device ID, Device Group ID, Application type ID, Application-specific ID, Application-specific user ID, and Application-specific group ID, and/or peer context information for to-be-discovered in proximity.

Peering Function: requests or responds to Peering (i.e. association), Peering Updates, De-peering, or Re-peering by using IDs such as Device ID, Device Group ID, Application type ID, Application-specific ID, Application-specific user ID, and Application-specific group ID, and/ or peer context information.

Channel Management Function: manages the radio resource or channel allocation among P2P networks based on the services or applications; manages channel allocation and/or accessing within a P2P network based on the peer information.

Power Control Function: performs transmitting power control and interference management based on power control information.

Data Transceiving Function: conducts reliable data transmitting and/or receiving based on the QoS required by service or application.

Measurement and Report Function: conducts measurements of channel, QoS, etc., and sends data reports from other logic functions to higher layer.

Scheduler Function: manages or controls the sequence of events in  MAC and/or PHY layer, for example, transmission queuing, collision avoidance, etc.

Encoder Function: performs encoding and other data processing such as interleaving, scrambling, etc. at PHY to aid in reliable reception

Modulator Function: performs modulation and other data processing such as mapping, precoding, etc. at PHY

Decoder Function: performs decoding and other data processing, such as de-interleaving, de-scrambling, etc. at PHY to aid in reliable reception

Demodulator Function: performs demodulation and other data processing such as equalization, de-mapping, etc. at PHY

</380r2>

<377r0>

PAC application at PD1 requests application-specific discovery operation via PAC middleware. PAC middleware has a API to PAC module including PAC PHY and MAC layer, and requests to send Peer Discovery Identity (PDI) to PAC module. PAC MAC constructs Peer Discovery Message (PDM) including PDI from higher layer and transmits the PDM via Discovery Slot determined at PHY layer. PDM is received by PAC module at PD2. PAC module matched the received PDI to the pre-stored PDI and it delivered to PAC middleware if it meets the matching constraints.

Therfore, only PDs with same PDI shall be discovered each other.

PDI may be pre-installed or given from network to higher layer.

The content of PDM including application-specific ID, application-specific user ID or application-specific group ID is decided by higher layer. Different peer discovery types such as “Advertise/Monitor”, “Search/Response or “Publish-Subscribe” may be supported.

</377r0>

<392r1+>

PAC provides functionalities optimized for scalable peer to peer communications with fully distributed coordination. PAC also provides mechanisms that enable low power consumption, discovery of peer information before association, efficient resource allocation in a distributed manner, and coexistence with other 802 systems.

</392r1+>

<395r1>

IEEE 802.15.8 shall support a fully distributed, decentralized, and self-organized system composed of PDs.

Some of these devices may be able to connect on an opportunistic basis to infrastructure, which is out of scope for IEEE 802.15.8.

IEEE 802.15.8 shall support one-to-one and one-to-many communications.

IEEE 802.15.8 shall support scalable data rate to accommodate many applications such as listed in the Application Matrix (document number 15-12-0684-00-0008).

Possibly aided by higher layers, a PD shall support data transfers between itself and identified PDs or groups.

IEEE 802.15.8 shall support both one-way and two-way communications.

</395r1>

Topology

<368r2>

A PAC device may have connections to multiple peers in a peer group and may participate in multiple peer groups concurrently. A PAC device provides one-to-one communication and one-to-many communication in a peer group. A PAC device provides one-to-one communication to multiple peer groups and one-to-many communication to multiple peer groups.

IEEE 802.15.8 shall support a PD having simultaneous communication sessions for same or different applications. IEEE 802.15.8 shall support a PD participation in at least two independent one-to-many communications with different peers at the same time.

A PAC device may have star topology to peers in a peer group or may have mesh topology to peers in a peer group. A PAC device provides at least two hops relay to peers in a peer group.

Figure 4.3. An example of concurrent communication

</368r2>

<380r2>

Figure X: Typical P2P Network Topology

As required in TGD, a P2P network (P2PNW) is formed by a desired application. There may be multiple applications existing simultaneously, therefore multiple P2PNWs coexist at the same time in proximity. In addition, a peer may run multiple applications simultaneously, and therefore joins multiple P2PNWs at the same time. As shown in the figure X, there are 4 applications (P2PNWs), peer 6 and peer 9 join 2 P2PNWs for 2 applications, respectively. Application 3 is pair communication between two peers, while application 1 is multi-hop broadcast for commercial advertisement with a tree topology, and application 4 is gaming with a mesh topology. With many different P2PNWs co-existing in the proximity, the topology is more heterogeneous than homogenous.

</380r2>

<395r1>

Several topologies are considered to support various service interactions within PDs. One-to-one and one-to-many topologies shall be supported. IEEE 802.15.8 shall support a PD participation in at least two independent one-to-many communications with different peers at the same time. IEEE 802.15.8 shall support a PD having simultaneous communication sessions for same or different applications. Mesh topology may be supported.

Figure 1. An example of concurrent communication

</395r1>

Reference model

<368r2>

In accordance with the ISO/OSI-IEEE Std 802-2001 reference model, all PDs are internally partitioned into a physical (PHY) layer, a medium access control (MAC) sublayer of the data link layer, and a MAC relaying sublayer (RELAY) resided below the Service Specific Convergence Sublayer (SSCS) of IEEE Std 802.2-1998 Logical Link Control (LLC). Direct communications between PDs are to transpire at the PHY layer and MAC sublayer as specified in this standard; Message security services are to occur at the MAC sublayer, and security operations are to take place inside and/or outside the MAC sublayer.

Within a PD, the RELAY provides its service to the higher layer through the RELAY service access point (SAP) located immediately above the RELAY sublayer, while the MAC provides its service to the RELAY through the MAC SAP located between them. On transmission, the higher layer passes MAC service data units (MSDUs) to the RELAY sublayer via the RELAY SAP and the RELAY sublayer passes MSDUs to the MAC sublayer via the MAC SAP, and the MAC sublayer passes MAC frames (also known as MAC protocol data units or MPDUs) to the PHY layer via the PHY SAP. On reception, the PHY layer passes MAC frames to the MAC sublayer via the PHY SAP, and the MAC sublayer passes MSDUs to the RELAY sublayer via the RELAY SAP, and the RELAY sublayer passes MSDUs to the higher layer via the RELAY SAP or relays MSDUs to the MAC sublayer via the MAC SAP.

MAC and PHY SAPs also pass control information between the layers.

Figure 4.4. Reference model

There may be a logical PD management entity (PDME) that exchanges network management information with the PHY and MAC as well as with other layers or exchanges network management information with the external network management entity.

</368r2>

<395r1>

All PDs are internally partitioned into a physical (PHY) layer and a medium access control (MAC) sublayer of the data link layer, in accordance with the ISO/OSI-IEEE Std 802-2001 reference model. Direct communications between PDs are to transpire at the PHY layer and MAC sublayer as specified in this standard; Message security services are to occur at the MAC sublayer, and security operations are to take place inside and/or outside the MAC sublayer.

Within a PD, the MAC provides its service to the higher layer through the MAC service access point (SAP) located immediately above the MAC sublayer, while the PHY provides its service to the MAC through the PHY SAP located between them. On transmission, the higher layer passes MAC service data units (MSDUs) to the MAC sublayer via the MAC SAP, and the MAC sublayer passes MAC frames (also known as MAC protocol data units or MPDUs) to the PHY layer via the PHY SAP. On reception, the PHY layer passes MAC frames to the MAC sublayer via the PHY SAP, and the MAC sublayer passes MSDUs to the higher layer via the MAC SAP.

MAC and PHY SAPs also pass control information between the layers.

Figure 2. Reference model

There may be a logical PD management entity (PDME) that exchanges network management information with the PHY and MAC as well as with other layers.

</395r1>

MAC layer

  - Overview

This section defines MAC mechanism for Wireless Personal Area Networks (WPAN) Peer Aware Communications (PAC) optimized for peer-to-peer and infrastructure-less communications with fully distributed coordination.

The MAC functions that is defined in the TGD[ref] are described in this clause.

- Synchronization

- Discovery

- Peering

- Communication

Synchronization

In synchronized mode, PAC operates in fully distributed synchronization.

[Def. 1] Synchronization among PDs can be achieved by transmitting/receiving synchronization reference signal. When there is already synchronization among PDs, each PD may send synchronization reference signal independently to maintain the synchronization. When there is no synchronization already established, a PD can initiate it by transmitting synchronization reference signal.

[Def. 2] Synchronization among PDs can be achieved by transmitting/receiving synchronization reference signal. The synchronization reference signal may be transmitted by a single PD elected by a group of PDs. When there is no synchronization already established, a PD can initiate it by transmitting synchronization reference signal.

<380r2>

The synchronization procedure can be triggered by the request from other procedures such as Discovery, Peering, Data transceiving, higher layer etc. The synchronization procedure starts by scanning for the common synchronization reference signal, which may be carried on common beacon and or sent on common channel.  If the common synchronization reference signal is not detected the scanning process continues until a maximum scan count is reached or a time out has occurred.  If the common synchronization reference signal is detected, then the next step is to synchronize with the superframe boundary via the common synchronization reference signal and then scan for the dedicated synchronization reference signal which may be carried on dedicated application beacon and/or sent on dedicated channel for the desired application.  Once the dedicated synchronized reference for the desired application is detected then the next step is to synchronize with the application frame boundary via the dedicated synchronization reference signal.  Upon synchronized with the application frame boundary the peer has successfully synchronized with the desired application.

The synchronization procedure may be repeated before time out or the maximum count of retries due to failed synchronization attempt or loss of synchronization response. If the synchronization still fails after timeout or reaches the maximum count of retries, the synchronization procedure is aborted and a failure report will be sent to the entity or procedure that requested the synchronization.

</380r2>

<708r0>

Distributed time synchronization for PAC

PDs in PAC network shall be synchronized by using distributed manners. When a new PD listens to neighbouring PD’s beacon frame, it will synchronize to PAC network. Synchronized PD shall keep synchronization with periodic beacon frame. Since the new PD tracks its time synchronization from beacon frame, it shall determine the next receiving time of beacon frame.

a) before transmitting beacon frame for synchronization, every PD in the PAC network have to initiate prioritized backoff algorithm to get a chance to transmit beacon frame.

b) beacon frame shall be transmitted periodically every beacon interval

c) all PDs which participated in PAC network shall keep time synchronization using timestamp value of beacon frame.

d) beacon frame shall set the timestamp value including transmitting PD’s local delay

e) PDs in the PAC network shall not be allowed to transmit a frame before receiving beacon frame.

f) a PD transmitting beacon frame can be selected sequentially among PDs in the PAC network.

Figure-1 Distributed time synchronization

</708r0>

<Billy>

UWB PHY PLACEHOLDER: In general MAC layer procedures should be applicable on top of the UWB PHY, however this place holder is to note that there may be some PHY specific elements to the synchronization procedures that need to be considered and noted here – e.g. MAC provisions for ranging and localization.

</Billy>

Discovery

PAC should support three discovery types of the following:

- Advertisement: In Advertise type discovery, a PD broadcasts its own identity and does not expect responses.

- Publish/Subscribe: In Publish/Subscribe type discovery, a PD broadcasts its own identity and expects responses from PDs that have discovered the broadcast message.

- Query/Reply: In Query/Reply type discovery, a PD broadcasts the identity of the PD or PDs being queried and expects a response or responses from the PD or PDs, accordingly.

For the purpose of discovery of PDs, the discovery signal conveys information that may represent one or more of the following IDs such as Device ID, Device Group ID, Application type ID, Application-specific ID, Application-specific user ID, and Application-specific group ID.

The discovery procedure should support mechanisms to ensure privacy that a PD is not tracked.

The discovery procedure should support protection of identity from impersonation.

<Billy>

UWB PHY PLACEHOLDER: In general MAC layer procedures should be applicable on top of the UWB PHY, however this place holder is to note that there may be some PHY specific elements to the discovery procedures that need to be considered and noted here – e.g. MAC provisions for ranging and localization.

</Billy>

<380r2>

Figure: Discovery Procedure

The proposed discovery procedure consists of three main stages:

1) Peer Discovery Scan;

2) Peer Discovery Evaluation;

3) Peer Discovery Request.

“To Discover” means that the peer wants to discover other peer(s) with a desired application; while “to be discovered” implies that the peer wants to be discovered by other peer(s) through broadcasting discovery message in proximity.

The Peer Discovery Scan is triggered if a peer wants “To Discover” for a desired application. The Peer Discovery Request stage is triggered if a peer wants “To Be Discovered” for a desired application.

During the Peer Discovery Scan stage, the peer scans the discovery message carrying the desired application information. If any peer is detected, Peer Discovery Evaluation stage is triggered for qualification check i.e. if the detected peer meets the discovery criteria. The qualified peer(s) is added to the PD list which may be used for peering. The peer may repeat scan and evaluate stage until it discovers required number of peers for desired application. If no peer is detected during the scan stage for a period of time, the peer switches to “To Be Discovered” status for the desired application, and enters the Peer Discovery Request stage. The peer may periodically broadcast the discovery message with the application information.

</380r2>

Peering

Peering is the procedure to establish a link between a pair of PDs or links among multiple PDs discovered during the discovery procedure.

Re-peering is the procedure to re-establish a link between a pair of PDs or links among multiple PDs which peered previously. In the re-peering procedure, discovery may be simplified or omitted.

De-peering is the procedure to disconnect the link established by peering. The disconnected link through de-peering may be re-established by re-peering procedure.

<380r2>

Peering Identifier (PID) is the identity of each established link or peering relationship between two or more PDs. Considering different types of applications, link(s) may be established at different levels of peering:

- Device-based peering

- Service-based peering

- User-based peering

Peering Procedure

Figure: Peering Procedure

The peering procedure is initiated by sending a peering request message including requested peering information. Acknowledgement to peering request message may be optional for responder. Responder may send a peering response message to requestor for indicating if the peering request is accepted or not. The response message may include peering information as well as the peering ID if the request is accepted. Acknowledgement to response message may be optional as well.

Re-peering procedure:

Figure: Re-peering Procedure

Re-peering procedure is similar to peering procedure. The main difference is: 1) the peering ID should be included in request message since the re-peering is to re-establish a link or links among PDs that peered previously; 2) the PD(s) receives the request need to check the peering log before making decision if accepts the re-peering request.

De-peering procedure:

Figure: De-peering Procedure

De-peering procedure starts with a de-peering request, which is replied by a de-peering response message. Both acknowledgement and de-peering response may be optional. Accordingly, the peer may triggers peer discovery process with updated parameters after de-peering procedure.

</380r2>

<Billy>

UWB PHY PLACEHOLDER: In general MAC layer procedures should be applicable on top of the UWB PHY, however this place holder is to note that there may be some PHY specific elements to the peering procedures that need to be considered and noted here – e.g. MAC provisions for ranging and localization.

</Billy>

<377r0>

Peering is the link establishment between the discovering PD and the discovered PD. Peering Request message and Peering Response message shall be exchanged to establish a link. A PD exchanges information such as device capability for setup a link, and determines link related parameters such as Link ID, QoS class, link range, or etc. Network protocol such as routing shall be operated only over connected links.

</377r0>

<395r1>

Peering phase

It is phase for discovering PD to fit slot sync and make peering with discovered PDs. The phase size is flexible because slot sync may be changed.

Figure 6. PAC peering frame structure

Peering procedure

Discovering PD sets up peering phase when it discover a or more PDs.

Peering procedure is as below:

  - Wait for timing1

    - Discovering PD moves to the channel of discovered PD. When the remaining duration of the hopping slot of discovered PD is less than time of 3 messages exchange, discovering PD waits and do peering in the next hopping slot.

  - Slot-sync & peering

    - Discovering PD transmits ‘Peering request message’ with device ID of discovered PD, which is ‘I want to do peering to you and follow your slot timing’.

    - Discovered PD receives the message and check its device ID, and transmits ‘Peering response message’, which is ‘OK’ and peering information.

    - Discovering PD transmits ‘Peering confirmation’ message, which is peering information.

  - Wait for timing2

    - Discovering PD waits the newly revised slot timing before communication phase is started.

</395r1>

<392r1>

Peering procedure is to establish a link between a pair of PDs discovered during the discovery procedure.

Peering procedure consists of peering request / response message exchange at Peering- REQ/Peering-RSP interval and the usage check of the entire PIDs at the PID broadcast interval in the peering region of the frame.

Figure 4. Resource structure of peering

After successful peering procedure, a pair of PDs shares a PID to be used for multiple access to the data region. For orthogonal use of PID in distributed communication environment, a PD should notify other PDs of the use of PIDs at the fixed position specified by each PID in the PID broadcast interval.

A PD performs the state check of the entire PIDs before sending a message related to the peering. Based on the PID usage information acquired in the PID broadcast interval, the list of available PIDs (i.e. unused PIDs) is included in the peering request message by the requesting PD. A PD which received the peering request message sends the peering response message including the selected PID. Through these messages, a pair of PDs shares a PID.

A PD selects a Peering-REQ RU at random. A PD which transmits the peering request message waits the reception of the peering response message at the corresponding Peering-RSP RU. If a PD fails to receive the peering response message at the Peering-RSP RU corresponding to the selected Peering-REQ RU, it reselects a Peering-REQ RU and retransmit the peering request message through newly selected Peering-REQ RU.

    - Resource Shuffling

A resource shuffling is used to vary the configuration of PDs which use the RUs blocked by a specific PD. Shuffling pattern for PID broadcast interval of peering region is configured using the entire RUs of a superfame. One of two patterns (i.e. s(0) and s(1)) is applied to two consecutive superframes and the same pattern is applied at eight times per ultraframe.

Figure 5. Configuration of shuffling pattern for PID broadcast interval

Figure 6. Change of shuffling pattern

</392r1>

<396r1>

- I-PD Procedure

- J-PD Procedure

</396r1>

Communications

Data communication may be conducted as unicast between a pair of PDs, or as multicast within a group of PDs, or as broadcast to any PD.

Unicast

Unicast is a one-to-one data communication between a pair of PDs. For reliable unicast transmission, ACK may be used for acknowledging a successful data transmitting.

Multicast

Multicast is a one way one-to-many data communication to a group or groups of PDs which may be addressed by an ID specific to the group or by multiple IDs. For reliable multicast transmission, ACK may be used for acknowledging a successful multicasting.

<380r2>

Reliable multicast is to provide reliable data transmission in multicast from one PD to multiple or all PDs with a group. Different ACK schemes may be explicitly indicated in MAC header for different levels of reliability.

- Full ACK: All members/peers need to acknowledge, referred to as 100% ACK.

- Partial ACK: Only a portion of peers needs to send back ACK, referred to as partial ACK. In this case, additional information will be contained in this field such as the percentage of peers for sending back ACK.

- Any ACK: ACK is only required from any one of members/peers. In other words, if one member/peer sends back ACK successfully, other members/peers do not need to send back ACK.

- Location-based ACK: Only peers around a location need to send back ACK, referred to as location-based ACK. In this case, additional location information will be contained in this field.

.

Figure Y: Multicast Scenarios

The figure Y shows one-hop and multi-hop multicast scenarios, respectively. For one-hop scenario, the sender directly sends out multicast data to a group of peers. In multi-hop scenario, the intermediate peer may relay the multicast data to a sub-group of peers. .

Figure: Distributed One-Hop Multicast

</380r2>

Broadcast

Broadcast is a one way one-to-many data communication to any PD.

<368r1>

The part of devices in a peer group may establish a group of the peer group. The MAC sublayer provides the group of the peer group formation primitives and the group-cast primitives to the next higher layer. The formation of a group of the peer group is performed with the implicit inviting and the explicit grouping.

</368r1>

<388r0>

- 5.10.1 Multicast Group Management

5.10.1.1 Finding/Joining Multicast Group

A multicast group consists of two or more PDs with the same application type ID, application specific ID, application specific group ID, and device group ID. It can be formed only if two or more PDs can recognize themselves. Before a PD joins a multicast group, it has to find the multicast group within K-hop coverage. If the PD cannot find the group, then it finds the group periodically. In order to find a multicast group, a PD broadcasts an Advertisement Command Frame (ACF) after random timer Tj where the maximum TTL is set to K. Range of Tj is [0, Tjmax ]. If a PD receive the ACF, it stores the ACF in order to forward it to other PDs and saves backward path in the routing table during expiration timer TW where TW is calculated by one-hop RTT and K if it is relay-enabled (Backward path is originator of the ACF, one-hop PD sending the ACF, Device Group ID & Application type ID & Application-specific ID & Application-specific group ID.) It compares the receiving frame’s Device Group ID & Application type ID & Application-specific ID & Application-specific group ID with its own. If they all are same, it replies an ARCF (Advertisement Reply Command Frame) to the PD sending the ACF (Reply of the ARCF depends upon the MGNF explained in the following pages). If any of them is not same, it decrements the TTL of the ACF and forwards the ACF.

In order to limit the duplicate ARCF, PDs replying the ARCF multicast a Multicast Group Notification Frame (MGNF) after random time. (MGNF is explained detail in later section)  Then, the PD multicasting the MGNF replies source PD with an ARCF by using backward path. A PD receiving both ACF and MGNF does not reply an ARCF. A PD receiving the ARCF whose destination is not itself saves the route information of ID of the source PD sending the ARCF, ID of the one-hop PD sending the ARCF, Device Group ID, Application type ID, Application-specific ID, Application-specific group ID. Then, this PD is referred to as “FORWARDING PD.” A PD receiving the ARCF whose destination is itself saves the route information same as FORWARDING PD. Then, this PD is referred to as “JOINED MULTICAST GROUP MEMBER.”

When a PD wants to join a multicast group, it broadcasts an ACF to its neighbor PDs. When a PD which is not in the multicast group receives the ACF, it saves the route information and forwards to the others. When a multicast group member PD receives an ACF, it broadcasts an ARCF to it. If there is a duplicate ARCF, the member PD broadcasts a MGNF to the other member PD by notifying that it will send an ARCF instead of it. The node sending MGNF may send KEKs (key encryption keys) to the node which wants to join the multicast group after authentication for secure multicast if needed. It also may send rekeying messages to update an existing group key for backward secrecy. Hereby, a PD receiving the ARCF joins the multicast group.

For example, in Figure 1,

- If PD A wants to join the multicast group, it broadcasts an ACF,

- PD C is not in the multicast group. Therefore, it saves the route information and forwards to the others (e.g., F and E). PD D already received the ACF and PD F now receives the ACF. Then, there will be a duplicate ARCF from both of D and F to A. In order to limit the duplicate ARCF, PD D multicasts a MGNF notifying that D will send an ARCF instead of F.

- PD E forwards the ACF to B and G. D replies with ARCF to A. D may send KEKs to A if needed. D may send rekeying messages also. F does not transmit an ARCF to A.

- When PD A receives the ARCF, A joins D’s multicast group. Also, PD B receives the ACF from PD E. Then, B multicasts a MGNF to limit a duplicate ARCF from G.

- PD B replies with an ARCF to A through E. B may send KEKs to A if needed. B may send rekeying messages also. G does not transmit an ARCF to A since the PD G already received the MGNF from B.

- E forwards the ARCF to PD C.

- C forwards the ARCF to PD A. Then, PD A is aware of route between PD A and PD B. Now, PD A joins PD B’s multicast group. Although group of B and G are not aware of group of D and F currently, multicasting service still works by simply forwarding multicast data frames.

Figure 1. Finding/Joining Multicast Group

5.10.1.2 Device Group ID Creation

A multicast group is determined by application type ID, application specific ID, and application specific group ID. Therefore, it is inefficient if transmitting all IDs in a frame and managing routing table. Therefore, we propose a device group ID creation scheme. Device group ID should be unique and distributed by a PD sending the first ARCF in the group. The PD manages/updates group keys for secure and dynamic multicast group communications after authentication procedures described in security section. The PD generates device group ID based on its unicast ID. Since a PD’s unicast ID is unique, prefix concatenated by PD’s unicast ID is also unique.

Figure 2. Device Group ID

When two or more multicast groups are merged, the device group ID should be same. The PD recognizing the existence of two or more multicast groups determines the device group ID for those groups randomly. Then, the PD sends MGNF (notification type: 3 and TTL: ∞) to the group that does not have the selected group ID to update multicast group ID.

In Figure. 3, A recognizes that two device group IDs exist in networks. A determines that device group ID is 8. A sends MGNF to D and E to change device group ID. A may send rekeying messages to update an existing group key for backward secrecy.

Figure 3. Device Group ID Creation

5.10.1.3 Multicast Group Notification Frame (MGNF)

The purposes of MGNF are limiting duplicate ARCF, management of routing table, notifying leaving multicast group, device group ID creation, request for unicast routing, reply for unicast routing, mobility support, local repair for relay-enabled PDs and notification of removed routing entry.

MGNF has eight notification types for above purposes. These are;

Table 5. Type of MGNF

| Type | Description |
| --- | --- |
| 0 | Limiting duplicate ARCF |
| 1 | Management of routing table |
| 2 | Notifying leaving multicast |
| 3 | Device group ID creation |
| 4 | Request for unicast routing |
| 5 | Reply for unicast routing |
| 6 | Mobility support |
| 7 | Local repair |
| 8 | Notification of removed routing entry |

- Limiting duplicate ARCF(Notification Type: 0): A PD receiving an ACF multicasts MGNF with notification type set to 0 with random timer Tj. Range of Tj  is [0, Tjmax ].  Although missing MGNF can increase duplicate ARCF, MGNFs are not retransmitted to avoid flooding. In this case, payload of MGNF contains source of the ACF.

- Management of routing table (Notification Type: 1): In order to reduce routing entries in the table, each PD maintains entries only for PDs that has exchanged ACFs and ARCFs in its routing table. Each PD in multicast group multicasts MGNFs periodically (Tp(Period)+Tj) with notification type set to 1. Range of random timer Tj is [0, Tjmax]. Upon receiving a MGNF, a forwarding PD updates the entries of the originator of the MGNF and one-hop PD sending the MGNF in its routing table, and forwards the MGNF. Upon receiving a MGNF, a non-forwarding PD updates the entries of the originator of the MGNF and one-hop PD sending the MGNF in its routing table, but does not forward the MGNF. Also it can be used for detection and routing table update for link breakage. When a link between two nodes is broken, the other member node in the group which is aware of the link breakage removes the entry of the node which has a broken link to its routing table. For example, in Figure 4, we suppose that the link between A and B is broken. E does not receive MGNF from A. E is aware that the link is broken, because A’s MGNF does not arrive before E’s routing table expiration timer is expired. Then, E removes the entry of A to E’s routing table.

Figure 4. Detection and Routing Table Update For Link Breakage

- Notifying leaving multicast (Notification Type: 2): will be explained in 5.10.1.5 section.

- Device group ID creation (Notification Type: 3): Explained in the 5.10.1.2 section.

- Request for unicast routing (Notification Type: 4): will be explained in 5.12.1 section.

- Reply for unicast routing (Notification Type: 5): will be explained in 5.12.1 section.

- Mobility support (Notification Type: 6): will be explained in 5.10.1.6 section.

- Local repair (Notification Type: 7): will be explained in 5.10.1.6 section.

- Notification of removed routing entry (Notification Type: 8): will be explained in 5.10.1.4.2 section.

5.10.1.3.1 Reliable MGNF Transmission

We classify the level of reliable MGNF transmission in two types. These are ACK-based MGNF transmission and no ACK-based transmission. The node which transmits MGNF has to receive ACK-based MGNF from its neighbor nodes for notification types 2,3,4,5 and 8 for reliable MGNF transmission. But it does not have to receive ACK-based MGNF for notification types 0, 1, 6 and 7. Because, the notification type 0 is for limiting duplicate ARCF, thus the PD does not need to receive it. The notification type 1 is for management of routing table and MGNF is transmitted periodically. The notification type 6 is for mobility support. Due to changed route information, the node does not need to receive the ACK-based MGNF from the moving node.

5.10.1.3.2 Reliable MGNF Transmission

When a PD belongs to multiple multicast groups, it multicasts MGNF multiple times. In order to resolve the above problem, a PD may set Destination Address field of MGNF as United Multicast Address (UMA). The UMA unifies multiple different groups into a single one. The UMA can reduce the MGNF traffic as the following pages.

5.10.1.3.3 Redundant MGNF Transmission

When a PD multicast its MGNF to the other nodes, if there are nodes which are member of multiple groups simultaneously, there will be a redundant MGNF transmission.

For example, in Figure 5, we suppose that A, B, C, and D are in group 2 while B, C and E are in group 1. B and C are in groups 1 and 2 simultaneously.

- B starts to multicast its MGNF for group 1 to C and D (forwarding to E). After C and D verify device group ID field in PD B’s MGNF, PDs C and D recognize that the MGNF belongs to group 1.

- Then, D forwards the MGNF from B to E. After PD E verifies device group ID in B’s MGNF, E recognizes that the MGNF belongs to group 1.

- B starts to transmit its MGNF for group 2 to PDs A, C, and D.

- Then A, C, and D verify their device group ID in PD B’s MGNF. They recognize that the MGNF belongs to group 2. There are 3 transmissions for MGNF (B transmitted MGNF twice).

Figure 5. Redundant MGNF Transmission

5.10.1.3.4 Redundant MGNF Transmission

In order to prevent redundant MGNF transmission, we proposed a UMA-based MGNF transmission. In this proposal, we set the destination address fields to UMA for nodes which are member of two or more groups simultaneously. When a node receives MGNF, it checks its destination address and if it finds that the destination address is equal to UMA and two or more device group IDs that have the same value (forwarding PD for that group ID) and those device group ID have the same destination address with the originator of the received MGNF, then, the PD forwards the received MGNF.

For example, in Figure 6,

- PD B starts to transmit its MGNF whose destination address field set to UMA for Groups 1 and 2 to A, C, and D.

- Then A, C, and D verify the device group ID field in PD B’s MGNF, they recognize that the MGNF belongs to UMA and they forward the received MGNF. PD E receives the MGNF from D. E does not forward the MGNF. There are2 transmissions for MGNF. (B transmitted MGNF just once).

Figure 6. UMA-Based MGNF Transmission

5.10.1.4 Creation and Management of Routing Table

5.10.1.4.1 Creation of Routing Table

Whenever a relay-enabled PD receives ACF or ARCF, a routing entry is created in the routing table. Also, the routing table is updated by receiving MGNF. A routing table contains; destination address, next-hop address, expiration timer, number of hops, current_SN, device group ID, and last ACF reception time (T0).

For example, in Figure 8,

- If A wants to join a multicast group, it broadcasts an ACF (assume K=4 hop). The initial routing table for A is as the following table (Note that originator of ACF has to save the ACF Tx time). When C receives the ACF, it creates the routing entry of A in its routing table.

- Since C is not in the multicast group, C forwards the ACF to the others (e.g., D and E). When D and E receive the ACF, they create the routing entry of A in their routing table.

- Since D is in the multicast group, D replies A with ARCF by using routing table. C creates the routing entry of D in its routing table. Suppose C forwarded an ACF at T0. Suppose C receives an ARCF from D at T1. Expiration timer of route entry to D in C’s routing table is updated to Trdst=Tp+Tjmax+T1-T02+(safety margin). At the same time, since E is not in the multicast group, it creates the routing entry of A in its routing table and forwards to the others (e.g., B).  When B receives the ACF, it creates the routing entry of A in its routing table.

- Since B is in the multicast group, B replies A with ARCF. E creates the routing entry of B in its routing table. C forwards ARCF from D to A. A updates a routing entry with expiration timer TrD.

- E forwards the ARCF received from B to C. C creates the routing entry of B in its routing table.

- PD C forwards the ARCF received from E to A.  A creates a routing entry with expiration timer TrB.

- Finally, A is aware of route to B and D.

Figure 7. Creation of Routing Table

5.10.1.4.2 Management of Routing Table

When a node’s routing table is full of routing entries, it chooses a node from its routing table with shortest distance (hop-based) firstly. It sends a MGNF (type 8) to the chosen node and sets timer. When the node receives the MGNF, it breaks the link between the node and itself, and sends an ACF. The node which has timer ignores the ACF and another node which receives ACF sends ARCF to it and creates a new link between the node which sends ACF and itself.

For example, in figure 8,

- A has limited routing entry in its routing table and H wants to join the multicast group. A chooses B in its routing tree randomly. A sends MGNF to B and sets a timer.

- When B receives the MGNF, it breaks the link between A and B.

- B sends ACF. A ignores B’s ACF.

- C receives the ACF and sends ARCF to B.

- Then C creates a link between C and B. Hereby, H can join the multicast group.

Figure 8. Management of Routing Table

Since MGNF (notification type: 1) is sent periodically, a MGNF implosion problem can occur in the network. To prevent this problem, we proposed an adaptive MGNF transmission technique based on the distance between nodes. The distance can be measured from received signal strength indicator. (RSSI) This technique is processed when the distance between nodes becomes longer or shorter. If the node moves away from the network, the MGNF transmission gets more often since there can be a link breakage. Vice versa, if the node moves within one-hop coverage (e.g mobility support), the MGNF transmission gets rarer to prevent redundant MGNF traffic.

5.10.1.5 Leaving From Multicast Group

There are several reasons for a PD to leave from the network: (i) by its intention, (ii) by mobility, (iii) by limited resources. If a PD wants to leave from a multicast group, it multicasts (within K-hop) a MGNF with notification type set to 2. A recipient of MGNF may send rekeying messages to valid group members for forward secrecy. Upon receiving the MGNF, a forwarding PD deletes the entry of the originator of the MGNF, and forward the MGNF. Upon receiving the MGNF, a non-forwarding PD deletes the entry of the originator of the MGNF, but does not forward the MGNF.

In the following figure 9,

- If PD C wants to leave from a multicast group, it multicasts a MGNF with notification type set to 2.

- B receives the MGNF from C and deletes the entry of the originator of the MGNF, and forwards the MGNF. PD D receives the MGNF and it deletes the entry of the originator of the MGNF. Finally, PD C leaves from the multicast group.

Figure 9. Leaving From Multicast Group

5.10.1.6 Mobility Support for Multicast

When a node moves within the one-hop coverage of another node, both of the nodes are aware of each other within the one-hop by MGNF’s TTL and they update their routing table. Then one of them sends a MGNF (notification type: 6) to another neighbor node. Therefore, it can be aware of that the nodes became closer. But, if the node is a multicast group member, it does not do anything. If it is a forwarding PD, it deletes routing entry whose destination field is the both of that nodes.

In figure 10, previously, B was 2-hop away from A. If B moves within the one-hop coverage of A, both of A and B are aware of each other and update their routing table. Then, A or B sends a MGNF (notification type: 6) to C. When C receives that MGNF, C is aware of that A and B became closer. Since C is a forwarding PD, it deletes routing entries whose destination field is A and B.

Figure 10. Mobility Support for Multicast

Since expiration timer is proportional to the number of hops from the impaired PD, the closest multicast group member detects link breakage. A PD starts local repair if it detects link breakage between multicast group members (due to expiration timer). Then, the PD multicasts MGNF (notification type: 7). If PDs receiving the MGNF create routing entry of originator of the MGNF during Tw , the PD performing local repair broadcasts an ACF within K-hop coverage.

Figure 11. Local Repair Procedure for Multicast

If a PD receives the ACF, it compares the receiving frame’s Device Group ID & Application type ID & Application-specific ID & Application-specific group ID with its own. If they all are same, it finds entry the originator of the ACF in its routing table. When it finds the entry, it checks the expiration timer. If the expiration timer is less than Tm, then it replies the PD sending the ACF with an ARCF. (Tm is threshold time to decide reply) In other case, the PD receiving the ACF does not reply. If any of them is not same, it decrements the TTL of the ACF and forwards the ACF.

5.10.1.7 Merging Multicast Groups

Find/join procedure by using ACFs and ARCFs can help merging disjoint groups. If a PD in a group receives an ACF from different disjoint groups with the same Device Group ID & Application type ID & Application-specific ID & Application-specific group ID, it can initiate merging process. Then the PD replies the ACF originator with ARCF. Then, these two disjoint groups are merged by Device ID Creation Scheme. Local repair by using ACFs and ARCFs also can help merging disjoint groups. Each group member performs local repair periodically during TL (long duty cycle) in order to merge disjoint groups.

- 5.10.2 Multicast Data Transmission

If a PD receives a multicast data frame, it has to decide forwarding the frame or not. The PD receiving the multicast data frame compares the source address of the data frame and next-hop address entries of its routing table. PD checks the next hop addresses in its routing table. If it finds one or more next-hop entries which are not overlapped with the source address of the received frame and those next-hop entries have same device group ID with the received frame, then, the PD forwards the incoming data frame to other PDs. Otherwise, the PD does not forward the incoming frame.

For the multicast data transmission, we have to know destination address, source address, originator address, sequence number and time to live (TTL) from multicast data frame. This information should be included in all multicast data frames. Therefore, the header in multicast data frames should contain the followings.

Figure 12. Multicast Data frame

For example, in Figure 13, we assume that 1) A, B, F, and G are in a same group which ID is 1, 2) A, D and H are in a same group which ID is 2, 3) A has routing information of B, D, and G, 4) B has routing information of A and F, and 5) D has routing information of A and H. A multicasts data frame to group 1.(Device Group ID: 1, Application type ID: 3, Application-specific ID: 2, Application-specific group ID: 5).

- C receiving the data frame compares the source address of the data frame and next-hop address entries in its routing table which has same group IDs from the received data frame. C finds any routing entry matching with the source address of the received frame and it searches another routing entry that has same multicast group with different next-hop address fields. Since C found, it will forward the data.

- C forwards the data. E receiving the data frame checks the condition whether forward or not. E will forward the data frame.

- E forwards the data. B and F do not forward the multicast data frame because B and F do not have next-hop address in their routing table, except E.

Figure 13. Multicast Data Transmission

Figure 14. Multicast flowchart

- 5.10.3	Prevention Loopback Problem

PDs can prevent multicast/broadcast loopback problem by using SN (Sequence Number) of data frames. When a PD forwards data with SN = n, the PD sets current_SN field in its routing table to n. If the receiving frame’s SN is not greater than the current_SN , the PD discards the frame.

- 5.10.4	Reliable Multicast

We proposed a Block ACK technique for reliable multicast to know whether the nodes receive Multicast Data Frame fully or not. Thus, when sender PD sends Multicast Data Frame, it chooses the groups in the multicast group by depending on the order of the information of the nodes in its routing table. If the sender wants to have ‘n’ groups, it reached it by dividing the numbers of all nodes in the multicast group to n. ‘n’ is decided dynamically. Then, the sender transmits Multicast Data Frame including the information of the group which sends the block ACK.

When the nodes in the group receive all multicast data frames successfully, they transmit Block ACK to the sender by notifying that they received all frames from it. If they did not receive any data frame or they did not receive it fully, they do not transmit Block ACK to the sender. If sender does not receive Block ACK or receives a negative notification, then it retransmits the data frame to the group. These steps are processed for all groups in the multicast group respectively. If the sender does not have any data frame to transmit, it sets a timer. If the timer is expired, it transmits Request ACK to the groups respectively. The advantage of sending Block ACK is that; nodes send fewer frames than when they send implicit ACK. Thus, there is less collision.

In figure 14, sender PD S transmits Multicast Data Frame to other nodes and chooses the groups. Suppose that S decided that PDs A, B and C are in group1, D, E and F are in group2, G and H are in group3. Then A, B and C transmit Block ACK to S. If S does not have any data frame for PDs D, E and F, then it transmits Request ACK, and they reply S with a Block ACK.

Figure 15. Reliable Multicast

- 5.10.5	Multicast Protocol Using Directional Antenna

If a PD can support directional antenna, it has to have additional routing table field which is describes its beam number forwarding to destination in order to transmit frame. That routing table contains; destination address, next-hop address, expiration timer , number of hops, current_SN, device Group ID, last ACF reception time (T0) and beam number. If each PD wants to join a multicast group, it performs find/join procedure by using all of beams.  During finding/joining procedure, PDs receiving a ACF check the SNR per beams and saves the specific beam with the highest SNR measured in order to find communication beam. If non-multicast group member receives the ACF forwards the ACF by using all of beams. If multicast group member receives the ACF, it replies ARCF by using saved specific beam with the highest SNR measured to the originator of the ACF. When PD receives ARCF, it saves the specific beam with the highest SNR measured into its routing table and forwards the ARCF by using backward path. After updating routing table, all of frames, except broadcast frame, can be transmitted directionally.

For example in Figure 15,

- If PD A wants to join a multicast, it broadcasts ACF by using all of beams.

- Then PD D and C receive the ACF. Since D is a multicast group member, it replies ARCF by using beam 3. Then, A finds the beam with the highest SNR measured and updates its routing table. C is a non-multicast group member then forwards ACF by using all of beams.

- Since F is a multicast group member, it replies ARCF by using beam 4. Then, C receives ARCF and updates its routing table.  Also, E forwards its ACF by using all of beams.

- When A receives ARCF from C, it updates its routing table. Also, B replies ARCF by using beam 1. Then, E receives ARCF and updates its routing table.

- Also, E forwards ARCF by using beam 1. Then, C receives ARCF and updates its routing table.

- C forwards ARCF by using beam 1. Then, A receives ARCF and updates its routing table.

Figure 16. Multicast Protocol Using Directional Antenna

</388r0>

<396r1>

Group Operation

Group operation with an operation mode is based on a superframe structure. A superframe consists of a temporary beacon (TB), a contention free period (CFP), and a contention access period (CAP). The clock of TB is synchronized to the first TS as shown in Figure 4. The length of TB needs to be decided according to the data size assigned but it is much larger than TS. CAP is used by J-PD as backoff interval in the group operation to be described shortly. CFP is used for group communication among PDs in the group.

Contention Access Period

Joining nodes will choose a random time in the contention period to transmit a JoinRequest packet. They will repeat the transmission if they are not included in the next GroupAnnounce transmission from the PD.

Contention-Free Period (TBD)

After joining the group, nodes will transmit packets in the contention free part of the superframe. The contention-free period could use token-based polling, as in the PCF of IEEE802.11, or fixed slot assignment.

Figure 4 Superframe structure.

Figure 5 Group operation procedure.

The group operation procedure is shown in Figure 5. The I-PD broadcasts group forming invitation (active scanning) within a TS including information of necessary IDs, selected operation mode (including channel), and group clock.

When a J-PD scans the invitation (passive scanning) at common mode, it moves to the selected operation mode that was announced by I-PD. Accounting from the start of TS, the J-PD waits for a duration of TB+CFP. Then, it sends joining request including its ID information with a random backoff within a duration of CAP.

When a J-PD scans the TS in an iteration, it will calculate the difference between t_now and t_start. If t_start is within a CAP, the J-PD takes random backoff between t_start and the end of CAP and sends joining request. If t_start is out of a CAP, it waits until the next CAP before sending request.

I-PD scans the selected operation mode (passive scanning), registers the J-PDs and distributes a list of the registered PDs within the temporary beacon (TB) at the selected operation mode. J-PDs that had sent joining requests but are not included in the distributed PDs list should repeat the joining process.

</396r1>

5.4.3

5.4.4

<Billy>

UWB PHY PLACEHOLDER: In general MAC layer procedures should be applicable on top of the UWB PHY, however this place holder is to note that there may be some PHY specific elements to the communications procedures that need to be considered and noted here – e.g. MAC provisions for ranging and localization.

</Billy>

MPDU structure

<368r2>

The MPDU consist of MAC frame header, MAC frame information, MAC frame payload, and MAC frame tail. The MAC frame header contains frame control, peer network identifier, peer device addressing fields, and peer network authenticator. The MAC frame information contains peer network information elements.

The peer network identifier consists of service class of peer network, peer group service profile identifier, and local peer network identifier.

Figure 5.1- Structure of MAC frame MPDU

</368r2>

<380r2>

General MAC Frame:

Frame Type and Subtype fields together indicate the type of a frame, i.e., the function of a frame.

Required ACK Type field in frame control specifies what type of acknowledge frame is expected. For example, no ACK or aggregated ACK.

Addressing field Indication field indicates the presence of the transmitting hop address and receiving hop address in the addressing fields.

Addressing fields consist of the following addresses: source address, destination address, transmitting hop address and receiving hop address. Transmitting hop address and receiving hop address are optional.

P2PNW/APP ID field shows the P2P network ID or application ID in the MHR. All the peers joining a P2PNW will have a locally unique P2PNW/APP ID. If P2PNW ID is not determined when a frame is sent, this field will carry application ID.

Application Type field indicates the application/service category, such as emergency service, social networking, smart office, etc.

Hopper Indication is used to indicate if the frame sender is willing to relay other frames for the multi-hop discovery.

</380r2>

<388r0>

Packet Format

- Table 1. ACF (Advertisement Command Frame)

| Name | Type | Description |
| --- | --- | --- |
| Originator Address | Integer | Originator PD’s Address |
| Destination Address | Integer | Destination PD’s Address |
| Sender Address | Integer | Sender PD’s Address |
| Application Type ID | Integer | Application Type ID |
| Application Specific ID | Integer | Application Specific ID |
| Application Specific Group ID | Integer | Application Specific Group ID |
| Device Group ID | Integer | Device Group ID |
| Time To Live (TTL) | Integer | Time To Live (TTL) |
| Sequence Number | Integer | Frame’s Sequence Number |
| Frame Type | Integer | Frame Type |
| Sub Type | Integer | Frame’s Sub Type |

- Table 2. ARCF (Advertisement Reply Command Frame)

| Name | Type | Description |
| --- | --- | --- |
| Originator Address | Integer | Originator PD’s Address |
| Destination Address | Integer | Destination PD’s Address |
| Sender Address | Integer | Sender PD’s Address |
| Receiver Address | Integer | Receiver PD’s Address |
| Application Type ID | Integer | Application Type ID |
| Application Specific ID | Integer | Application Specific ID |
| Application Specific Group ID | Integer | Application Specific Group ID |
| Device Group ID | Integer | Device Group ID |
| Sequence Number | Integer | Frame’s Sequence Number |
| Frame Type | Integer | Frame Type |
| Sub Type | Integer | Frame’s Sub Type |

- Table 3. MGNF (Multicast Group Notification Frame)

| Name | Type | Description |
| --- | --- | --- |
| Originator Address | Integer | Originator PD’s Address |
| Destination Address | Integer | Destination PD’s Address |
| Sender Address | Integer | Sender PD’s Address |
| Receiver Address | Integer | Receiver PD’s Address |
| Application Type ID | Integer | Application Type ID |
| Application Specific ID | Integer | Application Specific ID |
| Application Specific Group ID | Integer | Application Specific Group ID |
| Device Group ID | Integer | Device Group ID |
| Time To Live (TTL) | Integer | Time To Live (TTL) |
| Sequence Number | Integer | Frame’s Sequence Number |
| Notification Type | Integer | Notification Type (Type: 0, 1, 2, 3, 4, 5, 6, 7, 8) |
| Frame Type | Integer | Frame Type |
| Sub Type | Integer | Frame’s Sub Type |

- Table 4. Multicast Data Frame

| Name | Type | Description |
| --- | --- | --- |
| Originator Address | Integer | Originator PD’s Address |
| Destination Address | Integer | Destination PD’s Address |
| Sender Address | Integer | Sender PD’s Address |
| Receiver Address | Integer | Receiver PD’s Address |
| Application Type ID | Integer | Application Type ID |
| Application Specific ID | Integer | Application Specific ID |
| Application Specific Group ID | Integer | Application Specific Group ID |
| Device Group ID | Integer | Device Group ID |
| Time To Live (TTL) | Integer | Time To Live (TTL) |
| Sequence Number | Integer | Frame’s Sequence Number |
| Originator Sequence Number | Integer | Originator Sequence Number |
| Notification Type | Integer | Notification Type (Type: 0, 1, 2, 3, 4, 5, 6, 7, 8) |
| Frame Type | Integer | Frame Type |
| Sub Type | Integer | Frame’s Sub Type |
| Payload | Integer | Multicast Data |

</388r0>

<395r1>

Figure 3. PAC Frame formats

- PPDU structure

PPDU consists of preamble, physical header, and PSDU. Preamble is sequence(s) for timing offset, frequency offset and channel estimation, etc. Physical header is physical information such as bandwidth, PPDU length, etc.

- MPDU structure

MPDU consists of MAC header, frame body, and FCS. MAC header comprises data frame type, device IDs, service type IDs, MCS, MPDU length, etc. Frame body is data information. FCS contains an IEEE 32-bit CRC.

</395r1>

<396r1>

A MAC frame consists of a fixed-length MAC header, a variable-length MAC frame body, and a fixed-length FCS field as shown in Figure 2. The MAC frame body has an octet length L_FB such that 0 ≤ L_FB ≤ pMaxFrameBodyLength, and is present only if it has a nonzero length.

Figure 2 MAC frame format

</396r1>

<Billy>

Frame Check Sequence (FCS)

<Note to editor: the text below is the standard text for FCS; this should be placed as a sub-clause within the clauses that deal with the general MAC frame format>

The FCS comes at the end of all frames. The FCS is 2 octets in length and contains a 16-bit ITU-T CRC. The FCS is calculated over the complete frame beginning with the Frame Control (FC) octet(s).  The FCS shall be calculated using the following standard generator polynomial of degree 16:

G16(x) = x16+x12+x5+1

The FCS shall be calculated for transmission using the following algorithm:

- Let M(x) = b0xk–1 + b1xk – 2 +…+ bk–2x + bk – 1 be the polynomial representing the sequence of bits for which the checksum is to be computed.

- Multiply M(x) by x16, giving the polynomial x16 × M(x).

- Divide x16 × M(x) modulo 2 by the generator polynomial, G16(x), to obtain the remainder polynomial, R(x) = r0x15 + r1x14 +…+ r14x + r15.

- The FCS field is given by the coefficients of the remainder polynomial, R(x).

Here, binary polynomials are represented as bit strings, in highest polynomial degree first order.

As an example, consider an acknowledgment frame with no payload and the following 3 byte header:

0100 0000 0000 0000 0101 0110 	[leftmost bit (b0) transmitted first in time]

b0................................................................b23

The FCS for this case would be the following:

0010 0111 1001 1110				[leftmost bit (r0) transmitted first in time]

r0.......................................r15

A typical implementation is depicted in Figure 14.

1. Initialize the remainder register (r0 through r15) to zero.

2. Shift header and payload into the divider in the order of transmission (LSB first).

3. After the last bit of the data field is shifted into the divider, the remainder register contains the FCS.

4. The FCS is appended to the data field so that r0 is transmitted first.

Figure 14 – typical FCS implementation

</Billy>

Multiple access

e.g. Contention-based access, Contention-free access

<368r1>

The PAC enabled network provides resource allocation configuration primitives to the next higher layer of the peer network initiator. The resources, which may be temporal or spectral or spatial, are slotted and allocated to the components of the peer group on the whole time frame from the start to the end of the peer network.

The life cycle of the peer group from forming to disbanding is specified by combining the phases: synchronization phase, discovering phase, peering phase, data phase, disbanding phase, and handshaking phase. Any combinations are possible and a combination of the phases can be iterated except the disbanding phase. In each phase, the slotted resources are allocated to initiator, proxy initiator, peers, relay, and inactive period. Any combinations of allocation order are possible and a combination of the allocation order can be iterated.

The combination of the phase and the combination of allocation order of each phase are designed prior to support a service which is specified with the technical attributes defined in application matrix of IEEE 802.15.8 (15-12-0684). According to the application matrix, the generic configurations of resource allocation are registered as peer group service profile identifiers. The peer group service profile identifier is contained in the peer network identifier and the observers of the peer network may listen and can recognize the schedule of resource allocation. By obtaining the neighbor peer groups’ resource allocation information with the implicit and distributed manner, the PAC enabled devices can perform the proactive interference avoidance.

Figure 5.2- Combination of phase and the combination of resource allocation order

Figure 5.3- Comparison with IEEE 802.15.4 superframe and 15.4e multi-superframe

</368r1>

<373r1>

Basic Operation

A PD operating under PAC random access method transmits a pending MPDU when the device determines that the medium is idle for a DIFS period, or an EIFS period if the immediately preceding medium-busy event was caused by detection of a frame that was not received at this PD with a correct MAC FCS value. In these conditions, the PD shall follow the backoff procedure described in 5.2.2 for the pending MPDU transmission. It is necessary that a PD that has completed network entry procedure shall maintain time and frequency synchronization with neighbouring devices operating under PAC random access method. The basic access mechanism is illustrated in Figure 1.

Figure 1 – Basic access mechanism.

Backoff Procedure

All PDs participating in multiple channel access shall maintain a CW (Contention Window). A device attempting to access wireless channel shall select an integer drawn from uniform distribution over [0, CW-1]. Then, the chosen number is decreased by one every idle slot. The device shall stop the decrement when it senses the wireless channel busy. The device transmits the pending frame when its CW reaches 0.

PDs shall detect frame transmission collisions in the air by monitoring the channel. A collision occurs if more than one PDs attempt to transmit their frames at the same time. The collision detection information shall be used by PDs to increase or decrease their CWs. The change of CW follows EIED (Exponential Increase Exponential Decrease) algorithm described in the following paragraphs.

When a PD monitoring the channel detects a collision, it shall increase its CW exponentially with factor rI (>1). On the other hand, a PD monitoring the channel shall decrease its CW exponentially with factor rD (>1) when it does not detect a collision for DP (Decrement Period) since the last CW update. The upper and lower bounds of CW are CWmax and CWmin, respectively.

In case CW is not an integer value, the value is rounded to the largest integer smaller than the CW by floor function, and the integer value is used to determine the uniform distribution of the random variable used for random access . The parameters are given in Table 1:

Table 1 – Multiple access parameters.

| Parameters | value |
| --- | --- |
| rI | TBD |
| rD | TBD |
| DP | TBD |
| CWmin | TBD |
| CWmax | TBD |
| Slot time | TBD |
| SIFS time | TBD |
| PIFS time | SIFS + one slot time |
| DIFS time | PIFS + one slot time |

Collision Detection

The PHY of a PD overhearing wireless channel shall report collision detection information to the MAC of the PD. If the PHY does not detect a collision for DP since the last report, the PHY shall report the information to the MAC of the PD. Whenever the information is reported, the device shall adjust its CW according to the backoff procedure described in 5.2.2.

RTS/CTS Frame Exchange Procedure

A PD exchanges RTS/CTS frames prior to sending the MPDU. The CTS corresponding to an RTS is expected to arrive at the device SIFS time after or at the latest by SIFS plus [TBD]. The data rates for the RTS and CTS frames are [TBD]:

Unicast Frame Transmission Procedure

A PD with an individually addressed pending frame shall wait for corresponding ACK frame from the addressing device. In the absence of an ACK frame reception, the PD shall retry the frame transmission by following the random backoff procedure described in 5.2.2. The ACK corresponding to a unicast frame is expected to arrive at the PD SIFS time after or at the latest by SIFS plus [TBD]. If the PD fails to transmit unicast frame successfully, it repeats the transmission trials [TBD] times until successful frame transmission.

</373r1>

<377r0>

IEEE802.15.8 PAC shall consist of several types of frames to serve different operations, based on contention-free channel-access scheme. The several different frames constructs PAC superframe. IEEE802.15.8 PAC may have contention-based channel-access during a separated frame.

    - PAC Frame Structure

IEEE802.15.8 PAC superframe comprises of Sync frame, Discovery frame, Peering frame, and Data frame.

</377r0>

<395r1>

The fundamental access method of the IEEE802.15.8 PAC is CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance).

For a PD to transmit, it shall sense the medium to determine if another station is transmitting. If the medium is not determined to be busy, the transmission may proceed. The CSMA/CA distributed algorithm mandates that a gap of a minimum specified duration exists between contiguous frame sequences. A transmitting PD shall verify that the medium is idle for this required duration before attempting to transmit. If the medium is determined to be busy, the PD shall defer until the end of the current transmission. After deferral, or prior to attempting to transmit again immediately after a successful transmission, the PD shall select a random backoff interval and shall decrement the backoff interval counter while the medium is idle. A transmission is successful when an ACK frame is received from the STA addressed of the transmitted frame.

</395r1>

<392r1>

The multiple access scheme of IEEE 802.15.8 is contention-free multiple access with orthogonal requests. Contention-free multiple access consists of exchanging DS-REQ (Distributed Scheduling – Request) and DS-RSP (Distributed Scheduling – Response) between the peered PDs.

Once peering is over, available data channels used are assigned to the peered PDs by data channel mapping. In the assigned data channel, resources for request and response signals are determined by SP(Scheduling Priority) mapping.

For a PD trying to send DS-REQ, it shall sense the air medium to check to see if interferences are produced from heterogeneous devices. If the air medium is determined to have no interference, the PD may transit DS-REQ.

The PD shall transmit SRI(Scheduling Request Indicator) before transmitting DS-REQ. After the transmission of SRI, the PD shall transmit DS-REQ in the scheduling interval to request resource allocation in the data interval for data transmission. The other peered PD shall transmit DS-RSP in the scheduling interval in a response to the received DS-REQ signal. The information delivered by the exchange of DS-REQ and DS-RSP is used by the distributed scheduling algorithm.

</392r1>

<380r2>

PAC Frame Structure

Figure: General Hierarchical Frame Structure

- Superframe Beacon: start of a Superframe. It also indicates the superframe structure, such as the length of common channel, number of application frames, and length of reserved time duration. It may be used as synchronization reference also.

- Common Channel: shared by all peers and applications in proximity - both public broadcasting / multicasting and private pair communications via contention based accessing.

- Application Frame: dedicated to an application. There may be one or multiple Application Frames within a Superframe. An application frame consists of a dedicated channel and contention free period. Dedicated channel is shared among all peers within the application group, and is contention based for accessing. Contention free period is allocated to individual peers.

- Reserved Time: reserved for the insertion of other application frames.

- Inactive Period: optionally as the gap or guard time between Superframes.

- Hyperframe: top level frame. It may include several Superframes.

</380r2>

Synchronization procedure

<368r1>

The MAC sublayer provides a network synchronization procedure for peers to align to the current phase. In synchronization phase, initiator or the proxy initiator of the peer group transmits the peer group advertisement frame every Td over minimum synchronization adjust interval. The peers receive the sequential advertisement frame and adjust the length of unit resource slot.

Figure 5.4- Synchronization for unit resource slot length adjustment

</368r1>

269r2

<373r1>

It is necessary that a PD that has completed network entry procedure shall maintain time and frequency synchronization with neighbouring devices operating under PAC random access method. Timing and frequency synchronization is achieved by overhearing the frames transmitted by neighbouring PDs.

The accuracy of timing synchronization is in slot level, and the maximum timing error should be less than half the CP duration.

</373r1>

<377r0>

IEEE802.15.8 PAC follows distributed synchronization procedure without any single master PD to get reference timing. The distributed synchronization is proper to flat and scalable network to be supported by PAC. Because two PDs not being discovered are not able to make connection, synchronization procedure shall be prior to peering (link establishment). Moreover, it is better to be prior to discovery procedure to enhance peer discovery performance. Obviously, it helps to get higher throughput by efficient signalling and data transmission as well.

A PD shall be in synchrony state prior to peer discovery procedure and peering procedure.

IEEE802.15.8 PAC has two synchronization mode including Initial Synchronization mode and Maintaining Synchronization mode.

    - PAC synchronization modes and procedure

Initial Synchronization mode:

- Start in Initial Synchronization mode.

- PD monitors Synchronization Signals (SSs) during synchronization period.

- If at least one SS is detected during synchronization period, perform according to distributed synchronization mechanism.

- Else, start PAC operations based on frame structure in Maintaining Synchronization mode.

Maintaining Synchronization mode:

- PD sends SS periodically, but checks synchrony state via Blank subframe sometimes without sending SS.

- If in-synchrony, PD adjusts oscillator for phase drift compensation.

- If out-of-synchrony, go to initial synchronization mode.

Figure 1. Synchronization Frame Structure

Synchronization frame consists of several redundant SSs to be robust to channel fading. Blank subframe is in the middle of several SSs.

The distributed synchronization mechanism is designed based on classical PCO (Pulse Coupled Oscillator) synchronization algorithm. According to PCO algorithm, a PD assumes to have an oscillator which can fasten the own phase when receiving pulse as Figure 1. This adjustment is controlled by the predefined function  which has the own phase value as an input. If there is no other pulse detected, there is no change but normal phase increment according to time advance.

Figure 2. Oscillator Phase Transition

The overall PCO synchronization steps can be imagined from Figure 2. The phase value of each node is mapped to on the edge of circle. At first (a) phase, all nodes start randomly, so each node has a different phase value at a certain instant time. When node A increases the phase and reaches the maximum value (1 as normalized one), it fires Synchronization Signal (SS) to medium as the same role to pulse of original algorithm. Other nodes receiving the SS adjust the own oscillator to change the phase value according to predefined rule. Through these interactions with coupled oscillator, all nodes can achieve to reach the synchrony in a time as shown from Figure 2 (c).

Figure 3. PCO Synchronization Steps

- All nodes have oscillator with the same phase increment rate

- One node fires, then other nodes adjust oscillator according to the predefined function without state other than it’s internal phase

- Finally, all nodes converges to the same time base

The equation for phase adjustment is as follows:

To get synchrony, all nodes follow the same rule based on phase adjustment curve . The phase adjustment curve  is described by the non linear curve to represent mapping relation between the value  and the corresponding phase. The curve should be concave down for synchrony condition. The dissipation factor  has to be larger than zero. Using phase adjustment curve, adjusted phase value is calculated by following rule:

To provide fast convergence, selective update is adopted as following rule:

If  is met, the adjusted phase value is determined by the following rule:

If  is not met, there is no phase update.

To avoid ping-pong effect in scalable network environment, refractory period is decided during the time when the phase value has the following condition:

There is no phase update during refractory period.

    - Synchronization procedure for operations in unlicensed band

The synchronization procedure with energy sensing is designed for operations in unlicensed band to coexist with different systems sharing the same band.

This procedure is enabled only in initial synchronization mode.

- A PD senses energy level while doing operation for distributed synchronization.

- If medium is busy, the PD pends synchronization operation. Else, the PD keeps synchronization operation.

- If medium is not busy and SS is not detected during synchronization period, Superframe starts in Maintaining Synchronization mode.

Figure 4. Synchronization Operation with Energy Sensing

</377r0>

396r1: No text proposal found

380r2

<392r1>

IEEE 802.15.8 follows a synchronization procedure in the distributed manner. A PD shall perform a synchronization procedure before performing a discovery procedure. A PD shall search the start of frame and ultraframe in sequence. A PD shall transmit or receive the synchronization signal for distributed synchronization.

</392r1>

Discovery procedure

<368r1>

The peer discovery is carrying out during discovery phase by transmitting a peer discovery frame, which contains peer network identifier and peer network information elements. The peer group service profile is identified by the peer network identifier. The details on the combination of phase and allocation order are contained in peer network descriptor. The peer discovery frame is transmitted every Td over minimum discovery interval.

</368r1>

<377r0>

IEEE802.15.8 PAC shall have a periodic Discovery frame per a superframe.

Discovery frame is comprised of multiple Discovery slots as Figure 5. A Discovery Slot delivers single Peer Discovery Message (PDM). A PDM contains a PDI(Peer Discovery Identity) and a PDI label to present the type or usage of PDI.

The procedure to select a Discovery Slot is as follows:

- A PD selects one Discovery Slot.

- The PD broadcasts Peer Discovery Message at the selected Discovery Slot.

- The PD monitors congestion level by energy sensing.

- If congested, the PD selects different Discovery Slot for next transmission.

- If not congested, the PD keeps the current Discovery Slot.

</377r0>

<379r0>

    - Low Energy Service Discovery (LESD) protocol

Some PDs in a PAC WPAN may have multiple PHYs for PAC services. They shall either join an existing PAC WPAN among WPANs operating in different channels (or frequency bands) or start new PAC WPAN to initiate PAC service. To join an existing PAN, PD needs to search services available and frequency channels in use. PDs may use LESD to discover PAC service of interest and operating channel for communication if macLESDenabled is true.

In this sub-clause, operation of LESD protocol is described. Figure 1 illustrates state transition of LESD protocol. If macLESDcapable is TRUE and a PD wishes to use LESD protocol, macLESDenabled is set to TRUE. The PD with macLESDenabled of TRUE shall perform passive LESD scan as described in 5.4.1.1. If service of interest is discovered, the PD switches to channel sampling (CS) state as described in 5.4.1.3 to help neighboring PDs to discover the service and save energy consumption devoted to service discovery procedure. Meanwhile, the PD joins the PAC WPAN discovered during passive LESD scan.

If the PD fails to discover the service during passive LESD scan, PD performs an active LESD scan as described in 5.4.1.2. During active scan, the PD broadcasts LESD request command (as described in 5.4.2.1) to neighbor PDs. If LESD response commands are received, the sender PD updates corresponding MAC PIB attributes and reports the reception of command frame to the higher layer. Also, MAC sublayer broadcasts LESD notification command so that PDs nearby are notified of the service available. The neighbor PDs may not send LESD request command if LESD response commands received or LESD notification commands received contain Service ID field that the PDs are searching for. If the PD fails to receive response command or notification command for at most macLESDResponseWaitTime, the PD broadcast LESD request command again. The PD repeatedly attempts to send LESD request command if the number of retrial does not exceed macMaxLESDRequestRetries. When service of interest is discovered, the PD switches to channel sampling (CS) state to help neighboring PDs to discover the service and save energy consumption devoted to service discovery procedure.

If the PD fails to discover the service during active LESD scan, the PD starts new PAC WPAN. Meanwhile, the PD switches to channel sampling (CS) state to notify neighboring PDs that new service is available by response to the LESD request command. Details of each state are described in the following sub-clauses.

Figure 1—LESD state diagram

      - Passive LESD scan

LESD-enabled PDs (i.e., macLESDenbaled is TRUE) shall be capable of performing passive scan. A PD is instructed to begin a channel scan for service discovery through the MLSDE-LESD-SCAN.request primitive as described in 5.4.3.1. The next higher layer should submit a scan request for PAC services to discover specified by ServiceID over the channel specified by LESDChannelID.

On the receipt of SCAN.request primitive, the MLSDE of the PD shall update macServiceID to the value of ServiceID parameter and macAvailableChannelID to the value of AvailableChannelID parameter. For the duration of the scan, LESD PHY shall not attempt to decode frame being received and discard it whose SFD and preamble are not relevant to the LESD response or LESD notification command frame. The MAC sublayer shall discard all frames received whose Service ID field do not match to macServiceID. Otherwise, MLSDE of the PD shall report that LESD response command frame via MLSDE-LESD.confirm primitive as described in 5.4.3.6, or indicate LESD notification command frame via MLSDE-LESD-NOTIFY.indication primitive as described in 5.4.3.4.

PD performs passive LESD scan for at most macCSInterval. The results of the scan shall be reported to the higher layer via the MLSDE-LESD-SCAN.confirm primitive as described in 5.4.3.2.

If PD successfully discovers the PAC service and channel IDs in use for the service, the higher layer may wish to join the PAN by issuing MLME-ASSOCIATE primitives. While the PD attempts to join the PAN, MLSDE is instructed to switch to CS state by the higher layer via MLSDE-LESD-CS.request primitive as described in 5.4.3.8.

If PD does not discover the PAC service when it completes the passive LESD scan, PD may search the service by performing an active LESD scan procedure described in xxx.

Figure 2 illustrates the message sequence chart for passive LESD scan.

Figure 2—Passive LESD scan message sequence chart

      - Active LESD scan

If PD fails to discover the PAC service during passive LESD scan, the device attempts to search the service using active LESD scan. The higher layer commences active LESD scan by issuing MLSDE-LESD.request primitive as described in 5.4.3.3. On the receipt of MLSDE-LESD.request primitive, MLSDE generates LESD request command as described in 5.4.2.1 and attempts to broadcast the command frame via CSMA-CA algorithm.

On the receipt of LESD request command frame, MLSDE of the neighboring PDs shall discard the frame if the Service ID field does not match to the macServiceID. Otherwise, MLSDE shall report the reception of the command frame to the higher layer via MLSDE-LESD.indication primitive as described in 5.4.3.4.

In response to MLSDE-LESD.indication primitive, the higher layer of the neighboring PD shall issue MLSDE-LESD.response primitive as described in 5.4.3.5 to MLSDE. On the receipt of the primitive, MLSDE shall generate LESD response command frame as described in 5.4.2.2 and broadcast it via CSMA-CA algorithm.

On the receipt of the response command frame, MLSDE of PDs shall discard the frame if the Service ID field does not match to the macServiceID. If the value of Service ID field of the LESD response command matches to macServiceID, MLSDE updates macCommunicationChannelID with the value of CommunicationChannelID field in the received command, and issues MLSDE-LESD.confirm primitive as described in 5.4.3.6 to the higher layer to report the reception of the command frame.

Also, MLSDE generates LESD notification command as described in 5.4.2.3 and broadcast it, so that the neighboring PDs that wish to discover the PAC service with ServiceID of interest can locate the service.

If a PD receives LESD notification with the Service ID field matching to macServiceID, MLSDE of the device updates macCommunicationChannelID with the value of CommunicationChannelID field in the received notification command.

If the PD does not receive response command frame for at most macLESDResponseWaitTime after sending LESD request command, MLSDE shall increase macNumLESDRequestRetries and broadcast LESD request command if macNumLESDRequestRetries does not exceed macMaxLESDRequestRetries. If macNumLESDRequestRetries exceeds macMaxLESDRequestRetries., MLSDE reports to the higher layer that active LESD scan does not complete successfully by issuing MLSDE-SCAN.confirm with the Status parameter of EXCEED_NUM_TRIAL.

If a PD does not discover the PAC service successfully, the higher layer may initiate the PAN by issuing MLME-START primitives. While the PD attempts to start the PAN, MLSDE is instructed to switch to CS state by the higher layer via MLSDE-LESD-CS.request primitive as described in 5.4.3.8.

Figure 3 illustrates the message sequence chart for active LESD scan.

Figure 3—Active LESD scan message sequence chart

      - Channel Sampling (CS)

Channel Sampling (CS) procedure allows PDs to save the energy devoted to service discovery. For PDs that discover the PAC service successfully, the higher layer initiate CS procedure by issuing MLSDE-CS.request primitive described in 5.4.3.8.

On the receipt of MLSDE-CS.request primitive, MLSDE shall set the macLESDdone to TRUE and updates the MAC PIB attributes, macCSInterval and macCSDuration with the values of CSInterval parameter and CSDuration parameter of the primitive. The device is instructed to sample the channel every macCSInterval with the duration of macCSDuration. During macCSDuration, PD waits LESD request command containing service ID field matching to macServiceID. If LESD request command is not received during macCSDuration, the PD shall turn off its radio as illustrated in Figure 4.

Figure 4—Illustraton of Channel Sampling Procedure

On the receipt of LESD request command frame, MLSDE of the recipient PDs shall discard the frame if the Service ID field does not match to the macServiceID. Otherwise, MLSDE shall report the reception of the command frame to the higher layer via MLSDE-LESD.indication primitive.

In response to MLSDE-LESD.indication primitive, the higher layer of the neighboring PD shall issue MLSDE-LESD.response primitive to MLSDE. On the receipt of the primitive, MLSDE shall generate LESD response command frame and broadcast it via CSMA-CA algorithm. PD shall enable the receiver until the transmission of LESD response command is completed.

Figure 5 illustrates the message sequence chart for CS procedure.

Figure 5—message sequence chart for CS procedure

    - LESD MAC command frames

      - LESD request command

The LESD request command allows a PD to request service discovery to neighboring PDs to response service and RF channels matches to the requesting PD. The LESD request command shall be formatted as illustrated in Figure 6.

| Octets: | 1 | 1 |
| --- | --- | --- |
| MHR fields | Service ID | Available Channel ID |

Figure 6—LESD request command format

        - MHR field

        - Service ID field

The Service ID field shall indicate the ID of PAC service that the PD wishes to search. The value of this field is set to the value of ServiceID parameter of MLSDE-LESD.request primitive as described in 5.4.3.3.

        - Available Channel ID field

The Available Channel ID field shall indicate the channel numbers of the PD is capable of use. The value of this field is set to the value of AvailableChannelID parameter of MLSDE-LESD.request primitive as described in 5.4.3.3.

      - LESD response command

The LESD response command allows a PD to reply the result of a LESD request. The LESD response command shall be formatted as illustrated in Figure 7.

| Octets: | 1 | 1 |
| --- | --- | --- |
| MHR fields | Service ID | Communication Channel ID |

Figure 7—LESD response command format

        - MHR field

        - Service ID field

The Service ID field shall indicate the service ID of the WPAN that the PD is associated with. The value of this field is set to the value of Service ID parameter of MLSDE-LESD.response primitive as described in 5.4.3.5.

        - Communication Channel ID field

The Communication Channel ID field shall indicate the operating channel number for the PAC service of the WPAN that the PD is associated with. The value of this field is set to the value of CommunicationChannelID parameter of MLSDE-LESD.response primitive as described in 5.4.3.5.

      - LESD notification command

The LESD notification command allows a PD to announce the result of its service discovery request to the neighboring PDs. The LESD request command shall be formatted as illustrated in Figure 8.

| Octets: | 1 | 1 |
| --- | --- | --- |
| MHR fields | Service ID | Communication Channel ID |

Figure 8—LESD request command format

        - MHR field

        - Service ID field

The Service ID field shall indicate the service ID of the PAC service. The value of this field is set to the value of Service ID field of LESD response command as described in 5.4.2.2.

        - Communication Channel ID field

The Communication Channel ID field shall indicate the operating channel number for the PAC service. . The value of this field is set to the value of Communication Channel ID field of LESD response command as described in 5.4.2.2.

    - LESD MAC Primitives

      - MLSDE-LESD-SCAN.request

This primitive allows a LESD-enabled PD (i.e., macLESDenabled is TRUE) to perform a passive scan for PAC service discovery.

The semantics of this primitive are:

MLSDE-LESD-SCAN.request	(

ServiceID,

LESDChannelID,

AvailableChannelID

)

Table 1 specifies the parameters for the MLSDE-LESD-SCAN.request primitive.

Table 1—MLSDE-LESD-SCAN.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ServiceID | Integer | 0x0-0xf | PAC service ID to search. |
| LESDChannelID | Integer | 0x0000-0xffff | Channel number for service discovery. |
| AvailableChannelID | Set of octets | 0x0000-0xffff for each element | List of channel numbers that PD is capable of use. |

The MLSDE-LESD-SCAN.request primitive is generated by the higher layer of a PD and issued to its MLSDE to perform a passive scan.

On receipt of the MLSDE-LESD-SCAN.request primitive, the MLSDE of the device shall update MAC PIB attributes macLESDChannelID, macServiceID and macAvailableChannelID to the values of LESDChannelID, ServiceID parameter and AvailableChannelID parameter respectively. Then, MLSDE shall set a timer for macCSInterval and turn the radio on to perform the passive scan on the channel specified by macLESDChannelID.

      - MLSDE-LESD-SCAN.confirm

This primitive reports the results of the LESD scan request to the higher layer of the device.

The semantics of this primitive are:

MLSDE-LESD-SCAN.confirm	(

Status

)

Table 2 specifies the parameters for the MLSDE-LESD-SCAN.confirm primitive.

Table 2—MLSDE-LESD-SCAN.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, SCAN_IN_PROGRESS, INVALID_PARAMETER | The result of the passive LESD scan request. |

If LESD response commands or LESD notification commands with Service ID parameter matching to macServiceID are received during the scan duration, MLSDE shall generate corresponding primitives and report the reception of the commands to the higher layer. When timer for passive scan expires, MLSDE shall set the Status parameter to SUCCESS and report the completion of scan procedure to the higher layer via MLSDE-LESD-SCAN.confirm primitive.

If PD does not receive a LESD response command or a LESD notification commands with Service ID parameter matching to macServiceID until the timer for passive scan expires, MLSDE shall set the Status parameter to NO_SERVICE_FOUND and report the completion of scan procedure to the higher layer via MLSDE-LESD-SCAN.confirm primitive.

If the MLSDE receives the MLSDE-LESD-SCAN.request primitive while performing a previously initiated scan operation, the MLSDE shall not perform the passive scan and the status parameter will be set to SCAN_IN_PROGRESS.

      - MLSDE-LESD.request

This primitive allows a LESD-enabled PD (i.e., macLESDenabled is TRUE) to request a service discovery.

The semantics of this primitive are:

MLSDE-LESD.request	(

ServiceID,

AvailableChannelID

)

Table 3 specifies the parameters for the MLSDE-LESD.request primitive.

Table 3—MLSDE-LESD.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ServiceID | Integer | 0x0-0xf | PAC service ID to search. |
| AvailableChannelID | Set of octets | 0x0000-0xffff for each element | List of channel numbers that PD is capable of use. |

The MLSDE-LESD.request primitive is generated by the higher layer of a PD to initiate active scan and issued to its MLSDE to discover PAC service specified by ServiceID parameter.

On receipt of the MLSDE-LESD.request primitive, the MLSDE of the device shall update MAC PIB attributes macServiceID and macAvailableChannelID to the values of ServiceID parameter and AvailableChannelID parameter respectively. MLSDE shall generate LESD request command as described in 5.4.2.1 and broadcast the command frame. Then, MLSDE shall increase macMaxLESDRequestRetries by one and set a timer for macLESDResponseWaitTime to wait for a LESD response command frame.

      - MLSDE-LESD.indication

This primitive reports the reception of a LESD request command.

The semantics of this primitive are:

MLSDE-LESD.indication	(

ServiceID,

AvailableChannelID,

)

Table 4 specifies the parameters for the MLSDE-LESD.request primitive.

Table 4—MLSDE-LESD.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ServiceID | Integer | 0x0-0xf | PAC service ID to search. |
| AvailableChannelID | Set of octets | 0x0000-0xffff for each element | List of channel numbers that PD is capable of use. |

On receipt of LESD request command, the MLSDE shall check the value of Service ID field of the command. If the value matches to the macServiceID of the PD, the MLSDE shall generate MLSDE-LESD.indication primitive and notify the reception of the command. If the value does not match to the macServiceID of the PD, the MLSDE discard the received command.

On receipt of the MLSDE-LESD.indication primitive, the higher layer may issue MLSDE-LESD.response primitive to MLSDE if the value of ServiceID parameter matches to macServiceID of the recipient PD.

      - MLSDE-LESD.response

This primitive allows the next higher layer of a device to respond to the MLSDE-LESD.indication primitive.

The semantics of this primitive are:

MLSDE-LESD.response	(

PANID

ServiceID,

CommunicationChannelID

)

Table 5 specifies the parameters for the MLSDE-LESD.response primitive.

Table 5—MLSDE-LESD.response parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PANId | Integer | 0x0000–0xffff | The PAN identifier used by the PD. |
| ServiceID | Integer | 0x0-0xf | The Service ID of the PAC WPAN that the PD is associated with. |
| CommunicationChannelID | Set of octets | 0x0000-0xffff for each element | List of channel numbers in use at the PAC WPAN that the PD is associated with. |

The MLSDE-LESD.response primitive is generated by the next higher layer and issued to its MLSDE.

On receipt of the MLSDE-LESD.response primitive, the MLSDE of the device shall update the MAC PIB attribute macCommunicationChannelID with the value of CommunicationChannelID parameter. Then MLSDE shall generate a LESD response command frame as described in 5.4.2.2 and broadcast the command to the neighbor PDs.

      - MLSDE-LESD.confirm

This primitive reports the results of the LESD request to the higher layer of the device.

The semantics of this primitive are:

MLSDE-LESD.confirm	(

PANID,

ServiceID,

CommunicationChannelID,

Status

)

Table 6 specifies the parameters for the MLSDE-LESD.request primitive.

Table 6—MLSDE-LESD.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PANId | Integer | 0x0000–0xffff | The PAN identifier used in the PAC WPAN |
| ServiceID | Integer | 0x0-0xf | PAC service ID used in the PAC WPAN |
| CommunicationChannelID | Set of octets | 0x0000-0xffff for each element | List of channel numbers used in the PAC WPAN. |
| Status | Enumeration | SUCCESS, TRANSACTION_EXPIRED, INVALID_PARAMETER | The result of the LESD request. |

On receipt of a LESD response command, the MLSDE shall check the value of Service ID field of the command. If the value matches to the macServiceID of the PD, the MLSDE shall update the MAC PIB attribute macCommunicationChannelID with the value of Communication Channel ID field in the command and generate MLSDE-LESD.confirm primitive with the Status parameter of SUCCESS and report the results of the LESD request to the higher layer. Then, MLSDE shall generate LESD notification command as described in 5.4.2.3 and broadcast it to neighboring PDs. If the value of Service ID field of the LESD response command received does not match to the macServiceID of the PD, the MLSDE discard the received command.

The MLSDE of the PD shall set the value of Status to TRANSACTION_EXPIRED and report to the higher layer if the LESD response command with Service ID field matching to the macServiceID is not received for at most macLESDResponseWaitTime after the PD sends LESD request command. The value of Status parameter shall be set to INVALID_PARAMETER otherwise.

      - MLSDE-LESD-NOTIFY.indication

This primitive reports the reception of a LESD notification command.

The semantics of this primitive are:

MLSDE-LESD-NOTIFY.indication	(

PANID

ServiceID,

CommunicationChannelID,

)

Table 7 specifies the parameters for the MLSDE-LESD-NOTIFY.indication primitive.

Table 7—MLSDE-LESD-NOTIFY.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PANId | Integer | 0x0000–0xffff | The PAN identifier used in the PAC WPAN |
| ServiceID | Integer | 0x0-0xf | PAC service ID used in the PAC WPAN |
| CommunicationChannelID | Set of octets | 0x0000-0xffff for each element | List of channel numbers used in the PAC WPAN. |

This primitive is generated by the MLSDE of a device and issued to its next higher layer upon the reception of a LESD notification command frame with Service ID field matching to macServiceID of the PD. If the value of the Service ID field of the LESD notification command does not match, MLSDE discard the command.

On receipt of the MLSDE-LESD-NOTIFY.indication primitive, the higher layer is notified of the reception of a LESD notification command. The higher layer may be informed the PAC service and channel numbers in use among the PDs nearby.

      - MLSDE-LESD-CS.request

This primitive allows the PD to initiate CS procedure.

The semantics of this primitive are:

MLSDE-LESD-CS.request	(

PANID,

CSInterval,

CSDuration

ServiceID,

CommunicationChannelID,

)

Table 8 specifies the parameters for the MLSDE-LESD-CS.request primitive.

Table 8—MLSDE-LESD-CS.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PANId | Integer | 0x0000–0xffff | The PAN identifier to be used by the device. |
| CSInterval | Integer | 0x0000-0xffff | Time duration that PD alternates sampling the channel and sleeping as illustrated in Figure 4. |
| CSDuration | Integer | 0x0000-0xffff | Time duration that PD listens to on the channel specified by macLESDChannelID. |
| ServiceID | Integer | 0x0-0xf | PAC service ID to be used. |
| CommunicationChannelID | Set of octets | 0x0000-0xffff for each element | Channel numbers to be used. |

This primitive is generated by the next higher layer to the MLSDE when a PD discovers the PAC service successfully or if it does not discovery the service through passive and active LESD scan.

On the receipt of the primitive, the MLSDE of the PD shall update the corresponding MAC PIB attributes. The macLESDdone is set to TRUE and macPANID, macCSInterval, macCSDuration, macServiceID, macCommunicationChannelID are updated respectively as the values of PANID, CSInterval, CSDuartion, ServiceID, CommunicationID parameters.

      - MLSDE-LESD-CS.confirm

This primitive reports the results of the LESD-CS request to the higher layer of the device.

The semantics of this primitive are:

MLSDE-LESD-CS.confirm	(

Status

)

Table 9 specifies the parameters for the MLSDE-LESD.request primitive.

Table 9—MLSDE-LESD-CS.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, FAILURE, INVALID_PARAMETER | The result of the LESD CS request. |

The MLSDE shall generate MLSDE-LESD-CS.confirm primive with the Status parameter of SUCCESS if it successfully updates the MAC PIB attributes with the values of corresponding parameters of MLSDE-LESD-CS.request primitive as described in 5.4.3.8.

If the MLSDE receives the MLSDE-LESD-CS.request primitive while performing other LESD MAC operation, the MLSDE shall not perform CS procedure and report the result to the higher layer via MLSDE-LESD-CS.confirm primitive with the status parameter of FAILURE. The value of Status parameter shall be set to INVALID_PARAMETER otherwise.

    - LESD MAC constants and PIB attributes

      - LESD MAC constants

      - LESD MAC PIB attributes

Table 10—MAC PIB attributes

| Attribute | Type | Range | Description | Default |
| --- | --- | --- | --- | --- |
| macLESDcapable | Boolean | TRUE or FALSE | If TRUE, the device is capable of functionality specific to LESD. | Implementation specific |
| macLESDenabled | Boolean | TRUE or FALSE | If TRUE, the device is using functionality specific to LESD. | Implementation specific |
| macPANID | Integer | 0x0000-0xffff | The identifier of the PAN on which the PD is operating. If this value is 0xffff, the device is not associated. | 0xffff |
| macServiceID | Integer | 0x0-0xf | PAC service ID used in the PAC WPAN. | 0xf |
| macAvailableChannelID | Integer | 0x0000-0xfffff for each elemets | List of channel numbers that PD is capable of use. | - |
| macLESDResponseWaitTime | Integer | 0x0000-0xffff | The maximum time that a device shall wait for a LESD response command to be available following a LESD request command frame. | - |
| macCSInterval | Integer | 0x0000-0xffff | Time duration that PD alternates sampling the channel and sleeping as illustrated in Figure 4. | - |
| macCSDuration | Integer | 0x0000-0xffff | Time duration that PD listens to on the channel specified by macLESDChannelID. | - |
| macCommunicationChannelID | Integer | 0x0000-0xffff | List of channel numbers in use at the PAC WPAN | - |
| macLESDChannelID | Integer | 0x0000-0xffff | The channel number to use for LESD procedure. | - |
| macLESDdone | Boolean | TRUE or FALSE | Indicates if PAC service specified by macServiceID is found. TRUE if the service is found; FALSE otherwise. |  |
| macMaxLESDRequestRetries | Integer | 0x00-0xff | The maximum number of retries to send LESD request command | 10 |
| macNumLESDRequestRetries | Integer | 0x00-0xff | The number of retries to send LESD request command | 0 |

</379r0>

<396r1>

Common mode/channel and operation mode/channel

PAC system has a common channel, and multiple operation channels. Common channel is used by the common mode. To decrease scanning latency, common mode with a specified RF channel shall be used only for starting discovery, or for initiating a PAC group through broadcasting short signal/message. Common mode shall not be used in operations or communications for an established PAC group. Emergency message shall be broadcasted at the common mode to guarantee efficient delivery.

There is no global clock for common channel. An I-PD broadcasts a short signal/message on its own clock. Hereafter, we refer to the short signal/message broadcasted over common channel as a trigger signal (TS). A TS shall be used under the following constraints and as illustrated in Figure 3.

- Length of a TS that is broadcasted via the common channel shall be within the maximum length TcoMAX_send.

- Re-broadcasting of a TS shall be after a minimum duration TcoMIN_duration.

- An I-PD shall perform CCA before broadcasting a TS. The minimum CCA time is TcoMIN_cca.

  - When CCA reports a clear channel at time tk, the I-PD shall start broadcast of a TS at tk +TcoMIN_duration and stop broadcast after broadcasting  the last TS at tk + Nco_send ×TcoMIN_duration.

where, Nco_send is an integer randomly selected within [1, NcoMAX_send], and NcoMAX_send is the maximum number that a TS can be repeatedly broadcasted in an iteration. After an iteration, Nco_send is reset for the next iteration.

  - After 3A), the I-PD shall perform CCA again before further broadcasting. When CCA reports a clear channel, the I-PD repeats (iteration) procedure 3A) again.

  - Procedure of 3B) will be iterated until the upper layers stop the iteration.

  - In each iteration, the group clock provides the information of start time, t_start, of TS which is synchronized to the first broadcasted TS, as well as the information of the current TS time, t_now. The latter is the start time of TS in the current iteration.

    - Illustration of constraint 3 (A)

    - Illustration of constraint 3 (B), (C)

Figure 3 Constraints of using TS.

</396r1>

380r2

<395r1>

PAC structure consists of discovery phase, peering phase, and communication phase as figure 1. PDs perform discovery procedure, peering procedure and communication procedure in discovery phase, peering phase, and communication phase, respectively.

Figure 4. PAC structure

    - Discovery operation

      - Discovery phase

It is phase for discovering PD to discover other neighbour PD(s). The phase size is unit(s) of hopping slot. Discovery phase consists of multiple discovery slots. Discovering PD may be in different channel in each slot.

Figure 5. Discovery phase structure

      - Discovery procedure

Discovering PD sets up discovery phase when it wants to discover others. Discovered PD may not be in discovery phase.

Discovery procedure is as below:

- Discovering PD transmits ‘discovery request message’ and waits ‘discovery response message’ in first discovery slot.

- Discovered PD(s) receiving the message transmit(s) ‘discovery response message’ within the discovery slot.

- Discovering PD transmits ACK message right after the response message. (it’s omitted in figure 2)

- Repeat 1~3 in the next discovery slot.

Discovery phase is pre-announced to already communicated PDs.

When discovering PD discovers multiple PDs over several channels, the discovering PD performs peering in multiple peering phases without re-discovery. Discovered PDs wait peering message until peering timer is expired. Peering timer sets to zero from when discovered PD receives ACK message.

</395r1>

<392r1>

IEEE 802.15.8 follows a periodic discovery operation based on the broadcasting manner. The discovery operation consists of transmission of a PD’s discovery signal and reception of other PD’s discovery signals for a period. A PD transmits own discovery signal through a selected discovery RU and receives other PD’s discovery signals through the remained discovery RUs in the discovery region.

The period of discovery resource is an ultraframe. The selection of discovery RU is performed on the basis of the resource structure of discovery. After monitoring the utilization of discovery RUs during multiple periods before selecting a discovery RU, a PD selects a discovery RU which is unused (or least congested) from each PD’s perspective. The collision between PDs because of using the same resource is resolved by the following procedure. At the arbitrarily selected time, a PD receives the signals without transmitting own discovery signal through a selected discovery RU. It is to determine whether there are signals transmitted by other PDs. If a collision is detected, a PD performs the discovery RU reselection and transmits own discovery signal periodically through newly selected discovery RU.

Figure 1. Resource structure of discovery

A PD performs both device discovery and service discovery of other PDs using the insertion of different contents in a discovery signal to be periodically transmitted. Device discovery is performed periodically by a PD for the presence discovery of other PDs. And a device ID of a PD is included in the discovery signal. In addition, service discovery is performed in the query-based manner (using request and response message) to obtain the information related to other PD’s service. And Service information of a PD is included in the discovery signal. A resource for device discovery of a PD is used infrequently for the exchange of request and response messages to obtain the service information of other PD. To support aperiodic service discovery in the query-based manner, a parameter named SIV (Service Information Version) is also included in the discovery signal for device discovery. SIV is used to indicate the change in the service provided by each PD. By providing the parameter related to service information (i.e. SIV) periodically, the exchange of request and response messages is executed only if the information update is required.

Service discovery should be also supported through message transfer in the data region.

    - Resource shuffling

A resource shuffling is used to vary the configuration of PDs which uses the RUs in a BU. The positions of RUs are shuffled via a specific pattern. A PD calculates the current position of a selected RU using the shuffling pattern and the previous position.

A Shuffling pattern for discovery region is configured using the discovery RUs of entire BUs in a suprerfame. And the shuffling pattern for a discovery region has the form of a square matrix. The value of N is the total number of rows (or columns) in the matrix. And it is the same as the number of RUs in a BU or the number of BUs in a superframe. One shuffling pattern is used for all superframes in an ultraframe. The pattern is changed every ultraframe.

Figure 2. Configuration of shuffling pattern for discovery region

Figure 3. Change of shuffling pattern

</392r1>

Scheduling

<368r1>

The resource scheduling is followed the resource allocation configuration pre-determined by the peer group service. The unit resource slot of a peer group is n times of the base slotted resource. The unit resource slot size may be different from other peer groups to support the designated grade of service.

Figure 5.5- Base slotted resource and unit resource slot specified by each peer network

</368r1>

<377r0>

A PD shall determine Resource Slot based on the predetermined distributed scheduling algorithm.

Data transmission is performed during Data frame which is accessed by only peered PDs. For unicast transmission, Link ID is determined via peering procedure.

A PD shall determine one or multiple Resource Slot during Scheduling subframe as contention-free channel access scheme.

Scheduling subframe is comprised of Scheduling Request subframe and Scheduling Response subframe.

Scheduling Request signal represents Link ID, Resource Slot Star Index, and Resource Slot Length.

Scheduling Response signal represents Link ID, Resource Slot Adjusted Index, and Resource Slot Length.

Both signal contains resource information relating to resource assignment and is broadcasted to nearby PDs.

The flowchart of operation for distributed scheduling is as follows:

Link1 has a transmitter PD1(TxPD1) and a receiver PD1(RxPD1) and Link2 has a transmitter PD2(TxPD2) and a receiver PD2(RxPD2). Initially, TxPD1 and TxPD2 determine one or multiple RSs by initial configuration respectively. The candidate RS information is transmitted to the corresponding RxPD and neighboring RxPD as well. RxPD receives multiple Scheduling Request message and has resource information including RS Start Index and RS Length. RxPD modifies resource information to avoid resource assignment confliction based on RS Star Index information from neighbouring TxPDs. The modified resource information including RS Adjusted Index and RS Adjusted Length are transmitted by the RxPD to the corresponding TxPD and neighboring TxPDs. The TxPD determines the assigned RSs to transmit data packets on.

</377r0>

<380r2>

Qing: This is the procedure for channel management which is not exactly scheduling. But cannot find the clause for the Channel Management therefore inserted it here.

Channel Management consists of three parts, which are Peer Detection, Channel Allocation for a New Peer, and Channel Allocation for the 1st Peer.

Peer Detection consists of building a list of peers within proximity which is then used by the channel allocation procedures. Once the peer detection procedure is unable to detect any additional peers within predefined detection window and the peer list is not empty, the channel allocation procedure for a new peer is performed. If the peer list is empty then the channel allocation procedure for the 1st peer is performed, to form a new peer-to-peer network.

</380r2>

<395r1>

Communication phase

It is phase for PD to communicate with peered PDs. The phase size is units of hopping slot. The operating channel of each hopping slot is changed by hopping pattern.

Figure 7. PAC communication frame structure

Communication procedure

After peering procedure, peered PDs have the same hopping slot timing. The PDs has own hopping pattern and when they meet in the hopping slot with the same channel, they may communicate each other.

The PDs in communication phase also may listen the discovery request message from other PDs in discovery phase.

Hopping Pattern in 2.4 GHz band

In 2.4 GHz band, there are 6 possible patterns in Table 1 for PAC device. Each pattern has its Pattern ID. If a PD use hopping pattern ID 2, it operates in channel 1 at slot 1, channel 3 at slot 2, and channel 2 at slot 3. At slot 1, it can communicate with a PD which use pattern ID 1 at channel 1. In a similar way, the PD which use pattern ID 2 can communicate with PDs which use pattern ID 1, 4, 5. That ID set is Communication Possible Pattern (CP-Pattern) in Table 1.

| Pattern ID | Hopping Pattern | Communication Possible Pattern (CP-Pattern) |
| --- | --- | --- |
| 1 | (1, 2, 3) | 2 (in Ch.1 at Slot 1) 3 (in Ch.3 at Slot 3) 6 (in Ch.2 at Slot 2) |
| 2 | (1, 3, 2) | 1 (in Ch.1 at Slot 1) 4 (in Ch.3 at Slot 2) 5 (in Ch.2 at Slot 3) |
| 3 | (2, 1, 3) | 1 (in Ch.3 at Slot 3) 4 (in Ch.2 at Slot 1) 5 (in Ch.1 at Slot 2) |
| 4 | (2, 3, 1) | 2 (in Ch.3 at Slot 2) 3 (in Ch.1 at Slot 1) 6 (in Ch.2 at Slot 3) |
| 5 | (3, 1, 2) | 2 (in Ch.2 at Slot 3) 3 (in Ch.1 at Slot 2) 6 (in Ch.3 at Slot 1) |
| 6 | (3, 2, 1) | 1 (in Ch.2 at Slot 2) 4 (in Ch.1 at Slot 3) 5 (in Ch.3 at Slot 1) |

Table 1. Hopping Pattern of communication phase in 2.4 GHz band

Hopping Pattern decision rule

A PD shall following rule for decision of its hopping pattern.

If a PD doesn't have peered link:

- Discover hopping pattern ID of target PD and PDs which are peered with target PD.

- Subtract pattern IDs used by PDs which are peered with target PD from CP-Pattern of target PD.

- If there are remaining IDs, choose a pattern ID from them.

- If there are not reaming IDs, choose a pattern ID from CP-Pattern of target PD.

If a PD has peered link(s):

- Discover hopping pattern ID of target PD.

- If the CP-Pattern of target PD include own using pattern ID, use the pattern ID as it is.

- If the CP-Pattern of target PD doesn't include own using pattern ID, change the pattern ID to the identical ID of target PD.

Example:

- Case 1: No peered link. Pattern ID of target PD = 3, Pattern ID of PD which is peered with target PD = 1

  - CP-Pattern of target PD is 1, 4, 5 and subtract 1. And then we have 4, 5. And choose a pattern ID from 4 and 5.

- Case 2: Pattern ID of target PD = 3, Pattern IDs of PD which are peered with target PD = 1, 4, 5

  - CP-Pattern of target PD is 1, 4, 5 and subtract 1, 4, 5. And then we have nothing. And choose a pattern ID from 1, 4 and 5.

- Case 3: Having peered link. Pattern ID of target PD = 3, Using pattern ID = 1

  - CP-Pattern of target PD is 1, 4, 5 and include 1. Use pattern ID 1 as it is.

- Case 4: Having peered link. Pattern ID of target PD = 3, Using pattern ID = 2

  - CP-Pattern of target PD is 1, 4, 5 and doesn't include 2. Use pattern ID 2.

</395r1>

<392r1>

IEEE 802.15.8 supports the priority-based fully distributed scheduling among the contending PDs.

    - Resource mapping

After successful peering, the peered PDs share the same orthogonal PID. The orthogonal PID in conjunction with synchronization information such as frame number and superframe number determines available resources for sending DS-REQ/DS-RSP and data transmission of the peered PDs. The determination of available resource for sending DS-REQ/DS-RSP and data transmission of the peered PDs is referred to as resource mapping.

      - Data channel mapping

Data channel is a fundamental unit used for sending DS-REQ/DS-RSP and data transmission occurring in the scheduling interval and the data interval, respectively. The data channel mapping is a function that determines available data channels for the peered PDs based on PID, frame number, and superframe number.

The available data channel index l (0 ~ 15) for the peered PDs with PID p (0 ~ 127), frame number n (0 ~ 9), and superframe number s (0~15) is given by

l = (floor(p / 8) + sⅹ10 + n) modulo 16.

(In frame type 0 (n = 0), data channels corresponding to l=0, 1, 2 are not defined.)

      - SP mapping

SP(scheduling Priority) is the priority in access to the data interval over other contending PDs. SP is also used to determine the resource for both DS-REQ and DS-RSP in the scheduling interval of the data channel assigned by the data channel mapping. SP ranges from 0 to 7 and 7 corresponds to the highest priority.

The assigned SP for the peered PDs with data channel index l (0 ~ 15) for the peered PDs with PID p (0 ~ 127), frame number n (0 ~ 9), and superframe number s (0~15) is given by

    - Resource allocation

In this clause, the PD transmitting DS-REQ is referred to as the originator, and the peered PD of the originator as recipient.

      - Normal allocation

Normal allocation is the fundamental allocation mechanism of IEEE 802.15.8. Normal allocation enables peered PDs to participate in multiple access to the resource for data transmission.

After transmitting SRI, the originator sends DS-REQ using the resource for DS-REQ corresponds to the determined SP. In the blocking unit for DS-REQ, the originator shall transmit blocking signals, before and after the transmission of DS-REQ.

Figure 7. SP relevance to the resources for DS-REQ and DS-RSP

The originator sets Required slots field of DS-REQ payload to the required resource in the data interval in terms of OFDM slots. OFDM slot is a basic unit of data interval and one OFDM slot is 4 OFDM symbols. The required resource shall be the sum of data burst to be transmitted, ACK, and two GIs which is one for GI between data burst and ACK and the other for GI between ACK and the next data burst.

Figure 8. DS-REQ payload format

The recipient shall try to receive the DS-REQ that may be transmitted by the originator. Once the recipient succeeds in decoding the DS-REQ payload, it shall receive all the DS-REQs with higher SP than its own. If the recipient fails to decode the DS-REQ from the originator, it does not need to receive any other DS-REQs. The recipient shall accumulate the value of Required slots field of all the received DS-REQs with higher SP than its own.

The resource for DS-RSP is also determined by SP. The recipient shall transmit DS-RSP using the mapped resource for it in a response to the received DS-REQ from its originator. In the blocking unit for DS-RSP, the recipient shall transmit blocking signals, before and after the transmission of DS-RSP.

The recipient shall set Offset field of DS-RSP payload to the accumulated value of Required slots field of all the received DS-REQs with higher SP than its own. If the Offset field exceeds data interval boundary, recipient shall not send DS-RSP.

Allocated slots field of DS-RSP payload is set to the Required slots field of the DS-REQ received from its originator. If (Offset + Required slots) exceeds data interval boundary, the recipient shall adjust Allocated slots field in order not to exceed the data interval boundary.

Figure 9. DS-RSP payload format

The originator shall try to receive the DS-RSP from its recipient. Once the originator succeeds in decoding the DS-RSP payload, it shall receive all the DS-RSPs with higher SP than its own. If the originator fails to decode the DS-RSP form its recipient, it does not need to receive any other DS-RSPs.

After the successful exchange of DS-REQ and DS-RSP, the originator shall check to see whether its allocated ranging from Offset to (Offset+ Allocated slots) overlaps with resources allocated to other PDs with higher SP than its own. The originator shall utilize the allocated resource for its data transmission only when there is no conflict with other resources for PDs with higher SP than its own. Otherwise, the originator shall not use the allocated resource and stop the normal allocation process.

The originator shall transmit its data burst at Offset. The recipient shall transmit ACK at (Offset + Allocated slots- GI between ACK and the next data burst).

      - Consecutive allocation

Consecutive allocation is an allocation mechanism enabling any PDs performed normal allocation in a data channel to have an opportunity of another normal allocation in the following data channel which is not assigned by the data channel mapping.

The originator trying consecutive allocation in the data channel #(n+1) shall set CAR (Consecutive allocation request) bit to 1 when it sends DS-REQ to the recipient in data channel #n. Only if the originator succeeds in receiving DS-RSP from the recipient, it can precede consecutive allocation process in data channel #(n+1) in addition to the normal allocation in data channel #n.

In data channel #(n+1), after interference sensing, the originator shall check to see whether there is SRI signal in the scheduling interval. If SRI signal is detected, the originator immediately stops consecutive allocation process. Otherwise, the originator may continue consecutive allocation process, which is same to another normal allocation as an originator in data channel #(n+1).

If the recipient receives DS-REQ with CAR bit set to 1 in data channel #n, it shall also check to see whether there is CI signal in data channel #(n+1). If CI signal is detected, the recipient immediately stops consecutive allocation process. Otherwise, the recipient shall precede consecutive allocation process, which is same to another normal allocation as a recipient in data channel #(n+1).

</392r1>

QoS

<368r1>

The MAC sublayer provides the resource allocation features and the data primitives for supporting multiple grades of service to the next higher layer. The grades of service are categorized with the technical attributes specified in the application matrix for IEEE 802.15.8 (15-12-0684-00-008).

</368r1>

Interference management

<368r1>

To avoid interference caused of unscheduled accesses from multiple peer groups, the MAC sublayer provides two interference avoidance capabilities. During peer group formation, the initiator observes the frames from neighbour peer groups and gathers the resource allocation schedules. The initiator adjusts the resource allocation configuration and selects the start time to cause low probability of interference. The peers who serve to multiple peer networks schedule the transmission by selecting time-slot to avoid contention among multiple peer networks. In case of contention, the peers select appropriate access control algorithm to the peer’s priority.

Figure 5.6- Proactive interference avoidance at a PD which serves to multiple peer groups

</368r1>

<377r0>

Interference among multiple links is managed by threshold level which is used to identify neighbouring PDs. The number of concurrent link goes many as the threshold level goes low. Conversely, the number of concurrent link goes small as the threshold level goes high.

</377r0>

Transmit power control

<380r2>

As shown in the Figure xyz, the Transmit Power Control may be triggered by higher layer, Discovery, Peering, Data Transceiving, or other MAC logic function. The Transmit Power Control contains the following steps.

- Power Control Information (PCI) detection in the proximity;

- Inter Peer-to-Peer Networks Transmit Power Control;

- Intra-Peer-to-Peer Network Transmit Power Control.

Figure xyz Transmitting Power Control

</380r2>

Multicast

<368r2>

The part of devices in a peer group may establish a geo-group of the peer group. The MAC sublayer provides the geo-group of the peer group formation primitives and the geo-groupcast primitives to the next higher layer. The formation of a geo-group of the peer group is performed with the implicit inviting and the explicit grouping.

</368r2>

Broadcast

Multi-hop operation

<368r2>

To extend the coverage of the peer group, peer group relay capable PDs provide hop-relaying. The hop-relaying is performed at the RELAY sublayer by transmitting the received frames not destined to the device to the other side neighbour PD. The allocation of resources for relaying frames and the relaying procedure can be adopted from the IEEE 802.15.4k Timeslot Relaying based Link Extension.

</368r2>

<388r0>

- 5.12.1	Multi-hop Unicast Data Transmission

When a PD wants to unicast data frame, the PD searches routing entry of destination address in its routing table. If the PD finds the routing entry of destination address, it starts to unicast immediately. If it does not find routing entry of destination address, it multicasts a MGNF (Notification type: 4) to group. When the other PD receives the MGNF, it saves backward route information in its routing table during Tw. Then, it starts to find a routing entry of destination address in its routing table. If a PD receiving MGNF finds routing entry of destination address, it unicasts a MGNF (notification type: 5) to the PD which wants to unicast.

In the following figure,

- A wants to unicast to F within same multicast group which ID is 1, but A does not have routing information of F in A’s routing table. Then, A multicasts a MGNF (Notification type: 4) to group 1. When C receives the MGNF, it saves backward route information in the routing table during Tw. It starts to find a routing entry of F in its routing table.

- Since C did not find the routing entry, C forwards the MGNF. When E receives the MGNF, it saves backward route information in a routing table during Tw, it starts to find a routing entry of F in its routing table.

- Since E finds routing information of F, E unicasts a MGNF (notification type: 5) to A. C creates routing entry of F in its routing table.

- C forwards the MGNF to A. When A receives the MGNF, it creates a routing entry of F in its routing table. Now, A can unicast data to F.

Figure 17. Unicast Data Transmission

Figure 18. Unicast Flowchart

</388r0>

Relative positioning

<368r1>

The MAC sublayer provides geographical relation information among PDs which are resided within two hops from the device. The geographical relation presents relative angular distance among the neighbour devices. The relation matrix generated from the geographical relation information is advertised to the peer group according to the request.

</368r1>

Power management

Security

<388r0>

Security layer provides users with privacy, authentication, and authorization across the network.

- 5.15.1 Security modes

PAC security layer provides three different security modes on the basis of security requirements of network connections.

5.15.1.1 Security mode 1 (non-secure)

When a PAC device is in security mode 1, it shall never initiate any security procedure.

5.15.1.2 Security mode 2 (service level enforced security)

When a PAC device is in security mode 2, it shall initiate security procedures after a channel establishment request has been received or a channel establishment procedure has been initiated by itself. Whether a security procedure is initiated or not depends on the security requirements of the requested channel of service.

A PAC device in security mode 2 should classify the security requirements of its services using the following attributes.

| Security requirement | Description |
| --- | --- |
| Authentication required | - Before connecting to the application, the remote device must be authenticated |
| Authorization required | - Access is only granted automatically to trusted PAC devices, or untrusted devices after an authorization procedure - Always requires authentication to verify that the device is the right one |
| Encryption required | - The link must be changed to encrypted mode, before access to the service is possible |

Security mode 1 can be considered as a special case of security mode 2 where o service has registered any security requirements.

5.15.1.3 Security mode 3 (link level enforced security)

When a PAC device is in security mode 3, it shall initiate security procedures before the channel is established.

5.15.2 Security parameters

PAC security layer uses the following security parameters.

| Parameters | Description |
| --- | --- |
| PAC_ADDR | PAC device address (unique for each device) |
| AUTH_KEY | Authentication key used for authentication purposes |
| ENC_KEY | Encryption key for secure unicast |
| GENC_KEY | Group encryption key for secure group communication |
| RAND | Frequently changing random or pseudo-random number |

5.15.3 Key Derivation

For secure communications between PAC devices in networks, several key materials are derived using the shared secret information between the devices.

Following figure shows the key derivation procedure between devices using PIN secretly shared during the peering phase.

Figure 19. Key derivation

Each key material shall be derived selectively on the basis of the security mode. For example, when a PAC device is in security mode 1, it shall never initiate any security procedure. When a PAC device is in security mode 3, it shall derive all of the above key materials, such as AUTH_KEY and ENC_KEY between the devices.

5.15.4 Authentication

Authentication is the process of verifying ‘who’ is at the other end of the link. In PAC security layer, authentication is performed for devices, or services.

Technically, authentication is achieved based on the stored authentication key (AUTH_KEY) or by peering (entering a PIN).

Flowchart for authentication is shown as a follow.

Figure 20. Flowchart for authentication

5.15.4.1 Infrastructureless authentication

In PAC networks where there is no coordinator or AAA(Authentication, authorization, accountability) server, authentication between PAC devices are done using PIN, or certificate issued by the trusted authority.

Figure 21. Infrastructureless architecture

5.15.4.1.1 One-way authentication procedure

Some applications might require only one-way authentication. Following figure shows the one-way authentication procedure between PAC devices A (verifier) and B (claimant).

Figure 22. One-way authentication

When a verifier A requires to authenticate B, (1) A sends a random number A.RAND to B,  (2) A and B computes a secret information SRES’ and SRES, respectively, using pseudo-random function(PRF) on inputs of A’s random number (A.RAND), PAC address of B (B.PAC_ADDR), and authentication key shared between them (AUTH_KEY), (3) B sends SRES, (4) A checks if SRES is equal to SRES’. If they match, authentication succeeds; if not, authentication fails.

5.15.4.1.2 Mutual authentication procedure

Some applications might require mutual authentication. Following figure shows the mutual authentication procedure between PAC devices A and B.

Figure 3 Mutual authentication

Figure 23. Mutual authentication procedure

When PAC devices A and B authenticate each other, (1) A sends its address (A.PAC_ADDR) with a random number (A.RAND) to B, (2) B also sends its address (B.PAC_ADDR) with a random number (B.RAND) to A, (3) A and B optionally computes encryption key (ENC_KEY) on the input of all of the previously exchanged information and AUTH_KEY. Then, they shall verify each other when they communicate using a secure channel protected by the shared ENC_KEY.

5.15.4.2 Infrastructure authentication

In PAC networks where there is an AAA(Authentication, authorization, accountability) server and a dynamic coordinator, which is a PAC device with intermittent connection to the AAA server, authentication between PAC devices are done using symmetric master key, or certificate issued by the AAA server

Figure 24. Infrastructure architecture

When a PAC device A and B (coordinator) authenticate each other, the mutual authentication procedure shall progress as a following figure.

Figure 25. Infrastructure authentication

The authentication procedure consists of EAP authentication between a PAC device A and an AAA server, authentication key generation and 3-way handshake between PAC device A and B, and encryption key/group key delivery process.

5.15.4.2 .1EAP authentication

EAP (Extensible authentication protocol) authentication uses Extensible Authentication Protocol [IETF RFC 3748] in conjunction with an operator-selected EAP Method (e.g. EAP-TLS [IETF RFC 2716]). The EAP method will use a particular kind of credential – such as an X.509 certificate in the case of EAP-TLS, or a Subscriber Identity Module in the case of EAP-SIM.

The particular credentials and EAP methods that are to be used are outside of the scope of this specification. However, the EAP method selected should fulfill the following mandatory criteria listed in section 2.2 of RFC 4017: (1) mutual authentication, (2) protection against the man-in-the-middle attack. Use of an EAP method not meeting these criteria may lead to security vulnerabilities.

In the PAC authentication procedure, EAP yields the512-bit  master secret key (MSK), which is delivered to a PAC device A by an AAA server.

Then the other key encryption keys (KEK) and HMAC/CMAC keys are derived from the MSK

5.15.4.2.2 Authentication key generation

After EAP authentication, the PAC device A and AAA server generate PMK using a truncation function as a follow.

PMK = Truncate(MSK, 160)

Then, the AAA server sends PMK to the coordinator B securely. On receipt of it, the coordinator B and the PAC device A generate authentication key using PMK.

AUTH_KEY = PRF(PMK, A.PAC_ADDR, B.PAC_ADDR)

Then, PD A and coordinator B derive shared KEK, HMAC/CMAC key from the AUTH_KEY.

5.15.4.2.3 SA-ENC 3-way handshake

SA-ENC 3-way handshake consists of the following messages.

(1) SA-ENC-Challenge message (A ← B): B.random, sequence number, PMK lifetime, HMAC/CMAC digest

(2) SA-ENC-Request message (A → B): A.random, B.random, sequence number, security capabilities, security negotiation parameters, HMAC/CMAC digest

(3) SA-ENC-Challenge message (A ← B): A.random, B.random, sequence number, [SA-ENC_KEY-update,] SA-descriptor, security negotiation parameters, HMAC/CMAC digest

Integrity of the above handshake messages are protected by MAC digest against forgery attack. Optional SA-ENC_KEY-update contains all the keying materials for the ENC_KEY update and distribution, which is encrypted with KEK

SA-ENC 3-way handshake provides the following security guarantees:

- Full mutual authentication,

- Message (2) indicates to the coordinator B that a PD A is alive and that A possesses the AUTH_KEY

- Message (3) indicates to the PD A that the coordinator B is alive

- The coordinator B is guaranteed that SA-ENC-Update is sent by the PD A and is fresh

5.15.4.2.4 Encryption key delivery

After a successful authorization, the PAC device A shall dynamically requests parameters for SA(security association) including ENC_KEY through KEY_Request and KEY_Reply messages. When a secure communication is required between the devices, the connections are encrypted using the ENC_KEY.

5.15.4.2.5 Group key delivery

In a network environment where the secure multicast and broadcast service (MBS) is supported, additional group key (GENC_KEY)  generation and delivery process shall be performed optionally after ENC_KEY distribution procedure. Then, the group communication are encrypted using the GENC_KEY.

5.15.4.3 Authorization

Authorization is the process of deciding if device X is allowed to have access to service Y.

Authorization always includes authentication, and grants access rights to devices on the basis of their trust levels.

| Device trust level | Description |
| --- | --- |
| Trusted device | - The device has been previously authenticated - An authentication key is stored - The device is marked as “trusted” in the device DB |
| Untrusted device | - The device has been previously authenticated - An authentication key is stored - But, the device is not marked as “trusted” in the device DB |
| Unknown device | - No security information is available for this device - This is also an untrusted device |

Trusted devices (authenticated) are allowed to services, but untrusted or unknown devices may require authorization based on interaction before access to services is granted.

Technically, key would be derived or given (established) to the authorized user during the authorization procedure.

Flowchart for authorization is shown as a follow.

Figure 26. Flowchart for authorization

</388r0>

Coexistence

<392r1>

An interference sensing before transmission, using a blocking signal and low power transmission of the essential control signal at the fixed position without interference sensing is a basic coexistence scheme.

    - Interference sensing

If interference from heterogeneous devices is sensed during IS, the PD shall give up the transmission. In consideration of the WiFi, interference sensing should be performed at least 16us.

Figure 10. an example scenario of interference sensing

    - Blocking signal

To prevent resource occupancy of heterogeneous devices, the blocking signal can be transmitted by PDs. One or more subcarriers can be allocated for transmission of the blocking signal.

Figure 11. Conflict avoidance using the blocking

PDs can transmit the blocking signal, before data transmission (Forward Blocking) and after data transmission (Backward Blocking). A blocking concept is applied to the transmission of the control signal (Discovery, Peering, Scheduling). The size of blocking unit is the size of the resources that PDs want to reserve prior to heterogeneous devices.

    - Low power transmission

To guarantee the presence of essential control signals at the fixed position, PDs can use any resource whenever PDs need it with the sufficiently low power. For reliability of the transmitted signal, time domain repetition can be applied.

Figure 12. Low power and repeated transmission

</392r1>

<395r1>

PAC shall coexistence systems such as IEEE802 using unlicensed bands in section 7.1.1.

</395r1>

Higher layer interaction

<380r2>

Figure: Hierarchical Context Information

PD Management Entity manages the application and/or PD information, and provide the context-awareness to device at MAC/PHY.

</380r2>

Physical layer

Block Diagram

—Schematic diagram of common mode PHY

Note: Diagram is so specific. Modify it more generally. Describe each block. Remove Buffer. MIMO will be discussed. Add schematic diagram for Sub 1GHz band and UWB band.

Channelization

Sub 1GHz band

<Park>

- Proposed channelization

| Band | Central freq (MHz) | n | No of channels | Max Tx power (mW) |
| --- | --- | --- | --- | --- |
| Japan-A | fc=917+n | 0,1,…,10 | 11 channels of 1 MHz | 1 |
| Japan-B | fc=922+n | 0,1,…,5 | 6 channels of 1 MHz | 20 |
| Japan-C | fc=921.5+n | 0,1 | 2 channels of 1 MHz | 250 |
| Japan-D | fc=928.7+n | 0,1 | 2 channels of 500 kHz | 1 |
| Korea-A | fc=917.1+nⅹ0.2 | 1, 3, 4, 6, 7, 9, 10, 12, 13, 15, 16, 18 | 12 channels of 200 kHz | 3 mW |
| Korea-B | fc=917.1+nⅹ0.2 | 2, 5, 8, 11, 14, 17, 19, …, 32 | 20 channels of 200 kHz | 10 mW |

<Park>

<384r0>

| Band(MHz) | Modulation | Modulation index | Channel spacing (kHz) | Data rate (kb/s) |
| --- | --- | --- | --- | --- |
| Sub-GHz | Filtered 2FSK | 1 | 200 | 50 |

</384r0>

Note: Park will provide channel information based on Korea regulation.

<369r1>

- Proposed channelization

2.4 GHz band

- Channelization of 2.4 GHz band

  - Such frequency band ranges from 2.4 GHz to 2.5 GHz, which is divided into 9 channels of 10 MHz. By regulation, the maximum transmit power at the input antenna is 1 W. The central frequencies are given by

fc = 2410 MHz + 10n          for n = 0, 1, ..., 8

PAC uses unlicensed bands with multiple channels. E.g. PAC can use all or partial channels among 3 channels in 2.4GHz and 8 channels in 5GHz (UNII-1, UNII-3) when bandwidth is 20MHz per channel.

5 GHz band

- Channelization of 5.7 GHz band

  - Such frequency band ranges from 5.725 GHz to 5.875 GHz, which is divided into 14 channels of 10 MHz. By regulation, the maximum transmit power at the input antenna is 1 W. The central frequencies are given by

fc = 5735 MHz + 10n       for n = 0, 1, ..., 13

PAC uses unlicensed bands with multiple channels. E.g. PAC can use all or partial channels among 3 channels in 2.4GHz and 8 channels in 5GHz (UNII-1, UNII-3) when bandwidth is 20MHz per channel.

</369r1>

6.2.4. UWB band

<382r0>

We are using higher UWB band of 6 – 10.25 GHz as specified in “Technical Guidance for 802.15.8 Proposals.” We are proposing a single channel in upper UWB band for the system to maximize allowed Tx power level. Channel location and bandwidth are determined by regulation at a given Geo.

</382r0>

Note: Decawave and NICT will provide channelization for UWB

Duplex schemes

TDD

<395r1 [7.2]>

PAC is TDD system.

</395r1>

<392r1>

PAC should use TDD scheme.

</392r1>

<369r1>

PAC support TDD scheme.

</369r1>

Marco provide text

FDD

<369r1>

PAC support FDD scheme.

</369r1>

Multiplex schemes

(e.g. CDMA, OFDMA)

PAC should support OFDM.

<395r1>

PAC multiplexing scheme is OFDM.

</395r1>

<369r1>

The resource block for either OFDMA or SC-FDMA is given by

where                     ,                                       , and

BW is obtained by concatenating RBs:

Proposed bandwidths are given by

</369r1>

<377r0>

IEEE802.15.8 PAC should use OFDM to support high efficient control and data transmission.

</377r0>

<392r1>

PAC shall use OFDM scheme.

</392r1>

<707r0>

Time-frequency multiplexing

</707r0>

Frame structure

Note: This clause will be discussed by email reflector. And members should provide harmonized text by January meeting.

<707r0>  Supersedes 369r1

- Frame structure in TDD mode

where L1=PD1 transmits and PD2 receives, L2=PD2 transmits and PD1 receives, S-L1=synchronization for L1, RS-L1=reference signals for L1, RA=random access or channel sounding for MIMO or beamforming, GP=guard period.

The GP is computed as GP=(73+1024)Ts=71.42 µsec and satisfies

Tp+Tdec+Tsw+Tcomp+Tp=GP

  - Tdec=time to detect the last symbol

  - Tsw=time to switch from Rx to Tx or vice-versa

  - Tcomp=compensation time to align to GP

where Tp=1 Km/3x108m/s=3.3 µsec (worst case).

The TDD frame configuration is given in the table

</707r0>

<369r1>

- Frame structure in FDD mode

- One slot contains 7 DTF-S OFDM or OFDM symbols.

- Tslot=7680Ts=0.5 msec

</369r1>

648r0

<373r1>

The transmitted RF signal is generated by modulating the complex baseband signal, which is composed of multifarious fields. The fields are delimited by timing boundaries. The general PHY frame format is shown in Figure 2.

Figure 2 – General PHY frame format

The Preamble field is composed of STF and LTF, and is a part of PPDU that is used for packet detection, AGC, time and frequency synchronization, indication of transmission mode (OFDM or SC), collision detection, and channel estimation. A detailed description of the Preamble field is in 6.2.1

The PHY Header field is used to describe the content in the MPDU as well as the protocol used to transfer it.

The MPDU consists of MAC header, payload, and FCS.

SSF (Self Spatial Filtering) uses a technique called beam jittering. The Beam Jitter field is transmitted at the end of SSF request frame. The presence of Beam Jitter field is indicated in the MAC header in the MPDU, and payload of MPDU includes IE that contains the threshold for the correlation level used in SSF which is defined in 6.2.2.

Preamble

[Option 1]

The preamble consists of two fields named STF (Short Training Field) and LTF (Long Training Field). Figure 3 is a time-domain illustration of the preamble structure.

Figure 3 – Proposed preamble structure (option 1)

[Option 2]

The preamble consists of two fields named STF (Short Training Field) and LTF (Long Training Field). Figure 4 is a time-domain illustration of the preamble structure.

Figure 4 – Proposed preamble structure (option 2)

STF

[Option 1]

The STF is used for AGC (automatic gain control), packet detection, initial timing and frequency synchronization, and implicit transmission mode indication (e.g., OFDM or SC). The length of STF is equivalent to two OFDM symbols, and contains 5 repetitions of a periodic signal. The sign of the 4th period in the STF can be used to indicate the transmission mode.

There are three candidate sequences that can be used to generate the periodic signal used in the STF. The standard shall choose the best sequence considering performance, implementation complexity, and capability. The three candidates are MZC1 (Modified Zadoff-Chu sequence 1), MZC2 (Modified Zadoff-Chu sequence 2), and GSW (Gold Sine Wave), which are described in 6.2.1.3

[Option 2]

The STF is used for AGC (automatic gain control), packet detection, initial timing and frequency synchronization, and implicit transmission mode indication (e.g., OFDM or SC), and also for the first collision detection. The length of STF is equivalent to two OFDM symbols, and contains 5 repetitions of a periodic signal. The sign of the 4th period in the STF can be used to indicate the transmission mode.

There are two candidate sequences that can be used to generate the periodic signal used in the STF. The standard shall choose the best sequence considering performance, implementation complexity, and capability. The two candidates are MZC1 (Modified Zadoff-Chu sequence 1) and MZC2 (Modified Zadoff-Chu sequence 2), which are described in 6.2.1.3.

The last period of the STF is used for 1st stage collision detection. The same collision detection principle described in 6.2.1.2 is used for the STF.

LTF

[Option 1]

The LTF consists of two OFDM symbols. The first symbol of LTF is used for refined timing/frequency synchronization and channel estimation.

There are four candidate sequences that can be used to generate the signal used in the first symbol of LTF. The standard shall choose the best sequence considering performance, implementation complexity, and capability. The four candidates are Base Zadoff-Chu sequence, MZC1 (Modified Zadoff-Chu sequence 1), MZC2 (Modified Zadoff-Chu sequence 2), and GSW (Gold Sine Wave) which are described in 6.2.1.3

The second symbol of LTF is used for collision detection. The sub-carriers of the second symbol of LTF are divided into two groups by DC sub-carrier. A PD selects a random sub-carrier in each group of sub-carriers and transmits a busy tone. Figure 5 illustrates an example of the frequency domain structure of the second symbol of LTF.

Figure 5 – The second symbol of LTF for collision detection.

When a PD receives a frame, the PD checks the second symbol of LTF to detect a collision. For example, if one of the groups of sub-carriers contains more than one busy tone, the PD decides a collision occurred and may decide not to process the remainder of the radio frame. When there are P sub-carriers in each group of sub-carriers, the probability of collision detection failure is 1/P2.

[Option 2]

The LTF is two OFDM symbols long, and consists of a long CP followed by two repetitions of a periodic signal, where the combination of the sign of each period of the signal can be used to indicate transmission mode. LTF is used for refined timing/frequency synchronization, collision detection, and channel estimation.

Either Base ZC sequence or MZCs in frequency domain is used for LTF.

The 2nd period of the periodic signal in the LTF is used for 2nd stage collision detection. The collision detection procedure is illustrated in Figure 6.

Figure 6 – Collision detection procedure using preamble sequence

The collision detection is performed in two stages, the 1st stage using the STF and 2nd stage using the LTF. PHY reports collision detection if collision is detected in any of the two stages.

Preamble sequences for PAC

In this sub-clause, candidate sequences that can be used to generate the STF and the LTF of the Preamble field of PAC PHY frame.

Base Zadoff-Chu sequence:

A length-P ZC sequence in frequency domain is defined as follows

bu,Pm=e-jπm(m+1)uP

where u is the root index of base ZC sequence, odd P is the sequence length, and the sequence element index is m = 0, 1, …, P-1.

MZC1 (Modified Zadoff-Chu sequence 1):

Modified ZC sequence 1 in frequency domain is obtained by interleaving a base ZC sequence with it negative. Let Su,N[m] be a length-N modified ZC sequence 1, then Su,N[m] can be written as follows

- For odd m,

Su,Nm=bu,Pm-12=bm,1 ≤m< P+1-bm,P+2 ≤ m<N

- For even m,

Su,Nm=-bm,1≤m<P+1bm,P+2≤m<N0,m=0, P+1 (DC sub-carrier)

where N=2P+2.

MZC2 (Modified Zadoff-Chu sequence 2):

Modified ZC sequence 2 in frequency domain is obtained by concatenating a base ZC sequence with its complex conjugate. Let Su,N[m] be a length-N modified ZC sequence 2, then Su,N[m] can be written as follows

Su,Nm=bu,Pm-1,1≤m<P+1bu,P*m-P-2, P+2≤m<N0,m=0, P+1 (DC sub-carrier)

where N=2P+2.

GSW (Gold Sine Wave sequence):

Gold Sine Wave sequence in time domain is obtained by multiplying a Gold sequence with a half sine wave window function. Let su,N[n] be a length-N GSW sequence, then

su,Nn=2(N-2)NWu(n)sinnπN,  0≤n<N,

where

Wun=G1n-1G2n-1+uN,1≤n<P+1Wun=Wun+1-P,P≤n<N1,n=0, N-1

and N=2P+2, G1 and G2 are m-sequences of same order, u is the index of GSW sequence, and [ ]N is a modulo-N operator. The term 2(N-2)/N is applied to maintain the same average power as MZCs.

Beam Jitter field

SSF request frame includes a Beam Jitter field. Beam Jitter field comprises of a single OFDM symbol. A sequence (TBD) is assigned to the sub-carriers of the OFDM symbol.

When a PD transmits an SSF request frame, the Beam Jitter field is transmitted using beam jittering described in 6.2.2.1.

When a PD is receiving an SSF request frame, the Beam Jitter field is received without channel equalization. On receiving an SSF request frame, a PD calculates the correlation coefficient defined as follows.

ρ=x,rx,x∙r,r

where x is a vector of the known sequence that is assigned to the sub-carriers of the OFDM symbol of Beam Jitter field, and r is a vector the OFDM sub-carriers of the received Beam Jitter field.

The cross-correlation coefficient ρ is compared with the threshold found in the IE received in the SSF request frame. If ρ is larger than the threshold, the PD transmits an SSF response frame to the transmitter of the SSF request frame with the calculated ρ.

Beam jittering can be implemented for single carrier systems with minor modification. The detailed structure of beam jittering for single carrier system is TBD.

Beam Jittering.

Beam jittering is an open-loop transmit beamforming technique that uses an array antenna, where each sub-carrier of an OFDM symbol is transmitted with a beam pattern independently selected from a set of K predefined beam patterns.

The predefined beam patterns are designed so that all the beam patterns have an identical array gain in the boresight direction of the array, while the array gains in other directions are random.

The number of predefined beam patterns (K), design of beam patterns, and beam selection pattern are implementation specific, and is outside of the scope of this document.

Figure 7 and Table 2 show plots and array parameters of an example of a set of pre-defined beam patterns, respectively, where K=2. SSF with K=2 shows good performance when the beam patterns are well designed.

| Amplitude of array response | Phase of array response |
| --- | --- |

Figure 7 – An example of pre-defined beam patterns.

Table 2 – An example of array parameters for K pre-defined beam patterns.

| Antenna configuration | 4 antenna ULA |
| --- | --- |
| Antenna spacing | 0.5λ |
| K | 2 |
| Null locations | Beam L: sin-12i-14, i=-1,1,2. |
|  | Beam R: sin-12i-14, i=-1,0,2. |

</373r1>

<377r0>

    - Discovery frame structure

Discovery frame is comprised of multiple Discovery slots. A Discovery Slot delivers single Peer Discovery Message (PDM).

Figure 5. Discovery Frame Structure

</377r0>

<377r0>

    - Data frame structure

A Data frame is comprised of one or multiple scheduling subframe and the multiple resource slots (RSs) which are associated to one scheduling subframe.

A RS consists of Preamble Signal duration, Channel Feedback Signal duration, Data Packet duration, and ACK Signal duration.

</377r0>

<392r1>

The ultraframe has a fixed length and appears repeatedly. Ultraframe, superframe, frame has a hierarchical structure.

Figure 13. Hierarchical structure of ultraframe

Every frame consists of a synchronization region, a discovery region, a peering region and a data region. The type of a frame can be the frame type 0 of the frame type 1 according to the configuration. The frame type 0 consists of a synchronization region, a discovery region, a peering region and a data region. Frame type 1 consists of a synchronization region and a data region. The frame type 0 is used if the frame number is 0 and the frame type 1 is used otherwise.

    - Synchronization region

The synchronization region is located at the head of a frame. The synchronization signal for the distributed synchronization is sent in the synchronization region. A PD transmits the synchronization signal without interference sensing but the synchronization signal is transmitted by low power for coexistence with heterogeneous devices. The synchronization signal is transmitted repeatedly for reliability. There are two kinds of synchronization signals. One is used at the synchronization region of the first frame in an ultraframe and another is used at other frame.

Figure 14. Synchronization region

    - Discovery region

In the discovery region, a discovery signal is transmitted. The discovery region consists of multiple discovery RUs, ISs and GIs. The discovery RU consists of a preamble signal and a discovery signal. The Forward Blocking scheme is used for coexistence. The discovery region consists of 8 blocking unit and one blocking unit consists of 8 discovery RUs. The IS interval is present in front of every blocking unit.

Figure 15. Discovery region

    - Peering region

The peering region consists of the Peering-REQ/RSP interval and the PID broadcast interval. The Peering-REQ/ RSP interval consists of multiple PID-REQ RUs, PID-RSP RUs, ISs and GIs. The Peering-REQ RU consists of a preamble signal and a Peering-REQ signal. The Peering-RSP RU consists of a preamble signal and a Peering-RSP signal. Forward & Backward Blocking scheme is applied at Peering-REQ blocking unit, and Forward Blocking scheme is applied at Peering-RSP blocking unit. Four Peering-REQ blocking units and four Peering-RSP blocking units exist in the Peering-REQ/RSP interval. A Peering-REQ blocking unit has four Peering-REQ RUs and a Peering-RSP blocking unit has four Peering-RSP RUs.

In the PID broadcast interval, multiple PID broadcast RUs mapped to PID exist. Forward Blocking scheme is used.

Figure 16. Peering region

    - Data region

The data region is comprised of multiple data channels with a fixed size. In case of frame type 0, the data region has 13 data channels because 3 data channels (0~2) are used for the control such as Discovery and Peering. In case of frame type 1, data region has 16 data channels.

A data channel consists of a scheduling interval and a data interval. The scheduling interval consists of DS-REQ RUs, DS-RSP RUs, IS, GIs, and SRI. In case of the DS-REQ RU and the DS-RSP RU, a preamble signal is followed by a DS-REQ signal and a DS-RSP signal, respectively. Forward and Backward Blocking schemes are utilized in both DS-REQ and DS-RSP blocking units. The data interval consists of GIs and multiple pairs of data burst and ACK. The data burst consists of a preamble signal, a BCI (Burst Control Indicator) signal, and a data signal. The BCI signal is used to indicate the modulation order, the code rate, and the length of data signal. ACK consists of a preamble signal and a ACK signal. The number of data bursts and each burst size are determined by the distributed scheduling.

Figure 17. Data region

</392r1>

    - Discovery frame structure

<384r0>

The LESD Mode PPDU shall be formatted as illustrated.

|  |  | Octets |  |
| --- | --- | --- | --- |
|  |  | 1 | Variable |
| Preamble | SFD | PHY Header | PHY Payload |
| SHR |  | PHR | PSDU |

      - Preamble field

The Preamble field shall contain multiples of the 8-bit sequence “01010101”.

      - SFD

The SFD shall be the selected 16-bit sequence selected from the list of values shown in the following Table.

| Message type | SFD value for uncoded (PHR+PSDU) | SFD value for coded (PHR+PSDU) |
| --- | --- | --- |
| Request | 1011 0001 1001 1100 | 1011 0001 1011 0001 |
| Response | 0100 1110 1001 1100 | 0100 1110 0100 1110 |
| Notification | 0110 0011 1001 1100 | 0110 0011 0110 0011 |

Devices that support FEC shall support the SFD associated with coded (PHR + PSDU) and Devices that do not support FEC shall support the SFD associated with uncoded (PHR + PSDU).

      - PHR

The format of the PHR is shown in the following Table.

If the frame type is not “preamble”, the Frame Length field (L5–L0) specifies the total number of octets contained in the PSDU (prior to FEC encoding, if enabled). Otherwise, The field (L5–L0) represents the remaining time of repeated preamble set.

| Bit string index | 0–1 | 2–7 |
| --- | --- | --- |
| Bit mapping | T1–T0 | L5–L0 |
| Field name | Frame Type | Frame Length / Remaining time |

| (T1 T0) | Frame Type |
| --- | --- |
| 0 0 | Request |
| 0 1 | Response |
| 1 0 | Notification |
| 1 1 | Preamble(No PSDU field) |

      - PSDU field

The PSDU field carries the data of the PPDU.

</384r0>

<370r0>

We propose to use a discovery preamble (DP) based on a ZC sequence and a discovery resource block (DRB) from a modified (DTF-S) OFDM signal.

  - The DP and DRB formed the Discovery Signal (DS).

Moreover, we propose to use one channel (from the proposed channelization for sub-GHz, 2.4 GHz and 5.7 GHz bands) for only discovery of devices.

  - PAC devices can either transmit or receive the DS in this unique channel asynchronously.

The discovery signal (DS) sent over a discovery shared channel (DSCH):

The DS consists of a preamble sequence plus a DRB formed by Nfs frequency slots and Nts time slots.

Once synchronized, a receiver knows the location of the discovery resource block (DRS) to scan for possible peers or to pick time-frequency slots to transmit its DS.

The proposed ZC sequence for initial synchronization during discovery (or communication mode) is a ZC sequence with the following parameters:

  - Sequence length N = 63, relative prime r = 62 and q = 0.

Such ZC sequence is mapped onto 62 sub-carriers

  - The first subcarrier and the DC subcarrier are empty.

  - Of course, several repetitions may be transmitted.

For discovery, the nth symbol transmitted over the kth subcarrier is given by

  - xnl,k=1NXnkexp(j2πlk/N)

  - for l=-L+1,,…,0,1,…,N-1 and L the CP

The DS is transmitted with a predefined duty-cycle. From upper layers, terminals pick time-frequency slots in the DRB to transmit the DS.

A set of 126 symbols are transmitted per time slot:

  - such frame contains information about a given peer (peer ID, service ID, application ID, etc.) and which channel is used for association (different from the SDCH).

The number of frequency slots Nfs =1024 (IFFT size) and the number of time slots Nts is chosen as 20.

Consequently, the DRB can support up to NfsxNts=20,480 users for discovery per Group.

The discovery data is formatted as illustrated

Append 57 bits for user ID, device ID, Group ID, etc.

Append 6 bits from CRC-6-ITU error detection code

Append 63 bits from shorten BCH(126,63) code

</370r0>

<707r0>

We propose to use a discovery preamble  (DP) based on a ZC sequence and a discovery resource block (DRB) from a modified (DTF-S) OFDM signal.

  - The DP and DRB formed the Discovery Signal (DS).

Moreover, we propose to use one channel (from the proposed channelization for sub-GHz, 2.4 GHz and 5.7 GHz bands) for only discovery of devices.

  - PAC devices can either transmit or receive the DS in this unique channel asynchronously.

The discovery signal (DS) sent over a discovery shared channel (DSCH):

The DS consists of a preamble sequence plus a DRB formed by Nfs frequency slots and Nts time slots.

Once synchronized, a receiver knows the location of the discovery resource block (DRS) to scan for possible peers or to pick time-frequency slots to transmit its DS.

The proposed ZC sequence for initial synchronization during discovery (or communication mode) is a ZC sequence with the following parameters:

  - Sequence length N = 63, relative prime r = 62 and q = 0.

Such ZC sequence is mapped onto 62 sub-carriers

  - The first subcarrier and the DC subcarrier are empty.

  - Of course, several repetitions may be transmitted.

For discovery, the nth symbol transmitted over the kth subcarrier is given by

  - xnl,k=1NXnkexp(j2πlk/N)

  - for l=-L+1,,…,0,1,…,N-1 and L the CP

The DS is transmitted with a predefined duty-cycle. From upper layers, terminals pick time-frequency slots in the DRB to transmit the DS.

A set of 126 symbols are transmitted per time slot:

  - such frame contains information about a given peer (peer ID, service ID, application ID, etc.) and which channel is used for association (different from the SDCH).

The number of frequency slots Nfs =1024 (IFFT size) and the number of time slots Nts is chosen as 20.

Consequently, the DRB can support up to NfsxNts=20,480 users for discovery per Group.

The discovery data is formatted as illustrated

Append 57 bits for user ID, device ID, Group ID, etc.

Append 6 bits from CRC-6-ITU error detection code

Append 63 bits from shorten BCH(126,63) code

</707r0>

Data frame structure

<707r0>  Supersedes 369r1

The physical layer protocol data unit (PPDU) is formed by concatenating synchronization header (SHR), Discovery header (DIS), physical layer header (PHR) and physical layer service data unit (PSDU)

Reference signals for demodulation/equalization are embedded in the PSDU.

The MAC protocol data unit (MPDU) is passed to the PHY. Such data is encoded by QC-LDPC codes.

</707r0>

Modulation and coding scheme (MCS)

Note: NICT’s proposal is so detail for PFD. Members will discuss depth of text for PFD.

Modulation

BPSK, QPSK, 16QAM, GFSK, OOK, and 2PPM BPSK are included in proposals.

64QAM may be considered.

Sub 1 GHz band

GFSK, OFDM

2.4 GHz and 5 GHz band

BPSK, QPSK, 16QAM, 64QAM

UWB band

OOK, 2PPM BPSK

Note: Harmonized text will be provided.

Coding Scheme

Note: NICT will simplify this text for coding scheme.

Note: Another coding scheme may be provided in response to call for contribution. This will be addressed in January meeting.

<707r0>  Supersedes 369r1

Quasi-cyclic low density parity check codes.

Quasi-cyclic LDPC codes are systematic, linear codes satisfying

where c  is the codeword and H is the parity check matrix.

H is constructed from the prototype matriz Hp by replacing each entry [Hp]i,j with either a cyclic shift matrix Pc, identity or null matrices of size ZxZ. The final size of H is MpZxNpZ.

Details of encoding can be found in document with DCN 369r1.

QC-LDPC parameters for different coding rates to enable link adaptation:

The prototype matrices Hp for different coding rates can be found in document with DCN 369r1.

</707r0>

<707r0>  Supersedes 369r1

Modulation

In the modulation mapper, the scrambled coded bits    for i=0,1,…,n-1 are modulated with either BSPK, QPSK or 16QAM modulations, resulting in the a block of complex modulation symbols    di  for i=0,1,…,Nsym-1, where di=I+jQ

BPSK mapping

| bi | I | Q |
| --- | --- | --- |
| 0 | 1/√2 | 1/√2 |
| 1 | -1/√2 | -1/√2 |

QPSK mapping

| bi,bi+1 | I | Q |
| --- | --- | --- |
| 00 | 1/√2 | 1/√2 |
| 01 | 1/√2 | -1/√2 |
| 10 | -1/√2 | 1/√2 |
| 11 | -1/√2 | -1/√2 |

16QAM mapping

| bi,bi+1,bi+2,bi+3 | I | Q |
| --- | --- | --- |
| 0000 | 1/√10 | 1/√10 |
| 0001 | 1/√10 | 3/√10 |
| 0010 | 3/√10 | 1/√10 |
| 0011 | 3/√10 | 3/√10 |
| 0100 | 1/√10 | -1/√10 |
| 0101 | 1/√10 | -3/√10 |
| 0110 | 3/√10 | -1/√10 |
| 0111 | 3/√10 | -3/√10 |
| 1000 | -1/√10 | 1/√10 |
| 1001 | -1/√10 | 3/√10 |
| 1010 | -3/√10 | 1/√10 |
| 1011 | -3/√10 | 3/√10 |
| 1100 | -1/√10 | -1/√10 |
| 1101 | -1/√10 | -3/√10 |
| 1110 | -3/√10 | -1/√10 |
| 1111 | -3/√10 | -3/√10 |

DFT-S OFDM or OFDM

The Parameters are given by

is constant and equal to 15 KHz.

Maximum FFT size M=1024

Sampling time

Timing based on a common clock at

Optional GFSK Modulation

An optional and very low power PHY based on CP-2FSK modulation is contemplated for the sub-GHz band.

No support for MIMO technologies, i.e., layer mapper and precoding are not necessary.

The proposed channel encoder, bit interleaver and scrambler are used as well. The modulation mapper is CP-2FSK.

CP-2FSK modulation is given by

where V is amplitude, S(t)=sin(2πfct) is the modulating-carrier signal, fc is  the carrier frequency,                       is the peak frequency deviation, β=1 is the modulation index, Tsym is the symbol time and φ0 is the initial phase of the modulating-carrier signal

The information bearing signal is given by

where gm is information bit, p(t) is a Gaussian pulse shape of bandwidth-symbol duration product of 0.8

</707r0>

<686r0>

Pulse shape and duration

We do not define a specific pulse shape in order to allow different low-complexity pulse generators.

Furthermore, operating bands are not defined will be different at different Geos.

Pulse shape is constrained

In spectrum by the local regulations.

In duration by the Duty Cycle (DC) of  no more than DC=1/32=3.1%.

Packet structure

SHR

PHR

PPDU

Fig. 1: Packet structure.

As Fig, 1. shows, packet consists of SHR – Synchronization Header, PHR – PHY Header and PPDU – Physical Layer Protocol Data Unit.

SHR Structure

Preamble

SFD

Fig. 2. SHR structure.

Preamble consists of M=8 times repetition of the sequence Si. Si is one of the Gold sequences of length 31. Since set of Gold sequences of length 31 has 33 sequences, there are 33 virtual channels available per physical channel. Gold sequences of length 31 have relatively short length with good circular autocorrelation properties.

Synchronization Frame Delimiter (SFD) represents inversion of Si used in the preamble.

Symbol structure

Fig. 3. Symbol structure

Symbol structure is shown in Fig. 3. The whole packet, is transmitted using the same symbol structure regardless of the data rate used with On-Off Keying (OOK) modulation. For , where  is the symbol index, the same symbol duration of  is used. Transmitted pulse waveform is denoted . Hence, transmitted signal for the -th transmitted symbol is

(1)

where  is transmitted bit and  is pseudo-random sequence used in  to flatten the spectrum of the transmitted signal, while for   is set. Pseudo-random sequence used is the same Gold sequence used in PR and SFD for .

</686r0>

Data rates

<686r0>

Channel coding and data rates

RS coder

Bit interleaver

Conv. coder

Modulator

Fig. 4.: Concatenated coding scheme used.

Coding is concatenation of outer Reed-Solomon RS6(63,55) codes and inner convolutional codes. Different data rates have different convolutional coding as in Table 1.

| Data rate (Kbps) | 54.56 | 109.12 | 218.25 | 436.51 | 873.02 |
| --- | --- | --- | --- | --- | --- |
| Conv. coding rate | 1/4 | 1/4 | 1/4 | 1/2 | 1/1 |
| Chips per symbol | 4 | 2 | 1 | 1 | 1 |

Table 1.: Data rates used with convolutional coding rates and number of chips per symbol.

Table 2. provides detailed specification of the convolutional codes used. Bit interleaver used is algebraic bit interleaver specified in the UWB PHY of the IEEE 802.15.6-2012 standard for Body Area Networks.

| Conv. coding rate | ½ | ¼ |
| --- | --- | --- |
| Constrained length | 3 | 4 |
| Generators in octal | 5 7 | 13 15 15 17 |
| Free distance | 8 | 13 |

Table 2.: Convolutional codes used.

Symmetrical double-sided two-way ranging

Tround1

Tprop

Treply1

t

Tprop

Tprop

t

Tround2

Treply2

Device 1

Device 2

Tprop

Fig. 4. Principle of symmetrical double-sided two-way ranging.

As Fig. 4. shows, symmetrical double-sided two-way ranging is achieved by measuring two round trip times: round trip time from device 1 to device 2 denoted  and round trip time from device 2 to device 1 denoted .

denotes propagation time that is being estimated, while  and  represent reply times of device 1 and 2 respectively. Hence,  estimate is calculated as follows

.	(2)

If relative errors of timing at device 1 and 2 are denoted  and  it follows that

(3)

(4)

(5)

(6)

Thus error of  estimation is:

(7)

Hence, relative error of the range estimation is also in the same order of magnitude as  and , which for crystal oscillators is usually around 20 ppm.

</686r0>

Discovery mode

<384r0>

The modulation for the LESD Mode is a 2-level filtered FSK.

The following Table shows the modulation and channel parameters for the LESD Mode PHY.

| Frequency Band (MHz) | Modulation | Modulation index | Channel spacing (kHz) | Data rate (kb/s) |
| --- | --- | --- | --- | --- |
| Sub-GHz | Filtered 2FSK | 1 | 200 | 50 |

a Data rates shown are over-the-air data rates (the data rate transmitted over the air regardless of whether the FEC is enabled).

</384r0>

Multiple antennas

Note: NICT&ETRI will simplify and provide the harmonized text. by January meeting

<707r0>  Supersedes 369r1 below

6.6.1 Layer mapping

Two MIMO technologies are supported: open loop spatial multiplexing and transmit diversity (SFBC) for 2 and 4 antennas.

The [complex] modulation symbols per codeword di for i=0,1,…,Nsym-1 are mapped into several layers

  - Layer=independent stream of symbols in a MIMO configuration.

  - Rank=number of layers transmitted.

As                                                             for

  - where     is the number of layers and        is the number of symbols per layer for the qth codeword.

6.6.1.1 Open loop spatial multiplexing (parallel data streams)

The mapping of codeword symbols onto layer streams can be found in document with DCN 369r1.

6.6.1.2 Transmit diversity (same information is transmitted from multiple antennas)

The mapping of codeword symbols onto layer streams can be found in document with DCN 369r1

6.6.2 Precoding

Precoding allows to increase system performance and robustness by feeding back to the transmitter channel state information.

Schematic diagram of MIMO support with precoding is illustrated as

6.6.2.1  Open loop spatial multiplexing

Precoding for open loop spatial multiplexing increases robustness by feeding back the channel’s rank (RI=rank indicator). The transmitter chooses a pre-fixed codeword according to the RI.

Single antenna mapping is given by

ypi=x0i

where  pϵ{0,1,2,3},                                and

Multiple antennas mapping is given by

The transmitter chooses a codeword according to reported ν. The codebook for 2 and 4 antennas can be found in document with DCN 369r1.

6.6.2.2 Transmit diversity

Transmit diversity is aimed to increase robustness in scenarios with low SNR, low delay tolerance or no feedback to the transmitter is available or reliable.

Case of 2 antennas

  - STBC (space-time block code) for DTF-Spread OFDM

  - SFBC (space-frequency block code) for OFDM

SFBC for 2 antennas is given by

In case of 4 antennas a combination of SFBC (2 antennas) with frequency switch transmission diversity is given by

</707r0>

Bit interleaver

<707r0>

In order to minimize latency and integration on parallel architectures within the encoder/decoder implementation, an algebraic interleaver is proposed.

A maximum contention-free quadratic permutation interleaver is defined as

where i=0,1,…,NI-1. NI is the length of interleaver.

Elements of codewords of length n (ci for i=0,1,…,n-1) are interleaved in blocks of NI bits as c∏(j) for j=0,1,…,NI-1

Short length interleaver:

Long length interleaver:

</707r0>

Scrambling

<707r0>

A scrambler is used to shape the data spectrum and to randomize data across users in order to reduce interference.

A Gold code generator of length 63 is proposed as scrambler

  - PN sequence with period 263 (truly random for long packets)

  - Different initialization seeds, enables a different Gold code per user with low correlation respect to other user using a different seed.

The Gold code generator outputs si for i=0,1,…,263-1, which is used to scramble the interleaved codeword bits ci for i=0,1,…,n-1 prior to modulation as

Shift registers initialization at start of a packet is given by

  - User ID (1st register) and group ID (2nd register)

  - Fast forward both registers 100 times to reduce PAPR

</707r0>

<707r0>

Discovery

The discovery preamble  (DP) is based on a ZC sequence and a discovery resource block (DRB) from a modified DTF-S OFDM or OFDM signal.

  - The DP and DRB formed the Discovery Signal (DS).

Moreover, we propose to use one channel (from the proposed channelization for sub-GHz, 2.4 GHz and 5.7 GHz bands) for only discovery of devices.

PAC devices can either transmit or receive the DS in this unique channel asynchronously. Such unique channel for discovery is named Shared Discovery Channel (SDCH).

The discovery signal (DS) sent over a discovery shared channel (DSCH) is illustrated as

The DS consists of a preamble sequence plus a DRB formed by Nfs frequency slots and Nts time slots.

Once synchronized, a receiver knows the location of the discovery resource block (DRS) to scan for possible peers or to pick time-frequency slots to transmit its DS.

The proposed ZC sequence for initial synchronization during discovery (or communication mode) is a ZC sequence with the following parameters: Sequence length N = 63, relative prime r = 62 and q = 0.

Such ZC sequence is mapped onto 62 sub-carriers

  - The first subcarrier and the DC subcarrier are empty.

Of course, several repetitions may be transmitted

The discovery process is energy intensive. In order to minimize power consumption only one subcarrier of the N-point IFFT is used per user.

  - The PAPR is set to the minimum.

  - Across the frequency domain, users are orthogonal (OFDM)

For discovery, the nth symbol transmitted over the kth subcarrier is given by

  - xnl,k=1NXnkexp(j2πlk/N)

  - for l=-L+1,,…,0,1,…,N-1 and L the CP

Contiguous subcarriers  are orthogonal as illustrated in the figure:

The DS is transmitted with a predefined duty-cycle (see NICT MAC pre-proposal). From upper layers, terminals pick time-frequency slots in the DRB to transmit the DS.

A set of 126 symbols are transmitted per time slot:

  - such frame contains information about a given peer (peer ID, service ID, application ID, etc.) and which channel is used for association (different from the SDCH).

The number of frequency slots Nfs =1024 (IFFT size) and the number of time slots Nts is chosen as 20.

Consequently, the DRB can support up to NfsxNts=20,480 users for discovery per Group.

The discovery data is formatted as illustrated in  the figure:

Append 57 bits for user ID, device ID, Group ID, etc.

Append 6 bits from CRC-6-ITU error detection code

Append 63 bits from shorten BCH(126,63) code

</707r0>

<707r0>

Discovery procedure

Devices are in three possible states:

  - RDM –receiving discovery mode,

  - TDM –transmitting discovery mode,

  - Sleep -idle.

Also, devices are equipped with clear channel assessment (CCA).

1) At start up or after idle or after command, a device enters into receiving discovery mode (RDM) and listens for a DS.

Synchronization subsystem detects the preamble (and hence the position of the DRB in time) and detection subsystem scans the DRB for DRB usage and detection of peer ID, service ID, etc. Go to Sleep mode.

2) After command, a device enters into transmitting discovery mode (TDM)

Device chooses a free time-frequency slot to transmit its  DS over the next DRB transmission. Go to Sleep mode.

Of course, there are intermediate steps like time-out for DRB scanning, miss detection, etc. Here, we present the core algorithm only.

6.10.1 Peering procedure

For association or peering, intended devices that want to establish a communication link, request that through a random access preamble transmitted in the ACH.

The random access association preambles are named Random Access Preambles (RAPs).

Such RAPs are formed with ZC sequences as well.

Hence, a pool of orthogonal RAPs is formed

  - in order to reduce interference from competing terminals for random access.

A unique RAP is assigned to every device in a Group.

Such unique RAP is used for fine synchronization and control messages (how a communication link is granted).

The RAP signal is illustrated as

where GI is guard interval and RA preamble is set of orthogonal preambles.

Thus, a set of orthogonal preamble sequences of length 1024 can be generated from ZC sequences for random access, which satisfy maximum round-trip time and coverage performance for a maximum distance of 500m. Details of preamble design can be found in document with DCN 369r1.

The CP and GI lengths are given by

Once initial synchronization and decoding of discovery RB are achieved by Terminal 2 over Terminal 1:

  - Terminal 2 requests association by a random access procedure based on an orthogonal RAP.

The process is initiated and control by upper layers.

1) Terminal 2 sends RAP over the ACH.

  - It is randomly selected from a pool of orthogonal ZC sequences.

  - It contains finer frequency granularity for Terminal 1 to acquire fine time and frequency synchronization of Terminal 2, plus information about the resources needed to transmit in 3).

2) Terminal 1 replies with RA response message.

  - It contains timing information (round-trip delay), RAP-ID, RB  grant to transmit in 3), Group identifier, etc.

3) Scheduling request

  - It contains scheduling request information for transmission.

  - If this message is successfully detected in Terminal 1, still contention remains unsolved for other terminals. A quick resolution is needed.

4) Contention resolution

  - Terminal 1 echoes Terminal 2 ID contained in 3)

  - Terminal 2 detects its ID and sends ACK (RA terminated) a communication link is scheduled and established.

  - Terminal 2 detects another ID (RA terminated, starts a new one)

  - Terminal 2 fails to detect ID (RA terminated, starts a new one)

</707r0>

<707r0>

Reference signals

Reference signals are required to perform channel estimation in time and frequency domain for detection and equalization.

Channel time dispersion

Considering a maximum speed of v=100 Km/h (27.78 m/s)

The Doppler spread is given by fd=fc v/c

Min. sampling time to reconstruct the channel is Tc=1/2fd

The slot time Tslot=0.5 msec. Then, one reference symbol per slot is needed in the time domain to estimate the channel correctly.

Channel frequency dispersion

Considering 90%, 50% coherence bandwidth as                               and

where      is the RMS delay spread.

The outdoor RMS delay spread is estimated as

For a distance d=500m, we have 90%, 50% coherence bandwidth given in the table

If                   then frequency selective fading and frequency domain equalization is needed.

We propose that the spacing between 2 reference symbols in frequency in a RB is 30 KHz to resolve frequency variations.

ZC sequences

Preambles or beacons for synchronization and reference signals (RSs) are formed with Zadoff-Chu (ZC) sequences of length N.

  - Constant-amplitude zero-correlation (CAZAC) sequences

where                      is a primitive nth root of unity, r is a relative prime to N, sequence index k = 0, 1, ...,N-1and q is any integer.

ZC sequences have constant amplitude and so its N-point DFT.

  - This limits the PAPR and simplifies implementation as only phases have to be generated and stored, besides of bypassing the FFT at transmitter for DFT-S OFDM.

ZC sequences have ideal cyclic autocorrelation, i.e., the correlation with its shifted version is a delta function

  - Thus, a [large] set of orthogonal preambles or reference signals is possible to generate.

</707r0>

<Billy>

UWB Physical (PHY) layer specification

General

The UWB PHY employs a mean PRF that is nominally 16 MHz or optionally nominally 64 MHz.  The UWB PHY waveform is based upon an impulse radio signaling scheme using band-limited data pulses. The UWB PHY supports two independent bands of operation:

- The low band, which consists of four channels and occupies the spectrum from 3.1 GHz to 4.8 GHz

- The high band, which consists of eleven channels and occupies the spectrum from 6.0 GHz to 10.6 GHz

Within each channel, there is support for at least two complex channels that have unique length 31 SHR preamble codes. The combination of a channel and a preamble code is termed a complex channel.

A combination of burst position modulation (BPM) and binary phase-shift keying (BPSK) is used to support both coherent and non-coherent receivers using a common signaling scheme. The combined BPM-BPSK is used to modulate the symbols, with each symbol being composed of an active burst of UWB pulses. The various data rates are supported through the use of variable-length bursts.

Figure 1 shows the sequence of processing steps used to create and modulate a packet. The sequence of steps indicated here for the transmitter is used as a basis for explaining the creation of the UWB waveform. Note that the receiver portion of Figure 1 is informative and meant only as a guide to the essential steps that any compliant UWB receiver needs to implement in order to successfully decode the transmitted signal.

Figure 1 - signal flow

PPDU format

Figure 2 shows the format for the UWB frame, which is composed of three major components: the SHR preamble, the PHR, and the PSDU. For convenience, the PPDU packet structure is presented so that the leftmost field as written in this standard shall be transmitted or received first. All multiple octet fields shall be transmitted or received least significant octet first, and each octet shall be transmitted or received LSB first. The same transmission order should apply to data fields.

The SHR preamble is first, followed by the PHR, and finally the PSDU.  The SHR preamble is always sent at the base rate for the preamble code.  The PHR is sent at a nominal rate of 850 kb/s for all data rates above 850 kb/s and at a nominal of 110 kb/s for the nominal data rate of 110 kb/s. The PSDU is sent at the desired information data rate as defined in Table 3 – rate-dependent and timing dependent parameters.

The PSDU contains MAC layer messages.

PPDU encoding process

The encoding process is composed of many steps as illustrated in Figure 2.  The details of these steps are fully described in later sub-clauses, as noted in the following list, which is intended to facilitate an understanding of those details:

- Perform Reed-Solomon encoding on PSDU as described in 2.3.3.1.

- Produce the PHR as described in 2.2.6.1.

- Add SECDED check bits to PHR as described in 2.2.6.2 and prepend to the PSDU.

- Perform further convolutional coding as described in 2.3.3.2. Note that in some instances at the 27 Mb/s data rate, the convolutional encoding of the data field is effectively bypassed and two data bits are encoded per BPM-BPSK symbol.

- Modulate and spread PSDU according to the method described in 2.3.1 and 2.3.2. The PHR is modulated using BPM-BPSK at 850 kb/s or at 110 kb/s (for the 110 kb/s data rate) and the data field is modulated at the rate specified in the PHR.

- Produce the SHR preamble field from the SYNC field (used for AGC convergence, diversity selection, timing acquisition, and coarse frequency acquisition) and the SFD field (used to indicate the start of frame). The SYNC and SFD fields are described in 2.2.5.1 and 2.2.5.2, respectively.

Figure 2 – PPDU encoding process

Table 1 and Table 2 show how the 19 header bits (H0-H18), N data bits (D0-DN-1), and two tail bits (T0-T1) are mapped onto the symbols. In these tables, the polarity bit column operation is an XOR. The tables also show when the transition from the header bit rate to the data bit rate takes place. Note that the delay line of the convolutional code is initialized to zero. For this reason, the position bit of Symbol 0 shall always be zero. This means that Symbol 0 is always transmitted in the first half of the first header symbol.

Table 1 – mapping of header bits, data bits and tail bits onto symbols with Viterbi rate 0.5

Symbol structure

In the BPM-BPSK modulation scheme, a symbol is capable of carrying two bits of information: one bit is used to determine the position of a burst of pulses while an additional bit is used to modulate the phase (polarity) of this same burst.

The structure and timing of a symbol is illustrated in Figure 3. Each symbol shall consist of an integer number of possible chip positions, Nc, each with duration Tc. The overall symbol duration denoted by Tdsym is given by Tdsym = NcTc. Furthermore, each symbol is divided into two BPM intervals each with duration TBPM =Tdsym /2, which enables binary position modulation.

A burst is formed by grouping Ncpb consecutive chips and has duration Tburst = NcpbTc. The location of the burst in either the first half or second half of the symbol indicates one bit of information. Additionally, the phase of the burst (either –1 or +1) is used to indicate a second bit of information.

In each symbol interval, a single burst event shall be transmitted. The fact that burst duration is typically much shorter than the BPM duration, i.e., Tburst << TBPM, provides for some multi-user access interference rejection in the form of time hopping. The total number of burst durations per symbol, Nburst, is given by Nburst = Tdsym /Tburst. In order to limit the amount of inter-symbol interference caused by multipath, only the first half of each TBPM period shall contain a burst. Therefore, only the first Nhop= Nburst/4 possible burst positions are candidate hopping burst positions within each BPM interval. Each burst position can be varied on a symbol-to-symbol basis according to a time hopping code as described in 2.3.

Table 2 –mapping of header bits, data bits and tail bits onto symbols with Viterbi rate 1

PSDU timing parameters

The PSDU rate-dependent parameters and timing-related parameters are summarized in Table 3. Within each channel {0:15}, the peak PRF shall be 499.2 MHz. This rate corresponds to the highest frequency at which a compliant transmitter shall emit pulses. Additionally, the mean PRF is defined as the total number of pulses emitted during a symbol period divided by the length of the symbol duration. During the SHR preamble portion of a UWB frame, the peak and mean PRFs are essentially the same since pulses are emitted uniformly during each preamble symbol. During the data portion of a PPDU, however, the peak and mean PRFs differ due to the grouping of pulses into consecutive chip durations.

There are two possible preamble code lengths (31 or 127) and two mean PRFs (15.6 MHz or 62.4 MHz). A compliant device shall implement support for the preamble code length of 31 and the 15.6 MHz mean PRFs for the PSDU as depicted in Table 3. The use of the length 127 code is optional; when implemented, the mean PRF of the PSDU shall be 62.4 MHz.

UWB channels {4, 7, 11, and 15} are all optional channels and are differentiated from other UWB channels by the larger bandwidth (> 500 MHz) of the transmitted signals. These channels overlap the lower bandwidth channels. The larger bandwidth enables devices operating in these channels to transmit at a higher power (for fixed PSD constraints), and thus they may achieve longer communication range. The larger bandwidth pulses offer enhanced multipath resistance. Additionally, larger bandwidth leads to more accurate range estimates. The admissible data rates, preamble code lengths, PRFs, and modulation timing parameters are listed in Table 3.  Each UWB channel allows for several data rates that are obtained by modifying the number of chips within a burst while the total number of possible burst positions remains constant. Therefore, the symbol duration, Tdsym, changes to obtain the stated symbol rate and bit rates.

Figure 3 – symbol structure

Table 3 – rate-dependent and timing dependent parameters

| Preamble Code Length | Modulation & Coding |  |  | Data Symbol Structure |  |  |  |  |  | Data |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Viterbi Rate | RS Rate | Overall FEC Rate | #Burst Positions per Symbol Nburst | # Hop Bursts Nhop | # Chips Per Burst Ncpb | #Chips Per Symbol | Burst Duration T burst (ns) | Symbol Duration Tdsym (ns) | Symbol Rate (MHz) | Bit Rate Mb/s | Mean PRF (MHz) |
| 31 31 31 31 | 0.5 0.5 0.5 1.0 | 0.87 0.87 0.87 0.87 | 0.44 0.44 0.44 0.87 | 32 32 32 32 | 8 8 8 8 | 128 16 2 1 | 4096 512 64 32 | 256.41 32.05 4.01 2.00 | 8205.13 1025.64 128.21 64.10 | 0.12 0.98 7.80 15.60 | 0.11 0.85 6.81 27.24 | 15.60 15.60 15.60 15.60 |
| 127 127 127 127 | 0.5 0.5 0.5 0.5 | 0.87 0.87 0.87 0.87 | 0.44 0.44 0.44 0.44 | 8 8 8 8 | 2 2 2 2 | 512 64 8 2 | 4096 512 64 16 | 1025.64 128.21 16.03 4.01 | 8205.13 1025.64 128.21 32.05 | 0.12 0.98 7.80 31.20 | 0.11 0.85 6.81 27.24 | 62.40 62.40 62.40 62.40 |

The peak PRF is 499.2 MHz. This is the highest frequency in megahertz at which a compliant transmitter shall emit pulses. The peak PRF is also used to derive the chip duration Tc by the formula Tc = 1/peakPRF. The value of Tc is approximately 2 ns.  The channel center frequencies and bandwidths are given in Table 11.  Note that the bandwidth is not necessarily the inverse of the chip duration Tc. Pulse shape and bandwidth are further defined in 2.4.3.1.

The UWB PHY contains several optional data rates, preamble code lengths, and PRF. Table 3 describes the remaining timing parameters of Figure 3 for each permitted combination of preamble code length and PRF.

The preamble code length parameter denotes the length of the preamble code to be used during the SHR portion of a data frame. The code together with the channel number defines a complex channel. Individual codes to be used on each channel are given in Table 6 (length 31) and Table 7 (length 127).

The Viterbi rate parameter determines the rate of the convolutional code applied to the PSDU data bits. A value of 1 indicates that no convolutional coding is applied while a value of 0.5 indicates that a rate 1/2 code as described in 2.3.3.2 is applied to the PSDU data bits.

The RS rate parameters indicates the (63, 55) Reed-Solomon code rate, which is approximately 0.87. The Reed-Solomon code is applied to all the PSDU data bits that are transmitted by the UWB PHY. Reed-Solomon encoding is further described in 2.3.3.1.

The overall FEC rate is determine by the product of the Viterbi rate and the Reed-Solomon rate and has either a value of 0.44 or 0.87.

The burst-positions-per-symbol parameter is the total number of possible burst positions within the data symbol duration. Nburst has been chosen so that for each mean PRF a data symbol consists of a fixed number of burst durations.

The hop bursts parameter is the number of burst positions that may contain an active burst, that is, a burst containing UWB pulses. The value is computed as Nhop = Nburst/4.

The chips per burst parameter is the number of chip Tc durations within each burst period Tburst. Each burst consists of a multiple number of consecutive chips, as illustrated in Figure 3. Depending on the data rate to be used in the transmission of the PSDU, the number of chips in a burst varies, e.g., for low data rates, the burst consists of more chip periods than for high data rates. Particular, values of Ncpb have been selected so that the following is a valid data rate: (2 × Overall FEC rate)/(Ncpb × Nburst × Tc).

The burst duration is computed as Tburst = Ncpb × Tc.

The symbol duration is the duration of a modulated and coded PSDU symbol on the air and is computed as follows: Tdsym = Nburst × Tburst.

The symbol rate is the inverse of the PSDU symbol duration 1/Tdsym.

The bit rate is the user information rate considering FEC and is computed as follows:

Bit Rate = 2 × (Overall FEC Rate)/Tdsym

The mean PRF is the average PRF during the PSDU portion of a PHY frame and is computed as follows:

Mean PRF = Ncpb/Tdsym

Preamble timing parameters

Due to the variability in the preamble code length and the PRF, there are several admissible values for the timing parameters of a preamble symbol. These values are summarized in Table 4. In this subclause, a preamble symbol is defined as the waveform consisting of one whole repetition of the modulated preamble code (either length 31 or 127). Details on the construction of the preamble symbol for various code lengths and PRFs are given in 2.2.5. For each target PRF, the preamble is constructed from a preamble code, Ci, by inserting a number of chip durations between code symbols. The number of chip durations to insert is denoted by δL, values for each code length and PRF are given in Table 4, and the chip insertion is detailed in 2.2.5.1.

Table 4 presents the timing parameters during the SHR portion of a UWB PHY frame while Table 3 presents the timing parameters for the PSDU portion of the frame. First, note that the preamble is sent at a slightly higher mean PRF than the data as defined in Table 3. This is due to the fact that length 31 or 127 ternary codes are being used within the SHR, and the number of chips within the SHR is no longer a power of 2. For example, for the 16 MHz PRF in channels {0:3, 5:6, 8:10, 12:14), the peak PRF during the preamble is 31.2 MHz, and the corresponding mean PRF during the preamble is 16.10 MHz. The mean PRF during the data (PSDU) is 15.60 MHz. The remaining peak and mean PRF values for other optional UWB channels and the optional length 127 code are listed in Table 4.

Table 4 – preamble parameters

| Ci Code Length | Peak PRF (MHz) | Mean PRF (MHz) | Delta Length δL | #Chips Per Symbol | Symbol Duration Tpsym(ns) | Base Rate Msymbol/s |
| --- | --- | --- | --- | --- | --- | --- |
| 31 | 31.20 | 16.10 | 16 | 496 | 993.59 | 1.01 |
| 127 | 124.80 | 62.89 | 4 | 508 | 1017.63 | 0.98 |

The base symbol rate is defined as the rate at which the preamble symbols are sent. The base rate corresponding to the (default) mean PRF of 16.10 MHz is 1 Msymbol/s.  This symbol rate corresponds to preamble symbol duration, Tpsym, of 993.59 ns.

Table 5 – frame-dependent parameters

| Parameter | Description | Value |  |
| --- | --- | --- | --- |
| Channel | UWB PHY channel number | {1:15} |  |
| PRFmean | Mean PRF (MHz) | 16.10 | 62.89 |
| Nc | Number of chips per preamble symbol | 496 | 508 |
| Tpsym | Preamble symbol duration (ns) | 993.6 | 1017.6 |
| Nsync | Number of symbols in the packet sync sequence. | 64 to 4096 |  |
| Tsync | Duration of the packet sync sequence (μs) | 63.6 to 4069.7 | 65.1 to 4168.2 |
| Nsfd | Number of symbols in the SFD | 8 (or 64) |  |
| Tsfd | Duration of the SFD (μs) | 7.9 (or 63.6) | 8.1 (or 65.1) |
| Npre | Number of symbols in the SHR preamble | 72 to 4104 (or 128 to 4160) |  |
| Tpre | Duration of the SHR preamble (μs) | 71.5 to 4077.7 (or 127.2 to 4133.3) | 73.3 to 4176.3 (or 319.5 to 4422.6) |
| NCCA_PHR | Number of multiplexed preamble symbols in PHR | 4 or 32 |  |
| NCCA_data | Number of multiplexed preamble symbols in the data field | Tpre/(4 × Tdsym/M) |  |

Note: the values in brackets apply to the 110 Kb/s data rate.

Finally, for each UWB frame consisting of the SHR, SFD, PHR, and a data field, there are four possible durations of the SHR. This is due to the four possible lengths of SYNC field in the SHR, as described in 2.2.5. The SYNC field consists of repetitions of the preamble symbol. The number of preamble symbol repetitions may be 64, 1024, or 4096, with additional optional values of 128, 256, 512, 1536 and 2048. These different SYNC field lengths yield different time durations of the UWB frame. The relationship between SYNC field length and frame duration is shown in Table 5. After the insertion of the SFD (the SFD may be either 8 or 64 preamble symbols long), the total length (in preamble symbols) of the SHR is Npre as shown in Table 5, and this in turn leads to the possible SHR durations denoted as Tpre. After creation of the SHR, the frame is appended with the PHR whose length, Nhdr, is 16 symbols and duration is denoted as Thdr. The values of the frame duration parameters are shown in Table 5.

SHR preamble

A SHR preamble shall be added prior to the PHR to aid receiver algorithms related to AGC setting, antenna diversity selection, timing acquisition, coarse and fine frequency recovery, packet and frame synchronization, channel estimation, and leading edge signal tracking for ranging.

In this subclause, four different mandatory preambles are defined: a default preamble, a short preamble, a medium preamble, and a long preamble. The preamble to be used in the transmission of a frame is decided by the application layer.

Figure 4 shows the structure of the SHR preamble. The preamble can be subdivided into two distinct portions: SYNC (packet synchronization, channel estimation, and ranging sequence) and SFD (frame delimiter sequence). The duration of these portions are provided in Table 5. Subclauses 2.2.5.1 and 2.2.5.2 detail the different portions of the preamble.

Figure 4 – SHR preamble structure

SHR SYNC field

Each network operating on one of the UWB PHY channels {1–15} is also identified by a preamble code. The preamble code is used to construct symbols that constitute the SYNC portion of the SHR preamble as shown in Figure 4.

The UWB PHY supports two lengths of preamble code: a length 31 code and an optional length 127 code. Each preamble code is a sequence of code symbols drawn from a ternary alphabet {-1,0,1} and selected for use in the UWB PHY because of their perfect periodic autocorrelation properties. The length 31 code sequences are shown in Table 6 while the length 127 code sequences are shown in Table 7 where they are indexed from 1–24 (Ci i = 1,2,...24). The first 8 codes (index 1–8) are length 31 while the remaining 16 (index 9–24) are length 127. Which codes may be used in each of the channels is restricted, and the particular code assignments are made in Table 6 and Table 7. Specifically, the last column in each table indicates the set of channel numbers that permit use of the code. This restriction of codes is to ensure that codes with the lowest cross-correlation are used in the same channel. Additionally, 8 of the length 127 codes are reserved for use with the private ranging protocol only and are not used during normal operation. This restriction is indicated in the third column of Table 7 as well.

Table 6 – length 31 ternary codes

| Code index | Code sequence | Channel numbera |
| --- | --- | --- |
| 1 | -0000+0-0+++0+-000+-+++00-+0-00 | 1, 8, 12 |
| 2 | 0+0+-0+0+000-++0-+---00+00++000 | 1, 8, 12 |
| 3 | -+0++000-+-++00++0+00-0000-0+0- | 2, 5, 9, 13 |
| 4 | 0000+-00-00-++++0+-+000+0-0++0- | 2, 5, 9, 13 |
| 5 | -0+-00+++-+000-+0+++0-0+0000-00 | 3, 6, 10, 14 |
| 6 | ++00+00---+-0++-000+0+0-+0+0000 | 3, 6, 10, 14 |
| 7 | +0000+-0+0+00+000+0++---0-+00-+ | 4, 7, 11, 15 |
| 8 | 0+00-0-0++0000--+00-+0++-++0+00 | 4, 7, 11, 15 |
| a- Note code indices 1 through 6 may also be used for channels 4, 7, 11, and 15 (i.e., channels with bandwidth wider than 500 MHz) if interchannel communication is desired. |  |  |

Table 7 – length 127 ternary codes

| Code index | Code sequence | Channel numbera |
| --- | --- | --- |
| 9 | +00+000-0--00--+0+0+00-+-++0+0000++-000+00-00--0-+0+0--0-+++0++000+-0+00-0++-0+++00-+00+0+0-0++-+--+000000+00000-+0000-0-000--+ | 1–3, 5, 6, 8–10, 12–14 |
| 10 | ++00+0-+00+00+000000-000-00--000-0+-+0-0+-0-+00000+-00++0-0+00--+00++-+0+-0+0000-0-0-0-++-+0+00+0+000-+0+++000----+++0000+++0-- | 1–3, 5, 6, 8–10, 12–14 |
| 11 | -+-0000+00--00000-0+0+0+-0+00+00+0-00-+++00+000-+0+0-0000+++++-+0+--0+-0++--0-000+0-+00+0+----000-000000-+00+-0++000++-00++-0-0 | 1–3, 5, 6, 8–10, 12–14 |
| 12 | -+0++000000-0+0-+0---+-++00-+0++0+0+0+000-00-00-+00+-++000-+-0-++0-0++++0-00-0++00+0+00++-00+000+-000-0--+0000-0000--0+00000+-- | 1–3, 5, 6, 8–10, 12–14 |
| 13 | +000--0000--++0-++++0-0++0+0-00-+0++00++-0++0+-+0-00+00-0--000-+-00+0000-0++-00000+-0-000000-00-+-++-+000-0+0+0+++-00--00+0+000 | 1–15; DPS only |
| 14 | +000++0-0+0-00+-0-+0-00+0+0000+0+-0000++00+0+++++-+0-0+-0--+0++--000---0+000+0+0-+-000000+-+-0--00++000-00+00++-00--++-00-00000 | 1–15; DPS only |
| 15 | 0+-00+0-000-++0000---++000+0+-0-+00-+000--0-00--0--+++-+0-++00+-++0+00000+0-0+++-00+00+000-0000+00--+0++0+0+0-00-0-+-0+0++00000 | 1–15; DPS only |
| 16 | ++0000+000+00+--0+-++0-000--00+-0+00++000+++00+0+0-0-+-0-0+00+00+0++----+00++--+0+-0--+000000-0-0000-+0--00+00000+-++000-0-+0+0 | 1–15; DPS only |
| 17 | +--000-0-0000+-00000+000000+--+-++0-0+0+00+-00+++0-++0-00+0-+000++0+++-0--0+0+-0--00-00+000-++0000+0++-+-00+0+0+--00--0-000+00+ | 4, 7, 11, 15 |
| 18 | --0+++0000+++----000+++0+-000+0+00+0+-++-0-0-0-0000+0-+0+-++00+--00+0-0++00-+00000+-0-+0-0+-+0-000--00-000-000000+00+00+-0+00++ | 4, 7, 11, 15 |
| 19 | -0-++00-++000++0-+00+-000000-000----+0+00+-0+000-0--++0-+0--+0+-+++++0000-0+0+-000+00+++-00-0+00+00+0-+0+0+0-00000--00+0000-+-0 | 4, 7, 11, 15 |
| 20 | --+00000+0--0000-0000+--0-000-+000+00-++00+0+00++0-00-0++++0-0++-0-+-000++-+00+-00-00-000+0+0+0++0+-00++-+---0+-0+0-000000++0+- | 4, 7, 11, 15 |
| 21 | +0+00--00-+++0+0+0-000+-++-+-00-000000-0-+00000-++0-0000+00-+-000--0-00+00-0+-+0++0-++00++0+-00-0+0++0-0++++-0++--0000--000+000 | 1–15; DPS only |
| 22 | 0-00-++--00-++00+00-000++00--0-+-+000000-+-0+0+000+0---000--++0+--0-+0-0+-+++++0+00++0000-+0+0000+0+00-0+-0-+00-0+0-0++000+0000 | 1–15; DPS only |
| 23 | 000++0+0-+-0-00-0+0+0++0+--00+0000-000+00+00-+++0-0+00000+0++-+00++-0+-+++--0--00-0--000+-00+-0-+0+000++---0000++-000-0+00-+000 | 1–15; DPS only |
| 24 | +0+-0-000++-+00000+00--0+-0000-0-000000+--0-+0+--++00+----++0+00+00+0-0-+-0-0+0+00+++000++00+0-+00--000-0++-+0--+00+000+0000++0 | 1–15; DPS only |
| a- Note code indices 9 through 13 may also be used for UWB channels 4, 7, 11, and 15 (i.e., channels with bandwidth wider than 500 MHz) if interchannel communication is desired. |  |  |

Note that the assignment of preamble codes to channels has been done to enable interchannel communication. In other words, it is possible that a device operating on a wideband channel {4, 7, 11 or 15} may communicate with a device on a channel with which it overlaps.

When using the ternary code indexed by i, the SYNC field shall consist of Nsync repetitions of the symbol Si, where Si is the code Ci spread by the delta function δL of length L as shown in Table 4. The spreading operation, where code Ci is extended to the preamble symbol duration indicated in Table 4, is described mathematically by

where the operator  indicates a Kronecker product. After the Kronecker operation, a preamble symbol is formed as depicted in Figure 5, where L – 1 zeros have been inserted between each ternary element of Ci.

The spreading factor L, number of chips per symbol, preamble symbol duration Tpsym, and base symbol rate for different channels are given in Table 4.

Figure 5 – construction of symbol Si from code Ci

SHR SFD

An SFD shall be added to establish frame timing. The UWB PHY uses a short SFD for default and medium data rates and a long SFD for the optional low data rate of 110 kb/s as shown in Figure 4.  The short SFD shall be [0 +1 0 -1 +1 0 0 -1] spread by the preamble symbol Si, where the leftmost bit shall be transmitted first in time. The optional long SFD shall be obtained by spreading the sequence [0 +1 0 -1 +1 0 0 -1 0 +1 0 -1 +1 0 0 -1 -1 0 0 +1 0 -1 0 +1 0 +1 0 0 0 -1 0 -1 0 -1 0 0 +1 0 -1 -1 0 -1 +1 0 0 0 0 +1 +1 0 0 -1 -1 -1 +1 -1 +1 +1 0 0 0 0 +1 +1] by the preamble sequence Si. Note that the long SFD is eight times longer than the short SFD and consists of 64 preamble symbols, only 32 of which are active, and the other 32 are zeros. The structure of the SHR preamble and the two possible SFDs are shown in Figure 4.

PHY header (PHR)

A PHR, as shown in Figure 6, shall be added after the SHR preamble. The PHR consists of 19 bits and conveys information necessary for a successful decoding of the packet to the receiver. The PHR contains information about the data rate used to transmit the PSDU, the duration of the current frame’s preamble, and the length of the frame payload. Additionally, six parity check bits are used to further protect the PHR against channel errors.

Figure 6 – PHR bit assignment

The PHR shall be transmitted using the BPM-BPSK modulation outlined in 2.3. The PHR shall be transmitted at the nominal rate of 850 kb/s for all data rates above 850 kb/s and at the nominal rate of 110 kb/s for the nominal low data rates of 110 kb/s.

PHR rate, length, ranging, extension, preamble duration fields

The Data Rate field shall consist of two bits (R1, R0) that indicate the data rate of the received PSDU. The bits R1–R0 shall be set according to Table 9. The default value of the bits R1–R0 shall be set to 01 as this is the only mandatory data rate that is supported by a UWB-compliant PHY implementation. Support for other data rates listed in Table 9 is optional.

The Frame Length field, L6–L0, shall be an unsigned 7-bit integer number that indicates the number of octets in the PSDU that the MAC sublayer is currently requesting the PHY to transmit.

The Ranging Packet bit, RNG, indicates that the current frame is an RFRAME if it is set to 1; otherwise, it is set to 0.

The Header Extension bit, EXT, is reserved for future extension of the PHR. This bit shall be set to 0.

The Preamble Duration field, P1–P0, represents the length (in preamble symbols) of the SYNC portion of the SHR. P1–P0 shall be set according to Table 8. The default Preamble Duration setting is 01, which corresponds to a SYNC field of length 64 preamble symbols.

Table 8 – Preamble Duration field values

| P1–P0 | SYNC length (symbols) (Si) |
| --- | --- |
| 01 | 64 |
| 10 | 1024 |
| 11 | 4096 |

The Preamble Duration field is intended for use during ranging operations and is used by a receiver of the PHY frame to help determine at which preamble symbol the UWB PHY acquired and began tracking the preamble. A receiver may use the Preamble Duration field to set the value of its own preamble duration based upon the received value when communicating a ranging ACK packet.   The optional values of 128, 256, 512, 1536 and 2048 cannot be encoded within the PHY header (PHR), but may be ascertained in the receiver by counting the number of preamble symbols received.  Where one of these optional values is used the PHR encoding shall be the nearest smaller length value.

Table 9 – nominal data rates

| R1–R0 | Data rate Mb/s |
| --- | --- |
| 00 | 0.11 |
| 01 | 0.85 |
| 10 | 6.81 |
| 11 | 27.24 |

PHR SECDED check bits

The SECDED (single error correct, double error detect) field, C5–C0, is a set of six parity check bits that are used to protect the PHR from errors caused by noise and channel impairments. The SECDED bits are a simple Hamming block code that enables the correction of a single error and the detection of two errors at the receiver. The SECDED bit values depend on PHR bits 0–12 and are computed as follows:

C0 = XOR (R0, R1, L0, L2, L4, L5, EXT, P1)

C1 = XOR (R1, L2, L3, L5, L6, RNG, EXT, P0)

C2 = XOR (R0, L0, L1, L5, L6, RNG, EXT)

C3 = XOR (L0, L1, L2, L3, L4, RNG, EXT)

C4 = XOR (P0, P1)

C5 = XOR (R1, R0, L6, L5, L4, L3, L2, L1, L0, RNG, EXT, P1, P0, C4, C3, C2, C1, C0)

Data field

The Data field is the last component of the PPDU and is encoded as shown in Figure 7.

Figure 7 – data field encoding process

The data field shall be formed as follows:

- Encode the PSDU using systematic Reed-Solomon block code, which adds 48 parity bits as described in 2.3.3.1.

- Encode the output of the Reed-Solomon block code using a systematic convolutional encoder as described in 2.3.3.2, except in the cases where the Viterbi rate for the modulation is 1.0 in Table 3. In these cases the convolutional encoder is bypassed.

- Spread and modulate the encoded block using BPM-BPSK modulation as described in 2.3.

UWB PHY modulation

Modulation mathematical framework

The transmit waveform during the kth symbol interval may be expressed as

This equation describes the time hopping with polarity scrambling, which improves interference rejection capabilities of the UWB PHY. The kth symbol interval carries two information bits  and . Bit  is encoded into the burst position whereas bit  is encoded into the burst polarity. The sequence  is the scrambling code used during the kth symbol interval,  is the kth burst hopping position, and p(t) is the transmitted pulse shape at the antenna input. The burst hopping sequence  provides for multiuser interference rejection. The chip scrambling sequence  provides additional interference suppression among coherent receivers as well as spectral smoothing of the transmitted waveform. Note that equation defines the transmitted signal during the valid burst interval; at all other possible burst positions, no signal shall be transmitted. A reference modulator illustrating the BPM-BPSK modulation is shown in Figure 8.

Figure 8 – reference symbol modulator

Note here that the FEC Encoder is not included if the modulation Viterbi rate is 1.0, as described in 2.2.7. In this case, the FEC encoder is replaced by a multiplexer which shall apply even bits to the position input and odd bits to the polarity input.

Spreading

The time-varying spreader sequence  and the time-varying burst hopping sequence  shall be generated from a common PRBS scrambler.

The polynomial for the scrambler generator shall be

where D is a single chip delay, Tc, element. This polynomial forms not only a maximal length sequence, but also is a primitive polynomial. By the given generator polynomial, the corresponding scrambler output is generated as

where  denotes modulo-2 addition.

A linear feedback shift register (LFSR) realization of the scrambler is shown in Figure 9. The LFSR shall be initialized upon the transmission of bit 0 of the PHR. Note that Ncpb may change depending on the data rate and PRF in use during the PSDU. The LFSR shall not be reset after transmission of the PHR.

Figure 9 – LFSR implementation of the scrambler

The initial state of the LFSR shall be determined from the preamble code by first removing all the 0s in the ternary code and then replacing all the –1s with a zero. The first 15 bits of the resulting binary state shall be loaded into the LFSR. Table 10 shows an example of the above procedure for preamble code, C6 (length 31, preamble code index 6, see Table 6).  Table 10 shows the initial state as well as the first 16 output bits from the scrambler.

Table 10 – Example LFSR initial state for preamble code 6

| Initial state (s-15, s-14, …, s-1) | LFSR output: First 16 bits s0, s1, …, s15 (s0 first in time) |
| --- | --- |
| 111000101101101 | 0010011101101110 |

Note that even though each device within a network use the same initial LFSR setting, the communication is asynchronous so that the hopping and scrambling provides interference rejection.

The LFSR shall be clocked at the peak PRF of 499.2 MHz as specified in Table 3. During the kth symbol interval, the LFSR shall be clocked Ncpb times, and the scrambler output shall be the kth scrambling code . Furthermore, the kth burst hopping position, shall be computed as follows:

where

As shown in Table 3, the number of hopping burst Nhop is always a power of two, and consequently m is always an integer. Note that for Ncpb < m, the LFSR is clocked Ncpb times, not m times.

For the mandatory mode with mean data PRF of 15.60 MHz, the numbers of hopping bursts is 8, as indicated in Table 3, and consequently m takes on the values 3 and the corresponding hopping sequence is as follows:

h

k





s

k

N

c

p

b

2

s

1

k

N

c

p

b

+

4

s

2

k

N

c

p

b

+

+

+

=

Forward error correction (FEC)

The FEC used by the UWB PHY is a concatenated code consisting of an outer Reed-Solomon systematic block code and an inner half-rate systematic convolutional code. The inner convolutional code is not necessarily enabled at all data rates; the rows of  Table 3 that have a Viterbi rate of 1 indicate that the inner convolutional code is disabled for the PSDU part of the PHY frame.

The FEC encoding of a block of M PSDU bits, b0, b1, …, bM-1, is shown in Figure 10. The Reed-Solomon encoder shall append 48 parity bits, p0, p1, …, p47, to the original block. This results in a Reed-Solomon encoded block of length M + 48. Where the Viterbi rate is 0.5, a half-rate systematic convolutional encoder shall encode the Reed-Solomon encoded block into a systematic coded block of length 2M + 96 bits. The convolutional systematic bits shall be used to encode the position of the burst whereas the convolutional parity bits shall be used to encode the polarity of the pulses within a burst. Where the Viterbi rate is 1.0, even outputs of the Reed-Solomon encoder (b0, b2,..., bM–2, p0, p2,…, p46) shall be used to encode the position of the burst, and odd outputs (b1, b3,..., bM–1, p1, p3,…, p47) shall be used to encode the polarity of the pulses. Note here that M is always an even number.

A noncoherent receiver cannot see the convolutional parity bits (parity bits), and consequently a noncoherent receiver may use only a Reed-Solomon decoder to improve its performance. A coherent receiver may use either or both Reed-Solomon and convolutional decoding algorithms. Note here that since both the Reed-Solomon and the convolutional codes are both systematic, a receiver (either coherent or noncoherent) may be implemented without an FEC decoder. In this case, the information bits are simply recovered by demodulating the position of the burst. There will be additional parity check bits as a result of the Reed-Solomon encoding, but these may be simply ignored.

Figure 10 – FEC encoding process

Reed-Solomon encoding

The systematic Reed-Solomon code is over Galois field, GF(26), which is built as an extension of GF(2). The systematic Reed-Solomon code shall use the generator polynomial

where α = 010000 is a root of the binary primitive polynomial  in GF(26).

In Reed-Solomon encoding RS6(K + 8, K), a block of I bits (with ) is encoded into a codeword of I + 48 bits. The Reed-Solomon encoding procedure is performed in the following five steps:

- Addition of dummy bits. The  block  of  I  information  bits  is  expanded  to  330 bits by adding 330 – I dummy (zero) bits to the beginning of the block. The expanded block is denoted as {d0, d1, ..., d329} where d0 is the first in time.

- Bit-to-symbol conversion. The 330 bits {d0, d1, ..., d329} are converted into 55 Reed-Solomon symbols {D0, D1, ..., D54} having the following polynomial representation:

Resulting 6-bit symbols are presented as , where d6k+5 is the MSB and d6k is the LSB.

- Encoding. The information symbols {D0, D1, ..., D54} are encoded by systematic RS6(63,55) code with output symbols {U0, U1, ..., U62} ordered as follows:

where Pk are parity check symbols added by RS6(63,55) encoder.

The information polynomial associated with the information symbols {D0, D1, ..., D54} is denoted as . The parity check polynomial associated with the parity check symbols is denoted as . The parity check symbols are calculated as:

- Symbol-to-bit conversion. The output symbols {U0, U1, ..., U62} are converted into binary form with LSB coming out first, resulting in a block of 378 bits {u0, u1, ..., u377}.

- Removal of dummy bits. The 330 – I dummy bits added in the first step are removed. Only the last I + 48 bits are transmitted, i.e., {u330-I, u331-I, ..., u377} with u330-I being first in time.

Systematic convolutional encoding

The inner convolutional encoder shall use the rate R = ½ code with generator polynomials g0 = [010]2 and g1 = [101]2 as shown in Figure 11. Upon transmission of each PPDU, the encoder shall be initialized to the all zero state. Additionally, the encoder shall be returned to the all zero state by appending two zero bits to the PPDU. Note that since the generator polynomials are systematic, they are also noncatastrophic.

Figure 11 – systematic convolutional encoder

UWB PHY RF requirements

Operating frequency bands

The set of operating frequency bands are as defined in Table 11.  Default channel is channel 5.

Channel assignments

For each of the 15 operating frequency bands defined in Table 11, at the nominal 16 MHz PRF, two preamble codes are assigned, as per Table 6, giving a total of 30 complex channels.  At the optional nominal 64 MHz PRF, an additional 60 complex channels are available by employing the four preamble codes assigned for each channel as per Table 7.

Table 11 – band allocation

| Channel number (decimal) | Center frequency, fc (MHz) | Band width (MHz) |
| --- | --- | --- |
| 1 | 3494.4 | 499.2 |
| 2 | 3993.6 | 499.2 |
| 3 | 4492.8 | 499.2 |
| 4 | 3993.6 | 1331.2 |
| 5 | 6489.6 | 499.2 |
| 6 | 6988.8 | 499.2 |
| 7 | 6489.6 | 1081.6 |
| 8 | 7488.0 | 499.2 |
| 9 | 7987.2 | 499.2 |
| 10 | 8486.4 | 499.2 |
| 11 | 7987.2 | 1331.2 |
| 12 | 8985.6 | 499.2 |
| 13 | 9484.8 | 499.2 |
| 14 | 9984.0 | 499.2 |
| 15 | 9484.8 | 1354.97 |

Transmitter specification

Baseband impulse response

The transmitted pulse shape p(t) of the UWB PHY shall be constrained by the shape of its cross-correlation function with a standard reference pulse, r(t). The normalized cross-correlation between two waveforms is defined as

In the above, Er and Ep are the energies of r(t) and p(t), respectively. The reference r(t) pulse used in the calculation of is a root raised cosine pulse with roll-off factor of β = 0.5. Mathematically this is

In the above equation, Tp is the reciprocal of the chip frequency.  Table 12 shows the required pulse duration for each channel.

Table 12 – required reference pulse durations in each channel

| Channel number | Pulse duration, Tp (ns) | Main lobe width, Tw (ns) |
| --- | --- | --- |
| {1:3, 5:6, 8:10, 12:14} | 2.00 | 0.5 |
| 7 | 0.92 | 0.2 |
| {4, 11} | 0.75 | 0.2 |
| 15 | 0.74 | 0.2 |

In order for a UWB PHY transmitter to be compliant with this standard, the transmitted pulse p(t) shall have a magnitude of the cross-correlation function  whose main lobe is greater or equal to 0.8 for a duration of at least Tw (see Table 12), and any sidelobe shall be no greater than 0.3. For the purposes of testing a pulse for compliance, the following are defined: Let  be the magnitude of the cross-correlation of p(t) and r(t), and let  i = 1,2,... be a set of critical points, i.e. points at which . The maximum of the function occurs at one of these critical points,  where  for all values of . The requirement above thus states that for some continuous set of values that contain the point  the function  is greater than 0.8. In addition, the second constraint on the value of sidelobes may be stated mathematically as  for all .

Figure 12 shows an example UWB-compliant pulse, p(t) (left plot), along with the root raised cosine reference pulse r(t) (middle plot) with Tp = 2.0 ns and the magnitude of the cross-correlation  (right plot). The pulse p(t) is an 8 order butterworth pulse with a 3 dB bandwidth of 500 MHz. The figure is intended to show that this example pulse meets the requirements for compliance. Specifically, the main lobe is above 0.8 for nearly 1 ns, and no sidelobe is greater than 0.3 (in this case, the largest sidelobe peak is 0.2). The pulse p(t) is a compliant pulse for channels {1:3, 5:6, 8:10, 12:14}.

Figure 12 – compliant pulse example

Note that it is not the intention of this standard to imply that pulse shaping shall occur at baseband, only that the measurements described here occur on the pulse envelope if shaping is done at passband.

Transmit PSD mask

The transmitted spectrum shall be less than –10 dBr (dB relative to the maximum spectral density of the signal) for  and –18 dBr for . For example, the transmit spectrum mask for channel 4 is shown in Figure 13. The measurements shall be made using 1 MHz resolution bandwidth and a 1 kHz video bandwidth.

Figure 13 – transmit spectrum mask for band 4

Chip rate clock and chip carrier alignment

A UWB transmitter shall be capable of chipping at the peak PRF given in Table 3 with an accuracy of ± 20 parts per million. In addition, for each UWB PHY channel, the center of transmitted energy shall be at the values listed in Table 11 also with an accuracy of ± 20 parts per million. The measurements shall be made using 1 MHz resolution bandwidth and a 1 kHz video bandwidth.

Timestamps and time units

The UWB PHY supports precision ranging and localization through the capability of time stamping, using a ranging counter, the precise instant that RMARKERS are received (and transmitted) at the device antenna.

Time units

The time units used for ranging timestamps is defined by the least significant bit (LSB) of the time values which represents 1/128 of a chip time at the mandatory chipping rate of 499.2 MHz.

Note: The LSB of the ranging counter represents a time interval so small that an actual physical counter would have to run at a nominal 64 GHz to produce values with this resolution. An actual physical realization is not expected. Instead it is assumed that computational techniques will be used to generate sufficient of the less significant bits to yield the desired operational precision.

Antenna delays

The time of arrival and time of sending events relate to the RMARKER being at the antenna.

The receive timestamp will naturally occur in the digital domain of the receiver some period of time after the RMARKER arrives at the antenna.  To calculate when the RMARKER was at the antenna, and generate an accurate time of arrival, all the system delays between the antenna and the internal digital receive timestamp need to be accounted for. This receive antenna delay then needs to be subtracted from the internal digital receive timestamp to give the time of arrival value.

Similarly in the transmitter all the system delays between the internal digital transmit timestamp and the antenna need to be accounted.  This transmit antenna delay then needs to be added to the internal digital transmit timestamp to give the time of sending value, when the transmit RMARKER is at the antenna.

The mechanisms for determining these antenna delays are beyond the scope of this standard

</Billy>

- Appendix A

<395r1>

Table A. List of Service types

| Value | Meaning |
| --- | --- |
| 0 | All Service Types |
| 1 | real-time streaming |
| 2 | display |
| 3 | talking (VoIP) |
| 4 | two-way gaming |
| 5 – 254 | Reserved |
| 255 | Vendor Specific |

</395r1>

<390r1>

Table: Payload of discovery signal

| Contents | Size(bits) | Description | Notes |
| --- | --- | --- | --- |
| Type | 3 | Type of discovery signal 0: device advertisement 1: service advertisement 2: service info request 3: service info response 4: peer search request 5: peer search response |  |
| ID | 48 | Identifier of PD Type=0 : (own)device ID (e.g. mac address) Type=1: (own) (app. Type ID + app. specific ID+ app. Specific user ID) Type=2: (target) device ID Type=3: (own) (app. Type ID + app. specific ID+ app. Specific user ID) Type=4:(target) (app. Type ID + app. specific ID+((opt.)app. Specific user ID)) Type=5:(own) (app. Type ID + app. specific ID+ app. Specific user ID) | In case type=4, (target) app. Specific user ID can be included. ID bits are provided from upper layer (or management block) based on information of application layer |
| SIV(Service Information Version) | 5 | Version of service information provided by each PD .value:0~31(modulo 32) .value can be changed due to addition/deletion of application(s) or change of user | In case Type=0, type=2, Provided from upper layer based on information of application layer |
| Request range | 1 | Request Range of service information - 1: Delta with (pervious) service info. Ver. - 0: Full with (received) service info. Ver. | In case Type=2 |
| SN | 5 | Sequence number | In case Type=1or 3 |
| End indicator | 1 | end indication (0: continue, 1:end) | In case Type=1or 3 |
| GI | 1 | Service info. for group communication (0: individual, 1:group) | In case Type=1,3, 4 or 5 |
| Reserved | 7 or 6 | Reserved bits |  |
| total | 61 |  |  |

</390r1>