IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | NICT PHY Proposal |
| Date Submitted | July 6th, 2013 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Re: | In response to Call for Proposals to TG8 |
| Abstract | PHY proposal |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Content

1. Overview	3

1.1 Scope	3

1.2 Purpose	3

2. Normative references	3

3. Discovery	3

Annex A (informative) Bibliography	4

- Common mode PHY Description

- —Schematic diagram of common mode PHY

- Physical Channels

Frequency bands of operation are licensed bands and unlicensed bands. The target unlicensed bands are sub-GHz, 2.4 GHz and 5.7 GHz bands.

  - Channelization of 5.7 GHz band

Such frequency band (5.725 GHz to 5.875 GHz) is divided into 14 channels of 10 MHz. The central frequencies are given by

fc = 5735MHz + 10n

where n=0,1,…,13.

  - Channelization of 2.4 GHz band

Such frequency band (2.4 GHz to 2.5 GHz) is divided into 9 channels of 10 MHz. The central frequencies are given by

fc = 2410MHz + 10n

where n=0,1,…,8.

  - Channelization of 920 MHz band (Japan)

The proposed channelization of sub-GHz band in Japan is given in Table 1.

- — Channelization of sub-GHz band for Japan

| Frequency band (MHz) | Central frequency (MHz) | n | No of channels | Max Tx power (mW) |
| --- | --- | --- | --- | --- |
| 915.9 – 928.1 | fc=917+n | 0,1,…,10 | 11 channels of 1 MHz | 1 |
| 920.5 – 928.1 | fc=922+n | 0,1,…,5 | 6 channels of 1 MHz | 20 |
| 920.5 – 923.5 | fc=921.5+n | 0,1 | 2 channels of 1 MHz | 250 |
| 928.1 – 929.7 | fc=928.7+n | 0,1 | 2 channels of 500 kHz | 1 |

- Reference Signals

Reference signals are employed for channel estimation in coherence detection and equalization.  As PAC devices are conceived to be mobile and transmission bandwidths are up to the order of 10 MHz. Hence, the radio channels are frequency selective and time variant.

  - Zadoff-Chu sequences

Preambles or beacons and reference signals (RSs) are formed with Zadoff-Chu (ZC) sequences of length N. ZC sequences belong to the family of Constant-Amplitude Zero-AutoCorrelation (CAZAC) sequences. Such ZC sequences have two important properties for multicarrier-like systems:

- ZC sequences and its N-point DFT have constant amplitude.

This limits the PAPR and simplifies implementation as only phases have to be generated and stored.

- ZC sequences have ideal cyclic autocorrelation, i.e., the correlation with a circularly shifted version of itself is a delta function.

rkk(τ)=n=0N−1ak(n)ak(n+τ)=δ(τ)

Thus, a [large] set of orthogonal sequences for preambles and reference signals is possible to generate.

ZC sequences are given by

ak=e−j2πrN+k22+qkN evene−j2πrN+k(k+1)2+qkN odd

where r is a relative prime to N, sequence index k = 0, 1, ...,N-1and q is any integer.

- Discovery

Peer aware communication networks are aimed to operate without central control. Hence, transmission for initial discovery or re-discovery among peers is asynchronous. Thus, a preamble sequence is required for time and frame synchronization at start up or when re-synchronization is required for discovery or communication.

We propose to use a discovery preamble (DP) based on a ZC sequence and a discovery resource block (DRB) from a modified DTF-S OFDM signal.

The DP and DRB formed the Discovery Signal (DS).

Moreover, we propose to use one channel (from the proposed channelization for sub-GHz, 2.4 GHz and 5.7 GHz bands) for only discovery of devices.

PAC devices can either transmit or receive the DS in this unique channel asynchronously.

Such unique channel for discovery is named Shared Discovery Channel (SDCH).

# (informative)
Bibliography