IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Comment and resolution to TGD |
| Date Submitted | July 2013 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Re: | In response to Technical Guidance Document contributions |
| Abstract | Comments to TGD |
| Purpose | Discussion |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1.1	Comments to clause 9.4.1	3

## Comments to clause 9.4.1

The interference term in the link budget analysis to obtain PER metric is incorrect. The PER curves are in function of Es/N0, which can be transformed to SNR. On the other hand, the common assumption of interference modeled as aggregate noise, i.e. SINR=PSPN+PI is not provided in the PER curves and such the interference term is incorrectly introduced in the link budget.

Recommended resolution:

Either delete the interference term or replace it as aggregate noise with the following procedure:

- The SNR measured after the receiving filter in linear scale is given by

PSPN=P'tG'tG'rPNL's(d)M'NF'                                   1

where ‘ denotes value in linear scale, for instance power in W.

- Add interference as aggregate noise

PSPN+PI=P'tG'tG'rPNL's(d)M'NF'+PI                 2

where PI=i=1NIP'ihiL's(di) is the total interference power after the receiving filter assuming the interference bandwidth Bi≤B. hi is the channel’s gain, but assume 1 as the channel’s gain effect is counted in the fade margin M.

The key point to observe is the addition/subtraction of powers needs to be done in the linear domain as log(a+b)≠log(a)+log(b) .

- Noise power plus interference power is model as PN’ (interference as aggregate noise) such that

PSPN'=P'tG'tG'rPNL's(d)M'NF'+PI        3

- Noise density is N0=PNB=kT0 and energy per bit is Eb=PSR, where R is bit rate such that

EbRN0B=P'tG'tG'rPNL's(d)M'NF'+PI         4

EbN0=BP'tG'tG'rPNL's(d)M'NF'R+PIR      5

At room temperature 10log10kT01 mW=−174 dBm.

- Multiplying and diving by 1/B and substituting PNB=kT0in the right hand term:

EbN0=P'tG'tG'rkT0L's(d)M'NF'R+PIRB          6

If PI=0(no interference), the equation is the usual link budget formula, only given in the linear domain.

Finally:  EsN0|dB=10log10bEbN0  , where b=log2(M)and M the cardinality of constellation of modulation. For instance M=2 (BPSK), M=4 (QPSK), M=16 (16QAM), etc.