IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Draft 0.01 |  |
| Date Submitted | [September 7, 2013] |  |
| Source | [Liang Li, Eggert Dietmar, Kenichi Mori, Masahiro Kuroda, Andy Bottomley, Shin Hara, Rick Powell] [] | Voice: [ 949-813-7909] Fax: [ ] E-mail: [liangli@vinnotech.com, Dietmar.Eggert@atmel.com mori.ken1@jp.panasonic.com, andy.bottomley@microsemi.com shinsukehara0122@gmail.com marsh@nict.go.jp Rick.Powell@microsemi.com] |
| Abstract | This document is a contribution to work done thus far by TG4n |  |
| Purpose | This document is contribution in preparing a draft for letter ballot. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

IEEE Standard for

Local and metropolitan area networks—

Part 15.4: Low-Rate Wireless Personal Area

Networks (WPANs)

Amendment X: Physical Layer Utilizing Dedicated Chinese Medical Bands (176-214, 407-425, 608-630 MHz)

NOTE—The editing instructions contained in this amendment define how to merge the material contained therein into

the existing base standard and its amendments to form the comprehensive standard.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace.

Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change

and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material).

Delete removes existing material. Insert adds new material without disturbing the existing material. Deletions and

insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used

to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one.

Editing instructions, change markings, and this NOTE will not be carried over into future editions because the changes

will be incorporated into the base standard.

Abstract: This amendment to IEEE Std 802.15.4-2011 provides two PHYs (OQPSK-DSSS and GFSK) and the Discrete Chirp Spread Spectrum (DCSS) PHYs that support Device monitoring, control and ranging applications on Chinese approved radio bands for medical signal transmission (176-214 MHz, 407-425 MHz, and 608 – 630 MHz). In addition, the amendment describes only those MAC modifications needed to support the implementation of the PHYs.

Keywords: low data rate, low power, LR-WPAN, PAN, personal area network, radio frequency, RF, wireless personal area network, WPAN, MBAN, CMB.

# 2. Normative references

Insert the following new reference alphabetically into Clause 2:

China, Ministry of Industry and Information Technology Doc: 423-2005 ---- Technical Requirements for   Micro-power (short- distance) Radio Devices in the 174 – 214, 407-425, 608-630 MHz Band.

# 3. Definitions, Acronyms, and Abbreviations

## 3.2 Acronyms and abbreviations

Insert the following acronyms alphabetically into 3.2:

DCSS		discrete chirp spread spectrum

DSSS		Direct Sequence Spread Spectrum

MBAN 		medical body area network

PCA		 priority channel access

QPSK		quadrature phase-shift keying

GFSK 		Gaussian filtered frequency-shift keying

CMB		China Medical Band

TB 		total backoff

TID 		transaction identifier

TRLE 		time-slot relaying based link extension

BT		bandwidth time duration product

# 4. General description

Insert the following new sub clause (4.1b) after 4.1a:

## 4.1b  Introduction to medical body area network (MBAN) services on Chinese approved bands

The Ministry of Industry and Information Technology of the People's Republic of China has approved the 174-216 MHz, 407-425 MHz and 608-630 MHz bands for medical information transmission.

CMB devices operating within these bands must conform to a set of rules specified in above rules issued by Ministry of Industry and Information Technology of the People's Republic of China, which restrict use of the band to only medical, non-voice use under direction of a healthcare practitioner, among other requirements.

In the amendment, CMB is applied to define the devices operating on Chinese approved bands for medical signals.

As a one user in this band, CMB devices are required to protect all users and accept possible interference from those users (such as Digital TV).

Use of the band by the Digital TV is, in general, scheduled well in advance allowing CMB users to share the band in an orderly manner.

#### 4.5.1.1 General

Insert the following item at the end of the list in 4.5.1.1:

— Support for priority channel access (PCA) in the contention access period (CAP) of the superframe structure, as described in (5.1.1.4)

### 4.5.4 Improving probability of successful delivery

Insert the following after 4.5.4.1:

In the CMB environment, such as hospitals, some critical medical messages have the higher priority than the networking maintain messages and others. Even among medical messages, the priorities of messages are different and may be defined by users.

These critical medical message or higher priority message  are equal to critical even message in IEEE802.15.4K. Now, the definition: critical even message, is adopted in this amendment.

The amendment also adopts the same MAC processing procedure and for critical even messages as ones in IEEE802.15.4K.

# 5. MAC protocol

## 5.1 MAC functional description

### 5.1.8 Ranging

Insert the following new subclauses (5.1.8b–5.1.8b.4) after 5.1.8.6:

### 5.1.8b DCSS ranging using phase difference measurement

DCSS ranging using phase difference measurement is an optional feature and is primarily for stationary objects.

#### 5.1.8b.1 Initialization of phase difference measurement

The process of a phase difference measurement is initiated at the ranging originator upon receipt of an MLME-RANGING.request primitive with the requested ranging mode and the requested number of ranging measurements.

If the initiation of a ranging measurement is currently supported (based on macPMRangingEnabled), the MAC sublayer generates a Ranging command frame of subtype Range request to the ranging recipient (see 5.3.9a.2) based on the settings of the required ranging attributes. If, upon successful transmission of the frame, the device does not receive a Ranging command frame of subtype Range response (see 5.3.9a.3) within macPMRangeResponseWaitTime, the device shall initiate an MLME-RANGING.confirm primitive indicating a timeout condition.

Upon reception of the Ranging command frame of subtype Range request at the ranging recipient, the recipient checks whether it is currently able to perform a ranging measurement. If not, it shall reject the ranging measurement by returning a Ranging command frame of subtype Range response including the Range acceptance status indicating that ranging is currently not supported.

If ranging is supported, the recipient verifies the requested ranging protocol version (see 5.3.9a.2.2 and 5.3.9a.1), ranging mode (see 5.3.9a.2.2), and ranging parameters (see 5.3.9a.2.2). If any of these parameters are not supported by the recipient, it shall reject the ranging measurement by returning a Ranging command frame of subtype Range response including the Range acceptance status (see 5.3.9a.3.2) indicating an unsupported ranging protocol, unsupported ranging mode, undefined entry in phyPMDeviceTable (see Table 71a), or invalid ranging parameters.

In case the Range acceptance status indicates that only the dynamic ranging parameters are not accepted, and proposed supported dynamic ranging parameters are appended, the originator may store these received parameters and repeat the ranging procedure using the proposed dynamic ranging parameters.

At this point the recipient shall verify the requested number of ranging measurements (see 5.3.9a.2.2) and the Requested Ranging Configuration field (see 5.3.9a.2.2). If any of these requested fields are not supported by the recipient, it shall update these parameters in the Accepted Number of Ranging Measurements field and/or in the Accepted Ranging Configuration field of the returned Ranging command frame of subtype Range response (see 5.3.9a.3.2).

If, upon successful transmission of the Range response frame, the recipient does not receive a Ranging command frame of subtype Range sync request (see 5.3.9a.5) or Range abort (see 5.3.9a.4) within macPMRangeResponseWaitTime, the device shall abort the current ranging measurement.

Upon reception of the Ranging command frame of subtype Range response at the ranging originator, the originator shall check the received Range acceptance status. If the status indicates, that ranging is rejected, the originator shall abort the current ranging measurement and issue an MLME-RANGING.confirm primitive indicating the ranging rejections reason.

If the status indicates, that ranging is accepted but the recipient has returned a parameter that is not acceptable by the originator, the originator shall transmit a Ranging command frame of subtype Range abort and initiate an MLME-RANGING.confirm primitive indicating ranging abort.

If the status indicates, that ranging is accepted, and the returned parameter from the recipient within this frame are fully acceptable by the originator, the originator shall transmit a Ranging command frame of subtype Range sync request in order to gain synchronization with the recipient for the phase difference measurement.

If, upon successful transmission of the subtype Range sync request frame, the device does not receive a Ranging command frame of subtype Range start (see 5.3.9a.6) within macPMSyncWaitTime, the device shall abort the current ranging measurement and initiate an MLME-RANGING.confirm primitive indicating an unsuccessful synchronization.

If, after the transmission of a Range response frame, the recipient receives a Ranging command frame of subtype Range abort, the recipient shall abort the current ranging measurement.

If the recipient receives a Ranging command frame of subtype Range sync request, it shall check the received ranging protocol version and remaining ranging measurement counter (see 5.3.9a.5.2). If the received protocol version is not supported, or the received remaining measurement counter value is larger than the counter in the previously received Range sync request frame, the recipient shall return a Ranging command frame of subtype Range abort to the originator and abort the current ranging procedure.

If the received parameters within the Range sync request frame are acceptable by the recipient, it shall initiate the transmission of a Ranging command frame of subtype Range start to gain synchronization with the ranging originator (see 21.3.1) and start the actual phase difference measurement (see 21.3.2).

If the ranging originator receives a Range abort frame instead of a Range start frame, the originator shall abort the current ranging measurement and initiate an MLME-RANGING.confirm primitive indicating the ranging abort condition.

If the originator receives the expected Ranging command frame of subtype Range start, it synchronizes with the recipient (see 21.3.1) and starts the actual phase difference measurement (see 21.3.2).  After finishing the measurement, the procedures continues as described in 5.1.8b.2.

Figure 34a, Figure 34b, Figure 34c, and Figure 34d depict the initialization of a phase difference measurement.

Figure 34a— Successful initialization of a phase difference measurement procedure

Note on Figure 34a: The final MLME-RANGING.confirm primitive is only issued after finalizing the phase measurement and the subsequent data exchange (see Figure 34e).

Figure 34b—Timeout during initialization of a phase difference measurement procedure

Figure 34c—Error handling during Range response handling

Figure 34d—Error handling during ranging synchronization procedure

#### 5.1.8b.2 Data exchange after a phase difference measurement

Ranging may be performed using either compressed or uncompressed phase measurement result data as negotiated in the initial Range Request/Range response handshake. A compressed phase measurement value represents the average of phyPMCurrNumbersOfPhaseMeasPerFreq individual phase measurement values per measurement frequency, see Table 71a.

In case compression is not used, the recipient shall immediately enable its receiver for macPMRangeExtWaitTime, in order to allow for the reception of the first Range result request frame from the originator. If the recipient does not receive a Ranging command frame of subtype Range result request within macPMRangeExtWaitTime, the device shall abort the current ranging measurement.

When using compression, both devices shall perform the compression. In this case the recipient also shall immediately enable its receiver (at the beginning of the result compression), since the originator may finish its compression faster than the recipient, but the recipient shall already be able to receive the first Range result request frame, even if still busy with compression.

If compression is finished at the recipient, it shall leave its receiver enabled for an additional macPMRangeExtWaitTime, in order to allow for the reception of the first Range result request frame from the originator. If the recipient does not receive a Ranging command frame of subtype Range result requestwithin macPMRangeExtWaitTime, the device shall abort the current ranging measurement.

Based on the negotiated parameters from Table V.1 and Table V.2 both the originator and the reflector calculate the exact amount of phase measurement data to the exchange. For a more detailed description of the actual result exchange and an example see Annex V.

Immediately finishing the phase measurement (when not using compression), or after finishing the compression, the originator shall generate the initial Ranging command frame of subtype Range result request to the ranging recipient, in order to request the phase measurement result data from the recipient. If the originator does not receive a Ranging command frame of subtype Range result confirm within macPMRangeResponseWaitTime, the device shall initiate an MLME-RANGING.confirm primitive with the status of TIMEOUT.

In case the recipient has not finished its result compression at reception of the initial Range result request frame, it shall reply using a Range result confirm frame indicating a busy condition by setting the Result Data Type field to 0x00 (see 5.3.9a.8.2). After having finished the result compression, the recipient shall initiate the transmission using a second Range result confirm frame (including the values of Current Outer Loop Counter, Current Inner Loop Counter, and Current Result Data Start Address from the first Range result confirm frame) but with the Result Data Type field set to the proper result data type.

Upon reception of the Ranging command frame of subtype Range result request at the ranging recipient (i.e., the recipient has finished the result compression), the recipient shall check the requested phase measurement result data. If the Range result request frame contains invalid information (e.g., an invalid value in the requested outer loop counter that exceeds the actual measurement, see 5.3.9a.7.2), the recipient shall return a Ranging command frame of subtype Range abort to the originator and abort the current ranging procedure.

If the received Range result request frame is correct, the recipient shall return the requested phase measurement result data to the originator by transmitting a Range result confirm frame. If the recipient has more phase measurement result data to be transmitted to the originator, it shall await the next Ranging command frame of subtype Range result request within macPMRangeResponseWaitTime. If the recipient does not receive the expected frame within this time, the device shall abort the current ranging measurement.

If the originator expects a Range result confirm frame but instead receives a Range abort frame, the originator shall abort the current ranging measurement and issue an MLME-RANGING.confirm primitive indicating the ranging abort condition.

Otherwise, if the originator receives the expected Ranging command frame of subtype Range result  confirm, it shall extract the delivered phase measurement result data. If the Result Data Type field of the received Range result confirm frame is set to 0x00 (Busy condition - see Table 5k), the originator shall wait for macPMRangeExtWaitTime to again receiveto a second Range result confirm frame with the requested measurement result data. If a second Range result confirm frame is not received from the recipient within macPMRangeExtWaitTime, the originator shall issue an MLME-RANGING.confirm primitive indicating a timeout condition.

If the received phase measurement result data are correct, and if the originator has not yet requested all phase measurement results, it shall repeat the described sequence using the Range result request / Range result confirm handshake, until the completion of the result data or an abort condition has occurred by utilizing the macPMRangeResponseWaitTime for each subsequent frame exchange.

After successful completion of the data exchange, i.e., the originator has received all required phase measurement result data, the recipient has completed the ranging procedure. The originator shall calculate the final ranging results based on its own measured data and the received data from the recipient. After finishing the result calculation, the originator shall inform the next higher layer of the ranging originator by issuing an MLME-RANGING.confirm primitive indicating a status of SUCCESS including the final ranging results.

Figure 34e, Figure 34f, Figure 34g, and Figure 34h depict the data exchange after a phase difference measurement procedure.

Figure 34e—Data exchange

Figure 34f—Error handling due to ranging data compression delay

Figure 34g—Timeout during data exchange

Figure 34h—Error handling during of invalid Range result request

#### 5.1.8b.3 Retrieval of phase difference measurement ranging capabilities

In order to allow for any ranging device to retrieve the available ranging capabilities from any other ranging device before initiating a ranging measurement with appropriate ranging parameters, the next higher layer of a device may initiate an MLME-RANGING-CAPABILITIES.request primitive. This results in the transmission of a Ranging command frame of subtype Ranging capabilities request to the corresponding ranging device.

If, upon successful transmission of the frame, the device does not receive a Ranging command frame of subtype Ranging capabilities response within macPMRangeResponseWaitTime, the device shall initiate an MLME-RANGING-CAPABILITIES.confirm primitive indicating a timeout condition.

Upon reception of the Ranging command frame of subtype Ranging capabilities request, the recipient shall generate a Ranging command frame of subtype Ranging capabilities response including its own ranging capabilities based on the values in phyPMCapTable.

The originator informs the next higher layer about the results of the ranging capabilities retrieval by issuing an MLME-RANGING-CAPABILITIES.confirm primitive. If it has received the expected Ranging command frame of subtype Ranging capabilities response; the status is SUCCESS.

Figure 34i depicts the ranging capabilities retrieval.

Figure 34i—Successful ranging capabilities retrieval

## 5.3 MAC command frames

Insert the following new row at the end of Table 5:

Table 5—MAC command frames

| Command frame identifier | Command name | RFD |  | Subclause |
| --- | --- | --- | --- | --- |
|  |  | Tx | Rx |  |
| 0x21 (next non-reserved identifier) | Ranging command | X | X | 5.3.9a |

Insert the following new subclauses (5.3.9a–) after 5.3.9.2:

5.3.9a Ranging command

The Ranging command is used by a device that is performing any kind of ranging measurement activity. The Ranging command shall be formatted as illustrated in Figure 59a.

The list of Ranging Command Subtype Identifier field values are given in Table 5a.

| Octets: variable | 1 | 1 | variable |
| --- | --- | --- | --- |
| MHR fields | Command frame identifier (defined in Table 5) | Ranging Command Subtype Identifier (defined in Table 5a) | Command Payload (as defined for each Ranging command subtype) |

Figure 59a—Formatting of Ranging command frame

Table 5a—Ranging Command Subtype Identifier field values

| Ranging command subtype identifier | Subtype name | Direction |  | Subclause |
| --- | --- | --- | --- | --- |
|  |  | Ranging originator | Ranging recipient |  |
| 0x00 | Range request | Tx | Rx | 5.3.9a.2 |
| 0x01 | Range response | Rx | Tx | 5.3.9a.3 |
| 0x02 | Range abort | Tx/Rx | Tx/Rx | 5.3.9a.4 |
| 0x03 | Range sync request | Tx | Rx | 5.3.9a.5 |
| 0x04 | Range start | Rx | Tx | 5.3.9a.6 |
| 0x05 | Range result request | Tx | Rx | 5.3.9a.7 |
| 0x06 | Range result confirm | Rx | Tx | 5.3.9a.8 |
| 0x07 | Ranging capabilities request | Tx/Rx | Tx/Rx | 5.3.9a.9 |
| 0x08 | Ranging capabilities response | Tx/Rx | Tx/Rx | 5.3.9a.10 |

5.3.9a.1 Handling of Ranging Protocol Version field

Ranging command frames of subtypes Range request and Range sync request contain a field indicating the ranging protocol version specifying the applied ranging protocol version. This field shall be set to 0x00. All other field values are reserved.

If a device receives either of these Ranging command frame types with an incompatible ranging protocol version, it shall reject the frame. In the case of a received Range request frame this is done by initiating a Range respond frame including the range acceptance status indicating an unsupported ranging protocol version. In the case of a received Range sync request frame, this is done by initiating a Range abort frame.

5.3.9a.2 Range request command

The Ranging command frame of subtype Range request is used to initiate a ranging measurement and shall be sent by the ranging originator to the ranging recipient.

5.3.9a.2.1 MHR fields

The Source Addressing Mode, the Source Address, and the Source PAN Identifier fields shall be set according to the OrigAddrMode, OrigPANId, and OrigAddr parameters specified by the MLME-RANGING.request primitive.

The Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier fields shall be set according to the RecAddrMode, RecPANId, and RecAddr parameters specified by the MLME-RANGING.request primitive.

The PAN ID Compression field shall be handled according to 5.2.1.1.5.

The Frame Pending field shall be set to zero and ignored upon reception, and the AR field shall be set to one.

5.3.9a.2.2 Command Payload fields

The Command Payload fields of the Ranging command frame of subtype Range request shall be formatted as illustrated in Figure 59b.

| Octets: 1 | 1 | 1 | 1/11 | 1/7 | 1 | 1/2 |
| --- | --- | --- | --- | --- | --- | --- |
| Ranging Protocol Version | Ranging Mode | Requested Number of Ranging Measurements | Static Ranging Parameter | Dynamic Ranging Parameter | Requested Ranging Configuration | Requested Ranging Measurement Transmit Power |

Figure 59b—Command Payload fields of Ranging command frame of subtype Range

request

The Ranging Protocol Version field is defined in 5.3.9a.1.

The Ranging Mode field is defined according to the RangingMode parameter specified by the MLME-RANGING.request primitive in 6.2.25.1.

The Requested Number of Ranging Measurements field contains the requested number of ranging measurements during the current ranging measurement between the originator and the recipient. The value shall be set according to the NumberOfMeasurements parameter specified by the MLME-RANGING.request primitive.

The values of the Requested Number of Ranging Measurements field are defined in Table 5b.

Table 5b—Values of the Requested Number of Ranging Measurement field

| Requested number of ranging measurements | Description |
| --- | --- |
| 0x00 | Invalid (shall be considered as 0x01 on reception) |
| 0x01 | Single-shot ranging measurement |
| 0x02-0xff | Multi-shot ranging measurement |

The Static Ranging Parameter field shall be formatted as illustrated in Figure 59c.

Valid values of the Static Ranging Parameter Handling Mode field are defined in Table 5c.

| Octets: 1 | 0/10 |
| --- | --- |
| Static Ranging Parameter Handling Mode | Requested static ranging parameters |

Figure 59c—Static Ranging Parameter field

Table 5c—Valid values of the Static Ranging Parameter Handling Mode field

| Static Ranging Parameter Handling Mode | Description | Comment |
| --- | --- | --- |
| 0x00 | Request usage of default values for static ranging parameters as included within this frame | The originator has included its phyPMDefaultTable-based parameters in this frame, which shall be extracted by the recipient and used for this ranging measurement. |
| 0x01 | Request usage of network-based values for static ranging parameters | The originator implicitly requests the usage of the phyPMNetworkTable-based parameters but does not include these parameters in this frame explicitly. For error handling in case the requested parameters are not available at the recipient see 5.3.9a.3.2 Table 5f and Table 5h. |
| 0x02 | Request usage of device-based values for static ranging parameters | The originator implicitly requests the usage of the phyPMDeviceTable-based parameters but does not include these parameters in this frame explicitly. For error handling in case the requested parameters are not available at the recipient see 5.3.9a.3.2 Table 5f and Table 5h. |

The actual value of the Static Ranging Parameter Handling Mode field is a direct mapping of the current value of macPMAttributeUsageMode.

The requested static ranging parameters are only present if the Static Ranging Parameter Handling Mode field indicates that the requested static ranging parameters are included in this frame (see Table 5c), and the parameters shall be formatted as illustrated in Figure 59d. The actual values are taken from the phyPMDefaultTable attributes.

| Octets: 1 | 1 | 1 | 1 | 1 | … |
| --- | --- | --- | --- | --- | --- |
| phyPM- SetupDuration (in us) | phyPM-ShiftFreq (in 100kHz) | phyPM-FreqSettleDuration (in us) | phyPM-TxSetupDurationRec (in us) | phyPM-SamplingDurationOrig (in us) | … |

| … | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- |
| … | phyPM-TxSetupDurationOrig (in us) | phyPM-SamplingDurationRec (in us) | phyPM-InnerLoop-Repetitions | phyPM-OuterLoop-Repetitions | phyPM-CurrNumbersOfPhaseMeas- PerFreq |

Figure 59d—Requested static ranging parameters

The Dynamic Ranging Parameter field shall be formatted as illustrated in Figure 59e.

| Octets: 1 | 0/6 |
| --- | --- |
| Dynamic Ranging Parameter Handling Mode | Requested Dynamic Ranging Parameters |

Figure 59e—Dynamic Ranging Parameter field

Valid values of the Dynamic Ranging Parameter Handling Mode field are defined in Table 5d.

Table 5d—Valid values of the Dynamic Ranging Parameter Handling Mode field

| Dynamic Ranging Parameter Handling Mode field value | Description | Comment |
| --- | --- | --- |
| 0x00 | Request usage of dynamic ranging parameters as included within this frame | The originator has included its phyPMDefaultTable-based parameters in this frame, which shall be extracted by the recipient and used for this ranging measurement. |
| 0x01 | Request usage of network-based values for dynamic ranging parameters | The originator implicitly requests the usage of the phyPMNetworkTable-based parameters but does not include these parameters in this frame explicitly. |
| 0x02 | Request usage of device-based values for dynamic ranging parameters | The originator implicitly requests the usage of the phyPMDeviceTable-based parameters but does not include these parameters in this frame explicitly. |

The actual value of the Dynamic Ranging Parameter Handling Mode field is a direct mapping of the current

value of macPMAttributeUsageMode.

The requested dynamic ranging parameters are only present if the Dynamic Ranging Parameter Handling

Mode field indicates that the requested dynamic ranging parameters are included in this frame (see

Table 5d), and the parameters shall be formatted as illustrated in Figure 59f.

| Octets: 1 | 2 | 2 | 1 |
| --- | --- | --- | --- |
| phyPMMeasMode (0x00) | phyPMStartFreq (in 100 kHz) | phyPMStopFreq (in 100kHz) | phyPMStepFreq (in 100kHz) |

Figure 59f—Requested dynamic ranging parameters

The Requested Ranging Configuration field shall be formatted as illustrated in Figure 59g.

| Bits: 0 | 1-7 |
| --- | --- |
| Uncompressed Phase Measurement Results | Reserved |

Figure 59g—Requested Ranging Configuration field

The Uncompressed Phase Measurement Results field requests the provisioning of uncompressed phase measurement results by the ranging recipient to the ranging originator. If this value is set to zero, the phase measurement results are provided compressed, resulting in minimum frame exchanges during the data exchange phase of the ranging measurement. If this value is set to one, the recipient forwards the complete uncompressed measurement values to the recipient, resulting in extended frame exchanges during the data exchange phase of the ranging measurement.  If the atttribute macPMSuppressResultCompression is TRUE at the originator, this field is set ot one, otherwise to zero.

The Reserved field shall be set to zero and ignored upon reception.

The Requested Ranging Measurement Transmit Power field shall be formatted as illustrated in Figure 59h.

| Octets: 1 | 0/1 |
| --- | --- |
| Requested Ranging Measurement Transmit Power Handling Mode | Requested Ranging Measurement Transmit Power |

Figure 59h—Requested Ranging Measurement Transmit Power field

Valid values of the Requested Ranging Measurement Transmit Power Handling Mode field are defined in

Table 5e.

Table 5e—Valid values of the Requested Ranging Measurement Transmit Power Handling

Mode field

| Requested Ranging Measurement Transmit Power Handling Mode field value | Description |
| --- | --- |
| 0x00 | Requested ranging measurement transmit power not included in frame |
| 0x01 | Requested ranging measurement transmit power included in frame |

The actual value of the Requested Ranging Measurement Transmit Power Handling Mode field shall be derived from the current value of the PIB attribute macPMProvideRangingTxPower.

The Requested Ranging Measurement Transmit Power field is only available if the Requested Ranging Measurement Transmit Power Handling Mode field indicates that the requested ranging measurement transmit power is included in this frame, and the parameter shall set as defined in phyPMRangingTxPower.

5.3.9a.3 Range response command

The Ranging command frame of subtype Range response is used to either accept or reject a previously requested ranging measurement and shall be sent by the ranging recipient to the ranging originator.

5.3.9a.3.1 MHR fields

The Source Addressing Mode, the Source Address, and the Source PAN Identifier fields shall be set according to the parameters Destination Addressing Mode, Destination Address, and Destination PAN Identifier within the previously received Range request frame.

The Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier fields shall be set according to the Source Addressing Mode, Source Address, and Source PAN Identifier parameters within the previously received Range request frame.

The PAN ID Compression field shall be handled according to 5.2.1.1.5.

The Frame Pending field shall be set to zero and ignored upon reception, and the AR field shall be set to one.

5.3.9a.3.2 Command Payload fields

The Command Payload fields of the Ranging command frame of subtype Range response shall be formatted as illustrated in Figure 59i.

| Octets: 1 | 1 | 1 | 1 | 1/7 | 1 |
| --- | --- | --- | --- | --- | --- |
| Range Acceptance Status | Ranging Mode | Accepted Number of Ranging Measurements | Static Ranging Parameter Acceptance Status | Dynamic Ranging Parameter Acceptance | Accepted Ranging Configuration |

Figure 59i—Command Payload fields of Ranging command frame of subtype Range

response

Valid values of the Range Acceptance Status field are defined in Table 5f.

Table 5f—Valid values of the Range Acceptance Status field

| Range Acceptance Status field value | Description |
| --- | --- |
| 0x00 | Ranging request accepted |
| 0x01 | Ranging request rejected since ranging is currently not supported (macPMRangingEnabled is set to FALSE) |
| 0x02 | Ranging rejected since ranging is already ongoing |
| 0x03 | Ranging rejected due to unsupported ranging protocol version |
| 0x04 | Ranging rejected due to unsupported ranging mode |
| 0x05 | Ranging request rejected due to requested usage of device based ranging parameters but device not in phyPMDeviceTable |
| 0x06 | Ranging request rejected due to unsupported dynamic ranging parameters; proposed changes are appended |
| 0x07 | Ranging request rejected due to invalid ranging parameters |

If the contents of the retrieved Ranging Mode field in the previously received Range request frame is accepted by the ranging recipient, it shall return the same value for the ranging mode in the Ranging Mode field of the Range response command.

Otherwise, the ranging recipient shall reject the ranging request by setting the Range Acceptance Status  field to indicate an unsupported ranging mode, and the ranging recipient shall update the Ranging Mode field with a proposed ranging mode.

If the requested number of ranging measurements within the previously received Range request frame is accepted by the ranging recipient, it shall return the same value in the Accepted Number of Ranging Measurements field.

Otherwise, the ranging recipient shall update the Accepted Number of Ranging Measurements field with a supported value less than originally requested; the ranging request shall not be rejected solely due to this condition.

Valid values of the Accepted Number of Ranging Measurements field are defined in Table 5g.

Table 5g—Valid values of the Accepted Number of Ranging Measurements field

| Accepted Number of Ranging Measurements field value | Description |
| --- | --- |
| 0x00 | Invalid (shall be considered as 0x01 on reception) |
| 0x01 | Single-shot ranging measurement |
| 0x02-0xff | Multi-shot ranging measurement |

The Static Ranging Parameter Acceptance Status field shall contain one of the following valid values as

defined in Table 5h.

Table 5h—Valid values of the Static Ranging Parameter Acceptance Status field

| Static Ranging Parameter Acceptance field value | Description |
| --- | --- |
| 0x00 | Requested static ranging parameters are accepted, no changes required |
| 0x01 | Requested static ranging parameters are not accepted |
| 0x02 | No static ranging parameters stored |

If the recipient has accepted the requested static ranging parameters and no changes are required, the recipient shall set the Static Ranging Parameter Acceptance Status field to 0x00. This is independent of whether the requested static ranging parameters were included in the previously received Range request frame.

If the recipient cannot accept the requested static ranging parameters, the recipient sets the Static Ranging Parameter Acceptance Status field to 0x01. Furthermore, the ranging recipient shall reject the ranging request by setting the range acceptance status indicating ranging rejected due to invalid ranging parameters.

If the recipient requested to use either the phyPMDeviceTable or the phyPMNetworkTable-based static ranging parameters but this cannot be fulfilled (due to the capabilities based on phyPMParameterStorage and/or an undefined entry in phyPMDeviceTable), the recipient sets the Static Ranging Parameter Acceptance Status field to 0x02. Furthermore, the ranging recipient shall reject the ranging request by setting the range acceptance status indicating ranging rejected due to invalid ranging parameters (in case of requested phyPMNetworkTable) or to invalid device (in case of requested phyPMDeviceTable).

The Dynamic Ranging Parameter Acceptance field shall be formatted as illustrated in Figure 59j.

| Octets: 1 | 0/6 |
| --- | --- |
| Dynamic Ranging Parameter Acceptance Status | Proposed Dynamic Ranging Parameters |

Figure 59j—Dynamic Ranging Parameter Acceptance field

The Dynamic Ranging Parameter Acceptance Status field shall contain one of the following valid values defined in Table 5i.

Table 5i—Valid values of the Dynamic Ranging Parameter Acceptance Status field

| Dynamic Ranging Parameter Acceptance field value | Description |
| --- | --- |
| 0x00 | Requested dynamic ranging parameters are accepted, no changes required |
| 0x01 | Requested dynamic ranging parameters are not accepted; proposed changes are appended |
| 0x02 | No dynamic ranging parameters stored |

If the recipient has accepted the requested static ranging parameters and no changes are required, the recipient sets the Dynamic Ranging Parameter Acceptance Status field to 0x00. This is independent of whether the requested dynamic ranging parameters were included in the previously received Range request frame.

If the recipient cannot accept the requested dynamic ranging parameters, the recipient sets the Dynamic Ranging Parameter Acceptance Status field to 0x01, and the recipient shall fill in the Proposed Dynamic Ranging Parameter field to allow for the originator to extract the supported dynamic ranging parameters  and initiate a new ranging procedure with supported dynamic ranging parameters. Furthermore, the ranging recipient shall reject the ranging request by setting the range acceptance status indicating ranging rejected due to invalid ranging parameters.

If the recipient requested to use either the phyPMDeviceTable or the phyPMNetworkTable-based dynamic ranging parameters but this cannot be fulfilled (due to the current capabilities based on phyPMParameterStorage and/or an undefined entry in phyPMDeviceTable), the recipient sets the Dynamic Ranging Parameter Acceptance Status field to 0x02. Furthermore, the ranging recipient shall reject the ranging request by setting the range acceptance status indicating ranging rejected due to invalid ranging parameters (in case of requested phyPMNetworkTable) or to invalid device (in case of requested phyPMDeviceTable).

The proposed dynamic ranging parameters are only present, if the dynamic ranging parameter acceptance status is set to 0x01 (see Table 5i), and shall be formatted as illustrated in Figure 59k.

| Octets: 1 | 2 | 2 | 1 |
| --- | --- | --- | --- |
| phyPMMeasMode (0x00) | phyPMStartFreq (in 100kHz) | phyPMStopFreq (in 100kHz) | phyPMStepFreq (in 100kHz) |

Figure 59k—Proposed dynamic ranging parameters

The Accepted Ranging Configuration field shall be formatted as illustrated in Figure 59l.

| Bits: 0 | 1-7 |
| --- | --- |
| Uncompressed Phase Measurement Results | Reserved |

Figure 59l—Accepted Ranging Configuration field

The Uncompressed Phase Measurement Results field defines whether the phase measurement results by the ranging recipient to the ranging originator shall be uncompressed or compressed. If this value is set to zero, the phase measurement results are provided compressed, resulting in minimum frame exchanges during the data exchange phase of the ranging measurement. If this value is set to one, the recipient forwards the complete uncompressed measurement values to the recipient, resulting in extended frame exchanges during the data exchange phase of the ranging measurement.

This value shall only be set to one if provisioning of uncompressed data was requested from the originator and uncompressed phase measurement data provisioning is supported by the recipient. In all other cases, this field shall be set to zero.

The Reserved field shall be set to zero and ignored upon reception.

5.3.9a.4 Range abort command

The Ranging command frame of subtype Range abort shall be used by the originator if the recipient has accepted the Range request but the originator detects a condition where it cannot continue with the current ranging procedure (e.g., due to invalid returned ranging parameters from the recipient). This command  shall be sent by the ranging originator to the ranging recipient.

The frame shall also be used by the recipient if the device has received a Range sync request frame containing invalid parameters or a Range result request frame containing invalid parameters. In this case, the command shall be sent by the ranging recipient to the ranging originator.

5.3.9a.4.1 MHR fields

If the frame is sent by the originator to the recipient, the Source Addressing Mode, the Source Address, and the Source PAN Identifier fields shall be set according to the OrigAddrMode, OrigPANId, and OrigAddr parameters specified by the MLME-RANGING.request primitive. The Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier fields shall be set according to the RecAddrMode, RecPANId, and RecAddr parameters specified by the MLME-RANGING.request primitive.

If the frame is sent by the recipient to the originator, the Source Addressing Mode, the Source Address, and the Source PAN Identifier fields shall be set according to the parameters Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier within the previously received Ranging command frame. The Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier fields shall be set according to the parameters Source Addressing Mode, the Source Address, and the Source PAN Identifier within the previously received Ranging command frame.

The PAN ID Compression field shall be handled according to 5.2.1.1.5.

The Frame Pending field shall be set to zero and ignored upon reception, and the AR field shall be set to one.

5.3.9a.4.2 Command Payload fields

The Range abort frame does not contain any payload fields.

5.3.9a.5 Range sync request command

The Ranging command frame of subtype Range sync request is used:

— after the successful Range request/Range response handshake to start a single-shot ranging measurement, or

— when the Accepted Number of Ranging Measurements field within the Range response frame was larger than one (indicating a multi-shot ranging measurement) after finishing the previous data exchange phase to start the next ranging measurement of the same originator-recipient device pair (without another Range request / Range response handshake) using the same ranging parameters (but with a decremented value of the Remaining Ranging Measurements Counter field; see 5.3.9a.5.2).

The frame shall be sent by the ranging originator to the ranging recipient.

5.3.9a.5.1 MHR fields

The Source Addressing Mode, the Source Address, and the Source PAN Identifier fields shall be set according to the OrigAddrMode, OrigPANId, and OrigAddr parameters specified by the MLME-RANGING.request primitive.

The Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier fields shall be set according to the RecAddrMode, RecPANId, and RecAddr parameters specified by the MLME-RANGING.request primitive.

The PAN ID Compression field shall be handled according to 5.2.1.1.5.

The Frame Pending field shall be set to zero and ignored upon reception, and the AR field shall be set to one.

5.3.9a.5.2 Command Payload fields

The Command Payload fields of the Ranging command frame of subtype Range sync request shall be

formatted as illustrated in Figure 59m.

| Octets: 1 | 1 |
| --- | --- |
| Ranging Protocol Version (as defined in 5.3.9a.1) | Remaining Ranging Measurements Counter |

Figure 59m—Command Payload fields of Ranging command frame of subtype Range sync

request

The Remaining Ranging Measurements Counter field contains the remaining number of ranging measurements to be performed after the current ranging measurement between the originator and recipient node. A value of zero indicates a single-shot ranging measurement or the initiation of the last multi-shot ranging measurement procedure.

5.3.9a.6 Range start command

The Ranging command frame of subtype Range start is used to gain synchronization between the ranging originator and recipient for the current ranging procedure and start the actual phase difference measurement. The command shall be sent by the ranging recipient to the ranging originator.

5.3.9a.6.1 MHR fields

The Source Addressing Mode, the Source Address, and the Source PAN Identifier fields shall be set according to the parameters Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier within the previously received Range sync request frame.

The Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier fields shall be set according to the parameters Source Addressing Mode, the Source Address, and the Source PAN Identifier within the previously received Range sync request frame.

The PAN ID Compression field shall be handled according to 5.2.1.1.5.

The Frame Pending field shall be set to zero and ignored upon reception, and the AR field shall be set to zero.

5.3.9a.6.2 Command payload fields

The Range start frame does not contain any payload fields.

5.3.9a.7 Range result request command

The Ranging command frame of subtype Range result request is used after finishing a single phase difference measurement (even when operating a multi-shot ranging) to initiate the retrieval of phase measurement data. The command shall be sent by the ranging originator to the ranging recipient.

5.3.9a.7.1 MHR fields

The Source Addressing Mode, the Source Address, and the Source PAN Identifier fields shall be set according to the OrigAddrMode, OrigPANId, and OrigAddr parameters specified by the  MLME-RANGING.request primitive.

The Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier fields shall be set according to the RecAddrMode, RecPANId, and RecAddr parameters specified by the MLME-RANGING.request primitive.

The PAN ID Compression field shall be handled according to 5.2.1.1.5.

The Frame Pending field shall be set to zero and ignored upon reception, and the AR field shall be set to one.

5.3.9a.7.2 Command Payload fields

The Command Payload fields of the Ranging command frame of subtype Range result request shall be

formatted as illustrated in Figure 59n.

| Octets: 1 | 1 | 1 | 2 |
| --- | --- | --- | --- |
| Result Data Type | Requested Outer Loop Counter | Requested Inner Loop Counter | Requested Result Data Start Address |

Figure 59n—Command Payload fields of Ranging command frame of subtype Range result request

The Result Data Type field contains the requested type of result data.  This value shall be set to one indicating phase measurement data.

The Requested Outer Loop Counter and Requested Inner Loop Counter fields contain the current outer and inner loop counter values for which the result data are requested. Both loop counts start with zero and are updated after successful data retrieval for each range result data exchange.

The Requested Inner Loop Counter value starts at zero and is incremented for each new Range result request frame (after successful reception of the previous Range result confirm frame) until is has reached its maximum. After this maximu the counter is reset to zero within in the next Range result request frame.

The Requested Outer Loop Counter value starts at zero and is incremented once the Inner Loop Counter has reached its maximum.

The maximum of the Inner and Outer Loop Counters are negotiated during the initial Range request / Range response handshake within the static ranging parameters (see Figure 59d). The actual calcuation of these counters for a specific example is given in Table 5j and Annex V.

As an example, the originator and the recipient have agreed on an outer loop count of two (i.e., phyPMOuterLoopRepetitions is one) and an inner loop count of three (i.e., phyPMInnerLoopRepetitions is two). Furthermore, it is assumed that the originator is able to receive and process all requested Range result confirm frames and, thus, does not initiate any retransmissions of already received result data.

The actual values of the outer and inner loop counter fields for this example are illustrated in Table 5j.

Table 5j—Values of the outer and inner loop counter fields for given example

| Range result request frame number | Requested Outer Loop Counter field value | Requested Inner Loop Counter field value |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 0 | 1 |
| 3 | 0 | 2 |
| 4 | 1 | 0 |
| 5 | 1 | 1 |
| 6 | 1 | 2 |

The actual number of phase measurement values per specific Inner and Outer Loop Count (see Figure 171) depends on the resulting number of measurement frequencies (N = [phyPMStopFreq – phyPMStartFreq] / phyPMStepFreq + 1) and the actual number of phase measurement on a given frequency (M = phyPMCurrNumbersOfPhaseMeasPerFreq) (see Table V.1 and Table V.2). This number of values may exceed the length of one Range result confirm frame. In order to exactly define the expected data, the Requested Result Data Start Address field specifies the start address of the requested result data of the

measurement data based on the specified outer and inner loop counters (see Annex V for an example). This value is updated for each transmitted Range result request frame. If the previous result retrieval was not successful, the originator may repeat the last result data start address to repeat the retrieval of the same result data.

5.3.9a.8 Range result confirm command

The Ranging command frame of subtype Range result confirm is used in response to a received Range  result request frame to provide the requested phase measurement data. The command shall be sent by the ranging recipient to the ranging originator.

5.3.9a.8.1 MHR fields

The Source Addressing Mode, the Source Address, and the Source PAN Identifier fields shall be set according to the parameters Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier within the previously received Range request frame.

The Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier fields shall be set according to the parameters Source Addressing Mode, the Source Address, and the Source PAN Identifier within the previously received Range result request frame.

The PAN ID Compression field shall be handled according to 5.2.1.1.5.

The Frame Pending field shall be set to zero and ignored upon reception, and the AR field shall be set to one.

5.3.9a.8.2 Command Payload fields

The Command Payload fields of the Ranging command frame of subtype range confirm shall be formatted as illustrated in Figure 59o.

| Octets: 1 | 1 | 1 | 2 | 2 | Variable |
| --- | --- | --- | --- | --- | --- |
| Result Data Type | Current Outer Loop Counter | Current Inner Loop Counter | Number of Included Range Result Data Values | Current Result Data Start Address | Range Result Data |

Figure 59o—Command Payload fields of Ranging command frame of subtype Range result

confirm

The Result Data Type field contains the provided type of result data. Valid values of the Result Data Type

field are defined in Table 5k.

Table 5k—Valid values for the Result Data Type field

| Requested ranging mode | Description |
| --- | --- |
| 0x00 | Busy condition at recipient (e.g., due to unfinished phase measurement data compression). Recipient shall repeat a second Range result confirm frame (including the values of Current Outer Loop Counter, Current Inner Loop Counter, and Current Result Data Start Address from the first Range result confirm frame) after finishing the busy condition (see Figure 34f). All further included data in this frame shall be ignored upon reception. |
| 0x01 | Phase measurement data. Only in this case all other payload fields of the frame are valid and shall be extracted upon reception. |
| 0x02-0xff | Reserved. All further included data in this frame shall be ignored upon reception. |

The Current Outer Loop Counter and Current Inner Loop Counter fields contain the current outer and inner loop counter values for which the result data are provided. These fields shall be derived from the previously received Range result request frame.

The Number of Included Range Result Data Values field contains the actually included number of range result data values. The length of this field allows for using larger frames than 127 octets.

The Current Result Data Start Address field specifies the start address of the current result data for a given outer and inner loop counter and shall be derived from the previously received Range result request frame, i.e., the Range result confirm frame shall contain exactly the requested result data from the Range result request frame.

The Range Result Data field contains the actual phase measurement data values. The length of this field is based on the value of the Number of Included Range Result Data Values field. The size of these data values is 1 octet for ranging protocol version 0x00 (see 5.3.9a.1).

5.3.9a.9 Ranging capabilities request command

The Ranging command frame of subtype Ranging capabilities request is used to initiate the retrieval of the ranging capabilities from a ranging peer device and can be sent by any ranging device.

5.3.9a.9.1 MHR fields

The Source Addressing Mode shall be set according to the SrcAddrMode parameter specified by the MLME-RANGING-CAPABILITIES.request primitive.

The Source Address shall contain the address of the device based on the corresponding address mode according to the SrcAddrMode parameter specified by the MLME-RANGING-CAPABILITIES.request primitive.

The Source PAN Identifier fields shall contain the value of macPANId.

The Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier fields shall be set according to the PeerAddrMode, PeerPANId, and PeerAddr parameters specified by the MLME-RANGING-CAPABILITIES.request primitive.

The PAN ID Compression field shall be handled according to 5.2.1.1.5.

The Frame Pending field shall be set to zero and ignored upon reception, and the AR field shall be set to one.

5.3.9a.9.2 Command Payload fields

The Ranging capabilities request Command Payload fields contain only the Requested Ranging Mode field, which is one octet in length.

Valid values of the Requested Ranging Mode field are defined in Table 5l.

Table 5l—Valid values of the Requested Ranging Mode field

| Requested Ranging Mode field values | Description |
| --- | --- |
| 0x00 | Request to provide capabilities of all supported ranging modes |
| 0x01 | Request to provide capabilities of ranging mode PM_RANGING |
| 0x02-0xff | Reserved |

5.3.9a.10 Ranging capabilities response command

The Ranging command frame of subtype Ranging capabilities response is used to provide the requested ranging capabilities to the requesting device. The command can be sent by any ranging device as response to a received Ranging capabilities request frame.

5.3.9a.10.1 MHR fields

The Source Addressing Mode, the Source Address, and the Source PAN Identifier fields shall be set according to the parameters Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier within the previously received Ranging capabilities request frame.

The Destination Addressing Mode, the Destination Address, and the Destination PAN Identifier fields shall be set according to the parameters Source Addressing Mode, the Source Address, and the Source PAN Identifier within the previously received Ranging capabilities request frame.

The PAN ID Compression field shall be handled according to 5.2.1.1.5.

The Frame Pending field shall be set to zero and ignored upon reception, and the AR field shall be set to one.

5.3.9a.10.2 Command Payload fields

The Command Payload fields of the Ranging command frame of subtype Ranging capabilities response shall be formatted as illustrated in Figure 59p.

| Octets: 1 | 1/variable | 0/variable |
| --- | --- | --- |
| Ranging Protocol Version (defined in 5.3.9a.1) | Supported Ranging Modes | PM Ranging Capabilities |

Figure 59p—Command Payload fields of Ranging command frame of subtype Ranging

capabilities response

The Supported Ranging Modes field shall be formatted as illustrated in Figure 59q.

| Octets: 1 | variable |
| --- | --- |
| Number of Included Ranging Modes | Included Ranging Modes |

Figure 59q—Supported Ranging Modes field

The Number of Included Ranging Modes field shall be set according to the requested ranging modes in the received Ranging capabilities request frame.

If the received Ranging capabilities request frame requests the capabilities of all supported ranging modes, the device shall include its number of supported ranging modes. If the device does not support any ranging mode, this field shall be set to zero. If a node supports only one single ranging mode, for example only PM_RANGING, this field shall be set to one.

If the received Ranging capabilities request frame requests the retrieval the capabilities of one specific ranging mode (e.g., PM_ RANGING, as described in 5.3.9a.9.2), which is actually supported by the device, this field shall be set to one. If the requested ranging mode is not supported by the device, this field shall be set to zero.

If the Number of Included Ranging Modes field is set to zero, no further octets containing the Included Ranging Modes field are appended. If the Number of Included Ranging Modes field is set to one, exactly one octet specifying the Included Ranging Modes is appended. If the Number of Included Ranging Modes field is set to any other value, the Included Ranging Modes field comprises one octet for each supported ranging mode containing the actually supported ranging mode.

Currently, only PM_RANGING is supported.

The PM (Phase Difference Measurement) Ranging Capabilities field shall be formatted as illustrated in Figure 59r.

| Octets: 1 | 1 | 1 | 1 | 1 | 1 | … |
| --- | --- | --- | --- | --- | --- | --- |
| PM_RANGING = 0x01 | Length of PM Ranging Capabilities | phyPMMax- NumOfRangings | Minimum phyPM-SetupDuration (in us) | Maximum phyPM-SetupDuration (in us) | phyPMSupported- MeasModes | … |

| … | 1 | 1 | 1 | 1 | 1 | 1 | 1 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | Minimum phyPM-ShiftFreq (in 100 kHz) | Maximum phyPM-ShiftFreq (in 100 kHz) | Minimum phyPM-FreqSettle- Duration (in us) | Maximum phyPM-FreqSettle- Duration (in us) | Minimum phyPM-TxSetup- Duration- Rec (in us) | Maximum phyPM-TxSetup- Duration- Rec (in us) | Minimum phyPM-Sampling- Duration- Orig (in us) | … |

| … | 1 | 1 | 1 | 1 | 1 | … |
| --- | --- | --- | --- | --- | --- | --- |
| … | Maximum phyPMSampling- DurationOrig (in us) | Minimum phyPMTxSetup- DurationOrig (in us) | Maximum phyPMTxSetup- DurationOrig (in us) | Minimum phyPMSampling- DurationRec (in us) | Maximum phyPMSampling- DurationRec (in us) | … |

| … | 1 | 1 | 1 | 1 | 1 | Variable (<=8) | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| … | Minimum phyPM-InnerLoop- Repetitions (in us) | Maximum phyPM-InnerLoop- Repetitions (in us) | Minimum phyPM-OuterLoop- Repetitions (in us) | Maximum phyPM-OuterLoop- Repetitions (in us) | phyPM-LenOf- NumbersOf- Phase- Meas- PerFreq | Set of octets containing a supported number of phase measurements per frequency (phyPM-NumbersOf- PhaseMeas- PerFreq) | phyPM-Parameter- Storage |

Figure 59r—PM Ranging Capabilities field

The actual values to be included within the PM Ranging Capabilities field shall be taken from the phyPMCapTable (see Table 71 and Table 71b).

The Length of PM Ranging Capabilities field is variable and depends on the length of the set of octets containing the supported numbers of phase measurements per frequency.

as an MD.

# 6 MAC services

## 6.2 MAC management service

Insert the following new rows at the end of Table 8:

Table 8—Summary of the primitives accessed through the MLME-SAP

| Name | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| MLME-RANGING | 6.2.25.1 |  |  | 6.2.25.2 |
| MLME-RANGING-CAPABILITIES | 6.2.25.3 |  |  | 6.2.25.4 |

### 6.2.25 Ranging primitives

#### 6.2.25.1 MLME-RANGING.request

The MLME-RANGING.request primitive allows the next higher layer to request a ranging measurement between an originator and a recipient node.

The semantics of this primitive are:

MLME-RANGING.request 	(

OrigAddrMode,

OrigPANId,

OrigAddr,

RecAddrMode,

RecPANId,

RecAddr,

RangingMode,

NumberOfMeasurements,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44ii.

Table 44ii—MLME-RANGING.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| OrigAddrMode | Enumeration | SHORT_ADDRESS, EXTENDED_ADDRESS | The addressing mode of the requested ranging originator. |
| OrigPANId | Integer | 0x0000–0xffff | The PAN identifier of the requested ranging originator. |
| OrigAddr | Device address | As specified by the OrigAddrMode parameter | The individual device address of the requested ranging originator. |
| RecAddrMode | Enumeration | SHORT_ADDRESS, EXTENDED_ADDRESS | The addressing mode of the ranging recipient. |
| RecPANId | Integer | 0x0000–0xffff | The PAN identifier of the ranging recipient. |
| RecAddr | Device address | As specified by the RecAddrMode parameter | The individual device address of the ranging recipient. |
| RangingMode | Enumeration | PM_RANGING = 0x01 Reserved (0x02-0xff) | The requested ranging mode to be used during the ranging measurement. A value of PM_RANGING indicates Phase Difference Measurement. |
| NumberOfMeasurements | Integer | 0x01-0xff | The requested number of ranging measurements between the originator and recipient based on the initial parameter negotiation. |
| SecurityLevel | Integer | As defined in Table 46 | As defined in Table 46. |
| KeyIdMode | Integer | As defined in Table 46 | As defined in Table 46. |
| KeySource | Set of octets | As defined in Table 46 | As defined in Table 46. |
| KeyIndex | Integer | As defined in Table 46 | As defined in Table 46. |

On receipt of the MLME-RANGING.request primitive, the MAC sublayer entity initiates a ranging measurement procedure.

The SecurityLevel parameter specifies the level of security to be applied to the outgoing Ranging command

frames.

#### 6.2.25.2 MLME-RANGING.confirm

The MLME-RANGING.confirm primitive allows the  MAC layer to inform the next higher layer about the result of a finished ranging measurement.

The semantics of the MLME-RANGING.confirm primitive are:

MLME-RANGING.confirm 	(

status,

RangingResultListSize,

RangingResultList,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44jj.

Table 44jj—MLME-RANGING.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, UNSUPPORTED_RANGING, RANGING_IN_PROGRESS, RANGING_IN_PROGRESS_AT_REC, INVALID_ADDRESS, INVALID_DEVICE, CHANNEL_ACCESS_FAILURE, NO_ACK, INVALID_PARAMETER, UNSUPPORTED_RANGING_MODE, RANGING_TIMEOUT, UNSUPPORTED_RANGING_PROTOCOL, RANGING_ABORT, RANGE_NO_SYNC, FRAME_TOO_LONG, IMPROPER_KEY_TYPE, IMPROPER_SECURITY_LEVEL, SECURITY_ERROR, UNAVAILABLE_KEY, UNSUPPORTED_LEGACY, UNSUPPORTED_SECURITY | The status of the last ranging measurement. |
| RangingResultList- Size | Integer | 0x00-0xff | The number of ranging measurement results in the provided list of ranging result. This parameter is only valid if status is SUCCESS. |
| RangingResultList | Set of ranging result values | As defined in Table 44kk | The list of ranging results. This parameter is only valid if status is SUCCESS. |
| SecurityLevel | Integer | As defined in Table 46 or Table 48 | If the primitive were generated following failed outgoing processing of a frame to be transmitted by the ranging originator, then it is as defined in Table 46. If the primitive were generated following receipt of response command from the ranging recipient, then it is as defined in Table 48. |
| KeyIdMode | Integer | As defined in Table 46 or Table 48 | If the primitive were generated following failed outgoing processing of a frame to be transmitted by the ranging originator, then it is as defined in Table 46. If the primitive were generated following receipt of response command from the ranging recipient, then it is as defined in Table 48. |
| KeySource | Set of octets | As defined in Table 46 or Table 48 | If the primitive were generated following failed outgoing processing of a frame to be transmitted by the ranging originator, then it is as defined in Table 46. If the primitive were generated following receipt of response command from the ranging recipient, then it is as defined in Table 48. |
| KeyIndex | Integer | As defined in Table 46 or Table 48 | If the primitive were generated following failed outgoing processing of a frame to be transmitted by the ranging originator, then it is as defined in Table 46. If the primitive were generated following receipt of response command from the ranging recipient, then it is as defined in Table 48. |

Table 44kk—Elements of Ranging Result

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Distance | Integer | 0x0000 0000–0xffff ffff | The measured distance between originator and recipient in mm. |
| DistanceQuality | Integer | 0x00–0x64 | The confidence level of the ranging measurement in %. |

The MLME-RANGING.confirm primitive is generated by the MAC sublayer entity in response to an MLME-RANGING.request primitive. The MLME-RANGING.confirm primitive returns a status of either SUCCESS, indicating that the request to transmit was successful, or the appropriate error code.

If the OrigAddrMode or the RecAddrMode parameter is set to NO_ADDRESS in the MLME-RANGING.request primitive, the ranging measurement shall not be started, and the status shall be set to INVALID_ADDRESS.

If the ranging measurement is requested while another ranging procedure is still ongoing, the new ranging measurement shall not be started, and the status shall be set to RANGING_IN_PROGRESS.

If the macPMRangingEnabled attribute is set to FALSE the ranging measurement shall be aborted, and the status shall be set to UNSUPPORTED_RANGING.

If any ranging measurement relevant frame transmission uses CSMA-CA and the CSMA-CA algorithm failed due to adverse conditions on the channel, the status shall be set to CHANNEL_ACCESS_FAILURE.

If any ranging measurement relevant frame transmission uses acknowledged transmission and the acknowledgment is not received after macMaxFrameRetries retransmissions, the status shall be set to NO_ACK.

If the RangingMode parameter in the MLME-RANGING.request primitive is set to any mode not supported by the originator, the ranging measurement shall be aborted, and the status shall be set to UNSUPPORTED_RANGING_MODE.

If the current value of the PIB attribute macPMAttributeUsageMode requests the usage of phyPMDeviceTable, but the phyPMDeviceTable does not contain a valid entry for the requested recipient device, the ranging measurement shall be aborted, and the status shall be set to INVALID_DEVICE.

If the ranging recipient rejects the ranging measurement with the range acceptance status indicating ranging not supported, the status shall be set to UNSUPPORTED_RANGING_PROTOCOL.

If the ranging recipient rejects the ranging measurement with the range acceptance status indicating ranging already ongoing, the status shall be set to RANGING_IN_PROGRESS_AT_REC.

If the ranging recipient rejects the ranging measurement with the range acceptance status indicating unsupported ranging protocol version, the status shall be set to UNSUPPORTED_RANGING_PROTOCOL.

If the ranging recipient rejects the ranging measurement with the range acceptance status indicating unsupported ranging protocol mode, the status shall be set to UNSUPPORTED_RANGING_MODE.

If the ranging recipient rejects the ranging measurement with the range acceptance status indicating that the originator is not defined in phyPMDeviceTable, the status shall be set to INVALID_DEVICE.

If the ranging recipient rejects the ranging measurement with the range acceptance status indicating invalid ranging parameters, the status shall be set to INVALID_PARAMETER.

If the ranging originator does not receive an expected ranging response within macPMRangeResponseWaitTime, the ranging measurement shall be aborted, and the status shall be set to RANGING_TIMEOUT.

If the ranging originator is about to synchronize with the recipient, but does not receive an expected  Ranging command frame of subtype Range start within macPMSyncWaitTime, the ranging measurement shall be aborted, and the status shall be set to RANGING_NO_SYNC.

If the recipient has accepted the Range request, but the originator detects a condition where it cannot continue with the current ranging procedure (e.g. due to invalid returned ranging parameters from the recipient), the status shall be set to RANGING_ABORT.

6.2.25.3 MLME-RANGING-CAPABILITIES.request

The MLME-RANGING-CAPABILITIES.request primitive allows the next higher layer to request the current phase difference measurement based ranging capabilities of a peer device.

The semantics of this primitive are:

MLME-RANGING-CAPABILITIES.request 	(

SrcAddrMode,

PeerAddrMode,

PeerPANId,

PeerAddr,

RangingMode,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44ll.

Table 44ll—MLME-RANGING-CAPABILITIES.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SrcAddrMode | Enumeration | SHORT_ADDRESS, EXTENDED_ADDRESS | The addressing mode of the device initiating the ranging capabilities request. |
| PeerAddrMode | Enumeration | SHORT_ADDRESS, EXTENDED_ADDRESS | The addressing mode of the requested ranging peer device. |
| PeerPANId | Integer | 0x0000-0xffff | The PAN identifier of the requested ranging peer device. |
| PeerAddress | Device address | As specified by the PeerAddrMode parameter | The individual device address of the requested ranging peer device. |
| RangingMode | Enumeration | All supported ranging modes = 0x00 PM_RANGING = 0x01 Reserved (0x02–0xff) | The ranging mode for which the ranging capabilities are requested. A value of PM_RANGING indicates Phase Difference Measurement. A value of 0x00 indicates the request to retrieve the capabilities of all supported ranging modes. |
| SecurityLevel | Integer | As defined in Table 46 | As defined in Table 46. |
| KeyIdMode | Integer | As defined in Table 46 | As defined in Table 46. |
| KeySource | Set of octets | As defined in Table 46 | As defined in Table 46. |
| KeyIndex | Integer | As defined in Table 46 | As defined in Table 46. |

On receipt of the MLME-RANGING-CAPABILITIES.request primitive, the MAC sublayer entity initiates the retrieval of the ranging capabilities from the requested ranging peer device by transmitting a Range command frame of subtype Ranging capabilities request.

The SecurityLevel parameter specifies the level of security to be applied to the outgoing Ranging command frame of subtype Ranging capabilities request.

The device receiving the Range command frame of subtype Ranging capabilities request and supporting the ranging shall generate a Range command frame of subtype Ranging capabilities response.

6.2.25.4 MLME-RANGING-CAPABILITIES.confirm

The MLME-RANGING-CAPABILITIES.confirm primitive allows the MAC layer to inform the next  higher layer about the result of a request to retrieve the ranging capabilities from a ranging peer node.

The semantics of the MLME-RANGING-CAPABILITIES.confirm primitive are:

MLME-RANGING-CAPABILITIES.confirm 	(

status,

PeerAddrMode,

PeerPANId,

PeerAddr,

OwnAddrMode,

NumberOfIncludedRangingModes,

RangingCapabilitiesList,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44mm.

Table 44mm—MLME-RANGING-CAPABILITIES.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, INVALID_ADDRESS, CHANNEL_ACCESS_FAILURE, NO_ACK | The status of the last ranging capabilities retrieval procedure. |
| PeerAddrMode | Enumeration | SHORT_ADDRESS, EXTENDED_ADDRESS | The addressing mode of the device requesting the ranging capabilities. |
| PeerPANId | Integer | 0x0000-0xffff | The PAN identifier of the device requesting the ranging capabilities. |
| PeerAddress | Device address | As specified by the PeerAddrMode parameter | The individual device address of the device requesting the ranging capabilities. |
| OwnAddrMode | Enumeration | SHORT_ADDRESS, EXTENDED_ADDRESS | The addressing mode of the originating device used in the received Ranging capabilities response frame. |
| NumberOfIncluded-RangingModes | Integer | 0x00-0x08 | The number of Ranging modes included in the RangingCapabilitiesList. A value of 0x00 indicates no supported ranging mode. |
| RangingCapabilitiesList | Set of ranging capabilities | As defined in Table 44nn | The list of ranging modes including their ranging capabilities. If NumberOfIncludedRanging Modes is set to 0x00, this parameter is empty. |
| SecurityLevel | Integer | As defined in Table 46 or Table 48 | If the primitive were generated following failed outgoing processing of a Ranging command frame of subtype Ranging capabilities request, then it is as defined in Table 46. If the primitive were generated following receipt of a Ranging command frame of subtype Ranging capabilities response, then it is as defined in Table 48. |
| KeyIdMode | Integer | As defined in Table 46 or Table 48 | If the primitive were generated following failed outgoing processing of a Ranging command frame of subtype Ranging capabilities request, then it is as defined in Table 46. If the primitive were generated following receipt of a Ranging command frame of subtype Ranging capabilities response, then it is as defined in Table 48. |
| KeySource | Set of octets | As defined in Table 46 or Table 48 | If the primitive were generated following failed outgoing processing of a Ranging command frame of subtype Ranging capabilities request, then it is as defined in Table 46. If the primitive were generated following receipt of a Ranging command frame of subtype Ranging capabilities response, then it is as defined in Table 48. |
| KeyIndex | Integer | As defined in Table 46 or Table 48 | If the primitive were generated following failed outgoing processing of a Ranging command frame of subtype Ranging capabilities request, then it is as defined in Table 46. If the primitive were generated following receipt of a Ranging command frame of subtype Ranging capabilities response, then it is as defined in Table 48. |

If the SrcAddrMode or the PeerAddrMode parameter is set to NO_ADDRESS in the MLME-RANGING-CAPABILITIES.request primitive, the retrieval of ranging capabilities shall be aborted, and the status shall be set to INVALID_ADDRESS.

If the Ranging capabilities request frame transmission uses CSMA-CA and the CSMA-CA algorithm failed due to adverse conditions on the channel, the status shall be set to CHANNEL_ACCESS_FAILURE.

If any Ranging capabilities request frame transmission uses acknowledged transmission and the acknowledgment is not received after macMaxFrameRetries retransmissions, the status shall be set to NO_ACK.

If the device initiating the retrieval of ranging capabilities does not receive the expected Ranging capabilities response within macPMRangeResponseWaitTime, the retrieval of ranging capabilities shall be aborted, and the status shall be set to RANGING_TIMEOUT.

The included parameters NumberOfIncludedRangingModes and RangingCapabilitiesList are only valid if the status indicates SUCCESS.

Table 44nn—Elements of Ranging capabilities

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Supported RangingMode | Integer | PM_RANGING = 0x01 Reserved (0x02-0xff) | The supported ranging mode. |
| Length of ranging capabilties | Integer | Variable | The length of the ranging capabilties for the supported ranging mode |
| Maximum supported number of ranging measurements | Integer | 0x01-0xff | The maximum supported number of ranging measurements. |
| Minimum phyPMSetupDuration time | Integer | 0x00-0xff <= Maximum phyPMSetupDuration | The minimum phyPMSetupDuration time in us. |
| Maximum phyPMSetupDuration time | Integer | 0x00-0xff | The maximum phyPMSetupDuration time in us. |
| phyPMSupportedMeasModes | Integer | 0x00-0xff | Bit field containing the supported PM measurement modes according to phyPMMeasMode. Each set bit indicates a supported PM measurement mode, e.g., a device supporting only PM_UseFreq shall set only bit 0. |
| Minimum phyPMShiftFreq | Integer | 0x00-0xff <= Maximum phyPMShiftFreq | The minimum phyPMShiftFreq in 100kHz. |
| Maximum phyPMShiftFreq | Integer | 0x00-0xff | The maximum phyPMShiftFreq in 100kHz |
| Minimum phyPMFreqSettleDuration time | Integer | 0x00-0xff <= Maximum phyPMFreqSettleDuration | The minimum phyPMFreqSettleDuration time in us. |
| Maximum phyPMFreqSettleDuration time | Integer | 0x00-0xff | The maximum phyPMFreqSettleDuration time in us. |
| Minimum phyPMTxSetupDurationRec time | Integer | 0x00-0xff <= Maximum phyPMTxSetupDurationRec | The minimum phyPMTxSetupDurationRec time in us. |
| Maximum phyPMTxSetupDurationRec time | Integer | 0x00-0xff | The maximum phyPMTxSetupDurationRec time in us. |
| Minimum phyPMSamplingDurationOrig time | Integer | 0x00–0xff <= Maximum phyPMSamplingDuration- Orig | The minimum phyPMSamplingDurationOrig time in us. |
| Maximum phyPMSamplingDurationOrig time | Integer | 0x00-0xff | The maximum phyPMSamplingDurationOrig time in us. |
| Minimum phyPMTxSetupDurationOrig time | Integer | 0x00–0xff <= Maximum phyPMTxSetupDuration- Orig | The minimum phyPMTxSetupDurationOrig time in us. |
| Maximum phyPMTxSetupDurationOrig time | Integer | 0x00-0xff | The maximum phyPMTxSetupDurationOrig time in us. |
| Minimum phyPMSamplingDurationRec time | Integer | 0x00–0xff <= Maximum phyPMSamplingDuration- Rec | The minimum phyPMSamplingDurationRec time in us. |
| Maximum phyPMSamplingDurationRec time | Integer | 0x00-0xff | The maximum phyPMSamplingDurationRec time in us. |
| Minimum phyPMInnerLoopRepetitions | Integer | 0x00–0xff <= Maximum phyPMInnerLoop- Repetitions | The minimum phyPMInnerLoopRepetitions |
| Maximum phyPMInnerLoopRepetitions | Integer | 0x00-0xff | The maximum phyPMInnerLoopRepetitions. |
| Minimum phyPMOuterLoopRepetitions | Integer | 0x00–0xff <= Maximum phyPMOuterLoop- Repetitions | The minimum phyPMOuterLoopRepetitions. |
| Maximum phyPMOuterLoopRepetitions | Integer | 0x00-0xff | The maximum phyPMOuterLoopRepetitions. |
| Appended octets containing supported number of phase measurements per frequency | Integer | 1-8 | The number of appended octets indicating containing supported number of phase measurements per frequency. |
| Set of octets containing supported number of phase measurements per frequency | Integer | 0x01-0xff | The set of octets containing supported number of phase measurements per frequency. Each octet contains one supported number of phase measurements per frequency. |
| phyPMParameterStorage | Integer | 0x00-0x02 | The capability to store sets of ranging parameters. 0x00: Device is neither able to store network based nor device based ranging parameters; only the default parameter set is used. 0x01: Device is able to store network based ranging parameters. 0x02: Device is able to store both network based and device based set of ranging parameters. |

## 6.4 MAC constants and PIB attributes

### 6.4.2 MAC PIB attributes

Insert the following new row at the end of Table 52:

| Attribute | Type | Range | Description | Default |
| --- | --- | --- | --- | --- |
| macPMRangingEnabled | Boolean | TRUE, FALSE | Indication of whether a node is currently supporting phase difference measurement as a ranging method. A value of TRUE indicates that phase difference measurement is permitted. A value of FALSE indicates that phase difference measurement is not permitted. | FALSE |
| macPMRangeResponse-WaitTime | Integer | 2–64 | The maximum time, in multiples of aBaseSuperframeDuration, a device shall wait for a Range response command frame to be available following a Range request command frame. | macResponseWaitTime |
| macPMSyncWaitTime | Integer | 0x0001–0xffff | The maximum time in us, an originator device shall wait for an expected Range command frame of subtype Range start following a transmitted Range Sync request. This time starts immediately at the end of the successfully finished transmission of the Range Sync Request frame including the corresponding Acknowledgement frame. | 0x00ff |
| macPMRangeExtWaitTime | Integer | 2–4 | The maximum time, in multiples of macPMRangeResponseWaitTime, a recipient shall wait for the initial Range result request command frame after the phase measurement and phase measurement result compression if required, and for the originator to wait for a subsequent Range result response frame after receiving a busy indication from the recipient | 2 |
| macPMAttribute-UsageMode | Enumeration | 0x00-0x02 | The usage mode of the phase difference measurement attributes. 0x00: PM attributes from phyPMDefaultTable to be used. 0x01: PM attributes from phyPMNetworkTable to be used. 0x02: PM attributes from phyPMDeviceTable to be used. The value of this attribute must not be larger than the value of phyPMParameterStorage. | 0x00 |
| macPMSuppress-ResultCompression | Boolean | TRUE, FALSE | Indication of whether Phase Measurement result compression (per frequency base) shall be suppressed. | FALSE |
| macPMProvide-RangingTxPower | Boolean | TRUE, FALSE | Indication of whether the ranging originator shall send the ranging recipient its current value of the ranging transmit power. If this attribute is set, the originator will sent the recipient its own value of the PIB attribute to be applied at the recipient during the phase difference measurement. The recipient shall then apply the received ranging transmit power during the phase difference measurement, but must not update its own value of phyPMRangingTxPower. If this attribute is not set, the originator will not provide its own phyPMRangingTxPower, and the recipient shall apply its own value of phyPMRangingTxPower. | FALSE |

# 8. General PHY requirements

## 8.1 General requirements and definitions

Change the following items at the end of the second dashed list in 8.1 as shown:

- CMB OQPSK PHY: direct sequence spread spectrum (DSSS) PHY employing offset quadrature phase-shift keying (OQPSK) modulation, operating in the 195MHz, 416MHz, 619MHz,, as defined in Clause 21.1.

- CMB GFSK PHY: Gaussian frequency-shift keying  (GFSK) PHY  operating in the 195MHz,416MHz, 619MHz, , as defined in Clause 21.2

- CMB DCSS PHY: a discrete chirp spread spectrum (DCSS) ranging PHY, operating in the 195MHz, 416MHz, 619MHz, 915 MHz, 2380 MHz, and 2450 MHz, employing phase difference measurement techniques, as defined in Clause 21.3

### 8.1.1 Operating frequency range

A compliant device shall operate in one or several frequency bands using the modulation and spreading

formats summarized in Table 66. Table 66a shows frequency bands for devices supporting the CMB DSSS PHY. Table 66b shows frequency bands for devices supporting the CMB FSK PHY.

Insert the following new row before 780 DSSS as shown in Table 66:

Table 66a—Frequency bands and data rates for CMB OQPSK DSSS PHY

| Band Identifier | Frequency band(MHz) | Spreading parameters |  | Data parameters |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Chip rate (kchip/s) | Modulation | Bit rate (kb/s) | Symbol rate(ksymbol/s) | Symbols |
| 195 | 174-216 | 1000 | O-QPSK | 250 | 62.5 | 16-ary orthogonal |
|  | 174-216 | 1000 | O-QPSK | 500 | 125 | 8-ary orthogonal |
| 416 | 407-425 | 1000 | O-QPSK | 250 | 62.5 | 16-ary orthogonal |
|  | 407-425 | 1000 | O-QPSK | 500 | 125 | 8-ary orthogonal |
| 619 | 608-630 | 1000 | O-QPSK | 250 | 62.5 | 16-ary orthogonal |
|  | 608-630 | 1000 | O-QPSK | 500 | 125 | 8-ary orthogonal |

Table 66b — Frequency bands and data rates for CMB GFSK PHY

| Band Identifier | Frequency band(MHz) | Spreading parameters |  | Data parameters |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Chip rate (kchip/s) | Modulation | Bit rate (kb/s) | Modulation Index | Symbols |
| 195 | 174-216 | ___ | GFSK (common) | 50 | 1.0 | Binary |
|  | 174-216 | ___ | GFSK (Option) | 100 | 0.5 or 1.0 | Binary |
|  | 174-216 | ___ | GFSK (Option) | 200 | 0.5 or 1.0 | Binary |
| 416 | 407-425 | ___ | GFSK (common) | 50 | 1.0 | Binary |
|  | 407-425 | ___ | GFSK (Option) | 100 | 0.5 or 1.0 | Binary |
|  | 407-425 | ___ | GFSK (Option) | 200 | 0.5 or 1.0 | Binary |
| 619 | 608-630 | ___ | GFSK (common) | 50 | 1.0 | Binary |
|  | 608-630 | ___ | GFSK (Option) | 100 | 0.5 or 1.0 | Binary |
|  | 608-630 | ___ | GFSK (Option) | 200 | 0.5 or 1.0 | Binary |

### 8.1.2 Channel assignments

Insert the following new subclause (8.1.2.12) after 8.1.2.5:

#### 8.1.2.15 Channel numbering for CMB PHY

A channel page (phyCurrentPage; 9.3) value of 14 indicates the CMB PHY

##### 8.1.2.15.1  Channel numbering for O-QPSK PHY

For OQPSK DSSS PHY, each communication channel of MBAN occupies 2MHz band,

- For channel page 14, 21 channels numbered zero to twenty are available across the 195 MHz band.

The center frequencies of these channels are defined as follows:

Fc = 175 + 2 k in megahertz, 		for k = 0, 1, …, 20

- For channel page 14, 9 channels numbered zero to eight are available across the 416 MHz band.

The center frequencies of these channels are defined as follows:

Fc = 408 + 2 k in megahertz, 		for k = 0, 1, …, 8

- For channel page 14, 11 channels numbered zero to ten are available across the 619 MHz band.

The center frequencies of these channels are defined as follows:

Fc = 608 + 2 k in megahertz,		 for k = 0, 1, …, 10

##### 8.1.2.16.2  Channel numbering for GFSK PHY

For GFSK PHY, no matter common or optional each communication channel of CMB occupies 500 kHz band,

- For channel page 11, 84 channels numbered zero to eighty three  are available across the 195 MHz band.

The center frequencies of these channels are defined as follows:

Fc = 174.5+0.5*k megahertz, 		for k = 0, 1, …, 83

- For channel page 14, 36 channels numbered zero to thirty five are available across the 416 MHz band.

The center frequencies of these channels are defined as follows:

Fc = 407.5 + 0.5 k in megahertz, 		for k = 0, 1, …, 35

- For channel page 14, 44 channels numbered zero to forty three are available across the 619 MHz band.

The center frequencies of these channels are defined as follows:

Fc = 607.6 + 0.5 k in megahertz, 		for k = 0, 1, …, 43

where k is the channel number.

##### 8.1.2.15.3  Channel numbering for DCSS PHY

The DCSS PHY does not require an explicit channel numbering. The allocation of frequency band resources during a phase difference measurement is specified with PM attributes phyPMStartFreq, phyPMStopFreq, and phyPMStepFreq as defined in Table 71a. The handling of these parameters is described in Section 5.1.8b.1.

## 8.2 General radio specifications

### 8.2.7 Clear channel assessment (CCA)

Change the third paragraph of 8.2.7 as indicated:

The PHY PIB attribute phyCCAMode, as described in 9.3, shall indicate the appropriate operation mode.

The CCA parameters are subject to the following criteria:

Except for the CMB OQPSK PHY, the ED threshold shall correspond to a received signal power of

at most 10 dB greater than the specified receiver sensitivity for that PHY. For the CMB OQPSK

PHY, the ED threshold shall comply with the specification in 21.1.3.4.13.

# 9. PHY services

## 9.2 PHY constants

Change Table 70 (the entire table is not shown) as indicated:

Table 70—PHY constants

| Constant | Description | Value |
| --- | --- | --- |
| aMaxPHYPacketSize | The maximum PSDU size (in | For CMB GFSK |
|  | octets) the PHY shall be able to receive. | PHYs ??? For DSSS PHY, 127 |
| aTurnaroundTime | RX-to-TX or TX-to-RX turn- | For the CMB GFSK PHYs, the value is 1 ms expressed in symbol periods, rounded up to the next integer number of symbol periods using the ceiling() function.* For the CMB DSSS PHY, the value is 1 ms expressed in modulation symbol periods, rounded up to the next integer number of symbol periods using the ceiling() function. The value is 12 for all other PHYs. ?? |
|  | around time (in symbol periods), as defined in 8.2.1 and 8.2.2. |  |

*The function ceiling() returns the smallest integer value greater than or equal to its argument value.

## 9.3 PHY PIB attributes

Change Table 71 (the entire table is not shown) as indicated:

Table 71— PHY PIB attributes

| Attribute | Type | Range | Description |
| --- | --- | --- | --- |
| phyCurrentChannel | Integer | As defined in 8.1.2 | The logical channel to use for all following transmissions and receptions, 8.1.2. |
| phyCurrentPage | Integer | Any valid channel page | This is the current PHY channel page. This is used in conjunction with phyCurrentChannel to uniquely identify the channel currently being used. |
| phyCMBCurrentBand | Enumeration | 176, 416,619 | The operating frequency band currently selected. |
| phyCMBDSSSSymbolRate | Float | See Table 66 | The currently selected symbol rate in ksymbols per second. The valid symbol rates per band are given in Table 66. |
| phyCurrentCMBPHYType | Enumeration | DSSS, GFSK | Specifies the CMB PHY type in use. |
| phyChannelSpacing | Enumeration | 500, 2000 | The channel spacing, measured in kHz, that is used with phyCurrentBand and phyCurrentChannel to specify the frequency channel being used. |
| phyPMDefaultTable | Set of PM attributes, as defined in Table 71a | – | The current default Phase difference measurements attributes for this device in case the default ranging parameters shall be used. |
| phyPMCapTable | Set of PM capabilities, as defined in Table 71b | – | The relevant Phase difference measurement capabilities for this device. |
| phyPMNetworkTable | Set of PM attributes, as defined in Table 71a | - | The Phase difference measurements attributes for this ranging network in case network based ranging parameters shall be used. |
| phyPMDeviceTable | Set of PM device entries, as defined in Table71c | - | The Phase difference measurements attributes for a list of known ranging devices in case devices based ranging parameters shall be used. |

Insert Table 71a, Table 71b, and Table 71c after Table 71:

Table 71a—Elements of PM attributes

| Attribute | Type | Range | Description |
| --- | --- | --- | --- |
| phyPMSetupDuration | Integer | 0x00-0xff | The setup duration required to synchronize the start of a new phase difference measurement procedure, in us, for the originator and recipient nodes. |
| phyPMMeasMode | Integer | 0-7 | Utilized Phase Measurement Mode: 0: PM_UseFreq (mandatory, enables phyPMStartFreq, phyPMStopFreq, and phyPMStep- Freq; see 21.3.1 and 21.3.2) 1–7: Reserved |
| phyPMStartFreq | Integer | 0x0000-0xffff | Start frequency for phase difference measurement in 100 kHz. |
| phyPMStopFreq | Integer | 0x0000-0xffff | Stop frequency for phase difference measurement in 100 kHz. |
| phyPMStepFreq | Integer | 0x00-0xff | Frequency step for phase difference measurement in 100 kHz. |
| phyPMShiftFreq | Integer | 0x00-0xff | Transmit frequency shift between phase 1 and phase 2 during a phase difference measurement in 100 kHz; fPhase2 = fPhase1 + phyPMShiftFreq |
| phyPMFreqSettleDuration | Integer | 0x00-0xff | Settle duration required for initializing a new frequency during a phase difference measurement in us. |
| phyPMTxSetupDurationRec | Integer | 0x00-0xff | Time required for transmitter settling of recipient node within phase difference measurement phase 1 in us. |
| phyPMSamplingDurationOrig | Integer | 0x00-0xff | Time required for actual phase measurement of originator node within phase difference measurement phase 1 in us. |
| phyPMTxSetupDurationOrig | Integer | 0x00-0xff | Time required for transmitter settling of originator node within phase difference measurement phase 2 in us. |
| phyPMSamplingDurationRec | Integer | 0x00-0xff | Time required for actual phase measurement of recipient node within phase difference measurement phase 2 in us. |
| phyPMInnerLoopRepetitions | Integer | 0x00-0xff | Repetition count for inner Phase Difference measurement loop including - phyPMTxSetupDurationOrig - phyPMTxSetupDurationRec - phyPMSamplingDurationOrig - phyPMSamplingDurationRec. |
| phyPMOuterLoopRepetitions | Integer | 0x00-0xff | Repetition count for outer Phase Difference measurement loop, including an entire Phase Difference measurement procedure consisting of ((phyPMStopFreq – phyPMStartFreq)/phyPMStepFreq + 1) individual cycles. One cycle incorporates the following sequences: - phyPMFreqSettleDuration - phyPMTxSetupDurationOrig - phyPMTxSetupDurationRec - phyPMSamplingDurationOrig - phyPMSamplingDurationRec. |
| phyPMCurrNumbersOf-PhaseMeasPerFreq | Integer | 0x01-0xff | The currently supported number of phase measurements per frequency for a given inner and outer loop count. |
| phyPMRangingTxPower | Signed integer | — | The transmit power of the device, in dBm, to be used during the actual phase difference measurement. The value is handled similar to phyTXPower, and the corresponding tolerance on the transmit power setting is taken from phyTXPowerTolerance. |

Table 71b—Elements of PM capabilities

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| phyPMMaxNumOfRangings | Integer | 0x01-0xff | The maximum supported number of ranging measurements. |
| phyPMMinSetupDuration | Integer | 0x00-0xff <= phyPMMaxSetupDuration | The minimum phyPMSetupDuration time in us. |
| phyPMMaxSetupDuration | Integer | 0x00-0xff | The maximum phyPMSetupDuration time in us. |
| phyPMSupportedMeasModes | Integer | 0x00-0xff | The bit field containing the supported PM measurement modes according to phyPMMeasMode; each set bit indicates a supported PM measurement mode, e.g., a device supporting only PM_UseFreq shall set only bit 0. |
| phyPMMinShiftFreq | Integer | 0x00-0xff <= phyPMMaxShiftFreq | The minimum phyPMShiftFreq in 100kHz. |
| phyPMMaxShiftFreq | Integer | 0x00-0xff | The maximum phyPMShiftFreq in 100kHz. |
| phyPMMinFreqSettleDuration | Integer | 0x00-0xff <= phyPMMaxFreqSettleDuration | The minimum phyPMFreqSettleDuration time in us. |
| phyPMMaxFreqSettleDuration | Integer | 0x00-0xff | The maximum phyPMFreqSettleDuration time in us. |
| phyPMMinTxSetupDurationRec | Integer | 0x00-0xff <= phyPMMaxTxSetupDurationRec | The minimum phyPMTxSetupDurationRec time in us. |
| phyPMMaxTxSetupDurationRec | Integer | 0x00-0xff | The maximum phyPMTxSetupDurationRec time in us. |
| phyPMMinSamplingDurationOrig | Integer | 0x00-0xff <= phyPMMaxSamplingDurationOrig | The minimum phyPMSamplingDurationOrig time in us. |
| phyPMMaxSamplingDurationOrig | Integer | 0x00-0xff | The maximum phyPMSamplingDurationOrig time in us. |
| phyPMMinTxSetupDurationOrig | Integer | 0x00-0xff <= phyPMMaxTxSetupDurationOrig | The minimum phyPMTxSetupDurationOrig time in us. |
| phyPMMaxTxSetupDurationOrig | Integer | 0x00-0xff | The maximum phyPMTxSetupDurationOrig time in us. |
| phyPMMinSamplingDurationRec | Integer | 0x00-0xff <= phyPMMaxSamplingDurationRec | The minimum phyPMSamplingDurationRec time in us. |
| phyPMMaxSamplingDurationRec | Integer | 0x00-0xff | The maximum phyPMSamplingDurationRec time in us. |
| phyPMMinInnerLoopRepetitions | Integer | 0x00-0xff <= phyPMMaxInnerLoopRepetitions | The minimum phyPMInnerLoopRepetitions. |
| phyPMMaxInnerLoopRepetitions | Integer | 0x00-0xff | The maximum phyPMInnerLoopRepetitions. |
| phyPMMinOuterLoopRepetitions | Integer | 0x00-0xff <= phyPMMaxOuterLoopRepetitions | The minimum phyPMOuterLoopRepetitions. |
| phyPMMaxOuterLoopRepetitions | Integer | 0x00-0xff | The maximum phyPMOuterLoopRepetitions. |
| phyPMLenOf- NumbersOf- PhaseMeasPerFreq | Integer | 1-8 | The list size of the supported numbers of phase measurements per frequency. |
| phyPMNumbersOf-PhaseMeasPerFreq | Set of phyPM-LenOf-NumbersOf-PhaseMeas-PerFreq octets | 0x01-0xff | The set of octets containing the supported number of phase measurements per frequency. Each octet contains one supported number of phase measurements per frequency. |
| phyPMParameterStorage | Integer | 0x00-0x02 | The capability to store sets of ranging parameters. 0x00: Device is neither able to store network-based nor device-based ranging parameters; only the default parameter set is used. 0x01: Device is able to store network-based ranging parameters. 0x02: Device is able to store both network-based and device-based set of ranging parameters. |

Table 71c—Elements of PM device entries

| Attribute | Type | Range | Description |
| --- | --- | --- | --- |
| PANId | Device PAN ID | 0x0000–0xffff | The PAN identifier of the device in this device entry. |
| ShortAddress | Device short address | 0x0000-0xffff | The short address of the device in this device entry. A value of 0xfffe indicates that this device is using only its extended address. A value of 0xffff indicates that this value is unknown. |
| ExtAddress | IEEE address | Any valid extended IEEE address | The extended IEEE address of the device. |
| phyPMDeviceTable | Set of PM attributes, as defined in Table 71a | – | The phase difference measurements attributes for this device in case the device based ranging parameters shall be used. |

# 21. CMB PHY

Two PHYs are specified in order to support CMB applications: OQPSK , DSSS, as described in 21.1, and GFSK, as described in 21.2.

## 21.1 OQPSK  PHY specification

The O-QPSK DSSS PHY is described in the following subclauses.

### 21.1.1 PPDU format

For convenience, the PPDU structure is presented so that the leftmost field as written in this standard shall

be transmitted or received first. All multiple octet fields shall be transmitted or received least significant

octet first, and each octet shall be transmitted or received least significant bit (LSB) first.

The PPDU shall be formatted as illustrated in Figure 21.1.1

| Octets |  |  |  |
| --- | --- | --- | --- |
| 4 | 2 | 3 | variable |
| Preamble | SFD | As defined in 21.1.1.3 | PSDU |
| SHR |  | PHR | PHY payload |

Figure 21.1.1 Format of the OQPSK PHY PPDU

#### 21.1.1.1 Preamble field

The length of the preamble for the O-QPSK PHYs shall be 8 symbols (i.e., 4 octets), and the bits in the

Preamble field shall be binary zeros.

#### 21.1.1. 2 SFD field

The SFD is a field indicating the end of the SHR and the start of the packet data. The SFD shall be formatted as illustrated in Table 21.1.1

Table 21.1.1—Format of the SFD field for the CMB O-QPSK DSSC PHY

| Bit 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |

#### 21.1.1.3 PHR field

The PHR field  is  descripted in Figure 21.1.2. All multi-bit fields are unsigned integers and shall be

processed MSB first.

| Bit string index | 0 | 1 | 2 | 3 | 4 | 5 - 15 | 16 - 23 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit mapping | SM | RM1 | RM0 | R1 | R0 | L10–L0 | H7–H0 |
| Field name | Spreading mode | Rate Mode |  | Reserved |  | Frame Length | HCS |

Figure 21.1.2 Format of the PHR for O-QPSK-DSSC

For the 195 MHz, 416 MHz, and 618 MHz frequency bands, the SpreadingMode (SM) field

shall be set to one if DSSS is used for PSDU spreading (variable SpreadingMode set to DSSS, as

described in 21.3.2.4). Otherwise, the SM field shall be set to zero if DSSS is used for PSDU spreading

(variable SpreadingMode set to DSSS, as described in 21 .3.2.4).

The OQPSK PHY supports up to two different PSDU rate modes within each frequency band, and the

rate mode is given by the RateMode field (RM1–RM0). Table 21.1.2 shows the mapping of the bit values to

the RateMode. The Frame Length field (L10–L0) specifies the total number of octets contained in the PSDU (prior to FEC encoding).

Table 21.1.2 —Rate mode mapping of the CMB O-QPSK PHY

| (RM1, RM0) | RateMode |
| --- | --- |
| (0,0) | 0 |
| (0,1) | 1 |
| (10) | 2 |
| (1,1) | 3 |

The Header Check Sequence (HCS) field (H7–H0) is calculated over the first 16 PHR bits, where b0is the PHR bit at bit string index 0 and is the PHR bit atb15 bit string index 15, as described in Figure 21.1.2. The HCS field is defined as

(H7,H7,...,H1,H0)=(r0,r1,...,r6,r7)                                                                          (21.1.1)

for certain coefficientsr0,r1,...,r6,r7 . The computation of those coefficients is shown by the following algorithm.

The HCS shall be calculated using the following standard generator polynomial of degree 8:

G8(x)=x8+x2+x+1

The HCS shall be calculated as follows:

— LetM(x)=b0x15+b1x14+...+b14x+b15  be the polynomial representing the sequence of bits for which the checksum is to be computed.

— Multiply M(x) by x8 , giving the polynomial  .

— Divide modulo 2 by the generator polynomial,G8(x) to obtain the remainder polynomial, R(x)=r0x7+r1x6+...+r6x+r7

The HCS field is given by the coefficients of the remainder polynomial as shown in Equation (21.1.1).  An

example HCS is shown in Figure 21.1.3.

| Bit string index | 0 | 1 | 2 | 3 | 4 | 5-15 | 16-23 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit mapping | SM | RM1 | RM0 | R1 | R0 | L10-L0 | H7-H0 |
| Example value | 0 | 0 | 1 | 0 | 0 | 00000101010 | 01111000 |

Figure 21.1.3：Example HCS for CMB OQPSK

A typical implementation is depicted in Figure 21.1.4

CRC-8 Generator Polynomial: G8(x)=x8+x2+x+1

.

1)   Initialize the remainder register(r0,r1,...,r0,r7) to zero.

2)   Shift the sequenceb0,b1,...,b15 into the divider beginning withb0 .

3)   After the last bit, b15 , is shifted into the divider, the remainder register contains

the HCS: (r0,r1,...,r6,r7)=(H7,H7,...,H1,H0) .

Figure 21.1.4—Typical HCS implementation for CMB OQPSK

##### 21.1.3.1.4 PSDU field

The PSDU field carries the payload data of the PHY packet.

#### 21.1.3.2 Modulation and coding for CMB O-QPSK

##### 21.1. 3.2.1 Reference modulator diagram

Figure 21.1.5 shows the reference modulator diagram for the CMB O-QPSK PHY.

The inputs to the reference modulator are the bit sequences of the SHR field, the PHR field, and the PSDU

field.

Figure 21.1.5 Reference Diagram of CMB OQPSK

##### 21.1.3.2.2 SHR coding and spreading

A SHR sequence includes SFD bits and preamble bits. The spreading is (16,4) DSSS bit-to-chip mapping . The parameters is shown in Table 21.1.5

21.1.3.2.3 PHR coding and spreading

The PHR field, consisting of 24 information bits, as described in Figure 21.1.2。A SHR sequence includes SFD bits and preamble bits. The spreading is (16,4) DSSS bit-to-chip mapping . The parameters is shown in Table 21.1.5

21.1.3.2.4 PSDU coding and spreading for DSSS

Figure 21.1.5 shows the signal flow when DSSS is applied to the PSDU (Spreading Mode set to DSSS). The supported PSDU parameters for SpreadingMode DSSS are shown in Table 21.1.3.

Depending on the frequency band and RateMode, the output sequence of the bit-to-chip mapper shall be

whitened, as described in 21.1.3.2.6.

The relationship between the RateMode and the DataRate parameter of the MCPS-DATA.request primitive

is described in Table 46.

21.1.3.2.5 DSSS bit-to-chip mapping

Table 21.1.3, and Table 21.1.4 show (N,4) DSSS supported by the CMB OQPSK PHY.

Table 21.1.3—(8,4) DSSS bit-to-chip mapping

| Data Symbol (decimal) | Chip Values for (8,4) DSSS (c0 c1 … c6 c7) |
| --- | --- |
| 0 | 0 0 0 0 0 0 0 1 |
| 1 | 1 1 0 1 0 0 0 0 |
| 2 | 0 1 1 0 1 0 0 0 |
| 3 | 1 0 1 1 1 0 0 1 |
| 4 | 1 1 1 0 0 1 0 1 |
| 5 | 0 0 1 1 0 1 0 0 |
| 6 | 1 0 0 0 1 1 0 0 |
| 7 | 0 1 0 1 1 1 0 1 |
| 8 | 1 0 1 0 0 0 1 0 |
| 9 | 0 1 1 1 0 0 1 1 |
| 10 | 1 1 0 0 1 0 1 1 |
| 11 | 0 0 0 1 1 0 1 0 |
| 12 | 0 1 0 0 0 1 1 0 |
| 13 | 1 0 0 1 0 1 1 1 |
| 14 | 0 0 1 0 1 1 1 1 |
| 15 | 1 1 1 1 1 1 1 0 |

Table 21.1.4—(16,4) DSSS bit-to-chip mapping

| Data Symbol (decimal) | Chip Values for (16,4) DSSS (c0 c1 … c14 c15) |
| --- | --- |
| 0 | 0 0 1 1 1 1 1 0 0 0 1 0 0 1 0 1 |
| 1 | 0 1 0 0 1 1 1 1 1 0 0 0 1 0 0 1 |
| 2 | 0 1 0 1 0 0 1 1 1 1 1 0 0 0 1 0 |
| 3 | 1 0 0 1 0 1 0 0 1 1 1 1 1 0 0 0 |
| 4 | 0 0 1 0 0 1 0 1 0 0 1 1 1 1 1 0 |
| 5 | 1 0 0 0 1 0 0 1 0 1 0 0 1 1 1 1 |
| 6 | 1 1 1 0 0 0 1 0 0 1 0 1 0 0 1 1 |
| 7 | 1 1 1 1 1 0 0 0 1 0 0 1 0 1 0 0 |
| 8 | 0 1 1 0 1 0 1 1 0 1 1 1 0 0 0 0 |
| 9 | 0 0 0 1 1 0 1 0 1 1 0 1 1 1 0 0 |
| 10 | 0 0 0 0 0 1 1 0 1 0 1 1 0 1 1 1 |
| 11 | 1 1 0 0 0 0 0 1 1 0 1 0 1 1 0 1 |
| 12 | 0 1 1 1 0 0 0 0 0 1 1 0 1 0 1 1 |
| 13 | 1 1 0 1 1 1 0 0 0 0 0 1 1 0 1 0 |
| 14 | 1 0 1 1 0 1 1 1 0 0 0 0 0 1 1 0 |
| 15 | 1 0 1 0 1 1 0 1 1 1 0 0 0 0 0 1 |

21.1.3.2.6 Chip whitening

When SpreadingMode is set to DSSS, the PSDU chip sequence shall be whitened, depending on the

frequency band and RateMode, as shown in Table 21.1.6. This improves spectral properties of modes with low spreading gain or insufficient spectral properties (i.e., notches) of the spreading codes. For all other modes, no chip whitening shall be applied.

Table 21.1.5 Chip Whitening for DSSS

| Frequency band (MHz) | RateMode |
| --- | --- |
| 176-214 | 1 |
| 407-425 | 1 |
| 608-630 | 1 |

Chip whitening is the modulo-2 addition of a chip of the PSDU at the output of the bit-to-chip mapper with

the value of a cyclic m-sequence of length for m = 9. This shall be performed by the transmitter and is described by

Where

is the raw PSDU chip being whitened

is the whitened chip

Index k starts at 0, referring to the first chip of the PSDU at the output of the bit-to-chip mapper and is

increased by one at every chip interval. Figure 21.1.6 shows the whitening process. At k = 0, the register shall be initialized with

Figure 21.1.6-Chip whitening

21.1.3.2.7 Modulation parameters for O-QPSK

A chip value shall be mapped into a binary real-valued symbol out of {–1,1} by the mapping

ζ(c)=−1,c=0+1,c=1

In the 195 MHz, 416 MHz, and 619 MHz bands, a raised cosine pulse shape with roll-off factor of r = 0.8 is used to represent each baseband symbol and is described by

#### 21.1.3.4 CMB O-QPSK PHY RF requirements

##### 21.1.3.4.1 Operating frequency range

The CMB O-QPSK PHY operates in the following Chinese approved Medical bands:

— 174–216 MHz

— 407-425 MHz

— 608-630 MHz

##### 21.1.3.4.2 Transmit power spectral density (PSD) mask

In general, when operating in the 176-214 MHz, 407-425 MHz and 608-630 MHz band, the transmitted spectral products shall be less than the limits specified in Table 21.1.7. For both relative and absolute limits, average spectral power shall be measured using a 100 kHz resolution bandwidth. For the relative limit, the reference level shall be the highest average spectral power measured within ±600 kHz of the carrier frequency fc.

Table 21.1.6 PSD Limitation among Channels.

| Frequency | Relative Limit | Absolute limit |
| --- | --- | --- |
| ¦f-fc¦ > 1.2 MHz | -20 dB | -20dB |

However, there are some special limitation for spurious radio emission on the Chinese Radio regulation Annex I. These spurious emission related to approved Chinese are shown in Annex J.

The CMB O-QPSK transmits PSD mask shall conform with Chinese Radio regulations, which are shown in Annex I and Annex J.

##### 21.1.3.4.3 Receiver sensitivity

Under the conditions specified in 8.1.7, a compliant device shall be capable of achieving the sensitivity values given in Table 21.1.8 or better.

Table 21.1.7 Required receiver sensitivity for SpreadingMode DSSS

| Frequency band (MHz) | RateMode |  |
| --- | --- | --- |
|  | 0 | 1 |
| 176-214 | -85 | -82 |
| 407-425 | -85 | -82 |
| 608-630 | -85 | -82 |

##### 21.1.3.4.4 Receiver interference rejection

There are multiple receiver interference caused by CMB O-QPSK compliant signals,  and TV , industry and commercial in the same bands.

The interfering signal shall be an CMBO-QPSK compliant signal with the following characteristics:

— Pseudo-random PSDU

— SpreadingMode set to DSSS

— The same chip rate as the desired signal

— Chip-whitening enabled

The TV, industry and commercial interference are:

- On 176-216 MHz band, the major interferences are sourced from: Digital TV, and Wireless Microphone

- On 407-425 MHz band, the major interference are sources from Interphone (Walkie-talkie)

- On 608-630 MHz band, the major interference are sources from Digital TV.

The details of TV, industry and commercial are described in Annex K.

The interference-to-signal ratio (ISR) is the ratio of the signal power of an interferer relative to the signal

power of the desired signal.

The adjacent channel rejection shall be measured as follows: the desired signal shall be a CMB O-QPSK compliant signal of pseudo-random PSDU data. For a given RateMode, the desired signal is input to the receiver at a level 3 dB above the maximum allowed receiver sensitivity of Table 21.8.

The TV rejection shall be measured as follows: at CCA detection time, if the receiver ED level is higher ED threshold desired by user, there may be TV or other industry or commercial interference on same channel or band.

The detection methods of TV, industry and commercial interference are beyond IEEE802.15.4N standard.

##### 21.1.3.4.5 Tx-to-Rx turnaround time

The CMB O-QPSK PHY shall meet the requirements for TX-to-RX turnaround time as defined in 8.2.1.

##### 21.1.3.4.6  Rx-to-Tx turnaround time

The CMB O-QPSK PHY shall meet the requirements for RX-to-TX turnaround time as defined in 8.2.2.

##### 21.1.3.4.7  Error-vector magnitude (EVM) definition

A transmitter shall have EVM values of less than 35% when measured for 1000 chips. The EVM measurement shall conform with 8.2.3.

##### 21.1.3.4.8 Transmit center frequency and symbol tolerance

The transmit center frequency tolerance shall be ±40 ppm maximum.

##### 21.1.3.4.9 Transmit power

A transmitter shall be capable of transmitting at least –3 dBm. Devices should transmit lower power when

possible in order to reduce interference to other devices and systems.

The maximum transmit power is limited by local regulatory bodies.

##### 21.1.3.4.10 Receiver maximum input level of desired signal

The CMB O-QPSK PHY shall have a receiver maximum input level greater than or equal to –20 dBm using the measurement defined in 8.2.4.

##### 21.1.3.4.11 Receiver ED

The CMB O-QPSK PHY shall provide the receiver ED measurement as described in 8.2.5.

##### 21.1.3.4.12 Link quality indicator

The CMB O-QPSK PHY shall provide the LQI measurement as described in 8.2.6.

##### 21.1.3.4.13 Clear channel assessment (CCA)

The CMB O-QPSK PHY shall use one of the CCA methods as described in 8.2.7.

The detection time, aCCATime (as defined in 9.2), for clear channel assessment (CCA) is shown in

Table 188;

The ED threshold shall correspond to a received signal power of at most –79 dBm, when applying CCA Mode 1 or CCA Mode 3, as defined in 8.2.7.

Table 21.1.8—CCA duration for CMB O-QPSK PHY

| Frequency band (MHz) | aCCA Time (# of symbols) |
| --- | --- |
| 176-216 | 8 |
| 407-425 | 8 |
| 608-630 | 8 |

## 21.2 CMB GFSK PHY

The CMB GFSK PHY is described in the following subclauses.

### 21.2.1 PPDU format for CMB GFSK PHY

The CMB GFSK PPDU shall support the format shown in Figure 21.2.1.

The SHR, PHY header (PHR), and PHY payload are treated as bit strings of length n, numbered b0 on the left and bn-1 on the right. When transmitted, they are processed b0 first to bn-1 last, without regard to their content or structure.

All reserved fields shall be set to set to zero upon transmission and shall be ignored upon reception.

Figure 21.2.1— Format of the CMB GFSK PHY PPDU

#### 21.2.1.1 Preamble field

The Preamble field shall contain phyFSKPreambleLength (as defined in 9.3) multiples of the 8-bit sequence “01010101” for CMB GFSK.

#### 21.2.1.2 SFD

The SFD for CBM GFSK shall be a 2-octet sequence selected from the list of values shown in Table 21.2.1. Devices that do not support FEC shall support the SFD associated with uncoded (PHR + PSDU) and a value of zero for the PIB attribute phyMRFSKSFD, as defined in 9.3; these devices may also support the SFD associated with uncoded (PHR + PSDU) and a value of one for the PIB attribute phyMRFSKSFD. Devices that support FEC shall support both SFDs associated with a value of zero for the PIB attribute phyMRFSKSFD; these devices may additionally support both SFDs associated with a value of one for the PIB attribute phyMRFSKSFD.

The SFD is transmitted starting from the leftmost bit (i.e., starting with b0).

Table 21.2.1—CMB GFSK PHY SFD values for G

|  | SFD value for coded (PHR + PSDU) (b0–b15) | SFD value for uncoded (PHR + PSDU) (b0–b15) |
| --- | --- | --- |
| phyMRFSKSFD = 0 | 0110 1111 0100 1110 | 1001 0000 0100 1110 |
| phyMRFSKSFD = 1 | 0110 0011 0010 1101 | 0111 1010 0000 1110 |

#### 21.2.1.3 PHR

The format of the PHR is shown in Figure 21.2.2. All multi-bit fields are unsigned integers and shall be processed MSB first.

The FCS Type field (FCS) indicates the length of the FCS field described in 5.2.1.9 that is included in the MPDU. Table 21.2.2 shows the relationship between the contents of the FCS Type field and the length of the transmitted FCS.

Figure 21.2.2—Format of the PHR for CMB GFSK

Table 21.2.2—Relationship between FCS Type field and transmitted FCS length

| FCS Type field value | Transmitted FCS length |
| --- | --- |
| 0 | 4-octets |
| 1 | 2-octets |

The Data Whitening field (DW) indicates whether data whitening of the PSDU is used upon transmission. When data whitening is used, the Data Whitening field shall be set to one. It shall be set to zero otherwise. Data whitening shall not be applied to the SHR or PHR.

The Frame Length field (L10–L0) specifies the total number of octets contained in the PSDU (prior to FEC encoding, if enabled). The most significant bit (leftmost) shall be transmitted first.

#### 21.2.1.4 PSDU field

The PSDU field carries the data of the PPDU.

### 21.2.2 Modulation and coding for CMB GFSK PHY

#### 21.2.2.1 Gaussian Filter definition

The transmission pulse shape p(t) for CMB GFSK PHY shall be constrained by the shape of a normalized cross-correlation function (t) with a standard reference Gaussian filter whose pulse shape r(t) given by the next formula.

where BT is 0.7 and t is normalized by the symbol duration.

The normalized cross-correlation (t) between two waveforms, p(t) and r(t), is defined as

where Er and Ep are the energies of r(t) and p(t) respectively.

In order for CMB GFSK PHY transmitter to be compliant with this standard, the transmitted pulse p(t) shall have a magnitude of the cross-correlation function |  (τ)| whose main lobe is greater than or equal to TBD1, when integrated over a period of at least 2xTBD2 time.

The modulation for the CMB GFSK PHY is a 2-level Gaussian filtered FSK that meets the transmit spectral mask, as defined in 18.1.5.6. GFSK with a BT value of 0.7 should be used in Mode #1, #2 and #3.

Table 21.23 shows the modulation and channel parameters for the standard-defined PHY operating modes for the 174 MHz, 407 MHz, and 608 MHz bands. A device shall support operating mode #1 and may additionally support operating modes #2 and #3.

Table 21.2.3—CMB GFSK modulation and channel parameters

| Frequency band (MHz) | Parameter | Operating mode #1 | Operating mode #2 | Operating mode #3 |
| --- | --- | --- | --- | --- |
| 174-216MHz, 407-425MHz, 608-630MHz | Data rate (kb/s) | 50 | 100 | 200 |
|  | Modulation index | 1.0 | 0.5 or 1.0 | 0.5 or 1.0 |
|  | Channel spacing (KHz) | 500 | 500 | 500 |

#### 21.2.2.2 Reference modulator diagram

The functional block diagram in Figure 21.2.3 is provided as a reference for specifying the CMB GFSK PHY data flow processing functions. The subclause number in each block refers to the subclause that describes that function. Each bit shall be processed using the bit order rules defined in 20.2.1.

When FEC is enabled, the PHR and PSDU shall be processed for coding as a single block of data, as described in 20.2.2.3.3. When data whitening is enabled, the scrambling shall be only applied over the PSDU, as described in 20.2.3.

All fields in the PPDU shall use the same symbol rate and modulation order, unless otherwise specified elsewhere in this standard.

Figure 21.2.3—CMB GFSK FEC, data whitening, and modulator functions

#### 21.2.2.3 Bit-to-symbol mapping

#### The nominal frequency deviation, △f, shall be

(symbol rate × modulation index2)

The symbol encoding for CMB GFSK PHY is shown in Table 21.2.4, where the frequency deviation, fdev, is equal to △f  for filtered 2FSK.

Table 21.2.4—CMB GFSK symbol encoding

| 2-level |  |
| --- | --- |
| Symbol (binary) | Frequency deviation |
| 0 | –fdev |
| 1 | +fdev |

#### 21.2.2.4 Modulation quality

Modulation quality shall be measured by observing the frequency deviation tolerance and the zero crossing tolerance of the eye diagram caused by a PN9 sequence of length 511 bits.

#### 21.2.2.5 Frequency deviation tolerance

Modulation frequency tolerance is measured as a percentage of the frequency deviation, fdev, dictated by the modulation index. In the case of filtered 2FSK, the measured frequency deviation, f, at Ts / 2 shall be constrained to the range TBD% fdev < |f| < TBD% fdev, as shown in Figure 21.24, where Ts is the symbol time.

Figure 21.2.4—Eye diagram for filtered 2FSK

#### 21.2.2.6 Zero crossing tolerance

In the case of filtered 2FSK, the excursions for the zero crossings for all trajectories of the eye diagram shall be constrained to within ±TBD% of the symbol time Ts, as shown in Figure 21.24.

#### 21.2.2.7 Forward error correction (FEC)

FEC is optional. If the SFD indicates that FEC is used, as described in Table 21.21, then the FEC is applied to the PHR and PSDU as a single block of data.

Two types of FEC may be applied: a recursive and systematic code (RSC) or a nonrecursive and nonsystematic code (NRNSC). The use of RSC or NRNSC coding shall be controlled by the PIB attribute phyFSKFECScheme, as defined in 9.3.

When the SFD value indicates a coded packet, FEC shall be employed on the PHR and PSDU bits, applying either a 1/2-rate systematic or nonsystematic convolution coding with constraint length K = 4, and using the following two generator polynomials:

G0 x  = 1+x+x2+x3

G1 x  = 1+x2+x3

The total number of bits to be encoded, N, is obtained by summing up the size of the PHR (LPHR), the length of the PSDU (LPSDU is equal to the content of the Frame Length field in Figure 114), the number of tail bits(LTAIL), and the number of padding bits (LPAD). N shall be computed as follows:

N=LPHR+LPSDU+LTAIL+LPAD

Note that LPSDU is zero in the case of a mode switch packet.

Immediately after encoding the PHR and PSDU, a termination sequence with length LTAIL = 3 bits shall be inserted into the encoder, as shown in Figure 21.2.6. The tail bits are required to return the encoder to the zero state.

Figure 21.2.6—Data block extension with tail bits prior to coding

The value of the tail bits are dependent on the coding scheme and shall be set as shown in Table 21.2.5.

Table 21.2.5—Tail bit pattern for the RSC and NRNSC encoders

| Memory state (M0–M2) | Tail bits |  |
| --- | --- | --- |
|  | RSC (T0 T1 T2) | NRNSC (T0 T1 T2) |
| 000 | 000 | 000 |
| 001 | 100 | 000 |
| 010 | 110 | 000 |
| 011 | 010 | 000 |
| 100 | 111 | 000 |
| 101 | 011 | 000 |
| 110 | 001 | 000 |
| 111 | 101 | 000 |

When interleaving is used in conjunction with convolutional coding, a padding sequence of LPAD bits shall be further inserted into the encoder immediately after the tail bits. The padding bits are required to fill up the last interleave buffer completely, as described in 20.2.2.3.4. LPAD shall be computed as follows:

LPAD=5, when LPHR+LPSDU8 is odd

LPAD=13, when LPHR+LPSDU8is even

Padding bit patterns should not contain a long series of ‘1’s or ‘0’s. Figure  21.2.6 and Figure  21.2.7 illustrate examples of such patterns. The RSC encoder is shown in Figure 21.2.8, and the NRNSC encoder is shown in Figure 21.2.9.

Figure 21.2.6—An example of extension with padding bits prior to encoding, when (LPHR+LPSDU)/8 is odd

Figure 21.2.7—An example of extension with padding bits prior to encoding, when (LPHR+LPSDU)/8 is even

Figure 21.2.8—The recursive and systematic code (RSC) encoder

Figure 21.2.9—Non-recursive and non-systematic code (NRNSC) encoder

For an input sequence of bits with length N, B={bi, I∊ [0,1,2,…,N-1]} , the ith input bit shall be represented as bi and be fed into memory state M0, M1, and M2 in that order. The tail bits Ti and the pad bits shall be inserted once the encoding of PHR and PSDU is complete. The output sequence S also comprises N code-symbols.

S={s(0), s(1), s(2), ….,s(N-1)}={u01, u00,…,ui1, ui0, ui+11,  ui+10,…, uN-11,uN-10   }

Each code-symbol is denoted by s(i)={ ui1, ui0}, for all i=0,…N-1,  where s(i) is the ith output code-symbol due to the ith input bit ui1 and ui0 indicate the first and second output bits of the convolutional encoder, respectively. The code-symbol s(i) shall precede the code-symbol s(i + 1) and the code bit ui1  shall precede the code bit ui0 .

For the RSC encoder, the first and the second output bits of the encoder shall be generated in the following

way:

ui1=bi⊕(bi−1⊕bi−2⊕bi−3)⊕bi−2⊕bi−3

ui0=bi

where ⊕ stands for modulo-2 addition.

For the NRNSC encoder, the first and the second output bits of the encoder shall be generated in the following way:

ui1=bi⊕bi−2⊕bi−3

ui0=bi⊕bi−1⊕bi−2⊕bi−3

where the “overline” indicates the complement of the modulo-2 addition.

### 21.2.3 Data whitening for CBM GFSK PHY

Support for data whitening is optional.

When data whitening is enabled at the transmitter, the Data Whitening field of the PHR shall be set to one, as described in 20.2.1.3, and the whitened data shall be the exclusive or (XOR) of the PSDU with the PN9 sequence, as described by the following equation:

En=Rn⊕PN9n

Where

En is the whitened bit

Rn is the data bit being whitened

PN9n is the PN9 sequence bit

For each packet transmitted with data whitening enabled, R0 is the first bit of the PSDU and the index n increments for subsequent bits of the PSDU.

For packets received with the Data Whitening field of the PHR set to one, the receiver decodes the scrambled data in the following way:

Rn=REn⊕PN9n

Where

Rn is the PSDU bit after de-whitening

REn is the PSDU bit at the output of the filtered FSK demodulator

The PN generator is defined by the schematic in Figure 21.2.10.

Figure 21.2.10—Schematic of the PN9 sequence generator

The seed in the PN9 generator shall be all ones: “111111111.” The PN9 generator shall be reinitialized to the seed after each packet (either transmit or receive).

The PN9 generator is clocked using the seed as the starting point and enabled after the first clock cycle. For example, the first 30 bits out of the PN9 generator, once it is enabled, would be as follows:

PN9n= 00, 01, 02, 03, 14, 15, 16, 17, 08, 19, 110, 111, 012, 013, 014, 015, 116, 017, 118, 119, 020, 021, 122, 123, 024, 125, 126, 027, 128, 129.

### 21.2.4 CMB GFSK PHY RF requirements

#### 21.2.4.1 Operating frequency range

The CMB GFSK PHY operates in the bands given in Table 66b.

#### 21.2.4.2 Radio frequency tolerance

The single-sided clock frequency tolerance T at the transmitter, in ppm, shall be as follows: +/- 50 ppm.

#### 21.2.4.4 Transmit power spectral density mask

The transmit spectral content is the ratio of the total transmitted out-of-channel power to the total transmitted in-channel power in a given integration bandwidth.

The integration bandwidth shall be equal to1.5×R, where R is the symbol rate, expressed in units of hertz.

Out-of-channel power shall be measured at two offset frequencies relative to the carrier frequency. The offset frequencies M1 and M2 are defined as follows:

M1=1.5×R×(1+h)

M2=3×R×(1+h)

where h is the modulation index for 2-level modulation and three times the modulation index for 4-level modulation.

The transmit spectral content at M1 and M2 shall be less than –25 dB and –35 dB, respectively.

The modulated signal shall use a PN data pattern of 511 bits or longer.

The spectrum analyzer settings for this measurement shall be as follows: the resolution bandwidth is 1 kHz, the video bandwidth is 1 kHz or greater, and the detector is RMS.

In addition,  the transmit spectrum density of CMB GFSK signal shall comply with the PSD mask requirement of Chinese Radio regulations as provided in Annex I and Annex J.

#### 21.2.4.5 Receiver sensitivity

The CMB GFSK PHY receiver sensitivity shall be better than S, where S, for binary modulation, is defined as follows:

S=(S0+10log[RR0])dBm

where

S0 	is –91 without FEC and –97 with FEC

R0 	is 50 kb/s

R 	is the bit rate in kb/s

See 8.1.7 for additional information on receiver sensitivity.

#### 21.2.4.6 Receiver interference rejection

The adjacent designated channels are those on either side of the desired designated channel that are closest in frequency to the desired designated channel. The alternate designated channel is more than one removed from the desired designated channel in the operational frequency band.

The adjacent channel rejection shall be measured as follows: the desired signal shall be a compliant CMB GFSK PHY signal, as defined in 18.1.2, of pseudo-random data at the center frequency of the desired channel. The desired signal is input to the receiver at a level 3 dB above the receiver sensitivity given in 18.1.5.7.

In either the adjacent or the alternate channel, an unmodulated carrier in the center of that channel is input at the following level relative to the level of the desired signal:

— The adjacent channel rejection shall be greater than or equal to 10 dB.

— The alternate channel rejection shall be greater than or equal to 30 dB.

The test shall be performed for only one interfering signal at a time. The receiver shall meet the error rate criteria defined in 8.1.7 under these conditions.

#### 21.2.4.7 TX-to-RX turnaround time

The CMB GFSK PHY shall meet the requirements for TX-to-RX turnaround time as defined in 8.2.1.

#### 21.2.4.8 RX-to-TX turnaround time

The CMB GFSK PHY shall meet the requirements for RX-to-TX turnaround time as defined in 8.2.2.

#### 21.2.4.9 Transmit power

A transmitter shall be capable of transmitting at least –3 dBm. Devices should transmit lower power when

possible in order to reduce interference to other devices and systems.

The maximum transmit power is limited by local regulatory bodies.

#### 21.2.4.10 Receiver maximum input level of desired signal

The CMB GFSK PHY shall have a receiver maximum input level greater than or equal to –20 dBm using the measurement defined in 8.2.4.

#### 21.2.4.11 Receiver ED

The CMB GFSK PHY shall provide the receiver ED measurement as described in 8.2.5.

#### 21.2.4.12 Link quality indicator

The CMB GFSK PHY shall provide the LQI measurement as described in 8.2.6.

#### 21.2.4.13 Clear channel assessment (CCA)

The CMB GFSK PHY shall use one of the CCA methods as described in 8.2.7.

## 21.3 CMB DCSS PHY

The DCSS PHY provides a method to determine the distance between an originator node and a recipient node. The general ranging procedure is shown in Figure 21.3.1.

Figure 21.3.1 – General ranging procedure

A ranging procedure is initiated by the originator node by sending a Range request frame on the communication channel containing initialization data to the recipient node. The recipient node responds on this communication channel with a Range responseframe and either acknowledges or declines the ranging request depending on its abilities. During the ranging period, both nodes transmit and receive signals based on a DCSS modulation and perform phase measurements. At the end of this ranging procedure, the originator retrieves the phase measurement results from the recipient node using the initial communication channel.

Along with its own phase measurement results, the originator node is able to calculate the distance between both nodes. The communication channel used for initiating and configuring a distance measurement and exchanging measurement data uses one of the non-ranging PHYs defined in this standard. Both the DCSS PHY and the non-ranging PHY shall operate in the same frequency band.

#### 21.3.1 DCSS ranging method

To start a ranging sequence, the recipient transmits a start frame to the originator. A Ranging command frame of subtype Range start, as described in 5.3.9a.6, is sent by the ranging recipient to the ranging originator to gain time synchronization between the ranging nodes. Immediately following the end of the transmission and reception of the Range start frame and a period phyPMSetupDuration, the ranging originator and ranging recipient are synchronized by means of dedicated timers. The synchronized timers control the outer and inner loop repetitions, settling times, setup and sampling durations. The originator sets its PLL to the phyPMStartFreq frequency while the recipient sets its PLL to the phyPMStartFreq + phyPMShiftFreq frequency. This frequency offset relates to the intermediate frequency of a low-IF receiver, where the originator receiver operates on an inverse IF position.

First, the recipient transmits a signal and the originator performs a phase measurement. The phase measurement duration at the originator is specified by phyPMSamplingDurationOrig. While both PLLs are running continuously, both devices switch their RX/TX direction and the recipient performs phase measurements on the signal transmitted by the originator.

As shown in Figure 21.3.2, the inner loop is surrounded by an outer loop, while the frequencies of each PLL are stepped by phyPMStepFreq, starting from phyPMStartFreq to phyPMStopFreq on the originator and with an offset of phyPMShiftFreq on the recipient.

Figure 21.3.2 – Inner and outer loop of ranging sequence

Figure 21.3.3 shows the receive and transmit activities of both nodes over time and the corresponding carrier frequencies. The start frame activates the ranging procedure in both nodes according to the PIB attributes that were exchanged previously. Both nodes set their PLLs to the carrier frequency, activate RX and TX mode accordingly, and wait phyPMFreqSettleDurationOrig/Rec. After the setup time of phyPMTxSetupDurationOrig/Rec, the receiver performs the phase measurement. Then both nodes exchange their roles regarding TX and RX. This sequence of transmitting once in both directions is designated as the inner loop and is repeated for each frequency defined by phyPMStartFreq, phyPMStopFreq, and phyPMStepFreq.

This sequence of transmitting once in both directions is designated as the inner loop and is repeated for each frequency defined by phyPMStartFreq, phyPMStopFreq, and phyPMStepFreq, as given in Table 71b.

#### 21.3.2 Phase measurement

Phase measurement is performed on the received carrier signal relative to a reference clock signal derived from the crystal oscillator having the same frequency as specified by phyPMShiftFreq. Therefore, the phase measurement represents the phase shift of two signals with nominally the same frequency.

Figure 21.3.3 – Ranging sequence

# Annex I Chinese MIIT DOC; 423-2005:Technical Requirements for   Micro-power (short- distance) Radio Devices (Translation from Chinese Doc)

## I.1 Specific technical coefficients

### I.1.1 General micro-power (short distance) radio devices

Category A equipment

  - Operation frequency: 9 kHz-190 kHz

Emission limits of magnetic field strength:

9 kHz-50 kHz: 72dBuA/m (10 m Distance) (quasi-peak)

50 kHz-190 kHz: 72dBuA/m (10 m Distance) (3dB decreased in every octave band) (quasi-peak)

Category B equipment

- Operation frequency: 1.7-2.1MHz, 2.2-3.0MHz,

3.1-4.1MHz, 4.2-5.6MHz,

5.7-6.2MHz, 7.3-8.3MHz,

8.4-9.9MHz

- Emission limits of magnetic field strength 10 m distance away from the equipment is no more than 9dBuA/m(quasi-peak).

- Frequency offset tolerance: 100×10-6

- 6dB bandwidth of no more than 200 kHz.

Category C equipment

- Operation frequency: 6.765-6.795MHz, 13.553-13.567MHz, 26.957-27.283MHz

- Emission limits of magnetic field strength 10 m distance away from Tx device is no more than 42dBuA/m (quasi-peak).

- Frequency offset tolerance: 100×10-6

- Spurious radiation emission: For a Tx device operating at 13.553-13.567 MHz frequency, at both ends the emission limits of spurious radiation offsetting 140 kHz, is 9dBuA/m (at 10 m distance, quasi-peak).

Category D equipment

- Operation frequency: 315 kHz-30 MHz except the frequency bands of A, B, C Category equipment.

- Emission of magnetic field strength at 10 m distance away to one device :

315 kHz-1MHz: no more than -5dBuA/m (quasi-peak);

1MHz-30MHz: no more than -15dBuA/m (quasi-peak);

Category E equipment

- Operation frequency: 40.66-40.70MHz

- Transmitting power limit: 10mW (ERP)

- Frequency offset tolerance: 100×10-6

Category F equipment

Category F equipment refers to the short-distance radio devices working at 2400-2483.5MHz, rather than digital cordless telephone, Bluetooth device and wireless LAN equipment

- Operation frequency: 2400-2483.50MHz

- Transmitting power limit: 10mW (EIRP)

- Frequency offset tolerance: 75 kHz

Category G equipment

- Operation frequency: 24.00-24.25GHz

- Transmitting power limit: no more than 20mW (EIRP)

### I.1.2 General wireless remote control devices

General Radio controlled devices are prohibited to apply on toys;

If the operation frequency of wireless remote control device is the same as ones of local voice and television broadcasting station s, this remote control device is prohibited locally.

If the wireless control device interfere local voice and television broadcasting, the device must be immediately stopped until the elimination of these interference, or movement to non-interference frequency bands.

- Operation frequency: 470-566MHz, 614-787MHz

- Transmitting power limit: 5mW (ERP)

- Occupied bandwidth: no greater than 1.0MHz

### I.1.3 Wireless MIC, civilian, radio, measuring instruments and

Wireless MIC are applied for audio-visual training of education and cultural department, or disabled hearing aid at cinemas, concert halls, conference rooms and other public places, or a small radio device at tourist areas.

For less 5 sec duty cycle data transmission, 470-510 MHz is the operation frequency for civilian radio measuring instruments.

If above wireless device interferes local voice and television broadcasting, the device must be immediately stopped until the elimination of these interference, or movement to non-interference frequency bands.

For protecting biomedical telemetry equipment from the interference, the wireless MIC is not permitted in the hospital, which should be specified in a product manual of wireless MIC.

- Operation frequency and transmitting power:

  - Operation frequency: 87-108MHz. Transmitting power limit: 3mW (ERP)

  - Operation frequency: 75.4-76.0MHz, 84-87MHz. Transmitting power limit: 10mW (ERP)

  - Operation frequency: 189.9-223.0MHz. Transmitting power limit: 10mW (ERP)

  - Operation frequency: 470-510MHz, 630-787MHz. Transmitting power limit: 50Mw (ERP)

- Occupied bandwidth: no greater than 200kHz

- Frequency offset  tolerance: 100×10-6

### I.1.4 Biomedical telemetry device

Radio transmitting devices for the transmission of human or animal physiological phenomenon measured signals, used only in hospital or medical research institution. It should not interfere with radio astronomy services.

- Operation frequency: 174-216MHz, 407-425MHz, 608-630MHz

- Transmitting power limit: 10mW (ERP)

- Frequency offset tolerance: 100×10-6

### I.1.5 Analog cordless telephone

  - Operation frequency:

| Channel number | Panel Frequency (MHz) | Cell phone Frequency (MHz) |
| --- | --- | --- |
| 1 | 45.000 | 48.000 |
| 2 | 45.025 | 48.025 |
| 3 | 45.050 | 48.050 |
| 4 | 45.075 | 48.075 |
| 5 | 45.100 | 48.100 |
| 6 | 45.125 | 48.125 |
| 7 | 45.150 | 48.150 |
| 8 | 45.175 | 48.175 |
| 9 | 45.200 | 48.200 |
| 10 | 45.225 | 48.225 |
| 11 | 45.250 | 48.250 |
| 12 | 45.275 | 48.275 |
| 13 | 45.300 | 48.300 |
| 14 | 45.325 | 48.325 |
| 15 | 45.350 | 48.350 |
| 16 | 45.375 | 48.375 |
| 17 | 45.400 | 48.400 |
| 18 | 45.425 | 48.425 |
| 19 | 45.450 | 48.450 |
| 20 | 45.475 | 48.475 |

  - Plane and cell phone transmitting power limit: 20mW (ERP)

  - Maximum frequency offset: 5KHz

  - Occupied bandwidth: no greater than 16 kHz

  - Frequency offset tolerance: 1.8 kHz

  - Antenna must be permanently fixed to phone or cell phone, and its length should not be more than 1 meters.

### I.1.6   2.4 GHz band  digital cordless telephone

- Operation frequency: 2400-2483.5MHz

- Average EIRP limit: 25mW

- Frequency tolerance: 20×10-6

- Digital cordless telephony whose operation frequency is 2400-2483.5MHz must employ FH mode, and the number of hopping channels is at least 75.

- The duty period one channel must be no more than 0.4 second.

### I.1.7  Dedicated wireless rremote control device for control device for crane and mechanical transmission

Electromagnetic environment test must be carried out before device installation, in order to avoid interference or interfering with the operation of other similar devices, even resulting in accidents.

When dedicated wireless remote control device receives harmful interference, should stop using it immediately until the elimination of interference or adjustment to non-interference frequency.

For protection of radio astronomy service of Beijing Observatory and Ping Tang, Guizhou (Province) province, it is prohibited that devices whose operation frequency is within following range:

- Operation frequency:

230.700MHz, 223.700MHz, 230.975MHz, 223.975MHz,

231.600MHz, 224.600MHz, 232.325MHz, 225.325MHz,

230.100MHz, 223.100MHz, 232.025MHz, 225.025MHz

- Transmitting power limit: 20mW (ERP)

- Occupied bandwidth: no greater than 16 kHz

- Frequency tolerance: 4×10-6

### I.1.8 Wireless industry remote control device

The device operation is limited at industrial plants (or buildings).

- Operation frequency

418.950MHz, 418.975MHz, 419.000MHz, 419.025MHz,

419.050MHz, 419.075MHz, 419.100MHz, 419.125MHz,

419.150MHz, 419.175MHz, 419.200MHz, 419.250MHz, 419.275MHz

- Transmitting power limit: 20mW (ERP)

- Occupied bandwidth: no greater than 16 kHz

- Frequency offset tolerance: 4×10-6

### I.1.9   Wireless data transmission devices

For protection of radio astronomy service of Beijing Observatory and Ping Tang, Guizhou (Province) province, it is prohibited that devices whose operation frequency

The operation of wireless data transmission devices is limited in buildings.

  - Operation  frequency:

228.050MHz, 228.100MHz, 228.200MHz, 228.275MHz,

228.425MHz, 228.575MHz, 228.600MHz, 228.800MHz,

223.150MHz/230.150MHz, 223.250MHz/230.250MHz

223.275MHz/230.275MHz, 224.050MHz/231.050MHz

223.350MHz/230.350MHz, 224.250MHz/231.250MHz

  - Transmitting power limit: 10mW (ERP)

  - Occupied bandwidth: no greater than 16 kHz

  - Frequency offset tolerance: 4×10-6

### I.1.10 Dedicated wwireless transmission  equipment for electronic hanging scale

- Operation frequency and occupied bandwidth:

  - Operation frequency: 223.300MHz, 224.900MHz, 230.050MHz, 233.050MHz, 234.050MHz. Occupied bandwidth: no greater than 50 kHz

  - Operation frequency: 450.0125MHz, 450.0625MHz, 450.1125MHz, 450.1625MHz, 450.2125MHz. Occupied bandwidth: no greater than 20 kHz

- Effective radiation power limit: 50mW (ERP)

- Frequency offset tolerance: 4×10-6

### I.1.11Wireless  control device for various  types of civilian devices

These radio control devices are prohibited for toys and models.

- Operation frequency: 314-316MHz, 430-432MHz, 433.00-434.79MHz.  Transmitting power limit: 10mW (ERP). Occupied bandwidth: no greater than 400 kHz

- Operation frequency: 779-787MHz. Transmitting power limit: 10mW (ERP)

### I.1.12 Wireless remote control devices for toys and models control devices for toys and models

Radio controlled devices are limited for aircraft model, surface model of the vessel, the car model and other non-manned models as well as toys, but can’t be used for other kinds of wireless equipment.

- Operation frequency

26-27MHz band	operation on vessel and the car models:

26.975MHz, 26.995MHz, 27.025MHz, 27.045MHz, 27.075MHz,

27.095MHz, 27.125MHz, 27.145MHz, 27.175MHz, 27.195MHz,

27.225MHz, 27.255MHz

40MHz band		operation on vessel and the car models:

40.61MHz, 40.63MHz, 40.65MHz, 40.67MHz, 40.69MHz,

40.71MHz, 40.73MHz, 40.75MHz

40MHz band 		operation on aircraft model:

40.77MHz, 40.79MHz, 40.81MHz, 40.83MHz, 40.85MHz

72MHz band		operation on aircraft model:

72.13MHz, 72.15MHz, 72.17MHz, 72.19MHz, 72.21MHz,

72.79MHz, 72.81MHz, 72.83MHz, 72.85MHz, 72.87MHz

- Transmitting power limit: 750mW (ERP)

- Occupied bandwidth: for the frequency band of 26-27MHz, no greater than 8 kHz. For the frequency band of 40MHz and 72MHz, no greater than 20 kHz

- Frequency offset tolerance: for the frequency band of 26-27MHz, no greater than 100×10-6.  for the frequency band of 40MHz and 72MHz,no greater than 30×10-6

- Remote control devices for models must be one-way controller, and radio transmitting equipment should not be set on a model.

- For the requirements of electromagnetic environment, various types of remote control for models shall not be used in the range locating in a radius of 5000 m circle, around airport runway

- The remote control devices of models shall not transmit speech communication signals.

- After the release of radio control by the relevant state departments, remote control devices of models should not be used at a specific time, region.

### I.1.13 Interphone (walk-talk)

The usage of interphone must follow the radio management regulations relevant to interphone.

- Operation frequency (Unit: MHz)

409.7500, 409.7625, 409.7750, 409.7875, 409.8000, 409.8125,

409.8250, 409.8375, 409.8500, 409.8625, 409.8750, 409.8875,

409.9000, 409.9125, 409.9250, 409.9375, 409.9500, 409.9625,

409.9750, 409.9875

- Modulation: F3E

- Transmitting power limit: 500mW (ERP)

- Frequency offset tolerance: 5×10-6

- Channel spacing: 12.5 kHz

### I.1.14 Radar for vehicle location

- Operation frequency: 76-77GHz

- Peak EIRP limit: 55dBm

## I.2 General requirements

  - Frequency Ranges for Measurement of Spurious radiation emission

| Frequency range | Spurious radiation emission measurement frequency range: |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| 9kHz-100MHz | 9kHz | 1GHz |
| 100MHz-600MHz | 30MHz | 10 harmonic |
| 600MHz-2.5GHz | 30MHz | 12.75GHz |
| 2.5-13GHz | 30MHz | 26GHz |
| >13GHz | 30MHz | 2 harmonic |

  - Spurious radiation emission limits (the demarcation between spurious radiation and out of band radiation is the bandwidth, which is  ±2.5 multiples of  operation channel band on carrier frequency)

    - Transmitter in transmission state at the maximum power

| Frequency range | Test bandwidth | Limit | Detection method |
| --- | --- | --- | --- |
| 9kHz-150kHz | 200Hz (6dB) | 27dBuA/m (10 meters) (3 dB decreased per octave) | Quasi-peak |
| 150kHz-10MHz | 9kHz (6dB) |  | Quasi-peak |
| 10MHz-30MHz | 9kHz (6dB) | －3.5dBuA/m (10 meters) | Quasi-peak |
| 30MHz-1GHz | 100kHz (3dB) | －36dBm | Effective value |
| 1GHz-40GHz | 1MHz (3dB) | －30dBm | Effective value |
| >40GHz | 1MHz (3dB) | －20dBm | Effective value |

    - Transmitter in standby or idle state

| Frequency range | Test bandwidth | Limit | Detection method |
| --- | --- | --- | --- |
| 9kHz-150kHz | 200Hz(6dB) | 6dBuA/m (10 meters) (3 dB decreased per octave) | Quasi-peak |
| 150kHz-10MHz | 9kHz(6dB) |  |  |
| 10MHz-30MHz | 9kHz(6dB) | －24.5dBuA/m (10 meters) | Quasi-peak |
| 30MHz-1GHz | 100kHz(3dB) | －47dBm | Quasi-peak |
| >1GHz | 1MHz(3dB) |  |  |

NOTE---Magnetic field measurement is performed at open area and radiation power measurement is performed at full anechoic chamber.

For the equipment working frequency below 30 MHz, the emission state can be set to single frequency (SF) emission.

If specific technical specification does not match to general requirements, the specific technical specification is adopted.

  - Spurious radiation emission, in 48.5MHz-72.5MHz, 76MHz-108MHz, 167MHz-223MHz, 470MHz-566MHz or 606MHz-798MHz bands, should be less －54dBm.

  - The conduction harassment emission of power port, signal ports and telecommunication ports should be accordance with GB9254-1998 standard.

  - For the frequency bands above 30 MHz, the radiation power at the upper/lower limits of the specified working frequency is no more than －80dBm/Hz (EIRP). For the frequency bands below 30 MHz, the upper and lower boundaries of any working channel bandwidth (99% energy) can’t exceed the permitted specified ranges.

The manufacturers of micro-power (short distance) radio device should declare the extreme environmental conditions in normal usage, and in such conditions transmitting power and frequency tolerance should meet the requirements of this R

# Annex J Further  explanation for Chinese radio regulation on approved bands for medical signal transmission

## J.1 The spurious radiation emission on different bands

- Transmitter in transmission state at the maximum power the spurious power level should be:

| Frequency range | Test bandwidth | Limit | Detection method |
| --- | --- | --- | --- |
| 30MHz-1GHz | 100kHz (3dB) | －36dBm | Effective value |
| 1GHz-40GHz | 1MHz (3dB) | －30dBm | Effective value |
| >40GHz | 1MHz (3dB) | －20dBm | Effective value |

Effective value is equal to average value.

- Transmitter in standby or idle state, the spurious power level should be:

| Frequency range | Test bandwidth | Limit | Detection method |
| --- | --- | --- | --- |
| 30MHz-1GHz | 100kHz(3dB) | －47dBm | Quasi-peak |
| >1GHz | 1MHz(3dB) |  |  |

- The spurious radiation emission of any Transmitter, in 48.5MHz-72.5MHz, 76MHz-108MHz, 167MHz-223MHz, 470MHz-566MHz or 606MHz-798MHz bands, should be less than -54dBm/100KHz

The spurious radiation limitations are drawn in Figure J1.

Figure J.1

## J.2  The Tx spectrum mask on approved bands for medical signal transmission

  - On 174-216MHz and 608-630MHz Bands

Figure J.2

  - On 407-425MHz and 608-630MHz Bands

Figure J.3

  - CHBW: Channel Bandwidth.

  - 2.5 x CHBW:

    - Spurious radiation emission limits (the demarcation between spurious radiation and out of band radiation is the bandwidth, which is  ±2.5 multiple of  operation channel band on carrier frequency)

## J.3  The radio power at upper/lower limits of operation bands

- In any Transmitter state (max, idle or standby), for the frequency bands above 30 MHz, the radiation power at the upper/lower limits of the specified working frequency is no more than －80dBm/Hz (EIRP). -80dBm/Hz (EIRP)  is equal to -82.2dBm/Hz (ERP)

- The upper/lower limits of approved bands for medical signal transmission are {176MHz, 214 MHz }, {407 MHz, 425 MHz } and {608 MHz, 630 MHz } for Chinese medical bands.

Figure J.4

Figure J.5

# Annex K Interferences and  their models on approved models on approved bands for medical signal transmission

Table K.1 lists the possible  interference on Chinese approved bands for medical signals

Figure K.1, Interference on 174-216, 407-425 and 608-630 MHz Bands

## K.1 Inference on 176-214 MHZ channel

On this band, the major interference are sourced from

- VHF Radio and TV

- Wireless Microphone

The inference signal model and spectrum of VHF Radio and TV, including its Chinese standard numbers are shown in Figure K.2:

|  | Channel | China Standard |
| --- | --- | --- |
| Digital radio and TV | DS-6：174-175MHz DS-7：175-183MHz DS-8：183-191MHz DS-9：191-199MHz DS-100：199-207MHz DS-11：207-215MHz DS-12：215-216MHz | DTMB(OFDM) GB 20600—2006 Framing structure, Channel coding and modulation for digital television terrestrial broadcasting system |
| Analog radio and TV |  | CMMB(OFDM) GY/T 220.1-2006 Mobile Multimedia Broadcasting Part 1：Framing Structure Channel Coding and Modulation for Broadcasting Channel |
| Remark: The analog radio and TV will stop in 2015. |  |  |

Figure K.2, Interference spectrum and related China standards

The model of microphone is shown in Figure K.3 and K.4,

Figure K.3 is reference from FCC occupied bandwidth limit section 74.861 (e) (6)

Figure K.4 is reference from ETSI EN 300 422-1 V1.2.2 (2000-08)

Figure K.3 FCC OCCUPIED BANDWIDTH MASK

Figure K.4 Spectral mask - Normalized to channel bandwidth

## K.2 Inference on 407-425 MHZ channel

| Interphone | Different Frequency Bands | Dedicated interphone (Pt<30W) | 137-144、146-167、403-409.75、409.9875-423.5MHz |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Public interphone (Pt<500mW) | 409.7500-409.9875MHz | Analog | Channel number:20 Operation frequency:409.7500-409.9875MHz Channel spacing:12.5kHz Modulation:8K5F3E |
|  |  |  |  | Digital | Channel spacing:12.5kHz 6.25kHz multi-access mode : FDMA/TDMA |
|  | Individual Technology | Analog interphone |  |  |  |
|  |  | Digital interphone Standards: | DMR （base on TDMA）、DPMR （base on FDMA）、DSRR ,P25 standard |  |  |

Figure K.5 Inference on 407-425 MHZ band

The major interference on this band are sourced from

- Interphone (Walkie-Talkie)

The model of interphone is described as:

Baseband data in time domain

Carrier

modulation

RF signal

Baseband 4FSK

Modulation

Baseband signal

- This baseband data is based on frame format which set by DPMR standard.

Math model

x(t) is the baseband signal though the 4FSK

| Information bits |  | Symbol | 4FSK deviation |
| --- | --- | --- | --- |
| Bit 1 | Bit 0 |  |  |
| 0 | 1 | +3 | +1.944kHz |
| 0 | 0 | +1 | +0.648kHz |
| 1 | 0 | -1 | -0.648kHz |
| 1 | 1 | -3 | -1.944kHz |

Each of the four information bits is mapping with a cosine signal as ,Δfis 4FSK deviation in the above figure, is selected as 30kHz here according to the DPMR standard

## K.3 Inferences on 608-630 MHZ band

The major interferences on this band are sourced from:

  - UHF Radio and TV

  - General Wireless Remote Control Devices

The model of digital TV is described in the Figure K6

|  | Channel | China Standard |
| --- | --- | --- |
| Digital radio and TV | DS-25：608-614MHz DS-26：614-622MHz DS-27：622-630MHz | DTMB(OFDM) GB 20600—2006 Framing structure, Channel coding and modulation for digital television terrestrial broadcasting system |
| Analog radio and TV |  | CMMB(OFDM) GY/T 220.1-2006 Mobile Multimedia Broadcasting Part 1：Framing Structure Channel Coding and Modulation for Broadcasting Channel |
| Remark: The analog radio and TV will stop in 2015. |  |  |

Figure K.6, Interference spectrum and related China standards

Insert after Annex U the following new annex (Annex V):

Annex V

(informative)

Description of phase measurement result storage and data exchange

V.1 General

This annex describes the logical data handling of phase measurement (PM) results in a ranging device.

V.2 Logical data handling of compressed PM results

The total number of PM values depends on the PIB attributes phyPMOuterLoopRepetitions and phyPMInnerLoopRepetitions.

The logical data handling of compressed PM data should be treated as shown in Figure V.1. The example assumes an outer loop count (OLC) of two (phyPMOuterLoopRepetitions = 1) and an inner loop count (ILC) of three (phyPMInnerLoopRepetitions = 2). The measurement is performed over N frequencies.

Figure V.1—Example of logical data handling of compressed PM data

V.3 Logical data handling of uncompressed PM results

In the case of uncompressed PM results, several PM values (phyPMCurrNumbersOfPhaseMeasPerFreq) are stored per utilized measurement frequency. The total number of PM values depends on the variables phyPMOuterLoopRepetitions, phyPMInnerLoopRepetitions, and phyPMCurrNumbersOfPhaseMeasPerFreq.

The logical data handling of uncompressed PM data shall be treated as shown in Figure V.2.

The example assumes an OLC of two (phyPMOuterLoopRepetitions = 1) and an ILC of three (phyPMInnerLoopRepetitions = 2). The measurement is performed over N frequencies. A number M PM values are stored per frequency.

Figure V.2—Example of logical data handling of uncompressed PM data

V.4 Retrieval of compressed PM data

The data exchange procedure of compressed PM data, using Range result request and Range result confirm frames, is shown in Figure V.3. The example uses PM parameters as given in Table V.1. These parameters are selected for better representation only; actual PM parameters are different.

Range result request frames are sent from the ranging originator to the recipient, and Range result confirm frames are sent from the ranging recipient to the originator.

Figure V.3—Example of retrieval of compressed PM data

Table V.1—Example parameters for compressed PM data exchange

| Parameter | Example value | Comment |
| --- | --- | --- |
| phyPMStartFreq | 2403MHz |  |
| phyPMStopFreq | 2413MHz |  |
| phyPMStepFreq | 1MHz |  |
| Resulting number of used frequencies N | 11 | (2403MHz – 2413MHz / 1MHz) + 1 |
| phyPMOuterLoopRepetitions | 1 |  |
| phyPMInnerLoopRepetitions | 2 |  |
| Maximum resulting OLC | 2 | phyPMOuterLoopRepetitions + 1 |
| Maximum resulting ILC | 3 | phyPMInnerLoopRepetitions + 1 |
| macPMSuppressResultCompression | FALSE |  |
| phyPMCurrNumbersOfPhaseMeasPerFreq M | n/a | Value ignored |
| Number of Included Range Result Data Values | 8 | This value is set by the recipient. It is usually as large as possible, in order to transfer as much PM data as possible using one Range result confirm frame. |

V.5 Retrieval of uncompressed PM data

The data exchange procedure for uncompressed PM data, using Range result request and Range result confirm frames, is shown in Figure V.4. The example uses the PM parameters given in Table V.2. These parameters are selected for better representation only; actual PM parameters are different.

Range result request frames are sent from the ranging originator to the recipient, and Range result confirm frames are sent from the ranging recipient to the originator.

Figure V.4—Example of retrieval of uncompressed PM data

Table V.2—Example parameters for uncompressed PM data exchange

| Parameter | Example value | Comment |
| --- | --- | --- |
| phyPMStartFreq | 2403MHz |  |
| phyPMStopFreq | 2413MHz |  |
| phyPMStepFreq | 1MHz |  |
| Resulting number of used frequencies N | 11 | (2403MHz – 2413MHz / 1MHz) + 1 |
| phyPMOuterLoopRepetitions | 1 |  |
| phyPMInnerLoopRepetitions | 2 |  |
| Maximum resulting OLC | 2 | phyPMOuterLoopRepetitions + 1 |
| Maximum resulting ILC | 3 | phyPMInnerLoopRepetitions + 1 |
| macPMSuppressResultCompression | TRUE |  |
| phyPMCurrNumbersOfPhaseMeasPerFreq M | 4 |  |
| Number of Included Range Result Data Values | 16 | This value is set by the recipient. It is usually as large as possible, in order to transfer as much PM data as possible using one Range result confirm frame. |