IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Comment and resolution for system level simulations in TGD |
| Date Submitted | September 2013 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Re: | In response to Technical Guidance Document contributions |
| Abstract | Comments to TGD |
| Purpose | Discussion |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1.1	Comment	3

## Comment

There is an issue with the TGD parameters for system level simulations, in particular communication. The maximum distance for deployment scenarios is set to 50 m (clause 9.4.3.2). However, if you use the receiver’s parameters in the TGD:

Maximum transmission power: 20 dBm

Antenna gains: Gt=2.5 dBi and Gr=2.5 dBi

Fade margin: 8 dB

Receiver’s noise figure:  NF=7 dB

Noise density:  kT0=-174 dBm/Hz

Data rate R=1 MHz up to 10 MHz

Using the path loss under clause 2.2.6 channel model document, you can obtain

Es/N0 for a given distance link d, equation 6 in the TGD with Pi=0 (no interference).

Using such Es/N0 in the curves provided in the spreadsheet 15-13-0058-01-0008-candidate-link-curve-snr-per.xlsx, you will find that links with distances up to 50 m, the PER is zero for BPSK, QPSK and even 16QAM and 1/2 rate CC. At maximum distance (d=50 m), the supposed modulation to be employed is BPSK.

I consider these will give unrealistic results for MAC proposals.

Recommended resolution:

A quick fix is to increase the maximum distance to 60m.