IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | The process of interleaving for TVWS-OFDM |  |
| Date Submitted | [13 September 2013] |  |
| Source | [Cheolho SHIN, Byoung Hak kim and Sangsung CHOI] [ETRI] | Voice: [ ] Fax: [ ] E-mail: [chshin@etri.re.kr] |
| Re: |  |  |
| Abstract | Working doc of TVWS-OFDM Comment Resolutions for Comment ID 303 |  |
| Purpose | Comment Resolution |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Comment ID 303 for SB

The structure of the interleaver is not clear, the process of interleaving should be illustrated.

Proposed Change

Insert the figure to show the corresponding process of interleaving.

20.2.3.4 Interleave

The interleaving process consists of two permutations. The index of the coded bit before the first permutation shall be denoted as k; i shall be the index after the first and before the second permutation; and j shall be the index after the second permutation, just prior to modulation mapping. The coded bits are written at the index given by j, and read out sequentially. The index i is defined as follows:

i = (Ncbps / Nrow) ⅹk mod(Nrow) + floor(k / Nrow)

where

Ncbps is the number of coded bits per symbol.

k =0,1,2,...,(Ncbps - 1)

Nrow is 20

The process of interleaving for the first permutation is illustrated in the following Figure 180.

Figure 180 -The process of interleaving for the first permutation

The index j is defined as follows:

j = s ⅹ floor(i / s) + [i + Ncbps - floor((Nrow ⅹ i) / Ncbps)]mod(s)

where

Ncbps is the number of coded bits per symbol.

k =0,1,2,...,(Ncbps - 1)

Nrow is 20

and

s = max(Nbpsc/2, 1)

where Nbpsc is the number of bits per subcarrier, and has the values 1, 2, and 4 for BPSK, QPSK, and 16-QAM, respectively. Ncbps is defined as follows: 100 bits for BPSK, 200 bits for QPSK, and 400 bits for 16-QAM