IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Sponsor Ballot CID 353 Comments Resolution |  |
| Date Submitted | 23 Sept, 2013 |  |
| Source | [Alina Lu Liru, Keiichi Mizutani, Chin Sean Sum, Fumihide Kojima, Hiroshi Harada(NICT)] | Voice: [[+65-67711006] Fax: [[+65-67795753] E-mail: [liru@nict.com.sg] |
| Re: | Submission for resolution of TVWS-NB-OFDM comment in IEEE 802.15.4m Sponsor Ballot |  |
| Abstract | Proposed comment resolution for TG4m TVWS-NB-OFDM Sponsor Ballot comments |  |
| Purpose | To address sponsor ballot comments for TG4m TVWS-NB-OFDM |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

CID 353, Comment: Rearrange the order of sub-clauses under 20.3.3 to follow the order of reference modulator blocks as shown in Figure 185 for users' easy understanding

Proposed Change. Change accordingly

Proposed resolution. AP

Change the following texts in draft document.

Change page 93 line 31 to the following sentence.

Subclauses 20.3.3.2.1 through 20.3.3.2.5 describe scrambling, outer encoding, inner encoding, pad bit insertion and spreader.

Move subclause 20.3.3.2.4 to subclause 20.3.3.2.1.

20.3.3.2.1 Scrambler and scrambler seed

The input to the scrambler is the data bits followed by tail bits and then pad bits. The scrambler uses a PN9 sequence that is shown in Figure 102. The PN9 scrambler is initialized by the scrambling seed specified by 9 bits in the PHR. The leftmost value of the scrambling seed is placed into the leftmost delay element in Figure 102.

Change subclause number for the following titles.

20.3.3.2.2 Outer encoding

20.3.3.2.3 Inner encoding

20.3.3.2.4 Pad bit insertion

Change page 97 line 33 reference to the following highlighted subclause number.

20.3.3.2.5 Spreader

The spreader only applies to the PHR. The 44 PHR bits are encoded using the rate-1/2 recursive convolutional code as described in 20.3.3.2.3 to create an 88-bit sequence (d0, d1, d2, … d87). The 88-bit sequence is spread by using a spreading sequence [1111] to generate the 352-bit sequence (d0, d0, d0, d0,

d1, d1, d1, d1, d2, d2, d2, d2, …, d87, d87, d87, d87). The 352-bit sequence is then interleaved as described in 20.3.3.3.1 and mapped as BPSK as described in 20.3.3.3.2. The resulting values are interleaved as described in 20.3.3.4, pilots are inserted as described in 20.3.3.5, and then modulated as an OFDM symbol.