IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Comment Resolution for CID 183 |  |
| Date Submitted | 1 October, 2013 |  |
| Source | Alina Lu Liru, Keiichi Mizutani, Chin Sean Sum, Fumihide Kojima, Ming-Tuo Zhou, Verotiana Rabarijaona, , Hiroshi Harada (NICT) | E-Mail: [liru@nict.com.sg; mingtuo@nict.com.sg; sum@nict.go.jp; f-kojima@nict.go.jp; rverotiana@nict.go.jp; mizk@nict.go.jp; harada@nict.go.jp] |
| Re: |  |  |
| Abstract | [Comment resolution for sponsor ballot] |  |
| Purpose | [to provide comment resolution to sponsor ballot] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

CID 183: Channel Aggregation is mentioned in "Table 4ia--TVWS PHY operating mode description field", and in Clause 20 on the PHY. I do not find any mention of Channel Aggregation in 6.2 in any of the MLME definitions. Is channel aggregation completely autonomous? Shouldn't there be a means to manage it through MLME or in the PIB?

Suggested change: Add management capability for Channel Aggregation to MLME (perhaps in MLME.Start) and in the PHY PIB to enable the initialization of this parameter.

Proposed resolution: AP

Channel aggregation is meant for subchannels within 6M or 8M available TVWS channel. Hence it does not require to obtain multiple TVWS channel information. Suggest to add PIB attribute as shown below.

Add following item at the end of Table 71—PHY PIB attributes:

| Attribute | Type | Valid range | Description |
| --- | --- | --- | --- |
| phyTvwsChannelAggregation | Boonlean | TURE,FALSE | A value of TRUE indicates that channel aggregation is enabled. A value of FALSE indicates that channel aggregation is disabled. This attribute is only valid for the TVWS-NB-OFDM PHYs. |

Replace the following text in 20.3.4

The use of channel aggregation is controlled by the PIB atTable 217 shows channel aggregation phyTvwsChannelAggregation. When channel aggregation is enabled. At least one of the bandwidths, 6 MHz or 8 MHz, shall be supported. The maximal number of aggregated channels depends on the availability of channel bandwidth..