IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | NICT PHY proposal document (short version) |
| Date Submitted | November, 2013 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Re: | In response to Call for Proposals to TG8 |
| Abstract | Text for NICT PHY proposal (short version) |
| Purpose | In response to Call for Proposals to TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1	h	3

The enumeration is based on the PAC framework document DCN 328r4.

6

NICT PHY architecture

6.1 Channelization

Frequency bands of operation are: Sub-GHz, 2.4 GHz and 5.7 GHz bands.

Channelization of 5.7 GHz band

  - Such frequency band ranges from 5.725 GHz to 5.875 GHz, which is divided into 14 channels of 10 MHz. By regulation, the maximum transmit power at the input antenna is 1 W. The central frequencies are given by

fc = 5735 MHz + 10n       for n = 0, 1, ..., 13

Channelization of 2.4 GHz band

  - Such frequency band ranges from 2.4 GHz to 2.5 GHz, which is divided into 9 channels of 10 MHz. By regulation, the maximum transmit power at the input antenna is 1 W. The central frequencies are given by

fc = 2410 MHz + 10n          for n = 0, 1, ..., 8

  - The basic channelization by regulations in Japan is summarized in the following table:

  - 1bandwidth rule tolerance: 200 n  kHz, where n=1,2,3,4,5.

  - 2bandwidth rule tolerance: 100 n  kHz, where n=1,2,3,4,5.

Proposed channelization

6.2  Duplex

6.2.1 Frame structure in FDD mode

One slot contains 7 DTF-S OFDM or OFDM symbols.

Tslot=7680Ts=0.5 msec

6.2.1 Frame structure in TDD mode

where L1=PD1 transmits and PD2 receives, L2=PD2 transmits and PD1 receives, S-L1=synchronization for L1, RS-L1=reference signals for L1, RA=random access or channel sounding for MIMO or beamforming, GP=guard period.

The GP is computed as GP=(73+1024)Ts=71.42 µsec and satisfies

Tp+Tdec+Tsw+Tcomp+Tp=GP

  - Tdec=time to detect the last symbol

  - Tsw=time to switch from Rx to Tx or vice-versa

  - Tcomp=compensation time to align to GP

where Tp=1 Km/3x108m/s=3.3 µsec (worst case).

The TDD frame configuration is given in the table

6.3 Multiplex scheme

Time-frequency multiplexing

6.4 Frame structure

6.4.1 Frame structure for discovery

We propose to use a discovery preamble  (DP) based on a ZC sequence and a discovery resource block (DRB) from a modified (DTF-S) OFDM signal.

  - The DP and DRB formed the Discovery Signal (DS).

Moreover, we propose to use one channel (from the proposed channelization for sub-GHz, 2.4 GHz and 5.7 GHz bands) for only discovery of devices.

  - PAC devices can either transmit or receive the DS in this unique channel asynchronously.

The discovery signal (DS) sent over a discovery shared channel (DSCH):

The DS consists of a preamble sequence plus a DRB formed by Nfs frequency slots and Nts time slots.

Once synchronized, a receiver knows the location of the discovery resource block (DRS) to scan for possible peers or to pick time-frequency slots to transmit its DS.

The proposed ZC sequence for initial synchronization during discovery (or communication mode) is a ZC sequence with the following parameters:

  - Sequence length N = 63, relative prime r = 62 and q = 0.

Such ZC sequence is mapped onto 62 sub-carriers

  - The first subcarrier and the DC subcarrier are empty.

  - Of course, several repetitions may be transmitted.

For discovery, the nth symbol transmitted over the kth subcarrier is given by

  - for l=-L+1,,…,0,1,…,N-1 and L the CP

The DS is transmitted with a predefined duty-cycle. From upper layers, terminals pick time-frequency slots in the DRB to transmit the DS.

A set of 126 symbols are transmitted per time slot:

  - such frame contains information about a given peer (peer ID, service ID, application ID, etc.) and which channel is used for association (different from the SDCH).

The number of frequency slots Nfs =1024 (IFFT size) and the number of time slots Nts is chosen as 20.

Consequently, the DRB can support up to NfsxNts=20,480 users for discovery per Group.

The discovery data is formatted as illustrated

Append 57 bits for user ID, device ID, Group ID, etc.

Append 6 bits from CRC-6-ITU error detection code

Append 63 bits from shorten BCH(126,63) code

6.4.2 Data frame structure

The physical layer protocol data unit (PPDU) is formed by concatenating synchronization header (SHR), Discovery header (DIS), physical layer header (PHR) and physical layer service data unit (PSDU)

Reference signals for demodulation/equalization are embedded in the PSDU.

The MAC protocol data unit (MPDU) is passed to the PHY. Such data is encoded by QC-LDPC codes.

6.5 Modulation and coding scheme

6.5.1 FEC

Quasi-cyclic low density parity check codes.

HcT=0Quasi-cyclic LDPC codes are systematic, linear codes satisfying

where c  is the codeword and H is the parity check matrix.

H is constructed from the prototype matriz Hp by replacing each entry [Hp]i,j with either a cyclic shift matrix Pc, identity or null matrices of size ZxZ. The final size of H is MpZxNpZ.

Details of encoding can be found in document with DCN 369r1.

QC-LDPC parameters for different coding rates to enable link adaptation:

The prototype matrices Hp for different coding rates can be found in document with DCN 369r1.

bi6.5.2 Modulation

In the modulation mapper, the scrambled coded bits    for i=0,1,…,n-1 are modulated with either BSPK, QPSK or 16QAM modulations, resulting in the a block of complex modulation symbols    di  for i=0,1,…,Nsym-1, where di=I+jQ

BPSK mapping

| bi | I | Q |
| --- | --- | --- |
| 0 | 1/√2 | 1/√2 |
| 1 | -1/√2 | -1/√2 |

QPSK mapping

| bi,bi+1 | I | Q |
| --- | --- | --- |
| 00 | 1/√2 | 1/√2 |
| 01 | 1/√2 | -1/√2 |
| 10 | -1/√2 | 1/√2 |
| 11 | -1/√2 | -1/√2 |

16QAM mapping

| bi,bi+1,bi+2,bi+3 | I | Q |
| --- | --- | --- |
| 0000 | 1/√10 | 1/√10 |
| 0001 | 1/√10 | 3/√10 |
| 0010 | 3/√10 | 1/√10 |
| 0011 | 3/√10 | 3/√10 |
| 0100 | 1/√10 | -1/√10 |
| 0101 | 1/√10 | -3/√10 |
| 0110 | 3/√10 | -1/√10 |
| 0111 | 3/√10 | -3/√10 |
| 1000 | -1/√10 | 1/√10 |
| 1001 | -1/√10 | 3/√10 |
| 1010 | -3/√10 | 1/√10 |
| 1011 | -3/√10 | 3/√10 |
| 1100 | -1/√10 | -1/√10 |
| 1101 | -1/√10 | -3/√10 |
| 1110 | -3/√10 | -1/√10 |
| 1111 | -3/√10 | -3/√10 |

6.5.2.1 DFT-S OFDM or OFDM

The Parameters are given by

Δfis constant and equal to 15 KHz.

Ts=1Δf1024=65.10 nsecMaximum FFT size M=1024

Sampling time

Rc=1Ts=15.36 MHzTiming based on a common clock at

6.5.2.2 Optional GFSK modulation

An optional and very low power PHY based on CP-2FSK modulation is contemplated for the sub-GHz band.

No support for MIMO technologies, i.e., layer mapper and precoding are not necessary.

The proposed channel encoder, bit interleaver and scrambler are used as well. The modulation mapper is CP-2FSK.

CP-2FSK modulation is given by

s(t)=VS2πfct+2πΔf−∞tb(t')dt'+φ0

Δf=β2Tsymwhere V is amplitude, S(t)=sin(2πfct) is the modulating-carrier signal, fc is  the carrier frequency,                       is the peak frequency deviation, β=1 is the modulation index, Tsym is the symbol time and φ0 is the initial phase of the modulating-carrier signal

The information bearing signal is given by

b(t)=m(1−2gm)p(t−mTsym)

where gm is information bit, p(t) is a Gaussian pulse shape of bandwidth-symbol duration product of 0.8

6.6 Multiple antennas

6.6.1 Layer mapping

Two MIMO technologies are supported: open loop spatial multiplexing and transmit diversity (SFBC) for 2 and 4 antennas.

The [complex] modulation symbols per codeword di for i=0,1,…,Nsym-1 are mapped into several layers

  - Layer=independent stream of symbols in a MIMO configuration.

  - Rank=number of layers transmitted.

[d0q,...,dNsym−1q]→[x0(i),...,xυ−1(i)]Ti=0,1,...,NsymL−1As                                                             for

  - υNsymLwhere     is the number of layers and        is the number of symbols per layer for the qth codeword.

6.6.1.1 Open loop spatial multiplexing (parallel data streams)

The mapping of codeword symbols onto layer streams can be found in document with DCN 369r1.

#### 6.6.1.2 Transmit diversity (same information is transmitted from multiple antennas)

The mapping of codeword symbols onto layer streams can be found in document with DCN 369r1

6.6.2 Precoding

Precoding allows to increase system performance and robustness by feeding back to the transmitter channel state information.

Schematic diagram of MIMO support with precoding is illustrated as

6.6.2.1  Open loop spatial multiplexing

Precoding for open loop spatial multiplexing increases robustness by feeding back the channel’s rank (RI=rank indicator). The transmitter chooses a pre-fixed codeword according to the RI.

Single antenna mapping is given by

i=0,1,...,NsymP−1NsymP=NsymL          where  ,                                and

Multiple antennas mapping is given by

y(i)|Px1=W(i)|Pxνx(i)|νx1

The transmitter chooses a codeword according to reported ν. The codebook for 2 and 4 antennas can be found in document with DCN 369r1.

6.6.2.2 Transmit diversity

Transmit diversity is aimed to increase robustness in scenarios with low SNR, low delay tolerance or no feedback to the transmitter is available or reliable.

Case of 2 antennas

  - STBC (space-time block code) for DTF-Spread OFDM

  - SFBC (space-frequency block code) for OFDM

SFBC for 2 antennas is given by

y0(2i)y0(2i+1)y1(2i)y1(2i+1)=12x0(i)x1(i)−x1(i)x0(i)

In case of 4 antennas a combination of SFBC (2 antennas) with frequency switch transmission diversity is given by

y0(4i)y0(4i+1)y0(4i+2)y0(4i+3)y1(4i)y1(4i+1)y1(4i+2)y1(4i+3)y2(4i)y2(4i+1)y2(4i+2)y2(4i+3)y3(4i)y3(4i+1)y3(4i+2)y3(4i+3)=12x0(i)x1(i)0000x2(i)x3(i)−x1(i)x0(i)0000−x3(i)x2(i)

6.7 Bit interleaver

In order to minimize latency and integration on parallel architectures within the encoder/decoder implementation, an algebraic interleaver is proposed.

A maximum contention-free quadratic permutation interleaver is defined as

(i)=f1i+f2i2 Mod NI

where i=0,1,…,NI-1. NI is the length of interleaver.

Elements of codewords of length n (ci for i=0,1,…,n-1) are interleaved in blocks of NI bits as c∏(j) for j=0,1,…,NI-1

(j)=31j+64j2 Mod 1024Short length interleaver:

(j)=11j+21j2 Mod 15120Long length interleaver:

6.8 Scrambling

A scrambler is used to shape the data spectrum and to randomize data across users in order to reduce interference.

A Gold code generator of length 63 is proposed as scrambler

  - PN sequence with period 263 (truly random for long packets)

  - Different initialization seeds, enables a different Gold code per user with low correlation respect to other user using a different seed.

bi=ci+skn+i Mod 2The Gold code generator outputs si for i=0,1,…,263-1, which is used to scramble the interleaved codeword bits ci for i=0,1,…,n-1 prior to modulation as

Shift registers initialization at start of a packet is given by

  - User ID (1st register) and group ID (2nd register)

  - Fast forward both registers 100 times to reduce PAPR

6.9 Discovery procedure

The discovery preamble  (DP) is based on a ZC sequence and a discovery resource block (DRB) from a modified DTF-S OFDM or OFDM signal.

  - The DP and DRB formed the Discovery Signal (DS).

Moreover, we propose to use one channel (from the proposed channelization for sub-GHz, 2.4 GHz and 5.7 GHz bands) for only discovery of devices.

PAC devices can either transmit or receive the DS in this unique channel asynchronously. Such unique channel for discovery is named Shared Discovery Channel (SDCH).

The discovery signal (DS) sent over a discovery shared channel (DSCH) is illustrated as

The DS consists of a preamble sequence plus a DRB formed by Nfs frequency slots and Nts time slots.

Once synchronized, a receiver knows the location of the discovery resource block (DRS) to scan for possible peers or to pick time-frequency slots to transmit its DS.

The proposed ZC sequence for initial synchronization during discovery (or communication mode) is a ZC sequence with the following parameters: Sequence length N = 63, relative prime r = 62 and q = 0.

Such ZC sequence is mapped onto 62 sub-carriers

  - The first subcarrier and the DC subcarrier are empty.

Of course, several repetitions may be transmitted

The discovery process is energy intensive. In order to minimize power consumption only one subcarrier of the N-point IFFT is used per user.

  - The PAPR is set to the minimum.

  - Across the frequency domain, users are orthogonal (OFDM)

For discovery, the nth symbol transmitted over the kth subcarrier is given by

  - for l=-L+1,,…,0,1,…,N-1 and L the CP

Contiguous subcarriers  are orthogonal as illustrated in the figure:

The DS is transmitted with a predefined duty-cycle (see NICT MAC pre-proposal). From upper layers, terminals pick time-frequency slots in the DRB to transmit the DS.

A set of 126 symbols are transmitted per time slot:

  - such frame contains information about a given peer (peer ID, service ID, application ID, etc.) and which channel is used for association (different from the SDCH).

The number of frequency slots Nfs =1024 (IFFT size) and the number of time slots Nts is chosen as 20.

Consequently, the DRB can support up to NfsxNts=20,480 users for discovery per Group.

The discovery data is formatted as illustrated in  the figure:

Append 57 bits for user ID, device ID, Group ID, etc.

Append 6 bits from CRC-6-ITU error detection code

Append 63 bits from shorten BCH(126,63) code

6.10 Discovery procedure

Devices are in three possible states:

  - RDM –receiving discovery mode,

  - TDM –transmitting discovery mode,

  - Sleep -idle.

Also, devices are equipped with clear channel assessment (CCA).

1) At start up or after idle or after command, a device enters into receiving discovery mode (RDM) and listens for a DS.

Synchronization subsystem detects the preamble (and hence the position of the DRB in time) and detection subsystem scans the DRB for DRB usage and detection of peer ID, service ID, etc. Go to Sleep mode.

2) After command, a device enters into transmitting discovery mode (TDM)

Device chooses a free time-frequency slot to transmit its  DS over the next DRB transmission. Go to Sleep mode.

Of course, there are intermediate steps like time-out for DRB scanning, miss detection, etc. Here, we present the core algorithm only.

6.10.1 Peering procedure

For association or peering, intended devices that want to establish a communication link, request that through a random access preamble transmitted in the ACH.

The random access association preambles are named Random Access Preambles (RAPs).

Such RAPs are formed with ZC sequences as well.

Hence, a pool of orthogonal RAPs is formed

  - in order to reduce interference from competing terminals for random access.

A unique RAP is assigned to every device in a Group.

Such unique RAP is used for fine synchronization and control messages (how a communication link is granted).

The RAP signal is illustrated as

where GI is guard interval and RA preamble is set of orthogonal preambles.

Thus, a set of orthogonal preamble sequences of length 1024 can be generated from ZC sequences for random access, which satisfy maximum round-trip time and coverage performance for a maximum distance of 500m. Details of preamble design can be found in document with DCN 369r1.

The CP and GI lengths are given by

t=2⋅500m3x108m/s=3.33 μsec

Once initial synchronization and decoding of discovery RB are achieved by Terminal 2 over Terminal 1:

  - Terminal 2 requests association by a random access procedure based on an orthogonal RAP.

The process is initiated and control by upper layers.

1) Terminal 2 sends RAP over the ACH.

  - It is randomly selected from a pool of orthogonal ZC sequences.

  - It contains finer frequency granularity for Terminal 1 to acquire fine time and frequency synchronization of Terminal 2, plus information about the resources needed to transmit in 3).

2) Terminal 1 replies with RA response message.

  - It contains timing information (round-trip delay), RAP-ID, RB  grant to transmit in 3), Group identifier, etc.

3) Scheduling request

  - It contains scheduling request information for transmission.

  - If this message is successfully detected in Terminal 1, still contention remains unsolved for other terminals. A quick resolution is needed.

4) Contention resolution

  - Terminal 1 echoes Terminal 2 ID contained in 3)

  - Terminal 2 detects its ID and sends ACK (RA terminated) a communication link is scheduled and established.

  - Terminal 2 detects another ID (RA terminated, starts a new one)

  - Terminal 2 fails to detect ID (RA terminated, starts a new one)

6.11 Reference signals

Reference signals are required to perform channel estimation in time and frequency domain for detection and equalization.

6.11.1 Channel time dispersion

Considering a maximum speed of v=100 Km/h (27.78 m/s)

The Doppler spread is given by fd=fc v/c

Min. sampling time to reconstruct the channel is Tc=1/2fd

The slot time Tslot=0.5 msec. Then, one reference symbol per slot is needed in the time domain to estimate the channel correctly.

6.11.2 Channel frequency dispersion

BC,90=150στBC,50=15στConsidering 90%, 50% coherence bandwidth as                               and

στwhere      is the RMS delay spread.

στ=CadγaThe outdoor RMS delay spread is estimated as

For a distance d=500m, we have 90%, 50% coherence bandwidth given in the table

BC,50<BWIf                   then frequency selective fading and frequency domain equalization is needed.

We propose that the spacing between 2 reference symbols in frequency in a RB is 30 KHz to resolve frequency variations.

6.11.2 ZC sequences

Preambles or beacons for synchronization and reference signals (RSs) are formed with Zadoff-Chu (ZC) sequences of length N.

  - Constant-amplitude zero-correlation (CAZAC) sequences

ak=WNk22+qkN evenWNk(k+1)2+qkN odd

WN=e−j2πrNwhere                      is a primitive nth root of unity, r is a relative prime to N, sequence index k = 0, 1, ...,N-1and q is any integer.

ZC sequences have constant amplitude and so its N-point DFT.

  - This limits the PAPR and simplifies implementation as only phases have to be generated and stored, besides of bypassing the FFT at transmitter for DFT-S OFDM.

ZC sequences have ideal cyclic autocorrelation, i.e., the correlation with its shifted version is a delta function

  - Thus, a [large] set of orthogonal preambles or reference signals is possible to generate.