IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | BRC Internal Comment Resolution |  |
| Date Submitted | 2 December, 2013 |  |
| Source | Alina Lu Liru, Keiichi Mizutani, Chin Sean Sum, Fumihide Kojima, Ming-Tuo Zhou, Verotiana Rabarijaona, Lan Zhou , Hiroshi Harada (NICT) | E-Mail: [liru@nict.com.sg; mizk@nict.go.jp; mingtuo@nict.com.sg; sum@nict.go.jp; f-kojima@nict.go.jp; rverotiana@nict.go.jp; lan@nict.go.jp; harada@nict.go.jp] |
| Re: |  |  |
| Abstract | [Comment resolution for BRC internal comment resolution] |  |
| Purpose | [to provide comment resolution to BRC internal comments] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Comment:

The HCS procedure is incomplete; for once, the part about where the contents of the calculation field are actually used is missing. Need to add the logic on the generation of the remainder derived from the calculation field contents, treated as a polynomial, multiplied by x8 and then divided (modulo 2) by G8(x), and how the two remainders are complement XORed to produce the HCS.

Suggested change:

Suggest replacing description of HCS computation with reference to HCS algorithm in 18.2.1.3, where instead of 22 bits, the HCS is computed over the first 30 bits of the PHR. Confirm HCS procedure used in computing the bit stream for the example.

Proposed resolution: AP

Change line 22-26 on page 99

"The Header Check Sequence (HCS) field (H7-H0) is an 8-bit CRC taken over the PHY header (PHR) fields. The HCS shall be computed using the first 30 bits of the PHR. The HCS shall be calculated using the polynomial G8(x) = x8 + x2 + x + 1.The HCS is the remainder resulting from [xk(x7 + x6 +…+1)] divided (modulo 2) by G8(x), where the value k is the number of bits in the calculation field. At the transmitter, the initial remainder shall be preset to all zeros."

to

The Header Check Sequence (HCS) field (H7-H0) is an 8-bit CRC taken over the PHY header (PHR) fields.

The HCS shall be computed using the first 30 bits of the PHR with the polynomial G8(x)=x8+x2+x+1. The HCS is the remainder resulting from [x8(b0x29+b1x28+...+b28x+b29)] divided(modulo 2) by G8(x), where b0x29+b1x28+...+b28x+b29is the polynomial representing the first 30 bits of the PHR for which the checksum is to be computed. At the transmitter, the initial remainder shall be preset to all zeros.