IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | TG4n Coexistence Assurance Document |
| Date Submitted | 08 Nov. 2014 |
| Source | Liang Li, Arthur Astrin, Ken Mori |
| Re: | IEEE 802.15.4n |
| Abstract | Analysis on coexistence of IEEE 802.15.4n with other IEEE 802 systems within the same frequency band |
| Purpose | To address the coexistence capability of IEEE 802.15.4n to satisfy requirements of the IEEE 802.19 Work Group and IEEE 802 Executive Committee to determine if a proposed IEEE 802 standard has made a reasonable effort to be able to coexist with devices compliant to other IEEE 802 standards in their operating band. |
| Notice | This document has been prepared to assist IEEE P802.15 coexistence. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Table of Contents

1	Scope	1

2	References	1

3	Acronyms and abbreviations	2

4	IEEE 802.15.4n amendment overview	2

4.1	Operating frequency bands	2

4.2	Modulation parameters	2

4.3	Coexistence mechanisms	4

5	Other IEEE 802 standards occupying the same frequency bands	4

6	Coexistence scenarios and analysis	4

6.1	Methodology	4

6.1.1	PER for IEEE 802.15.4n	5

6.2	Coexistence with IEEE O-QPSK PHY operating in the 779–787 band devices	5

6.3	Transmit power spectral density (PSD) mask	5

6.4	Receiver sensitivity	5

6.5	Receiver interference rejection	6

7	Discussion and conclusion	6

# Scope

The IEEE 802.19 Work Group has mandated that new wireless standards developed under IEEE 802 be accompanied by a Coexistence Assurance document. In [3], guidelines are provided for how coexistence can be quantified based on predicted packet error rates among IEEE 802 wireless devices. A detailed discussion of coexistence and coexistence methods can be found in IEEE Standard 802.15.2-2003 [2]. Hence, this coexistence assurance document is provided by the IEEE 802.15.4n Task Group to satisfy the requirements of the IEEE 802.19 Work Group and IEEE 802 Executive Committee.

The Ministry of Industry and Information Technology (MIIT) of the People's Republic of China has approved the 174-216 MHz, 407-425 MHz and 608-630 MHz bands for medical information transmission.

The IEEE 802.15 Task Group 4n defines an amendment to the IEEE Standard 802.15.4 [4], including changes to the Medium Access Control (MAC) layer and additions to the PHY layer to support these bands. This amendment will enable devices to be used on or near a body in compliance with the China medical band (CMB) Rules. The IEEE 802.15.4n Amendment is only valid for the People's Republic of China (CN) and therefore this document will only address coexistence within the People's Republic of China. At the time of writing of this document, this band cannot be used by CMB devices outside of the People's Republic of China.

China medical band (CMB) devices operating within these bands conform to a set of rules [1] specified in China, Ministry of Industry and Information Technology Doc: 423-2005, which restrict use of the band to only medical, non-voice use under direction of a healthcare practitioner, among other requirements.

This document addresses the coexistence of the IEEE 802.15.4n CMB band systems with other IEEE 802 standards operating in the same frequency bands. It addresses the interference caused by IEEE 802.15.4n devices to these existing systems, and the interference from these existing standards with IEEE 802.15.4n.

# References

- Ministry of Industry and Information Technology of the People’s Republic of China document No. 423 (2005) available at: http://www.miit.gov.cn/n11293472/n11295310/n11297428/11637344.html

- IEEE Std 802.15.2TM-2003, “Coexistence of Wireless Personal Area Networks with Other Wireless Devices Operating in Unlicensed Frequency Band”

- S. Shellhammer, “Writing a Coexistence Assurance Document,” IEEE 802.19-09/0001r0, 2009.

- IEEE Std 802.15.4TM-2011.

- S. Shellhammer, “Estimating of Packet Error Rate Caused by Interference – A Coexistence Assurance Methodology,” IEEE 802.19-05/0028r2, 2005.

- J .G. Proakis, Digital Communications 4th edition, 2000.

- Doc 15-12-0105-02-4n-Translaiton of Chinese-MIIT-Doc423-2005.doc

- Doc 15-13-0313-03-4n-Chinese-Radio-Regulation-Discussion.doc

- Doc 15-12-0471-04-4n-summary-of-interference-on-Chinese-Medical-bands.doc

# Acronyms and abbreviations

AWN 			Affected Wireless Network

BER			Bit Error Rate

CCA			Clear Channel Assessment

DSSS			Direct Sequence Spread Spectrum

ED			Energy Detection

IWN 			Interfering Wireless Network

LQI			Link Quality Indicator

CMB			China medical band

O-QPSK		Offset Quadrature Phase Shift Keying

PER			Packet Error Rate

SIR			Signal to Interference Ratio

# IEEE 802.15.4n amendment overview

## Operating frequency bands

The allocated frequency band for the IEEE 802.15.4n Amendment is:

| Band Identifier (MHz) | Frequency band (MHz) |
| --- | --- |
| 195 | 174-216 |
| 416 | 407-425 |
| 619 | 608-630 |

Table 1 – IEEE 802.15.4n Frequency bands for CMB

## Modulation parameters

The IEEE 802.15.4n CMB consists of two PHYs: O-QPSK and GFSK. Modulation parameters in the frequency bands are shown in Table 2 – IEEE 802.15.4n Frequency bands and data rates for CMB O-QPSK PHY and Table 3 – IEEE 802.15.4n Frequency bands and data rates for CMB GFSK PHY.

| Band Identifier (MHz) | Frequency band(MHz) | Spreading parameters |  | Data parameters |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Chip rate (kchip/s) | Modulation | Bit rate (kb/s) | Symbol rate (ksymbol/s) | Symbols |
| 195 | 174-216 | 1000 | O-QPSK | 250 (mandatory) | 62.5 | 16-ary orthogonal |
|  | 174-216 | 1000 | O-QPSK | 500 (optional) | 125 | 8-ary orthogonal |
| 416 | 407-425 | 1000 | O-QPSK | 250 (mandatory) | 62.5 | 16-ary orthogonal |
|  | 407-425 | 1000 | O-QPSK | 500 (optional) | 125 | 8-ary orthogonal |
| 619 | 608-630 | 1000 | O-QPSK | 250 (mandatory) | 62.5 | 16-ary orthogonal |
|  | 608-630 | 1000 | O-QPSK | 500 (optional) | 125 | 8-ary orthogonal |

Table 2 – IEEE 802.15.4n Frequency bands and data rates for CMB O-QPSK PHY

| Band Identifier (MHz) | Frequency band(MHz) | Spreading parameters |  | Data parameters |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Chip rate (kchip/s) | Modulation | Bit rate (kb/s) | Modulation Index | Symbols |
| 195 | 174-216 | --- | GFSK (mandatory) | 50 | 1.0 | Binary |
|  | 174-216 | --- | GFSK (optional) | 100 | 0.5 or 1.0 | Binary |
|  | 174-216 | --- | GFSK (optional) | 200 | 0.5 or 1.0 | Binary |
| 416 | 407-425 | --- | GFSK (mandatory) | 50 | 1.0 | Binary |
|  | 407-425 | --- | GFSK (optional) | 100 | 0.5 or 1.0 | Binary |
|  | 407-425 | --- | GFSK (optional) | 200 | 0.5 or 1.0 | Binary |
| 619 | 608-630 | --- | GFSK (Mandatory) | 50 | 1.0 | Binary |
|  | 608-630 | --- | GFSK (optional) | 100 | 0.5 or 1.0 | Binary |
|  | 608-630 | --- | GFSK (optional) | 200 | 0.5 or 1.0 | Binary |

Table 3 – IEEE 802.15.4n Frequency bands and data rates for CMB GFSK PHY

Table 3 – IEEE 802.15.4n Frequency bands and data rates for CMB GFSK PHY shows the modulation and channel parameters for the standard-defined PHY operating modes for the 195 MHz, 416 MHz, and 619 MHz bands. A device will support mode #5 and may additionally support modes #1 to #4.

| Frequency band | Parameter | Mode #1 | Mode #2 | Mode #3 | Mode #4 | Mode #5 |
| --- | --- | --- | --- | --- | --- | --- |
| 174-216MHz 407-425MHz 608-630MHz | Data rate (kb/s) | 100 | 100 | 200 | 200 | 50 |
|  | Modulation index | 0.5 | 1.0 | 0.5 | 1.0 | 1.0 |
|  | Channel spacing (kHz) | 500 | 500 | 500 | 500 | 500 |

Table 4 – IEEE 802.15.4n CMB GFSK modulation and channel parameters

## Coexistence mechanisms

The coexistence mechanisms for the CMB band specified in the IEEE 802.15.4n standard are:

- Clear Channel Assessment (CCA)

  - Energy detection and/or carrier sense

- Dynamic channel selection

- Low Duty cycle

- Limited transmission power

- Modulation

  - Direct sequence spread spectrum (DSSS) with half-sine pulse shaping

  - GFSK

- ED and LQI

- Channelization Scheme

  - Channel alignment between IEEE 802.15.4n O-QPSK and IEEE 802.15.4n GFSK

# Other IEEE 802 standards occupying the same frequency bands

The frequency bands are 195 MHz, 416 MHz and 619 MHz bands. There are no other frequency bands used by the IEEE 802.15.4n Amendment. There are no other IEEE 802 standards occupying same frequency bands used by the Amendment.

# Coexistence scenarios and analysis

This clause presents the analysis of the interference caused by IEEE 802.15.4n CMB devices to the IEEE 802 standard devices identified in section 5, and vice versa. Note that the interferer parameters chosen in the following sub-clauses are chosen for the purpose of coexistence analysis in this CA document and are a sub-set of the available modes.

## Methodology

In order to quantify the coexistence performance, the techniques described in Shellhammer [3] and [5] have been adopted. The coexistence assurance methodology predicts the PER of an affected wireless network (AWN), in the presence of an interfering wireless network (IWN). In its simplest form, the methodology assumes an AWN and an IWN, each composed of a single transmitter and a receiver. The methodology takes as input a path loss model, a BER function for the AWN, and in some case predicted temporal models for packets generated by the AWN and for packets generated by the IWN. Based on these inputs, the methodology predicts the PER of the AWN as a function of the physical separation between the IWN transmitter and the AWN receiver.

The appeal of the coexistence assurance methodology is that multiple networking standards can be characterized and compared with just a few parameters, notably:

- Bandwidth of AWN and IWN devices

- Path loss model for the networks

- PER as a function of SIR of AWN devices

### PER for IEEE 802.15.4n

The PER for the O-QPSK PHY in IEEE 802.15.4 is calculated from a BER model. The BER (Pb) is given as a function of SIR as follows [4]:

Pb=815116j=216−1j16jexp20SIR1j−1    (1)

From the computed BER, the PER is obtained as shown in the Equation (2).

PER=1−1−PbL                               (2)

where L is the number of bits in a packet and the typical length of an IEEE 802.15.4n packet is 20 octets [4].

## Coexistence with IEEE O-QPSK PHY operating in the 779–787 band devices

The 802.15.4 standard defines operation of the O-QPSK as defined in Clause 10. It is therefore important to consider the likely impact of CMB devices that are operating adjacent to other IEEE 802.15.4 wireless systems. The top edge of the CMB band at 619 MHz (608MHz - 630MHz) is separated from the 779 - 787 MHz band by 149 MHz. This, combined with the MIIT rules [1] for CMB devices that transmitter out-of-band (OOB) emissions are attenuated in accordance with [7] is sufficient to assure coexistence.

## Transmit power spectral density (PSD) mask

In general, when operating in the 176 MHz, 407MHz and 608MHz bands, the transmitted spectral products will be less than the limits specified in following table.

For both relative and absolute limits, average spectral power will be measured using a 100 kHz resolution bandwidth. For the relative limit, the reference level will be the highest average spectral power measured within ±600 kHz of the carrier frequency fc.

| Frequency | Relative Limit | Absolute limit |
| --- | --- | --- |
| ¦f-fc¦ > 1.2 MHz | -20 dB | -20dBm |

Table 5 – IEEE 802.15.4n PSD Limitation among channels

However, there are some special limitations for spurious radio emission on the Chinese radio regulation in Bibliography. These spurious emission details related to Chinese approved radio bands for medical signal transmission are shown in [7], [8] and [9].

## Receiver sensitivity

Under the conditions specified in Table 69 (8.1.7), a compliant device will be capable of achieving the sensitivity values given in Table 6

| Frequency band (MHz) | Rate Mode |  |
| --- | --- | --- |
|  | 0 | 1 |
| 176-214 | -85 | -82 |
| 407-425 | -85 | -82 |
| 608-630 | -85 | -82 |

Table 6 – IEEE 802.15.4n required receiver sensitivity (dBm) for both Rate Mode modulations

## Receiver interference rejection

There are multiple receiver interference caused by CMB O-QPSK compliant signals, and TV, industry and commercial in the same bands.

The interfering signal will be a CMB O-QPSK compliant signal with the following characteristics:

- Pseudo-random PSDU

- SpreadingMode can be set to DSSS

- Data Whitenning is used in GFSK

- The same chip rate as the desired signal

- O-QPSK Chip-whitening can be enabled

The TV, industry and commercial interference are:

- On 176-216 MHz band, the major interferences are sourced from: Digital TV, and Wireless Microphone

- On 407-425 MHz band, the major interferences are sources from Interphone (Walkie-Talkie)

- On 608-630 MHz band, the major interferences are sources from Digital TV.

The interference-to-signal ratio (ISR) is the ratio of the signal power of an interferer relative to the signal power of the desired signal.

The adjacent channel rejection will be measured as follows: the desired signal will  be a CMB O-QPSK compliant signal of pseudo-random PSDU data. For a given Rate Mode, the desired signal is input to the receiver at a level 3 dB above the maximum allowed receiver sensitivity.

The TV rejection will be measured as follows: at CCA detection time, if the receiver ED level is higher ED threshold desired by user, there may be TV or other industry or commercial interference on same channel or band.

The detection methods of TV, industry and commercial interference are outside the scope of this standard.

# Discussion and conclusion

This document has presented an analysis of the coexistence between the IEEE 802.15.4n Amendment and IEEE 802.15.4 systems. The results of the analysis are that frequency difference and transmit mask protect IEEE 802.15.4n and IEEE 802.15.4 systems respectively.  This is notable since it is expected that these two systems may be used together in close proximity around the human body. The separation of frequency to protect IEEE 802.15.4n is unaffected by the data rate that is used.

For IEEE 802.15.4n devices the relative limit on average spectral power (using a 100 kHz resolution bandwidth) at frequency f such that |f-fc| > .6 MHz has to be less than -20dBm from the TX power at fc.

For IEEE 802.15.4n devices the absolute limit on average spectral power (using a 100 kHz resolution bandwidth) at frequency f such that  |f-fc| > 1.2 MHz has to be less than -20dBm for any TX power.

As a victim, 802.15.4n GFSK has comparable BER performance with the other 802 GFSK systems; 802.15.4n DSSS has much better BER performance than the other 802 DSSS systems due to the high spreading factor values. As an interferer, either 802.15.4n GFSK or 802.15.4n DSSS has similar performance impact to the other 802 systems at the same transmitting power level. However the performance degradation to the other systems can become significant as the transmitting power is increased up to the possible maximum 10dBm. This requires more physical distance from other 802 systems if an 802.15.4n system is designed to operate at a high transmitting power level.

IEEE 802.15.4n devices can be operated at a transmit power of up to 10 dBm in the CMB bands and this combined with receiver rejection, protects IEEE 802.15.4n and IEEE 802.15.4 devices.