IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | NICT PHY Specification Proposal |
| Date Submitted | May 4th, 2014 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Response | In response to Call for Contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1. Symbols and Abbreviations	3

1.1 Symbols	3

1.2 Abbreviations	3

2. PHY specification	3

3. PHY frame structure	3

3.1 TDD frame	4

3.2 Guard interval	5

3.3 Preamble	5

3.4 Control channel	5

4. Synchronization preamble	6

5. PHY header	7

5.1 Data rate	7

5.2 PSDU length	7

5.3 Burst mode	8

5.4 Scrambler seeds	8

5.5 Frame type	8

5.6 PSDU type	8

5.7 Header check sequence	8

6. Discovery	9

6.1 Discovery signal	9

7. Peering	11

7.1 Random access preamble	11

7.2 Random access procedure	13

8. PSDU construction	14

8.1 QC-LDPC encoder parameters	16

8.2 Interleaver	16

8.3 Scrambler	17

8.4 Modulation mapper	17

9. Layer mapping	18

9.1 Layer mapping for one antenna	18

9.2 Open loop spatial multiplexing	18

9.3 Transmit diversity	19

10. Precoding	19

10.1 Single antenna mapping	20

10.2 Open loop spatial multiplexing	20

10.3 Transmit diversity	21

11. Multicarrier modulation	22

11.1 Cyclic prefix	22

11.2 Resource block	23

11.3 Filtered OFDM symbols for sub-1 GHz band	24

11.4 Carrier aggregation	24

11.5 Data rates	24

12. Reference signals	25

12.1 Time domain	25

12.2 Frequency domain	26

13. Optional GFSK modulation	26

14. Operating frequency bands	26

14.1 Channelization of 920 MHz band	27

14.2 Channelization of 2.4 GHz band	28

14.3 Channelization of 5.7 GHz band	28

Annex A (Normative) QC-LDPC prototype matrices	29

Annex B (Normative) Modulation mapping	31

Bibliography	34

- Symbols and Abbreviations

  - Symbols

  - Abbreviations

- PHY specification

The proposed PHY specification is designed to offer robust performance for PAC systems and to provide a large scope for implementation opportunities for high performance, robustness, low complexity, and low power operation.

The proposed PHY provides a data interface to the MAC layer under the control of the physical layer convergence protocol (PLCP).

The proposed PHY provides four levels of functionality, as follows:

- Activation and deactivation of the radio transceiver.

- The transmission and reception of synchronization preambles to mantain network synchronization.

- An interface to the MAC for trasnmission and reception of discovery, peering, scheduling and data information.

- It may provide clear channel assessment (CCA) indication to the MAC in order to verify activity in the wireless medium.

- PHY frame structure

The PHY frame format or physical layer protocol data unit (PPDU) can be formed for 2 types of PPDU. The PPDU ultra frame type 1 is formed by concatenating the synchronization preamble, the physical layer header (PHR), the discovery interval, peering interval, and the physical layer service data unit type (PSDU), respectively, as illustrated in Figure 1. The PPDU ultra frame type 2 is formed by concatenating the synchronization preamble, the PHR, and PSDU, as illustrated in Figure 1.

- —PPDU ultra frame structures type 1 and 2.

The PPDU type 1 conveys discovery, peering and data information. The PPDU ultra frame type 2 is used in burst mode indicated in the PHR, where discovery and peering are not required.

The PSDU is formed by N data-channels. Each data channel is sub-divided into scheduling and data frame. Each data frame consists of 10 TDD slots: two switching slots (denoted as S) and eight data slots.  Each data slot consists of seven OFDM symbols with duration of 0.5 msec, as illustrated in Figure 2.

- —Generic PSDU structure

  - TDD frame

The TDD frame consists of two switching slots and eight data slots. For asymmetric traffic, three types of TDD frames are defined in Table 1.

- —TDD asymmetric frame types

| Type | Frame structure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 1 2 | S | Lt | Lt | Lt | Lt | S | Lr | Lr | Lr | Lr |
|  | S | Lt | Lt | S | Lr | Lr | Lr | Lr | Lr | Lr |
|  | S | Lt | Lt | Lt | Lt | Lt | Lt | S | Lr | Lr |

In frame type 1, the first four data slots, denoted as Lt, the PAC device (PD) transmits. After switching, in the remaining four data slots, the PD receives, Lr.

The TDD switching slot (denoted as S) consists of a guard interval, preamble for fine synchronization and channel sounding and control channel as illustrated in Figure 3. Note that the preamble and control channel are transmitted by the PD that was in receiving mode before the switching. The duration of S is 0.5 msec as well.

- —TDD switching slot

  - Guard interval

The last transmitted symbol of PD1 arrives to PD2 after a propagation time, Tp. Such symbol is detected over Tdec. PD2 switches from receiver to transmitter in Tsw. Then, the first transmitted symbol of PD2 arrives to PD1 after Tp. Consequently, the guard interval is given by

GI=Tp+Tdec+Tsw+Tp+Tcom

where Tcom is compensation time to align to FFT sampling.

Considering Tp=1 Km/3x108 m/s = 3.3 µsec (worst case), or Tp=10 m/3x108 m/s = 0.033 µsec (typical case), Tsw=500 nsec, Tdec=0.9 µsec, then GI=10 µsec.

  - Preamble

The preamble is formed of a ZC sequence.

  - Control channel

Control channels passes information for the PD in receiving mode about previous channel’s rank and TDD frame type.

- —Channel’s rank

| C0 C1 | Channel’s rank |
| --- | --- |
| 00 | 1 |
| 01 | 2 |
| 10 | 3 |
| 11 | 4 |

- —TDD frame type

| F0 F1 | Frame type |
| --- | --- |
| 00 | 0 |
| 01 | 1 |
| 10 | 2 |
| 11 | r |

- Synchronization preamble

The synchronization preamble is repetitions of Zadoff-Chu (ZC) sequences. Those belong to family of Constant-amplitude zero-correlation (CAZAC) sequences. ZC sequences have good correlation properties and easy implementation for devices with multicarrier modulation. As ZC sequences have constant amplitude and so its N-point DFT, this limits the PAPR and simplifies implementation as only phases have to be generated and stored.

ZC sequences are well documented in the literature and it is possible to test and design a large set of orthogonal preambles or reference signals.

ZC sequences of length N is given by

ak=WNk22+qkN evenWNk(k+1)2+qkN odd

where WN=e−j2πrN is a primitive nth root of unity, r is a relative prime to N, q is any integer and  sequence index k = 1, 0,…,N−1.

The synchronization preamble is formed by repetitions of the ZC sequence with length N = 63, relative prime r = 62 and q = 0.  The autocorrelation properties is illustrated in Figure 4.

- —Autocorrelation of synchronization preamble.

- —Implementation of synchronization preamble.

- PHY header

The PHY header structure is illustrated in Figure 6, where the transmit order is from bit 0 to bit 26.

| Bit 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R0 | R1 | R2 | r | L0 | L1 | L2 | L3 | L4 | L5 | L6 | L7 | r | r |
| Data rate |  |  |  | PSDU length |  |  |  |  |  |  |  |  |  |

| 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B | S10 | S11 | S12 | S13 | S14 | S15 | S20 | S21 | S22 | S23 | S24 | S25 |
| Burst | 1st seed |  |  |  |  |  | 2nd seed |  |  |  |  |  |

| 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- |
| F | P1 | P2 | r | r |
| Frame type | PSDU type |  | Reserved |  |

- —PHY header structure

  - Data rate

Data rates are indicated by (R0, R1, R2), where R2 is the most significant bit (MSB) and R0 is the least significant bit (LSB).

  - PSDU length

A variable frame length is indicated by (L0, L1, L2, L3, L4, L5, L6, L7), where L7 is the MSB and L0 is the LSB.

  - Burst mode

The burst mode is indicated by (B) and defined in Table 56. The burst mode supports higher throughput by allowing the transmission of consecutive frames without ACK.

- —Burst mode

| B | Status |
| --- | --- |
| 0 | Next package is not part of burst |
| 1 | Next package is part of burst |

  - Scrambler seeds

The two scrambler seeds (initial conditions) for the Gold code generator are indicated by (S10 S11S12 S13 S14 S15) for user ID and (S20 S21 S22 S23 S24 S25) for group ID.

  - Frame type

The frame type is indicated by (F) and defined in Table 5.

- —Frame type

| F | Status |
| --- | --- |
| 0 | Next package is type 1 |
| 1 | Next package is type 2 |

  - PSDU type

For frame type 2, the PSDU may convey the discovery or peering or data information. This is indicated by (P1, P2) and defined in Table 6.

- —PSDU type

| P1 P2 | PSDU status |
| --- | --- |
| 00 | Discovery |
| 01 | Peering |
| 10 | Data |
| 11 | Reserved |

  - Header check sequence

The PLCP shall append 4-bits from CRC-4 ITU error detection coding to the PHR information. The CRC-4-ITU shall be the one’s complement of the remainder generated by the modulo-2 division of the PHR information by the polynomial:

1+x+x4

The HCS bits shall be obtained in the transmit order as shown in Figure 7 after the PHR bits are processed in the shift register. The shift register stages shall be initialized to all ones.

- —CRC-4 ITU data processing.

- Discovery

For ultra-frame type 1, the discovery information is conveyed by 4 resource blocks denoted as discovery RB signal (DRS).

- —Discovery RB

  - Discovery signal

The discovery signal contains the discovery information of PDs. It consists of a discovery resource blocks (DRBs) formed by 2 slots (14 OFDM symbols) in the time domain and 12 subcarriers in the frequency domain. As the FFT size is 1024, there are 85 DRBs. A PD shall use a DRB to transmit 168 QPSK symbols with half rate FEC or 168 bits (21 bytes).

Once synchronized, a receiver PD knows the location of the DRB to scan for possible peers’ discovery information or for a transmitter PD to pick a DRB to transmit its discovery signal.   The DRB signal parameters are described in clause 9.2.34.

- Peering

After the discovery phase, PDs that intend to establish a communication link with a discovered PD request that with a random access preamble. According to the frame structure in clause 3, after discovery, PDs can transmit in the peering frame period for association with another PD. As such PDs can transmit at any moment; a set of orthogonal preambles is required in order to reduce interference from competing terminals.

The random access association preambles are named Random Access Preambles (RAPs). Such RAPs are formed with ZC sequences as well. Hence, a pool of orthogonal RAPs is formed A unique RAP is assigned to every device in a Group. Such unique RAP is used for fine synchronization and control messages (how a communication link is granted or how resources are assigned to PDs).

  - Random access preamble

The RAP signal structure is illustrated in Figure 9. There is a CP for and guard interval at the beginning and end, respectively.

- —Random access preamble structure

The design of the RAP takes into account:

- The RAP length must fit a slot time of 0.5 msec.

- Maximum round-trip delay.

- Granularity of subcarriers spacing.

Tseq, must allow round-trip estimation at the largest expected distance of d=500 m (1 km round-trip).

the maximum round-trip time and coverage performance for a maximum distance of 500m as follows:

The RAP duration,

Tseq≥1000 m3x108 m/s+στ

According to the Channel Model Document, the RMS delay spread is computed asστ=Cadγa and values for a distance of 500 m are given in Table 7.

Consequently Tseq≥5.33 μsec

- —RMS delay spread for 500m

| Frequency band | στ |
| --- | --- |
| 5.7 GHz | 339 nsec |
| 2.4 GHz | 355 nsec |
| 920 MHz | 2 µsec |

The coverage performance can be estimated from the link budget. It is possible to show that

Tcp=NFkT0L(d)EpN0

where NF is the noise figure, kT0 is the noise temperature, L(d) is the path loss at distance d and Ep is the required preamble energy to meet a PFA of 10−3.

According to the parameters d=500m, fc=2.4 GHz, Pt=1 W, Gt=Gr=5 dBi, NF=5 dB, kT0=−204 dB, the path loss model in clause 2.2.6 of the Channel Model Document, and through simulations the value Ep/N0=18 dB. Consequently, Tseq=0.433 msec.

The CP and GI lengths at 500m are approximately 2(500 m)/3x108 m/s = 3.33µsec. Then, Tseq is upper bounded by

Tseq≤0.433 msec−2(3.33 μsec)=0.4263 msec

The FFT size to implement the RAP must be a natural number multiple of a power of 2:

NFFT=fsTseq=k

In order to minimize the subcarriers orthogonality loss between the RAP and subcarriers use to data, the subcarrier spacing, ∆f,  is a integer multiple of the RAP subcarrier spacing ∆fRAP computed as

ΔfRAP=Δfk

where k is a natural number.

The sequence length that satisfies Equation (7), Equation (8) and Equation (11) simultaneously is Tseq=0.4 msec. Consequently, ∆fRAP=1/Tseq=2.5 KHz and from Equation (11), k=6 or granularity increment. The sampling time is given by ts=1/(1024 ∆fRAP). Finally, the RAP preamble length is computed as 0.4 msec/ts=1024.

The CP duration is given by

TCP=0.5(Tslot−Tseq)+0.5στ

The maximum RMS delay spread of 2 µsec is considered for maximum coverage and so protection against multipath interference. Hence, TCP=51µsec and in terms of number of sampling points:TCPts=131.

The maximum round trip delay (RTD) is the value for the GI:

TGI=0.5(Tslot−Tseq)

Hence, TGI=50 µsec and shown in Figure 10.

- —RAP dimensioning

Conclusion: a set of orthogonal preamble sequences of length 1024 can be generated from ZC sequences for random access, which satisfy maximum round-trip time and coverage performance for a maximum distance of 500m. Moreover, such set of orthogonal preambles is divided into Groups. Tentatively, 100 Groups with 64 RAPs each. Every PD identifies the supported RAP Group ID in the discovery signal.

  - Random access procedure

Once network synchronization and decoding of discovery RB information are achieved by PDs, such PDs may request association (peering) to a target PD at any time.  Consequently, the random access can be requested during the peering period, in which control signals are interchanged in order to schedule the intended PDs for transmission (channel band, modulation, slot time, etc.).

As a general case, we assume the decoding of discovery RB are achieved by PD2 over PD1, PD2 requests association (peering) to PD1 by a random access procedure based on an orthogonal RAP. Moreover, after the discovery phase, PD2 can estimate the transmission power of PD1, channel band used for association and RAP-ID Group handled by PD1.

- —Random access procedure

The peering process is initiated and control by upper layers.

  - PD2 sends a RAP to PD1 requesting association, which is contention based. It is randomly selected from a pool of orthogonal ZC sequences that belong to the RAP Group supported by PD1. Moreover, such RAP contains finer frequency granularity for PD1 to acquire fine time and frequency synchronization of PD2, plus information about the resources needed to transmit in 3).

  - PD1 replies with a RA response message. It is broadcast and contains timing information (round-trip delay), RAP-ID of PD2, plus resources, like time slot or time-frequency slot, to transmit in 3), etc.

  - PD2 sends a scheduling request (note that it is contention free). It contains scheduling request information for transmission. If this message is successfully detected in PD1, still contention remains unsolved for other terminals.

  - Contention resolution. PD1 echoes PD2 ID contained in 3) PD2 detects its ID and sends ACK (RA terminated) a communication link is scheduled and established.  PD2 detects another ID (RA terminated, starts a new one) PD2 fails to detect ID (RA terminated, starts a new one)

- PSDU construction

The PSDU contains the MAC protocol data unit (MPDU) and forward error correction (FEC) bits. The PSDU construction process is illustrated in Figure 12.

The MPDU is passed to the PHY from the MAC. Such data is encoded by quasi-cyclic low density parity check codes (QC-LDPC). Such QC-LDPC FEC codes allow performance close to turbo codes, besides that encoder/decoder enable high throughput and low implementation complexity (efficient implementation in parallel architectures). That is, it is a better option than convolutional codes.

Quasi-cyclic LDPC codes are systematic, linear codes satisfying

HcT=0

where Hn−k x k is the parity check matrix. The codeword, c=(i0,i1,...,ik−1,p0,p1,...,pn−k−1) of length n, consists of k data bits and n-k parity bits.

- —PSDU construction schematic diagram

The number of codewords in the PSDU is given by

NCW=NMPDUk

where NMPDU is the number of information bits in the MPDU. If the rem(NMPDU ,k)≠0 , the last codeword in the PSDU requires Nbs=NCW k−NMPDU  bits stuffing. Otherwise, Nbs=0. The total number of uncoded bits is NPSDU=NMPDU+Nbs.

Total number of coded bits in a packet is NTcw=NCW n and such coded bits are indexed as:

cnq+i    for i=0,1,…,n−1 and q=0,1,…,NCW−1.

H is constructed from a prototype matrix Hp|Mp x Np by replacing each entry of the prototype matrix, denoted as [Hp]i,j, with either a cyclic shift matrix, Pc, or identity matrix or null matrix of size ZxZ. The final size of H is MpZ x NpZ.

Hpij=IZxZIf Hpij=00ZxZIf Hpij='−'PcIf Hpij=p

where p is integer number larger or equal to zero, and ‘-’ denotes a character.

The cyclic-permutation matrix Pc is obtained by cyclically shifting the columns of P0=IZxZ to the right c times, for instance:

P0=1000010000100001       P1=0100001000011000       P2=0010000110000100

  - QC-LDPC encoder parameters

The QC-LDPC coding rates are indicated in Table 8, where k is the number of information bits, n is the number of coded bits and n−k is the number of parity bits.

- —Coding rates

| Coding rate (CR) | k | n |
| --- | --- | --- |
| 1/2 | 972 | 1944 |
| 1/2 | 324 | 648 |
| 2/3 | 1296 | 1944 |
| 2/3 | 432 | 648 |
| 3/4 | 1458 | 1944 |
| 3/4 | 486 | 648 |
| 5/6 | 1620 | 1944 |
| 5/6 | 540 | 648 |

The prototype matrices for the different data rates are in Annex A.

  - Interleaver

The interleaver is based on a maximum contention-free quadratic permutation interleaver. The objective is to minimize latency and the interleaver’s integration on parallel architectures within the encoder/decoder chip implementation.

A maximum contention-free quadratic permutation interleaver is defined as:

(i)=f1i+f2i2 Mod NI

where NI is the interleaver’s length, and the interleaver’s index i=0,1,…,NI −1.

If NI is even, f1 is odd and relative prime to NI and all prime factors of NI are also factors of f2.

The short length interleaver is given by

(i)=31i+64i2 Mod 1024

The long length interleaver is given by

(i)=11i+21i2 Mod 15120

The NTcw coded bits are interleaved in blocks of NI bits as:

c(i Mod NI)    for i=0,1,…,NTcw−1

  - Scrambler

The Gold code generator of length 63 shall be employed as scrambler. A scrambler is used to shape the data spectrum and randomize data across users in order to reduce interference.

As the Gold code generator is formed by two PN sequences with period L=263, its output remains random for long packet sizes. Moreover, different initialization seeds, enables a different Gold code sequence per user and consequently low correlation respect to other user using a different seed.

The Gold code generator shall be constructed by two PN sequence generators with polynomials x6+x+1 and x6+x5+x2+x+1. The Gold code generator output is indicated by si, which is used to scramble the interleaved-coded bits as:

bi=c(i Mod NI)+si Mod L Mod 2       for i=0,1…,NTcw−1

The 63 shift register initialization shall be done by user ID for the first PN generator and group ID for the second PN generator. Fast forward both PN generators 100 times to reduce PAPR.

  - Modulation mapper

The scrambled-interleaved-coded-bits, bi, shall be modulated with BPSK, QPSK, 16QAM or 64QAM.

    - Pad bits

Pad bits shall be appended at the end of the input bit stream to align on a symbol boundary. The number of pad bits is given by

Npad=log2(M)NPSDU+(n−k)NCWlog2(M)−[NPSDU+(n−k)NCW]

where M is the cardinality of the modulation. In case of uncoded transmission NCW =0.

The total number of bits on the air per PHY frame is given by

NT=NPSDU+(n−k)NCW+Npad

    - Modulations

The complex modulation symbols, denoted as dl, as function of the input bits, are given in Table 9 and the modulation mappings are in Annex B, for bit index i=0,…,NT−1 and symbol index l=0,…,Nsym−1 where Nsym=NT /Log2(M) is an integer.

The modulations symbols per codeword may be expressed as

dl+qNCW=dlq   for l=0,1,…,Nsymq−1 and q=0, 1,…,NCW−1.

where Nsymq=Nsym/NCW is the number of symbols for the qth codeword.

- —Modulations

| Modulation | Complex symbol | Log2(M) |
| --- | --- | --- |
| BPSK | dl(bi)=I+jQ | 1 |
| QPSK | dl(bi, bi+1)=I+jQ | 2 |
| 16QAM | dl(bi, bi+1, bi+2, bi+3)=I+jQ | 4 |
| 64QAM | dl(bi, bi+1, bi+2, bi+3 ,bi+4, bi+5)=I+jQ | 6 |

- Layer mapping

Two MIMO technologies are supported: open loop spatial multiplexing and transmit diversity for 2 and 4 antennas. The mandatory transmission in MIMO mode depends on the number of antennas that can be implemented in a PD.

The complex modulation symbols for the qth codeword, dlq for l=0,1,…,Nsymq−1 are mapped into several layers as

[d0q,...,dNsymq−1q]→[x0(i),...,xν−1(i)]T    for  i=0,1,…,NsymL−1.

where ν is the number of layers and NsymLis the number of symbols per layer. Layer stands for an independent stream of symbols in a MIMO configuration. Rank is defined as the number of layers transmitted.

  - Layer mapping for one antenna

In case of one antenna in a PD, only one layer is used,  ν=1, and the mapping is given by the first row of Table 10.

  - Open loop spatial multiplexing

Spatial multiplexing represents the transmission of multiple parallel streams. The mapping of modulation symbols to layers is shown in Table 10. The number of layers is less or equal to the number of antennas, ν ≤ P.

- —Mapping for spatial multiplexing

| No of layers | No of codewords | Mapping | Parameter |
| --- | --- | --- | --- |
| 1 | 1 | x0(i)=di0 | NsymL=Nsym0 |
| 2 | 1 | x0(i)=d2i0 x1(i)=d2i+10 | NsymL=Nsym02 |
| 2 | 2 | x0(i)=di0 x1(i)=di1 | NsymL=Nsym0=Nsym1 |
| 4 | 1 | x0(i)=d4i0 x1(i)=d4i+10 x2(i)=d4i+20 x3(i)=d4i+30 | NsymL=Nsym04 |
| 4 | 2 | x0(i)=d2i0 x1(i)=d2i+11 x2(i)=d2i0 x3(i)=d2i+11 | NsymL=Nsym02=Nsym12 |

  - Transmit diversity

Transmit diversity is created by transmitting the same information from multiple antennas. The mapping of modulation symbols to layers is shown in Table 11.

- —Mapping for transmit diversity

| No of layers | No of codewords | Mapping | Parameter |
| --- | --- | --- | --- |
| 2 | 1 | x0(i)=d2i0 x1(i)=d2i+11 | NsymL=Nsym02 |
| 4 | 1 | x0(i)=d4i0 x1(i)=d4i+10 x2(i)=d4i+20 x0(i)=d4i+30 | NsymL=Nsym04Nsym0+m4 If NsymMod4=0 If NsymMod4≠0 |

If NsymqMod4≠0 add m null symbols at the end such that Nsymq+m Mod4=0.

- Precoding

The block of symbols obtained from the layer mapping, x, are mapped onto the block of symbols, y, to be transmitted by P antennas as illustrated in Figure 13.

- —Precoding mapping

  - Single antenna mapping

Transmission with a single antenna, precoding is defined by

y0(i)=x0(i)

where i=0,1,…,NsymP−1 and NsymP=NsymLis the number of symbols transmitted per antenna.

  - Open loop spatial multiplexing

Precoding for open loop spatial multiplexing delivers performance robustness by feeding back the channel’s rank. Such channel’s rank is indicated by 2 bits in the control channel of the TDD frame.  Consequently, transmitter can choose a pre-fixed codeword according to the channel’s rank.

Precoding for multiple antennas is defined by

y0(i)⋮yP−1(i)=W(i)x0(i)⋮xν−1(i)

where i=0,1,…,NsymP−1 and NsymP=NsymL. The precoding codeword matrix W is chosen by the transmitter according to the reported channel’s rank.

The codebook for P=2 antennas is given in Table 12 and P=4 antennas is given in Table 13.

- —Codebook for transmission on 2 antennas

| Index | ν=1 | ν=2 |
| --- | --- | --- |
| 0 | 1211 | 121001 |
| 1 | 121−1 | 12111−1 |
| 2 | 121j | 1211j−j |
| 3 | 121−j |  |

j=eπ2

The codebook for 4 antennas is based on the Householder theorem: If x and y are vectors with the same norm, then exists an orthogonal symmetric matrix W such that y=Wx, where  W=I−2uuT and ||u||=1.

Since W is orthogonal and symmetric, then W=W−1, simplifying the implementation complexity considerably.

- —Codebook for transmission on 4 antennas

| n | un | ν=1 | ν=2 | ν=3 | ν=4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1−1−1−1T | W01 | W0142 | W01243 | W012342 |
| 1 | 1−j−1jT | W11 | W1122 | W11233 | W112342 |
| 2 | 11−11T | W21 | W2122 | W21233 | W232142 |
| 3 | 1j1−jT | W31 | W3122 | W31233 | W332142 |
| 4 | 1(−1−j)2−j1T | W41 | W4142 | W41243 | W412342 |
| 5 | 1(−1+j)2−j(−1−j)2T | W51 | W5142 | W51243 | W512342 |
| 6 | 1(1+j)2−j(−1+j)2T | W61 | W6132 | W61343 | W613242 |
| 7 | 1(−1+j)2j(1+j)2T | W71 | W7132 | W71343 | W713242 |
| 8 | 1−111T | W81 | W8122 | W81243 | W812342 |
| 9 | 1−j−1−jT | W91 | W9142 | W91343 | W912342 |
| 10 | 111−1T | W101 | W10132 | W101233 | W1013242 |
| 11 | 1j−1jT | W111 | W11132 | W111343 | W1113242 |
| 12 | 1−1−11T | W121 | W12122 | W121233 | W1212342 |
| 13 | 1−11−1T | W131 | W13132 | W131233 | W1313242 |
| 14 | 11−1−1T | W141 | W14132 | W141233 | W1432142 |
| 15 | 1111T | W151 | W15122 | W151233 | W1512342 |

W is conformed for the codebook for 4 antennas as follows: Wic1,...,cm denotes the matrix formed by the columns {c1,…,cm} of the matrix Wi=I4x4−2uiuiH||ui||.

  - Transmit diversity

Transmit diversity is aimed to increase robustness in scenarios with low SNR, low delay tolerance or no feedback to the transmitter is available or reliable.

In case of 2 antennas, the space-frequency block codes (SFBC) are defined by

y0(2i)y0(2i+1)y1(2i)y1(2i+1)=12x0(i)x1(i)−x1(i)x0(i)

for i=0, 1,…, NsymL−1 and NsymP=2NsymL.

In case of 4 antennas a combination of SFBC for 2 antennas with frequency switch transmission diversity is employed and defined as

y0(4i)y0(4i+1)y0(4i+2)y0(4i+3)y1(4i)y1(4i+1)y1(4i+2)y1(4i+3)y2(4i)y2(4i+1)y2(4i+2)y2(4i+3)y3(4i)y3(4i+1)y3(4i+2)y3(4i+3)=12x0(i)x1(i)0000x2(i)x3(i)−x1(i)x0(i)0000−x3(i)x2(i)

for i=0, 1,…, NsymL−1 and NsymP=4NsymL.

- Multicarrier modulation

The multicarrier modulation parameters for either DFT-spread OFDM or OFDM are given in Table 14. The subcarrier spacing of 15 KHz ensures a good compromise for handling delay spread in radio channels and implementation availability.

- —Multicarrier parameters

| Description | Notation |
| --- | --- |
| Total No of subcarriers | MFFT=1024 |
| Subcarrier spacing | ∆f=15 KHz |
| Sampling time | Ts=1/(∆f MFFT)=65.1 nsec |
| Clock rate | Rc=1/Ts=15.36 MHz |

  - Cyclic prefix

The cyclic prefix is chosen according to the typical RMS delay spread of the ISM and sub-GHz bands computed according to the Channel Model Document and shown in Table 15.

- —Typical RMS delay spread

| Frequency | Scenario | RMS delay spread |
| --- | --- | --- |
| 5.2 GHz | Indoor commercial | 190 nsec |
| 5.2 GHz | Indoor office | 60 nsec |
| 5.2 GHz | Indoor residential | 23 nsec |
| 2.4 GHz | Outdoor | 295 nsec |
| 900 MHz | Indoor | 30.55 nsec |
| 900 MHz | Urban | 1.82 usec |

The cyclic prefix length covers 73 sampling points with duration 73Ts=4.75 µsec. this enables to design a slot time of 0.5 msec consisting of 7 DTF-Spread OFDM or OFDM symbols as shown in Figure 14.

- —Slot structure

  - Resource block

Resource block (RB) is a set of time-frequency slots for data communication and enabling multiple access. A RB is formed by a slot time of 0.5 msec (Nsymb=7 OFDM symbols as in Figure 14 and Figure 15) and NscRB=6 subcarriers or 90 KHz spectrum as show in Figure 15. The total number of RBs is NRB=170 (the 2 upper and lower subcarriers are empty).

- —Resource block parameters

Transmission bandwidth (BW) is obtained by concatenating RBs as

BW=nNscRBΔf

where 12 ≤ n ≤ 111.

Several transmission bandwidths are available as shown in Table 16. These cover sub-GHz band as well as 2.4 and 5.7 GHz bands.

- —Transmission bandwidths

| BW (MHz) | No of RBs | No subcarriers | FFT size | Sampling rate |
| --- | --- | --- | --- | --- |
| 1 | 12 | 72 | 128 | 1.92 MHz |
| 3 | 33 | 198 | 256 | 3.84 MHz |
| 5 | 56 | 336 | 512 | 7.68 MHz |
| 10 | 111 | 666 | 1024 | 15.36 MHz |
| 15 | 166 | 996 | 1024 | 15.36 MHz |

For the sub-GHz band, the transmission bandwidth is 1 MHz, while for the 2.4 and 5.7 GHz bands, the maximum transmission considered is 10 MHz. the main reason is that PAC applications require as many channel resources for multiplexing (and consequently multiple access) as possible. For instance, it is preferable to have 15 channels of 10 MHz rather than 7 channels of 20 MHz to accommodate more users.

  - Filtered OFDM symbols for sub-1 GHz band

In order to improve performance, the cyclic prefix is replaced by the coefficients of symmetric Blackman-Harris filter given by

w(n)=a0−a1cos2πnN−1+a2cos4πnN−1+a3cos6πnN−1 ;           0≤n≤N−1a0=0.35875;a1=0.48829;a2=0.14128;a3=0.01168

where N=73/74. The window is applied around a resource block as illustrated in Figure 16.

- —Multicarrier modulation for sub-1 GHz band

  - Carrier aggregation

However, we propose to use carrier aggregation, where one more channel of 10 MHz can be added together to increase the transmission bandwidth to 20 MHz as shown in Figure 17, if the scenario allows it. Such carrier aggregation is granted by the MAC and signaled to the PHY by one bit, G, in the control channel of the TDD frame (see sub-clause 3.1).

- —Carrier aggregation

The aggregated channel can be considered by the PD as a single enlarged channel of 20 MHz from the RF viewpoint. Hence, the same RF front end can be used without modifications.

  - Data rates

Data rates depend on the employed spectrum (number of subcarriers and carrier aggregation), modulation, coding rate, MIMO technology and overhead (pilots, control information, etc.).

The number of subcarriers is given by nNscRBNsym, where NscRB=6subcarriers per RB, Nsym=7OFDM symbols and 12≤n≤111. Every subcarrier conveys a modulation symbol. The number of bits per symbol is Log2(M)where M is the cardinality of modulation. The coding rate CR values are shown in Table 17. The carrier aggregation CA may double the number of employed subcarriers. Open loop spatial multiplexing MIMO, CM, may double or quadruple the capacity.

The different combinations are shown in Table 17 without overhead.

- —Data rate parameters

| n | Log2(M) | Modulation | CA | CA mode | CR | CM | CM mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 1 | BPSK | 1 | disable | 1/2 | 1 | disable |
| ⋮ | 2 | QPSK | 2 | enable | 2/3 | 2 | 2x2 |
|  | 4 | 16QAM |  |  | 3/4 | 4 | 4x4 |
| 111 | 6 | 64QAM |  |  | 5/6 |  |  |

The different data rates are given by

Rb=nNscRBNsym Log2(M)0.5 msecCRCACM

Example: the peak data rate is  given by using all RBs, n=111, carrier aggregation, 64QAM, coding rate of 5/6 and 4x4 MIMO is Rb=372.96 Mbps.

- Reference signals

Reference signals for channel estimation and equalization are based on ZC sequences. Such reference signals must be considered in the time and frequency domain.

  - Time domain

Considering a maximum speed of v=100 Km/h (27.78 m/s), the Doppler spread, fd=fc v/c is given in Table 18.

- —Doppler spread

| fc | fd (Hz) | Tc (msec) |
| --- | --- | --- |
| 5.7 GHz | 527.82 | 0.947 |
| 2.4 GHz | 222.24 | 2.2 |
| 920 MHz | 85.2 | 6 |

The minimum sampling time to reconstruct the channel is computed as Tc=1/2fd , which is also given in Table 18.  The slot time is 0.5 msec. Then, one reference symbol per slot is needed in the time domain to estimate the channel correctly.

  - Frequency domain

Considering the Channel Model Document, the 90% and 50% coherence bandwidth as Bc,90=150στand BC,50=15στ, where στ is the RMS delay spread, such coherence bandwidths can be computed  as

στ=Cadγa

Such coherence bandwidths are shown in Table 19. If BC,50<BWthen the radio channel contains frequency selective fading and equalization is needed.

We propose that the spacing between 2 reference symbols in the frequency domain is 30 KHz to resolve frequency variations.

- —RMS delay spread

| Frequency band | Ca | γa | στ | Bc,90 (KHz) | BC,50 (KHz) |
| --- | --- | --- | --- | --- | --- |
| 5.7 GHz | 10 | 0.51 | 238 nsec | 84 | 840 |
| 2.4 GHz | 55 | 0.27 | 295 nsec | 67 | 678 |
| 920 MHz | 1254.3 | 0.06 | 1.82 usec | 11 | 110 |

- Optional GFSK modulation

An optional and very low power PHY based on CP-2FSK modulation is contemplated for the sub-GHz band with no support for MIMO technologies, i.e., layer mapper and precoding are not necessary. The proposed channel encoder, bit interleaver and scrambler are used as well. The modulation mapper is CP-2FSK that is given by

s(t)=Vs2πfct+2πΔf'−∞tb(t')dt'+ϕ0

where V is amplitude, S(t)=sin(2πfct) is the modulating-carrier signal, fc is  the central carrier frequency, ∆f=β/2Tsym is the peak frequency deviation, Tsym is the symbol time, β=1 is the modulation index, and φ0 is the initial phase of the modulating-carrier signal.

The information bearing signal is given by

b(t)=m(1−2gm)p(t−mTsym)

where gm is information bits, p(t) is a Gaussian pulse shape of bandwidth-symbol duration product of 0.8.

- Operating frequency bands

The frequency bands of operation are Sub-GHz, 2.4 GHz and 5.7 GHz.

Those are selected because they do not require operation license. Hence, implementers need only to comply with local regulations. Moreover, those bands cover all PAC applications in terms of capacity, mobility and operational distance.

PAC applications require discovery and data communication links for many PAC users as possible at moderate data rate.  That is, sacrifice bandwidth against number of users.  This is a different requirement as compared to other standards like WiFi, which requires sacrificing number of users against bandwidth, as it is well documented in the CSMA performance literature.

Hence, PAC applications require as many channel resources for multiplexing (and consequently multiple access) as possible. That is, it is preferable to have 15 channels of 10 MHz rather than 7 channels of 20 MHz to accommodate more users.

Multiplexing is how multiple users communicate simultaneously sharing a common wireless medium without interfering each other. Example: frequency division multiplexing (FDM), time division multiplexing (TDM), space division multiplexing (SDM), etc., or combinations like time-frequency division multiplexing, etc. Multiplexing is provided by the PHY.

Multiple access or channel access is how to allocate such resources in time, frequency or both, to users, even if there are more users than available resources. Example: time division multiple access (TDMA), frequency division multiple access (FDMA), OFDMA, SC-FDMA, CDMA, etc. A multiple access scheme is based on a multiplexing method and control by the MAC.

Using 10 MHz channels with high order modulations and possibly MIMO technologies, PAC applications can have high throughput. We consider that support for high number of user rather than high data rate is a distinct requirement for PAC as compared to other standards, especially Wi-Fi. Coexistence with Wi-Fi and other systems can be achieved with power control, low duty cycle, etc., rather than using the same bandwidth.

However, as stated in clause 11.4, one carrier aggregation can be used to increase the bandwidth to 20 MHz.

  - Channelization of 920 MHz band

The channelization of 920 MHz band by regulations in Japan is shown in Table 20.

- —Sub-GHz channelization by regulations in Japan

| Band | Max Tx power (mW) | Frequency band (MHz) | Basic channelization |
| --- | --- | --- | --- |
| A1 | 1 | 915.9 – 928.1 | 61 channels of 200 KHz |
| B1 | 20 | 920.5 – 928.1 | 38 channels of 200 KHz |
| C1 | 250 | 920.5 – 923.5 | 15 channels of 200 KHz |
| D2 | 1 | 928.1 – 929.7 | 16 channels of 100 KHz |

1bandwidth rule tolerance: (200 n) KHz, where n=1,2,3,4,5.

2bandwidth rule tolerance: (100 n)  KHz, where n=1,2,3,4,5.

The proposed channelization for sub-GHz band in Japan is shown in Table 21.

- —Proposed Sub-GHz channelization (Japan)

| Band | Central frequency (MHz) | n | No of channels | Max Tx power (mW) |
| --- | --- | --- | --- | --- |
| A | fc=917+n | 0,1,…,10 | 11 channels of 1 MHz | 1 |
| B | fc=922+n | 0,1,…,5 | 6 channels of 1 MHz | 20 |
| C | fc=921.5+n | 0,1 | 2 channels of 1 MHz | 250 |
| D | fc=928.7+n | 0,1 | 2 channels of 500 KHz | 1 |

  - Channelization of 2.4 GHz band

The 2.4 GHz band ranges from 2.4 GHz to 2.4835 GHz and it is divided into 8 channels of 10 MHz. The central frequencies are given by

fc=2405 MHz +10n        for n=0,1,…,7

  - Channelization of 5.7 GHz band

The 5.7 GHz band ranges from 5.725 GHz to 5.875 GHz and it is divided into 15 channels of 10 MHz. The central frequencies are given by

fc=5730 MHz +10n      for n=0,1,…,14

# (Normative)
QC-LDPC prototype matrices

Table A.1— Hp for n=648, Z=27 and R=1/2.

| 0 | - | - | - | 0 | 0 | - | - | 0 | - | - | 0 | 1 | 0 | - | - | - | - | - | - | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22 | 0 | - | - | 17 | - | 0 | 0 | 12 | - | - | - | - | 0 | 0 | - | - | - | - | - | - | - | - | - |
| 6 | - | 0 | - | 10 | - | - | - | 24 | - | 0 | - | - | - | 0 | 0 | - | - | - | - | - | - | - | - |
| 2 | - | - | 0 | 20 | - | - | - | 25 | 0 | - | - | - | - | - | 0 | 0 | - | - | - | - | - | - | - |
| 23 | - | - | - | 3 | - | - | - | 0 | - | 9 | 11 | - | - | - | - | 0 | 0 | - | - | - | - | - | - |
| 24 | - | 23 | 1 | 17 | - | 3 | - | 10 | - | - | - | - | - | - | - | - | 0 | 0 | - | - | - | - | - |
| 25 | - | - | - | 8 | - | - | - | 7 | 18 | - | - | 0 | - | - | - | - | - | 0 | 0 | - | - | - | - |
| 13 | 24 | - | - | 0 | - | 8 | - | 6 | - | - | - | - | - | - | - | - | - | - | 0 | 0 | - | - | - |
| 7 | 20 | - | 16 | 22 | 10 | - | - | 23 | - | - | - | - | - | - | - | - | - | - | - | 0 | 0 | - | - |
| 11 | - | - | - | 19 | - | - | - | 13 | - | 3 | 17 | - | - | - | - | - | - | - | - | - | 0 | 0 | - |
| 25 | - | 8 | - | 23 | 18 | - | 14 | 9 | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 0 |
| 3 | - | - | - | 16 | - | - | 2 | 25 | 5 | - | - | 1 | - | - | - | - | - | - | - | - | - | - | 0 |

Table A.2— Hp for n=648, Z=27, R=2/3.

| 25 | 26 | 14 | - | 20 | - | 2 | - | 4 | - | - | 8 | - | 16 | - | 18 | 1 | 0 | - | - | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 9 | 15 | 11 | - | 0 | - | 1 | - | - | 18 | - | 8 | - | 10 | - | - | 0 | 0 | - | - | - | - | - |
| 16 | 2 | 20 | 26 | 21 | - | 6 | - | 1 | 26 | - | 7 | - | - | - | - | - | - | 0 | 0 | - | - | - | - |
| 10 | 13 | 5 | 0 | - | 3 | - | 7 | - | - | 26 | - | - | 13 | - | 16 | - | - | - | 0 | 0 | - | - | - |
| 23 | 14 | 24 | - | 12 | - | 19 | - | 17 | - | - | - | 20 | - | 21 | - | 0 | - | - | - | 0 | 0 | - | - |
| 6 | 22 | 9 | 20 | - | 25 | - | 17 | - | 8 | - | 14 | - | 18 | - | - | - | - | - | - | - | 0 | 0 | - |
| 14 | 23 | 21 | 11 | 20 | - | 24 | - | 18 | - | 19 | - | - | - | - | 22 | - | - | - | - | - | - | 0 | 0 |
| 17 | 11 | 11 | 20 | - | 21 | - | 26 | - | 3 | - | - | 18 | - | 26 | - | 1 | - | - | - | - | - | - | 0 |

Table A.3— Hp for n=648, Z=27, R=3/4.

| 16 | 17 | 22 | 24 | 9 | 3 | 14 | - | 4 | 2 | 7 | - | 26 | - | 2 | - | 21 | - | 1 | 0 | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 12 | 12 | 3 | 3 | 26 | 6 | 21 | - | 15 | 22 | - | 15 | - | 4 | - | - | 16 | - | 0 | 0 | - | - | - |
| 25 | 18 | 26 | 16 | 22 | 23 | 9 | - | 0 | - | 4 | - | 4 | - | 8 | 23 | 11 | - | - | - | 0 | 0 | - | - |
| 9 | 7 | 0 | 1 | 17 | - | - | 7 | 3 | - | 3 | 23 | - | 16 | - | - | 21 | - | 0 | - | - | 0 | 0 | - |
| 24 | 5 | 26 | 7 | 1 | - | - | 15 | 24 | 15 | - | 8 | - | 13 | - | 13 | - | 11 | - | - | - | - | 0 | 0 |
| 2 | 2 | 19 | 14 | 24 | 1 | 15 | 19 | - | 21 | - | 2 | - | 24 | - | 3 | - | 2 | 1 | - | - | - | - | 0 |

Table A.4—Hp for n=648, Z=27, R=5/6.

| 17 | 13 | 8 | 21 | 9 | 3 | 18 | 12 | 10 | 0 | 4 | 15 | 19 | 2 | 5 | 10 | 26 | 19 | 13 | 13 | 1 | 0 | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 12 | 11 | 14 | 11 | 25 | 5 | 18 | 0 | 9 | 2 | 26 | 26 | 10 | 24 | 7 | 14 | 20 | 4 | 2 | - | 0 | 0 | - |
| 22 | 16 | 4 | 3 | 10 | 21 | 12 | 5 | 21 | 14 | 19 | 5 | - | 8 | 5 | 18 | 11 | 5 | 5 | 15 | 0 | - | 0 | 0 |
| 7 | 7 | 14 | 14 | 4 | 16 | 16 | 24 | 24 | 10 | 1 | 7 | 15 | 6 | 10 | 26 | 8 | 18 | 21 | 14 | 1 | - | - | 0 |

Table A.5—Hp for n=1944, Z=81, R=1/2.

| 57 | - | - | - | 50 | - | 11 | - | 50 | - | 79 | - | 1 | 0 | - | - | - | - | - | - | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | - | 28 | - | 0 | - | - | - | 55 | 7 | - | - | - | 0 | 0 | - | - | - | - | - | - | - | - | - |
| 30 | - | - | - | 24 | 37 | - | - | 56 | 14 | - | - | - | - | 0 | 0 | - | - | - | - | - | - | - | - |
| 62 | 53 | - | - | 53 | - | - | 3 | 35 | - | - | - | - | - | - | 0 | 0 | - | - | - | - | - | - | - |
| 40 | - | - | 20 | 66 | - | - | 22 | 28 | - | - | - | - | - | - | - | 0 | 0 | - | - | - | - | - | - |
| 0 | - | - | - | 8 | - | 42 | - | 50 | - | - | 8 | - | - | - | - | - | 0 | 0 | - | - | - | - | - |
| 69 | 79 | 79 | - | - | - | 56 | - | 52 | - | - | - | 0 | - | - | - | - | - | 0 | 0 | - | - | - | - |
| 65 | - | - | - | 38 | 57 | - | - | 72 | - | 27 | - | - | - | - | - | - | - | - | 0 | 0 | - | - | - |
| 64 | - | - | - | 14 | 52 | - | - | 30 | - | - | 32 | - | - | - | - | - | - | - | - | 0 | 0 | - | - |
| - | 45 | - | 70 | 0 | - | - | - | 77 | 9 | - | - | - | - | - | - | - | - | - | - | - | 0 | 0 | - |
| 2 | 56 | - | 57 | 35 | - | - | - | - | - | 12 | - | - | - | - | - | - | - | - | - | - | - | 0 | 0 |
| 24 | - | 61 | - | 60 | - | - | 27 | 51 | - | - | 16 | 1 | - | - | - | - | - | - | - | - | - | - | 0 |

Table A.6—Hp for n=1944, Z=81, R=2/3.

| 61 | 75 | 4 | 63 | 56 | - | - | - | - | - | - | 8 | - | 2 | 17 | 25 | 1 | 0 | - | - | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56 | 74 | 77 | 20 | - | - | - | 64 | 24 | 4 | 67 | - | 7 | - | - | - | - | 0 | 0 | - | - | - | - | - |
| 28 | 21 | 68 | 10 | 7 | 14 | 65 | - | - | - | 23 | - | - | - | 75 | - | - | - | 0 | 0 | - | - | - | - |
| 48 | 38 | 43 | 78 | 76 | - | - | - | - | 5 | 36 | - | 15 | 72 | - | - | - | - | - | 0 | 0 | - | - | - |
| 40 | 2 | 53 | 25 | - | 52 | 62 | - | 20 | - | - | 44 | - | - | - | - | 0 | - | - | - | 0 | 0 | - | - |
| 69 | 23 | 64 | 10 | 22 | - | 21 | - | - | - | - | - | 68 | 23 | 29 | - | - | - | - | - | - | 0 | 0 | - |
| 12 | 0 | 68 | 20 | 55 | 61 | - | 40 | - | - | - | 52 | - | - | - | 44 | - | - | - | - | - | - | 0 | 0 |
| 58 | 8 | 34 | 64 | 78 | - | - | 11 | 78 | 24 | - | - | - | - | - | 58 | 1 | - | - | - | - | - | - | 0 |

Table A.7—Hp for n=1944, Z=81, R=3/4.

| 48 | 29 | 28 | 39 | 9 | 61 | - | - | - | 63 | 45 | 80 | - | - | - | 37 | 32 | 22 | 1 | 0 | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 49 | 42 | 48 | 11 | 30 | - | - | - | 49 | 17 | 41 | 37 | 15 | - | 54 | - | - | - | 0 | 0 | - | - | - |
| 35 | 76 | 78 | 51 | 37 | 35 | 21 | - | 17 | 64 | - | - | - | 59 | 7 | - | - | 32 | - | - | 0 | 0 | - | - |
| 9 | 65 | 44 | 9 | 54 | 56 | 73 | 34 | 42 | - | - | - | 35 | - | - | - | 46 | 39 | 0 | - | - | 0 | 0 | - |
| 3 | 62 | 7 | 80 | 68 | 26 | - | 80 | 55 | - | 36 | - | 26 | - | 9 | - | 72 | - | - | - | - | - | 0 | 0 |
| 26 | 75 | 33 | 21 | 69 | 59 | 3 | 38 | - | - | - | 35 | - | 62 | 36 | 26 | - | - | 1 | - | - | - | - | 0 |

Table A.8—Hp for for n=1944, Z=81, R=5/6.

| 13 | 48 | 80 | 66 | 4 | 74 | 7 | 30 | 76 | 52 | 37 | 60 | - | 49 | 73 | 31 | 74 | 73 | 23 | - | 1 | 0 | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69 | 63 | 74 | 56 | 64 | 77 | 57 | 65 | 6 | 16 | 51 | - | 64 | - | 68 | 9 | 48 | 62 | 54 | 27 | - | 0 | 0 | - |
| 51 | 15 | 0 | 80 | 24 | 25 | 42 | 54 | 44 | 71 | 71 | 9 | 67 | 35 | - | 58 | - | 29 | - | 53 | 0 | - | 0 | 0 |
| 16 | 29 | 36 | 41 | 44 | 56 | 59 | 37 | 50 | 24 | - | 65 | 4 | 65 | 52 | - | 4 | - | 73 | 52 | 1 | - | - | 0 |

# (Normative)
Modulation mapping

Table B.9—BPSK mapping

| bi | I | Q |
| --- | --- | --- |
| 0 | 1/√2 | 1/√2 |
| 1 | −1/√2 | −1/√2 |

Table B.10—QPSK mapping

| bi,bi+1 | I | Q |
| --- | --- | --- |
| 0 0 | 1/√2 | 1/√2 |
| 0 1 | 1/√2 | −1/√2 |
| 1 0 | −1/√2 | 1/√2 |
| 1 1 | −1/√2 | −1/√2 |

Table B.11—16QAM mapping

| bi,bi+1,bi+2,bi+3 | I | Q |
| --- | --- | --- |
| 0 0 0 0 | 1/√10 | 1/√10 |
| 0 0 0 1 | 1/√10 | 3/√10 |
| 0 0 1 0 | 3/√10 | 1/√10 |
| 0 0 1 1 | 3/√10 | 3/√10 |
| 0 1 0 0 | 1/√10 | −1/√10 |
| 0 1 0 1 | 1/√10 | −3/√10 |
| 0 1 1 0 | 3/√10 | −1/√10 |
| 0 1 1 1 | 3/√10 | −3/√10 |
| 1 0 0 0 | −1/√10 | 1/√10 |
| 1 0 0 1 | −1/√10 | 3/√10 |
| 1 0 1 0 | −3/√10 | 1/√10 |
| 1 0 1 1 | −3/√10 | 3/√10 |
| 1 1 0 0 | −1/√10 | −1/√10 |
| 1 1 0 1 | −1/√10 | −3/√10 |
| 1 1 1 0 | −3/√10 | −1/√10 |
| 1 1 1 1 | −3/√10 | −3/√10 |

Table B.12—64QAM mapping

| bi bi+1 bi+2 bi+3 bi+4 bi+5 | I | Q | bi bi+1 bi+2 bi+3 bi+4 bi+5 | I | Q |
| --- | --- | --- | --- | --- | --- |
| 0 0 0 0 0 0 | 342 | 342 | 1 0 0 0 0 0 | −342 | 342 |
| 0 0 0 0 0 1 | 342 | 142 | 1 0 0 0 0 1 | −342 | 142 |
| 0 0 0 0 1 0 | 142 | 342 | 1 0 0 0 1 0 | −142 | 342 |
| 0 0 0 0 1 1 | 142 | 142 | 1 0 0 0 1 1 | −142 | 142 |
| 0 0 0 1 0 0 | 342 | 542 | 1 0 0 1 0 0 | −342 | 542 |
| 0 0 0 1 0 1 | 342 | 742 | 1 0 0 1 0 1 | −342 | 742 |
| 0 0 0 1 1 0 | 142 | 542 | 1 0 0 1 1 0 | −142 | 542 |
| 0 0 0 1 1 1 | 142 | 742 | 1 0 0 1 1 1 | −142 | 742 |
| 0 0 1 0 0 0 | 542 | 342 | 1 0 1 0 0 0 | −542 | 342 |
| 0 0 1 0 0 1 | 542 | 142 | 1 0 1 0 0 1 | −542 | 142 |
| 0 0 1 0 1 0 | 742 | 342 | 1 0 1 0 1 0 | −742 | 342 |
| 0 0 1 0 1 1 | 742 | 142 | 1 0 1 0 1 1 | −742 | 142 |
| 0 0 1 1 0 0 | 542 | 542 | 1 0 1 1 0 0 | −542 | 542 |
| 0 0 1 1 0 1 | 542 | 742 | 1 0 1 1 0 1 | −542 | 742 |
| 0 0 1 1 1 0 | 742 | 542 | 1 0 1 1 1 0 | −742 | 542 |
| 0 0 1 1 1 1 | 742 | 742 | 1 0 1 1 1 1 | −742 | 742 |
| 0 1 0 0 0 0 | 342 | −342 | 1 1 0 0 0 0 | −342 | −342 |
| 0 1 0 0 0 1 | 342 | −142 | 1 1 0 0 0 1 | −342 | −142 |
| 0 1 0 0 1 0 | 142 | −342 | 1 1 0 0 1 0 | −142 | −342 |
| 0 1 0 0 1 1 | 142 | −142 | 1 1 0 0 1 1 | −142 | −142 |
| 0 1 0 1 0 0 | 342 | −542 | 1 1 0 1 0 0 | −342 | −542 |
| 0 1 0 1 0 1 | 342 | −742 | 1 1 0 1 0 1 | −342 | −742 |
| 0 1 0 1 1 0 | 142 | −542 | 1 1 0 1 1 0 | −142 | −542 |
| 0 1 0 1 1 1 | 142 | −742 | 1 1 0 1 1 1 | −142 | −742 |
| 0 1 1 0 0 0 | 542 | −342 | 1 1 1 0 0 0 | −542 | −342 |
| 0 1 1 0 0 1 | 542 | −142 | 1 1 1 0 0 1 | −542 | −142 |
| 0 1 1 0 1 0 | 742 | −342 | 1 1 1 0 1 0 | −742 | −342 |
| 0 1 1 0 1 1 | 742 | −142 | 1 1 1 0 1 1 | −742 | −142 |
| 0 1 1 1 0 0 | 542 | −542 | 1 1 1 1 0 0 | −542 | −542 |
| 0 1 1 1 0 1 | 542 | −742 | 1 1 1 1 0 1 | −542 | −742 |
| 0 1 1 1 1 0 | 742 | −542 | 1 1 1 1 1 0 | −742 | −542 |
| 0 1 1 1 1 1 | 742 | −742 | 1 1 1 1 1 1 | −742 | −742 |

| bi bi+1 bi+2 bi+3 bi+4 bi+5 | I | Q | bi bi+1 bi+2 bi+3 bi+4 bi+5 | I | Q |
| --- | --- | --- | --- | --- | --- |
| 0 0 0 0 0 0 | 342 | 342 | 1 0 0 0 0 0 | −342 | 342 |
| 0 0 0 0 0 1 | 342 | 142 | 1 0 0 0 0 1 | −342 | 142 |
| 0 0 0 0 1 0 | 142 | 342 | 1 0 0 0 1 0 | −142 | 342 |
| 0 0 0 0 1 1 | 142 | 142 | 1 0 0 0 1 1 | −142 | 142 |
| 0 0 0 1 0 0 | 342 | 542 | 1 0 0 1 0 0 | −342 | 542 |
| 0 0 0 1 0 1 | 342 | 742 | 1 0 0 1 0 1 | −342 | 742 |
| 0 0 0 1 1 0 | 142 | 542 | 1 0 0 1 1 0 | −142 | 542 |
| 0 0 0 1 1 1 | 142 | 742 | 1 0 0 1 1 1 | −142 | 742 |
| 0 0 1 0 0 0 | 542 | 342 | 1 0 1 0 0 0 | −542 | 342 |
| 0 0 1 0 0 1 | 542 | 142 | 1 0 1 0 0 1 | −542 | 142 |
| 0 0 1 0 1 0 | 742 | 342 | 1 0 1 0 1 0 | −742 | 342 |
| 0 0 1 0 1 1 | 742 | 142 | 1 0 1 0 1 1 | −742 | 142 |
| 0 0 1 1 0 0 | 542 | 542 | 1 0 1 1 0 0 | −542 | 542 |
| 0 0 1 1 0 1 | 542 | 742 | 1 0 1 1 0 1 | −542 | 742 |
| 0 0 1 1 1 0 | 742 | 542 | 1 0 1 1 1 0 | −742 | 542 |
| 0 0 1 1 1 1 | 742 | 742 | 1 0 1 1 1 1 | −742 | 742 |
| 0 1 0 0 0 0 | 342 | −342 | 1 1 0 0 0 0 | −342 | −342 |
| 0 1 0 0 0 1 | 342 | −142 | 1 1 0 0 0 1 | −342 | −142 |
| 0 1 0 0 1 0 | 142 | −342 | 1 1 0 0 1 0 | −142 | −342 |
| 0 1 0 0 1 1 | 142 | −142 | 1 1 0 0 1 1 | −142 | −142 |
| 0 1 0 1 0 0 | 342 | −542 | 1 1 0 1 0 0 | −342 | −542 |
| 0 1 0 1 0 1 | 342 | −742 | 1 1 0 1 0 1 | −342 | −742 |
| 0 1 0 1 1 0 | 142 | −542 | 1 1 0 1 1 0 | −142 | −542 |
| 0 1 0 1 1 1 | 142 | −742 | 1 1 0 1 1 1 | −142 | −742 |
| 0 1 1 0 0 0 | 542 | −342 | 1 1 1 0 0 0 | −542 | −342 |
| 0 1 1 0 0 1 | 542 | −142 | 1 1 1 0 0 1 | −542 | −142 |
| 0 1 1 0 1 0 | 742 | −342 | 1 1 1 0 1 0 | −742 | −342 |
| 0 1 1 0 1 1 | 742 | −142 | 1 1 1 0 1 1 | −742 | −142 |
| 0 1 1 1 0 0 | 542 | −542 | 1 1 1 1 0 0 | −542 | −542 |
| 0 1 1 1 0 1 | 542 | −742 | 1 1 1 1 0 1 | −542 | −742 |
| 0 1 1 1 1 0 | 742 | −542 | 1 1 1 1 1 0 | −742 | −542 |
| 0 1 1 1 1 1 | 742 | −742 | 1 1 1 1 1 1 | −742 | −742 |

# Bibliography

Bibliographical references are resources that provide additional or helpful material but do not need to be understood or used to implement this standard. Reference to these resources is made for informational use only.