IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Frame Structure Supporting Multi-hop Communications for PAC |
| Date Submitted | May 5th, 2014 |
| Source | [Zhuo Chen, Hongkun Li, Qing Li, Chonggang Wang, Tao Hao] Voice: [610-878-5695 ] [InterDigital Communications] Fax: [610-878-7885 ] [781 3rd Ave. King of Prussia, PA 19406] E-mail: [Qing.Li@InterDigital.com] |
| Re: | In response to Call for Final Contribution to TG8 |
| Abstract | MAC proposal |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Content

1. Definitions, Acronyms, and Abbreviations	3

2. Background	3

3. Overview	4

3.1 Frame Structure for Multi-hop Communication with Dedicated Multi-hop Period	4

3.2 Frame Structure for Multi-hop Communication with Time Reuse	4

4. Detailed Descriptions	4

4.1 Frame Structure with Multi-hop Period	4

4.2 Frame Structure with Time Reuse	8

5. References	16

- Definitions, Acronyms, and Abbreviations

For the purposes of this document, the following terms and definitions apply.

  - Definitions

| Peer | A user or device, or a group of users or devices sharing a group ID, i.e. an MS in 2G, a UE in 3G, or a group of FFDs or RFDs in IEEE802.15 (WPAN). In the multi-hop scenario, a peer could be a hop peer or an End Peer. |
| --- | --- |
| End Peer | In the context of multi-hop, an End Peer is a peer that does not relay or hop any message or data to the other peers in proximity. |
| Hop Peer (Hopper) | In the context of multi-hop, a hop peer is a peer that relays or hops a message/messages or data to the other peers in proximity to extend the radio coverage. The terms “Hop Peer” and “Hopper” are interchangeable in this disclosure. |
| Initiator | The first peer to initiate the first service or application in proximity or the first peer to initiate a service or application in proximity. An Initiator could be either AppInitiator or ProxInitiator as defined below. |
| Context | Special situation information such as service, application, location, time, power, etc. |
| Tier | Ring based divisions of two-dimensional space to guarantee peers in non-adjacent Tiers do not interfere with each other |
| Sector | Angular divisions of Tiers to guarantee peers in non-adjacent sectors do not interfere with each other. |

  - Acronyms and abbreviations

CAP	Contention Access Period

CFP 	Contention Free Period

P2P	Peer-to-Peer

P2PNW	Peer-to-Peer Network

PAC	Peer Aware Communications

- Background

Multi-hop operation is required by both Technique Guidance Document (TGD) [2] as well as PAC Framework Document (PFD) [1] to extend the network coverage for PAC. A frame structure shall be developed to fully support multi-hop operation at MAC, especially for distributed and infrastructure-less P2P networks (P2PNWs).

The general superframe in 802.15.4, shown in Figure 1, consists of contention period and contention free period.  Contention free period is pre-allocated to specific users. The allocation is determined within CAP.

Figure 1: An Example of General Superframe Structure in 802.15.4

- Overview

This document presents frame structures that are defined for supporting multi-hop communication in peer aware communication.

  - Frame Structure for Multi-hop Communication with Dedicated Multi-hop Period

A multi-hop period is inserted in superframe for supporting multi-hop communication with the following benefits:

- Reducing the impact on one-hop communication. Many use cases in PAC require only one-hop communication.

- Reducing the contention and potential collision among different hops, especially for those high data rate and high QoS applications.

- Introducing more flexibility of frame formation for distributed P2PNW.

The multi-hop period is reserved for only multi-hop communication, and all the one-hop communications are scheduled in one-hop period.

  - Frame Structure for Multi-hop Communication with Time Reuse

The two-dimensional space is divided into several areas and multi-hop frame structures are formalized based on location information. The frame structure enable pairs of communications that are far away from each other can happen simultaneously without interference, thus increases the capacity of PAC via time reuse.

- Detailed Descriptions

  - Frame Structure with Multi-hop Period

This subsection introduces 2 frame structures for multi-hop P2P communication by inserting a dedicated Multi-hop Period for multi-hop communication. Figure 2 presents a simple multi-hop topology as an example for further illustration of the frame formation.

Figure 2: An Example of Multi-hop Communication

    - Multi-hop Period at the End of a Superframe

Figure 3 shows the frame structure with a Multi-hop Period at the end of a Superframe for multi-hop P2P communications in proximity, where a Superframe consists of:

- A Superframe Beacon to indicate the start of a Superframe

- A Superframe (SF) Common Period after the Superframe Beacon for communications among different P2P networks in proximity

- A One-hop Period contains a frame or frames, which may be allocated to same application or different applications (e.g. Frame1 is for Application1, and Frame 2 is for Application2) for One-hop P2P communications

- A Multi-hop Period dedicated for multi-hop P2P communications. For example, Hopper B relays the packets from Initiator A to Hopper C. Hopper C relays the 3-hop packets originated from Initiator A to End Peer D.

- An Inactive period reserved for future allocation

More specifically, an initiator defines the Superframe structure by broadcasting the Superframe Beacon, which may include the following information:

- The length of the Superframe Common Period

- The number of frames in this Superframe, and the length for each defined frame

- The length of the Multi-hop Period

- The length of Inactive Period

The Superframe Beacon should indicate the beginning of a Superframe, which is followed by Superframe Common Period. The “common” implies that this period is accessible for each P2PNW in proximity. This Common Period could be used for requesting channel resource in the Superframe. A frame beacon should be used to indicate the beginning of a frame within the one-hop period. The frame beacon may define the structure of a frame, such as

- Frame length

- Time slot allocation, i.e. which slot is allocated to which peer

The frame structure is flexible, and is determined by the peer which sends out the frame beacon, i.e. the peer formulating the frame. The frame(s) in the one-hop period shall be used only for one-hop communication, for example, the communication between initiator A and hopper B in Figure 2. The multi-hop period follows the one-hop period, and only P2PNWs requiring multi-hop communication should be active in the multi-hop period.

The multi-hop period is reserved for all the multi-hop communications, for example, communication between hopper B and hopper C, and communication between hopper C and end peer D. Each hopper/end peer should follow a hopper beacon from its associated hopper. The allocation of multi-hop period is determined in the superframe common period, which means each P2PNW could request for channel resource in the multi-hop period as needed. The hopper beacon should specify the length of period allocated for the hopper and its peers, for example, hopper B in Figure 2 should manage hopper C through hopper beacon, so that hopper C knows which time period to communicate with hopper B. Hopper C manages the end peer D in the same manner.

Figure 3: Superframe with a Multi-hop Period Inserted at the End of a Superframe

Figure 4 shows an example on how to formulate the superframe, one-hop period and multi-hop period, and how to use different types of beacon to maintain the superframe structure. Since some hopper and end peer may not hear the superframe beacon or frame beacon, they should be aware of only a part of time period in multi-hop period by obtaining the hopper beacon. For example, end peer D is aware of only time period defined by the hopper beacon from hopper C, and communicates with hopper C within that period.

Figure 4: An Example of Frame Formation with Multi-hop Period at the End of a Superframe

    - Multi-hop Period at the End of each Frame

Figure 5 shows the frame structure with a Multi-hop Period at the end of each frame for the multi-hop peer-to-peer communications, where a Superframe consists of:

- A Superframe Beacon to indicate the start of the Superframe

- A Superframe (SF) Common Period after the Superframe Beacon

- A frame or frames for P2P communications, each frame consists of:

  - A Frame Common Period for all the peers active in the frame, which is contention based

  - A contention free One-hop Period  allocated to peers for one-hop communications

  - A Multi-hop Period reserved to the peers for multi-hop communications

- An Inactive period reserved for future allocation

Figure 5: Superframe with a Multi-hop Period Inserted at the End of each Frame

The multi-hop period is moved to the end of a frame, which means frame beacon defines the multi-hop period for each frame. For example, a frame beacon could indicate whether there is a multi-hop period at the end of this frame or the length of multi-hop period if there is a multi-hop period. Superframe beacon should not contain the multi-hop period information. The multi-hop period in a frame is allocated to different peers, which require the multi-hop communication. This allocation process is done during the frame common period, following the frame beacon.

Figure 6 shows an example on how to formulate the superframe and maintain the structure by using different types of beacons. In the frame common period, peers should request the channel resource at the end of this frame, and send out hopper beacon to indicate the time period for multi-hop communication at designated position. For example, hopper B is allocated for a time period in multi-hop period of frame 2. The allocation is determined in frame common period, and hopper B broadcasts its hopper beacon at designated time. Hopper C receives the hopper beacon, and starts communication with hopper B. Hopper C and end peer D communicate with each other following the same manner.

Figure 6: An Example of Frame Formation with Multi-hop Period at the End of each Frame

  - Frame Structure with Time Reuse

The topology in Figure 7 shows a multi-hop communication scenario for one application. The topology is used as an example to describes multi-hop frame structures with Time reuse. In this topology, peer 1 is the Initiator; peer 1.1.1.1and peer 1.2.1.1.1 are End Peers; all other peers are Hoppers. Table 1 summarizes notations used in this document.

Figure 7: An Example of Multi-Hop Topology

Table 1: Notations Used in this document

| Parameter | Description |
| --- | --- |
| R | The radius of Tier 1, or the difference between outer radius and inner radius of Tier p (p > 1) |
| N | Number of non-overlap Tier-frames in a superframe |
| M | Number of Tier-frames overlapped |
| d | The distance between a peer and the Initiator |
| i | Tier-frame number |
| Ih | Hopper’s Tier-frame number |
| p | Tier number |
| Sectorp,q | The qth Sector in Tier p |
| ASetp,q | Adjacent Sector Set of Sectorp,q |
| Subi,s | The sth Hopper-subframe in Tier-frame i |
| PSeti,s | Peers communicate in the sth Hopper-subframe in Tier-frame i |
| SSeti,s | Sector Set of the sth Hopper-subframe in Tier-frame i |
| ISeti,s | Interference Sector Set of the sth Hopper-subframe in Tier-frame i |

    - Tier Based Frames Structure for Inter-Tier Time Reuse

The two dimensional space is divided into Tiers as shown in. This division guarantees that peers in non-adjacent Tiers will not interfere with each other. The division should obey following rules:

- The Initiator denoted as Tier 0 is at the center of all Tiers.

- The area of Tier 1 is a circle with radius R centered at the Initiator. All peers located in this range are in Tier 1 except the Initiator.

- The area of Tier p (p>1) is a region bounded by two concentric circles centered at the Initiator, where the radius of larger circle is p*R and the radius of the smaller one is (p-1)*R. In other words, a peer is in Tier p (p >0) if its distance d from the Initiator is within the range ((p -1)R, p*R].

- The minimum value of R, Rmin, is the maximum transmission range allowed of all peers in the network, which is calculated by

  - is the maximum transmission power allowed for the application in the network.

  - is the receiver sensitivity for peers in the network.

  - is inverse of the Path Loss function

Table 2 gives some typical values of Rmin for PAC.

Table 2: Example Value of Rmin Using 802.15.8 Path Loss Function

| Rmin (meter) | Maximum transmission power for the application (dBm) | Receiver sensitivity (dBm) |
| --- | --- | --- |
| 52 | 10 | -82 |
| 57 | 20 | -82 |
| 61 | 20 | -92 |

Figure 8: Two-dimensional Space is Divided into Several Tiers

Peers in different Tiers that are at least 2 Tiers away from each other can use the same channel without interfering with each other. For example in Figure 8, peers in Tier 1 and peers in Tier 4 can communicate simultaneously without interfering with each other.

A superframe should consist of several Tier-frames and/or reserved frame which can overlap in time based on the following rule.

Tier-frame p and Tier-frame p’ can overlap in time, if

,

where N is the number of non-overlapped Tier-frames in a superframe.

For example, if N equals to 3, the Tier-frame 0, Tier-frame 3 and Tier-frame 3*M can overlap in time as shown in Figure 9.

A Tier-frame consists of several Hopper-subframes and/or unallocated subframes except Tier-frame 0 which only has an Initiator-subframe and/or unallocated subframe.

To maintain the network

- The Initiator sends Superframe Beacon at the beginning of a Superframe. The Superframe beacon contains but is not limited to fields in Table 3. The Initiator and its peers communicate within Initiator-subframe.

- A Hopper in Tier p will allocate a Hopper-subframe in Tier-frame p. The Hopper sends Hopper-beacon at the beginning of the allocated Hopper-subframe. The Hopper beacon contains but is not limited to fields in Table 4. The Hopper and its peers communicate within its Hopper-subframe.

In this document, we assume each peer knows its own location, e.g. geographic coordinates or relative location via Geographic Coordinate System (GPS) or Inertial Navigation System (INS). How a peer to get its location is out of scope of this document. The Initiator shall periodically broadcast its current location to all peers in the network via Hoppers. When a peer receives a Superframe Beacon or Hopper Beacon, it shall update its Tier number

p = ,

where d is the distance between the peer and the Initiator.

Note: in this document,  is the smallest integer not less than x and  is the largest integer not greater than x.

Figure 9: Tier Based Frame Structure for Inter-Tier Time Reuse

Table 3: Fields in Superframe Beacon Frame..

| Field | Description |
| --- | --- |
| Initiator location | Indicate the location, e.g. geographic coordinates or relative location (e.g. via GPS or INS). |
| R | The radius of Tier 1, or the difference between outer radius and inner radius of Tier p (p > 1) |
| N | Number of non-overlapped Tier-frames in a superframe |
| Tier-frame 1 start offset | Indicate the offset from the beginning of the superframe to the start of Tier-frame 1 |
| Tier-frame 1 end offset | Indicate the offset from the beginning of the superframe to the end of Tier-frame 1 |
| … |  |
| Tier-frame N-1 start offset | Indicate the offset from the beginning of the superframe to the start of Tier-frame N-1 |
| Tier-frame N-1 end offset | Indicate the offset from the beginning of the superframe to the end of Tier-frame N-1 |

Note: The offset in the table can be slots or time

Table 4: Fields in Hopper Beacon Frame.

| Field | Description |
| --- | --- |
| Initiator location | Indicate the location, e.g. geographic coordinates or relative location (e.g. via GPS or INS). |
| Hopper’s Tier-frame number | Indicate the Tier-frame number of the Hopper. The field is set to 0 for Superframe Beacon |
| R | The radius of Tier 1, or the difference between outer radius and inner radius of Tier p (p > 1) |
| Beacon start offset | Indicate the offset of the beacon from the start of its Tier-frame. |
| Next Tier-frame offset | Indicate the offset of next Tier-frame from the beacon |
| N (optional) | Number of non-overlapped Tier-frames in a superframe |
| Tier-frame 1 start offset | Indicate the offset from the beginning of the superframe to the start of Tier-frame 1 |
| Tier-frame 1 end offset | Indicate the offset from the beginning of the superframe to the end of Tier-frame 1 |
| … |  |
| Tier-frame N-1 start offset | Indicate the offset from the beginning of the superframe to the start of Tier-frame N-1 |
| Tier-frame N-1 end offset | Indicate the offset from the beginning of the superframe to the end of Tier-frame N-1 |

Note: The offset in the table can be slots or time.

    - Sector Based Hopper-subframe Structure for Intra-Tier Time Reuse

Hopper-subframes within the same Tier, i.e. Intra-Tier, can also overlap in time if peers in any of Hopper-subframes will not interfere with peers in all other Hopper-subframes as shown in Figure 10.

Figure 10 Hopper-subframe Structure for Intra-Tier Time Reuse

In order to quickly determine whether two Hopper-subframes can overlap without complicated computations, a Tier is further divided into several sectors. For example as shown in Figure 11, we uniformly divide Tier p into

- Sectors

- Tier 0 is not divided into Sectors.

This division guarantees the shortest distance between non-adjacent boundary lines of a sector is always larger than R.

Figure 11: A Tier is Divided into Several Sectors

Sector can be number based on following rules

- If a polar coordinate that chooses the Initiator as the pole is deployed, Sectors within a Tier can be numbered counter clockwise starting from azimuth 0.

- The qth Sector in Tier p is denoted as Sectorp,q. For example, Sector4,2, it the 2nd Sector in Tier 4.

- Peers on the boundary of two Sectors in the same Tier always choose the Sector with smaller index number.

- Peers will obtain or update its Sector number when it receives a Hopper beacon from its Hopper or a Superframe beacon from the Initiator. In this specification, Hopper selection process is done. In other words, a peer knows who its Hopper is. How to select a hopper is out of scope of this specification.

After dividing the space into Sectors, following concepts can be defined

- Adjacent Sector: Two sectors are adjacent sectors if they have a common line or point on the boundary.

- Adjacent Sector Set (ASetp,q): Each sector Sectorp,q has an Adjacent Sector Set ASetp,q which contains all Sectors that are adjacent to it.

  - If p is odd and q is odd,

    - ASetp,q ={Sectorp-1,(q-1)/2, Sectorp-1,(q+1)/2, Sectorp,q-1, Sectorp,q+1, Sectorp+1,q-1, Sectorp+1,q, Sectorp+1,q+1};

  - If p is odd and q is even,

    - ASetp,q ={Sectorp-1,q/2, Sectorp-1,q/2+1, Sectorp,q-1, Sectorp,q+1, Sectorp+1,q-1, Sectorp+1,q, Sectorp+1,q+1};

  - If p is even,

    - ASetp,q ={Sectorp-1,q-1, Sectorp-1,q, Sectorp-1,q+1, Sectorp,q-1, Sectorp,q+1, Sectorp+1,2q-2, Sectorp+1,2q-1, Sectorp+1,2q, Sectorp+1,2q+1};

For example, The Adjacent Sectors Set of Sector3,2

ASet3,2 = { Sector2,1, Sector2,2, Sector3,1, Sector3,3, Sector4,1, Sector4,2, Sector4,3}

as shown in Figure 11. Peers in Sector3,2 will not interfere with peers that are not in its sector and Adjacent Sectors Set, since the shortest distance between non adjacent boundary lines of a sector is always larger than R.

- Peer Set (PSeti,s):  The sth Hopper-subframe in Tier-frame i has a Peer Set PSeti,s which contains all peers that communicate in the Hopper-subframe.  For example as shown in Figure 11, the PSet of the Hopper-subframe allocated to Hopper 1.2.1.1 is {Hopper 1.2.1.1, End Peer 1.2.1.1.1}. Each Hopper is allocated a Hopper-subframe in its Tier-frame, i.e.Tier-frame i, for communication among peers in its PSet.

- Sector Set (SSeti,s): The sth Hopper-subframe in Tier-frame i has a Sector Set SSeti,s which is the union of sectors of all peers in its PSeti,s. For example as shown in Figure 11, the Sector Set of Hopper-subframe allocated to Hopper 1.2.1.1 is {Sector3,11, Sector4,11}.

- Interference Sector Set (ISeti,s):  The sth Hopper-subframe in Tier-frame i has an Interference Sector Set ISeti,s, which is the union of Adjacent Sectors Set of peers in its PSeti,s. For example as shown in Figure 11, the Interference Sector Set of Hopper-subframe allocated to Hopper 1.2.1.1  is

ASet3,1  ASet3,2 = { Sector2,6, Sector2,5, Sector3,11, Sector3,10,  Sector3,12 Sector4,10, Sector4,11, Sector4,12}.

Peers that communicate in Hopper 1.2.1.1 subframe will not interfere with peers that are not in the Interference Set.

Therefore, two Hopper-subframe Subi,s and Subi,s’ in the same Tier frame i can overlap in time if and only if peers that communicate in Hopper-subframe Subi,s  do not interfere peers that communicate in Hopper-subframe Subi,s’. In others words, two Hopper-subframe Subi,s and Subi,s’ can overlap in time if either

- no peer that communicate in Hopper-subframe Subi,s is in ISeti,s’, i.e. SSeti,s ∩ ISeti,s’ =, or

- no peer that communicate in Hopper-subframe Subi,s’ is in ISeti,s, i.e. SSeti,s’ ∩ ISeti,s =).

Figure 12 gives an example of Multi-hop Frame Structure with Inter and Intra Tier Frame Time Reuse for Network Topology in Figure 7. The blue circle highlights the Tier based inter-Tier frame time reuse and the yellow circle highlights the Sector based intra-Tier frame time reuse.

Figure 12: Multi-hop Frame Structure with Inter and Intra Tier Frame Time Reuse for Network Topology in Figure 7

- References

- IEEE 802.15.8, TG8 PAC Framework Document (Draft), doc.: IEEE 802.15-14-0085-01-0008, January 2014.

- IEEE Std 802.15.4-2011, IEEE Standard for Information technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements – Part 15.4: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Low-Rate Wireless Personal Area Networks (WPANs).