IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3d Technical Requirements Document |  |
| Date Submitted | [September, 2014] |  |
| Source | Thomas Kürner | E-mail: t.kuerner@tu-bs.de |
| Re: |  |  |
| Abstract | TG3d System Requirements to be developed |  |
| Purpose | Supporting document for the development of the amendment 3d of IEEE 802.15.3 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

| List of contributors |  |
| --- | --- |
| Kiyoshi Toshimitsu | Toshiba Corporation |
| Ichiro Seto | Toshiba Corporation |
| Ken Hiraga | NTT Corporation |
| Keiji Akiyama | Sony Corporation |
| Thomas Kürner | TU Braunschweig |

Table of Contents

1	Definitions:	4

2	General Guidelines [1]	4

3	Introduction	7

4	Protocol Reference Model	7

5	Use case summary	8

6	TRD Summary	8

7	Topology	9

9	Data Rates	10

10	Operational frequency bands	11

11	Coexistence/protection existing services and other use cases	11

11.1	Potentially Critical Interference Scenarios in the THz frequency band	11

11.1.1	Nomadic devices operated outdoors in rural environment	12

11.1.2	Nomadic devices operated outdoors in urban environment	12

11.1.3	Fixed links with scattering objects close to direct ray path	13

11.1.4	Relevant emission from antenna sidelobes	13

11.1.5	Airborne operated transmitters	14

11.1.6	Satellite in limb scanning mode	15

11.2	Spectral Masks between Active Services and Earth Exploration Services (EESS)	15

11.2.1	Maximum equivalent isotropically radiated power (EIRP)	15

11.2.2	Definition of transmit power spectrum masks	16

12	Regulatory requirements	18

13	Transmission range	19

14	Antenna gain and required alignment accuracy	19

14.1	Outdoor Fixed Wireless Gain and Accuracy	19

15	Channel models	22

16	Link budget and SNR analysis	22

17	Size, Weight and Power	24

18	Link Switching	25

19	Beam steered or control channel assisted device discovery	25

20	Media access mechanism and superframe expectations	25

20.1	Multiband operation	25

20.2	Half-/Full-duplex Operation	25

20.3	“Touch and Get” operation	26

21	Security mechanisms	27

22	I/O Interfaces and Memory Buffer Considerations	28

23	Fast connection setup scheme	28

24	References	29

# Definitions:

# General Guidelines [1]

This technical requirement document (TRD) describes the technical aspects that TG3d standard must fulfill, such as performance-related issues, reliability issues and availability issues. These types of requirements are often called quality of service (QoS) requirements; other requirements are usually maintenance-level requirements or external constraints, sometimes called compliance. Technical requirements are summarized as any other specifications; they have a name and a unique identifier. Technical requirements are documented in the same manner as any specifications, including a description, an example, a source or references to related technical requirements and a revision history.

TG3d  needs to effectively define and manage requirements to ensure they are meeting needs of the applications, while proving compliance.

Ideally, requirements are:

• Correct (technically and legally possible)

• Complete (express a whole idea or statement)

• Clear (unambiguous and not confusing)

• Consistent (not in conflict with other requirements)

• Verifiable (it can be determined that the system meets the requirement)

• Traceable (uniquely identified and trackable)

• Feasible (can be accomplished within cost and schedule)

• Modular (can be changed without excessive impact)

• Design-independent (does not pose a specific solution on design)

Each requirement must first form a complete sentence, containing a subject and a predicate. These sentences must consistently use the verb “shall”, “will” or “must” to show the requirement's mandatory nature, and “should” or “may” to show that the requirement is optional. The whole requirement specifies a desired end goal or result and contains a success criterion or other measurable indication of the quality.

The TRD needs to capture these levels of user requirements, maintaining intelligent traceability and change impact analysis between them.

Typical constraint requirements can specify:

• Performance

• Interfaces

• Security

• Safety

• Reliability

• Availability

• Maintainability

An efficient way of writing better requirements is to ensure they are clearly mapped to test cases. Making sure each requirement is clearly verifiable from the start, not only helps prepare later phases of the project, it also puts the developer in the correct state of mind. Requirements and their associated tests must also indicate what the system should not do, and what happens at the limits (degraded mode).

This rule also applies for compliance requirements: indicating how they shall be tested is a good way to write better requirements.

The TRD need to implement a reliable and repeatable change control process that helps turn this challenge into an opportunity.

By providing examples and counter-examples of good requirements and documents, IEEE can enhance the quality, consistency, and completeness of the requirements. These can originally be templates, industry standards and rules inside a repository, such as the IEEE server.

Requirement typical sentence construction

Defects to avoid:

- Vagueness

- Weakness

- Over specification

- Subjectivity

- Multiplicity

- Unclear meaning

- Implicit meaning

Some words to be used with caution:

“adequate”, “applicable”, “appropriate”, “approximate”, “bad”, “best practice”, “between”, “clearly”, “compatible”, “completely”, “consider”, “could”, “down to”, “easy/easily”, “effective”, “efficient”, “equivalent”, “excellent”, “good”, “his/her”, “however”, “ideal”, “etc”, “in order to”, “include but shall not be limited to”, “least”, “like”, “low”, “maximise”, “may”, “most”, “minimum/mal”, “must”, “nearly”, “necessary”, “needed”, “normal”, “or”, “possible/bly“, “practicable”, “provide”, “quality”, “readily”, “relevant”, “safe/ly“, “same”, “should”, “significant”, “similar”, “so as”, “subject to”, “substantial”, “sufficient”, “suitable”, “support”, “target”, “typical”, “up to”, “user friendly”, “whether”, “will”, “with”, “worse”.

# Introduction

This document provides the technical contents of the project to develop the amendment 3d to IEEE 802.15.3 to enable data rates of 100 Gbps for switched point-to-point according to the PAR and CSD of this project [2,3]. This document will provide guidance on how to respond to a call for proposals.

# Protocol Reference Model

The communication protocol reference model used for this document is shown in Figure 1.

PMD: Physical Medium Dependent (radio)

PHY_SAP: PHY Service Access Point

PLCP: PHY Layer Convergence Protocol

MAC_SAP: MAC Service Access Point

Figure 1: Reference partitioning

# Use case summary

The use cases and applications with performance and functional requirements are described in the Applicatons Requirements Document (ARD) [4].

# TRD Summary

This clause contains an overall summary of the rest of the document, mentioning the highlights such as PHY types (assuming multiple options), data rates, topology options (e.g. star, point-to-point, etc.), and MAC frame formats. (Abstracts of chapter 7 and thereafter will be written here.)

# Topology

This clause identifies operational topologies

Close Proximity P2P (60 GHz): For the point-to-point (P2P) system the topology is always limited to two active devices. Topology is required to be limited to Point-to-Point (P2P).

# Data Rates

The data rates should be sufficient to support the proposed use cases in conjunction with the operational frequency plan and channel model.  This section discusses the data rates, which usually is the bases for different PHY type classes.

Data rates are described in the ApplicationsRequirementsDocument (ARD)[4].

# Operational frequency bands

This section secribes the operational frequency bands. The prject foresees various different bands including 60 GHz, the THz frequency band and free space optics.

The THz frequency range is quite large, with some frequencies exhibiting better propagation qualities than others.  Also influencing this issue are practical limits on implementation physics.  In this section the operational frequency bands are discussed.  Each use case may operate in a different band.

Close Proximity P2P (60 GHz): The system uses the 60GHz unlicensed band. The channel plan is the same as that of IEEE802.15.3c. Ch2 and Ch3 in this band aredesignated as unlicensed bands in many countries. Hence the system shall support the use of these two channels.

# Coexistence/protection existing services and other use cases

The THz bands are currently used for earth science research (mainly passive) and coexistence/protection of these users is necessary to enable the broad acceptance of the technology.  This section discusses coexistence and protection issues [6], [7].

Also, each use case outlined in [4] must coexist with all other use cases.  In some instances this coexistence will be explicit (i.e. different operation bands), but in other instances the coexistence mechanism might be more implicit.

Close-Proximity P2P:

As for the close proximity P2P system, the individual proposers shall consider coexistence with the following wireless devices: IEEE802.15.3c, IEEE802.11ad, Wireless HD, millimeter wave analog broadcasting systems, and millimeter wave vehicular radars. By guaranteeing the maximum transmission range of this close proximity P2P system, it can coexist with those existing services.

## Potentially Critical Interference Scenarios in the THz frequency band

This is an ideal estimation of maximum occurring interference powers using worst case assumptions throughout all scenarios with no additional attenuation due to the impact of weather.

### Nomadic devices operated outdoors in rural environment

The assumptions are that the nomadic TX is accidentally pointed in immediate skyward direction as a satellite is passing overhead, directly looking at the ground (0° zenith angle, shortest path).  Also it is assumed that a ground reflection may superimpose constructively with the direct path towards the satellite

### Nomadic devices operated outdoors in urban environment

In this scenario we assume that no objects shadow the direct ray path and that additionally the reflections from buildings around the TX coherently combine at the satellite.

Indoor-operated devices are not relevant due to transmission losses; therefore, indoor setups are implicitly covered by the outdoor scenario.

### Fixed links with scattering objects close to direct ray path

In this scenario, objects close to the direct ray path may scatter/reflect power in a skyward direction towards the satellite.  This scenario is possible despite highly directive antennas.

### Relevant emission from antenna sidelobes

High sidelobe levels may cause non-negligible radiation in a skyward direction with fixed links become especially critical because of potentially high output powers.

### Airborne operated transmitters

In this scenario, radiation through the fuselage or windows is possible, and the transmitted rays may not be attenuated due to the use of composite fuselages.  At higher altitudes we can hardly expect any atmospheric absorption.

### Satellite in limb scanning mode

THz waves may be radiated in the immediate direction of the limb scanner.  Ground-based transmitters do not become relevant due to the long horizontal path.

## Spectral Masks between Active Services and Earth Exploration Services (EESS)

### Maximum equivalent isotropically radiated power (EIRP)

The clause shows how to calculate the maximum equivalent isotropically radiated power (EIRP), which is scenario path loss specific, and includes the satellite antenna gain.  If the TX radiated signal does not exceed the maximum allowed interference power mask, then there will be no interference.

### Definition of transmit power spectrum masks

This clause provides the definition of transmit power spectrum masks and constraints for the isotropically radiated power spectral density.  A simple approximation with line segments  (line parameter sets given in [8]) is used to calculate the effective limitation to several 10 dBm EIRP for bandwidths of several 10 GHz.

#### Nomadic devices in rural and urban areas

#### Fixed links and sidelobes

Fixed links are far less critical than nomadic transmitters and can support higher output powers and longer distances with no significant sidelobe limitations.  These relevant constraints are only applicable below 500 GHz.

#### Airborne TX and limb scanning

Airborne transmitters are most critical with constraints in almost the entire THz range resulting in restrictions requiring lower power and shorter ranges.

# Regulatory requirements

Close Proximity P2P (60 GHz):

The output RF power and other regulationsin each countryallowing the use of the unlicensed 60GHz band shall be followed.

THz:

Devices will need to comply with the regulatory requirements specified for THz devices.  Unique to THz is that these regulatory requirements will be evolving over the next few years.  In this section these issues can be discussed.  All regulatory assumptions should be clearly stated.

Active THz systems will have to comply with the ITU Radio Regulations footnote 5.565 [9].

# Transmission range

The transmission range is driven by use cases in conjunction with the operational frequency plan and channel model.  This section discusses the transmission range, which often is the justification for different PHY types.

Close Proximity P2P (60 GHz): The maximum transmission range should be set to a value is 50 mm [TBD].

# Antenna gain and required alignment accuracy

The antenna gain and required alignment accuracy is driven by use cases.  This section discusses the antenna gain and required alignment accuracy, which may be part of the justification for different PHY types.

Close Proximity P2P (60 GHz): These items are out of scope on this document since they are implementation dependent.

## Outdoor Fixed Wireless Gain and Accuracy

Due to the very high path loss accompanied with THz transmission, for outdoor applications high antenna gains are required. The antenna gain depends on the distance, transmitted data rate, carrier frequency and application. However, an example for a fixed wireless link with a distance of 1 km under worst environmental conditions of a rain rate of 50 mm/h is given in Figure 7. For shorter distances, or better atmospheric conditions, antennas with lower gain can be used. If a transmit power of 10 dBm, a noise figure of 10 dB, and an ambient temperature of 300 K is assumed, the maximum transmittable data rates per GHz bandwidth in a 1 km link are shown in Figure 7 [10].

From Figure 7, very high data rates can only be transmitted if the antenna gains are respectively high. For an antenna gain of 50 dBi for the transmitter and receiver antennas a maximum data rate of 25 Gbps can be transmitted in the 76 GHz bandwidth available in the first window. However, if the antenna gain is increased to 70 dBi, the maximum data rate can be increased to about 860 Gbps in the first transmission window.

Figure 8: Ideal antenna diameter (left and blue) and diameter of the radiated area (right and red) in dependence of the antenna gain for the first THz window and a distance of 1 km

For 70 dBi antenna gain the angle for loss of connectivity due to fluctuation of the pole and the pole twist is just 0.13°. The requirement for adaptive antenna alignments or control mechanisms to compensate for pole sway/twist depends on the grade of sway/twist impairments, given by the antenna installations, e.g. type of pole or building.

# Channel models

The channel models are described in the Channel Modeling Document [5]

# Link budget and SNR analysis

It is likely that each use case will have a unique link budget analysis due to differences in required performance and complexity of modulation formats.  Reference [11] contains useful information in regards to link budget calculations.

Example Link Budget:

Close Proximity P2P (60 GHz):

The usage model is assumed to be P2P communication under line-of-sight propagation in millimeterwave of 60 GHz band. The propagation loss is based on free-space communication under far field, including no obstacles except for terminal cases. Coding gain and target packet error rate would be determined depending on PHY and protocol technologies. The reference table below contains useful information in regards to link budget calculations.

The proposer shall show evidence that the system meets the requirement in section 12 (Transmission Range). For example, the proposer may show the maximum transmission range by system margin being below 0 dB at that range.

Example Link Budget:

# Size, Weight and Power

Size, weight and power (SWaP) concerns arise from use case expectations and influences range and data rate. The antenna is included in the SWaP estimate.   Of primary concern is mobile operation battery life.  This section discusses these issues.

Close-Proximity P2P (60 GHz): The close proximity P2Psystem in millimeter wave is expected to have extremelyshort active time and at least either one of the P2P devices in the system is expected to be mobile. From that sense, the mobile device in this system shall be designed to have low power consumption in stand-by state. The device on the other side, especially such as kiosk, should be designed to have low average power consumption.

Other features regarding this concern are out of scope on this document because it is implementation dependent

# Link Switching

There may be a need to support link switching due to physical movement or interference.  This section discusses these issues.  Obviously this is use case dependent since for some use cases link switching may not be appropriate.

# Beam steered or control channel assisted device discovery

There may be a need to support device discovery, which includes aligning the antenna to close the link.  This section discusses these issues. These parameters may be use case dependent.

# Media access mechanism and superframe expectations

## Multiband operation

- Antenna alignment

- Fast session transfer, for example from 60 GHz to THz

Close Proximity P2P (60 GHz):

The following features may be considered:

- MIMO

- Channel bonding

## Half-/Full-duplex Operation

Methods for a full-duplex MAC were discussed in [12].   These include features such as

- Superframe configuration (i.e. CAP only)

- Beaconing methods

- PNC designation

- PMD modifications to accommodate full-duplex

- Link establishment beaconing protocol

- Asynchronous vs. synchronous full-duplex superframes

- PHY/MAC header fields and preamble definitions

Close Proximity P2P (60 GHz):

The following features may be considered:

- FDD

- TDD

## “Touch and Get” operation

Close Proximity P2P (60 GHz): The close proximity P2P system shall have the following features to achieve its basic requirements:

- Connection setup without any network identifiers

- Network topology always limited to two active devices

- Fast connection setup time prior to active state to meet application requirement

- A means of ensuring spatial division from other systems without beamforming

- CSMA/CA not required prior to data transmission

- No periodic management frame transmission after connection establishment.

- A method to estimate whether a peer device drew apart and a procedure to promptly dissolve connection and change to a standby state when such estimation is made.

The fifthand sixth features are derived from the requirement that the close proximity P2P system shall have an access mechanism which achieves high and secure throughput. The last feature is derived from the requirement to limit the communication range.

# Security mechanisms

Close Proximity P2P (60 GHz): Because of the limitation of the transmission range, security mechanismscan be omitted for a close proximity P2P system.If required, it can be done at the higher layers. Note that security setup may give impact on the application requirement by imposing longer connection setup time.

# I/O Interfaces and Memory Buffer Considerations

Close Proximity P2P (60 GHz): Choice of features regarding these items areup to the implementer.

# Fast connection setup scheme

Close Proximity P2P (60 GHz): Time forconnection setup is the time required to changefrom the standby state to the connected state where each device is ready for data transfer.

To realize “touch and get” user experience, a fast connection setup scheme having a capability to establish connectionswithin20 ms [TBD]is required.

# References

[1] TG6 Technical Requirements Document IEEE 802. 15-08-0644-08-0006

[2] 100G PAR, https://mentor.ieee.org/802.15/dcn/13/15-13-0523-07-0thz-100g-working-draft-par.docx

[3]100G CSD, https://mentor.ieee.org/802.15/dcn/13/15-13-0522-06-0thz-100g-working-draft-5c.docx

[4] Applications Requirmements Document (ARD), https://mentor.ieee.org/802.15/dcn/14/15-14-0304-00-003d-applications-requirement-document-ard.docx

[5] Channel Modelling Document, https://mentor.ieee.org/802.15/dcn/14/15-14-0310-00-003d-channel-modeling-document.docx

[6] S. Priebe, D. M. Britz, M. Jacob, S. Sarkozy, K. M. K. H. Leong, J. E. Logan, B. S. Gorospe, T. Kürner: Interference Investigations of Active Communications and Passive Earth Exploration Services in the THz Frequency Range“, accepted for publication in IEEE Transactions on THz Science and Technology, 14 pages, 2012

[7] S. Priebe: Interference between THz Communications and Spaceborne Earth Exploration Services, IEEE 802.15-12-0324-00-0thz, San Diego, July 2012

[8] S. Priebe, C. Jastrow, M. Jacob, T. Kleine-Ostmann, T. Schrader, T. Kürner: Channel and Propagation Measurements at 300 GHz, IEEE Transactions on Antennas and Propagation, vol. 59, no. 5, pp. 1688–1698, 2011.

[9] Ref. to Radio Regulations

[10] T. Schneider, A. Wiatrek, S. Preußler, M. Grigat, R. P. Braun: Link Budget Analysis for Terahertz Fixed Wireless Links, IEEE Transactions on THz Science and Technology 2, 250 – 256 (2012)

[11] M. Grigat, T. Schneider, S. Preußler, R. P. Braun: Link Budget Considerations for THz Fixed Wireless Links, IEEE 802.15-12-0582-01-0thz, San Antonio, November 2012

[12] Rick Roberts, et. al., 100 Gbps Optical Wireless Tutorial,

http://griffin.meeting.verilan.com/docs/802.15/14/15-14-0016-00-0thz-100-gbps-optical-wireless-tutorial.pdf