IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3d Technical Requirements Document |  |
| Date Submitted | [July2015] |  |
| Source | Thomas Kürner | E-mail: t.kuerner@tu-bs.de |
| Re: |  |  |
| Abstract | TG3d System Requirements to be developed |  |
| Purpose | Supporting document for the development of the amendment 3d of IEEE 802.15.3 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

| List of contributors |  |
| --- | --- |
| Kiyoshi Toshimitsu | Toshiba Corporation |
| Ichiro Seto | Toshiba Corporation |
| Ken Hiraga | NTT Corporation |
| Keiji Akiyama | Sony Corporation |
| Thomas Kürner | TU Braunschweig |
| Sebastian Rey | TU Braunschweig |

Table of Contents

1	Definitions:	4

2	General Guidelines [1]	4

3	Introduction	7

4	Protocol Reference Model	8

5	Use case summary	9

6	TRD Summary	10

7	Topology and Link Switching	11

8	Data Rates	11

9	Transmission range	13

10	Operational frequency bands	14

11	Regulatory Requirements and Coexistence	15

11.1	Requirements from the Radio Regulations	15

11.1.1	Potentially Critical Interference Scenarios in the THz frequency band	15

11.2	Coexistence among different use cases	20

12	Channel models	21

13	Link budget and SNR analysis	22

13.1	Antenna gain and required alignment accuracy	22

13.2	Outdoor Fixed Wireless Gain and Accuracy	23

14	Media Access Mechanism	25

14.1	Beam steered or control channel assisted device discovery	25

14.2	Link Establishment Beaconing Protocol	25

14.3	PHY/MAC header fields and preamble definitions	25

14.4	Requirements MAC for an established link	25

15	I/O Interfaces and Memory Buffer Considerations	26

16	Fast connection setup scheme	27

17	Security mechanisms 	28

18	Size, Weight and Power	29

References	30

# Definitions:

# General Guidelines [1]

This technical requirement document (TRD) describes the technical aspects that TG3d standard must fulfill, such as performance-related issues, reliability issues and availability issues. These types of requirements are often called quality of service (QoS) requirements; other requirements are usually maintenance-level requirements or external constraints, sometimes called compliance. Technical requirements are summarized as any other specifications; they have a name and a unique identifier. Technical requirements are documented in the same manner as any specifications, including a description, an example, a source or references to related technical requirements and a revision history.

TG3d  needs to effectively define and manage requirements to ensure they are meeting needs of the applications, while proving compliance.

Ideally, requirements are:

• Correct (technically and legally possible)

• Complete (express a whole idea or statement)

• Clear (unambiguous and not confusing)

• Consistent (not in conflict with other requirements)

• Verifiable (it can be determined that the system meets the requirement)

• Traceable (uniquely identified and trackable)

• Feasible (can be accomplished within cost and schedule)

• Modular (can be changed without excessive impact)

• Design-independent (does not pose a specific solution on design)

Each requirement must first form a complete sentence, containing a subject and a predicate. These sentences must consistently use the verb “shall”, “will” or “must” to show the requirement's mandatory nature, and “should” or “may” to show that the requirement is optional. The whole requirement specifies a desired end goal or result and contains a success criterion or other measurable indication of the quality.

The TRD needs to capture these levels of user requirements, maintaining intelligent traceability and change impact analysis between them.

Typical constraint requirements can specify:

• Performance

• Interfaces

• Security

• Safety

• Reliability

• Availability

• Maintainability

An efficient way of writing better requirements is to ensure they are clearly mapped to test cases. Making sure each requirement is clearly verifiable from the start, not only helps prepare later phases of the project, it also puts the developer in the correct state of mind. Requirements and their associated tests must also indicate what the system should not do, and what happens at the limits (degraded mode).

This rule also applies for compliance requirements: indicating how they shall be tested is a good way to write better requirements.

The TRD need to implement a reliable and repeatable change control process that helps turn this challenge into an opportunity.

By providing examples and counter-examples of good requirements and documents, IEEE can enhance the quality, consistency, and completeness of the requirements. These can originally be templates, industry standards and rules inside a repository, such as the IEEE server.

Requirement typical sentence construction

Defects to avoid:

- Vagueness

- Weakness

- Over specification

- Subjectivity

- Multiplicity

- Unclear meaning

- Implicit meaning

Some words to be used with caution:

“adequate”, “applicable”, “appropriate”, “approximate”, “bad”, “best practice”, “between”, “clearly”, “compatible”, “completely”, “consider”, “could”, “down to”, “easy/easily”, “effective”, “efficient”, “equivalent”, “excellent”, “good”, “his/her”, “however”, “ideal”, “etc”, “in order to”, “include but shall not be limited to”, “least”, “like”, “low”, “maximise”, “may”, “most”, “minimum/mal”, “must”, “nearly”, “necessary”, “needed”, “normal”, “or”, “possible/bly“, “practicable”, “provide”, “quality”, “readily”, “relevant”, “safe/ly“, “same”, “should”, “significant”, “similar”, “so as”, “subject to”, “substantial”, “sufficient”, “suitable”, “support”, “target”, “typical”, “up to”, “user friendly”, “whether”, “will”, “with”, “worse”.

# Introduction

This document provides the technical requirements of the project to develop the amendment 3d to IEEE 802.15.3 to enable data rates of 100 Gbps for switched point-to-point according to the PAR and CSD of this project [2,3]. This document will provide guidance and requirements on how to respond to a call for proposals.

# Protocol Reference Model

The communication protocol reference model used for this document is shown in Figure 1.

PMD: Physical Medium Dependent (radio)

PHY_SAP: PHY Service Access Point

PLCP: PHY Layer Convergence Protocol

MAC_SAP: MAC Service Access Point

Figure 1: Reference partitioning

# Use case summary

The use cases and applications with performance and functional requirements are described in the Applicatons Requirements Document (ARD) [4].

# TRD Summary

This clause contains an overall summary of the rest of the document, mentioning the highlights such as PHY types (assuming multiple options), data rates, topology options (e.g. star, point-to-point, etc.), and MAC frame formats. (Abstracts of chapter 7 and thereafter will be written here.)

# Topology and Link Switching

This clause identifies operational topologies

In all use cases the topology will be  point-to-point (P2P) and limited to two active devices.  Each of the devices will contain both a transmitter and receiver and is thefore denoted as a transceiver (TRX). It may be necessary to switch between several links, see Figure 7.1. The possibility for link switching between two connections is mandatory. Link switching during a running connection is optional. Obviously this is use case dependent since for some use cases link switching may not be appropriate.

Figure 7.1: Switched Point-to-Point Links; Link 1 TRX1 to TRX2 switched to link 2 TRX1 to TRX 3

# Data Rates

The data rates should be sufficient to support the proposed use cases in conjunction with the operational frequency plan and channel model.  This section discusses the data rates, which usually is the basis for different PHY type classes.

The PHY should at least support the data rates as defined in Table 8.1. For better compatibility with Ethernet a data rate of 40 Gbps may be considered as well

Table 8.1: Data Rates for different use cases

| Use case | Minimum Data Rate in Gb/s | Maximum Data Rate in Gb/s | Required BER after error correction |
| --- | --- | --- | --- |
| Intra-Device Communication | 1 | 100 | 10-12 |
| Close Proximity Communication | tbd | 100 | tbd |
| Wireless Fronthauling | 10 | 100 | 10-12 |
| Wireless Backhauling | tbd | 100 | 10-12 |
| Data Center | 1 | 100 | 10-12 |

# Transmission range

The transmission range is driven by use cases in conjunction with the operational frequency plan and channel model.  This section discusses the transmission range, which often is the justification for different PHY types.

The data rates specified in section 8 shall be achieved for transmissions over the distances specified in table 9.2. For the various use cases different antennas may be applied. In the proposal the antenna characteristics (gain, antenna diagrams) to achieve the transmission distance have to be given (see also section 13).

Table 9.2: Transmission Ranges for different use cases

| Use case | Transmission Distance |
| --- | --- |
| Intra-Device Communication | 10 cm or less |
| Close Proximity Communication | 50 mm or less |
| Wireless Fronthauling | a few hundred meters |
| Wireless Backhauling | a few hundred meters up to a few kilometers |
| Wireless Data Center | 10-100 m |

# Operational frequency bands

This section describes the operational frequency bands.

The THz frequency range is quite large, with some frequencies exhibiting better propagation qualities than others.  Also influencing this issue are practical limits on implementation physics.  In this section the operational frequency bands are discussed.  Each use case may operate in a different band.

The operational frequency bands are from 252 to 325 GHz. The frequency bands 252 to 275 GHz have already an allocation for MOBILE and FIXED services in the radio regulations. The band 275 – 325 GHz is not allocated to any servcie in the radio regulations, but may be used for active services, if the conditions in footnote 5.565 of the radio regulations are met.

<tbd: include options for channelization>

# Regulatory Requirements and Coexistence

The THz bands are currently used for earth science research (mainly passive) and coexistence/protection of these users is necessary to enable the broad acceptance of the technology.  This section discusses coexistence and protection issues [6], [7].

The proposals have to include an estimation of the received power level as a function of the distance and direction of interference given the operational characteristics, e. g. transmission power and antenna characteristics, for the interference scenarios described in 11.1 and 11.2.

## Requirements from the Radio Regulations

Devices will need to comply with the regulatory requirements specified for THz devices.  Unique to THz is that these regulatory requirements will be evolving over the next few years.  In this section these issues can be discussed.  All regulatory assumptions should be clearly stated.

Active THz systems will have to comply with the ITU Radio Regulations footnote 5.565 [9].

### Potentially Critical Interference Scenarios in the THz frequency band

This is an ideal estimation of maximum occurring interference powers using worst case assumptions throughout all scenarios with no additional attenuation due to the impact of weather.

Nomadic devices operated outdoors in rural environment

The assumptions are that the nomadic TX is accidentally pointed in immediate skyward direction as a satellite is passing overhead, directly looking at the ground (0° zenith angle, shortest path), see Fig. 11.1  Also it is assumed that a ground reflection may superimpose constructively with the direct path towards the satellite. This scenario is potentially relevant for the intra-device and kiosk downloading when operated outdoors.

Figure 11.1: Interference situation for nomadic devices operated outdoors in rural environment

Nomadic devices operated outdoors in urban environment

In this scenario we assume that no objects shadow the direct ray path and that additionally the reflections from buildings around the TX coherently combine at the satellite, see Figure 11.2.

Figure 11.2: Interference situation for nomadic devices operated outdoors in urban environment

Indoor-operated devices are not relevant due to transmission losses; therefore, indoor setups are implicitly covered by the outdoor scenario. This scenario is potentially relevant for the intra-device and kiosk downloading when operated outdoors.

Fixed links with scattering objects close to direct ray path

In this scenario, objects close to the direct ray path may scatter/reflect power in a skyward direction towards the satellite, see Figure 11.3.  This scenario is possible despite highly directive antennas. This scenario is relevant for the backhauling/fronthaulimg use case.

Figure 11.3: Interference situation for fixed links with scattering objects close to direct ray path

Relevant emission from antenna sidelobes

High sidelobe levels may cause non-negligible radiation in a skyward direction. With fixed links these become especially critical because of potentially high output powers, see Figure 11.4. This scenario is relevant for the backhauling/fronthauling scenario.

Figure 11.4: Interference situation due to relevant emission from antenna sidelobes

### Spectral Masks between Active Services and Earth Exploration Services (EESS)

Maximum equivalent isotropically radiated power (EIRP)

The clause shows how to calculate the maximum equivalent isotropically radiated power (EIRP), which is scenario path loss specific, and includes the satellite antenna gain.  If the TX radiated signal does not exceed the maximum allowed interference power mask, then there will be no interference. The calculation procedure is displayed in Figure 11.5.

Figure 11.5: Calculation of the maximum equivalent isotripically radiated power (EIRP)

Definition of transmit power spectrum masks

This clause provides the definition of transmit power spectrum masks and constraints for the isotropically radiated power spectral density.  A simple approximation with line segments  (line parameter sets given in [8]) is used to calculate the effective limitation to several 10 dBm EIRP for bandwidths of several 10 GHz.

Nomadic devices in rural and urban areas

Figure 11.6: Spectrum masks for nomadic devices

Fixed links and sidelobes

Fixed links are far less critical than nomadic transmitters and can support higher output powers and longer distances with no significant sidelobe limitations.  These relevant constraints are only applicable below 500 GHz.

Figure 11.7: Spectrum masks for fixed links and sidelobes

## Coexistence among different use cases

Also, each use case outlined in [4] must coexist with all other use cases.  In some instances this coexistence will be explicit (i.e. different operation bands), but in other instances the coexistence mechanism might be more implicit.

# Channel models

The channel models are described in the Channel Modelling Document [5].

# Link budget and SNR analysis

It is likely that each use case will have a unique link budget analysis due to differences in required performance and complexity of modulation formats.  Reference [11] contains useful information in regards to link budget calculations.

Proposals have to include link budget calculations, which show, that the requirements defined in sections 8 and 9 can be met. This shall include also requirements on the antenna gain and the required alignment accuracy, which is use case dependent. An example link budget is given in Table 13.1.

Table 13.1: Exemplary Link Budget

## Antenna gain and required alignment accuracy

The antenna gain and required alignment accuracy is driven by use cases.  This section discusses the antenna gain and required alignment accuracy, which may be part of the justification for different PHY types.

## Outdoor Fixed Wireless Gain and Accuracy

Due to the very high path loss accompanied with THz transmission, for outdoor applications high antenna gains are required. The antenna gain depends on the distance, transmitted data rate, carrier frequency and application. However, an example for a fixed wireless link with a distance of 1 km under worst environmental conditions of a rain rate of 50 mm/h is given in Figure 13.1. For shorter distances, or better atmospheric conditions, antennas with lower gain can be used. If a transmit power of 10 dBm, a noise figure of 10 dB, and an ambient temperature of 300 K is assumed, the maximum transmittable data rates per GHz bandwidth in a 1 km link are shown in Figure 13.1 [10].

Figure 13.1: Achievable data rates per GHz bandwidth for a 1km link in the THz range for several rain rates [10].

From Figure 13.2, very high data rates can only be transmitted if the antenna gains are respectively high. For an antenna gain of 50 dBi for the transmitter and receiver antennas a maximum data rate of 25 Gbps can be transmitted in the 76 GHz bandwidth available in the first atmospheric window (I in Fig. 13.1). However, if the antenna gain is increased to 70 dBi, the maximum data rate can be increased to about 860 Gbps in the first transmission window.

Figure 13.2: Ideal antenna diameter (left and blue) and diameter of the radiated area (right and red) in dependence of the antenna gain for the first THz window and a distance of 1 km

For 70 dBi antenna gain the angle for loss of connectivity due to fluctuation of the pole and the pole twist is just 0.13°. The requirement for adaptive antenna alignments or control mechanisms to compensate for pole sway/twist depends on the grade of sway/twist impairments, given by the antenna installations, e.g. type of pole or building.

# Media Access Mechanism

## Beam steered or control channel assisted device discovery

There may be a need to support device discovery, which includes aligning the antenna to close the link.  This section discusses these issues. These parameters may be use case dependent.

## Link Establishment Beaconing Protocol

## PHY/MAC header fields and preamble definitions

## Requirements MAC for an established link

# I/O Interfaces and Memory Buffer Considerations

The choice of features regarding these items are up to the implementer. However, proposals should include a statement on the expected numbers.

# Fast connection setup scheme

This is relevant for the close proximity use case only. However the consideration of this feature is not part of the amendment developed in this TG. The fast connection setup will be developped in the parallel running TG 3e.

# Security mechanisms

Proposals should indicate how security can be integrated, supporting both MAC security and Authentication/Key Management. Diffentiation may be made between

  - the links for the wireless backhaul/fronthaul, data center and intra-device communication use cases, where links will be establised on a longer time period

  - the links for the close proximity communication, where the links survive only a short time period.

  - using existing 802 security services which can be those already in 802.15 or alignment with 802.3 or 802.11 security services.

  - using 802.15.3 specific security services, built on current best practices.

# Size, Weight and Power

Size, weight and power (SWaP) concerns arise from use case expectations and influences range and data rate. The antenna is included in the SWaP estimate.   Of primary concern is mobile operation battery life.  This section discusses these issues.

Typical values for size, weight and power consumption, differentiated per use case have to be given.

# References

[1] TG6 Technical Requirements Document IEEE 802. 15-08-0644-08-0006

[2] 100G PAR, https://mentor.ieee.org/802.15/dcn/13/15-13-0523-07-0thz-100g-working-draft-par.docx

[3]100G CSD, https://mentor.ieee.org/802.15/dcn/13/15-13-0522-06-0thz-100g-working-draft-5c.docx

[4] Applications Requirmements Document (ARD), https://mentor.ieee.org/802.15/dcn/14/15-14-0304-16-003d-applications-requirement-document-ard.docx

[5] Channel Modelling Document, https://mentor.ieee.org/802.15/dcn/14/15-14-0310-07-003d-channel-modeling-document.docx

[6] S. Priebe, D. M. Britz, M. Jacob, S. Sarkozy, K. M. K. H. Leong, J. E. Logan, B. S. Gorospe, T. Kürner: Interference Investigations of Active Communications and Passive Earth Exploration Services in the THz Frequency Range“, accepted for publication in IEEE Transactions on THz Science and Technology, 14 pages, 2012

[7] S. Priebe: Interference between THz Communications and Spaceborne Earth Exploration Services, IEEE 802.15-12-0324-00-0thz, San Diego, July 2012

[8] S. Priebe, C. Jastrow, M. Jacob, T. Kleine-Ostmann, T. Schrader, T. Kürner: Channel and Propagation Measurements at 300 GHz, IEEE Transactions on Antennas and Propagation, vol. 59, no. 5, pp. 1688–1698, 2011.

[9] Ref. to Radio Regulations

[10] T. Schneider, A. Wiatrek, S. Preußler, M. Grigat, R. P. Braun: Link Budget Analysis for Terahertz Fixed Wireless Links, IEEE Transactions on THz Science and Technology 2, 250 – 256 (2012)

[11] M. Grigat, T. Schneider, S. Preußler, R. P. Braun: Link Budget Considerations for THz Fixed Wireless Links, IEEE 802.15-12-0582-01-0thz, San Antonio, November 2012

10 Gbit/s is the maximum data rate available today in CIPRI. Hence, this shall be the minimum data rate targeted in the amendment.