IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG4s Technical Guidance Document |  |
| Date Submitted | [12 November, 2015] |  |
| Source | [Shoichi Kitazawa] [ATR] [2-2-2, Hikaridai, Seika, Kyoto Japan] | Voice: [ ] Fax: [ ] E-mail: [ kitazawa@atr.jp ] |
| Re: | [ ] |  |
| Abstract | [Working draft for TG4s technical guidance document] |  |
| Purpose | [Summaries of technical topics discussed at TG4s] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Document Overview

This technical document provides summary of contributions to SG SRU and TG4s regarding Spectrum Resource Measurement and Management.

| List of contributors |  |
| --- | --- |
| Masayuki Ariyoshi |  |
| Shusaku Shimada |  |
| Mineo Takai |  |
| Takeshi Yamamoto |  |
| Mitsuru Iwaoka |  |
| Hidetoshi Yokota |  |
| Ruben Salazar |  |

Table of Contents

1	Overview	5

2	Definitions	5

3	Abbreviation and acronyms	5

4	General descriptions	6

4.1	Summary of PAR	6

4.2	General requirement of SRMM	6

4.3	Spectrum Resource Measurement Report Structure	7

4.4	Use Case	8

4.4.1	Use Case1:Multiple PAN	8

4.4.2	Use Case 2: Anomaly Detection	8

4.4.3	Use Case 3: Resource management for Smart Utility Network (SUN)	9

4.4.4	Use Case 4: Cluster Tree	10

4.4.5	Use Case 5:	11

4.4.6	11

5	Functional requirements	12

5.1	Spectrum Resource Measurement	13

5.2	MAC PIB	14

5.2.1	General MAC PIB	14

5.2.2	SRM PIB	14

5.2.3	SRM metrics specific MAC PIB	15

5.3	PHY PIB	18

5.3.1	SRM metrics specific PHY PIB	18

6	Measurement metrics	19

6.1	PHY	19

6.2	MAC	19

7	SRM specific measurement usage	19

7.1	ED (Energy Detection)	19

7.2	Percentage of time of failed transmissions (No Ack)	19

7.3	Retry Histogram	20

7.4	Percentage of time of deferred transmissions (CCA)	20

7.5	Channel Utilization	20

7.6	LQI (Link Quality Indicator)	20

7.7	Transmit Power Used	20

7.8	Max Transmit Power	20

7.9	Received Channel Power Indicator (RCPI)	21

7.10	Received Signal Noise Indicator (RSNI)	21

7.11	Received Signal Strength Indicator (RSSI) (Table 129 [23])	21

7.12	Noise Histogram	22

7.13	Average Access Delay	22

7.14	MAC Performance Metrics specific MAC PIB attributes	22

References	22

Appendix A.	Applications	24

A.1	 Hospital/Medical/Healthcare	24

A.2	Industrial Automation	24

A.3	Infrastructure Monitoring	25

A.4	Advanced Metering Infrastructure (AMI)	25

Appendix B.	Related Standards	26

- Overview

This technical guidance document provides summary of MAC related functions to enable spectrum resource measurement and management for 802.15.4. The TG4s aims at building a standard to specify the MAC framework for exchanging information for the better use of spectrum resource usage.

- Definitions

This section provides definitions of terms that discussed in TG4s.

spectrum resource measurement:

spectrum resource measurement and management:

- Abbreviation and acronyms

The following abbreviations are used in this document.

LR-WPAN	Low-Rate WPAN

RFD		Reduced Function Device

SRM		Spectrum Resource Measurement

SRMM		Spectrum Resource Measurement and Management

SRU		Spectrum Resource Utilization

TGD		Technical Guidance Document

WPAN		Wireless Personal Area Network

- General descriptions

This section provides the basic framework for Spectrum Resource Measurement and Management (SRMM).

  - Summary of PAR

Title:

Standard for Low-Rate Wireless Networks: Amendment Enabling Spectrum Resource Measurement Capability

Scope of the project:

This amendment to IEEE Std 802.15.4 defines MAC related functions to enable spectrum resource management.

It specifies

- spectrum resource measurements and network performance metrics, such as packet error ratio, delay, etc,

- information elements and data structures to capture these measurements,

- procedures for collecting and exchanging spectrum resource measurement information with higher layers or other devices.

Need for the Project:

As various wireless systems are deployed in the shared and license exempt frequency bands including 2.4GHz and 915MHz bands, heavy interference has limited performance of the wireless systems. In order for these wireless systems to operate more effectively, a standardized set of spectrum resource measurements is needed that will facilitate management functions in these networks.

Stakeholders for the Standard:

The stakeholders include manufacturers and users of telecom, medical, environmental, energy, and consumer electronics equipment and manufacturers and users of equipment involving the use of wireless sensor and control networks.

The PAR can be found on the IEEE Standard Association Website.

https://development.standards.ieee.org/P875300033/par

  - General requirement of SRMM

This section describes Spectrum Resource Measurement and Management (SRMM)

    - A device should be able to request another device to conduct spectrum resource measurements.

    - A device should be able to conduct and report spectrum resource measurements at the time of receiving a request from another device, at a predefined interval or on a predefined condition.

    - A device should be able to  provide spectrum resource measurements information to another device before its joining (e.g., in the form of the beacon frame).

    - Intermediate device(s) between the requesting and requested (target) devices should be able to relay the spectrum resource measurement request.

    - Intermediate device(s) should be able to piggy back its own measurement information when relaying the report from the requested device to the requesting device.

  - Spectrum Resource Measurement Report Structure

SRM report header should include at least the following:

- Reporting device ID, PAN ID

- Measurement time

- Number of measurements included in the report

SRM report payload can include multiple pairs of measurement metric type and value as long as those measurements were performed by the same device at the same time.

All the measurement metric types and the details (precision, unit etc.) of their associated value(s) are to be defined in TG4s.

Header

Type

Value

Type

Value

Type

Value

…

Figure 1 SRM report payload

SRM request header should include the following:

- Requesting device ID, PAN ID

- Measurement type

- Measurement mode (e.g., autonomous, triggered)

- Measurement duration

- Relay mode (e.g., through, piggy back)

  - Use Case

The group discusses following categories of the SRM use case.

- The Network manager (Next Higher Layer) request to the PAN coordinators about spectrum resource information.

- PAN coordinator report to the Network manager (Next Higher Layer) about anomaly detection

    - Use Case1:Multiple PAN

This use case depicts coexistence with multiple PAN’s.

Each PAN coordinator notifies the spectrum resource usage information to the Network manager or a group of Network Managers. The network manager(s) may use the spectrum resource usage information for controls PHY and MAC parameter.

802.15.4

PAN Coordinator

802.15.4

PAN Coordinator

802.15.4

PAN Coordinator

Radio resource contention will be occur

between different piconet

Detecting spectrum resource usage and notifies to the network manager.

The Network Manager

controls PHY/MAC parameter based on spectrum resource usage.

Network manager

Figure 2 Multiple PAN

An Initial Proposal of Reference Architecture for TG4s (15-15-027)

    - Use Case 2: Anomaly Detection

- Co-existing networks, e.g. IEEE802.11 and 15.4e (ISA100.11a),  are able to share the alien system information each other.

  - Increased noise energy using energy detection

  - Overlapping network activity using carrier sense

- Jabber or jamming node may be detected.

  - Suspicious behavior or ordinary mobile demeanor of alien node can be estimated.

  - Sometimes notification storm has to be suppressed, e.g. mobile worker carrying wireless gadgets.

- Security threat may be alleviated, for instance;

  - detection of  “replay attack” node

  - detection of  “active spoofing” node

Figure 3 Anomaly detection

Vigilance of Spectrum Resource Usage in private facilities for network stability and security (15-15-040)

    - Use Case 3: Resource management for Smart Utility Network (SUN)

- Large-scale SUN is composed of multiple PANs

  - When a new smart meter (end point) is installed, it will join one of those PANs in their footprints

  - Selection and reselection of the PAN to be joined is determined with certain criteria

  - Structure of RF mesh changes from time to time

- Stable performance among multiple PANs is desired

  - Well-balanced SRU (Spectrum Resource Usage) is desired for each end point to show as much the same performance as possible regardless of the available PANs

  - Instantaneous QoS fluctuation should not affect the behavior of those end points

Figure 4 Smart Utility Network

- Figure 4 is one example configuration of Network Manager and it could be separated from the application server (Head End System).

- Which device collects and/or stores SRM information and which device analyzes it depend on the scale of the network and capability of each device. Frequency of collection (timeliness) and volume of information (detailedness) need to consider scalability.

- It is desirable to provide a mechanism to control SRM information traffic (e.g., pacing or discarding outdated information) in order not to affect application traffic.

Use case proposal for Technical Guidance Document (15-15-177)

    - Use Case 4: Cluster Tree

- Each PAN Coordinator collect SRM information from device.

  - RFD (Reduced Function Device) should forward SRM information

- The SRM information should include PAN ID.

Figure 5 Cluster Tree

Additional TG4s use case and consideration of SRM information (15-15-199)

    - Use Case 5:

- Functional requirements

This chapter describes functional requirement of SRM.

Table 1 Spectrum Resource Measurement Requirement

Spectrum Resource Measurement and Management requirement table (15-15-089r2)

  - Spectrum Resource Measurement

The Spectrum Resource Measurement provides the following

- Measurement results of channel load and/or link quality

- Anomaly detection

Figure 6 LR-WPAN device architecture

Figure 6 shows typical LR-WAPN device architecture.

  - MAC PIB

    - General MAC PIB

The MAC PIB attributes are described in 6.4.2[23] and 8.4.2[22].

The group define SRM capability PIB as follows.

General MAC PIB attributes for functional organization

| Attribute | Type | Range | Description | Default |
| --- | --- | --- | --- | --- |
| macSRMcapable | Boolean | TRUE, FALSE | If TRUE, the device is capable of functionality specific to SRM | FALSE |

    - SRM PIB

| Attribute | Type | Range | Description | Default |
| --- | --- | --- | --- | --- |
| macChannelLoad Enabled | Boolean | TRUE, FALSE | If TRUE, the device is using functionality specific to Channel Load | - |
| macLinkStatistics Enabled | Boolean | TRUE, FALSE | If TRUE, the device is using functionality specific to Link Statistics | - |
| macInterference MeasurementEnabled | Boolean | TRUE, FALSE | If TRUE, the device is using functionality specific to Interference Measurement | - |
| macNodeStatistics Enabled | Boolean | TRUE, FALSE | If TRUE, the device is using functionality specific to Node Statistics | - |
| macChannelOccupancy StatisticsEnabled | Boolean | TRUE, FALSE | If TRUE, the device is using functionality specific to Channel Occupancy Statistics | - |
| macWakeup StatisticsEnabled | Boolean | TRUE, FALSE | If TRUE, the device is using functionality specific to Wakeup Statistics | - |
| macTimeSynchError Enabled | Boolean | TRUE, FALSE | If TRUE, the device is using functionality specific to Time Synch Error | - |
| macSecurityEnabled | Boolean | TRUE, FALSE | If TRUE, the device is using functionality specific to SRMM Security | - |
| macLatencyEnabled | Boolean | TRUE, FALSE | If TRUE, the device is using functionality specific to Network Latency | - |
| macReportEnabled | Boolean | TRUE, FALSE | If TRUE, the device is using functionality specific to Management Report | - |

    - SRM metrics specific MAC PIB

| Attribute |  |  | Type |  |  |  | Range |  |  |  | Description |  |  |  | Default |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| macEd |  |  | Integer |  |  |  | 0x00-0xff |  |  |  | The received signal power within the bandwidth of the channel as defined 10.2.5. |  |  |  | - |  |  |  |
| macTxFailTime |  |  | Integer |  |  |  | 0x00-0xff |  |  |  | The total transmission attempt time of failed transmissions after all retries with no ACK over the measurement period, linearly scaled with 255 representing100%. |  |  |  | - |  |  |  |
| macRetryHistogram |  |  | Integer |  |  |  | 0-100 |  |  |  | Histogram of the number of retries for one transmission during the measurement time |  |  |  | - |  |  |  |
| macLqi |  |  | Integer |  |  |  | 0x00-0xff |  |  |  | Averaged LQI that indicates the strength and/or quality of a received packet as defined in 10.2.6. |  |  |  | - |  |  |  |
| macRcpi |  |  | Integer |  |  |  | 0x00-0xff |  |  |  | The total channel power (signal, noise, and interference) of a received frame measured on the channel and at the antenna connector used to receive the frame |  |  |  | - |  |  |  |
| macRsni |  |  | Integer |  |  |  | 0x00-0xff |  |  |  | The signal to noise plus interference ratio of a received frame, which is defined by the ratio of the received signal power to the noise plus interference power as measured on the channel and at the antenna connector used to receive the frame. |  |  |  | - |  |  |  |
| macRssi |  |  | Integer |  |  |  | 0x00-0xff |  |  |  | The RF power level at the input of the transceiver. |  |  |  | - |  |  |  |
| macNoiseHistogram |  |  | Integer |  |  |  | - |  |  |  | Power histogram measurement of non-IEEE 802.15 noise power |  |  |  | - |  |  |  |
| macFrameErrorCount |  |  | Integer |  |  |  | 0~28×macCounterOctets-1 |  |  |  | The number of received frames that were discarded due to any error except for FCS error (complementary to macFcsErrorCount). |  |  |  |  |  |  |  |
| macTimeOutCount |  |  | Integer |  |  |  | 0~28×macCounterOctets-1 |  |  |  | #### Same as macTxFailCount? |  |  |  |  |  |  |  |
| macCounterOctets | Integer | As defined in 8.4.2.6 | See 8.4.2.6 (existing metric) |  |
| macRetryCount |  |  | Integer |  |  |  | As defined in 8.4.2.6 |  |  |  | See 8.4.2.6 (existing metric) |  |  |  |  |  |  |  |
| macMultipleRetryCount |  |  | Integer |  |  |  | As defined in 8.4.2.6 |  |  |  | See 8.4.2.6 (existing metric) |  |  |  |  |  |  |  |
| macTxFailCount |  |  | Integer |  |  |  | As defined in 8.4.2.6 |  |  |  | See 8.4.2.6 (existing metric) |  |  |  |  |  |  |  |
| macTxSuccessCount |  |  | Integer |  |  |  | As defined in 8.4.2.6 |  |  |  | See 8.4.2.6 (existing metric) |  |  |  |  |  |  |  |
| macFcsErrorCount | Integer | As defined in 8.4.2.6 | See 8.4.2.6 (existing metric) |  |
| macSecurityFailure | Integer | As defined in 8.4.2.6 | See 8.4.2.6 (existing metric) |  |
| macDuplicateFrameCount |  |  | Integer |  |  |  | As defined in 8.4.2.6 |  |  |  | See 8.4.2.6 (existing metric) |  |  |  |  |  |  |  |
| macRxSuccessCount |  |  | Integer |  |  |  | As defined in 8.4.2.6 |  |  |  | See 8.4.2.6 (existing metric) |  |  |  |  |  |  |  |
| macNackCount |  |  | Integer |  |  |  | As defined in 8.4.2.6 |  |  |  | See 8.4.2.6 (existing metric) |  |  |  |  |  |  |  |
| macDeferredTxCount |  |  | Integer |  |  |  | 0~28×macCounterOctets-1 |  |  |  | The number of frames that were deferred transmission. |  |  |  |  |  |  |  |
| macAverageBufferUtilization |  |  | Integer |  |  |  | 0-100 |  |  |  | The average percentage of the used buffer space in Tx queue. |  |  |  |  |  |  |  |
| macMaximumBufferUtilization |  |  |  |  |  | Integer |  |  |  | 0-100 |  |  |  | The maximum percentage of the used buffer space in Tx queue. |  |  |  |  |  |  |  |
| macTxFragmentCount |  |  | Integer |  |  |  | 0~28×macCounterOctets-1 |  |  |  | The number of transmitted frames that were fragmented. |  |  |  |  |  |  |  |
| macRxFragmentCount |  |  | Integer |  |  |  | 0~28×macCounterOctets-1 |  |  |  | The number of received frames that were fragmented. |  |  |  |  |  |  |  |
| macTxMulticastCount |  |  | Integer |  |  |  | 0~28×macCounterOctets-1 |  |  |  | The number of transmitted multicast frames |  |  |  |  |  |  |  |
| macRxMulticastCount |  |  | Integer |  |  |  | 0~28×macCounterOctets-1 |  |  |  | The number of received multicast frames |  |  |  |  |  |  |  |
| macAckFailCount |  |  | Integer |  |  |  | 0~28×macCounterOctets-1 |  |  |  | #### Same as macTxFailCount? |  |  |  |  |  |  |  |
| macAverageAccessDelay |  |  | Integer |  |  |  | 0~28×macCounterOctets-1 |  |  |  | The average medium access delay for transmitted frames measured from the time the MPDU is ready for transmission until the actual frame transmission start time. |  |  |  |  |  |  |  |
| macChannelUtilization |  |  | Integer |  |  |  | 0~28×macCounterOctets-1 |  |  |  | Total channel used time for Tx and Rx and the occupied time for the other devices. |  |  |  |  |  |  |  |

  - PHY PIB

    - SRM metrics specific PHY PIB

| Attribute | Type | Range | Description | Default |
| --- | --- | --- | --- | --- |
| phyTxPower | Signed integer | - | Nominal transmit power of the device in dBm using the Maximum Transmit Power IE or predefined. See Table 181. | - |
| phyMaxTxPower | Signed integer | - | Nominal transmit power level allowed on a network. See Table 181. | - |

- SRM specific measurement usage

This chapter describes measurement metrics for TG4s which describes in Table 1.

  - PHY

    - Transmit Power Used

- Nominal transmit power of the device indicated as phyTxPower in dBm.

- The value shall be less than or equal to the Max Transmit Power, phyMaxTxPower (Table 181 [22]).

    - Max Transmit Power

- Nominal transmit power level allowed on a network indicated as phyMaxTxPower. The upper limit, in units of dBm, on the transmit power as measured at the output of the antenna connector to be used by the device on the current channel (defined in 8.4.1.19 [23])

  - MAC

    - ED (Energy Detection)

- The average of the received signal power over 8 symbol periods

- The minimum and maximum values of ED are 0x00 and 0xff

- The received power in units of dB is linearly mapped between these values

    - Percentage of time of failed transmissions (No Ack)

- macTxFailTime is calculated with the following equation:

macTxFailTime =tx fail time [i]Measurement Time

    - Retry Histogram

- Histogram of the number of retries for one transmission during the measurement time

- Range: from 0 to macMaxFrameRetries

  - 0  Tx success without retry

  - macMaxFrameRetries  Tx failure

    - Percentage of time of deferred transmissions (CCA)

- macDeferredTxTime= Deferred periodMeasurement Time

- Deferred period: total back-off period after first CCA

    - Channel Utilization

- Total channel used time for Tx and Rx and occupied time for the other devices over the measurement duration, linearly scaled with 255 representing100%

- Channel Busy Time:

  - Total time of Tx and Rx of all Frames and ACKs

  - Deferred periods (previous page) and reception periods of frames that are not destined for this device are also added

    - LQI (Link Quality Indicator)

- A characterization of the strength and/or quality of a received packet

- The minimum and maximum LQI values (0x00 and 0xff) should be associated with the lowest and highest quality compliant signals detectable by the receiver, and LQI values in between should be uniformly distributed between these two limits

    - Received Channel Power Indicator (RCPI)

- The total channel power (signal, noise, and interference) of a received frame measured on the channel and at the antenna connector used to receive the frame (defined in 20.3.21.6 [23])

- RCPI shall be a monotonically increasing, logarithmic function of the received power level defined in dBm. The allowed values shall be an 8 bit value in the range from 0 to 220

    - Received Signal Noise Indicator (RSNI)

- The signal to noise plus interference ratio of a received frame, which is defined by the ratio of the received signal power (RCPI-ANPI) to the noise plus interference power (ANPI) as measured on the channel and at the antenna connector used to receive the frame (defined 8.4.2.43 [23])

- ANPI can be calculated with Averaged IPI Density (IPI: Idle Power Indicator)

- IPI Density in the specified channel as a function of time over the measurement duration where the channel is idle (CCA indicates idle and no Tx or Rx)
(10.11.9.4 [23])

- IPI Density=Integer DIPI [μs]Channel Idle Time [μs]×255

- ANPI=averagede IPI Density values

    - Received Signal Strength Indicator (RSSI) (Table 129 [23])

- The RF power level at the input of the transceiver measured during the PHR and is valid after the SFD is detected

- The minimum and maximum values are 0x00 and 0xff and the values in between should be uniformly distributed

    - Noise Histogram

- A power histogram measurement of non-IEEE 802.15 noise power by sampling the channel when CCA indicates idle and the device is neither transmitting nor receiving a frame

- IPI densities observed in the channel for the IPI levels defined in 4.3.8.7 [23] and 10.11.9.4 [23]

    - Average Access Delay

- The average medium access delay for transmitted frames measured from the time the MPDU is ready for transmission until the actual frame transmission start time

- In the case of TSCH, macTsTxOffset can be used calculate Access Delay time

- Defined in 10.11.16 [23]

    - MAC Performance Metrics specific MAC PIB attributes

- 10 metric specific MAC PIB attributes are defined in 8.4.2.6 [22].  These attributes are reused for SRM.

- By using these attributes, the packet success rate can be calculated as follows:

Packet Success Rate=1-macTxFailCountmacTxSuccessCount++macRetryCount+macMultipleRetryCount+macTxFailCount

tx fail time [i]Measurement Time

- Deferred periodMeasurement Time

  - Transmit Power Used

  - Nominal transmit power of the device indicated as phyTxPower in dBm.

  - The value shall be less than or equal to the Max Transmit Power, phyMaxTxPower (Table 181 [22]).

  - Max Transmit Power

  - Nominal transmit power level allowed on a network indicated as phyMaxTxPower. The upper limit, in units of dBm, on the transmit power as measured at the output of the antenna connector to be used by the device on the current channel (defined in 8.4.1.19 [23])

IPI Density=Integer DIPI [μs]Channel Idle Time [μs]×255

ANPI=averagede IPI Density values

# Packet Success Rate=1-macTxFailCountmacTxSuccessCount++macRetryCount+macMultipleRetryCount+macTxFailCount

- IEs

  - SRM IEs

# References

- Kitazawa, Establishing a Study Group for a Spectrum Resource Utilization (SRU) through Radio Resource Measurement and Management for WPANs, IEEE 802.15-13-404r1

- Kitazawa, Overview of SG SRU, IEEE 802.15-13-543r0

- Additional use case of temporal and flexible industrial network deployment

- Additional use case of temporal and flexible industrial network deployment, IEEE 802.15-13-654r1

- Yamamoto et al., Simulation Methodology for SRU, IEEE 802.15-13-660r0

- Proposal of radio resource management architecture(15-13-0285r1)

- A Use Case of Self-Organizing Wireless Network for Medical System(15-13-306-0)

- IG SRU Working Draft RRMM-usecases and 5C(15-13-0294r1)

- IG SRU Usecase requirements table(15-13-0293r1)

- An Initial Proposal of Reference Architecture for TG4s (15-15-027)

- Overview of TG4s Spectrum Resource Usage (15-15-028)

- Vigilance of Spectrum Resource Usage in private facilities for network stability and security (15-15-040)

- Metrics used in some Wireless Sensor Network standards (15-15-72)

- Proposed Spectrum Resource Measurement Report Structure (15-15-086)

- Spectrum Resource Measurement and Management requirement table (15-15-089)

- Use case proposal for Technical Guidance Document (15-15-177)

- Additional TG4s use case and consideration of SRM information (15-15-199)

- Proposal for SRMM General Requirements in Technical Guidance Document (15-15-327)

- Proposal for SRMM Functional Requirements in Technical Guidance Document (15-15-333)

- Proposal for SRMM MAC PIB in Technical Guidance Document(15-15-508)

- Proposal for SRM specific MAC PIB in Technical Guidance Document (15-15-0673)

- IEEE802.15.4-REVc-D00

- IEEE802.11-2012

- Applications

## A.1	 Hospital/Medical/Healthcare

Various wireless systems (WLAN, Bluetooth etc.) are deployed in medical environments.

Requirement

- Self-organizing wireless system

- Spectrum sensing of whole band for searching vacant radio resources

- Radio resource assignment and network topology management

Figure 8 Image of medical information system in hospital

## A.2	Industrial Automation

Requirement

- Quick additional deployment

- Quick withdrawal, relocation

- Inexpensive and reusable

Figure 9 Industrial wireless network

## A.3	Infrastructure Monitoring

Requirement

Figure 10

## A.4	Advanced Metering Infrastructure (AMI)

- A number of smart meters construct Neighborhood Area Network (NAN) in the form of RF mesh in order to send meter reading data for billing or event data for management as well as to receive commands for energy and device control

- Requirements

  - Stable network performance under dynamic mesh structure change

  - Centralized and distributed resource usage control mechanism

  - Assured QoS provision as a commercial communication infrastructure

Figure 11

- Related Standards