IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Harmonized text for distributed power control during CFP |
| Date Submitted |  |
| Source | David Smith, Nicole Sawyer, Siyu Zhou (NICTA†) Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Response | In response to technical contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

† NICTA is funded by the Australian Government through the Department of Communications and the Australian Research Council through the ICT Centre of Excellence Program.

Contents

1. Physical layer procedures	2

1.1 Power control during CFP	2

- Physical layer procedures

This text should be placed in the PHY clause describing transmission of symbols:

For each antenna used for transmission of the PPDU, the block of complex-valued symbols shall be multiplied with the amplitude scaling factor TBD in order to conform to the transmit power specified in clause 1.1.

  - Power control during CFP

Power control controls the transmit power of the different physical channels.

The transmit power Pi defined in sub-clause 1.1.1, is first scaled by the ratio of the number of antennas with a non-zero PPDU transmission to the number of configured antenna for the transmission scheme.  The resulting scaled power is then split equally across the antennas on which the non-zero PPDU is transmitted.

    - PAC device behavior

The setting of a PAC device transmission power of PPDU is defined as follows:

According to communications priority, higher layers provide target packet delivery ratio indicator (TPDRI) or pd,i for PD i.

pd,i  ← Higer layers

Power control shall be applied in T stages as follows

Pi=minPmax,PCi(T)

where Pi is the transmit power for PD i. Pmax is the maximum transmit power and given in Table 1. In single hop, T is set when a stopping criterion is reached (flagi=2). In multi-hop TBD.

- —Maximum transmit power

| Frequency band | Pmax |
| --- | --- |
| 2.4 GHz | 1 W |
| 5.7 GHz | 1 W |
| Sub-1 GHz (band A, D) | 1 mW |
| Sub-1 GHz (band B) | 20 mW |
| Sub-1 GHz (band C) | 250 mW |

PCi(τ)is the transmit power control at stage τ for PD i and obtained as follows

Set: flagi= 0.

- Set: τ = 0 and according to TPDRI, compute equivalent target SINR, γt,i, as

γt,i=log(pd,i)a1b

where aand b depend on the modulation, coding scheme, coding rate and packet size Mp.

a=−Mp1501acbc

b=−bc

- —Modulation and coding rate

| Modulation | Coding rate | ac | bc |
| --- | --- | --- | --- |
| BPSK | 1/2 | 3.891 | 8.85 |
| BPSK | 3/4 | 2.13 | 9.48 |
| QPSK | 1/2 | 1.95 | 9.02 |
| QPSK | 3/4 | 1.066 | 9.29 |
| 16-QAM rate 1/2 | 1/2 | 0.62 | 8.31 |
| 16-QAM rate 3/4 | 3/4 | 0.275 | 8.45 |
| 64-QAM rate 2/3 | 2/3 | 0.125 | 8.34 |
| 64-QAM rate 3/4 | 3/4 | 0.085 | 7.81 |
| 64-QAM rate 5/6 | 5/6 | 0.06 | 8.034 |

Set: Pveca vector containing a discrete number of possible power levels in (0, Pmax].

while flagi ≠ 2  do

γiτ=PiRτPi,Noise+Interferenceτ

where PiR(τ) is the received power measured at the antenna input of PD i. Pi,Noise+InterferenceR(τ)is the AWGN plus interference measured at the antenna input of PD i.

γvec=PvecPiR(τ)Pi(τ)Pi,Noise+InterferenceRτ

PD=expaγvecb

τ=τ+1

If τ≥3and γi(τ−1)≈γi(τ−2) and Pi(τ−1)=Pi(τ−2) and flagi=1 then

flagi=2

Elseif τ≥2 and γi(τ−1)≈γi(τ−2) and Pi(τ−1)=Pi(τ−2) and flagi=0 then

flagi=1

End if

If flagi=0 then

Pi(τ)est=maxPvec≤γt,iγi(τ−1)Pi(τ−1)

Else

Pi(τ)est=minPvec≥γt,iγi(τ−1)Pi(τ−1)

End if

ki,est=argmaxkPvec(k)=Pi(τ)est

γe,i=γvec(ki,est)

Pe,i=Pvec(ki,est)

di,est=Pe,i3pd,i3abγe,ib

Ui=−Pvec+di,estPD3

ki=argmaxkUi(k)

PCi(τ)=Pvec(ki)

End while