IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Resource slot request command |
| Date Submitted | May 11th, 2015 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Response | In response to Call for Contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1. Reference signals	2

    - ReS  request command

The ReS request command is used by a peered PD that is requesting the allocation of a new ReS or the deallocation of an existing ReS.  .

The ReS request command shall be formatted as illustrated in Table 1.

- —ReS request command format

| X octets | 1 octet | 1 octet |
| --- | --- | --- |
| MHR fields | Command identifier | ReS characteristics |

      - MHR fields

MHR fields: destination addressing mode field, source addressing mode field, frame pending field, Source PAC identifier field, source address field, and AR field (acknowledgment request).

The destination addressing mode field shall be set to indicate that destination addressing information is not present, and the source addressing mode field shall be set to 1.    (

- —Addressing mode field

| *Addressing mode | Description |
| --- | --- |
| 00 | Device IDs are not present |
| 01 | Reserved |
| 10 | Reserved |
| 11 | Device ID is present |

*source, destination, multicast, link ID.

| *Addressing mode | Description |
| --- | --- |
| 0 | Address fields are not present |
| 1 | Address field is present |

*source, destination, multicast, link ID.

Frame pending field shall be set to zero and ignored upon reception, and the AR field shall be set to one.

The source PAC identifier field shall contain the value of macPACId TBD, and the Source Address field shall contain the value of source device ID.

      - ReS characteristic field

The ReS characteristics field shall be formatted as illustrated in Table 3.

- —ReS characteristic field format

| Bits | 0-3 | x-y | x-y | x-y |
| --- | --- | --- | --- | --- |
| Attribute | ReS length | ReS direction | Type | Reserved |

The ReS length field shall contain the number of superframe slots being requested for the ReS.

The ReS direction field shall be set to one if the ReS is to be a receive-only ReS. Conversely, this field shall be set to zero if the ReS is to be a transmit-only ReS. The ReS direction is defined relative to the direction of data frame transmissions by the PD.

The characteristics type field shall be set to one if the characteristics refer to a ReS allocation or zero if the characteristics refer to a ReS deallocation.