IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Random access for CFP |
| Date Submitted | July 13th, 2015 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Response | In response to Call for Contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1. Random access procedure for peering in CFP	2

2. Random access PHY generation	3

2.1 Random access preamble	3

2.2 Baseband signal	3

- Random access procedure for peering in CFP

Once network synchronization and decoding of discovery RB information are achieved by PDs, such PDs may request association or peering to a target PD at any time.  Consequently, the random access can be requested during the peering period, in which control signals are interchanged in order to schedule the intended PDs for transmission.

After discovery, PD2 requests association (peering) to PD1 by a random access procedure as illustrated in Figure 1.

- —Random access procedure flow diagram

  - PD2 sends a RAP to PD1 requesting association. The RAP is randomly selected from a pool of orthogonal ZC sequences that belong to the RAP Group supported by PD1.  PD1 selects randomly the time and frequency indexes of a resource slot over the control channel to transmit the RAP.

  - PD1 detects the RAP, estimates the radio channel, and composes the RA response (RAR) message. Such RAR comprises timing alignment (round-trip delay), first contention-free resource slot for transmission and temporal link ID.

  - After successful reception of RAR by PD2 over the time-frequency slot where the RAP was transmitted, PD2 adjust timing and composes RA message (RAM) with the temporal link ID for PD2 identification. As several PDs may use the same RA preamble and consequently transmit the RAM over the same granted ReS that result in contention.

  - Contention resolution: as only one RA message can be detected over all potential contending PDs, PD1 resolves such possible contention by sending the temporal link ID of the successful RA message detected, plus ReS assignment for data transfer in the contention resolution message (CRM). In case of congestion, PD1 may provide a back off indicator to PD2. If a PD in contention receives the contention resolution message with the right temporal link ID, the PD shall terminate the RA procedure as successful. If a PD in contention does not receive the contention resolution message or receives the wrong temporal link ID, such PD shall terminate RA procedure as failure and may repeat it.

- Random access PHY generation

After the discovery phase, PDs that intend to establish a communication link with a discovered PD request that with a random access preamble. According to the superframe structure in Clause 4, after discovery, PDs can transmit in the Peering period for peering with another PD. As such PDs can transmit at any moment; a set of orthogonal preambles is required in order to reduce interference from competing terminals.

The random access association preambles are named Random Access Preambles (RAPs). Such RAPs are formed with ZC sequences as well. Hence, a pool of orthogonal RAPs is formed A unique RAP is assigned to every device in a Group. Such unique RAP is used for fine synchronization and control messages (how a communication link is granted or how resources are assigned to PDs). The RAP signal structure is illustrated in Figure 2.

- — Random access preamble structure

  - Random access preamble

The random access preambles are generated from cyclic shifts of the Zadoff-Chu sequence defined by

an=e−jπrn(n+1)N

where N=197, r=1, and n=0,1,…,N-1.

Different random access preambles are generated with cyclic shift α (positive integer <N) as

bn=a(n+α)Mod N

  - Baseband signal

TBD