IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | MAC services |
| Date Submitted | September 15th |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) Billy Verso (DecaWave) |
| Response | Call for contributions |
| Abstract |  |
| Purpose | For reference in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1. MAC services	3

- MAC services

The MAC sublayer provides an interface to the next higher layer and PHY.  Such interface consists of a MAC sublayer management entity (MLME) and a MAC sublayer data entity (MLDE). Such entities are accessed through two service access points:

— The MAC sublayer data entity service access point (MLDE-SAP).

— The MAC sublayer management entity service access point (MLME-SAP).

- —MAC services reference model.

The PAC information base (PIB) consists of constants and variables required to manage the MAC sublayer or PHY layer of a PD. The PIB fields and attributes are presented in Table 38.  A field may be read only, or read/write.

The MLDE handles data transfer. The MLME handles access to the PIB, communication’s status, CFP management, channel scanning, reset of MAC sublayer, receiver enable, updating superframe configuration, PAC group management for discovery, peering,  de-peering, re-peering, loss of synchronization, channel sounding, and ranging calibration. The service primitives between MAC sublayer and MAC user are shown in Figure 2.

- —MAC service primitives reference model.

  - MLME-SAP primitives

The MLME-SAP allows to access management commands between the MAC sublayer and MAC user.     Table 1 summarizes the primitives supported by the MLME-SAP interface.

- —MLME-SAP primitives

| Name | Request | Confirm | Indication | Response |
| --- | --- | --- | --- | --- |
| MLME.Get |  |  | − | − |
| MLME.Set |  |  | − | − |
| MLME.Discovery |  |  |  |  |
| MLME.Peering |  |  |  |  |
| MLME. de-peering |  |  |  |  |
| MLME.re-peering |  |  |  |  |
| MLME.FrameErrorNotification | − | − |  | − |
| MLME.Reset |  |  | − | − |
| MLME.ReceiverEnable |  |  | − | − |
| MLME.Scan |  |  | − | − |
| MLME.GroupStart |  |  |  |  |
| MLME.CommunicationStatus |  |  | − | − |
| MLME.Synchronization |  | − | − | − |
| MLME.SynchronizationLoss | − | − |  | − |
| MLME.ChannelSounding |  |  | − | − |
| MLME.DPS |  |  |  | − |
| MLME.Calibrate |  |  | − | − |

    - PIB access

These primitives are used to read or write PIB values.

      - MLME.Get.Request

The primitive reads a given PIB field. The properties of this primitive are:

MLM.Get.Request{

PIBName;

}

The primitive parameter is defined in Table 2.

- —MLME.Get.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| PIBName | String | As defined in Table 38 | Name of the PIB field to read. |

      - MLME.Get.Confirm

This primitive reports the result requested by MLME.Get.Request. The properties of this primitive are:

MLME.Get.Confirm{

Status;

PIBName;

PIBValue;

}

The primitive parameters are defined in Table 3.

- —MLME.Get.Confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, FAIL, UNSUPPORTED | The status of the request for PIB field information. |
| PIBName | String | As defined in Table 38 | Name of the PIB field to read. |
| PIBValue | As defined in Table 38 | As defined in Table 38 | The value of the indicated PIB field to read. |

The property PIBValue is set to 0 when Status is UNSUPPORTED.

      - MLME.Set.Request

The primitive allows writing a given value to the indicated PIB field. The properties of this primitive are:

MLME.Set.Request{

PIBName;

PIBValue;

}

The primitive parameters are defined in Table 4.

- —MLME.Set.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| PIBName | String | As defined in Table 38 | Name of the PIB field to write. |
| PIBValue | Various types | As defined in Table 38 | The value of the indicated PIB field to write. |

      - MLME.Set.Confirm

This primitive reports the result requested by MLME.Set.Request. The properties of this primitive are:

MLME.Get.Confirm{

Status;

PIBName;

}

The primitive parameters are defined in Table 5.

- —MLME.Set.Confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, INVALID, READ_ONLY, UNSUPPORTED. | INVALID means PIBValue is out of range. UNSUPPORTED means PIBName is not defined in the PIB. |
| PIBName | String | As defined in Table 38 | Name of the written PIB field. |

    - Discovery primitives

These primitives are used for the discovery of neighboring PDs.

      - MLME.Discovery.Request

The primitive requests a PD to perform discovery. The properties of this primitive are:

MLME.Discovery.Request{

DiscoveryMode;

DiscoveryInfo;

ChannelPage;

ScanChannels;

}

The primitive parameters are defined in Table 6.

- —MLME.Discovery.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DiscoveryMode | Enumeration | TX, RX | Indicates transmission or reception of discovery information. |
| DiscoveryInfo | Array of PAC IDs descriptors | As defined in Table 7 | Indicates the discovery information. |
| ChannelPage | Integer | Any valid channel page as defined in Table TBD | The channel page on which to attempt discovery. |
| ChannelNumber | Integer | Any valid channel number as defined in Table TBD | The channel number on which to attempt discovery. |
| ScanChannels | Array of integers | Any valid channel number as defined in Table TBD | The channel numbers to be used for discovery. |

In RX mode, channelNumber is zero.

Discovery descriptors are defined in Table 7.

- —PAC ID descriptors

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Device_ID | MAC address | PD specific | MAC address of the PD. |
| MulticastGroup_ID | Integer | 0 to 216−1 | Multicast Group addresss of PAC network |
| Application_ID | char | 14 octets | Application name, user name, login. |

      - MLME.Discovery.Confirm

This primitive requests association with given PD. The properties of this primitive are:

MLME.Discovery.Confirm{

DiscoveryMode;

DiscoveryInfo;

ChannelPage;

ScanChannels;

Status;

}

The primitive parameters are defined in Table 8.

- —MLME.Discovery.Confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DiscoveryMode | Enumeration | TX, RX | Indicates transmission or reception of discovery information. |
| DiscoveryInfo | Array of PAC IDs descriptors | As defined in Table 7 | Indicates discovery information. |
| ChannelPage | Integer | Any valid channel page as defined in Table TBD | The channel page on which discovery was attempted. |
| ChannelNumber | Integer | Any valid channel number as defined in Table TBD | The channel number on which discovery was attempted |
| ScanChannels | Array of integers | Any valid channel number as defined in Table TBD | The channel numbers used for discovery. |
| Status | Enumeration | SUCCESS, FAIL, DISCOVERY_IN_PROGRESS | Status of the discovery request |

    - Peering primitives

These primitives are used when a PD becomes associated with another PD.

      - MLME.Peering.Request

This primitive requests association with given PD. The properties of this primitive are:

MLME.Peering.Request{

ChannelNumber;

ChannelPage;

MulticastGroup_ID;

DestinationAddress;

}

The primitive parameters are defined in Table 9.

- —MLME.Peering.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| ChannelNumber | Integer | As defined in Table TBD | The channel number on which to attempt peering. |
| ChannelPage | Integer | As defined in Table TBD | The channel page on which to attempt peering. |
| MulticastGroup_ID | Integer | 0 to 216 −1 | Group ID of destination PD. |
| DestinationAddress | MAC address | PD specific | Address of the PD with which to peer. |

      - MLME.Peering.Confirm

The primitive reports the result requested by MLME.Peering.Request of the initiating PD. The properties of this primitive are:

MLME.Peering.Confirm{

(DestinationAddress);

(SourceAddress);

MulticastGroup_ID;

Status;

}

The primitive parameters are defined in Table 10.

- —MLME.Peering.Confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DestinationAddress | MAC address | PD specific | Address of the PD with which to peer. |
| SourceAddress | MAC address | PD specific | The address of the PD requesting peering. |
| MulticastGroup_ID | Integer | 0 to 216 −1 | Group ID of destination PD. |
| Status | Enumeration | SUCCESS, CHANNEL_ACCESS_FAILURE, NO_ACK, ACCESS_DENIED, | The status of the peering attempt. |

If the peering request was successful, then the Status parameter will be set to SUCCESS. Otherwise, the Status parameter will be set to indicate the type of failure.

      - MLME.Peering.Indication

The primitive is used to indicate the reception of a peering request command. The properties of this primitive are:

MLME.Peering.Indication{

PD_ID;

PHYcapability;

}

The primitive parameters are defined in

- —MLME.Peering.Indication parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| PD_ID | MAC address | PD specific | Address of the PD requesting peering. |
| PHYcapability | Enumeration | LOW_MOBILITY, HIGH_MOBILITY, GFSK, UWB_BPM_BPSK, UWB_OOK | Operational capability of the PD requesting peering. |

      - MLME.Peering.Response

The primitive is used to initiate a response to an MLME.Peering.Indication primitive. The properties of this primitive are:

MLME.Peering.Response{

PD_ID;

MulticastGroup_ID;

Status;

}

The primitive parameters are defined in

- —MLME.Peering.Response parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| PD_ID | MAC address | PD specific | Address of the PD requesting peering. |
| MulticastGroup ID | Integer | 0 to 216 −1 | MulticastGroup address allocated after successful peering. This parameter is set to 0 if there is no multicastGroup address, or 1 if peering was unsuccessful |
| Status | Enumeration | SUCCESFUL, OUT_OF_CAPACITY, ACCESS_DENIED, | Status of the peering attempt. |

    - De-peering primitives

These primitives are used when a PD becomes de-peered from another PD.

      - MLME.Depeering.Request

This primitive requests de-peering of given PD. The properties of this primitive are:

MLME.Depeering.Request{

DestinationAddress;

SourceAddress;

MulticastGroup_ID;

Reason;

}

The primitive parameters are defined in Table 13.

- —MLME.Depeering.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DestinationAddress | MAC address | PD specific | Address of the PD with which to peer. |
| SourceAddresss | MAC address | PD specific | The address of the PD requesting peering. |
| MulticastGroup_ID | Integer | 0 to 216 −1 | Group ID of destination PD. |
| Reason | Integer | 0 to 1 | Reasons for dissociation: 0 - Source wants to leave. 1 - Source requests destination to leave. |

      - MLME.Depeering.Confirm

This primitive reports the result requested by MLME.Depeering.Request. The properties of this primitive are:

MLME.De-peering.Confirm{

DestinationAddress;

SourceAddress;

MulticastGroup_ID;

Status;

}

The primitive parameters are defined in Table 14.

- —MLME.Depeering.Confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DestinationAddresss | MAC address | PD specific | Address of the PD with which to de-peer. |
| SourceAddresss | MAC address | PD specific | The address of the PD requesting de-peering. |
| MulticastGroup_ID | Integer | 0 to 216 −1 | MulticastGroup ID of destination PD. |
| Status | Enumeration | SUCCESS, NO_ACK, CHANNEL_ACCESS_FAILURE, | The status of the de-peeringattempt. |

If the de-peering request was successful, then the Status parameter will be set to SUCCESS. Otherwise, the Status parameter will be set to indicate the type of failure.

      - MLME.De-peering.Indication

The primitive is used to indicate the reception of a de-peering request command.

MLME.De-peering.Indication{

PD_ID;

Reason;

}

The primitive parameters are defined in

- —MLME-De-peering.Indication

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| PD_ID | MAC address | PD specific | The address of the PD requesting de-peering. |
| Reason | Integer | 0 − 1 | 0 – source wants to leave 1 – source requests destination to leave |

    - Frame error notification primitives

This primitive is used to notify the next higher layer that an error has occurred during the processing of a frame.

      - MLME.FrameErrorNotification.Indication

This primitive indicates a communications status. The properties of this primitive are:

MLME.FrameErrorNotification.Indication{

SourceAddress;

DestinationAddress;

MulticastGroup_ID;

Status;

}

The primitive parameters are defined in Table 16.

- —MLME.FrameErrorNotification.Indication parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| SourceAddress | MAC address | PD specific | Address of the PD from which the frame causing an error originated. |
| DestinationAddress | MAC address | PD specific | Address of the PD from which the frame causing an error originated. |
| MulticastGroup_ID | Integer | 0 to 216 −1 | Group ID of destination PD. |
| Status | Enumeration | TRANSACTION_OVERFLOW, TRANSACTION_EXPIRED, CHANNEL_ACCESS_FAILURE, NO_ACK, TBD | The status of the communication error frame. |

TRANSACTION_OVERFLOW – The MLME does not have the capacity to store the superframe that was to be sent.

TRANSACTION_EXPIRED – The transaction was not handled within macTransactionPersistenceTime.

CHANNEL_ACCESS_FAILURE – There was a failure in the CAP, CFP while attempting to send the frame.

NO_ACK – An acknowledgment was expected but not received.

    - Reset MAC sublayer

These primitives are used to reset the MAC sublayer. The execution is implementation specific.

      - MLME.Reset.Request

This primitive is used by the next higher layer to request a reset operation. The properties of this primitive are:

MLME.Reset.Request{

ResetMode;

}

The primitive parameters are defined in Table 17.

- —MLME.Reset.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResetMode | Boolean | TRUE, FALSE | Reset the MAC sublayer. |

If TRUE, the MAC sublayer is reset and all MAC PIB fields are set to their default values. If FALSE, the MAC sublayer is reset, but all MAC PIB fields retain their values prior to the generation of the MLME-Reset.Request primitive.

      - MLME.Reset.Confirm

This primitive reports the results of the reset operation. The properties of this primitive are:

MLME.Reset.Confirm {

Status;

}

The primitive parameters are defined in Table 18.

- —MLME.Reset.Confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, FAIL | The result of the reset operation. |

    - Receiver enable

These primitives are used to enable or disable a receiver of a PD.

      - MLME.ReceiverEnable.Request

This primitive allows the next higher layer to request that the receiver is either enabled for a finite period of time or disabled. The properties of this primitive are:

MLME.ReceiverEnable.Request {

Defer;

RxOnTime;

RxOnDuration;

}

The primitive parameters are defined in Table 19.

- —MLME.ReceiverEnable.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Defer | Boolean | TRUE, FALSE. | TRUE if the requested operation can be deferred until the next superframe, if the requested time has already passed. FALSE if the requested operation is only to be attempted in the current superframe. |
| RxOnTime | Integer | 0 to (224 −1) | The number of symbols measured from the start of the superframe before the receiver is to be enabled or disabled. |
| RxOnDuration | Integer | 0 to (224 −1) | The number of symbols for which the receiver is to be enabled. This property is equal to 0, if the receiver is to be disabled. |

The primitive enables the radio to receiver mode for a fixed duration, at a time relative to the start of the current or next superframe, or immediately during the CAP, or according to the TDD receiver mode, and without conflict with another higher priority operation of the radio transceiver.

In case of conflicting operation with a higher priority task of the radio transceiver, the PD will interrupt the receive operation, until the completion of the superseded operation. The property RxOnDuration will be checked to determine whether the time has expired. If so, the ReceiverEnable primitive operation is complete. If not, the receiver is re-enabled until either the PD has another superseded task, or the time specified by the property RxOnDuration has expired. When the primitive is issued to disable the receiver, the PD will disable its receiver unless the radio transceiver has a higher priority task.

This primitive may also be generated to cancel a previously generated request to enable the receiver. The receiver is enabled or disabled exactly once per primitive request.

If the RxOnDuration parameter is equal to zero, the MLME requests the PHY to disable its receiver.

      - MLME.ReceiverEnable.Confirm

This primitive reports the results of the attempt to enable or disable the receiver. The properties of this primitive are:

MLME.ReceiverEnable.Confirm {

Status;

}

The primitive parameters are defined in Table 20.

- —MLME.ReceiverEnable.Confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, PAST_TIME, ON_TIME_TOO_LONG, INVALID_PARAMETER | The result of the request to enable or disable the receiver. |

The primitive is generated by the MLME and issued to its next higher layer in response to an MLME.ReceiverEnable.Request primitive. This primitive returns a status of either SUCCESS, if the request to enable or disable the receiver was successful or the appropriate error code.

Before attempting to enable the receiver, the MLME first determines whether (RxOnTime + RxOnDuration) is less than the synchronization period interval. If (RxOnTime + RxOnDuration) is not less than the synchronization period interval, the MLME issues the MLME.ReceiverEnable.Confirm primitive with the property Status=ON_TIME_TOO_LONG.

The MLME then determines whether the receiver can be enabled in the current superframe. If the current time measured from the start of the superframe is less than (RxOnTime – macSIFSPeriod), the MLME attempts to enable the receiver in the current superframe. If the current time measured from the start of the superframe is greater than or equal to (RxOnTime – macSIFSPeriod) and DeferPermit is equal to TRUE, the MLME defers until the next superframe and attempts to enable the receiver in that superframe. Otherwise, if the MLME cannot enable the receiver in the current superframe and is not permitted to defer the receive operation until the next superframe, the MLME issues the MLME.ReceiverEnable.Confirm with the property Status=PAST_TIME.

    - Channel scanning

This primitive is used to initiate a channel scan over a given list of channels. The properties of this primitive are:

      - MLME.Scan.Request

The MLME-SCAN.request primitive is used to initiate a channel scan over a given list of channels. The properties of this primitive are:

MLME.Scan.Request{

ScanType;

ChannelPage;

ScanChannels;

ScanDuration;

}

The primitive parameters are defined in Table 21.

- —MLME.Scan.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| ScanType | Enumeration | ED, ACTIVE, PASSIVE. | Indicates the type of scan performed. |
| ChannelPage | Integer | Any valid channel page as indicated in Table TBD | The channel page on which to perform the scan. |
| ScanChannels | Array of integers | Any valid channel number as indicated in Table TBD | The channel numbers to be scanned |
| ScanDuration | Integer | TBD | A value used to calculate the length of time to spend scanning each channel for ED, active, and passive scans. |

      - MMLE.Scan.Confirm

This primitive reports the result of the channel scan request. The properties of this primitive are:

MLME.Scan.Confirm{

Status,

ScanType;

ChannelPage;

UnscannedChannels;

ResultListSize;

EnergyDetectList;

DetectedCategory;

}

The primitive parameters are defined in Table 22.

- —MLME.Scan.Confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, LIMIT_REACHED, SCAN_IN_PROGRESS, FRAME_TOO_LONG, INVALID_PARAMETER | The status of the scan request. |
| ScanType | Enumeration | ED, ACTIVE, PASSIVE | Indicates the type of scan performed |
| UnscannedChannels | Array of integers | Any valid channel number as defined in Table TBD | A list of the channels given in the request which were not scanned. This parameter is not valid for ED scans |
| ResultListSize | Integer | Implementation specific | The number of elements returned in EnergyDetectList. |
| EnergyDetectList | Array of integers | 0 to 255 per element in the array. | The list of energy measurements, one for each channel searched during an ED scan. This parameter is null for active, passive scans. |
| DetectedCategory | Integer | 0 to 2 | Categorization of energy detection with the following values: 0: Category detection is not supported 1: UWB PHY detected 2: Non-UWB PHY signal source detected. |

If the MLME receives a MLME.Scan.Request primitive while performing a previously initiated scan operation, the MLME will not perform the scan and the Status property will be set to SCAN_IN_PROGRESS. If the MLME.Scan.Request primitive requested an active or passive scan, the EnergyDetectList property will be NULL.

    - Group configuration

These primitives are used to initiate a PAC Group or to begin using a new superframe configuration.

      - MMLE.GroupStart.Request

This primitive is also used by a PD already peered with an existing PAC Group to begin using a new superframe configuration. The properties of this primitive are:

MLME.GroupStart.Request{

GroupID;

InitiatorID;

ChannelNumber;

ChannelPage;

StartTime;

SuperframeOrder;

}

The primitive parameters are defined in Table 23.

- —MMLE.GroupStart.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Initiator_ID | MAC address | PD specific | The address of the initiator PD |
| MulticastGroup_ID | Integer | 0 to 216 −1 | Group ID of initiator PD. |
| ChannelNumber | Integer | Any valid channel number a defined in Table TBD | The channel number to use. |
| ChannelPage | Integer | Any valid channel page as defined in Table TBD | The channel page to use. |
| StartTime | Unsigned integer | 0 to (224 −1) | The time at which to begin transmitting the synchronization period. If this parameter is equal to 0, the synchronization period will begin immediately. Otherwise, the specified time is relative to the received synchronization period of the current initiator PD. |

      - MMLE.GroupStart.Confirm

This primitive reports the results of the attempt to start using a new superframe configuration. The properties of this primitive are:

MLME.GroupStart.Confirm{

Status;

}

The primitive parameters are defined in Table 24.

- —MMLE.GroupStart.Confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, SUPERFRAME_OVERLAP, INVALID_PARAMETER, FRAME_TOO_LONG, CHANNEL_ACCESS_FAILURE | The result of the attempt to start a PAC Group or an updated superframe configuration. |

The MLME.Start.Confirm primitive returns a status of either SUCCESS, indicating that the MAC sublayer has started a new PAC Group or using the new superframe configuration. Otherwise, an appropriate error code is return as follows:

CHANNEL_ACCESS_FAILURE – The transmission lost synchronization.

FRAME_TOO_LONG – The length of the MPDU exceeds MaxPHYPacketSize.

SUPERFRAME_OVERLAP – The outgoing superframe overlaps the incoming superframe.

      - MMLE.GroupStart.Indication

The primitive is used to indicate the reception of GroupStart request command. The properties of this primitive are:

MLME.GroupStart.Indication{

PD_ID;

PHYcapability;

}

The primitive parameters are defined in

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| PD_ID | MAC address | PD specific | Address of the PD requesting GroupStart. |
| PHYcapability | Enumeration | LOW_MOBILITY, HIGH_MOBILITY, GFSK, UWB_BPM_BPSK, UWB_OOK | Operational capability of the PD requesting GroupStart. |

      - MMLE.GroupStart.Response

The primitive is used to initiate a response to an MLME.StartGroup.Indication primitive. The properties of this primitive are:

MLME.Peering.Response{

PD_ID;

MulticastGroup_ID;

Status;

}

The primitive parameters are defined in

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| PD_ID | MAC address | PD specific | Address of the PD requesting GroupStart. |
| MulticastGroup ID | Integer | 0 to 216 −1 | MulticastGroup address. This parameter is set to 0 if there is no multicastGroup address. |
| Status | Enumeration | SUCCESFUL, OUT_OF_CAPACITY, ACCESS_DENIED, | Status of the GroupStart attempt. |

    - .Request parameters

- .Confirm parameters

    - Synchronization

These primitives are used to synchronize with other PDs and to communicate loss of synchronization to the next higher layer.

      - MLME.Synchronization.Request

The primitive requests to synchronize with other PDs by acquiring and tracking their synchronization signal. The properties of this primitive are:

MLME.Synchronization.Request{

ChannelNumber;

ChannelPage;

TrackMode;

}

The primitive parameters are defined in Table 27.

- —MLME.Synchronization.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| ChannelNumber | Integer | As defined in Table TBD | The channel number on which to attempt synchronization. |
| ChannelPage | Integer | As defined in Table TBD | The channel page on which to attempt synchronization. |
| TrackMode | Boolean | TRUE, FALSE | TRUE if the PD is to synchronize from the next synchronization period. FALSE if the PD is to synchronize with only the next synchronization period. |

When the primitive is received by the MLME, it will set phyCurrentPage and phyCurrentChannel equal to the values of the ChannelPage and ChannelNumber, respectively. If the TrackMode parameter is equal to TRUE, the MLME will track the synchronization period, i.e., enable its receiver just before the expected time of synchronization period. If the TrackMode parameter is equal to FALSE, the MLME will locate the synchronization period, but not continue to track it.

If this primitive is received by the MLME while it is currently tracking the synchronization period, the MLME will not discard the primitive but will treat it as a new synchronization request.

      - MLME.Synchronization.Confirm

      - MLME.SynchronizationLoss.Indication

The primitive indicates the loss of synchronization. The properties of this primitive are:

MLME.SynchronizationLoss.Indication{

LossReason;

GroupID;

PD_ID;

ChannelNumber;

ChannelPage;

}

The primitive parameters are defined in Table 28.

- —MMLE.Synchronization.Indication parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| LossReason | Enumeration | SYNC_LOST, SUPERFRAME_OVERLAP, TBD | The reason synchronization was lost. |
| MulticastGroup_ID | Integer | 0 to 255 | Group ID of the PD that has lost synchronization |
| PD_ID | MAC address | PD specific | The MAC address of the PD that has lost synchronization |
| ChannelNumber | Integer | Any valid channel number as defined in Table TBD | The channel number on which the PD that has lost synchronization operated. |
| ChannelPage | Integer | Any valid channel page as defined in Table TBD | The channel page on which the PD that has lost synchronization operated. |

The primitive is generated by the MLME of a PD and issued to its next higher layer in the event of a loss of synchronization. The reason for synchronization loss is expressed by the parameter of LossReason:

SYNC_LOST – The PD has missed several synchronization periods.

SUPERFRAME_OVERLAP – If a change in the superframe configuration causes the outgoing superframe overlaps the incoming superframe.

    - Channel sounding

These primitives are used to obtain the results of a channel sounding from PDs that supports the channel sounding capability.

      - MMLE.ChannelSounding.Request

The primitive is used by the next higher layer to request that the PHY performs channel sounding by PDs with that capability. The properties of this primitive are:

MLME.ChannelSounding.Request{

ChannelNumber;

ChannelPage;

}

The primitive parameters are defined in Table 29.

- —MMLE.ChannelSounding.Request

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| ChannelNumber | Integer | Any valid channel number as defined in Table TBD | The channel number on which to attempt channel sounding. |
| ChannelPage | Integer | Any valid channel page as defined in Table TBD | The channel page on which to attempt channel sounding. |

      - MMLE.ChannelSounding.Confirm

The primitive reports the result of a request to the PHY to provide channel sounding information. Such information is an estimate of the SINR at the input antenna of the requestee PD. The properties of this primitive are:

MLME.ChannelSounding.Confirm{

Status;

SINR;

CQI;

}

The primitive parameters are defined in Table 30.

- —MMLE.ChannelSounding.Confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, FAIL, UNSUPPORTED | The status of the attempt to return channel sounding data. |
| SINR | Integer | −40 to 40 | SINR estimate at the input antenna in dBm. |
| CQI | Integer | 0 to 6 | CQI value measured during reception of the PSDU. 0 – very good 1 – good 2 – medium 3– bad |

    - Dynamic preamble selection

The dynamic preamble selection (DPS) primitives apply to the UWB PHY allowing the temporary use of different preamble codes as a means to thwart a hostile third party attack on the ranging procedures.

      - MLME.DPS.Request

This requests that the UWB PHY temporarily use the specified preamble code in transmitter and/or receiver for a single use. The properties of this primitive are:

MLME.DPS.Request {

TxDPSIndex;
RxDPSIndex;
DPSIndexDuration;

}

The primitive parameters are defined in Table 31.

- — MLME.DPS.Request parameters

| Field | Type | Value range | Description |
| --- | --- | --- | --- |
| TxDPSIndex | Integer | 0 to 48 | The code index value for the UWB PHY transmitter to use in UWB SHR generation. A value of 0 indicates that the phyPreambleCode value is used. See note below. |
| RxDPSIndex | Integer | 0 to 48 | The code index value for the UWB PHY receiver to use in SHR detection and reception. A value of 0 indicates that the phyPreambleCode value is used. See note below. |
| DPSIndexDuration | Integer | 0 to (224 −1) | This specifies a time in µs after which the DPS selection will revert to use the phyPreambleCode value in both transmitter and receiver, assuming it is not ended earlier by the issuing of either an MCP-DATA.request or an MCP-DATA.indication primitive. |

Note:	The TxDPSIndex and RxDPSIndex values select codes as per the “Preamble Sequence ID” column of Table XX for the BPM-BPSK modulation mode, and as per parameter i of Table XX for the OOK modulation mode where the DPS index specified here is i + 1.

The MLME responds with an MLME.DPS.Confirm primitive with the appropriate status parameter, and assuming the MLME.DPS.Request has been successful, the resulting altered preamble codes persist until one of the follow events:

- The DPS is cancelled by issuing a new MLME.DPS.Request with zero values for both TxDPSIndex and RxDPSIndex parameters,

- A frame is received and the MCP-DATA.indication primitive is generated,

- A frame is transmitted via an MCP-DATA.request primitive and the MCP-DATA.confirm primitive is generated,

- The period specified by the DPSIndexDuration parameter expires and the MLME.DPS.indication primitive is issued.

The use of DPS is further explained in  .

      - MLME.DPS.Confirm

The MLME.DPS.confirm primitive reports the results of the attempt to enable or disable DPS.

The properties of this primitive are:

MLME.DPS.Confirm {

Status;

}

The primitive parameters are defined in table <insert correct reference>

- —MLME.DPS.Confirm parameters

| Field | Type | Value range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, DPS_NOT_SUPPORTED | TBD |

The MLME.DPS.confirm primitive is generated by the MLME and issued to its next higher layer in response to an MLME.DPS.Request primitive.

If any parameter value in the MLME.DPS.request primitive is out of range is not supported by the PD in its current PHY operating mode as selected by phyModeSelection, the status of DPS_NOT_SUPPORTED is returned.  If the request to enable or disable the DPS is successful, the MLME issues the MLME.DPS.confirm primitive with a status of SUCCESS.

      - MLME.DPS.Indication

The MLME.DPS.indication primitive informs the next higher layer that the DPSIndexDuration period, specified in the MLME.DPS.request, has expired and the preamble codes in both the transmitter and the receiver have reverted to use the code specified by the phyPreambleCode value.

The properties of this primitive are:

MLME.DPS.Indication { <empty> }

There are no parameters for this primitive.

    - Ranging calibration

A PD with a UWB PHY that supports ranging may optionally include the ability to dynamically characterize the internal delays that give rise to the phyTxRmarkerOffset and phyRxRmarkerOffset quantities.  These quantities represent the propagation time from the internal transmit timestamp to the transmit antenna, and the propagation time from the receive antenna to the internal receiver timestamp.  The MLME.Calibrate primitives are defined to support the characterization of these delays.

      - MLME.Calibrate.Request

The MLME.Calibrate.request primitive attempts to have the PD respond with ranging marker offset information. There are no parameters for this primitive.

MLME.Calibrate.Request { <empty> }

In response to the MLME.Calibrate.request the MLME issues an MLME.Calibrate.confirm primitive.

      - MLME.Calibrate.Confirm

The MLME.Calibrate.confirm primitive reports the result of a previous MLME.Calibrate.request,

The properties of this primitive are:

MLME.Calibrate.Confirm {

Status; 
CalTxRMARKEROffset;
CalRxRMARKEROffset;

}

The primitive parameters are defined in Table 33.

- —MLME.Calibrate.Confirm parameters

| Field | Type | Value range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, UNSUPPORTED | The status result of the attempt to return the delay calibration data. |
| CalTxRMARKEROffset | Unsigned integer | 0 to (232 −1) TBD | A count of the propagation time from the transmit ranging counter to the transmit antenna. The units of this are defined in 10.4.1. |
| CalRxRMARKEROffset | Unsigned integer | 0 to (232 −1) | A count of the propagation time from the receive antenna to the receive ranging counter. The units of this are defined in 10.4.1. |

The MLME.Calibrate.Confirm primitive is generated by the MLME and issued to its next higher layer in response to an MLME.Calibrate.Request primitive.

In some implementations the MLME.Calibrate.Confirm may return values calibrated into the PD during its manufacture, in other cases the DP may perform a physical measurement of the delays. Either way, since system delays may depend on the PHY configuration, it is recommended to configure the required phyModeSelection, phyPreambleCode, etc. before initiating the MLME.Calibrate.Request.

If the MLME.Calibrate feature is supported, the MLME issues the MLME-CALIBRATE.confirm primitive with a Status of SUCCESS, indicating that the CalTxRMARKEROffset and CalRxRMARKEROffset values are valid, otherwise the returned Status the status parameter will be set to UNSUPPORTED.

  - MAC data service

The MCPS-SAP supports the transport of data information. Table 34 lists the primitives maintained by the MCPS-SAP.

- —MCPS-SAP primitives

| Name | Request | Confirm | Indication | Response |
| --- | --- | --- | --- | --- |
| MCPS.Data |  |  |  | − |

    - MCPS.Data.Request

The primitive requests the transfer of data to another PD. The properties of this primitive are:

MCPS.Data.Request {

phyModeSelection;

SourceAddress;

DestinationAddress;

MulticastGroup_ID;

MSDUlength;

MSDUHandle;

TransmitTimeOut;

DataType;

DataRate;

PurgeMode;

AckTX;

Ranging;

IeList;

RequestRriTx;

}

The primitive parameters are defined in Table 35.

- —MCPS.Data.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| SourceAddress | MAC address | PD specific | The address of the PD from which the frame originated. |
| phyModeSelection | Enumeration | LOW-MOBILITY, HIGH_MOBILITY, GFSK, UWB_BPM_BPSK, UWB_OOK | PHY selection |
| DestinationAddress | MAC address | PD specific | Address of the PD for which the frame is intended. |
| MulticastGroup_ID | Integer | 0 to 216 −1 | Group ID of a PAC network. |
| MSDUlength | Integer | TBD | MDSU length in octets. |
| MSDUhandle | Integer | TBD | Integer number identifying the MSDU. |
| TransmitTimeOut | Integer | TBD | The maximum allowed delay in microseconds from when the data is presented to the SAP until the frame has finished transmission and the acknowledgment, if required, is received. |
| DataType | Enumeration | AUDIO, VIDEO, DATA | Indicates the type of data that is sent in the stream. |
| DataRate | Integer | TBD | Indicates the data rate |
| PurgeMode | Boolean | TRUE, FALSE | If TRUE, the MSDU in the transaction buffer corresponding to MSDUhandle, is discarded from the transaction buffer. |
| AckTX | Boolean | FALSE, TRUE | This parameter is present only when phyModeSelection is UWB. TRUE if acknowledged transmission is used, FALSE otherwise. |
| Ranging | Boolean | FALSE, TRUE | This parameter is present only when phyModeSelection is UWB. TRUE if ranging bit in PHR is to be set, FALSE otherwise. |
| IeList | Set if IEs as described in Array of integers (IEs) | As described in Table TBD | This parameter is present only when phyModeSelection is UWB. Determines/supplies the IEs to be sent, including: RRRT IE, RRTD IE, RPRT IE, RRTM IE, RCDT IE and RTOF IE |
| RequestRrtiTx | Boolean | TRUE, FALSE | This parameter is present only when phyModeSelection is UWB. This parameter requests that the PD inserts an RRTI IE in the sent data frame. If the MCPS-DATA.request is early enough the data frame shall be sent at the configured macRngRrtiTime otherwise shall be sent as soon as possible thereafter. |

    - MCPS.Data.Confirm

The primitive reports the results of a request to transfer data to another PD. The properties of this primitive are:

MCPS.Data.Confirm {

MSDUhandle;

Status;

PurgeMode;

TxRangingCounter;

RxRangingCounter;

IeList;

TxRrtiValue;

AoaAzimuth;

AoaElevation;

AoaPresent;

Rssi;

}

The primitive parameters are defined in Table 36.

- —MCPS.Data.Confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| MSDUhandle | Integer | TBD | Integer number identifying the MSDU. |
| Status | Enumeration | SUCCESS, TRANSACTION_EXPIRED, CHANNEL_ACCESS_FAILURE, NO_ACK, COUNTER_ERROR, INVALID_PARAMETER, PURGE_SUCESS, INVALID_HANDLE | The status of the last MSDU transmission. |
| TxRangingCounter | Unsigned Integer | TBD | This parameter is present only when phyModeSelection is UWB. The timestamp of the transmitted data frame |
| RxRangingCounter | Unsigned Integer | TBD | This parameter is present only when phyModeSelection is UWB. For an acknowledged transmission this is the timestamp of the received acknowledgement frame. For a non-acknowledged transmission this parameter is invalid. |
| IeList | IEs received in the ACK | As defined in Table TBD | This parameter is present only when phyModeSelection is UWB. For an acknowledged transmission this reports the IEs received in the ACK including: RRTI IE |
| TxRrtiValue | Unsigned Integer | 0 to (232 −1) | This parameter is present only when phyModeSelection is UWB. This reports the value sent in the RRTI IE where this was requested by the RequestRrtiTx parameter of the MCPS-DATA.request. |
| AoaAzimuth | Float | -π to +π | This parameter is present only when phyModeSelection is UWB. For an acknowledged transmission this is the AOA of the received signal in azimuth measured in radians, relative to some PD specific axis defined by its antenna arrangement or orientation. This parameter is valid only when the AoaPresent parameter is either AZIMUTH or BOTH. |
| AoaElevation | Float | -π to +π | This parameter is present only when phyModeSelection is UWB. For an acknowledged transmission this is the AOA of the received signal AOA of the received signal in elevation measured in radians. This parameter is valid only when the AoaPresent parameter is either ELEVATION or BOTH. |
| AoaPresent | Enumeration | NONE, BOTH, AZIMUTH, ELEVATION | This parameter is present only when phyModeSelection is UWB. Indicates validity of AoaAzimuth and AoaElevation parameter. For a non-acknowledged transmission or where AOA is not supported this parameter value shall be NONE. |
| Rssi | Integer | 0 to 255 | This parameter is present only when phyModeSelection is UWB. For an acknowledged transmission this reports the received signal strength for the ACK frame. This is a measure of the RF power level at the antenna based on the gain setting in the RX chain and the measured signal level in the channel. For the UWB PHY the RSSI value is measured during the frame Preamble and locked when a valid SFD is detected. A value of zero indicates that RSSI measurement is not supported or was not measured for this frame. |

The MCPS.Data.Confirm primitive returns a status of either SUCCESS, indicating that the request to transmit was successful, or the appropriate error code.

If the requested transaction is too large to fit in the CAP or CFP, the MAC sublayer shall discard the frame and issue the MCPS.Data.Confirm primitive with a status of FRAME_TOO_LONG.

If the transmission uses CSMA-CA or CFP and  those failed due to adverse conditions on the channel, the MAC sublayer will discard the MPDU and the Status property will be set to CHANNEL_ACCESS_FAILURE.

If MCPS.Data.Request.PurgeMode is set to TRUE, the MCPS.Data.Confirm.Status property will be set to either PURGE_SUCCESS if an MSDU, matching the MSDUhandle property, was removed from the transaction buffer; otherwise INVALID_HANDLE.

    - MCPS.Data.Indication

The primitive indicates the reception of data from another PD. The properties of this primitive are:

MCPS.Data.Indication{

phyModeSelection;

SourceAddress;

DestinationAddress;

MulticastGroup_ID;

MSDUlength;

CQI;

DataSequenceNumber;

DataRate;

TxRangingCounter;

RxRangingCounter;

IeList;

AoaAzimuth;

AoaElevation;

AoaPresent;

Rssi;

}

The primitive parameters are defined in Table 37.

- —MCPS.Data.Indication parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| SourceAddress | MAC address | PD specific | The address of the PD from which the frame originated. |
| DestinationAddress | MAC address | PD specific | Address of the PD for which the frame is intended. |
| MulticastGroup_ID | Integer |  | Group ID of PAC network. |
| MSDUlength | Integer | TBD | MDSU length in octets. |
| CQI | Integer | 0 – 4 | ##### CQI value measured during reception of the PSDU. TBD |
| DataSequenceNumber | Integer | TBD | Data number sequence of the received data frame. |
| DataRate | Integer | TBD | Indicates the data rate. |
| RxRangingCounter | Unsigned Integer | 0 to (232 −1) TBD | This parameter is present only when phyModeSelection is UWB. The timestamp of the received data frame |
| TxRangingCounter | Unsigned Integer | TBD | This parameter is present only when phyModeSelection is UWB. If the received frame had the AR bit set, then this is the timestamp of the transmitted acknowledgement frame, otherwise this parameter is invalid. |
| IeList | Set if IEs as described in Array of integers (IEs) | As described in | This parameter is present only when phyModeSelection is UWB. Reports the IEs received including: RRRT IE, RRTD IE, RRTI IE, RPRT IE, RRTM IE, RCDT IE and RTOF IE |
| AoaAzimuth | Float | -π to +π | This parameter is present only when phyModeSelection is UWB. AOA of the received signal in azimuth measured in radians, relative to some PD specific axis defined by its antenna arrangement or orientation. This parameter is valid only when the AoaPresent parameter is either AZIMUTH or BOTH. |
| AoaElevation | Float | -π to +π | This parameter is present only when phyModeSelection is UWB. AOA of the received signal in elevation measured in radians. This parameter is valid only when the AoaPresent parameter is either ELEVATION or BOTH. |
| AoaPresent | Enumeration | NONE, BOTH, AZIMUTH, ELEVATION | This parameter is present only when phyModeSelection is UWB. Indicates validity of AoaAzimuth and AoaElevation parameter. Where AOA is not supported this parameter value shall be NONE. |
| Rssi | Integer | 0 to 255 | This parameter is present only when phyModeSelection is UWB. The received signal strength for received frame. This is a measure of the RF power level at the antenna based on the gain setting in the RX chain and the measured signal level in the channel. For the UWB PHY the RSSI value is measured during the frame Preamble and locked when a valid SFD is detected. A value of zero indicates that RSSI measurement is not supported or was not measured for this frame. |

  - PAC information base (PIB)

The PIB comprises the fields required to manage the MAC sublayer and PHY layer of a PD, and presented in Table 38.

The fields associate to the PHY layer and MAC sublayer use the prefix phy and mac, respectively. Constants are READ_ONLY.

- —PIB fields

| Field | Type | Range | Default | Description |
| --- | --- | --- | --- | --- |
| PD_ID | IEEE EUI-48 | READ_ONLY | Implementation specific | PD MAC address |
| phyMaxPacketSize | Integer | READ_ONLY | TBD | Maximum PSDU size in octets |
| phyCurrentChannel | Integer | As defined in table TBD | Implementation specific | The RF channel number used for Tx and Rx |
| phyCurrentPage | Integer | As defined in table TBD | Implementation specific | The RF channel page used for Tx and Rx |
| phyTurnaroundTime | Integer | READ_ONLY | TBD | RX-to-TX or TX-to-RX turnaround time (in symbol periods), as defined in |
| phyTXPower | Integer | TBD | Implementation specific | The transmit power of the PD in dBm |
| phyCCAMode | Integer | TBD | Implementation specific | The CCA mode as defined in |
| phySHRduration | Integer | TBD | TBD | The duration of the synchronization period in symbols |
| phyDiscoveryDuration | Integer | TBD | TBD | The duration of the discovery period in symbols |
| phyPeeringDuration | Integer | TBD | TBD | The duration of the peering period in symbols |
| phySymbolsperOctect | Float | TBD | Implementation specific | Number of symbols per octet |
| phyModeSelection | Enumeration | LOW_MOBILITY, HIGH_MOBILITY, GFSK, UWB_BPMBPSK, UWB_OOK. | Implementation specific | PHY selection |
| phyUWBPreambleCode | Unsigned Integer | For UWB PHY: 1 to 48 for the BPM-BPSK modulation mode; and, 1 to 33 for the OOK modulation mode. | Implementation specific | For the UWB PHY the value here shall select the preamble code to employ. The value selects the code as per the “Preamble Sequence ID” column of Table XX for the BPM-BPSK modulation mode, and as per parameter i of Table XX for the OOK modulation mode, (actually the phyPreambleCode value specified here is i+1). For other PHY the value here is ignored. |
| phyUWBTxRmarkerOffset | Unsigned Integer | 0 to (232 −1) TBD | TBD | The propagation time from the internal transmit timestamp to the transmit antenna. The units of this are defined in 10.4.1. The PD adds this offset to its internal transmit timestamp to get the reported TxRangingCounter value. |
| phyUWBRxRmarkerOffset | Unsigned Integer | TBD | TBD | The propagation time from the receive antenna to the internal receiver timestamp. The units of this are defined in 10.4.1. The PD subtracts this offset from its internal receive timestamp to get the reported RxRangingCounter value. |
| macUWBRngAckRrtiSupport | Boolean | READ_ONLY | TBD | Read only value indicating whether the PD is able to generate an RRTI IE within the standard ACK response time and insert it into a ranging ACK frame. |
| macUWBRngAckRrtiEnable | Boolean | TRUE, FALSE | TBD | When set the PD MAC shall automatically generate and insert an RRTI IE in a ranging ACK frame, where the RFRAME being acknowledged carried an RRRT IE and where macRngAckRrtiSupport indicates that the PD is capable of doing this. |
| macUWBRngMinRrtiTime | Unsigned Integer | READ ONLY | TBD | This is a read only value reporting the minimum RX to TX response time that the PD supports for inserting an RRTI IE into a MAC data frame. The units of this are defined in 10.4.1. A value of zero shall indicate that the PD cannot support RRTI insertion, i.e. so the upper layers know to employ an RRTD IE in the ranging exchanges. |
| macUWBRngRrtiTime | Unsigned Integer | TBD | TBD | This sets the desired RX-to-TX response time for the PD to use when inserting an RRTI IE into a MAC data frame. The units of this are defined in 10.4.1. The value set should take account of the macRngMinRrtiTime and any upper layer delays in initiating transmission, be reflected in the preferred response time indicated via RPRT IE exchanges. |
| Group_IDList | Array of integers | 0 – 255 per element in the array | Implementation specific | List if Group IDs |
| PD_IDList | Array of MAC addresses | PD specific | Implementation specific | List of PD IDs per Group ID |