IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | MAC frame format assignments |
| Date Submitted | November 9th, 2015 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Response | In response to Call for Contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1. MAC frame format	2

- MPDU formats

The MPDUs in the MAC sublayer are described as a sequence of fields in a specific order, and depicted in the order in which they are transmitted by the PHY, from left to right, where the leftmost bit is transmitted first in time. For each field with k bits, such bits are numbered from 0 (leftmost and least significant) to k – 1 (rightmost and most significant).

  - General MPDU format

The general MPDU shall be formatted as illustrated in Figure 1. The general MPDU consists of MAC header (MHR), MAC service data unit (MSDU) or payload and MAC check sequence (MCS).

| Octets: 2 | 1 |  | 2 | 4 |  | 4 | 1/2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frame control | Sequence number | Destination Group ID | Multicast group address | Destination address | Source Group ID | Source address | Link ID |

MHR

| 1? | variable | variable | variable | 1/2 |
| --- | --- | --- | --- | --- |
| Security header | IEs header | IEs payload | Payload | CS |

MHR                                   MSDU                    MCS

- —General MPDU format

    - Frame control field

The Frame Control field contains information defining the frame type, addressing fields, and other control flags. The Frame Control field shall be formatted as illustrated in Error: Reference source not found.

| Bits: 0-2 | 3 | 4 | 5 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- |
| MPDU type | Security enabled | Frame pending | Acknowledgment request | Reserved | Sequence number suppression | IE present |

| 10-11 | 12-13 | 14-15 |
| --- | --- | --- |
| Destination addressing mode | Frame version | Source addressing mode |

- —Frame control field

      - MPDU type field

The Frame Type field shall be set as defined in Table 1.

- —MPDU type field values

| b1 b0 | Description |
| --- | --- |
| 00 | Data |
| 01 | Acknowledgment |
| 10 | MAC command |
| 11 | Reserved |

      - Security enabled field (BJ)

      - Frame pending field

The Frame Pending field shall be set to one if the PD sending the frame has more data for the recipient. This field shall be set to zero otherwise and ignored on reception.

      - Acknowledgment request field (Igor)

The AR field specifies whether an acknowledgment is required from the recipient device on receipt of a data or MAC command frame. If this field is set to one, the recipient device shall send an acknowledgment frame only if, upon reception, the frame passes the filtering described in 5.2.2.2. If this field is set to zero, the recipient device shall not send an acknowledgment frame.

      - Sequence number suppression field

      - IE present field

The IE Present field shall be set to one if IEs are contained in the MAC frame. This field shall be set to zero otherwise.

      - Destination addressing mode field (Billy)

      - Frame version (BJ)

      - Source addressing mode field (Billy)

    - Sequence number field (BJ)

    - Multicast group address field (Billy)

    - Destination address field (Billy)

64 bits!

    - Source address field (Billy)

    - Link ID field (Billy)

    - Security header field (BJ)

    - IE field

The IE field is variable length and contains one or more IE. This field is comprised of the IE header and IE payload subfields. IEs shall be present only if the IE present field in the Frame control field is set to one. The format of the IE field is shown in Figure 3. Each IE consists of a descriptor as part of the MHR and payload as part of the MSDU as described in . This standard limits the number of IEs to 1 octet (256).

| Octets: 1 | variable |  | variable | variable |  | variable |
| --- | --- | --- | --- | --- | --- | --- |
| No of IEs | IE header 1 | … | IE header N | IE payload 1 | … | IE payload N |

IE header                                                                      IE payload

- —IE field format

    - Payload field (BJ)

    - Check sequence field (BJ)

  - Format of MPDU types

    - MPDU data format

The MPDU for data shall be formatted as illustrated in Figure 4.

| Octets: 2 | 1 | variable | variable | variable | variable | variable |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frame control | Sequence number | Addressing fields | Security fields | IE header | IE payload | Data payload | CS |

MHR                                                                                MSDU                       MCS

- —MPDU data format

      - MHR fields

In the Frame control field, the MPDU type field shall contain the value that indicates a MPDU data frame, as shown in Table 1. All other fields in the Frame control field shall be set appropriately according to the intended use of the Data MPDU.

The sequence number field shall contain the current value of macDsn.

The Addressing fields contain the destination address field and/or the source address field, dependent on the settings in the Frame control field.

The Security field, if present, shall contain the information required for security processing of the Data MPDU frame, as specified in .

      - MSDU fields

The MSDU fields or payload shall contain the sequence of octets that the next higher layer has requested the MAC sublayer to transmit.

    - MPDU ACK format

The MPDU ACK shall be formatted as illustrated in Error: Reference source not found.

| Octets: 2 | 1 |  |
| --- | --- | --- |
| Frame control | Sequence number | CS |

MHR                             MCS

- —MPDU  ACK format

The MPDU for Enhanced ACK format shall be formatted as illustrated in

| Octets: 2 | 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frame control | Sequence number | Addressing fields | Security fields | IE header | IE payload | Data payload | CS |

It may be employed when the ACK is used to carry addition content back like synch info, etc. in the payloads.

    - MPDU MAC command format

The MAC command shall be formatted as illustrated

| Octets: 2 | 1 | variable |  | variable |
| --- | --- | --- | --- | --- |
| Frame control | Sequence number | Addressing fields | Security fields | IE header |

MHR

| variable | 1 | variable |  |
| --- | --- | --- | --- |
| IE payload | Command ID | Content | CS |

MSDU                             MCS

- —MPDU MAC command format

      - MHR fields

The Sequence number field, if present, shall contain the current value of macDsn.

The addressing fields shall comprise the destination address fields and/or the source address fields, dependent on the settings in the Frame control field.

The Security field, if present, shall contain the information required for security processing of the MAC command, as specified in

      - Command ID field

The Command ID field identifies the MAC command being used, whose value is defined in Table

      - Payload field

The Payload field contains the MAC command itself. The formats of the individual commands are described in

  - IEs

    - IEs header

The nth IE header shall be formatted as shown in Figure 7.

| Octets: 1 | 1 | Variable |
| --- | --- | --- |
| Length | ID | Content |

- —IE header n format

The Length field specifies the number of octets in the Content field.

The IE header ID field is defined in Table 2.

- —IE header IDs

| ID | Name | Content |
| --- | --- | --- |
| 0 | Vendor specific | 1.3.1.1 |
| 1 | Time correction | 1.3.1.2 |
| 2 | UWB | 1.3.1.3 |
| 3-255 | Reserved |  |

      - Vendor specific

It may be used in a MPDU data frame or MPDU MAC command frame. It is created by the MAC user with use description in .

      - Time correction

      - UWB

    - IEs payload

The IE payload n is shown in Figure 8.

| Octets: 1 | 1 | Variable |
| --- | --- | --- |
| Length | ID | Content |

- —IE payload n format

The Length field specifies the number of octets in the Content field.

The IE payload ID field and Content sub-clause are defined in Table 3.

- —IE payload IDs

| ID | Name | Content description |
| --- | --- | --- |
| 0 | Encapsulated data IE | 1.3.2.1 |
| 1 | MLME IE | 1.3.2.2 |
| 2 | UWB IE | 1.3.3.2 |
| 3-255 | Reserved |  |

      - Encapsulated data IE

Encapsulated data is a higher layer payload that is carried in the Content field. It may be used in a MPDU data frame or MPDU MAC command frame.

      - MLME IE

The Content field of the MLME IE shall be formatted as illustrated in

| Octets: variable | variable |  | variable |
| --- | --- | --- | --- |
| Nested IE | Nested IE | … | Nested IE |

The nested IE format is defined in 1.3.3.

    - Nested IE format

The Nested IE shall be formatted as illustrated in Error: Reference source not found.

- —Nested IE format

| Octets: 1 | 1 | variable |
| --- | --- | --- |
| Length | Nested ID | Content |

The Length field specifies the number of octets in the Content field.

The Nested ID field values are defined in

- —Nested ID field values

| Nested ID | Name | Content description |
| --- | --- | --- |
| 0 | PHY capabilities IE | 1.3.3.1 |
| 1 | UWB |  |
| 2 | Reserved |  |

      - PHY capabilities IE

The PHY Capabilities IE Content field shall be formatted as illustrated in Figure 9.

| Bits:0-3 | 4-7 | Octets: 2 |
| --- | --- | --- |
| PHY type | Frequency band ID | PHY mode |

- —PHY Capabilities IE Content field format

The PHY type values are defined in Table 6.

- —PHY type values

| PHY type | Description |
| --- | --- |
| 0 | Low mobility PHY |
| 1 | High mobility PHY |
| 2 | GFSK |
| 3 | UWB |

The frequency band ID values are defined in Table 7.

- —Frequency band ID values

| Frequency band ID | Designation |
| --- | --- |
| 0 | 2.4 GHz |
| 1 | 5.7 GHz |
| 2 | Sub-GHz |
| 3 | UWB |

The PHY mode values are defined in Table 8.

- —PHY mode values

| PHY mode | Description |
| --- | --- |
| 0 | Data rate=1 Mbps; QPSK; |
| 1 |  |

PHY mode [coded] bit rate [FEC rate] modulation [mod index] [spreading] [chip rate] FFT size inter-carrier spacing

      - UWB