IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Ethertype compression |  |
| Date Submitted | 09 November, 2015 |  |
| Source | Tero Kivinen INSIDE Secure Eerikinkatu 28 FI-00180 Helsinki Finland | Voice: +358 20 500 7800 Fax: +358 20 500 7801 E-mail: kivinen@iki.fi |
| Re: | Ethertype compression related to sponsor ballot comments I-8 – I-13, and I-41. |  |
| Abstract | This document describes enhancement to the current 802.15.9 multiplexing layer which will compress the ethertype in some cases, so we can save 2 octets for small single frame packets.. |  |
| Purpose | Resolve the I-8 – I-13, and I-41 sponsor ballot comments. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Proposed new section 7.2 with changes:

# 7.2 MP IE Content

The contents of a MP IE are formatted as shown in Figure 9.

| Octets: 1 | 0/1 | 0/2 | 0/2 | Variable |
| --- | --- | --- | --- | --- |
| Transaction Control | Fragment Number | Total Upper Layer Frame Size | Multiuplex ID | Upper Layer Frame Fragment |

Figure 9 – MP IE Contents

The Transaction Control field is always there, and the rest of the fields depend on the transfer type. The summary of different MP IE formats can be found in Table 16.

## 7.2.1 Transaction Control field

The Transaction Control octet is as follows:

### 7.2.1.1 Transfer Type Field

Where the Transfer type can have the values as described in Table 18.

Table 16 – Summary of different MP IE formats

| Bits: 0-1 | 2-7 | Octets: 1 | 0/2 | 0/2 | Variable |
| --- | --- | --- | --- | --- | --- |
| Transfer type | Transaction ID | Fragment Number | Total Upper Layer Frame Size | Multiplex ID | Upper layer frame fragment |
| 0b00 | 0x00-0x3f | Omitted | Omitted | 0x0000-0xffff | Full upper layer frame |
| 0b00 | 0x01-0x3f | Omitted | Omitted | Omitted | Full upper layer frame for protocols having small Multiplex ID. The Multiplex ID of 0x01-0x3f will be encoded inside the Transaction ID field, and Multiplex ID field is omitted. |
| 0b01 | 0x00-0x3f | 0x00 | 0x0000-0xffff | 0x0000-0xffff | First fragment and total size of the packet, and Multiplex ID |
| 0b01 | 0x00-0x3f | 0x01-0xfe | Omitted | Omitted | Middle fragment |
| 0b10 | 0x00-0x3f | 0x01-0xfe | Omitted | Omitted | Last fragment |
| 0b11 | 0x00-0x3f | Omitted | Omitted | Omitted | Omitted (abort without telling max size responder can handle) |
| 0b11 | 0x00-0x3f | Omitted | 0x0000-0xffff | Omitted | Omitted (abort with information what is max size responder can handle). |

| Bit: 0-1 | 2-7 |
| --- | --- |
| Transfer type | Transaction ID |

Figure 10—Transaction Control Field

### 7.2.1.2 Transaction ID field

Transaction ID 6-bit field that is used to match all frames related to this transaction. It is selected by the originator to be unique between a pair of devices, and it is cannot be reused until the transaction is finished or aborted. This allows 64 simultaneous transaction between two devices. Note, that Transaction ID needs to be unique regardless of who is originator and who is responder, this means that originator needs to make sure it does not pick Transaction ID which is in use for incoming transaction.

When full upper layer frame is sent inside one frame (transfer type 0b00) the Transaction ID field does not have any meaning, as the whole transaction is done inside the single frame. If the transfer type 0b00 is used and multiplex ID used is between 0x01-0x3f, then the Transaction ID field contains the lowest 6-bits of multiplex ID and Multiplex ID field is omitted. If the transfer type 0b00 is used and multiplex ID is not in range of 0x01-0x3f, then Transaction ID of 0x00 is used, and Multiplex ID field contains multiplex ID.

### 7.2.1.3 Fragment Number field

Fragment Number field is only present if the Transfer type is 0b01 or 0b10. If the Fragment Number field has value of 0x00, indicating first fragment, then the Total Upper Layer Frame Size and Multiplex ID fields are also present. This means that non-initial frames have only 2 octet overhead.

### 7.2.2 Total Upper Layer Frame Size field

Total Upper Layer Frame Size tells the size of the incoming upper layer frame. When responder receives it, it can verify whether it can process the incoming upper layer frame or not. Total Upper Layer Frame Size is only present if the Fragment Number is 0x00 and Transaction type is 0b01, and it can optionally be present if the Transaction type is 0b11.

### 7.2.3 Multiplex ID field

The Multiplex ID field is used to multiplex different upper layer protocols. If the Multiplex ID field is larger than 0x5dc then its value is Ethertype, otherwise it is local value allocated in this specification. If the multiplex ID is between 0x01-0x3f, and transfer type 0b00 is used, more efficient encoding of the multiplex ID can be used.

Current allocated local values are described in Table 17.

| Ethertype (decimal) | Ethertype (hex) | Description |
| --- | --- | --- |
| 1 | 0x0001 | Key management protocol |

The Multiplex ID field is present if the Frame number is 0x00 and Transaction type is 0b01, or if the Transaction type is 0b00. The Multiplex ID field is also present if the Transfer Type is 0b00 and the Multiplex ID is not in range of 0x01-0x3f. If the Multiplex ID is between range of 0x01-0x3f and the Transfer Type is 0b00, then the multiplex ID is stored inside the Transaction ID field, and the Multiplex ID field is omitted.

### 7.2.4 Upper Layer Frame Fragment field

The Upper Layer Frame Fragment field is present in all other Transaction types than 0b11, but it can also be empty. For example if the originator wants to first ask whether the other end is able to process this big transaction, i.e. it can send IE having only Transaction type 0b01, with Fragment Number of 0x00, Total Upper Layer Frame Size, and the Multiplex ID, without any data in the Upper Layer Frame Fragment field. The responder can then either send empty Enhanced-Acknowledgment back or send back Enhanced-Acknowledgment with Transaction type 0b11 asking other end to abort and it can then also include the maximum size it can cope.

Table 18—Transfer Type Field

| Transfer Type | Name | Description |
| --- | --- | --- |
| 0b00 | Full frame | The Fragment Number and Total upper layer frame size fields are omitted, Multiplex ID is present (either in the Multiplex ID field, or in the Transaction ID field), and the Upper Layer Frame Fragment contains the whole upper layer frame packet |
| 0b01 | Non-last fragment | The Fragment Number field is always present, and if it is 0x00 meaning this is first fragment then the Total upper layer frame size and Multiplex ID fields are also present. The rest of the IE is Upper Layer Frame Fragment. |
| 0b10 | Last fragment | The Fragment Number field is always present, and Total upper layer frame size and Multiplex ID fields are always omitted (as this cannot be first fragment, as in that case the frame could have been fit in one fragment, i.e the Full frame format transfer type would have been used). The rest of the IE is Upper Layer Frame Fragment. |
| 0b11 | Abort | This is message from the responder to the originator indicating that he wishes to abort the transfer. This can be sent at any time, including response to the first fragment. The Fragment Number field and Multiplex ID fields are always omitted, but the Total upper layer frame size field might be present and if it is present that tells the maximum size packet the responder is willing to accept. Whether the Total Upper Layer Frame Size field is present is known from the length of the IE, i.e. if the length is 1 octet, then it is omitted, if it is 3 octets, then it is there. This can also be sent in case when originator wants to abort the transfer in process. |

## Examples:

### Old format:

When sending small KMP frame:

| Octets: 1 |  | 2 | Variable |
| --- | --- | --- | --- |
| Bits: 0-1 | 2-7 |  |  |
| Transfer type = 0b00 | Transaction ID = ?? | Multiplex ID = 0x01 | KMP Payload |

Total length of the MP IE will be 3 + length of KMP payload.

### New format:

Same frame:

| Octets: 1 |  | Variable |
| --- | --- | --- |
| Bits: 0-1 | 2-7 |  |
| Transfer type = 0b00 | Transaction ID = 6-bit Multiplex ID = 0x01 | KMP Payload |

Total length of the MP IE will be 1 + length of the KMP payload.

Frame which have larger Multiplex ID is using the old format like this:

| Octets: 1 |  | 2 | Variable |
| --- | --- | --- | --- |
| Bits: 0-1 | 2-7 |  |  |
| Transfer type = 0b00 | Transaction ID = 0x00 | Multiplex ID > 63 | KMP Payload |