IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Draft text for cyclic-superframe structure of PAC MAC |  |
| Date Submitted | Nov. 11, 2015 |  |
| Source | Seong-Soon Joo (ETRI) | E-Mail: [ssjoo@etri.re.kr] |
| Re: |  |  |
| Abstract | This document is a draft for defining cyclic-superframe structure of the PAC. |  |
| Purpose | This document provides the input for the draft PAC specification. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.15 Working Group or IEEE 802.15.8 Task Group. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Draft txtGorRPorSTD for varTitlePAR

IMPORTANT NOTICE: IEEE Standards documents are not intended to ensure safety, security, health, or environmental protection, or ensure against interference with or from other devices or networks. Implementers of IEEE Standards documents are responsible for determining and complying with all appropriate safety, security, environmental, health, and interference protection practices and all applicable laws and regulations.

This IEEE document is made available for use subject to important notices and legal disclaimers. 
These notices and disclaimers appear in all publications containing this document and may 
be found under the heading “Important Notice” or “Important Notices and Disclaimers 
Concerning IEEE Documents.” They can also be obtained on request from IEEE or viewed at http://standards.ieee.org/IPR/disclaimers.html.

- Overview

- Normative references

- Definitions

  - Definitions

  - Acronyms and abbreviations

- General description

  - General

  - Components of IEEE 802.15.8 PAC

  - Network Topologies

  - Architecture

  - Functional Overview

- MAC protocol

  - MAC functional description

    - Channel access

      - Superframe

      - Cyclic-Superframe

The cyclic-superframe structure is used for a PD to adjust transceiver wakeup timing for energy efficient QoS supported transmission and for avoiding interference with other PAC groups.

A cyclic-superframe is a cycle of a set of consecutive superframes. A superframe consists of a synchronization period (SP), a discovery period (DP), a peering period (PP), a CAP, and a CFP. Each period of a superframe can be set to active or inactive state to form different types of superframe. The value of superframe type is described by the period state descriptor, as defined in 5.2.2.2. Figure 1 illustrates the several types of superframe.

A cyclic-superframe consists of superframes of two different types: primary superframe and secondary superframe. The primary superframe and the secondary superframe shall be set to one of superframe types.

The cyclic superframe structure is described by the duration of cyclic-superframe (DCS), the number of primary superframe (NPS), the number of secondary superframe (NSS), the type of primary superframe, and the type of secondary superframe. The format of a cyclic-superframe is defined by the initiator of synchronization procedure, discovery procedure, peering procedure, and data transmission.

Primary and secondary superframes shall be allocated in a cyclic-superframe with one of distribution types:

- Distribution type 0: after all primary superframes are appeared, secondary superframes are appeared sequentially

- Distribution type 1: primary superframes and secondary superframes are appeared evenly over a cyclic-superframe; a cyclic-superframe is started with a primary superframe and a secondary superframe is appeared after NPS/NSS primary superframes

The distribution type shall be determined by DCS and NPS. If NPS is equal or larger than the half of DCS, distribution type 1 is applied for allocating primary and secondary superframes. Otherwise, distribution type 0 is applied.

Figure 1— The cyclic-superframe structure for PAC,

The format of cyclic-superframe shall be set by the higher layer. The higher layer designs a cyclic-superframe for supporting the following questions:

- how many discovery command frames will be sent in a certain interval

- how many peering command frames will be sent in a certain interval

- on which period and how many data frames will be transmitted in a certain interval

- how to minimize the wake-up time

- how to avoid interference with other PAC groups

Several examples of a cyclic-superframe structure are shown in Figure 2.

For synchronization, if the type of primary superframe and the type of secondary superframe are set to type b4’1000, transceiver is on only for the SP of all the superframes of a cyclic-superframe.

According to the required data rate and QoS for a peer link or a peer group, the cyclic-superframe structure can support energy efficient QoS supported data transmission, as illustrated in Figure 2. For medium rate reliable data transmission, the higher layer may choose a CFP for transmitting a data frame and select eight superframes long cyclic-superframe, six superframes long primary superframe, CFP active for the type of primary superframe, and SP active for the type of secondary superframe. With this configuration, one data frame is transmitted for every three inactive superframes over CFP. For burst reliable data transmission, three consecutive data transmission before sleeping for six inactive superframes is carried over CFP by specifying the cyclic-superframe structure with the duration of cyclic-superframe set to 9, the number of primary superframe set to 3, the type of primary superframe set to type b4’0001, and the type of secondary superframe set to type b4’1000.

Figure 2—For example cyclic-superframe structures

A PD is instructed to set the cyclic-superframe structure through MLME-Synchronization.request primitive, MLME-Discovery.request primitive, MLME-GroupStart.request primitive, MLME-Peering.request primitive, and MCPS-Data.request primitive. The cyclic-superframe shall be repeated between one-to-one peers or one-to-many peers or multicast group peers until the cyclic-superframe structure is changed.

The specification of a cyclic-superframe structure shall be advertised with CS IE, which is four bytes long header IE with the duration of cyclic-superframe, the number of primary superframe, the type of primary superframe, the type of secondary superframe, and the sequential order of superframe sending CS IE. The CS IE can be contained in a discovery request command frame, peering request command frame, peering response command frame, or data request frame.

The start of a cyclic-superframe is detected by the sequential order of superframe sending CS IE.

    - Starting and maintaining PACs

  - MPDU formats

    - General MPDU format

      - Frame Check Sequence (FCS)

    - IEs

      - Header IEs

      - CS IE

The CS IE Content field shall be formatted as illustrated in Figure 3.

| Octets: 1 | 1 | 1 | Bits: 0-3 | 4-7 |
| --- | --- | --- | --- | --- |
| Sequential Order | CS Duration | PS Number | PS Type | SS Type |

Figure 3—The CS IE Content field format

The Sequential Order field contains the sequential order of superframe sending CS IE from the start of a cyclic-superframe.

The CS Duration field contains the duration of cyclic-superframe.

The PS Number field contains the number of primary superframe.

The PS Type field contains the type of primary superframe, which shall be set as indicated in Table 1.

The SS Type field contains the type of secondary superframe, which shall be set as indicated in Table 1.

| Bit : 0 | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Discovery Period State | Peering Period State | CAP State | CFP State |

Figure 4—Period state descriptor

The Discovery Period State field shall set to one when discovery period of a superframe is active.

The Peering Period State field shall set to one when peering  period of a superframe is active.

The CAP field shall set to one when CAP period of a superframe is active.

The CFP field shall set to one when CFP period of a superframe is active.

- MAC services

  - MLME-SAP primitives

    - PIB access

    - Discovery primitives

These primitives are used for the discovery of neighboring PDs.

      - MLME-Discovery.Request

The primitive requests a PD to perform discovery. The properties of this primitive are:

MLME.Discovery.Request{

DiscoveryMode;

DiscoveryInfo;

ChannelPage;

ScanChannels;

CyclicSuperframeStructure;

}

The primitive parameters are defined in Table 1.

Table 1—MLME.Discovery.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DiscoveryMode | Enumeration | TX, RX | Indicates transmission or reception of discovery information. |
| DiscoveryInfo | Array of PAC IDs descriptors | As defined in Table 2 | Indicates the discovery information. |
| ChannelPage | Integer | Any valid channel page as defined in Table TBD | The channel page on which to attempt discovery. |
| ChannelNumber | Integer | Any valid channel number as defined in Table TBD | The channel number on which to attempt discovery. |
| ScanChannels | Array of integers | Any valid channel number as defined in Table TBD | The channel numbers to be used for discovery. |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 18 | Indicates the structure of cyclic-superframe |

In RX mode, channelNumber is zero.

Discovery descriptors are defined in Table 2.

Table 2—PAC ID description

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Device_ID | MAC address | PD specific | MAC address of the PD. |
| MulticastGroup_ID | Integer | 0 to 216−1 | Multicast Group addresss of PAC network |
| Application_ID | char | 14 octets | Application name, user name, login. |

Cyclic-superframe structure descriptors are defined in Table 28.

Table 18—Cyclic-superframe structure descriptior

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| CS duration | Integer | 1 octet | Duration of cyclic-superframe. |
| PS number | Integer | 1 octet | Number of primary superframe. |
| PS type | Integer | As defined in Figure 4 (5.2.2.2) | Type of primary superframe. |
| SS type | Integer | As defined in Figure 4 (5.2.2.2) | Type of secondary superframe. |

      - MLME.Discovery.Confirm

This primitive requests association with given PD. The properties of this primitive are:

MLME.Discovery.Confirm{

DiscoveryMode;

DiscoveryInfo;

ChannelPage;

ScanChannels;

Status;

}

The primitive parameters are defined in Table 3.

Table 3—MLME.Discovery.Confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DiscoveryMode | Enumeration | TX, RX | Indicates transmission or reception of discovery information. |
| DiscoveryInfo | Array of PAC IDs descriptors | As defined in Table 2 | Indicates discovery information. |
| ChannelPage | Integer | Any valid channel page as defined in Table TBD | The channel page on which discovery was attempted. |
| ChannelNumber | Integer | Any valid channel number as defined in Table TBD | The channel number on which discovery was attempted |
| ScanChannels | Array of integers | Any valid channel number as defined in Table TBD | The channel numbers used for discovery. |
| Status | Enumeration | SUCCESS, FAIL, DISCOVERY_IN_PROGRESS | Status of the discovery request |

    - Peering primitives

These primitives are used when a PD becomes associated with another PD.

      - MLME.Peering.Request

MLME.Peering.Request{

….

CyclicSuperframeStructure;

}

….

Table 19—MLME.Peering.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DiscoveryMode | Enumeration | TX, RX | Indicates transmission or reception of discovery information. |
| . . . |  |  |  |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 18 | Indicates the structure of cyclic-superframe |

….

Cyclic-superframe structure descriptors are defined in Table 28.

      - MLME.Peering.Confirm

      - MLME.Peering.Indication

The primitive is used to indicate the reception of a peering request command. The properties of this primitive are:

MLME.Peering.Indication{

….

CyclicSuperframeStructure;

}

….

Table 21—MLME.Peering.Indicaiton parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| . . . |  |  |  |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 18 | Indicates the structure of cyclic-superframe |

….

Cyclic-superframe structure descriptors are defined in Table 28.

    - De-peering primitives

    - Frame error notification primitives

    - Reset MAC sublayer

These primitives are used to reset the MAC sublayer. The execution is implementation specific.

    - Receiver enable

These primitives are used to enable or disable a receiver of a PD.

    - Channel scanning

This primitive is used to initiate a channel scan over a given list of channels. The properties of this primitive are:

    - Group configuration

These primitives are used to initiate a PAC Group or to begin using a new superframe configuration.

      - MLME.GroupStart.Request

MLME.GroupStart.Request{

….

CyclicSuperframeStructure;

}

….

Table 33—MLME.GroupStart.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| . . . |  |  |  |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 18 | Indicates the structure of cyclic-superframe |

….

Cyclic-superframe structure descriptors are defined in Table 28.

      - MLME.GroupStart.Confirm

      - MLME.GroupStart.Indication

MLME.GroupStart.Indication{

….

CyclicSuperframeStructure;

}

….

Table 35—MLME.GroupStart.Indicaiton parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| . . . |  |  |  |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 18 | Indicates the structure of cyclic-superframe |

….

Cyclic-superframe structure descriptors are defined in Table 28.

    - Synchronization

      - MLME.Synchronization.Request

MLME.Peering.Indication{

….

CyclicSuperframeStructure;

}

….

Table 37—MLME.Synchronization.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| . . . |  |  |  |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 18 | Indicates the structure of cyclic-superframe |

….

Cyclic-superframe structure descriptors are defined in Table 28.

      - MLME.Synchronization.Confirm

      - MLME.SynchronizationLoss.Indication

    - Channel sounding

  - MAC data service

    - MCPS.Data.Request

MCPS.Data.Request {

….

CyclicSuperframeStructure;

}

Table 4—MCPS.Data.Request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| . . . |  |  |  |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 18 | Indicates the structure of cyclic-superframe |

    - MCPS.Data.Confirm

    - MCPS.Data.Indication

The primitive indicates the reception of data from another PD. The properties of this primitive are:

MCPS.Data.Indication{

….

CyclicSuperframeStructure;

}

….

Table 5—MCPS.Data.Indication parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| . . . |  |  |  |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 18 | Indicates the structure of cyclic-superframe |

….

Cyclic-superframe structure descriptors are defined in Table 28.

  - PAC information base (PIB)

The PIB comprises the fields required to manage the MAC sublayer and PHY layer of a PD, and presented in Table 6.

The fields associate to the PHY layer and MAC sublayer use the prefix phy and mac, respectively. Constants are READ_ONLY.

Table 6—PIB fields

| Field | Type | Range | Default | Description |
| --- | --- | --- | --- | --- |
| . . . |  |  |  | . . . |