IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | MAC commands |
| Date Submitted | November 10th, 2015 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Response | In response to Call for Contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1.1 MAC commands	2

  - MAC commands

The MPDU MAC command format is defined in TBD . The remaining MSDU fields for Command ID and Content are defined in this clause as well as relevant MHR fields.

The MAC command IDs and associated content sub-clauses are listed in Table 1.

- —MAC commands

| Command ID | Command name | sub-clause |
| --- | --- | --- |
| 1 | Discovery request | 1.1.1 |
| 2 | Discovery response | 1.1.2 |
| 3 | Peering request | 1.1.3 |
| 4 | Peering response | 1.1.4 |
| 5 | De-peering notification | 1.1.5 |
| 6 | Data request | 1.1.6 |
| 7 | ReS request | 1.1.9 |
| 8 | Reserved |  |
| 9 | Reserved |  |

    - Discovery request command

This command shall be sent by an unassociated PD that wishes to discover other PDs.

The Frame Pending field in the MHR shall be set to zero and ignored upon reception, and the AR field shall be set to one.

The Discovery request command Content field shall be formatted as illustrated in Figure 1.

| Bit: 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Power source | Receiver on when idle | Security enable | Reserved | Reserved | Reserved | Reserved |

- —Discovery request command Content field format

The Power source field shall be set to one if the device is receiving power from the alternating current source. Otherwise, the Power source field shall be set to zero.

The Receiver on when idle field shall be set to one if the device does not disable its receiver to conserve power during idle periods.

The Security enable field shall be set to one if the device is capable of sending and receiving cryptographically protected MAC frames.

    - Discovery response command

The Frame pending field in the MHR shall be set to zero and ignored upon reception, and the AR field shall be set to one.

The Peering response command Content field shall be formatted as illustrated in Figure 2.

Octets: 1

Discovery status

- —Discovery response command Content field format

The values of the Discovering status field are defined in Table 2.

- —Discovery status field values

| Discovery status value | Description |
| --- | --- |
| 0 | Success |
| 1 | Access denied |
| 2 | Channel access failure |
| 3 | Discovery in progress |
| 4 ─ 255 | Reserved |

    - Peering request command

This command shall be sent by an unassociated PD that wishes to peer with a PAC network.

The Frame Pending field in the MHR shall be set to zero and ignored upon reception, and the AR field shall be set to one.

The Peering request command Content field shall be formatted as illustrated in Figure 3

| Bit: 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Power source | Receiver on when idle | Security enable | Reserved | Reserved | Reserved | Reserved |

- —Peering request command Content field format

The Power source field shall be set to one if the device is receiving power from the alternating current source. Otherwise, the Power source field shall be set to zero.

The Receiver on when idle field shall be set to one if the device does not disable its receiver to conserve power during idle periods.

The Security enable field shall be set to one if the device is capable of sending and receiving cryptographically protected MAC frames.

    - Peering response command

The Frame pending field in the MHR shall be set to zero and ignored upon reception, and the AR field shall be set to one.

The Peering response command Content field shall be formatted as illustrated in Figure 4.

| Octets: 6 | 1 |
| --- | --- |
| PD ID | Peering status |

- —Peering response command Content field format

The values of the Peering status field are defined in Table 3.

- —Peering status field values

| Peering status value | Description |
| --- | --- |
| 0 | Success |
| 1 | PAC Group at capacity |
| 2 | Access denied |
| 3 | Channel access failure |
| 4 ─ 255 | Reserved |

    - De-peering notification command

The Frame pending field in the MHR shall be set to zero and ignored upon reception, and the AR field shall be set to one.

The Peering response command Content field shall be formatted as illustrated in Figure 5.

Octets: 1

Reason

- — De-peering notification command Content field format

The values of the Peering status field are defined in Table 4.

- —De-peering reason values

| De-peering reason value | Description |
| --- | --- |
| 0 | Source wants to leave |
| 1 | Source requests destination to leave |
| 2 | Destination wants to leave |
| 3 | Destination requests source to leave |
| 4 ─ 255 | Reserved |

    - Data request command

The Data request command has no Content field.

There are three cases for which this command is sent:

- If the Data Request command is triggered by the reception of a MCPS.Data.Request primitive from the next higher layer.

- If the Data request command is being sent following the acknowledgment to a Peering request command.

- If the Data request command is being sent in response to the receipt of an indication that data are pending for that PD.

    - Group ID Conflict notification command

The PAN ID Conflict Notification command has no Content field.

The Frame pending field shall be set to zero and ignored upon reception, and the AR field shall be set to one.

The Destination Group ID field shall contain the value of macGroupId, while the Source Group ID field shall be omitted.

    - Realignment command

The Realignment command Content field shall be formatted as illustrated in Figure 6.

| Octets: 2 |  | 1 | 1 | 1 |
| --- | --- | --- | --- | --- |
| Group ID | Multicast group address | Operational band | Channel number | Channel page |

- —Realignment command Content field

The Frame pending field shall be set to zero and ignored upon reception. The AR field shall be set to zero.

The Group ID field shall contain the Group ID intended to be used for all future communications.

The Multicast group address field shall contain the multicast group address intended to be used for all future communications.

The operation band values are indicated in Table 5. If the status value is “Same”, the channel number/page remains the same. If the status value is “New”, the channel number/page shall use a new channel number/page for all future communications.

The channel number field shall contain the new channel number.

The page number field shall contain the new page number.

- —Operational band values

| Operational band value | Channel number status | Channel page status |
| --- | --- | --- |
| 0 | Same | Same |
| 1 | New | Same |
| 2 | Same | New |
| 3 | New | New |
| 4 − 255 | Reserved | Reserved |

    - Resource Element request command

The Resource Element request command Content field shall be formatted as illustrated in Figure 7.

| Octets: 2 | 2 | Bits: 0 | 1 | 2 − 7 |
| --- | --- | --- | --- | --- |
| RE length | RE index | RE direction | Allocation | Reserved |

- —RE Request command Content field format

The 8 most significant bits of the RE length field shall contain the number of samples of the RE in the time domain. The least significant bits of the RE length field shall contain the number of subcarriers of the RE in the frequency domain.

The 8 most significant bits of the RE length field shall contain the number of samples of the RE in the time domain. The least significant bits of the RE length field shall contain the number of subcarriers of the RE in the frequency domain.

The RE Direction field shall be set to one if the RE is receive-only. Conversely, this field shall be set to zero if the RE is transmit-only.

The Allocation field shall be set to one for RE allocation or zero for RE deallocation.