IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | MAC command frames (harmonized) |
| Date Submitted | December 21st, 2015 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Response | In response to Call for Contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1.1 MAC commands	2

  - MAC commands

The MAC command frame format is defined in 5.2. In this clause, the MAC Command Identifier and MAC Command Content fields are defined, as well as relevant MHR fields. The MHR addressing fields are described in 5.1.

The MAC command identifiers or IDs are listed in Table 1.

- —MAC commands

| Command ID | Command Name | sub-clause |
| --- | --- | --- |
| 1 | Discovery transmission | 1.1.1 |
| 2 | Discovery reception | 1.1.4 |
| 3 | Discovered request | 1.1.3 |
| 4 | Discovered response | 1.1.4 |
| 5 | Peering request | 1.1.5 |
| 6 | Peering response | 1.1.6 |
| 7 | De-peering notification | 1.1.7 |
| 8 | Group ID conflict notification | 1.1.8 |
| 9 | Realignment | 1.1.9 |
| 10 | ReS request | 1.1.10 |
| 11 − 255 | Reserved | − |

    - Discovery transmission command

This command shall be sent by a PD that wishes to transmit its discovery information.

The AR field shall be set to zero.

The Discovery transmission command Content field shall be formatted as illustrated in Figure 1.

| Bit: 0 | 1 | 2 − 7 | Octets: 21 |
| --- | --- | --- | --- |
| Reserved | Security enable | Reserved | Discovery information |

- —Discovery request command Content field format

The Security enable field shall be set to one if the device is capable of sending and receiving cryptographically protected MAC frames.

The discovery information field shall be set to the PD’s discovery information.

    - Discovery reception command

This command shall be sent by a PD that wishes to receive the discovery information of other PDs through its own PHY.

The AR field shall be set to zero.

The Discovery response command Content field shall be formatted as illustrated in Figure 4.

| Octets: 1 | Octets: 1 | Octets: 21 | … | Octets: 21 |
| --- | --- | --- | --- | --- |
| Discovery status | Discovered blocks | Discovery information | … | Discovery information |

- —Discovery response command Content field format

The values of the Discovering status field are defined in Table 3.

- —Discovery status field values

| Discovery status value | Description |
| --- | --- |
| 0 | Success |
| 1 | Access denied |
| 2 | Channel access failure |
| 3 | Discovery in progress |
| 4 ─ 255 | Reserved |

The Discovered blocks field contains the number of discovered PDs during the discovery period.

The Discovery information field contains the discovery information of a PD.

    - Discovered information request command

This command shall be sent by a PD that wishes to obtain the Discovered information of other PDs through another PD. The AR field shall be set to one.

The Discovered request command Content field shall formatted as illustrated in Figure 3.

| Bit: 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Power source | Receiver on when idle | Security enable | Discovery info enable Reserved | Reserved | Reserved | Reserved |

Octets: 0 – 21 (Variable)

Discovered information

- —Discovered request command Content field format

The Power source field shall be set to one if the device is receiving power from the alternating current source. Otherwise, the Power source field shall be set to zero.

The Receiver on when idle field shall be set to one if the device does not disable its receiver to conserve power during idle periods.

The Security enable field shall be set to one if the device is capable of sending and receiving cryptographically protected MAC frames.

The Discovery enable field shall be set to one if the PD’s discovery information is to be sent. Zero otherwise.

The discovery information field shall be set to the PD’s discovery information if the Discovery info enable field is set to one. Otherwise, the discovery information field shall be set to zeros.

    - Discovered information response command

The AR field shall be set to one.

The Discovered response command Content field shall be formatted as illustrated in Figure 4.

| Octets: 1 | Octets: 1 | Octets: 21 | … | Octets: 21 |
| --- | --- | --- | --- | --- |
| Discovered status | Discovered blocks | Discovered information | … | Discovered information |

- —Discovered response command Content field format

The values of the Discovering status field are defined in Table 3.

- —Discovered status field values

| Discovered status value | Description |
| --- | --- |
| 0 | Success |
| 1 | Access denied |
| 2 | Channel access failure |
| 3 | Discovery in progress |
| 2 | More discovered information follows |
| 3 ─ 255 | Reserved |

The Discovered blocks field contains the number of discovered PDs during the discovery period.

The Discovery information field contains the discovery information of a PD.

    - Peering request command

This command shall be sent by an unassociated PD that wishes to peer with a PAC network.

The AR field shall be set to one.

The Peering request command Content field shall be formatted as illustrated in Figure 5.

| Bit: 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Power source | Receiver on when idle | Security enable | Group enable | Reserved | Reserved | Reserved |

- —Peering request command Content field format

The Power source field shall be set to one if the device is receiving power from the alternating current source. Otherwise, the Power source field shall be set to zero.

The Receiver on when idle field shall be set to one if the device does not disable its receiver to conserve power during idle periods.

The Security enable field shall be set to one if the device is capable of sending and receiving cryptographically protected MAC frames.

    - Peering response command

The peering response command allows a PD to communicate the results of a peering attempt back to the PD requesting peering.

The Peering response command Content field shall be formatted as illustrated in Figure 6.

| Octets: 1/2 | 1 |
| --- | --- |
| Multicast/link ID address | Peering status |

- —Peering response command Content field format

The multicast/link ID address field shall contain the multicast address (2 octets) of the PAC group, and optionally the link ID address (1 octet) of the PAC group.

The values of the Peering status field are defined in Table 4.

- —Peering status field values

| Peering status value | Description |
| --- | --- |
| 0 | Success |
| 1 | PAC Group at capacity |
| 2 | Access denied |
| 3 | Channel access failure |
| 4 ─ 255 | Reserved |

    - De-peering notification command

The Source Addressing Mode field and the Destination Addressing Mode field shall be set to indicate addressing.

The Frame Pending field shall be set to zero and ignored upon reception, and the AR field shall be set to one.

The Destination Device ID field shall contain the device ID of the PD to which to re-peer with. The Destination Address field shall contain the 6-byte unicast address of the PD to which the re-peering request command is being sent. The Source Device ID field shall contain PD’s own device ID. The Source Address field shall contain the value of 48-bit device address.

The de-peering response command Content field shall be formatted as illustrated in Figure 7.

Octets: 1

Reason

- — De-peering notification command Content field format

The values of the de-peering status field are defined in Table 5.

- —De-peering reason values

| De-peering reason value | Description |
| --- | --- |
| 0 | PD wants to leave |
| 1 | PD is requested to leave |
| 2 | PD requests destination/group to leave |
| 3 ─ 255 | Reserved |

    - Group ID Conflict notification command

The Group ID Conflict Notification command has no Content field.

The AR field shall be set to one.

When a group initiator PD hears a multicast message from a PD that is not a member of its own multicast group this is the conflict of multicast group. The group initiator PD picks a new group ID and informs to all its members by realignment command.

    - Realignment command

The Realignment command Content field shall be formatted as illustrated in Figure 8.

| Octets: 2 | 1 | 1 | 1 |
| --- | --- | --- | --- |
| Multicast Group ID | Operational band | Channel number | Channel page |

- —Realignment command Content field

The AR field shall be set to zero.

The Group ID field shall contain the Group ID intended to be used for all future communications.

The Multicast group address field shall contain the multicast group address intended to be used for all future communications.

The operation band values are indicated in Table 6. If the status value is “Same”, the channel number/page remains the same. If the status value is “New”, the channel number/page shall use a new channel number/page for all future communications.

The channel number field shall contain the new channel number.

The page number field shall contain the new page number.

- —Operational band values

| Operational band value | Channel number status | Channel page status |
| --- | --- | --- |
| 0 | Same | Same |
| 1 | New | Same |
| 2 | Same | New |
| 3 | New | New |
| 4 − 255 | Reserved | Reserved |

    - Resource Element request command

The Resource Element request command Content field shall be formatted as illustrated in Figure 9.

| Octets: 2 | 2 | Bits: 0 | 1 | 2 − 7 |
| --- | --- | --- | --- | --- |
| RE length | RE index | RE direction | Allocation | Reserved |

- —RE Request command Content field format

The 8 most significant bits of the RE length field shall contain the number of samples of the RE in the time domain. The least significant bits of the RE length field shall contain the number of subcarriers of the RE in the frequency domain.

The 8 most significant bits of the RE length field shall contain the number of samples of the RE in the time domain. The least significant bits of the RE length field shall contain the number of subcarriers of the RE in the frequency domain.

The RE Direction field shall be set to one if the RE is receive-only. Conversely, this field shall be set to zero if the RE is transmit-only.

The Allocation field shall be set to one for RE allocation or zero for RE deallocation.