IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | PIB for MAC |
| Date Submitted | January 18th, 2016 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Response | In response to Call for Contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1. PIB (continue)	2

- PIB (continue)

- —PIB fields (continue)

| Field | Type | Range | Default | Description |
| --- | --- | --- | --- | --- |
| macAddress | IEEE EUI-48 | READ_ONLY | Implementation specific | PD MAC address |
| macPeeringPermit | Boolean | TRUE, FALSE | FALSE | Indication of whether a PD is currently allowing peering. A value of TRUE indicates that peering is permitted. |
| macSyncOrder | Integer | TBD | TBD | Indicates the frequency with which the sync period is transmitted, as defined in TBD |
| macDSN | Integer | 0 ─ 255 | ─ | The sequence number added to the transmitted Data frame or MAC command. |
| macREpermit | Boolean | TRUE, FALSE | TRUE | TRUE if the PD accepts RE requests. FALSE otherwise. |
| macMaxBE | Integer | TBD | TBD | The maximum value of the back-off exponent, BE, in the CSMA-CA algorithm, as defined in TBD |
| macMinBE | Integer | 0 ─ macMaxBE | TBD | The minimum value of the back-off exponent, BE, in the CSMA-CA algorithm, as defined in TBD |
| macMaxBackoffs | Integer | TBD | TBD | The maximum number of backoffs the CSMA-CA algorithm will attempt before declaring a channel access failure. |
| macMaxFrameRetries | Integer | TBD | TBD | The maximum number of retries allowed after a transmission failure. |
| macLIFSperiod | Integer | TBD | Implementation dependent | The minimum time forming a LIFS period. |
| macSIFSperiod | Integer | TBD | Implementation dependent | The minimum time forming a SIFS period. |
| macPromiscuousMode | Boolean | TRUE, FALSE | FALSE | A TRUE value indicates that the MAC sublayer accepts all frames received from the PHY. FALSE otherwise. |
| macRangingSupport | Boolean | TRUE, FALSE | FALSE | This indicates whether the MAC sublayer supports ranging features. |
| macResponseWaitTime | Integer | TBD | TBD | The maximum time, in multiples of aBaseSuperframeDuration, a PD shall wait for a response command to be available following a request command. |
| macRxOnWhenIdle | Boolean | TRUE, FALSE | FALSE | Indication of whether the MAC sublayer enables its receiver during idle periods. |
| macSecurityEnabled | Boolean | TRUE, FALSE | FALSE | A value of TRUE indicates that security is enabled in the MAC sublayer. FALSE otherwise. |
| macSuperFrameOrder | Integer | TBD | TBD | The length of the active portion of the outgoing superframe as defined in TBD. |
| macPriorityChannelAccess | Boolean | TRUE, FALSE | FALSE | A value of TRUE indicates that PCA is enabled. FALSE otherwise. |