IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Frequency bands and channels assignment |
| Date Submitted | January 20th, 2016 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Response | In response to Call for Contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1. Frequency bands and channels assignment	2

    - Channel assignments

Channel assignments are defined through a combination of channel numbers and channel pages as shown in Table 1.

A compliant PD shall support all channels allowed by regulations for the region in which such PD operates. An exception to this is the UWB PHY where specific channel assignments are defined in 10.

- —Channel assignments

| Channel page | Channel numbers | Frequency band | Description |
| --- | --- | --- | --- |
| 0 | 0 − 10 | Sub-GHz | 1.1 |
| 1 | 0 − 5 | Sub-GHz | 1.1 |
| 2 | 0 − 1 | Sub-GHz | 1.1 |
| 3 | 0 − 1 | Sub-GHz | 1.1 |
| 4 | 0 – x | 2.4 GHz | 1.1.1.2 |
| 5 | 0 – 7 | 2.4 GHz | 1.1.1.3 |
| 6 | 0 – x | 5.7 GHz | 1.1.1.4 |
| 7 | 0 − 14 | 5.7 GHz | 1.1.1.5 |

      - Sub-GHz band

The channelization of the Sub-GHz or 920 MHz band by regulations in Japan is shown in Error: Reference source not found.

- —Sub-GHz channelization (Japan)

| Channel page | Band | Central frequency (MHz) | n | Max Tx power (mW) |
| --- | --- | --- | --- | --- |
| 0 | A | fc=917+n | 0,1,…,10 | 1 |
| 1 | B | fc=922+n | 0,1,…,5 | 20 |
| 2 | C | fc=921.5+n | 0,1 | 250 |
| 3 | D | fc=928.7+n | 0,1 | 1 |

Channel page zero contains 11 channels available in the 920 MHz or sub-GHz band.

Channel page one contains 6 channels available in the 920 MHz or sub-GHz band.

Channel page two contains 2 channels available in the 920 MHz or sub-GHz band.

Channel page three contains 2 channels available in the 920 MHz or sub-GHz band.

      - Channelization of 2.4 GHz band (Low mobility PHY)

      - Channelization of 2.4 GHz band (High mobility PHY)

Channel page five contains 8 channels available in the 2.4 GHz band for the high mobility PHY.

The 2.4 GHz band ranges from 2.4 GHz to 2.4835 GHz and it is divided into 8 channels of 10 MHz. The central frequencies are given by

fc=2405 MHz +10n        for n=0,1,…,7

      - Channelization of 5.7 GHz band (Low mobility PHY)

      - Channelization of 5.7 GHz band (High mobility PHY)

Channel page seven contains 15 channels available in the 5.7 GHz band for the high mobility PHY.

The 5.7 GHz band ranges from 5.725 GHz to 5.875 GHz and it is divided into 15 channels of 10 MHz. The central frequencies are given by

fc=5730 MHz +10n      for n=0,1,…,14

7.1.1