IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | PHY frame structure |
| Date Submitted | January 20th, 2016 |
| Source | Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura (NICT) |
| Response | In response to Call for Contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

2. Preamble	3

    - PHY frame format

The PHY frame format or physical layer protocol data unit (PPDU) can be formed for 2 types of PPDU. The PPDU type 1 is formed by concatenating the PAC network synchronization preambles as described in 9.1.5 in the PSDU. The PPDU type 2 is formed by concatenating preamble, the PHY header (PHR), and PHY service data unit (PSDU), as illustrated in  and Table 1.

- —PPDU structure

- —PPDU types

| Type | Preamble | PHR | PSDU | Remarks |
| --- | --- | --- | --- | --- |
| I | ─ | ─ | Variable | PAC synchronization preambles |
| II | 7 symbols | 4 octets | Variable | Discovery, peering or data information |

    - Preamble

The preamble consist of five repetitions of ZC sequences of length 32, r = 1 and q = 0, followed by two ZC sequences of length 52, r = 1 and q = 0, as illustrated in Figure 2. A ZC sequence of length N is obtained as

ak=WNk22+qkN evenWNk(k+1)2+qkN odd

where WN=e−j2πrN is a primitive nth root of unity, r is a relative prime to N, q is any integer and  sequence index k = 1, 0,…,N−1.

- —Preamble structure

    - PHY header

The PHY header structure is illustrated in Figure 3, where the transmit order is from bit 0 to bit 31.

| Bit 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R0 | R1 | R2 | r | L0 | L1 | L2 | L3 | L4 | L5 | L6 | L7 | r | r |
| Data rate |  |  |  | PSDU length |  |  |  |  |  |  |  |  |  |

| 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B | S10 | S11 | S12 | S13 | S14 | S15 | S20 | S21 | S22 | S23 | S24 | S25 |
| Burst | 1st seed |  |  |  |  |  | 2nd seed |  |  |  |  |  |

| 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- |
| F | P1 | P2 | r | r |
| Frame type | PSDU type |  | Reserved |  |

- —PHY header structure

    - Data rate

Data rates are indicated by (R0, R1, R2), where R2 is the most significant bit (MSB) and R0 is the least significant bit (LSB).

    - PSDU length

A variable frame length is indicated by (L0, L1, L2, L3, L4, L5, L6, L7), where L7 is the MSB and L0 is the LSB.

    - Burst mode

The burst mode is indicated by (B) and defined in Error: Reference source not found. The burst mode supports higher throughput by allowing the transmission of consecutive frames without ACK.

- —Burst mode

| B | Status |
| --- | --- |
| 0 | Next package is not part of burst |
| 1 | Next package is part of burst |

    - Scrambler seeds

The two scrambler seeds (initial conditions) for the Gold code generator are indicated by (S10 S11S12 S13 S14 S15) for user ID and (S20 S21 S22 S23 S24 S25) for group ID.

    - Frame type

The frame type is indicated by (F) and defined in Table 3.

- —Frame type

| F | Status |
| --- | --- |
| 0 | Next package is type 1 |
| 1 | Next package is type 2 |

    - PSDU type

For frame type 2, the PSDU may convey the discovery or peering or data information. This is indicated by (P1, P2) and defined in Table 4.

- —PSDU type

| P1 P2 | PSDU status |
| --- | --- |
| 00 | Discovery |
| 01 | Peering |
| 10 | Data |
| 11 | Reserved |

    - Header check sequence

The PLCP shall append 4-bits from CRC-4 ITU error detection coding to the PHR information. The CRC-4-ITU shall be the one’s complement of the remainder generated by the modulo-2 division of the PHR information by the polynomial:

1+x+x4

The HCS bits shall be obtained in the transmit order as shown in Figure 4 after the PHR bits are processed in the shift register. The shift register stages shall be initialized to all ones.

- —CRC-4 ITU data processing

    - PSDU

The PSDU is formed by N frames. Each frame is sub-divided into 10 TDD slots: two switching slots (denoted as S) and eight data slots.  Each data slot consists of seven OFDM symbols with duration of 0.5 msec, as illustrated in Figure 5.

- —Generic PSDU structure

    - PAC synchronization preamble

The PAC synchronization preamble is described in sub-clauses 9.1.5.1, 9.1.5.2, 9.1.5.3, 9.1.5.4.

    - TDD frame

The TDD frame consists of two switching slots and eight data slots. For asymmetric traffic, three types of TDD frames are defined in Table 5.

- —TDD asymmetric frame types

| Type | Frame structure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 1 2 | S | Lt | Lt | Lt | Lt | S | Lr | Lr | Lr | Lr |
|  | S | Lt | Lt | S | Lr | Lr | Lr | Lr | Lr | Lr |
|  | S | Lt | Lt | Lt | Lt | Lt | Lt | S | Lr | Lr |

In frame type 1, the first four data slots, denoted as Lt, the PAC device (PD) transmits. After switching, in the remaining four data slots, the PD receives, Lr.

The TDD switching slot (denoted as S) consists of a guard interval, preamble for fine synchronization and channel sounding and control channel as illustrated in Figure 6. Note that the preamble and control channel are transmitted by the PD that was in receiving mode before the switching. The duration of S is 0.5 msec as well.

- —TDD switching slot

    - Guard interval

The last transmitted symbol of PD1 arrives to PD2 after a propagation time, Tp. Such symbol is detected over Tdec. PD2 switches from receiver to transmitter in Tsw. Then, the first transmitted symbol of PD2 arrives to PD1 after Tp. Consequently, the guard interval is given by

GI=Tp+Tdec+Tsw+Tp+Tcom

where Tcom is compensation time to align to FFT sampling.

Considering Tp=1 Km/3x108 m/s = 3.3 µsec (worst case), or Tp=10 m/3x108 m/s = 0.033 µsec (typical case), Tsw=500 nsec, Tdec=0.9 µsec, then GI=10 µsec.

    - TDD Preamble

TBD

    - Control channel

Control channels passes information for the PD in receiving mode about previous channel’s rank and TDD frame type.

Table 1—Channel’s rank

| C0 C1 | Channel’s rank |
| --- | --- |
| 00 | 1 |
| 01 | 2 |
| 10 | 3 |
| 11 | 4 |

Table 2—TDD frame type

| F0 F1 | Frame type |
| --- | --- |
| 00 | 0 |
| 01 | 1 |
| 10 | 2 |
| 11 | r |

9.2.1 to 9.2.13