IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Revised draft of the cyclic-superframe structure |  |
| Date Submitted | May 16, 2016 |  |
| Source | Seong-Soon Joo (ETRI) | E-Mail: [ssjoo@etri.re.kr] |
| Re: |  |  |
| Abstract | This document is a revised draft of cyclic-superframe structure. |  |
| Purpose | This document provides the input for the TG8 WG letter ballot draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.15 Working Group or IEEE 802.15.8 Task Group. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Draft txtGorRPorSTD for varTitlePAR

IMPORTANT NOTICE: IEEE Standards documents are not intended to ensure safety, security, health, or environmental protection, or ensure against interference with or from other devices or networks. Implementers of IEEE Standards documents are responsible for determining and complying with all appropriate safety, security, environmental, health, and interference protection practices and all applicable laws and regulations.

This IEEE document is made available for use subject to important notices and legal disclaimers. 
These notices and disclaimers appear in all publications containing this document and may 
be found under the heading “Important Notice” or “Important Notices and Disclaimers 
Concerning IEEE Documents.” They can also be obtained on request from IEEE or viewed at http://standards.ieee.org/IPR/disclaimers.html.

- Overview

- Normative references

- Definitions

  - Definitions

cyclic-superframe: A periodic time interval comprised of multiple primary and secondary superframes.

  - Acronyms and abbreviations

DCS	duration of cyclic-superframe

DP	discovery period

NPS	number of primary superframe

NSS	number of secondary superframe

PP	 peering period

SP	synchronization period

- General description

  - General

  - Components of IEEE 802.15.8 PAC

  - Network Topologies

  - Architecture

  - Functional Overview

    - Superframe structure

The superframe structure is defined as illustrated in Figure 1. The superframe is divided into 5 periods in the following order: synchronization period (SP), discovery period (DP), peering period (PP), contention access period (CAP), and contention free period (CFP).

The duration of the superframe is fixed to [TBD]. The duration of each period is fixed and the value is [TBD].

Figure 1—The superframe structure

      - Cyclic-Superframe

A cyclic-superframe structure is used to reduce power consumption and to avoid interference among PAC groups, as defined in 5.1.1.2.

The PDs of PAC groups located within communication range are synchronized with a superframe structure. A PD monitors discovery period, peering period, CAP, and CFP of every superframe to receive a frame from peers. To allow PDs of a PAC group to sleep and to wake up synchronously at specific period of a superframe, a cyclic-superframe structure supports for a PAC group capable of adjusting accessible period of a superframe and interval of waking up. A PAC group may select the quality of communication by configuring a cyclic-superframe structure.

A PAC group contends with neighbor PAC groups to transmit a frame in a period of a superframe. To avoid interference among PAC groups, a cyclic-superframe structure supports for a PAC group capable of exhibiting a pattern of accessible period of multiple superframes to neighbors.

    - Communication types

- MAC protocol

  - MAC functional description

    - Channel access

      - Superframe

      - Cyclic-Superframe

A cyclic-superframe is a periodic time interval comprised of multiple superframes.

Each period of a superframe of a cyclic-superframe except the synchronization period can be configured whether a PAC group can access or not. Figure 2 a) illustrates twelve combinations of accessibl. periods of a superframe in a cyclic-superframe. The four-bit indicator is set to control the accessible states of DP, PP, CAP, and CFP of a superframe, respectively, as defined in Figure 5.

A superframe of a cyclic-superframe shall have primary type of accessible periods or secondary type of accessible periods. The first appeared superframe in a cyclic-superframe has primary type of accessible periods and is named primary superframe. After placing all the primary superframes from the start of a cyclic-superframe, secondary superframes, which have secondary type of accessible periods, are placed contiguously, as shown in Figure 2 b). Primary or secondary superframe can take any combination of accessible periods of a superframe

The cyclic superframe structure shall be described by the duration of cyclic-superframe (DCS), the number of primary superframe (NPS), the number of secondary superframe (NSS), the type of accessible periods of primary superframe, and the type of accessible periods of secondary superframe. The maximum duration of a cyclic-superframe is 255 superframes.

Figure 2—The cyclic-superframe structure for PAC

The cyclic-superframe structure of a PAC group is selected according to the service type of a PAC group and the phase of peer aware communication, such as synchronization, discovery, peering, and data transmission phase.

The cyclic-superframe structure is specified by the initiator of discovery phase or peering phase, or data transmission phase. A PD shall be instructed to set the cyclic-superframe structure through MLME-GROUP-START.request primitive, MLME-DISCOVERY.request primitive, MLME-PEERING.request primitive, and MCPS-DATA.request primitive with CyclicSuperframeStructure parameter, which consists of the duration of cyclic-superframe, the number of primary superframe, the type of primary superframe, and the type of secondary superframe. The cyclic-superframe shall be repeated between one-to-one peers or one-to-many peers, or many-to-many peers of a PAC group until the cyclic-superframe structure is changed.

The initiator of a communication phase shall configure the cyclic-superframe structure and select the time to start a new cyclic-superframe structure that minimizes the interference caused by this new cyclic-superframe to neighbor PAC groups. The initiator of a communication phase shall advertise the specification of a cyclic-superframe structure with Cyclic-superframe descriptor IE, as defined in 5.2.4.3.1. Cyclic-superframe descriptor IE is four bytes long header IE and consists of the duration of cyclic-superframe, the number of primary superframe, the type of primary superframe, the type of secondary superframe, and the sequence number of the superframe sending Cyclic-superframe descriptor IE in a cyclic-superframe. The next start of a cyclic-superframe is calculated from the duration of cyclic-superframe and the sequence number of the superframe sending Cyclic-superframe descriptor IE.

The Cyclic-superframe descriptor IE can be contained in a discovery request command frame or peering request command frame, or data frame. The neighbor PAC groups operating within communication range can collect advertised cyclic-superframe structure by scanning accessible periods of a cyclic-superframe and will store this information for selecting

the time to start a new cyclic-superframe structure in the future.

The cyclic-superframe structure shall be set by the higher layer, as described in Annex F. The higher layer designs a cyclic-superframe in the consideration of following questions:

- how long discovery phase will last

- how long peering phase will last

- on which period and how many data frames will be transmitted in a certain interval

- how to minimize the duration of wake-up period

- how to avoid interference with other PAC groups

Several examples of a cyclic-superframe structure are shown in Figure 3.

In synchronization phase, for transceiver to be accessible only SP of all the superframes, the type of primary superframe and the type of secondary superframe are set to default value, b4’0000, as shown in Figure 3 a). The synchronization phase is ended when the cyclic-superframe structure is changedrfor supporting the discovery phase. In discovery phase, to let DP and Pd be accessible for every four superframes, the values of DCS, NPS, the type of primary superframe, and the type of secondary superframe are set to 4, 3, b4’0000, and b4’1100 respectively, as shown in Figure 3 b).

In data transmission phase, according to the required data rate and QoS for the service of a PAC group, the cyclic-superframe structure can support QoS guaranteed energy efficient data transmission by configuring transmission interval and by selecting accessible periods. As an example, for medium rate best effort data transmission, the higher layer may choose a CAP for transmitting a data frame, select six superframes for the duration of cyclic-superframe, select five superframes for the number of primary superframe, set DP accessible for the primary superframe, and set CAP accessible for the secondary superframe, as shown in Figure 3 c). With this configuration, PDs of a PAC group try to transmit a data frame over CAP everyesix superframes.

For burst reliable data transmission, three consecutive data frames transmission over CFP before sleeping for the duration of six superframes is carried by configuring the cyclic-superframe structure with DCS set to 9, NPS set to 3, the type of primary superframe set to b4’1101, and the type of secondary superframe set to b4’0000, as shown in Figure 3 d).

The configuration of DCS and NPS for a cyclic-superframe depends on frequency of message transmission or data rate. If a discovery request message needs to be transmitted every X ms, DCS will be set as nearest integer of X divided by aSuperframeDuration and NPS will be set as 1, and the PP and CAP of primary superframe are set to accessible. If one-to-one peer needs a best effort Y bps data rate link with a data frame arrived every Z ms and the average transmission rate in a CAP of a superframe is AvgRateCAP, DCS will be set as nearest integer of Z divided by aSuperframeDuration and NPS will be set as nearest integer of Y divided by AvgRateCAP, and the CAP of primary superframe is set to accessible.

Figure 3—Example cyclic-superframe structures

    - Starting and maintaining PACs

  - MAC frame formats

    - Device addresses

    - General MAC frame format

    - Format of individual frame types

    - Information Elements (IEs)

      - General IE format

      - IE list termination

      - Header IEs

The Header IEs are defined in Table 1, Table 2 and Table 3, one table for each of the IE classes.  These tables are ordered by IE ID number with a description for each and reference to the sub-clause that defines the IE and its usage.

Table 1—Class 0 Header IEs

| Class 0 Header IE ID | IE Description | Acronym | Sub-clause |
| --- | --- | --- | --- |
| 0 | Header IE list terminator. | - | 5.2.4.2 |
| 1 | Cyclic-superframe descriptor IE | - | 5.2.4.3.1 |
| 2 to 15 | Reserved | - | - |

Table 2—Class 1 Header IEs

| Class 1 Header IE ID | IE Description | Acronym | Sub-clause |
| --- | --- | --- | --- |
| 0 | Link-ID Assignment IE | - | Error: Reference source not found |
| 0 to 511 | Reserved | - | - |

Table 3—Class 3 Header IEs

| Class 3 Header IE ID | IE Description | Acronym | Sub-clause |
| --- | --- | --- | --- |
| 0 to 15 | Reserved | - | - |

        - Cyclic-superframe descriptor IE

The Cyclic-superframe descriptor IE Content field shall be formatted as illustrated in Figure 4.

| Octets: 1 | 1 | 1 | Bits: 0-3 | 4-7 |
| --- | --- | --- | --- | --- |
| Sequence Number | Cyclic-superframe Duration | Primary superframe Number | Primary superframe Type | Secondary superframe Type |

Figure 4—The Cyclic-superframe descriptor IE Content field format

The Sequence Number field contains the sequence number of the superframe sending Cyclic-superframe descriptor IE counting from the start of a cyclic-superframe.

The Cyclic-superframe Duration field contains the number of superframes in a cyclic-superframe.

The Primary superframe Number field contains the number of primary superframe in a cyclic-superframe.

The Primary superframe Type field contains the type of accessible periods of primary superframe, which shall be set as indicated in Figure 5.

The Secondary superframe Type field contains the type of accessible periods of secondary superframe, which shall be set as indicated in Figure 5.

| Bit : 0 | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Discovery Period State | Peering Period State | CAP State | CFP State |

Figure 5—Accessible period indicator

The Discovery Period State field shall set to one when discovery period of a superframe is accessible. Otherwise it is set to zero.

The Peering Period State field shall set to one when peering period of a superframe is accessible. Otherwise it is set to zero.

The CAP State field shall set to one when contention access period of a superframe is accessible. Otherwise it is set to zero..

The CFP State field shall set to one when contention free period of a superframe is accessible. Otherwise it is set to zero..

  - Synchronization procedure

  - Discovery

  - Peering

  - Communication period

  - MAC commands

- MAC services

  - MLME-SAP primitives

    - PIB access

    - Discovery primitives

These primitives are used for the discovery of neighboring PDs.

      - MLME-DISCOVERY.request

The primitive requests a PD to perform discovery. The properties of this primitive are:

MLME-DISCOVERY.request{

DiscoveryMode;

DiscoveryInfo;

ChannelPage;

ScanChannels;

CyclicSuperframeStructure;

}

The primitive parameters are defined in Table 4.

                - Table 4—MLME-DISCOVERY.request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DiscoveryMode | Enumeration | TX, RX | Indicates transmission or reception of discovery information. |
| DiscoveryInfo | Array of PAC IDs descriptors | As defined in Table 5 | Indicates the discovery information. |
| ChannelPage | Integer | Any valid channel page as defined in Error: Reference source not found | The channel page on which to attempt discovery. |
| ChannelNumber | Integer | Any valid channel number as defined in Error: Reference source not found | The channel number on which to attempt discovery. |
| ScanChannels | Array of integers | Any valid channel number as defined in Error: Reference source not found | The channel numbers to be used for discovery. |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 6 | Indicates the structure of cyclic-superframe |

In RX mode, channelNumber is zero.

Discovery descriptors are defined in Table 5.

                - Table 5—PAC ID description

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Device_ID | MAC address | PD specific | MAC address of the PD. |
| MulticastGroup_ID | Integer | 0 to 216−1 | Multicast Group address of PAC network |
| Application_ID | char | 14 octets | Application name, user name, login. |

Cyclic-superframe structure descriptors are defined in Table 6. The default value of duration of cyclic-superframe, number of primary superframe, type of primary superframe, and type of secondary superframe are set to 1, 1, b4’0000, and b4’0000, respectively.

                - Table 6—Cyclic-superframe structure descriptor

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Duration of cyclic-superframe | Integer | 1 octet | Number of superframes in a cyclic-superframe. |
| Number of primary superframe | Integer | 1 octet | Number of primary superframe in a cyclic-superframe |
| Type of primary superframe. | Integer | As defined in Figure 5 (5.2.4.3.1) | Type of accessible periods combination of primary superframe |
| Type of secondary superframe | Integer | As defined in Figure 5 (5.2.4.3.1) | Type of accessible periods combination of secondary superframe |

When receiving the MLME-DISCOVERY.request primitive with the CyclicSuperfreamStructure parameter, the MAC sublayer sets the PIB with the value of the cyclic-superframe structure descriptor and selects the start time of a cyclic-superframe. After starting a cyclic-superframe, the MAC sublayer inserts the Cyclic-superframe descriptor IE to the MAC header of the Discovery Request command frame. The MAC sublayer schedules to access a period of a superframe of a cyclic-superframe.

      - MLME-DISCOVERY.confirm

This primitive requests association with given PD. The properties of this primitive are:

MLME-DISCOVERY.confirm{

DiscoveryMode;

DiscoveryInfo;

ChannelPage;

ScanChannels;

CyclicSuperframeStructure;

Status;

}

The primitive parameters are defined in Table 7.

Table 7—MLME-DISCOVERY.confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DiscoveryMode | Enumeration | TX, RX | Indicates transmission or reception of discovery information. |
| DiscoveryInfo | Array of PAC IDs descriptors | As defined in Table 5 | Indicates discovery information. |
| ChannelPage | Integer | Any valid channel page as defined in Error: Reference source not found | The channel page on which discovery was attempted. |
| ChannelNumber | Integer | Any valid channel number as defined in Error: Reference source not found | The channel number on which discovery was attempted |
| ScanChannels | Array of integers | Any valid channel number as defined in Error: Reference source not found | The channel numbers used for discovery. |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 6 | Indicates the structure of cyclic-superframe |
| Status | Enumeration | SUCCESS, FAIL, DISCOVERY_IN_PROGRESS | Status of the discovery request |

When receiving the Discovery Request command frame with the Cyclic-superframe descriptor IE, the MAC sublayer sets the PIB with the value of Cyclic-superframe Duration field, Primary superframe Number field, Primary superframe Type field, and Secondary superframe Type field. The MAC sublayer notifies the receiving the cyclic-superframe structure to the higher layer with MLME-DISCOVERY.confirm primitive. The MAC sublayer calculates the next start of a cyclic-superframe structure with the value of Sequence Number field and of Cyclic-superframe Duration field, and starts to schedule to access a period of a superframe of a cyclic-superframe.

    - Peering primitives

These primitives are used when a PD becomes associated with another PD.

      - MLME-PEERING.request

This primitive requests association with given PD. The properties of this primitive are:

MLME-PEERING.request{

ChannelNumber;

ChannelPage;

GroupMode

MulticastGroup_ID;

DestinationAddress;

CyclicSuperframeStructure;

}

The primitive parameters are defined in Table 8.

Table 8—MLME-PEERING.request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| ChannelNumber | Integer | Any valid channel number as defined in Error: Reference source not found | The channel number on which to attempt peering. |
| ChannelPage | Integer | Any valid channel page as defined in Error: Reference source not found | The channel page on which to attempt peering. |
| GroupMode | Integer | As defined in Error: Reference source not found | Group mode determines the type of peering procedure |
| MulticastGroup_ID | Integer | 0 to 216 −1 | Group ID provided by the application layer. |
| DestinationAddress | MAC address | IEEE 48 bit addressing | Address of the PD with which to peer for one-to-one peering. |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 6 | Indicates the structure of cyclic-superframe |

When receiving the MLME-PEERING.request primitive with the CyclicSuperfreamStructure parameter, the MAC sublayer sets the PIB with the value of the cyclic-superframe structure descriptor and selects the start time of a cyclic-superframe. After starting a cyclic-superframe, the MAC sublayer inserts the Cyclic-superframe descriptor IE to the MAC header of the Peering Request command frame. The MAC sublayer schedules to access a period of a superframe of a cyclic-superframe.

      - MLME-PEERING.confirm

      - MLME-PEERING.indication

The primitive is used to indicate the reception of a peering request command. The properties of this primitive are:

MLME-PEERING.indication{

SourceID;

GroupMode

MulticastGroupID

PHYcapability;

CyclicSuperfameStructure;

}

The primitive parameters are defined in Table 9.

Table 9—MLME-PEERING.indication parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| SourceID | MAC address | PD specific | Address of the PD requesting peering. |
| GroupMode | Integer | As defined in Error: Reference source not found | Group mode determines the type of peering procedure |
| MulticastGroup_ID | Integer | 0 to 216 −1 | Group ID of the requested group. |
| PHYcapability | Enumeration | LOW_MOBILITY, HIGH_MOBILITY, GFSK, UWB_BPM_BPSK, UWB_OOK | Operational capability of the PD requesting peering. |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 6 | Indicates the structure of cyclic-superframe |

When receiving the Peering Request command frame with the Cyclic-superframe descriptor IE, the MAC sublayer sets the PIB with the value of Cyclic-superframe Duration field, Primary superframe Number field, Primary superframe Type field, and Secondary superframe Type field. The MAC sublayer notifies the receiving the cyclic-superframe structure to the higher layer with MLME-PEERING.indication primitive. The MAC sublayer calculates the next start of a cyclic-superframe structure with the value of Sequence Number field and of Cyclic-superframe Duration field, and starts to schedule to access a period of a superframe of a cyclic-superframe.

      - MLME-PEERING.response

    - De-peering primitives

    - Frame error notification primitives

    - Reset MAC sublayer

    - Receiver enable

    - Channel scanning

    - Group configuration

These primitives are used to initiate a PAC Group or to begin using a new cyclic-superframe configuration.

      - MLME-GROUP-START.request

This primitive is also used by a PD already peered with an existing PAC Group to begin using a new cyclic-superframe configuration. The properties of this primitive are:

MLME-GROUP-START.request{

GroupID;

InitiatorID;

ChannelNumber;

ChannelPage;

StartTime;

;

CyclicSuperframeStructure;

}

The primitive parameters are defined in Table 10.

                - Table 10—MLME-GROUP-START.request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Initiator_ID | MAC address | PD specific | The address of the initiator PD |
| MulticastGroup_ID | Integer | 0 to 216 −1 | Group ID of initiator PD. |
| ChannelNumber | Integer | Any valid channel number a defined in Error: Reference source not found | The channel number to use. |
| ChannelPage | Integer | Any valid channel page as defined in Error: Reference source not found | The channel page to use. |
| StartTime | Unsigned integer | 0 to (224 −1) | The time at which to begin transmitting the synchronization period. If this parameter is equal to 0, the synchronization period will begin immediately. Otherwise, the specified time is relative to the received synchronization period of the current initiator PD. |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 6 | Indicates the structure of cyclic-superframe |

When receiving the MLME-GROUP-START.request primitive with the CyclicSuperfreamStructure parameter, the MAC sublayer sets the PIB with the value of the cyclic-superframe structure descriptor and sets the start time of a cyclic-superframe with the StartTime parameter. After starting a cyclic-superframe, the MAC sublayer inserts the Cyclic-superframe descriptor IE to the MAC header of the Data frame. The MAC sublayer schedules to access a period of a superframe of a cyclic-superframe.

      - MLME-GROUP-START.confirm

      - MLME-GROUP-START.indication

The primitive is used to indicate the reception of a new cyclic-superframe configuration. The properties of this primitive are:

MLME-GROUP-START.indication{

PD_ID;

PHYcapability;

CyclicSuperframeStructure;

}

The primitive parameters are defined in Table 11.

Table 11—MLME-GROUP-START.indication parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| PD_ID | MAC address | PD specific | Address of the PD requesting GroupStart. |
| PHYcapability | Enumeration | LOW_MOBILITY, HIGH_MOBILITY, GFSK, UWB_BPM_BPSK, UWB_OOK | Operational capability of the PD requesting GroupStart. |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 6 | Indicates the structure of cyclic-superframe |

When receiving the Data frame with the Cyclic-superframe descriptor IE, the MAC sublayer sets the PIB with the value of Cyclic-superframe Duration field, Primary superframe Number field, Primary superframe Type field, and Secondary superframe Type field. The MAC sublayer notifies the receiving the cyclic-superframe structure to the higher layer with MLME-GROUP-START.indication primitive. The MAC sublayer calculates the next start of a cyclic-superframe structure with the value of Sequence Number field and of Cyclic-superframe Duration field, and starts to schedule to access a period of a superframe of a cyclic-superframe.

      - MLME-GROUP-START.response

    - Synchronization

These primitives are used to synchronize with other PDs and to communicate loss of synchronization to the next higher layer.

      - MLME-SYNCHRONIZATIOHN.request

The primitive requests to synchronize with other PDs by acquiring and tracking their synchronization signal. The properties of this primitive are:

MLME-SYNCHRONIZATION.request{

ChannelNumber;

ChannelPage;

TrackMode;

}

The primitive parameters are defined in Table 12.

Table 12—MLME-SYNCHRONIZATION.request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| ChannelNumber | Integer | As defined in Error: Reference source not found | The channel number on which to attempt synchronization. |
| ChannelPage | Integer | As defined in Error: Reference source not found | The channel page on which to attempt synchronization. |
| TrackMode | Boolean | TRUE, FALSE | TRUE if the PD is to synchronize from the next synchronization period. FALSE if the PD is to synchronize with only the next synchronization period. |
|  |  | Table 6 |  |

When the primitive is received by the MLME, it will set phyCurrentPage and phyCurrentChannel equal to the values of the ChannelPage and ChannelNumber, respectively. If the TrackMode parameter is equal to TRUE, the MLME will track the synchronization period, i.e., enable its receiver just before the expected time of synchronization period. If the TrackMode parameter is equal to FALSE, the MLME will locate the synchronization period, but not continue to track it.

If this primitive is received by the MLME while it is currently tracking the synchronization period, the MLME will not discard the primitive but will treat it as a new synchronization request.

      - MLME-SYNCHRONIZATION.confirm

      - MLME-SYNCHRONIZATION-LOSS.indication

    - Channel sounding

    - Dynamic preamble selection

    - Ranging calibration

  - MAC data service

The MCPS-SAP supports the transport of data information. Table 13 lists the primitives maintained by the MCPS-SAP.

Table 13—MCPS-SAP primitives

| Name | Request | Confirm | Indication | Response |
| --- | --- | --- | --- | --- |
| MCPS.Data | − |  |  | − |

    - MCPS-DATA.request

The primitive requests the transfer of data to another PD. The properties of this primitive are:

MCPS-DATA.request {

phyModeSelection;

SourceAddress;

DestinationAddress;

MulticastGroupId;

EtherType;

MsduLength;

Msdu;

TransmitTimeOut;

DataType;

DataRate;

PurgeMode;

AckTX;

Ranging;

IeList;

RequestRriTx;

CyclicSuperframeStructure;

}

The primitive parameters are defined in Table 14.

                - Table 14—MCPS-DATA.request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| SourceAddress | MAC address | PD specific | The address of the PD from which the frame originated. |
| phyModeSelection | Enumeration | LOW-MOBILITY, HIGH_MOBILITY, GFSK, UWB_BPM_BPSK, UWB_OOK | PHY selection |
| DestinationAddress | MAC address | PD specific | Address of the PD for which the frame is intended. |
| MulticastGroupId | Integer | 0 to 216 −1 | Group ID. |
| EtherType | Unsigned | 0 to 216-1 | The EtherType value to use in the MAC data frame. |
| MsduLength | Integer | TBD | MDSU length in octets. |
| Msdu | array of octets | - | The octets to send in the Data Payload field of the MAC data frame. |
| TransmitTimeOut | Integer | TBD | The maximum allowed delay in microseconds from when the data is presented to the SAP until the frame has finished transmission and the acknowledgment, if required, is received. |
| DataType | Enumeration | AUDIO, VIDEO, DATA | Indicates the type of data that is sent in the stream. |
| DataRate | Integer | TBD | Indicates the data rate |
| PurgeMode | Boolean | TRUE, FALSE | If TRUE, the MSDU in the transaction buffer corresponding to MSDUhandle, is discarded from the transaction buffer. |
| AckTX | Boolean | FALSE, TRUE | This parameter is present only when phyModeSelection is UWB. TRUE if acknowledged transmission is used, FALSE otherwise. |
| Ranging | Boolean | FALSE, TRUE | This parameter is present only when phyModeSelection is UWB. TRUE if ranging bit in PHR is to be set, FALSE otherwise. |
| IeList | Set if IEs as described in Error: Reference source not found | As described in Error: Reference source not found | This parameter is present only when phyModeSelection is UWB. Determines/supplies the IEs to be sent, including: RRRT IE, RRTD IE, RPRT IE, RRTM IE, RCDT IE and RTOF IE |
| RequestRrtiTx | Boolean | TRUE, FALSE | This parameter is present only when phyModeSelection is UWB. This parameter requests that the PD inserts an RRTI IE in the sent data frame. If the MCPS-DATA.request is early enough the data frame shall be sent at the configured macRngRrtiTime otherwise shall be sent as soon as possible thereafter. |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 6 | Indicates the structure of cyclic-superframe |

When receiving the MCPS-DATA.request primitive with the CyclicSuperfreamStructure parameter, the MAC sublayer checks whether the cyclic-superframe structure descriptor is equal to the PIB or not. If not, the MAC sublayer sets the PIB with the value of the cyclic-superframe structure descriptor and selects the start time of a cyclic-superframe. After starting a cyclic-superframe, the MAC sublayer inserts the Cyclic-superframe descriptor IE to the MAC header of the Data frame. The MAC sublayer schedules to access a period of a superframe of a cyclic-superframe. If yes, the MAC sublayer inserts the Cyclic-superframe descriptor IE to the MAC header of the Data frame.

    - MCPS-DATA.confirm

    - MCPS-DATA.indication

The primitive indicates the reception of data from another PD. The properties of this primitive are:

MCPS-DATA.indication{

phyModeSelection;

SourceAddress;

DestinationAddress;

MulticastGroupId;

EtherType;

MsduLength;

Msdu;

CQI;

DataSequenceNumber;

DataRate;

TxRangingCounter;

RxRangingCounter;

IeList;

AoaAzimuth;

AoaElevation;

AoaPresent;

Rssi;

CyclicSuperframeStructure;

}

The primitive parameters are defined in Table 15.

                - Table 15—MCPS-DATA.indication parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| SourceAddress | MAC address | PD specific | The address of the PD from which the frame originated. |
| DestinationAddress | MAC address | PD specific | Address of the PD for which the frame is intended. |
| MulticastGroupId | Integer |  | Group ID. |
| EtherType | Unsigned | 0 to 216-1 | The EtherType value from the received data frame. |
| MsduLength | Integer | TBD | MDSU length in octets. |
| Msdu | array of octets | - | The octets from the Data payload field of the received data frame. |
| CQI | Integer | 0 – 4 | ##### CQI value measured during reception of the PSDU. TBD |
| DataSequenceNumber | Integer | TBD | Data number sequence of the received data frame. |
| DataRate | Integer | TBD | Indicates the data rate. |
| RxRangingCounter | Unsigned Integer | 0 to (232 −1) | This parameter is present only when phyModeSelection is UWB. The timestamp of the received data frame |
| TxRangingCounter | Unsigned Integer | to (232 −1) | This parameter is present only when phyModeSelection is UWB. If the received frame had the AR bit set, then this is the timestamp of the transmitted acknowledgement frame, otherwise this parameter is invalid. |
| IeList | Set if IEs as described in Error: Reference source not found | As described in tError: Reference source not found | This parameter is present only when phyModeSelection is UWB. Reports the IEs received including: RRRT IE, RRTD IE, RRTI IE, RPRT IE, RRTM IE, RCDT IE and RTOF IE |
| AoaAzimuth | Float | -π to +π | This parameter is present only when phyModeSelection is UWB. AOA of the received signal in azimuth measured in radians, relative to some PD specific axis defined by its antenna arrangement or orientation. This parameter is valid only when the AoaPresent parameter is either AZIMUTH or BOTH. |
| AoaElevation | Float | -π to +π | This parameter is present only when phyModeSelection is UWB. AOA of the received signal in elevation measured in radians. This parameter is valid only when the AoaPresent parameter is either ELEVATION or BOTH. |
| AoaPresent | Enumeration | NONE, BOTH, AZIMUTH, ELEVATION | This parameter is present only when phyModeSelection is UWB. Indicates validity of AoaAzimuth and AoaElevation parameter. Where AOA is not supported this parameter value shall be NONE. |
| Rssi | Integer | 0 to 255 | This parameter is present only when phyModeSelection is UWB. The received signal strength for received frame. This is a measure of the RF power level at the antenna based on the gain setting in the RX chain and the measured signal level in the channel. For the UWB PHY the RSSI value is measured during the frame Preamble and locked when a valid SFD is detected. A value of zero indicates that RSSI measurement is not supported or was not measured for this frame. |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 6 | Indicates the structure of cyclic-superframe |

When receiving the Data frame with the Cyclic-superframe descriptor IE, the MAC sublayer checks whether the cyclic-superframe structure descriptor is equal to the PIB or not. If yes, the MAC sublayer ignores the Cyclic-superframe descriptor IE. If not, the MAC sublayer sets the PIB with the value of Cyclic-superframe Duration field, Primary superframe Number field, Primary superframe Type field, and Secondary superframe Type field. The MAC sublayer notifies the receiving the cyclic-superframe structure to the higher layer with MCPS-DATA.indication primitive. The MAC sublayer calculates the next start of a cyclic-superframe structure with the value of Sequence Number field and of Cyclic-superframe Duration field, and starts to schedule to access a period of a superframe of a cyclic-superframe.

  - PAC information base (PIB)

The PIB comprises the fields required to manage the MAC sublayer and PHY layer of a PD, and presented in Table 16.

The fields associate to the PHY layer and MAC sublayer use the prefix phy and mac, respectively. Constants are READ_ONLY.

Table 16—PIB fields

| Field | Type | Range | Default | Description |
| --- | --- | --- | --- | --- |
| PD_ID | IEEE EUI-48 | READ_ONLY | Implementation specific | PD MAC address |
| phyMaxPacketSize | Integer | READ_ONLY | TBD | Maximum PSDU size in octets |
| phyCurrentChannel | Integer | As defined in table Error: Reference source not found | Implementation specific | The RF channel number used for Tx and Rx |
| phyCurrentPage | Integer | As defined in table Error: Reference source not found | Implementation specific | The RF channel page used for Tx and Rx |
| phyTurnaroundTime | Integer | READ_ONLY | TBD | RX-to-TX or TX-to-RX turnaround time (in symbol periods), as defined in |
| phyTXPower | Integer | TBD | Implementation specific | The transmit power of the PD in dBm |
| phyCCAMode | Integer | TBD | Implementation specific | The CCA mode as defined in |
| phySHRduration | Integer | TBD | TBD | The duration of the synchronization period in symbols |
| phyDiscoveryDuration | Integer | TBD | TBD | The duration of the discovery period in symbols |
| phyPeeringDuration | Integer | TBD | TBD | The duration of the peering period in symbols |
| phySymbolsperOctect | Float | TBD | Implementation specific | Number of symbols per octet |
| phyModeSelection | Enumeration | LOW_MOBILITY, HIGH_MOBILITY, GFSK, UWB_BPMBPSK, UWB_OOK. | Implementation specific | PHY selection |
| phyUWBPreambleCode | Unsigned Integer | For UWB PHY: 1 to 48 for the BPM-BPSK modulation mode; and, 1 to 33 for the OOK modulation mode. | Implementation specific | For the UWB PHY the value here shall select the preamble code to employ. The value selects the code as per the “Preamble Sequence ID” column of Error: Reference source not found for the BPM-BPSK modulation mode, and as per parameter i of Error: Reference source not found for the OOK modulation mode, (actually the phyPreambleCode value specified here is i+1). For other PHY the value here is ignored. |
| phyUWBTxRmarkerOffset | Unsigned Integer | 0 to (232 −1) | Implementation specific | The propagation time from the internal transmit timestamp to the transmit antenna. The units of this are defined in Error: Reference source not found. The PD adds this offset to its internal transmit timestamp to get the reported TxRangingCounter value. |
| phyUWBRxRmarkerOffset | Unsigned Integer | 0 to (232 −1) | Implementation specific | The propagation time from the receive antenna to the internal receiver timestamp. The units of this are defined in Error: Reference source not found. The PD subtracts this offset from its internal receive timestamp to get the reported RxRangingCounter value. |
| macUWBRngAckRrtiSupport | Boolean | READ_ONLY | TBD | Read only value indicating whether the PD is able to generate an RRTI IE within the standard ACK response time and insert it into a ranging ACK frame. |
| macUWBRngAckRrtiEnable | Boolean | TRUE, FALSE | TBD | When set the PD MAC shall automatically generate and insert an RRTI IE in a ranging ACK frame, where the RFRAME being acknowledged carried an RRRT IE and where macRngAckRrtiSupport indicates that the PD is capable of doing this. |
| macUWBRngMinRrtiTime | Unsigned Integer | READ ONLY | TBD | This is a read only value reporting the minimum RX to TX response time that the PD supports for inserting an RRTI IE into a MAC data frame. The units of this are defined in Error: Reference source not found. A value of zero shall indicate that the PD cannot support RRTI insertion, i.e. so the upper layers know to employ an RRTD IE in the ranging exchanges. |
| macUWBRngRrtiTime | Unsigned Integer | TBD | TBD | This sets the desired RX-to-TX response time for the PD to use when inserting an RRTI IE into a MAC data frame. The units of this are defined in Error: Reference source not found. The value set should take account of the macRngMinRrtiTime and any upper layer delays in initiating transmission, be reflected in the preferred response time indicated via RPRT IE exchanges. |
| macGroupIdList | Array of integers | 0 – 255 per element in the array | Implementation specific | List if Group IDs |
| macPDIdList | Array of MAC addresses | PD specific | Implementation specific | List of PD IDs per Group ID |
| macAddress | IEEE EUI-48 | READ_ONLY | Implementation specific | PD MAC address |
| macPeeringPermit | Boolean | TRUE, FALSE | FALSE | Indication of whether a PD is currently allowing peering. A value of TRUE indicates that peering is permitted. |
| macSyncOrder | Integer | TBD | TBD | Indicates the frequency with which the sync period is transmitted, as defined in TBD |
| macDSN | Integer | 0 ─ 255 | ─ | The sequence number added to the transmitted Data frame or MAC command. |
| macREpermit | Boolean | TRUE, FALSE | TRUE | TRUE if the PD accepts RE requests. FALSE otherwise. |
| macMaxBE | Integer | TBD | TBD | The maximum value of the back-off exponent, BE, in the CSMA-CA algorithm, as defined in TBD |
| macMinBE | Integer | 0 ─ macMaxBE | TBD | The minimum value of the back-off exponent, BE, in the CSMA-CA algorithm, as defined in TBD |
| macMaxBackoffs | Integer | TBD | TBD | The maximum number of backoffs the CSMA-CA algorithm will attempt before declaring a channel access failure. |
| macMaxFrameRetries | Integer | TBD | TBD | The maximum number of retries allowed after a transmission failure. |
| macLIFSperiod | Integer | TBD | Implementation dependent | The minimum time forming a LIFS period. |
| macSIFSperiod | Integer | TBD | Implementation dependent | The minimum time forming a SIFS period. |
| macPromiscuousMode | Boolean | TRUE, FALSE | FALSE | A TRUE value indicates that the MAC sublayer accepts all frames received from the PHY. FALSE otherwise. |
| macRangingSupport | Boolean | TRUE, FALSE | FALSE | This indicates whether the MAC sublayer supports ranging features. |
| macResponseWaitTime | Integer | TBD | TBD | The maximum time, in multiples of aBaseSuperframeDuration, a PD shall wait for a response command to be available following a request command. |
| macRxOnWhenIdle | Boolean | TRUE, FALSE | FALSE | Indication of whether the MAC sublayer enables its receiver during idle periods. |
| macSecurityEnabled | Boolean | TRUE, FALSE | FALSE | A value of TRUE indicates that security is enabled in the MAC sublayer. FALSE otherwise. |
| macSuperFrameOrder | Integer | TBD | TBD | The length of the active portion of the outgoing superframe as defined in TBD. |
| macPriorityChannelAccess | Boolean | TRUE, FALSE | FALSE | A value of TRUE indicates that PCA is enabled. FALSE otherwise. |
| macCyclicSuperframeStructure | List of Cyclic-superframe Structure Descriptor | Set as defined in Table 6 | Default Cyclic-superframe Structure Descriptor | The list of the cyclic-superframe structure of PAC groups operating within communication range |

(Normative)

QC-LDPC prototype matrices

(Normative)

Modulation mapping

# (informative)

# (informative)

## The mathematics of two-way ranging

# (informative)

## Design procedure of a primitive (n,k,t) BCH code

# (informative)

## Use of a cyclic-superframe

The IEEE 802.15.8 capable device will contain the IEEE 802.15.8 PHY and MAC sublayer, the PAC adaptation layer, and application services. The PAC adaptation layer performs as a higher layer of the IEEE 802.15.8 MAC sublayer and provides the PAC management service to application services.

The PAC adaptation layer interprets an application service request, selects a cyclic-superframe structure, and calls an appropriate PAC primitive with CyclicSuperframeStrcture parameter. The PAC adaptation layer receives a discovery request and a peering request from other PAC capable device, and filters off a request or adapts request message to notify the application service.

A sequential flow for supporting an application service in PAC adaptation layer is as follows:

- when an application is installed on a PAC capable device, PAC adaptation layer provides an interface for selecting the use of IEEE 802.15.8 as for communication and the discovery mode, and an interface for setting a peer or a peer group information

- PAC adaptation layer lets application service select a required communication quality by suggesting several typical profiles of peer aware communication or expects a communication situation with information gathered from the application service, such as required bandwidth, tolerable delay, and etc.

- PAC adaptation layer selects the type of primary superframe and the type of secondary superframe, and calculates the duration of cyclic-superframe and the number of primary superframe, as described in 5.1.1.2 according to which communication phase is in

- when receiving CyclicSuperframeStrcture parameter from the PAC adaptation layer, IEEE 802.15.8 MAC sublayer selects the time to start discovery phase or peering phase, or data transmission phase considering the cyclic-superframe structure of neighbor PAC groups stored in the PIB

- PAC adaptation layer schedules the procedure for synchronization, discovery of peers, and peering to peers, and after completing procedures notifies the application service that communication is ready

- PAC adaptation layer advertises the cyclic-superframe structure of a PAC group periodically by inserting CyclicSuperframeStrcture parameter to MCPS-DATA.request primitive

- IEEE 802.15.8 MAC sublayer scans access periods of superframes of a cyclic-superframe and collects the cyclic-superframe structure of PAC groups operating within communication range