IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3e Comment Resolution for i-17 of Sponsor Ballot |  |
| Date Submitted | 14, Sep. 2016 |  |
| Source | [ Kondou, Keitarou; Akiyama, Keiji] [Sony Semiconductor Solutions Corp.] [1-7-1 Konan, Minato-ku, Tokyo Japan] | Voice: [ +81-3-6748-4001] Fax: [ +81-3-6748-4268] E-mail: [Keitarou.Kondo@sony.com , Keiji.Akiyama@sony.com ] |
| Re: | In response to i-17 from consolidated sponsor ballot comments (15-16-0585-00-003e) |  |
| Abstract | Frequency Band expansion prepared for future use |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

List of contributors

| Name | Affiliation |
| --- | --- |
| Keitarou Kondou | Sony Semiconductor Solutions Corp. |
| Keiji Akiyama | Sony Semiconductor Solutions Corp. |
| Ken Hiraga | NTT Corporation |

Comment

| Comment # | Page | Subclause | Line | Comment | Must Be Satisfied | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| i-17 |  | 11a.1.4 |  | With the possibility of future channel expansion, what needs to be done now? | Yes | Revisit channel assignment table to ensure it can be extended when more channels become available. |

Resolution:

To support it, modify frequency table in 11a.1.4 and update and modify the IE for Capability and simplify it.

Update Strategy:

[802.15.3e Frequency Expansion]

- Add CHNL_ID 5-16 referring the style of proposal for 802.11ay.

- Channel assignments are as follows;

Channel BW

Frequency

(

GHz

)

2

.

16

GHz

4

.

32

GHz

6

.

48

GHz

8

.

64

GHz

#

1

#

2

#

3

#

4

#

5

#

6

#

7

#

9

#

11

#

8

#

10

#

12

#

13

#

14

#

15

#

16

Channel BW

For Single channel

For Bonding or

Frequency Aggregation

By using “SC Channel Aggregation field”,

it can be used as single channel aggregation

Aggregation Case 1: Pattern 0-1: Aggregation with single channel

Channel BW

Frequency

(

GHz

)

2

.

16

GHz

Channel BW

#

2

#

4

#

2

#

4

#

6

pattern0

pattern1

Aggregation Rule 1:

All Aggregation pattern shall include CHNL_ID 2.

Aggregation Rule 2:

For Single channel aggregation,  1 empty channel shall include

between channels.

(consecutive single channel aggregation are realized by Bonding

channel and “SC Channel Aggregation field”)

Aggregation Case 2: 2Ch or 3ch Bonding + Aggregation

Channel BW

Frequency

(

GHz

)

4

.

32

GHz

#

7

#

9

#

8

#

10

Channel BW

#

7

#

10

#

11

#

8

#

12

#

14

pattern2

pattern3

pattern4

pattern5

pattern6

Insert or modify the text as follows:

11a.1.4 RF channelization

Replace Table 11a-1 as follows;

Table 11a-1—HRCP PHY channelization

| CHNL_ID | Start frequency | Center frequency | Stop frequency |
| --- | --- | --- | --- |
| 1 | 57.240 GHz | 58.320 GHz | 59.400 GHz |
| 2 | 59.400 GHz | 60.480 GHz | 61.560 GHz |
| 3 | 61.560 GHz | 62.640 GHz | 63.720 GHz |
| 4 | 63.720 GHz | 64.800 GHz | 65.880 GHz |
| 5 | 65.880 GHz | 66.960 GHz | 68.040 GHz |
| 6 | 68.040 GHz | 69.120 GHz | 70.200 GHz |
| 7 | 57.240 GHz | 59.400 GHz | 61.560 GHz |
| 8 | 59.400 GHz | 61.560 GHz | 63.720 GHz |
| 9 | 61.560 GHz | 63.720 GHz | 65.880 GHz |
| 10 | 63.720 GHz | 65.880 GHz | 68.040 GHz |
| 11 | 65.880 GHz | 68.040 GHz | 70.200 GHz |
| 12 | 57.240 GHz | 60.480 GHz | 63.720 GHz |
| 13 | 59.400 GHz | 62.640 GHz | 65.880 GHz |
| 14 | 63.720 GHz | 66.960 GHz | 70.200 GHz |
| 15 | 57.240 GHz | 61.560 GHz | 65.880 GHz |
| 16 | 59.400 GHz | 63.720 GHz | 68.040 GHz |

Note: CHNL_ID 9,10,11,14 are only used for channel aggregation.

6.4.11a HRCP PNC Capability IE

Remove the following sentence;

The SC Supported Channel Bonding field indicates the number of bonded channels supported by the SCPHY DEV. The Supported OOK Channel bonding field shall be formatted as illustrated in Figure 6-88f.

Insert the following sentence;

The SC Channel Aggregation field indicates the combined channels described as CHNL_ID 7 to 16 in 11a.1.4 are used as aggregation of each 2.16GHz physical frequency channels or frequency bonding channels for whole frequency channel. The SC Channel Aggregation field shall be set zero for bonding, shall be set one for aggregation.

Remove Figure 6-88f;

Replace Figure 6-88g;

| Bits: b0 | b1 | b2 | b3 | b4 | b5 | b6 |
| --- | --- | --- | --- | --- | --- | --- |
| pattern0 | pattern1 | pattern2 | pattern3 | pattern4 | pattern5 | pattern6 |

Figure 6-88g—SC Supported Channel Aggregation pattern field format

Replace Table 6-17f

Table 6-17f—Channel Aggregation Patterns for SC PHY in Figure 6-88g

| CHNL_ID | 2 | 4 | 6 | 7 | 8 | 10 | 11 | 12 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pattern0 |  |  |  |  |  |  |  |  |  |
| pattern1 |  |  |  |  |  |  |  |  |  |
| pattern2 |  |  |  |  |  |  |  |  |  |
| pattern3 |  |  |  |  |  |  |  |  |  |
| pattern4 |  |  |  |  |  |  |  |  |  |
| pattern5 |  |  |  |  |  |  |  |  |  |
| pattern6 |  |  |  |  |  |  |  |  |  |