IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3e Comment Resolution for i-128 of Sponsor Ballot |  |
| Date Submitted | 13, Sep. 2016 |  |
| Source | [ Kondou, Keitarou; Akiyama, Keiji] [Sony Semiconductor Solutions Corp.] [1-7-1 Konan, Minato-ku, Tokyo Japan] | Voice: [ +81-3-6748-4001] Fax: [ +81-3-6748-4268] E-mail: [Keitarou.Kondo@sony.com , Keiji.Akiyama@sony.com ] |
| Re: | In response to i-128 from consolidated sponsor ballot comments (15-16-0585-00-003e) |  |
| Abstract | Modify MLME-START primitive parameters |  |
| Purpose | To realize Figure 4-3 case, MLME-START primitive parameter should be added. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Comment

| Comment # | Page | Subclause | Line | Comment | Must Be Satisfied | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| i-128 |  | 5.3.3 |  | It is possible to start a pairnet with both beacons. If so, there should be an option here. | Yes | Add "BOTH_HRCP_PHYS" or similar term.. |

Resolution:

Add “HRCP_BOTH_PHY” in “Valid range” of “PhyMode” parameter of MLME-START primitive

Add the text from following page.

Change the “PhyMode” line  of Table 5.8  as indicated

Table 5-8—MLME-START primitive parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PhyMode | Enumeration | 2.4_GHZ, SC_MMWAVE, HSI_MMWAVE, AV_MMWAVE, HRCP SC PHY,HRCP_OOK_PHY, HRCP_BOTH_PHY | The PHY that will be used for the beacons and the CP(s) in the piconet or pairnet that will be started. |