IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3e Comment Resolution for i-129 of Sponsor Ballot |  |
| Date Submitted | 13, Sep. 2016 |  |
| Source | [ Kondou, Keitarou; Akiyama, Keiji] [Sony Semiconductor Solutions Corp.] [1-7-1 Konan, Minato-ku, Tokyo Japan] | Voice: [ +81-3-6748-4001] Fax: [ +81-3-6748-4268] E-mail: [Keitarou.Kondo@sony.com , Keiji.Akiyama@sony.com ] |
| Re: | In response to i-129 from consolidated sponsor ballot comments (15-16-0585-00-003e) |  |
| Abstract | Change Resultcode of MLME-STOP |  |
| Purpose | . |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Comment

| Comment # | Page | Subclause | Line | Comment | Must Be Satisfied | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| i-129 | 15 | 5.3.4 | 34 | It is not possible for the higher layer to deny a shutdown request. The device receives the request from its higher layer and begins to shutdown. The other DEV(s) in a piconet or pairnet are not allowed to stop the shutdown (and indeed there is no message to do this). It makes no sense for one DEV to tell another that it can't shut down (for example, if their power runs out). | Yes | Delete "HIGHER_LAYER_DENIED" It is correct to add this to associate, but not shutdown. |

Resolution:

Stopping pairnet is due to higher layer request.

DEV can send dissasociate message.

To avoid confusion, change "Higher Layer Denied" reason-code to "Higher Layer Requested".

Change the text from following page.

Replace the text of the last line of Table 5-9 as indicated:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ReasonCode | Enumeration | NOT_A_PNC, HANDOVER_FAILED, HIGHER_LAYER_DENIEDHIGHER LAYER_REQUESTED, OTHER | Indicates the reason for a ResultCode. NOT_A_PNC and HANDOVER_FAILED are not a valid response for HRCP DEVs. |

Change the “PhyMode” line  of Table 5.8  as indicated

Table 5-8—MLME-START primitive parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PhyMode | Enumeration | 2.4_GHZ, SC_MMWAVE, HSI_MMWAVE, AV_MMWAVE, HRCP SC PHY,HRCP_OOK_PHY, HRCP_BOTH_PHY | The PHY that will be used for the beacons and the CP(s) in the piconet or pairnet that will be started. |