IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3e Comment Resolutions for i-204, 205 of Sponsor Ballot |  |
| Date Submitted | 21, Sep. 2016 |  |
| Source | [ Kondou, Keitarou; Akiyama, Keiji] [Sony Semiconductor Solutions Corp.] [1-7-1 Konan, Minato-ku, Tokyo Japan] | Voice: [ +81-3-6748-4001] Fax: [ +81-3-6748-4268] E-mail: [Keitarou.Kondo@sony.com , Keiji.Akiyama@sony.com ] |
| Re: | In response to i-204, 205, 206 from consolidated sponsor ballot comments (15-16-0585-00-003e) |  |
| Abstract | Add HRCP Aggregated Multi-protocol Data format |  |
| Purpose | To meet CSD requirement, data formats compatible to 802.1 are prepared. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Comment

| Comment # | Page | Subclause | Line | Comment | Must Be Satisfied | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| i-204 |  | 5.5.7 |  | The MAC-HRCP-DATA primitives do not pass addressing (or Ethertypes) explicitly. In order to meet the CSD requirements, compatibility with 802.1 requires 48 bit addressing and Ethertypes. | Yes | Add the MAC address of the source and destination to these primitives. Otherwise, you will need another way to notify the next higher layer of the addressing (you could specify this with the associate and disassociate primitives, but it may be easier here). Also, specify how the addressing is carried from end to end. Finally, you need to specify how the Ethertype is carried in this protocol. |
| i-205 |  | 6.3.4a.1 |  | The data frame format proposed violates the CSD for this project, which requires adherence to 802.1. To do that, the Ethertype and associated tags need to be carried in the data frame. In addition, this is just good protocol design. |  | Indicate that the Payload field for the MAC Subframe Body field is formatted as the Non-secure Multi-protocol Data frame Frame Payload field. |
| i-206 |  | 6.3.4a.2 |  | The formats of the Payload fields and MAC subheaders are not listed. | Yes | Add "The Payload field is formatted as the Non-secure Multi-protocol Data frame Frame Payload field in 6.3.5.1." and "The HRCP MAC Subheader field is defined in 6.3.4a.1" |

Resolution:

Add “MAC-HRCP-MUL-DATA” to MAC SAP primitives.

Add “non-secure Aggregated HRCP Multi-protocol Data Frame” and “secure Aggregated HRCP Multi-protocol Data Frame”.

5.5 MAC SAP

Change Table 5-30 as follows:

Table 5-30—Summary of MAC SAP primitives

| Name | Request | Confirm | Indication | Response |
| --- | --- | --- | --- | --- |
| MAC-ASYNC-DATA | 5.5.1 | 5.5.2 | 5.5.3 |  |
| MAC-ISOCH-DATA | 5.5.4 | 5.5.5 | 5.5.6 |  |
| MAC-HRCP-DATA | 5.5.7 | 5.5.8 | 5.5.9 |  |
| MAC-HRCP-MUL-DATA | 5.5.10 | 5.5.11 | 5.5.12 |  |

Insert the following row at the end of Table 5-31:

Table 5-31—MAC-ISOCH-DATA, MAC-ASYNC-DATA and MAC-HRCP-DATA primitive parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| LogicalChannel | Enumeration | CH0,CH1 | LogicalChannel value is available for use by the Higher Layer Protocol user and is therefore out of scope of this specification. This parameter is valid only for MAC-HRCP-DATA primitives. |
| DataID | Enumeration | ETHERTYPE_PROTOCOL_DISCRIMINATION,OUI_CID,ETHERTYPE | The DataID specifies Data header type |
| DestinationAddress | MAC Address | Any valid individual MAC address | The destination address representing the address of data destination when the DataID is “ETHERTYPE_PROTOCOL_DISCRIMINATOIN” |
| SourceAddress | MAC Address | Any valid individual MAC address | The source address representing the address of data origin when the DataID is “ETHERTYPE_PROTOCOL_DISCRIMINATOIN” |
| Ethertype | Octet string | Ethertype defined in IEEE Std 802-2014 | The Ethertype specified when DataID is “ETHERTYPE_PROTOCOL_DISCRIMINATOIN” or “ETHERTYPE” |
| OUI_CID | Octet string |  | The OUI or CID strings specified when DataID is “OUI_CID” |

After 5.5.9,insert the following new subclause as 5.5,10,5.5.11 and 5.5.12:

5.5.10 MAC-HRCP-MUL-DATA.request

This primitive is used to initiate the transfer of a Multi-protocol MSDU from one MAC entity to another MAC entity or entities using HRCP PHY. The semantics of this primitive are:

MAC-HRCP-MUL-DATA.request	` (

RequestID,

StreamIndex,

LogicalChannel,

ACKRequested,

ConfirmRequested,

Length,

DataID,

DestinationAddress

SourceAddress,

Ethertype,

OUI_CID

Data

)

The primitive parameters are defined in Table 5-31.

5.5.11 MAC-HRCP-MUL-DATA.confirm

This primitive is used to report the result of a request to transfer a Multi-protocol MSDU from one MAC entity to another MAC entity or entities. This primitive is only generated if the ConfirmRequested parameter in the MAC-HRCP-MUL- DATA.request with the same RequestID value is ALWAYS or is ON_ERROR and the ResultCode is FAILURE. The semantics of this primitive are:

MAC-HRCP-MUL-DATA.confirm		(

RequestID,

TransmitDelay,

ResultCode,

ReasonCode

)

The primitive parameters are defined in Table 5-31.

5.5.12 MAC-HRCP-MUL-DATA.indication

This primitive is used to indicate the reception of a Multi-protocol MSDU. The semantics of this primitive are:

MAC-HRCP-MUL-DATA.indication 	(

Length,

DataID,

DestinationAddress

SourceAddress,

Ethertype,

OUI_CID,

LogicalChannel,

Data

)

The primitive parameters are defined in Table 5-31.

6.2 General frame format

Replace Figure 6-3a as follows:

| Octet:2 | 2 | 1 | 1 | 3 | 1 |  | Variable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Pairnet ID | DestID | SrcID | TX and ACK Information | Stream Index |  | Frame Payload |
| MAC header |  |  |  |  |  |  | MAC Frame body |

Table 6-3a—MAC Header field and MAC Frame Body field format for pairnet

6.2.1.2 Frame Type field

Replace Table 6-1a as follows:

Table 6-1a—Valid frame type values for pairnet

| Table value b5 b4 b3 | Frame type description | Subclause |
| --- | --- | --- |
| 000 | Beacon frame | 6.3.1 |
| 001 | Reserved |  |
| 010 | Reserved |  |
| 011 | Command frame | 6.3.3a |
| 100 | Data frame | 6.3.4a |
| 101 | Multi-protocol Data frame | 6.3.5a |
| 110-111 | Reserved |  |

Insert text before 6.2.6 as follows:

6.2.5 Stream Index field

For pairnet, the Stream Index field assigned values are as follows:

— 0x00 is assigned for data

— 0x01 to 0xFF is reserved

6.3.1.1a Non-secure Beacon frame for pairnet

Insert a line at the end of Table 6-6a as follows:

Table 6-6a—MAC header settings for a beacon frame for HRCP

| Header field | Setting on transmission | Interpretation on reception |
| --- | --- | --- |
| Frame type | Beacon value in Table 6-1a | Decoded |
| SEC | 0 | Decoded |
| ACK policy | No-ACK value in Table 6-2a | May be ignored |
| DestID | BcstID | Decoded |
| SrcID | HRCP PNCID | Decoded |
| TX and ACK Information | 0x000000 | May be ignored |
| Stream Index | 0x00 | May be ignored |

6.3.1.2a Non-secure Beacon frame for pairnet

Insert a line at the end of Table 6-7a as follows:

Table 6-7a—MAC header settings for a Secure Beacon frame for HRCP

| Header field | Setting on transmission | Interpretation on reception |
| --- | --- | --- |
| Frame type | Beacon value in Table 6-1a | Decoded |
| SEC | 1 | Decoded |
| ACK policy | No-ACK value in Table 6-2a | May be ignored |
| DestID | BcstID | Decoded |
| SrcID | HRCP PNCID | Decoded |
| TX and ACK Information | 0x000000 | May be ignored |
| Stream Index | 0x00 | May be ignored |

6.3.3a.1 Non-secure Command frame

Insert a line at the end of Table 6-10a as follows:

Table 6-10a—MAC header settings for a non-secure command frame for pairnet

| Header field | Setting on transmission | Interpretation on reception |
| --- | --- | --- |
| Frame type | Command value in Table 6-1a | Decoded |
| SEC | 0 | Decoded |
| ACK policy | As appropriate | May be ignored |
| DestID | As appropriate | Decoded |
| SrcID | As appropriate | Decoded |
| TX and ACK Information | As appropriate | May be ignored |
| Stream Index | 0x00 | May be ignored |

6.3.3a.2 Secure command frame

Insert a line at the end of Table 6-11a as follows:

Table 6-11a—MAC header settings for a Secure command frame for pairnet

| Header field | Setting on transmission | Interpretation on reception |
| --- | --- | --- |
| Frame type | Command value in Table 6-1a | Decoded |
| SEC | 1 | Decoded |
| ACK policy | As appropriate | May be ignored |
| DestID | As appropriate | Decoded |
| SrcID | As appropriate | Decoded |
| TX and ACK Information | As appropriate | May be ignored |
| Stream Index | 0x00 | May be ignored |

6.3.4a.1 Non-Secure HRCP Aggregated Data frame

Insert the following text and table before section 6.3.4a.2

The MAC header settings for a non-secure HRCP Aggregated Data frame shall be set and interpreted as described in Table 6-xx

Table 6-xx—MAC header settings for a non-secure HRCP Aggregated Data frame for pairnet

| Header field | Setting on transmission | Interpretation on reception |
| --- | --- | --- |
| Frame type | Data value in Table 6-1a | Decoded |
| SEC | 0 | Decoded |
| ACK policy | As appropriate | Decoded |
| DestID | As appropriate | Decoded |
| SrcID | As appropriate | Decoded |
| TX and ACK Information | As appropriate | Decoded |
| Stream Index | 0x00 | May be ignored |

6.3.4a.2 Secure HRCP Aggregated Data frame

Insert the following text and table before section 6.3.5

The MAC header settings for a secure HRCP Aggregated Data frame shall be set and interpreted as described in Table 6-xx

Table 6-xx—MAC header settings for a secure HRCP Aggregated Data frame for pairnet

| Header field | Setting on transmission | Interpretation on reception |
| --- | --- | --- |
| Frame type | Data value in Table 6-1a | Decoded |
| SEC | 1 | Decoded |
| ACK policy | As appropriate | Decoded |
| DestID | As appropriate | Decoded |
| SrcID | As appropriate | Decoded |
| TX and ACK Information | As appropriate | Decoded |
| Stream Index | 0x00 | May be ignored |

After 6.3.4a.2, insert the following new subclause as 6.3.5a.1 and 6.3.5a.2:

6.3.5a.1 Non-Secure HRCP Aggregated Multi-protocol Data frame

The non-secure  HRCP Aggregated Multi-protocol Data frame is same frame format as Figure 6-58e but the Payload field is replaced by HRCP Multi-protocol Data Payload format illustrated in Figure 6-64a.

| Octets:1 | variable | variable |
| --- | --- | --- |
| Data ID | Data Header | Data Payload |

Figure 6-64a—HRCP Multi-protocol Data Payload format

The Frame Type field shall be set to the Multi-protocol Data frame value in Table 6-1a, and the SEC field shall be set to zero. The MAC header settings for a non-secure HRCP Multi-protocol Data frame shall be set and interpreted as described in Table 6-xx.

The Data ID field and the Data Header field are defined in 6.3.5.1.

Table 6-xx—MAC header settings for a non-secure HRCP Aggregated Multi-protocol Data frame for pairnet

| Header field | Setting on transmission | Interpretation on reception |
| --- | --- | --- |
| Frame type | Data value in Table 6-1a | Decoded |
| SEC | 0 | Decoded |
| ACK policy | As appropriate | Decoded |
| DestID | As appropriate | Decoded |
| SrcID | As appropriate | Decoded |
| TX and ACK Information | As appropriate | Decoded |
| Stream Index | As appropriate | Decoded |

6.3.5a.1 Secure HRCP Aggregated Multi-protocol Data frame

The Secure HRCP Aggregated Multi-protocol Data frame is same frame format as Figure 6-58g but the Payload 1 …  n  field are replaced by HRCP Multi-protocol Data Payload format illustrated in Figure 6-64a..

The Frame Type field shall be set to the secure HRCP Aggregated Multi-protocol Data frame value in Table 6-1a, and the SEC field shall be set to one. The MAC header settings for a secure HRCP Aggregated Multi-protocol Data frame shall be set and interpreted as described in Table 6-xx.

The SECID field is defined in 6.2.7.2.

The SFC field is defined in 6.2.7.3.

The Data ID field and the Data Header field are defined in 6.3.5.1.

The Integrity Code field is defined in 6.2.7.5

The FCS field is defined in 6.2.7.6.

Table 6-xx—MAC header settings for a secure HRCP Aggregated Multi-protocol Data frame for pairnet

| Header field | Setting on transmission | Interpretation on reception |
| --- | --- | --- |
| Frame type | Data value in Table 6-1a | Decoded |
| SEC | 1 | Decoded |
| ACK policy | As appropriate | Decoded |
| DestID | As appropriate | Decoded |
| SrcID | As appropriate | Decoded |
| TX and ACK Information | As appropriate | Decoded |
| Stream Index | As appropriate | Decoded |

7.8 Aggregation

Insert text before section 7.9 as follows:

For non-secure and secure Aggregated Multi-protocol Data frame, in the case of a single MSDU is fragmented to multi subframes, the Data ID and the Data Header field shall also copy to each relevant subframe fields.