IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3e Comment Resolution for i-222 of Sponsor Ballot |  |
| Date Submitted | 13, Sep. 2016 |  |
| Source | [ Kondou, Keitarou; Akiyama, Keiji] [Sony Semiconductor Solutions Corp.] [1-7-1 Konan, Minato-ku, Tokyo Japan] | Voice: [ +81-3-6748-4001] Fax: [ +81-3-6748-4268] E-mail: [Keitarou.Kondo@sony.com , Keiji.Akiyama@sony.com ] |
| Re: | In response to i-222 from consolidated sponsor ballot comments (15-16-0585-00-003e) |  |
| Abstract | Clarify Ack mode for Association Request Command |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Comment

| Comment # | Page | Subclause | Line | Comment | Must Be Satisfied | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| i-222 |  | 6.3.3a.1 |  | "When sending command frames the ACK Policy field in the Frame Control field shall be set to Stk-ACK." --> This is not true for some command frames such as Association Request and Array training command | No | Change the description accordingly as indicated in the comment |

Resolution:

Clarify  Association Request command be sent by No-ACK policy."

Add the text from following page.

Replace the first two sentences of 6.3.3a.1 as indicated:

6.3.3a.1 Non-secure Command frame

When sending command frames the ACK Policy field in the Frame Control field shall be set to Stk-ACK. Commands may be fragmented by Stk-ACK like a data frame.
The Non-secure command frame format shall be formatted as illustrated in Figure 6-56a.

Insert the text after the first sentence of 6.5.1.1 as indicated:

6.5.1.1 Association Request command

The Association Request command Payload field shall be formatted as illustrated in Figure 6-126 for piconet and Figure 6-126a for pairnet. The ACK Policy field shall be set to No-ACK.