IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Resolution for comment i-21, Multi-Rate support |  |
| Date Submitted | 13, Sep. 2016 |  |
| Source | [ Kondou, Keitarou] [Sony Semiconductor Solutions Corp.] [1-7-1 Konan, Minato-ku, Tokyo Japan] | Voice: [+81-50-3140-1435] Fax: [ +81-3-6748-4268] E-mail: [Keitarou.Kondo@sony.com] |
| Re: | In response to i-21 from consolidated sponsor ballot comments (15-16-0585-00-003e) |  |
| Abstract | Add description of HRCP PHY for Multi-Rate Support as specified in section 7.13 of the baseline spec. |  |
| Purpose | .Resolve the comment i-21 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Comment

| Comment # | Page | Subclause | Line | Comment | Must Be Satisfied | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| i-21 |  | 7.13 |  | Add description of HRCP PHY for Multi-Rate Support as specified in section 7.13 of the baseline spec. | Yes | As commented. |

Resolution:

Insert following descriptions as a part of the draft of 802.15.3e,

7.13 Multi-rate support

Change the dashed list after the first paragraph as indicated

— In 10.3 for the 2.4 GHz PHY

— In 11.2.2.1 for the SC PHY mode

— In 11.3.2.1 for the HSI PHY mode

— In 11.4 for the AV PHY mode

— In 11a.2 for the HRCP-SC PHY mode

— In 11a.3 for the HRCP-OOK PHY mode

Change the text at the second paragraph as indicated

In order to determine the rates that are supported by a target DEV in the piconet or pairnet, the DEV shall use one of the following three methods:

- Check the capabilities of the target DEV broadcast by the PNC when the DEV becomes a member of the piconet.

- Send a Probe Request command, as described in 6.5.4.5, to the target DEV to request its Capability IE, as described in 6.4.11.

- Request the information from the PNC using the PNC Information Request command, as described in 6.5.4.1.

- Check the capabilities of the target DEV exchanged during the association process of the pairnet as described in 5.3.5.