IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Resolution for comment i-89 and i-90, frame length filed correction |  |
| Date Submitted | 13, Sep. 2016 |  |
| Source | [ Kondou, Keitarou]] [Sony Semiconductor Solutions Corp.] [1-7-1 Konan, Minato-ku, Tokyo Japan] | Voice: [ +81-50-3140-1435] Fax: [ +81-3-6748-4268] E-mail: [Keitarou.Kondou@sony.com] |
| Re: | In response to i-89 from consolidated sponsor ballot comments (15-16-0585-00-003e) |  |
| Abstract | Correction for the bit-width of the frame length field in the SC-PHY header. |  |
| Purpose | .Resolve the comment i-89 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Comment

| # | Page | Subclause | Line | Comment | Must Be Satisfied | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| i-89 | 97 | 11a.2.3.2.1 | 6 | The 21-bit field of the frame length is not sufficient for maximum payload length allowed (2,099,200 octets). | Yes | Increase the bit width of the field to 22. Add some reserved bits if needed. |
| i-90 | 109 | 11a.2.8.7 | 58 | The Field order of HRCP-SC-MIMO PHY Header is quite different from that of HRCP-SC PHY Header. And number of bits assigned for frame length is smaller than that of HRCP-SC PHY Header. | Yes | Correct the order of the fields and change the number of bits for frame length to 22. |

Resolution:

Change the bit-width of the field suggested in the comment sheet. Some related items shall be updated. Apply following instruction to the current version of draft (D04).

11a.2.2.2 Header dependent parameters

Change the table 11a-7 as follows

| Header rate (Mb/s) | Moudlation Scheme | Spreading Factor, LSF | FEC | PW length (chips), LPW | Code bits per block, LCBPB | Number of occupied blocks, Nblock_hdr | Number of stuff bits, LSTUFF |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 162168 | π/2-shift BPSK | 4 | EH | 8 | 14 | 19 | 102 |

11a.2.2.3 Timing-related parameters

Change the row “Lhdp” in the table 11a-9 as follows

| Lhdp | Length of the encoded frame header in octets | 3233 |
| --- | --- | --- |

11a.2.3.2.1 HRCP-SC PHY header

Replace the figure 11a-7 as follows

| Bits: b0-b2 | b3 | b4-b7 | b8-b9 | b10-b31 | b32-b35 |
| --- | --- | --- | --- | --- | --- |
| MCS | Pilot word | Scrambler Seed ID | PPRE | Frame Length | Reserved |

Figure 11a-7—PHY header format for HRCP-SC

11a.2.8.7 HRCP-SC-MIMO PHY Header

Change the text of first paragraph as follows

Information for MIMO and channel aggregation bitstream processing, described in 11a.2.8.6, shall be included in the header as shown in Figure 11a-14. The HRCP-SC-MIMO PHY header shall be processed in the same manner as described in 11a.2.3.2.

Replace the figure 11a-14 as follows

| Bits: b0-b2 | b3 | b4-b7 | b8-b9 | b10-b31 | b32-b33 | b34-b37 | b38-b39 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS | Pilot word | Scrambler Seed ID | PPRE | Frame Length | Reserved | Spatial (MIMO) Stream Number 0 ~ 15 | Channel Number for Channel Aggregation, 0~3 |

Figure 11a-14— PHY header format for MIMO PHY