IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Proposed resolution for discovery MAC and PHY service primitives |
| Date Submitted | September 14th, 2016 |
| Source | Marco Hernandez, Huan-Bang Li, F. Kojima (NICT) |
| Response | In response to Call for Contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1. MAC service primitives	2

1.1 Discovery	2

2. PHY service primitives	6

2.1 Discovery	6

- MAC service primitives

  - Discovery

These primitives are used for the targeted   discovery of PDs.

      - MLME-DISCOVERY.request

The primitive requests a PD to perform discovery. The properties of this primitive are:

MLME-DISCOVERY.request{

DestinationAddress;

MulticastGroupId;

DiscoveryInfoMode;

CyclicSuperframeStructure;

}

The primitive parameters are defined in Table 1.

Table 1—MLME-DISCOVERY.request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
|  |  | Table 2 |  |
| ChannelPage | Integer | Any valid channel page as defined in Error: Reference source not found | The channel page on which to attempt discovery. |
| ChannelNumber | Integer | Any valid channel number as defined in Error: Reference source not found | The channel number on which to attempt discovery. |
|  |  | Error: Reference source not found |  |
| DestinationAddress | MAC address | IEEE 48 bit address | MAC address of the target PD requesting its discovery information. |
| MulticastGroupId | Integer | 0 to 216 −1 | Multicast address of the PAC group requesting their PDs discovery information. |
| DiscoveryInfoMode | Enumeration | PD_INFO, GROUP_INFO | Indicates if the request is for the PD discovery information or the PDs discovery information in the PAC group |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 3 | Indicates the structure of cyclic-superframe |

.

PAC ID description

Cyclic-superframe structure descriptors are defined in Table 3. The default value of duration of cyclic-superframe, number of primary superframe, type of primary superframe, and type of secondary superframe are set to 1, 1, b4’0000, and b4’0000, respectively.

Table 3—Cyclic-superframe structure descriptor

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Duration of cyclic-superframe | Integer | 1 octet | Number of superframes in a cyclic-superframe. |
| Number of primary superframe | Integer | 1 octet | Number of primary superframe in a cyclic-superframe |
| Type of primary superframe. | Integer | As defined in Error: Reference source not found (Error: Reference source not found) | Type of accessible periods combination of primary superframe |
| Type of secondary superframe | Integer | As defined in Error: Reference source not found (Error: Reference source not found) | Type of accessible periods combination of secondary superframe |

      - When generated

This primitive is generated by the next higher to request for the discovery information of another PD or other PDs in the PAC group.

      - Effect on receipt

On recepit of this primitive the MAC sublayer shall send a Discovery Request command frame to the PD with MAC address indicated in the DestinationAddress parameter if the DiscoveryInfoMode is set to PD_INFO, over the channel page and channel number indicated in ChannelPage and ChannelNumber parameters, respectively. If the DiscoveryInfo parameter is set to GROUP_INFO, the MAC sublayer shall send a Discovery Request command frame to the multicast address indicated in the MulticastGroupId parameter.

When receiving the MLME-DISCOVERY.request primitive with the CyclicSuperfreamStructure parameter, the MAC sublayer sets the PIB with the value of the cyclic-superframe structure descriptor and selects the start time of a cyclic-superframe. After starting a cyclic-superframe, the MAC sublayer inserts the Cyclic-superframe descriptor IE to the MAC header of the Discovery request command frame. The MAC sublayer schedules to access a period of a superframe of a cyclic-superframe.

      - MLME-DISCOVERY.indication

The primitive is used to indicate to the next higher layer the reception of a Discovery request command frame. The properties of this primitive are:

MLME-DISCOVERY.indication{

ChannelPage;

ChannelNumber;

SourceAddress;

MulticastGroupId;

CyclicSuperframeStructure;

}

The primitive parameters are defined in Table 4.

Table 4— MLME-DISCOVERY.indication parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
|  |  | Table 2 |  |
| ChannelPage | Integer | Any valid channel page as defined in Error: Reference source not found | The channel page on which to attempt discovery. |
| ChannelNumber | Integer | Any valid channel number as defined in Error: Reference source not found | The channel number on which to attempt discovery. |
|  |  | Error: Reference source not found |  |
| SourceAddress | MAC address | IEEE 48 bit address | MAC address of the PD requesting discovery information. |
| MulticastGroupId | Integer | 0 to 216 −1 | Multicast address of the PAC group to send the discovery information. |
| DiscoveryInfoMode | Enumeration | PD_INFO, GROUP_INFO | Indicates if the request is for the PD discovery information or the PDs discovery information in the PAC group |
| CyclicSuperframeStructure | Cyclic-superframe structure descriptor | As defined in Table 3 | Indicates the structure of cyclic-superframe |

      - When generated

This primitive is generated by the MAC sublayer of the PD receiving a Discovery Request command frame to the next higher layer requesting for discovery information.

      - Effect on receipt

On recepit of this primitive, the next higher layer shall inform the requested discovery information.

      - MLME-DISCOVERY.response

The primitive is used to initiate a response to an MLME-DISCOVERY.indication primitive. The properties of this primitive are:

MLME-DISCOVERY.response{

DiscoveryInfoList;

ChannelPage;

ScanChannels;

Status;

}

The primitive parameters are defined in Table 5.

Table 5—MLME-DISCOVERY.response parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DiscoveryInfoList | List of PDs disacovery information | As defined in Table 2 | Indicates discovery information. |
| ChannelPage | Integer | Any valid channel page as defined in Error: Reference source not found | The channel page on which discovery was attempted. |
| ChannelNumber | Integer | Any valid channel number as defined in Error: Reference source not found | The channel number on which discovery was attempted |
|  |  | Error: Reference source not found |  |
| DiscoveryInfoMode | Enumeration | PD_INFO, GROUP_INFO | Indicates if the request is for the PD discovery information or the PDs discovery information in the PAC group |
| Status | Enumeration | SUCCESS, FAIL | Status of the discovery request |

      - When generated

This primitive is generated by the next higger layer to the MAC sublayer to send a Discovery Response command frame containing the requested discovery information.

      - Effect on recepit

On recepit of this primitive, the MAC sublayer shall send a Discovery Response command frame with the requested discovery information to the PD with MAC address indicated in the SourceAddress parameter if the DiscoveryInfoMode is set to PD_INFO. If the DiscoveryInfo parameter is set to GROUP_INFO, the MAC sublayer shall send a Discovery Response command frame with the requested discovery information  to the multicast address indicated in the MulticastGroupId parameter over the channel page and channel number indicated in the ChannelPage and ChannelNumber parameters, respectively.

      - MLME-DISCOVERY.confirm

The primitive reports the result requested by MLME-DISCOVERY.request of the requestor PD. The properties of this primitive are:

MLME-DISCOVERY.confirm{

DiscoveryInfoList;

Status;

}

The primitive parameters are defined in Table 6.

Table 6—MLME-DISCOVERY.confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DiscoveryInfoList | List of PDs discovery information | As defined in Table 2 Table 1 | Indicates discovery information. |
|  |  | Error: Reference source not found |  |
|  |  | Error: Reference source not found |  |
|  |  | Error: Reference source not found |  |
| Status | Enumeration | SUCCESS, FAIL | Status of the discovery request |

      - When generated

This primitive is generated by the MAC sublayer upon reception of a Discovery Response command frame to inform the next higher layer of the requested discovery information.

      - Effect on receipt

On recepit of this primitive, the next higher layer is informed of other PDs discovery information in the DiscoveryInfoList parameter, if the Status parameter indicates SUCESSFUL. If the Status parameter indicates FAIL, the discoveryInfoList is an empty list.

- PHY service primitives

  - Discovery

These primitives are used for untargeted discovery of PDs.

    - PLME-DISCOVERY.request

This primitive is a request by the PD management entity to the PHY entity to either transmit or receive the PD’s discovery information. The semantics of this primitive are:

MLME-DISCOVERY.request{

DiscoveryMode;

DiscoveryInfo;

}

The primitive parameters are defined in Table xx.

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DiscoveryMode | Enumeration | TX, RX | Indicates transmission or reception of discovery information. |
| DiscoveryInfo | Integer | As defined in Table 1 | Indicates the discovery information of the PD |

      - When generated

This primitive is generated by the next higher layer via the PD management entity to PD’s PHY entity to transmit the PD’s discovery information, indicated in the DiscoveryInfo parameter, if the DiscoveryMode parameter is set to TX. On the other hand, if the DiscoveryMode parameter is set to RX, the PHY entity detects the discovery information of other PDs.

      - Effect on receipt

On recept of this primitive, the PHY entity shall turn the PD’s transmitter on, if the DiscoveryMode parameter is set to TX with the intention to transmit the discovery information. On the other hand, if the DiscoveryMode parameter is set to RX, the PHY entity shall turn the PD’s receiver on during the discovery period with the intention to detect other PDs discovery information.

    - PLME-DISCOVERY.confirm

This primitive is issued by the PHY entity in response to PLME-Discovery.request. The semantics of this primitive are:

MLME-DISCOVERY.confirm{

DiscoveryMode;

DiscoveryInfoList;

Status;

}

The primitive parameters are defined in Table xx.

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| DiscoveryMode | Enumeration | TX, RX | Indicates transmission or reception of discovery information. |
| DiscoveryInfoList | List of PD’s discovery information | As defined in Table 1 | Indicates the discovery information of other PDs |
| Status | Enumeration | SUCCESFUL, FAIL | Indicates the status of the discovery information transmission or reception. |

      - When generated

This primitive is generated by the PHY entity in response to PLME-Discovery.request to the next higher layer via the PD management entity. If DiscoveryMode parameter is set to TX, the Status parameter indicates if the transmission of the PD’s discovery information was SUCCESFUL. On the other hand if the DiscoveryMode parameter is set to RX, the DiscoveryInfoList contains a list of PDs discovery information if the Status parameter indicates SUCCESFUL. If the Status parameter indicates FAIL, the DiscoveryInfoList is an empty list.

      - Effect on recepit

On receipt of this primitive, the next higher layer is informed of other PDs discovery information in the surroundings, if the Status parameter indicates SUCESSFUL.