IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Resolution for service primitive definitions: GET & SET |
| Date Submitted | September 14, 2016 |
| Source | Byung-Jae Kwak (ETRI), Nah-Oak Song (KAIST) |
| Response |  |
| Abstract |  |
| Purpose | For discussion & Approval |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

- Overview

- Normative references

- Definitions

- General description

- MAC functional description

- MAC services

  - MLME-SAP primitives

    - PIB access

      - MLME-GET.request

The primitive reads a given PIB field. The properties of this primitive are:

MLME-GET.request{

PIBName;

}

The primitive parameter is defined in Table 1.

Table 1—MLME-GET.request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| PIBName | String | As defined in Error: Reference source not found | Name of the PIB field to read. |

        - When generated

This primitive is generated by the next higher layer to request MLME for a given PIB field.

        - Effect on receipt

MLME reports the requested PIB field using MLME-GET.confirm described in 6.1.1.2.

      - MLME-GET.confirm

This primitive reports the result requested by MLME-GET.request. The properties of this primitive are:

MLME-GET.confirm{

Status;

PIBName;

PIBValue;

}

The primitive parameters are defined in Table 2.

Table 2—MLME-GET.confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, FAIL, UNSUPPORTED | The status of the request for PIB field information. |
| PIBName | String | As defined in Error: Reference source not found | Name of the PIB field to read. |
| PIBValue | As defined in Error: Reference source not found | As defined in Error: Reference source not found | The value of the indicated PIB field to read. |

The property PIBValue is set to zero when Status is UNSUPPORTED.

        - When generated

This primitive is generated by MLME in response to an MLME-GET.request, described in 6.1.1.1, received from the next higher layer, to report the value of requested PIB field.

        - Effect on receipt

The next higher layer receives the status and the value of the PIB field it has requested using MLME-GET.request described in 6.1.1.1.

      - MLME-SET.request

The primitive allows writing a given value to the indicated PIB field. The properties of this primitive are:

MLME-SET.request{

PIBName;

PIBValue;

}

The primitive parameters are defined in Table 3.

Table 3—MLME-SET.request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| PIBName | String | As defined in Error: Reference source not found | Name of the PIB field to write. |
| PIBValue | Various types | As defined in Error: Reference source not found | The value of the indicated PIB field to write. |

        - When generated

This primitive is generated by the next higher layer to request MLME to write a given value to the indicated PIB field.

        - Effect on receipt

If successful, the given value is written to the indicated PIB field. The result is reported to the next higher layer using MLME-SET.confirm described in 6.1.1.4.

      - MLME-SET.confirm

This primitive reports the result requested by MLME-SET.request. The properties of this primitive are:

MLME-SET.confirm{

Status;

PIBName;

}

The primitive parameters are defined in Table 4.

Table 4—MLME-SET.confirm parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, INVALID, READ_ONLY, UNSUPPORTED. | INVALID means PIBValue is out of range. UNSUPPORTED means PIBName is not defined in the PIB. |
| PIBName | String | As defined in Error: Reference source not found | Name of the written PIB field. |

        - When generated

This primitive is generated by MLME in response to an MLME-SET.request, described in 6.1.1.3, received from the next higher layer, to report the result of writing a given value to the indicated PIB field.

        - Effect on receipt

The next higher layer receives the result of MLME-SET.request it has requested to MLME.

    - Discovery primitives