IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Resolution for service primitive definitions: Synchronization |
| Date Submitted | September 14, 2016 |
| Source | Byung-Jae Kwak (ETRI), Nah-Oak Song (KAIST) |
| Response |  |
| Abstract |  |
| Purpose | For discussion & Approval |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

- Overview

- Normative references

- Definitions

- General description

- MAC functional description

- MAC services

  - MLME-SAP primitives

    - PIB access

    - Discovery primitives

    - Peering primitives

    - De-peering primitives

    - Frame error notification primitives

    - Reset MAC sublayer

    - Receiver enable

    - Channel scanning

    - Group configuration

    - Synchronization

      - MLME-SYNCHRONIZATION.request

The primitive requests to synchronize with other PDs by acquiring and tracking their synchronization signal. The properties of this primitive are:

MLME-SYNCHRONIZATION.request{

ChannelNumber;

ChannelPage;

TrackMode;

}

The primitive parameters are defined in Table 1.

Table 1—MLME-SYNCHRONIZATION.request parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| ChannelNumber | Integer | As defined in Error: Reference source not found | The channel number on which to attempt synchronization. |
| ChannelPage | Integer | As defined in Error: Reference source not found | The channel page on which to attempt synchronization. |
| TrackMode | Boolean | TRUE, FALSE | TRUE if the PD is to synchronize from the next synchronization period. FALSE if the PD is to synchronize with only the next synchronization period. |

When the primitive is received by the MLME, it will set phyCurrentPage and phyCurrentChannel equal to the values of the ChannelPage and ChannelNumber, respectively. If the TrackMode parameter is equal to TRUE, the MLME will track the synchronization period, i.e., enable its receiver just before the expected time of synchronization period. If the TrackMode parameter is equal to FALSE, the MLME will locate the synchronization period, but not continue to track it.

If this primitive is received by the MLME while it is currently tracking the synchronization period, the MLME will not discard the primitive but will treat it as a new synchronization request.

        - When generated

This primitive is generated by the next higher layer to request MLME to synchronize with other PDs.

        - Effect on receipt

MLME attempts synchronization with other PDs by acquiring and tracking their synchronization signal. The result of synchronization is reported to the next higher layer using MLME-SYNCHRONIZATION.confirm described in 6.1.10.2.

      - MLME-SYNCHRONIZATION.confirm

The primitive reports the results of synchronization request to the higher layer. The properties of this primitive are:

MLME-SYNCHRONIZATION.confirm{

Status;

}

The primitive parameters are defined in Table 1.

Table 2—MLME-SYNCHRONIZATION.confirm parameter

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, FAILURE, CHANNEL_ACCESS_FAILURE | The result of the synchronization |

        - When generated

This primitive is generated by MLME in response to an MLME-SYNCHRONIZATION.request, described in 6.1.10.1, received from the next higher layer, to report the result of synchronization.

        - Effect on receipt

The next higher layer receives the result of MLME-SYNCHRONIZATION.request it has requested to MLME.

      - MLME-SYNCHRONIZATION-LOSS.indication

The primitive indicates the loss of synchronization. The properties of this primitive are:

MLME-SYNCHRONIZATION-LOSS.indication{

LossReason;

GroupID;

PD_ID;

ChannelNumber;

ChannelPage;

}

The primitive parameters are defined in Table 3.

Table 3—MLME-SYNCHRONIZATION-LOSS.indication parameters

| Property | Type | Valid range | Description |
| --- | --- | --- | --- |
| LossReason | Enumeration | SYNC_LOST | The reason synchronization was lost. |
| MulticastGroup_ID | Integer | 0 to 255 | Group ID of the PD that has lost synchronization |
| PD_ID | MAC address | PD specific | The MAC address of the PD that has lost synchronization |
| ChannelNumber | Integer | Any valid channel number as defined in Error: Reference source not found | The channel number on which the PD that has lost synchronization operated. |
| ChannelPage | Integer | Any valid channel page as defined in Error: Reference source not found | The channel page on which the PD that has lost synchronization operated. |

The primitive is generated by the MLME of a PD and issued to its next higher layer in the event of a loss of synchronization. The reason for synchronization loss is expressed by the parameter of LossReason:

SYNC_LOST – The PD has missed synchronization for five superframes.

        - When generated

This primitive is generated by MLME to indicate the loss of synchronization to the next higher layer.

        - Effect on receipt

The next higher layer shall generate MLME-SYNCHRONIZATION.request, described in 6.1.10.1, to request MLME to synchronize with other PDs.

    - Channel sounding