IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3e Comment Resolution for i-25 of Sponsor Ballot |  |
| Date Submitted | 13, Sep. 2016 |  |
| Source | [ Kondou, Keitarou; Akiyama, Keiji] [Sony Semiconductor Solutions Corp.] [1-7-1 Konan, Minato-ku, Tokyo Japan] | Voice: [ +81-3-6748-4001] Fax: [ +81-3-6748-4268] E-mail: [Keitarou.Kondo@sony.com , Keiji.Akiyama@sony.com ] |
| Re: | In response to i-25 from consolidated sponsor ballot comments (15-16-0585-00-003e) |  |
| Abstract | Add HigherLayerProtocolInformation to some MLME primitives |  |
| Purpose | To respond comment id i-25 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Comment

| Comment # | Page | Subclause | Line | Comment | Must Be Satisfied | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| i-25 |  | 5.3 |  | For MLME-START.request, MLME-SCAN.confirm, MLME-ASSOCIATION.request, MLME-ASSOCIATION.indication, MLME-ASSOCIATION.response, MLME-ASSOCIATION.confirm, a parameter to hand over Higher Layer Protocol Information Element is needed. (For HRCP, may be present) | Yes | As commented. |

Resolution:

Add the parameter to MLME-SCAN.confirm primitive

5.3.2 Scanning for piconets and pairnets

Insert a line at the end of Table  5-5 as indicated;

Table 5-5—MLME-SCAN primitive parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| HigherLayerProtocolInformation | As defined in the Higher Layer Protocol Information IE | As defined in 6.4.38 | Included 3 octets Unique ID and Higher Layer Protocol Information in variable length |

5.3.2.2 MLME-SCAN.confirm

Insert a parameter in MLME-SCAN.confirm as indicated;

MLME-SCAN.confirm (

NumberOfPiconets,

PiconetDescriptionSet,

NumberOfChannels,

ChannelRatingList,

HigherLayerProtocolInformation,

ResultCode,

ReasonCode

)

5.3.2 Starting a piconet or pairnet

Insert a line at the end of Table  5-8 as indicated;

Table 5-8—MLME-START primitive parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| HigherLayerProtocolInformation | As defined in the Higher Layer Protocol Information IE | As defined in 6.4.38 | Included 3 octets Unique ID and Higher Layer Protocol Information in variable length |

5.3.3.1 MLME-START.request

Insert a parameter in MLME-START.request as indicated;

MLME-START.request (

Bsid,

SecMode,

MinDepSuperframePercent,

DesiredDepSuperframePercent,

PhyMode,

HigherLayerProtocolInformation

)

5.3.5 Associating with a piconet or pairnet

Insert a line at the end of Table  5-10 as indicated;

Table 5-8—MLME-START primitive parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| HigherLayerProtocolInformation | As defined in the Higher Layer Protocol Information IE | As defined in 6.4.38 | Included 3 octets Unique ID and Higher Layer Protocol Information in variable length |

5.3.5.1 MLME-ASSOCIATE.request

Insert a parameter in MLME-ASSOCIATE.request as indicated;

MLME-ASSOCIATE.request	 (

Bsid,

Pnid,

PncAddress,

ChannelIndex,

NeighborPiconetRequest,

PiconetServicesInquiry,

HRCP DEV Capability,

Timeout,

HigherLayerProtocolInformation

)

5.3.5.2 MLME-ASSOCIATE.confirm

Insert a parameter in MLME-ASSOCIATE.confirm as indicated;

MLME-ASSOCIATE.confirm	 (

DevID,

VendorSpecificIE,

HRCP Operation Parameters,

HigherLayerProtocolInformation,

ResultCode,

ReasonCode

)

5.3.5.3 MLME-ASSOCIATE.indication

Insert a parameter in MLME-ASSOCIATE.indication as indicated;

MLME-ASSOCIATE.indication 	(

DevId,

DevAddress,

HRCPOperationParameters,

HigherLayerProtocolInformation

)