IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Resolution for security MAC service primitives |
| Date Submitted | September 15h, 2016 |
| Source | Marco Hernandez, Huan-Bang Li, F. Kojima (NICT) |
| Response | In response to Call for Contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1. MAC service primitives	2

1.1 Security	2

1.2 MLME-PEERKEY-START.request	6

- MAC service primitives

  - Security

These primitives are used for the security procedure.

    - MLME-PN-EXHAUSTION.indication

This primitive indicates that the PN associated with a temporal key exceeds PNExhaustionThreshold.

The primitive parameters are as follows:

MLME-PN-EXHAUSTION.indication{

Key ID,

Key Type;

}

Table 1—PN-EXHAUSTION parameters

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| Key ID | Integer | N/A | Key identifier |
| Key type | Integer | Pairwise, group | Defines whether this key is a pairwise key or group key. |

      - When generated

This primitive is generated by the MLME when the PN associated with a temporal key exceeds PNExhaustionThreshold.

      - Effect on receipt

On receipt of this primitive, the PD deletes the temporal key associated with the PN.

    - MLME-SETKEYS.request

This primitive causes the keys identified in the parameters of the primitive to be set in the MAC and enabled for use.

The primitive parameter is as follows:

MLME-SETKEYS.request{

KeyList;

}

- —KeyList parameters

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| KeyList | Key Descriptors | N/A | The list of keys to be used by the MAC sublayer. |

Table 2— Key Descriptors parameters

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| Key | Bit string | N/A | The temporal key value |
| Length | Integer | N/A | The number of bits in the Key to be used |
| Key ID | Integer | 0 GCM 1-255 reserved | Key identifier |
| Key type | Integer | Pairwise, group | Defines whether this key is a pairwise key or group key. |
| DestinationAddress | MAC address | IEEE 48 bits | This parameter is valid only when the Key Type value is Pairwise |
| MulticastGroupID | Integer | 0 – 216-1 | This parameter is valid only when the Key Type value is Group |
| Receive Sequence Count | 8 octets | N/A | Value to which the RSC(s) is initialized |
| Cipher Suite Selector | 4 octets | As defined in XXX | The cipher suite required for this procedure |

      - When generated

This primitive is generated at any time when one or more keys are to be set in the MAC sublayer.

      - Effect on receipt

Provided the MLME-SETPROTECTION.request primitive has been issued, on receipt of this primitive, the MAC sublayer applies the keys as follows

- The MAC sublayer uses the key information as defined by the Key Type, Key ID, address, and Cipher Suite Selector parameters, for the transmission of subsequent secured frames to which the key applies.

- The MAC sublayer installs the key with the associated Key ID, such that received secured frames for the cipher suite indicated by the Cipher Suite Selector parameter, and containing the matching Key ID are processed using that key, subject to validation based on the Receive Sequence Count parameter.

    - MLME-DELETEKEYS.request

This primitive causes the keys identified in the parameters of the primitive to be deleted from the MAC and thus disabled for use.

The primitive parameter is as follows:

MLME-DELETEKEYS.request{

DeleteKeyList;

}

- —DeleteKeyList parameters

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| DeleteKeyList | Delete Key Descriptors | N/A | The list of keys to be deleted from the MAC sublayer. |

- —Delete Key Descriptor parameters

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| Key ID | Integer | 0 GCM 1-255 reserved | Key identifier |
| Key type | Integer | Pairwise, group | Defines whether this key is a pairwise key or group key. |
| DestinationAddress | MAC address | IEEE 48 bits | This parameter is valid only when the Key Type value is Pairwise |
| MulticastGroupID | Integer | 0 – 216-1 | This parameter is valid only when the Key Type value is Group |

      - When generated

This primitive is generated by MLME at any time when keys for a security association are to be deleted in the MAC sublayer.

      - Effect on receipt

On receipt of this primitive, the MAC deletes all temporal keys identified by each Delete Key Descriptor in the DeleteKeyList, defined by the Key Type, Key ID and Address parameters, and to cease using them.

    - MLME-SKF.request

This primitive is generated by the MLME when the PD has a SKF frame to send.

The primitive parameters are as follows:

MLME-SKF.request {

KeyType

DestinationAddress;

MulticstGroupId;

Data;

}

- —SKF parameters

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| Key type | Integer | Pairwise, Group | Defines whether this key is a pairwise key or group key. |
| DestinationAddress | MAC address | IEEE 48 bits | This parameter is valid only when the Key Type value is Pairwise |
| MulticastGroupID | Integer | 0 – 216-1 | This parameter is valid only when the Key Type value is Group |
| Data |  | NA | Data is specified in Error: Reference source not found and Error: Reference source not found |

.

      - When generated

This primitive is generated by the MLME when the it has a SKF frame to send.

      - Effect on receipt

The MAC sends this SKF frame to the PD with MAC address in the DestinationAddress parameter if the Key type parameneter is set to Pairwise. If the Key type parameneter is set to Group, The MAC sends this SKF frame to the multicast address indicated in the MulticastGroupID parameter.

    - MLME-SKF.confirm

This primitive indicates that the SKF frame has been transmitted by the MAC sublayer.

The semantics of this primitive   are as follows:

MLME-SKF.confirm {

ResultCode;

}

- —ResultCode parameters

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, TRANSMISSION_FAILURE | Indicates whether the SKF frame has been transmitted to the target PD. |

      - When generated

This primitive is generated by the MAC sublayer to the MLME as a result of an MLME-SKF.request primitive being created to send a SKF frame.

      - Effect on receipt

The MLME is always notified whether the SKF frame has been transmitted.

  - MLME-PEERKEY-START.request

This primitive is generated by the MLME to start a PeerKey handshake with a PD.

The semantics of this primitive are as follows:

MLME-PEERKEY-START.request{

DestinationAddress;

CipherSuiteSelector

}

- —PEER-KEY-Start parameters

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| DestinationAddress | MAC address | IEEE 48 bits | This parameter is valid only when the Key Type value is Pairwise |
| Cipher Suite Selector | 4 octets | As defined in XXX | The cipher suite required for this procedure |

GCM-128, GCM-256

      - When generated

This primitive is generated by the MLME for a PD to initiate a PeerKey handshake with a specified peerMAC entity of another PD in order to create a secure link between the two PDs.

      - Effect on receipt

This primitive initiates the master key handshake (ECDH) as part of PeerKey handshake by sending a SKF frame.

    - MLME-SETPROTECTION.request

This primitive indicates whether protection is required for frames sent to and received from the indicated MAC address.

The primitive parameter is as follows:

MLME-SETPROTECTION.request{

ProtecDescriptor

}

Table 4— ProtecDescriptor parameters

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| ProtectType | Enumeration | None, Rx_Tx | The protection value for this MAC sublayer. |
| Key Type | Integer | Pairwise, group | Defines whether this key is a pairwise key, Group key. |
| DestinationAddress | MAC address | IEEE 48 bits | This parameter is valid only when the Key Type value is Pairwise |
| MulticastGroupID | Integer | 0 – 216-1 | This parameter is valid only when the Key Type value is Group |

      - When generated

This primitive is generated by the MLME when protection is required for frames sent to and received from  the PD with MAC address indicated in DestinationAddress parameter or PAC group with multicast address indicated in the MulticastGroupId parameter.

      - Effect on receipt

On receipt of this primitive, the MAC sublayer sets the protection to secure Data frames, as indicated in the ProtectType parameter:

- None: specifies that Data frames to and from the MAC address or multicast address are not protected.

- Rx_Tx: specifies that Data frames to and from the MAC address or multicast address are protected.

Once Data frames are protected to and from the specified MAC address or multicast address, the MLME-SETPROTECTION.request primitive is used to reset the prior setting. Invocation of the MLME-SETPROTECTION.request primitive with a ProtectType of None deletes a protection state.