IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Resolution for comment r01-71, update MAC PIB |  |
| Date Submitted | 21, Oct. 2016 |  |
| Source | [ Kondou, Keitarou] [Sony Semiconductor Solutions Corp.] [1-7-1 Konan, Minato-ku, Tokyo Japan] | Voice: [+81-50-3140-1435] Fax: [ +81-3-6748-4268] E-mail: [Keitarou.Kondo@sony.com] |
| Re: | In response to r01-71 from Consolidated Sponsor Ballot recirculation-1 comments (15-16-0726-01-003e) |  |
| Abstract | Defines the chip rate in the case channel bonding is used and provide the data processing procedure in the case channel aggregation is used. |  |
| Purpose | Resolve the comment r01-71 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Comment

| # | Page | Subclause | Line | Comment | Must Be Satisfied | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- |
| r01-71 | 36 | 5.4 | 10 | MAC PIB has not been updated for 15.3e. Characteristics of HRCP DEVs should be added to the table. | No | Consider to update the MAC PIB for 15.3e |

Resolution:

Accept the comments and update the MAC PIB.

Following texts and changes are applied to the baseline document, IEEE 802.15.3-2016.

5.4 MAC management

Change the title of 5.4.1 as follows:

5.4.1 MAC PIB PNC and PRC group

Insert the following text and table after Table 5-28:

The MAC PIB PRC group, Table 5-28a, describes both the DEV’s PRC capabilities as well as the characteristics of the current pairnet.

Table 5-28a—MAC PIB PRC group parameters

| Managed Object | Octets | Definition | Access |
| --- | --- | --- | --- |
| macSuperframeDuration | 2 | Duration of the superframe. | Read Only |
| macNumAssocSlots | 1 | Number of association slot. | Read Only |
| macDurAssocSlots | 1 | Duration of an association slot. | Read only |
| macPrcCapable | 1 bit | 1 if the DEV has the capability to become the PRC, 0. | Read Only |
| macPrcDesMode | 1 bit | 1 if it is desired that the DEV be the PRC. Provided only for PRC. | Read/Write |
| macSec | 1 bit | Indicates if the DEV is capable of operating a secure pairnet as the PRC. | Read Only |
| macAllowedChannelSet | Variable | A set of channel indices, one for each channel that the MAC is allowed to use for scanning and starting pairnet. | Read/Write |
| macAssocVendorSpecificIe | Variable | A Vendor Defined IE, as defined in 6.4.3, that is sent in the Association Response command, as described in 6.5.1.2, when the PRDEV is acting as the PRC. | Read/Write |
| macAssocHigherLayerIe | Variable | A Higher layer protocol information IE, as defined in 6.4.38, that is sent in the Beacon frame and association related commands, as described in 7.3a.3, when the PRDEV is acting as the PRC. Provided only for PRC. | Read/Write |
| macDesiredAtp | 2 | The ATP value to send in an Association Request command. | Read/write |
| macNextDevId | 1 | The Next DEVID value to send in a Beacon frame. | Read/write |

5.4.2 MAC PIB characteristic group

Insert the following text and table after Table 5-29:

The MAC PIB PRDEV characteristics group, Table 5-29a, contains information about the capabilities and characteristics of the PRDEV.

Table 5-29a—MAC PIB PRDEV characteristic group parameters

| Managed Object | Octets | Definition | Access |
| --- | --- | --- | --- |
| macDevAddress | 6 | The MAC address of the PRDEV. | Read Only |
| macDevId | 1 | The ID of the PRDEV. | Read Only |
| macPowerManagementMode | 1 | The current power management mode of the PRDEV. 0x00 = ACTIVE From 0x01 to 04, ignored 0x05 = LLPS | Read only |
| macLlpsSupported | 1 | 0x00 = PRDEV does not support LLPS mode. 0x01 = PRDEV supports LLPS mode. | Read Only |
| macPowerSource | 1 | 0x00 = Battery power. 0x01 = Mains power. | Read/Write |
| macSecurityOptionImplemented | 1 | 0x00 = Mode 0. 0x01 = Mode 1. | Read Only |
| macAggregationCapable | 1 | 0x00 = PRDEV does not support aggregation. 0x01 = PRDEV supports aggregation. | Read Only |