IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Correction of Draft 1 Annex D blurred equations |  |
| Date Submitted | December 2016 |  |
| Source | Billy Verso (Decawave Ltd), | billy.verso (at) decawave.com |
| Re: | Improvement of text readability 802.15.8 draft |  |
| Abstract | This document contains a reworking of the equations and realted text that appeared blurred in Annex D of the first letter ballot of the IEEE 802.15.8 draft standard. |  |
| Purpose | This document provides the details text for the IEEE 802.15.8 draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.15 Working Group or IEEE 802.15.8 Task Group. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Note to editor:

The text below is the draft text with everything except the Annex D text deleted, which caused the annex number to revert to A and equation numbering to start from 1.  You can choose to either paste the full text below to replace Annex D in the next draft or individually integrate the changes (which are tracked to facilitate this).

# (informative)
The mathematics of two-way ranging

## Single-sided two-way ranging error due to clock offset

With reference to Error: Reference source not found, the times Tround and Treply are measured independently by device A and B using their respective the local clocks, which have some clock offset error eA and eB from their nominal frequency.  As a result, Tprop is then actually:

Tprop=12Tround×1+eA−Treply×1+eB

And, the error in the range estimate equation (given in Error: Reference source not found) is:

Tprop−Tprop=12Tround−Treply−12Tround×1+eA−Treply×1+eB

Or:

Tprop−Tprop=12−Tround×eA+Treply×eB

Or, since Treply is 500 to 5000 times bigger than Tprop, for the purpose of simplifying the error estimate we can take Treply as (almost) the same as Tround and can be use Treply in place of Tround to give an error estimate:

error=Tprop−Tprop≈12eB−eA×Treply

Based on this equation, Table 1 presents the typical errors in SS-TWR time-of-flight estimation depending on the reply time, Treply, and the total clock offset error.

Table 1—typical clock induced error in SS-TWR time-of-flight estimation

| clock error Treply | 2 ppm | 5 ppm | 10 ppm | 20 ppm | 40 ppm |
| --- | --- | --- | --- | --- | --- |
| 100 µs | 0.1 ns | 0.25 ns | 0.5 ns | 1 ns | 2 ns |
| 200 µs | 0.2 ns | 0.5 ns | 1 ns | 2 ns | 4 ns |
| 500 µs | 0.5 ns | 1.25 ns | 2.5 ns | 5 ns | 10 ns |
| 1 ms | 1 ns | 2.5 ns | 5 ns | 10 ns | 20 ns |
| 2 ms | 2 ns | 5 ns | 10 ns | 20 ns | 40 ns |
| 5 ms | 5 ns | 12.5 ns | 25 ns | 50 ns | 100 ns |

Table 1 shows that quite accurate results are achievable when short messages are used (e.g. with shorter preamble lengths and higher data rates) to give small reply time and more accurate clock sources are employed in the PDs. Note: the reply time Treply is not just the RX-to-TX turnaround time but also includes the message length.    An error of 1 ns in time-of-flight is equivalent to a 30 cm error in distance.

## Deviation of double-sided two-way ranging formula

Figure 1 shows the two round trips of double sider ranging and defines the terms used for the round trip times, Raa and Rbb, and the reply times, Daa and Db, for the pair of devices, A and B, participating in the two-way ranging exchange to measure the time-of-flight, Tff.

Figure 1—terms used in deriving the double-sided two-way ranging formula

In Figure 1, device A transmits a message, P1, to device B.  Device B receives this message a short time later and after a time, Dbb, it transmits a message, P2, back to device A.  Message P2 arrives at device A at a time Raa after it transmitted message P1.  These then have the relationship:

Ra=2Tf+Db

So

Tf=12Ra−Db

In practice, the times are measured by real clocks in A and B, which will run independently, either faster or slower than an ideal clock, synchronized to their local reference frequency generator which can be assumed to be a constant frequency over the duration of the ranging exchange. Let us say that Clocks A and B run respectively at ka and kb times the frequency of an ideal, true, clock.  Any time measurements will be multiplied by these constants, ka or kb.  Denoting the actual time estimates for Ra and Da as Ra and Da respectively, and similarly Rb and Db as the estimates of Rb and Db. Then, since Ra is measured at A by A’s clock:

Ra=kaRa

And, similarly

Da=kaDa

Rb=kbRb

Db=kbDb

Using Ra,Da etc. as estimates for Ra, Da etc. we introduce a measurement error, so for example if Tf is an estimate of Tf we can say for a single round trip exchange

Tf=12Ra−Db=12kaRa−kbDa

And, the error in the estimation is

Tf−Tf=12ka−1Ra−kb−1Db

For the UWB physical layer the values of the expressions ka−1 and kb−1 may be up to 20 ppm, i.e. 20x10-6, and for accurate ranging it is important to keep the error below 100 ps (1x10-10) which means the delays e.g. Db must be kept below about 5 µs which is not possible where even short UWB frames are typically > 100 µs long.  The solution is to use two round trip delays.

We know from  that

Da=Daka

And similarly

Db=Dbkb

Then from  and  we can say

Ra=2Tf+Dbkb

And from  and

Ra=2kaTf+kaDbkb

And similarly

Rb=2kbTf+kbDaka

In , , and  we have quantities that we can measure, Ra, Rb, Da and Dbb. But we have no way of measuring ka or kb. And the errors in these quantities swamp the value of Tf.  There is however one thing we can do. If we multiply Ra by Rb, the bulk of the value of the product will be the product of Da and Db. For this term in the product, the ka and kb constants cancel each other out.

Then, from , , and

RaRb=4kakbTf2+kaDbkbkbDaka+2kbTfkaDbkb+2kaTfkbDaka

RaRb=4kakbTf2+DaDb+2TfkaDb+kbDa

RaRb−DaDb=4kakbTf2+2TfkaDb+kbDa

And, from , , and

RaRb−DaDbRa+Da=4kakbTf2+2TfkaDb+kbDa2kaTf+kaDbkb+Da

On left hand side, taking out 2Tf and multiplying above and below by kb

RaRb−DaDbRa+Da=2Tfkb2kakbTf+kaDb+kbDa2kakbTf+kaDb+kbDa

So finally

RaRb−DaDbRa+Da=2Tfkb≈2Tf

And similarly

RaRb−DaDbRb+Db=2Tfka≈2Tf

We now have two possible estimates for Tf, and since ka and kb are very close to unity, i.e. 0.99998 < ka, kb < 1.00002,  we can estimate Tf  as follows

Tfa=12RaRb−DaDbRb+Db

Tfb=12RaRb−DaDbRa+Da

These estimates are very close to the actual Tf because ka and kb are very close to one and, crucially, their accuracy is independent of the response delays employed at A and at B.

Whether a system should use formula  or formula  would depend on which clock it expects to be more accurate. For example, if the system knows B has a higher accuracy clocks then it should use formula .  If it expects neither to be more accurate than the other and it is more accurate to use the average result from  and  since this will always be as good as, or better than, the worst of the two  and .

This average can be approximated by the following formula that combines  and :

Tfab=RaRb−DaDbRa+Da+Rb+Db

For double-sided two-way ranging it is generally suggested that the delay times Da and Db have to be nearly equal for the overall error to be acceptably small, but this is not a restriction when employing equations ,  and , that burden is removed, so it is NOT required the use of the same response time at each end.  This gives much more implementation flexibility within the individual nodes participating in a ranging exchange and also facilitates more efficient implementations when a group of N nodes want to find the ½·N·(N-1) distances between each other.  Here, instead of separated ranging exchanges, messages can be combined, (e.g. a response from node B to Node A could also act as a ranging initiation message other nodes), to do the distance measurements with a much reduced over the air traffic, perhaps as few as 2·N messages.

## Asymmetric double-side two-way ranging error due to clock offset

Formulas  and  give the time of flight results (TOF) results including the clock offset errors. For the average formula then we can similarly say

Tfab=Tfka+kb2

Thus, the error in the estimated Tfab is:

Error=Tf−Tfab=Tf−Tfka+kb2

Or

Error=Tf×1−ka+kb2

To size this error, if devices A and B have clocks, where each are 20 ppm away from the nominal clock in directions make their combined error additive and equal to 40 ppm, then ka and kb might both be 0.99998 or  1.00002.

Even with a fairly large UWB operating range of say 100 m, the TOF is just 333 ns, so the error is:

20×10-6 × 333×10-9 seconds, which is 6.7×10-12 seconds or 6.7 picoseconds.

Again note that these error levels do NOT require the use of the same response time at each end.

At these error levels the precision of determining the arrival time of the RMARKER is actually the more significant source of error.  This depends on implementation but for the UWB PHY it might typically be expected to fall in a range from 1 ns down to 100 ps.