IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG 802.15.12 minutes for January, 2017 Interim Meeting |  |
| Date Submitted | [January 19th, 2017] |  |
| Source | [Charlie Perkins] [Futurewei] | Voice: [+1-408-330-4586] Fax: [ ] E-mail: [charliep@computer.org] |
| Re: | [TG 802.15.12 (ULI) meeting minutes] |  |
| Abstract | TG 802.15.12 minutes from the Atlanta IEEE 802 Wireless Interim Meeting |  |
| Purpose | [Record the transactions of the TG 802.15.12 Interim meeting during January, 2017] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Minutes for IEEE TG 802.15.12 Atlanta Interim Meeting

16-20 January, 2017

Chair – Pat Kinney

Vice Chair - Charlie Perkins

Meeting Objectives / Session Focus - TG12 -ULI

- Monday 16 Jan, PM1: Opening report, Agenda, Status and Functional decomposition review

- Monday 16 Jan, PM2: Session focus: Header Compression, PDE, MMI, Management Protocols Module (MPM), assignment of functional blocks not already assigned

- Tuesday 17 Jan, PM1: Detailed discussion on PDE

- Wednesday 18 Jan, PM1: Detailed discussion on MMI

- Wednesday 18 Jan, PM2: Detailed discussion on MMI

- Thursday 19 Jan, AM1: Detailed discussion on MPM

- Thursday 19 Jan, AM2: Discussion on Header Compression, Generic Module, Assigned module status, recap on week’s efforts, define the next steps, timetable for completion, phone call

- Adjourn

Monday PM1 (January 16) (Highland Ballroom V)

Chair called the meeting to order at 1:31pm.  Attendance: 15

Opening / Closing Report (DCN: 15-17-00xx-00-0000)

Agenda for Monday PM1 session:

- Call meeting to order - Opening Report - Approve agenda (15-17-0026-00-0000) - Approve minutes (15-16-0838-00-0000) - Review status from San Antonio - Discussion on ULI focus for Atlanta session - ULI presentation(s) - Recess

Patent Policy and call for Essential Patents

- Chair presents IEEE slides #1 to #4 of the IEEE patent and meeting conduct slides. Chair provides an opportunity for disclosure.  None was heard.

Review of meeting Goals

- Monday 16 Jan, PM1: Opening report, Agenda, Status and Functional decomposition review

- Monday 16 Jan, PM2: Session focus: Header Compression, PDE, MMI, Management Protocols Module (MPM), assignment of functional blocks not already assigned

- Tuesday 17 Jan, PM1: Detailed discussion on PDE

- Wednesday 18 Jan, PM1: Detailed discussion on MMI

- Wednesday 18 Jan, PM2: Detailed discussion on MMI

- Thursday 19 Jan, AM1: Detailed discussion on MPM

- Thursday 19 Jan, AM2: Discussion on Header Compression, Generic Module, Assigned module status, recap on week’s efforts, define the next steps, timetable for completion, phone call

Approve agenda (15-17-0026-00)

- No objections

Approve previous minutes (15-16-0838-00)

- No objections

Review status from San Antonio

- PDE (see doc# 15-16-656-02)

- MMI (see doc# 15-16-656-02)

- Revised Functional Decomposition

Discussion on ULI focus for Atlanta session

- Yokota to present management protocol on Thursday

- Pass-thru becomes mandatory, generic to be deleted

- Problem that ULI is not supposed to know frame size

- 802.15.12 Protocol Discrimination Entity (PDE) (see doc# 15-16-656-02)

- 802.15.12 Multiplexed MAC interface (MMI) (see also doc# 15-16-656-02). Uses 802.15.9 for multiplexed data service.

- The fragmentation services above and below will need interface descriptions

Recess at 2:42pm

Monday PM2 (January 16) (Highland Ballroom V)

Chairs called the meeting to order at 4:13pm. Attendance: 7.

Agenda for Monday PM2 session:

- Call meeting to order - Modify agenda to have MMI discussion instead - Recess

Discussion about MMI purpose/description

- Fragmentation within MMI only via 802.15.9

- Refining parameter descriptions for MCPS-DATA request on page 4-5 of 15-16-0656

- Problem to know maximum  frame length, since FRAME_TOO_LONG doesn’t say

- Discussion about when MAC layer adds IEs that are not visible to MMI SAP

Recess at 5:03pm

Tuesday PM1 (January 17) (Highland Ballroom V)

Chairs called the meeting to order at 1:34pm. Attendance: 10.

Agenda for PM1 session:

- Call meeting to order - Management module description - Recess

Management module description

- Review high-level definition of management module

- ULI “Profile” for Protocol Management

- Mapping between layers of different 802 specifications is not simple

- Use case 1: one-shot MAC/PHY configuration (via predefined profile)

- Pathway from ULM-SAP through Management Protocol module to/from MCPS-SAP

- ULI Profile identification and creation operations

- Discussion about which modules need to know about ULI Profiles (e.g., for 6top)

- Discussion whether Management Protocol block should sit between App and ULI_6top

- Decision to consider ULI Profile as a key mechanism for simplifying ULI use by applications

- Decision to use object definitions from openmobilealliance.org/release/lightweightm2m/ <filename OMA-SUP-XML_LWM2M_Device-V1_0-20160407-C.txt>

Discussion about PDE purpose/description

- Need to produce a module template

- Solicit help – volunteer found!

Recess at 2:51pm

Wednesday AM2 WG Midweek Interim (January 18) (Buckhead Ballroom 1)

Chair called the meeting to order at 10:31am, after delay due to spurious fire alarm. Attendance: about 31

- Administrivia (15-16-0857-05-0000-802-15-atl-2017-graphic-agenda)

- Task group reports

- 802.18 - Holcomb

- TG3d 100g - Kürner (DCN 15-17-0054)

- Closing: IG Thz - Kürner

- TG3e HRCP -Not Meeting In Atl – Estrada (DCN 15-17-0056)

- TG4s SRU (Spectrum Resource Utilization) - Kitasawa

- TG4t HRP (Higher Rate Phy) -Not Meeting In Atl - Powell

- TG10 L2r (Layer 2 Routing) -Not Meeting In Atl - Powell

- TG4v RSB (Regional Sub Gig Bands) - Beecher

- TG7m OWC (Optical Wireless Comms) - Jang

- TG8 PAC (Peer Aware Communications) - Lee

- IG LPWA (Low Power Wide Area) – Robert (DCN 15-17-0066-00-lpwa)

- IG HRRC (High Rate Rail) (Plenaries Only) - Kim

- Closing: IG Thz - KÜRNER

- Closing: IG VAT (Vehicular Assistance Technology) - Jang

- IG Dep (Dependability)-Not Meeting In Atl - Kohno

- TG15.12 ULI (Upper Layer Interface) - Kinney

- SCs IETF/SC-M/Rules - Kinney

- WNG – Kinney (No presentations scheduled)

- Project Related AOB – Heile

  - 802.15.3f  PAR – Ben Wolfe (15-17-0055-00-0000)

  - mmWave-channelization-channelization-CSD – Ben Wolfe (15-17-0049-00-0000)

- WG Motion: WG approval of PAR and CSD for 802.15.3f

  - Motion passes: 28-0-0

Adjourn at 11:11pm

Wednesday PM1 (January 18) (Highland Ballroom III)

Chairs called the meeting to order at 1:34pm. Attendance: 14.

Agenda for PM1 session:

- Call meeting to order - PDE purpose/description - PDE operation - Recess

Review of earlier minor revisions to Functional Decomposition Diagram

Discussion about PDE operation, specifically compression

- Visio diagram showing ULI header + Ethertype, IPv6/v4

- Examples using no header compression

- Assign a specific ULI IE ID for 6LoWPan compressed packets, to save 2 bytes

- There’s no IETF 4LoWPan, so 802.15.12 is not working on compressing IPv4 over 15.4

- 802.15.9 Transaction ID in Table 18 can be used to eliminate need for Multiplex ID

- For generic ULI, will use MPX IE. A specific ID will be assigned to 6LoWPan

Recess at 2:56pm

Wednesday PM2 (January 18) (Highland Ballroom III)

Chairs called the meeting to order at 4:10pm. Attendance: 10.

Agenda for PM2 session:

- Call meeting to order - Review week’s progress so far - MMI operation - Recess

Review week’s progress so far

- 802.15.9 fragmentation is “absorbed” into MMI layer (mandatory)

- RFC 2944 fragmentation is situated entirely within 6LoWPAN

- ULI IE ID dedicated to 6LoWPAN no EtherType required; saves two bytes

- ULI IE ID == MPX IE (2 bytes instead of transaction control + dispatch == 5 bytes)

- Since receiving device simply discards unknown IEs without error indication, discovery mechanisms are needed

- Dispatch values for each SAP

- Profile IDs to be enumerated

Recess at 4:41pm,  until ThursdayAM1

Thursday AM 1 (January 19) (Highland Ballroom III)

Chairs called the meeting to order at 8:32am. Attendance: 11.

Agenda for AM1 session:

- Call meeting to order - Review week’s progress so far - Recess

802.15.12 Header construction

- ULI IE ID (dedicated to 6LoWPAN traffic)

- MPX IE (used for all non-6LoWPAN traffic):

- Protocol Identifiers

Frame Composition

- IE-based versus payload-based

- 6LoWPan versus non-6LoWPan

Packet Construction

Recess at 8:57am

Thursday AM2 (January 19) (Highland Ballroom III)

Chair called the meeting to order at 10:31am. Attendance: 13

Agenda for AM2 session:

- Review Functional Decomposition diagram

- Ranging functional module

- Assignments of unassigned modules

- Recap on week's efforts

- Adjourn

Ranging functional module (DCN 15-17-0082-00-0000)

- Renamed from “Ranging” to “Ranging & Location Support (RLS)”

- Build upon the ideas presented in 15-16-0657-00-0012

- Localisation functional decomposition within the 802.15.12 framework

- Passive reporting

- Active sending of blink frames

- Active ranging – proposal overview

- Ranging mechanism: SS-TWR (single-sided two-way ranging)

- Ranging mechanism: DS-TWR (double-sided two-way ranging)

- Active DS-TWR

- Piggy-backed TWR (more details in next revision)

- Communicating ranging results

- Future work

- Discussion about updating the depicted function decomposition (slide 4)

- Discussion about whether all protocol modules have to use MAC and PHY primitives

- Discussion about whether solver lives in APP or else in Ranging module

- Ranging should work well with UWB, but other radios should also be possible

Meeting Accomplishments

- Discussion on the mandatory functional blocks for 802.15.12

  - Proposed management functional block details, profile IDs

  - PDE: Determined need for dispatch bytes, EtherType

  - Passthru module

  - MMI: Determined to use ULI IE, or KMP structure and MPX IE

- Discussion on the optional elements 802.15.12

  - Merged PDE fragmentation block back into 6LoWPAN

  - Ranging update (DCN 15-17-0082-00-0012)

    - renamed to be RLS (Ranging & Location Support)

Schedule

- Now targeting completion in March, 2019

Action Items

- Create a module document template

- Consider 6top as a “hands-on” effort for template creation

- Closing Plenary at 6:30pm Thursday

Adjourn at 11:44am