IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Resolutions for security clause |
| Date Submitted | February 1st, 2017 |
| Source | Marco Hernandez, Huan-Bang Li, F. Kojima (NICT) |
| Response | In response to Call for Contributions to TG8 |
| Abstract |  |
| Purpose | For discussion in TG8 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Contents

1. Security	2

1.1 Galois counter mode protocol	2

1.2 Key management	6

1.3 Extended Diffie-Hellman key agreement procedure	11

1.4 Signature scheme	15

- Security

For security, this standard uses Advanced Encryption Standard (AES) in Galois counter mode (GCM) with Galois Message Authentication Code (GMAC) henceforth named Galois counter mode protocol (GCMP), Elliptic Curve Cryptology (ECC) and Elliptic Curve Digital Signature Algorithm (ECDSA). The cryptographic public keys exchange between PDs is based on the Extended Diffie-Hellman (E-DH) protocol, which provides authentication, forward secrecy and cryptographic deniability.

For security, PDs are assumed to have already a communication link (send/receive PHY frames), where the discovery and peering procedures were successful. Henceforth such state is denominated as communication session or session. Such communication session is terminated when PDs end such communication link, like de-peering or drop links. This standard defines 3 possible secured communication sessions: unicast session, multicast session and selected broadcast session.

  - Galois counter mode protocol

GCMP provides data confidentiality and data origin authentication and integrity to the MAC payload field   MAC frame  of MAC frames.

The AES algorithm is defined in FIPS PUB 197.  All AES processing used in GCMP uses AES with a 128-bit key (GCMP-128) or a 256-bit key (GCMP-256).

GCM is a generic mode that can be used with any block-oriented encryption algorithm. GCM requires a new temporal key (TK) for every unicast, multicast, and selected broadcast session. GCM also requires a unique nonce value for each frame protected by a given temporal key, and GCMP uses a 96-bit nonce that includes a 48-bit packet number (PN) for this purpose. Reuse of a PN with the same temporal key voids all security guarantees. GCMP uses a 128-bit message integrity code (MIC).

    - Encrypted MAC frame format

GCMP processing expands the original MAC frame size by 23 octets: 7 octets for the GCMP Header field and 16 octets for the MIC field.

The GCMP Header field is constructed from the PN subfield. The 48-bit PN is represented as an array of 6 octets. PN5 is the most significant octet of the PN, and PN0 is the least significant octet. .

Figure 1—Expanded GCMP MAC frame

—Key ID field

    - GCMP cryptographic encapsulation

The GCMP cryptographic encapsulation process is depicted in Figure 2.

Figure 2—GCMP encapsulation block diagram

GCMP encrypts the MAC Payload of a plaintext MAC frame and encapsulates the resulting cipher text using the following steps:

- Increment the PN, to obtain a fresh PN for each MAC frame, so that the PN never repeats for the same temporal key. Retransmitted MAC frames shall be not modified on retransmission: MAC Header and MAC Payload are not modified; otherwise GMAC will fail.

- Use the fields in the MAC Header to construct the additional authentication data (AAD) for GCM. The GCM algorithm provides integrity protection for the fields included in the AAD. Additionally, during the Extended Diffie-Hellman key agreement protocol, the AAD shall include Additional Data (AD) as described in X.

- Construct the GCM nonce from the PN and the Source MAC Address from the MAC Header.

- Place the new PN  into the GCMP Header.

- Use the temporal key (TK), AAD, nonce, and MAC Payload to form the cipher text and Message Integrity Code (MIC).

- Form the encrypted MAC frame by combining the original MAC Header, GCMP Header, encrypted MAC Payload, MIC, and append a FCS.

      - PN processing

The PN shall be incremented by 1 for each new secured MAC frame. The PN shall never repeat for a series of encrypted MAC frames using the same temporal key.

If the PN is larger than macPNExhaustionThreshold (Table 97), an MLME-PNEXHAUSTION.indication primitive shall be generated. macPNExhastionThreshold is defined as an unsigned integer of 6 octets, whose value is implementation dependent. Vendors can choose the threshold value to which the process of refreshing keys is triggered. This avoids to introduce another exhaustion variable.

      - GCM AAD

The format of the AAD is shown Figure 3.

Figure 3—AAD format

The AAD is constructed from the MACHeader. Retransmitted MAC frames shall be not modified on retransmission: MAC Header is not modified. The MAC Header shall be present in the AAD always.

During the Extended Diffie-Hellman key agreement protocol, the AAD shall include the Additional Data (AD) field as described in X.

      - GCM nonce

The format of the GCM nonce is shown in Figure 4.

Figure 4—GCM nonce format

The Source MAC Address field value from the MAC Header. The octets of PN shall be ordered so that PN0 is at octet index 6 and PN5 is at octet index 11.

      - GMCP Header

The GCMP Header is formed by the PN as illustrated in Figure 120.

    - GCM encryption processing

GCM is a generic authenticate-and-encrypt block cipher mode, and in this standard, GCM is used with the AES block cipher.

There are four inputs to the GCM encryption processing:

- Key: the temporal key (16 octets or 32 octets) as described in X.

- Nonce: the nonce (12 octets) as described in 1.1.2.3.

- Data: the plaintext MAC Payload from a MAC frame.

- AAD: the additional authentication data  formed from the MAC Header and Additional Data (AD) during the Extended Diffie-Hellman key agreement protocol as described in X.

The GCM encryption processing provides authentication and integrity of the MAC Payload and AAD, as well as data confidentiality of the MAC Payload. The output from the GCM encryption processing consists of the encrypted data and encrypted MIC (16 octets).

The PN values sequentially number each MAC frame, where the PN is incremented by 1 for each new MAC frame. Each transmitter shall maintain a single PN (48-bit counter) for each unicast, multicast and selected broadcast session. The PN shall be implemented as a 48-bit strictly increasing integer, initialized to 1 when the corresponding temporal key is initialized or refreshed.

    - GCMP cryptographic decapsulation

The GCMP cryptographic decapsulation process is depicted in Figure 5.

Figure 5—GCMP decapsulation block diagram

GCMP decrypts the encrypted MAC Payload of a cipher text MAC frame and decapsulates a plaintext MAC frame using the following steps:

- The   cipher text MAC frame is parsed to construct the AAD and nonce.

- The AAD is formed from the MAC Header of the encrypted MAC frame, and Additional Data (AD) during the Extended Diffie-Hellman key agreement protocol as described in X

- The nonce is formed from the  Source MAC Address field of the MAC Header and PN field of the GMCP Header.

- The MIC is extracted for use in the GCM integrity checking.

- The GCM decryption processing uses the temporal key, AAD, nonce, MIC, and  encrypted MAC Payload to recover the plaintext MAC  Payload, as well as to check the integrity of the AAD and plaintext MACPayload.

- The received MAC Header and the plaintext  MAC Payload from the GCM decryption processing are concatenated to form a plaintext MAC frame.

- The decryption processing prevents replay of MAC frames by validating that the PN in the GCMP Header is greater than the replay counter (RCPN) maintained in a PD for the unicast or multicast session.

    - GCM decryption processing

GCM decryption processing shall use the same parameters as GCM encryption processing. There are five inputs to GCM recipient processing:

- TK: the temporal key (16 octets or 32 octets).

- Nonce: the nonce (12 octets) as described in 1.1.2.3.

- Encrypted Data: the encrypted MAC Payload and the MIC parsed from the encrypted MAC frame

- AAD: the AAD formed from the MAC Header, and Additional Data (AD) during the Extended Diffie-Hellman key agreement protocol as described in X.

The GCM decryption processing checks the authentication and integrity of both: the encrypted MAC Payload and AAD; as well as decrypting the encrypted MAC Payload. The plaintext MAC Payload is returned only if the MIC check is successful (the parsed MIC matches the MIC value obtained from decrypting the received encrypted MAC frame (MIC†)).

If successful, the GCM decryption processing concatenates the original MAC Header with the plaintext MAC Payload to form the plaintext MAC frame.

      - PN and replay detection

The following processing rules are used to detect replay:

- The receiver extracts the PN from the GCMP header.

- The receiver shall maintain a separate set of replay counters for each unicast, multicast and selected broadcast sessions in a PD. The receiver initializes these replay counters to 0 when a temporal key is reset. The replay counter is set to the PN value of accepted GCMP MAC frames.

- A replayed frame occurs when the PN extracted from a received MAC frame is less than or equal to the current replay counter (RCPN) value  and the receiver shall discard such frame.

  - Key management

The key agreement is performed via Elliptic-Curve Diffie-Hellman (ECDH) algorithm with digital signatures via the Elliptic-Curve Digital Signature Algorithm (ECDSA).

    - Elliptic Curve Domain Parameters

An elliptic curve over a finite field Fp is defined as

(x,y)∈Fp2|y2=x3+ax+b Mod p;  4a3+27b2≠0 Mod p∪0

where a,b∈Fp and 0 is the point at infinity (identity element required to define an Abelian group). Arithmetic operations in an EC over Fp are referred as EC arithmetic, whose definitions are out of the scope of this standard.

The domain parameters of an EC over Fp is defined as

T(p,a,b,G,n,h)

This defines public-key cryptographic schemes based on ECC.

- The odd prime number p that specifies the size of the finite field.

- The coefficients a,b of the EC equation.

- The base point G=(xG , yG) that generates the subgroup (over Mod operation).

- The prime order n of the subgroup (number of points in the subgroup).

- The cofactor h=N/n of the subgroup required to compute n. N is the No. of points in the EC.

For interoperability, this standard shall use the elliptic curves P-256 and P-384 defined in FIPS 186-4 for ECDSA and Curve 25519 defined in Error: Reference source not found for ECDH key exchange protocol.

    - Elliptic curve key pair generation

Elliptic curve key pairs should be generated as follows:

Input: Valid elliptic curve domain parameters T (p, a, b, G, n, h).

Output: An elliptic curve key pair (d, Q) associated with T.

Actions: Generate an elliptic curve key pair as follows:

1) The private key is a random integer d in the interval [1, n − 1].

2) The public key is the point Q = d G.

3) Output (d, Q).

    - Hash functions

This clause specifies the cryptographic hash functions supported in this Standard. Hash functions are used by the ECDSA.

The supported hash functions are SHA-256 and SHA-384 specified in FIPS 186-4. They map octet strings to hash values (message digest), which are octet strings of a fixed length. The security level associated with a hash function depends on its application.

Hash algorithm basic properties

| Algorithm | Message size (bits) | Message digest size (bits) |
| --- | --- | --- |
| SHA-256 | < 264 | 256 |
| SHA-384 | < 2128 | 384 |

Hash values shall be calculated as follows:

      - Hash256

Input: The input to the hash function is an octet string M.

Output: The hash value H or “error message”.

Actions:

- If message size |M| ≥ 264, then output: “error message”.

- If message size |M| < 264, then output: hash value H according to FIPS 186-4.

      - Hash384

Input: The input to the hash function is an octet string M.

Output: The hash value H or “error message”.

Actions:

- If message size |M| ≥ 2128, then output: “error message”.

- If message size |M| < 2128, then output: hash value H according to FIPS 186-4.

    - ECDH procedure

This clause describes how to derive a shared secret value between PDs with the ECDH key establishment scheme. This is part of the Extended Diffie-Hellman key agreement protocol described in clause X.

The concatenation of octet sequences X and Y is denoted as X || Y.

PD U shall employ the following process to calculate a shared secret value, Z, with PD V.

      - ECDH shared secret procedure

Input:

- Supported elliptic curve domain parameters T = (p, a, b, G, n, h)

- PD U’s private key dU

- PD V’s public key QV

Output: A shared secret EC element Z, or error indication.

Actions:

- Compute the elliptic curve pointP=(xP,yP)=hdUQV using EC arithmetic.

- If P=0 destroy P,h,dU, QV and output: “error message”.

- If P≠0 convert xP to Z using the EC element-to-octet string, destroy P and output: Z.

      - Key derivation function

Input:

- Shared secret Z

- KeyDataLength in bits (either 128 or 256).

- Hash function (either Hash256 or Hash384).

- OtherInformation= keyInfo|| PD U MAC address|| PD V MAC/Multicast address

Output: The temporal/session key K (octet string of length KeyDataLength/8), or error message.

Actions:

- If Hash = Hash256 then hashLength=256, maxHashInput=264 and set H=Hash256().

- If Hash = Hash384 then hashLength=384, maxHashInput=2128 and set H=Hash384().

- Compute a random integer c in [1,20] and set counter as a an integer with value 1.

- If ( Z || counter || OtherInformation) > maxHashInput, then output: “error message”

- Set rep=keyDataLengthhashLength

- For i=1 to rep do

Compute Hash(i)=H(Z || counter || OtherInformation).

counter=counter+1.

end

- MasterKey=Hash(1) || Hash (2) || … || Hash(rep).

- TK||KeyData=Parse(MasterKey,0,keylength)

        - Other information

OtherInformation=keyInfo|| PD U MAC address|| PD V MAC/Multicast address

        - Finite field element-to-Byte string conversion

Input: An element α in the finite field Fq.

Output: A byte string S of length n=t8 bytes, wheret=log2(q).

Actions:

- If q is an odd prime, then α must be an integer in the interval [0, q-1].

- Let (S1, S2,…, Sn) be the bytes of S from leftmost to rightmost, where

Si = (ai1, ai2, … , ai8), for i =1 to n

Assume: ai =ai1 27+ai2 26+…+ai7 2+ai8, for i =1 to n.

The bytes of S shall satisfy q=a128(n-1)+a228(n-2)+…+an-128+an.

Stop.

- Alternatively, if q = 2m, then we assume that α is already represented as a bit string of length m, with each bit indicating the coefficient (0 or 1) of a specific element of basis GF(2m) viewed as a vector space over GF(2). Let (s1, s2… sm) be the bits of α from leftmost to rightmost; and let (S1, S2,…, Sn) be the bytes of S from leftmost to rightmost. The rightmost bit sm shall become the rightmost bit of the last byte Sn, and so on until the leftmost bit s1, which shall become the (8n─ m + 1)th bit of the first byte S1. The leftmost (8n ─ m) bits of the first byte S1 shall be zero. Stop.

        - Finite element multiplicative inverse computation

Input:

- z: the value to be inverted modulo m

- m: the modulus.

Output:

- status: either “success” or “error”.

- z’: the multiplicative inverse of z Mod m, if it exists.

Actions:

- Verify that m and z are positive integers such that z < m; if not, output ERROR.

- Set i = m, j = z, y2 = 0 and y1 = 1.

- Setquotient=ij

- Set remainder=i−(j quotient)

- Set y=y2−(y1
            quotient)

- Set i=j , j=remainder , y2=y1 , y1=y

- If j > 0, then go to step 3.

- If i ≠ 1, then output ERROR

- Compute z'=y2 Mod m

- Output z’ and status=”success”.

      - HMAC-based key derivation function

Hashed-based message authentication code (HMAC)-based key derivation function is henceforth denoted as HKDF.

The concatenation of octet sequences X and Y is denoted as X || Y.

Input:

- Salt: a public or secret octet string used as key during the randomness extraction step*.

- Z: shared secret, an octet string established during the Extended Diffie Hellman public key agreement protocol.

- OtherInfo: optional context information (it can be a zero-length string).

- L: length of output key material produced by the HKDF ( ≤255*HashLen*8 ).

- Hash: the Hash function employed (SHA-256 or SHA-384) in the HMAC procedure, which is defined in FIPS 198-1**.

*The length of Salt can be of any length up to the maximum bit length permitted for input to the employed Hash function. The Salt can be a value computed from nonces exchanged as part of the key establishment protocol, a value already shared by the participant PDs, or a value pre-determined by the protocol. If there is no means to select the Salt, such Salt value shall be set to 0.

**The HMAC function has 2 inputs: an Elliptic Curve Cryptographic key and a Message. The HMAC’s output length is HashLen.

Output: Keying material (OKM) of L bits.

Actions:

K = K(1) || K(2) || K(3) || … || K(N)

where N = Ceil(L/HashLen) and the values of K(i) are defined as follows:

PRK = HMAC( Salt, Z )

K(0) = empty string of zero-length

K(1) = HMAC( PRK, K(0) || OtherInfo || 0x01 )

K(2) = HMAC( PRK, K(1) || OtherInfo || 0x02 )

⁞

K(N) = HMAC( PRK, K(N-1) || OtherInfo || N )

OKM = first L bits of K

  - Signature scheme

This clause specifies the signature schemes based on ECC supported in this standard.

The signature scheme is designed to be used by two entities:  a signer-PD U and a verifier-PD V, when U wants to send a message M in an authentic manner and V wants to verify the authenticity of M. In fact, once a message is signed, any PD V having a copy of U’s public key can verify the signature. In particular, the verifier may not be the entity to whom U originally sent the message. Such third party verification is optional and recommended when a PAC network may have access to infrastructure.

The signature scheme is described in terms of a signing operation, a verifying operation, associated setup and key deployment procedures. PDs U and V should use the schemes as follows: when they want to communicate, PDs U and V shall use the setup procedure to establish which signing option to use. Then PD U shall select an EC key pair and PD V should obtain PD U’s public key. PD U will use the key pair to control the signing operation, and PD V will use the public key to control the verifying operation. Then, each time PD U wants to send a message M, PD U should apply the signing operation to M under its key pair to obtain a signature S of message M, in order to form a signed message. Finally, when PD V receives the signed message, PD V should apply the verifying operation to the signed message under PD U’s public key to verify its authenticity. If the verifying operation is “valid”, PD V concludes the signed message is indeed authentic.

- Figure 6— ECC signature process

    - ECDSA signature procedure

Initialization:

- PD U shall establish which hash function to use when generating signatures, see clause 1.2.3. Let Hash denote the hash function and hashlen denote the length in octets of the hash values.

- PD U should establish elliptic curve domain parameters T = (p, a, b, G, n, h) at the desired security level. The elliptic curve domain parameters T should be generated using the primitive specified in clause 1.2.2.

- Entity V should obtain in an authentic manner the hash function Hash and elliptic curve domain parameters T established by PD U.

- PD U should establish an elliptic curve key pair (dU, QU) associated with T to use with the signature scheme. The key pair generation is specified in clause 1.2.2.

- PD V should obtain, in an authentic manner, the EC public key QU selected by PD U.

- PD U Shall sign messages using ECDSA using the keys and parameters established previously and the key deployment procedure as follows:

Input:  The message M as an octet string.

Output: The signature S = (r, s) on M consisting of a pair of integers r and s (each in [1, n-1]), or “error message”.

Actions:

- Compute an ephemeral EC key pair (k, R) where R = (xR, yR), associated with the elliptic curve domain parameters T established during the setup procedure, using the EC key pair generation specified in clause 1.2.2.

- Set r = xR mod n (n is the EC subgroup order associated to domain parameters T).

If r = 0 go to 1).

- Compute the hash function z=Hash(M) as specified in clause 1.2.3.

- Computes=k−1(z+dU) Mod n, where k-1 is the multiple inverse of k Mod n and dU is the private key of PD U.

- If s=0 go to 1).

- Output the pair S = (r, s) as the signature.

    - ECDSA signature verification procedure

PD V shall verify signed messages from PD U with ECDSA using the keys and parameters established during the setup procedure and the key deployment procedure

Initialization:

- Assurance of the signatory’s claimed identity, PD U.

- Assurance of the validity of the public key of PD U, QU.

- Assurance that the claimed signatory actually possessed the private key that was used to generate the digital signature at the time that the signature was generated.

Methods for the verifier to obtain these assurances are provided in SP800-89 in case the PAC network has access to infrastructure. Otherwise initialization can be skipped.

Input:

- The initialization procedure as specified in Error: Reference source not found.

- The message M as an octet string.

- PD U’s purported signature S = (r, s) on M.

Output: An indication of whether the purported signature on M is “valid” or “invalid”.

Actions:

- If r and s are not integers in the interval [1, n − 1], output “invalid” and stop.

- Use the hash function established during the initialization procedure to compute the hash value: z=Hash(M).

- Compute the integeru1=s−1z Mod n.

- Compute the integeru2=s−1r Mod n.

- Compute the EC pointP=(xP,yP)=u1G+u2QU.

- If r=xP Mod nthen output “valid”. Otherwise output “invalid”.

  - Extended Diffie-Hellman key agreement procedure

E-DH establishes a shared secret key between two parties who mutually authenticate each other based on public keys. E-DH provides forward secrecy and cryptographic deniability.

    - Notation

The concatenation of octet sequences X and Y is denoted as X || Y.

PK: Elliptic-curve public key.

Encode(PK): encodes an elliptic-curve public key PK into a octet sequence as defined in X.

DH(PK1, PK2): represents an octet sequence which is the shared secret output from an Elliptic-curve Diffie-Hellman function with public keys PK1 and PK2 (the corresponding private keys are placed in a secure location per PD).

Sign(PK, M): represents an octet sequence that is an ECDSA signature on the octet sequence M (and verifiable with public key PK) with PK’s corresponding private key, as defined in X.

HKDF: represents the HMAC-based key derivation function as defined in X.

    - Keys

The E-DH key agreement procedure uses the elliptic-curve public keys defined in Table X per PD.

Table X

| Name | Description |
| --- | --- |
| IK | Identity key |
| EK | Ephemeral key |
| SPK | Signed pre-key |
| Signature | Signature of SPK: Sign(IK, Encode(SPK)) |
| OPK | One-time pre-keys |

All public keys have a corresponding private key, but for the sake to simplify the description, private keys are omitted.

All pairwise keys shall be generated with the same elliptic-curve, chosen from EC 25519, or 256, or 384 as defined in X.

- Each PD has a long-term identity public key (IK).

- Each PD has a signed prekey (SPK), which will change periodically.

- Each PD has a set of one-time prekeys (OPKBi), which are each used only once per invocation of the E-DH procedure and delete afterwards.

- Each PD has an ephemeral key pair with public key EK, which are regenerated per invocation of the E-DH procedure.

The identity key (IK) can be generated once. A new signed prekey and prekey signature shall be generated at some time interval (once a day, or once a week) that is implementation specific. The new signed prekey and prekey signature shall replace the previous values (a PD may keep the private key corresponding to the previous signed prekey for some period of time to handle delayed messages, but after an expiration time, such pairwise keys shall be deleted). The use of one-time prekeys is optional and every one-time prekey shall be used only once per E-DH invocation and shall be deleted afterwards.

These keys shall be sent during the peering procedure and when new keys are required.

After a successful E-DH procedure, PDs will share a secret key (SK), which can be used for encrypted communication sessions or used to encrypt a symmetric key to be employed as a group key by a PAC group.

    - Sending the initial message

After successful peering procedure, or when new keys are required, a Requestor PD obtains the public keys defined in Table X from a Responder PD.

- Requestor PD verifies the Responder’s PD elliptic curve signature (ECDSA) with the Responder’s PD IK and SPK as Sign(IK, Encode(SPK)). If the verification fails, an error message shall be generate by issuing an MLME-SECURITYFAILURE.indication and MLME-COMM-STATUS.indication primitives with the Status parameter set to FAIL_SIGNATURE.

- Requestor PD generates an ephemeral key pair with public key EK.

- Requestor PD generates the following shared secret outputs from an elliptic-curve Diffie-Hellman function:

DH1=DH(IKRequestor, SPKResponder)

DH2=DH(EKRequestor, IKRespnder)

DH3= DH(EKRequestor, SPKRespnder)

DH4= DH(EKRequestor, OPKRespnder) ; if a OPKRespnder is available.

D=DH1||DH2||DH3 ; if OPKRespnder is not available.

D= DH1||DH2||DH3||DH4 ; if OPKRespnder is available.

- Requestor PD generates the shared secret key (SK) as follows:

SK=HKDF(Salt, D, OtherInfo, Hash256)

where Salt is a zero-filled octet sequence with length equal to the hash output length and OtherInfo is a pre-determined ASCII string. Hash384 is optional for stronger security.

- Requestor PD shall delete its ephemeral private key and D, DH1, DH2, DH3, and DH4 if available.

- Requestor PD generates the Associated Data as follows:

AD=Encode(IKRequestor) || Encode(IKResponder) || OtherInfo

OtherInfo is optional and contains pre-determined identifying information.

- Requestor PD constructs a Public Key Request command frame as defined in clause X. The Encrypted Data field, of the Content field, shall be cipher text corresponding to an implementation specific plaintext according to the GCMP described in X, using the AD defined in step 6)  to form the AAD field defined in X and using the SK as temporal key.

- Requestor PD sends the Public Key Request command frame to the Responder PD.

    - Receiving the initial message

Upon reception of the Public Key Request command frame, the Responder PD shall perform:

- Responder PD retrieves the Requestor’s PD IK and EK. Also, the Requestor PD retrieves its identity private key (private IK), its private signed prekey (private SPK) and its one-time private prekey (private OPK) if available.

- Responder PD generates the following shared secret outputs from an elliptic-curve Diffie-Hellman function:

DH1=DH(IKRequestor, private SPKResponder)

DH2=DH(EKRequestor, private IKRespnder)

DH3= DH(EKRequestor, private SPKRespnder)

DH4= DH(EKRequestor, private OPKRespnder) ; if a private OPKRespnder is available.

D=DH1||DH2||DH3 ; if private OPKRespnder is not available.

D= DH1||DH2||DH3||DH4 ; if private OPKRespnder is available.

- Responder PD generates the shared secret key (SK*) as follows:

SK*=HKDF(Salt, D, OtherInfo, Hash256)

where Salt is a zero-filled octet sequence with length equal to the hash output length and OtherInfo is a pre-determined ASCII string. Hash384 is optional for stronger security.

- Responder PD shall delete D, DH1, DH2, DH3, and DH4 if available

- Responder PD generates the Associated Data as follows:

AD=Encode(IKRequestor) || Encode(IKResponder) || OtherInfo

OtherInfo is optional and contains pre-determined identifying information.

- Responder PD decrypts the Encrypted Data field, of the Content field, according to the GCMP described in X, using the AD defined in step 5)  to form the AAD field defined in X and using the SK* as temporal key.

- If the initial ciphertext fails to decrypt or the GCMP authentication fails (MIC), an error message shall be generate by issuing an MLME-SECURITYFAILURE.indication and MLME-COMM-STATUS.indication primitives with the Status parameter set to the corresponding error code: FAIL or FAIL_MIC.

- If the initial ciphertext decrypts successfully and the MIC check is fine, the E-DH key agreement protocol is complete. Responder PD shall delete its private OPK for forward secrecy.

- Responder PD(s) constructs a Public Key Response command frame as defined in clause X to inform that the E-DH key agreement protocol is complete.

- Responder PD(s) sends the Public Key Response command frame to the Requestor PD.

    - Keys authentication

Before or after an E-DH key agreement procedure, PDs should compare their identity public keys IK, through some authenticated channel. For example, PDs may compare public key fingerprints manually, or by scanning a QR code. Methods for doing this are outside the scope of this standard.

    - Group key

The initiator PD constructs a symmetric key as group key when it starts a PAC group. The group key (GK) shall be sent per PD after the initiator PD and a Responder PD have successfully run the E-DH key agreement procedure.

- Compute the elliptic curve pointP=(xP,yP)=hdUQV using EC arithmetic and from supported elliptic curve domain parameters T = (p, a, b, G, n, h).

- If P=0 destroy P,h,dU, QV  and output: “error message”.

- If P≠0 convert xP to Z using the EC element-to-octet string, destroy P and output: Z.

- The group key GK is obtained as follows:

GK=HKDF(Salt, Z, OtherInfo, Hash256)

where Salt is a zero-filled octet sequence with length equal to the hash output length and OtherInfo is set to Group ID. Hash384 is optional for stronger security.

- Initiator PD constructs a Public Key Request command frame with the Key Type field set to GK and Key Data field set to the value of GK obtained in step 4).

- Initiator PD shall encrypt the Public Key Request command frame with the SK between the Initiator PD and Responder PD as temporal key TK according to the GCMP defined in clause X.

- Initiator PD sends the encrypted Public Key Request command frame to the Responder PD in the PAC Group corresponding to Group ID.

- Responder PD sends a Public Key Response command frame to confirm the successful reception of GK.

- Initiator PD repeats the steps 6) and 7) for every PD in the PAC Group.

If a new GK is required, the Initiator PD repeats the procedure from step 1), and all PDs using the old GK shall issue an MLME-DELETEKEYS.request primitive for that old group key.

      - MLME-PUBLICKEY.request

This primitive is used by a PD to transmit a public key to a specified PD or PAC group and to request the public key of the specified PD or PDs in the PAC group. The parameters of this primitive are:

MLME-PUBLICKEY.request{

Destination Address;

Multicast Address;

Request Type;

Key Descriptor;

}

Table 4—MLME-KEY.request parameters

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| Destination Address | MAC address | IEEE 48 bit MAC address | The MAC address of Responder PD. This parameter is only present in case of the connection is unicast. |
| Multicast Address | Multicast address | 2 octets | The multicast group address of the PAC group. This parameter is only present in case of the connection is multicast. |
| Request Type | Enumeration | PAIRWISE, GROUP, NEWKEYS | Indicates the type of key request. |
| Key Descriptor | As defined in Error: Reference source not found | As defined in Error: Reference source not found | Key descriptor parameters as defined in Error: Reference source not found. |

Table 5—Key Descriptor parameters

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| Key cipher suite | Unsigned char | 0: GCMP-128 1: GCMP-256 2─255: reserved | The cipher suite used for the key(s) |
| Key Type | Enumeration | Pairwise, Group | Indicates if the key is pairwise or group. |
| Elliptic curve | Unsigned char | 0: curve 25519 1: curve 256 2: curve 384 3 – 255: reserved | The standard elliptic curve used to derive the keys |
| Receive Sequence Count | Octet | 6 octets | Value to which the replay counter is initialized. |
| Key Data | String of Octets | Octets | Set of pairwise public keys or Group key |

Table 6—Key Data parameters

| Key Type | Key Data |
| --- | --- |
| Pairwise | IK, SPK, Signature, OPK, EK |
| Group | GK |

        - When generated

This primitive is generated by the MLME to send a Public Key Request command frame to another PD or PAC group. If the Request Type parameter is set to PAIRWISE, the event is generated as part of the E-DH key agreement protocol as described in clause Error: Reference source not found. If the Request Type parameter is set to GROUP, the event is generated to send a group key GK to a Responder PD as defined in clause Error: Reference source not found  (it is assumed the PDs have exchanged cryptographic public keys after sucesfull peering procedure). If the Request Type parameter is set to NEWKEYS, the event is generated to request for new public keys from a Responder PD or PAC group.

        - Effect on receipt

The MLME sends the Public Key Request command frame to the PD indicated in the Destination Address parameter, or PAC group indicated in Multicast Address parameter.

      - MLME-PUBLICKEY.response

MLME-PUBLICKEY.request{

Destination Address;

Request Type;

Key Descriptor;

Status;

}

Table 7—MLME-PUBLICKEY.response parameters

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| Destination Address | MAC address | IEEE 48 bit MAC address | The MAC address of Requestor PD. |
| Request Type | Enumeration | PAIRWISE, GROUP, NEWKEYS | Indicates the type of key request. |
| Key Descriptor | As defined in Error: Reference source not found | As defined in Error: Reference source not found | Key descriptor parameters as defined in Error: Reference source not found. |
| Status | Enumeration | SUCCESS, FAIL, FAIL_MIC | The Status of the security procedure. |

        - When generated

This primitive is generated by the MLME to send a Public Key Response command frame to another PD with MAC address in the Destination Address parameter.

If the Request Type parameter is set to PAIRWISE, the event is generated to confirm the E-DH key agreement protocol is complete with the Status parameter set to SUCCESS. The Key Descriptor parameter is empty. In case something went wrong, the correponding error code is as follows: FAIL indicates the decryption of the Encrypted Data field of the Public Key Request command frame failed. FAIL_MIC indicates the MIC check test failed.

If the Request Type parameter is set to GROUP, the event is generated to confirm the successful reception of the group key GK with the Status parameter set to SUCCESS. The Key Descriptor parameter is empty. In case something went wrong, the correponding error code is as follows: FAIL indicates the decryption of the Encrypted Data field (containing the GK) of the Public Key Request command frame failed. FAIL_MIC indicates the MIC check test failed.

If the Request Type parameter is set to NEWKEYS, the event is generated to send the public keys of the Responder PD in the Key Descriptor parameter and the Status parameter set to SUCCESS.

        - Effect on receipt

The MLME sends the Public Key Response command frame to the PD indicated in the Destination Address parameter.

    - Public Key  Request command

This command shall be sent by a PD in response to a MLME-PUBLICKEY.request primitive in order to send its cryptographic public keys to another PD or PAC group, and to obtain the public keys of such PD or PDs from a PAC group.  This command frame is part of the Extended Diffie-Hellman key agreement protocol described in clause Error: Reference source not found to send Requestor PD’s public keys defined in clause Error: Reference source not found and a probe Encrypted Data field. After the E-DH key agreement protocol has been run suscefully, the Public Key Request command frame can be used to send an encrypted group key, GK, to a Responder PD as defined in clause Error: Reference source not found.

      - Relevant MHR fields

The DAM field shall be set to either MAC address or Multicast address. The SAM field shall be set to MAC address. The AR/SNS field shall be set to Immediate Acknowledgment if the DAM field indicates MAC address. The SEC field shall be set to 0. The Destination Address field shall contain the MAC address of the Responder PD or multicast address of a PAC group. The Source address field shall contain the MAC address of the Requestor PD.

      - Relevant MAC Payload fields

The Content field shall be formatted as illustrated in Error: Reference source not found or empty depending of the Request Type parameter of the MLME-PUBLICKEY.request primitive.

| Octets: 1 | Octets: 1 | Bits: 0-1 | Bits: 2:7 | Octets: 6 | Octets: 1 | Variable | Octet: 1 | Variable |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request Type | Key Cipher Suite | Key Type | Elliptic Curve | RSC | Key Data Length | Key Data | Encrypted Data Length | Encrypted Data |

Figure 11—The generic Content field of the Public Key Request command

Table 8—Request Type field parameters

| Value | Description |
| --- | --- |
| 0 | Reserved |
| 1 | Pairwise |
| 2 | Group |
| 3 | New keys |
| 4 - 255 | Reserved |

If the Request Type field is set to 1 (Pairwise), the following fields: Key Cipher Suite, Key Type, Elliptic Curve, RSC, and Key Data shall convey the information defined in Error: Reference source not found, Key Descriptor, of the Requestor PD. The Key Data Length field contains the length in octets of the Key Data field. Such Key Data field is defined in Table 6. The Encrypted Data Length field contains the length in octets of the Encrypted Key Data field. The Encrypted Data field contains the ciphertext of a probe string of octets that is part of the Extended Diffie-Hellman key agreement protocol as defined in clause Error: Reference source not found.

If the Request Type field is set to 2 (Group), the following fields: Key Cipher Suite, Key Type, Elliptic Curve, RSC, and Key Data Length and Key Data shall be empty. The Encrypted Data Length field contains the length in octets of the Encrypted Key Data field. Such Encrypted Data field contains the encrypted group key, GK, (the Requestor PD and Responder PD had run sucesfully the E-DH key agreement protocol previously).

On the other hand, if the Request Type field is set to 3 (New keys), the Content Field shall contain only the Request Type field. The intention is to request for new public keys from the Responder PD or PAC group.

    - Public Key Response command

This command shall be sent by the Responder PD(s) in response to a MLME-PUBLICKEY.response primitive in order to send the requested public key to the Requestor PD, or the conformation the E-DH key agreement protocol is complete or the confirmation of the reception of a group key, GK.

      - Relevant MHR fields

The DAM field shall be set to MAC address. The SAM field shall be set to MAC address. The AR/SNS field shall be set immediate acknowledgment. The SEC field shall be set to 0. The Destination address field shall contain the MAC address of the Requestor PD. The Source address field shall contain the MAC address of the Responder PD.

      - Relevant MAC Payload fields

The Content field shall be formatted as illustrated in Figure 12.

| Octets: 1 | Octets: 1 | Bits: 0-1 | Bits: 2-7 | Octets: 1 | Variable | Octets: 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Request Type | Key Cipher Suite | Key Type | Elliptic Curve | Key Data Length | Key Data | Status |

Figure 12—The Content field of the Public Key Response command

Table 9—Status field parameters

| Status | Description |
| --- | --- |
| 0 | Reserved |
| 1 | Success |
| 2 | Decryption failed |
| 3 | MIC check failed |
| 4 - 255 | Reserved |

If the Request Type field is set to 1 (Pairwise), the following fields: Key Cipher Suite, Key Type, Elliptic Curve, Key Data Length and Key Data shall be empty. The Status field is set to 1 if the E-DH key agreement protocol is completed successfully. The Status field is set to 2 if the decryption of the Encrypted Data field of the Public Key Request command frame failed. The Status field is set to 3 if the corresponding MIC check test failed.

If the Request Type field is set to 2 (Group), the following fields: Key Cipher Suite, Key Type, Elliptic Curve, Key Data Length and Key Data shall be empty. The Status field is set to 1 if group key GK was received successfully. The Status field is set to 2 if the decryption of the Encrypted Data field of the Public Key Request command frame failed. The Status field is set to 3 if the corresponding MIC check test failed.

If the Request Type field is set to 3 (New keys), the following fields: Key Cipher Suite, Key Type, Elliptic Curve, and Key Data shall convey the information defined in Error: Reference source not found, Key Descriptor, of the Responder PD. The Key Data Length field contains the length in octets of the Key Data field. Such Key Data field is defined in Table 6. The Status field is set to 1.