IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Companion Document for comments resolution |
| Date Submitted | [August 2017] |
| Source | John Li (Huawei) |
| Re: |  |
| Abstract | Discussion on D0 comments’ resolution revision |
| Purpose | Resolution for D0 technical comments revision |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Abstract: A PHY and a MAC layer for short-range optical wireless communications in optically transparent media are defined using the light wavelength from 10,000 nm to 190 nm where transmitting and receiving devices incorporate light emitting sources and photodiodes as well as digital cameras with a lens and an image sensor. The standard is capable of delivering data rates sufficient to support audio and video multimedia services and robustness as well as low latency for communications between machines and also considers mobility of the optical link, compatibility with various light infrastructures, impairments due to noise and interference from sources like ambient light and a MAC layer that accommodates the unique needs of digital communications using the light. The standard adheres to applicable eye safety regulations.

Keywords: IEEE 802.15.13, laser diode, LD, LED, light-emitting diode, short-range optical wireless communications, visible light, visible-light communication, VLC, OWC, LiFi, discrete multitoned, DMT, orthogonal frequency-division multiplex, OFDM, adaptive multiple subcarrier transmission, multiple-input multiple-output, MIMO, coordinated wireless network, optical camera communications, OCC

The Institute of Electrical and Electronics Engineers, Inc.

3 Park Avenue, New York, NY 10016-5997, USA

Copyright © 20xx by the Institute of Electrical and Electronics Engineers, Inc.

All rights reserved. Published 6 September 2011. Printed in the United States of America.

IEEE and 802 are registered trademarks in the U.S. Patent & Trademark Office, owned by The Institute of Electrical and Electronics Engineers, Incorporated.

PDF:	ISBN 978-0-7381-6665-0	STD97117

Print:	ISBN 978-0-7381-6666-7	STDPD97117

IEEE prohibits discrimination, harassment and bullying. For more information, visit http://www.ieee.org/web/aboutus/whatis/policies/p9-26.html.

No part of this publication may be reproduced in any form, in an electronic retrieval system or otherwise, without the prior written permission

of the publisher.

IEEE Standards documents are developed within the IEEE Societies and the Standards Coordinating Committees of the IEEE Standards Association (IEEE-SA) Standards Board. The IEEE develops its standards through a consensus development process, approved by the American National Standards Institute, which brings together volunteers representing varied viewpoints and interests to achieve the final product. Volunteers are not necessarily members of the Institute and serve without compensation. While the IEEE administers the process and establishes rules to promote fairness in the consensus development process, the IEEE does not independently evaluate, test, or verify the accuracy of any of the information or the soundness of any judgments contained in its standards.

Use of an IEEE Standard is wholly voluntary. The IEEE disclaims liability for any personal injury, property or other damage, of any nature whatsoever, whether special, indirect, consequential, or compensatory, directly or indirectly resulting from the publication, use of, or reliance upon this, or any other IEEE Standard document.

The IEEE does not warrant or represent the accuracy or content of the material contained herein, and expressly disclaims any express or implied warranty, including any implied warranty of merchantability or fitness for a specific purpose, or that the use of the material contained herein is free from patent infringement. IEEE Standards documents are supplied “AS IS.”

The existence of an IEEE Standard does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to the scope of the IEEE Standard. Furthermore, the viewpoint expressed at the time a standard is approved and issued is subject to change brought about through developments in the state of the art and comments received from users of the standard. Every IEEE Standard is subjected to review at least every five years for revision or reaffirmation, or every ten years for stabilization. When a document is more than five years old and has not been reaffirmed, or more than ten years old and has not been stabilized, it is reasonable to conclude that its contents, although still of some value, do not wholly reflect the present state of the art. Users are cautioned to check to determine that they have the latest edition of any IEEE Standard.

In publishing and making this document available, the IEEE is not suggesting or rendering professional or other services for, or on behalf of, any person or entity. Nor is the IEEE undertaking to perform any duty owed by any other person or entity to another. Any person utilizing this, and any other IEEE Standards document, should rely upon the advice of a competent professional in determining the exercise of reasonable care in any given circumstances.

Interpretations: Occasionally questions may arise regarding the meaning of portions of standards as they relate to specific applications. When the need for interpretations is brought to the attention of IEEE, the Institute will initiate action to prepare appropriate responses. Since IEEE Standards represent a consensus of concerned interests, it is important to ensure that any interpretation has also received the concurrence of a balance of interests.For this reason, IEEE and the members of its societies and Standards Coordinating Committees are not able to provide an instant response to interpretation requests except in those cases where the matter has previously received formal consideration. A statement, written or oral, that is not processed in accordance with the IEEE -SA Standards Board Operations Manual shall not be considered the official position of IEEE or any of its committees and shall not be considered to be, nor be relied upon as, a formal interpretation of the IEEE.At lectures, symposia, seminars, or educational courses, an individual presenting information on IEEE standards shall make it clear that his or her views should be considered the personal views of that individual rather than the formal position, explanation, or interpretation of the IEEE. Comments for revision of IEEE Standards are welcome from any interested party, regardless of membership affiliation with IEEE. Suggestions for changes in documents should be in the form of a proposed change of text, together with appropriate supporting comments. Recommendations to change the status of a stabilized standard should include a rationale as to why a revision or withdrawal is required.

Comments and recommendations on standards, and requests for interpretations should be addressed to:

Secretary, IEEE-SA Standards Board

445 Hoes Lane

Piscataway, NJ 08854

USA

Authorization to photocopy portions of any individual standard for internal or personal use is granted by the Institute of Electrical and Electronics Engineers, Inc., provided that the appropriate fee is paid to Copyright Clearance Center. To arrange for payment of licensing fee, please contact Copyright Clearance Center, Customer Service, 222 Rosewood Drive, Danvers, MA 01923 USA; +1 978 750 8400. Permission to photocopy portions of any individual standard for educational classroom use can also be obtained through the Copyright Clearance Center.

Introduction

This introduction is not part of IEEE Std 802.15.13-2011, IEEE Standard for Local and metropolitan area networks— Part 15.13: Multi Giga bit/sec Optical Wireless Communications

Visible-light communication (VLC) transmits data by intensity modulating optical sources, such as light-emitting diodes (LEDs) and laser diodes (LDs), faster than the persistence of the human eye. VLC merges lighting and data communications in applications such as area lighting, signboards, streetlights, vehicles, traffic signals, and status indicators. This standard describes the use of VLC for wireless personal area networks (WPAN) and covers topics such as network topologies, addressing, collision avoidance, acknowledgement, performance quality indication, dimming support, visibility support, colored status indication and color-stabilization.

Notice to users

Laws and regulations

Users of these documents should consult all applicable laws and regulations. Compliance with the provisions of this standard does not imply compliance to any applicable regulatory requirements. Implementers of the standard are responsible for observing or referring to the applicable regulatory requirements. IEEE does not, by the publication of its standards, intend to urge action that is not in compliance with applicable laws, and these documents may not be construed as doing so.

Copyrights

This document is copyrighted by the IEEE. It is made available for a wide variety of both public and private uses. These include both use, by reference, in laws and regulations, and use in private self-regulation, standardization, and the promotion of engineering practices and methods. By making this document available for use and adoption by public authorities and private users, the IEEE does not waive any rights in copyright to this document.

Updating of IEEE documents

Users of IEEE standards should be aware that these documents may be superseded at any time by the issuance of new editions or may be amended from time to time through the issuance of amendments, corrigenda, or errata. An official IEEE document at any point in time consists of the current edition of the document together with any amendments, corrigenda, or errata then in effect. In order to determine whether a given document is the current edition and whether it has been amended through the issuance of amendments, corrigenda, or errata, visit the IEEE Standards Association website at http:// ieeexplore.ieee.org/xpl/standards.jsp, or contact the IEEE at the address listed previously.

For more information about the IEEE Standards Association or the IEEE standards development process, visit the IEEE-SA website at http://standards.ieee.org.

iv	Copyright © 20xx IEEE. All rights reserved.

Errata

Errata, if any, for this and all other standards can be accessed at the following URL: http:// standards.ieee.org/reading/ieee/updates/errata/index.html. Users are encouraged to check this URL for errata periodically.

Interpretations

Current interpretations can be accessed at the following URL: http://standards.ieee.org/reading/ieee/interp/ index.html.

Patents

Attention is called to the possibility that implementation of this amendment may require use of subject matter covered by patent rights. By publication of this amendment, no position is taken with respect to the existence or validity of any patent rights in connection therewith. The IEEE is not responsible for identifying Essential Patent Claims for which a license may be required, for conducting inquiries into the legal validity or scope of Patents Claims or determining whether any licensing terms or conditions provided in connection with submission of a Letter of Assurance, if any, or in any licensing agreements are reasonable or non-discriminatory. Users of this amendment are expressly advised that determination of the validity of any patent rights, and the risk of infringement of such rights, is entirely their own responsibility. Further information may be obtained from the IEEE Standards Association.

Copyright © 20xx IEEE. All rights reserved.	v

Participants

At the time this amendment was submitted to the IEEE-SA for approval, the IEEE P802.15.13 Working Group had the following membership:

Robert F. Heile, Chair

Rick Alfvin, Co-Vice Chair

Patrick W. Kinney, Co-Vice Chair and Secretary

James P. K. Gilb, Working Group Technical Editor

Yeong Min Jang, IEEE 802.15.7 Chair

Hideki Aoyama, Rick Roberts, Yu Zeng , IEEE 802.15.7 Vice Chairs

Richard Roberts, Dobroslav Tsonev, Jaesang Cha, Murat Uysal, IEEE 802.15.7 Technical Editors

TBD

vi	Copyright © 20xx IEEE. All rights reserved.

Major technical contributions were received from the following individuals:

TBD

Copyright © 20xx IEEE. All rights reserved.	vii

The following members of the individual balloting committee voted on this standard. Balloters may have voted for approval, disapproval, or abstention.

TBD

viii	Copyright © 20xx IEEE. All rights reserved.

When the IEEE-SA Standards Board approved this standard on TBD, it had the following membership:

Richard H. Hulett, Chair

John Kulick, Vice Chair

Robert M. Grow, Past Chair

Judith Gorman, Secretary

TBD

*Member Emeritus

Also included are the following nonvoting IEEE-SA Standards Board liaisons:

Satish Aggarwal, NRC Representative

Richard DeBlasio, DOE Representative

Michael Janezic, NIST Representative

Catherine Berger

IEEE Project Editor

Patricia Gerdon

IEEE Standards Program Manager, Technical Program Development

Copyright © 20xx IEEE. All rights reserved.	ix

x	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1.Overview | ....................................................................................................................................................... |  | 1 |
| --- | --- | --- | --- |
| 1.1 | Scope............................................................................................................................................ |  | 1 |
| 1.2 | Purpose......................................................................................................................................... |  | 1 |
| 2.Normative references .................................................................................................................................... |  |  | 2 |
| 3.Definitions, acronyms, and abbreviations2 |  |  |  |
| 3.1 | Definitions ................................................................................................................................... |  | 2 |
| 3.2 | Acronyms and abbreviations ....................................................................................................... |  | 6 |
| 4.General description ....................................................................................................................................... |  |  | 8 |
| 4.1 | Introduction.................................................................................................................................. |  | 8 |
| 4.2 | Network topologies...................................................................................................................... |  | 9 |
| 4.2.1 |  | Peer-to-peer topology .................................................................................................... | 11 |
| 4.2.2 |  | Star topology.................................................................................................................. | 11 |
| 4.2.3 |  | Broadcast topology ........................................................................................................ | 11 |
| 4.2.4 |  | Coordinated topology .................................................................................................... | 11 |
| 4.2.5 |  | Relay functionality......................................................................................................... | 11 |
| 4.2.6 |  | Heterogeneous RF & OWC functionality...................................................................... | 12 |
| 4.3 | Modulation-domain spectrum.................................................................................................... |  | 13 |
| 4.4 | Architecture ............................................................................................................................... |  | 13 |
| 4.4.1 |  | PHY layer ...................................................................................................................... | 14 |
| 4.4.2 |  | MAC sublayer................................................................................................................ | 21 |
| 4.4.3 |  | Dimming and flicker-mitigation support ....................................................................... | 21 |
| 4.5 | Functional overview .................................................................................................................. |  | 29 |
| 4.5.1 |  | Superframe structure...................................................................................................... | 29 |
| 4.5.2 |  | Data transfer model........................................................................................................ | 30 |
| 4.5.3 |  | Clock-rate selection ....................................................................................................... | 31 |
| 4.5.4 |  | Frame structure .............................................................................................................. | 32 |
| 4.5.5 |  | Improving probability of successful delivery ................................................................ | 32 |
| 4.6 | Security ...................................................................................................................................... |  | 33 |
| 4.7 | Concept of primitives................................................................................................................. |  | 33 |
| 5.MAC protocol specification........................................................................................................................ |  |  | 35 |
| 5.1 | MAC functional description ...................................................................................................... |  | 35 |
| 5.1.1 |  | Superframe structure...................................................................................................... | 36 |
| 5.1.2 |  | Channel access ............................................................................................................... | 42 |
| 5.1.3 |  | Starting a OWPAN ........................................................................................................ | 51 |
| 5.1.4 |  | Neighboring OWPAN status monitoring....................................................................... | 56 |
| 5.1.5 |  | Maintaining OWPANs................................................................................................... | 56 |
| 5.1.6 |  | Association and disassociation ...................................................................................... | 60 |
| 5.1.7 |  | Synchronization ............................................................................................................. | 63 |
| 5.1.8 |  | Transaction handling...................................................................................................... | 64 |
| 5.1.9 |  | Transmission, reception, and acknowledgment ............................................................. | 65 |
| 5.1.10 |  | GTS allocation and management ................................................................................... | 72 |
| 5.1.11 |  | Fast link recovery........................................................................................................... | 77 |
| 5.1.12 |  | Multiple channel resource assignment........................................................................... | 81 |
| 5.1.13 |  | OWC cell design and mobility support.......................................................................... | 84 |
| 5.1.14 |  | CSI feedback and link adaptation .................................................................................. | 88 |

- Relaying, ........................................................................................................................ 89

5.1.16	Color stabilization.......................................................................................................... 93

| Copyright © 20xx IEEE. All rights reserved. | 0 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 5.1.17 |  | Visibility and dimming support ..................................................................................... | 93 |
| --- | --- | --- | --- |
| 5.1.18 |  | Interference coordination ............................................................................................... | 98 |
| 5.1.19 |  | Mobility and handover................................................................................................. | 100 |
| 5.2 | MAC frame formats................................................................................................................. |  | 103 |
| 5.2.1 |  | General MAC frame format......................................................................................... | 103 |
| 5.2.2 |  | Format of individual frame types................................................................................. | 107 |
| 5.3 | MAC frame formats................................................................................................................. |  | 117 |
| 5.3.1 |  | Conventions ................................................................................................................. | 118 |
| 5.3.2 |  | General frame format................................................................................................... | 118 |
| 5.3.3 |  | Frame fields ................................................................................................................. | 118 |
| 5.3.4 |  | Specific Frame Formats ............................................................................................... | 122 |
| 5.4 | MAC command frames............................................................................................................ |  | 137 |
| 5.4.1 |  | Association request command ..................................................................................... | 138 |
| 5.4.2 |  | Association response command................................................................................... | 139 |
| 5.4.3 |  | Disassociation notification command .......................................................................... | 140 |
| 5.4.4 |  | Data request command................................................................................................. | 141 |
| 5.4.5 |  | Scan-over-backhaul request command ........................................................................ | 142 |
| 5.4.6 |  | Scan-over-backhaul confirmation command ............................................................... | 143 |
| 5.4.7 |  | OWPAN ID conflict notification command ................................................................ | 143 |
| 5.4.8 |  | Neighboring OWPAN report request command.......................................................... | 144 |
| 5.4.9 |  | Neighboring OWPAN report indication command ..................................................... | 144 |
| 5.4.10 |  | Beacon request command ............................................................................................ | 144 |
| 5.4.11 |  | Additional beacon request command........................................................................... | 144 |
| 5.4.12 |  | Coordinator realignment command ............................................................................. | 146 |
| 5.4.13 |  | GTS request command................................................................................................. | 148 |
| 5.4.14 |  | Blinking notification command ................................................................................... | 149 |
| 5.4.15 |  | Dimming notification command .................................................................................. | 150 |
| 5.4.16 |  | Fast link recovery command........................................................................................ | 150 |
| 5.4.17 |  | Mobility notification command ................................................................................... | 151 |
| 5.4.18 |  | GTS response command .............................................................................................. | 151 |
| 5.4.19 |  | Clock rate change notification command .................................................................... | 152 |
| 5.4.20 |  | Multiple channel assignment command ...................................................................... | 153 |
| 5.4.21 |  | Color stabilization timer notification command .......................................................... | 153 |
| 5.4.22 |  | Color stabilization information command ................................................................... | 153 |
| 5.4.23 |  | CVD disable command ................................................................................................ | 154 |
| 5.4.24 |  | CSI Control Frame....................................................................................................... | 154 |
| 5.4.25 |  | Dimming Control Frame.............................................................................................. | 156 |
| 5.4.26 |  | Information element command .................................................................................... | 156 |
| 6.MAC sublayer service specification ......................................................................................................... |  |  | 164 |
| 6.1 | Overview.................................................................................................................................. |  | 164 |
| 6.2 | MAC data service .................................................................................................................... |  | 165 |
| 6.2.1 |  | MCPS-DATA.request.................................................................................................. | 165 |
| 6.2.2 |  | MCPS-DATA.confirm................................................................................................. | 169 |
| 6.2.3 |  | MCPS-DATA.indication ............................................................................................. | 170 |
| 6.2.4 |  | MCPS-PURGE.request................................................................................................ | 172 |
| 6.2.5 |  | MCPS-PURGE.confirm............................................................................................... | 173 |
| 6.2.6 |  | Data service message sequence chart .......................................................................... | 173 |
| 6.2.7 |  | Primitives for heterogeneous network ......................................................................... | 174 |
| 6.3 | MAC management service....................................................................................................... |  | 177 |
| 6.3.1 |  | Association primitives ................................................................................................. | 177 |
| 6.3.2 |  | Disassociation primitives ............................................................................................. | 186 |
| 6.3.3 |  | Beacon notification primitive ...................................................................................... | 192 |

1	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 6.3.4 |  | Additional beacon primitives....................................................................................... | 195 |
| --- | --- | --- | --- |
| 6.3.5 |  | Primitives for reading PIB attributes ........................................................................... | 197 |
| 6.3.6 |  | GTS management primitives ....................................................................................... | 199 |
| 6.3.7 |  | Primitives for resetting the MAC sublayer .................................................................. | 204 |
| 6.3.8 |  | Primitives for specifying the receiver enable time ...................................................... | 206 |
| 6.3.9 |  | Primitives for channel scanning................................................................................... | 209 |
| 6.3.10 |  | Neighbor report primitives........................................................................................... | 214 |
| 6.3.11 |  | Communication status primitive .................................................................................. | 217 |
| 6.3.12 |  | Primitives for writing PIB attributes............................................................................ | 221 |
| 6.3.13 |  | Primitives for updating the superframe configuration ................................................. | 223 |
| 6.3.14 |  | Primitive for synchronizing with a coordinator ........................................................... | 227 |
| 6.3.15 |  | Primitive for synchronization loss with a coordinator................................................. | 229 |
| 6.3.16 |  | Primitives for requesting data from a coordinator ....................................................... | 233 |
| 6.4 MAC constants and PIB attributes........................................................................................... |  |  | 235 |
| 6.4.1 |  | MAC constants ............................................................................................................ | 235 |
| 6.4.2 |  | MAC PIB attributes ..................................................................................................... | 237 |
| 6.4.3 |  | MAC PIB Attributes .................................................................................................... | 248 |
| 6.5 | Optical-clock-rate selection ..................................................................................................... |  | 248 |
| 6.5.1 |  | Optical-clock-rate selection for P2P topology............................................................. | 248 |
| 6.5.2 |  | Optical-clock-rate selection for star topology ............................................................. | 251 |
| 6.5.3 |  | Clock-rate selection for multicast topology................................................................. | 253 |
| 6.6 Message sequence charts illustrating MAC-PHY interaction ................................................. |  |  | 255 |
| 7.Security suite specifications...................................................................................................................... |  |  | 263 |
| 7.1 | Overview.................................................................................................................................. |  | 263 |
| 7.2 | Functional description.............................................................................................................. |  | 264 |
| 7.2.1 |  | Outgoing frame security procedure ............................................................................. | 264 |
| 7.2.2 |  | Outgoing frame key retrieval procedure ...................................................................... | 265 |
| 7.2.3 |  | Incoming frame security procedure ............................................................................. | 266 |
| 7.2.4 |  | Incoming frame security material retrieval procedure................................................. | 267 |
| 7.2.5 |  | Key descriptor lookup procedure................................................................................. | 269 |
| 7.2.6 |  | Blacklist checking procedure....................................................................................... | 269 |
| 7.2.7 |  | Device descriptor lookup procedure ............................................................................ | 269 |
| 7.2.8 |  | Incoming security level checking procedure ............................................................... | 270 |
| 7.2.9 |  | Incoming key usage policy checking procedure.......................................................... | 270 |
| 7.3 | Security operations .................................................................................................................. |  | 270 |
| 7.3.1 |  | Integer and octet representation ................................................................................... | 271 |
| 7.3.2 |  | CCM* nonce ................................................................................................................ | 271 |
| 7.3.3 |  | CCM* prerequisites ..................................................................................................... | 271 |
| 7.3.4 |  | CCM* transformation data representation................................................................... | 271 |
| 7.3.5 |  | CCM* inverse transformation data representation ...................................................... | 273 |
| 7.4 | Auxiliary Security header ........................................................................................................ |  | 274 |
| 7.4.1 |  | Integer and octet representation ................................................................................... | 274 |
| 7.4.2 |  | Security Control field................................................................................................... | 274 |
| 7.4.3 |  | Frame Counter field ..................................................................................................... | 276 |
| 7.4.4 |  | Key Identifier field....................................................................................................... | 276 |
| 7.5 Security-related MAC PIB attributes....................................................................................... |  |  | 277 |
| 7.5.1 |  | PIB security material ................................................................................................... | 277 |
| 7.5.2 |  | Key table ...................................................................................................................... | 280 |
| 7.5.3 |  | Device table ................................................................................................................. | 281 |
| 7.5.4 |  | Minimum security level table ...................................................................................... | 281 |
| 7.5.5 |  | Frame counter .............................................................................................................. | 281 |
| 7.5.6 |  | Automatic request attributes ........................................................................................ | 281 |

| Copyright © 20xx IEEE. All rights reserved. | 2 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 7.5.7 |  | Default key source ....................................................................................................... | 281 |
| --- | --- | --- | --- |
| 7.5.8 |  | Coordinator address ..................................................................................................... | 282 |
| 8.PHY layer specification ............................................................................................................................ |  |  | 282 |
| 8.1 | Overview.................................................................................................................................. |  | 282 |
| 8.2 | Operating modes ...................................................................................................................... |  | 282 |
| 8.3 | General requirements ............................................................................................................... |  | 286 |
| 8.3.1 |  | Wavelength band plan ................................................................................................. | 286 |
| 8.3.2 |  | Optical mapping........................................................................................................... | 286 |
| 8.3.3 |  | Maximum error tolerance for multiple optical sources ............................................... | 287 |
| 8.3.4 |  | Minimum LIFS, SIFS, and RIFS periods .................................................................... | 288 |
| 8.3.5 |  | TX-to-RX turnaround time .......................................................................................... | 288 |
| 8.3.6 |  | RX-to-TX turnaround time .......................................................................................... | 288 |
| 8.3.7 |  | Transmit data clock frequency tolerance ..................................................................... | 288 |
| 8.3.8 |  | Wavelength quality indicator (WQI) ........................................................................... | 288 |
| 8.3.9 |  | Clear channel assessment (CCA)................................................................................. | 289 |
| 8.4 | Data modes .............................................................................................................................. |  | 289 |
| 8.5 | Dimming and flicker mitigation .............................................................................................. |  | 290 |
| 8.5.1 |  | Dimming during idle time............................................................................................ | 291 |
| 8.5.2 |  | Dimming during data transmission time...................................................................... | 292 |
| 8.5.3 |  | Flicker mitigation......................................................................................................... | 295 |
| 8.6 | PPDU format............................................................................................................................ |  | 295 |
| 8.6.1 |  | Preamble field .............................................................................................................. | 296 |
| 8.6.2 |  | PHY header.................................................................................................................. | 297 |
| 8.6.3 |  | Header check sequence (HCS) .................................................................................... | 300 |
| 8.6.4 |  | Optional fields.............................................................................................................. | 300 |
| 8.6.5 |  | PSDU field ................................................................................................................... | 301 |
| 8.7 | PPDU Format........................................................................................................................... |  | 302 |
| 8.7.1 |  | PPDU format for low bandwidth PHY ........................................................................ | 302 |
| 8.7.2 |  | PPDU Format for high bandwidth PHY ...................................................................... | 312 |
| 9.PHY service specifications ....................................................................................................................... |  |  | 318 |
| 9.1 | Overview.................................................................................................................................. |  | 318 |
| 9.2 | PHY management service........................................................................................................ |  | 318 |
| 9.2.1 |  | PLME-CCA.request..................................................................................................... | 319 |
| 9.2.2 |  | PLME-CCA.confirm.................................................................................................... | 319 |
| 9.2.3 |  | PLME-GET.request ..................................................................................................... | 320 |
| 9.2.4 |  | PLME-GET.confirm .................................................................................................... | 320 |
| 9.2.5 |  | PLME-SET.request...................................................................................................... | 321 |
| 9.2.6 |  | PLME-SET.confirm..................................................................................................... | 322 |
| 9.2.7 |  | PLME-SET-TRX-STATE.request............................................................................... | 323 |
| 9.2.8 |  | PLME-SET-TRX-STATE.confirm.............................................................................. | 323 |
| 9.2.9 |  | PLME-SWITCH.request.............................................................................................. | 324 |
| 9.2.10 |  | PLME-SWITCH.confirm ............................................................................................ | 325 |
| 9.3 | PHY data service ..................................................................................................................... |  | 325 |
| 9.3.1 |  | PD-DATA.request ....................................................................................................... | 325 |
| 9.3.2 |  | PD-DATA.confirm ...................................................................................................... | 326 |
| 9.3.3 |  | PD-DATA.indication ................................................................................................... | 327 |
| 9.4 | PHY enumeration description.................................................................................................. |  | 331 |
| 9.5 | PHY constants and PIB attributes............................................................................................ |  | 331 |
| 9.5.1 |  | PHY constants.............................................................................................................. | 332 |
| 9.5.2 |  | PHY PIB attributes ...................................................................................................... | 333 |

3	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

10.PHY II specifications	336

10.1 Reference modulator diagram	337

10.2 Forward error correction encoder	337

10.3 Run-length limiting encoder	338

10.4 Data mapping for VPPM	338

11.PHY VII Low-bandwidth PHY	338

11.1 Adaptive OFDM Concept	338

11.2 Waveform	338

11.2.1	Adaptive OFDM signal generation	338

11.2.2	Carrier mapping	338

11.2.3	IDFT	339

11.2.4	Cyclic prefix	340

11.2.5	Subcarrier modulation mapping	340

11.2.6	Single-carrier modulation	342

11.2.7	Unipolar modulation	343

11.2.8	Dimming	349

11.3 MIMO	351

11.3.1	Modified frame structure	351

11.3.2	Reference symbols for MIMO channel estimation	352

11.3.3	MIMO transmission modes	354

11.4 Channel coding	355

11.4.1	Channel coding for the header	356

11.4.2	Channel coding for the data	356

11.4.3	Data interleaving	358

11.5 Relaying	359

11.5.1	Amplify and forward	359

11.5.2	Decode and forward	360

11.6 PHY Service Specifications	362

11.6.1	Overview	362

11.6.2	PHY Management Service	362

12.PHY VIII High-bandwidth PHY	369

12.1 Adaptive OFDM	369

12.1.1	Adaptive OFDM Waveform	370

12.1.2	Single-carrier modulation	371

12.1.3	Unipolar modulation	375

12.1.4	Multiple-input multiple-output	378

12.1.5	Coordinated wireless network	384

12.1.6	Relaying	385

12.1.7	Payload data	385

12.1.8	Data scrambling	386

Annex A	389

A.1	General	389

A.2	Regulatory documents	390

Annex B	393

B.1	IEEE 802.2 convergence sublayer	393

B.1.1	MA-UNITDATA.request	393

Copyright © 20xx IEEE. All rights reserved.	4

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  | B.1.2 | MA-UNITDATA.indication ............................................................................................ | 394 |  |
| --- | --- | --- | --- | --- | --- |
| 2 |  | B.1.3 | MA-UNITDATA-STATUS.indication............................................................................ | 395 |  |
| 3 | Annex C |  |  | 397 |  |
| 4 |  | ...................................................................................................................................................... |  |  |  |
| 5 | Annex D |  |  | 398 |  |
| 6 |  | ...................................................................................................................................................... |  |  |  |
| 7 | Annex E |  |  | 404 |  |
| 8 |  | ...................................................................................................................................................... |  |  |  |
| 9 |  | E.1 Introduction—Dynamic displays vs. addressed displays |  | 404 |  |
| 10 |  | ................................................ |  |  |  |
| 11 |  | E.2 | Dynamic displays............................................................................................................. | 404 |  |
| 12 |  | E.2.1 | Operation mechanism ...................................................................................................... | 404 |  |
| 13 |  | E.2.2 Reduced brightness mitigation on OWC dynamic displays ............................................ |  | 405 |  |
| 14 |  | E.2.3 OWC application using dynamic displays....................................................................... |  | 406 |  |
| 15 |  | E.3 | Addressed displays .......................................................................................................... | 407 |  |
| 16 |  | E.3.1 LCD display using LED backlighting modulation .......................................................... |  | 407 |  |
| 17 |  | E.3.2 | LED pixel modulation ..................................................................................................... | 407 |  |
| 18 | Annex F |  |  | 409 |  |
| 19 |  | ....................................................................................................................................................... |  |  |  |
| 20 | Annex G |  |  | 413 |  |
| 21 |  | ...................................................................................................................................................... |  |  |  |
| 22 | Annex H |  |  | 414 |  |
| 23 |  | ...................................................................................................................................................... |  |  |  |
| 24 |  |  |  |  |  |
| 25 |  |  |  |  |  |
| 26 |  |  |  |  |  |
| 27 |  |  |  |  |  |
| 28 |  |  |  |  |  |
| 29 |  |  |  |  |  |
| 30 |  |  |  |  |  |
| 31 |  |  |  |  |  |
| 32 |  |  |  |  |  |
| 33 |  |  |  |  |  |
| 34 |  |  |  |  |  |
| 35 |  |  |  |  |  |
| 36 |  |  |  |  |  |
| 37 |  |  |  |  |  |
| 38 |  |  |  |  |  |
| 39 |  |  |  |  |  |
| 40 |  |  |  |  |  |
| 41 |  |  |  |  |  |
| 42 |  |  |  |  |  |
| 43 |  |  |  |  |  |
| 44 |  |  |  |  |  |
| 45 |  |  |  |  |  |
| 46 |  |  |  |  |  |
| 47 |  |  |  |  |  |
| 48 |  |  |  |  |  |
| 49 |  |  |  |  |  |
| 50 |  |  |  |  |  |
| 51 |  |  |  |  |  |
| 52 |  |  |  |  |  |
| 53 |  |  |  |  |  |
| 54 |  |  |  |  |  |
|  | 5 |  | Copyright © 20xx IEEE. All rights reserved. |  |  |
|  |  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |

IEEE Standard for

Local and metropolitan area networks —

Part 15.13: Multi Giga bit/s ec Optical Wireles s Communications

IMPORTANT NOTICE: This standard is not intended to ensure safety, security, health, or environmental protection. Implementers of the standard are responsible for determining appropriate safety, security, environmental, and health practices or regulatory requirements.

This IEEE document is made available for use subject to important notices and legal disclaimers. These notices and disclaimers appear in all publications containing this document and may be found under the heading “Important Notice” or “Important Notices and Disclaimers Concerning IEEE Documents.” They can also be obtained on request from IEEE or viewed at http://standards.ieee.org/IPR/disclaimers.html.

1. Overview

1.1 Scope

This standard defines a PHY and MAC layer for short-range optical wireless communications in optically transparent media using the light wavelengths using the light wavelength from 10,000 nm to 190 nm in wavelength. The standard is capable of delivering data rates sufficient to support audio and video multimedia services and also considers mobility of the optical link, compatibility with variable light infrastructures, impairments due to noise and interference from sources like ambient light and a MAC layer that accommodates visible links as well as the other targeted light wavelengths. It also accommodates optical communications for cameras where transmitting devices incorporate light emitting sourses and receivers are digital cameras with a lens and image sensor. The standard adheres to applicable eye safety regulations.

1.2 Purpos e

This standard provides a global standard for short-range optical wireless communication. The standard provides (i) access to several hundred THz of unlicensed spectrum; (ii) immunity to electromagnetic interference and noninterference with Radio Frequency (RF) systems; (iii) for optical light systems, additional security by allowing the user to see the communication channel; and (iv) communication augmenting and complementing existing services (such as illumination, display, indication, decoration, etc.).

| Copyright © 2011 IEEE. All rights reserved. | 1 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 2. Normative references |
| --- | --- |
| 2 |  |

- The following referenced documents are indispensable for the application of this document (i.e., they must

- be understood and used, so each referenced document is cited in text and its relationship to this document is

- explained). For dated references, only the edition cited applies. For undated references, the latest edition of

- the referenced document (including any amendments or corrigenda) applies.

| 7 |  |
| --- | --- |
| 8 | ANSI/INCITS 373: Fiber Channel Framing and Signaling Interface (FC-FS).1 |
| 9 |  |
| 10 | IEEE Std 802.15.4TM-2006, IEEE Standard for Information technology—Telecommunications and |

- information exchange between systems—Local and metropolitan area networks—Specific requirements—

- Part 15.4: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Low-Rate

- Wireless Personal Area Networks (LR-WPANs).2, 3

| 14 |  |
| --- | --- |
| 15 | ITU-T I.432.1, Series I: Integrated Services Digital Network, ISDN user-network interfaces—Layer |

- 1RecommendationsB-ISDN user-network interface—Physical layer specification: General characteristics,

- http://www.itu.int/rec/T-REC-I.432.1-199902-I/en.4

| 18 |  |
| --- | --- |
| 19 | IEEE Std 802.15.7TM-2011, IEEE Standard for Information technology—Telecommunications and |

- information exchange between systems—Local and metropolitan area networks—Specific requirements—

- Part 15.7: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Short-

- Range Wireless Optical Communication Using Visible Light.5

| 23 |  |  |
| --- | --- | --- |
| 24 |  |  |
| 25 | 3. Definitions , acronyms , and abbreviations |  |
| 26 |  |  |
| 27 | 3.1 Definitions |  |
| 28 |  |  |
| 29 |  |  |

- For the purposes of this document, the following terms and definitions apply. The IEEE Standards Dictio-

- nary: Glossary of Terms & Definitions should be consulted for terms not defined in this clause.6

32

33	baseband bandwidth: The bandwidth as measured in the electrical domain at the LED driver.

34

- color function: A function that provides information, such as device status and channel quality, to the

- human eye via color.

| 37 | color-shift keying (CSK): A modulation scheme for visible-light communication involving multiple light |  |  |  |
| --- | --- | --- | --- | --- |
| 38 |  |  |  |  |
| 39 | sources, which keeps the average emitted optical color and the total optical power constant during |  |  |  |
| 40 | communication. |  |  |  |
| 41 | color stabilization: A control loop for the stabilization of the color emitted by color-shift-keying |  |  |  |
| 42 |  |  |  |  |
| 43 | transmitters. |  |  |  |
| 44 |  |  |  |  |
| 45 |  |  |  |  |
| 46 |  |  |  |  |
|  | 1ANSI publications are available from the Sales Department, American National Standards Institute, 25 West 43rd Street, 4th Floor, |  |  |  |

- New York, NY 10036, USA (http://www.ansi.org/).

- 2IEEE publications are available from the Institute of Electrical and Electronics Engineers, Inc., 445 Hoes Lane, Piscataway, NJ 08854,

- USA (http://standards.ieee.org/).

- 3The IEEE standards or products referred to in this clause are trademarks of the Institute of Electrical and Electronics Engineers, Inc.

- 4ITU-T publications are available from the International Telecommunications Union, Place des Nations, CH-1211, Geneva 20, Switzer-land/Suisse (http://www.itu.int/).

- 5IEEE publications are available from the Institute of Electrical and Electronics Engineers, Inc., 445 Hoes Lane, Piscataway, NJ 08854,

- USA (http://standards.ieee.org/).

- 6The IEEE Standards Dictionary: Glossary of Terms & Definitions is available at http://shop.ieee.org/.

2	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| color visibility dimming (CVD) frame: A frame used for color, visibility and dimming support. The color |  | 1 |  |
| --- | --- | --- | --- |
| visibility dimming frame visually provides information such as communication status and channel quality to |  | 2 |  |
| the user via various colors. The color visibility dimming frame may also be sent during idle or receive |  | 3 |  |
| modes of operation for continuous visibility and dimming support. During the color visibility dimming |  | 4 |  |
| frame transmission, the device is still emitting light while not communicating, and it is thus able to fulfill its |  | 5 |  |
| lighting function. The payload of the frame consists of visibility patterns of appropriate intensity and color. |  | 6 |  |
| compensation time: The idle time inserted in the idle pattern or in the data frame, where the light is turned |  | 7 |  |
|  |  | 8 |  |
| “ON” or “OFF” with the appropriate ratio to meet dimming requirements. |  | 9 |  |
| dimming: Reducing the radiant power of a transmitter while preserving the color of the transmitted light. |  | 10 |  |
|  |  | 11 |  |
| field of view (FOV): The angular extent of coverage for the optical transmitter or receiver. |  | 12 |  |
|  |  | 13 |  |
| idle pattern: A pattern whose duty cycle variation results in a change of brightness for dimming support and |  | 14 |  |
|  |  | 15 |  |
| may be transmitted during idle or receive mode. |  | 16 |  |
| macro cell: An aggregate cell formed using all the cells available at the optical media and is used for device |  | 17 |  |
|  |  | 18 |  |
| discovery and association. |  | 19 |  |
| modulation-domain spectrum: The spectrum observed at the output of the receiver’s photodetector; |  | 20 |  |
|  |  | 21 |  |
| typically measured at the output of the trans impedance amplifier |  | 22 |  |
| on-off keying (OOK): A simple modulation technique that represents digital data as the presence (‘ON’) or |  | 23 |  |
|  |  | 24 |  |
| absence (‘OFF’) of a signal. Note that ‘ON’ and ‘OFF’ are simply two logic levels or two distinct amplitude |  | 25 |  |
| levels for the purposes of communication and does not necessarily require that the light source be turned |  | 26 |  |
| OFF completely. |  | 27 |  |
| optical clock rate: The frequency at which the data is clocked out to the optical source. |  | 28 |  |
|  |  | 29 |  |
| photodetector: A photodetector captures optical power and translates it into an output signal. Most |  | 30 |  |
|  |  | 31 |  |
| photodetectors convert optical power into an electrical current or an electrical voltage. |  | 32 |  |
| PHY switch: A switch at the transmission interface between the PHY and the optical SAP, used to send and |  | 33 |  |
|  |  | 34 |  |
| receive data to and from a single or multiple optical sources and photodetectors in a selective manner. |  | 35 |  |
| point-and-shoot: The alignment of devices by the transmission of a color visibility dimming frame for the |  | 36 |  |
|  |  | 37 |  |
| purpose of illuminating the target receiving device. |  | 38 |  |
| switching level: A distinct amplitude level that defines ‘ON’ and ‘OFF’ of the light source for the purpose |  | 39 |  |
|  |  | 40 |  |
| of communications and does not necessarily require that the light source be turned off completely. |  | 41 |  |
| variable pulse-position modulation (VPPM): A modulation scheme for visible-light communication that |  | 42 |  |
|  |  | 43 |  |
| allows pulse-width control for light dimming support, mitigating intra-frame flicker. |  | 44 |  |
| visibility pattern: An in-band idle pattern used in the payload of a color visibility dimming frame. |  | 45 |  |
|  |  | 46 |  |
| OWPAN cluster: OWPANs managed by the same master coordinator (MC). |  | 47 |  |
|  |  | 48 |  |
| spatial phase (S_Phase): the phase of a discrete waveform which is built from the states ofa group of LEDs |  | 49 |  |
|  |  | 50 |  |
| . |  | 51 |  |
| global phase shift: the phase value that all LEDs in a data group together are shifted to transmit data. |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 3 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | data group: A group of data LEDs those operate together to transmit a data symbol |  |
| --- | --- | --- |
| 2 | reference group: A group of reference LEDs those operate together to transmit a reference signal |  |
| 3 |  |  |
| 4 |  |  |

- S_Phase shift: the subtraction value between the spatial phase values of data group and of the reference

- group.

| 7 | bad-sampled image: an image sampling over ‘ON’ and ‘OFF’ transitioning state of a light source. |  |  |
| --- | --- | --- | --- |
| 8 |  |  |  |
| 9 | x_state (of a LED): an unclear state that observed from a bad-sampled image. |  |  |
| 10 |  |  |  |
| 11 | SM-PSK (e.g. S2-PSK; S8-PSK; etc.): Spatial Multiple-Phase Shift Keying |  |  |
| 12 |  |  |  |
| 13 | DSM-PSK (e.g. DS8-PSK): Dimmable SM-PSK |  |  |
| 14 |  |  |  |
| 15 | ISC/L-PD: TBD |  |  |
| 16 |  |  |  |
| 17 | VTASC: TBD |  |  |
| 18 |  |  |  |
| 19 | SS-VTASC: TBD |  |  |
| 20 |  |  |  |
| 21 | offset-VPWM: TBD |  |  |
| 22 |  |  |  |
| 23 | Spread Spectrum: TBD |  |  |
| 24 |  |  |  |
| 25 | SS Code: TBD |  |  |
| 26 |  |  |  |
| 27 | Invisible Data Embedding: TBD |  |  |
| 28 |  |  |  |
| 29 | Sequential Scalable 2D Code: TBD |  |  |
| 30 |  |  |  |
| 31 | Gold Sequence: TBD |  |  |
| 32 |  |  |  |
| 33 | Angle Free Communication: TBD |  |  |
| 34 |  |  |  |
| 35 | Scalable Bitrate Controller: TBD |  |  |
| 36 |  |  |  |
| 37 | distance adaptive data rate controller: TBD |  |  |
| 38 |  |  |  |
| 39 | M-PSK: TBD |  |  |
| 40 |  |  |  |
| 41 | M-FSK: TBD |  |  |
| 42 |  |  |  |
| 43 | 2D-CODER: TBD |  |  |
| 44 |  |  |  |
| 45 | SS-M-PSK: TBD |  |  |
| 46 |  |  |  |
| 47 | SS-M-FSK: TBD |  |  |
| 48 |  |  |  |
| 49 | SS-HYBRID: TBD |  |  |
| 50 |  |  |  |
| 51 | SS-2D-CODER: TBD |  |  |
| 52 |  |  |  |
| 53 | Hybrid-M-PSK-FSK: TBD |  |  |
| 54 |  |  |  |
|  | 4 | Copyright © 20xx IEEE. All rights reserved. |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Sequential Scalable QR Code: TBD |  | 1 |  |
| --- | --- | --- | --- |
| Sequential Scalable Color Code: TBD |  | 2 |  |
|  |  | 3 |  |
| Orthogonal Spreading Codes: TBD |  | 4 |  |
|  |  | 5 |  |
| Blending: TBD |  | 6 |  |
|  |  | 7 |  |
| Watermarking: TBD |  | 8 |  |
|  |  | 9 |  |
| ROI: TBD |  | 10 |  |
|  |  | 11 |  |
| Heterogeneous network: a network functionality where data transmission is done over a combination of |  | 12 |  |
|  |  | 13 |  |
| optical and RF wireless links. |  | 14 |  |
| editor’s note: the following definitions need to be checked carefully. Most of them are from OCC. |  | 15 |  |
|  |  | 16 |  |
| Definition |  | 17 |  |
|  |  | 18 |  |
| a) clock information (of a data packet): This scheme is similar to the C-OOK scheme in which asyn- |  | 19 |  |
|  |  | 20 |  |
| chronous bits (Ab) represent the form of clock information. However, the data packet with clock |  | 21 |  |
| information (called a symbol) is encoded using M-FSK technique. |  | 22 |  |
| b) Frequency symbol: a symbol modulated by a frequency to transmit a data packet |  | 23 |  |
| Definition |  | 24 |  |
|  |  | 25 |  |
| varying frame rate ISC mode: an ISC mode that supports a varying frame-rate receiver. |  | 26 |  |
|  |  | 27 |  |
| asynchronous decoding: a decoding procedure under presence of frame rate variation. |  | 28 |  |
|  |  | 29 |  |
| optical clock rate (modulation rate): The frequency at which the data is clocked out to the optical source. |  | 30 |  |
|  |  | 31 |  |
| In flicker-free mode, let assume the frequency no less than 200Hz to be invisible to human eyes. |  | 32 |  |
| asynchronous bit: a form of clock information in the temporal scheme helping a varying frame rate |  | 33 |  |
|  |  | 34 |  |
| receiver in asynchronous-decoding. Note that this is not only necessarily one single bit (bit 1 or bit 0), but |  | 35 |  |
| also can be a symbol (a set of bits in which symbol 1 and symbol 0 are orthogonal somehow) to operate at |  | 36 |  |
| high noise affected. |  | 37 |  |
| clock information (of a data packet/symbol): The information represents the state of a symbol clocked |  | 38 |  |
|  |  | 39 |  |
| out. The clock information is transmitted along with a symbol to help a receiver identifying an arrival |  | 40 |  |
| state of new symbol under presence of frame rate variation. |  | 41 |  |
| rolling exposure time: the time from the first line to the last line exposes to light in a rolling shutter image |  | 42 |  |
|  |  | 43 |  |
| sensor. |  | 44 |  |
| rolling sampling rate: how may row of pixels exposed to light in a rolling shutter camera |  | 45 |  |
|  |  | 46 |  |
| forward decoding: a decoding process in asynchronous decoding that taken from the position of the SF |  | 47 |  |
|  |  | 48 |  |
| (preamble of the OOK data frame structure) backward on a rolling image. |  | 49 |  |
| backward decoding: a decoding process in asynchronous decoding that taken from the position of the SF |  | 50 |  |
|  |  | 51 |  |
| (preamble of the OOK data frame structure) forward on a rolling image. |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 5 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- packet recovery: a process in asynchronous decoding that recovers a complete data packet from the

- incomplete data parts decoded, forward and backward part of one (two) data packet(s).

3

- data fusion: a process in asynchronous decoding to group data parts (forward and backward parts) those

- belong to one packet. There are two types of data fusion that happens according to the value of asynchro-

- nous bits:

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

- inter-frame fusion: to group data parts from different images (usually two images, but can be more than two)

- intra-frame fusion: to group data parts from an image.

DS rate: the frequency at which the data subframe is clocked out to the transmission medium.

3.2 Acronyms and abbreviations

A/D	analog-to-digital converter

ACK	acknowledgment

AES	advanced encryption standard

AR	acknowledgment request

BE	backoff exponent

BI	beacon interval

BO	beacon order

BP	beacon period

BSN	beacon-sequence number

CAP	contention access period

CC	convolutional code

CCA	clear channel assessment

CDR	clock and data recovery

CFP	contention-free period

CIE	Commission Internationale de l'Eclairage (International Commission on Illumination)

CRC	cyclic redundancy check

CSK	color-shift keying

CSMA/CA	carrier sense multiple access with collision avoidance

CVD	color visibility dimming

D/A	digital-to-analog converter

D/L	downlink

DC	direct current

DME	device management entity

DSN	data-sequence number

ED	energy detection

ENC	encryption mode

FCS	frame check sequence

FDM	frequency division multiplexing

FEC	forward error correction

FER	frame-error ratio

FLP	fast locking pattern

FLR	fast link recovery

FOV	field of view

GF	Galois field

GTS	guaranteed time slot

HCS	header-check sequence

HP	hopping pattern

IFS	interframe space

6	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| ID | identifier | 1 |
| --- | --- | --- |
| IE | information element | 2 |
| LD | laser diode | 3 |
| LED | light-emitting diode | 4 |
| LIFS | long interframe space | 5 |
| LLC | logical link control | 6 |
| LPDU | logical link control protocol data unit | 7 |
| LOS | line of sight | 8 |
| MAC | medium access control | 9 |
| MC | master coordinator | 10 |
| MCPS | medium-access-control common-part sublayer | 11 |
| MCS | modulation and coding scheme | 12 |
| MD | mobile device | 13 |
| MFB | MCS feedback | 14 |
| MFR | medium-access-control footer | 15 |
| MFTP | maximum flickering-time period | 16 |
| MHR | medium-access-control header | 17 |
| MIC | message-integrity code | 18 |
| MLME | medium-access-control link-management entity | 19 |
| MPDU | medium-access-control protocol-data unit | 20 |
| MSDU | medium-access-control service-data unit | 21 |
| MSI | MCS sequence index | 22 |
| NB | number of backoffs | 23 |
| OOK | on-off keying | 24 |
| OWC | Optical Wireless Communications | 25 |
| OWPAN | Optical Wireless Personal Area Network | 26 |
| PAN | personal-area network | 27 |
| PD | physical-layer data | 28 |
| PHR | physical-layer header | 29 |
| PHY | physical layer | 30 |
| PIB | physical-layer personal-area-network information base | 31 |
| PID | personal-area-network identifier | 32 |
| PLME | physical-layer management entity | 33 |
| PPDU | physical-layer data unit | 34 |
| PPM | pulse position modulation | 35 |
| PSDU | PHY service data unit | 36 |
| PWM | pulse-width modulation | 37 |
| P2MP | point-to-multipoint | 38 |
| P2P | peer-to-peer | 39 |
| QoS | quality of service | 40 |
| RIFS | reduced interframe space | 41 |
| RLL | run-length limited | 42 |
| ROI | Region of Interest | 43 |
| RS | Reed-Solomon | 44 |
| RX | receiver | 45 |
| SAP | service access point | 46 |
| SC | spreading code | 47 |
| SHR | synchronization header | 48 |
| SIFS | short interframe space | 49 |
| SPDU | session-protocol data unit | 50 |
| SO | superframe order | 51 |
| SS | Spread Spectrum | 52 |
| SSCS | service-specific convergence sublayer | 53 |
| TDP | topology dependent pattern | 54 |
|  | Copyright © 20xx IEEE. All rights reserved. | 7 |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |

|  | IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC |  | IEEE Draft P802.15.13/D0 |
| --- | --- | --- | --- |
| 1 | TRX | transceiver |  |
| 2 | TX | transmitter |  |
| 3 | U/L | uplink |  |
| 4 | OWPAN | Optical wireless personal area network |  |
| 5 | VLC | visible-light communication |  |
| 6 | VPPM | variable pulse-position modulation |  |
| 7 | WPAN | wireless personal area network |  |
| 8 | WQI | wavelength quality indication |  |

- ISC/L-PD: TBD

- VTASC: TBD

- SS-VTASC: TBD

- offset-VPWM: TBD

- SS Code: TBD

- M-PSK: TBD

- M-FSK: TBD

- 2D-CODER: TBD

- SS-M-PSK: TBD

- SS-M-FSK: TBD

- SS-HYBRID: TBD

- SS-2D-CODER: TBD

- Hybrid-M-PSK-FSK: TBD

- ROI: TBD

| 23 | FSK | Frequency Shift Keying |  |
| --- | --- | --- | --- |
| 24 | FL | Frequency Labeling |  |
| 25 | DS | Data Symbol |  |
| 26 | MCU | MicroController Unit |  |
| 27 | FLR | Frequency Labeling Ratio |  |
| 28 | DF | Data Frame |  |
| 29 | DSF | Data Sub-Frame |  |
| 30 | SS | Splitter Symbol |  |
| 31 | PF | Preamble Frequency |  |
| 32 | SF | Synchronization Frame |  |
| 33 | IF | Inactive Frame |  |
| 34 | SEQ | Sequence Number |  |
| 35 | DP | Data Packet |  |
| 36 | FP | FEC Packet |  |
| 37 | MFH | MAC Frame Header |  |
| 38 | MFT | MAC Frame Tail |  |
| 39 | RS-FSK | Rolling Shutter - Frequency Shift Keying |  |
| 40 |  |  |  |
| 41 | 4. General des cription |  |  |
| 42 |  |  |  |
| 43 |  |  |  |
| 44 | 4.1 Introduction |  |  |
| 45 |  |  |  |
| 46 | Visible-light communication (OWC) transmits data by intensity modulating optical sources, such as light- |  |  |
| 47 |  |  |  |
| 48 | emitting diodes (LEDs) and laser diodes (LDs), faster than the persistence of the human eye. OWC merges |  |  |
| 49 | lighting and data communications in applications such as area lighting, signboards, streetlights, vehicles, |  |  |
| 50 | and traffic signals. This standard describes the use of OWC for wireless personal area networks (WPAN). |  |  |
| 51 | Some of the characteristics found in this standard are as follows: |  |  |
| 52 | a) Star, peer-to-peer, or broadcast operation |  |  |
| 53 |  |  |  |
| 54 | b) 16-bit short or 64-bit extended addresses |  |  |
|  | 8 | Copyright © 20xx IEEE. All rights reserved. |  |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- Scheduled or slotted random access with collision avoidance transmission

- Fully acknowledged protocol for transfer reliability

- Wavelength quality indication (WQI)

- Dimming support

- Visibility support

- Color function support

- Color-stabilization support

4.2 Network topologies

As shown in Table 1, three classes of devices are considered for OWC: infrastructure, mobile, and vehicle.

Table 1—Device classification

|  | Infrastructure | Mobile | Vehicle |
| --- | --- | --- | --- |
| Fixed coordinator | Yes | No | No |
| Power supply | Ample | Limited | Moderate |
| Form factor | Unconstrained | Constrained | Unconstrained |
| Light source | Intense | Weak | Intense |
| Physical mobility | No | Yes | Yes |
| Range | Short/long | Short | Long |
| Data rates | High/low | High | Low |

The IEEE 802.15.13 optical wireless personal area network (OWPAN) standard maps the intended applications to four topologies: peer-to-peer, star, broadcast and coordinated, as shown in Figure 1. Moreover, two advanced network functionalities are supported, relaying and heterogeneous networking of OWC and RF, as shown in Figure 2.

In the star topology, the communication is established between devices and a single central controller, called the coordinator. In the peer-to-peer topology, one of the two devices in an association takes on the role of the coordinator. In the coordinated topology, multiple devices communicate with multiple coordinators, supervised by a master coordinator. The master coordinator is connected to each coordinator via backhaul link. Note that the functionality of the master coordinator is not part of this standard. In addition, two advanced network functionalities may be enabled: relaying and heterogeneous RF&OWC. With the relaying functionality, an intermediate relay node is introduced between the coordinator and the device. With the heterogeneous RF&OWC functionality, data transmission over the optical wireless link can be combined with a parallel radio-based wireless link.

Each device, coordinator or relay node has a unique 64-bit address. When a device associates with a coordinator or relay node it is allowed to be allocated a short 16-bit address. Either address is allowed to be used for communication within the OWPAN managed by the coordinator, the relay node or the master coordinator. The coordinator, the relay node and the master coordinator might often be mains powered, while the devices will often be battery powered.

| Copyright © 20xx IEEE. All rights reserved. | 9 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Peer to peer	Star

Visibility

Broadcast	Coordinated	support across

all topologies

| Master | Coordinator | Device |  |
| --- | --- | --- | --- |
| coordinator |  |  |  |

Figure 1—Supported MAC topologies

Relaying	Heterogeneous RF & OWC

RF Link

| Master | Coordinator | Relay | Device |  |
| --- | --- | --- | --- | --- |
| coordinator |  |  |  |  |
| OWC link |  |  | RF link |  |

Figure 2—Advanced network functionalities

Each independent OWPAN has an identifier, as defined in 5.2.1.3 and 5.2.1.5. This OWPAN identifier allows communication between devices within a network using short addresses. The mechanism by which OWPAN identifiers are chosen is outside the scope of this standard.

- The network formation is performed by the higher layer, which is not part of this standard. Apart from the

- peer-to-peer and star topologies, IEEE 802.15.13 devices are also allowed to operate in a broadcast-only

- topology without being part of a network, i.e., without being associated to any device or having any devices

10	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| associated to them. Moreover, the devices are allowed to operate in the coordinated topology, where a |  | 1 |  |
| --- | --- | --- | --- |
| master coordinator is introduced to perform higher layer functions such as handover between adjacent |  | 2 |  |
| coordinators and interference management. In addition, IEEE 802.15.13 devices are allowed to operate with |  | 3 |  |
| relay network functionality, where an additional relay is used between the device and the coordinator. |  | 4 |  |
| Finally, the devices are allowed to operate with heterogeneous RF&OWC network functionality, where an |  | 5 |  |
| additional bidirectional radio link is introduced, where the optical link is either unidirectional or |  | 6 |  |
| bidirectional. A brief overview on how each supported topology may be formed is provided in 4.2.1, 4.2.2, |  | 7 |  |
| 4.2.3, 4.2.4, 4.2.5 and 4.2.6. |  | 8 |  |
| In case illumination function is required, visibility support is also provided in the absence of communication |  | 9 |  |
|  |  | 10 |  |
| or in the idle or receive modes of operation. The purpose of this mode is to maintain illumination and |  | 11 |  |
| mitigate flicker. |  | 12 |  |
| 4.2.1 Peer-to-peer topology |  | 13 |  |
|  |  | 14 |  |
| The basic structure of a peer-to-peer topology is illustrated in Figure 1. In a peer-to-peer topology, each |  | 15 |  |
|  |  | 16 |  |
| device is capable of communicating with any other device within its coverage area. In a peer-to-peer |  | 17 |  |
| topology, one of the peers acts as a coordinator. One peer defaults as the coordinator, for instance, by virtue |  | 18 |  |
| of being the first device to communicate on the channel. |  | 19 |  |
| 4.2.2 Star topology |  | 20 |  |
|  |  | 21 |  |
| The basic structure of a star topology is illustrated in Figure 1. All star networks operate independently from |  | 22 |  |
|  |  | 23 |  |
| all other star networks currently in operation. This is achieved by choosing a OWPAN identifier that is not |  | 24 |  |
| currently used by any other network within the coverage area. Once the OWPAN identifier is chosen, the |  | 25 |  |
| coordinator allows other devices to join its network. The higher layer is allowed to use the procedures |  | 26 |  |
| described in 5.1.3 and 5.1.6 to form a star network. |  | 27 |  |
| 4.2.3 Broadcast topology |  | 28 |  |
|  |  | 29 |  |
| The basic structure of a broadcast topology is illustrated in Figure 1. The device in a broadcast mode can |  | 30 |  |
|  |  | 31 |  |
| transmit a signal to other devices without forming a network. The communication is uni-directional and the |  | 32 |  |
| destination address is not required. |  | 33 |  |
| 4.2.4 Coordinated topology |  | 34 |  |
|  |  | 35 |  |
| The basic structure of a coordinated topology is illustrated in Figure 1. In the coordinated topology, multiple |  | 36 |  |
|  |  | 37 |  |
| coordinators are connected to each other and to a master coordinator through backhaul network. The |  | 38 |  |
| backhaul is out of the scope of this specification. Multiple OWPANs are coordinated by the master |  | 39 |  |
| coordinator. The master coordinator may be in charge of various kinds of coordination among the multiple |  | 40 |  |
| OWPANs, e.g. handover, interference management, OWPAN status monitoring, etc. |  | 41 |  |
| 4.2.5 Relay functionality |  | 42 |  |
|  |  | 43 |  |
| With the relay functionality, an intermediate relay is used to assist a transmission via a direct optical wireless |  | 44 |  |
|  |  | 45 |  |
| link. With the relay functionality, each relay supports different duplexing and relay modes. For full duplex |  | 46 |  |
| (FD), the relay receives and transmits data simultaneously, while in half duplex (HD), the relay receives the |  | 47 |  |
| data in one time slot and retransmits it in another transmission slot. The relay supports two modes; amplify- |  | 48 |  |
| and-forward (AF), and decode-and-forward (DF). |  | 49 |  |
| a) In AF mode, the RD receives the data from the coordinator, which are then retransmitted after |  | 50 |  |
|  |  | 51 |  |
| amplification. |  | 52 |  |
| b) In DF mode, the received data is decoded by the relay and then retransmitted to the destination |  | 53 |  |
| device. |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 11 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- In case the device is disconnected from the coordinator, a relay search request is conducted, including the

- relay capabilities. The coordinator broadcasts a relay search request frame. Each relay replies back on the

- control channel with its own capabilities including duplexing and relaying modes. The coordinator selects

- the relay that provides the best connectivity. The coordinator sets up a relay link between itself and the

- device through the selected relay. A connection remains active until the direct link between the coordinator

- and the device is reinitiated and the coordinator requires a termination of the link between the coordinator

- and the relay.

8

9	4.2.6 Heterogeneous RF & OWC functionality

10

- The IEEE 802.15.13 specification supports heterogeneous RF&OWC functionality. The functionality may

- be used in conjunction with different network topologies as demonstrated in Figure 3. The RF AP may be

- co-located with a coordinator or a master coordinator. Each coordinator provides optical wireless links to the

- devices while the RF AP provides a parallel RF link.

| 15 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 16 |  |  |  |  |  |
| 17 |  |  |  |  |  |
| 18 |  |  |  |  |  |
| 19 |  |  |  |  |  |
| 20 |  |  |  |  |  |
| 21 |  |  |  |  |  |
| 22 |  |  |  |  |  |
| 23 | Broadcast |  | Coordinated |  |  |
| 24 |  |  |  |  |  |
|  |  |  |  | MC with co- |  |
| 25 |  |  |  |  |  |
|  |  |  |  | located RF node |  |
| 26 |  |  |  |  |  |
| 27 |  |  |  |  |  |
| 28 |  |  |  |  |  |
| 29 |  |  |  |  |  |
| 30 |  |  |  |  |  |
| 31 |  |  |  |  |  |
| 32 |  |  |  |  |  |
| 33 | Master |  |  |  |  |
| 34 |  | Coordinator | Relay | Device |  |
|  | coordinator |  |  |  |  |
| 35 |  |  |  |  |  |
|  | OWC link |  |  | RF link |  |
| 36 |  |  |  |  |  |
| 37 | Figure 3—Heterogeneous RF&OWC functionality |  |  |  |  |
| 38 |  |  |  |  |  |
| 39 |  |  |  |  |  |
| 40 |  |  |  |  |  |

- As shown in Table 2, three types of devices according to the capabilities in supporting OWC and RF are

- considered for IEEE 802.15.13. Type 1 devices support OWC only operations. Type 2 devices support OWC

- downlink operations as well as RF bidirectional operations. Type 3 devices support OWC bidirectional

- operations as well as RF bidirectional operations.

45

- Type 2 and type 3 devices can operate via OWC and RF simultaneously. In the downlink, aggregated

- transmission through both OWC link and RF link may be used, besides switching between OWC link and

- RF link. For the coordinated topology, command frames or ACK frames with destination address set to the

- coordinator may be first transmitted to the master coordinator through RF link and then forwarded to the

- coordinator through the backhaul link. Specification of these networking functionalities is out of scope in

- 802.15.13.

52

53

54

12	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  | Table 2— Device classification according to supported RF capabilities |  |  |  |  |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | 2 |  |
|  | Device Type | RF Down | RF Uplink |  | OWC |  | OWC Uplink |  | 3 |  |
|  |  |  |  |  |  |  |  |  | 4 |  |
|  |  | Link |  |  | Downlink |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 5 |  |
|  | Type 1 |  |  |  | x |  | x |  | 6 |  |
|  | Type 2 | x |  | x | x |  |  |  | 7 |  |
|  |  |  |  |  |  |  |  |  | 8 |  |
|  | Type 3 | x |  | x | x |  | x |  | 9 |  |
|  |  |  |  |  |  |  |  |  | 10 |  |
| 4.3 Modulation-domain s pectrum |  |  |  |  |  |  |  |  | 11 |  |
|  |  |  |  |  |  |  |  |  | 12 |  |
|  |  |  |  |  |  |  |  |  | 13 |  |
| Figure 4 illustrates the concept of the modulation-domain spectrum. In Figure 4, the optical light source is |  |  |  |  |  |  |  |  | 14 |  |
| “always on”; hence, the output of the photodetector can be observed for performing clear channel |  |  |  |  |  |  |  |  | 15 |  |
| assessment (CCA). Prior to time t = T1, the spectrum is all at DC. After t = T1, the spectrum is split between |  |  |  |  |  |  |  |  | 16 |  |
| DC and the modulating signal. |  |  |  |  |  |  |  |  | 17 |  |
|  |  |  |  |  |  |  |  |  | 18 |  |
|  |  |  |  |  |  |  |  |  | 19 |  |
|  |  |  | modulationdomain |  |  |  | modulationdomain |  | 20 |  |
|  |  |  |  |  | - |  |  | - | 21 |  |
|  |  |  |  | spectrum |  |  |  | spectrum |  |  |
|  |  | amplitude |  |  |  | amplitude |  |  | 22 |  |
|  |  |  |  | t < | T1 |  |  | t > T1 |  |  |
|  |  |  |  |  |  |  |  |  | 23 |  |
|  |  |  |  |  |  |  |  |  | 24 |  |
|  |  |  |  |  |  |  |  |  | 25 |  |
|  |  |  |  | frequency |  |  |  | Fm frequency | 26 |  |
|  |  | 0 |  |  |  |  | 0 |  | 27 |  |
| voltage |  |  |  | voltage |  |  |  |  | 28 |  |
|  |  |  |  |  |  |  |  |  | 29 |  |
| +V |  |  |  | +V |  |  |  |  | 30 |  |
|  |  |  |  |  |  |  |  |  | 31 |  |
| 0 |  |  |  | 0 |  |  |  |  | 32 |  |
|  | time |  |  |  | t = T1 |  | time |  | 33 |  |
|  | t = T1 |  |  |  |  |  |  |  | 34 |  |
|  | visible light source |  | photodetector |  |  |  |  |  | 35 |  |
|  |  |  |  |  |  |  |  |  | 36 |  |
|  |  |  |  |  |  |  |  |  | 37 |  |
|  | Figure 4—Illustration of modulation-domain spectrum |  |  |  |  |  |  |  | 38 |  |
|  |  |  |  |  |  |  |  |  | 39 |  |
|  |  |  |  |  |  |  |  |  | 40 |  |
| 4.4 Architecture |  |  |  |  |  |  |  |  | 41 |  |
|  |  |  |  |  |  |  |  |  | 42 |  |
| The IEEE 802.15.13 architecture is defined in terms of a number of layers and sublayers in order to simplify |  |  |  |  |  |  |  |  | 43 |  |
| the standard. Each layer is responsible for one part of the standard and offers services to the higher layers. |  |  |  |  |  |  |  |  | 44 |  |
| The interface between the layers serve to define the logical links that are described in this standard. |  |  |  |  |  |  |  |  | 45 |  |
|  |  |  |  |  |  |  |  |  | 46 |  |
| A OWPAN device comprises of a PHY layer, which contains the light transceiver along with its low-level |  |  |  |  |  |  |  |  | 47 |  |
| control mechanism, and a medium access control (MAC) sublayer that provides access to the physical |  |  |  |  |  |  |  |  | 48 |  |
| channel for all types of transfers. Figure 5 shows these layers in a graphical representation, which are |  |  |  |  |  |  |  |  | 49 |  |
| described in more detail in 4.4.1 and 4.4.2. |  |  |  |  |  |  |  |  | 50 |  |
|  |  |  |  |  |  |  |  |  | 51 |  |
|  |  |  |  |  |  |  |  |  | 52 |  |
|  |  |  |  |  |  |  |  |  | 53 |  |
|  |  |  |  |  |  |  |  |  | 54 |  |
|  |  | Copyright © 20xx IEEE. All rights reserved. |  |  |  |  |  | 13 |  |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |  |  |  |  |

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  |  |  | upper layers |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LLC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSCS |  |  |  |  |  |  |  |  |  |  |  | device |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | management |  | dimmer |  |
|  |  |  |  |  |  |  |  |  |  |  |  | entity |  |  |  |
| MCPS-SAP |  |  |  |  |  |  | MLME-SAP |  |  |  |  | (DME) |  |  |  |
|  |  |  |  |  | MAC |  |  |  |  |  |  |  |  |  |  |
| PD-SAP |  |  |  |  |  |  | PLME-SAP |  |  |  |  |  |  |  |  |
|  |  |  |  |  | PHY |  |  |  |  |  |  |  |  |  |  |
|  |  |  | PHY switch |  |  |  |  |  |  |  |  |  |  |  |  |

OPTICAL-SAP optical media

cell #1	cell #2	...	cell	cell #n

#n-1

Figure 5—OWPAN device architecture

The upper layers, shown in Figure 5, consist of a network layer, which provides network configuration, manipulation, and message routing, and an application layer, which provides the intended function of the device. The definition of these upper layers is outside the scope of this standard. A logical link control (LLC) layer can access the MAC sublayer through the service-specific convergence sublayer (SSCS), defined in Annex B.

A device management entity (DME) is also supported in the architecture. The DME can talk to the PLME and MLME for the purposes of interfacing the MAC and PHY with a dimmer. The DME can access certain dimmer related attributes from the MLME and PLME in order to provide dimming information to the MAC and PHY. The DME can also control the PHY switch using the PLME for selection of the optical sources and photodetectors. The details of the DME are outside the scope of this standard. The PHY switch interfaces to the optical SAP and connects to the optical media, which may consist of a single or multiple optical sources and photodetectors. Multiple optical sources and photodetectors are supported in the standard for PHY III as well for OWC cell mobility. The PLME controls the PHY switch in order to select a cell. The line going to the optical SAP from the PHY switch is a vector. The number of lines comprising the optical SAP has the dimension of n  m, where ‘n’ is the number of cells and ‘m’ is the number of distinct data streams from the PHY. The value of ‘m’ is three for PHY III.

4.4.1 PHY layer

The PHY layer supports multiple PHY types. (editor note: only three PHYs are maintained according to the motion agreed in May meeting 2017)

14	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| a) PHY I: This PHY type is intended for outdoor usage with low data rate applications. This mode uses |  |  | 1 |  |
| --- | --- | --- | --- | --- |
| on-off keying (OOK) and variable pulse position modulation (VPPM) with data rates in the tens to |  |  | 2 |  |
| hundreds of kb/s, as defined in Table 106. |  |  | 3 |  |
| b) PHY II: This PHY type is intended for indoor usage with moderate data rate applications. This mode |  |  | 4 |  |
| uses OOK and VPPM with data rates in the tens of Mb/s, as defined in Table 107. |  |  | 5 |  |
| c) PHY III: This PHY type is intended for applications using color-shift keying (CSK) that have |  |  | 6 |  |
| multiple light sources and detectors. This mode uses CSK with data rates in the tens of Mb/s, as |  |  | 7 |  |
| defined in Table 108. |  |  | 8 |  |
| d) PHY IV: This PHY is intended for using with global shutter cameras and photodiode detectors with |  |  | 9 |  |
| data rates in the tens of b/s using various modulations. |  |  | 10 |  |
| e) PHY V: This PHY is intended for using with rolling shutter cameras and photodiode detectors with |  |  | 11 |  |
| data rates in the hundreds of b/s using various modulations. |  |  | 12 |  |
| f) PHY VI: This PHY is intended for 2-D screen coding modulation with data rates in the kb/s using |  |  | 13 |  |
| various modulations. |  |  | 14 |  |
| g) PHY VII: This PHY is intended for low rate LiFi applications with data rates in the tens of Mb/s |  |  | 15 |  |
| using OFDM modulation. |  |  | 16 |  |
| h) PHY VIII: This PHY is intended for high rate LiFi applications with data rates in the hundreds of |  |  | 17 |  |
| Mb/s using OFDM modulation. |  |  | 18 |  |
| 4.4.1.1 PHY frame structure |  |  | 19 |  |
|  |  |  | 20 |  |
| The MAC protocol data unit (MPDU) at the output of the MAC sublayer passes through the PHY layer and |  |  | 21 |  |
|  |  |  | 22 |  |
| becomes the PHY service data unit (PSDU) at the output of the PHY layer after being processed via the |  |  | 23 |  |
| various PHY blocks such as channel coding and line coding. The PSDU is prefixed with a synchronization |  |  | 24 |  |
| header (SHR), containing the preamble sequence field; and a PHY header (PHR), which, among other |  |  | 25 |  |
| things, contains the length of the PSDU in octets. The preamble sequence enables the receiver to achieve |  |  | 26 |  |
| synchronization. The SHR, PHR, and PSDU together form the PHY frame or PHY layer data unit (PPDU). |  |  | 27 |  |
| The format of the PHY frame is shown in Figure 161. |  |  | 28 |  |
| 4.4.1.2 Interoperability and coexistence between PHY types |  |  | 29 |  |
|  |  |  | 30 |  |
| The PHY types coexist but do not interoperate. PHY I and PHY II occupy different spectral regions in the |  |  | 31 |  |
|  |  |  | 32 |  |
| modulation-domain spectrum, which enables frequency division multiplexing (FDM) as a coexistence |  |  | 33 |  |
| mechanism, as shown in Figure 6. PHY I and PHY III also occupy different spectral regions in the |  |  | 34 |  |
| modulation-domain spectrum, with different data rates and different optical rate support, providing |  |  | 35 |  |
| coexistence. However, the optical clock frequencies used for PHY II and PHY III overlap, causing |  |  | 36 |  |
| significant overlap in the frequency domain spectrum. In addition, not all devices support multiple optical |  |  | 37 |  |
| frequency bands needed for PHY III. Hence, all PHY III devices use a PHY II device for device discovery to |  |  | 38 |  |
| support coexistence with PHY II. |  |  | 39 |  |
|  |  |  | 40 |  |
| amplitude | modulation - domain |  | 41 |  |
|  |  |  | 42 |  |
|  | spectrum |  |  |  |
|  |  |  | 43 |  |
| PHY I | PHY II, III |  | 44 |  |
|  |  |  | 45 |  |
|  |  |  | 46 |  |
|  |  |  | 47 |  |
|  | modulation |  | 48 |  |
| ambient light |  |  | 49 |  |
|  | frequency |  | 50 |  |
| interference |  |  |  |  |
|  |  |  | 51 |  |
| Figure 6—FDM separation of the PHY types in the modulation domain |  |  | 52 |  |
|  |  |  | 53 |  |
|  |  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. |  | 15 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- Editor note: revisit the figure after TG13 agree on which PHYs are maintained. See comment 16 of https://

- mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx

3

4	4.4.1.3 High-bandwidth PHY Introduction

5

- The high-bandwidth PHY extends the capabilities and improves the transmission performance in order to

- address the specific requirements of new use cases in scenarios B1-B4 mentioned in the Technical Consider-

- ations Document (TCD) for 802.15.7r1 [8], such as optical wireless access in indoor/home/office, industrial

- wireless (with specific requirements for robustness, low latency and secure data transmission), communica-

- tions between vehicles and vehicle-to-the-roadside-infrastructure communications, and as a wireless back-

- haul technology.

12

- The high-bandwidth PHY supports fixed wireless links and multiple mobile user links via an OWC infra-

- structure, which consists of one or more optical wireless access points. The PHY is wavelength-agnostic and

- extends the optical wavelength range beyond the scope of 802.15.13 standard for Optical Wireless Commu-

- nications (OWC) also below and above the wavelengths of 380 nm and 780 nm, respectively hereby includ-

| 17 | ing the invisible light. A wide range of data rates (i.e., 1 Mb/s to 10 Gb/s) are supported, targeting an |
| --- | --- |
| 18 | efficient use of the optical bandwidth also under time-variant channel conditions. |
| 19 |  |

- The high-bandwidth PHY introduces modern wireless transmission technologies, such as orthogonal fre-

- quency-division multiplexing (OFDM), adaptive transmission, multiple-input multiple-output (MIMO) and

- coordinated wireless networking of multiple access points (APs) to provide mobility for mobile user devices

- (UDs) in an OWC network infrastructure. In addition, specific requirements for enhanced robustness and

- lower latency are addressed to support e.g. industrial wireless, vehicular and backhaul scenarios (B2, B3,

- B4).

26

- For coordinated networking, unified interfaces are introduced for the user plane and an open interface to the

- control plane, which can be used at the network layer to manage the interference between parallel optical

- wireless links and to support user mobility. These interfaces enable also the coexistence of OWC with radio

- based wireless links.

31

32	4.4.1.3.1 PHY based on G.hn in coax mode

33

- As the above transmission concept is already realized for other media in home environments, such as twisted

- pair, coax and transmission over plastic optical fibers, the following specifications take references to the

- ITU-T recommendation for home networking G.hn that has been recently developed for all these media. The

- requirements of the TCD can be met by

| 38 |  |
| --- | --- |
| 39 | - Starting from G.hn PHY baseband coax modes for 50, 100 and 200 MHz bandwidth |
| 40 |  |
| 41 | - adding further modes for scalability towards lower and higher bandwidths |
| 42 |  |
| 43 | - adding new features for the coordinated wireless (COW) topology |
| 44 |  |
| 45 | - adding new features for the wireless relaying (WREL) topology |
| 46 |  |

- Starting from the G.hn recommendation is efficient because optical wireless is an emerging technology

- expecting fast market growth. The home networking community has developed G.hn for several fixed net-

- working media and is adopting OWC as a next medium to support as an further evolution of G.hn. By align-

- ing the G.hn evolution with the high-bandwidth mode in 802.15.13, a powerful new specification is obtained

- that fulfills the requirements in the TCD by evolving an existing technology rather defining a new one.

52

53

54

16	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

In the following, an implementable overview of the necessary features taken over from 2015 release of G.hn is given it is demonstrated how the existing G.hn PHY can be further developed in order to enable the new COW and WREL modes over the optical wireless link.

4.4.1.3.2 Network topologies

coordinated network	relaying

network controller

access

point

user

device

relay

device

Figure 7—New network topologies coordinated wireless (COW) and wireless relaying (WREL)

The high-bandwidth PHY supports three new topologies, i.e. coordinated wireless (COW), heterogeneous network (HET) and relaying (WREL), as shown in Figure 7 in addition to the point-to-point (P2P), broad-cast (BC) and star (S) topologies already described in 802.15.7-2011. In COW topology, mobility among multiple access points (APs), i.e. handover and interference coordination is supported. In the HET topology, optical wireless and radio frequency (RF) transmissions can be combined. In the WREL topology, the high-bandwidth PHY supports data transport including intermediate relays.

The high-bandwidth PHY defines all methods at the PHY layer for operating the link in P2P, S, COW, HET and WREL topologies. It is defined by higher layers (above the PHY) in which topology the link is operated. The PHY supports the respective data transport and control signaling needed in the respective topology.

Point-to-point

In the P2P topology, two UDs can connect to each other and establish a wireless link. The P2P link is defined such that it may serve as a wireless replacement of an Ethernet cable in any computer or telecommu-nication networks. Besides specifying the fundamental PHY architecture for all topologies, the MAC layer is assumed to support an automatic link setup and a feedback path for closed-loop frequency-selective link adaptation.

Star

In the S topology, one UD acts as AP serving multiple other UDs in parallel. The AP aggregates the traffic from multiple UDs and coordinates their wireless transmission. The S topology requires additional function-

| Copyright © 20xx IEEE. All rights reserved. | 17 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- alities. The PHY supports time-division multiple access (TDMA) and frequency-division multiple access

- (FDMA). Feedback from UDs is transmitted in an orthogonal manner via the PHY, i.e., contention-free. An

- additional control channel is broadcast via the PHY to all UDs containing information about granted trans-

- mission resources for uplink and downlink directions.

5

6	Coordinated wireless network

7

- In the COW topology, multiple UDs are served by multiple APs, which are in turn coordinated by a network

- controller (NC). The NC is a device that has a fixed network link to the APs. The NC reroutes the traffic

- paths between NC and APs in case of handover and controls the transmission of all APs and UDs to manage

- interference. APs are time-synchronized, what can be achieved e.g. by the IEEE 1588 precision time proto-

- col (PTP). The NC aggregates the wireless traffic of UDs and APs. However, its functionality is not part of

- 802.15.13. Only the specific data transport and control signaling needed in the COW topology are defined.

- Based on cell-specific reference signal, UDs and APs estimate the interference channel in down- and uplink

- directions, respectively. The corresponding metrics reports are conveyed in the downlink over the wireless

- uplink and via the APs to the NC and ii) in the uplink via the APs to the NC where it is used for interference

- coordination and handover.

18

19	Heterogeneous network

20

- In the HET topology, there are 3 types of devices based on its capability of support OWC and RF. Table 3

- shows the 3 different types of devices.

23

24

| 25 |  | Table 3—Device types |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 26 |  |  |  |  |  |  |
| 27 | Device type | RF down link | RF uplink | OWC downlink | OWC uplink |  |
| 28 |  |  |  |  |  |  |
|  | Type 1 |  |  | √ | √ |  |
| 29 |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |
|  | Type 2 | √ | √ | √ |  |  |
| 31 |  |  |  |  |  |  |
|  | Type 3 | √ | √ | √ | √ |  |
| 32 |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  |

- Type 1 devices only have OWC transceivers, therefore only OWC links are available. Type 2 and type 3

- devices have both OWC transceivers and RF transceivers and both OWC links and RF links are available.

- Therefore a heterogeneous network comprised of both OWC links and RF links can be formed as shown in

- Figure 8. Coordinators, which are LED lamps, are located at the room ceiling. Each coordinator is connected

- to the master coordinator through the backhaul link. The backhaul link is probably based on wired link, e.g.,

- power line link. The master coordinator distributes downlink traffic to different coordinators and manages

- handover and interference coordination between different OWPANs. Each coordinator provides OWC

- access to one or multiple devices. Type 2 and type 3 devices are connected to the master coordinator through

- RF links. The RF access point (AP) can be co-located at the master coordinator.

44

- In the downlink direction, joint transmission through both OWC link and RF link and handover between

- OWC link and RF link are possible for both type 2 and type 3 devices. For type 2 devices, uplink traffic shall

- be transmitted through RF link and OWC command frames and ACK frames that need to be transmitted to

- the coordinator shall be first transmitted to the master coordinator through RF link and then forwarded to the

- coordinator through the backhaul link.

50

51

52

53

54

18	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Global controller (with RF AP)

Backhaul link

Coordinator 1   Coordinator 2

| VLC link | RF link |
| --- | --- |
| Device | Device |
| VPAN1 | VPAN2 |

Figure 8—Heterogeneous network comprising of both RF links and OWC links

A heterogeneous network can have devices of all the three types. The master coordinator and coordinators shall distribute traffic and transmission opportunities to devices based on their capabilities of support RF and OWC.

Wireless relaying

In the REL topology, an intermediate relay device (RD) is enabled to assist a transmission via a direct opti-cal wireless link which may be shaded or blocked or in a bad signal condition. Both, classical relaying as well as cooperative relaying are supported. In this topology, it is assumed that each RD has multiple capabil-ities including duplexing and relay modes. For duplexing mode, the RF supports either full duplex (FD), where the RF receives and transmits data simultaneously, while in half duplex (HD), the RF receives the data in one time slot and retransmits the data in another time slot. For the relaying mode, the RD supports two modes; amplify-and-forward (AF), and decode- and-forward (DF) modes. In AF mode, the RD receives the data from the AP, which are then retransmitted after amplification to mitigate the channel degradation occurred during transmission from AP to RD. In DF mode, the data received by the RD is fully decoded and then retransmitted from the RD to the UD. In case the link between the AP and UD is disconnected, the AP will initiate a RD search request. Each AP broadcasts a search RD request frame. Each RD replies back on the control channel with its own capabilities including duplexing and relaying modes. The AP selects the RD that provides the best performance according to a specific criterion. The AP initiates a relay setup link procedure between the AP, selected REL and the UD. A connection remains active until the direct link between the AP and UD is reinitiated and the AP requires a termination of the link between the AP and REL.

| Copyright © 20xx IEEE. All rights reserved. | 19 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 4.4.1.3.3 Essential Features |  |
| --- | --- | --- |
| 2 | Scalable data rates |  |
| 3 |  |  |
| 4 |  |  |

- The high-bandwidth PHY supports variable data rates from 1 Mbit/s to 10 Gbit/s by means of a scalable

- design. In all PHY modes, subcarrier spacing and cyclic prefix (CP) are the same. Used bandwidth is scal-

- able by adapting number of used subcarriers. Interoperability among all PHY modes is enabled, i.e. a trans-

- ceiver with a smaller bandwidth can synchronize with respect to, and exchange control information and data

- with, another transceiver having a higher bandwidth and vice versa. Bandwidth adaptation is supported to

- operate the link at the lowest bandwidth during link setup, and uses subcarriers at low frequencies only to

- transmit control information, before switching eventually to a higher bandwidth mode.

12

13	Waveform

14

- An adaptive, real-valued OFDM waveform denoted as DC-biased discrete multi-tone (DMT) is used. It can

- be extended by an optional preprocessing in order to improve energy efficiency. Moreover, adaptive bit- and

- power loading is supported using variable modulation formats on each subcarrier or on groups of subcarri-

- ers, depending on the channel-, interference- and noise-characteristics of the optical wireless link.

19

20	Efficient use of the optical bandwidth

21

- The high-bandwidth PHY supports the efficient use of the optical bandwidth by means of a highly scalable

- PHY layer design, together with closed-loop adaptive transmission and the efficient support of MIMO,

- cooperative transmission and relaying. This combination allows robustness in the multi-path propagation

- channel, in case of mobility and in interference scenarios. Moreover, PHY is defined so that one-way laten-

- cies of 1 ms are achievable.

27

28	Dimming support, coexistence

29

- The high-bandwidth PHY allows dimming for use cases based on visible light such as indoor office/home

- applications, data centers, industrial establishments, secure wireless and vehicular communications. Due to

- adaptive transmission, coexistence is supported with ambient light and other light sources.

33

34	Metrics reporting

35

- The high-bandwidth PHY provides metrics to be reported for efficient operation of higher layer protocols.

- Depending on the topology, metrics to be reported comprise signal strength of strongest APs and UDs, fre-

- quency-selective signal-to-interference-and-noise ratio (SINR) and channel state information (CSI) for

- strongest APs and UDs. Short time intervals between metric reports and control messages are enabled target-

- ing fast adaptation to the time-varying wireless channel, low latency and minimal overhead.

41

42	Advanced wireless networking, high availability

43

- The high-bandwidth PHY allows robust wireless transmission and thereby high availability for all channel

- conditions. Advanced wireless networking is supported in S, COW, HET and WREL topologies. The link is

- available in both, line-of-sight (LOS) and non-LOS (NLOS) scenarios, at low signal-to-noise-and-interfer-

- ence ratio (SINR) and in interference-limited conditions.

48

49

50

51

52

53

54

20	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 4.4.2 MAC sublayer |  | 1 |  |
| --- | --- | --- | --- |
| The MAC sublayer provides two services accessed through two service access points (SAPs). MAC data is |  | 2 |  |
|  |  | 3 |  |
| accessed through the MAC common part sublayer SAP (MCPS-SAP) while MAC management is accessed |  | 4 |  |
| through the MAC sublayer management entity SAP (MLME-SAP). The MAC data service enables the |  | 5 |  |
| transmission and reception of MPDUs across the PHY data service. |  | 6 |  |
| The features of the MAC sublayer are beacon management, channel access, guaranteed time slot (GTS) |  | 7 |  |
|  |  | 8 |  |
| management, frame validation, acknowledged frame delivery, association, and disassociation. The MAC |  | 9 |  |
| sublayer provides hooks for implementing application-appropriate security mechanisms. The MAC sublayer |  | 10 |  |
| also provides color function, visibility, color-stabilization, and dimming support. |  | 11 |  |
| Clause 5 contains the specifications for the MAC sublayer. |  | 12 |  |
|  |  | 13 |  |
| 4.4.3 Dimming and flicker-mitigation support |  | 14 |  |
|  |  | 15 |  |
| This subclause outlines the methods for dimming and flicker-mitigation support. An idle pattern can be |  | 16 |  |
|  |  | 17 |  |
| transmitted during MAC idle or RX states for infrastructure light sources for dimming support. This is |  | 18 |  |
| important since it is desired to maintain visibility and flicker-free operation during idle or RX periods at the |  | 19 |  |
| infrastructure. The idle pattern has the same duty cycle that is used during the active data communication so |  | 20 |  |
| that there is no flicker seen during idle periods. This idle pattern and its dependence on the dimmer setting is |  | 21 |  |
| shown in Figure 9. The transition of active operation and idle/RX operation can be in large time scale (block |  | 22 |  |
| active/idle/RX) or in a small time scale (within a communication session). In the large time scale block |  | 23 |  |
| session activity, when the OWC activity is “ON”, there can be small time scaled transition of active mode |  | 24 |  |
| and idle/RX mode. Dimmer setting for high brightness (a) in Figure 9 illustrates a higher duty cycle for |  | 25 |  |
| higher brightness. Dimmer setting for low brightness (b) illustrates a lower duty cycle for lower brightness. |  | 26 |  |
| The data and the idle pattern should have the same duty cycle in order to minimize flicker. |  | 27 |  |
|  |  | 28 |  |
|  |  | 29 |  |
|  |  | 30 |  |
|  |  | 31 |  |
|  |  | 32 |  |
|  |  | 33 |  |
|  |  | 34 |  |
|  |  | 35 |  |
|  |  | 36 |  |
|  |  | 37 |  |
|  |  | 38 |  |
|  |  | 39 |  |
|  |  | 40 |  |
|  |  | 41 |  |
|  |  | 42 |  |
|  |  | 43 |  |
|  |  | 44 |  |
|  |  | 45 |  |
|  |  | 46 |  |
|  |  | 47 |  |
|  |  | 48 |  |
|  |  | 49 |  |
|  |  | 50 |  |
|  |  | 51 |  |
|  |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 21 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| brightness |  |  |  |  |  |  |  | block active |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A : |  |  | active |  | I : idle/ RX |  |  |  |  |  |  |  |
| on |  |  |  |  |  |  |  |  |  |  |  | block |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | block |  |  |
| VLC activity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | A |  | I | A | I | …… . | I | A |  | idle/ |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | active |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | RX |  |  |  |
| off |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

time

active	idle/RX	active

MAC scheduled TX

idle pattern

infrastructure TX output

Dimmer setting for high brightness (a)

active	idle/RX	active

MAC scheduled TX

idle pattern

infrastructure TX output

Dimmer setting for low brightness (b)

Figure 9—Adapting dimmer pattern and data duty cycle depending on dimmer setting

22	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

4.4.3.1 Light dimming

Light dimming is defined as controlling the perceived brightness of the light source according to the user’s requirement and is a cross layer function between the PHY and MAC. The details on the light dimming function of MAC sublayer are discussed in 5.4.15.

4.4.3.1.1 Idle pattern and compensation time dimming

The standard allows an idle pattern to be inserted between the data frames for light dimming, as shown in Figure 10. The duty cycle of the idle pattern can be varied to provide brightness variation. The idle pattern selection is not specified in this standard. An idle pattern can either be in-band or out-of-band as defined by the modulation-domain spectrum and both types of idle patterns are supported in this standard. An in-band idle pattern does not require any change in the clock and can be seen by the receiver. An out -of-band idle pattern is typically sent at a much lower optical clock rate (including the option of maintaining visibility via a DC bias only) and is not seen by the receiver (i.e., it does not lie in the receiver’s modulation-domain bandpass). The standard also allows a compensation time (“ON” or “OFF” time of a light source) to be inserted into either the idle pattern or into the data frame to reduce or increase the average brightness of a light source.

| idle pattern |  | VLC data frame |  | idle pattern |  |
| --- | --- | --- | --- | --- | --- |
| 001001001001001 |  | The compensation time ("ON" or |  |  |  |
| i.e. duty cycle of 1/3 |  | "OFF" time) can be inserted for |  |  |  |
|  |  | dimming. |  |  |  |

Figure 10—Example of idle pattern and compensation time dimming

4.4.3.1.2 Visibility pattern dimming

Visibility patterns are in-band idle patterns that are used in the payload of a CVD frame. The visibility patterns are used for supporting features such as flicker mitigation, continuous visibility, device discovery, and color stabilization. The visibility patterns are not encoded in the PHY layer and do not have a frame check sequence (FCS) associated with them. In order to generate high resolution visibility patterns from 0% to 100% in steps of 0.1%, there are certain constraints that need to be used in the design criteria for visibility patterns.

- The number of transitions between ones and zeros can be maximized to provide high-frequency switching in order to avoid flicker and to help the clock and data recovery (CDR) circuit at receiver for synchronization purposes, if used.

- Visibility pattern generation can be made in a simple manner. Designing a thousand patterns to support low resolutions (as low as 0.1% resolution) is not practical and makes visibility pattern generation and use very complex.

- Since visibility patterns are transmitted without changing the clock frequency (in-band), the patterns avoiding conflicts with existing RLL code words are recommended.

The generation of the visibility patterns and their usage is defined in 8.5.1.2.

4.4.3.1.3 Color-shift keying (CSK) dimming

CSK supports OWC using multi-color light sources and photo detectors. CSK has the following advantages:

| Copyright © 20xx IEEE. All rights reserved. | 23 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

|  | IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC |  | IEEE Draft P802.15.13/D0 |
| --- | --- | --- | --- |
| 1 | a) | Information is provided by the color coordinates: CSK channels are defined by mixed colors that are |  |
| 2 |  | allocated in the color coordinate plane; therefore, the connectivity is facilitated by the color coding. |  |
| 3 | b) | Total average power is constant: The total average power of all CSK light sources is constant; there- |  |
| 4 |  | fore, the envelope of the sum of all light signals is constant. |  |
| 5 | c) | Variable bit rate: CSK enables variable bit rate due to higher order modulation support; that is, the |  |
| 6 |  | raw bit rate equals the optical clock rate times the bits per CSK symbol. |  |
| 7 |  |  |  |

- CSK dimming employs amplitude dimming and controls the brightness by changing the current driving the

- light source. However, a color shift of the light source may arise from improper control of driving current for

- amplitude dimming. For a given dimmer setting, the average optical power from the light sources is

- constant. This implies that the center color of the color constellation is constant.

12

13	4.4.3.1.4 OOK dimming

14

- Since OOK modulation is always sent with a symmetric Manchester symbol, compensation time may need

- to be inserted into the data frame to adjust the average intensity of the perceived source. The structure for the

17	OOK dimming frame is as shown in Figure 11. This process breaks the frame into subframes and each

- subframe can be preceded by a resync field that aids in readjusting the data clock after the compensation

- time. The data frame is fragmented into subframes of the appropriate length after the FCS has been

- calculated and the forward error correction (FEC) has been applied. An example of OOK dimming to

- increase brightness by adding compensation symbols is as shown in Figure 12.

| 22 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 23 |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |
| 25 |  |  |  | frame length |  |  |  |
| 26 |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |
| 31 |  |  | dimmed | compensation |  |  |  |
| 32 |  | PHY |  |  | resync field |  |  |
|  | preamble |  | OOK |  |  | data subframe |  |
| 33 |  | header |  | symbols | i.e. 1010 pattern |  |  |
|  |  |  | extension |  |  |  |  |
| 34 |  |  | Figure 11—OOK dimming structure |  |  |  |  |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  |  |  |
| 38 |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  |  |
| 41 |  |  |  |  |  |  |  |
| 42 |  |  |  |  |  |  |  |
| 43 |  |  |  |  |  |  |  |
| 44 |  |  |  |  |  |  |  |
| 45 |  |  |  |  |  |  |  |
| 46 |  |  |  |  |  |  |  |
| 47 |  |  |  |  |  |  |  |
| 48 |  |  |  |  |  |  |  |
| 49 |  |  |  |  |  |  |  |
| 50 |  |  |  |  |  |  |  |
| 51 |  |  |  |  |  |  |  |
| 52 |  |  |  |  |  |  |  |
| 53 |  |  |  |  |  |  |  |
| 54 |  |  |  |  |  |  |  |
|  | 24 |  | Copyright © 20xx IEEE. All rights reserved. |  |  |  |  |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |
| brightness |  | AB = N% |  | AB = N% |  |  | AB = N% |  |  | AB = N% | AB = N% |  |  | AB = N% | 3 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
| N% |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 |  |
| 50% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |
| idle |  | CS P | H | CS | RF | DS | CS | RF | DS |  | CS | RF | DS | idle | 8 |  |
| patterns |  |  |  |  |  |  |  |  |  |  |  |  |  | patterns | 9 |  |
|  |  |  |  |  |  |  |  | frame length |  |  |  |  |  | time | 10 |  |
|  |  |  |  |  |  |  | PHY frame |  |  |  |  |  |  |  | 11 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12 |  |
|  |  |  |  |  |  |  |  |  |  |  | P: preambles |  |  |  | 13 |  |
|  |  |  |  |  |  |  |  |  |  |  | H: PHY headers & extension |  |  |  | 14 |  |
|  |  |  |  |  |  |  |  |  |  |  | CS: compensation symbols |  |  |  | 15 |  |
|  |  |  |  |  |  |  |  |  |  |  | RF: resync field |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | DS: data subframe |  |  |  | 16 |  |
|  | Figure 12—Example of OOK dimming to increase brightness |  |  |  |  |  |  |  |  |  |  |  |  |  | 17 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 18 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 19 |  |
| 4.4.3.1.5 Low-Clock-Rate OOK amplitude dimming |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 21 |  |
| Since the flicker on OOK modulation is mitigated by using symmetric symbols, a number of ones and zeros |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 22 |  |
| in bits "1" and "0" are equal maintaining the average bright of the light source at 50%. For some Image |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 23 |  |
| Sensor Communication PHY modes, the clock rate of modulation is constant at a low frequency, hundreds |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 24 |  |
| of Hz on S2-PSK and S8-PSK PHY modes or less than 5kHz on OOK PHY modes; hence, the pulse |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 |  |
| amplitude modulation is performed to adjust the average brightness. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 26 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 27 |  |
| Figure 13 shows an example of amplitude dimming for OOK modulation. The brightness of ones is |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |
| controlled to achieve the average brightness at a desired dimming level. The output dimming level is the |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 29 |  |
| average value of the one and the zero brightness. Compared to the OOK dimming approach by inserting |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |  |
| compensation symbols, the data rate of Low-Clock-Rate OOK amplitude modulation approach is |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 31 |  |
| maintained during dimming. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 32 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 33 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 34 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 35 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 36 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 37 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 38 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 39 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 41 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 42 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 43 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 44 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 45 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 46 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 47 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 48 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 49 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 51 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 52 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 53 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 54 |  |
|  |  |  |  | Copyright © 20xx IEEE. All rights reserved. |  |  |  |  |  |  |  |  |  | 25 |  |  |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

Figure 13—Example of OOK amplitude dimming applied for low-clock-rate OOK PHY modes (need editable figure)

4.4.3.1.6 VPPM dimming

- VPPM is a modulation scheme adapted for pulse width based light dimming and offers protection from

- intra-frame flicker. It does not create a color-shift in the light source that can arise from amplitude dimming

- because the pulse amplitude in VPPM is always constant and the dimming control is performed by the pulse

- width, not the amplitude.

33

34 VPPM makes use of the characteristics of 2-PPM (pulse position modulation) for non-flicker and PWM 35 (pulse-width modulation) for dimming control and full brightness. Bits “1” and “0” in VPPM are 36 distinguished by the pulse position within a unit period and have the same pulse width within their

- respective unit periods. The non-flicker characteristic in VPPM is obtained from the property that the

- average brightness on bits “1” and “0” is constant.

39

- Dimming and full brightness in VPPM is achieved by controlling the “ON” time pulse width. Figure 14

- describes the dimming control mechanism by VPPM. It is possible to adjust the pulse width for VPPM

- based on the dimming requirements. Therefore, a user can achieve the full brightness that can be provided

- by the light source.

44

- As shown in Figure 15, the light intensity for the payload can be adjusted by adapting the pulse width of

- VPPM symbols. The light intensity for the preamble and header can be adjusted by inserting compensation

- symbols of the appropriate length and intensity before the frame. The details on high resolution dimming

- using VPPM are described in 8.5.2.3.

49

50

51

52

53

54

26	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 14—Schematic mechanism for VPPM dimming

Figure 15—Example of VPPM dimming

4.4.3.1.7 RS-FSK Dimming

RS-FSK supports dimming by changing the duty cycle of the transmitted signal. This allows the system to adjust the observed average brightness by human eyes. Note that the duty cycle is independent of and does not affect the transmitted signal frequency, allowing the same demodulation scheme across different dim-ming settings (Figure 16). However, as the duty cycle setting is configured to be further from 50% (i.e., very bright or very dark), the symbol error rate is expected to increase as in these cases it is more difficult to accu-rately determine the strip width from the captured image.

| Copyright © 20xx IEEE. All rights reserved. | 27 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 16—RS-FSK dimming with 25%, 50%, and 75% duty cycles

4.4.3.2 Flicker mitigation

Flicker is defined as the fluctuation of the brightness of light that can cause noticeable physiological changes in humans. This standard strives for the mitigation of flicker that may be caused due to modulation of the light sources for communication. The maximum flickering time period (MFTP) is defined as the maximum time period over which the light intensity can be changing, but for which the resulting flicker is not

28	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

perceivable by the human eye (Berman, et al. [B13]). To avoid flicker, the brightness changes over periods longer than MFTP must be avoided.

The flicker in OWC is classified into two categories according to its generation mechanism: intra-frame flicker and interframe flicker. Intra-frame flicker is defined as the perceivable brightness fluctuation within a frame. Interframe flicker is defined as the perceivable brightness fluctuation between adjacent frame transmissions.

4.4.3.2.1 Intra-frame flicker mitigation

Intra-frame flicker mitigation is accomplished by either the use of run length limiting coding, modulation scheme, or both. Specifically, these schemes are manchester encoding as specified in 10.5.2, 4B6B encoding as specified in 10.5.1, 8B10B encoding as specified in 10.3, or VPPM as specified in 10.6.

4.4.3.2.2 Interframe flicker mitigation

The scheme used for interframe flicker mitigation is the transmission of an idle pattern between data frames whose average brightness is equal to that of the data frames, as defined in 8.5.1.1.

4.5 Functional overview

This clause provides a brief overview of the general functions of a OWPAN MAC sublayer and includes information on the superframe structure, data transfer model, data frame structure, acknowledgments, and security.

4.5.1 Superframe structure

This standard allows the optional use of a superframe structure. The format of the superframe is defined by the coordinator and varies with the network topologies. The superframe is bounded by beacons sent by the coordinator, as shown in Figure 17a, and is divided into equally sized slots. Optionally, the superframe can have an active and an inactive portion, as shown in Figure 17b. The beacon frame is transmitted in the first slot of each superframe. If a coordinator does not wish to use a superframe structure, it will turn off the beacon transmissions. Each superframe can have two or three portions: beacon period (BP), contention access period (CAP) and optional contention-free period (CFP), as shown in Figure 21. The beacons aresent in the BP and are used to synchronize the attached devices, to identify the OWPAN, and to describe the structure of the superframes. Any device wishing to communicate during the contention access period (CAP) between two beacons competes with other devices via slotted random access.The standard defines four random access methods: unslotted random access, slotted random access, unslotted CSMA/CA, and slotted CSMA/CA. These methods are described in 5.1.2.1.

All transactions are completed by the time of the next BP.

| Copyright © 20xx IEEE. All rights reserved. | 29 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 17—Superframe structure without GTSs

For low- latency applications or applications requiring specific data bandwidth, the coordinator is allowed to dedicate portions of the active superframe to that application. These portions are called guaranteed time slots (GTSs). The GTSs form the contention -free period (CFP), which always appears at the end of the active superframe starting at a slot boundary immediately following the CAP. More information on the GTS structure is provided in Figure 19. The coordinator is allowed to allocate a number of these GTSs, and a GTS may occupy more than one slot period. More information on the GTS slots and the maximum number available can be found in 5.2.2.1. All contention-based transactions are completed before the CFP begins. Also each device transmitting in a GTS ensures that its transaction is complete before the time of the next GTS or the end of the CFP. More information on the superframe structure as described in 5.1.1.

4.5.2 Data transfer model

Five types of data transfer transactions exist:

- The data transfer to a coordinator in which a device transmits the data.

- The data transfer from a coordinator in which the device receives the data.

- The data transfer between two peer devices.

- The data transfer between a device and a relay node.

- The data transfer between a relay node and a coordinator.

In a star or broadcast topology, only the first two transaction types are used, while in a peer-to-peer topology, all three transaction types are allowed.

The mechanisms for each transfer type depend on whether the network supports the transmission of beacons. A beacon-enabled OWPAN is used in networks that either require synchronization or support for low-latency devices. If the network does not need synchronization or support for low latency devices, it can elect not to use the beacon for normal transfers. However, the beacon is still required for network discovery. The structure of the frames used for data transfer is specified in 5.2.

30	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 4.5.2.1 Data transfer to a coordinator |  | 1 |  |
| --- | --- | --- | --- |
| When a device wishes to transfer data to a coordinator in a beacon-enabled OWPAN, it first listens for the |  | 2 |  |
|  |  | 3 |  |
| beacon. When the beacon is found, the device synchronizes to the superframe structure. At the appropriate |  | 4 |  |
| time, the device transmits its data frame, using slotted random access, to the coordinator. The coordinator is |  | 5 |  |
| allowed to acknowledge the successful reception of the data by transmitting an optional acknowledgment |  | 6 |  |
| frame. |  | 7 |  |
| When a device wishes to transfer data in a non beacon-enabled OWPAN, it simply transmits its data frame, |  | 8 |  |
|  |  | 9 |  |
| using unslotted random access, to the coordinator. The coordinator acknowledges the successful reception of |  | 10 |  |
| the data by transmitting an optional acknowledgment frame. The transaction is now complete. |  | 11 |  |
| 4.5.2.2 Data transfer from a coordinator |  | 12 |  |
|  |  | 13 |  |
| When the coordinator wishes to transfer data to a device in a beacon-enabled OWPAN, it indicates in the |  | 14 |  |
|  |  | 15 |  |
| beacon that the data message is pending. The device periodically listens to the beacon and, if a message is |  | 16 |  |
| pending, transmits a MAC command requesting the data, using slotted random access. The coordinator |  | 17 |  |
| acknowledges the successful reception of the data request by transmitting an acknowledgment frame. The |  | 18 |  |
| pending data frame is then sent using slotted random access, or; if possible, immediately after the |  | 19 |  |
| acknowledgment as described in 5.1.9.3. The device is allowed to acknowledge the successful reception of |  | 20 |  |
| the data by transmitting an optional acknowledgment frame. The transaction is now complete. Upon |  | 21 |  |
| successful completion of the data transaction, the message is removed from the list of pending messages in |  | 22 |  |
| the beacon. |  | 23 |  |
| When a coordinator wishes to transfer data to a device in a non beacon-enabled OWPAN, it stores the data |  | 24 |  |
|  |  | 25 |  |
| and waits for the appropriate device to make contact and request the data. A device is allowed to make |  | 26 |  |
| contact by transmitting a MAC command requesting the data, using unslotted random access, to its |  | 27 |  |
| coordinator. The coordinator acknowledges the successful reception of the data request by transmitting an |  | 28 |  |
| acknowledgment frame. If a data frame is pending, the coordinator transmits the data frame, using unslotted |  | 29 |  |
| random access to the device. If a data frame is not pending, the coordinator indicates this fact either in the |  | 30 |  |
| acknowledgment frame following the data request or in a data frame with a zero-length payload as described |  | 31 |  |
| in 5.1.9.3. If requested, the device acknowledges the successful reception of the data frame by transmitting |  | 32 |  |
| an acknowledgment frame. |  | 33 |  |
| 4.5.2.3 Peer-to-peer data transfers |  | 34 |  |
|  |  | 35 |  |
| In a peer-to-peer OWPAN, every device is allowed to communicate with every other device in its coverage |  | 36 |  |
|  |  | 37 |  |
| area. In order to do this effectively, the devices wishing to communicate will need to either receive |  | 38 |  |
| constantly or synchronize with each other. In the former case, the device can simply transmit its data using |  | 39 |  |
| unslotted random access. In the latter case, other measures need to be taken in order to achieve |  | 40 |  |
| synchronization. Such measures are beyond the scope of this standard. |  | 41 |  |
| 4.5.3 Clock-rate selection |  | 42 |  |
|  |  | 43 |  |
| The standard supports multiple optical clock rates in order to accommodate a wide variety of optical sources |  | 44 |  |
|  |  | 45 |  |
| and receivers. The standard also supports the use of asymmetric clock rates between two devices since the |  | 46 |  |
| transmitter and receiver in a device are independent and may support different clock-rate ranges. As an |  | 47 |  |
| example, the infrastructure transmitter may be unable to switch rapidly but may be able to transmit with high |  | 48 |  |
| power and require lower error correction while the mobile device transmitter may be able to switch rapidly |  | 49 |  |
| but may require higher error correction support due to its lower transmit power. The optical clock rate for |  | 50 |  |
| communication is established using the MAC and can be communicated to the receiver prior to data transfer. |  | 51 |  |
| The clock-rate selection and negotiation procedure is described in 6.5. |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 31 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 4.5.4 Frame structure |
| --- | --- |
| 2 |  |

- The frame structures have been designed to keep the complexity to a minimum while providing for error

- protection for transmission on a noisy channel. Each successive protocol layer adds to the structure with

- layer-specific headers and footers.

6

- a)   A beacon frame, used by a coordinator to transmit beacons.

- b)   A data frame, used for all transfers of data.

- c)   An acknowledgment frame, used for confirming successful frame reception.

- d)   A MAC command frame, used for handling all MAC peer entity control transfer.

- e)   A control frame, used for RTS/CTS procedure.

- f)   A CVD frame, used to maintain the proper light intensity between data frames, support dimming

| 13 | and for visually providing information such as communication status and channel quality to the user. |  |
| --- | --- | --- |
| 14 | 4.5.5 Improving probability of successful delivery |  |
| 15 |  |  |
| 16 |  |  |

- The IEEE 802.15.13 OWPAN employs various mechanisms to improve the probability of successful data

- transmission. These mechanisms are random access, frame acknowledgment, and data verification.

19

20	4.5.5.1 Random access mechanism

21

- The IEEE 802.15.13 OWPAN uses four types of channel access mechanism, depending on the network

- configuration. Non-beacon-enabled OWPANs use an unslotted random channel access mechanism, with or

- without CSMA/CA, as described in 5.1.2.1. Each time a device wishes to transmit data frames or MAC

- commands, it waits for a random back off period. Following the random back off, the device transmits its

- frame of data. If the optional carrier sense mechanism is active and the channel is found to be busy following

- the random back off, the device waits for another random period before trying to access the channel again.

- Acknowledgment frames are sent without using a random access mechanism (i.e., scheduled).

29

- Beacon-enabled OWPANs use a slotted random channel access mechanism, with or without CSMA/CA,

- where the back off slots are aligned with the start of the beacon transmission. Each time a device wishes to

- transmit data frames during the CAP, it locates the boundary of the next back off slot and then waits for a

- random number of back off slots. If the optional collision avoidance mechanism is active, and the channel is

- busy, following this random back off the device waits for another random number of back off slots before

- trying to access the channel again. If the channel is idle or the optional carrier sense mechanism is not active,

- the device begins transmitting on the next available back off slot boundary. Acknowledgment and beacon

- frames are sent without using a random access mechanism (i.e., scheduled).

| 38 | RTS/CTS mechanism is used in addition, if configured. |  |
| --- | --- | --- |
| 39 |  |  |
| 40 | 4.5.5.2 Frame acknowledgment |  |
| 41 |  |  |
| 42 |  |  |

- A successful reception and validation of a data or MAC command frame can be optionally confirmed with

- an acknowledgment, as described in 5.1.9.4. If the receiving device is unable to handle the received data

- frame for any reason, the message is not acknowledged.

46

- If the originator does not receive an acknowledgment after some period, it assumes that the transmission was

- unsuccessful. When the acknowledgment is not required, the originator assumes the transmission was

- successful.

50

51	4.5.5.3 Data verification

52

- A cyclic redundancy check is included in the MAC frame and the PHY header, as defined in Annex C, to

- verify the validity of the received data.

32	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

4.6 Security

From a security perspective, IEEE 802.15.13 OWPAN is slightly different from other wireless networks, due to directionality and visibility because of the choice of the visible optical spectrum. Because of directionality and visibility, if an unauthorized receiver is in the path of the communication signal, it can be recognized. Also, the signal will not travel across medium such as walls, unlike other radio frequency based wireless networks. However, security algorithms are still provided in the standard for features such as data confidentiality, authentication and replay protection.

Devices can be low-cost and have limited capabilities in terms of computing power, available storage, and power drain; and it cannot always be assumed they have a trusted computing base nor a high-quality random number generator aboard. Communications cannot rely on the online availability of a fixed infrastructure and might involve short-term relationships between devices that may never have previously communicated. These constraints limit the choice of cryptographic algorithms and protocols and influence the design of the security architecture because the establishment and maintenance of trust relationships between devices need to be addressed with care. In addition, battery lifetime and cost constraints can put severe limits on the security overhead these networks can tolerate, something that is of far less concern with higher bandwidth networks. Most of these security architectural elements can be implemented at higher layers and may, therefore, be considered to be outside the scope of this standard.

The cryptographic mechanism in this standard is based on symmetric-key cryptography and uses keys that are provided by higher layer processes. The establishment and maintenance of these keys is outside the scope of this standard. The mechanism assumes a secure implementation of cryptographic operations and secure and authentic storage of keying material.

The cryptographic mechanism provides particular combinations of the following security services:

- Data confidentiality: Assurance that transmitted information is only disclosed to parties for whom it is intended.

- Data authenticity: Assurance of the source of transmitted information (and, thereby, that information was not modified in transit).

- Replay protection: Assurance that duplicate information is detected.

The actual frame protection provided can be adapted on a frame-by-frame basis and allows for varying levels of data authenticity (to minimize security overhead in transmitted frames where required) and for optional data confidentiality. When nontrivial protection is required, replay protection is always provided.

Cryptographic frame protection is allowed to use a key shared between two peer devices (link key) or a key shared among a group of devices (group key), thus allowing some flexibility and application-specific trade-off between key storage and key maintenance costs versus the cryptographic protection provided. If a group key is used for peer-to-peer communication, protection is provided only against outsider devices and not against potential malicious devices in the key-sharing group.

The cryptographic security mechanisms used for protected MAC frames is described in Clause 7.

4.7 Concept of primitives

This subclause provides a brief overview of the concept of service primitives. The services of a layer are the capabilities it offers to the next higher layer or sublayer by building its functions on the services of the next lower layer. This concept is illustrated in Figure 18, showing the service hierarchy and the relationship of the two correspondent users and their associated layer (or sublayer) peer protocol entities.

| Copyright © 20xx IEEE. All rights reserved. | 33 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  |
| --- | --- | --- |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 13 |  |  |
| 14 |  |  |
| 15 | Figure 18—Service primitives |  |
| 16 |  |  |
| 17 |  |  |
| 18 | The services are specified by describing the information flow between the user and the layer. This |  |
| 19 |  |  |

- information flow is modeled by discrete, instantaneous events, which characterize the provision of a service.

- Each event consists of passing a service primitive from one layer to the other through a layer service access

- point (SAP) associated with an user. Service primitives convey the required information by providing a

- particular service. These service primitives are an abstraction because they specify only the provided service

| 24 | rather than the means by which it is provided. This definition is independent of any interface |
| --- | --- |
| 25 | implementation. |
| 26 |  |

- Services are specified by describing the service primitives and parameters that characterize it. A service may

- have one or more related primitives that constitute the activity that is related to that particular service. Each

- service primitive may have zero or more parameters that convey the information required to provide the

- service.

31

32 A primitive can be one of the following four generic types: 33

- a)   Request: The request primitive is used to request a service to be initiated.

- b)   Confirm: The confirm primitive is used to convey the results of one or more associated previous

36	service requests.

- c)   Indication: The indication primitive is used to indicate the next higher layer of an internal event.

- d)   Response: The response primitive is used to complete a procedure previously invoked by an

| 39 | indication primitive. |
| --- | --- |
| 40 |  |
| 41 |  |
| 42 |  |
| 43 |  |
| 44 |  |
| 45 |  |
| 46 |  |
| 47 |  |
| 48 |  |
| 49 |  |
| 50 |  |
| 51 |  |
| 52 |  |
| 53 |  |
| 54 |  |
| 34 | Copyright © 20xx IEEE. All rights reserved. |

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5. MAC protocol s pecification

This clause specifies the MAC sublayer of this standard. The MAC sublayer handles all access to the physical layer and is responsible for the following tasks:

- Generating network beacons if the device is a coordinator

- Synchronizing to network beacons

- Supporting OWPAN association and disassociation

- Supporting visibility

- Supporting dimming

- Flicker-mitigation scheme

- Supporting device security

- Providing a reliable link between two peer MAC entities

- Supporting mobility

Peer-to-peer, star and broadcasting and coordinated capabilities, as shown in Figure 1, are provided with a single MAC frame structure. All of these diverse modes are supported via a single low complexity integrated frame structure.

Constants and attributes that are specified and maintained by the MAC sublayer are written in the text of this clause in italics. Constants have a general prefix of “a”, e.g., aBaseSlotDuration, and are listed in Table 90 (see 6.4.1). Attributes have a general prefix of “mac”, e.g., macAckWaitDuration, and are listed in Table 91 (see 6.4.2), while the security attributes are listed in Table 99 (see 7.5.1).

5.1 MAC functional des cription

editor’s note: the following introduction paragraphs should be revisited after clause 5 gets stable.

This subclause provides a detailed description of the MAC functionality. Subclause 5.1.2 describes the following two mechanisms for channel access: contention based and contention free. Contention-based access allows devices to access the channel in a distributed fashion using an unslotted random access backoff algorithm. Contention-free access is controlled entirely by the coordinator through the use of GTSs.

The mechanisms used for starting and maintaining a OWPAN are respectively described in 5.1.3 and 5.1.5. Channel scanning is used by a device to assess the current state of a channel (or channels), locate all beacons within its operating space, or locate a particular beacon with which it has lost synchronization. Before starting a new OWPAN, the results of a channel scan can be used to select an appropriate logical channel, as well as a OWPAN identifier that is not being used by any other OWPAN in the area. Because it is still possible for the operating space of two OWPANs with the same OWPAN identifier to overlap, a procedure exists to detect and resolve this situation. Following a channel scan and suitable OWPAN identifier selection, operation as a coordinator shall commence. Also described in the subclause is a method to allow coordinator beaconing to discover other such devices during normal operations, i.e., when not scanning.

The mechanisms to allow devices to join or leave a OWPAN are defined in 5.1.6. The association procedure describes the conditions under which a device may join a OWPAN and the conditions necessary for a coordinator to permit devices to join. Also described is the disassociation procedure, which can be initiated by the associated device or its coordinator.

The mechanisms to allow devices to acquire and maintain synchronization with a coordinator are described in 5.1.7. Synchronization on a beacon-enabled OWPAN is described after first explaining how a coordinator generates beacon frames. Following this explanation, synchronization on a nonbeacon- enabled OWPAN is described. Also described is a procedure to reestablish communication between a device and its coordinator,

| Copyright © 20xx IEEE. All rights reserved. | 35 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | as it is possible that a device may lose synchronization in the case of either a beacon-enabled or a |
| --- | --- |
| 2 | nonbeacon-enabled OWPAN. |
| 3 |  |

- This standard has been designed so that application data transfers can be controlled by the devices on a

- OWPAN rather than by the coordinator. The procedures the coordinator uses to handle multiple transactions

- while preserving this requirement are described in 5.1.8.

| 7 | The mechanisms for transmitting, receiving, and acknowledging frames, including frames sent using |  |
| --- | --- | --- |
| 8 |  |  |
| 9 | indirect transmission, are described in 5.1.9. In addition, methods for retransmitting frames are also |  |
| 10 | described. |  |
| 11 |  |  |

- The mechanisms for allocating and deallocating a GTS are described in 5.1.10. The deallocation process

- may result in the fragmentation of the GTS space, i.e., an unused slot or slots. The subclause describes a

- mechanism to resolve fragmentation.

15

16 The MAC sublayer uses the mechanisms described in Clause 7 for all incoming and outgoing frames. 17

- Throughout this subclause, the receipt of a frame is defined as the successful receipt of the frame by the

- PHY and the successful verification of the FCS by the MAC sublayer, as described in 5.2.1.9.

20

- The mechanisms to allow devices to recover quickly in case of temporary interference using a fast link

- recovery process are defined in 5.1.11. The fast link recovery process also enables devices to save power by

- letting the infrastructure initiate the link recovery.

24

- The mechanisms to allow devices to use multiple channels in case of limited time resources or interference

- are defined in 5.1.12. Multiple channel resource assignment uses information about multiple channel

- support and band hopping in order to support more users or improve performance.

28

- The mechanisms to support mobility of the device under an infrastructure that supports multiple optical

- elements over a wide coverage area are defined in 5.1.13. The concept of a cell is introduced and the support

- for mobility across multiple cells supported by the infrastructure is presented.

32

33 The mechanisms to visually indicate to the user the various states using various colors are defined in 5.1.16. 34 The various states such as device discovery (scan, association, disassociation), file transfer status,

- wavelength quality indication and acknowledgments can be visually indicated to the user to help with device

- alignment for communication.

37

- The mechanisms to stabilize the optical color emitted by the transmitter are defined in 5.1.16. The CVD

- frames are used to estimate the change in color and this information can be provided as feedback to the

- transmitter to stabilize its color.

41

- The mechanisms for using the visibility and dimming information in the MAC are defined in 5.1.17.

- Features such as an extended preamble mode for providing visibility with improved synchronization

| 44 | performance, dimming overrides, adjusting the MAC layer transmission schedule to accommodate |  |
| --- | --- | --- |
| 45 | dimming, association and link adaptation in the presence of dimming are provided. |  |
| 46 | 5.1.1 Superframe structure |  |
| 47 |  |  |
| 48 | A coordinator on a OWPAN can optionally bound its channel time using a superframe structure. A |  |
| 49 |  |  |

- superframe is bounded by the transmission of a beacon frame and can have an active portion and an inactive

- portion. The coordinator may enter a low-power (sleep) mode during the inactive portion.

52

- The structure of this superframe is described by the values of macBeaconOrder and macSuperframeOrder.

- The MAC PIB attribute macBeaconOrder, describes the interval at which the coordinator shall transmit its

36	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| beacon frames. The value of macBeaconOrder, BO, and the beacon interval, BI, are related as follows: for 0 |  | 1 |  |
| --- | --- | --- | --- |
|  BO  14, BI = aBaseSuperframeDuration  2BO optical clocks. If BO = 15, the coordinator shall not |  | 2 |  |
| transmit beacon frames except when requested to do so, such as on receipt of a beacon request command. |  | 3 |  |
| The value of macSuperframeOrder shall be ignored if BO = 15. |  | 4 |  |
| The MAC PIB attribute macSuperframeOrder describes the length of the active portion of the superframe, |  | 5 |  |
|  |  | 6 |  |
| which includes the beacon frame. The value of macSuperframeOrder, SO, and the superframe duration, SD, |  | 7 |  |
| are related as follows: for 0  SO  BO, SD = aBaseSuperframeDuration  2SO optical clocks. If SO = |  | 8 |  |
| 15, the superframe shall not remain active after the beacon. If BO = 15, the superframe shall not exist (the |  | 9 |  |
| value of macSuperframeOrder shall be ignored), and macRxOnWhenIdle shall define whether the receiver is |  | 10 |  |
| enabled during periods of transceiver inactivity. |  | 11 |  |
| The active portion of each superframe shall be divided into aNumSuperframeSlots equally spaced slots of |  | 12 |  |
|  |  | 13 |  |
| duration 2SO  aBaseSlotDuration and is composed of three parts: a BP, a CAP and a CFP. The BP exists in |  | 14 |  |
| coordinated topologies when comb division is not used in PHY VII. Beacon interference avoidance using |  | 15 |  |
| comb division for PHY VII coordinated topology is described in TBD. The beacon shall be transmitted, |  | 16 |  |
| without the use of any random access, at the start of slot 0, if BP doesn’t exist. If BP exits, beacon is |  | 17 |  |
| transmitted without the use of any random access, in one beacon slot of BP. The CAP shall commence |  | 18 |  |
| immediately following the beacon. The CFP, if present, follows immediately after the CAP and extends to |  | 19 |  |
| the end of the active portion of the superframe. Any allocated GTSs shall be located within the CFP. |  | 20 |  |
|  |  | 21 |  |
| The MAC sublayer shall ensure that the integrity of the superframe timing is maintained, e.g., compensating |  | 22 |  |
| for clock drift error. |  | 23 |  |
|  |  | 24 |  |
| OWPANs that wish to use the superframe structure (referred to as a beacon-enabled OWPAN) shall set |  | 25 |  |
| macBeaconOrder to a value between 0 and 14, both inclusive, and macSuperframeOrder to a value between |  | 26 |  |
| 0 and the value of macBeaconOrder, both inclusive. |  | 27 |  |
|  |  | 28 |  |
| OWPANs that do not wish to use the superframe structure (referred to as a nonbeacon-enabled OWPAN) |  | 29 |  |
| shall set both macBeaconOrder and macSuperframeOrder to 15. In this case, a coordinator shall not transmit |  | 30 |  |
| beacons, except upon receipt of a beacon request command; all transmissions, with the exception of |  | 31 |  |
| acknowledgment frames and any data frame that quickly follows the acknowledgment of a data request |  | 32 |  |
| command, see 5.1.9.3, shall use an unslotted random access mechanism to access the channel. In addition, |  | 33 |  |
| GTSs shall not be permitted. |  | 34 |  |
| An example of a superframe structure is shown in Figure 19. In this case, the beacon interval, BI, is twice as |  | 35 |  |
|  |  | 36 |  |
| long as the active superframe duration, SD, and the CFP contains two GTSs. |  | 37 |  |
|  |  | 38 |  |
|  |  | 39 |  |
|  |  | 40 |  |
|  |  | 41 |  |
|  |  | 42 |  |
|  |  | 43 |  |
|  |  | 44 |  |
|  |  | 45 |  |
|  |  | 46 |  |
|  |  | 47 |  |
|  |  | 48 |  |
|  |  | 49 |  |
|  |  | 50 |  |
| Figure 19—An example of the superframe structure |  | 51 |  |
|  |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 37 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

|  | IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC |  |  |  |  |  |  |  |  | IEEE Draft P802.15.13/D0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | peer-to-peer mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ( a ) |  | all slots in frame used for same data |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | star |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  | (b) beacon, contention, uplink, downlink slots |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | broadcast |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  | (c) broadcast slots (beacon, downlink) |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | visibility mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  | ( d ) visibility slots |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 | legend for slot indication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  | data |  | beacon |  | contention |  |  | uplink | downlinkvisibility |  |  |  |  |  |
| 26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 27 | Figure 20—Example usage of frame structure for multiple topologies |  |
| --- | --- | --- |
| 28 |  |  |
| 29 |  |  |

- Figure 20 provides an example usage of frame structure configuration for multiple topologies such as peer-

- to-peer, star, broadcast and visibility modes. The beacon slots are used for the beacons and the contention

- slots are used in the CAP period. The uplink and downlink GTS slots are used in the CFP periods. Visibility

- or idle patterns can be sent in the visibility slots during idle or RX modes of the infrastructure to ensure

- continuous output and mitigate flicker and are also used for point-and-shoot mode to ensure visibility.

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

38	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

beacon	beacon

（a）

Peer-to-Peer / Broadcast topology

beacon	beacon

CAP	              CFP

（b）

|  |  | Star topology |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | beacon |  |  |  | beacon |  |
|  | BP | CAP |  | CFP |  |  |
| （c） | … | GTS | GTS | GTS | GTS |  |

Coordinated topology

Figure 21—Superframe structure for difference topologies

5.1.1.1 Beacon period (BP)

The BP exists in coordinated topology when comb division is not used, in PHY VII. Beacon interference avoidance using comb division for coordinated topology is specified in section 12. (editor note: please Volker provide text for comb division in high bandwidth PHY) The BP shall start at slot 0 of a superframe and complete before the beginning of the CAP on a superframe slot boundary. BP is further divided into multiple beacon slots as shown in Figure 22. The duration of each beacon slot is equal to the sum of the duration of a beacon physical layer data unit (PPDU) and the subsequent beacon-to-beacon inter-frame space. In peer-to-peer mode and star mode, the BP shall contain only one beacon slot. In coordinated network, the BP consists one to a maximal of MaxBeaconSlot beacon slots as shown in Figure 22. The structure of a BP is notified in the beacon frame (see 5.2.2.1.3).

| Copyright © 20xx IEEE. All rights reserved. | 39 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Beacon Period

Beacon Slot 1	Beacon Slot 2	Beacon Slot 3	Beacon Slot 4

Beacon PPDU

B2BIFS

Figure 22—An example of the structure of beacon period

5.1.1.2 Contention access period (CAP)

The CAP shall start immediately following the beacon and complete before the beginning of the CFP on a superframe slot boundary. If the CFP is zero length, the CAP shall complete at the end of the active portion of the superframe. The CAP shall be at least aMinCAPLength optical clocks, unless additional space is needed to temporarily accommodate the increase in the beacon frame length needed to perform GTS maintenance (see 5.2.2.1.5) and shall shrink or grow dynamically to accommodate the size of the CFP.

All frames, except acknowledgment frames and any data frame that quickly follows the acknowledgment of a data request command (see 5.1.9.3) transmitted in the CAP shall use a slotted random access mechanism to access the channel. A device transmitting within the CAP shall ensure that its transaction is complete (i.e., including the reception of any acknowledgment) one interframe space (IFS) period (see 5.1.1.4) before the end of the CAP. If this is not possible, the device shall defer its transmission until the CAP of the following superframe.

In the case when a polling mechanism is employed, a poll by the coordinator to all devices can be used to indicate the beginning of a transmission slot for slotted random access. The poll synchronizes and enables each transmission of the different devices. In the case when a polling mechanism is not employed, the devices will be configured to synchronize according to the beacon frame and will be responsible for keeping track of the valid transmission slots in case slotted random access is employed.

5.1.1.3 Contention-free period (CFP)

The CFP shall start on a slot boundary immediately following the CAP and it shall complete before the end of the active portion of the superframe. If any GTSs have been allocated by the coordinator, they shall be located within the CFP and occupy contiguous slots. The CFP shall therefore grow or shrink depending on the total length of all of the combined GTSs. Communication between devices can take a variable number of slots. A single device or user can have access to more than a single slot for sustained data transfer in the frame, if there are slots available.

40	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| No transmissions within the CFP shall use a unslotted random access mechanism to access the channel. A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| device transmitting in the CFP shall ensure that its transmissions are complete one IFS period (see 5.1.1.4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |
| before the end of its GTS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |
| In the case when a polling mechanism is employed, a poll by the coordinator to a specific device shall be |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |
| used to indicate the beginning of a transmission slot allocated solely and specifically to the device of choice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 |  |
| The polling mechanism synchronizes and enables each transmission of a device, thus, completely |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |
| eliminating contention. No restrictions shall be imposed on the order and/or the number of times devices are |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 |  |
| being polled. The polling mechanism allows contention access to be enabled at random points of the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| superframe by issuing a poll to all stations as described in 6.2.1.1.1. Furthermore, transmission slots during |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |  |
| the CFP can be allocated dynamically as the coordinator chooses at its own discretion which device shall be |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11 |  |
| polled. The algorithm used to determine the polling sequence is outside the scope of this specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12 |  |
| 5.1.1.4 Interframe spacing (IFS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 13 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 14 |  |
| The MAC sublayer needs a finite amount of time to process data received by the PHY. To allow for this, two |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16 |  |
| successive frames transmitted from a device shall be separated by at least an IFS period; if the first |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 17 |  |
| transmission requires an acknowledgment, the separation between the acknowledgment frame and the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 18 |  |
| second transmission shall be at least an IFS period. The length of the IFS period is dependent on the size of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 19 |  |
| the frame that has just been transmitted. Frames (i.e., MPDUs) of up to aMaxSIFSFrameSize octets in length |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |
| shall be followed by a SIFS period of a duration of at least macMinSIFSPeriod optical clocks. Frames (i.e., |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 21 |  |
| MPDUs) with lengths greater than aMaxSIFSFrameSize octets shall be followed by a long interframe space |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 22 |  |
| (LIFS) period of a duration of at least macMinLIFSPeriod optical clocks. Burst frames shall have a reduced |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 23 |  |
| interframe space (RIFS) of exactly macMinRIFSPeriod. The IFS for the different modes are defined in 8.3.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 24 |  |
| and the concepts are illustrated in Figure 23. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 |  |
| The slotted random access algorithm shall take this requirement into account for transmissions in the CAP. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 26 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 27 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |
|  | acknowledged transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 29 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |  |
|  | long frame |  |  |  |  |  | ACK |  |  |  |  |  | short frame |  |  |  |  |  |  |  |  | ACK |  |  | 31 |  |
|  |  |  | tack |  |  |  |  |  |  | LIFS |  |  |  |  |  |  |  | tack |  |  |  |  | SIFS |  | 32 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 33 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 34 |  |
|  | unacknowledged transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 35 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 36 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 37 |  |
|  | long frame |  |  |  |  |  | short frame |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 38 |  |
|  |  |  | LIFS |  |  |  |  |  |  | SIFS |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 39 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  |
|  | burst transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 41 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 42 |  |
|  | long frame |  |  |  |  |  | burst frame |  |  | burst frame |  |  |  |  |  | burst frame |  |  |  |  |  |  |  |  | 43 |  |
|  |  |  | LIFS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 44 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 45 |  |
|  |  |  |  |  |  |  |  | RIFS |  |  |  |  |  | RIFS |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Figure 23—Interframe spacing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 46 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 47 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 48 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 49 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 51 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 52 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 53 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 54 |  |
|  |  |  | Copyright © 20xx IEEE. All rights reserved. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 41 |  |  |  |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 5.1.2 Channel access |
| --- | --- |
| 2 |  |

- This subclause describes the mechanisms for accessing the physical optical channel. The standard provides a

- single OWC MAC frame structure that can be configured for multiple modes. The frame is composed of a

- variable number of slots. A slot can be defined as the minimum time needed to communicate to send the

- smallest data to a device and is fixed.

7

8	5.1.2.1 Random access algorithm

9

- The slotted random access algorithm shall be used before the transmission of data or MAC command frames

- transmitted within the CAP, unless the frame can be quickly transmitted following the acknowledgment of a

| 12 | data request command (as defined | in 5.1.9.3 for timing requirements). None of the random access |
| --- | --- | --- |
| 13 | algorithms shall be used for the | transmission of beacon frames in a beacon-enabled OWPAN, |
| 14 | acknowledgment frames, or data frames transmitted in the CFP. |  |
| 15 |  |  |

- If periodic beacons are being used in the OWPAN, the MAC sublayer shall employ the slotted version of the

- random access algorithm for transmissions in the CAP of the superframe. Conversely, if periodic beacons

- are not being used in the OWPAN or if a beacon could not be located in a beacon-enabled OWPAN, the

- MAC sublayer shall transmit using the unslotted version of the random access algorithm. In both cases, the

- algorithm is implemented using units of time called backoff periods, where one backoff period shall be equal

- to aUnitBackoffPeriod optical clocks.

22

- In slotted random access, the backoff period boundaries of every device in the OWPAN shall be aligned with

- the superframe slot boundaries of the coordinator, i.e., the start of the first backoff period of each device is

- aligned with the start of the beacon transmission. In slotted random access, the MAC sublayer shall ensure

- that the PHY commences all of its transmissions on the boundary of a backoff period. In unslotted random

- access, the backoff periods of one device are not related in time to the backoff periods of any other device in

- the OWPAN. In the case a polling mechanism is used, each poll by the coordinator node simultaneously

- addressed to all device nodes will indicate the beginning of the next transmission slot.

30

- Each device shall maintain two variables for each transmission attempt: NB and BE. NB is the number of

- times the access algorithm was required to backoff while attempting the current transmission; this value

- shall be initialized to zero before each new transmission attempt. The variable BE is the backoff exponent,

- which is related to how many backoff periods a device shall wait before attempting to access/assess a

- channel. BE shall be initialized to the value of macMinBE.

36

- Figure 24 illustrates the steps of the access algorithm. The MAC sublayer shall first initialize NB and BE for

- slotted random access then locate the boundary of the next backoff period. The MAC sublayer shall delay

- for a random number of complete backoff periods in the range 0 to 2BE – 1 [step (2)] and then request that

- the PHY perform a transmission or optionally a CCA. In a slotted random access system, the transmission,

- or CCA if active, shall start on a backoff period boundary. In an unslotted system, the transmission, or CCA

- if active, shall start immediately.

43

- In a slotted random access system, the MAC sublayer shall ensure that, after the random backoff, the

- remaining slotted random access operations can be undertaken and the entire transaction can be transmitted

- before the end of the CAP. Note that any bit padding used by the supported PHY shall be considered in

- making this determination. If the number of backoff periods is greater than the remaining number of backoff

- periods in the CAP, the MAC sublayer shall pause the backoff countdown at the end of the CAP and resume

- it at the start of the CAP in the next superframe. If the number of backoff periods is less than or equal to the

- remaining number of backoff periods in the CAP, the MAC sublayer shall apply its backoff delay and then

- evaluate whether it can proceed. The MAC sublayer shall proceed if the remaining unslotted random access

- algorithm steps, the frame transmission, and any acknowledgment can be completed before the end of the

- CAP. If the MAC sublayer can proceed and CCA is active, it shall request that the PHY perform the CCA in

42	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

| IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC |  |  |  | IEEE Draft P802.15.13/D0 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| the current superframe. If the MAC sublayer cannot proceed, it shall wait until the start of the CAP in the |  |  |  |  | 1 |  |
| next superframe and apply a further random backoff delay before evaluating whether it can proceed again. |  |  |  |  | 2 |  |
| If CCA is active and the channel is assessed to be busy, the MAC sublayer shall increment both NB and BE |  |  |  |  | 3 |  |
|  |  |  |  |  | 4 |  |
| by one, ensuring that BE shall be no more than macMaxBE. If the value of NB is less than or equal to |  |  |  |  | 5 |  |
| macMaxRABackoffs, the access algorithm shall return to perform a random backoff as shown in Figure 24. If |  |  |  |  | 6 |  |
| the value of NB is greater than macMaxRABackoffs, the access algorithm shall terminate with a channel |  |  |  |  | 7 |  |
| access failure status. |  |  |  |  | 8 |  |
|  |  |  |  |  | 9 |  |
|  |  |  |  |  | 10 |  |
|  | random access |  |  |  | 11 |  |
|  |  |  |  |  | 12 |  |
|  |  |  |  |  | 13 |  |
|  | NB=0 |  |  |  | 14 |  |
|  | BE=macMinBE |  |  |  | 15 |  |
|  |  |  |  |  | 16 |  |
| no | slotted? |  |  |  | 17 |  |
|  |  |  |  |  | 18 |  |
|  | yes |  |  |  | 19 |  |
|  | locate backoff |  |  |  | 20 |  |
|  | period boundary |  |  |  | 21 |  |
|  |  |  |  |  | 22 |  |
|  | delay for |  |  |  | 23 |  |
|  | random (2BE-1) unit |  |  |  | 24 |  |
|  | backoff periods |  |  |  | 25 |  |
| no | carrier sense |  |  |  | 26 |  |
|  |  |  |  |  | 27 |  |
|  | active? |  |  |  |  |  |
|  |  |  |  |  | 28 |  |
|  | yes |  |  |  |  |  |
|  |  |  |  |  | 29 |  |
|  |  |  |  |  | 30 |  |
|  | perform CCA |  |  |  | 31 |  |
|  |  |  |  |  | 32 |  |
|  |  |  |  |  | 33 |  |
|  |  | no |  |  | 34 |  |
|  | channel idle? |  |  |  | 35 |  |
|  | yes |  |  |  | 36 |  |
|  |  |  |  |  | 37 |  |
|  | transmit |  |  |  | 38 |  |
|  |  |  | no |  | 39 |  |
|  |  |  | NB | yes | 40 |  |
|  | ACK required? |  | ≤macMaxRABackoffs? |  | 41 |  |
|  | yes |  |  |  | 42 |  |
|  |  |  |  |  | 43 |  |
|  | ACK received within |  | NB=NB+1, |  | 44 |  |
|  | macAckWaitDuration? |  | BE=min(BE+1,macMaxBE) |  | 45 |  |
|  | yes |  |  |  | 46 |  |
|  |  |  |  |  | 47 |  |
|  | success |  |  | failure | 48 |  |
|  | Figure 24—Random access flowchart |  |  |  | 49 |  |
|  |  |  |  |  | 50 |  |
|  |  |  |  |  | 51 |  |
|  |  |  |  |  | 52 |  |
|  |  |  |  |  | 53 |  |
|  |  |  |  |  | 54 |  |
|  | Copyright © 20xx IEEE. All rights reserved. |  |  | 43 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 5.1.2.2 RTS / CTS protocol |
| --- | --- |
| 2 |  |

- RTS/CTS protocol maybe optionally used as indicated in the beacon frame (see 5.2.2.1). A device may

- compete for the transmission of the RTS frame first. When the device gains the right for transmission after

- contention it shall transmit an RTS frame.

6

- After receiving the RTS frame successfully, the recipient shall transmit a CTS frame to the device that has

- sent the RTS frame. CTS frame carries a Duration field that holds a time value that indicates the duration for

- which the sending device expects the medium to be busy referenced from the end of the last symbol of the

- PPDU carrying the MAC frame. Any device other than the addressed device that successfully demodulates

- the frame should stop competing for the transmission until the time indicated by the Duration field expires.

| 16 | S I F S S I F S |  |  | A I F S |  |
| --- | --- | --- | --- | --- | --- |
| 17 | C W |  |  |  |  |
| 18 |  |  |  |  |  |
| 19 |  |  |  |  |  |
| 20 |  |  |  |  |  |
| 21 | R | C |  | A |  |
| 22 | T | T | D a ta / C o m m a n d fra m e | C |  |
| 23 |  |  |  |  |  |
| 24 | S | S |  | K |  |
| 25 |  |  |  |  |  |
| 26 |  |  |  |  |  |
| 27 |  |  |  |  |  |
| 28 |  | Figure 25—An example of the usage of RTS/CTS |  |  |  |
| 29 |  |  |  |  |  |
| 30 |  |  |  |  |  |
| 31 | 5.1.2.3 Multiple modulation bandwidths supporting |  |  |  |  |
| 32 |  |  |  |  |  |
| 33 | Multiple modulation bandwidths supporting |  |  |  |  |
| 34 |  |  |  |  |  |

- The coordinator and the devices in the OWPAN may support different modulation bandwidths and the

- coordinator has no prior knowledge of which device will win during the contention in the CAP. To support

- multiple modulation bandwidths, the RTS/CTS mechanism shall be used (see 5.1.2).

38

- The coordinator records the supported modulation bandwidth of each associated device in its OWPAN

- during association (see clause 5.1.6).

| 41 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 42 | The coordinator and a device shall set the operation modulation bandwidth according to the following rules: |  |  |  |  |  |  |
| 43 |  |  |  |  |  |  |  |
| 44 | The downlink PPDUs shall be modulated with the modulation bandwidth of BW_SEL1 that is equal to or |  |  |  |  |  |  |
| 45 | lower | than | MAX_BW_SEL1. | MAX_BW_SEL1 | equals | to | min{max |
| 46 | COODINATOR_Tx_modulation_bandwidth, max DEVICE_Rx_modulation_bandwidth} , the PPDU |  |  |  |  |  |  |
| 47 | carrying the corresponding ACK frames shall be modulated on the modulation bandwidth of BW_SEL2 that |  |  |  |  |  |  |
| 48 | is equal to or lower than MAX_BW_SEL2. MAX_BW_SEL2 equals to |  |  |  |  |  | min{max |
| 49 | COODINATOR_Rx_modulation_bandwidth, max DEVICE_Tx_modulation_bandwidth}. |  |  |  |  |  |  |
| 50 |  |  |  |  |  |  |  |

- The "max COODINATOR_Tx_modulation_bandwidth" represents the maximum modulation bandwidth

- that the transmitter of the coordinator can support. "max DEVICE_Rx_modulation bandwidth" represents

| 53 | the maximum modulation bandwidth that the receiver of a device can support. The "max |  |
| --- | --- | --- |
| 54 | COODINATOR_Rx_modulation_bandwidth" represents the maximum modulation bandwidth that the |  |
|  | 44 | Copyright © 20xx IEEE. All rights reserved. |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| receiver of the coordinator can support. "max DEVICE_Tx_modulation_bandwidth" represents the |  |  |  |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| maximum modulation bandwidth that the transmitter of a device can support. |  |  |  |  |  |  | 2 |  |
| The uplink PPDUs shall be modulated on the modulation bandwidth of BW_SEL2 that is equal to or lower |  |  |  |  |  |  | 3 |  |
|  |  |  |  |  |  |  | 4 |  |
| than MAX_BW_SEL2. MAX_BW_SEL2 equals to min{max COODINATOR_Rx_modulation_bandwidth, |  |  |  |  |  |  | 5 |  |
| max DEVICE_Tx_modulation_bandwidth}. And the PPDU carrying the corresponding ACK frames shall |  |  |  |  |  |  | 6 |  |
| be on the modulation bandwidth of BW_SEL1 that is equal to or lower than MAX_BW_SEL1. |  |  |  |  |  |  | 7 |  |
| MAX_BW_SEL1 | equals | to | min{max | COODINATOR_Tx_modulation | bandwidth, | max | 8 |  |
| DEVICE_Rx_modulation bandwidth}. |  |  |  |  |  |  | 9 |  |
| Regarding the broadcast downlink transmission, the coordinator shall modulate the frames on the minimum |  |  |  |  |  |  | 10 |  |
|  |  |  |  |  |  |  | 11 |  |
| bandwidth supported by any device. This minimum bandwidth is configured by the network and not in the |  |  |  |  |  |  | 12 |  |
| scope of this specification. |  |  |  |  |  |  | 13 |  |
| The following RTS/CTS protocol may be applied for bandwidth negotiation. The coordinator shall indicate |  |  |  |  |  |  | 14 |  |
|  |  |  |  |  |  |  | 15 |  |
| the application of the RTS/CTS protocol in the in the CAP Descriptor field of the beacon, see 5.2.2.1.4. If RTS/CTS protocol is disabled, the minimum bandwidth shall be used for transmissions in the CAP |  |  |  |  |  |  | 16 |  |
| A device may compete for the transmission of the RTS frame first. When the device gains the right for |  |  |  |  |  |  | 17 |  |
|  |  |  |  |  |  |  | 18 |  |
| transmission after contention it shall transmit an RTS frame. RTS is transmitted using the minimal |  |  |  |  |  |  | 19 |  |
| bandwidth. The RTS frame shall include the the modulation bandwidth that is recommended by the device, which |  |  |  |  |  |  | 20 |  |
| will be used by the coordinator to determine the bandwidth for the device to transmit the subsequent data/ |  |  |  |  |  |  | 21 |  |
| command frame. |  |  |  |  |  |  | 22 |  |
| Editor's note: John will provide a flowchart to explain the procedure of bandwidth switching using RTS/ |  |  |  |  |  |  | 23 |  |
|  |  |  |  |  |  |  | 24 |  |
| CTS, see comment 52 of https://mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m- |  |  |  |  |  |  | 25 |  |
| d1.xlsx |  |  |  |  |  |  | 26 |  |
| After receiving the RTS frame successfully, the coordinator shall transmit a CTS frame to the device that has |  |  |  |  |  |  | 27 |  |
|  |  |  |  |  |  |  | 28 |  |
| sent the RTS frame. Upon the evaluation of the recommended bandwidth in the received RTS frame, |  |  |  |  |  |  | 29 |  |
|  |  |  |  |  |  |  | 30 |  |
| (editor note: in subsection where CTS is specified. Please John provide text for CTS, see comment 56 of |  |  |  |  |  |  | 31 |  |
| https://mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx), the coordinator |  |  |  |  |  |  | 32 |  |
| shall inform the device about the bandwidth assigned for mutual communications using the CTS frame. |  |  |  |  |  |  | 33 |  |
| The CTS frame shall include the modulation bandwidth that is assigned by the coordinator to the device, and |  |  |  |  |  |  | 34 |  |
| which will be used for the device to transmit the subsequent data/command frame.. |  |  |  |  |  |  | 35 |  |
| The device shall transmit the data/command frame using the modulation bandwidth assigned in the CTS |  |  |  |  |  |  | 36 |  |
|  |  |  |  |  |  |  | 37 |  |
| frame, after if it has received the corresponding CTS frame after the RTS frame it has sent. If the device does |  |  |  |  |  |  | 38 |  |
| not receive the CTS frames during SIFS after it sent the RTS frame, it shall not transmit the data/command |  |  |  |  |  |  | 39 |  |
| frame and shall try to compete to resend the RTS frame. The coordinator may send an ACK frame following |  |  |  |  |  |  | 40 |  |
| the data/command frame, if requested. Transmission of a frame sequence including a data/command frame |  |  |  |  |  |  | 41 |  |
| with ACK using RTS/CTS is presented in Figure 25. |  |  |  |  |  |  | 42 |  |
| In this RTS/CTS mechanism, the coordinator and a device shall set the operation modulation bandwidth |  |  |  |  |  |  | 43 |  |
|  |  |  |  |  |  |  | 44 |  |
| according to the following rules: |  |  |  |  |  |  | 45 |  |
| The PPDU carrying a RTS/CTS/ACK frame shall be modulated on the minimum of the maximum |  |  |  |  |  |  | 46 |  |
|  |  |  |  |  |  |  | 47 |  |
| modulation bandwidths supported by both devices. The PPDUs carrying data/command shall be modulated |  |  |  |  |  |  | 48 |  |
| on the modulation bandwidth that is indicated in the CTS frame. |  |  |  |  |  |  | 49 |  |
| For the downlink transmissions, the coordinator shall use the same RTS/CTS protocol described above. |  |  |  |  |  |  | 50 |  |
|  |  |  |  |  |  |  | 51 |  |
|  |  |  |  |  |  |  | 52 |  |
|  |  |  |  |  |  |  | 53 |  |
|  |  |  |  |  |  |  | 54 |  |
|  |  | Copyright © 20xx IEEE. All rights reserved. |  |  |  | 45 |  |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 5.1.2.4 Polling |
| --- | --- |
| 2 |  |

- The start of a transmission period is marked by the transmission of a beacon frame, which carriers the infor-

- mation required for new STAs to initiate an association procedure. Following the beacon frame transmis-

- sion, a contention-based transmission period is enabled. The duration of the period is measured in terms of

- transmission slots, which the coordinator node enables via its polling mechanism. Each frame transmission

- (other than the Beacon frame transmission) by the coordinator can be used to poll one or multiple STAs.

- During the contention-based period, all STAs are effectively polled by the coordinator and, as soon as they

- recognize the beginning of the polling frame (polling information is contained in the frame header at the

- beginning of the frame), they have the possibility to transmit.  Collisions result in failed frame reception,

- which results in lack of acknowledgments and retransmission of the respective frames. Acknowledgments

- and retransmissions are described in subsection ….. A STA which does not take the opportunity to transmit

- when polled during the CP needs to wait for the next valid frame transmitted by the coordinator, which

- marks the beginning of the next contention slot, in order to have the opportunity to transmit again. The poll-

- ing mechanism enables very low-complexity synchronization of the transmission slots and also enables the

- coordinator to dynamically adapt and assign the slots for transmission. The duration of the CP is measured

- in terms of transmission slots and can be configured at the coordinator node. Following the CP, a CFP is

- enabled, during which the coordinator node polls each STA individually and allows it to transmit without

- any interference from other STAs. The specific polling algorithm and order is outside the scope of this spec-

- ification and is left as a system designer's choice. STA association and disassociation begins in the CP, but

- can be completed during the subsequent CFP. The concept is presented in Fig. 46.

| 22 |  |  |
| --- | --- | --- |
| 23 |  |  |
| 24 |  |  |
| 25 |  |  |
| 26 |  |  |
| 27 |  |  |
| 28 |  |  |
| 29 |  |  |
| 30 |  |  |
| 31 |  |  |
| 32 |  |  |
| 33 |  |  |
| 34 |  |  |
| 35 |  |  |
| 36 |  |  |
| 37 |  |  |
| 38 |  |  |
| 39 | Figure 26—Superframe structure example with four STAs |  |
| 40 |  |  |
| 41 |  |  |
| 42 | 5.1.2.4.1 Association example using polling |  |
| 43 |  |  |
| 44 |  |  |

- The AP sends a Beacon frame periodically at the beginning of a polling cycle. In a short contention period

- after receiving the Beacon frame, the STA can reply with a "connection request". The first valid downlink

- frame after the beacon marks the beginning of the contention period, i.e., the first valid frame after the bea-

- con is interpreted as a poll to all stations that have not been connected to transmit their "connection request".

- All stations that have already been associated with the AP may ignore any polling during the contention

- period. If no stations have been connected to the AP, the first valid frame would be the subsequent beacon

- frame (see Fig. 47 for a description of this procedure). The AP starts polling a station in the next polling

- round after receiving the "connection request". In case several STAs want to connect at the same time, a col-

- lision occurs. As a solution, a random backoff is introduced, for example, one STA retries to establish a con-

- nection after 2 beacons, another STA retries after 5 beacons, etc. If it is not polled during the next polling

46	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

round, a STA assumes a collision has occurred and proceeds accordingly with a back-off and an attempt to reconnect. A STA transmits a poll request frame immediately after it decodes a valid MAC frame header. Hence, as depicted in Fig. 47, it will not wait for the entire frame on the downlink from the AP to be trans-mitted. Upon a successful polling request (the AP has successfully received the station's request to be polled and has indicated this by polling the station within the next transmission round), a station exchanges the nec-essary association and authentication information with the AP via association and authentication control frames. Upon successful authentication and association, the STA is assigned a unique 16-bit address which will identify the STA among all other devices connected to the same AP via the VLC link.

(a) Case1: No STAs are connected to the AP

(b) Case2: At least one STA is connected to the AP

Figure 27—A STA sending a connection request to the AP

5.1.2.4.2 Multiple access example using polling

The multiple access in the presented MAC protocol is based on the function of a PCF which provides CF frame transfer. The PC shall reside in the AP. It is also a requirement for a STA to be able to respond to a poll request received from a PC. A STA should also be able to request to be polled by an active PC as described in subclause 6.2.1.9.1. When polled by the PC, a STA may transmit only one MPDU, which can be sent to the PC but may have any destination. The acknowledgments for any data packet "piggyback" on the trans-mission of any management or data frame (including Null frames). If a frame is not acknowledged, the STA shall not retransmit the frame unless it is polled again by the PC. If a polled STA does not have any data to transmit, it simply ignores the polling request. The request times out after a specified period of time, which indicates to the PC that the polled station might either be out of range or will not take advantage of the pos-sibility to transmit.

Any frames transmitted from the AP to the STA are treated as polling frames, except for the Beacon frame. This can be assumed because every packet transmission contains a polled STA number, which enables the stations to keep track of the polling order. A newly connected STA is made aware of its queue number with the first valid packet it receives from the AP, which contains both its queue number and its MAC address. Furthermore, the AP will never transmit a frame before the uplink from a STA due to a previous poll has

| Copyright © 20xx IEEE. All rights reserved. | 47 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- been completed. Hence, no collisions in the uplink are possible and each donwlink packet transmission can

- function as a polling frame. In case the response to a poll request is not detected by the AP, the AP will

- attempt to poll the next STA in the queue. Upon detection of a subsequent downlink packet by the STA

- whose response was not detected, the STA will stop transmission immediately in order to avoid any colli-

- sions in the uplink as illustrated in Figure 28. This functionality is necessary because the AP does not expect

- to always receive a response to its poll. If a STA has no acknowledgment or information to transmit, it will

7	simply ignore its possibility to transmit. An AP always sends polling frames in order. In case there is no

- information to be transmitted to a STA, which is next in the queue, a Data Null frame is used as a polling

- frame.

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

Figure 28—Consequences of the AP failing to recognize a poll response

5.1.2.4.3 Acknowledgement and Retransmission Example Using Polling

The reception of every packet at the MAC layer level has to be acknowledged by the receiving side (AP or STA). If a packet is not acknowledged by the STA or the AP at the following polling round, then it is retrans-mitted during the subsequent polling cycle. Packets may arrive out of order. Hence, even if a STA or an AP does not receive the expected acknowledgment during a given polling round, it can proceed with the trans-mission of the next packet. If packets are not acknowledged in the course of a predetermined time period equal to four transmission opportunities for the acknowledging device, they are considered lost and are dropped from the transmission. Figure 29 illustrates the acknowledgment procedure.

- The principle of operation is illustrated in Figure 29 for transmission on the downlink. The principle of oper-

- ation is analogous for the uplink direction. Two STAs are already connected to the AP. The AP sends a data

- frame to the first STA, which also polls the STA. The STA responds to the poll with a payload frame or a

- management frame as soon as it manages to decode the MAC header of the received frame. Note that if the

- high-reliability MAC header encoding at the PHY layer is implemented, then the STA responds as soon as

- this information is decoded at the PHY layer and the MLME is notified even before the entire DATA portion

- of the PHY packet is received and decoded. After both the downlink and the uplink packet transmissions are

- complete, the AP sends a data frame to the second STA, which also replies to the poll with a payload frame

- as soon as the PHY header is decoded. After completion of the transmission in both directions, the AP

- attempts to send a second data frame to the first STA. This time a CRC error is detected at the PHY layer, but

- the packet transmission is correctly detected by the STA, which interprets this as an invitation to transmit

- and responds with an acknowledgment to the first data packet in the downlink (note that if the high-reliabil-

- ity MAC header encoding at the PHY layer is not used, the entire packet would be dropped without the STA

- being polled, so uplink transmission would not be possible). The acknowledgment in this case is part of a

- Null frame, but can also be transmitted with a data frame if the STA had any data to upload. After transmis-

- sion is complete in both directions, the AP sends a second data packet to the second STA. The STA detects

48	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| this and after decoding the header, transmits an acknowledgment for the reception of the first data packet. |  | 1 |
| --- | --- | --- |
| Upon completion of both transmissions, the AP sends a third data packet to the first STA. The STA responds |  | 2 |
| with a management frame, but does not provide any acknowledgments as the second data packet was not |  | 3 |
| successfully received during the previous transmission cycle. In this case, the STA could simply ignore the |  | 4 |
| possibility to transmit if no management information or data is available for transmission. In both cases, the |  | 5 |
| AP would recognize the lack of an acknowledgment for the second data packet which failed to successfully |  | 6 |
| reach the first STA. Upon completion of transmission in both directions, the AP sends a third data packet to |  | 7 |
| the second STA. This time an error at the PHY layer causes the packet to not get detected at the PHY layer. |  | 8 |
| As a result, the STA does not recognize the poll and does not transmit any data. After completing transmis- |  | 9 |
| sion, the AP transmits another data frame to the first STA. In this case, the second packet is retransmitted |  | 10 |
| since no acknowledgment was received at the AP during the previous transmission cycle. At this point, the |  | 11 |
| AP will keep transmitting the second data packet at each polling cycle, until an acknowledgment is received |  | 12 |
| from the STA or until enough unsuccessful attempts are made so that the AP decides to drop the packet. The |  | 13 |
| STA correctly identifies the packet header and transmits a response with an acknowledgment for the third |  | 14 |
| data packet, which was received during the previous transmission round. Upon completion of both transmis- |  | 15 |
| sions, the AP sends the fourth data packet to the second STA. The STA recognizes the header and sends a |  | 16 |
| response in a management frame. In this case, again, the STA could simply ignore the possibility to transmit |  | 17 |
| if no management information or data is available for transmission. In both cases, the AP would recognize |  | 18 |
| the lack of an acknowledgment for the third data packet which failed to successfully reach the second STA. |  | 19 |
| Upon completion of the bidirectional transmission, the AP transmits the fourth data packet to the first STA. |  | 20 |
| The STA recognizes the header and transmits an acknowledgment for the second data packet, which was |  | 21 |
| successfully retransmitted during the previous round. After completion of the bidirectional transmission, the |  | 22 |
| AP retransmits the third data packet to the second STA for which it did not receive an acknowledgment in |  | 23 |
| the previous round. The STA recognizes the header and attempts to transmit an acknowledgment for the |  | 24 |
| fourth data packet which was successfully received during the previous round. The transmission, however, |  | 25 |
| contains at least one error, which causes the PHY layer algorithm at the AP to drop the data, and, hence, the |  | 26 |
| MAC layer algorithm does not receive the acknowledgment for the fourth data packet. Upon completion of |  | 27 |
| both data transmissions, the AP continues with transmission of the fifth data packet to the first STA. It |  | 28 |
| should be noted that if for any reason the AP failed to recognize the end of the uplink transmission from the |  | 29 |
| second STA, which could be due to the fact that the packet was not detected at all or due to the fact that |  | 30 |
| because of a PHY layer error the packet length was not correctly interpreted, and began transmitting the next |  | 31 |
| packet to the first STA prematurely, the second STA would have discontinued its transmission immediately |  | 32 |
| upon detecting the next donwlink packet transmission. This example was explained in subclause 6.2.1.9.2 |  | 33 |
| and illustrated in Fig. 49. All subsequent packet transmissions in Fig. 50 are successful and analogous to the |  | 34 |
| already explained examples. It is interesting to note that since the acknowledgment for the fourth data packet |  | 35 |
| to the second STA was unsuccessful, the AP transmits this packet a second time (in the last illustrated |  | 36 |
| frame), so the STA receives the packet successfully twice. The MAC layer protocol is able to handle the |  | 37 |
| packet redundancy as well as reordering based on the packet sequence numbers described in subclause |  | 38 |
| 6.4.1.2. |  | 39 |
|  |  | 40 |
|  |  | 41 |
|  |  | 42 |
|  |  | 43 |
|  |  | 44 |
|  |  | 45 |
|  |  | 46 |
|  |  | 47 |
|  |  | 48 |
|  |  | 49 |
|  |  | 50 |
|  |  | 51 |
|  |  | 52 |
|  |  | 53 |
|  |  | 54 |
| Copyright © 20xx IEEE. All rights reserved. | 49 |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

  - No packet loss

- Various packet loss and retransmission scenarios.

Figure 29—Downlink transmission with two STAs

5.1.2.5 Visibility support during channel access

|  |  |  | beacon |  |  |  | contention |  |  |  |  |  |  | uplink |  |  |  |  |  |  |  | downlink |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| star topology |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (infrastructure) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | send visibility pattern during |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | contention/uplink/unused |  |
|  |  |  |  |  |  |  |  |  |  | visibility for |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | slots in star toplogy |  |
|  |  |  |  |  |  |  |  | infrastructure (downlink) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| full-duplex |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| for visibility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | (a) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | unused slots |  |  |  |  |  |  |  |  | unused slots |  |  |  |  |  |  |  |  |  |
| all modes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | send visibility pattern during |  |
|  |  |  |  |  |  |  |  |  | visibility for point-and-shoot (uplink) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | unused slots in all |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | applications |  |
|  |  |  |  |  |  |  |  |  | and for infrastructure (downlink) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| full-duplex |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| for visibility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

(b)

Figure 30—Usage of CVD frames during idle or RX modes of operation

The visibility slots can be used during contention and uplink slots in star topology mode and unused slots in all modes to maintain visibility, reduce flicker and keep the transmitter always “ON” for the infrastructure. This is shown in Figure 30. Visibility support is a very important distinguishing feature for OWC. One may need to transmit idle patterns during receive and idle modes. This can be done by simultaneous reception of

50	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| data and the transmission of visibility or idle patterns. This is possible due to spatial separation of the light |  | 1 |  |
| --- | --- | --- | --- |
| source and the receiving circuitry. As shown in Figure 30, idle patterns are sent during contention, uplink |  | 2 |  |
| slots and unused downlink slots by the infrastructure to maintain visibility. Idle patterns are also sent during |  | 3 |  |
| unused slots by the mobile device to help with pointing and alignment for optimal data transfer. |  | 4 |  |
| If the continuous visibility bit is set in the capabilities field shown in Table 38, then infrastructure devices |  | 5 |  |
|  |  | 6 |  |
| shall provide continuous visibility. |  | 7 |  |
|  |  | 8 |  |
|  |  | 9 |  |
| 5.1.3 Starting a OWPAN |  | 10 |  |
|  |  | 11 |  |
| 5.1.3.1 Scanning through channels |  | 12 |  |
|  |  | 13 |  |
| A scan procedure shall be performed by a device or a prospective coordinator to detect any OWPANs and |  | 14 |  |
|  |  | 15 |  |
| coordinators that are operating in its vicinity. Two types of scanning are specified, active scan, passive scan, |  | 16 |  |
| and a third type, scan-over-backhaul, is enabled via the use of specified primitives. All devices shall be |  | 17 |  |
| capable of performing passive scans across a specified list of channels. In addition, a coordinator shall be |  | 18 |  |
| able to perform active scans. A device is instructed to begin a channel scan through the MLME- |  | 19 |  |
| SCAN.request primitive. For the duration of the scan, the device shall suspend beacon transmissions, if |  | 20 |  |
| applicable, and shall only accept frames received over the PHY data service that are relevant to the scan |  | 21 |  |
| being performed. Upon the conclusion of the scan, the device shall recommence beacon transmissions. The |  | 22 |  |
| results of the scan shall be returned via the MLME-SCAN.confirm primitive. |  | 23 |  |
| 5.1.3.1.1 Active channel scan |  | 24 |  |
|  |  | 25 |  |
| It is anticipated that the active channel scan is used with the peer-to-peer topology and the coordinated |  | 26 |  |
|  |  | 27 |  |
| topology. |  | 28 |  |
| An active scan allows a device to locate any coordinator transmitting beacon frames within its coverage |  | 29 |  |
|  |  | 30 |  |
| area. This could be used by a prospective OWPAN coordinator to select a OWPAN identifier prior to starting |  | 31 |  |
| a new OWPAN, or it could be used by a device prior to association. This could also be used by a device |  | 32 |  |
| when it is requested by the coordinator to report a neighboring OWPANs descriptors list. |  | 33 |  |
| During an active scan, the MAC sublayer shall discard all frames received over the PHY data service that |  | 34 |  |
|  |  | 35 |  |
| are not beacon frames. If a beacon frame is received that contains the address of the scanning device in its |  | 36 |  |
| list of pending addresses, the scanning device shall not attempt to extract the pending data. |  | 37 |  |
| Before commencing an active scan, the MAC sublayer shall store the value of macOWPANId and then set it |  | 38 |  |
|  |  | 39 |  |
| to 0xffff for the duration of the scan. This enables the receive filter to accept all beacons rather than just the |  | 40 |  |
| beacons from its current OWPAN, see 5.1.9.2. On completion of the scan, the MAC sublayer shall restore |  | 41 |  |
| the value of macOWPANId to the value stored before the scan began. An active scan |  | 42 |  |
| is requested using the MLME-SCAN.request primitive with the ScanType parameter set to |  | 43 |  |
| indicate an active scan. The device shall |  | 44 |  |
| send a beacon request command, see 5.4.10. Upon successful |  | 45 |  |
| transmission of the beacon request command, the device shall enable its receiver for | at most | 46 |  |
| [aBaseSuperframeDuration × (2n + 1)] optical clocks, where n is the value of the ScanDuration parameter. |  | 47 |  |
| During this time, the device shall reject all nonbeacon frames and record the information contained in all |  | 48 |  |
| unique beacons in a OWPAN descriptor structure, see 6.3.3.1, |  | 49 |  |
| . If a beacon frame is received when macAutoRequest is set to TRUE, the list of OWPAN |  | 50 |  |
| descriptor structures shall be stored by the MAC sublayer until the scan is complete; at this time, the list |  | 51 |  |
| shall be sent to the next higher layer in the OWPANDescriptorList parameter of the MLME-SCAN.confirm |  | 52 |  |
| primitive. A device shall be able to store between one and an implementation-specified maximum number of |  | 53 |  |
| OWPAN descriptors. A beacon frame shall be assumed to be unique if it contains both a OWPAN identifier |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 51 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- and a source address that has not been seen before during the scan. If a beacon frame

- is received when macAutoRequest is set to FALSE, each recorded OWPAN descriptor is sent to the next

| 3 | higher | layer in a separate MLME-BEACON-NOTIFY. indication primitive. A received beacon frame |
| --- | --- | --- |
| 4 | containing one or more octets of payload shall also cause the OWPAN descriptor to be sent to the next |  |
| 5 | higher | layer via the MLME-BEACON-NOTIFY.indication primitive. If a protected beacon frame is |

- received, i.e., the Security Enabled subfield in the frame control field is set to one, the device shall attempt to

- unsecure the beacon frame using the unsecuring process described in 7.2.3. The security-related elements of

8	the OWPAN descriptor corresponding to the beacon, see 6.3.3.1, shall be set to the corresponding

- parameters returned by the unsecuring process. The SecurityFailure element of the OWPAN descriptor shall

- be set to SUCCESS if the status from the unsecuring process is SUCCESS and set to one of the other status

- codes indicating an error in the security processing otherwise. The information from the unsecured frame

- shall be recorded in the OWPAN descriptor even if the status from the unsecuring process indicated an error.

- If a coordinator of a beacon-enabled OWPAN receives the beacon request command, it shall ignore the

- command and continue transmitting its periodic beacons as usual. If a coordinator of a nonbeacon-enabled

- OWPAN receives this command, it shall transmit a single beacon frame using unslotted random access or

- unslotted CSMA-CA.

17

- If macAutoRequest is set to TRUE, the active scan shall terminate when the number

- of beacons found equals the implementation-specified limit or the channel has been scanned for the full

- time, as specified in 5.1.3.1.1. If macAutoRequest is set to FALSE, the active scan

- shall terminate when the channel has been scanned for the full time. If a channel was not scanned for the full

- time, it shall be considered to be unscanned.

23

| 28 | 5.1.3.1.2 Passive channel scan |  |
| --- | --- | --- |
| 29 |  |  |
| 30 | It is anticipated that the passive channel scan is used with the star or broadcast topology. |  |
| 31 |  |  |
| 32 |  |  |

- A passive scan, like an active scan, allows a device to locate any coordinator transmitting beacon frames

- within its coverage area. The beacon request command, however, is not transmitted. This type of scan could

- be used by a device prior to association. During a passive scan, the MAC sublayer shall discard all frames

- received over the PHY data service that are not beacon frames. If a beacon frame is received that contains

- the address of the scanning device in its list of pending addresses, the scanning device shall not attempt to

- extract the pending data.

39

- Before commencing a passive scan, the MAC sublayer shall store the value of macOWPANId and then set it

- to 0xffff for the duration of the scan. This enables the receive filter to accept all beacons rather than just the

- beacons from its current OWPAN, see 5.1.9.2. On completion of the scan, the MAC sublayer shall restore

- the value of macOWPANId to the value stored before the scan began. A passive scan

- is requested using the MLME-SCAN.request primitive with the ScanType parameter set to

- indicate a passive scan. the device shall

- enable its receiver for at most [aBaseSuperframeDuration × (2n +

- 1)] optical clocks, where n is the value of the ScanDuration parameter. During this time, the device shall

- reject all nonbeacon frames and record the information contained in all unique beacons in a OWPAN

- descriptor structure, see 6.3.3.1. If a beacon frame is received when macAutoRequest is set to TRUE, the list

- of OWPAN descriptor structures shall be stored by the MAC sublayer until the scan is complete; at this time,

- the list shall be sent to the next higher layer in the OWPANDescriptorList parameter of the MLME-

- SCAN.confirm primitive. A device shall be able to store between one and an implementation- specified

- maximum number of OWPAN descriptors. A beacon frame shall be assumed to be unique if it contains both

- a OWPAN identifier and a source address that has not been seen before during the scan of the current

52	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| channel. If a beacon frame is received when macAutoRequest is set to FALSE, each recorded OWPAN |  | 1 |  |
| --- | --- | --- | --- |
| descriptor is sent to the next higher layer in a separate MLME-BEACON-NOTIFY. indication primitive. |  | 2 |  |
| Once the scan is complete, the MLME-SCAN.confirm shall be issued to the next higher layer with a null |  | 3 |  |
| OWPANDescriptorList. A received beacon frame containing one or more octets of payload shall also cause |  | 4 |  |
| the OWPAN descriptor to be sent to the next higher layer via the MLME-BEACON-NOTIFY. indication |  | 5 |  |
| primitive. |  | 6 |  |
| If a protected beacon frame is received (i.e., the Security Enabled subfield in the frame control field is set to |  | 7 |  |
|  |  | 8 |  |
| one), the device shall attempt to unsecure the beacon frame using the unsecuring process described in 7.2.3. |  | 9 |  |
| The security-related elements of the OWPAN descriptor corresponding to the beacon, as shown in 6.3.3.1, |  | 10 |  |
|  |  | 11 |  |
| shall be set to the corresponding parameters returned by the unsecuring process. The SecurityFailure |  | 12 |  |
| element of the OWPAN descriptor shall be set to SUCCESS if the status from the unsecuring process is |  | 13 |  |
| SUCCESS and set to one of the other status codes indicating an error in the security processing otherwise. |  | 14 |  |
| The information from the unsecured frame shall be recorded in the OWPAN descriptor even if the status |  | 15 |  |
|  |  | 16 |  |
| from the unsecuring process indicated an error. |  | 17 |  |
| If macAutoRequest is set to TRUE, the passive scan shall terminate when the number |  | 18 |  |
|  |  | 19 |  |
| of beacons found equals the implementation specified limit or the channel has been scanned for the full time. |  | 20 |  |
| If macAutoRequest is set to FALSE, the passive scan shall terminate when the |  | 21 |  |
| channel has been scanned for the full time. If a channel was not scanned for the full time, it shall be |  | 22 |  |
| considered to be unscanned. |  | 23 |  |
|  |  | 24 |  |
|  |  | 25 |  |
|  |  | 26 |  |
|  |  | 27 |  |
|  |  | 28 |  |
| 5.1.3.1.3 Scan-over-backhaul |  | 29 |  |
|  |  | 30 |  |
| Scan-over-backhaul shall be performed by a prospective coordinator that plans to establish a OWPAN if |  | 31 |  |
|  |  | 32 |  |
| inter-coordinator communication over backhaul is feasible. |  | 33 |  |
| Scan-over-backhaul is requested by the next higher layer of the prospective coordinator using the MLME- |  | 34 |  |
|  |  | 35 |  |
| SCAN.request primitive with the ScanType parameter set to indicate a scan-over-backhaul, as defined in |  | 36 |  |
| TBD. On reception of the primitive, the MLME of the prospective coordinator shall generate a scan-over- |  | 37 |  |
| backhaul request command (see TBD) and send it to neighboring coordinators that are connected to the |  | 38 |  |
| prospective coordinator through the backhaul. After the prospective coordinator transmitted the scan-over- |  | 39 |  |
| backhaul request command, it shall enable its receiver (on the backhaul) to receive scan-over-backhaul |  | 40 |  |
| confirmation commands (as specified in TBD) sent by other coordinators for ScanDuration. Coordinators |  | 41 |  |
| that have received the scan-over-backhaul request command through the backhaul shall respond with a scan- |  | 42 |  |
| over-backhaul confirmation command, with its own OWPAN descriptor embedded in it. When the MLME |  | 43 |  |
| of the prospective coordinator receives a scan-over-backhaul confirmation command, it shall record the |  | 44 |  |
| information contained in the unique scan-over-backhaul confirmation command in a local neighboring |  | 45 |  |
| OWPAN descriptor list. A scan-over-backhaul confirmation command frame shall be assumed to be unique |  | 46 |  |
| if it contains both a OWPAN ID and a source address that has not been seen before. When the ScanDuration |  | 47 |  |
| time expires, the MLME of the prospective coordinator shall report the local neighboring OWPAN |  | 48 |  |
| descriptor list to the next higher layer by issuing a MLME-SCAN.confirm primitive. |  | 49 |  |
| 5.1.3.2 OWPAN initiation |  | 50 |  |
|  |  | 51 |  |
|  |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 53 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

- exchange capabilities

- channel selection

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

54	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  |  | 1 |  |
| --- | --- | --- | --- |
|  |  | 2 |  |
|  |  | 3 |  |
|  |  | 4 |  |
|  |  | 5 |  |
|  |  | 6 |  |
|  |  | 7 |  |
|  |  | 8 |  |
|  |  | 9 |  |
| A prospective coordinator shall perform the following procedures to establish an OWPAN. On receipt of the primitive MLME-RESET.request, the MLME of the prospective coordinator shall turn off the transceiver by issuing a PLME-SET-TRX-STATE.request to the PHY layer, and then the MAC sublayer is set to its initial conditions, clearing all internal variables to their default values. The MLME of the device shall respond with a MLME-RESET.confirm primitive to notify the next higher layer the result of the reset operation. After the reset, the next higher layer of the prospective coordinator shall issue a MLME-SCAN.request primitive to require the MLME to perform a scan to discover other OWPANs in its vicinity. Two types of scan may be performed. If inter-coordinator communication over backhaul is feasible, then a scan-over-backhaul is performed. Otherwise, a passive scan shall be performed. The specific scan type shall be indicated by the next higher layer in the MLME-SCAN.request primitive. After the requested scan is done, the MLME shall issue a MLME-SCAN.confirm primitive to report the result of the scan. The next higher layer of the prospective coordinator shall select an OWPAN ID and the short address of the coordinator based on the OWPAN IDs and the short addresses recorded in the local neighboring OWPANs descriptor list obtained via the scan once it has received the MLME-SCAN.confirm. The MLME of the prospective coordinator shall select an OWPAN ID and short address that are different from the OWPAN IDs and short addressed obtained via the scan. Then the next higher layer shall provide the selected OWPAN ID and short address to the MLME by issuing a MLME-START.request primitive. On receipt of the MLME-START.request, the MLME of the prospective coordinator shall start a superframe and broadcast beacon frames with the selected parameters (e.g., OWPAN ID, short address) periodically. |  | 10 |  |
|  |  | 11 |  |
| 5.1.3.3 Beacon generation |  | 12 |  |
|  |  | 13 |  |
| A device shall be permitted to transmit beacon frames only if macShortAddress is not equal to 0xffff. |  | 14 |  |
|  |  | 15 |  |
| A coordinator shall use the MLME-START.request primitive to begin transmitting beacons only if the |  | 16 |  |
|  |  | 17 |  |
| BeaconOrder parameter is less than 15. The coordinator may begin beacon transmission as the coordinator |  | 18 |  |
| of a new OWPAN. |  | 19 |  |
| For the coordinator (i.e., the OWPANCoordinator parameter is set to TRUE), the MAC sublayer shall ignore |  | 20 |  |
|  |  | 21 |  |
| the StartTime parameter and begin beacon transmissions immediately. Setting the StartTime parameter to |  | 22 |  |
| zero shall also cause the MAC sublayer to begin beacon transmissions immediately. |  | 23 |  |
| On receipt of the MLME-START.request primitive, the MAC sublayer shall set the OWPAN identifier in |  | 24 |  |
|  |  | 25 |  |
| macOWPANId and use this value in the Source OWPAN Identifier field of the beacon frame. The address |  | 26 |  |
| used in the Source Address field of the beacon frame shall contain the value of aExtendedAddress if |  | 27 |  |
| macShortAddress is equal to 0xfffe or macShortAddress otherwise. |  | 28 |  |
| The time of transmission of the most recent beacon shall be recorded in macBeaconTxTime and shall be |  | 29 |  |
|  |  | 30 |  |
| computed so that its value is taken at the same position in each beacon frame, the location of which is |  | 31 |  |
| implementation specific. The position, which is specified by the macTimeStampOffset attribute, is the same |  | 32 |  |
| as that used in the timestamp of the incoming beacon frame, as described in 5.1.7.1. |  | 33 |  |
| All beacon frames shall be transmitted at the beginning of each superframe at an interval equal to aBase- |  | 34 |  |
|  |  | 35 |  |
| SuperframeDuration  2n optical clocks, where n is the value of macBeaconOrder (the construction of the |  | 36 |  |
| beacon frame is specified in 5.2.2.1). |  | 37 |  |
|  |  | 38 |  |
| Beacon transmissions shall be given priority over all other transmit and receive operations. |  | 39 |  |
| 5.1.3.4 Device discovery |  | 40 |  |
|  |  | 41 |  |
|  |  | 42 |  |
| The coordinator indicates its presence on a OWPAN to other devices by transmitting beacon frames. This |  | 43 |  |
| allows the other devices to perform device discovery. |  | 44 |  |
|  |  | 45 |  |
| Device discovery shall be performed at 11.67 Kbps with a 200 KHz optical clock for PHY I and at 1.25 |  | 46 |  |
| Mbps with a 3.75 MHz optical clock for PHY II. PHY III does not provide device discovery support and |  | 47 |  |
| shall rely on device discovery using PHY II before operating in that mode. The dimmed OOK mode can be |  | 48 |  |
| used to support dimming in the device discovery process. This mode is indicated using the MAC PIB |  | 49 |  |
| attribute, macUseDimmedOOKmode, as defined in Table 91. The MAC and PHY capabilities are exchanged |  | 50 |  |
| in the device discovery process. The clock rate support capabilities are also exchanged. Once the capabilities |  | 51 |  |
| are exchanged, regular data transmission mode resumes for all three PHY types. Device discovery requires |  | 52 |  |
| bi-directional communication and is not applicable for broadcasting. |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 55 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 5.1.4 Neighboring OWPAN status monitoring |
| --- | --- |
| 2 |  |

- Neighboring OWPANs status monitoring procedure allows the coordinator and the devices to detect and

- maintain neighboring OWPANs information. Neighboring OWPAN status monitoring maybe performed for

- the purposes of OWPAN maintenance, interference coordination and handover.

6

7	Both coordinators and the devices shall participate in the neighboring OWPANs status monitoring

- procedure. The information of neighboring OWPANs that can be detected shall be recorded in a neighboring

- OWPANs descriptor list.

10

- Both the coordinator and the devices shall each maintain a local neighboring OWPANs descriptor list

- respectively. The local neighboring OWPAN descriptor list is maintained by receiving beacon frames or

- other frames from the neighboring OWPANs. When a beacon frame or any other frame from neighboring

- OWPANs is received for the first time, a new record corresponding to the neighboring OWPAN shall be

- added to the list. The record in the list has an ageing time of macAgeingTime superframes. Whenever a

- beacon frame or any other frame from a neighboring OWPAN corresponding to a record in the list has been

- received, the device shall update the LastTimeDetected of the record instead of adding a new record. Once

- the beacon frames or any other frame from a neighboring OWPAN corresponding to a record in the list has

- not been received within the ageing time since last time it was detected, the record corresponding to that

- neighboring OWPAN shall be deleted from the list.

21

- The devices shall update their local neighboring OWPANs descriptor list and send a neighboring OWPAN

- report indication command to the coordinator to report its local neighboring OWPANs descriptor list

- whenever any of the following events occurs:

| 25 | a) It has received a beacon frame or any other frames sent by the neighboring coordinators or devices |  |
| --- | --- | --- |
| 26 |  |  |
| 27 | for the first time; |  |

- b)   It has been macLastDetect superframes since the neighboring OWPAN was detected last time;

- c)   A neighboring OWPAN report request command is received by the device from the coordinator;

- The coordinator shall update its local neighboring OWPAN descriptor list whenever any of the following

| 32 | events occurs: |  |  |
| --- | --- | --- | --- |
| 33 | a) | It has received a beacon frame or any other frames sent by the neighboring coordinators or devices |  |
| 34 |  |  |  |
| 35 |  | for the first time; |  |
| 36 | b) | It has been macLastDetect superframes since the neighboring OWPAN was detected last time; |  |
| 37 |  |  |  |

- The coordinator shall also maintain a global neighboring OWPAN descriptor list. The global neighboring

- OWPAN descriptor list is maintained and updated by gathering the reported local neighboring OWPAN

- descriptor lists from devices and its own local neighboring OWPAN descriptor list. The global neighboring

- OWPANs descriptor list is shown in Table 4.

42

- The coordinator shall update its global neighboring OWPAN descriptors list whenever it has received a

- neighboring OWPAN report indication command from devices in the OWPAN, or when its own local

- neighboring OWPAN descriptors list has been updated. The coordinator shall also update the global

- neighboring OWPAN descriptors list by deleting the corresponding information if the device which has

- reported the corresponding information is disassociated from the OWPAN.

48

49	5.1.5 Maintaining OWPANs

50

- In some instances a situation could occur in which two OWPANs exist in the same operating space with the

- same OWPAN identifier. If this conflict happens, the coordinator and its devices shall perform the OWPAN

- identifier conflict resolution procedure.

54

56	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 4—Global neighboring OWPANs description list

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
| NumberNeighbor- |  |  | The number of records of neighboring OWPANs |  |
| ingOWPANRe- | Integer | 0x00 - 0xff |  |  |
|  |  |  | that are maintained by the coordinator. |  |
| cords |  |  |  |  |
| OWPANDescrip- |  | Refer to Table yy | The record of the first neighboring OWPAN that can |  |
| tion[0] |  |  | be detected by devices. |  |
| NumberDetectDe- |  |  | The number of devices that have detected the neigh- |  |
|  | Integer | 000 - oxff | boring OWPAN specified by OWPANDescriptor |  |
| vice |  |  |  |  |
|  |  |  | [0]. Assuming the total number is K. |  |
|  |  |  | The short address of the first device that has |  |
| DeviceAddr[0] | Integer |  | reported it can detect the neighboring OWPAN |  |
|  |  |  | specified by OWPANDescriptor [0]. |  |
|  |  |  | The RSS at which the first device receives the bea- |  |
| LinkQuality[0] |  |  | con frame or other frames from the OWPAN speci- |  |
|  |  |  | fied by OWPANDescriptor [0]. |  |
|  |  |  | The short address of the Kth device that has |  |
| DeviceAddr[k-1] | Integer |  | reported it can detect the neighboring OWPAN |  |
|  |  |  | specified by OWPANDescriptor [0]. |  |
|  |  |  | The RSS at which the Kth device receives the bea- |  |
| LinkQuality[K-1] |  |  | con frame or other frames from the OWPAN speci- |  |
|  |  |  | fied by OWPANDescriptor [0]. |  |
| OWPANDescrip- |  | Refer to Table yy | The record of the Nth neighboring OWPAN that can |  |
| tor[N-1] |  |  | be detected by devices. |  |
| NumberDetectDe- |  |  | The number of devices that have detected the neigh- |  |
|  | Integer | 0x00 - 0xff | boring OWPAN specified by OWPANDescriptor |  |
| vice |  |  |  |  |
|  |  |  | [N-1]. Assuming the total number is L. |  |
|  |  |  | The short address of the first device that has |  |
| DeviceAddr[0] | Integer |  | reported it can detect the neighboring OWPAN |  |
|  |  |  | specified by OWPANDescriptor [N-1]. |  |
|  |  |  | The RSS at which the first device receives the bea- |  |
| LinkQuality[0] |  |  | con frame or other frames from the OWPAN speci- |  |
|  |  |  | fied by OWPANDescriptor [N-1]. |  |
|  |  |  | The short address of the Lth device that has reported |  |
| DeviceAddr[L-1] | Integer |  | it can detect the neighboring OWPAN specified by |  |
|  |  |  | OWPANDescriptor [N-1]. |  |
|  |  |  | The RSS at which the Lth device receives the bea- |  |
| LinkQuality[L-1] |  |  | con frame or other frames from the OWPAN speci- |  |
|  |  |  | fied by OWPANDescriptor [N-1]. |  |

This procedure is optional for a device.

5.1.5.1 Detection

The OWPAN coordinator shall conclude that a OWPAN identifier conflict is present if either of the following apply:

— A beacon frame is received by the OWPAN coordinator with the OWPAN Coordinator subfield, see 5.2.2.1.2, set to one and the OWPAN identifier equal to macOWPANId.

| Copyright © 20xx IEEE. All rights reserved. | 57 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1 — A OWPAN ID conflict notification command, see 5.4.7, is received by the OWPAN coordinator 2 from an associated device on its OWPAN.

3

- A device that is associated through the OWPAN coordinator (i.e., macAssociatedOWPANCoord is set to

- TRUE) shall conclude that a OWPAN identifier conflict is present if the following applies:

| 6 | — A beacon frame is received by the device with the OWPAN Coordinator subfield set to one, the |  |
| --- | --- | --- |
| 7 |  |  |
| 8 | OWPAN identifier equal to macOWPANId, and an address that is equal to neither |  |
| 9 | macCoordShortAddress nor macCoordExtendedAddress. |  |
| 10 | 5.1.5.2 Resolution |  |
| 11 |  |  |
| 12 | On the detection of a OWPAN identifier conflict by a device, it shall generate the OWPAN ID conflict noti- |  |
| 13 | fication command, defined in 6.7.5, and send it to its coordinator. Since the OWPAN ID conflict notification |  |
| 14 | command contains an acknowledgment request (see 6.7.3.1), the coordinator shall confirm its receipt by |  |
| 15 | sending an acknowledgment frame. Once the device has received the acknowledgment frame from the coor- |  |
| 16 | dinator, the MLME shall issue an MLME -SYNC -LOSS.indication primitive with the LossReason parame- |  |
| 17 | ter set to OWPAN_ID_CONFLICT. If the device does not receive an acknowledgment frame, the MLME |  |
| 18 | shall not inform the next higher layer of the OWPAN identifier conflict. |  |
| 19 |  |  |
| 20 | On the detection of a OWPAN identifier conflict by the coordinator, the MLME shall issue an MLME- |  |
| 21 | SYNC-LOSS.indication to the next higher layer with the LossReason parameter set to OWPAN_ID_CON- |  |
| 22 | FLICT. The next higher layer of the coordinator may then perform an active scan and, using the information |  |
| 23 | from the scan, select a new OWPAN identifier. |  |
| 24 |  |  |

- In addition, the next higher layer of the coordinator shall issue a MLME-NeighborReport.request primitive

- to its MLME to trigger a neighboring OWPAN report. On receipt of the MLME-NeighborReport.request

- primitive, the MLME of the coordinator shall generate a neighboring OWPAN report request command and

- send it to the device(s) indicated in the ReportDevicesList of the MLME-NeighborOWPANReport.request

- primitive. To which device(s) the neighboring OWPAN request command shall be sent is determined by the

- next higher layer and is out of the scope of the specification. Then the coordinator shall wait for the neigh-

- boring OWPAN report indication commands sent by the devices for macResponseWaitTime optical clocks.

| 33 | Devices that have received the neighboring OWPAN report request command from the coordinator may first |  |
| --- | --- | --- |
| 34 | perform an active scan. The device updates its local neighboring OWPAN descriptors list according to the |  |
| 35 | result obtained from the active scan. When the active scan requested by the coordinator is done, the device |  |
| 36 | shall generate a neighboring OWPAN report indication command based on its updated local neighboring |  |
| 37 | OWPAN descriptor list and send the neighboring OWPAN report indication command to the coordinator. If |  |
| 38 | the device choose not to perform the active scan after it receives the neighboring OWPAN report request |  |
| 39 | command, it shall generate a neighboring OWPAN report indication command which include its local neigh- |  |
| 40 | boring OWPAN descriptor list and send the command to the coordinator. |  |
| 41 |  |  |
| 42 | When the coordinator has received a unique neighboring OWPAN report indication command from the |  |
| 43 | device, it shall update its global neighboring OWPAN descriptor list. When macResponseWaitTime optical |  |
| 44 | clocks elapsed, the MLME of the coordinator shall notify its next higher layer the result of the requested |  |
| 45 | neighboring OWPAN report by issuing a MLME-NeighborReport.confirm primitive. The next higher layer |  |
| 46 | of the coordinator shall select a new OWPAN ID which is different from the OWPAN IDs recorded in its updated global |  |
| 47 | neighboring OWPAN descriptor list. The algorithm for selecting a suitable OWPAN identifier is out of the |  |
| 48 | scope of this standard. If the next higher layer does select a new OWPAN identifier, it may then issue an |  |
| 49 | MLME-START.request with the CoordRealignment parameter set to TRUE in order to realign the OWPAN, |  |
| 50 | as described in 6.2.3.3. |  |
| 51 |  |  |
| 52 |  |  |
| 53 |  |  |
| 54 |  |  |
|  | 58 | Copyright © 20xx IEEE. All rights reserved. |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 5.1.5.3 Realigning a OWPAN |  | 1 |  |
| --- | --- | --- | --- |
| If a coordinator receives the MLME-START.request primitive (see 7.3.11.1) with the CoordRealignment |  | 2 |  |
|  |  | 3 |  |
| parameter set to TRUE, the coordinator shall attempt to transmit a coordinator realignment command |  | 4 |  |
| containing the new parameters (e.g. OWPANId) and EffectiveTime. |  | 5 |  |
| When the coordinator is already transmitting beacons and the CoordRealignment parameter is set to TRUE, |  | 6 |  |
|  |  | 7 |  |
| the next scheduled beacon shall be transmitted on the current channel using the current superframe |  | 8 |  |
| configuration, with the frame pending subfield of the frame control field set to one. Immediately following |  | 9 |  |
| the transmission of the beacon, the coordinator realignment command shall also be transmitted on the |  | 10 |  |
| current channel using unslotted random access. |  | 11 |  |
| When the coordinator is not already transmitting beacons and the CoordRealignment parameter is set to |  | 12 |  |
|  |  | 13 |  |
| TRUE, the coordinator realignment command shall be transmitted immediately on the current channel using |  | 14 |  |
| unslotted random access. |  | 15 |  |
| If the transmission of the coordinator realignment command fails due to a channel access failure, the MLME |  | 16 |  |
|  |  | 17 |  |
| shall notify the next higher layer by issuing the MLME -START.confirm primitive with a status of |  | 18 |  |
| CHANNEL_ACCESS_FAILURE. The next higher layer may then choose to issue the MLME- |  | 19 |  |
| START.request primitive again. |  | 20 |  |
| Upon successful transmission of the coordinator realignment command, the new superframe configuration |  | 21 |  |
|  |  | 22 |  |
| and channel parameters shall be put into operation as described below. The coordinator realignment |  | 23 |  |
| command shall indicate in which superframe the new parameters will take effect by setting a proper value to |  | 24 |  |
| the Effective Time field. The coordinator shall also indicate it by setting the CountDown field in the |  | 25 |  |
| following beacon frames. Both the coordinator and devices shall make sure the new parameters are properly |  | 26 |  |
| set and start to use the new parameters (e.g., the selected OWPAN ID) when the effective time arrives. |  | 27 |  |
| If the coordinator is not already transmitting beacons, the new superframe configuration and channel |  | 28 |  |
|  |  | 29 |  |
| parameters shall be put into operation immediately. |  | 30 |  |
| Then the MAC sublayer shall issue the MLME-START.confirm primitive with a status of SUCCESS. |  | 31 |  |
|  |  | 32 |  |
| 5.1.5.4 Realignment in a OWPAN |  | 33 |  |
|  |  | 34 |  |
| If a device has received the coordinator realignment command (see 5.4.12) from the coordinator through |  | 35 |  |
|  |  | 36 |  |
| which it is associated, the MLME shall issue the MLME-SYNC-LOSS.indication primitive with the |  | 37 |  |
| LossReason parameter set to REALIGNMENT and the OWPANId, LogicalChannel, and the security- |  | 38 |  |
| related parameters set to the respective fields in the coordinator realignment command. The next higher |  | 39 |  |
| layer of a coordinator may then issue an MLME-START.request primitive with the CoordRealignment |  | 40 |  |
| parameter set to TRUE. The next higher layer of a device that is not a coordinator may instead change the |  | 41 |  |
| superframe configuration or channel parameters through use of the MLME-SET.request primitive. |  | 42 |  |
| 5.1.5.5 Updating superframe configuration and channel PIB attributes |  | 43 |  |
|  |  | 44 |  |
| To update the superframe configuration and channel attributes, the MLME shall assign values from the |  | 45 |  |
|  |  | 46 |  |
| MLME-START.request primitive parameters to the appropriate PIB attributes. The MLME shall set |  | 47 |  |
| macBeaconOrder to the value of the BeaconOrder parameter. If macBeaconOrder is equal to 15, the MLME |  | 48 |  |
| will also set macSuperframeOrder to 15. In this case, this primitive configures a nonbeacon-enabled |  | 49 |  |
| OWPAN. If macBeaconOrder is less than 15, the MAC sublayer will set macSuperframeOrder to the value |  | 50 |  |
| of the SuperframeOrder parameter. The MAC sublayer shall also update macOWPANId with the value of the |  | 51 |  |
| OWPANId parameter and update phyCurrentChannel with the values of the LogicalChannel parameters by |  | 52 |  |
| issuing the PLME-SET.request primitive. |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 59 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 5.1.6 Association and disassociation |  |
| --- | --- | --- |
| 2 | This subclause specifies the procedures for association and disassociation. |  |
| 3 |  |  |
| 4 | 5.1.6.1 Association |  |
| 5 |  |  |
| 6 | A device shall attempt to associate only after having first performed a MAC sublayer reset, by issuing the |  |
| 7 |  |  |
| 8 | MLME-RESET.request primitive with the SetDefaultPIB parameter set to TRUE, and then having |  |

- completed either an active channel scan, see 5.1.3.1.1, or a passive channel scan as shown in 5.1.3.1.2. The

- results of the channel scan would have then been used to choose a suitable OWPAN. The algorithm for

- selecting a suitable OWPAN with which to associate from the list of OWPAN descriptors returned from the

- channel scan procedure is out of the scope of this standard.

13

- If no OWPAN is detected, the next higher layer of the device may request the MLME to send an additional

- beacon request command (see 9.5.8) by issuing a MLME-AdditionalBeacon.request primitive. After sending the additional beacon request command, the device shall

- start a passive scan to discover beacon frames or additional beacon frames during Tcoordscan, as descriped in 5.1.3.1.2. The results  of  the  channel  scan  during Tcoordscan would  have  then  been  also used  to  choose  a  suitable  OWPAN. If

- the device cannot detect any beacon when the Tcoordscan expires, the next higher layer of the device may request attempt to send another additional

- beacon request command.

19

- Following the selection of a OWPAN with which to associate, the next higher layers shall request through

- the MLME-ASSOCIATE.request primitive that the MLME configures the following PHY and MAC PIB

- attributes to the values necessary for association:

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

— macOWPANId shall be set equal to the CoordOWPANId parameter of the MLME-ASSOCIATE.request primitive.

— macCoordExtendedAddress or macCoordShortAddress, depending on which is known from the beacon frame from the coordinator through which it wishes to associate, shall be set equal to the CoordAddress parameter of the MLME-ASSOCIATE.request primitive.

A coordinator shall allow association only if macAssociationPermit is set to TRUE. Similarly, a device should attempt to associate only with a OWPAN through a coordinator that is currently allowing association, as indicated in the results of the scanning procedure. If a coordinator with macAssociationPermit set to FALSE receives an association request command from a device, the command shall be ignored.

A coordinator receives the additional beacon request commands from a device may start to send additional beacons in each superframe from the next superframe in addition to the original beacons. The coordinator shall allocate a GTS in the CFP to transmit the additional beacon frames. The content of the additional beacon frame shall be the same as the beacon sent in the BP in the same superframe except the beacon type and shall be modulated on the minimum bandwidth. The BeaconType field in the beacon frame can be used to identify the type of the beacons. If the coordinator receives another additional beacon request command from the same device within macResponseWaitTime optical clocks, it may allocate a different GTS to transmit in the CFP to transmit additional beacon frames from the next superframe. If the coordinator receives an association request from the device that sent the additional beacon request command within macResponseWaitTime optical clocks, it shall handle the association request. If the coordinator does not receive any association request from the device that sent the additional beacon request command within macResponseWaitTime optical clocks, it may stop transmitting the additional beacons and only transmit the original beacons.

In order to optimize the association procedure on a beacon-enabled OWPAN, a device may begin tracking the beacon of the coordinator through which it wishes to associate. This is achieved by the next higher layer issuing the MLME-SYNC.request primitive with the TrackBeacon parameter set to TRUE.

60	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| A device that is instructed to associate with a OWPAN, through the MLME-ASSOCIATE.request primitive, |  | 1 |  |
| --- | --- | --- | --- |
| shall try to associate only with an existing OWPAN and shall not attempt to start its own OWPAN. |  | 2 |  |
| The MAC sublayer of an unassociated device shall initiate the association procedure by sending an |  | 3 |  |
|  |  | 4 |  |
| association request command, see 5.4.1, to the coordinator of an existing OWPAN; if the association request |  | 5 |  |
| command cannot be sent due to a channel access failure, the MAC sublayer shall notify the next higher |  | 6 |  |
| layer. Because the association request command contains an acknowledgment request (see 5.4.1.1), the |  | 7 |  |
| coordinator shall confirm its receipt by sending an acknowledgment frame. |  | 8 |  |
| The acknowledgment to an association request command does not mean that the device has associated. The |  | 9 |  |
|  |  | 10 |  |
| next higher layer of the coordinator needs time to determine whether the current resources available on the |  | 11 |  |
| OWPAN are sufficient to allow another device to associate. The next higher layer should make this decision |  | 12 |  |
| within macResponseWaitTime optical clocks. If the next higher layer of the coordinator finds that the device |  | 13 |  |
| was previously associated on its OWPAN, all previously obtained device-specific information should be |  | 14 |  |
| removed. If sufficient resources are available, the next higher layer should allocate a 16-bit short address to |  | 15 |  |
| the device, and the MAC sublayer shall generate an association response command, see 5.4.2, containing the |  | 16 |  |
| new address and a status indicating a successful association. If sufficient resources are not available, the next |  | 17 |  |
| higher layer of the coordinator should inform the MAC sublayer, and the MLME shall generate an |  | 18 |  |
| association response command containing a status indicating a failure as shown in Table 30. The association |  | 19 |  |
| response command shall be sent to the device requesting association using indirect transmission, i.e., the |  | 20 |  |
| association response command frame shall be added to the list of pending transactions stored on the |  | 21 |  |
| coordinator and extracted at the discretion of the device concerned using the method described in 5.1.9.3. |  | 22 |  |
| If the allocate address subfield of the capability information field (see 5.4.26.1.1) of the association request |  | 23 |  |
|  |  | 24 |  |
| command is set to one, the next higher layer of the coordinator shall allocate a 16-bit address with a range |  | 25 |  |
| depending on the addressing mode supported by the coordinator, as described in Table 5. If the Allocate |  | 26 |  |
| Address subfield of the association request command is set to zero, the 16-bit short address shall be equal to |  | 27 |  |
| 0xfffe. A short address of 0xfffe is a special case that indicates that the device has associated, but has not |  | 28 |  |
| been allocated a short address by the coordinator. In this case, the device shall use only its 64-bit extended |  | 29 |  |
| address to operate on the network. |  | 30 |  |
| On receipt of the acknowledgment to the association request command, the device shall wait for at most |  | 31 |  |
|  |  | 32 |  |
| macResponseWaitTime optical clocks for the coordinator to make its association decision; the PIB attribute |  | 33 |  |
| macResponseWaitTime is a network-topology-dependent parameter and may be set to match the specific |  | 34 |  |
| requirements of the network that a device is trying to join. If the device is tracking the beacon, it shall |  | 35 |  |
| attempt to extract the association response command from the coordinator whenever it is indicated in the |  | 36 |  |
| beacon frame. If the device is not tracking the beacon, it shall attempt to extract the association response |  | 37 |  |
| command from the coordinator after macResponseWaitTime optical clocks. If the device does not extract an |  | 38 |  |
| association response command frame from the coordinator within macResponseWaitTime optical clocks, the |  | 39 |  |
| MLME shall issue the MLME-ASSOCIATE.confirm primitive with a status of NO_DATA, and the |  | 40 |  |
| association attempt shall be deemed a failure. In this case, the next higher layer shall terminate any tracking |  | 41 |  |
| of the beacon. This is achieved by issuing the MLME-SYNC.request primitive with the TrackBeacon |  | 42 |  |
| parameter set to FALSE. |  | 43 |  |
| The MLME-ASSOCIATE.response and the subsequent Association response (see 5.4.2) also contain |  | 44 |  |
|  |  | 45 |  |
| information about what capabilities the device and the coordinator will and will not use during future |  | 46 |  |
| communication. |  | 47 |  |
| Because the association response command contains an acknowledgment request (see 5.4.2.1), the device |  | 48 |  |
|  |  | 49 |  |
| requesting association shall confirm its receipt by sending an acknowledgment frame. If the Association |  | 50 |  |
| Status field of the command indicates that the association was successful, the device shall store the address |  | 51 |  |
| contained in the 16-bit Short Address field of the command in macShortAddress; communication on the |  | 52 |  |
| OWPAN using this short address shall depend on its range, as described in Table 5. If the original beacon |  | 53 |  |
| selected for association following a scan contained the short address of the coordinator, the extended address |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 61 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- of the coordinator, contained in the MHR of the association response command frame, shall be stored in

- macCoordExtendedAddress.

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

|  | Table 5—Usage of the 16-bit short address |  |
| --- | --- | --- |
| Value of macShortAddress |  | Description |
| 0x0000–0xfffd |  | If a source address is included, the device shall use short source addressing |
|  |  | mode for beacon and data frames and the appropriate source addressing |
|  |  | mode specified in 5.4 for MAC command frames. |
| 0xfffe |  | If a source address is included, the device shall use extended source |
|  |  | addressing mode for beacon and data frames and the appropriate source |
|  |  | addressing mode specified in 5.4 for MAC command frames. |
| 0xffff |  | The device is not associated and, therefore, shall not perform any data |
|  |  | frame communication. The device shall use the appropriate source |
|  |  | addressing mode specified in 5.4 for MAC command frames. |

- If the Association Status field of the command indicates that the association was unsuccessful, the device

- shall set macOWPANId to the default value (0xffff).

| 20 |  |
| --- | --- |
| 21 | 5.1.6.2 Disassociation |
| 22 |  |
| 23 | The disassociation procedure is initiated by the next higher layer by issuing the MLME- |
| 24 | DISASSOCIATE.request primitive to the MLME. |
| 25 |  |

- When a coordinator wants one of its associated devices to leave the OWPAN, the MLME of the coordinator

- shall send the disassociation notification command in the manner specified by the TxIndirect parameter of

- the MLME-DISASSOCIATE.request primitive previously sent by the next higher layer. If TxIndirect is

- TRUE, the MLME of the coordinator shall send the disassociation notification command to the device using

- indirect transmission, i.e., the disassociation notification command frame shall be added to the list of

- pending transactions stored on the coordinator and extracted at the discretion of the device concerned using

- the method described in 5.1.9.3. If the command frame is not successfully extracted by the device, the

- coordinator should consider the device disassociated. Otherwise, the MLME shall send the disassociation

- notification command to the device directly. In this case, if the disassociation notification command cannot

- be sent due to a channel access failure, the MAC sublayer shall notify the next higher layer.

36

- Because the disassociation command contains an acknowledgment request (see 5.4.3.1), the receiving

- device shall confirm its receipt by sending an acknowledgment frame. If the direct or indirect transmission

- fails, the coordinator should consider the device disassociated.

40

- If an associated device wants to leave the OWPAN, the MLME of the device shall send a disassociation

- notification command to its coordinator. If the disassociation notification command cannot be sent due to a

- channel access failure, the MAC sublayer shall notify the next higher layer. Because the disassociation

- command contains an acknowledgment request (see 5.4.3.1), the coordinator shall confirm its receipt by

- sending an acknowledgment frame. However, even if the acknowledgment is not received, the device should

- consider itself disassociated.

| 47 |  |
| --- | --- |
| 48 | If the source address contained in the disassociation notification command is equal to macCoord- |
| 49 | ExtendedAddress, the device should consider itself disassociated. If the command is received by a |

- coordinator and the source is not equal to macCoordExtendedAddress, it shall verify that the source address

- corresponds to one of its associated devices; if so, the coordinator should consider the device disassociated.

- If none of the conditions for disassociation are satisfied, the command shall be ignored.

53

54

62	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| An associated device shall disassociate itself by removing all references to the OWPAN; the MLME shall set |  |  |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| macOWPANId, | macShortAddress, | macAssociatedOWPANCoord, | macCoordShortAddress |  | and | 2 |  |
| macCoordExtendedAddress to the default values. The next higher layer of a coordinator should disassociate |  |  |  |  |  | 3 |  |
| a device by removing all references to that device. |  |  |  |  |  | 4 |  |
| The next higher layer of the requesting device shall be notified of the result of the disassociation procedure |  |  |  |  |  | 5 |  |
|  |  |  |  |  |  | 6 |  |
| through the MLME-DISASSOCIATE.confirm primitive. |  |  |  |  |  | 7 |  |
| 5.1.7 Synchronization |  |  |  |  |  | 8 |  |
|  |  |  |  |  |  | 9 |  |
| This subclause specifies the procedures for coordinators to generate beacon frames and for devices to |  |  |  |  |  | 10 |  |
|  |  |  |  |  |  | 11 |  |
| synchronize with a coordinator. For OWPANs supporting beacons, synchronization is performed by |  |  |  |  |  | 12 |  |
| receiving and decoding the beacon frames. For OWPANs not supporting beacons, synchronization is |  |  |  |  |  | 13 |  |
| performed by polling the coordinator for data. |  |  |  |  |  | 14 |  |
| 5.1.7.1 Synchronization with beacons |  |  |  |  |  | 15 |  |
|  |  |  |  |  |  | 16 |  |
| All devices operating on a beacon-enabled OWPAN (i.e., macBeaconOrder < 15) shall be able to acquire |  |  |  |  |  | 17 |  |
|  |  |  |  |  |  | 18 |  |
| beacon synchronization in order to detect any pending messages or to track the beacon. Devices shall be |  |  |  |  |  | 19 |  |
| permitted to acquire beacon synchronization only with beacons containing the OWPAN identifier specified |  |  |  |  |  | 20 |  |
| in macOWPANId. If macOWPANId specifies the broadcast OWPAN identifier (0xffff), a device shall not |  |  |  |  |  | 21 |  |
| attempt to acquire beacon synchronization. |  |  |  |  |  | 22 |  |
| A device is instructed to attempt to acquire the beacon through the MLME-SYNC.request primitive. If |  |  |  |  |  | 23 |  |
|  |  |  |  |  |  | 24 |  |
| tracking is specified in the MLME-SYNC.request primitive, the device shall attempt to acquire the beacon |  |  |  |  |  | 25 |  |
| and keep track of it by regular and timely activation of its receiver. If tracking is not specified, the device |  |  |  |  |  | 26 |  |
| shall either attempt to acquire the beacon only once or terminate the tracking after the next beacon if |  |  |  |  |  | 27 |  |
| tracking was enabled through a previous request. |  |  |  |  |  | 28 |  |
| To acquire beacon synchronization, |  | a device shall enable its receiver and |  | search for at | most | 29 |  |
|  |  |  |  |  |  | 30 |  |
| [aBaseSuperframeDuration  (2n + 1)] optical clocks, where n is the value of macBeaconOrder. If a beacon |  |  |  |  |  | 31 |  |
| frame containing the current OWPAN identifier of the device is not received, the MLME shall repeat this |  |  |  |  |  | 32 |  |
| search. Once the number of missed beacons reaches aMaxLostBeacons, the MLME shall notify the next |  |  |  |  |  | 33 |  |
| higher layer by issuing the MLME-SYNC-LOSS.indication primitive with |  |  |  | a loss reason of |  | 34 |  |
| BEACON_LOSS. |  |  |  |  |  | 35 |  |
|  |  |  |  |  |  | 36 |  |
| The MLME shall timestamp each received beacon frame at the same symbol boundary within each frame, |  |  |  |  |  | 37 |  |
| the location of which is described by the macTimeStampOffset attribute. The position shall be the same as |  |  |  |  |  | 38 |  |
| that used in the timestamp of the outgoing beacon frame, stored in macBeaconTxTime. The timestamp value |  |  |  |  |  | 39 |  |
| shall be that of the local clock of the device at this position. The timestamp is intended to be a relative time |  |  |  |  |  | 40 |  |
| measurement that may or may not be made absolute, at the discretion of the implementer. |  |  |  |  |  | 41 |  |
|  |  |  |  |  |  | 42 |  |
| If a protected beacon frame is received (i.e., the Security Enabled subfield in the frame control field is set to |  |  |  |  |  | 43 |  |
| one), the device shall attempt to unsecure the beacon frame using the unsecuring process described in 7.2.3. |  |  |  |  |  | 44 |  |
|  |  |  |  |  |  | 45 |  |
| If the status from the unsecuring process is not SUCCESS, the MLME shall issue an MLME-COMM- |  |  |  |  |  | 46 |  |
| STATUS.indication primitive with the status parameter set to the status from the unsecuring process, |  |  |  |  |  | 47 |  |
| indicating the error. |  |  |  |  |  | 48 |  |
|  |  |  |  |  |  | 49 |  |
| The security-related elements of the OWPAN descriptor corresponding to the beacon (see Table 64) shall be |  |  |  |  |  | 50 |  |
| set to the corresponding parameters returned by the unsecuring process. The SecurityFailure element of the |  |  |  |  |  | 51 |  |
| OWPAN descriptor shall be set to SUCCESS if the status from the unsecuring process is SUCCESS and set |  |  |  |  |  | 52 |  |
| to one of the other status codes indicating an error in the security processing otherwise. |  |  |  |  |  | 53 |  |
|  |  |  |  |  |  | 54 |  |
|  | Copyright © 20xx IEEE. All rights reserved. |  |  |  | 63 |  |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1	If a beacon frame is received, the MLME shall discard the beacon frame if the Source Address and the

- Source OWPAN Identifier fields of the MHR of the beacon frame do not match the coordinator source

- address (macCoordShortAddress or macCoordExtendedAddress, depending on the addressing mode) and the

- identifier of the device (macOWPANId).

5

- If a valid beacon frame is received and macAutoRequest is set to FALSE, the MLME shall indicate the

- beacon parameters to the next higher layer by issuing the MLME-BEACON-NOTIFY.indication primitive.

- If a beacon frame is received and macAutoRequest is set to TRUE, the MLME shall first issue the MLME-

- BEACON-NOTIFY.indication primitive if the beacon contains any payload. The MLME shall then compare

- its address with those addresses in the Address List field of the beacon frame. If the Address List field

- contains the 16-bit short or 64-bit extended address of the device and the source OWPAN identifier matches

- macOWPANId, the MLME shall follow the procedure for extracting pending data from the coordinator as

- shown in 5.1.9.3.

14

- If beacon tracking is activated, the MLME shall enable its receiver at a time prior to the next expected

- beacon frame transmission, i.e., just before the known start of the next superframe. If the number of

- consecutive beacons missed by the MLME reaches aMaxLostBeacons, the MLME shall respond with the

- MLME-SYNC-LOSS.indication primitive with a loss reason of BEACON_LOST.

19

20	5.1.7.2 Synchronization without beacons

21

- All devices operating on a nonbeacon-enabled OWPAN (macBeaconOrder = 15) shall be able to poll the

- coordinator for data at the discretion of the next higher layer.

24

- A device is instructed to poll the coordinator when the MLME receives the MLME-POLL.request primitive.

- On receipt of this primitive, the MLME shall follow the procedure for extracting pending data from the

- coordinator as shown in 5.1.9.3.

28

29	5.1.8 Transaction handling

30

- Transactions can be instigated from the devices themselves rather than from the coordinator. In other words,

- either the coordinator needs to indicate in its beacon when messages are pending for devices or the devices

- themselves need to poll the coordinator to determine whether they have any messages pending. Such

- transfers are called indirect transmissions.

35

- The coordinator shall begin handling a transaction on receipt of an indirect transmission request either via

- the MCPS-DATA.request primitive or via a request from the MLME to send a MAC command instigated by

- a primitive from the next higher layer, such as the MLME-ASSOCIATE.response primitive as shown in

- 6.3.1.3. On completion of the transaction, the MAC sublayer shall indicate a status value to the next higher

- layer. If a request primitive instigated the indirect transmission, the corresponding confirm primitive shall be

- used to convey the appropriate status value. Conversely, if a response primitive instigated the indirect

- transmission, the MLME-COMM-STATUS.indication primitive shall be used to convey the appropriate

- status value. The MLME-COMM-STATUS.indication primitive can be related to its corresponding response

- primitive by examining the Destination Address field.

45

- The information contained in the indirect transmission request forms a transaction, and the coordinator shall

- be capable of storing at least one transaction. On receipt of an indirect transmission request, if there is no

- capacity to store another transaction, the MAC sublayer shall indicate to the next higher layer a status of

- TRANSACTION_OVERFLOW in the appropriate corresponding primitive.

50

- If the coordinator is capable of storing more than one transaction, it shall ensure that all the transactions for

- the same device are sent in the order in which they arrived at the MAC sublayer. For each transaction sent, if

- another exists for the same device, the MAC sublayer shall set its frame pending subfield to one, indicating

- the additional pending data.

64	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Each transaction shall persist in the coordinator for at most macTransactionPersistenceTime. If the |  | 1 |  |
| --- | --- | --- | --- |
| transaction is not successfully extracted by the appropriate device within this time, the transaction |  | 2 |  |
| information shall be discarded and the MAC sublayer shall indicate to the next higher layer a status of |  | 3 |  |
| TRANSACTION_EXPIRED in the appropriate corresponding primitive. In order to be successfully |  | 4 |  |
| extracted, an acknowledgment shall be received if one was requested. |  | 5 |  |
| If the transaction was successful, the transaction information shall be discarded, and the MAC sublayer shall |  | 6 |  |
|  |  | 7 |  |
| indicate to the next higher layer a status of SUCCESS in the appropriate corresponding primitive. |  | 8 |  |
| If the coordinator transmits beacons, it shall list the addresses of the devices to which each transaction is |  | 9 |  |
|  |  | 10 |  |
| associated in the Address List field and indicate the number of addresses in the Pending Address |  | 11 |  |
| Specification field of the beacon frame. If the coordinator is able to store more than seven pending |  | 12 |  |
| transactions, it shall indicate them in its beacon on a first-come-first-served basis, ensuring that the beacon |  | 13 |  |
| frame contains at most seven addresses. For transactions requiring a GTS, the coordinator shall not add the |  | 14 |  |
| address of the recipient to its list of pending addresses in the beacon frame. Instead it shall transmit the |  | 15 |  |
| transaction in the GTS allocated for the device as shown in 5.1.10.3. |  | 16 |  |
| On a beacon-enabled OWPAN, if there is a transaction pending for the broadcast address, the frame pending |  | 17 |  |
|  |  | 18 |  |
| subfield of the frame control field in the beacon frame shall be set to one, and the pending message shall be |  | 19 |  |
| transmitted immediately following the beacon using the unslotted random access algorithm. If there is a |  | 20 |  |
| second message pending for the broadcast address, its transmission shall be delayed until the following |  | 21 |  |
| superframe. Only one broadcast message shall be allowed to be sent indirectly per superframe. |  | 22 |  |
| On a beacon-enabled OWPAN, a device that receives a beacon containing its address in the list of pending |  | 23 |  |
|  |  | 24 |  |
| addresses shall attempt to extract the data from the coordinator. On a nonbeacon-enabled OWPAN, a device |  | 25 |  |
| shall attempt to extract the data from the coordinator on receipt of the MLME-POLL.request primitive. The |  | 26 |  |
| procedure for extracting pending data from the coordinator is described in 5.1.9.3. If a device receives a |  | 27 |  |
| beacon with the frame pending subfield set to one, it shall leave its receiver enabled for up to |  | 28 |  |
| macMaxFrameTotalWaitTime optical clocks to receive the broadcast data frame from the coordinator. |  | 29 |  |
| 5.1.9 Transmission, reception, and acknowledgment |  | 30 |  |
|  |  | 31 |  |
| This subclause describes the fundamental procedures for transmission, reception, and acknowledgment. |  | 32 |  |
|  |  | 33 |  |
| 5.1.9.1 Transmission |  | 34 |  |
|  |  | 35 |  |
| Each device shall store its current data-sequence number (DSN) value in the MAC PIB attribute macDSN |  | 36 |  |
|  |  | 37 |  |
| and initialize it to a random value; the algorithm for choosing a random number is out of the scope of this |  | 38 |  |
| standard. Each time a data or a MAC command frame is generated, the MAC sublayer shall copy the value |  | 39 |  |
| of macDSN into the Sequence Number field of the MHR of the outgoing frame and then increment it by one. |  | 40 |  |
| Each device shall generate exactly one DSN regardless of the number of unique devices with which it |  | 41 |  |
| wishes to communicate. The value of macDSN shall be permitted to roll over. |  | 42 |  |
| Each coordinator shall store its current beacon-sequence number (BSN) value in the MAC PIB attribute |  | 43 |  |
|  |  | 44 |  |
| macBSN and initialize it to a random value; the algorithm for choosing a random number is out of the scope |  | 45 |  |
| of this standard. Each time a beacon frame is generated, the MAC sublayer shall copy the value of macBSN |  | 46 |  |
| into the Sequence Number field of the MHR of the outgoing frame and then increment it by one. The value |  | 47 |  |
| of macBSN shall be permitted to roll over. |  | 48 |  |
|  |  | 49 |  |
| It should be noted that both the DSN and BSN are 8-bit values and, therefore, have limited use to the next |  | 50 |  |
| higher layer (e.g., in the case of the DSN, in detecting retransmitted frames). |  | 51 |  |
|  |  | 52 |  |
| The Source Address field, if present, shall contain the address of the device sending the frame. When a |  | 53 |  |
| device has associated and has been allocated a 16-bit short address (i.e., macShortAddress is not equal to |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 65 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 0xfffe or 0xffff), it shall use that address in preference to its 64-bit extended address (i.e., |
| --- | --- |
| 2 | aExtendedAddress) wherever possible. When a device has not yet associated to a OWPAN or |
| 3 | macShortAddress is equal to 0xffff, it shall use its 64-bit extended address in all communications requiring |
| 4 | the Source Address field. If the Source Address field is not present, the originator of the frame shall be |
| 5 | assumed to be the coordinator, and the Destination Address field shall contain the address of the recipient. |
| 6 |  |

- The Destination Address field, if present, shall contain the address of the intended recipient of the frame,

- which may be either a 16-bit short address or a 64-bit extended address. If the Destination Address field is

- not present, the recipient of the frame shall be assumed to be the coordinator, and the Source Address field

- shall contain the address of the originator.

11

- If both destination and source addressing information is present, the MAC sublayer shall compare the

- destination and source OWPAN identifiers. If the OWPAN identifiers are identical the source OWPAN

- identifier shall be omitted from the transmitted frame. If the OWPAN identifiers are different, both the

- Destination OWPAN Identifier and Source OWPAN Identifier fields shall be included in the transmitted

- frame. If only either the destination or the source addressing information is present the OWPAN identifier

- field of the single address shall be included in the transmitted frame.

18

- If the frame is to be transmitted on a beacon-enabled OWPAN, the transmitting device shall attempt to find

- the beacon before transmitting. If the beacon is not being tracked, as shown in 5.1.7.1, and hence the device

21	does not know where the beacon will appear, it shall enable its receiver and search for at most

- [aBaseSuperframeDuration  (2n + 1)] optical clocks, where n is the value of macBeaconOrder, in order to

- find the beacon. If the beacon is not found after this time, the device shall transmit the frame following the

- successful application of the unslotted version of the random access algorithm as shown in 5.1.2.1. Once the

- beacon has been found, either after a search or due to its being tracked, the frame shall be transmitted in the

- appropriate portion of the superframe. Transmissions in the CAP shall follow a successful application of the

- slotted version of the random access algorithm, see 5.1.2.1, and transmissions in a GTS shall not use any

- random access.

29

- If the frame is to be transmitted on a nonbeacon-enabled OWPAN, the frame shall be transmitted following

- the successful application of the unslotted version of the random access algorithm as shown in 5.1.2.1.

32

- For either a beacon-enabled OWPAN or a nonbeacon-enabled OWPAN, if the transmission is direct and

- originates due to a primitive issued by the next higher layer and the access algorithm fails, the next higher

- layer shall be notified. If the transmission is indirect and the access algorithm fails, the frame shall remain in

- the transaction queue until it is requested again and successfully transmitted or until the transaction expires.

| 38 | The device shall process the frame using the outgoing frame security procedure described in 7.2.1. |
| --- | --- |
| 39 |  |

- If the status from the outgoing frame security procedure is not SUCCESS, the MLME shall issue the

- corresponding confirm or MLME-COMM-STATUS.indication primitive with the status parameter set to the

- status from the outgoing frame security procedure, indicating the error.

43

- To transmit the frame, the MAC sublayer shall first enable the transmitter by issuing the PLME-SET-

- TRX-STATE.request primitive with a state of TX_ON to the PHY. On receipt of the PLME-SET-TRX-

- STATE.confirm primitive with a status of either SUCCESS or TX_ON, the constructed frame shall then be

47	transmitted by issuing the PD-DATA.request primitive. Finally, on receipt of the PD-DATA.confirm

- primitive, the MAC sublayer shall disable the transmitter by issuing the PLME-SET-TRX-STATE.request

- primitive with a state of RX_ON or TRX_OFF to the PHY, depending on whether the receiver is to be

- enabled following the transmission. In the case where the Acknowledgment Request subfield of the frame

51	control field is set to one, the MAC sublayer shall enable the receiver immediately following the

- transmission of the frame by issuing the PLME-SET-TRX-STATE.request primitive with a state of RX_ON

- to the PHY.

54

66	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 5.1.9.2 Reception and rejection |  | 1 |  |
| --- | --- | --- | --- |
| Each device may choose whether the MAC sublayer is to enable its receiver during idle periods. During |  | 2 |  |
|  |  | 3 |  |
| these idle periods, the MAC sublayer shall still service transceiver task requests from the next higher layer. |  | 4 |  |
| A transceiver task shall be defined as a transmission request with acknowledgment reception, if required, or |  | 5 |  |
| a reception request. On completion of each transceiver task, the MAC sublayer shall request that the PHY |  | 6 |  |
| enables or disables its receiver, depending on the values of macBeaconOrder and macRxOnWhenIdle. If |  | 7 |  |
| macBeaconOrder is less than 15, the value of macRxOnWhenIdle shall be considered relevant only during |  | 8 |  |
| idle periods of the CAP of the incoming superframe. If macBeaconOrder is equal to 15, the value of |  | 9 |  |
| macRxOnWhenIdle shall be considered relevant at all times. |  | 10 |  |
| A device with its receiver enabled will be able to receive and decode transmissions from all devices |  | 11 |  |
|  |  | 12 |  |
| complying with this standard that are currently operating on the same channel and are in its operating space, |  | 13 |  |
| along with interference from other sources. The MAC sublayer shall, therefore, be able to filter incoming |  | 14 |  |
| frames and present only the frames that are of interest to the upper layers. |  | 15 |  |
| The MAC sublayer shall discard all received frames that do not contain a correct value in their FCS field in |  | 16 |  |
|  |  | 17 |  |
| the MFR (see 5.2.1.9). The FCS field shall be verified on reception by recalculating the purported FCS over |  | 18 |  |
| the MHR and MSDU of the received frame and by subsequently comparing this value with the received FCS |  | 19 |  |
| field. The FCS field of the received frame shall be considered to be correct if these values are the same and |  | 20 |  |
| incorrect otherwise. |  | 21 |  |
| The MAC sublayer shall accept only frames that satisfy all of the following filtering requirements: |  | 22 |  |
|  |  | 23 |  |
| — The Frame Type subfield shall not contain a reserved frame type. |  | 24 |  |
|  |  | 25 |  |
| — If a destination OWPAN identifier is included in the frame, it shall match macOWPANId or shall be |  | 26 |  |
| the broadcast OWPAN identifier (0xffff). |  | 27 |  |
| — If a short destination address is included in the frame, it shall match either macShortAddress or the |  | 28 |  |
|  |  | 29 |  |
| broadcast address (0xffff). Otherwise, if an extended destination address is included in the frame, it |  |  |  |
|  |  | 30 |  |
| shall match aExtendedAddress. |  |  |  |
|  |  | 31 |  |
| — If the frame type indicates that the frame is a beacon frame, the source OWPAN identifier shall |  |  |  |
|  |  | 32 |  |
| match macOWPANId unless macOWPANId is equal to 0xffff, in which case the beacon frame shall |  | 33 |  |
| be accepted regardless of the source OWPAN identifier. |  | 34 |  |
| — If only source addressing fields are included in a data or MAC command frame, the frame shall be |  | 35 |  |
| accepted only if the device is the coordinator and the source OWPAN identifier matches |  | 36 |  |
| macOWPANId. |  | 37 |  |
| If any of the third-level filtering requirements are not satisfied, the MAC sublayer shall discard the incoming |  | 38 |  |
|  |  | 39 |  |
| frame without processing it further. If all of the third-level filtering requirements are satisfied, the frame |  | 40 |  |
| shall be considered valid and processed further. For valid frames that are not broadcast, if the Frame Type |  | 41 |  |
| subfield indicates a data or MAC command frame and the Acknowledgment Request subfield of the frame |  | 42 |  |
| control field is set to one, the MAC sublayer shall send an acknowledgment frame. Prior to the transmission |  | 43 |  |
| of the acknowledgment frame, the sequence number included in the received data or MAC command frame |  | 44 |  |
| shall be copied into the Sequence Number field of the acknowledgment frame. This step will allow the |  | 45 |  |
| transaction originator to know that it has received the appropriate acknowledgment frame. |  | 46 |  |
|  |  | 47 |  |
| If both the destination and source addressing information is included in the frame, the MAC sublayer shall |  | 48 |  |
| assume that the omitted Source OWPAN Identifier field is identical to the Destination OWPAN Identifier |  | 49 |  |
| field. |  | 50 |  |
|  |  | 51 |  |
| The device shall process the frame using the incoming frame security procedure described in 7.2.3. |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 67 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- If the status from the incoming frame security procedure is not SUCCESS, the MLME shall issue the

- corresponding confirm or MLME-COMM-STATUS.indication primitive with the status parameter set to the

| 3 | status from the incoming frame security procedure, indicating the error, and with the security-related |
| --- | --- |
| 4 | parameters set to the corresponding parameters returned by the unsecuring process. |
| 5 |  |

- If the valid frame is a data frame, the MAC sublayer shall pass the frame to the next higher layer. This is

- achieved by issuing the MCPS-DATA.indication primitive containing the frame information. The security-

- related parameters of the MCPS-DATA.indication primitive shall be set to the corresponding parameters

- returned by the unsecuring process.

10

- If the valid frame is a MAC command or beacon frame, it shall be processed by the MAC sublayer

- accordingly, and a corresponding confirm or indication primitive may be sent to the next higher layer. The

| 13 | security-related parameters of the corresponding confirm or indication primitive shall be set to the |  |
| --- | --- | --- |
| 14 | corresponding parameters returned by the unsecuring process. |  |
| 15 | 5.1.9.3 Extracting pending data from a coordinator |  |
| 16 |  |  |
| 17 |  |  |

- A device on a beacon-enabled OWPAN can determine whether any frames are pending for it by examining

- the contents of the received beacon frame, as described in 5.1.7.1. If the address of the device is contained in

- the Address List field of the beacon frame and macAutoRequest is TRUE, the MLME of the device shall

- send a data request command, see 5.4.4, to the coordinator during the CAP with the Acknowledgment

- Request subfield of the frame control field set to one; the only exception to this is if the beacon frame is

- received while performing an active or passive scan as shown in 5.1.5.1. There are two other cases for which

- the MLME shall send a data request command to the coordinator. The first case is when the MLME receives

- the MLME-POLL.request primitive. In the second case, a device may send a data request command

- macResponseWaitTime optical clocks after the acknowledgment to a request command frame, such as during

| 27 | the association procedure. If the data request is intended for the coordinator, the destination address |
| --- | --- |
| 28 | information may be omitted. |
| 29 |  |

- If the data request command originated from an MLME-POLL.request primitive, the MLME shall perform

- the security process on the data request command based on the SecurityLevel, KeyIdMode, KeySource, and

- KeyIndex parameters of the MLME-POLL.request primitive, according to 7.2.1. Otherwise, the MLME

- shall perform the security process on the data request command based on the macAutoRequestSecurityLevel,

- macAutoRequestKeyIdMode, macAutoRequestKeySource, and macAutoRequestKeyIndex PIB attributes,

- according to 7.2.1.

36

- On successfully receiving a data request command, the coordinator shall send an acknowledgment frame,

- thus confirming its receipt. If the coordinator has enough time to determine whether the device has a frame

- pending before sending the acknowledgment frame (see 5.1.9.4.2), it shall set the frame pending subfield of

- the frame control field of the acknowledgment frame accordingly to indicate whether a frame is actually

- pending for the device. If this is not possible, the coordinator shall set the frame pending subfield of the

- acknowledgment frame to one.

43

- On receipt of the acknowledgment frame with the frame pending subfield set to zero, the device shall

- conclude that there are no data pending at the coordinator.

46

- On receipt of the acknowledgment frame with the frame pending subfield set to one, a device shall enable its

- receiver for at most macMaxFrameTotalWaitTime CAP optical clocks in a beacon-enabled OWPAN, or in a

- nonbeacon-enabled OWPAN, to receive the corresponding data frame from the coordinator. If there is an

- actual data frame pending within the coordinator for the requesting device, the coordinator shall send the

- frame to the device using one of the mechanisms described in this subclause. If there is no data frame

52	pending  for  the  requesting  device,  the  coordinator  shall  send  a  data  frame  without  requesting

- acknowledgment to the device containing a zero length payload, indicating that no data are present, using

- one of the mechanisms described in this subclause.

68	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The data frame following the acknowledgment of the data request command shall be transmitted using one of the following mechanisms:

— Without using slotted random access, if the MAC sublayer can commence transmission of the data frame between aTurnaroundTime-RX-TX and (aTurnaroundTime-RX-TX + aUnitBackoffPeriod) optical clocks, on a backoff slot boundary, and there is time remaining in the CAP for the message, appropriate IFS, and acknowledgment as defined in 9.5.1. If a requested acknowledgment frame is not received following this data frame, the process shall begin anew following the receipt of a new data request command.

—  Using slotted random access, otherwise.

If the requesting device does not receive a data frame from the coordinator within macMaxFrameTotalWaitTime CAP optical clocks in a beacon-enabled OWPAN, or in a nonbeacon -enabled OWPAN, or if the requesting device receives a data frame from the coordinator with a zero length payload, it shall conclude that there are no data pending at the coordinator. If the requesting device does receive a data frame from the coordinator, it shall send an acknowledgment frame, if requested, thus confirming receipt.

If the frame pending subfield of the frame control field of the data frame received from the coordinator is set to one, the device still has more data pending with the coordinator. In this case it may extract the data by sending a new data request command to the coordinator.

5.1.9.4 Use of acknowledgments and retransmissions

A data or MAC command frame shall be sent with the Acknowledgment Request subfield of its frame control field set appropriately for the frame. A beacon or acknowledgment frame shall always be sent with the Acknowledgment Request subfield set to zero. Similarly, any frame that is broadcast shall be sent with its Acknowledgment Request subfield set to zero.

5.1.9.4.1 No acknowledgment

A frame transmitted with its Acknowledgment Request subfield set to zero shall not be acknowledged by its intended recipient. The originating device shall assume that the transmission of the frame was successful.

The message sequence chart in Figure 32 shows the scenario for transmitting a single frame of data from an originator to a recipient without requiring an acknowledgment. In this case, the originator transmits the data frame with the Acknowledgment Request (AR) subfield of the frame control field equal to zero.

originator next	originator	recipient	recipient next

higher layer	MAC	MAC	higher layer

MCPS-DATA.request (AR=0)

Data (AR=0)

MCPS-DATA.indication

MCPS-DATA.confirm

Figure 32—Successful data transmission without an acknowledgment

| Copyright © 20xx IEEE. All rights reserved. | 69 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 5.1.9.4.2 Acknowledgment |
| --- | --- |
| 2 |  |

- A frame transmitted with the Acknowledgment Request subfield of its frame control field set to one shall be

- acknowledged by the recipient. If the intended recipient correctly receives the frame, it shall generate and

- send an acknowledgment frame containing the same DSN from the data or MAC command frame that is

- being acknowledged.

7

- The transmission of an acknowledgment frame in a nonbeacon-enabled OWPAN or in the CFP shall

- commence aTurnaroundTime-RX-TXoptical clocks after the last optical clock of the data or MAC command

- frame. The transmission of an acknowledgment frame in the CAP shall commence either aTurnaroundTime-

- RX-TX optical clocks after the reception of the last optical clock of the data or MAC command frame or at a

- backoff slot boundary. In the latter case, the transmission of an acknowledgment frame shall commence

- between aTurnaroundTime-RX-TX and (aTurnaroundTime-RX-TX + aUnitBackoffPeriod) optical clocks

| 14 | after the reception of the last optical clock of the data or MAC command frame. The constants |
| --- | --- |
| 15 | aTurnaroundTime-RX-TX and aTurnaroundTime-TX-RX are defined in Table 143. |
| 16 |  |

- The message sequence chart in Figure 33 shows the scenario for transmitting a single frame of data from an

- originator to a recipient with an acknowledgment. In this case, the originator indicates to the recipient that it

- requires an acknowledgment by transmitting the data frame with the Acknowledgment Request (AR)

- subfield of the frame control field set to one.

21

| 22 | originator next |  |  |  |  | originator |  |  |  |  | recipient |  |  | recipient next |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | higher layer |  |  |  |  | MAC |  |  |  |  | MAC |  |  | higher layer |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 |  |  |  | MCPS-DATA.request (AR=1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |  |  | data (AR=1) |  |  |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |  |  | acknowledgment |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |  |  |  |  |  |  | MCPS-DATA.indication |  |  |  |  |  |
| 31 |  |  |  |  | MCPS-DATA.confirm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 35 |  |  |  | Figure 33—Successful data transmission with an acknowledgment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38 | 5.1.9.4.3 Retransmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- A device that sends a frame with the Acknowledgment Request subfield of its frame control field set to zero

- shall assume that the transmission was successfully received and shall hence not perform the retransmission

- procedure.

44

- A device that sends a data or MAC command frame with its Acknowledgment Request subfield set to one

- shall wait for at most macAckWaitDuration optical clocks for the corresponding acknowledgment frame to

- be received. If an acknowledgment frame is received within macAckWaitDuration optical clocks and

- contains the same DSN as the original transmission, the transmission is considered successful, and no

- further action regarding retransmission shall be taken by the device. If an acknowledgment is not received

- within macAckWaitDuration optical clocks or an acknowledgment is received containing a DSN that was

- not the same as the original transmission, the device shall conclude that the single transmission attempt has

- failed.

53

54

70	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

If a single transmission attempt has failed and the transmission was indirect, the coordinator shall not retransmit the data or MAC command frame. Instead, the frame shall remain in the transaction queue of the coordinator and can only be extracted following the reception of a new data request command. If a new data request command is received, the originating device shall transmit the frame using the same DSN as was used in the original transmission.

If a single transmission attempt has failed and the transmission was direct, the device shall repeat the process of transmitting the data or MAC command frame and waiting for the acknowledgment, up to a maximum of macMaxFrameRetries times. The retransmitted frame shall contain the same DSN as was used in the original transmission. Each retransmission shall only be attempted if it can be completed within the same portion of the superframe, i.e., the CAP or a GTS in which the original transmission was attempted. If this timing is not possible, the retransmission shall be deferred until the same portion in the next superframe. If an acknowledgment is still not received after macMaxFrameRetries retransmissions, the MAC sublayer shall assume the transmission has failed and notify the next higher layer of the failure.

5.1.9.5 Transmission scenarios

Due to the imperfect nature of the wireless medium, a transmitted frame does not always reach its intended destination. Figure 34 illustrates three different data transmission scenarios:

— Successful data transmission. The originator MAC sublayer transmits the data frame to the recipient via the PHY data service. In waiting for an acknowledgment, the originator MAC sublayer starts a timer that will expire after macAckWaitDuration optical clocks. The recipient MAC sublayer receives the data frame, sends an acknowledgment back to the originator, and passes the data frame to the next higher layer. The originator MAC sublayer receives the acknowledgment from the recipient before its timer expires and then disables and resets the timer. The data transfer is now complete, and the originator MAC sublayer issues a success confirmation to the next higher layer.

— Lost data frame. The originator MAC sublayer transmits the data frame to the recipient via the PHY data service. In waiting for an acknowledgment, the originator MAC sublayer starts a timer that will expire after macAckWaitDuration optical clocks. The recipient MAC sublayer does not receive the data frame and so does not respond with an acknowledgment. The timer of the originator MAC sublayer expires before an acknowledgment is received; therefore, the data transfer has failed. If the transmission was direct, the originator retransmits the data, and this entire sequence may be repeated up to a maximum of macMaxFrameRetries times; if a data transfer attempt fails a total of (1 + macMaxFrameRetries) times, the originator MAC sublayer will issue a failure confirmation to the next higher layer. If the transmission was indirect, the data frame will remain in the transaction queue until either another request for the data is received and correctly acknowledged or until macTransactionPersistenceTime is reached. If macTransactionPersistenceTime is reached, the transaction information will be discarded, and the MAC sublayer will issue a failure confirmation to the next higher layer.

— Lost acknowledgment frame. The originator MAC sublayer transmits the data frame to the recipient via the PHY data service. In waiting for an acknowledgment, the originator MAC sublayer starts a timer that will expire after macAckWaitDuration optical clocks. The recipient MAC sublayer receives the data frame, sends an acknowledgment back to the originator, and passes the data frame to the next higher layer. The originator MAC sublayer does not receive the acknowledgment frame, and its timer expires. Therefore, the data transfer has failed. If the transmission was direct, the originator retransmits the data, and this entire sequence may be repeated up to a maximum of macMaxFrameRetries times. If a data transfer attempt fails a total of (1 + macMaxFrameRetries) times, the originator MAC sublayer will issue a failure confirmation to the next higher layer. If the transmission was indirect, the data frame will remain in the transaction queue either until another request for the data is received and correctly acknowledged or until macTransactionPersistenceTime is reached. If macTransactionPersistenceTime is reached, the transaction information will be discarded, and the MAC sublayer will issue a failure confirmation to the next higher layer.

| Copyright © 20xx IEEE. All rights reserved. | 71 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 34—Transmission scenarios, using direct transmission, for frame reliability

5.1.10 GTS allocation and management

A GTS allows a device to operate on the channel within a portion of the superframe that is dedicated (on the OWPAN) exclusively to that device. A GTS shall be allocated only by the coordinator, and it shall be used only for communications between the coordinator and a device associated with the OWPAN through the coordinator. A single GTS may extend over one or more superframe slots. The coordinator may allocate a number of GTSs at the same time, provided there is sufficient capacity in the superframe.

A GTS shall be allocated before use, with the coordinator deciding whether to allocate a GTS based on the requirements of the GTS request and the current available capacity in the superframe. GTSs shall be allocated on a first-come-first- served basis, and all GTSs shall be placed contiguously at the end of the superframe and after the CAP. Each GTS shall be deallocated when the GTS is no longer required, and a GTS can be deallocated at any time at the discretion of the coordinator or by the device that originally requested the GTS. A device that has been allocated a GTS may also operate in the CAP.

A data frame transmitted in an allocated GTS shall use only short addressing.

The management of GTSs shall be undertaken by the coordinator only. To facilitate GTS management, the coordinator shall be able to store all the information necessary to manage seven GTSs. For each GTS, the coordinator shall be able to store its starting slot, length, direction, and associated device address.

The GTS direction, which is relative to the data flow from the device that owns the GTS, is specified as either transmit or receive. The device address and direction shall, therefore, uniquely identify each GTS. Each device may request one transmit GTS and/or one receive GTS. For each allocated GTS, the device shall be able to store its starting slot, length, and direction. If a device has been allocated a receive GTS, it shall enable its receiver for the entirety of the GTS. In the same way, the coordinator shall enable its receiver

72	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| for the entirety of the GTS if a device has been allocated a transmit GTS. If a data frame is received during a |  |  | 1 |  |
| --- | --- | --- | --- | --- |
| receive GTS and an acknowledgment is requested, the device shall transmit the acknowledgment frame as |  |  | 2 |  |
| usual. Similarly, a device shall be able to receive an acknowledgment frame during a transmit GTS. |  |  | 3 |  |
| A device shall attempt to allocate and use a GTS only if it is currently tracking the beacons. The MLME is |  |  | 4 |  |
|  |  |  | 5 |  |
| instructed to track beacons by issuing the MLME-SYNC.request primitive with the TrackBeacon parameter |  |  | 6 |  |
| set to TRUE. If a device loses synchronization with the coordinator, all its GTS allocations shall be lost. |  |  | 7 |  |
| The use of GTSs is optional. |  |  | 8 |  |
|  |  |  | 9 |  |
| 5.1.10.1 CAP maintenance |  |  | 10 |  |
|  |  |  | 11 |  |
| The coordinator shall preserve the minimum CAP length of aMinCAPLength and take preventative action if |  |  | 12 |  |
|  |  |  | 13 |  |
| the minimum CAP is not satisfied. However, an exception shall be allowed for the accommodation of the |  |  | 14 |  |
| temporary increase in the beacon frame length needed to perform GTS maintenance. If preventative action |  |  | 15 |  |
| becomes necessary, the action chosen is left up to the implementation, but may include one or more of the |  |  | 16 |  |
| following: |  |  | 17 |  |
| — Limiting the number of pending addresses included in the beacon. |  |  | 18 |  |
|  |  |  | 19 |  |
| — | Not including a payload field in the beacon frame. |  | 20 |  |
| — | Deallocating one or more of the GTSs. |  | 21 |  |
|  |  |  | 22 |  |
| 5.1.10.2 GTS allocation |  |  | 23 |  |
|  |  |  | 24 |  |
| A device is instructed to request the allocation of a new GTS through the MLME-GTS.request primitive, |  |  | 25 |  |
|  |  |  | 26 |  |
| with GTS characteristics set according to the requirements of the intended application. |  |  | 27 |  |
| To request the allocation of a new GTS, the MLME shall send the GTS request command, see 5.4.18, to the |  |  | 28 |  |
|  |  |  | 29 |  |
| coordinator. The Characteristics Type subfield of the GTS Characteristics field of the request shall be set to |  |  | 30 |  |
| one (GTS allocation), and the length and direction subfields shall be set according to the desired |  |  | 31 |  |
| characteristics of the required GTS. Because the GTS request command contains an acknowledgment |  |  | 32 |  |
| request (see 5.4.3.1), the coordinator shall confirm its receipt by sending an acknowledgment frame. |  |  | 33 |  |
| On receipt of a GTS request command indicating a GTS allocation request, the coordinator shall first check |  |  | 34 |  |
|  |  |  | 35 |  |
| if there is available capacity in the current superframe, based on the remaining length of the CAP and the |  |  | 36 |  |
| desired length of the requested GTS. The superframe shall have available capacity if the maximum number |  |  | 37 |  |
| of GTSs has not been reached and allocating a GTS of the desired length would not reduce the length of the |  |  | 38 |  |
| CAP to less than aMinCAPLength. GTSs shall be allocated on a first-come-first-served basis by the |  |  | 39 |  |
| coordinator provided there is sufficient bandwidth available. The coordinator shall make this decision within |  |  | 40 |  |
| aGTSDescPersistenceTime superframes. |  |  | 41 |  |
| On receipt of the acknowledgment to the GTS request command, the device shall continue to track beacons |  |  | 42 |  |
|  |  |  | 43 |  |
| and wait for at most aGTSDescPersistenceTime superframes. If no GTS descriptor for the device appears in |  |  | 44 |  |
| the beacon within this time, the MLME of the device shall notify the next higher layer of the failure. This |  |  | 45 |  |
| notification is achieved when the MLME issues the MLME-GTS.confirm primitive, see 6.3.6.3, with a |  |  | 46 |  |
| status of NO_DATA. |  |  | 47 |  |
| When the coordinator determines whether capacity is available for the requested GTS, it shall generate a |  |  | 48 |  |
|  |  |  | 49 |  |
| GTS descriptor with the requested specifications and the 16-bit short address of the requesting device. If the |  |  | 50 |  |
| GTS was allocated successfully, the coordinator shall set the start slot in the GTS descriptor to the |  |  | 51 |  |
| superframe slot at which the GTS begins and the length in the GTS descriptor to the length of the GTS. In |  |  | 52 |  |
| addition, the coordinator shall notify the next higher layer of the new GTS. This notification is achieved |  |  | 53 |  |
| when the MLME of the coordinator issues the MLME-GTS.indication primitive, see 6.3.6.2, with the |  |  | 54 |  |
|  | Copyright © 20xx IEEE. All rights reserved. | 73 |  |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- characteristics of the allocated GTS. If there was not sufficient capacity to allocate the requested GTS, the

- start slot shall be set to zero and the length to the largest GTS length that can currently be supported. The

- coordinator shall then include this GTS descriptor in its beacon and update the GTS Specification field of

4	the beacon frame accordingly. The coordinator shall also update the Final CAP Slot subfield of the

- Superframe Specification field of the beacon frame, indicating the final superframe slot utilized by the

- decreased CAP. The GTS descriptor shall remain in the beacon frame for aGTSDescPersistenceTime

- superframes, after which it shall be removed automatically. The coordinator shall be allowed to reduce its

- CAP below aMinCAPLength to accommodate the temporary increase in the beacon frame length due to the

- inclusion of the GTS descriptor.

10

- On receipt of a beacon frame containing a GTS descriptor corresponding to macShortAddress, the device

- shall process the descriptor. The MLME of the device shall then notify the next higher layer of whether the

- GTS allocation request was successful. This notification is achieved when the MLME issues the MLME-

- GTS.confirm primitive with a status of SUCCESS (if the start slot in the GTS descriptor was greater than

- zero) or DENIED (if the start slot was equal to zero or if the length did not match the requested length).

16

17	5.1.10.3 GTS usage

18

- When the MAC sublayer of a device that is not the coordinator receives an MCPS-DATA.request primitive,

- see 6.2.1, with the TxOptions parameter indicating a GTS transmission, it shall determine whether it has a

- valid transmit GTS. If a valid GTS is found, the MAC sublayer shall transmit the data during the GTS, i.e.,

- between its starting slot and its starting slot plus its length. At this time, the MAC sublayer shall transmit the

- MPDU immediately without using any random access, provided the requested transaction can be completed

- before the end of the GTS. If the requested transaction cannot be completed before the end of the current

- GTS, the MAC sublayer shall defer the transmission until the specified GTS in the next superframe. Note

- that the MAC shall allow for the PHY overhead in making this determination.

27

- If the device has any receive GTSs, the MAC sublayer of the device shall ensure that the receiver is enabled

- at a time prior to the start of the GTS and for the duration of the GTS, as indicated by its starting slot and its

- length.

31

32 When the MAC sublayer of the coordinator receives an MCPS-DATA.request primitive with the TxOptions 33 parameter indicating a GTS transmission, it shall determine whether it has a valid receive GTS

- corresponding to the device with the requested destination address. If a valid GTS is found, the coordinator

- shall defer the transmission until the start of the receive GTS. In this case, the address of the device with the

- message requiring a GTS transmission shall not be added to the list of pending addresses in the beacon

- frame as shown in 5.1.8. At the start of the receive GTS, the MAC sublayer shall transmit the data without

- using any random access, provided the requested transaction can be completed before the end of the GTS. If

- the requested transaction cannot be completed before the end of the current GTS, the MAC sublayer shall

- defer the transmission until the specified GTS in the next superframe.

41

- For all allocated transmit GTSs (relative to the device), the MAC sublayer of the coordinator shall ensure

- that its receiver is enabled at a time prior to the start and for the duration of each GTS.

44

- Before commencing transmission in a GTS, each device shall ensure that the data transmission, the

- acknowledgment, if requested, and the IFS, suitable to the size of the data frame, can be completed before

- the end of the GTS.

48

- If a device misses the beacon at the beginning of a superframe, it shall not use its GTSs until it receives a

- subsequent beacon correctly. If a loss of synchronization occurs due to the loss of the beacon, the device

- shall consider all of its GTSs deallocated.

52

53

54

74	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 5.1.10.4 GTS deallocation |  | 1 |  |
| --- | --- | --- | --- |
| A device is instructed to request the deallocation of an existing GTS through the MLME-GTS.request |  | 2 |  |
|  |  | 3 |  |
| primitive specified in 6.3.6.1, using the characteristics of the GTS it wishes to deallocate. From this point |  | 4 |  |
| onward, the GTS to be deallocated shall not be used by the device, and its stored characteristics shall be |  | 5 |  |
| reset. |  | 6 |  |
| To request the deallocation of an existing GTS, the MLME shall send the GTS request command, specified |  | 7 |  |
|  |  | 8 |  |
| in 5.4.18, to the coordinator. The Characteristics Type subfield of the GTS Characteristics field of the |  | 9 |  |
| request shall be set to zero (i.e., GTS deallocation), and the length and direction subfields shall be set |  | 10 |  |
| according to the characteristics of the GTS to deallocate. Because the GTS request command contains an |  | 11 |  |
| acknowledgment request, specified in 5.4.3.1, the coordinator shall confirm its receipt by sending an |  | 12 |  |
| acknowledgment frame. On receipt of the acknowledgment to the GTS request command, the MLME shall |  | 13 |  |
| notify the next higher layer of the deallocation. This notification is achieved when the MLME issues the |  | 14 |  |
| MLME-GTS.confirm primitive, see 6.3.6.3, with a status of SUCCESS and a GTSCharacteristics parameter |  | 15 |  |
| with its Characteristics Type subfield set to zero. If the GTS request command is not received correctly by |  | 16 |  |
| the coordinator, it shall determine that the device has stopped using its GTS by the procedure described in |  | 17 |  |
| 5.1.10.6. |  | 18 |  |
| On receipt of a GTS request command with the Characteristics Type subfield of the GTS Characteristics |  | 19 |  |
|  |  | 20 |  |
| field set to zero (GTS deallocation), the coordinator shall attempt to deallocate the GTS. If the GTS |  | 21 |  |
| characteristics contained in the GTS request command do not match the characteristics of a known GTS, the |  | 22 |  |
| coordinator shall ignore the request. If the GTS characteristics contained in the GTS request command |  | 23 |  |
| match the characteristics of a known GTS, the MLME of the coordinator shall deallocate the specified GTS |  | 24 |  |
| and notify the next higher layer of the change. This notification is achieved when the MLME issues the |  | 25 |  |
| MLME-GTS.indication primitive, see 6.3.6.2, with a GTSCharacteristics parameter containing the |  | 26 |  |
| characteristics of the deallocated GTS and a Characteristics Type subfield set to zero. The coordinator shall |  | 27 |  |
| also update the Final CAP Slot subfield of the Superframe Specification field of the beacon frame, |  | 28 |  |
| indicating the final superframe slot utilized by the increased CAP. It shall not add a descriptor to the beacon |  | 29 |  |
| frame to describe the deallocation. |  | 30 |  |
| GTS deallocation may be initiated by the coordinator due to a deallocation request from the next higher |  | 31 |  |
|  |  | 32 |  |
| layer, the expiration of the GTS (see 5.1.10.6), or maintenance required to maintain the minimum CAP |  | 33 |  |
| length, aMinCAPLength (see 5.1.10.1). |  | 34 |  |
| When a GTS deallocation is initiated by the next higher layer of the coordinator, the MLME shall receive the |  | 35 |  |
|  |  | 36 |  |
| MLME-GTS.request primitive with the GTS Characteristics field of the request set to zero (i.e. GTS |  | 37 |  |
| deallocation) and the length and direction subfields set according to the characteristics of the GTS to |  | 38 |  |
| deallocate. |  | 39 |  |
| When a GTS deallocation is initiated by the coordinator either due to the GTS expiring or due to CAP |  | 40 |  |
|  |  | 41 |  |
| maintenance, the MLME shall notify the next higher layer of the change. This notification is achieved when |  | 42 |  |
| the MLME issues the MLME-GTS.indication primitive with a GTSCharacteristics parameter containing the |  | 43 |  |
| characteristics of the deallocated GTS and a Characteristics Type subfield set to zero. |  | 44 |  |
| In the case of any deallocation initiated by coordinator, the coordinator shall deallocate the GTS and add a |  | 45 |  |
|  |  | 46 |  |
| GTS descriptor into its beacon frame corresponding to the deallocated GTS, but with its starting slot set to |  | 47 |  |
| zero. The descriptor shall remain in the beacon frame for aGTSDescPersistenceTime superframes. The |  | 48 |  |
| coordinator shall be allowed to reduce its CAP below aMinCAPLength to accommodate the temporary |  | 49 |  |
| increase in the beacon frame length due to the inclusion of the GTS descriptor. |  | 50 |  |
| On receipt of a beacon frame containing a GTS descriptor corresponding to macShortAddress and a start slot |  | 51 |  |
|  |  | 52 |  |
| equal to zero, the device shall immediately stop using the GTS. The MLME of the device shall then notify |  | 53 |  |
| the next higher layer of the deallocation. This notification is achieved when the MLME issues the MLME- |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 75 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | GTS.indication primitive with a GTSCharacteristics parameter containing the characteristics of the |  |
| --- | --- | --- |
| 2 | deallocated GTS and a Characteristics Type subfield set to zero. |  |
| 3 | 5.1.10.5 GTS reallocation |  |
| 4 |  |  |
| 5 | The deallocation of a GTS may result in the superframe becoming fragmented. For example, Figure 35 |  |
| 6 |  |  |

- shows three stages of a superframe with allocated GTSs. In stage 1, three GTSs are allocated starting at slots

- 14, 10, and 8, respectively. If GTS 2 is now deallocated (stage 2), there will be a gap in the superframe

- during which nothing can happen. To solve this, GTS 3 will have to be shifted to fill the gap, thus increasing

- the size of the CAP (stage 3).

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Figure 35—CFP defragmentation on GTS deallocations

The coordinator shall ensure that any gaps occurring in the CFP, appearing due to the deallocation of a GTS, are removed to maximize the length of the CAP.

When a GTS is deallocated by the coordinator, it shall add a GTS descriptor into its beacon frame indicating that the GTS has been deallocated. If the deallocation is initiated by a device, the coordinator shall not add a GTS descriptor into its beacon frame to indicate the deallocation. For each device with an allocated GTS having a starting slot lower than the GTS being deallocated, the coordinator shall update the GTS with the new starting slot and add a GTS descriptor to its beacon corresponding to this adjusted GTS. The new starting slot is computed so that no space is left between this GTS and either the end of the CFP, if the GTS appears at the end of the CFP, or the start of the next GTS in the CFP.

76	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

In situations where multiple reallocations occur at the same time, the coordinator may choose to perform the reallocation in stages. The coordinator shall keep each GTS descriptor in its beacon for aGTSDescPersistenceTime superframes.

On receipt of a beacon frame containing a GTS descriptor corresponding to macShortAddress and a direction and length corresponding to one of its GTSs, the device shall adjust the starting slot of the GTS corresponding to the GTS descriptor and start using it immediately.

In cases where it is necessary for the coordinator to include a GTS descriptor in its beacon, it shall be allowed to reduce its CAP below aMinCAPLength to accommodate the temporary increase in the beacon frame length. After aGTSDescPersistenceTime superframes, the coordinator shall remove the GTS descriptor from the beacon.

5.1.10.6 GTS expiration

The MLME of the coordinator shall attempt to detect when a device has stopped using a GTS using the following rules:

— For a transmit GTS, the MLME of the coordinator shall assume that a device is no longer using its GTS if a data frame is not received from the device in the GTS at least every 2n superframes, where n is defined below.

— For receive GTSs, the MLME of the coordinator shall assume that a device is no longer using its GTS if an acknowledgment frame is not received from the device at least every 2n superframes, where n is defined below. If the data frames sent in the GTS do not require acknowledgment frames, the MLME of the coordinator will not be able to detect whether a device is using its receive GTS. However, the coordinator is capable of deallocating the GTS at any time.

| The value of n is defined as follows: |  |  |
| --- | --- | --- |
| n = 2(8-macBeaconOrder) | 0 | macBeaconOrder 8 |
| n = 1 | 9 |  macBeaconOrder 14 |

5.1.11 Fast link recovery

In the star topology, a fast link recovery process may be triggered at the device end during communication. The trigger may be initiated when the device does not receive ACKs for a number of times given by the MAC PIB attribute macNumAcks, as defined in Table 91. In the fast link recovery process, the device may decide on its own to stop sending data. The device may also send the fast link recovery (FLR) signal repeatedly (within the allocated resource) to the coordinator if the device is connected to mains power. Upon receiving the FLR signal, the coordinator shall send a FLR response to the device. The communication resumes after the device receives the response. If there is bi-directional data transfer during communication, the device may wait after stopping sending data. If the device does not receive any FLR response signal within a timer given by the MAC PIB attribute macLinkTimeOut, the device may assume the link is broken and may disassociate.

The FLR signal and response are defined in 5.4.16. The FLR signal and response shall be sent at the lowest data rate corresponding to the currently negotiated optical rate.

Figure 36 shows an example of the process of device stopping sending data based on the retransmission count.

| Copyright © 20xx IEEE. All rights reserved. | 77 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 36—An example of the process of device stopping data transmission based on the retransmission count, and triggering FLR

In the star topology, a fast link recovery may also be triggered by the coordinator. The trigger may be initiated when the coordinator does not receive contiguous ACKs for a number of times given by the MAC PIB attribute macNumAcks. In the fast link recovery process, the coordinator may stop sending data to the device. The coordinator then sends fast link recovery (FLR) signal repeatedly to the device. The coordinator may hold the uplink grant allocated to the device. Upon receiving FLR signal, the device shall send a FLR response to the coordinator. The communication resumes after the device receives the response.

Figure 37 shows an example of the process of the coordinator stopping sending data based on the retransmission count.

78	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 37—An example of the process of the coordinator stopping sending data based on the retransmission count, and triggering FLR

In peer-to-peer OWC, the devices may let each other know their battery life. If the conditions to trigger the fast link recovery process are satisfied, the device may further compare its own battery life with the battery life of its peer (the one it is communicating). If the battery life of the device is shorter than its peer’s, then the device stops sending data, and waits. If the battery life of the device is longer than its peer’s, then the device stops sending data and initiates the fast link recovery process.

When the fast link recovery is triggered, and if the device has spare wavelength bands, some or all of the spare bands may also start sending fast link recovery signals to recover the link. The device then shall choose a band that gets the fast link recovery response to continue the communication.

The address field of MHR in FLR signal and response may include the address or the identifier of the color bands.

Figure 38 shows a flowchart of the process for color band assisted fast link recovery.

| Copyright © 20xx IEEE. All rights reserved. | 79 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22	Figure 38—Flowchart of process for color band assisted fast link recovery

23

24

25

- When the fast link recovery is triggered, if the device has other communication directions/angles (e.g., a

- light with multiple LEDs with different angles) some or all of the other angles may also start sending fast

- link recovery signaling to recover the link. The device then shall choose an angle that gets the fast link

- recovery response to continue the communication. The process of fast link recovery on other directions/

- angles is done successively (i.e., one direction after another). The direction is indicated in the link recovery

- mechanism provided by the command frame structure.

32

- The address field of MHR in FLR signal and response may include the address or the identifier of the angles

- or directions.

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Figure 39 shows a flowchart of the process for multiple angle assisted fast link recovery.

80	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 39—Flowchart showing process of multiple angles assisted fast link recovery

5.1.12 Multiple channel resource assignment

5.1.12.1 Multiple channel information

.

| Copyright © 20xx IEEE. All rights reserved. | 81 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

82	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Copyright © 20xx IEEE. All rights reserved. | 83 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

|  | IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC | IEEE Draft P802.15.13/D0 |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

6

10

- .

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

5.1.13 OWC cell design and mobility support

There may be a need to support link switching due to physical movement or interference. Mobility can be of two types: physical and logical. Physical mobility occurs when the OWC device M1 changes its position due to the movement within the coverage area of infrastructure I1 while logical mobility occurs when the device M1 changes its communication link from a link with infrastructure I2 to one with infrastructure I3 due to interference or deliberate channel switching, as shown in Figure 42.

Figure 42—Physical and logical mobility

A coordinator DME can separate the optical media into multiple cells for supporting applications such as location-based services.

84	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.1.13.1 Mobility using boundary information

A single coordinator can support mobility of the device through multiple cells using the PHY switch, controlled by the DME, as shown in Figure 43. Each optical element in a cell is denoted by cell_ID(i,j), where j is the index of the element in the ith cell. The size and the position of the cell in the optical media shown in Figure 5 can be variable and can be programmed by the DME. The actual size and position determination for the cell by the coordinator DME is not defined in the standard. If device 1 moves to the next cell, for example, from cell_ID(i,j) to cell_ID(i+1,j), the coordinator can detect the mobility of the device using the uplink signal (i.e., acknowledgment frame).

1

2

3

4

5

6

7

8

9

10

11

12

time slot n for device 1

PHY switch

13

14

15

16

| Cell_ID(i,j) | Cell_ID(i+1,j) | Cell_ID(i+2,j) |  |
| --- | --- | --- | --- |

| D/L | U/L | D/L | U/L |
| --- | --- | --- | --- |

device 1

Figure 43—Cell configuration for OWC mobility

Figure 44 shows the mobility support for a device through multiple cells. When device 1 moves out from Cell_ID(i,j) to Cell_ID(i+1,j), the coordinator may not receive the uplink transmission (for example, acknowledgment frame or CVD frame) from Cell_ID(i,j). The coordinator may then search for the device through the adjacent cells such as Cell_ID(i+1,j) and Cell_ID(i-1,j) during the same time slots assigned to device 1 in the superframe. The other devices in cell_ID(i,j) will continue communication in the same cell. The coordinator may also expand the cell size in order to provide coverage for mobility of the device. The coordinator can decide on the new cell selection for the device on receiving the uplink transmission from device 1. Thus, if the coordinator can resume communication with the device in cell_ID(i+1,j), the

| Copyright © 20xx IEEE. All rights reserved. | 85 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

coordinator DME may set the PHY switch to use cell_ ID(i+1,j) for device 1 during the time slots allocated for device 1 and then switch back to cell_ ID(i,j) to service any existing devices in cell_ID(i,j) in the remaining time slots. The searching process can be terminated if the device is not found within the link timeout period, defined in MAC PIB attribute macLinkTimeOut in Table 91, and the device can then be considered to be disassociated from the coordinator.

|  |  |  |  |  |  |  | coordinator |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cell_ID (i,j) |  | Cell_ID (i+1,j) |  |  | Cell_ID (i+2,j) |  |  |
| Cell_ID (i,1) | Cell_ID (i,2) | Cell_ID (i,3) | Cell_ID (i+1,1) | Cell_ID (i+1,2) | Cell_ID (i+2,1) | Cell_ID (i+2,2) | Cell_ID (i+2,3) | Cell_ID (i+2,4) |
| Cell_ID (i,4) | Cell_ID (i,5) | Cell_ID (i,6) | Cell_ID (i+1,3) | Cell_ID (i+1,4) | Cell_ID (i+2,5) | Cell_ID (i+2,6) | Cell_ID (i+2,7) | Cell_ID (i+2,8) |
|  |  |  |  |  |  |  | Device 2 |  |
|  | Device 1 |  | Device 1 |  |  |  |  |  |
|  | Time slot n |  | Time slot n+2 |  |  |  | Time slot n+1 |  |
| Cell_ID (i,7) | Cell_ID (i,8) | Cell_ID (i,9) | Cell_ID (i+1,5) | Cell_ID (i+1,6) | Cell_ID (i+2,9) | Cell_ID (i+2,10) | Cell_ID (i+2,11) | Cell_ID (i+2,12) |

Figure 44—Mobility support for a device through multiple cells

5.1.13.2 Cell configuration during superframe

In order to support access for new devices through the entire superframe, the entire optical media shall be configured to a single macro cell during the beacon and CAP periods. Once devices are discovered and associated, the cell sizes and positions can be determined and the cell structure can be applied to the individual device(s) for communication, as shown in Figure 45.

86	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  |  |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 2 |  |
|  |  | CFP |  |  | 3 |  |
| beacon | CAP |  |  |  | 4 |  |
|  |  |  |  |  | 5 |  |
|  |  |  |  |  | 6 |  |
|  |  |  |  |  | 7 |  |
|  |  |  |  |  | 8 |  |
| macro cell |  |  |  |  | 9 |  |
|  |  |  |  |  | 10 |  |
|  |  |  |  |  | 11 |  |
|  | device 2 |  |  | device 2 | 12 |  |
|  | Cell_ID (1,j) | Cell_ID (2,j) | Cell_ID (1,j) |  | 13 |  |
|  |  |  |  | Cell_ID (2,j) | 14 |  |
|  |  |  |  |  | 15 |  |
| device 1 | device 1 |  |  |  | 16 |  |
|  |  |  |  |  | 17 |  |
|  |  | Cell_ID (4,j) | Cell_ID (3,j) | Cell_ID (4,j) |  |  |
|  |  |  |  |  | 18 |  |
|  | Cell_ID (3,j) |  |  |  | 19 |  |
|  | Cell_ID (i,j) |  |  |  | 20 |  |
|  |  |  |  |  | 21 |  |
|  | Figure 45—Superframe configuration for mobility support |  |  |  | 22 |  |
|  |  |  |  |  | 23 |  |
|  |  |  |  |  | 24 |  |
| 5.1.13.3 Cell size and location search procedure |  |  |  |  | 25 |  |
|  |  |  |  |  | 26 |  |
| Once a device is associated with a coordinator using the beacon and CAP, the coordinator may establish the |  |  |  |  | 27 |  |
|  |  |  |  |  | 28 |  |
| size and location of the cell in order to service the new device in the CFP with a smaller cell size. In order to |  |  |  |  |  |  |
|  |  |  |  |  | 29 |  |
| determine the size and location of the cell, the coordinator first sets the cellSearchEn bit in the superFrame |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |
| specification field of the beacon frame as defined in Figure 66. If the cellSearchEn bit is set, the |  |  |  |  |  |  |
|  |  |  |  |  | 31 |  |
| cellSearchLength is transmitted as an additional field in the beacon frame, as shown in Figure 63. If the |  |  |  |  |  |  |
|  |  |  |  |  | 32 |  |
| cellSearchEn bit is set, the coordinator readjusts its superframe GTS allocation to ensure the first |  |  |  |  |  |  |
|  |  |  |  |  | 33 |  |
| cellSearchLength slots of the CFP are allocated for cell size and location search. |  |  |  |  |  |  |
|  |  |  |  |  | 34 |  |
| The first cellSearchLength slots are used as visibility slots by the coordinator and the devices. During the |  |  |  |  | 35 |  |
|  |  |  |  |  | 36 |  |
| first cellSearchLength slots, the coordinator sequentially cycles through the cellSearchLength cells and |  |  |  |  |  |  |
|  |  |  |  |  | 37 |  |
| transmits CVD frames in all the cells. Figure 46 shows an example of the sequential search for 4 cells. CS1 |  |  |  |  |  |  |
|  |  |  |  |  | 38 |  |
| to CS4 are the 4 cell search slots that are made available for searching via setting the cellSearchLength to 4 |  |  |  |  |  |  |
|  |  |  |  |  | 39 |  |
| and setting the cellSearchEn bit in the beacon frame. |  |  |  |  |  |  |
|  |  |  |  |  | 40 |  |
|  |  |  |  |  | 41 |  |
|  |  |  |  |  | 42 |  |
|  |  |  |  |  | 43 |  |
|  |  |  |  |  | 44 |  |
|  |  |  |  |  | 45 |  |
|  |  |  |  |  | 46 |  |
|  |  |  |  |  | 47 |  |
|  |  |  |  |  | 48 |  |
|  |  |  |  |  | 49 |  |
|  |  |  |  |  | 50 |  |
|  |  |  |  |  | 51 |  |
|  |  |  |  |  | 52 |  |
|  |  |  |  |  | 53 |  |
|  |  |  |  |  | 54 |  |

| Copyright © 20xx IEEE. All rights reserved. | 87 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

|  | IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC |  |  |  |  |  |  |  | IEEE Draft P802.15.13/D0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | CFP |  |  |  |  |
| 4 |  |  |  | cell search |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | beacon | CAP | CS1 | CS2 | CS3 | CS4 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | macro cell |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | device 2 | Cell_ID (1,j) | Cell_ID (2,j) |  |  | Cell_ID (1,j) | Cell_ID (2,j) | Cell_ID (1,j) | Cell_ID (2,j) | Cell_ID (1,j) | Cell_ID (2,j) |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  | device 1 |  |  |  |  |  |  |
| 16 |  | device 1 | Cell_ID (4,j) |  |  |  | Cell_ID (4,j) | device 1 | Cell_ID (4,j) | device 1 | Cell_ID (4,j) |  |
| 17 | device 1 | Cell_ID (3,j) |  |  |  | Cell_ID (3,j) |  | Cell_ID (3,j) |  | Cell_ID (3,j) |  |  |
| 18 |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | Cell_ID (i,j) |  |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 | Figure 46—Cell size and location search procedure |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  |  |  |  |  |

- If a device receives a beacon with the cellSearchEn bit set to 1, the device shall also continuously transmit

- CVD frames during the cellSearchEn slots while also monitoring the CVD frame reception from the

- coordinator. The device shall note the WQI during each of the cellSearchLength slots and shall report this

- information back to the coordinator using the mobility notification command frame, as described in 5.4.17.

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

The coordinator makes the determination of the cell sizes and location based on the information from the mobility notification command and its own reception of the CVD frames from the device during the cell search slots.

5.1.14 CSI feedback and link adaptation

5.1.14.1 CSI feedback for MCS selection

In MAC header, a link adaptation control field is defined.

Table 8—Link adaptation control field

| MCS request | MCS sequence | Feedback sequence | MCS feedback |  |
| --- | --- | --- | --- | --- |
|  |  | index |  |  |
| 1 bit | 3 bits | 3 bits | 3 bits |  |
| 0 no MCS request |  | To which MCS request |  |  |
|  | The index of the MCS request | the MCS feedback | Suggest MCS level |  |
| 1 MCS request |  |  |  |  |
|  |  | applies |  |  |

Two types of CSI feedbacks are supported.

88	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1) Solicited feedback: a CSI requester request a CSI responder to feedback CSI. |  | 1 |  |
| --- | --- | --- | --- |
| 2) Unsolicited feedback: a coordinator/device report CSI to another device/coordinator without a |  | 2 |  |
| request. |  | 3 |  |
| For solicited feedback, the CSI requester may set the MCS request to 1 to request a responder to provide |  | 4 |  |
|  |  | 5 |  |
| MCS recommendation. In each MCS request, the requester shall set the MSI (MCS sequence index) subfield |  | 6 |  |
| in the link adaptation control field to a value in the range 0 to 6. How the requester chooses the MSI value is |  | 7 |  |
| implementation dependent. |  | 8 |  |
| On receipt of a frame with the MCS request subfield equal to 1, an CSI responder initiates computation of |  | 9 |  |
|  |  | 10 |  |
| the MCS estimate and labels the result of this computation with the MSI value. The MFB responder includes |  | 11 |  |
| the received MSI value in the feedback sequence index field of the corresponding response frame. The sug- |  | 12 |  |
| gested MCS level is transmitted in MCS feedback field. |  | 13 |  |
| For unsolicited feedback, a device/coordinator feedbacks a suggest MCS level without being requested. In |  | 14 |  |
|  |  | 15 |  |
| this case, feedback sequence index is set to 7, and suggested MCS level is transmitted in MCS feedback |  | 16 |  |
| field. |  | 17 |  |
| After CSI feedback is obtained by the transmitter, it selects a MCS level for future transmissions. The |  | 18 |  |
|  |  | 19 |  |
| selected MCS level may or may not be the one suggested by the receiver. |  | 20 |  |
| The selected MCS level for data transmission is indicated in PHY header. |  | 21 |  |
|  |  | 22 |  |
| 5.1.14.2 CSI feedback for bit-loading |  | 23 |  |
|  |  | 24 |  |
| Editor note: Volker will provide text in the next round of comment resolution, see comment 80 of https://men- |  | 25 |  |
|  |  | 26 |  |
| tor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx |  | 27 |  |
| 5.1.14.3 CSI feedback for MIMO operation |  | 28 |  |
|  |  | 29 |  |
| Editor note: Volker will provide text in the next round of comment resolution, see comment 83 of https://men- |  | 30 |  |
|  |  | 31 |  |
| tor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx |  | 32 |  |
| 5.1.14.4 CSI feedback for coordinated topology |  | 33 |  |
|  |  | 34 |  |
| Editor note: Volker will provide text in the next round of comment resolution, see comment 81 of https://men- |  | 35 |  |
|  |  | 36 |  |
| tor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx |  | 37 |  |
| 5.1.15 Relaying, |  | 38 |  |
|  |  | 39 |  |
| 5.1.15.1 Relay Discovery Procedure |  | 40 |  |
|  |  | 41 |  |
| The MAC protocol supports optional relaying mechanism for the cases when dedicated relay terminals are |  | 42 |  |
|  |  | 43 |  |
| available. The relay terminals are named as "relay STAs". The AP keeps track of the relay STAs in the |  | 44 |  |
| environment. At any given point in time, the AP can send an Advanced Modulation Control frame to a STA, |  | 45 |  |
| which contains the APs capabilities, but alos serves as a signal to the STA to transmit its own capabilities |  | 46 |  |
| using an Advanced Modulation Control frame. All the STAs in the environment, that have received an |  | 47 |  |
| Advanced Modulation Control frame including the relay STAs, reply with their capabilities. The AP collects |  | 48 |  |
| the relaying capabilities of the STAs along with the full set of capabilities included in the Advanced |  | 49 |  |
| Modulation Control frame. Subclause 5.8.4.3.1.5 describes the signalling mechanism for the relaying |  | 50 |  |
| capabilities using the Advanced Modulation control frame. Note that this procedure can occur at different |  | 51 |  |
| points in time for the different STAs. |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 89 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 47—Protocol for establishing STA relay capabilities

The PHY layer protocol at each STA measures the signal to interference and noise (SINR) on each subcarrier for each link and signals it to the AP through a CSI control frame. The AP obtains CQI reports, which can also be used for the additional advanced modulation capabilities. Subclause 5.8.4.3.2 describes the structure of the CSI control frame. Note that this procedure can occur at different points in time for the different STAs and also is not obligatory before every relaying instruction, given that the CSI available to the AP is still applicable. The relay discovery procedure is illustrated in Figure 47.

90	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 48—Relay selection procedure

5.1.15.2 Relay Selection Procedure

The relay selection procedure defines selection of the best relay STA and relaying mode among multiple relay STAs. The relay selection procedure is based on the capabilities for each relay STA and the CSI available at the AP. Figure 48 shows the best relay selection algorithm based on the predefined constraints and performance metrics. The AP calculates the constraint metric for each relay with HD/FD and AF/DF relaying modes. Then, the transmitter decides the best relay together with a relaying mode that satisfies the constraint.

5.1.15.3 Relay Link Setup (RLS) Procedure

Following the selection of the relay to be used between the AP and STA, a RLS request frame is sent to the selected relay. The RLS Request frame includes the capabilities and the IDs of the AP, the STA and the relay transfer parameter set. Upon receiving the RLS Request frame, the relay STA shall transmit a RLS Request frame to the destination containing the same information as received within the frame body of the source RLS Request frame. RLS response frame indicates whether the STA can participate in the RLS. Finally, the AP transmits the RLS announcement that indicates the RLS procedure was successfully completed.

5.1.15.4 Frame Exchange Rules

Figure 50 pictures the packet loss and retransmission scenarios for HD relaying. In Figure 50, there is a single user in the network. The AP transmits the first data packet to the STA. The packet is received by both

| Copyright © 20xx IEEE. All rights reserved. | 91 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- the STA and the relay STA. The STA responds as soon as it decodes the header. The response is received by

- both the AP and the relay STA. The AP is aware of the relay STA and waits for its response. The response

- from the relay STA is also received by the AP. The relay STA stores the first data packet until it receives its

- acknowledgment.

5

- Then in the second transmission cycle, the AP sends the second data packet which is received by both the

- STA and the relay STA. The relay STA starts to store the second data packet. First, the STA responds with

- the ACK for the first packet. Then, the relay STA sends its response with the ACK of the first packet and

- deletes the first package from its memory. In the next transmission cycle, the AP transmits the third data

- packet which is this time received by the relay STA only. The STA does not respond to this packet. The relay

- STA waits for the response from the STA over a predetermined amount of time and then sends its response

- to the AP. The response from the relay STA includes the information that it did not receive a response from

- the STA and will retransmit the third packet to the STA. The AP starts to wait for the next response from the

- relay STA. The relay STA retransmits the third packet. The STA responds with the ACK for the second data

- packet. The relay STA also sends its response with ACK from STA. Then, the relay STA deletes the second

- data packet.

17

18	5.1.15.5 RLS Teardown Procedure

19

- The RLS Teardown procedure is to terminate the relay operation. The RLS teardown frame is sent by the AP

- and received by the relay STA. Then, the relay STA terminates the relaying operation for any subsequent

- packets.

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Figure 49—RLS procedure

Figure 50—Packet loss and retransmission scenario in HD relaying

92	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 5.1.16 Color stabilization |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- |
| When a device joins a network (administrated by a coordinator), it advertises its capability of color |  |  |  | 2 |  |
|  |  |  |  | 3 |  |
| stabilization in CSK links as shown in Table 38. It is assumed that at least one link is functioning as a CSK |  |  |  | 4 |  |
| bidirectional link. Otherwise, no color-stabilization functionality is invoked in the network. Also, for the |  |  |  | 5 |  |
| sake of simplicity, it is assumed that only the device will be requested to send color-stabilization updates. |  |  |  | 6 |  |
| The device and the coordinator go through the steps of association as in 5.1.6. Upon the issuance of a |  |  |  | 7 |  |
|  |  |  |  | 8 |  |
| MLME-ASSOCIATE.request the device sends an Association request, among other things advertising its |  |  |  | 9 |  |
| capability for Rx-side CSK-color stabilization.7 Upon reception of this request, the coordinator MLME |  |  |  | 10 |  |
| creates an MLME-ASSOCIATE.indication to the next higher layer in the coordinator. There, a decision is |  |  |  | 11 |  |
| made whether and where color stabilization will be invoked. If the link to be established is a duplex CSK |  |  |  | 12 |  |
| link, the coordinator can also choose to stabilize the color of the device Tx. (As already mentioned, we are |  |  |  | 13 |  |
| describing the case of color stabilization of the coordinator, but the other possible cases can be inferred from |  |  |  | 14 |  |
| the description in a straight-forward manner). After this decision has been made, the pertinent capability |  |  |  | 15 |  |
| negotiation response field in the MLME-ASSOCIATE.response is set according to Table 58 and the |  |  |  | 16 |  |
| pertinent information is then translated by the coordinator MLME into the MAC association response |  |  |  | 17 |  |
| message. Upon reception of this message, the device MLME creates the MLME-ASSOCIATE.confirm and |  |  |  | 18 |  |
| sends it to the next higher layer in the device for further processing. |  |  |  | 19 |  |
|  |  |  |  | 20 |  |
| When the coordinator starts sending CVD frames to the device (identified by the pertinent MAC header as |  |  |  | 21 |  |
| shown in 5.1.16), the device sends color stabilization information back to the coordinator. The MAC |  |  |  | 22 |  |
| command frame used for this can be found in 5.4.22. After a time set in the variable |  |  |  | 23 |  |
| macColorStabilizationTimer, as shown in Table 91, the current information is sent again from the device to |  |  |  | 24 |  |
| the coordinator. If the coordinator wants to change the time between two such updates, it can send a color- |  |  |  | 25 |  |
| stabilization-timer notification command (see 5.4.21) to the device, upon which the device MLME sets the |  |  |  | 26 |  |
| pertinent timer, which is not further described in this standard. |  |  |  | 27 |  |
| Upon dissociation, the macColorStabilization variable is set back to its default value '00'. |  |  |  | 28 |  |
|  |  |  |  | 29 |  |
| 5.1.17 Visibility and dimming support |  |  |  | 30 |  |
|  |  |  |  | 31 |  |
|  |  |  |  | 32 |  |
| The standard supports visibility for the following purposes: |  |  |  | 33 |  |
|  |  |  |  | 34 |  |
|  | a) Alignment (device discovery, negotiation, connection) |  |  | 35 |  |
|  | b) Visible guiding for user alignment |  |  | 36 |  |
|  | c) Infrastructure continuous light output |  |  | 37 |  |
|  | d) Blinking for unexpected interference, disconnection warnings |  |  | 38 |  |
| 5.1.17.1 Visibility pattern |  |  |  | 39 |  |
|  |  |  |  | 40 |  |
| The MAC passes the visibility pattern requirement to the PHY layer via the PLME interface using the |  |  |  | 41 |  |
|  |  |  |  | 42 |  |
| phyDim PIB attribute as shown in Table 144. Sending an idle pattern is a mandatory requirement for |  |  |  | 43 |  |
| infrastructure during idle or receive operation to ensure continuous illumination. Sending an idle pattern is |  |  |  | 44 |  |
| optional for the mobile device. |  |  |  | 45 |  |
| 5.1.17.2 Extended preamble mode for visibility |  |  |  | 46 |  |
|  |  |  |  | 47 |  |
| The MAC provides an extended preamble mode for visibility. The advantage of this mode is to provide |  |  |  | 48 |  |
|  |  |  |  | 49 |  |
| additional time for synchronization while simultaneously providing visibility. |  |  |  | 50 |  |
|  |  |  |  | 51 |  |
|  |  |  |  | 52 |  |
|  |  |  |  | 53 |  |
| 7If not otherwise mentioned, an acknowledgment shall be sent after the reception of each message. |  |  |  | 54 |  |
|  |  | Copyright © 20xx IEEE. All rights reserved. | 93 |  |  |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  |
| --- | --- | --- |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 | Figure 51—Extended preamble mode provided by the MAC |  |
| 12 |  |  |
| 13 |  |  |
| 14 | The MAC uses the knowledge of the idle time and may increase the number of preamble repetitions during |  |
| 15 |  |  |
| 16 | the frame transmission to cover the idle time period. The extended preamble is made continuous to the |  |
| 17 | existing preamble of the next frame transmission. There is a possibility that the idle time may not be an |  |
| 18 | integral multiple of the preamble length. In such cases, it is acceptable to transmit a fraction of the preamble |  |
| 19 | (the latter part) in order to maintain visibility. This fraction of the preamble can be called a truncated |  |
| 20 | preamble. |  |
| 21 | The MAC can choose to either transmit a idle pattern or an extended preamble in the idle mode during |  |
| 22 |  |  |
| 23 | regular operation. The choice is made by the DME and is indicated to the PHY via PLME access to the PHY |  |
| 24 | PIB attribute phyUseExtendedMode (see Table 144). |  |
| 25 | The fast locking pattern (FLP) part of the preamble sequence (1010...) shall be used in the extended |  |
| 26 |  |  |
| 27 | preamble mode, as shown in Figure 52. Since idle time is not an integral multiple of the preamble, only a |  |
| 28 | fraction of the preamble pattern such as '010' can be sent to complete the idle time. |  |
| 29 |  |  |
| 30 |  |  |
| 31 |  |  |
| 32 |  |  |
| 33 |  |  |
| 34 |  |  |
| 35 |  |  |
| 36 |  |  |
| 37 |  |  |
| 38 |  |  |
| 39 |  |  |
| 40 |  |  |
| 41 |  |  |
| 42 | Figure 52—Truncated preamble in extended preamble mode for utilizing idle time for |  |
| 43 |  |  |
|  | visibility |  |
| 44 |  |  |
| 45 |  |  |

- NOTE—the dimming requirements shall be met, even during the preambles. The implementer needs to be mindful of

- this when doing repetitions of the preamble.8

| 48 | 5.1.17.3 Transmitting visibility pattern during uplink for star topology mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 49 |  |  |  |  |  |
| 50 | For the star topology mode, assuming the visibility pattern is sent “in-band” as described in the modulation |  |  |  |  |
| 51 |  |  |  |  |  |
| 52 | domain |  | (see 4.3), the point-and-shoot visibility signal from the mobile device cannot be transmitted |  |  |
| 53 |  |  |  |  |  |
| 54 | 8Notes in text, tables, and figures are given for information only and do not contain requirements needed to implement the standard. |  |  |  |  |
|  | 94 |  |  | Copyright © 20xx IEEE. All rights reserved. |  |
|  |  |  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

continuously since multiple users could be pointing to the infrastructure fixed coordinator. This makes the visibility signal difficult to attain due to the low duty cycle. Hence, the knowledge of idle periods (unused slots) is transmitted by the beacons and the mobile device uses the idle periods for transmitting the visibility pattern to the fixed coordinator. All mobile devices talking to a coordinator can share the empty slots for the CVD frame transmission during uplink.

Figure 53—Usage of CVD frames during star topology operation

5.1.17.4 Dimming override capability

This standard supports bypassing the dimmer functionality during OWC operation. The dimmer control can be set to maximum brightness to facilitate OWC communication. As soon as the OWC communication is completed, the dimmer regains control of the optical source driver and resumes normal operation.

A dimmer override capability request signal is added to the MLME SAP and provided to the external dimmer interface, using the MAC PIB attribute, macDimOverrideRequest, as shown in Table 91. This dimmer override request attribute shall be set to ‘1’ during OWC operation and shall be set to ‘0’ after the communication has been completed. The dimmer circuit can decide whether to accept or reject this request. The response to this dimmer override request signal by the external dimmer circuit is out-of-scope of this standard. The MLME-GET (see 6.3.5) and MLME-SET(see 6.3.12) primitives are used to read and write PIB attributes for dimming.

5.1.17.5 PWM signal override

A PWM signal override request signal is added to the MLME SAP, using the MAC PIB attribute, macDimPWMOverrideRequest, as defined in Table 91 and provided to the external dimmer interface. This PWM override request attribute shall be set to ‘1’ to inform the dimmer circuit that the OWC PHY will be responsible for dimming and to disable any PWM circuit present in the dimmer. The duty cycle for dimming

| Copyright © 20xx IEEE. All rights reserved. | 95 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- is then driven by modulation mode provided by the OWC PHY (such as VPPM). The response to this PWM

- override request signal by the external dimmer circuit is out-of-scope of this standard. The MLME-GET (see

- 6.3.5) and MLME-SET (see 6.3.12) primitives are used to read and write PIB attributes for dimming.

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

5.1.17.6 MAC layer transmission adjustment for dimming

Referring to Figure 54, the infrastructure MAC adjusts the data transmission to match the duty cycle requirements from the dimmer.

Figure 54—MSC for dimming

5.1.17.7 Device discovery and association in the presence of dimming and visibility

The visibility pattern can help with device discovery when the idle pattern or the data has been modified because of the PHY and MAC layer modulation changes to support dimming. Based on the dimming pattern change and duty cycle, the OWC device may choose to associate with a different coordinator that is currently not being dimmed or has a higher duty cycle (more illumination). The visibility pattern is uncoded as shown in Figure 79. The header for the CVD frame is sent at the lowest data rate corresponding to the

96	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

currently negotiated clock rate. Figure 55 shows an example of using the visibility pattern as a signal to establish the best connectivity to an infrastructure device.

Figure 55—Example of using the visibility pattern to establish best connectivity to an infrastructure device

5.1.17.8 Link adaptation for dimming support

Dimming requirements of the infrastructure should be notified to OWC RX device, so that the OWC receiver can adapt to the dimming pattern of the data when VPPM is used. The infrastructure coordinator may receive an external dimming request. A dimming adaptation timer is used that delays the time between the dimming request and the actual dimming of the light source. With this knowledge of an incoming dimming, the link between the devices can be adapted to work at a new (lower) data rate (if dimmed) without requiring the link to be interrupted or possible link failure. This dimming adaptation is indicated and supported by the MAC dimming notification command frame in 5.4.15. Figure 56 shows an example of delay dimming and adapt resources for uninterrupted link.

| Copyright © 20xx IEEE. All rights reserved. | 97 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

Figure 56—Usage of MAC layer to delay dimming and adapt resources for uninterrupted link

5.1.18 Interference coordination

5.1.18.1 General description

- The OWPANs managed by the same master coordinator (MC) forms a OWPAN cluster. In this clause, only

- intra-cluster interference coordination are specified.

32

- At the beginning of the OWPAN establishment, the boundary of the superframe of all the OWPANs in the

- same cluster should be aligned, and different OWPANs may use the same beacon slot for beacon frame

- transmission.

36

- A device and the coordinator should be capable of detecting the presence of other neighboring OWPANs

- that have the overlapped coverage with the OWPAN it associated with. The coordinator is responsible for

39	collect ing the interference information from all devices associated with it and report to the master

- coordinator. The coordinator should be capable of receive the resource coordination information from the

- master coordinator, which will be used for scheduling the allocations in the superframe.

42

- The interference coordination is executed based on the coordination period. The interference information

- that is reported in current coordination period should be used for the resource coordination during next coor-

| 45 | dination period. The duration of a coordination period Tcoordination shall equal to the length of |  |
| --- | --- | --- |
| 46 | macCoordinationTime superframe. |  |
| 47 | The interference coordination procedure includes the following mechanisms. |  |
| 48 |  |  |
| 49 |  |  |

- a)   Interference measurement and report

- b)   Resource coordination

- c)   Interference parameters/resource coordination update

98	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 5.1.18.2 Interference measurement and report |  | 1 |  |
| --- | --- | --- | --- |
| The interference measurement and report shall follow the neighboring OWPANs status monitoring |  | 2 |  |
|  |  | 3 |  |
| procedure, see 5.1.4. |  | 4 |  |
| The coordinator shall report the interference information to the master coordinator every |  | 5 |  |
|  |  | 6 |  |
| macCoordinationTime superframes (i.e., a coordination period). The value of the coordination period can be |  | 7 |  |
| determined by the master coordinator. |  | 8 |  |
| 5.1.18.3 Resource Coordination |  | 9 |  |
|  |  | 10 |  |
| The resource coordination shall be executed for both BP and non-BP (i.e, CAP and CFP) by the global con- |  | 11 |  |
|  |  | 12 |  |
| troller according to the interference information received from the coordinators. For the BP, the global con- |  | 13 |  |
| troller shall coordinate the transmissions of beacons in a TDM manner and allocate different beacon slots for |  | 14 |  |
| the OWPANs that interfere with each other. The structure of the beacon period of a OWPAN refers to section |  | 15 |  |
| 7. For the non-BP part, the master coordinator may coordinate the resources in TDM/WDM/FDM manners, |  | 16 |  |
| which is vendor discretionary. Other rules and criteria for resource coordination is out of the scope of this |  | 17 |  |
| standard. |  | 18 |  |
| The master coordinator allocates resources for the OWPANs that it manages and notify the coordinators |  | 19 |  |
|  |  | 20 |  |
| about the allocations. The coordinator that does not receive the allocations information from the master |  | 21 |  |
| coordinator can makes the scheduling by itself; the coordinator that receive the allocations information from |  | 22 |  |
| the network controller shall take the allocation information into account for scheduling. |  | 23 |  |
| 5.1.18.4 Interference coordination update |  | 24 |  |
|  |  | 25 |  |
| Both the device and the coordinator shall be capable of updating the interference information. The coordina- |  | 26 |  |
|  |  | 27 |  |
| tor shall update the global record of neighboring OWPANs according to the reports from the associated |  | 28 |  |
| devices and its detection of neighboring OWPANs, see 5.1.4. The coordinator shall report the updated inter- |  | 29 |  |
| ference infor-mation to the master coordinator for the coordination in the next coordination period. The mas- |  | 30 |  |
| ter coordinator shall then update the resource coordination using the latest interference information. |  | 31 |  |
|  |  | 32 |  |
|  |  | 33 |  |
|  |  | 34 |  |
|  |  | 35 |  |
|  |  | 36 |  |
|  |  | 37 |  |
|  |  | 38 |  |
|  |  | 39 |  |
|  |  | 40 |  |
|  |  | 41 |  |
|  |  | 42 |  |
|  |  | 43 |  |
|  |  | 44 |  |
|  |  | 45 |  |
|  |  | 46 |  |
|  |  | 47 |  |
|  |  | 48 |  |
|  |  | 49 |  |
|  |  | 50 |  |
|  |  | 51 |  |
|  |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 99 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| GC distributes the allocation |  | GC distributes the allocation | GC distributes the allocation |  |  |
| --- | --- | --- | --- | --- | --- |
| information |  | information | information |  |  |
| GC |  |  |  |  |  |
| Coordinator reports the |  | Coordinator reports the | Coordinator reports the | T |  |
| interference info to GC |  | interference info to GC | interference info to GC |  |  |
| Coordinator |  |  |  |  |  |
| Device1 reports the |  | Device1 updates the |  | T |  |
| interference info to coordinator | interference info to coordinator |  |  |  |  |
| Device1 |  |  |  |  |  |
| Device2 reports the |  |  |  | T |  |
| interference info to coordinator |  |  |  |  |  |

Device2

T

Tcoordination

Figure 57—An example of the interference coordination

5.1.19 Mobility and handover

Handover is used when a device moves from the coverage of one OWPAN to other. Two types of handover procedures are specified,

- handover initiated by device

- handover initiated by master coordinator

5.1.19.1 Handover initiated by device

After association to a OWPAN, a device may search the area for available neighboring coordinators and per-form received signal strength (RSS) measurement. The measurement is based on beacons or reference sig-nals.

A device may perform alpha-filtering on the measurements based on

| RSSn =  1 – RSSn – 1 + Mn | (1) |
| --- | --- |

Where Mn is the latest received measurement result from the physical layer; RSSn is the updated filtered measurement result, that is used for evaluation of reporting criteria or for measurement reporting; RSSn-1 is the old filtered measurement result; α is a filtering-coefficient that can be configured.

If the RSS of neighbor cells satisfy

| RSSn, tg – RSSn, associate = th1 | (2) |
| --- | --- |

100	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Then the device should initiate the handover to the target coordinator. Here RSS tg is the RSS of the target coordinator and RSSassociate is the RSS of the associated coordinator and th1 is a predefined threshold.

Once the handover is initiated by the device, it sends a re-association request command (see 9.5.3) to the tar-get coordinator. The device uses the re-association request to request association as well as to send its pre-ferred QoS requirements to the target coordinator.

In the reassociation response command (see 9.5.4), the target coordinator indicates whether the request is permitted. Besides, the target coordinator also inform the QoS resources allocated to the device, or suggests alternate level of QoS the target coordinator can support.

The previous coordinator may continue to send the packets that have been store in the buffer to the device. The device may receive these packets to its best effort. If the previous coordinator does not received acknowledgement from the device for N consecutive frames, then the previous coordinator consider the device has left the OWPAN and the transmission is ceased.

| Device |  | Old |  | New |  |
| --- | --- | --- | --- | --- | --- |
|  |  | coordinator |  | coordinator |  |

Association and data exchange

Search for Neighbor VPAN and RSS measurement

Re-association request (with QoS requirement)

Re-association response (with QoS resource allocation)

Association and data exchange

Figure 58—Handover initiated by device

5.1.19.2 Handover initiated by master coordinator

After association to a OWPAN, a device may scan the area for available neighboring coordinators and perform received signal strength (RSS) measurement. The measurement is based on beacons or reference signals.

A device may perform alpha-filtering on the measurements based on

| RSSn =  1 – RSSn – 1 + Mn | (3) |
| --- | --- |

| Copyright © 20xx IEEE. All rights reserved. | 101 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- Where Mn is the latest received measurement result from the physical layer; RSSn is the updated filtered

- measurement result, that is used for evaluation of reporting criteria or for measurement reporting; RSSn-1 is

- the old filtered measurement result; α is a filtering-coefficient that can be configured.

| 4 |  |  |
| --- | --- | --- |
| 5 | The device may report the measured RSS of neighboring OWPANs to the coordinator using the procedure |  |
| 6 |  |  |
|  | specified in 5.1.4. |  |
| 7 |  |  |
| 8 | The coordinator can send the measurement report to the master coordinator together with the QoS |  |

- requirement of the device. If the master coordinator decides to handover the device to the target coordinator,

- it sends its decision to the current coordinator. It also notify the target coordinator about the upcoming

- handover together with QoS requirement. The procedures for the communications between master

| 12 | coordinator and the coordinator are out the scope of this specification. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13 |  |  |  |  |  |
| 14 | Current coordinator sends handover command frame to the device. |  |  |  |  |
| 15 |  |  |  |  |  |
| 16 | Then the device sends re-association request (see TBD) to the target device. |  |  |  |  |
| 17 |  |  |  |  |  |
| 18 | Editor note: reassociation request has not been specified. Please John provide text for reassociation request |  |  |  |  |
| 19 |  |  |  |  |  |
|  | comment then refer to it. see comment 102 of https://mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013- |  |  |  |  |
| 20 |  |  |  |  |  |
|  | comment-against-7m-d1.xlsx |  |  |  |  |
| 21 |  |  |  |  |  |
| 22 | In the re-association response command, the target coordinator confirms the handover. Besides, the target |  |  |  |  |
| 23 |  |  |  |  |  |
|  | coordinator also informs the QoS resources allocated to the device, or suggests alternate level of QoS the |  |  |  |  |
| 24 |  |  |  |  |  |
|  | target coordinator can support. |  |  |  |  |
| 25 |  |  |  |  |  |
| 26 |  |  |  |  |  |
| 27 | Device | Old | New | Master |  |
| 28 |  |  |  |  |  |
|  |  | coordinator | coordinator | coordinator |  |
| 29 |  |  |  |  |  |
| 30 | Association and data exchange |  |  |  |  |
| 31 |  |  |  |  |  |
| 32 | Search for Neighbor |  |  |  |  |
| 33 | VPAN and RSS |  |  |  |  |
|  | measurement |  |  |  |  |
| 34 |  |  |  |  |  |
|  | RSS measurement report |  |  |  |  |
| 35 |  |  |  |  |  |
| 36 |  |  | RSS measurement report and QoS requirement |  |  |
| 37 |  |  |  |  |  |
| 38 |  |  |  |  |  |
| 39 |  |  |  | Handover |  |
| 40 |  |  |  | decision |  |
| 41 |  |  | Handover decision |  |  |
| 42 |  |  |  |  |  |
| 43 | Handover command |  | Handover decision |  |  |
| 44 |  |  |  |  |  |
| 45 |  | Re-association request |  |  |  |
| 46 |  |  |  |  |  |
| 47 |  |  |  |  |  |
| 48 | Re-association response with QoS resource allocation |  |  |  |  |
| 49 |  |  |  |  |  |
| 50 |  |  |  |  |  |
| 51 |  |  |  |  |  |
| 52 | Figure 59—Handover initiated by master coordinator |  |  |  |  |
| 53 |  |  |  |  |  |
| 54 |  |  |  |  |  |

102	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.2 MAC frame formats

This subclause specifies the format of the MAC frame (MPDU). Each MAC frame consists of the following basic components:

- A MHR, which comprises frame control, sequence number, address information, and security-related information.

- A MSDU, of variable length, which contains information specific to the frame type. Acknowledgment frames do not contain a payload.

- A MFR, which contains a FCS.

The frames in the MAC sublayer are described as a sequence of fields in a specific order. All frame formats in this subclause are depicted in the order in which they are transmitted by the PHY, from left to right, where the left most bit is transmitted first in time. Bits within each field are numbered from 0 (left most and least significant) to k – 1 (right most and most significant), where the length of the field is k bits. Fields that are longer than a single octet are sent to the PHY in the order from the octet containing the lowest numbered bits to the octet containing the highest numbered bits.

For every MAC frame, all reserved bits shall be ignored upon receipt.

5.2.1 General MAC frame format

The MAC frame format is composed of a MHR, a MSDU, and a MFR. The fields of the MHR appear in a fixed order; however, the addressing fields may not be included in all frames. The general MAC frame shall be formatted as illustrated in Figure 60.

| Octets | 1 | 0/2 | 0/2/8 | 0/2 | 0/2/8 | 0/3/9 | 0/2/8 | 0/5/6/ | variab | 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| : 2 |  |  |  |  |  |  |  | 10/14 | le |  |  |
| Frame | Sequen | Destina- | Destina- | Source | Sourc | Ackno | Poll- | Auxil- | Frame | FCS |  |
| Con- | ce | tion | tion | OWPA | e | wledg | ing | iary | Pay- |  |  |
| trol | Number | OWPAN | Address | N | Addre | e | Field | Security | load |  |  |
|  |  | Identifier |  | Identi- | ss | Field |  | Header |  |  |  |
|  |  |  |  | fier |  |  |  |  |  |  |  |
|  |  |  | Addressing fields |  |  |  |  |  |  |  |  |
|  |  |  |  | MHR |  |  |  |  | MSDU | MFR |  |

Figure 60—General MAC frame format

5.2.1.1 Frame control field

The frame control field is 2 octets in length and contains information defining the frame type, addressing fields, and other control flags. The frame control field shall be formatted as illustrated in Figure 61. Reserved bits are set to zero on transmission and ignored on reception.

5.2.1.1.1 Frame Version subfield

The Frame Version subfield specifies the version number corresponding to the frame. This subfield shall be set to 0b00 to indicate a frame compatible with IEEE Std 802.15.13. All other subfield values shall be reserved for future use.

| Copyright © 20xx IEEE. All rights reserved. | 103 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

| Bits: | 2–5 | 6–8 | 9 | 10 | 11 | 12–13 | 14–15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0–1 |  |  |  |  |  |  |  |  |
| Frame | Reserved | Frame | Security | Frame | Ack | Dest | Source |  |
| Version |  | Type | Enabled | Pending | Request | Addressing | Addressing |  |
|  |  |  |  |  |  | Mode | Mode |  |

Figure 61—Format of the frame control field

5.2.1.1.2 Frame type subfield

The Frame Type subfield shall be set to one of the nonreserved values listed in Table 9.

Table 9—Values of the Frame Type subfield

| Frame type value | Description |  |
| --- | --- | --- |
| b2 b1 b0 |  |  |
| 000 | Beacon |  |
| 001 | Data |  |
| 010 | Acknowledgment |  |
| 011 | Command |  |
| 100 | CVD |  |
| 101 | Control |  |
| 110–111 | Reserved |  |

5.2.1.1.3 Security Enabled subfield

- The Security Enabled subfield is 1 bit in length, and it shall be set to one if the frame is protected by the

- MAC sublayer and shall be set to zero otherwise. The Auxiliary Security Header field of the MHR shall be

- present only if the Security Enabled subfield is set to one.

38

39	5.2.1.1.4 Frame Pending subfield

40

- The Frame Pending subfield is 1 bit in length and shall be set to one if the device sending the frame has more

- data for the recipient. This subfield shall be set to zero otherwise (see 5.1.9.3).

43

- The Frame Pending subfield shall be used only in beacon frames or frames transmitted either during the

- CAP by devices operating on a beacon-enabled OWPAN or at any time by devices operating on a

- nonbeacon-enabled OWPAN.

| 47 |  |
| --- | --- |
| 48 | At all other times, it shall be set to zero on transmission and ignored on reception. |
| 49 |  |
| 50 | 5.2.1.1.5 Acknowledgment Request subfield |
| 51 |  |

- The Acknowledgment Request subfield is 1 bit in length and specifies whether an acknowledgment is

- required from the recipient device on receipt of a data or MAC command frame. If this subfield is set to one,

- the recipient device shall send an acknowledgment frame only if, upon reception, the frame passes the third

104	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

level of filtering as shown in 5.1.9.2. If this subfield is set to zero, the recipient device shall not send an acknowledgment frame.

5.2.1.1.6 Destination Addressing Mode subfield

The Destination Addressing Mode subfield shall be set to one of the nonreserved values listed in Table 62.

If this subfield is equal to zero and the Frame Type subfield does not specify that this frame is an acknowledgment or beacon frame, the Source Addressing Mode subfield shall be nonzero, implying that the frame is directed to the OWC coordinator with the OWPAN identifier as specified in the Source OWPAN Identifier field. If this subfield is equal to 01, the Source Addressing Mode subfield shall be equal to 01, implying that the frame is a broadcast frame, and no source or destination address fields are present in the frame.

Figure 62—Possible values of the Destination Addressing Mode and

Source Addressing Mode subfields

| Addressing mode value | Description |  |
| --- | --- | --- |
| b1 b0 |  |  |
| 00 | OWPAN identifier and address fields are not present. |  |

No address field (broadcast only mode with no address

- fields present). Addresses with all ones of 16 bits or 64 bits are defined as broadcast.

  - Address field contains a 16-bit short address.

  - Address field contains a 64-bit extended address.

- Source Addressing Mode subfield

The Source Addressing Mode subfield shall be set to one of the nonreserved values listed in Table 62.

If this subfield is equal to zero and the Frame Type subfield does not specify that this frame is an acknowledgment frame, the Destination Addressing Mode subfield shall be nonzero, implying that the frame has originated from the coordinator with the OWPAN identifier as specified in the Destination OWPAN Identifier field.

If this subfield is equal to 01, the Source Addressing Mode subfield shall be equal to 01, implying that the frame is a broadcast frame, and no source or destination address fields are present in the frame.

5.2.1.2 Sequence Number field

The Sequence Number field is 1 octet in length and specifies the sequence identifier for the frame.

For a beacon frame, the Sequence Number field shall specify a BSN. For a data, acknowledgment, or MAC command frame, the Sequence Number field shall specify a DSN that is used to match an acknowledgment frame to the data or MAC command frame.

5.2.1.3 Destination OWPAN Identifier field

The Destination OWPAN Identifier field, when present, is 2 octets in length and specifies the unique OWPAN identifier of the intended recipient of the frame. A value of 0xffff in this field shall represent the

| Copyright © 20xx IEEE. All rights reserved. | 105 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- broadcast OWPAN identifier, which shall be accepted as a valid OWPAN identifier by all devices currently

- listening to the channel.

| 3 | This field shall be included in the MAC frame only if the Destination Addressing Mode subfield of the |  |
| --- | --- | --- |
| 4 |  |  |
| 5 | frame control field is 10 or 11. |  |
| 6 | 5.2.1.4 Destination Address field |  |
| 7 |  |  |
| 8 |  |  |

- The Destination Address field, when present, is either 2 octets or 8 octets in length, according to the value

- specified in the Destination Addressing Mode subfield of the frame control field, see 5.2.1.1.6, and specifies

- the address of the intended recipient of the frame. A 16-bit value of 0xffff in this field shall represent the

- broadcast short address, which shall be accepted as a valid 16-bit short address by all devices currently

- listening to the channel.

14

- This field shall be included in the MAC frame only if the Destination Addressing Mode subfield of the

- frame control field is nonzero.

17

18	5.2.1.5 Source OWPAN Identifier field

19

- The Source OWPAN Identifier field, when present, is 2 octets in length and specifies the unique OWPAN

- identifier of the originator of the frame. This field shall be included in the MAC frame only if the Source

- Addressing Mode is nonzero.

23

- The OWPAN identifier of a device is initially determined during association on a OWPAN, but may change

- following a OWPAN identifier conflict resolution as discussed in 5.1.5.

| 26 | 5.2.1.6 Source Address field |  |
| --- | --- | --- |
| 27 |  |  |
| 28 | The Source Address field, when present, is either 2 octets or 8 octets in length, according to the value |  |
| 29 |  |  |

- specified in the Source Addressing Mode subfield of the frame control field, as shown in 5.2.1.1.7, and

- specifies the address of the originator of the frame. This field shall be included in the MAC frame only if the

- Source Addressing Mode subfield of the frame control field is 10 or 11.

33

34	5.2.1.7 Auxiliary Security Header field

35

- The Auxiliary Security Header field has a variable length and specifies information required for security

- processing, including how the frame is actually protected (security level) and which keying material from

- the MAC security PIB is used (see 7.5.1). This field shall be present only if the Security Enabled subfield is

- set to one. For details on formatting, see 7.4.

40

41	5.2.1.8 Frame Payload field

42

- The Frame Payload field has a variable length and contains information specific to individual frame types. If

- the Security Enabled subfield is set to one in the frame control field, the frame payload is protected as

- defined by the security suite selected for that frame.

46

47	5.2.1.9 FCS field

48

- The FCS field is 2 octets in length and is explained in Annex C. The FCS is calculated over the MHR and

- MSDU parts of the frame. The FCS shall be only generated for payloads greater than zero bytes.

51

52

53

54

106	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.2.2 Format of individual frame types

Six frame types are defined: beacon, data, acknowledgment, command, control and CVD. These frame types are discussed in 5.2.2.1 through 5.2.2.4.3.

5.2.2.1 Beacon frame format

The beacon frame shall be formatted as illustrated in Figure 63.

| Octet | 1 |  | 4/10 | 0/5/6/ | 3 | 0/1 | variab | variab | variab | 0/1 | varia | 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s: 2 |  |  |  | 10/14 |  |  | le | le | le |  | ble |  |  |
| Fram | Seque |  | Addres | Auxil- | Super- | BP | CAP | GTS | Pend- | cell- | Bea- | FC |  |
| e | nce |  | sing | iary | frame | Descri | Descri | fields | ing | Search | con | S |  |
| Con- | Num- |  | fields | Secu- | Spec | ptor | ptor | (Figure | addres | Lengt | Pay- |  |  |
| trol | ber |  |  | rity |  |  |  | 64) | s | h | load |  |  |
|  |  |  |  | Header |  |  |  |  | fields |  |  |  |  |
|  |  |  |  |  |  |  |  |  | (Figure |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 65) |  |  |  |  |
|  |  | MHR |  |  |  |  |  | MSDU |  |  |  | M |  |
|  |  |  |  |  |  |  |  |  |  |  |  | FR |  |

Figure 63—Beacon frame format

The GTS fields shall be formatted as illustrated in Figure 64, and the pending address fields shall be formatted as illustrated in Figure 65.

| Octets: 1 | 0/1 | variable |
| --- | --- | --- |
| GTS Specification | GTS Directions | GTS List |

Figure 64—Format of the GTS information fields

| Octets: 1 | variable |
| --- | --- |
| Pending Address Specification | Address List |

Figure 65—Format of the pending address information fields

The order of the fields of the beacon frame shall conform to the order of the general MAC frame as illustrated in Figure 60.

5.2.2.1.1 Beacon frame MHR fields

The MHR for a beacon frame shall contain the frame control field, the Sequence Number field, the Source OWPAN Identifier field, and the Source Address field.

| Copyright © 20xx IEEE. All rights reserved. | 107 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- In the frame control field, the Frame Type subfield shall contain the value that indicates a beacon frame, as

- shown in Table 9, and the Source Addressing Mode subfield shall be set as appropriate for the address of the

- coordinator transmitting the beacon frame. If protection is used for the beacon, the Security Enabled subfield

- shall be set to one. If a broadcast data or command frame is pending, the frame pending subfield shall be set

- to one. All other subfields shall be set to zero by the sender and ignored on reception.

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

The Sequence Number field shall contain the current value of macBSN.

The addressing fields shall comprise only the source address fields. The Source OWPAN Identifier and Source Address fields shall contain the OWPAN identifier and address, respectively, of the device transmitting the beacon.

The Auxiliary Security Header field, if present, shall contain the information required for security processing of the beacon frame, as specified in 5.2.1.7.

5.2.2.1.2 Superframe Specification field

The Superframe Specification field shall be formatted as illustrated in Figure 66.

| Bits: | 4-7 | 8-10 | 8- | 15 | 16 | 17 | 18 | 19- | 12 | 13 | 14 | 15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0–3 |  |  | 11 |  |  |  |  | 22 |  |  |  |  |  |
| Bea- | Super- | OWP | Fin | Bea- | OWP | Associ- | cell- | Cou | Re |  |  |  |  |
| con | frame | AN | al | con | AN | ation | Search | nt | ser |  |  |  |  |
| Order | Order | mode | CA | type | Coor- | Permit | En | dow | ve |  |  |  |  |
|  |  |  | P |  | dina- |  |  | n | d |  |  |  |  |
|  |  |  | Slot |  | tor |  |  |  |  |  |  |  |  |

| Bits: 0-3 | 4-7 | 8-10 | 11-14 | 15 | 16 | 17 | 18 | 19 | 20-23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BeaconOrder | SuperframeOrder | OWPANmode | FinalCAPSlot | RTS/CTS | Beacontype | OWPANCoordinator | AssociationPermit | cellSearch | Countdown |

Figure 66—Format of the Superframe Specification field

The Beacon Order subfield shall specify the transmission interval of the beacon. Refer to 5.1.1 for an explanation of the relationship between the beacon order and the beacon interval.

The Superframe Order subfield shall specify the length of time during which the superframe is active (i.e., receiver enabled), including the beacon frame transmission time. Refer to 5.1.1 for an explanation of the relationship between the superframe order and the superframe duration.

The OWPAN Mode subfield specifies the topology mode of the OWPAN, and shall be contain one of the nonreserved values listed in Table 10.

Table 10—Valid values of the OWPAN Mode subfield

| OWPAN Mode | Description |
| --- | --- |
| 0b000 | Peer-to-Peer Mode |
| 0b001 | Star Mode |

108	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 10—Valid values of the OWPAN Mode subfield

| OWPAN Mode | Description |
| --- | --- |
| 0b010 | Broadcast Mode |
| 0b011 | Relay Mode |
| 0b100 | Coordinated Mode |
| 0b101 - 0b111 | Reserved |

The Final CAP Slot / Broadcast bandwidth subfield defines the modulation bandwidth for broadcast when OWPAN mode is set to 0b010. Editor note: Detailed signaling mapping for broadcast bandwidth is FFS. The Final CAP Slot / Broadcast bandwidth subfield defines the final superframe slot utilized by the CAP if OWPAN mode is set to values other than 0b010. The duration of the CAP, as implied by this subfield, shall be greater than or equal to the value specified by aMinCAPLength. However, an exception is allowed for the accommodation of the temporary increase in the beacon frame length needed to perform GTS maintenance, as in 5.2.2.1.5.

The RTS/CTS subfield shall be set to one if RTS/CTS protocol specified in 5.1.2.2 is enabled in the OWPAN, and set to zero otherwise.

The Beacon Type subfield shall be set to one if the beacon frame is a normal beacon which is transmitted in the beacon period regularly, and set to zero if the beacon frame is an additional beacon which is transmitted in a GTS in the CFP period.

The OWPAN Coordinator subfield shall be set to one if the beacon frame is being transmitted by the coordinator. Otherwise, the OWPAN Coordinator subfield shall be set to zero.

The Association Permit subfield shall be set to one if macAssociationPermit is set to TRUE (i.e., the coordinator is accepting association to the OWPAN). The association permit bit shall be set to zero if the coordinator is currently not accepting association requests on its network.

If the cellSearchEn bit is set, the cellSearchLength is transmitted as an additional field in the beacon frame, as shown in Figure 63.

The Countdown field indicates the number of superframes after which the previously announced new configuration parameters of the OWPAN shall take effect. The value of this field shall decrement by one in each superframe until the Countdown is complete.

5.2.2.1.3 BP Descriptor field

The BP Descriptor field only exists when the OWPAN Mode sub-field in Superframe Specification field is set to 10, i.e., the coordinated mode.

The BP Descriptor field shall be formatted as illustrated in Figure 67.

| Bits: 0 - 3 | 4 - 7 |
| --- | --- |
| Beacon Slot Number | Beacon Slot Used |

Figure 67—BP descriptor field

The Beacon Slot Number subfield shall contain the number of the beacon slots in the BP.

The Beacon Slot Used subfield shall indicates which beacon slot is used by this OWPAN.

| Copyright © 20xx IEEE. All rights reserved. | 109 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

5.2.2.1.5 GTS Specification field

The GTS Specification field shall be formatted as illustrated in Figure 69.

| Bits: 0–2 | 3–6 | 7 |
| --- | --- | --- |
| GTS Descriptor Count | Reserved | GTS Permit |

Figure 69—Format of the GTS Specification field

- The GTS Descriptor Count subfield specifies the number of 3-octet GTS descriptors contained in the GTS

- List field of the beacon frame. If the value of this subfield is greater than zero, the size of the CAP shall be

- allowed to dip below aMinCAPLength to accommodate the temporary increase in the beacon frame length

- caused by the inclusion of the subfield. If the value of this subfield is zero, the GTS Directions field and

- GTS List field of the beacon frame are not present.

54

110	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The GTS Permit subfield shall be set to one if macGTSPermit is equal to TRUE (i.e., the coordinator is accepting GTS requests). Otherwise, the GTS Permit field shall be set to zero.

5.2.2.1.6 GTS Directions field

The GTS Directions field shall be formatted as illustrated in Figure 70.

| Bits: 0–6 | 7 |
| --- | --- |
| GTS Directions Mask | Reserved |

Figure 70—Format of the GTS Directions field

The GTS Directions Mask subfield contains a mask identifying the directions of the GTSs in the superframe. The lowest bit in the mask corresponds to the direction of the first GTS contained in the GTS List field of the beacon frame, with the remainder appearing in the order that they appear in the list. Each bit shall be set to one if the GTS is a receive-only GTS or to zero if the GTS is a transmit-only GTS. GTS direction is defined relative to the direction of the data frame transmission by the device.

5.2.2.1.7 GTS List field

The size of the GTS List field is defined by the values specified in the GTS Specification field of the beacon frame and contains the list of GTS descriptors that represents the GTSs that are being maintained. The maximum number of GTS descriptors shall be limited to seven.

Each GTS descriptor shall be formatted as illustrated in Figure 71.

| Bits: 0–15 | 16–19 | 20–23 |
| --- | --- | --- |
| Device Short Address | GTS Starting Slot | GTS Length |

Figure 71—Format of the GTS descriptor

The Device Short Address subfield shall contain the short address of the device for which the GTS descriptor is intended.

The GTS Starting Slot subfield contains the superframe slot at which the GTS is to begin.

The GTS Length subfield contains the number of contiguous superframe slots over which the GTS is active.

5.2.2.1.8 Pending Address Specification field

The Pending Address Specification field shall be formatted as illustrated in Figure 72.

The Number of Short Addresses Pending subfield indicates the number of 16-bit short addresses contained in the Address List field of the beacon frame.

The Number of Extended Addresses Pending subfield indicates the number of 64-bit extended addresses contained in the Address List field of the beacon frame.

| Copyright © 20xx IEEE. All rights reserved. | 111 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Bits: 0–2 | 3 | 4–6 | 7 |  |
| 2 |  |  |  |  |  |  |
| 3 |  | Number of Short | Reserved | Number of Extended | Reserved |  |
| 4 |  |  |  |  |  |  |
|  |  | Addresses Pending |  | Addresses Pending |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  | Figure 72—Format of the Pending Address Specification field |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 | 5.2.2.1.9 Address List field |  |  |  |  |  |
| 9 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |

- The size of the Address List field is determined by the values specified in the Pending Address Specification

- field of the beacon frame and contains the list of addresses of the devices that currently have messages

- pending with the coordinator. The address list shall not contain the broadcast short address 0xffff.

14

- The maximum number of addresses pending shall be limited to seven and may comprise both short and

- extended addresses. All pending short addresses shall appear first in the list followed by any extended

- addresses. If the coordinator is able to store more than seven transactions, it shall indicate them in its beacon

- on a first-come-first-served basis, ensuring that the beacon frame contains at most seven addresses.

19

20	5.2.2.1.10 Beacon Payload field

21

- The Beacon Payload field is an optional sequence of up to aMaxBeaconPayloadLength octets specified to be

- transmitted in the beacon frame by the next higher layer. The set of octets contained in macBeaconPayload

- shall be copied into this field.

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

5.2.2.2 Data frame format

The data frame shall be formatted as illustrated in Figure 73.

| Octets: 2 | 1 |  | (As defined in | 0/5/6/10/14 | variable | 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5.2.2.2.1) |  |  |  |  |
| frame control | Sequence |  | Addressing | Auxiliary | Data Payload | FCS |  |
|  | Number |  | fields | Security Header |  |  |  |
|  |  | MHR |  |  | MSDU | MFR |  |

Figure 73—Data frame format

- The order of the fields of the data frame shall conform to the order of the general MAC frame as illustrated

- in Figure 60.

44

45	5.2.2.2.1 Data frame MHR fields

46

- The MHR for a data frame shall contain the frame control field, the Sequence Number field, the destination

- OWPAN identifier/address fields, and/or the source OWPAN identifier/address fields.

49

- In the frame control field, the Frame Type subfield shall contain the value that indicates a data frame, as

- shown in Table 9. If protection is used for the data, the Security Enabled subfield shall be set to one. All

| 52 | other subfields shall be set appropriately according to the intended use of the data frame. All reserved |  |
| --- | --- | --- |
| 53 | subfields shall be set to zero by the sender and ignored on reception. |  |
| 54 |  |  |
|  | 112 | Copyright © 20xx IEEE. All rights reserved. |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The Sequence Number field shall contain the current value of macDSN.

The addressing fields shall comprise the destination address fields and/or the source address fields, dependent on the settings in the frame control field.

The Auxiliary Security Header field, if present, shall contain the information required for security processing of the data frame, as specified in 5.2.1.7.

5.2.2.2.2 Data Payload field

The payload of a data frame shall contain the sequence of octets that the next higher layer has requested the MAC sublayer to transmit. The data type field is 1 byte and is explained in Table 12.

Table 12—Data Payload field

| Bits 0–1 | Bits 2–7 | variable |
| --- | --- | --- |
| 00—Single | Number of PPDUs per data frame | Data payload |
| 01—Packed |  |  |
| 10—Burst |  |  |
| 11—Reserved |  |  |

The data type field mentions the format used for sending the data—single, packed, or burst. It also mentions the number of PPDUs that are associated for this data frame.

The payload of a data frame shall contain the sequence of octets that the next higher layer has requested the MAC sub layer to transmit.

5.2.2.3 Acknowledgment frame format

The acknowledgment frame shall be formatted as illustrated in Figure 74.

| Octets: 2 |  | 1 | variable | 2 |
| --- | --- | --- | --- | --- |
| frame control |  | sequence number | B-ACK frame payload | FCS |
|  |  |  | (optional) |  |
|  | MHR |  | MSDU | MFR |

Figure 74—Acknowledgment frame format

The order of the fields of the acknowledgment frame shall conform to the order of the general MAC frame as illustrated in Figure 60. The sequence number is defined in 5.2.2.1.1.

In B- ACK frames, the DestAddr field is set to the SrcAddr of the frame that requested the B- ACK. The B-ACK frame acknowledges correct or incorrect receipt of the previous sequence of frames and provides information for the transmission of the next sequence of frames as described in 5.2.2.3. The B-ACK frame payload is defined in Figure 75.

| Copyright © 20xx IEEE. All rights reserved. | 113 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Octets: 2 | 1 | 1 | 2 | 0 – n |  |
| 2 |  |  |  |  |  |  |
| 3 | buffer size | frame count | reserved | sequence control | frame bitmap |  |
| 4 |  |  |  |  |  |  |
| 5 |  | Figure 75—B-ACK frame payload |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |

- The Buffer Size field specifies the maximum number of octets in the sum of the frame payloads of all frames

- in the next B-ACK sequence. The Frame Count field specifies the maximum number of frames in the next

- B-ACK sequence. The Sequence Control and frame bitmap fields together specify an acknowledgment

- window of MSDU fragments and their reception status. The Sequence Control field specifies the Sequence

- Number and Fragment Number that start the acknowledgment window.

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

| Bits: b15–b14 | b13–b3 | b2–b0 |
| --- | --- | --- |
| reserved | sequence number | fragment number |

Figure 76—B-ACK frame bitmap

The frame bitmap field varies in length. A zero-length frame bitmap field indicates an acknowledgment window of length zero. Otherwise, the least-significant octet of the frame bitmap field corresponds to the MSDU indicated by the Sequence Control field, and each bit of the octet corresponds to a fragment of that MSDU. The least-significant bit in each octet corresponds to the first fragment and successive bits correspond to successive fragments. Successive octets present in the frame bitmap field correspond to successive MSDUs, and each bit corresponds to a fragment of the MSDU. The acknowledgment window ends at fragment seven of the MSDU that corresponds to the most-significant octet in the frame bitmap. For all bits within the frame bitmap, a value of ONE indicates that the corresponding fragment was received in either the current sequence or an earlier one. A value of ZERO indicates that the corresponding fragment was not received in the current sequence (although it may have been received in an earlier one). Bits of the least-significant octet of the frame bitmap field corresponding to fragments prior to the start of the acknowledgment window are undefined. Frames with a Sequence Number earlier than the Sequence Number indicated in the Sequence Control field were not received in the last B-ACK sequence. Such frames were previously received or are no longer expected.

The block ACK is applicable to the packed data type. The bitmap and sequence number is repeated for every frame in the burst mode (multiple frames)

The order of the fields of the acknowledgment frame shall conform to the order of the general MAC frame as illustrated.

The MHR for an acknowledgment frame shall contain only the frame control field and the Sequence Number field.

- In the frame control field, the Frame Type subfield shall contain the value that indicates an acknowledgment

- frame, as shown in Table 9. If the acknowledgment frame is being sent in response to a received data request

- command, the device sending the acknowledgment frame shall determine whether it has data pending for the

- recipient. If the device can determine this before sending the acknowledgment frame (see 5.1.9.4.2), it shall

- set the frame pending subfield according to whether there is pending data. Otherwise, the frame pending

- subfield shall be set to one. If the acknowledgment frame is being sent in response to either a data frame or

- another type of MAC command frame, the device shall set the frame pending subfield to zero. All other

- subfields, except the security enabled subfield, shall be set to zero by the sender and ignored on reception.

114	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The Sequence Number field shall contain the value of the sequence number received in the frame for which the acknowledgment is to be sent.

5.2.2.4 Command frame format

The command frame shall be formatted as illustrated in Figure 77.

| Octets: 2 | 1 |  | (As defined in | 0/5/6/10/14 | 1 | variable | 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5.2.2.4.1) |  |  |  |  |  |
| Frame | Sequence |  | Addressing | Auxiliary | Command | Command | FCS |  |
| Control | Number |  | fields | Security | Frame | Payload |  |  |
|  |  |  |  | Header | Identifier |  |  |  |
|  |  | MHR |  |  | MSDU |  | MFR |  |

Figure 77—Command frame format

The order of the fields of the MAC command frame shall conform to the order of the general MAC frame as illustrated in Figure 60.

5.2.2.4.1 MAC command frame MHR fields

The MHR for a MAC command frame shall contain the frame control field, the Sequence Number field, the destination OWPAN identifier/address fields and/or the source OWPAN identifier/address fields.

In the frame control field, the Frame Type subfield shall contain the value that indicates a MAC command frame, as shown in Table 9. If the frame is to be secured, the Security Enabled subfield of the frame control field shall be set to one and the frame secured according to the process described in 7.5.4. Otherwise the Security Enabled subfield of the frame control field shall be set to zero. All other subfields shall be set appropriately according to the intended use of the MAC command frame. All reserved subfields shall be set to zero by the sender and ignored on reception.

The Sequence Number field shall contain the current value of macDSN.

The addressing fields shall comprise the destination address fields and/or the source address fields, dependent on the settings in the frame control field.

The Auxiliary Security Header field, if present, shall contain the information required for security processing of the MAC command frame, as specified in 5.2.1.7.

5.2.2.4.2 Command Frame Identifier field

The Command Frame Identifier field identifies the MAC command being used. This field shall be set to one of the nonreserved values listed in Table 29.

5.2.2.4.3 Command Payload field

The Command Payload field contains the MAC command itself. The formats of the individual commands are described in 5.4.

| Copyright © 20xx IEEE. All rights reserved. | 115 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.2.2.5 Control frame format

The Control frame shall be formatted as illustrated in Figure 78.

| Octets: |  |  |  | 1 | 2 |  |  | 1 | 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Band- |  |  |
| Frame | Sequen- |  |  | Control | Dura- |  |  |  |  |  |
|  | ceNum- |  |  | frame |  | RA | TA | width | FCS |  |
| control |  |  |  |  | tion |  |  |  |  |  |
|  | ber |  |  | type |  |  |  | Info |  |  |
|  | MHR |  |  |  |  | MSDU |  |  | MFR |  |

Figure 78—Control frame format

The order of the fields of the control frame shall conform to the order of the general MAC frame as illus-trated in Figure 60.

5.2.2.5.1 Control frame MHR fields

The MHR for a control frame shall contain the frame control field, the SequenceNumber field.

In the frame control field, the Frame Type subfield shall contain the value that indicates a control frame, as shown in Table 9. The Source Addressing Mode and Destination Addressing Mode shall be set to 01, to indicate that the control frames are broadcast frames.

5.2.2.5.2 Control frame type field

The control frame type field identifies the type of the MAC control frame being used. This field shall be set to one of the nonreserved values in Table yy.

Table yy Control frames

| Control frame type hex value | Control frame name |
| --- | --- |
| 0x01 | RTS |
| 0x02 | CTS |
| 0x03-0xff | Reserved |

5.2.2.5.3 Duration field

If the Control frame type subfield is set to 0x01, i.e., for all RTS frames, the duration time value is the time, in microseconds, required to transmit the pending PPDU, plus one CTS frame, plus one ACK frame (if acknowledge is required), plus the IFSs: two SIFS and a tack (if acknowledge is required). If the calculated duration includes a fractional microsecond, that value is rounded up to the next higher integer.

If the Control frame type subfield is set to 0x02, i.e., for CTS frames, the duration value is the value obtain from the Duration subfield of the immediately previous RTS frame, minus the time, in microseconds, required to transmit the CTS frame and its SIFS interval. If the calculated duration includes a fractional microsecond, that value is rounded up to the next higher integer.

5.2.2.5.4 RA field

The RA field of a RTS frame is the 64-bit extended address of the device or coordinator that is the intended immediate recipient of the pending individually addressed data, command frame.

The RA field of a CTS frame is set to the 64-bit extended address of the device or coordinator that sent the immediately previous RTS frame.

5.2.2.5.5 TA field

The TA field of a RTS frame is the 64-bit extended address of the device or coordinator that sends this control frame.

5.2.2.5.6 Bandwidth Info

In a RTS frame, this subfield is used to indicate the recommended bandwidth that will be used to modulate the immediately pending PPDU and shall be set to the one of the nonreserved values in Table zz.

In a CTS frame, this subfield is used to indicate the bandwidth that is assigned to the receiver of the CTS frame by the coordinator for the modulation of immediately pending PPDU.

Table zz - Bandwidth Info

| Bandwidth Info values hex value | Bandwidth |
| --- | --- |
| 0x01 | 5MHz |
| 0x02 | 10MHz |
| 0x03 | 15MHz |
| 0x04 | 20MHz |
| 0x05 | 40MHz |
| 0x06 | 50MHz |
| 0x07 | 100MHz |
| 0x08 | 200MHz |
| 0x09 | 600MHz |
| 0x0A | 1000MHz |
| 0x0B-0xff | Reserved |

5.2.2.6 CVD frame format

The structure of the CVD frame is as shown in Figure 79. The CVD frame is used to visually provide information on the communication status, such as misalignment between the two devices, transmission

116	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Octets: 2 | Octet: 2 | variable |
| --- | --- | --- |
| Frame control | FCS | Visibility pattern |
| MHR | MFR |  |

Figure 79—CVD frame

direction, or sending data status; the data transmission quality; and the transferred file size and remaining file size. The visibility pattern has no error protection. The length of the visibility pattern shall be set in the PHY header and the FCS shall not include the visibility pattern of the CVD frame. The FCS is only calculated over the frame control field (MHR) using the cyclic redundancy check (CRC) described in Annex C. The visibility pattern will be generated based on the dimming level requirements and is described in 8.5.1.2. The CVD frame is used by the infrastructure to maintain visibility at all times and by the mobile device for point-and-shoot. The CVD frame can also be used for color stabilization for PHY III as explained in 8.6. It should be noted that the CVD frame is not used for communicating the dimming level; rather, the dimming notification command is used for this function as described in 5.4.15.

The CVD frame is sent at the currently negotiated optical clock.

5.2.2.7 Data Null Frame

Editor note: whether Data Null Frame is a new MAC frame type, a new command frame or a new type of acknowledgment frame should be further decided. See comment 123 of https://mentor.ieee.org/802.15/dcn/ 17/15-17-0297-02-0013-comment-against-7m-d1.xlsx

The data null frame is used for acknowledgment and polling when no data or control / management information should be exchanged between two communication nodes. The frame header contains the MAC header fields defined in subclause 6.4.1 and the frame body is left empty.

5.3 MAC frame formats

Editor note: Nikola need to instruct how to merge clause 5.2 and clause 5.3. See comment 124 of https:// mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx

The format of the MAC frames is specified in this clause. A STA shall be able to properly construct a subset of the frames specified in this clause for transmission and to decode a subset of the frames specified in this clause upon validation following reception. All STAs shall be able to validate every received frame using the frame check sequence (FCS) and to interpret certain fields from the MAC headers of all frames.

Each frame consists of the following basic components:

- A MAC header, which comprises frame control, acknowledgment information, address, \and sequence control information;

- A variable length frame body, which contains information specific to the frame type and subtype;

- A FCS, which contains an IEEE 32-bit CRC.

| Copyright © 20xx IEEE. All rights reserved. | 117 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 5.3.1 Conventions |
| --- | --- |
| 2 |  |

- The MPDUs or frames in the MAC sublayer are described as a sequence of fields in specific order. Each fig-

- ure in this clause depicts the fields/subfields as they appear in the MAC frame and in the order in which they

- are passed to the physical layer convergence procedure (PLCP), from left to right.

6

- In figures, all bits within fields are numbered, from 0 to k, where the length of the field is k + 1 bits. The

- octet boundaries within a field can be obtained by taking the bit numbers of the field modulo 8. Octets

- within numeric fields that are longer than a single octet are depicted in increasing order of significance, from

- lowest numbered bit to highest numbered bit. The octets in fields longer than a single octet are sent to the

- PLCP in order from the octet containing the lowest numbered bits to the octet containing the highest num-

- bered bits.

13

- Any field containing a CRC is an exception to this convention and is transmitted commencing with the coef-

- ficient of the highest-order term.

16

- MAC addresses are assigned as ordered sequences of bits. The Individual/Group bit is always transferred

- first and is bit 0 of the first octet.

19

- Values specified in decimal are coded in natural binary unless otherwise stated. The values in Table 1 are in

- binary, with the bit assignments shown in the table. Values in other tables may be shown in decimal notation.

- Reception, in references to frames or fields within frames (e.g., received Beacon frames or a received Dura-

- tion/ID field), applies to MPDUs or MAC management protocol data units (MMPDUs) indicated from the

- PHY layer without error and validated by FCS within the MAC sublayer. Without further qualification,

- reception by the MAC sublayer implies that the frame contents are valid, and that the protocol version is

- supported, with no implication regarding frame addressing or regarding whether the frame type or other

- fields in the MAC header are meaningful to the MAC entity that has received the frame.

29

30	5.3.2 General frame format

31

- The MAC frame format comprises a set of fields that occur in a fixed order in all frames. Figure 5.8.2.1

- depicts the general MAC frame format. All fields, except for the Frame Body (used for delivering payload

- information and data), are present in all types of MAC frames. Each field is defined in 5.8.3. The format of

- each of the individual subtypes of each frame type is defined in 5.8.4.

36

- The Frame Body field is of variable size. The maximum frame body size is determined by the maximum

- MSDU size (2304 octets) plus any overhead from security encapsulation.

39

40

41

42

43

44

45

46

47

48

49

50

51

Figure 80—General MAC frame format.

5.3.3 Frame fields

5.3.3.1 Frame Control field

- The Frame Control field consists of the following subfields: Protocol Version, Type, Subtype, and a

- Reserved subfield. The format of the Frame Control field is illustrated in Figure 81.

54

118	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 81—Frame control field format.

5.3.3.1.1 Protocol Version

Two-bit field indicating the MAC protocol frame version. The 802.15.7 protocol version is indicated with the value of '00' for this field, while the current 802.15.7r1 protocol version is indicated with the value '10'.

5.3.3.1.2 Type and Subtype fields

The Type field is 2 bits in length, and the Subtype field is 4 bits in length. The Type and Subtype fields together identify the function of the frame. There are two frame types in the current MAC layer protocol description: data, and management. Each of the frame types has several defined subtypes. Table 13 defines the valid combinations of type and subtype. (The numeric values in Table 1 are shown in binary.)

Table 13—Type and Subtype field values for different frame types

| Type value | Type description | Subtype value | Subtype description |  |
| --- | --- | --- | --- | --- |
| b3 b2 |  | b7 b6 b5 b4 |  |  |
| 00 | Management | 0000 | Association request |  |
| 00 | Management | 0001 | Association response |  |
| 00 | Management | 0010 | Reassociation request |  |
| 00 | Management | 0011 | Reassociation response |  |
| 00 | Management | 0100 | Probe request |  |
| 00 | Management | 0101 | Probe response |  |
| 00 | Management | 0110 - 0111 | Reserved |  |
| 00 | Management | 1000 | Beacon |  |
| 00 | Management | 1001 | ATIM |  |
| 00 | Management | 1010 | Disassociation |  |
| 00 | Management | 1011 | Authentication |  |
| 00 | Management | 1100 | Deauthentication |  |
| 00 | Management | 1101 | Action |  |
| 00 | Management | 1101 - 1111 | Reserved |  |
| 10 | Data | 0000 | Data |  |
| 10 | Data | 0001 - 0011 | Reserved |  |
| 10 | Data | 0100 | Null (no data) |  |

| Copyright © 20xx IEEE. All rights reserved. | 119 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

|  | IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC |  |  |  |  | IEEE Draft P802.15.13/D0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | Table 13—Type and Subtype field values for different frame types |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  | Type value |  | Type description | Subtype value | Subtype description |  |  |
| 4 |  |  |  |  |  |  |  |  |
|  |  | b3 b2 |  |  | b7 b6 b5 b4 |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
|  |  | 10 |  | Data | 0101 - 1111 | Reserved |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
|  |  | 01 |  | Reserved | 0000 - 1111 | Reserved |  |  |
| 8 |  |  |  |  |  |  |  |  |
|  |  | 11 |  | Control | 0000 | Waveform Control |  |  |
| 9 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |
|  |  | 11 |  | Control | 0001 | Advanced Modulation Control |  |  |
| 11 |  |  |  |  |  |  |  |  |
|  |  | 11 |  | Control | 0010 | CSI Control |  |  |
| 12 |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |
|  |  | 11 |  | Control | 0011 | Dimming Control |  |  |
| 14 |  |  |  |  |  |  |  |  |
|  |  | 11 |  | Reserved | 0100 - 1111 | Reserved |  |  |
| 15 |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |
| 17 | 5.3.3.1.3 Reserved field |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |

- The Reserved field constitutes the rest of the Frame Control field and has been introduced for possible

- extension of the frame control field.

| 22 | 5.3.3.2 Reserved field |  |
| --- | --- | --- |
| 23 |  |  |
| 24 | Two octets reserved for possible protocol frame extension. |  |
| 25 |  |  |
| 26 | 5.3.3.3 Acknowledgment Information (Ack Info) field |  |
| 27 |  |  |
| 28 |  |  |

- In all existing frame types and subtypes (except for the Beacon frame), the Ack Info field contains the infor-

- mation necessary to identify the MSDU or MMPDU sequence number and the station which transmitted the

- acknowledged packet. The information in this field has the format depicted in Figure 82.

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Figure 82—Ack Info field format.

5.3.3.3.1 Sequence #

Bits 0 to 7 contain the address of the STA which transmitted the packet which is being acknowledged. In the uplink transmission, these bits identify the STA transmitting the current packet as the acknowledgment can be only for packets transmitted by the AP.

5.3.3.3.2 Reserved

Bits 8 to 11 are reserved for possible frame extension or functionality extension for packet fragmentation support.

120	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 5.3.3.3.3 Sequence Number |  |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bits 12 to 23 identify the number of the packet which is being acknowledged. |  |  |  |  | 2 |  |
|  |  |  |  |  | 3 |  |
| 5.3.3.3.4 ACK |  |  |  |  | 4 |  |
|  |  |  |  |  | 5 |  |
| Bit 24 is set to '1' when a packet is being acknowledged with the current frame, and set to '0' otherwise. |  |  |  |  | 6 |  |
|  |  |  |  |  | 7 |  |
| Bit 25 is set to '1' when the last Beacon frame reception is being acknowledged, and set to '0' otherwise. |  |  |  |  | 8 |  |
|  |  |  |  |  | 9 |  |
| 5.3.3.3.5 Reserved |  |  |  |  | 10 |  |
|  |  |  |  |  | 11 |  |
| Bits 26 to 31 are reserved for possible future extension of the functionality specification. |  |  |  |  | 12 |  |
|  |  |  |  |  | 13 |  |
| 5.3.3.4 Receiver Address field |  |  |  |  | 14 |  |
|  |  |  |  |  | 15 |  |
| The Receiver Address field is a 48-bit field indicating the network address of the STA for which the trans- |  |  |  |  | 16 |  |
|  |  |  |  |  | 17 |  |
| mitted packet is intended. |  |  |  |  | 18 |  |
| 5.3.3.5 Transmitter Address field |  |  |  |  | 19 |  |
|  |  |  |  |  | 20 |  |
| The Transmitter Address field is a 48-bit field indicating the network address of the STA transmitting the |  |  |  |  | 21 |  |
|  |  |  |  |  | 22 |  |
| packet. |  |  |  |  | 23 |  |
| 5.3.3.6 Reserved field |  |  |  |  | 24 |  |
|  |  |  |  |  | 25 |  |
| This field contains 48 bits which are reserved for future references in an extension of the MAC protocol. |  |  |  |  | 26 |  |
|  |  |  |  |  | 27 |  |
| 5.3.3.7 Sequence Control field |  |  |  |  | 28 |  |
|  |  |  |  |  | 29 |  |
| In data frames, the Sequence Control field contains the information necessary to identify the MSDU or |  |  |  |  | 30 |  |
|  |  |  |  |  | 31 |  |
| MMPDU sequence number. The information in this field has the format depicted in Figure 83. Bits 4 to 15 |  |  |  |  | 32 |  |
| identify the packet sequence number, while bits 0 to 3 are reserved for possible future extensions of the pro- |  |  |  |  | 33 |  |
| tocol functionality such as packet fragmentation support. This same information is used by the receiving sta- |  |  |  |  | 34 |  |
| tion and placed in the Ack Info field of a subsequent frame used for package acknowledgment. |  |  |  |  | 35 |  |
|  |  |  |  |  | 36 |  |
|  |  |  |  |  | 37 |  |
|  |  |  |  |  | 38 |  |
|  |  |  |  |  | 39 |  |
|  |  |  |  |  | 40 |  |
|  |  |  |  |  | 41 |  |
|  | Figure 83—Sequence Control field format. |  |  |  | 42 |  |
|  |  |  |  |  | 43 |  |
|  |  |  |  |  | 44 |  |
| 5.3.3.8 Reserved field |  |  |  |  | 45 |  |
|  |  |  |  |  | 46 |  |
| This field contains 16 bits which are reserved for future references in an extension of the MAC protocol. |  |  |  |  | 47 |  |
|  |  |  |  |  | 48 |  |
| 5.3.3.9 5.8.3.9 Frame Body field |  |  |  |  | 49 |  |
|  |  |  |  |  | 50 |  |
| The Frame Body is a variable length field that contains information specific to individual frame types and |  |  |  |  | 51 |  |
|  |  |  |  |  | 52 |  |
| subtypes as defined in subclause 5.8.4. The minimum frame body is 0 octets. The maximum length frame |  |  |  |  |  |  |
|  |  |  |  |  | 53 |  |
|  |  |  |  |  | 54 |  |
|  | Copyright © 20xx IEEE. All rights reserved. |  |  | 121 |  |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- body is defined by the maximum length (MSDU + ICV + higher layer overhead), where ICV stands for

- integrity check value.

3

4	5.3.3.10 Frame check sequence (FCS) field

5

- The FCS field is a 32-bit field containing a 32-bit CRC. The FCS is calculated over all the fields of the MAC

- header and the Frame Body field. These are referred to as the calculation fields.

| 8 | The FCS is calculated using the following standard generator polynomial of degree 32: |  |
| --- | --- | --- |
| 9 |  |  |
| 10 | G(x) = x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1 |  |
| 11 |  |  |
| 12 |  |  |
| 13 | The FCS is the ones complement of the sum (modulo 2) of the following: |  |
| 14 |  |  |

- a) The remainder of xk × (x31 + x30 + x29 + …+ x2 + x + 1) divided (modulo 2) by G(x), where k is the num-

- ber of bits in the calculation fields, and

17

- b) The remainder after multiplication of the contents (treated as a polynomial) of the calculation fields by

- x32 and then division by G(x).

20

21 The FCS field is transmitted commencing with the coefficient of the highest-order term. 22

- As a typical implementation, at the transmitter, the initial remainder of the division is preset to all ones and

- is then modified by division of the calculation fields by the generator polynomial G(x). The ones comple-

- ment of this remainder is transmitted, with the highest-order bit first, as the FCS field.

26

- At the receiver, the initial remainder is preset to all ones and the serial incoming bits of the calculation fields

- and FCS, when divided by G(x), results in the absence of transmission errors, in a unique nonzero remainder

- value. The unique remainder value is the polynomial:

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

x31 + x30 + x26 + x25 + x24 + x18 + x15 + x14 + x12 + x11 + x10 + x8 + x6 + x5 + x4 + x3 + x + 1

5.3.4 Specific Frame Formats

5.3.4.1 Management Frames Format

5.3.4.1.1 Management Frames Information Components

5.8.4.1.1.1 Beacon Frame Format

The Beacon frame body contains the following information in the presented order:

Table 14—Beacon Frame Body

| Order | Information |
| --- | --- |
| 1 | Timestamp |
| 2 | Beacon Interval |
| 3 | Capability |
| 4 | Service Set Identifier (SSID) |
| 5 | Supported Rates |

122	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 14—Beacon Frame Body

| Order | Information |
| --- | --- |
| 6 | DS Parameter Set |
| 7 | Traffic Indication Map |
| 8 | Country |
| 9 | Extended Support Rates |
| 10 | Vendor Specific Information |

5.8.4.1.1.2 IBSS ATIM Frame Format

The body of an IBSS ATIM management frame is null.

5.8.4.1.1.3 Disassociation Frame Format

The Disassociation frame body contains the following information in the presented order:

Table 15—Disassociation Frame Body

Order	Information

  - Reason Code

  - Vendor Specific Information

- Association Request Frame Format

The Association Request frame body contains the following information in the presented order:

Table 16—Association Request Frame Body

| Order | Information |
| --- | --- |
| 1 | Capability |
| 2 | Listen Interval |
| 3 | SSID |
| 4 | Supported Rates |
| 5 | Extended Supported Rates |
| 6 | Vendor Specific Information |

| Copyright © 20xx IEEE. All rights reserved. | 123 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.8.4.1.1.5 Association Response Frame Format

The Association Response frame body contains the following information in the presented order:

Table 17—Association Response Frame Body

| Order | Information |
| --- | --- |
| 1 | Capability |
| 2 | Status Code |
| 3 | AID |
| 4 | Supported Rates |
| 5 | Extended Supported Rates |
| 6 | Vendor Specific Informatino |

5.8.4.1.1.6 Reassociation Request Frame Format

The Reassociation Request frame body contains the following information in the presented order:

Table 18—Reassociation Request Frame Body

| Order | Information |
| --- | --- |
| 1 | Capability |
| 2 | Listen Interval |
| 3 | Current AP address |
| 4 | SSID |
| 5 | Supported Rates |
| 6 | Extended Supported Rates |
| 7 | Vendor Specific Information |

5.8.4.1.1.7 Reassociation Response Frame Format

The Reassociation Response frame body contains the following information in the presented order: 5.8.4.1.1.8 Probe Request Frame Format

The Probe Request frame body contains the following information in the presented order:

124	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 19—Reassociation Response Frame Body

| Order | Information |
| --- | --- |
| 1 | Capability |
| 2 | Status Code |
| 3 | AID |
| 4 | Supported Rates |
| 5 | Extended Supported Rates |
| 6 | Vendor Specific Information |

Table 20—Probe Request Frame Body

| Order | Information |
| --- | --- |
| 1 | SSID |
| 2 | Supported Rates |
| 3 | Request Information |
| 4 | Extended Supported Rates |

  - Vendor Specific Information

- Probe Response Frame Format

The Probe Response frame body contains the following information in the presented order:

Table 21—Probe Response Frame Body

| Order | Information |
| --- | --- |
| 1 | Timestamp |
| 2 | Beacon Interval |
| 3 | Capability |
| 4 | Service Set Identifier (SSID) |
| 5 | Supported Rates |
| 6 | Country |
| 7 | Extended Supported Rates |
| 8 | Vendor Specific Information |
| 9 | Requested information elements |

| Copyright © 20xx IEEE. All rights reserved. | 125 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.8.4.1.1.10 Authentication Frame Format

The Probe Request frame body contains the following information in the presented order:

Table 22—Authentication Frame Body

| Order | Information |
| --- | --- |
| 1 | Authentication Algorithm Number |
| 2 | Authentication Transaction Sequence Number |
| 3 | Status Code |
| 4 | Challenge Text |
| 5 | Vendor Specific |

Table 23—Challenge text information

| Authentication | Authentication Transaction | Status Code | Challenge Text |  |
| --- | --- | --- | --- | --- |
| Algorithm | Sequence No. |  |  |  |
| Open System | 1 | Reserved | Not present |  |
| Open System | 2 | Status | Not present |  |
| Shared Key | 3 | Reserved | Not present |  |
| Shared Key | 4 | Status | Present |  |
| Shared Key | 5 | Reserved | Present |  |
| Shared Key | 6 | Status | Not present |  |

5.8.4.1.1.11 Deauthentication Frame Format

The Deauthentication frame body contains the following information in the presented order:

Table 24—Deauthentication Frame Body

Order	Information

- Reason Code

- Vendor Specific Information

126	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.8.4.1.1.12 Action Frame Format

The Action frame body contains the following information in the presented order:

Table 25—Action Frame Body

Order	Information

  - Action

  - Vendor Specific Information

- Management Frame Body Fields

5.8.4.1.2.1 Authentication Algorithm Number Field

The Authentication algorithm number is a 16 -bit field, which indicates a single algorithm used for authenti-cation (see Figure 84). Two values have been defined:

'0' Open System

'1'  Shared Key

'2-65536'  Reserved for future use

Figure 84—Authentication algorithm number field.

5.8.4.1.2.2 Authentication Transaction Sequence Number Field

The authentication transaction sequence number is a 16-bit field, which indicates the current step in a multi-step authentication procedure (see Figure 85).

Figure 85—Authentication transaction sequence number field.

5.8.4.1.2.3 Beacon Interval Field

| Copyright © 20xx IEEE. All rights reserved. | 127 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

Figure 86—Beacon interval field.

The beacon interval field is a 16-bit field, which indicates the interval in time units between two consecutive beacon frames (see Figure 86).

5.8.4.1.2.4 Capability Information Field

The beacon interval field is a 16- bit field, which indicates the AP capabilities with regards to optional ser-vices (see Figure 87). The field is left unspecified and reserved for future use.

Figure 87—Capability Information field.

5.8.4.1.2.5 Current AP Address Field

- The current AP address field is a 48-bit field, which indicates the address of the AP with which the STA is

- currently associated. The field is illustrated in Figure 88.

| 31 |  |  |  |
| --- | --- | --- | --- |
| 32 |  |  |  |
| 33 |  |  |  |
| 34 |  |  |  |
| 35 |  |  |  |
| 36 |  |  |  |
| 37 |  | Figure 88—Current AP address field. |  |
| 38 |  |  |  |
| 39 |  |  |  |
| 40 | 5.8.4.1.2.6 Listen Interval Field |  |  |
| 41 |  |  |  |
| 42 |  |  |  |

- The listen interval field is a 16-bit field, which indicates to the AP how often the STA wakes up to listen for

- Beacon management frames. The field is illustrated in Figure 89. The value of the field is expressed in units

- of Beacon Intervals.

46

47

48

49

50

51

52

53

54

Figure 89—Listen interval field.

128	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.8.4.1.2.7 Reason Code Field

The Reason Code is a 16-bit field, which indicates the reason a notification management frame of type Dis-association or Deauthentication has been generated. The field is illustrated in Figure 90. The values of the field are specified in Table 26.

Figure 90—Reason Code field.

|  | Table 26—Reason codes |  |
| --- | --- | --- |
| Reason Code | Meaning |  |
| 0 | Reserved |  |
| 1 | Unspecified reason |  |
| 2 | Previous authentication no longer valid |  |
| 3 | Deauthentication because sending STA is leaving (or has left) IBSS or ESS |  |
| 4 | Disassociated due to inactivity |  |
| 5 | Disassociated because AP is unable to handle all currently associated STAs |  |
| 6 | Class 2 frame received from nonauthenticated STA |  |
| 7 | Class 3 frame received from nonassociated STA |  |
| 8 | Disassociated because sending STA is leaving (or has left) BSS |  |
| 9 | STA requesting (re)association is not authenticated with responding STA |  |
| 10 - 12 | Reserved |  |
| 13 | Invalid information element |  |
| 14 | Message integrity code (MIC) failure |  |
| 15 | 4-Way Handshake timeout |  |
| 16 | Group Key Handshake timeout |  |
| 17 | Information element in 4-Way Handshake different from (Re)Association |  |
|  | Request / Probe Response / Beacon frame |  |
| 18 | Invalid group cipher |  |
| 19 | Invalid pairwise cipher |  |
| 20 | Invalid AKMP |  |
| 21 | Unsupported RSN information element version |  |
| 22 | Invalid RSN information element capabilities |  |

| Copyright © 20xx IEEE. All rights reserved. | 129 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

|  | Table 26—Reason codes |
| --- | --- |
| Reason Code | Meaning |
| 23 | IEEE 802.1X authentication failed |
| 24 | Cipher suite rejected because of the security policy |
| 25 - 35 | Reserved |
| 36 | Requested from peer STA as the STA is leaving the BSS (or resetting) |
| 37 | Requested from peer STA as it does not want to use the mechanism |
| 38 | Requested from peer STA as the STA using wrong mechanism |
| 39 | Requested from peer STA due to timeou |
| 40 - 44 | Reserved |
| 45 | Peer STA does not support the requested cipher suite |
| 46 - 65535 | Reserved |

5.8.4.1.2.8 Association ID (AID) Field

The AID field is a 16-bit field, which is a station ID assigned by an AP during association. The field is illus-trated in Figure 91. The value of the field is in the range 1 - 2007 and is placed in the 14 LSBs of the AID field, with the two MSBs of the field set to '11'.

Figure 91—Association ID field.

5.8.4.1.2.9 Status Code Field

- The Status Code field is a 16-bit field used in response to a management frame in order to indicate the suc-

- cess or the failure of the operation. The field is illustrated in Figure 92. The possible values of this field are

- presented in Table 27.

| 41 |  |  |
| --- | --- | --- |
| 42 |  |  |
| 43 |  |  |
| 44 |  |  |
| 45 |  |  |
| 46 |  |  |
| 47 | Figure 92—Status Code field. |  |
| 48 |  |  |
| 49 |  |  |
| 50 |  |  |

- 5.8.4.1.2.10 Timestamp Field

54

130	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  | Table 27—Status codes |
| --- | --- |
| Reason Code | Meaning |
| 0 | Successful |
| 1 | Unspecified failure |
| 2 - 9 | Reserved |
| 10 | Cannot support all requested capabilities in the Capability Information field |
| 11 | Reassociation denied due to inability to confirm that association exists |
| 12 | Association denied due to reasons outside the scope of this standard |
| 13 | Responding STA does not support the specified authentication algorithm |
| 14 | Received an unexpected authentication transaction sequence number |
| 15 | Authentication rejected because of challenge failure |
| 16 | Authentication rejected due to timeout waiting for next frame in sequence |
| 17 | Association denied because AP is unable to handle additional associated STAs |
| 18 | Association denied due to requesting STA not supporting all basics data rates |
| 19 - 33 | Reserved |
| 34 | Association denied due to excessive frame loss rate / poor channel conditions |
| 35 - 36 | Reserved |
| 37 | The request has been declined |
| 38 | The request has not been successful due to invalid parameter values |
| 39 | Reserved |
| 40 | Invalid information element (content is invalid) |
| 41 | Invalid group cipher |
| 42 | Invalid pairwise cipher |
| 43 | Invalid AKMP |
| 44 - 45 | Reserved |
| 46 | Cipher suite rejected because of security policy |
| 47 - 48 | Reserved |
| 49 | The destination STA is not within this BSS |
| 50 | Reserved |
| 51 | Association denied because the Listen Interval is too large |
| 52 - 65535 | Reserved |

The Timestamp field is a 64-bit field representing the value of a timing synchronization function. The field is illustrated in Figure 93.

| Copyright © 20xx IEEE. All rights reserved. | 131 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  |  |
| --- | --- | --- | --- |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
|  |  | Figure 93—Timestamp field. |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 | 5.8.4.1.3. Management Frame Information Elements |  |  |
| 9 |  |  |  |
| 10 |  |  |  |

- The information elements are defined to have a common general format which consists of 1 octet specifying

- the element ID, 1 octet specifying the length of the element field in octets, and a variable-length element

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

field with the information contained in the respective element. The overall structure is presented in Figure 94. The set of valid information elements is defined in Table 28.

Figure 94—Information element format.

Table 28—Valid set of information elements

| Information Element | Element ID |  | Length (in octets) |
| --- | --- | --- | --- |
| SSID |  | 0 | 2 to 34 |
| Supported rates |  | 1 | 3 to 10 |
| Reserved | 2 | - 6 |  |
| Country |  | 7 | 8 to 256 |
| Reserved | 8 | - 9 |  |
| Request | 10 |  | 2 - 256 |
| BSS Load | 11 |  | 7 |
| Reserved | 12 | - 15 | Reserved |
| Challenge Text | 16 |  | 3 to 255 |
| Reserved | 17 | - 43 |  |
| TCLAS Processing | 44 |  | 3 |
| Reserved | 45 | - 49 |  |
| Extended Supported Rates | 50 |  | 3 to 257 |
| Reserved | 51 - 126 |  |  |

132	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 28—Valid set of information elements

| Information Element | Element ID | Length (in octets) |
| --- | --- | --- |
| Extended Capabilities | 127 | 2 to 257 |
| Reserved | 128 - 220 |  |
| Vendor Specific | 221 | 3 to 257 |
| Reserved | 222 - 255 |  |

- Data Frames Format

- Control Frames Format

5.3.4.3.1 Advanced Modulation Control Frame

A control frame indicating the advanced modulation capabilities of a communication node. The frame is structured as described in Figure 95.

Figure 95—Format of the advanced modulation control frame.

5.8.4.3.1.1 Adaptive Loading

A single bit indicating whether the communication node transmitting the advanced modulation control frame supports adaptive bit and energy loading:

1  Adaptive bit and energy loading is supported.

0  Adaptive bit and energy loading is not supported.

5.8.4.3.1.2 eU

Four bits indicating if the node supports eU-OFDM. The bit value at a given position out of the four posi-tions indicates whether eU-OFDM implementation with the same number of streams as the bit position is supported. Positions are counted from left to right. For example:

1000  eU-OFDM with one stream only is supported.

1100  eU-OFDM with one and two streams only is supported

1010  eU-OFDM with one and three streams only is supported

1111  eU-OFDM with all possible streams is supported

0000  eU-OFDM is not supported

| Copyright © 20xx IEEE. All rights reserved. | 133 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 5.8.4.3.1.3 RPO |
| --- | --- |
| 2 |  |

- A single bit indicating whether the communication node transmitting the advanced modulation control rame

- supports RPO-OFDM:

| 5 | '1'  RPO-OFDM is supported. |  |
| --- | --- | --- |
| 6 |  |  |
| 7 | '0'  RPO-OFDM is not supported. |  |
| 8 |  |  |
| 9 | 5.8.4.3.1.4 Analog Dimming |  |
| 10 |  |  |
| 11 |  |  |

- A single bit indicating whether the communication node transmitting the advanced modulation control

- frame supports analog dimming (adjusting the analog signal bias level):

| 14 | '1'  Analog dimming is supported. |  |
| --- | --- | --- |
| 15 |  |  |
| 16 | '0'  Analog dimming is not supported. |  |
| 17 |  |  |
| 18 | 5.8.4.3.1.5 Relaying |  |
| 19 |  |  |
| 20 |  |  |

- Four bits indicating types of relaying operations the communication node transmitting the advanced modula-

- tion control frame supports. The first bit (B7) indicates whether relaying in FD is supported:

| 23 | '1'  Relaying in FD is supported. |  |
| --- | --- | --- |
| 24 |  |  |
| 25 | '0'  Relaying in FD is not supported. |  |
| 26 |  |  |
| 27 | The second bit (B8) indicates whether relaying in HD is supported: |  |
| 28 |  |  |
| 29 | '1'  Relaying in HD is supported. |  |
| 30 |  |  |
| 31 | '0'  Relaying in HD is not supported. |  |
| 32 |  |  |
| 33 | The third bit (B9) indicates whether AF relaying is supported: |  |
| 34 |  |  |
| 35 | '1'  AF relaying is supported. |  |
| 36 |  |  |
| 37 | '0'  AF relaying is not supported. |  |
| 38 |  |  |
| 39 | The fourth bit (B10) indicates whether DF relaying is supported: |  |
| 40 |  |  |
| 41 | '1'  DF relaying is supported. |  |
| 42 |  |  |
| 43 | '0'  DF relaying is not supported. |  |
| 44 |  |  |
| 45 | 5.8.4.3.1.6 MIMO |  |
| 46 |  |  |
| 47 |  |  |

- A single bit indicating whether the communication node transmitting the advanced modulation control

- frame supports MIMO communication:

| 50 | '1'  MIMO is supported. |  |  |
| --- | --- | --- | --- |
| 51 |  |  |  |
| 52 | '0'  MIMO is not supported. |  |  |
| 53 |  |  |  |
| 54 |  |  |  |
|  | 134 | Copyright © 20xx IEEE. All rights reserved. |  |

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.8.4.3.1.7 # of MIMO Channels

Four bits indicating the maximum number of MIMO communication channels which the communication node transmitting the advanced modulation control frame supports. A value of '0000' corresponds to 1 chan-nel, and a value of '1111' corresponds to 16 channels.

5.8.4.3.1.7 SC-FDMA

An IDFT_SIZE of bits, where IDFT_ SIZE is the size of the IDFT operation used for OFDM time -domain signal generation. The bit at position k indicates whether a DFT transform of size k is supported for SC-FDMA pre-coding. For example, if IDFT_SIZE = 16, then:

'10000000000000000' - A DFT of size 1 is supported.

'00100000000000000' - A DFT of size 3 is supported.

'10100100000000000' - A DFT of size 1, 3 and 6 is supported.

'00000000000000000' - No DFT size is supported, so SC-FDMA is not supported.

5.8.4.3.2 CSI Control Frame

The CSI control frame contains the standard MAC header information defined in subclause 5.8.2. The frame body described in this clause is the only specific field. It is structured as described in Fig. 5.8.4.3.2.1.

Figure 96—Format of the CSI control frame.

5.8.4.3.2.1 MIMO Mode

A one bit value indicating whether the current CSI frame contains information relevant for the SISO or MIMO mode of operation:

1  MIMO mode of operation.

0  SISO mode of operation.

5.8.4.3.2.2 Adaptive Loading

A one-bit field which indicates whether the CSI in the current frame refers to the adaptive bit loading mode.

1  The CSI is relevant for the adaptive bit loading scheme

0  The CSI is relevant for the fixed bit loading scheme

5.8.4.3.2.3 Frame Info

A one-bit field indicating the type of information carried by the current frame:

| Copyright © 20xx IEEE. All rights reserved. | 135 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | '0'  The frame carries CQIs |  |
| --- | --- | --- |
| 2 | '1'  The frame carrier bit loading (RATE) information |  |
| 3 |  |  |
| 4 | 5.8.4.3.2.4 Number of MIMO Channels TX |  |
| 5 |  |  |
| 6 |  |  |

- The number of MIMO channels of the transmitting node in the MIMO configuration. Values range between

- 0 and 15 corresponding to 1 and 16 channels, respectively. If the current CSI frame carries CQIs, then this

- field contains the number of the MIMO channels for the node receiving the frame. If the frame carriers bit

- loading (RATE) information, this field contains the number of the MIMO channels for the node transmitting

- the frame. This information is expected to have been exchanged via the advanced modulation control frame

- prior to any node issuing a CSI control frame.

13

14	5.8.4.3.2.5 Number of MIMO Channels RX

15

- The number of MIMO channels of the receiving node in the MIMO configuration. Values range between 0

- and 15 corresponding to 1 and 16 channels, respectively. If the current CSI frame carries CQIs, then this

- field contains the number of the MIMO channels for the node transmitting the frame. If the frame carriers bit

- loading (RATE) information, the this field contains the number of the MIMO channels for the node receiv-

- ing the frame. This information is expected to have been exchanged via the advanced modulation control

- frame prior to any node issuing a CSI control frame.

22

23	5.8.4.3.2.6 Channel Number

24

- The MIMO channel to which the subsequent RATE/CSI information refers. In a SISO mode, this field is set

- to '0000'. If the Adaptive Loading field is set to '1', then this field occurs only once and is followed by the

- CQI / RATE information for every subcarrier of the respective MIMO channel. If the Adaptive Loading

- field is set to '0', then this field occurs as many times as there are MIMO channels for which relevant infor-

- mation needs to be exchanged followed by the relevant information as CQI / RATE values. If the Frame Info

- is set to '0', then this field occurs Number of MIMO Channels TX .  Number of MIMO Channels RX times

- where the 6-bit CQI following each occurrence corresponds to the average SNR achieved for the respective

- combination between the corresponding MIMO transmitter and MIMO receiver. The order in which the

- information is arranged is | TX1  RX | TX1  RX2 |…| TX1  RXN_MIMO| TX2  RX1 | TX2 

-  RX2 | … |TX2  RXN_MIMO| … | TXM_MIMO  RX1 | TXM_MIMO  RX2 | … |TXM_MIMO 

- RXN_MIMO |. If the Frame Info is set to '1', then this field occurs #_of_MIMO_Channels_TX times where

- the information is arranged in the order | TX1 | TX2 | TX3 | … | TXM_MIMO|. In each case, the occurrence of

- the Channel # is followed by the respective 4-bit RATE value for that TX channel.

38

39	5.8.4.3.2.6 RATE / CSI

40

- The RATE / CSI values for the MIMO channel specified in Channel #. If the Adaptive Loading field is set to

- '1', then this field occurs once for every subcarrier modulated with information on the respective MIMO

- channel. If the Adaptive Loading field is set to '0', then this field occurs as many times as there are MIMO

- channels for which relevant information needs to be exchanged and is preceded by the relevant MIMO

- Channel # field. If the Frame Info is set to '0', then this field occurs #_of_MIMO_Channels_TX ? x_of_MI-

- MO_Channels_RX times where a 6-bit CQI value is specified corresponding to the average SNR achieved

- for the respective combination between the corresponding MIMO transmitter and MIMO receiver. The order

- in which the information is arranged is | TX1  RX | TX1  RX2 |…| TX1  RXN_MIMO| TX2 

- RX1 | TX2  RX2 | … |TX2  RXN_MIMO | … | TXM_MIMO  RX1 | TXM_MIMO  RX2 | …

- |TXM_MIMO  RXN_MIMO |. If the Frame Info is set to '1', then this field occurs # of MIMO Channels TX

- times where the 4-bit RATE information is arranged in the order | TX1 | TX2 | TX3 | … | TXM_MIMO |. In

- each case, the occurrence of the RATE field is preceded by the respective Channel # value for that TX chan-

- nel.

54

136	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.8.4.3.2 Dimming Control Frame

The frame contains control information which needs to be exchanged for the purposes of realizing dimming techniques such as RPO-OFDM and/or analog dimming.

5.4 MAC command frames

The command frames defined by the MAC sublayer are listed in Table 29. A coordinator shall be capable of transmitting and receiving all command frame types, with the exception of the GTS request command, while the requirements for a device are indicated by an “X” in Table 29. A P2P device functioning as a coordinator shall be capable of transmitting and receiving all supported command frames in a device. MAC commands shall only be transmitted in the CAP for beacon-enabled OWPANs or at any time for nonbeacon-enabled OWPANs.

How the MLME shall construct the individual commands for transmission is detailed in 5.4.1 through 5.4.23. MAC command reception shall abide by the procedure described in 5.1.9.2.

Table 29—Command frames

|  |  | Device |  |  | P2P |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Command frame | Command name |  |  | coordinator |  |  | Subclause |  |
| identifier |  |  |  |  |  |  |  |  |
|  |  | Tx | Rx | Tx |  | Rx |  |  |
| 0x01 | Association request | X |  | X |  | X | 5.4.1 |  |
| 0x02 | Association response |  | X | X |  | X | 5.4.2 |  |
| 0x03 | Disassociation notification | X | X | X |  | X | 5.4.3 |  |
| 0x04 | Data request | X |  | X |  | X | 5.4.4 |  |
| 0x05 | OWPAN ID conflict notification | X |  | X |  | X | 5.4.7 |  |
| 0x06 | Beacon request |  |  |  |  |  | 5.4.10 |  |
| 0x07 | Coordinator realignment |  | X | X |  | X | 5.4.12 |  |
| 0x08 | GTS request |  |  |  |  |  | 5.4.13 |  |
| 0x09 | Blinking notification |  |  |  |  |  | 5.4.14 |  |
| 0x0a | Dimming notification |  | X | X |  | X | 5.4.15 |  |
| 0x0b | Fast link recovery |  |  |  |  |  | 5.4.16 |  |
| 0x0c | Mobility notification |  |  |  |  |  | 5.4.17 |  |
| 0x0d | GTS Response |  |  |  |  |  | 5.4.18 |  |
| 0x0e | Clock rate change notification |  | X | X |  | X | 5.4.19 |  |
| 0x0f | Multiple channel assignment |  |  |  |  |  | 5.4.20 |  |
| 0x10 | Band hopping |  |  |  |  |  | 5.1.12.2 |  |
| 0x11 | Color stabilization timer notification | X | X |  |  |  | 5.4.21 |  |

| Copyright © 20xx IEEE. All rights reserved. | 137 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 29—Command frames (continued)

|  |  | Device |  |  | P2P |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Command frame | Command name |  |  | coordinator |  |  | Subclause |  |
| identifier |  |  |  |  |  |  |  |  |
|  |  | Tx | Rx | Tx |  | Rx |  |  |
| 0x12 | Color stabilization information | X | X |  |  |  | 5.4.22 |  |
| 0x13 | CVD disable |  |  |  |  |  | 5.4.23 |  |
| 0x14 | Information element | X | X | X |  | X | 5.4.26 |  |
| 0x15–0xff | Reserved |  |  |  |  |  | — |  |

5.4.1 Association request command

The association request command allows a device to request association with a OWPAN through the coordinator. This command shall only be sent by an unassociated device that wishes to associate with a OWPAN. A device shall only associate with a OWPAN through the coordinator as determined through the scan procedure.

All devices shall be capable of transmitting this command, although a device is not required to be capable of receiving it.

The association request command shall be formatted as illustrated in Figure 97.

| Octets: (see 5.2.2.4) | 1 | 1 (see 5.4.26.1) |
| --- | --- | --- |
| MHR fields | Command | Capability |
|  | Frame Identifier | Information |
|  | (as defined in |  |
|  | Table 29) |  |

Figure 97—Association request command format

5.4.1.1 MHR fields

The Source Addressing Mode subfield of the frame control field shall be set to three (64-bit extended addressing). The Destination Addressing Mode subfield shall be set to the same mode as indicated in the beacon frame to which the association request command refers.

The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and the Acknowledgment Request subfield shall be set to one.

The Destination OWPAN Identifier field shall contain the identifier of the OWPAN to which to associate. The Destination Address field shall contain the address from the beacon frame that was transmitted by the coordinator to which the association request command is being sent. The Source OWPAN Identifier field shall contain the broadcast OWPAN identifier (i.e., 0xffff). The Source Address field shall contain the value of aExtendedAddress.

138	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.4.2 Association response command

The association response command allows the coordinator or a coordinator to communicate the results of an association attempt back to the device requesting association.

This command shall only be sent by the coordinator or a coordinator to a device that is currently trying to associate.

All devices shall be capable of receiving this command, although a device is not required to be capable of transmitting it.

The association response command shall be formatted as illustrated in Figure 98.

| Octets: (see 5.2.2.4) | 1 | 2 | 1 | 1 |
| --- | --- | --- | --- | --- |
| MHR fields | Command Frame | Short | Association | Capability- |
|  | Identifier | Address | Status | negotiation |
|  | (as defined in Table 29) |  |  | response |

Figure 98—Association response command format

The capability-negotiation response is the same as that of the color-stabilization scheme in Table 42.

5.4.2.1 MHR fields

The Destination Addressing Mode and Source Addressing Mode subfields of the frame control field shall each be set to three (i.e., 64-bit extended addressing).

The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and the Acknowledgment Request subfield shall be set to one.

The Destination OWPAN Identifier field shall contain the value of macOWPANId, while the Source OWPAN Identifier field shall be omitted. The Destination Address field shall contain the extended address of the device requesting association. The Source Address field shall contain the value of aExtendedAddress.

5.4.2.2 Short Address field

If the coordinator was not able to associate this device to its OWPAN, the Short Address field shall be set to 0xffff, and the Association Status field shall contain the reason for the failure. If the coordinator was able to associate the device to its OWPAN, this field shall contain the short address that the device may use in its communications on the OWPAN until it is disassociated.

A Short Address field value equal to 0xfffe shall indicate that the device has been successfully associated with a OWPAN, but has not been allocated a short address. In this case, the device shall communicate on the OWPAN using only its 64-bit extended address.

5.4.2.3 Association Status field

The Association Status field shall contain one of the nonreserved values listed in Table 30.

| Copyright © 20xx IEEE. All rights reserved. | 139 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 30—Valid values of the Association Status field

| Association status | Description |
| --- | --- |
| 0x00 | Association successful. |
| 0x01 | OWPAN at capacity. |
| 0x02 | OWPAN access denied. |
| 0x03–0x7f | Reserved. |
| 0x80–0xff | Reserved for MAC primitive enumeration values. |

5.4.2.4 Capability negotiation response field

The capability negotiation response field describes if and where (device and/or coordinator) color stabilization is performed. All allowed settings are shown in Table 31.

Table 31—Capability negotiation response field

| Bits: 0–1 | Bits: 2–7 |
| --- | --- |
| 00: No color stabilization |  |
| 01: Color-stabilization information to be sent from |  |
| device to coordinator upon reception of CVD frames |  |
| 10: Color-stabilization information to be sent from | Reserved |
| coordinator to device upon reception of CVD frames |  |
| 11: Color-stabilization information to be sent from |  |
| device to coordinator and from coordinator to device |  |
| when either receives CVD frames |  |

5.4.3 Re-association request command

The re-association request command allows a device to request re-association with a target OWPAN through the coordinator. This command shall only be sent by a device that is already associate with an original coordinator and that wishes to associate with another OWPAN. A device shall only re-associate with an OWPAN through the coordinator as determined through neighboring OWPAN measurements. All devices shall be capable of transmitting this command, although a device is not required to be capable of receiving it.

The re-association request command shall be formatted as illustrated in Figure 98.

| Octet: (see 5.2.2.4) | 1 | 1 (see 5.4.26.1) | Variable |
| --- | --- | --- | --- |
| MHR field | Command Frame Identifier (as defined in Table 29) | Capability information | Originating OWPAN information |

5.4.2.1 MHR field

The Source Addressing Mode subfield of the frame control field shall be set to three (64-bit extended addressing). The Destination Addressing Mode subfield shall be set to the same mode as indicated in the beacon frame to which the association request command refers.

The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and the Acknowledgment Request subfield shall be set to one. The Destination OWPAN Identifier field shall contain the identifier of the OWPAN to which to associate.

The Destination Address field shall contain the address from the beacon frame that was transmitted by the coordinator to which the association request command is being sent. The Source OWPAN Identifier field shall contain the value of macOWPANId. The Source Address field shall contain the value of aExtendedAddress.

5.4.2.2 Originating OWPAN information

The Originating OWPAN information contains the information of the OWPAN with which the device is currently associate, see Table 64.

5.4.3 Disassociation notification command

The OWC coordinator or an associated device may send the disassociate notification command.

All devices shall implement this command.

The disassociation notification command shall be formatted as illustrated in Figure 99.

| Octets: (see 5.2.2.4) | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier | Disassociation reason |
|  | (as defined in Table 29) |  |

Figure 99—Disassociation notification command format

140	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.4.3.1 MHR fields

The Destination Addressing Mode subfield of the frame control field shall be set according to the addressing mode specified by the corresponding primitive. The Source Addressing Mode subfield shall be set to three (i.e., 64-bit extended addressing).

The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and the Acknowledgment Request subfield shall be set to one.

The Destination OWPAN Identifier field shall contain the value of macOWPANId, while the Source OWPAN Identifier field shall be omitted. If the coordinator wants an associated device to leave the OWPAN, then the Destination Address field shall contain the address of the device being removed from the OWPAN. If an associated device wants to leave the OWPAN, then the Destination Address field shall contain the value of either macCoordShortAddress, if the Destination Addressing Mode subfield is equal to two, or macCoordExtendedAddress, if the Destination Addressing Mode subfield is equal to three. The Source Address field shall contain the value of aExtendedAddress.

5.4.3.2 Disassociation Reason field

The Disassociation Reason field shall contain one of the nonreserved values listed in Table 32.

Table 32—Valid disassociation reason codes

|  | Disassociate reason | Description |  |
| --- | --- | --- | --- |
|  | 0x00 | Reserved. |  |
|  | 0x01 | The coordinator wishes the device to leave the OWPAN. |  |
|  | 0x02 | The device wishes to leave the OWPAN. |  |
|  | 0x03 | Device cannot support communications for the requested |  |
|  |  | dimming value. |  |
|  | 0x04f–0x7f | Reserved. |  |
|  | 0x80–0xff | Reserved for MAC primitive enumeration values. |  |

5.4.4 Data request command

The data request command is sent by a device to request data from the coordinator.

There are three cases for which this command is sent. On a beacon -enabled OWPAN, this command shall be sent by a device when macAutoRequest is equal to TRUE and a beacon frame indicating that data are pending for that device is received from its coordinator. The coordinator indicates pending data in its beacon frame by adding the address of the recipient of the data to the Address List field. This command shall also be sent when instructed to do so by the next higher layer on reception of the MLME-POLL.request primitive. In addition, a device may send this command to the coordinator macResponseWaitTime optical clocks after the acknowledgment to an association request command.

All devices shall be capable of transmitting this command, although a device is not required to be capable of receiving it.

The data request command shall be formatted as illustrated in Figure 100.

| Copyright © 20xx IEEE. All rights reserved. | 141 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  |  |
| --- | --- | --- | --- |
|  | Octets: (see 5.2.2.4) | 1 |  |
| 2 |  |  |  |
| 3 | MHR fields | Command Frame Identifier |  |
| 4 |  |  |  |
|  |  | (as defined in Table 29) |  |
| 5 |  |  |  |
| 6 | Figure 100—Data request command format |  |  |
| 7 |  |  |  |
| 8 |  |  |  |

- If the data request command is being sent in response to the receipt of a beacon frame indicating that data are

- pending for that device, the Destination Addressing Mode subfield of the frame control field may be set to

- zero (i.e., destination addressing information not present) if the beacon frame indicated in its Superframe

- Specification field (see 5.2.2.1.2) that it originated from the coordinator (see 5.2.1.1.6) or set otherwise

- according to the coordinator to which the data request command is directed. If the destination addressing

- information is to be included, the Destination Addressing Mode subfield shall be set according to the value

- of macCoordShortAddress. If macCoordShortAddress is equal to 0xfffe, extended addressing shall be used:

- the Destination Addressing Mode subfield shall be set to three, and the Destination Address field shall

- contain the value of macCoordExtendedAddress. Otherwise, short addressing shall be used: the Destination

- Addressing Mode subfield shall be set to two, and the Destination Address field shall contain the value of

- macCoordShortAddress.

20

- If the data request command is being sent in response to the receipt of a beacon frame indicating that data are

- pending for that device, the Source Addressing Mode subfield shall be set according to the addressing mode

- used for the pending address. If the Source Addressing Mode subfield is set to two, short addressing shall be

- used: the Source Address field shall contain the value of macShortAddress. Otherwise, extended addressing

- shall be used: the Source Addressing Mode subfield shall be set to three, and the Source Address field shall

- contain the value of aExtendedAddress.

27

- If the data request command is triggered by the reception of an MLME-POLL.request primitive from the

- next higher layer, then the destination addressing information shall be the same as that contained in the

- primitive. The Source Addressing Mode subfield shall be set according to the value of macShortAddress. If

- macShortAddress is less than 0xfffe, short addressing shall be used. Extended addressing shall be used

- otherwise.

33

- If the data request command is being sent following the acknowledgment to an association request command

- frame, the Destination Addressing Mode subfield of the frame control field shall be set according to the

- coordinator to which the data request command is directed. If macCoordShortAddress is equal to 0xfffe,

- extended addressing shall be used. Short addressing shall be used otherwise. The Source Addressing Mode

- subfield shall be set to use extended addressing.

39

- If the Destination Addressing Mode subfield is set to zero (i.e., destination addressing information not

- present), the source OWPAN identifier shall contain the value of macOWPANId. Otherwise, the Destination

- OWPAN Identifier field shall contain the value of macOWPANId, while the Source OWPAN Identifier field

- shall be omitted.

44

- The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and

- the Acknowledgment Request subfield shall be set to one.

47

48	5.4.5 Scan-over-backhaul request command

49

- The scan-over-backhaul request command shall be send by a prospective coordinator to other coordinators

- to request the OWPAN descriptor from the recipient through the backhaul link  when inter-coordinator communication over backhaul is feasible and scan over backhaul is performed (see 5.1.3.1.3)

52

53 The scan-over-backhaul request command shall be formatted as illustrated in Figure 101. 54

142	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Octets: | 1 |
| --- | --- |
| MHR fields | Command Frame identifier |

Figure 101—Scan-over-backhaul request command

5.4.6 Scan-over-backhaul confirmation command

The scan-over-backhaul confirmation command shall be sent by a coordinator as a response to a previous received scan-over-backhaul request command through the backhaul.

The scan-over-backhaul confirmation command shall be formatted as illustrated in Figure 102.

|  | Octets: | 1 | Variable |
| --- | --- | --- | --- |
|  | MHR fields | Command Frame identifier | OWPANDescriptor |

Figure 102—Scan-over-backhaul confirmation command

5.4.6.1 OWPAN Descriptor

The OWPANDescriptor contains the information of the OWPAN whose coordinator generates the scan-over-backhaul confirmation command, see Table 64. The LinkQuality field is set to 0xff, the TimeStamp field is set to 0xffffff.

5.4.7 OWPAN ID conflict notification command

The OWPAN ID conflict notification command is sent by a device to the coordinator to report a OWPAN identifier conflict when  the conflict is detected by the device.

All devices shall be capable of transmitting this command, although a device is not required to be capable of receiving it.

The OWPAN ID conflict notification command shall be formatted as illustrated in Figure 103.

| Octets: (see 5.2.2.4) | 1 |
| --- | --- |
| MHR fields | Command Frame Identifier |
|  | (as defined in Table 29) |

Figure 103—OWPAN ID conflict notification command format

The Destination Addressing Mode and Source Addressing Mode subfields of the frame control field shall both be set to three (i.e., 64-bit extended addressing).

The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and the Acknowledgment Request subfield shall be set to one.

The Destination OWPAN Identifier field shall contain the value of macOWPANId, while the Source OWPAN Identifier field shall be omitted. The Destination Address field shall contain the value of macCoordExtendedAddress. The Source Address field shall contain the value of aExtendedAddress.

| Copyright © 20xx IEEE. All rights reserved. | 143 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 5.4.8 Neighboring OWPAN report request command |
| --- | --- |
| 2 |  |

- The neighboring OWPAN report request command is sent by the coordinator to devices to request a report

- of neighboring OWPAN information detected by the devices.

5

6

7

8

9

10

11

12

13

14

15

16

17

The neighboring OWPAN report request command shall be formatted as illustrated in Figure 104.

| Octets | 1 |
| --- | --- |
| MHR fields | Command Frame identifier |

Figure 104—Neighboring OWPAN report request command

5.4.9 Neighboring OWPAN report indication command

- The neighboring OWPAN report indication command is sent by the device to the coordinator to report

- neighboring OWPAN information. The neighboring OWPAN information is recorded in a local neighboring

- OWPAN descriptor list, each record in the list corresponding to the information of one neighboring OWPAN

- that can be detected by the device.

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

The neighboring OWPAN report indication command shall be formatted as illustrated in Figure 105.

| Octets | 1 | Variable |  |
| --- | --- | --- | --- |
| MHR fields | Command Frame identifier | Local neighboring |  |
|  |  | OWPAN descriptor list |  |

Figure 105—Neighboring OWPAN report indication command

The local neighboring OWPANs descriptor list is shown in Table 33.

5.4.10 Beacon request command

- The beacon request command is used by a device to locate all coordinators within its operating space during

- an active scan.

| 40 |  |
| --- | --- |
| 41 | This command is optional for a device. |
| 42 |  |
| 43 | The beacon request command shall be formatted as illustrated in Figure 106. |
| 44 |  |
| 45 | 5.4.11 Additional beacon request command |
| 46 |  |

- The additional beacon request command is sent by a device to request a coordinator to send additional

- beacon frames.

| 49 |  |  |
| --- | --- | --- |
| 50 | The command is optional for a device. |  |
| 51 |  |  |
| 52 | The additional beacon request shall be formatted as illustrated in Figure 107. |  |
| 53 |  |  |
| 54 |  |  |
|  | 144 | Copyright © 20xx IEEE. All rights reserved. |

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 33—Local neighboring OWPANs description list

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |

|  |  |  | The number of records |  |
| --- | --- | --- | --- | --- |
| Number NeighboringOW- |  |  | of neighboring |  |
|  | Integer | 0x00 - 0xff | OWPANs that are |  |
| PANRecords |  |  |  |  |
|  |  |  | maintained by the |  |
|  |  |  | device. |  |
|  |  |  | The record of the first |  |
| OWPANDescriptor[0] |  | Refer to Table 34 | neighboring OWPAN |  |
|  |  |  | that can be detected by |  |
|  |  |  | the device. |  |
|  |  |  | The measured RSS of the |  |
| LinkQuality[0] | Integer | 0x00 - 0xff | first neighboring |  |
|  |  |  | OWPAN. |  |
|  |  |  | The sequence number |  |
|  |  |  | of the superframe |  |
|  |  |  | during which the |  |
| LastTimeDetected | Integer | 0x00 - 0xff | device received a frame |  |
|  |  |  | from the first neighbor- |  |
|  |  |  | ing OWPAN for the |  |
|  |  |  | last time. |  |
|  |  |  | The record of the N-th |  |
| OWPANDescriptor[N-1] |  | Refer to Table 34 | neighboring OWPAN |  |
|  |  |  | that can be detected by |  |
|  |  |  | the device. |  |
|  |  |  | The measured RSS of the |  |
| LinkQuality[N-1] | Integer | 0x00 - 0xff | Nth neighboring |  |
|  |  |  | OWPAN. |  |
|  |  |  | The sequence number |  |
|  |  |  | of the superframe |  |
|  |  |  | during which the |  |
| LastTimeDetected | Integer | 0x00 - 0xff | device received a frame |  |
|  |  |  | from the first neighbor- |  |
|  |  |  | ing OWPAN for the |  |
|  |  |  | last time. |  |

5.4.11.1 Reason field

The Reason field shall contain one of the non-reserved values listed in Table 35.

The Destination Addressing Mode subfield of the frame control field shall be set to two (i.e., 16-bit short addressing), and the Source Addressing Mode subfield shall be set to zero (i.e., source addressing information not present).

The frame pending subfield of the frame control field shall be set to zero and ignored upon reception. The Acknowledgment Request subfield and Security Enabled subfield shall also be set to zero.

The Destination OWPAN Identifier field shall contain the broadcast OWPAN identifier (i.e., 0xffff). The Destination Address field shall contain the broadcast short address (i.e., 0xffff).

| Copyright © 20xx IEEE. All rights reserved. | 145 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 34—The format of the OWPAN descriptor

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |

|  |  |  |  |  |  |  | The OWPAN ID of the |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | neighboring OWPAN |  |  |
| OWPANid |  | integer |  |  | 0x0000 - 0xffff |  | as specified in the |  |  |
|  |  |  |  |  |  |  | received beacon |  |  |
|  |  |  |  |  |  |  | frames. |  |  |
|  |  |  |  |  |  |  | The 16 bits short |  |  |
|  |  |  |  |  |  |  | address of the coordi- |  |  |
| CoordShortAddress |  | Integer |  |  |  |  | nator as specified in the |  |  |
|  |  |  |  |  |  |  | received beacon |  |  |
|  |  |  |  |  |  |  | frames. |  |  |
|  |  |  |  |  |  |  | The 64 bits extended |  |  |
|  |  |  |  |  |  |  | address of the coordi- |  |  |
| CoordExtendedAddress |  | Integer |  |  |  |  | nator as specified in the |  |  |
|  |  |  |  |  |  |  | received beacon |  |  |
|  |  |  |  |  |  |  | frames. |  |  |
|  |  |  |  |  |  |  | Indicates whether a |  |  |
|  |  |  |  |  |  |  | coordinator is cur- |  |  |
|  |  |  |  |  |  |  | rently allowing associ- |  |  |
| AssociationPermit |  | Boolean |  |  | TRUE or FALSE |  | ation. A value of |  |  |
|  |  |  |  |  |  |  | TRUE indicates that |  |  |
|  |  |  |  |  |  |  | association is permit- |  |  |
|  |  |  |  |  |  |  | ted. |  |  |
|  |  |  | Octets: 7 |  | 1 |  |  |  |  |
|  |  |  | MHR fields |  | Command Frame Identifier |  |  |  |  |
|  |  |  |  |  | (as defined in Table 29) |  |  |  |  |
|  |  | Figure 106—Beacon request command format |  |  |  |  |  |  |  |
|  | Octets (see 5.2.2.4) |  |  |  | 1 |  | 1 |  |  |
|  |  | MHR fields |  | Command Frame identi- |  |  | Reason |  |  |
|  |  |  |  |  | fier |  |  |  |  |
|  |  | Figure 107—Additional beacon request command |  |  |  |  |  |  |  |

5.4.12 Coordinator realignment command

The coordinator realignment command is sent by a coordinator when any of its OWPAN configuration attributes change (i.e., OWPAN identifier, short address) due to the receipt of an MLME-START.request primitive.

it is broadcast to the OWPAN.

146	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  | Table 35—Reason field |  |
| --- | --- | --- |
| Value | Function |  |
| 0x00 | A new device that may be located at the interferefering area of multiple |  |
|  | OWPANs fails to detect the beacon frame due to interference. |  |
| 0x01 | An associated device that is located at the interferefering area of multi- |  |
|  | ple OWPANs fails to detect the beacon frame due to interference. |  |
| 0x02 - 0xFF | reserved |  |

All devices shall be capable of receiving this command, although a device is not required to be capable of transmitting it.

The coordinator realignment command shall be formatted as illustrated in Figure 108.

| Octets: 17/18/23/24 | 1 | 2 | 2 |  | 2 | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| MHR | Command | OWPAN | Coordinator |  | Short | Effec- |
| fields | Frame Identifier | Identifier | Short |  | Address | tive Time |
|  | (as defined in |  | Address |  |  |  |
|  | Table 29) |  |  |  |  |  |

Figure 108—Coordinator realignment command format

5.4.12.1 MHR fields

The Destination Addressing Mode subfield of the frame control field shall be set to two (e.g., 16-bit short addressing) if it is to be broadcast to the OWPAN. The Source Addressing Mode subfield of the frame control field shall be set to three (e.g., 64-bit extended addressing).

The frame pending subfield of the frame control field shall be set to zero and ignored upon reception.

The Acknowledgment Request subfield of the frame control field shall be set to zero if the command is to be broadcast to the OWPAN.

The Destination OWPAN Identifier field shall contain the broadcast OWPAN identifier (e.g., 0xffff). The Destination Address field shall contain the broadcast short address (e.g., 0xffff). The Source OWPAN Identifier field shall contain the value of macOWPANId, and the Source Address field shall contain the value of aExtendedAddress.

5.4.12.2 OWPAN Identifier field

The OWPAN Identifier field shall contain the OWPAN identifier that the coordinator intends to use for all future communications.

5.4.12.3 Coordinator Short Address field

The Coordinator Short Address field shall contain the value of macShortAddress.

| Copyright © 20xx IEEE. All rights reserved. | 147 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |
| --- | --- |
| 2 |  |

- e

- .

5

6	5.4.12.5 Short Address field

7

- If the coordinator realignment command is broadcast to the OWPAN, the Short Address field shall be set to

- 0xffff and ignored on reception.

10

11	5.4.12.6 Effective Time field

12

- The effective time field shall contain the BSN of the corresponding superframe in which the new parameters

- (OWPAN IDs, short address, e.g.) shall take effect.

15

16	5.4.13 GTS request command

17

- The GTS request command is used by an associated device that is requesting the allocation of a new GTS or

- the deallocation of an existing GTS from the coordinator. Only devices that have a 16-bit short address less

- than 0xfffe shall send this command.

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

This command is optional.

The GTS request command shall be formatted as illustrated in Figure 109.

| Octets: 7 | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier | GTS Characteristics |
|  | (as defined in Table 29) |  |

Figure 109—GTS request command format

5.4.13.1 MHR fields

- The Destination Addressing Mode subfield of the frame control field shall be set to zero (e.g., destination

- addressing information not present), and the Source Addressing Mode subfield shall be set to two (e.g., 16-

- bit short addressing).

40

- The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and

- the Acknowledgment Request subfield shall be set to one.

43

- The Source OWPAN Identifier field shall contain the value of macOWPANId, and the Source Address field

- shall contain the value of macShortAddress.

| 46 | 5.4.13.2 GTS Characteristics field |  |  |
| --- | --- | --- | --- |
| 47 |  |  |  |
| 48 | The GTS Characteristics field shall be formatted as illustrated in Figure 110. |  |  |
| 49 |  |  |  |
| 50 | The GTS Length subfield shall contain the number of superframe slots being requested for the GTS. |  |  |
| 51 |  |  |  |
| 52 |  |  |  |
| 53 |  |  |  |
| 54 |  |  |  |
|  | 148 | Copyright © 20xx IEEE. All rights reserved. |  |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Bits: 0–3 | 4 | 5 | 6–7 |
| --- | --- | --- | --- |
| GTS Length | GTS Direction | Characteristics Type | Reserved |

Figure 110—GTS Characteristics field format

The GTS Direction subfield shall be set to one if the GTS is to be a receive-only GTS. Conversely, this subfield shall be set to zero if the GTS is to be a transmit-only GTS. GTS direction is defined relative to the direction of data frame transmissions by the device.

The Characteristics Type subfield shall be set to one if the characteristics refers to a GTS allocation or zero if the characteristics refers to a GTS deallocation.

5.4.14 Blinking notification command

The blinking notification command (see Figure 111) is sent by a coordinator when the device is no longer responding. A reason for this might be the misalignment between the device TX and the coordinator RX (limited FOV of receiver, low device TX power, mobility of the device, etc.). In such cases, the device can change the visibility indication from continuous visibility for point-and-shoot to blinking indication. The device can then change from point-and-shoot mode to blinking mode in order to indicate to the user that the uplink to the coordinator is disconnected. This indication can be applied to both P2MP and P2P modes of operation.

| Octets: 7 | 1 | 1 |  |
| --- | --- | --- | --- |
| MHR fields | Command Frame Identifier | Blinking frequency |  |
|  | (see Table 29) |  |  |

Figure 111—Blinking notification command

The blinking notification bit shall be set when the MAC PIB attribute, macUseBlinkingNotification and macBlinkingNotificationFrequency, as defined in Table 91 indicates the blinking notification usage.

To support the blinking notification, the frequency shall be chosen from the phyBlinkingNotificationFrequency PHY PIB attribute as shown in Table 144, using the MLME-SET.request and PLME-SET.request primitives.

This feature can help to align the link and is only intended for mobile devices.

5.4.14.1 Blinking frequency

The frequency subfield shall contain the frequency for blinking (see Figure 112).

| Bits: 0–3 | 4–7 |
| --- | --- |

Frequency	Reserved

Figure 112—Blinking frequency field format

| Copyright © 20xx IEEE. All rights reserved. | 149 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 5.4.15 Dimming notification command |
| --- | --- |
| 2 |  |

- The DME indicates the dimming level to the MAC using the MAC PIB attribute, macDim, as defined in

- Table 91. The dimming notification command is used to communicate the dimming level set by the macDim

- PIB attribute to the receiver. The dimming notification command (see Figure 113) shall be sent at the lowest

- data rate corresponding to the currently negotiated optical clock rate. The symbol shape information for

- VPPM is derived using the algorithm of Figure 158 after the dimming level is obtained.

| 8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 9 |  |  |  |  |  |
| 10 | Octets: 7 | 1 | 2 | 2 |  |
| 11 |  |  |  |  |  |
| 12 | MHR fields | Command frame identifier | Dimming level | Dimmer adaptation timer |  |
| 13 |  | (see Table 29) |  | (see 5.1.17.8) |  |
| 14 |  |  |  |  |  |
|  |  | Figure 113—Dimming notification command |  |  |  |
| 15 |  |  |  |  |  |
| 16 |  |  |  |  |  |
| 17 |  |  |  |  |  |

- The dimming level is two bytes long and contains a value between 0 and 1000, where 0 represents 0%

- visibility and 1000 represents 100% visibility. The dimming levels are defined with a resolution of 0.1%.

20	The dimmer adaptation timer provides a resolution of 0–16383 MAC clock cycles. The dimming

- notification command transmits the dimmer level from the TX to the RX along with the dimmer adaptation

- timer information. VPPM by default uses only 50% duty cycle, so if dimming is supported using VPPM as

- in 8.5.2.3, the VPPM pulse shape is obtained using the dimmer notification command in conjunction with

- the algorithm shown in Figure 158. Before dimming is supported using VPPM, the dimming notification

- command needs to be sent by the MAC to the receiver.

26

27	5.4.16 Fast link recovery command

28

- Fast link recovery command is used for the device or coordinator to send the fast link recovery (FLR) signal

- and the fast link recovery response (FLR RSP), to help the link recovery.

31

- Fast link recovery signal and response use the fast link recovery command format. The fast link recovery

- command shall be formatted as illustrated in Figure 114.

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

| Octets: | 1 | 1 |  |
| --- | --- | --- | --- |
| (as defined in 5.2.2.4) |  |  |  |
| MHR fields | Command Frame Identifier | FLR field |  |
|  | (as defined in Table 29) |  |  |
|  |  | FLR field explanation |  |
|  |  | Bit 0: ‘0’ indicating it is FLR signal, ‘1’ indicat- |  |
|  |  | ing it is FLR RSP |  |
|  |  | Bits 1–3: index of FLR signal direction, if bit 0 |  |
|  |  | is ‘0’. received FLR signal direction index if bit 0 |  |
|  |  | is ‘1’. |  |
|  |  | Bits 4–7: reserved |  |

Figure 114—Fast link recovery command

150	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| The FLR signal and the FLR response (RSP) are differentiated by the first bit (bit 0) of the FLR field in the |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fast link recovery command frame. The device can indicate the index of FLR signal direction by using bits 1 |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |
| to 3 of the FLR field in the command frame. If the device receives the FLR signal and needs to send FLR |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |
| RSP, it repeats the received FLR signal direction index by using bits 1 to 3 of the FLR field in the command |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
| frame. If the device is uni-direction, it uses ‘000’ as the index of the direction. |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |
| The usage of the FLR is presented in 5.1.11. |  |  |  |  |  |  |  |  |  |  |  |  | 6 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |
| 5.4.17 Mobility notification command |  |  |  |  |  |  |  |  |  |  |  |  | 8 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| The mobility notification command is shown in Figure 115. The concept of OWC cell mobility is defined in |  |  |  |  |  |  |  |  |  |  |  |  | 10 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 11 |  |
| 5.1.13. |  |  |  |  |  |  |  |  |  |  |  |  | 12 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 13 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 14 |  |
|  |  |  | Octets: 7 |  |  | 1 |  | variable |  |  |  |  | 15 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 16 |  |
|  |  |  | MHR fields | Command Frame Identifier |  |  |  | cellSearchQuality |  |  |  |  | 17 |  |
|  |  |  |  |  | (Table 29) |  |  | (see 5.1.13.3) |  |  |  |  | 18 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 19 |  |
|  |  |  | Figure 115—Mobility notification command |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 21 |  |
| The results from the cell search are provided in the mobility notification command as shown in Figure 115. |  |  |  |  |  |  |  |  |  |  |  |  | 22 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 23 |  |
| The WQI values (in octets) obtained for the current channel during the cell search procedure defined in |  |  |  |  |  |  |  |  |  |  |  |  | 24 |  |
| 5.1.13.3 shall be included in the command frame. The number of octets sent shall be equal to |  |  |  |  |  |  |  |  |  |  |  |  | 25 |  |
| cellSearchLength, as defined in 5.1.13.3. |  |  |  |  |  |  |  |  |  |  |  |  | 26 |  |
| 5.4.18 GTS response command |  |  |  |  |  |  |  |  |  |  |  |  | 27 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |
| The optional GTS.response primitive is generated in response to a GTS.request primitive. When used, the |  |  |  |  |  |  |  |  |  |  |  |  | 29 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |  |
| GTS response command shall be formatted as illustrated in Figure 116. |  |  |  |  |  |  |  |  |  |  |  |  | 31 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 32 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 33 |  |
|  | Octets: 7 |  |  | 1 |  |  | 1 |  |  | 1 |  |  | 34 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 35 |  |
|  | MHR fields |  | Command Frame Identifier |  |  |  | GTS characteristics |  |  | GTS Starting Slot |  |  | 36 |  |
|  |  |  | as defined in Table 29) |  |  |  |  |  |  | (see 5.2.2.1.7) |  |  | 37 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 38 |  |
|  |  |  | Figure 116—GTS response command format |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 39 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  |
| 5.4.18.1 MHR fields |  |  |  |  |  |  |  |  |  |  |  |  | 41 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 42 |  |
| The Destination Addressing Mode subfield of the frame control field shall be set to zero (e.g., destination |  |  |  |  |  |  |  |  |  |  |  |  | 43 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 44 |  |
| addressing information not present), and the Source Addressing Mode subfield shall be set to two (e.g., 16- |  |  |  |  |  |  |  |  |  |  |  |  | 45 |  |
| bit short addressing). |  |  |  |  |  |  |  |  |  |  |  |  | 46 |  |
| The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and |  |  |  |  |  |  |  |  |  |  |  |  | 47 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 48 |  |
| the Acknowledgment Request subfield shall be set to one. |  |  |  |  |  |  |  |  |  |  |  |  | 49 |  |
| The Source OWPAN Identifier field shall contain the value of macOWPANId, and the Source Address field |  |  |  |  |  |  |  |  |  |  |  |  | 50 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 51 |  |
| shall contain the value of macShortAddress. |  |  |  |  |  |  |  |  |  |  |  |  | 52 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 53 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 54 |  |
|  |  |  | Copyright © 20xx IEEE. All rights reserved. |  |  |  |  |  |  | 151 |  |  |  |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |  |  |  |  |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

5.4.18.2 GTS Characteristics field

The GTS Characteristics field shall be formatted as illustrated in Figure 117.

| Bits: 0–3 | 4 | 5 | 6–7 |
| --- | --- | --- | --- |
| GTS Length | GTS Direction | Characteristics Type | Reserved |

Figure 117—GTS Characteristics field format

The GTS Length subfield shall contain the number of superframe slots being requested for the GTS.

- The GTS Direction subfield shall be set to one if the GTS is to be a receive-only GTS. Conversely, this

- subfield shall be set to zero if the GTS is to be a transmit-only GTS. GTS direction is defined relative to the

- direction of data frame transmissions by the device.

18

- The Characteristics Type subfield shall be set to one if the characteristics refers to a GTS allocation or zero

- if the characteristics refers to a GTS deallocation.

| 21 |  |
| --- | --- |
| 22 | 5.4.19 Clock rate change notification command |
| 23 |  |
| 24 | The command format for the clock rate change notification is as shown in Figure 118. This clock rate |

- change notification is sent at the current clock rate negotiated between the devices. All future transmissions

- from the current device to the receiving device will occur at this new clock rate.

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

| Octets: 7 | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier (as | New clock rate for future |
|  | defined in Table 29) | TX |

Figure 118—Clock rate change notification format

The modulation and coding scheme (MCS) ID from Table 118 shall be used to indicate the optical clock rate. Any MCS ID corresponding to the chosen future clock rate can be used. The 6 LSBs shall be set to the MCS ID corresponding to the future clock rate. The other bits are set to 0 and reserved for future use.

152	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.4.20 Multiple channel assignment command

| Octets: 7 | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier (as | Multiple Channels |
|  | defined in Table 29) |  |

Figure 119—Multiple channel assignment command format

Multiple channels should be used in the OWC system when time slot resources are not enough to cover all the currents users. These channels should be assigned based on the band-plan in Table 111. Refer to Table 6 for the contents of the Multiple Channels field.

5.4.21 Color stabilization timer notification command

The color stabilization timer notification command shall be formatted as illustrated in Figure 120. This command is used to inform a device or coordinator about the minimum time between two color-stabilization updates (upon reception of CVD frames).

| Octets: 7 | 1 | 2 | 4 |  |
| --- | --- | --- | --- | --- |
| MHR fields | Command frame identifier | Short address | Color stabilization timer |  |
|  | (see Table 29) |  |  |  |

Figure 120—Color stabilization timer notification command format

The color stabilization timer field has the same format as the macColorStabilizationTimer (see Table 91).

5.4.22 Color stabilization information command

The color stabilization information command shall be formatted as illustrated in Figure 121. This command is used for relaying the color-stabilization updates (upon reception of CVD frames) back to the pertinent CSK transmitter (see Figure 168).

| Octets: 7 | 1 | 2 | Color stabilization information |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 | 2 | 2 |  |
| MHR fields | Command frame identifier | Short address | band i | band j | band k |  |
|  | (see Table 29) |  |  |  |  |  |

Figure 121—Color stabilization information command format

The color stabilization information per band is 2 octets long and is used by the color-stabilization module (see Figure 169). It consists of the received signal levels in each of the three CSK bands. Two octets are used for each of the bands. A linear scale is used for each band, where the highest value corresponds to the maximum receive signal and the lowest value to the minimum receive signal. These fields are sent LSB first.

| Copyright © 20xx IEEE. All rights reserved. | 153 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

5.4.23 CVD disable command

| Octets: 7 | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command frame identifier (see Table 29) | CVD disable |

Figure 122—CVD disable command format

- The CVD frame can be transmitted depending on bi-directional, multicasting, and broadcasting capabilities.

- A device shall not transmit a CVD frame after the device has received a frame from an associated device that

- has the “CVD usage option” bit set to ‘0’ as defined in Table 36. A device may resume sending CVD frames

- after it has received a frame from associated devices that have the “CVD usage option” bit set to ‘1’. When

- the coordinator transmits and receives data with a device, if another device transmits an in-band CVD frame,

- interference may occur in the link between the coordinator and device. Out-of-band idle patterns may be

- used to maintain visibility when interference is seen in devices due to use of the CVD frame. In this case, the

- coordinator may indicate the transmission of “CVD usage option” with the CVD usage option bit set to ‘0’.

- The CVD frame should be used prudently so as to cause minimal interference and prolong battery life. In

- many cases, a light source is used for illumination, which takes precedence over the use for communication.

- The CVD usage option subfield is 1 bit in length and shall be set to ‘1’ if the device is sending a CVD frame.

- This subfield shall be set to ‘0’ otherwise.

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

|  |  | Table 36—CVD disable field |  |  |
| --- | --- | --- | --- | --- |
| Command frame | Bit |  | Usage/Description |  |
| payload |  |  |  |  |
| CVD usage option | b0 |  | logic 1 indicates that the device shall transmit the CVD frame |  |
|  |  |  | logic 0 indicates that the device shall not transmit the CVD frame |  |
|  |  |  | and may use out-of-band idle patterns if visibility needs to be |  |
|  |  |  | maintained |  |
|  | b1–b7 |  | Reserved |  |

5.4.24 CSI Control Frame

Editor’s note: need to register the new command frame in Table 29, the format Figure 123 need to be adjusted to be compatible with other command frames

The CSI control frame contains the standard MAC header information defined in subclause 5.8.2. The frame body described in this clause is the only specific field. It is structured as described in Fig. 5.8.4.3.2.1.

5.4.24.1 MIMO Mode

A one bit value indicating whether the current CSI frame contains information relevant for the SISO or MIMO mode of operation:

154	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  |  |  |  |  |  |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: 1 | 1 |  | 1 | 4 |  | 4 | 4/8 | 4/6 |  |  |
|  |  |  |  |  |  |  |  |  | 2 |  |
| MIMO | Adaptive |  | Frame | Number of MIMO |  | Number of MIMO | Channel | RATE/CSI | 3 |  |
|  |  |  |  |  |  |  |  |  | 4 |  |
| Mode | Loading |  | Info | Channels TX |  | Channels RX | Number |  |  |  |
|  |  |  |  |  |  |  |  |  | 5 |  |
|  |  |  | Figure 123—Format of the CSI control frame body |  |  |  |  |  | 6 |  |
|  |  |  |  |  |  |  |  |  | 7 |  |
| 1  MIMO mode of operation. |  |  |  |  |  |  |  |  | 8 |  |
|  |  |  |  |  |  |  |  |  | 9 |  |
| 0  SISO mode of operation. |  |  |  |  |  |  |  |  | 10 |  |
|  |  |  |  |  |  |  |  |  | 11 |  |
| 5.4.24.2 Adaptive Loading |  |  |  |  |  |  |  |  | 12 |  |
|  |  |  |  |  |  |  |  |  | 13 |  |
| A one-bit field which indicates whether the CSI in the current frame refers to the adaptive bit loading mode. |  |  |  |  |  |  |  |  | 14 |  |
|  |  |  |  |  |  |  |  |  | 15 |  |
| 1  The CSI is relevant for the adaptive bit loading scheme |  |  |  |  |  |  |  |  | 16 |  |
|  |  |  |  |  |  |  |  |  | 17 |  |
| 0  The CSI is relevant for the fixed bit loading scheme |  |  |  |  |  |  |  |  | 18 |  |
|  |  |  |  |  |  |  |  |  | 19 |  |
| 5.4.24.3 Frame Info |  |  |  |  |  |  |  |  | 20 |  |
|  |  |  |  |  |  |  |  |  | 21 |  |
| A one-bit field indicating the type of information carried by the current frame: |  |  |  |  |  |  |  |  | 22 |  |
|  |  |  |  |  |  |  |  |  | 23 |  |
| '0'  The frame carries CQIs |  |  |  |  |  |  |  |  | 24 |  |
|  |  |  |  |  |  |  |  |  | 25 |  |
| '1'  The frame carrier bit loading (RATE) information |  |  |  |  |  |  |  |  | 26 |  |
|  |  |  |  |  |  |  |  |  | 27 |  |
| 5.4.24.4 Number of MIMO Channels TX |  |  |  |  |  |  |  |  | 28 |  |
|  |  |  |  |  |  |  |  |  | 29 |  |
| The number of MIMO channels of the transmitting node in the MIMO configuration. Values range between |  |  |  |  |  |  |  |  | 30 |  |
|  |  |  |  |  |  |  |  |  | 31 |  |
| 0 and 15 corresponding to 1 and 16 channels, respectively. If the current CSI frame carries CQIs, then this |  |  |  |  |  |  |  |  | 32 |  |
| field contains the number of the MIMO channels for the node receiving the frame. If the frame carriers bit |  |  |  |  |  |  |  |  | 33 |  |
| loading (RATE) information, this field contains the number of the MIMO channels for the node transmitting |  |  |  |  |  |  |  |  | 34 |  |
| the frame. This information is expected to have been exchanged via the advanced modulation control frame |  |  |  |  |  |  |  |  | 35 |  |
| prior to any node issuing a CSI control frame. |  |  |  |  |  |  |  |  | 36 |  |
| 5.4.24.5 Number of MIMO Channels RX |  |  |  |  |  |  |  |  | 37 |  |
|  |  |  |  |  |  |  |  |  | 38 |  |
| The number of MIMO channels of the receiving node in the MIMO configuration. Values range between 0 |  |  |  |  |  |  |  |  | 39 |  |
|  |  |  |  |  |  |  |  |  | 40 |  |
| and 15 corresponding to 1 and 16 channels, respectively. If the current CSI frame carries CQIs, then this |  |  |  |  |  |  |  |  | 41 |  |
| field contains the number of the MIMO channels for the node transmitting the frame. If the frame carriers bit |  |  |  |  |  |  |  |  | 42 |  |
| loading (RATE) information, the this field contains the number of the MIMO channels for the node receiv- |  |  |  |  |  |  |  |  | 43 |  |
| ing the frame. This information is expected to have been exchanged via the advanced modulation control |  |  |  |  |  |  |  |  | 44 |  |
| frame prior to any node issuing a CSI control frame. |  |  |  |  |  |  |  |  | 45 |  |
| 5.4.24.6 Channel Number |  |  |  |  |  |  |  |  | 46 |  |
|  |  |  |  |  |  |  |  |  | 47 |  |
| The MIMO channel to which the subsequent RATE/CSI information refers. In a SISO mode, this field is set |  |  |  |  |  |  |  |  | 48 |  |
|  |  |  |  |  |  |  |  |  | 49 |  |
| to '0000'. If the Adaptive Loading field is set to '1', then this field occurs only once and is followed by the |  |  |  |  |  |  |  |  | 50 |  |
| CQI / RATE information for every subcarrier of the respective MIMO channel. If the Adaptive Loading |  |  |  |  |  |  |  |  | 51 |  |
| field is set to '0', then this field occurs as many times as there are MIMO channels for which relevant infor- |  |  |  |  |  |  |  |  | 52 |  |
| mation needs to be exchanged followed by the relevant information as CQI / RATE values. If the Frame Info |  |  |  |  |  |  |  |  | 53 |  |
| is set to '0', then this field occurs Number of MIMO Channels TX . Number of MIMO Channels RX times |  |  |  |  |  |  |  |  | 54 |  |
|  |  |  | Copyright © 20xx IEEE. All rights reserved. |  |  |  |  | 155 |  |  |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- where the 6-bit CQI following each occurrence corresponds to the average SNR achieved for the respective

- combination between the corresponding MIMO transmitter and MIMO receiver. The order in which the

- information is arranged is | TX1  RX | TX1  RX2 |…| TX1  RXN_MIMO| TX2  RX1 | TX2 

-  RX2 | … |TX2  RXN_MIMO| … | TXM_MIMO  RX1 | TXM_MIMO  RX2 | … |TXM_MIMO 

- RXN_MIMO |. If the Frame Info is set to '1', then this field occurs #_of_MIMO_Channels_TX times where

- the information is arranged in the order | TX1 | TX2 | TX3 | … | TXM_MIMO|. In each case, the occurrence of

- the Channel # is followed by the respective 4-bit RATE value for that TX channel.

8

9	5.4.24.7 RATE / CSI

10

- The RATE / CSI values for the MIMO channel specified in Channel #. If the Adaptive Loading field is set to

- '1', then this field occurs once for every subcarrier modulated with information on the respective MIMO

- channel. If the Adaptive Loading field is set to '0', then this field occurs as many times as there are MIMO

- channels for which relevant information needs to be exchanged and is preceded by the relevant MIMO

- Channel # field. If the Frame Info is set to '0', then this field occurs #_of_MIMO_Channels_TX ? x_of_MI-

- MO_Channels_RX times where a 6-bit CQI value is specified corresponding to the average SNR achieved

- for the respective combination between the corresponding MIMO transmitter and MIMO receiver. The order

- in which the information is arranged is | TX1  RX | TX1  RX2 |…| TX1  RXN_MIMO| TX2 

- RX1 | TX2  RX2 | … |TX2  RXN_MIMO | … | TXM_MIMO  RX1 | TXM_MIMO  RX2 | …

- |TXM_MIMO  RXN_MIMO |. If the Frame Info is set to '1', then this field occurs # of MIMO Channels TX

- times where the 4-bit RATE information is arranged in the order | TX1 | TX2 | TX3 | … | TXM_MIMO |. In

- each case, the occurrence of the RATE field is preceded by the respective Channel # value for that TX chan-

- nel.

| 24 | 5.4.25 Dimming Control Frame |  |
| --- | --- | --- |
| 25 |  |  |
| 26 | editor’s note: the text is not complete. |  |
| 27 |  |  |
| 28 |  |  |

- The frame contains control information which needs to be exchanged for the purposes of realizing dimming

- techniques such as RPO-OFDM and/or analog dimming.

31

32	5.4.26 Information element command

33

- The format of an individual information element (IE) is shown in Figure 124. The first octet is the Element

- ID and the second octet is the Length (Ln) of the payload of the IE in octets. The following Ln octets are the

- payload for the IE. Unless otherwise specified, these elements may appear in any order in the frames that are

- allowed to include more than one of these elements.

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

| Octets: 1 | 1 | Ln |
| --- | --- | --- |
| Element ID | Length (=Ln) | IE payload |

Figure 124—Information element format

The information elements defined in this standard are listed in Table 37.

When the information elements are used, they shall be added at the end of command frame format. Multiple information elements can be part of a single command frame. IEs can be added to any command frame.

156	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 37—Information elements

| Element ID | Element | Subclause |  |
| --- | --- | --- | --- |
| hex value |  |  |  |
| 0x01 | Capabilities | 5.4.26.1 |  |
| 0x02 | Wavelength quality | 5.4.26.2 |  |
|  | indication |  |  |

5.4.26.1 Capabilities IE

The capabilities IE is used to convey device MAC and PHY capabilities to peer devices. The capabilities IE, as shown in Figure 125, consists of two fields: the capability information field, refer to Table 38, which indicates general capabilities of the device; and the aggregation bitmap field, which is specified in Figure 125.

| Octets: 8 | variable: 8n | variable: 8n |
| --- | --- | --- |
| Capability Information Field | Aggregation bitmap field | Guard bitmap field |

Figure 125—Capabilities IE

5.4.26.1.1 Capability information field

The capability information field is illustrated in Table 38.

|  |  | Table 38—Capability information field |  |
| --- | --- | --- | --- |
|  | Bit | Function |  |
|  | position |  |  |
|  | 0 | Power source |  |
|  | 1–2 | Battery information |  |
|  | 3 | Security capability |  |
|  | 4 | Coordinator capability |  |
| MAC layer | 5 | Traffic support |  |
| capabilities | 6–8 | Topology support |  |
|  | 9–10 | Device type |  |
|  | 11 | Beacon support |  |
|  | 12 | Dimming support |  |
|  | 13 | Continuous visibility transmission (for infrastructure) |  |
|  | 14 | CVD support |  |
|  | 15–23 | Reserved |  |

| Copyright © 20xx IEEE. All rights reserved. | 157 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

Table 38—Capability information field (continued)

|  | Bit | Function |  |
| --- | --- | --- | --- |
|  | position |  |  |
|  | 24 | PHY I support |  |
|  | 25 | PHY II support |  |
|  | 26 | PHY III support |  |
| PHY layer |  |  |  |
|  | 27–28 | Color stabilization capability |  |
| capabilities |  |  |  |
|  | 29–31 | Max supported TX clock |  |
|  | 32–34 | Max supported RX clock |  |
|  | 35 | Explicit clock notification request |  |
|  | 36 | CCA support |  |
|  | 37–39 | Reserved |  |
| Physical | 40–42 | Number of optical sources |  |
|  | 43–45 | Multiple direction support |  |
| device |  |  |  |
| capabilities |  |  |  |
|  | 46–55 | Number of cells supported (n) |  |
| Band | 56–63 | Bands used for PHY III (any 3 bits of the bits set to 1 can be used) |  |
| capabilities |  |  |  |

- The power source subfield is 1 bit in length and shall be set to one if the device is receiving power from the

- alternating current mains. Otherwise, the power source subfield shall be set to zero.

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

The battery information subfield, shown in Table 39, is set to reserved (11) if the power source is set to 1.

Table 39—Battery Indication

| Bits (b2 b1) | Battery indication |
| --- | --- |
| 00 | Unknown |
| 11 | < 50% (low battery) |
| 10 | 50% (sufficient battery) |
| 11 | Reserved |

The security capability subfield is 1 bit in length and shall be set to one if the device is capable of sending and receiving cryptographically protected MAC frames; otherwise, it shall be set to zero.

The coordinator capability subfield is 1 bit in length and shall be set to 1 if the device is capable of functioning as a coordinator; otherwise, it shall be set to zero.

The traffic support capability subfield is 1 bit in length. It shall be set to 0 if the device is only capable of broadcasting (unidirectional) communication. Otherwise, it shall be set to 1.

158	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The topology support capability subfield can support multiple topologies via the bit maps of Table 39.

Table 40—Topology support capability

| Bits (b8 b7 b6) | Topology indication |
| --- | --- |
| b8 | P2MP |
| b7 | P2P |
| b6 | Broadcast |

The device-type capability subfield is set according to Table 41. This information is provided to assist upper layers.

Table 41—Device-type capability

| Bits (b10 b9) | Device capability |
| --- | --- |
| 00 | Infrastructure |
| 01 | Mobile |
| 10 | Vehicle |
| 11 | Unknown/reserved |

The beacon support capability subfield is 1 bit in length. It shall be set to 1 if the device is capable of sending beacons. Otherwise, it shall be set to 0.

The dimming support in MAC capability subfield is 1 bit in length. It shall be set to 1 if the device is capable of supporting dimming in the MAC using duty cycling and idle patterns. Otherwise, it shall be set to 0. A device shall honor all dimming requests. If the dimming support bit is not set then the device shall not attempt to communicate when a dimming request is received and shall comply with the dimming request even if the device must disassociate from the network as discussed in 5.1.6.2. Even if the device supports dimming but is unable to communicate during dimming, it shall set the macDimDataFailureIndication MAC PIB attribute as mentioned in Table 91, but shall still comply with the dimming request at the expense of loss of communication.

The continuous visibility transmission subfield is one bit in length. It shall be set to 1 if the device will be continuously transmitting to maintain illumination. Otherwise, it shall be to 0.

The CVD support subfield is 1 bit in length. It shall be set to 1 if the device is capable of transmitting various colors; otherwise, it shall be set to 0.

The PHY I support subfield is 1 bit in length. It shall be set to 1 if the device supports PHY I.

The PHY II support subfield is 1 bit in length. It shall be set to 1 if the device supports PHY II.

The PHY III support subfield is 1 bit in length. It shall be set to 1 if the device supports PHY III.

| Copyright © 20xx IEEE. All rights reserved. | 159 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

The color-stabilization capability subfield describes if and where (device and/or coordinator) color stabilization is performed. All allowed settings are shown in Table 42.

|  | Table 42—Color-stabilization capability |
| --- | --- |
| Bits (b28 b27) | Color-stabilization scheme |
| 00 | No color stabilization |

- Color-stabilization information to be sent from device to coordinator upon reception of CVD frames

- Color-stabilization information to be sent from coordinator to device upon reception of CVD frames

- Color-stabilization information to be sent from device to coordinator and from coordinator to device when either receives CVD frames

- The max supported TX clock subfield and max RX clock subfields follow the usage as indicated in Table 43.

- Support for 200 kHz is mandatory for PHY I and support of 3.75 MHz is mandatory for PHY II. Support for

- 12 MHz is mandatory for PHY III and shall be indicated using bits ‘100’ as in Table 43.

| 22 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 23 | Table 43—Maximum supported optical clock frequency |  |  |  |  |
| 24 |  |  |  |  |  |
| 25 |  |  |  |  |  |
| 26 |  | Bits (b31 b30 b29) | Description |  |  |
| 27 |  |  |  |  |  |
| 28 |  | 000 | 200 kHz |  |  |
| 29 |  |  |  |  |  |
|  |  | 001 |  400 kHz |  |  |
| 30 |  |  |  |  |  |
| 31 |  | 010 | 3.75 MHz |  |  |
| 32 |  |  |  |  |  |
|  |  | 011 | 7.5 MHz |  |  |
| 33 |  |  |  |  |  |
| 34 |  | 100 | 15 MHz |  |  |
| 35 |  |  |  |  |  |
|  |  | 101 | 30 MHz |  |  |
| 36 |  |  |  |  |  |
| 37 |  | 110 | 60 MHz |  |  |
| 38 |  |  |  |  |  |
|  |  | 111 | 120 MHz |  |  |
| 39 |  |  |  |  |  |
| 40 |  |  |  |  |  |
| 41 |  |  |  |  |  |

- The explicit clock notification subfield is 1 bit in length. The subfield shall be set to 1 if the receiving device

- needs an explicit clock change notification from the transmitter before any change of clock frequency.

44

45 If CCA is supported, then the CCA Support bit is set to 1; otherwise, the bit is set to 0. 46

- The number of optical sources subfield indicates the number of optical sources in the transmitter of the

- device that have distinct frequency responses.

49

- The multiple direction support subfield indicates the number of distinct directions supported by the device

- transmitter supported by the multiple optical sources. This is used for fast link recovery as defined in 5.4.16.

52

- The number of cells n indicates the maximum number of cells supported in the device. The number of cells

- supported shall not be more than 1023.

160	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

In regards to the bands used for PHY III, bit 7 is reserved and bits 0–6 map to the bits corresponding to the bandplan. Only 3 bits shall be set to indicate PHY III usage. If the device supports more colors and wants to change the PHY III usage, it needs to send the capabilities information again with the new bitmap.

editor note: the following advanced modulation capabilities should be merged into capabilities IE, see com-ment 119 of https://mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx

Advanced Modulation Control Frame

This control frame shall indicate the advanced modulation capabilities of a communication node. The frame header contains the MAC header fields defined in subclause 6.4.1 and the frame body shall be structured as described in Fig. 1.

| Bits: 1 | 4 | 1 | 1 | 4 | 1 | 4 | IDFT_SIZE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Adative | eU | RPO | Analog |  |  | # of |  |  |
|  |  |  |  | Relayingh | MIMO | MIMO | SC-FDMA |  |
| Loading |  |  | Dimming |  |  |  |  |  |
|  |  |  |  |  |  | Channel |  |  |

Figure 126—Format of the advanced modulation control frame body

5.4.26.1.2 Adaptive Loading

A single bit indicating whether the communication node transmitting the advanced modulation control frame supports adaptive bit and energy loading:

1  Adaptive bit and energy loading is supported.

0  Adaptive bit and energy loading is not supported.

5.4.26.1.3 eU

Four bits indicating if the node supports eU-OFDM. The bit value at a given position out of the four posi-tions indicates whether eU-OFDM implementation with the same number of streams as the bit position is supported. Positions are counted from left to right. For example:

1000  eU-OFDM with one stream only is supported.

1100  eU-OFDM with one and two streams only is supported

1010  eU-OFDM with one and three streams only is supported

1111  eU-OFDM with all possible streams is supported

0000  eU-OFDM is not supported

5.4.26.1.4 RPO

A single bit indicating whether the communication node transmitting the advanced modulation control rame supports RPO-OFDM:

'1'  RPO-OFDM is supported.

| Copyright © 20xx IEEE. All rights reserved. | 161 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | '0'  RPO-OFDM is not supported. |  |
| --- | --- | --- |
| 2 | 5.4.26.1.5 Analog Dimming |  |
| 3 |  |  |
| 4 |  |  |

- A single bit indicating whether the communication node transmitting the advanced modulation control

- frame supports analog dimming (adjusting the analog signal bias level):

| 7 | '1'  Analog dimming is supported. |  |
| --- | --- | --- |
| 8 |  |  |
| 9 | '0'  Analog dimming is not supported. |  |
| 10 |  |  |
| 11 | 5.4.26.1.6 Relaying |  |
| 12 |  |  |
| 13 |  |  |

- Four bits indicating types of relaying operations the communication node transmitting the advanced modula-

- tion control frame supports. The first bit (B7) indicates whether relaying in FD is supported:

| 16 | '1'  Relaying in FD is supported. |  |
| --- | --- | --- |
| 17 |  |  |
| 18 | '0'  Relaying in FD is not supported. |  |
| 19 |  |  |
| 20 | The second bit (B8) indicates whether relaying in HD is supported: |  |
| 21 |  |  |
| 22 | '1'  Relaying in HD is supported. |  |
| 23 |  |  |
| 24 | '0'  Relaying in HD is not supported. |  |
| 25 |  |  |
| 26 | The third bit (B9) indicates whether AF relaying is supported: |  |
| 27 |  |  |
| 28 | '1'  AF relaying is supported. |  |
| 29 |  |  |
| 30 | '0'  AF relaying is not supported. |  |
| 31 |  |  |
| 32 | The fourth bit (B10) indicates whether DF relaying is supported: |  |
| 33 |  |  |
| 34 | '1'  DF relaying is supported. |  |
| 35 |  |  |
| 36 | '0'  DF relaying is not supported. |  |
| 37 |  |  |
| 38 | 5.4.26.1.7 MIMO |  |
| 39 |  |  |
| 40 |  |  |

- A single bit indicating whether the communication node transmitting the advanced modulation control

- frame supports MIMO communication:

| 43 | '1'  MIMO is supported. |  |
| --- | --- | --- |
| 44 |  |  |
| 45 | '0'  MIMO is not supported. |  |
| 46 |  |  |
| 47 | 5.4.26.1.8 Number of MIMO Channels |  |
| 48 |  |  |
| 49 |  |  |

- Four bits indicating the maximum number of MIMO communication channels which the communication

- node transmitting the advanced modulation control frame supports. A value of '0000' corresponds to 1 chan-

- nel, and a value of '1111' corresponds to 16 channels.

53

54

162	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 5.4.26.1.9 SC-FDMA |  | 1 |  |
| --- | --- | --- | --- |
| An IDFT_SIZE of bits, where IDFT_SIZE is the size of the IDFT operation used for OFDM time-domain |  | 2 |  |
|  |  | 3 |  |
| signal generation. The bit at position k indicates whether a DFT transform of size k is supported for SC- |  | 4 |  |
| FDMA pre-coding. For example, if IDFT_SIZE = 16, then: |  | 5 |  |
| '10000000000000000' - A DFT of size 1 is supported. |  | 6 |  |
|  |  | 7 |  |
| '00100000000000000' - A DFT of size 3 is supported. |  | 8 |  |
|  |  | 9 |  |
| '10100100000000000' - A DFT of size 1, 3 and 6 is supported. |  | 10 |  |
|  |  | 11 |  |
| '00000000000000000' - No DFT size is supported, so SC-FDMA is not supported. |  | 12 |  |
|  |  | 13 |  |
|  |  | 14 |  |
|  |  | 15 |  |
| 5.4.26.1.10 Aggregation and guard channel |  | 16 |  |
|  |  | 17 |  |
| The aggregation and guard channels are used to support any optical light optical source for OWC that may |  | 18 |  |
|  |  | 19 |  |
| have variable spectral widths and center frequencies. The aggregation and guard bitmap for a single optical |  | 20 |  |
| source type is as shown in Figure 127. The bit map is variable in length. The length of the aggregation and |  | 21 |  |
| guard bit maps are ‘n’ octets each, where ‘n’ is the number of optical source types. The aggregation and |  | 22 |  |
| guard channel bit usage are defined by an 8-bit bitmap for every optical source type supported by the |  | 23 |  |
| transmitter of the device. The 8-bit bitmap is indexed by the bandplan identification number. The bit position |  | 24 |  |
| ‘m’ is set to a ‘1’ for band 'm' if that band is used by the optical source. The reserved bit in Figure 127 shall |  | 25 |  |
| be set to 0. |  | 26 |  |
|  |  | 27 |  |
|  |  | 28 |  |
|  |  | 29 |  |
|  |  | 30 |  |
|  |  | 31 |  |
|  |  | 32 |  |
|  |  | 33 |  |
|  |  | 34 |  |
|  |  | 35 |  |
|  |  | 36 |  |
|  |  | 37 |  |
|  |  | 38 |  |
| Figure 127—Aggregation and guard bitmap per optical source type |  | 39 |  |
|  |  | 40 |  |
|  |  | 41 |  |
| For example, |  | 42 |  |
|  |  | 43 |  |
| if band 1 and band 2 need to be aggregated [assuming a blue light-emitting diode (LED)], the |  | 44 |  |
|  |  | 45 |  |
| aggregation bit-map is indicated as 0110000 and the guard bit-map is indicated as 0000000. |  |  |  |
|  |  | 46 |  |
| if band 1 is being used but there is leakage in bands 3, 4, 5 (assuming a white LED, which is |  | 47 |  |
|  |  | 48 |  |
| realized via a blue LED with yellow phosphor), the aggregation bit map is indicated as 0100000 and the |  |  |  |
|  |  | 49 |  |
| guard bit-map is indicated as 0001110. |  |  |  |
|  |  | 50 |  |
|  |  | 51 |  |
|  |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 163 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 5.4.26.2 Wavelength quality indication (WQI) IE |
| --- | --- |
| 2 |  |

- WQI is communicated to another device using the WQI Information Element. The WQI value to be sent in

- the Information Element may be an average value across a number of packets, and WQI value sets for a

5	number of band plan ID's can be reported using the WQI information element as shown in Table 44. The

- wavelength quality indication IE is 7 octets in length and the WQI information is provided for all band plan

- IDs. If a band plan ID is not supported, WQI of 0 shall be reported.

| 8 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 9 |  | Table 44—Wavelength quality indication IE |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  | Band plan code | WQI value |  |
| 13 |  |  |  |  |
| 14 |  | 0x00 | 0x00 to 0xff |  |
| 15 |  |  |  |  |
|  |  | 0x01 | 0x00 to 0xff |  |
| 16 |  |  |  |  |
| 17 |  | 0x02 | 0x00 to 0xff |  |
| 18 |  |  |  |  |
|  |  | 0x03 | 0x00 to 0xff |  |
| 19 |  |  |  |  |
| 20 |  | 0x04 | 0x00 to 0xff |  |
| 21 |  |  |  |  |
|  |  | 0x05 | 0x00 to 0xff |  |
| 22 |  |  |  |  |
| 23 |  | 0x06 | 0x00 to 0xff |  |
| 24 |  |  |  |  |
| 25 |  |  |  |  |
| 26 | 6. MAC s ublayer s ervice s pecification |  |  |  |
| 27 |  |  |  |  |
| 28 | 6.1 Overview |  |  |  |
| 29 |  |  |  |  |
| 30 |  |  |  |  |

- The MAC sublayer provides an interface between the SSCS, DME and the PHY. The MAC sublayer

- conceptually includes a management entity called the MLME. This entity provides the service interfaces

- through which layer management functions may be invoked. The MLME is also responsible for maintaining

- a database of managed objects pertaining to the MAC sublayer. This database is referred to as the MAC

- sublayer PIB.

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Figure 128 depicts the components and interfaces of the MAC sublayer.

Figure 128—MAC sublayer reference model

164	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The MAC sublayer provides the following two services, accessed through two SAPs:

- The MAC data service, accessed through the MAC common part sublayer (MCPS) data SAP (MCPS-SAP), and

- The MAC management service, accessed through the MLME-SAP.

These two services provide the interface between the SSCS and the PHY, via the PD-SAP and PLME-SAP interfaces (see Clause 9). In addition to these external interfaces, an implicit interface also exists between the MLME and the MCPS that allows the MLME to use the MAC data service.

6.2 MAC data s ervice

The MCPS-SAP supports the transport of SSCS protocol data units (SPDUs) between peer SSCS entities. Table 45 lists the primitives supported by the MCPS-SAP. These primitives are discussed in the subclauses referenced in this table.

Table 45—MCPS-SAP primitives

| MCPS-SAP primitive | Request | Confirm | Indication |
| --- | --- | --- | --- |
| MCPS-DATA | 6.2.1 | 6.2.2 | 6.2.3 |
| MCPS-PURGE | 6.2.4 | 6.2.5 | — |

6.2.1 MCPS-DATA.request

The MCPS-DATA.request primitive requests the transfer of a data SPDU (i.e., MSDU) from a local SSCS entity to a single peer SSCS entity. In the packed mode, multiple MSDU are passed via a local SSCS entity to a single peer SSCS entity.

The semantics of the MCPS-DATA.request primitive are as follows:

MCPS-DATA.request	(

SrcAddrMode,

DstAddrMode,

DstOWPANId,

DstAddr,

MsduLength,

Msdu,

MsduHandle,

TxOptions,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

DataRate,

BurstMode,

ColorReceived,

ColorNotReceived

)

| Copyright © 20xx IEEE. All rights reserved. | 165 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 46 specifies the parameters for the MCPS-DATA.request primitive.

Table 46—MCPS-DATA.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SrcAddrMode | Integer | 0x00–0x03 | The source addressing mode for this primitive and |
|  |  |  | subsequent MPDU. This value can take one of the |
|  |  |  | following values: |
|  |  |  | 0x00 = No address (addressing fields omitted, as |
|  |  |  | defined in 5.2.1.1.7). |
|  |  |  | 0x01 = Reserved. |
|  |  |  | 0x02 = 16-bit short address. |
|  |  |  | 0x03 = 64-bit extended address. |
| DstAddrMode | Integer | 0x00–0x03 | The destination addressing mode for this primitive |
|  |  |  | and subsequent MPDU. This value can take one of |
|  |  |  | the following values: |
|  |  |  | 0x00 = No address (addressing fields omitted, as |
|  |  |  | defined in 5.2.1.1.6). |
|  |  |  | 0x01 = No address field (broadcast only mode with |
|  |  |  | no address fields present). |
|  |  |  | 0x02 = 16-bit short address. |
|  |  |  | 0x03 = 64-bit extended address. |
| DstOWPANId | Integer | 0x0000–0xffff | The 16-bit OWPAN identifier of the entity to which |
|  |  |  | the MSDU is being transferred. |
| DstAddr | Device | As specified by the | The individual device address of the entity to which |
|  | address | DstAddrMode parameter | the MSDU is being transferred. |
| MsduLength | Integer |  aMaxMACPayloadSize | The number of octets contained in the MSDU to be |
|  |  |  | transmitted by the MAC sublayer entity. |
| Msdu | Set of octets | — | The set of octets forming the MSDU to be |
|  |  |  | transmitted by the MAC sublayer entity. |
| MsduHandle | Integer | 0x00–0xff | The handle associated with the MSDU to be |
|  |  |  | transmitted by the MAC sublayer entity. |
| TxOptions | Bitmap | 3-bit field | The 3 bits (b0, b1, b2) indicate the transmission |
|  |  |  | options for this MSDU. |
|  |  |  | For b0, 1 = acknowledged transmission, |
|  |  |  | 0 = unacknowledged transmission. |
|  |  |  | For b1, 1 = GTS transmission, 0 = CAP transmission |
|  |  |  | for a beacon-enabled OWPAN. |
|  |  |  | For b2, 1 = indirect transmission, 0 = direct |
|  |  |  | transmission. |
|  |  |  | For a non-beacon-enabled OWPAN, bit b1 should |
|  |  |  | always be set to 0. |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in Table 97 |
|  |  |  | in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (as |
|  |  |  | defined in Table 98 in 7.4.2.2). This parameter is |
|  |  |  | ignored if the SecurityLevel parameter is set to |
|  |  |  | 0x00. |
| KeySource | Set of 0, 4, | As specified by the | The originator of the key to be used (see 7.4.4.1). |
|  | or 8 octets | KeyIdMode parameter | This parameter is ignored if the KeyIdMode |
|  |  |  | parameter is ignored or set to 0x00. |

166	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 46—MCPS-DATA.request parameters (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). This |
|  |  |  | parameter is ignored if the KeyIdMode parameter is |
|  |  |  | ignored or set to 0x00. |
| DataRate | Enumeration | 6-bit field | The data rate of the PHY frame to be transmitted by |
|  |  |  | the PHY entity as shown in Table 118. |
| BurstMode | Boolean | TRUE or FALSE | The BurstMode bit shall be set TRUE if the burst |
|  |  |  | mode is being used (as discussed in 8.6.1); |
|  |  |  | otherwise, the BurstMode bit shall be set FALSE. |
| ColorReceived | Boolean | TRUE or FALSE | ColorReceived shall be set as TRUE, when the |
|  |  |  | ACK frame is sent and the color function for the |
|  |  |  | ACK state indication is used by the CVD frame. |
|  |  |  | ColorReceived shall be set as FALSE when the |
|  |  |  | ACK frame is sent but the color function for the |
|  |  |  | ACK state indication is not used by the CVD frame. |
| ColorNotReceived | Boolean | TRUE or FALSE | ColorNotReceived shall be set as TRUE, when the |
|  |  |  | ACK frame is not sent but the color function for the |
|  |  |  | non-ACK state indication is used by the CVD |
|  |  |  | frame. |
|  |  |  | ColorNotReceived shall be set as FALSE when the |
|  |  |  | ACK frame is not sent and the color function for the |
|  |  |  | non-ACK state indication is not used by the CVD |
|  |  |  | frame. |

6.2.1.1 Appropriate usage

The MCPS-DATA.request primitive is generated by a local SSCS entity when a data SPDU (i.e., MSDU) is to be transferred to a peer SSCS entity.

6.2.1.2 Effect on receipt

On receipt of the MCPS-DATA.request primitive, the MAC sublayer entity begins the transmission of the supplied MSDU.

The MAC sublayer builds an MPDU to transmit from the supplied arguments. The flags in the SrcAddrMode and DstAddrMode parameters correspond to the addressing subfields in the frame control field, as shown in 5.2.1.1, and are used to construct both the frame control and addressing fields of the MHR. If both the SrcAddrMode and the DstAddrMode parameters are set to 0x00 (i.e., addressing fields omitted), the MAC sublayer will issue the MCPS-DATA.confirm primitive with a status of INVALID_ADDRESS.

The TxOptions parameter indicates how the MAC sublayer data service transmits the supplied MSDU. If the TxOptions parameter specifies that an acknowledged transmission is required, the Acknowledgment Request subfield of the frame control field will be set to one (see 5.1.9.4).

If the TxOptions parameter specifies that a GTS transmission is required, the MAC sublayer will determine whether it has a valid GTS (for GTS usage rules, as defined in 5.1.10.3) . If a valid GTS could not be found, the MAC sublayer will issue the MCPS-DATA.confirm primitive with a status of INVALID_GTS. If a valid GTS was found, the MAC sublayer will defer, if necessary, until the GTS. If the TxOptions parameter specifies that a GTS transmission is not required, the MAC sublayer will transmit the MSDU using either

| Copyright © 20xx IEEE. All rights reserved. | 167 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1	slotted random access in the CAP for a beacon-enabled OWPAN or unslotted random access for a

- nonbeacon-enabled OWPAN. Specifying a GTS transmission in the TxOptions parameter overrides an

- indirect transmission request.

4

- If the TxOptions parameter specifies that an indirect transmission is required and this primitive is received

- by the MAC sublayer of a coordinator, the data frame is sent using indirect transmission, i.e., the data frame

- is added to the list of pending transactions stored on the coordinator and extracted at the discretion of the

- device concerned using the method described in 5.1.9.3. Transactions with a broadcast destination address

- will be transmitted using the mechanism described in 5.2.1.1.4. Transactions with a unicast destination

- address can then be extracted at the discretion of each device concerned using the method described in

- 5.1.9.3. If there is no capacity to store the transaction, the MAC sublayer will discard the MSDU and issue

- the MCPS-DATA.confirm primitive with a status of TRANSACTION_OVERFLOW. If there is capacity to

- store the transaction, the coordinator will add the information to the list. If the transaction is not handled

14	within macTransactionPersistenceTime, the transaction information will be discarded and the MAC

- sublayer will issue the MCPS-DATA.confirm primitive with a status of TRANSACTION_EXPIRED. The

- transaction handling procedure is described in 5.1.8. If the TxOptions parameter specifies that an indirect

- transmission is required and if the device receiving this primitive is not a coordinator, the destination address

- is not present, or the TxOptions parameter also specifies a GTS transmission, the indirect transmission

- option will be ignored.

20

- If the TxOptions parameter specifies that an indirect transmission is not required, the MAC sublayer will

- transmit the MSDU using slotted random access either in the CAP for a beacon-enabled OWPAN or

23	immediately for a nonbeacon-enabled OWPAN. If the TxOptions parameter specifies that a direct

- transmission is required and the MAC sublayer does not receive an acknowledgment from the recipient after

- macMaxFrameRetries retransmissions (see 5.1.9.4), it will discard the MSDU and issue the MCPS-

- DATA.confirm primitive with a status of NO_ACK.

27

- If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for

- this frame, the MAC sublayer will set the Security Enabled subfield of the frame control field to one. The

- MAC sublayer will perform outgoing processing on the frame based on the DstAddr, SecurityLevel,

- KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs during

- outgoing frame processing, the MAC sublayer will discard the frame and issue the MCPS-DATA.confirm

- primitive with the error status returned by outgoing frame processing.

34

- If the requested transaction is too large to fit in the CAP or GTS, as appropriate, the MAC sublayer shall

- discard the frame and issue the MCPS-DATA.confirm primitive with a status of FRAME_TOO_LONG.

37

- If the transmission attempts a random access (either slotted or unslotted) and the random access algorithm

- failed due to adverse conditions on the channel, and the TxOptions parameter specifies that a direct

- transmission is required, the MAC sublayer will discard the MSDU and issue the MCPS-DATA.confirm

- primitive with a status of CHANNEL_ACCESS_FAILURE.

42

- If the MAC sublayer receives the request while transmission is prohibited, it shall delay transmission until

- transmission is permitted.

45

- If the MPDU was successfully transmitted and, if requested, an acknowledgment was received, the MAC

- sublayer will issue the MCPS-DATA.confirm primitive with a status of SUCCESS.

48

- If any parameter in the MCPS-DATA.request primitive is not supported or is out of range, the MAC sublayer

- will issue the MCPS-DATA.confirm primitive with a status of INVALID_PARAMETER.

51

52

53

54

168	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.2.2 MCPS-DATA.confirm

The MCPS-DATA.confirm primitive reports the results of a request to transfer a data SPDU (MSDU) from a local SSCS entity to a single peer SSCS entity.

The semantics of the MCPS-DATA.confirm primitive are as follows:

MCPS-DATA.confirm	(

MsduHandle,

status,

Timestamp

)

Table 47 specifies the parameters for the MCPS-DATA.confirm primitive.

Table 47—MCPS-DATA.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MsduHandle | Integer | 0x00–0xff | The handle associated with the MSDU |
|  |  |  | being confirmed. |
| status | Enumeration | SUCCESS, | The status of the last MSDU |
|  |  | TRANSACTION_OVERFLOW, | transmission. |
|  |  | TRANSACTION_EXPIRED, |  |
|  |  | CHANNEL_ACCESS_FAILURE, |  |
|  |  | INVALID_ADDRESS, |  |
|  |  | INVALID_GTS, NO_ACK, |  |
|  |  | COUNTER_ERROR, |  |
|  |  | FRAME_TOO_LONG, |  |
|  |  | UNAVAILABLE_KEY, |  |
|  |  | UNSUPPORTED_SECURITY or |  |
|  |  | INVALID_PARAMETER |  |
| Timestamp | Integer | 0x000000–0xffffff | Optional. The time, in optical clocks, at |
|  |  |  | which the data were transmitted (see |
|  |  |  | 5.1.7.1). |
|  |  |  | The value of this parameter will be |
|  |  |  | considered valid only if the value of the |
|  |  |  | status parameter is SUCCESS; if the |
|  |  |  | status parameter is not equal to |
|  |  |  | SUCCESS, the value of the Timestamp |
|  |  |  | parameter shall not be used for any other |
|  |  |  | purpose. The boundary is described by |
|  |  |  | macTimeStampOffset (as defined in |
|  |  |  | Table 91). |
|  |  |  | The time stamp is a 24-bit value, and |
|  |  |  | the precision of this value shall be a |
|  |  |  | minimum of 20 bits, with the lowest |
|  |  |  | 4 bits being the least significant. |

6.2.2.1 When generated

The MCPS-DATA.confirm primitive is generated by the MAC sublayer entity in response to an MCPS-DATA.request primitive. The MCPS-DATA.confirm primitive returns a status of either SUCCESS,

| Copyright © 20xx IEEE. All rights reserved. | 169 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- indicating that the request to transmit was successful, or the appropriate error code. The status values are

- fully described in 6.2.1.2 and subclauses referenced by 6.2.1.2.

3

4	6.2.2.2 Appropriate usage

5

- On receipt of the MCPS-DATA.confirm primitive, the SSCS of the initiating device is notified of the result

- of its request to transmit. If the transmission attempt was successful, the status parameter will be set to

- SUCCESS. Otherwise, the status parameter will indicate the error.

9

10	6.2.3 MCPS-DATA.indication

11

- The MCPS-DATA.indication primitive indicates the transfer of a data SPDU (i.e., MSDU) from the MAC

- sublayer to the local SSCS entity. In the packed mode, multiple MSDU are passed via a local SSCS entity to

- a single peer SSCS entity.

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

The semantics of the MCPS-DATA.indication primitive are as follows:

MCPS-DATA.indication	(

SrcAddrMode,

SrcOWPANId,

SrcAddr,

DstAddrMode,

DstOWPANId,

DstAddr,

MsduLength,

Msdu,

MpduLinkQuality,

DSN,

Timestamp,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

DataRate,

BurstMode,

ColorReceived,

ColorNotReceived

)

Table 48 specifies the parameters for the MCPS-DATA.indication primitive.

6.2.3.1 When generated

The MCPS-DATA.indication primitive is generated by the MAC sublayer and issued to the SSCS on receipt of a data frame at the local MAC sublayer entity that passes the appropriate message filtering operations as described in 5.1.9.2.

6.2.3.2 Appropriate usage

On receipt of the MCPS-DATA.indication primitive, the SSCS is notified of the arrival of data at the device.

170	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 48—MCPS-DATA.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SrcAddrMode | Integer | 0x00–0x03 | The source addressing mode for this primitive |
|  |  |  | corresponding to the received MPDU. This value can |
|  |  |  | take one of the following values: |
|  |  |  | 0x00 = no address (addressing fields omitted). |
|  |  |  | 0x01 = reserved. |
|  |  |  | 0x02 = 16-bit short address. |
|  |  |  | 0x03 = 64-bit extended address. |
| SrcOWPANId | Integer | 0x0000–0xffff | The 16-bit OWPAN identifier of the entity from which |
|  |  |  | the MSDU was received. |
| SrcAddr | Device | As specified by the | The individual device address of the entity from which |
|  | address | SrcAddrMode | the MSDU was received. |
|  |  | parameter |  |
| DstAddrMode | Integer | 0x00–0x03 | The destination addressing mode for this primitive |
|  |  |  | corresponding to the received MPDU. This value can |
|  |  |  | take one of the following values: |
|  |  |  | 0x00 = no address (addressing fields omitted). |
|  |  |  | 0x01 = no address field (broadcast only mode with no |
|  |  |  | address fields present). |
|  |  |  | 0x02 = 16-bit short device address. |
|  |  |  | 0x03 = 64-bit extended device address. |
| DstOWPANId | Integer | 0x0000–0xffff | The 16-bit OWPAN identifier of the entity to which |
|  |  |  | the MSDU is being transferred. |
| DstAddr | Device | As specified by the | The individual device address of the entity to which |
|  | address | DstAddrMode | the MSDU is being transferred. |
|  |  | parameter |  |
| MsduLength | Integer | aMaxMacPayloadSize | The number of octets contained in the MSDU being |
|  |  |  | indicated by the MAC sublayer entity. |
| Msdu | Set of octets | — | The set of octets forming the MSDU being indicated |
|  |  |  | by the MAC sublayer entity. |
| MpduLinkQuality | Integer | 0x00–0xff | WQI value measured during reception of the MPDU. |
|  |  |  | Lower values represent lower WQI (see 5.4.26.2) |
| DSN | Integer | 0x00–0xff | The DSN of the received data frame. |
| Timestamp | Integer | 0x000000–0xffffff | Optional. The time, in optical clocks, at which the data |
|  |  |  | were received (see 5.1.7.1). |
|  |  |  | The boundary is described by macTimeStampOffset (as |
|  |  |  | defined in Table 91). |
|  |  |  | The time stamp is a 24-bit value, and the precision of |
|  |  |  | this value shall be a minimum of 20 bits, with the |
|  |  |  | lowest 4 bits being the least significant. |
| SecurityLevel | Integer | 0x00–0x07 | The security level purportedly used by the received |
|  |  |  | data frame (as defined in Table 97 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key purportedly used by |
|  |  |  | the originator of the received frame (as defined in |
|  |  |  | Table 98 in 7.4.2.2). This parameter is invalid if the |
|  |  |  | SecurityLevel parameter is set to 0x00. |

| Copyright © 20xx IEEE. All rights reserved. | 171 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

Table 48—MCPS-DATA.indication parameters (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| KeySource | Set of 0, 4, | As specified by the | The originator of the key purportedly used by the |
|  | or 8 octets | KeyIdMode parameter | originator of the received frame (see 7.4.4.1). This |
|  |  |  | parameter is invalid if the KeyIdMode parameter is |
|  |  |  | invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key purportedly used by the |
|  |  |  | originator of the received frame (see 7.4.4.2). This |
|  |  |  | parameter is invalid if the KeyIdMode parameter is |
|  |  |  | invalid or set to 0x00. |
| DataRate | Enumeration | 6-bit field | The data rate of the PHY frame to be transmitted by |
|  |  |  | the PHY entity as shown in Table 118. |
| BurstMode | Boolean | TRUE or FALSE | The BurstMode bit shall be set TRUE if the burst |
|  |  |  | mode is being used (as discussed in 8.6.1); otherwise, |
|  |  |  | the BurstMode bit shall be set FALSE. |
| ColorReceived | Boolean | TRUE or FALSE | ColorReceived shall be set as TRUE, if CVD frame is |
|  |  |  | sent when data frame is successfully received. |
| ColorNotReceived | Boolean | TRUE or FALSE | ColorNotReceived shall be set as TRUE, if CVD |
|  |  |  | frame is sent when data frame is not received. |

6.2.4 MCPS-PURGE.request

- The MCPS-PURGE.request primitive allows the next higher layer to purge an MSDU from the transaction

- queue.

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

This primitive is optional for a device.

The semantics of the MCPS-PURGE.request primitive are as follows:

MCPS-PURGE.request	(

MsduHandle

)

Table 49 specifies the parameters for the MCPS-PURGE.request primitive.

Table 49—MCPS-PURGE.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MsduHandle | Integer | 0x00–0xff | The handle of the MSDU to be purged from |
|  |  |  | the transaction queue. |

6.2.4.1 Appropriate usage

The MCPS-PURGE.request primitive is generated by the next higher layer whenever a MSDU is to be purged from the transaction queue.

172	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.2.4.2 Effect on receipt

On receipt of the MCPS-PURGE.request primitive, the MAC sublayer attempts to find in its transaction queue the MSDU indicated by the MsduHandle parameter. If an MSDU has left the transaction queue, the handle will not be found, and the MSDU can no longer be purged. If an MSDU matching the given handle is found, the MSDU is discarded from the transaction queue, and the MAC sublayer issues the MCPS-PURGE.confirm primitive with a status of SUCCESS. If an MSDU matching the given handle is not found, the MAC sublayer issues the MCPS-PURGE.confirm primitive with a status of INVALID_HANDLE.

6.2.5 MCPS-PURGE.confirm

The MCPS-PURGE.confirm primitive allows the MAC sublayer to notify the next higher layer of the success of its request to purge an MSDU from the transaction queue.

This primitive is optional for a device.

The semantics of the MCPS-PURGE.confirm primitive are as follows:

MCPS-PURGE.confirm	(

MsduHandle,

status

)

Table 50 specifies the parameters for the MCPS-PURGE.confirm primitive.

Table 50—MCPS-PURGE.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MsduHandle | Integer | 0x00–0xff | The handle of the MSDU requested to be |
|  |  |  | purge from the transaction queue. |
| status | Enumeration | SUCCESS or | The status of the request to be purged an |
|  |  | INVALID_HANDLE | MSDU from the transaction queue. |

6.2.5.1 When generated

The MCPS-PURGE.confirm primitive is generated by the MAC sublayer entity in response to an MCPS-PURGE.request primitive. The MCPS-PURGE.confirm primitive returns a status of either SUCCESS, indicating that the purge request was successful, or INVALID_HANDLE, indicating an error. The status values are fully described in 6.2.5.2.

6.2.5.2 Appropriate usage

On receipt of the MCPS-PURGE.confirm primitive, the next higher layer is notified of the result of its request to purge an MSDU from the transaction queue. If the purge request was successful, the status parameter will be set to SUCCESS. Otherwise, the status parameter will indicate the error.

6.2.6 Data service message sequence chart

Figure 129 illustrates a sequence of messages necessary for a successful data transfer between two devices. Figure 150 and Figure 151 also illustrate this, including the steps taken by the PHY.

| Copyright © 20xx IEEE. All rights reserved. | 173 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 129—Message sequence chart describing the MAC data service

6.2.7 Primitives for heterogeneous network

As described in section xx, ACK frames from type 2 devices to a coordinator shall be send trough RF link first and then relayed to the coordinator through a wired backhaul link (power line, Ethernet, cable, etc.) in a heterogeneous network, which is a different way than it is sent through the VLC link. Therefore, particular primitives are needed so as to send the ACK frame successfully through the RF link and backhaul link.

MCPS- SAP ACK primitives defines how an ACK frame shall be transmitted in a heterogeneous network when a type 2 device has an ACK frame that needs to be sent to the coordinator by the RF link and the backhaul link. For ACK frames sent by type 1 devices and type 3 devices, MCPS-SAP DATA primitives shall be used.

6.2.7.1 MCPS-ACK.indication

The MCPS-ACK.indication primitive allows the MAC sublayer of the device to provide an ACK frame to its next higher layer.

The semantics of the MCPS-ACK.indication primitive are as follows:

MCPS-ACK.indication	(

DstAddr

SrcAddr

ACKFrame

)

Table 51 specifies the parameters for the MCPS-ACK.indication primitive.

174	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 51—MCPS-ACK.indication primitive

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
|  |  | From 0x0000-0xffff, 0x0000 rep- |  |  |
| DstAddr | Integer | resents without a short address | The short address of the coordinator |  |
|  |  | assigned, 0xffff represents a broad- | that the ACK frame is designated to. |  |
|  |  | cast transmission. |  |  |
|  |  | From 0x0000-0xffff, 0x0000 rep- |  |  |
| SrcAddr | Integer | resents without a short address | The short address of the device that |  |
|  |  | assigned, 0xffff represents a broad- | has generated the ACK frame. |  |
|  |  | cast transmission. |  |  |
| ACKFrame | Set of |  | The set of octets forming the ACK |  |
|  | octets |  | frames to be transmitted. |  |

6.2.7.1.1 When generated

When the MAC sublayer of the device generates an ACK frame that needs to be sent to the coordinator, it shall generate a MCPS-ACK.indication primitive and issue the MCPS-ACK.indication primitive to its next higher layer.

6.2.7.1.2 Effect on receipt

On receipt of the MCPS-ACK.indication primitive, the next higher layer of the device may generate a next higher layer message based on the provided arguments and send it to the next higher layer of the coordinator through the RF link and backhaul link. The ACK frame generated by the MAC sublayer shall be embedded in the next higher layer message.

6.2.7.2 MCPS-ACK.response

The MCPS-ACK.response primitive is used to initiate a response to an MCPS-ACK.indication primitive. The semantics of the MCPS-ACK.response primitive are as follows:

MCPS-ACK.response	(

Status,

)

Table 52—MCPS-ACK.response

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
|  |  | SUCCESS, |  |  |
| Status | Enumeration | CHANNEL ACCESS FAILURE, | The status of the uplink |  |
|  |  | NO ACKNOWLEDGEMENT, | transmission request. |  |
|  |  | TRANSACTION TIME EXPIRE |  |  |

| Copyright © 20xx IEEE. All rights reserved. | 175 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 6.2.7.2.1 When generated |
| --- | --- |
| 2 |  |

- The next higher layer of the device shall issue a MCPS-ACK.response primitive when it has finished the

- uplink transmission requested by the MCPS of the device, or when the transaction time of the uplink

- transmission has expired.

6

7	6.2.7.2.2 Effect on receipt

8

- The MCPS of the device shall be notified of the result of the requested uplink transmission through the RF

- link and backhaul link.

11

12	6.2.7.3 MCPS-ACK.request

13

- The MCPS-ACK.request primitive allows the next higher layer of the coordinator to request its MAC

- sublayer to process an ACK frame received through the backhaul link.

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

The semantics of the MCPS-ACK.request primitive are as follows:

MCPS-ACK.request	(

DstAddr

SrcAddr

ACKFrame

)

Table 53 specifies the parameters for the MCPS-ACK. request primitive.

Table 53—MCPS-ACK. request primitive

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
|  |  | From 0x0000-0xffff, 0x0000 rep- |  |  |
| DstAddr | Integer | resents without a short address | The short address of the coordinator that |  |
|  |  | assigned, 0xffff represents a broad- | the ACK frame is designated to. |  |
|  |  | cast transmission. |  |  |
|  |  | From 0x0000-0xffff, 0x0000 rep- |  |  |
| SrcAddr | Integer | resents without a short address | The short address of the device that has |  |
|  |  | assigned, 0xffff represents a broad- | generated the ACK frame. |  |
|  |  | cast transmission. |  |  |
| ACKFrame | Set of |  | The set of octets forming the ACK frames |  |
|  | octets |  | to be transmitted. |  |

6.2.7.3.1 When generated

- The next higher layer of the coordinator shall generate a MCPS-ACK.request primitive when it has received

- the next higher layer message through the backhaul link and issue the MCPS-ACK.request primitive to its

- MAC sublayer.

53

54

176	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.2.7.3.2 Effect on receipt

On receipt of the MCPS-ACK.request primitive, the MAC sublayer of the coordinator shall process the ACK frame provided in this primitive the same way as it was received through the VLC link.

6.2.7.4 MCPS-ACK.confirm

The MCPS-ACK.confirm primitive reports the process result of the received ACK frame.

The semantics of the MCPS-ACK.confirm primitive are as follows:

MCPS-ACK.confirm	(

Result,

)

Table 54 specifies the parameters for the MCPS-ACK.confirm primitive.

Table 54—parameters for the MCPS-ACK.confirm primitive

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
| Result | Enumer- | SUCCESS, | The process result of the received ACK frame. |  |
|  | ation | INVALID FRAME |  |  |

6.2.7.4.1 When generated

After the FCS re-calculation and the MAC frame header identification, the MCPS of the coordinator shall provide the process result of the received frame to its next higher layer by issuing an MCPS-ACK.response primitive.

6.2.7.4.2 Effect on receipt

The next higher layer of the coordinator is notified of the process result of the received ACK frame via the backhaul link.

6.3 MAC management s ervice

The MLME-SAP allows the transport of management commands between the next higher layer and the MLME. Table 55 summarizes the primitives supported by the MLME through the MLME -SAP interface. Primitives marked with a diamond () are optional for an RFD. Primitives marked with an asterisk (*) are optional for both device types (i.e., RFD and FFD). The primitives are discussed in the subclauses referenced in this table.

6.3.1 Association primitives

MLME-SAP association primitives define how a device becomes associated with a OWPAN.

All devices shall provide an interface for the request and confirm association primitives. The indication and response association primitives are optional for a device.

| Copyright © 20xx IEEE. All rights reserved. | 177 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

Table 55—Summary of the primitives accessed through the MLME-SAP

| Name | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| MLME-ASSOCIATE | 6.3.1.1 | 6.3.1.2 | 6.3.1.3 | 6.3.1.4 |
| MLME-DISASSOCIATE | 6.3.2.1 | 6.3.2.2 |  | 6.3.2.3 |
| MLME-BEACON-NOTIFY |  | 6.3.3.1 |  |  |
| MLME-GET | 6.3.5.1 |  |  | 6.3.5.2 |
| MLME-GTS | 6.3.6.1 | 6.3.6.2 |  | 6.3.6.3 |
| MLME-RESET | 6.3.7.1 |  |  | 6.3.7.2 |
| MLME-RX-ENABLE | 6.3.8.1 |  |  | 6.3.8.2 |
| MLME-SCAN | 6.3.9.1 |  |  | 6.3.9.2 |
| MLME-COMM-STATUS |  | 6.3.11.1 |  |  |
| MLME-SET | 6.3.12.1 |  |  | 6.3.12.2 |
| MLME-START | 6.3.13.1 |  |  | 6.3.13.2 |
| MLME-SYNC | 6.3.14.1* |  |  |  |
| MLME-SYNC-LOSS |  | 6.3.15.1 |  |  |
| MLME-POLL | 6.3.16.1 |  |  | 6.3.16.2 |

6.3.1.1 MLME-ASSOCIATE.request

The MLME-ASSOCIATE.request primitive allows a device to request an association with a coordinator.

The semantics of the MLME-ASSOCIATE.request primitive are as follows:

MLME-ASSOCIATE.request	(

,

CoordAddrMode,

CoordOWPANId,

CoordAddress,

CapabilityInformation,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

ColorAssoc

)

Table 56 specifies the parameters for the MLME-ASSOCIATE.request primitive.

6.3.1.1.1 Appropriate usage

- The MLME-ASSOCIATE.request primitive is generated by the next higher layer of an unassociated device

- and issued to its MLME to request an association with a OWPAN through a coordinator. If the device wishes

- to associate through a coordinator on a beacon-enabled OWPAN, the MLME may optionally track the

- beacon of that coordinator prior to issuing this primitive.

178	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 56—MLME-ASSOCIATE.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| CoordAddrMode | Integer | 0x02–0x03 | The coordinator addressing mode for this |
|  |  |  | primitive and subsequent MPDU. This value |
|  |  |  | can take one of the following values: |
|  |  |  | 2 = 16-bit short address. |
|  |  |  | 3 = 64-bit extended address. |
| CoordOWPANId | Integer | 0x0000–0xffff | The OWPAN identifier of the coordinator as |
|  |  |  | specified in the received beacon frame. |
| CoordAddress | Device | As specified by the | The address of the coordinator with which to |
|  | address | CoordAddrMode | associate. |
|  |  | parameter |  |
| CapabilityInformation | Bitmap | As defined in 5.4.26.1 | Specifies the operational capabilities of the |
|  |  |  | associating device. |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in |
|  |  |  | Table 97 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used |
|  |  |  | (as defined in Table 98 in 7.4.2.2). This |
|  |  |  | parameter is ignored if the SecurityLevel |
|  |  |  | parameter is set to 0x00. |
| KeySource | Set of 0, | As specified by the | The originator of the key to be used (see |
|  | 4, or 8 | KeyIdMode parameter | 7.4.4.1). This parameter is ignored if the |
|  | octets |  | KeyIdMode parameter is ignored or set to |
|  |  |  | 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2) . |
|  |  |  | This parameter is ignored if the KeyIdMode |
|  |  |  | parameter is ignored or set to 0x00. |
| ColorAssoc | Boolean | TRUE or FALSE | ColorAssoc shall be set as TRUE if the color |
|  |  |  | CVD frame is to be transmitted after the |
|  |  |  | association request command is sent. |

6.3.1.1.2 Effect on receipt

On receipt of the MLME-ASSOCIATE.request primitive, the MLME of an unassociated device first updates the appropriate PHY and MAC PIB attributes and then generates an association request command as shown in 5.4.1, as dictated by the association procedure described in 5.1.6.1.

The SecurityLevel parameter specifies the level of security to be applied to the association request command frame. Typically, the association request command should not be implemented using security. However, if the device requesting association shares a key with the coordinator, then security may be specified.

If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC sublayer will perform outgoing processing on the frame based on the CoordAddress, SecurityLevel, KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs during

| Copyright © 20xx IEEE. All rights reserved. | 179 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- outgoing frame processing, the MLME will discard the frame and issue the MLME-ASSOCIATE.confirm

- primitive with the error status returned by outgoing frame processing.

3

- If the association request command cannot be sent to the coordinator due to the unslotted random access

- algorithm indicating a busy channel, the MLME will issue the MLME-ASSOCIATE.confirm primitive with

- a status of CHANNEL_ACCESS_FAILURE.

7

8	If the MLME successfully transmits an association request command, the MLME will expect an

- acknowledgment in return. If an acknowledgment is not received, the MLME will issue the MLME-

- ASSOCIATE.confirm primitive with a status of NO_ACK (see 5.1.9.4).

11

- If the MLME of an unassociated device successfully receives an acknowledgment to its association request

- command, the MLME will wait for a response to the request (see 5.1.6.1). If the MLME of the device does

- not receive a response, it will issue the MLME-ASSOCIATE.confirm primitive with a status of NO_DATA.

- If the MLME of the device extracts an association response command frame from the coordinator, it will

- then issue the MLME-ASSOCIATE.confirm primitive with a status equal to the contents of the Association

- Status field in the association response command as shown in 5.4.2.3.

| 19 | On receipt of the association request command, the MLME of the coordinator issues the MLME- |  |
| --- | --- | --- |
| 20 |  |  |
| 21 | ASSOCIATE.indication primitive. |  |
| 22 |  |  |

- If any parameter in the MLME-ASSOCIATE.request primitive is either not supported or out of range, the

- MLME will issue the MLME-ASSOCIATE.confirm primitive with a status of INVALID_PARAMETER.

| 26 | 6.3.1.2 MLME-ASSOCIATE.indication |
| --- | --- |
| 27 |  |

- The MLME-ASSOCIATE.indication primitive is used to indicate the reception of an association request

- command.

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

The semantics of the MLME-ASSOCIATE.indication primitive are as follows:

MLME-ASSOCIATE.indication	(

DeviceAddress,

CapabilityInformation,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table 57 specifies the parameters for the MLME-ASSOCIATE.indication primitive.

Table 57—MLME-ASSOCIATE.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device | An extended 64-bit | The address of the device requesting |
|  | address | IEEE address | association. |
| CapabilityInformation | Bitmap | Refer to 5.4.26.1 | The operational capabilities of the device |
|  |  |  | requesting association. |

180	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 57—MLME-ASSOCIATE.indication parameters (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SecurityLevel | Integer | 0x00–0x07 | The security level purportedly used by the |
|  |  |  | received MAC command frame (as defined in |
|  |  |  | Table 97 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key purportedly |
|  |  |  | used by the originator of the received frame (as |
|  |  |  | defined in Table 98 in 7.4.2.2). This parameter |
|  |  |  | is invalid if the SecurityLevel parameter is set |
|  |  |  | to 0x00. |
| KeySource | Set of 0, | As specified by the | The originator of the key purportedly used by |
|  | 4, or 8 | KeyIdMode | the originator of the received frame (see |
|  | octets | parameter | 7.4.4.1). This parameter is invalid if the |
|  |  |  | KeyIdMode parameter is invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key purportedly used by the |
|  |  |  | originator of the received frame (see 7.4.4.2). |
|  |  |  | This parameter is invalid if the KeyIdMode |
|  |  |  | parameter is invalid or set to 0x00. |

6.3.1.2.1 When generated

The MLME-ASSOCIATE.indication primitive is generated by the MLME of the coordinator and issued to its next higher layer to indicate the reception of an association request command (as defined in 5.4.1).

6.3.1.2.2 Appropriate usage

When the next higher layer of a coordinator receives the MLME- ASSOCIATE.indication primitive, the coordinator determines whether to accept or reject the unassociated device using an algorithm outside the scope of this standard. The next higher layer of the coordinator then issues the MLME-ASSOCIATE.response primitive to its MLME.

The association decision and the response should become available at the coordinator within a time of macResponseWaitTime (see 5.1.6.1). After this time, the device requesting association attempts to extract the association response command frame from the coordinator, using the method described in 5.1.9.3, in order to determine whether the association was successful.

6.3.1.3 MLME-ASSOCIATE.response

The MLME-ASSOCIATE.response primitive is used to initiate a response to an MLME-ASSOCIATE.indication primitive.

| Copyright © 20xx IEEE. All rights reserved. | 181 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

The semantics of the MLME-ASSOCIATE.response primitive are as follows:

MLME-ASSOCIATE.response	(

DeviceAddress,

AssocShortAddress,

status,

CapabilityNegotiationResponse,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table 58 specifies the parameters for the MLME-ASSOCIATE.response primitive.

Table 58—MLME-ASSOCIATE.response parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device | An extended 64-bit | The address of the device requesting |
|  | address | IEEE address | association. |
| AssocShortAddress | Integer | 0x0000–0xffff | The 16-bit short device address allocated by |
|  |  |  | the coordinator on successful association. |
|  |  |  | This parameter is set to 0xffff if the |
|  |  |  | association was unsuccessful. |
| status | Enumeration | Refer to 5.4.2.3 | The status of the association attempt. |
| CapabilityNegotiatio | Integer | 00–11 | The coordinator indicates who will send |
| nResponse |  |  | color compensation information (same |
|  |  |  | definitions and usage as the color |
|  |  |  | stabilization scheme subfield in Table 42). |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in |
|  |  |  | Table 97 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be |
|  |  |  | used (as defined in Table 98 in 7.4.2.2). This |
|  |  |  | parameter is ignored if the SecurityLevel |
|  |  |  | parameter is set to 0x00. |
| KeySource | Set of 0, 4, or | As specified by the | The originator of the key to be used (see |
|  | 8 octets | KeyIdMode parameter | 7.4.4.1). This parameter is ignored if the |
|  |  |  | KeyIdMode parameter is ignored or set to |
|  |  |  | 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see |
|  |  |  | 7.4.4.2). This parameter is ignored if the |
|  |  |  | KeyIdMode parameter is ignored or set to |
|  |  |  | 0x00. |

6.3.1.3.1 Appropriate usage

- The MLME-ASSOCIATE.response primitive is generated by the next higher layer of a coordinator and

- issued to its MLME in order to respond to the MLME-ASSOCIATE.indication primitive.

54

182	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 6.3.1.3.2 Effect on receipt |  |  | 1 |  |
| --- | --- | --- | --- | --- |
| When the MLME of a coordinator receives the MLME-ASSOCIATE.response primitive, it generates an |  |  | 2 |  |
|  |  |  | 3 |  |
| association response command as shown in 5.4.2. The command frame is sent to the device requesting |  |  | 4 |  |
| association using indirect transmission, i.e., the command frame is added to the list of pending transactions |  |  | 5 |  |
| stored on the coordinator and extracted at the discretion of the device concerned using the method described |  |  | 6 |  |
| in 5.1.9.3. |  |  | 7 |  |
| If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for |  |  | 8 |  |
|  |  |  | 9 |  |
| this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC |  |  | 10 |  |
| sublayer will perform outgoing processing on the frame based the DeviceAddress, SecurityLevel, |  |  | 11 |  |
| KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs during |  |  | 12 |  |
| outgoing frame processing, the MLME will discard the frame and issue the MLME-COMM- |  |  | 13 |  |
| STATUS.indication primitive with the error status returned by outgoing frame processing. |  |  | 14 |  |
| Upon receipt of the MLME-ASSOCIATE.response primitive, the coordinator attempts to add the |  |  | 15 |  |
|  |  |  | 16 |  |
| information contained in the primitive to its list of pending transactions. If there is no capacity to store the |  |  | 17 |  |
| transaction, the MAC sublayer will discard the frame and issue the MLME-COMM-STATUS.indication |  |  | 18 |  |
| primitive with a status of TRANSACTION_OVERFLOW. If there is capacity to store the transaction, the |  |  | 19 |  |
| coordinator will add the information to the list. If the transaction is not handled within |  |  | 20 |  |
| macTransactionPersistenceTime, the transaction information will be discarded and the MAC sublayer will |  |  | 21 |  |
| issue the MLME-COMM-STATUS.indication primitive with a status of TRANSACTION_EXPIRED. The |  |  | 22 |  |
| transaction handling procedure is described in 5.1.8. |  |  | 23 |  |
| If the frame was successfully transmitted and an acknowledgment was received, if requested, the MAC |  |  | 24 |  |
|  |  |  | 25 |  |
| sublayer will issue the MLME-COMM-STATUS.indication primitive with a status of SUCCESS. |  |  | 26 |  |
| If any parameter in the MLME-ASSOCIATE.response primitive is not supported or is out of range, the |  |  | 27 |  |
|  |  |  | 28 |  |
| MAC sublayer will issue the MLME-COMM-STATUS.indication primitive with a |  | status of | 29 |  |
| INVALID_PARAMETER. |  |  | 30 |  |
| 6.3.1.4 MLME-ASSOCIATE.confirm |  |  | 31 |  |
|  |  |  | 32 |  |
| The MLME-ASSOCIATE.confirm primitive is used to inform the next higher layer of the initiating device |  |  | 33 |  |
|  |  |  | 34 |  |
| whether its request to associate was successful or unsuccessful. |  |  | 35 |  |
| The semantics of the MLME-ASSOCIATE.confirm primitive are as follows: |  |  | 36 |  |
|  |  |  | 37 |  |
|  |  |  | 38 |  |
| MLME-ASSOCIATE.confirm | ( |  | 39 |  |
|  | AssocShortAddress, |  | 40 |  |
|  | status, |  | 41 |  |
|  | CapabilityNegotiationResponse, |  | 42 |  |
|  | SecurityLevel, |  | 43 |  |
|  | KeyIdMode, |  | 44 |  |
|  | KeySource, |  | 45 |  |
|  | KeyIndex |  | 46 |  |
|  | ) |  | 47 |  |
|  |  |  | 48 |  |
| Table 59 specifies the parameters for the MLME-ASSOCIATE.confirm primitive. |  |  | 49 |  |
|  |  |  | 50 |  |
|  |  |  | 51 |  |
|  |  |  | 52 |  |
|  |  |  | 53 |  |
|  |  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. |  | 183 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 59—MLME-ASSOCIATE.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| AssocShortAddress | Integer | 0x0000–0xffff | The short device address allocated by |
|  |  |  | the coordinator on successful |
|  |  |  | association. This parameter will be |
|  |  |  | equal to 0xffff if the association |
|  |  |  | attempt was unsuccessful. |
| status | Enumeration | The value of the Status field of the | The status of the association attempt. |
|  |  | association response command |  |
|  |  | (as defined in 5.4.2.3), |  |
|  |  | SUCCESS, |  |
|  |  | CHANNEL_ACCESS_FAILURE, |  |
|  |  | NO_ACK, |  |
|  |  | NO_DATA, |  |
|  |  | COUNTER_ERROR, |  |
|  |  | FRAME_TOO_LONG, |  |
|  |  | IMPROPER_KEY_TYPE, |  |
|  |  | IMPROPER_SECURITY_LEVEL, |  |
|  |  | SECURITY_ERROR, |  |
|  |  | UNAVAILABLE_KEY, |  |
|  |  | UNSUPPORTED_LEGACY, |  |
|  |  | UNSUPPORTED_SECURITY |  |
|  |  | INVALID_PARAMETER |  |
| CapabilityNegotiati | Integer | 00–11 | Coordinator indicates who will send |
| onResponse |  |  | (see Table 42). |
| SecurityLevel | Integer | 0x00–0x07 | If the primitive was generated |
|  |  |  | following failed outgoing processing |
|  |  |  | of an association request command: |
|  |  |  | The security level to be used (as |
|  |  |  | defined in Table 97 in 7.4.2.1). |
|  |  |  | If the primitive was generated |
|  |  |  | following receipt of an association |
|  |  |  | response command: |
|  |  |  | The security level purportedly used |
|  |  |  | by the received frame (as defined in |
|  |  |  | Table 97 in 7.4.2.1). |

184	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 59—MLME-ASSOCIATE.confirm parameters (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| KeyIdMode | Integer | 0x00–0x03 | If the primitive was generated |
|  |  |  | following failed outgoing processing |
|  |  |  | of an association request command: |
|  |  |  | The mode used to identify the key to |
|  |  |  | be used (as defined in Table 98 in |
|  |  |  | 7.4.2.2). This parameter is ignored if |
|  |  |  | the SecurityLevel parameter is set to |
|  |  |  | 0x00. |
|  |  |  | If the primitive was generated |
|  |  |  | following receipt of an association |
|  |  |  | response command: |
|  |  |  | The mode used to identify the key |
|  |  |  | purportedly used by the originator of |
|  |  |  | the received frame (as defined in |
|  |  |  | Table 98 in 7.4.2.2). This parameter |
|  |  |  | is invalid if the SecurityLevel |
|  |  |  | parameter is set to 0x00. |
| KeySource | Set of 0, 4, | As specified by the KeyIdMode | If the primitive was generated |
|  | or 8 octets | parameter | following failed outgoing processing |
|  |  |  | of an association request command: |
|  |  |  | The originator of the key to be used |
|  |  |  | (see 7.4.4.1). This parameter is |
|  |  |  | ignored if the KeyIdMode parameter |
|  |  |  | is ignored or set to 0x00. |
|  |  |  | If the primitive was generated |
|  |  |  | following receipt of an association |
|  |  |  | response command: |
|  |  |  | The originator of the key purportedly |
|  |  |  | used by the originator of the received |
|  |  |  | frame (see 7.4.4.1). This parameter is |
|  |  |  | invalid if the KeyIdMode parameter |
|  |  |  | is invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | If the primitive was generated |
|  |  |  | following failed outgoing processing |
|  |  |  | of an association request command: |
|  |  |  | The index of the key to be used (see |
|  |  |  | 7.4.4.2). This parameter is ignored if |
|  |  |  | the KeyIdMode parameter is ignored |
|  |  |  | or set to 0x00. |
|  |  |  | If the primitive was generated |
|  |  |  | following receipt of an association |
|  |  |  | response command: |
|  |  |  | The index of the key purportedly |
|  |  |  | used by the originator of the received |
|  |  |  | frame (see 7.4.4.2). This parameter is |
|  |  |  | invalid if the KeyIdMode parameter |
|  |  |  | is invalid or set to 0x00. |

| Copyright © 20xx IEEE. All rights reserved. | 185 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 6.3.1.4.1 When generated |
| --- | --- |
| 2 |  |

- The MLME-ASSOCIATE.confirm primitive is generated by the initiating MLME and issued to its next

- higher layer in response to an MLME-ASSOCIATE.request primitive. If the request was successful, the

- status parameter will indicate a successful association, as contained in the Status field of the association

| 6 | response command. Otherwise, the status parameter indicates either an error code from the received |  |
| --- | --- | --- |
| 7 | association response command or the appropriate error code from Table 59. The status values are fully |  |
| 8 | described in 6.3.1.1.2 and subclauses referenced by 6.3.1.1.2. |  |
| 9 | 6.3.1.4.2 Appropriate usage |  |
| 10 |  |  |
| 11 |  |  |

- On receipt of the MLME-ASSOCIATE.confirm primitive, the next higher layer of the initiating device is

- notified of the result of its request to associate with a coordinator. If the association attempt was successful,

- the status parameter will indicate a successful association, as contained in the Status field of the association

- response command, and the device will be provided with a 16-bit short address as specified in Table 5 in

- 5.1.6.1. If the association attempt was unsuccessful, the address will be equal to 0xffff, and the status

- parameter will indicate the error.

18

19	6.3.1.5 Association-message sequence charts

20

- Figure 130 illustrates a sequence of messages that may be used by a device that is not tracking the beacon of

- the coordinator, specified in 5.1.9.3, to successfully associate with a OWPAN. Figure 146 and Figure 147,

- and described in 6.6, illustrate this same scenario, including steps taken by the PHY, for a device associating

- with a coordinator and for a coordinator allowing association by a device, respectively.

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Figure 130—Message sequence chart for association

6.3.2 Disassociation primitives

The MLME-SAP disassociation primitives define how a device can disassociate from a OWPAN.

All devices shall provide an interface for these disassociation primitives.

186	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.2.1 MLME-DISASSOCIATE.request

The MLME-DISASSOCIATE.request primitive is used by an associated device to notify the coordinator of its intent to leave the OWPAN. It is also used by the coordinator to instruct an associated device to leave the

OWPAN.

The semantics of the MLME-DISASSOCIATE.request primitive are as follows:

MLME-DISASSOCIATE.request	(

DeviceAddrMode,

DeviceOWPANId,

DeviceAddress,

DisassociateReason,

TxIndirect,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

ColorDisAssoc

)

Table 60 specifies the parameters for the MLME-DISASSOCIATE.request primitive.

Table 60—MLME-DISASSOCIATE.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddrMode | Integer | 0x02–0x03 | The addressing mode of the device to which |
|  |  |  | to send the disassociation notification |
|  |  |  | command. |
| DeviceOWPANId | Integer | 0x0000–0xffff | The OWPAN identifier of the device to which |
|  |  |  | to send the disassociation notification |
|  |  |  | command. |
| DeviceAddress | Device | As specified by the | The address of the device to which to send the |
|  | address | DeviceAddrMode | disassociation notification command. |
|  |  | parameter. |  |
| DisassociateReason | Integer | 0x00–0xff | The reason for the disassociation (as defined |
|  |  |  | in 5.4.3.2). |
| TxIndirect | Boolean | TRUE or FALSE | TRUE if the disassociation notification com- |
|  |  |  | mand is to be sent indirectly. |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in |
|  |  |  | Table 97 in 7.4.2.1). |

| Copyright © 20xx IEEE. All rights reserved. | 187 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  | Table 60—MLME-DISASSOCIATE.request parameters (continued) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  |  |  |  |  |  |  |
| 3 |  | Name |  | Type | Valid range | Description |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  | KeyIdMode |  | Integer | 0x00–0x03 | The mode used to identify the key to be used |  |
| 6 |  |  |  |  |  | (as defined in Table 98 in 7.4.2.2). This |  |
| 7 |  |  |  |  |  | parameter is ignored if the SecurityLevel |  |
| 8 |  |  |  |  |  | parameter is set to 0x00. |  |
| 9 |  | KeySource |  | Set of 0, 4, or | As specified by the | The originator of the key to be used (see |  |
| 10 |  |  |  | 8 octets | KeyIdMode parameter | 7.4.4.1). This parameter is ignored if the |  |
| 11 |  |  |  |  |  | KeyIdMode parameter is ignored or set to |  |
| 12 |  |  |  |  |  | 0x00. |  |
| 13 |  | KeyIndex |  | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). |  |
| 14 |  |  |  |  |  | This parameter is ignored if the KeyIdMode |  |
| 15 |  |  |  |  |  | parameter is ignored or set to 0x00. |  |
| 16 |  |  |  |  |  |  |  |
|  |  | ColorDisAssoc |  | Boolean | TRUE or FALSE | ColorDisAssoc shall be set as TRUE if the |  |
| 17 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | color CVD frame is to be transmitted after the |  |
| 18 |  |  |  |  |  | disassociation notification command is sent. |  |
| 19 |  |  |  |  |  |  |  |
| 20 | 6.3.2.1.1 Appropriate usage |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |

- The MLME-DISASSOCIATE.request primitive is generated by the next higher layer of an associated device

- and issued to its MLME to request disassociation from the OWPAN. It is also generated by the next higher

- layer of the coordinator and issued to its MLME to instruct an associated device to leave the OWPAN.

26

27	6.3.2.1.2 Effect on receipt

28

- On receipt of the MLME-DISASSOCIATE.request primitive, the MLME compares the DeviceOWPANId

- parameter with macOWPANId. If the DeviceOWPANId parameter is not equal to macOWPANId, the MLME

- issues the MLME-DISASSOCIATE.confirm primitive with a status of INVALID_PARAMETER. If the

- DeviceOWPANId parameter is equal to macOWPANId, the MLME evaluates the primitive address fields.

34	If the DeviceAddrMode parameter is equal to 0x02 and the DeviceAddress parameter is equal to

- macCoordShortAddress or if the DeviceAddrMode parameter is equal to 0x03 and the DeviceAddress

- parameter is equal to macCoordExtendedAddress, the TxIndirect parameter is ignored, and the MLME sends

- a disassociation notification command (see 5.4.3) to its coordinator in the CAP for a beacon-enabled

- OWPAN or immediately for a nonbeacon-enabled OWPAN.

39

- If the DeviceAddrMode parameter is equal to 0x02 and the DeviceAddress parameter is not equal to

- macCoordShortAddress or if the DeviceAddrMode parameter is equal to 0x03 and the DeviceAddress

- parameter is not equal to macCoordExtendedAddress, and if this primitive was received by the MLME of a

- coordinator with the TxIndirect parameter set to TRUE, the disassociation notification command will be

- sent using indirect transmission, i.e., the command frame is added to the list of pending transactions stored

- on the coordinator and extracted at the discretion of the device concerned using the method described in

- 5.1.9.3.

47

- If the DeviceAddrMode parameter is equal to 0x02 and the DeviceAddress parameter is not equal to

- macCoordShortAddress or if the DeviceAddrMode parameter is equal to 0x03 and the DeviceAddress

- parameter is not equal to macCoordExtendedAddress, and if this primitive was received by the MLME of a

- coordinator with the TxIndirect parameter set to FALSE, the MLME sends a disassociation notification

- command to the device in the CAP for a beacon-enabled OWPAN or immediately for a nonbeacon-enabled

- OWPAN.

54

188	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Otherwise, the MLME issues the MLME-DISASSOCIATE.confirm primitive with a status of |  |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
| INVALID_PARAMETER and does not generate a disassociation notification command. |  |  |  |  | 2 |  |
| If the disassociation notification command is to be sent using indirect transmission and there is no capacity |  |  |  |  | 3 |  |
|  |  |  |  |  | 4 |  |
| to store the transaction, the MLME will discard the frame and issue the MLME-DISASSOCIATE.confirm |  |  |  |  | 5 |  |
| primitive with a status of TRANSACTION_OVERFLOW. If there is capacity to store the transaction, the |  |  |  |  | 6 |  |
| coordinator will add the information to the list. If the transaction is not handled within macTransaction- |  |  |  |  | 7 |  |
| PersistenceTime, the transaction information will be discarded, and the MLME |  | will | issue | the | 8 |  |
| MLME-DISASSOCIATE.confirm with a status of TRANSACTION_EXPIRED. The transaction handling |  |  |  |  | 9 |  |
| procedure is described in 5.1.8. |  |  |  |  | 10 |  |
| If the disassociation notification command cannot be sent due to an unslotted random access algorithm |  |  |  |  | 11 |  |
|  |  |  |  |  | 12 |  |
| failure and this primitive was received either by the MLME of a coordinator with the TxIndirect parameter |  |  |  |  | 13 |  |
| set to FALSE or by the MLME of a device, the MLME will issue the MLME-DISASSOCIATE.confirm |  |  |  |  | 14 |  |
| primitive with a status of CHANNEL_ACCESS_FAILURE. |  |  |  |  | 15 |  |
| If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for |  |  |  |  | 16 |  |
|  |  |  |  |  | 17 |  |
| this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC |  |  |  |  | 18 |  |
| sublayer will perform outgoing processing on the frame based on the DeviceAddress, SecurityLevel, |  |  |  |  | 19 |  |
| KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs during |  |  |  |  | 20 |  |
| outgoing frame processing, the MLME will discard the frame and issue the MLME- |  |  |  |  | 21 |  |
| DISASSOCIATE.confirm primitive with the error status returned by outgoing frame processing. |  |  |  |  | 22 |  |
| If the MLME successfully transmits a disassociation notification command, the MLME will expect an |  |  |  |  | 23 |  |
|  |  |  |  |  | 24 |  |
| acknowledgment in return. If an acknowledgment is not received and this primitive was received either by |  |  |  |  | 25 |  |
| the MLME of a coordinator with the TxIndirect parameter set to FALSE or by the MLME of a device, the |  |  |  |  | 26 |  |
| MLME will issue the MLME-DISASSOCIATE.confirm primitive with a status of NO_ACK (see 5.1.9.4). |  |  |  |  | 27 |  |
| If the MLME successfully transmits a disassociation notification command and receives an |  |  |  |  | 28 |  |
|  |  |  |  |  | 29 |  |
| acknowledgment in return, the MLME will issue the MLME-DISASSOCIATE.confirm primitive with a |  |  |  |  | 30 |  |
| status of SUCCESS. |  |  |  |  | 31 |  |
| On receipt of the disassociation notification command, the MLME of the recipient issues the MLME- |  |  |  |  | 32 |  |
|  |  |  |  |  | 33 |  |
| DISASSOCIATE.indication primitive. |  |  |  |  | 34 |  |
| If any parameter in the MLME-DISASSOCIATE.request primitive is not supported or is out of range, the |  |  |  |  | 35 |  |
|  |  |  |  |  | 36 |  |
| MLME will issue the MLME-DISASSOCIATE.confirm primitive with |  | a | status | of | 37 |  |
| INVALID_PARAMETER. |  |  |  |  | 38 |  |
| 6.3.2.2 MLME-DISASSOCIATE.indication |  |  |  |  | 39 |  |
|  |  |  |  |  | 40 |  |
| The MLME-DISASSOCIATE.indication primitive is used to indicate the reception of a disassociation |  |  |  |  | 41 |  |
|  |  |  |  |  | 42 |  |
| notification command. |  |  |  |  | 43 |  |
| The semantics of the MLME-DISASSOCIATE.indication primitive are as follows: |  |  |  |  | 44 |  |
|  |  |  |  |  | 45 |  |
| MLME-DISASSOCIATE.indication | ( |  |  |  | 46 |  |
|  |  |  |  |  | 47 |  |
|  | DeviceAddress, |  |  |  | 48 |  |
|  | DisassociateReason, |  |  |  | 49 |  |
|  | SecurityLevel, |  |  |  | 50 |  |
|  | KeyIdMode, |  |  |  | 51 |  |
|  | KeySource, |  |  |  | 52 |  |
|  | KeyIndex |  |  |  | 53 |  |
|  | ) |  |  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. |  |  |  | 189 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |  |

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 61 specifies the parameters for the MLME-DISASSOCIATE.indication primitive.

Table 61—MLME-DISASSOCIATE.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device | An extended 64-bit | The address of the device requesting |
|  | address | IEEE address | disassociation. |
| DisassociateReason | Integer | 0x00–0xff | The reason for the disassociation (as defined |
|  |  |  | in 5.4.3.2). |
| SecurityLevel | Integer | 0x00–0x07 | The security level purportedly used by the |
|  |  |  | received MAC command frame (as defined in |
|  |  |  | Table 97 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key |
|  |  |  | purportedly used by the originator of the |
|  |  |  | received frame (as defined in Table 98 in |
|  |  |  | 7.4.2.2). This parameter is invalid if the |
|  |  |  | SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or | As specified by the | The originator of the key purportedly used by |
|  | 8 octets | KeyIdMode parameter | the originator of the received frame (see |
|  |  |  | 7.4.4.1). This parameter is invalid if the |
|  |  |  | KeyIdMode parameter is invalid or set to |
|  |  |  | 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key purportedly used by the |
|  |  |  | originator of the received frame (see 7.4.4.2). |
|  |  |  | This parameter is invalid if the KeyIdMode |
|  |  |  | parameter is invalid or set to 0x00. |

6.3.2.2.1 When generated

The MLME-DISASSOCIATE.indication primitive is generated by the MLME and issued to its next higher layer on receipt of a disassociation notification command.

6.3.2.2.2 Appropriate usage

The next higher layer is notified of the reason for the disassociation.

6.3.2.3 MLME-DISASSOCIATE.confirm

The MLME-DISASSOCIATE.confirm primitive reports the results of an MLME-DISASSOCIATE.request primitive.

The semantics of the MLME-DISASSOCIATE.confirm primitive are as follows:

MLME-DISASSOCIATE.confirm ( status,

DeviceAddrMode,

DeviceOWPANId, DeviceAddress

)

190	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 62 specifies the parameters for the MLME-DISASSOCIATE.confirm primitive.

Table 62—MLME-DISASSOCIATE.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, | The status of the disassociation |
|  |  | TRANSACTION_OVERFLOW, | attempt. |
|  |  | TRANSACTION_EXPIRED, |  |
|  |  | NO_ACK, |  |
|  |  | CHANNEL_ACCESS_FAILURE, |  |
|  |  | COUNTER_ERROR, |  |
|  |  | FRAME_TOO_LONG, |  |
|  |  | UNAVAILABLE_KEY, |  |
|  |  | UNSUPPORTED_SECURITY, |  |
|  |  | INVALID_PARAMETER |  |
| DeviceAddrMode | Integer | 0x02–0x03 | The addressing mode of the device |
|  |  |  | that has either requested |
|  |  |  | disassociation or been instructed to |
|  |  |  | disassociate by its coordinator. |
| DeviceOWPANId | Integer | 0x0000–0xffff | The OWPAN identifier of the device |
|  |  |  | that has either requested |
|  |  |  | disassociation or been instructed to |
|  |  |  | disassociate by its coordinator. |
| DeviceAddress | Device | As specified by the | The address of the device that has |
|  | address | DeviceAddrMode parameter. | either requested disassociation or |
|  |  |  | been instructed to disassociate by its |
|  |  |  | coordinator. |

6.3.2.3.1 When generated

The MLME-DISASSOCIATE.confirm primitive is generated by the initiating MLME and issued to its next higher layer in response to an MLME -DISASSOCIATE.request primitive. This primitive returns a status of either SUCCESS, indicating that the disassociation request was successful, or the appropriate error code. The status values are fully described in 6.3.2.1.2 and subclauses referenced by 6.3.2.1.2.

6.3.2.3.2 Appropriate usage

On receipt of the MLME-DISASSOCIATE.confirm primitive, the next higher layer of the initiating device is notified of the result of the disassociation attempt. If the disassociation attempt was successful, the status parameter will be set to SUCCESS. Otherwise, the status parameter indicates the error.

| Copyright © 20xx IEEE. All rights reserved. | 191 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 6.3.2.4 Disassociation-message sequence charts |
| --- | --- |
| 2 |  |

- The request to disassociate may originate either from a device or from the coordinator through which the

- device has associated. Figure 131 illustrates the sequence of messages necessary for a device to successfully

- disassociate itself from the OWPAN.

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Figure 131—Message sequence chart for disassociation initiated by a device

Figure 132 illustrates the sequence necessary for a coordinator in a beacon-enabled OWPAN to successfully disassociate a device from its OWPAN using indirect transmission.

6.3.3 Beacon notification primitive

The MLME-SAP beacon notification primitive defines how a device may be notified when a beacon is received during normal operating conditions.

All devices shall provide an interface for the beacon notification primitive.

6.3.3.1 MLME-BEACON-NOTIFY.indication

The MLME-BEACON-NOTIFY.indication primitive is used to send parameters contained within a beacon frame received by the MAC sublayer to the next higher layer. The primitive also sends a measure of the WQI and the time the beacon frame was received.

The semantics of the MLME-BEACON-NOTIFY.indication primitive are as follows:

MLME-BEACON-NOTIFY.indication ( BSN,

OWPANDescriptor,

PendAddrSpec,

AddrList,

sduLength, sdu

)

Table 63 specifies the parameters for the MLME-BEACON-NOTIFY.indication primitive.

192	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 132—Message sequence chart for disassociation initiated by a coordinator, using indirect transmission, in a beacon-enabled OWPAN

Table 63—MLME-BEACON-NOTIFY.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| BSN | Integer | 0x00–0xff | The beacon sequence number. |
| OWPANDescript | OWPANDescript | Refer to Table 64 | The OWPANDescriptor for the |
| or | or value |  | received beacon. |
| PendAddrSpec | Bitmap | Refer to 5.2.2.1.8 | The beacon pending address |
|  |  |  | specification. |
| AddrList | List of device | — | The list of addresses of the devices for |
|  | addresses |  | which the beacon source has data. |
| sduLength | Integer | 0 – aMaxBeaconPayloadLength | The number of octets contained in the |
|  |  |  | beacon payload of the beacon frame |
|  |  |  | received by the MAC sublayer. |
| sdu | Set of octets | — | The set of octets comprising the beacon |
|  |  |  | payload to be transferred from the MAC |
|  |  |  | sublayer entity to the next higher layer. |

| Copyright © 20xx IEEE. All rights reserved. | 193 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 64 describes the elements of the OWPANDescriptor type.

Table 64—Elements of OWPAN descriptor

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| CoordAddrMode | Integer | 0x02–0x03 | The coordinator addressing mode |
|  |  |  | corresponding to the received beacon frame. |
|  |  |  | This value can take one of the following |
|  |  |  | values: |
|  |  |  | 2 = 16-bit short address. |
|  |  |  | 3 = 64-bit extended address. |
| CoordOWPANId | Integer | 0x0000–0xffff | The OWPAN identifier of the coordinator as |
|  |  |  | specified in the received beacon frame. |
| CoordAddress | Device | As specified by the | The address of the coordinator as specified in |
|  | address | CoordAddrMode parameter | the received beacon frame. |
| SuperframeSpec | Bitmap | Refer to 5.2.2.1.2 | The superframe specification as specified in the |
|  |  |  | received beacon frame. |
| GTSPermit | Boolean | TRUE or FALSE | TRUE if the beacon is from the coordinator |
|  |  |  | that is accepting GTS requests. |
| LinkQuality | Integer | 0x00–0xff | The WQI at which the network beacon was |
|  |  |  | received. Lower values represent lower WQI |
|  |  |  | (see 5.4.26.2). |
| TimeStamp | Integer | 0x000000–0xffffff | The time at which the beacon frame was |
|  |  |  | received, in symbols. This value is equal to the |
|  |  |  | timestamp taken when the beacon frame was |
|  |  |  | received, as described in 5.1.7.1. |
|  |  |  | This is a 24-bit value, and the precision of this |
|  |  |  | value shall be a minimum of 20 bits, with the |
|  |  |  | lowest 4 bits being the least significant. |
| SecurityFailure | Enumer- | SUCCESS, | SUCCESS if there was no error in the security |
|  | ation | COUNTER_ERROR, | processing of the frame. One of the other status |
|  |  | IMPROPER_KEY_TYPE, | codes indicating an error in the security |
|  |  | IMPROPER_SECURITY_LE | processing otherwise (see 7.2.3). |
|  |  | VEL, SECURITY_ERROR, |  |
|  |  | UNAVAILABLE_KEY, |  |
|  |  | UNSUPPORTED_LEGACY, |  |
|  |  | UNSUPPORTED_SECURITY |  |
| SecurityLevel | Integer | 0x00–0x07 | The security level purportedly used by the |
|  |  |  | received beacon frame (as defined in Table 97 |
|  |  |  | in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key purportedly |
|  |  |  | used by the originator of the received frame (as |
|  |  |  | defined in Table 98 in 7.4.2.2). This parameter |
|  |  |  | is invalid if the SecurityLevel parameter is set |
|  |  |  | to 0x00. |

194	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 64—Elements of OWPAN descriptor (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| KeySource | Set of 0, | As specified by the | The originator of the key purportedly used by |
|  | 4, or 8 | KeyIdMode parameter | the originator of the received frame (see |
|  | octets |  | 7.4.4.1). This parameter is invalid if the |
|  |  |  | KeyIdMode parameter is invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key purportedly used by the |
|  |  |  | originator of the received frame (see 7.4.4.2). |
|  |  |  | This parameter is invalid if the KeyIdMode |
|  |  |  | parameter is invalid or set to 0x00. |

6.3.3.1.1 When generated

The MLME -BEACON-NOTIFY.indication primitive is generated by the MLME and issued to its next higher layer upon receipt of a beacon frame either when macAutoRequest is set to FALSE or when the beacon frame contains one or more octets of payload.

6.3.3.1.2 Appropriate usage

On receipt of the MLME-BEACON- NOTIFY.indication primitive, the next higher layer is notified of the arrival of a beacon frame at the MAC sublayer.

6.3.4 Additional beacon primitives

MLME-SAP additional beacon primitive defines how an unassociated device request coordinators to send additional beacons before initiating an association with a OWPAN.

6.3.4.1 MLME-AdditionalBeacon.requet

The MLME-AdditionalBeacon.request primitive allows an unassociated device to generate and transmit an additional beacon frame request command.

The semantics of the MLME-AdditionalBeacon.request primitive are as follows:

MLME-AdditionalBeacon.request (

Reason

)

Table 65—MLME-AdditionalBeacon.request primitive

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
| Reason | Integer | 0x00 - 0xff | The reason for the additional beacon |  |
|  |  |  | request (as defined in Table 34). |  |

| Copyright © 20xx IEEE. All rights reserved. | 195 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 6.3.4.1.1 When generated |
| --- | --- |
| 2 |  |

- The MLME-AdditionalBeacon.request primitive is generated by the next higher layer of an unassociated

- device when the next higher layer of the unassociated device has obtained the result of a requested passive

- scan which indicates no beacon frames has been detected.

6

7	6.3.4.1.2 Effect on receipt

8

- The MLME of the unassociated device shall generate and transmit an additional beacon request command,

- then it shall set a timer and enable its receiver to receive beacon frames.

11

12	6.3.4.1.3 MLME-AdditionalBeacon.indication

13

- The MLME-AdditionalBeacon.indication primitive is used to indicate the reception of an additional beacon

- request command.

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

The semantics of the MLME-AdditionalBeacon.indication primitive are as follows:

MLME-AdditionalBeacon.indication (

Reason

DeviceAddr

)

Table 66 specifies the parameters for the MLME-AdditionalBeacon.indication primitive.

Table 66—MLME-AdditionalBeacon.indication primitive

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |

| Reason | Integer | 0x00 - 0xff | The reason for the additional beacon |  |
| --- | --- | --- | --- | --- |
|  |  |  | request (as defined in Table 34). |  |
| DeviceAddr | Device | 64 bit extended address | The address of the device sending this |  |
|  | address |  | additional beacon request. |  |

6.3.4.1.4 When generated

The MLME-AdditionalBeacon.indication primitive is generated by the MLME of a coordinator and issued to its next higher layer to indicate the reception of an additional beacon request command received by the PHY.

6.3.4.1.5 Effect on receipt

On receipt of the MLME-AdditionalBeacon.indication primitive, the next higher layer of the coordinator is notified of the additional beacon request command sent by an unassociated device. The next higher layer may then allocate a GTS for additional beacon transmission.

196	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.5 Primitives for reading PIB attributes

The MLME-SAP get primitives define how to read values from the PIB.

All devices shall provide an interface for these get primitives.

6.3.5.1 MLME-GET.request

The MLME-GET.request primitive requests information about a given PIB attribute. The semantics of the MLME-GET.request primitive are as follows:

MLME-GET.request ( PIBAttribute,

PIBAttributeIndex

)

Table 67 specifies the parameters for the MLME-GET.request primitive.

Table 67—MLME-GET.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PIBAttribute | Integer | Refer to Table 91 | The identifier of the PIB attribute to read. |
| PIBAttributeIndex | Integer | Attribute specific; | The index within the table of the specified PIB |
|  |  | as defined in Table 91 | attribute to read. This parameter is valid only |
|  |  |  | for MAC PIB attributes that are tables; it is |
|  |  |  | ignored when accessing PHY PIB attributes. |

6.3.5.1.1 Appropriate usage

The MLME -GET.request primitive is generated by the next higher layer and issued to its MLME to obtain information from the PIB.

6.3.5.1.2 Effect on receipt

On receipt of the MLME-GET.request primitive, the MLME checks to see if the PIB attribute is a MAC PIB attribute or PHY PIB attribute. If the requested attribute is a MAC attribute, the MLME attempts to retrieve the requested MAC PIB attribute from its database. If the identifier of the PIB attribute is not found in the database, the MLME will issue the MLME-GET.confirm primitive with a status of UNSUPPORTED_ATTRIBUTE. If the PIBAttributeIndex parameter specifies an index for a table that is out of range, the MLME will issue the MLME-GET.confirm primitive with a status of INVALID_INDEX. If the requested MAC PIB attribute is successfully retrieved, the MLME will issue the MLME-GET.confirm primitive with a status of SUCCESS.

If the requested attribute is a PHY PIB attribute, the request is passed to the PHY by issuing the PLME-GET.request primitive. Once the MLME receives the PLME-GET.confirm primitive, it will translate the received status value because the status values used by the PHY are not the same as those used by the MLME (e.g., the status values for SUCCESS are 0x00 and 0x07 in the MAC and PHY enumeration tables, respectively). Following the translation, the MLME will issue the MLME-GET.confirm primitive to the next

| Copyright © 20xx IEEE. All rights reserved. | 197 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

higher layer with the status parameter resulting from the translation and the PIBAttribute and PIBAttributeValue parameters equal to those returned by the PLME primitive.

6.3.5.2 MLME-GET.confirm

The MLME-GET.confirm primitive reports the results of an information request from the PIB.

The semantics of the MLME-GET.confirm primitive are as follows:

MLME-GET.confirm	(

status,

PIBAttribute,

PIBAttributeIndex,

PIBAttributeValue

)

Table 68 specifies the parameters for the MLME-GET.confirm primitive.

Table 68—MLME-GET.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, | The result of the request for PIB |
|  |  | UNSUPPORTED_ATTRIBUTE | attribute information. |
|  |  | or INVALID_INDEX |  |
| PIBAttribute | Integer | Refer to Table 91 | The identifier of the PIB attribute that |
|  |  |  | was read. |
| PIBAttributeIndex | Integer | Attribute specific; | The index within the table or array of |
|  |  | as defined in Table 91 | the specified PIB attribute to read. This |
|  |  |  | parameter is valid only for MAC PIB |
|  |  |  | attributes that are tables or arrays; it is |
|  |  |  | ignored when accessing PHY PIB |
|  |  |  | attributes. |
| PIBAttributeValue | Various | Attribute specific; | The value of the indicated PIB attribute |
|  |  | as defined in Table 91 | that was read. |
|  |  |  | This parameter has zero length when |
|  |  |  | the status parameter is set to |
|  |  |  | UNSUPPORTED_ATTRIBUTE. |

6.3.5.2.1 When generated

The MLME-GET.confirm primitive is generated by the MLME and issued to its next higher layer in response to an MLME- GET.request primitive. This primitive returns a status of either SUCCESS, indicating that the request to read a PIB attribute was successful, or an error code of UNSUPPORTED_ATTRIBUTE. When an error code of UNSUPPORTED_ATTRIBUTE is returned, the PIBAttribute value parameter will be set to length zero. The status values are fully described in 6.3.5.1.2.

198	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.5.2.2 Appropriate usage

On receipt of the MLME -GET.confirm primitive, the next higher layer is notified of the results of its request to read a PIB attribute. If the request to read a PIB attribute was successful, the status parameter will be set to SUCCESS. Otherwise, the status parameter indicates the error.

6.3.6 GTS management primitives

The MLME-SAP GTS management primitives define how GTSs are requested and maintained. A device wishing to use these primitives and GTSs in general will already be tracking the beacons of its coordinator.

These GTS management primitives are optional.

6.3.6.1 MLME-GTS.request

The MLME-GTS.request primitive allows a device to send a request to the coordinator to allocate a new GTS or to deallocate an existing GTS. This primitive is also used by the coordinator to initiate a GTS deallocation.

The semantics of the MLME-GTS.request primitive are as follows:

MLME-GTS.request	(

GTSCharacteristics,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table 69 specifies the parameters for the MLME-GTS.request primitive.

Table 69—MLME-GTS.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| GTSCharacteristics | GTS | Refer to 5.4.18.2 | The characteristics of the GTS request, including |
|  | characteristics |  | whether the request is for the allocation of a new |
|  |  |  | GTS or the deallocation of an existing GTS. |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in Table 97 |
|  |  |  | in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (as |
|  |  |  | defined in Table 98 in 7.4.2.2). This parameter is |
|  |  |  | ignored if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or | As specified by | The originator of the key to be used (see 7.4.4.1). |
|  | 8 octets | the KeyIdMode | This parameter is ignored if the KeyIdMode |
|  |  | parameter | parameter is ignored or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). This |
|  |  |  | parameter is ignored if the KeyIdMode parameter is |
|  |  |  | ignored or set to 0x00. |

| Copyright © 20xx IEEE. All rights reserved. | 199 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 6.3.6.1.1 Appropriate usage |
| --- | --- |
| 2 |  |

- The MLME-GTS.request primitive is generated by the next higher layer of a device and issued to its MLME

- to request the allocation of a new GTS or to request the deallocation of an existing GTS. It is also generated

- by the next higher layer of the coordinator and issued to its MLME to request the deallocation of an existing

- GTS.

| 7 | 6.3.6.1.2 Effect on receipt |  |
| --- | --- | --- |
| 8 |  |  |
| 9 | On receipt of the MLME-GTS.request primitive by a device, the MLME of a device attempts to generate a |  |
| 10 |  |  |
| 11 | GTS request command, specified in 5.4.18, with the information contained in this primitive and, if |  |
| 12 | successful, sends it to the coordinator. |  |
| 13 |  |  |

- If macShortAddress is equal to 0xfffe or 0xffff, the device is not permitted to request a GTS. In this case, the

- MLME issues the MLME-GTS.confirm primitive containing a status of NO_SHORT_ADDRESS.

16

- If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for

- this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC

- sublayer will perform outgoing processing on the frame based on macCoordExtendedAddress and the

- SecurityLevel, KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs

- during outgoing frame processing, the MLME will discard the frame and issue the MLME-GTS.confirm

- primitive with the error status returned by outgoing frame processing.

23

- If the GTS request command cannot be sent due to an unslotted random access algorithm failure, the MLME

- will issue the MLME-GTS.confirm primitive with a status of CHANNEL_ACCESS_FAILURE.

26

- If the MLME successfully transmits a GTS request command, the MLME will expect an acknowledgment in

- return. If an acknowledgment is not received, the MLME will issue the MLME-GTS.confirm primitive with

- a status of NO_ACK (see 5.1.9.4).

30

- If a GTS is being allocated (see 5.1.10.2) and the request has been acknowledged, the device will wait for a

- confirmation via a GTS descriptor specified in a beacon frame from its coordinator. If the MLME of the

- coordinator can allocate the requested GTS, it will issue the MLME-GTS.indication primitive with the

- characteristics of the allocated GTS and generate a GTS descriptor with the characteristics of the allocated

- GTS and the 16-bit short address of the requesting device. If the MLME of the coordinator cannot allocate

- the requested GTS, it will generate a GTS descriptor with a start slot of zero and the short address of the

- requesting device.

38

- If the device receives a beacon frame from its coordinator with a GTS descriptor containing a 16-bit short

- address that matches macShortAddress, the device will process the descriptor. If no descriptor for that device

- is received, the MLME will issue the MLME-GTS.confirm primitive with a status of NO_DATA.

42

- If a descriptor is received that matches the characteristics requested (indicating that the coordinator has

- approved the GTS allocation request), the MLME of the device will issue the MLME-GTS.confirm

- primitive with a status of SUCCESS and a GTSCharacteristics parameter with a characteristics type equal to

- one, indicating a GTS allocation.

47

- If the descriptor is received with a start slot of zero (indicating that the coordinator has denied the GTS

- allocation request), the device requesting the GTS issues the MLME-GTS.confirm primitive with a status of

- DENIED, indicating that the GTSCharacteristics parameter is to be ignored.

51

- If a GTS is being deallocated (see 5.1.10.4) at the request of a device and the request has been acknowledged

- by the coordinator, the device will issue the MLME-GTS.confirm primitive with a status of SUCCESS and a

- GTSCharacteristics parameter with a characteristics type equal to zero, indicating a GTS deallocation. On

200	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

receipt of a GTS request command with a request type indicating a GTS deallocation, the coordinator will acknowledge the frame and deallocates the GTS. The MLME of the coordinator will then issue the MLME-GTS.indication primitive with the appropriate GTS characteristics. If the coordinator does not receive the deallocation request, countermeasures can be applied by the coordinator to ensure consistency is maintained as discussed in 5.1.10.6.

If the MLME of the coordinator receives an MLME-GTS.request primitive indicating deallocation, the coordinator will deallocate the GTS and issue the MLME- GTS.confirm primitive with a status of SUCCESS and a GTSCharacteristics parameter with a characteristics type equal to zero.

If the device receives a beacon frame from its coordinator with a GTS descriptor containing a short address that matches macShortAddress and a start slot equal to zero, the device immediately stops using the GTS. The MLME of the device then notifies the next higher layer of the deallocation by issuing the MLME-GTS.indication primitive with a GTSCharacteristics parameter containing the characteristics of the deallocated GTS.

If any parameter in the MLME-GTS.request primitive is not supported or is out of range, the MLME will issue the MLME-GTS.confirm primitive with a status of INVALID_PARAMETER.

6.3.6.2 MLME-GTS.indication

The MLME-GTS.indication primitive indicates that a GTS has been allocated or that a previously allocated GTS has been deallocated.

The semantics of the MLME-GTS.indication primitive are as follows:

MLME-GTS.indication	(

DeviceAddress,

GTSCharacteristics,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table 70 specifies the parameters for the MLME-GTS.indication primitive.

Table 70—MLME-GTS.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device address | 0x0000–0xfffd | The 16-bit short address of the device that |
|  |  |  | has been allocated or deallocated a GTS. |
| GTSCharacteristics | GTS | Refer to 5.4.18.2 | The characteristics of the GTS. |
|  | characteristics |  |  |

| Copyright © 20xx IEEE. All rights reserved. | 201 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 70—MLME-GTS.indication parameters (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SecurityLevel | Integer | 0x00–0x07 | If the primitive was generated when a GTS |
|  |  |  | deallocation is initiated by the coordinator |
|  |  |  | itself, the security level to be used is set to |
|  |  |  | 0x00. |
|  |  |  | If the primitive was generated whenever a |
|  |  |  | GTS is allocated or deallocated following |
|  |  |  | the reception of a GTS request command: |
|  |  |  | The security level purportedly used by the |
|  |  |  | received MAC command frame (as defined |
|  |  |  | in Table 97 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | If the primitive was generated when a GTS |
|  |  |  | deallocation is initiated by the coordinator |
|  |  |  | itself, this parameter is ignored. |
|  |  |  | If the primitive was generated whenever a |
|  |  |  | GTS is allocated or deallocated following |
|  |  |  | the reception of a GTS request command: |
|  |  |  | The mode used to identify the key |
|  |  |  | purportedly used by the originator of the |
|  |  |  | received frame (as defined in Table 98 in |
|  |  |  | 7.4.2.2). This parameter is invalid if the |
|  |  |  | SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 | As specified by the | If the primitive was generated when a GTS |
|  | octets | KeyIdMode parameter | deallocation is initiated by the coordinator |
|  |  |  | itself, this parameter is ignored. |
|  |  |  | If the primitive was generated whenever a |
|  |  |  | GTS is allocated or deallocated following |
|  |  |  | the reception of a GTS request command: |
|  |  |  | The originator of the key purportedly used |
|  |  |  | by the originator of the received frame (see |
|  |  |  | 7.4.4.1). This parameter is invalid if the |
|  |  |  | KeyIdMode parameter is invalid or set to |
|  |  |  | 0x00. |
| KeyIndex | Integer | 0x01–0xff | If the primitive was generated when a GTS |
|  |  |  | deallocation is initiated by the coordinator |
|  |  |  | itself, this parameter is ignored. |
|  |  |  | If the primitive was generated whenever a |
|  |  |  | GTS is allocated or deallocated following |
|  |  |  | the reception of a GTS request command: |
|  |  |  | The index of the key purportedly used by the |
|  |  |  | originator of the received frame (see |
|  |  |  | 7.4.4.2). This parameter is invalid if the |
|  |  |  | KeyIdMode parameter is invalid or set to |
|  |  |  | 0x00. |

202	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.6.2.1 When generated

The MLME -GTS.indication primitive is generated by the MLME of the coordinator to its next higher layer whenever a GTS is allocated or deallocated following the reception of a GTS request command by the MLME as discussed in 5.4.18. The MLME of the coordinator also generates this primitive when a GTS deallocation is initiated by the coordinator itself. The Characteristics Type field in the GTSCharacteristics parameter will be equal to one if a GTS has been allocated or zero if a GTS has been deallocated.

This primitive is generated by the MLME of a device and issued to its next higher layer when the coordinator has deallocated one of its GTSs. In this case, the Characteristics Type field of the GTSCharacteristics parameter is equal to zero.

6.3.6.2.2 Appropriate usage

On receipt of the MLME-GTS.indication primitive the next higher layer is notified of the allocation or de-allocation of a GTS.

6.3.6.3 MLME-GTS.confirm

The MLME-GTS.confirm primitive reports the results of a request to allocate a new GTS or deallocate an existing GTS.

The semantics of the MLME-GTS.confirm primitive are as follows:

MLME-GTS.confirm ( GTSCharacteristics, status

)

Table 71 specifies the parameters for the MLME-GTS.confirm primitive.

Table 71—MLME-GTS.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| GTSCharacteristics | GTS | Refer to 5.4.18.2 | The characteristics of the GTS. |
|  | characteristics |  |  |
| status | Enumeration | SUCCESS, | The status of the GTS request. |
|  |  | DENIED, |  |
|  |  | NO_SHORT_ADDRESS, |  |
|  |  | CHANNEL_ACCESS_FAILURE, |  |
|  |  | NO_ACK, |  |
|  |  | NO_DATA, |  |
|  |  | COUNTER_ERROR, |  |
|  |  | FRAME_TOO_LONG, |  |
|  |  | UNAVAILABLE_KEY, |  |
|  |  | UNSUPPORTED_SECURITY, |  |
|  |  | INVALID_PARAMETER |  |

6.3.6.3.1 When generated

The MLME- GTS.confirm primitive is generated by the MLME and issued to its next higher layer in response to a previously issued MLME-GTS.request primitive.

| Copyright © 20xx IEEE. All rights reserved. | 203 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- If the request to allocate or deallocate a GTS was successful, this primitive will return a status of SUCCESS

- and the Characteristics Type field of the GTSCharacteristics parameter will have the value of one or zero,

- respectively. Otherwise, the status parameter will indicate the appropriate error code. The reasons for these

- status values are fully described in 6.3.6.1.2 and subclauses referenced by 6.3.6.1.2.

5

6	6.3.6.3.2 Appropriate usage

7

- On receipt of the MLME-GTS.confirm primitive the next higher layer is notified of the result of its request

- to allocate or deallocate a GTS. If the request was successful, the status parameter will indicate a successful

- GTS operation. Otherwise, the status parameter will indicate the error.

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

6.3.6.4 GTS management message sequence charts

Figure 133 and Figure 134 illustrate the sequence of messages necessary for successful GTS management. The first depicts the message flow for the case in which the device initiates the GTS allocation. The second depicts the message flow for the two cases for which a GTS deallocation occurs, first, by a device (a) and, second, by the coordinator (b).

Figure 133—Message sequence chart for GTS allocation initiated by a device

6.3.7 Primitives for resetting the MAC sublayer

MLME-SAP reset primitives specify how to reset the MAC sublayer to its default values.

All devices shall provide an interface for these reset primitives.

6.3.7.1 MLME-RESET.request

The MLME-RESET.request primitive allows the next higher layer to request that the MLME performs a reset operation.

204	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 134—Message sequence chart for GTS deallocation initiated by a device (a) and the PAN coordinator (b)

The semantics of the MLME-RESET.request primitive are as follows:

MLME-RESET.request	(

SetDefaultPIB

)

Table 72 specifies the parameter for the MLME-RESET.request primitive.

Table 72—MLME-RESET.request parameter

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SetDefaultPIB | Boolean | TRUE or FALSE | If TRUE, the MAC sublayer is reset, and all MAC |
|  |  |  | PIB attributes are set to their default values. If |
|  |  |  | FALSE, the MAC sublayer is reset, but all MAC PIB |
|  |  |  | attributes retain their values prior to the generation of |
|  |  |  | the MLME-RESET.request primitive. |

6.3.7.1.1 Appropriate usage

The MLME-RESET.request primitive is generated by the next higher layer and issued to the MLME to request a reset of the MAC sublayer to its initial conditions. The MLME-RESET.request primitive is issued prior to the use of the MLME-START.request or the MLME-ASSOCIATE.request primitives.

| Copyright © 20xx IEEE. All rights reserved. | 205 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 6.3.7.1.2 Effect on receipt |
| --- | --- |
| 2 |  |

- On receipt of the MLME-RESET.request primitive, the MLME issues the PLME-SET-TRX-STATE.request

- primitive with a state of FORCE_TRX_OFF. On receipt of the PLME-SET-TRX-STATE.confirm primitive,

- the MAC sublayer is then set to its initial conditions, clearing all internal variables to their default values. If

- the SetDefaultPIB parameter is set to TRUE, the MAC PIB attributes are set to their default values.

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

The MLME-RESET.confirm primitive with a status of SUCCESS is issued on completion.

6.3.7.2 MLME-RESET.confirm

The MLME-RESET.confirm primitive reports the results of the reset operation.

The semantics of the MLME-RESET.confirm primitive are as follows:

MLME-RESET.confirm ( status

)

Table 73 specifies the parameter for the MLME-RESET.confirm primitive.

Table 73—MLME-RESET.confirm parameter

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS | The result of the reset operation. |

6.3.7.2.1 When generated

The MLME -RESET.confirm primitive is generated by the MLME and issued to its next higher layer in response to an MLME-RESET.request primitive and following the receipt of the PLME-SET-TRX-STATE.confirm primitive.

6.3.7.2.2 Appropriate usage

- On receipt of the MLME-RESET.confirm primitive, the next higher layer is notified of its request to reset

- the MAC sublayer. This primitive returns a status of SUCCESS indicating that the request to reset the MAC

- sublayer was successful.

| 41 |  |
| --- | --- |
| 42 | 6.3.8 Primitives for specifying the receiver enable time |
| 43 |  |
| 44 | MLME-SAP receiver state primitives define how a device can enable or disable the receiver at a given time. |
| 45 |  |
| 46 | These receiver state primitives are optional. |
| 47 |  |
| 48 | 6.3.8.1 MLME-RX-ENABLE.request |
| 49 |  |

- The MLME-RX-ENABLE.request primitive allows the next higher layer to request that the receiver is either

- enabled for a finite period of time or disabled.

52

53

54

206	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The semantics of the MLME-RX-ENABLE.request primitive are as follows:

MLME-RX-ENABLE.request ( DeferPermit, RxOnTime, RxOnDuration

)

Table 74 specifies the parameters for the MLME-RX-ENABLE.request primitive.

Table 74—MLME-RX-ENABLE.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeferPermit | Boolean | TRUE or FALSE | TRUE if the requested operation can be deferred until |
|  |  |  | the next superframe if the requested time has already |
|  |  |  | passed. FALSE if the requested operation is only to be |
|  |  |  | attempted in the current superframe. This parameter is |
|  |  |  | ignored for nonbeacon-enabled OWPANs. |
|  |  |  | If the issuing device is the OWPAN coordinator, the |
|  |  |  | term superframe refers to its own superframe. |
|  |  |  | Otherwise, the term refers to the superframe of the |
|  |  |  | coordinator through which the issuing device is |
|  |  |  | associated. |
| RxOnTime | Integer | 0x000000–0xffffff | The number of optical clocks measured from the start |
|  |  |  | of the superframe before the receiver is to be enabled |
|  |  |  | or disabled. This is a 24-bit value, and the precision of |
|  |  |  | this value shall be a minimum of 20 bits, with the |
|  |  |  | lowest 4 bits being the least significant. This |
|  |  |  | parameter is ignored for nonbeacon-enabled |
|  |  |  | OWPANs. |
|  |  |  | If the issuing device is the OWPAN coordinator, the |
|  |  |  | term superframe refers to its own superframe. |
|  |  |  | Otherwise, the term refers to the superframe of the |
|  |  |  | coordinator through which the issuing device is |
|  |  |  | associated. |
| RxOnDuration | Integer | 0x000000–0xffffff | The number of optical clocks for which the receiver is |
|  |  |  | to be enabled. |
|  |  |  | If this parameter is equal to 0x000000, the receiver is |
|  |  |  | to be disabled. |

6.3.8.1.1 Appropriate usage

The MLME- RX-ENABLE.request primitive is generated by the next higher layer and issued to the MLME to enable the receiver for a fixed duration, at a time relative to the start of the current or next superframe on a beacon- enabled OWPAN or immediately on a nonbeacon-enabled OWPAN. This primitive may also be generated to cancel a previously generated request to enable the receiver. The receiver is enabled or disabled exactly once per primitive request.

| Copyright © 20xx IEEE. All rights reserved. | 207 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 6.3.8.1.2 Effect on receipt |
| --- | --- |
| 2 |  |

- The MLME will treat the request to enable or disable the receiver as secondary to other responsibilities of

- the device (e.g., GTSs, coordinator beacon tracking, beacon transmissions). When the primitive is issued to

- enable the receiver, the device will enable its receiver until either the device has a conflicting responsibility

- or the time specified by RxOnDuration has expired. In the case of a conflicting responsibility, the device will

- interrupt the receive operation. After the completion of the interrupting operation, the RxOnDuration will be

- checked to determine whether the time has expired. If so, the operation is complete. If not, the receiver is re-

- enabled until either the device has another conflicting responsibility or the time specified by RxOnDuration

- has expired. When the primitive is issued to disable the receiver, the device will disable its receiver unless

- the device has a conflicting responsibility.

12

- On a nonbeacon-enabled OWPAN, the MLME ignores the DeferPermit and RxOnTime parameters and

- requests that the PHY enable or disable the receiver immediately. If the request is to enable the receiver, the

- receiver will remain enabled until RxOnDuration symbols have elapsed.

16

17 Before attempting to enable the receiver on a beacon-enabled OWPAN, the MLME first determines whether 18 (RxOnTime + RxOnDuration) is less than the beacon interval, as defined by macBeaconOrder. If

- (RxOnTime + RxOnDuration) is not less than the beacon interval, the MLME issues the MLME-RX-

- ENABLE.confirm primitive with a status of ON_TIME_TOO_LONG.

21

- The MLME then determines whether the receiver can be enabled in the current superframe. The OWPAN

- coordinator issuing this primitive makes the determination based on its own superframe. A device that is not

- the OWPAN coordinator makes the determination based on the superframe of the coordinator through which

- it is associated. If the current number of optical clocks measured from the start of the superframe is less than

- RxOnTime, the MLME attempts to enable the receiver in the current superframe. If the current number of

- optical clocks measured from the start of the superframe is greater than or equal to RxOnTime and

- DeferPermit is equal to TRUE, the MLME defers until the next superframe and attempts to enable the

- receiver in that superframe. Otherwise, if the MLME cannot enable the receiver in the current superframe

- and is not permitted to defer the receive operation until the next superframe, the MLME issues the MLME-

- RX-ENABLE.confirm primitive with a status of PAST_TIME.

32

33 If the RxOnDuration parameter is equal to zero, the MLME requests that the PHY disable its receiver. 34

- If any parameter in the MLME-RX-ENABLE.request primitive is not supported or is out of range, the MAC

- sublayer will issue the MLME-RX-ENABLE.confirm primitive with a status of INVALID_PARAMETER.

| 38 | If the request to enable or disable the receiver was successful, the MLME issues the MLME-RX- |  |
| --- | --- | --- |
| 39 | ENABLE.confirm primitive with a status of SUCCESS. |  |
| 40 | 6.3.8.2 MLME-RX-ENABLE.confirm |  |
| 41 |  |  |
| 42 |  |  |

- The MLME-RX-ENABLE.confirm primitive reports the results of the attempt to enable or disable the

- receiver.

45

46

47

48

49

50

51

52

53

54

The semantics of the MLME-RX-ENABLE.confirm primitive are as follows:

MLME-RX-ENABLE.confirm	(

status

)

Table 75 specifies the parameter for the MLME-RX-ENABLE.confirm primitive.

208	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 75—MLME-RX-ENABLE.confirm parameter

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, | The result of the request to enable or disable the |
|  |  | PAST_TIME, | receiver. |
|  |  | ON_TIME_TOO_LONG, |  |
|  |  | INVALID_PARAMETER |  |

6.3.8.2.1 When generated

The MLME-RX- ENABLE.confirm primitive is generated by the MLME and issued to its next higher layer in response to an MLME-RX-ENABLE.request primitive.

6.3.8.2.2 Appropriate usage

On receipt of the MLME-RX- ENABLE.confirm primitive, the next higher layer is notified of its request to enable or disable the receiver. This primitive returns a status of either SUCCESS, if the request to enable or disable the receiver was successful, or the appropriate error code. The status values are fully described in 6.3.8.1.2.

6.3.8.3 Message sequence chart for changing the state of the receiver

Figure 135 illustrates the sequence of messages necessary for enabling the receiver for a fixed duration when the device does not have any conflicting responsibilities. Figure 135a) illustrates the case for a beacon-enabled OWPAN where it is assumed both that the MLME-RX-ENABLE.request has been received by the MLME without sufficient time available to enable the receiver in the current superframe and that the DeferPermit parameter is TRUE. Figure 135b) illustrates the case for a nonbeacon-enabled OWPAN where the receiver is enabled immediately.

6.3.9 Primitives for channel scanning

MLME-SAP scan primitives define how a device can determine the energy usage or the presence or absence of OWPANs in a communications channel.

All devices shall provide an interface for these scan primitives.

6.3.9.1 MLME-SCAN.request

The MLME-SCAN.request primitive is used to initiate a channel scan over a given list of channels. A device can use a channel scan to measure the energy on the channel, search for the coordinator with which it associated, or search for all coordinators transmitting beacon frames within the coverage area of the scanning device.

| Copyright © 20xx IEEE. All rights reserved. | 209 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 135—Message sequence chart for changing the state of the receiver

The semantics of the MLME-SCAN.request primitive are as follows:

MLME-SCAN.request	(

ScanType,

ScanDuration,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

ColorScan

)

Table 76 specifies the parameters for the MLME-SCAN.request primitive.

210	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 76—MLME-SCAN.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ScanType | Integer | 0x00–0x01 | Indicates the type of scan performed: |
|  |  |  | 0x00 = active scan (optional for a device). |
|  |  |  | 0x01 = passive scan. |
|  |  |  | 0x10 = scan-over-backhaul |
| ScanDuration | Integer | 0–14 | The time spent scanning each channel is |
|  |  |  | [aBaseSuperframeDuration  (2n + 1)] optical |
|  |  |  | clocks, where n is the value of the |
|  |  |  | ScanDuration parameter. |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in |
|  |  |  | Table 97 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (as |
|  |  |  | defined in Table 98 in 7.4.2.2). This parameter |
|  |  |  | is ignored if the SecurityLevel parameter is set |
|  |  |  | to 0x00. |
| KeySource | Set of 0, 4, | As specified by the | The originator of the key to be used (see |
|  | or 8 octets | KeyIdMode parameter | 7.4.4.1). This parameter is ignored if the |
|  |  |  | KeyIdMode parameter is ignored or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). |
|  |  |  | This parameter is ignored if the KeyIdMode |
|  |  |  | parameter is ignored or set to 0x00. |
| ColorScan | Boolean | TRUE or FALSE | ColorScan shall be set as TRUE if the color |
|  |  |  | CVD frame is to be transmitted either during |
|  |  |  | passive scan or after the beacon request |
|  |  |  | command is sent (see 5.1.3.1) for an active |
|  |  |  | scan. |

6.3.9.1.1 Appropriate usage

The MLME-SCAN.request primitive is generated by the next higher layer and issued to its MLME to initiate a channel scan to search for activity within the coverage area of the device. This primitive can be used to perform an active, passive scanor scan-over-backhaul to locate beacon frames containing any OWPAN identifier. Refer to 5.1.5.1 for a description of each type of scan in detail.

All devices shall be capable of performing passive scans, while active scans are optional for a device. However, a device may support active scanning to participate in a nonbeacon-enabled network.

6.3.9.1.2 Effect on receipt

If the MLME receives the MLME-SCAN.request primitive while performing a previously initiated scan operation, it issues the MLME-SCAN.confirm primitive with a status of SCAN_IN_PROGRESS. Otherwise, the MLME initiates a scan in all channels specified in the ScanChannels parameter.

The active scan is performed on each channel by the MLME first sending a beacon request command as specified in 5.4.10. The MLME then enables the receiver and records the information contained in each received beacon in a OWPAN descriptor structure as shown in Table 64. The active scan on a particular channel terminates when the number of OWPAN descriptors stored equals an implementation-specified

| Copyright © 20xx IEEE. All rights reserved. | 211 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | maximum or when [aBaseSuperframeDuration  (2n + 1)] optical clocks, where n is the value of the |
| --- | --- |
| 2 | ScanDuration parameter, have elapsed, whichever comes first. Refer to 5.1.3.1.1 for more detailed |
| 3 | information on the active channel scan procedure. |
| 4 |  |

- The passive scan is performed on each channel by the MLME enabling its receiver and recording the

- information contained in each received beacon in a OWPAN descriptor structure as specified in Table 64.

- The passive scan on a particular channel terminates when the number of OWPAN descriptors stored equals

- an implementation-specified maximum or when [aBaseSuperframeDuration  (2n + 1)] optical clocks,

- where n is the value of the ScanDuration parameter, have elapsed, whichever comes first. Refer to 5.1.3.1.2

- for more detailed information on the passive channel scan procedure.

11

- The scan-over-backhaul is performed on each channel by the MLME first sending a scan-over-backhaul

- request command to other coordinators through the backhaul as specified in 6.7.6. The MLME then enables

- the receiver and records the information contained in each received scan-over-backhaul confirmation

- command. The scan-over-backhaul on a particular channel terminates when the number of OWPAN

- descriptors stored equals an implementation-specified maximum or when [aBaseSuperframeDuration ? (2n

- + 1)] optical clocks, where n is the value of the ScanDuration parameter, have elapsed, whichever comes

- first. Refer to 6.2.2.1.1 for more detailed information on the scan-over-backhaul procedure.

19

- If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for

- this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC

22	sublayer will perform outgoing processing on the frame based on macCoordExtendedAddress, the

- SecurityLevel, KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs

- during outgoing frame processing, the MLME will discard the frame and issue the MLME-SCAN.confirm

- primitive with the error status returned by outgoing frame processing.

26

- The results of an active, or passive scan or scan-over-backhaul are reported to the next higher layer through

- the MLME-SCAN.confirm primitive. If the scan is successful and macAutoRequest is set to TRUE, the

29	primitive  results  will  include  a  set  of  OWPAN  descriptor  values.  If  the  scan  is  successful  and

- macAutoRequest is set to FALSE, the primitive results will contain a null set of OWPAN descriptor values;

- each OWPAN descriptor value will be sent individually to the next higher layer using separate MLME-

- BEACON-NOTIFY (see 6.3.3.1) primitives. In both cases, the MLME-SCAN.confirm primitive will

- contain a list of unscanned channels and a status of SUCCESS.

34

- If, during an active scan, the MLME is unable to transmit a beacon request command on a channel specified

- by the ScanChannels parameter due to a channel access failure, the channel will appear in the list of

- unscanned channels returned by the MLME-SCAN.confirm primitive. If the MLME was able to send a

38	beacon request command on at least one of the channels but no beacons were found, the MLME-

- SCAN.confirm primitive will contain a null set of OWPAN descriptor values, regardless of the value of

- macAutoRequest, and a status of NO_BEACON.

| 41 |  |
| --- | --- |
| 42 | If, during an active or passive scan, the implementation-specified maximum is reached thus terminating the |
| 43 | scan procedure, the MAC sublayer will issue the MLME-SCAN.confirm primitive with a status of |
| 44 | LIMIT_REACHED. |
| 45 |  |

- If any parameter in the MLME-SCAN.request primitive is not supported or is out of range, the MAC

- sublayer will issue the MLME-SCAN.confirm primitive with a status of INVALID_PARAMETER.

| 48 | 6.3.9.2 MLME-SCAN.confirm |  |  |
| --- | --- | --- | --- |
| 49 |  |  |  |
| 50 |  |  |  |
| 51 | The MLME-SCAN.confirm primitive reports the result of the channel scan request. |  |  |
| 52 |  |  |  |
| 53 | The semantics of the MLME-SCAN.confirm primitive are as follows: |  |  |
| 54 |  |  |  |
|  | 212 | Copyright © 20xx IEEE. All rights reserved. |  |

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

MLME-SCAN.confirm	(

status,

ScanType,

ResultListSize,

OWPANDescriptorList

)

Table 77 specifies the parameters for the MLME-SCAN.confirm primitive.

Table 77—MLME-SCAN.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, LIMIT_REACHED, | The status of the scan request. |
|  |  | NO_BEACON, |  |
|  |  | SCAN_IN_PROGRESS, |  |
|  |  | COUNTER_ERROR, |  |
|  |  | FRAME_TOO_LONG, |  |
|  |  | UNAVAILABLE_KEY, |  |
|  |  | UNSUPPORTED_SECURITY |  |
|  |  | or INVALID_PARAMETER |  |
| ScanType | Integer | 0x00–0x01 | Indicates the type of scan performed: |
|  |  |  | 0x00 = active scan |
|  |  |  | 0x01 = passive scan |
|  |  |  | 0x10 = scan-over-backhaul |
| ResultListSize | Integer | Implementation specific | The number of elements returned in |
|  |  |  | the appropriate result lists. |
| OWPANDescriptorL | List of | Refer to Table 64 | The list of OWPAN descriptors, one |
| ist | OWPAN |  | for |
|  | descriptor |  | each beacon found during a |
|  | values |  | scan if macAutoRequest is set |
|  |  |  | to TRUE. This parameter is null when |
|  |  |  | macAutoRequest is set to FALSE |
|  |  |  | during a scan. |

6.3.9.2.1 When generated

The MLME-SCAN.confirm primitive is generated by the MLME and issued to its next higher layer when the channel scan initiated with the MLME -SCAN.request primitive has completed. If the MLME-SCAN.request primitive requested a scan with macAutoRequest set to FALSE, the OWPANDescriptorList parameter will be null.

The MLME-SCAN.confirm primitive returns a status of either SUCCESS, indicating that the requested scan was successful, or the appropriate error code. The status values are fully described in 6.3.9.1.2 and subclauses referenced by 6.3.9.1.2.

| Copyright © 20xx IEEE. All rights reserved. | 213 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 6.3.9.2.2 Appropriate usage |
| --- | --- |
| 2 |  |

- On receipt of the MLME-SCAN.confirm primitive, the next higher layer is notified of the results of the scan

- procedure. If the requested scan was successful, the status parameter will be set to SUCCESS. Otherwise,

- the status parameter indicates the error.

6

7	6.3.9.3 Channel scan message sequence charts

8

- Figure 148 and Figure 149 illustrate the sequence of messages necessary to perform a passive scan and an

- active scan. These figures include steps taken by the PHY.

| 11 | 6.3.10 Neighbor report primitives |  |
| --- | --- | --- |
| 12 |  |  |
| 13 | The MLME-SAP NeighborReport management primitives define how the coordinator triggers a |  |
| 14 |  |  |
| 15 | neighboring OWPAN report and how the requested devices to perform the report. |  |
| 16 | 6.3.10.1 MLME-NeighborReport.request |  |
| 17 |  |  |
| 18 |  |  |

- The MLME-NeighborReport.request primitive allows the coordinator to generate and send a Neighboring

- OWPAN report request command to devices.

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

The semantics of the MLME-NeighborReport.request primitive are as follows:

MLME-NeighborReport.request (

ReportDevicesList,

ReportType,

)

Table 78—MLME-NeighborReport.request

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
| ReportDevicesList | List of short |  | Indicates to which device(s) the coordinator shall |  |
|  | address |  | send a neighboring OWPAN report command. |  |
|  |  |  | 0x00 = only update on neighboring OWPAN sta- |  |
| ReportType | Integer | 0x00 - 0x01 | tus is requested; |  |
|  |  |  | 0x01= full report on neighboring OWPAN status |  |
|  |  |  | is requested; |  |

6.3.10.1.1 When generated

On receipt of the MLME-SYNC-LOSS.indication primitive with the LossReason parameter set to OWPAN_ID_CONFLICT by the next higher layer of the coordinator, it shall issue a MLME-NeighborReport.request primitive to its MLME.

214	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 6.3.10.1.2 Effect on receipt |  | 1 |  |
| --- | --- | --- | --- |
| The MLME of the coordinator shall generate a neighboring OWPAN report request command and send the |  | 2 |  |
|  |  | 3 |  |
| command to the device specified by the ReportDeviceList. If multiple devices have been requested by the |  | 4 |  |
| next higher layer, the MLME shall generate multiple neighboring OWPAN report request command and |  | 5 |  |
| send the command to the multiple devices respectively. |  | 6 |  |
| 6.3.10.1.3 MLME-NeighborReport.indication |  | 7 |  |
|  |  | 8 |  |
| The MLME-NeighborReport.indication primitive indicates to the next higher layer of the device that the |  | 9 |  |
|  |  | 10 |  |
| MLME has received a neighboring OWPAN report request command from the coordinator. |  | 11 |  |
| The semantics of the MLME-NeighborReport.indication primitive are as follows: |  | 12 |  |
|  |  | 13 |  |
| MLME-NeighborReport.indication ( |  | 14 |  |
|  |  | 15 |  |
| ReportType, |  | 16 |  |
|  |  | 17 |  |
| ) |  | 18 |  |
|  |  | 19 |  |
| 6.3.10.1.4 When generated |  | 20 |  |
|  |  | 21 |  |
| On receipt of a neighboring OWPAN report request command by the MLME of the device, it shall issue the |  | 22 |  |
|  |  | 23 |  |
| MLME-NeighborReport.indication primitive to its next higher layer. |  | 24 |  |
| 6.3.10.1.5 Effect on receipt |  | 25 |  |
|  |  | 26 |  |
| On receipt of the MLME-NeighborReport.indication primitive, the next higher layer of the device is notified |  | 27 |  |
|  |  | 28 |  |
| of the neighboring OWPAN report request of the coordinator. The next higher layer shall then decide |  | 29 |  |
| whether to perform an active scan or not and then provide the updated local neighboring OWPAN descriptor |  | 30 |  |
| list to the MLME by issuing a MLME- NeighborReport.response primitive. |  | 31 |  |
| 6.3.10.1.6 MLME-NeighborReport.response |  | 32 |  |
|  |  | 33 |  |
| The MLME-NeighborReport.response primitive is used to initiate a response to an MLME- |  | 34 |  |
|  |  | 35 |  |
| NeighborReport.indication primitive. |  | 36 |  |
| The semantics of the MLME-NeighborReport.response primitive are as follows: |  | 37 |  |
|  |  | 38 |  |
| MLME-NeighborReport.response ( |  | 39 |  |
|  |  | 40 |  |
| ReportType, |  | 41 |  |
|  |  | 42 |  |
| NumberNeighboringOWPAN Descriptor, |  | 43 |  |
|  |  | 44 |  |
| NeighboringOWPANDescriptor[0], |  | 45 |  |
|  |  | 46 |  |
| …… |  | 47 |  |
|  |  | 48 |  |
| NeighboringOWPANDescriptor[N-1], |  | 49 |  |
|  |  | 50 |  |
| ) |  | 51 |  |
|  |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 215 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  | Table 79—MLME-NeighborReport.response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 2 |  |  |  |  |  |
| 3 | Name | Type | Valid range | Description |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  | 0x00= only update on neighboring OWPAN |  |
| 6 |  |  |  | status is requested; |  |
| 7 | ReportType | Integer | 0x00 - 0x001 |  |  |
| 8 |  |  |  | 0x01=full report on neighboring OWPAN sta- |  |
| 9 |  |  |  | tus is requested; |  |
| 10 |  |  |  |  |  |
|  | NumberNeighborin- |  |  | Indicates the number of Neighboring OWPAN |  |
| 11 |  | Integer |  |  |  |
|  | gOWPANDescriptor |  |  | Descriptors. |  |
| 12 |  |  |  |  |  |
| 13 | NeighboringOW- |  |  | Indicates the basic information of the neigh- |  |
| 14 | PANDescriptor[x] |  |  | boring OWPAN. |  |
| 15 |  |  |  |  |  |
| 16 |  |  |  |  |  |
| 17 | 6.3.10.1.7 When generated |  |  |  |  |
| 18 |  |  |  |  |  |

- The MLME-NeighborReport.response primitive is generated by the next higher layer of a device and issued

- to its MLME in order to respond the MLME-NeighborReport.indication primitive.

21

22	6.3.10.1.8 Effect on receipt

23

- On receipt of the MLME- NeighborReport.response primitive, the MLME of the device shall generate a

- neighboring OWPAN report indication command based on the provided arguments and send the command

- to the coordinator.

27

28	6.3.10.2 MLME-NeighborReport.confirm

29

- The MLME-NeighborReport.confirm primitive provides the result of a requested neighboring OWPAN

- report to the next higher layer of the coordinator.

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

The semantics of the MLME-NeighborReport.confirm primitive are as follows:

MLME-NeighborReport.confirm (

Result,

UnacknowledgedDevices,

NumberNeighboringOWPAN Descriptor,

NeighboringOWPANDescriptor[0],

DetectDeviceList[0],

……

NeighboringOWPANDescriptor[N-1],

DetectDeviceList[N-1],

)

216	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 80—MLME-NeighborReport.confirm

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
|  |  | SUCCESS, |  |  |
| Result | Numeration | PARTIAL_AC- | Indicates the result of the requested report. |  |
|  |  | KNOWLEDGE |  |  |
|  |  |  | Indicates the short address of the devices from |  |
|  |  |  | which the corresponding neighboring OWPAN |  |
| Unacknowledged- | List of short |  | report indication commands have not been received |  |
|  |  |  | by the coordinator within the macNeighborRepor- |  |
| DevicesList | addresses |  | tResponseTime optical clocks. |  |
|  |  |  | The field is valid only when the Result equals to |  |
|  |  |  | "PARTIAL_ACKNOWLEDGED". |  |
| NumberNeighbor- |  |  | Indicates the number of Neighboring OWPAN |  |
| ingOWPAN | Integer | 0x00 - 0xff |  |  |
|  |  |  | Descriptors. |  |
| Descriptor |  |  |  |  |
| NeighboringOW- | lists of Neigh- |  | Indicates the basic information of the neighboring |  |
| PANDescriptorL- | boringOWPAN | Refer to Table 4 |  |  |
|  |  |  | OWPAN. |  |
| ist | Descriptor |  |  |  |

6.3.10.2.1 When generated

When the neighboring OWPAN report indication commands from each requested device have been received by the MLME of the coordinator, or when the macNeighborReportResponseTime optical clocks time expires, the MLME of the coordinator shall issue the MLME-NeighborReport.confirm primitive to its next higher layer to report the result of the requested report.

6.3.10.2.2 Effect on receipt

The next higher layer of the coordinator shall be notified of the result of the requested neighboring OWPAN report on receipt of the MLME-NeighborReport.confirm primitive.

6.3.11 Communication status primitive

The MLME -SAP communication status primitive defines how the MLME communicates to the next higher layer about transmission status, when the transmission was instigated by a response primitive, and about security errors on incoming packets.

All devices shall provide an interface for this communication status primitive.

6.3.11.1 MLME-COMM-STATUS.indication

The MLME-COMM-STATUS.indication primitive allows the MLME to indicate a communications status.

The semantics of the MLME-COMM-STATUS.indication primitive are as follows:

| Copyright © 20xx IEEE. All rights reserved. | 217 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

MLME-COMM-STATUS.indication	(

OWPANId,

SrcAddrMode,

SrcAddr,

DstAddrMode,

DstAddr,

status,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table 81 specifies the parameters for the MLME-COMM-STATUS.indication primitive.

Table 81—MLME-COMM-STATUS.indication parameters

| Name | Type | Valid range |  | Description |
| --- | --- | --- | --- | --- |
| OWPANId | Integer | 0x0000–0xffff | The 16-bit OWPAN identifier of the |  |
|  |  |  | device from which the frame was |  |
|  |  |  | received or to which the frame was being |  |
|  |  |  | sent. |  |
| SrcAddrMode | Integer | 0x00–0x03 | The source addressing mode for this |  |
|  |  |  | primitive. This value can take one of the |  |
|  |  |  | following values: |  |
|  |  |  | 0 = no address (addressing fields |  |
|  |  |  | omitted). |  |
|  |  |  | 0x01 | = no address field (broadcast only |
|  |  |  | mode with no address fields present). |  |
|  |  |  | 0x02 | = 16-bit short address. |
|  |  |  | 0x03 | = 64-bit extended address. |
| SrcAddr | Device | As specified by the | The individual device address of the |  |
|  | address | SrcAddrMode | entity from which the frame causing the |  |
|  |  | parameter | error originated. |  |
| DstAddrMode | Integer | 0x00–0x03 | The destination addressing mode for this |  |
|  |  |  | primitive. This value can take one of the |  |
|  |  |  | following values: |  |
|  |  |  | 0x00 | = no address (addressing fields |
|  |  |  | omitted). |  |
|  |  |  | 0x01 | = reserved. |
|  |  |  | 0x02 | = 16-bit short address. |
|  |  |  | 0x03 | = 64-bit extended address. |
| DstAddr | Device | As specified by the | The individual device address of the |  |
|  | address | DstAddrMode | device for which the frame was intended. |  |
|  |  | parameter |  |  |

218	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 81—MLME-COMM-STATUS.indication parameters (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, | The communications status. |
|  |  | TRANSACTION_OVERFLOW, |  |
|  |  | TRANSACTION_EXPIRED, |  |
|  |  | CHANNEL_ACCESS_FAILURE, |  |
|  |  | NO_ACK, COUNTER_ERROR, |  |
|  |  | FRAME_TOO_LONG, |  |
|  |  | IMPROPER_KEY_TYPE, |  |
|  |  | IMPROPER_SECURITY_LEVEL, |  |
|  |  | SECURITY_ERROR, |  |
|  |  | UNAVAILABLE_KEY, |  |
|  |  | UNSUPPORTED_LEGACY, |  |
|  |  | UNSUPPORTED_SECURITY, |  |
|  |  | INVALID_PARAMETER |  |
| SecurityLevel | Integer | 0x00–0x07 | If the primitive was generated following |
|  |  |  | a transmission instigated through a |
|  |  |  | response primitive: |
|  |  |  | The security level to be used (as defined |
|  |  |  | in Table 97 in 7.4.2.1). |
|  |  |  | If the primitive was generated on receipt |
|  |  |  | of a frame that generates an error in its |
|  |  |  | security processing: |
|  |  |  | The security level purportedly used by |
|  |  |  | the received frame (as defined in |
|  |  |  | Table 97 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | If the primitive was generated following |
|  |  |  | a transmission instigated through a |
|  |  |  | response primitive: |
|  |  |  | The mode used to identify the key to be |
|  |  |  | used (as defined in Table 98 in 7.4.2.2). |
|  |  |  | This parameter is ignored if the |
|  |  |  | SecurityLevel parameter is set to 0x00. |
|  |  |  | If the primitive was generated on receipt |
|  |  |  | of a frame that generates an error in its |
|  |  |  | security processing: |
|  |  |  | The mode used to identify the key |
|  |  |  | purportedly used by the originator of the |
|  |  |  | received frame (as defined in Table 98 in |
|  |  |  | 7.4.2.2). This parameter is invalid if the |
|  |  |  | SecurityLevel parameter is set to 0x00. |

| Copyright © 20xx IEEE. All rights reserved. | 219 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

Table 81—MLME-COMM-STATUS.indication parameters (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| KeySource | Set of 0, 4, | As specified by the KeyIdMode | If the primitive was generated following |
|  | or 8 octets | parameter | a transmission instigated through a |
|  |  |  | response primitive: |
|  |  |  | The originator of the key to be used (see |
|  |  |  | 7.4.4.1). This parameter is ignored if the |
|  |  |  | KeyIdMode parameter is ignored or set to |
|  |  |  | 0x00. |
|  |  |  | If the primitive was generated on receipt |
|  |  |  | of a frame that generates an error in its |
|  |  |  | security processing: |
|  |  |  | The originator of the key purportedly |
|  |  |  | used by the originator of the received |
|  |  |  | frame (see 7.4.4.1). This parameter is |
|  |  |  | invalid if the KeyIdMode parameter is |
|  |  |  | invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | If the primitive was generated following |
|  |  |  | a transmission instigated through a |
|  |  |  | response primitive: |
|  |  |  | The index of the key to be used (see |
|  |  |  | 7.4.4.2). This parameter is ignored if the |
|  |  |  | KeyIdMode parameter is ignored or set to |
|  |  |  | 0x00. |
|  |  |  | If the primitive was generated on receipt |
|  |  |  | of a frame that generates an error in its |
|  |  |  | security processing: |
|  |  |  | The index of the key purportedly used by |
|  |  |  | the originator of the received frame (see |
|  |  |  | 7.4.4.2). This parameter is invalid if the |
|  |  |  | KeyIdMode parameter is invalid or set to |
|  |  |  | 0x00. |

6.3.11.1.1 When generated

- The MLME-COMM-STATUS.indication primitive is generated by the MLME and issued to its next higher

- layer either following a transmission instigated through a response primitive or on receipt of a frame that

- generates an error in its security processing (see 7.2.3).

43

- The MLME-COMM-STATUS.indication primitive is generated by the MAC sublayer entity following the

- MLME-ASSOCIATE.response primitive. This primitive returns a status of either SUCCESS, indicating that

| 46 | the request to transmit was successful, an error code of TRANSACTION_OVERFLOW, |  |
| --- | --- | --- |
| 47 | TRANSACTION_EXPIRED, CHANNEL_ACCESS_FAILURE, NO_ACK, or INVALID_PARAMETER |  |
| 48 | (these status values are fully described in 6.3.1.3.2), or an error code resulting from failed security |  |
| 49 | processing (these status values are fully described in 7.2.1 and 7.2.3). |  |
| 50 |  |  |
| 51 |  |  |
| 52 |  |  |
| 53 |  |  |
| 54 |  |  |
|  | 220 | Copyright © 20xx IEEE. All rights reserved. |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.11.1.2 Appropriate usage

On receipt of the MLME -COMM-STATUS.indication primitive, the next higher layer is notified of the communication status of a transmission or notified of an error that has occurred during the secure processing of incoming frame.

6.3.12 Primitive for heterogeneous RF & OWC functionality

As described in 4.2.6, command frames (e.g. association request command.) from type 2 devices with destination address set to the coordinator may be may be first transmitted through RF link and then forwarded to the coordinator through the backhaul link, if RF&OWC functionality is enabled. MLME-SAP command primitives are specified to provide interface to the next higher layer so as to route the command frame through RF link.

      - MLME-COMMAND.indication

The MLME-COMMAND.indication primitive allows the MLME of the device to provide a command frame to its next higher layer.

The semantics of the MLME-COMMAND.indication primitive are as follows:

MLME-COMMAND.indication (

DstAddr,

SrcAddr,

CommandFrame

)

Table 10-22 specifies the parameters for the MLME-COMMAND.indication primitive.

Table 10-22 MLME-COMMAND.indication primitive

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DstAddr | Integer | From 0x0000-0xffff, 0x0000 represents without a short address assigned, 0xffff represents a broadcast transmission. | The short address of the coordinator that the command frame is designated to. |
| DSrcAddr | Integer | From 0x0000-0xffff, 0x0000 represents without a short address assigned, 0xffff represents a broadcast transmission. | The short address of the device that has generated the command frame. |
| CommandFrame | Set of octets |  | The set of octets forming the command frames to be transmitted. |

        - When generated

When the MLME of the device generates a command frame that needs to be sent to the coordinator, it shall generate a MLME-COMMAND.indication primitive and issue the MLME-COMMAND.indication primitive to its next higher layer.

        - Effect on receipt

On receipt of the MLME-COMMAND.indication primitive, the next higher layer of the device may generate a next higher layer message based on the provided arguments and shall send the message to the the next higher layer of the coordinator through the RF link and backhaul link. The command frame generated by the MLME shall be embedded in the next higher layer message.

      - MLME-COMMAND.request

The MLME-COMMAND.request primitive allows the next higher layer of the coordinator to request its MLME to identify and process a command frame received through the RF link and backhaul link.

The semantics of the MLME-COMMAND.request primitive are as follows:

MLME-COMMAND.request (

DstAddr,

SrcAddr,

CommandFrame

)

Table 10-23 specifies the parameters for the MLME-COMMAND. request primitive.

Table 10-23 MLME-COMMAND. request primitive

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DstAddr | Integer | From 0x0000-0xffff, 0x0000 represents without a short address assigned, 0xffff represents a broadcast transmission. | The short address of the coordinator that the command frame is designated to. |
| DSrcAddr | Integer | From 0x0000-0xffff, 0x0000 represents without a short address assigned, 0xffff represents a broadcast transmission. | The short address of the device that has generated the command frame. |
| CCommandFrame | Set of octets |  | The set of octets forming the command frames to be transmitted. |

        - When generated

The next higher layer of the coordinator shall generate a MLME-COMMAND.request primitive when it has received the next higher layer message and issue the MLME-COMMAND.request primitive to its MLME.

        - Effect on receipt

On receipt of the MLME-COMMAND.request primitive, the MLME of the coordinator shall process the command frame provided in this primitive the same way as it was received through the VLC link.

6.3.12 Primitives for writing PIB attributes

MLME-SAP set primitives define how PIB attributes may be written.

All devices shall provide an interface for these set primitives.

6.3.12.1 MLME-SET.request

The MLME-SET.request primitive attempts to write the given value to the indicated PIB attribute.

6.3.12.1.1 Semantics of the primitive

The semantics of the MLME-SET.request primitive are as follows:

MLME-SET.request	(

PIBAttribute,

PIBAttributeIndex,

PIBAttributeValue

)

Table 82 specifies the parameters for the MLME-SET.request primitive.

Table 82—MLME-SET.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PIBAttribute | Integer | Refer to Table 91 and | The identifier of the PIB attribute to write. |
|  |  | Table 99 |  |
| PIBAttributeIndex | Integer | Attribute specific; | The index within the table of the specified |
|  |  | as defined in Table 91 | PIB attribute to write. This parameter is valid |
|  |  | and Table 99 | only for MAC PIB attributes that are tables; it |
|  |  |  | is ignored when accessing PHY PIB |
|  |  |  | attributes. |
| PIBAttributeValue | Various | Attribute specific; | The value to write to the indicated PIB |
|  |  | as defined in Table 91 | attribute. |
|  |  | and Table 99 |  |

6.3.12.1.2 Appropriate usage

The MLME-SET.request primitive is generated by the next higher layer and issued to its MLME to write the indicated PIB attribute.

6.3.12.1.3 Effect on receipt

On receipt of the MLME-SET.request primitive, the MLME checks to see if the PIB attribute is a MAC PIB attribute or PHY PIB attribute. If the requested attribute is a MAC attribute, the MLME attempts to write the

| Copyright © 20xx IEEE. All rights reserved. | 221 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1 given value to the indicated MAC PIB attribute in its database. If the PIBAttribute parameter specifies an 2 attribute that is a read- only attribute, shown in Table 91, the MLME will issue the MLME-SET.confirm 3 primitive with a status of READ_ONLY. If the PIBAttribute parameter specifies an attribute that is not 4 found in the database, the MLME will issue the MLME-SET.confirm primitive with a status of

- UNSUPPORTED_ATTRIBUTE. If the PIBAttributeIndex parameter specifies an index for a table that is out

- of range, the MLME will issue the MLME-SET.confirm primitive with a status of INVALID_INDEX. If the

- PIBAttributeValue parameter specifies a value that is out of the valid range for the given attribute, the

- MLME will issue the MLME-SET.confirm primitive with a status of INVALID_PARAMETER. If the

- requested MAC PIB attribute is successfully written, the MLME will issue the MLME-SET.confirm

- primitive with a status of SUCCESS.

11

- If the PIBAttribute parameter indicates that macBeaconPayloadLength is to be set and the length of the

- resulting beacon frame exceeds aMaxPHYFrameSize (e.g., due to the additional overhead required for

- security processing), the MAC sublayer shall not update macBeaconPayloadLength and will issue the

- MLME-GET.confirm primitive with a status of INVALID_PARAMETER.

16

- If the requested attribute is a PHY PIB attribute, the request is passed to the PHY by issuing the PLME-

- SET.request primitive. Once the MLME receives the PLME-SET.confirm primitive, it will translate the

- received status value because the status values used by the PHY are not the same as those used by the

- MLME (e.g., the status values for SUCCESS are 0x00 and 0x07 in the MAC and PHY enumeration tables,

- respectively). Following the translation, the MLME will issue the MLME-SET.confirm primitive to the next

- higher layer with the status parameter resulting from the translation and the PIBAttribute parameter equal to

- that returned by the PLME primitive.

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

6.3.12.2 MLME-SET.confirm

The MLME-SET.confirm primitive reports the results of an attempt to write a value to a PIB attribute.

The semantics of the MLME-SET.confirm primitive are as follows:

MLME-SET.confirm	(

status,

PIBAttribute,

PIBAttributeIndex

)

Table 83 specifies the parameters for the MLME-SET.confirm primitive.

Table 83—MLME-SET.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, READ_ONLY, | The result of the request to write the |
|  |  | UNSUPPORTED_ATTRIBUTE, | PIB attribute. |
|  |  | INVALID_INDEX, |  |
|  |  | INVALID_PARAMETER |  |
| PIBAttribute | Integer | Refer to Table 91 and Table 99 | The identifier of the PIB attribute that |
|  |  |  | was written. |
| PIBAttributeIndex | Integer | Attribute specific; | The index within the table of the |
|  |  | as defined in Table 91 and | specified PIB attribute to write. This |
|  |  | Table 99 | parameter is valid only for MAC PIB |
|  |  |  | attributes that are tables; it is ignored |
|  |  |  | when accessing PHY PIB attributes. |

222	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.12.2.1 When generated

The MLME-SET.confirm primitive is generated by the MLME and issued to its next higher layer in response to an MLME-SET.request primitive. The MLME -SET.confirm primitive returns a status of either SUCCESS, indicating that the requested value was written to the indicated PIB attribute, or the appropriate error code. The status values are fully described in 6.3.12.1.3.

6.3.12.2.2 Appropriate usage

On receipt of the MLME-SET.confirm primitive, the next higher layer is notified of the result of its request to set the value of a PIB attribute. If the requested value was written to the indicated PIB attribute, the status parameter will be set to SUCCESS. Otherwise, the status parameter indicates the error.

6.3.13 Primitives for updating the superframe configuration

MLME-SAP start primitives define how a coordinator can request to start using a new superframe configuration in order to initiate a OWPAN, begin transmitting beacons on an already existing OWPAN, thus facilitating device discovery, or to stop transmitting beacons.

These start primitives are optional for a device.

6.3.13.1 MLME-START.request

The MLME -START.request primitive allows the OWC coordinator to initiate a new OWPAN or to begin using a new superframe configuration. This primitive may also be used by a device already associated with an existing OWPAN to begin using a new superframe configuration.

The semantics of the MLME-START.request primitive are as follows:

MLME-START.request	(

OWPANId,

LogicalChannel,

StartTime,

BeaconOrder,

SuperframeOrder,

OWPANCoordinator,

CoordRealignment,

CoordRealignSecurityLevel,

CoordRealignKeyIdMode,

CoordRealignKeySource,

CoordRealignKeyIndex,

BeaconSecurityLevel,

BeaconKeyIdMode,

BeaconKeySource,

BeaconKeyIndex

)

Table 84 specifies the parameters for the MLME-START.request primitive.

| Copyright © 20xx IEEE. All rights reserved. | 223 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 84—MLME-START.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| OWPANId | Integer | 0x0000–0xffff | The OWPAN identifier to be used by |
|  |  |  | the device. |
| LogicalChannel | Integer | Selected from the available | The logical channel on which to start |
|  |  | logical channels | using the new superframe |
|  |  |  | configuration. |
| StartTime | Integer | 0x000000–0xffffff | The time at which to begin |
|  |  |  | transmitting beacons. If this |
|  |  |  | parameter is equal to 0x000000, |
|  |  |  | beacon transmissions will begin |
|  |  |  | immediately. Otherwise, the specified |
|  |  |  | time is relative to the received beacon |
|  |  |  | of the coordinator with which the |
|  |  |  | device synchronizes. |
|  |  |  | This parameter is ignored if either the |
|  |  |  | BeaconOrder parameter has a value |
|  |  |  | of 15 or the OWPANCoordinator |
|  |  |  | parameter is TRUE. |
|  |  |  | The time is specified in optical clocks |
|  |  |  | and is rounded to a backoff slot |
|  |  |  | boundary. This is a 24-bit value, and |
|  |  |  | the precision of this value shall be a |
|  |  |  | minimum of 20 bits, with the lowest |
|  |  |  | 4 bits being the least significant. |
| BeaconOrder | Integer | 0–15 | How often the beacon is to be |
|  |  |  | transmitted. A value of 15 indicates |
|  |  |  | that the coordinator will not transmit |
|  |  |  | periodic beacons. |
|  |  |  | Refer to 5.1.1 for an explanation of |
|  |  |  | the relationship between the beacon |
|  |  |  | order and the beacon interval. |
| SuperframeOrder | Integer | 0–BO or 15 | The length of the active portion of the |
|  |  |  | superframe, including the beacon |
|  |  |  | frame. If the BeaconOrder parameter |
|  |  |  | (BO) has a value of 15, this parameter |
|  |  |  | is ignored. |
|  |  |  | Refer to 5.1.1 for an explanation of |
|  |  |  | the relationship between the |
|  |  |  | superframe order and the superframe |
|  |  |  | duration. |
| OWPANCoordinator | Boolean | TRUE or FALSE | If this value is TRUE, the device will |
|  |  |  | become the coordinator of a new |
|  |  |  | OWPAN. If this value is FALSE, the |
|  |  |  | device will begin using a new |
|  |  |  | superframe configuration on the |
|  |  |  | OWPAN with which it is associated. |
| CoordRealignment | Boolean | TRUE or FALSE | TRUE if a coordinator realignment |
|  |  |  | command is to be transmitted prior to |
|  |  |  | changing the superframe |
|  |  |  | configuration or FALSE otherwise. |

224	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 84—MLME-START.request parameters (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| CoordRealignSecurity- | Integer | 0x00–0x07 | The security level to be used for |
| Level |  |  | coordinator realignment command |
|  |  |  | frames (as defined in Table 97 in |
|  |  |  | 7.4.2.1). |
| CoordRealignKeyId- | Integer | 0x00–0x03 | The mode used to identify the key to |
| Mode |  |  | be used (as defined in Table 98 in |
|  |  |  | 7.4.2.2). This parameter is ignored if |
|  |  |  | the CoordRealignSecurityLevel |
|  |  |  | parameter is set to 0x00. |
| CoordRealignKey- | Set of 0, 4, | As specified by the | The originator of the key to be used |
| Source | or 8 octets | CoordRealignKeyIdMode | (see 7.4.4.1). This parameter is |
|  |  | parameter | ignored if the CoordRealignKeyId- |
|  |  |  | Mode parameter is ignored or set to |
|  |  |  | 0x00. |
| CoordRealignKeyIndex | Integer | 0x01–0xff | The index of the key to be used (see |
|  |  |  | 7.4.4.2). This parameter is ignored if |
|  |  |  | the CoordRealignKeyIdMode |
|  |  |  | parameter is ignored or set to 0x00. |
| BeaconSecurityLevel | Integer | 0x00–0x07 | The security level to be used for |
|  |  |  | beacon frames (as defined in Table 97 |
|  |  |  | in 7.4.2.1). |
| BeaconKeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to |
|  |  |  | be used (as defined in Table 98 in |
|  |  |  | 7.4.2.2). This parameter is ignored if |
|  |  |  | the BeaconSecurityLevel parameter is |
|  |  |  | set to 0x00. |
| BeaconKeySource | Set of 0, 4, | As specified by the | The originator of the key to be used |
|  | or 8 octets | BeaconKeyIdMode | (see 7.4.4.1). This parameter is |
|  |  | parameter | ignored if the BeaconKeyIdMode |
|  |  |  | parameter is ignored or set to 0x00. |
| BeaconKeyIndex | Integer | 0x01–0xff | The index of the key to be used (see |
|  |  |  | 7.4.4.2). This parameter is ignored if |
|  |  |  | the BeaconKeyIdMode parameter is |
|  |  |  | ignored or set to 0x00. |

6.3.13.1.1 Appropriate usage

The MLME-START.request primitive is generated by the next higher layer and issued to its MLME to request that a device start using a new superframe configuration.

6.3.13.1.2 Effect on receipt

If the MLME-START.request primitive is received when macShortAddress is set to 0xffff, the MLME will issue the MLME-START.confirm primitive with a status of NO_SHORT_ADDRESS.

When the CoordRealignment parameter is set to TRUE, the coordinator attempts to transmit a coordinator realignment command frame as described in 5.1.5.3. If the transmission of the coordinator realignment command fails due to a channel access failure, the MLME will not make any changes to the superframe configuration (i.e., no PIB attributes will be changed) and will issue an MLME-START.confirm with a status of CHANNEL_ACCESS_FAILURE. If the coordinator realignment command is successfully transmitted,

| Copyright © 20xx IEEE. All rights reserved. | 225 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | the MLME updates the appropriate PIB parameters with the values of the BeaconOrder, SuperframeOrder, |
| --- | --- |
| 2 | OWPANId, and LogicalChannel parameters, as described in 5.1.5.5, and will issue an MLME- |
| 3 | START.confirm with a status of SUCCESS. |
| 4 |  |

- When the CoordRealignment parameter is set to FALSE, the MLME updates the appropriate PIB parameters

- with the values of the BeaconOrder, SuperframeOrder, OWPANId, and LogicalChannel parameters, as

- described in 5.1.5.5.

| 8 | The address used by the coordinator in its beacon frames is determined by the current value of |  |
| --- | --- | --- |
| 9 |  |  |
| 10 | macShortAddress, which is set by the next higher layer before issuing this primitive. |  |
| 11 |  |  |

- If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for

- this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC

- sublayer will perform outgoing processing on the frame, as described in 7.2.1. If the CoordRealignment

- parameter is set to TRUE, the CoordRealignSecurityLevel, CoordRealignKeyIdMode, CoordRealignKey-

- Source, and CoordRealignKeyIndex parameters will be used to process the MAC command frame. If the

- BeaconOrder parameter indicates a beacon-enabled network, the BeaconSecurityLevel, BeaconKeyIdMode,

- BeaconKeySource, and BeaconKeyIndex parameters will be used to process the beacon frame. If any error

- occurs during outgoing frame processing, the MLME will discard the frame and issue the MLME-

- START.confirm primitive with the error status returned by outgoing frame processing.

| 21 | If the length of the beacon frame exceeds aMaxPHYFrameSize (e.g., due to the additional overhead required |  |
| --- | --- | --- |
| 22 |  |  |
| 23 | for security processing), the MAC sublayer shall discard the beacon frame and issue the MLME- |  |
| 24 | START.confirm primitive with a status of FRAME_TOO_LONG. |  |
| 25 |  |  |

- The MLME shall ignore the StartTime parameter if the BeaconOrder parameter is equal to 15 because this

- indicates a nonbeacon-enabled OWPAN. If the BeaconOrder parameter is less than 15, the MLME examines

- the StartTime parameter to determine the time to begin transmitting beacons; the time is defined in optical

- clocks and is rounded to a backoff slot boundary. If the OWC coordinator parameter is set to TRUE, the

30	MLME ignores the StartTime parameter and begins beacon transmissions immediately. Setting the

- StartTime parameter to 0x000000 also causes the MLME to begin beacon transmissions immediately. If the

- OWPANCoordinator parameter is set to FALSE and the StartTime parameter is nonzero, the MLME

- calculates the beacon transmission time by adding StartTime optical clocks to the time, obtained from the

- local clock, when the MLME receives the beacon of the coordinator through which it is associated. If the

- time calculated causes the outgoing superframe to overlap the incoming superframe, the MLME shall not

- begin beacon transmissions. In this case, the MLME issues the MLME-START.confirm primitive with a

- status of SUPERFRAME_OVERLAP. Otherwise, the MLME then begins beacon transmissions when the

- current time, obtained from the local clock, equals the number of calculated optical clocks.

39

- If the StartTime parameter is nonzero and the MLME is not currently tracking the beacon of the coordinator

- through which it is associated, the MLME will issue the MLME-START.confirm primitive with a status of

- TRACKING_OFF.

43

- On completion of this procedure, the MLME responds with the MLME-START.confirm primitive. If the

- attempt to start using a new superframe configuration was successful, the status parameter will be set to

- SUCCESS. If any parameter is not supported or is out of range, the status parameter will be set to

- INVALID_PARAMETER.

48

49	6.3.13.2 MLME-START.confirm

50

- The MLME-START.confirm primitive reports the results of the attempt to start using a new superframe

- configuration.

53

54

226	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The semantics of the MLME-START.confirm primitive are as follows:

MLME-START.confirm	(

status

)

Table 85 specifies the parameters for the MLME-START.confirm primitive.

Table 85—MLME-START.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, | The result of the attempt to start using an updated |
|  |  | NO_SHORT_ADDRESS, | superframe configuration. |
|  |  | SUPERFRAME_OVERLAP, |  |
|  |  | TRACKING_OFF, |  |
|  |  | INVALID_PARAMETER, |  |
|  |  | COUNTER_ERROR, |  |
|  |  | FRAME_TOO_LONG, |  |
|  |  | UNAVAILABLE_KEY, |  |
|  |  | UNSUPPORTED_SECURITY, |  |
|  |  | CHANNEL_ACCESS_FAILURE |  |

6.3.13.2.1 When generated

The MLME-START.confirm primitive is generated by the MLME and issued to its next higher layer in response to an MLME-START.request primitive. The MLME-START.confirm primitive returns a status of either SUCCESS, indicating that the MAC sublayer has started using the new superframe configuration, or the appropriate error code. The status values are fully described in 6.3.13.1.2 and subclauses referenced by 6.3.13.1.2.

6.3.13.2.2 Appropriate usage

On receipt of the MLME-START.confirm primitive, the next higher layer is notified of the result of its request to start using a new superframe configuration. If the MAC sublayer has been successful, the status parameter will be set to SUCCESS. Otherwise, the status parameter indicates the error.

6.3.13.3 Message sequence chart for updating the superframe configuration

Figure 136 illustrates the sequence of messages necessary for initiating beacon transmissions as a coordinator. Figure 145 illustrates the sequence of messages necessary for the OWC coordinator to start beaconing on a new OWPAN; this figure includes steps taken by the PHY.

6.3.14 Primitive for synchronizing with a coordinator

MLME-SAP synchronization primitives define how synchronization with a coordinator may be achieved and how a loss of synchronization is communicated to the next higher layer.

All devices shall provide an interface for the indication primitive. The request primitive is optional.

| Copyright © 20xx IEEE. All rights reserved. | 227 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 136—Message sequence chart for updating the superframe configuration

6.3.14.1 MLME-SYNC.request

The MLME-SYNC.request primitive requests to synchronize with the coordinator by acquiring and, if specified, tracking its beacons.

The semantics of the MLME-SYNC.request primitive are as follows:

MLME-SYNC.request	(

LogicalChannel,

TrackBeacon

)

Table 86 specifies the parameters for the MLME-SYNC.request primitive.

Table 86—MLME-SYNC.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| LogicalChannel | Integer | Selected from the available logical | The logical channel on which to attempt |
|  |  | channels supported by the PHY | coordinator synchronization. |
| TrackBeacon | Boolean | TRUE or FALSE | TRUE if the MLME is to synchronize with the |
|  |  |  | next beacon and attempt to track all future |
|  |  |  | beacons. FALSE if the MLME is to |
|  |  |  | synchronize with only the next beacon. |

228	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.14.1.1 Appropriate usage

The MLME-SYNC.request primitive is generated by the next higher layer of a device on a beacon-enabled OWPAN and issued to its MLME to synchronize with the coordinator.

6.3.14.1.2 Effect on receipt

If the MLME-SYNC.request primitive is received by the MLME on a beacon-enabled OWPAN, it will first set phyCurrentChannel equal to the values of the LogicalChannel parameters, respectively; both attributes are updated by issuing the PLME-SET.request primitive. If the TrackBeacon parameter is equal to TRUE, the MLME will track the beacon, i.e., enable its receiver just before the expected time of each beacon so that the beacon frame can be processed. If the TrackBeacon parameter is equal to FALSE, the MLME will locate the beacon, but not continue to track it.

If this primitive is received by the MLME while it is currently tracking the beacon, the MLME will not discard the primitive, but rather treat it as a new synchronization request.

If the beacon could not be located either on its initial search or during tracking, the MLME will issue the MLME-SYNC-LOSS.indication primitive with a loss reason of BEACON_LOST.

6.3.15 Primitive for synchronization loss with a coordinator

6.3.15.1 MLME-SYNC-LOSS.indication

The MLME-SYNC-LOSS.indication primitive indicates the loss of synchronization with a coordinator.

The semantics of the MLME-SYNC-LOSS.indication primitive are as follows:

MLME-SYNC-LOSS.indication ( LossReason, OWPANId,

LogicalChannel,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table 87 specifies the parameters for the MLME-SYNC-LOSS.indication primitive.

Table 87—MLME-SYNC-LOSS.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| LossReason | Enumeration | OWPAN_ID_CONFLICT, | The reason that synchronization was lost. |
|  |  | REALIGNMENT, or |  |
|  |  | BEACON_LOST |  |
| OWPANId | Integer | 0x0000–0xffff | The OWPAN identifier with which the |
|  |  |  | device lost synchronization or to which it |
|  |  |  | was realigned. |

| Copyright © 20xx IEEE. All rights reserved. | 229 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 87—MLME-SYNC-LOSS.indication parameters (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| LogicalChannel | Integer | Selected from the available logical | The logical channel on which the device |
|  |  | channels supported by the PHY | lost synchronization or to which it was |
|  |  | (see Table 111). | realigned. |
| SecurityLevel | Integer | 0x00–0x07 | If the primitive was either generated by |
|  |  |  | the device itself following loss of |
|  |  |  | synchronization or generated by the |
|  |  |  | coordinator upon detection of a OWPAN |
|  |  |  | ID conflict, the security level is set to |
|  |  |  | 0x00. |
|  |  |  | If the primitive was generated following |
|  |  |  | the reception of either a coordinator |
|  |  |  | realignment command or a OWPAN ID |
|  |  |  | conflict notification command: |
|  |  |  | The security level purportedly used by |
|  |  |  | the received MAC frame (as defined in |
|  |  |  | Table 97 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | If the primitive was either generated by |
|  |  |  | the device itself following loss of |
|  |  |  | synchronization or generated by the |
|  |  |  | coordinator upon detection of a OWPAN |
|  |  |  | ID conflict, this parameter is ignored. |
|  |  |  | If the primitive was generated following |
|  |  |  | the reception of either a coordinator |
|  |  |  | realignment command or a OWPAN ID |
|  |  |  | conflict notification command: |
|  |  |  | The mode used to identify the key |
|  |  |  | purportedly used by the originator of the |
|  |  |  | received frame (as defined in Table 98 in |
|  |  |  | 7.4.2.2). This parameter is invalid if the |
|  |  |  | SecurityLevel parameter is set to 0x00. |

230	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 87—MLME-SYNC-LOSS.indication parameters (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| KeySource | Set of 0, 4, or 8 | As specified by the KeyIdMode | If the primitive was either generated by |
|  | octets | parameter | the device itself following loss of |
|  |  |  | synchronization or generated by the |
|  |  |  | coordinator upon detection of a OWPAN |
|  |  |  | ID conflict, this parameter is ignored. |
|  |  |  | If the primitive was generated following |
|  |  |  | the reception of either a coordinator |
|  |  |  | realignment command or a OWPAN ID |
|  |  |  | conflict notification command: |
|  |  |  | The originator of the key purportedly |
|  |  |  | used by the originator of the received |
|  |  |  | frame (see 7.4.4.1). This parameter is |
|  |  |  | invalid if the KeyIdMode parameter is |
|  |  |  | invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | If the primitive was either generated by |
|  |  |  | the device itself following loss of |
|  |  |  | synchronization or generated by the |
|  |  |  | coordinator upon detection of a OWPAN |
|  |  |  | ID conflict, this parameter is ignored. |
|  |  |  | If the primitive was generated following |
|  |  |  | the reception of either a coordinator |
|  |  |  | realignment command or a OWPAN ID |
|  |  |  | conflict notification command: |
|  |  |  | The index of the key purportedly used by |
|  |  |  | the originator of the received frame (see |
|  |  |  | 7.4.4.2). This parameter is invalid if the |
|  |  |  | KeyIdMode parameter is invalid or set to |
|  |  |  | 0x00. |

6.3.15.2 Message sequence chart for synchronizing with a coordinator

Figure 137 illustrates the sequence of messages necessary for a device to synchronize with a coordinator. In Figure 137a), a single synchronization request is issued. The MLME then searches for a beacon and, if found, determines whether the coordinator has any data pending for the device. If so, the data are requested as described in 5.1.9.3. In Figure 137b), a track synchronization request is issued. The MLME then searches for a beacon and, if found, attempts to keep track of it using a timer that expires just before the expected time of the next beacon.

For both examples Figure 137a) and Figure 137b), the received beacon frames do not contain payload, and macAutoRequest is set to TRUE. The MLME also checks for any data pending in the coordinator for the device when a beacon frame is received.

6.3.15.2.1 When generated

The MLME -SYNC-LOSS.indication primitive is generated by the MLME of a device and issued to its next higher layer in the event of a loss of synchronization with the coordinator. It is also generated by the MLME of the OWC coordinator and issued to its next higher layer in the event of a OWPAN ID conflict.

| Copyright © 20xx IEEE. All rights reserved. | 231 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | device next | device | coordinator |  |  |
| --- | --- | --- | --- | --- | --- |
| 2 |  |  |  |  |  |
|  | higher layer | MLME | MLME |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 | a) | MLME-SYNC.request(FALSE) |  |  |  |
| 7 |  |  |  | single |  |
| 8 |  |  |  |  |  |
|  |  |  | beacon frame (with data pending) | synchronization |  |
| 9 |  |  |  | request |  |
| 10 |  |  |  |  |  |
| 11 |  |  | data request |  |  |
| 12 |  |  |  |  |  |
| 13 |  |  |  |  |  |
| 14 |  |  |  |  |  |
| 15 | b) | MLME-SYNC.request(TRUE) |  |  |  |
| 16 |  |  |  |  |  |
| 17 |  |  | beacon |  |  |
| 18 |  |  |  |  |  |
| 19 |  |  |  | track |  |
| 20 |  | timer to expire |  |  |  |
| 21 |  | before the next |  | synchronization |  |
|  |  |  |  | request |  |
| 22 |  | beacon |  |  |  |
| 23 |  |  |  |  |  |
| 24 |  |  | beacon |  |  |
| 25 |  |  |  |  |  |
| 26 |  |  |  |  |  |
| 27 |  |  |  |  |  |
| 28 |  |  |  |  |  |
| 29 |  |  |  |  |  |
| 30 | Figure 137—Message sequence chart for synchronizing to a coordinator in a beacon- |  |  |  |  |
| 31 |  |  |  |  |  |
| 32 |  |  |  |  |  |

- If a device that is associated through the OWC coordinator has detected a OWPAN identifier conflict and

- communicated it to the OWC coordinator, the MLME will issue this primitive with the LossReason

- parameter set to OWPAN_ID_CONFLICT. Similarly, if the OWC coordinator receives a OWPAN ID

| 36 | conflict notification command, as specified in 5.4.7, the MLME will issue this primitive with the |
| --- | --- |
| 37 | LossReason parameter set to OWPAN_ID_CONFLICT. |
| 38 |  |

- If a device has received the coordinator realignment command, specified in 5.4.12, from the coordinator

- through which it is associated, the MLME will issue this primitive with the LossReason parameter set to

- REALIGNMENT and the OWPANId, LogicalChannel, and security-related parameters set as described in

- 5.1.5.4.

43

- If a device has not heard the beacon for aMaxLostBeacons consecutive superframes following an MLME-

- SYNC.request primitive, either initially or during tracking, the MLME will issue this primitive with the

- LossReason parameter set to BEACON_LOST. The OWPANId, LogicalChannel parameters shall be set

- according to the coordinator with which synchronization was lost. The SecurityLevel parameter shall be set

- to zero and the KeyIdMode, KeySource, and KeyIndex parameters shall be ignored. If the beacon was being

- tracked, the MLME will not attempt to track the beacon any further.

50

51	6.3.15.2.2 Appropriate usage

52

- On receipt of the MLME-SYNC-LOSS.indication primitive, the next higher layer is notified of a loss of

- synchronization.

232	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.16 Primitives for requesting data from a coordinator

MLME-SAP polling primitives define how to request data from a coordinator.

All devices shall provide an interface for these polling primitives.

6.3.16.1 MLME-POLL.request

The MLME-POLL.request primitive prompts the device to request data from the coordinator.

The semantics of the MLME-POLL.request primitive are as follows:

MLME-POLL.request	(

CoordAddrMode,

CoordOWPANId,

CoordAddress,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table 88 specifies the parameter for the MLME-POLL.request primitive.

Table 88—MLME-POLL.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| CoordAddrMode | Integer | 0x02–0x03 | The addressing mode of the coordinator to |
|  |  |  | which the poll is intended. This parameter can |
|  |  |  | take one of the following values: |
|  |  |  | 2 = 16-bit short address, |
|  |  |  | 3 = 64-bit extended address. |
| CoordOWPANId | Integer | 0x0000–0xfffe | The OWPAN identifier of the coordinator to |
|  |  |  | which the poll is intended. |
| CoordAddress | Device- | As specified by the | The address of the coordinator to which the |
|  | Address | CoordAddrMode | poll is intended. |
|  |  | parameter |  |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in |
|  |  |  | Table 97 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used |
|  |  |  | (as defined in Table 98 in 7.4.2.2). This |
|  |  |  | parameter is ignored if the SecurityLevel |
|  |  |  | parameter is set to 0x00. |
| KeySource | Set of 0, | As specified by the | The originator of the key to be used (see |
|  | 4, or 8 | KeyIdMode parameter | 7.4.4.1). This parameter is ignored if the |
|  | octets |  | KeyIdMode parameter is ignored or set to |
|  |  |  | 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). |
|  |  |  | This parameter is ignored if the KeyIdMode |
|  |  |  | parameter is ignored or set to 0x00. |

| Copyright © 20xx IEEE. All rights reserved. | 233 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 6.3.16.1.1 Appropriate usage |
| --- | --- |
| 2 |  |

- The MLME-POLL.request primitive is generated by the next higher layer and issued to its MLME when

- data are to be requested from a coordinator.

5

6	6.3.16.1.2 Effect on receipt

7

- On receipt of the MLME-POLL.request primitive, the MLME generates and sends a data request command,

- as specified in 5.4.4. If the poll is directed to the coordinator, the data request command may be generated

- without any destination address information present. Otherwise, the data request command is always

- generated with the destination address information in the CoordOWPANId and CoordAddress parameters.

- If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for

- this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC

15	sublayer will perform outgoing processing on the frame based on the CoordAddress, SecurityLevel,

- KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs during

- outgoing frame processing, the MLME will discard the frame and issue the MLME-POLL.confirm primitive

- with the error status returned by outgoing frame processing.

19

- If the data request command cannot be sent due to an unslotted random access algorithm failure, the MLME

- will issue the MLME-POLL.confirm primitive with a status of CHANNEL_ACCESS_FAILURE.

22

- If the MLME successfully transmits a data request command, the MLME will expect an acknowledgment in

- return. If an acknowledgment is not received, the MLME will issue the MLME-POLL.confirm primitive

- with a status of NO_ACK (see 5.1.9.4).

26

27 If an acknowledgment is received, the MLME will request that the PHY enable its receiver if the frame 28 pending subfield of the acknowledgment frame is set to one. If the frame pending subfield of the

- acknowledgment frame is set to zero, the MLME will issue the MLME-POLL.confirm primitive with a

- status of NO_DATA.

31

- If a frame is received from the coordinator with a zero length payload or if the frame is a MAC command

- frame, the MLME will issue the MLME-POLL.confirm primitive with a status of NO_DATA. If a frame is

34	received  from  the  coordinator  with  nonzero  length  payload,  the  MLME  will  issue  the  MLME-

- POLL.confirm primitive with a status of SUCCESS. In this case, the actual data are indicated to the next

- higher layer using the MCPS-DATA.indication primitive as specified in 6.2.3.

37

- If a frame is not received within macMaxFrameTotalWaitTime CAP optical clocks in a beacon-enabled

- OWPAN, or optical clocks in a nonbeacon-enabled OWPAN, even though the acknowledgment to the data

- request command has its frame pending subfield set to one, the MLME will issue the MLME-POLL.confirm

- primitive with a status of NO_DATA.

42

- If any parameter in the MLME-POLL.request primitive is not supported or is out of range, the MLME will

- issue the MLME-POLL.confirm primitive with a status of INVALID_PARAMETER.

| 45 | 6.3.16.2 MLME-POLL.confirm |  |  |
| --- | --- | --- | --- |
| 46 |  |  |  |
| 47 | The MLME-POLL.confirm primitive reports the results of a request to poll the coordinator for data. |  |  |
| 48 |  |  |  |
| 49 | The semantics of the MLME-POLL.confirm primitive are as follows: |  |  |
| 50 |  |  |  |
| 51 | MLME-POLL.confirm | ( |  |
| 52 |  | status |  |
| 53 |  | ) |  |
| 54 |  |  |  |
|  | 234 | Copyright © 20xx IEEE. All rights reserved. |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 89 specifies the parameters for the MLME-POLL.confirm primitive.

Table 89—MLME-POLL.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Integer | SUCCESS, | The status of the data request. |
|  |  | CHANNEL_ACCESS_FAILURE, |  |
|  |  | NO_ACK, NO_DATA, |  |
|  |  | COUNTER_ERROR, |  |
|  |  | FRAME_TOO_LONG, |  |
|  |  | UNAVAILABLE_KEY, |  |
|  |  | UNSUPPORTED_SECURITY or |  |
|  |  | INVALID_PARAMETER |  |

6.3.16.2.1 When generated

The MLME-POLL.confirm primitive is generated by the MLME and issued to its next higher layer in response to an MLME-POLL.request primitive. If the request was successful, the status parameter will be equal to SUCCESS, indicating a successful poll for data. Otherwise, the status parameter indicates the appropriate error code. The status values are fully described in 6.3.16.1.2 and the subclauses referenced by 6.3.16.1.2.

6.3.16.2.2 Appropriate usage

On receipt of the MLME-POLL.confirm primitive, the next higher layer is notified of the status of the procedure to request data from the coordinator.

6.3.16.3 Message sequence chart for requesting data from a coordinator

Figure 138 illustrates the sequence of messages necessary, including the layer behavior of the device and the over-the-air interface, for a device to request data from a coordinator.

In both scenarios Figure 138a) and Figure 138b), a poll request is issued to the MLME, which then sends a data request command to the coordinator. In Figure 138a), the corresponding acknowledgment has the frame pending (FP) subfield set to zero and the MLME issues the poll request confirmation immediately. In Figure 138b), the corresponding acknowledgment has the frame pending subfield set to one and the MLME enables the receiver in anticipation of the data frame from the coordinator. On receipt of this data frame, the MLME issues a poll request confirmation followed by a data indication containing the data of the received frame.

6.4 MAC cons tants and PIB attributes

This subclause specifies the constants and attributes required by the MAC sublayer.

6.4.1 MAC constants

The constants that define the characteristics of the MAC sublayer are presented in Table 90.

| Copyright © 20xx IEEE. All rights reserved. | 235 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 138—Message sequence chart for requesting data from a coordinator

Table 90—MAC sublayer constants

| Constant | Description | Value |
| --- | --- | --- |
| aBaseSlotDuration | The number of optical clocks forming a superframe | 60 |
|  | slot when the superframe order is equal to 0 (see |  |
|  | 5.1.1). |  |
| aBaseSuperframeDuration | The number of optical clocks forming a superframe | aBaseSlotDuration  |
|  | when the superframe order is equal to 0. | aNumSuperframeSlots |
| aExtendedAddress | The 64-bit (IEEE) address assigned to the device. | Device specific |
| aGTSDescPersistenceTime | The number of superframes in which a GTS descriptor | 4 |
|  | exists in the beacon frame of the coordinator. |  |
| aMaxBeaconOverhead | The maximum number of octets added by the MAC | 75 |
|  | sublayer to the MSDU of a beacon frame. |  |
| aMaxBeaconPayloadLength | The maximum size, in octets, of a beacon payload. | aMaxPHYFrameSize – |
|  |  | aMaxBeaconOverhead |
| aMaxLostBeacons | The number of consecutive lost beacons that will | 4 |
|  | cause the MAC sublayer of a receiving device to |  |
|  | declare a loss of synchronization. |  |
| aMaxMACSafePayloadSize | The maximum number of octets that can be | aMaxPHYFrameSize – |
|  | transmitted in the MSDU field of an unsecured MAC | aMaxMPDUUnsecure |
|  | frame that will be guaranteed not to exceed | dOverhead |
|  | aMaxPHYFrameSize. |  |
| aMaxMACPayloadSize | The maximum number of octets that can be | aMaxPHYFrameSize – |
|  | transmitted in the MSDU field. | aMinMPDUOverhead |

236	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 90—MAC sublayer constants (continued)

| Constant | Description | Value |
| --- | --- | --- |
| aMaxMPDUUnsecuredOverhead | The maximum number of octets added by the MAC | 25 |
|  | sublayer to the PSDU without security. |  |
| aMaxSIFSFrameSize | The maximum size of an MPDU, in octets, that can be | 18 |
|  | followed by a SIFS period. |  |
| aMinCAPLength | The minimum number of optical clocks forming the | 440 |
|  | CAP. This ensures that MAC commands can still be |  |
|  | transferred to devices when GTSs are being used. |  |
|  | An exception to this minimum shall be allowed for the |  |
|  | accommodation of the temporary increase in the |  |
|  | beacon frame length needed to perform GTS |  |
|  | maintenance (as defined in 5.2.2.1.5). |  |
| aMinMPDUOverhead | The minimum number of octets added by the MAC | 9 |
|  | sublayer to the PSDU. |  |
| aNumSuperframeSlots | The number of slots contained in any superframe. | 16 |
| aUnitBackoffPeriod | The number of optical clocks forming the basic time | 20 |
|  | period used by the unslotted random access algorithm. |  |

6.4.2 MAC PIB attributes

The MAC PIB comprises the attributes required to manage the MAC sublayer of a device. The attributes contained in the MAC PIB are presented in Table 91 and Table 99. Attributes marked with a dagger (†) are read-only attributes (i.e., attribute can only be set by the MAC sublayer), which can be read by the next higher layer using the MLME-GET.request primitive. All other attributes can be read or written by the next higher layer using the MLME-GET.request or MLME-SET.request primitives, respectively. Higher layers may impose additional constraints on read/write operations, without making devices non-compliant. Attributes marked with a diamond () are optional for a device (i.e., not operating as a coordinator).

The read- only attribute macAckWaitDuration is dependent on a combination of constants and PHY PIB attributes. The formula for relating the constants and attributes is shown in Equation (4).

| AckWaitTime =backoff period + aTurnaroundTime-RX-TX + clock period  numSymAckFrame | (4) |
| --- | --- |

where numSymAckFrame is the number of bits in the acknowledgment frame and is equal to 103 for PHY I and II and 111 for PHY III. For B-ACK mode, the AckWaitTime would be larger, depending on the number of acknowledgments in the B-ACK mode as explained in 5.2.2.2. The clock period is obtained via the optical rates specified in Table 106, Table 107, and Table 108.

The attribute macMaxFrameTotalWaitTime may be set by the next higher layer and is dependent upon a combination of PHY and MAC PIB attributes and constants. The formula relating the attributes and constants is shown in Equation (5).

| macMaxFrameTotalWaitTime = |  |  |  |  |  |  | (5) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  m – 1 |  |  |  |  |  |  |  |
|  |  | macMinBE + k | +  2 | macMaxBE | – 1 |  macMaxCSMABackoffs – m |  aUnitBackoffPeriod + phyMaxFrameDuration |  |
|  | 2 |  |  |  |  |  |  |  |
|  |  k = 0 |  |  |  |  |  |  |  |

where

| Copyright © 20xx IEEE. All rights reserved. | 237 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

m is min(macMaxBE-macMinBE, macMaxCSMABackoffs)

Table 91—MAC PIB attributes

| Attribute | Identifier | Type | Range | Description | Default |  |
| --- | --- | --- | --- | --- | --- | --- |
| macAckWaitDuration† | 0x40 | Integer | Refer to | The maximum number of | Dependent |  |
|  |  |  | Equation (4) | optical clocks to wait for an | on |  |
|  |  |  |  | acknowledgment frame to | currently |  |
|  |  |  |  | arrive following a transmitted | selected |  |
|  |  |  |  | data frame. | PHY. |  |
|  |  |  |  | This value is dependent on |  |  |
|  |  |  |  | the supported PHY, which |  |  |
|  |  |  |  | determines both the selected |  |  |
|  |  |  |  | logical channel. The |  |  |
|  |  |  |  | calculated value is the time to |  |  |
|  |  |  |  | commence transmitting the |  |  |
|  |  |  |  | ACK plus the length of the |  |  |
|  |  |  |  | ACK frame. The |  |  |
|  |  |  |  | commencement time is |  |  |
|  |  |  |  | described in 5.1.9.4.2. |  |  |
| macAssociatedOWPAN- | 0x41 | Boolean | TRUE or | Indication of whether the | FALSE |  |
| Coord |  |  | FALSE | device is associated to the |  |  |
|  |  |  |  | OWPAN through the |  |  |
|  |  |  |  | coordinator. A value of |  |  |
|  |  |  |  | TRUE indicates the device |  |  |
|  |  |  |  | has associated through the |  |  |
|  |  |  |  | coordinator. Otherwise, the |  |  |
|  |  |  |  | value is set to FALSE. |  |  |
| macAssociation- | 0x42 | Boolean | TRUE or | Indication of whether a | FALSE |  |
| Permit |  |  | FALSE | coordinator is currently |  |  |
|  |  |  |  | allowing association. A value |  |  |
|  |  |  |  | of TRUE indicates that |  |  |
|  |  |  |  | association is permitted. |  |  |
| macAutoRequest | 0x43 | Boolean | TRUE or | Indication of whether a | TRUE |  |
|  |  |  | FALSE | device automatically sends a |  |  |
|  |  |  |  | data request command if its |  |  |
|  |  |  |  | address is listed in the beacon |  |  |
|  |  |  |  | frame. A value of TRUE |  |  |
|  |  |  |  | indicates that the data request |  |  |
|  |  |  |  | command is automatically |  |  |
|  |  |  |  | sent. |  |  |
|  |  |  |  | This attribute also affects the |  |  |
|  |  |  |  | generation of the MLME- |  |  |
|  |  |  |  | BEACON- |  |  |
|  |  |  |  | NOTIFY.indication primitive |  |  |
|  |  |  |  | (see 6.3.3.1.1). |  |  |
| macBeaconPayload | 0x44 | Set of | — | The contents of the beacon | NULL |  |
|  |  | octets |  | payload. |  |  |
| macBeaconPayload- | 0x45 | Integer | 0 – aMax- | The length, in octets, of the |  |  |
| Length |  |  | Beacon- | beacon payload. | 0 |  |
|  |  |  | PayloadLength |  |  |  |

238	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 91—MAC PIB attributes (continued)

| Attribute | Identifier | Type | Range | Description | Default |  |
| --- | --- | --- | --- | --- | --- | --- |
| macBeaconOrder | 0x46 | Integer | 0–15 | Specification of how often the |  |  |
|  |  |  |  | coordinator transmits its |  |  |
|  |  |  |  | beacon. If BO = 15, the |  |  |
|  |  |  |  | coordinator will not transmit | 15 |  |
|  |  |  |  | a periodic beacon. Refer to |  |  |
|  |  |  |  | 5.1.1 for an explanation of the |  |  |
|  |  |  |  | relationship between the |  |  |
|  |  |  |  | beacon order and the beacon |  |  |
|  |  |  |  | interval. |  |  |
| macBeaconTxTime† | 0x47 | Integer | 0x000000– | The time that the device |  |  |
|  |  |  | 0xffffff | transmitted its last beacon |  |  |
|  |  |  |  | frame, in symbol periods. The |  |  |
|  |  |  |  | measurement shall be taken at |  |  |
|  |  |  |  | the same symbol boundary |  |  |
|  |  |  |  | within every transmitted |  |  |
|  |  |  |  | beacon frame, the location of |  |  |
|  |  |  |  | which is implementation | 0x000000 |  |
|  |  |  |  | specific. |  |  |
|  |  |  |  | This is a 24-bit value, and the |  |  |
|  |  |  |  | precision of this value shall |  |  |
|  |  |  |  | be a minimum of 20 bits, with |  |  |
|  |  |  |  | the lowest four bits being the |  |  |
|  |  |  |  | least significant. |  |  |
| macBSN | 0x48 | Integer | 0x00–0xff | The sequence number added | Random |  |
|  |  |  |  | to the transmitted beacon | value from |  |
|  |  |  |  | frame. | within the |  |
|  |  |  |  |  | range |  |
| macCoordExtended- | 0x49 | IEEE | An extended | The 64-bit address of the |  |  |
| Address |  | address | 64-bit IEEE | coordinator through which | — |  |
|  |  |  | address | the device is associated. |  |  |
| macCoordShort-Address | 0x4a | Integer | 0x0000–0xffff | The 16-bit short address |  |  |
|  |  |  |  | assigned to the coordinator |  |  |
|  |  |  |  | through which the device is |  |  |
|  |  |  |  | associated. A value of | 0xffff |  |
|  |  |  |  | 0xfffe indicates that the |  |  |
|  |  |  |  | coordinator is only using its |  |  |
|  |  |  |  | 64-bit extended address. A |  |  |
|  |  |  |  | value of 0xffff indicates that |  |  |
|  |  |  |  | this value is unknown. |  |  |
| macDSN | 0x4b | Integer | 0x00–0xff | The sequence number added | Random |  |
|  |  |  |  | to the transmitted data or | value from |  |
|  |  |  |  | MAC command frame. | within the |  |
|  |  |  |  |  | range |  |
| macGTSPermit | 0x4c | Boolean | TRUE or | TRUE if the coordinator is to |  |  |
|  |  |  | FALSE | accept GTS requests. FALSE | TRUE |  |
|  |  |  |  | otherwise. |  |  |
| macMaxBE | 0x4d | Integer | 3–15 | The maximum value of the | 5 |  |
|  |  |  |  | backoff exponent, BE, in the |  |  |
|  |  |  |  | unslotted random access |  |  |
|  |  |  |  | algorithm. Refer to 5.1.2.1 for |  |  |
|  |  |  |  | a detailed explanation of the |  |  |
|  |  |  |  | backoff exponent. |  |  |

| Copyright © 20xx IEEE. All rights reserved. | 239 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 91—MAC PIB attributes (continued)

| Attribute | Identifier | Type | Range | Description | Default |  |
| --- | --- | --- | --- | --- | --- | --- |
| macMaxCSMABackoffs | 0x4e | Integer | 0–5 | The maximum number of |  |  |
|  |  |  |  | backoffs the unslotted | 4 |  |
|  |  |  |  | random access algorithm will |  |  |
|  |  |  |  | attempt before declaring a |  |  |
|  |  |  |  | channel access failure. |  |  |
| macMaxFrameTotal- | 0x4f | Integer | Refer to | The maximum number of |  |  |
| WaitTime |  |  | Equation (5) | optical clocks in a beacon- |  |  |
|  |  |  |  | enabled OWPAN, or in a |  |  |
|  |  |  |  | nonbeacon-enabled OWPAN, |  |  |
|  |  |  |  | to wait either for a frame |  |  |
|  |  |  |  | intended as a response to a |  |  |
|  |  |  |  | data request frame or for a |  |  |
|  |  |  |  | broadcast frame following a | Dependent |  |
|  |  |  |  | beacon with the frame |  |  |
|  |  |  |  |  | on |  |
|  |  |  |  | pending subfield set to one. |  |  |
|  |  |  |  |  | currently |  |
|  |  |  |  | This attribute, which shall | selected |  |
|  |  |  |  |  | PHY |  |
|  |  |  |  | only be set by the next higher |  |  |
|  |  |  |  | layer, is dependent upon |  |  |
|  |  |  |  | macMinBE, macMaxBE, |  |  |
|  |  |  |  | macMaxCSMABackoffs and |  |  |
|  |  |  |  | the number of optical clocks |  |  |
|  |  |  |  | per octet. Refer to 6.4.2 for |  |  |
|  |  |  |  | the formula relating the |  |  |
|  |  |  |  | attributes. |  |  |
| macMaxFrameRetries | 0x50 | Integer | 0–7 | The maximum number of | 3 |  |
|  |  |  |  | retries allowed after a |  |  |
|  |  |  |  | transmission failure. |  |  |
| macMaxRABackoffs | TBD | Integer | 0-5 | The maximum number of |  |  |
|  |  |  |  | backoffs the unslotted | 4 |  |
|  |  |  |  | random access algorithm will |  |  |
|  |  |  |  | attempt before declaring a |  |  |
|  |  |  |  | channel access failure. |  |  |
| macMinBE | 0x51 | Integer | 0–macMaxBE | The minimum value of the |  |  |
|  |  |  |  | backoff exponent (BE) in the |  |  |
|  |  |  |  | unslotted random access | 3 |  |
|  |  |  |  | algorithm. Refer to 5.1.2.1 for |  |  |
|  |  |  |  | a detailed explanation of the |  |  |
|  |  |  |  | backoff exponent. |  |  |
| macMinLIFSPeriod† | 0x52 | Integer | As defined in | The minimum number of | Dependent |  |
|  |  |  | Table 112 in | optical clocks forming a LIFS | on |  |
|  |  |  | 8.3.4 | period. | currently |  |
|  |  |  |  |  | selected |  |
|  |  |  |  |  | PHY. |  |
| macMinSIFSPeriod† | 0x53 | Integer | As defined in | The minimum number of | Dependent |  |
|  |  |  | Table 112 in | optical clocks forming a SIFS | on |  |
|  |  |  | 8.3.4 | period. | currently |  |
|  |  |  |  |  | selected |  |
|  |  |  |  |  | PHY. |  |
| macOWPANId | 0x54 | Integer | 0x0000–0xffff | The 16-bit identifier of the |  |  |
|  |  |  |  | OWPAN on which the device |  |  |
|  |  |  |  | is operating. If this value is | 0xffff |  |
|  |  |  |  | 0xffff, the device is not |  |  |
|  |  |  |  | associated. |  |  |

240	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 91—MAC PIB attributes (continued)

| Attribute | Identifier | Type | Range | Description | Default |  |
| --- | --- | --- | --- | --- | --- | --- |
| macResponseWaitTime | 0x55 | Integer | 2–64 | The maximum time, in | 32 |  |
|  |  |  |  | multiples of |  |  |
|  |  |  |  | aBaseSuperframeDuration, a |  |  |
|  |  |  |  | device shall wait for a |  |  |
|  |  |  |  | response command frame to |  |  |
|  |  |  |  | be available following a |  |  |
|  |  |  |  | request command frame. |  |  |
| macRxOnWhenIdle | 0x56 | Boolean | TRUE or | Indication of whether the |  |  |
|  |  |  | FALSE | MAC sublayer is to enable its |  |  |
|  |  |  |  | receiver during idle periods. |  |  |
|  |  |  |  | For a beacon-enabled |  |  |
|  |  |  |  | OWPAN, this attribute is | FALSE |  |
|  |  |  |  | relevant only during the CAP |  |  |
|  |  |  |  | of the incoming superframe. |  |  |
|  |  |  |  | For a nonbeacon-enabled |  |  |
|  |  |  |  | OWPAN, this attribute is |  |  |
|  |  |  |  | relevant at all times. |  |  |
| macSecurityEnabled | 0x57 | Boolean | TRUE or | Indication of whether the |  |  |
|  |  |  | FALSE | MAC sublayer has security |  |  |
|  |  |  |  | enabled. |  |  |
|  |  |  |  | A value of TRUE indicates | TRUE |  |
|  |  |  |  | that security is enabled, while |  |  |
|  |  |  |  | a value of FALSE indicates |  |  |
|  |  |  |  | that security is disabled. |  |  |
| macShortAddress | 0x58 | Integer | 0x0000–0xffff | The 16-bit address that the |  |  |
|  |  |  |  | device uses to communicate |  |  |
|  |  |  |  | in the OWPAN. If the device |  |  |
|  |  |  |  | is the coordinator, this value |  |  |
|  |  |  |  | shall be chosen before a |  |  |
|  |  |  |  | OWPAN is started. |  |  |
|  |  |  |  | Otherwise, the address is |  |  |
|  |  |  |  | allocated by a coordinator | 0xffff |  |
|  |  |  |  | during association. |  |  |
|  |  |  |  | A value of 0xfffe indicates |  |  |
|  |  |  |  | that the device has associated |  |  |
|  |  |  |  | but has not been allocated an |  |  |
|  |  |  |  | address. A value of 0xffff |  |  |
|  |  |  |  | indicates that the device does |  |  |
|  |  |  |  | not have a short address. |  |  |
| macSuperframe-Order† | 0x59 | Integer | 0–15 | The length of the active | 15 |  |
|  |  |  |  | portion of the outgoing |  |  |
|  |  |  |  | superframe, including the |  |  |
|  |  |  |  | beacon frame. If superframe |  |  |
|  |  |  |  | order, SO, = 15, the |  |  |
|  |  |  |  | superframe will not be active |  |  |
|  |  |  |  | following the beacon. Refer |  |  |
|  |  |  |  | to 5.1.1 for an explanation of |  |  |
|  |  |  |  | the relationship between the |  |  |
|  |  |  |  | superframe order and the |  |  |
|  |  |  |  | superframe duration. |  |  |

| Copyright © 20xx IEEE. All rights reserved. | 241 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 91—MAC PIB attributes (continued)

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macTimestamp- | 0x5a | Boolean | TRUE or | Indication of whether the | Imple- |
| Supported† |  |  | FALSE | MAC sublayer supports the | mentation |
|  |  |  |  | optional time stamping | specific |
|  |  |  |  | feature for incoming and |  |
|  |  |  |  | outgoing data frames. |  |
| macTransaction- | 0x5b | Integer | 0x0000–0xffff | The maximum time (in unit | 0x01f4 |
| PersistenceTime |  |  |  | periods) that a transaction is |  |
|  |  |  |  | stored by a coordinator and |  |
|  |  |  |  | indicated in its beacon. |  |
|  |  |  |  | The unit period is governed |  |
|  |  |  |  | by macBeaconOrder, BO, as |  |
|  |  |  |  | follows: For 0  BO  14, the |  |
|  |  |  |  | unit period will be aBase- |  |
|  |  |  |  | SuperframeDuration * 2BO. |  |
|  |  |  |  | For BO = 15, the unit period |  |
|  |  |  |  | will be aBaseSuperframe- |  |
|  |  |  |  | Duration. |  |
| macDim | 0x5c | Integer | 0–1000 | Percentage dimming; 0 is 0% | 0 |
|  |  |  |  | visibility and 1000 is 100% |  |
|  |  |  |  | visibility. |  |
| macNumAcks | 0x5d | Integer | 0–15 | Maximum number of times | 3 |
|  |  |  |  | not receiving ACKs to trigger |  |
|  |  |  |  | fast link recovery procedure. |  |
| macLinkTimeOut | 0x5e | Integer | 0–255 | A timer initiated when the | 63 |
|  |  |  |  | link recovery procedure is |  |
|  |  |  |  | triggered. If the timer expires |  |
|  |  |  |  | while the device has not |  |
|  |  |  |  | received any fast link |  |
|  |  |  |  | recovery response (FLR RSP) |  |
|  |  |  |  | signal since the fast link |  |
|  |  |  |  | recovery procedure is |  |
|  |  |  |  | triggered, the device assumes |  |
|  |  |  |  | that the link is broken and |  |
|  |  |  |  | cannot be recovered. The |  |
|  |  |  |  | range for macLinkTimeOut is |  |
|  |  |  |  | defined in terms of the |  |
|  |  |  |  | number of superframes. |  |
| macDimOverrideRequest | 0x5f | Boolean | TRUE or | shall be set to ‘1’ after OWC | 0 |
|  |  |  | FALSE | device association and shall |  |
|  |  |  |  | be set to ‘0’ after the OWC |  |
|  |  |  |  | device disassociation |  |
| macDimPWMOverrideRe | 0x60 | Boolean | TRUE or | shall be set to ‘1’ to inform | 0 |
| quest |  |  | FALSE | the dimmer circuit that the |  |
|  |  |  |  | OWC device will be |  |
|  |  |  |  | responsible for dimming and |  |
|  |  |  |  | to disable any PWM circuit |  |
|  |  |  |  | present in the dimmer |  |
| macDimDataFailureIndic | 0x61 | Boolean | TRUE or | shall be set to ‘1’ when the | 0 |
| ation |  |  | FALSE | device is unable to perform |  |
|  |  |  |  | data communication under |  |
|  |  |  |  | dimming |  |

242	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 91—MAC PIB attributes (continued)

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macDuringASSOCColor | 0x62 | Unsigned | 0–255 | Use macDuringASSOCColor | 0 |
|  |  |  |  | for the color assignment of |  |
|  |  |  |  | the CVD frame when the |  |
|  |  |  |  | color function for the |  |
|  |  |  |  | association MAC state |  |
|  |  |  |  | indication between MLME- |  |
|  |  |  |  | ASSOCIATE.request and |  |
|  |  |  |  | MLME- |  |
|  |  |  |  | ASSOCIATE.confirm is used |  |
|  |  |  |  | by the CVD frame. |  |
|  |  |  |  | The unsigned integer is the |  |
|  |  |  |  | index for the look-up table for |  |
|  |  |  |  | the color function table, |  |
|  |  |  |  | phyColorFunction, as shown |  |
|  |  |  |  | in Table 144, PHY PIB |  |
|  |  |  |  | attributes. |  |
| macDuringDISASSOCCo | 0x63 | Unsigned | 0–255 | Use | 0 |
| lor |  |  |  | macDuringDISASSOCColor |  |
|  |  |  |  | for the color assignment of |  |
|  |  |  |  | the CVD frame when the |  |
|  |  |  |  | color function for the |  |
|  |  |  |  | disassociation MAC state |  |
|  |  |  |  | indication between MLME- |  |
|  |  |  |  | DISASSOCIATE.request and |  |
|  |  |  |  | MLME- |  |
|  |  |  |  | DISASSOCIATE.confirm is |  |
|  |  |  |  | used by the CVD frame. |  |
|  |  |  |  | The unsigned integer is the |  |
|  |  |  |  | index for the look-up table for |  |
|  |  |  |  | the color function table, |  |
|  |  |  |  | phyColorFunction, as shown |  |
|  |  |  |  | in Table 144, PHY PIB |  |
|  |  |  |  | attributes. |  |
| macDuringSCANColor | 0x64 | Unsigned | 0–255 | Use macDuringSCANColor | 0 |
|  |  |  |  | for the color assignment of |  |
|  |  |  |  | the CVD frame when the |  |
|  |  |  |  | color function for the scan |  |
|  |  |  |  | MAC state indication |  |
|  |  |  |  | between MLME- |  |
|  |  |  |  | SCAN.request and MLME- |  |
|  |  |  |  | SCAN.confirm is used by the |  |
|  |  |  |  | CVD frame. |  |
|  |  |  |  | The unsigned integer is the |  |
|  |  |  |  | index for the look-up table for |  |
|  |  |  |  | the color function table, |  |
|  |  |  |  | phyColorFunction, as shown |  |
|  |  |  |  | in Table 144, PHY PIB |  |
|  |  |  |  | attributes. |  |

| Copyright © 20xx IEEE. All rights reserved. | 243 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 91—MAC PIB attributes (continued)

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macColorReceived | 0x65 | Unsigned | 0–255 | Use macColorReceived for | 0 |
|  |  |  |  | the color assignment oft he |  |
|  |  |  |  | CVD Frame when the ACK |  |
|  |  |  |  | frame is sent and the color |  |
|  |  |  |  | function for the ACK state |  |
|  |  |  |  | indication is used by the CVD |  |
|  |  |  |  | frame. |  |
|  |  |  |  | The unsigned integer is the |  |
|  |  |  |  | index for the look-up table for |  |
|  |  |  |  | the color function table, |  |
|  |  |  |  | phyColorFunction, as shown |  |
|  |  |  |  | in Table 144, PHY PIB |  |
|  |  |  |  | attributes. |  |
| macColorNotReceived | 0x66 | Unsigned | 0–255 | Use macColorNotReceived | 0 |
|  |  |  |  | for the color assignment of |  |
|  |  |  |  | the CVD Frame when the |  |
|  |  |  |  | ACK frame is not sent but the |  |
|  |  |  |  | color function for the non- |  |
|  |  |  |  | ACK state indication is used |  |
|  |  |  |  | by the CVD frame. |  |
|  |  |  |  | The unsigned integer is the |  |
|  |  |  |  | index for the look-up table for |  |
|  |  |  |  | the color function table, |  |
|  |  |  |  | phyColorFunction, as shown |  |
|  |  |  |  | in Table 144, PHY PIB |  |
|  |  |  |  | attributes. |  |
| macCQIColorLFER | 0x67 | Unsigned | 0–255 | Use macCQIColorLFER for | 0 |
|  |  |  |  | the color assignment of the |  |
|  |  |  |  | CVD frame when the color |  |
|  |  |  |  | function for the channel |  |
|  |  |  |  | quality indication showing |  |
|  |  |  |  | the low FER is used by the |  |
|  |  |  |  | CVD frame. |  |
|  |  |  |  | The unsigned integer is the |  |
|  |  |  |  | index for the look-up table for |  |
|  |  |  |  | the color function table, |  |
|  |  |  |  | phyColorFunction, as shown |  |
|  |  |  |  | in Table 144, PHY PIB |  |
|  |  |  |  | attributes. |  |
| macCQIColorMFER | 0x68 | Unsigned | 0–255 | Use macCQIColorMFER for | 0 |
|  |  |  |  | the color assignment of the |  |
|  |  |  |  | CVD frame when the color |  |
|  |  |  |  | function for the channel |  |
|  |  |  |  | quality indication showing |  |
|  |  |  |  | the medium FER is used by |  |
|  |  |  |  | the CVD frame. |  |
|  |  |  |  | The unsigned integer is the |  |
|  |  |  |  | index for the look-up table for |  |
|  |  |  |  | the color function table, |  |
|  |  |  |  | phyColorFunction, as shown |  |
|  |  |  |  | in Table 144, PHY PIB |  |
|  |  |  |  | attributes. |  |

244	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 91—MAC PIB attributes (continued)

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macCQIColorHFER | 0x69 | Unsigned | 0–255 | Use macCQIColorHFER for | 0 |
|  |  |  |  | the color assignment of the |  |
|  |  |  |  | CVD frame when the color |  |
|  |  |  |  | function for the channel |  |
|  |  |  |  | quality indication showing |  |
|  |  |  |  | the high FER is used by the |  |
|  |  |  |  | CVD frame. |  |
|  |  |  |  | The unsigned integer is the |  |
|  |  |  |  | index for the look-up table for |  |
|  |  |  |  | the color function table, |  |
|  |  |  |  | phyColorFunction, as shown |  |
|  |  |  |  | in Table 144, PHY PIB |  |
|  |  |  |  | attributes. |  |
| macCFAppColor | 0x6a | Unsigned | 0–255 | Use macCFAppColor for the | 0 |
|  |  |  |  | color assignment of the CVD |  |
|  |  |  |  | frame when the color function |  |
|  |  |  |  | for the indication of |  |
|  |  |  |  | application-dependent |  |
|  |  |  |  | information is used by the |  |
|  |  |  |  | CVD frame. |  |
|  |  |  |  | The unsigned integer is the |  |
|  |  |  |  | index for the look-up table for |  |
|  |  |  |  | the color function table, |  |
|  |  |  |  | phyColorFunction, as shown |  |
|  |  |  |  | in Table 144, PHY PIB |  |
|  |  |  |  | attributes. |  |
| macColorStabilization | 0x6b | Binary | 00–11 | The color stabilization action | 0 |
|  |  | Integer |  | entailed when receiving CVD |  |
|  |  |  |  | frames. The information for |  |
|  |  |  |  | setting these two bits is found |  |
|  |  |  |  | in Table 42. |  |
| macColorStabilizationTi | 0x6c | Integer | 0x0–0xffffffff | Minimum time between two | 0x004000 |
| mer |  |  |  | stabilization measurements | 00 |
|  |  |  |  | (see 8.6) that are send back to |  |
|  |  |  |  | the corresponding CSK Tx. |  |
|  |  |  |  | The time is measured in |  |
|  |  |  |  | multiples of |  |
|  |  |  |  | aMaxPHYFrameSize frames |  |
|  |  |  |  | for color stabilization. |  |
| macUseDimmedOOKmod | 0x6d | Boolean | TRUE or | Shall be set to 1 when | 0 |
| e |  |  | FALSE | dimming is to be performed |  |
|  |  |  |  | in the dimmed OOK mode in |  |
|  |  |  |  | conjunction with OOK. |  |
| macTimeStampOffset | 0x6e | Octet | 0x00–0xff | The location of the time | 0 |
|  |  |  |  | stamp after the end of the |  |
|  |  |  |  | preamble in optical clocks. |  |
| macUseBlinkingNotificati | 0x6f | Boolean | TRUE or | Shall be set to 0 when | 1 |
| on |  |  | FALSE | blinking notification is to be |  |
|  |  |  |  | performed. |  |

| Copyright © 20xx IEEE. All rights reserved. | 245 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 91—MAC PIB attributes (continued)

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macBlinkingNotificationF | 0x70 | Integer | 0–10 | The frequency of blinking | 0 |
| requency |  |  |  | notification |  |
|  |  |  |  | 0: 0.25Hz |  |
|  |  |  |  | 1: 0.5Hz |  |
|  |  |  |  | 2: 0.75Hz |  |
|  |  |  |  | 3: 1Hz |  |
|  |  |  |  | 4: 1.25Hz |  |
|  |  |  |  | 5: 1.5Hz |  |
|  |  |  |  | 6: 1.75Hz |  |
|  |  |  |  | 7: 2Hz |  |
|  |  |  |  | 8: 2.25Hz |  |
|  |  |  |  | 9: 2.5Hz |  |
|  |  |  |  | 10: 2.75Hz |  |
| macLedIdAmbiguityResol | 0x71 | Unsigned | 0-255 | This attribute resolves the | 0 |
| ution |  |  |  | ambiguity of a short LED ID |  |
|  |  |  |  | by appending it to a SSID as |  |
|  |  |  |  | shown in table TBD. |  |
| mac_symbol_rate |  |  | 5-20 | To adapt TX symbol rate | 10 |
|  |  |  |  | versus RX frame rate |  |
|  |  |  |  | variation when RX has the |  |
|  |  |  |  | minimum frame rate below |  |
|  |  |  |  | the symbol rate. |  |
| mac_bandwidth |  |  | 2 kHz - 8 kHz | To adapt the number of TX |  |
|  |  |  |  | LEDs spatially (resolution) |  |
|  |  |  |  | versus RX image resolution |  |
|  |  |  |  | mode. |  |
| mac_LEDs_spatial_resol |  | int |  | To adapt the number of TX | 2 |
| ution |  |  |  | LEDs spatially (resolution) |  |
|  |  |  |  | versus RX image resolution |  |
|  |  |  |  | mode. |  |
| mac_DS_rate |  | int | 60 - 120 | To control the length of a data | 100 |
|  |  |  |  | subframe. Shorter data frame |  |
|  |  |  |  | can support for longer |  |
|  |  |  |  | distance transmission. |  |
| macTxMode | 0x91 | Unsigned | 0-255 | This attribute indicates the | 0 |
|  |  |  |  | MAC transmission mode is |  |
|  |  |  |  | visible or Invisible. |  |
|  |  |  |  | 0 : Visible VTASC Mode |  |
|  |  |  |  | 1 : Invisible Mode - Blending |  |
|  |  |  |  | Method |  |
|  |  |  |  | 2 : Invisible Mode - |  |
|  |  |  |  | Watermarking Method |  |
| macTxCamerEnable | 0x92 | Unsigned | 0-255 | This attribute indicates the | 0 |
|  |  |  |  | Transmitter is Enabled with |  |
|  |  |  |  | Camera or not for Interactive |  |
|  |  |  |  | Receiver distance specific |  |
|  |  |  |  | data transfer control. |  |
|  |  |  |  | 0 : Camera not connected |  |
|  |  |  |  | 1 : Camera connected |  |
| macRxDistance | 0x93 | Unsigned | 0-255 | This attribute notify the | 0 |
|  |  |  |  | Receiver distance from |  |
|  |  |  |  | Transmitter |  |

246	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 91—MAC PIB attributes (continued)

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macTxDataType | 0x94 | Unsigned | 0-255 | This attribute indicates the | 0 |
|  |  |  |  | type of data to be transmitted. |  |
|  |  |  |  | 0 : Normal Data (Media |  |
|  |  |  |  | Content, Information Content |  |
|  |  |  |  | based on the Application used |  |
|  |  |  |  | for) |  |
|  |  |  |  | 1 : ID Data |  |
|  |  |  |  | 2 : Authentication Data |  |
| maxDataLength | 0x95 | Integer | 0-65535 | This attribute specify the | 0 |
|  |  |  |  | length of the data to be |  |
|  |  |  |  | transmitted |  |
| macLEDIDusage | 0x81 | Unsigned | 0-255 | This attribute indicates the | 0 |
|  |  |  |  | type of data transmitted using |  |
|  |  |  |  | Flash Light Transmitter. |  |
|  |  |  |  | 0 : LED IT |  |
|  |  |  |  | 1 : With or Without LED ID |  |
|  |  |  |  | and IP address |  |
| macCoordinationTime | 0x82 | Integer | 0-127 | This attribute indicates the | 10 |
|  |  |  |  | number of superframes that |  |
|  |  |  |  | consist of a coordination |  |
|  |  |  |  | period |  |
| macNeighborReportResp | 0x83 | Integer | 2-128 | The maximum time, in multi- | 64 |
| onseTime |  |  |  | ples of aBaseSuperframeDur- |  |
|  |  |  |  | ation, a coordinator shall wait |  |
|  |  |  |  | for the neighboring VPAN |  |
|  |  |  |  | report indication command |  |
|  |  |  |  | frames to be available follow- |  |
|  |  |  |  | ing the neighboring VPAN |  |
|  |  |  |  | report request command |  |
|  |  |  |  | frame. |  |
| macAgeingTime | 0x84 | Integer | 0-31 | Ageing time for neighboring | 10 |
|  |  |  |  | OWPAN monitoring records |  |

Table 92—Ambiguity Resolution Method

| PIB Attribute Value | Method Name | Method |
| --- | --- | --- |
| 0 | ID without payload | Preappend ID to SSID |
| 1 | ID and SSID hash | Preappend ID to hash resolved SSID |
| 2 | w/ or w/o ID and IP address | Preappend ID to provided IP address |

| Copyright © 20xx IEEE. All rights reserved. | 247 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 6.4.3 MAC PIB Attributes |
| --- | --- |
| 2 |  |

- The MAC PIB comprises the attributes required to manage the MAC sublayer of a device. The attributes

- contained in the IEEE802.15.13 MAC PIB are presented in Table 91- MAC PIB Attributes.

5

- The additional MAC PIB attributes added on IEEE802.15.7r1 for Offset Variable Pulse Width Modulation

- for Smart Device Flash Light is presented the Table 93.

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

Table 93—IEEE802.15.7r1 MAC PIB Attributes Additions

| Attribute | Identifier | Type | Range | Description | Default |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | This attribute |  |  |
|  |  |  |  | indicates the |  |  |
|  |  |  |  | type of data |  |  |
|  |  |  |  | transmitted |  |  |
|  |  |  |  | using Flash |  |  |
| macLEDI- | 0x81 | Unsigned | 0-255 | Light Transmit- | 0 |  |
| Dusage |  |  |  | ter. |  |  |
|  |  |  |  | 0 : LED IT |  |  |
|  |  |  |  | 1 : With or |  |  |
|  |  |  |  | Without LED |  |  |
|  |  |  |  | ID and IP |  |  |
|  |  |  |  | address |  |  |

6.5 Optical-clock-rate s election

The standard supports multiple optical clock rates in order to accommodate a wide variety of optical sources and receivers. The standard also supports the use of asymmetric clock rates between transmitter and receiver since they constitute independent chains and may support different clock-rate ranges. The multiple clocks associated with each PHY type are respectively shown in Table 106, Table 107, and Table 108.

- Support for the minimum clock rate for a given PHY type shall be mandatory for all TX and RX devices. All

- specified clock rates less than the maximum supported clock rate in a given device shall also be supported in

- that device. If a clock rate is supported, all data rates associated with that clock rate shall be supported. The

- preamble, headers, and payload in the PHY shall have the same clock rate. The header shall be sent at lowest

- data rate for the chosen clock rate. The payload can choose any data rate belonging to the chosen clock rate.

37

38

39

40

41

42

43

44

45

The clock-rate negotiation can be supported with or without explicit clock-rate negotiation, as indicated in the capabilities information field in Table 38. Explicit clock-rate negotiation implies that the devices shall transmit a clock rate change notification command as in 5.4.19 before a new clock rate is used. If explicit clock-rate negotiation is not used, the device shall have the capability to perform synchronization at all supported optical clock rates without any prior knowledge of the clock rate chosen at the transmitter for communication.

6.5.1 Optical-clock-rate selection for P2P topology

- Let us assume that Device 1 supports clock rates at the transmitter (C1, C2, … Ct1), where Ct1 is the

- maximum clock rate supported at the transmitter at of the coordinator. Also, C1 < C2 < ... < Ct1. Within a

- PHY type, the clock rates are integral multiples of each other to make the clock generation and selection

- simple at the transmitter (i.e., Ci+1/Ci = m, which is an integer). The receiver may support more or less clock

- rates than the transmitter since the receiver optronics is physically independent of the transmitter clock. Let

- the clocks supported by the receiver of device 1 be C1, C2, … Cr1, where Cr1 is the maximum clock rate

- supported at the receiver of device 1. Similarly, let Ct2 and Cr2 be the maximum clock rates supported by the

- device 2. Support for the lowest clock rate C1 is mandatory at both the transmitter and receiver for all

248	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| devices i.e., t1, t2, r1, r2  1. For every clock rate, there is an associated set of data rates at the physical layer. |  | 1 |  |
| --- | --- | --- | --- |
| This data rate is dependent on the modulation, RLL coding, and FEC used at the physical layer for a given |  | 2 |  |
| clock rate. Let the data rate be represented by rate{Ci,p}, where Ci is the chosen clock rate and 1  p  N(Ci), |  | 3 |  |
| where N(Ci) is the number of physical-layer data rates associated with clock rate Ci. |  | 4 |  |
| 6.5.1.1 Explicit notification |  | 5 |  |
|  |  | 6 |  |
| In Figure 139, a device sends the association request at the lowest clock C1 |  | 7 |  |
|  | at a physical layer data rate | 8 |  |
| rate{C1,k}. The data rate index k is typically chosen to be the lowest data rate to guarantee maximum range |  | 9 |  |
| and reliability for the given clock rate. In this association request, the device also informs the coordinator of |  | 10 |  |
| the maximum clock rate supported by its transmitter and receiver (Ct2, Cr2). The maximum-clock-rate |  | 11 |  |
| information is provided by the capabilities IE as shown in Table 38, which shall also be transmitted during |  | 12 |  |
| this association request. The coordinator receives the association request and compares the received |  | 13 |  |
| information about the supported clocks at the device and compares it with its supported clocks. In order for |  | 14 |  |
| it to communicate, it shall select a clock rate C_SEL1 that is equal to or lower than MAX_C_SEL1, which is |  | 15 |  |
| the minimum of its maximum transmitter clock and the maximum receiver clock supported by the |  | 16 |  |
| coordinator. The decision to use clock rates lower than MAX_C_SEL1 and MAX_C_SEL2 at the coordinator |  | 17 |  |
| and the device for, respectively, the transmission depends on the performance and throughput needs of the |  | 18 |  |
| coordinator and the devices. The coordinator also sends an association grant back to the device at the same |  | 19 |  |
| lowest clock rate C1 supported by all devices. The devices then exchange the selected clock frequencies by |  | 20 |  |
| using the clock-rate-change notification command for future communication before they switch to the |  | 21 |  |
| selected clock frequencies. The devices may also decide to change the clock rate anytime in future |  | 22 |  |
| communication, as long as it is below MAX_C_SEL1 and MAX_C_SEL2 for transmission at the coordinator |  | 23 |  |
| and device, respectively. |  | 24 |  |
|  |  | 25 |  |
|  |  | 26 |  |
|  |  | 27 |  |
|  |  | 28 |  |
|  |  | 29 |  |
|  |  | 30 |  |
|  |  | 31 |  |
|  |  | 32 |  |
|  |  | 33 |  |
|  |  | 34 |  |
|  |  | 35 |  |
|  |  | 36 |  |
|  |  | 37 |  |
|  |  | 38 |  |
|  |  | 39 |  |
|  |  | 40 |  |
|  |  | 41 |  |
|  |  | 42 |  |
|  |  | 43 |  |
|  |  | 44 |  |
|  |  | 45 |  |
|  |  | 46 |  |
|  |  | 47 |  |
|  |  | 48 |  |
|  |  | 49 |  |
|  |  | 50 |  |
|  |  | 51 |  |
|  |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 249 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  |  |
| --- | --- | --- | --- |
| 2 |  | clock-rate selection for P2P topology |  |
| 3 |  |  |  |
|  | coordinator | device |  |
| 4 |  |  |  |
| 5 | (supports clocks | (supports clocks |  |
|  | TX: C1,C2,...Ct1 | TX: C1,C2,...Ct2 |  |
| 6 |  |  |  |
|  | RX: C1,C2,...Cr1) | RX: C1,C2,...Cr2) |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  | association request (clock C1, rate{C1,k}) |  |
| 11 |  | inform max clock of device (Ct2,Cr2) |  |
| 12 |  |  |  |
| 13 |  | association grant (clock C1, rate{C1,j}) |  |
| 14 |  |  |  |
| 15 |  | inform max clock of coordinator(Ct1,Cr1) |  |
| 16 | MAX_C_SEL1 = min (Cr2, Ct1) | MAX_C_SEL2 = min (Cr1, Ct2) |  |
| 17 |  |  |  |
|  | C_SEL1 ≤ MAX_C_SEL1 | C_SEL2 ≤ MAX_C_SEL2 |  |
| 18 |  |  |  |
| 19 |  | inform selected clock C_SEL1 (clock C1) |  |
| 20 |  | inform selected clock C_SEL2 (clock C1) |  |
| 21 |  | device to coordinator (clock C_SEL2) |  |
| 22 |  |  |  |
|  |  | coordinator to device (clock C_SEL1) |  |
| 23 |  |  |  |
| 24 |  |  |  |
| 25 |  |  |  |
| 26 |  |  |  |
| 27 |  |  |  |
| 28 | There is a data rate table rate{Ci,j} associated with each clock rate |  |  |
| 29 |  |  |  |
| 30 | Ci where j is the data rate index associated with clock rate Ci |  |  |
| 31 | Figure 139—Clock-rate selection for P2P topology (explicit notification) |  |  |
| 32 |  |  |  |
| 33 |  |  |  |
| 34 | 6.5.1.2 Without explicit notification |  |  |
| 35 |  |  |  |

- It is also possible for the coordinator to send the association grant at the new clock rate C_SEL2 and not have

- to explicitly exchange notification information, as shown in Figure 140, if the coordinator has the capability

- to detect all clock rates less than its maximum receive clock rate. In this case, communication can occur

- without overhead for explicit notification.

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

250	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

coordinator (supports clocks TX: C1,C 2,...Ct1 RX: C1,C2,...Cr1)

MAX_C_SEL1 = min (Cr2, Ct1) C_SEL1 ≤ MAX_C_SEL1

clock-rate selection for P2P topology

association request (clock C1, rate{C1,k})

inform max clock of device (Ct2,Cr2)

device (supports clocks TX: C1,C 2,...Ct2 RX: C1,C2,...Cr2)

2

3

4

5

6

7

8

9

10

11

12

13

association grant(clock C_SEL1, rate{C_SEL2,j})

inform max clock of coordinator (Ct1,Cr1)

MAX_C_SEL2 = min (Cr1, Ct2)

C_SEL2 ≤ MAX_C_SEL2

device to coordinator(clock C_SEL2)

coordinator to device(clock C_SEL1)

There is a data rate table rate{Ci,j} associated with each clock rate

Ci where j is the data rate index associated with clock rate Ci

Figure 140—Clock-rate selection for P2P topology (without explicit notification)

6.5.2 Optical-clock-rate selection for star topology

6.5.2.1 Explicit notification

Figure 141 shows the optical clock-rate selection for a star topology. In this case, let us assume device 1 to be a coordinator. The coordinator will send a broadcast message via a beacon to all nodes, such as devices 2 and 3, and inform them of its supported clock rates. The CAP always uses the lowest clock rate C1 for uplink contention. The coordinator and the devices communicate the selected clock frequencies during the CFP using clock rate C1 before switching to the selected clock frequencies. The information about the coordinator capabilities is broadcast using the capabilities IE. The current clock in use, and any change of clock, is communicated via the clock rate change notification.

| Copyright © 20xx IEEE. All rights reserved. | 251 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  | clock-rate selection for star topology |  |
| --- | --- | --- |
| coordinator | device 2 | device 3 |
| (supports clocks | (supports clocks | (supports clocks |
| TX: C1,C2,...Ct1 | TX: C1,C2,...Ct2 | TX: C1,C2,...Ct3 |
| RX: C1,C2,...Cr1) | RX: C1,C2,...Cr2) | RX: C1,C2,...Cr3) |

beacon (clock C1,rate{C1,k})

inform max clock of coordinator (Ct1,Cr1)

contentionaccess period (clock C1,rate{C1,j} )

inform max clock of device 2 (Ct2,Cr2)

contentionaccess period (clock C1,rate{C1,m})

inform max clock of device 3 (Ct3,Cr3)

| C_SEL12 | ≤ min (Cr2, Ct1) |  |  |  | C_SEL2 ≤ min (Cr1, Ct2) |  |  |  | C_SEL3 ≤ min (Cr1, Ct3) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C_SEL13 | ≤ min (Cr3, Ct1) |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CFP: inform clock C_SEL12 (clock C1) |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | CFP: inform clock C_SEL13 (clock C1) |  |  |  |  |
|  |  |  | CFP: inform clock C_SEL2 (clock C1) |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | CFP: inform clock C_SEL3 (clock C1) |  |  |  |  |

CFP: device 2 to coordinator (clock C_SEL2)

CFP: coordinator to device 2 (clock C_SEL12)

CFP: device 3 to coordinator (clock C_SEL3)

CFP: coordinator to device 3 (clock C_SEL13)

Figure 141—Clock-rate selection for star topology (explicit notification)

6.5.2.2 Without explicit notification

Similar to the P2P topology, the clock-rate selection for the star topology can also occur without explicit notification, as shown in Figure 142.

252	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  |  |  |  |  |  | clock-rate selection for star topology |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| coordinator |  |  |  |  |  |  | device 2 |  |  |  |  |  | device 3 |  |  |  |  |  |  |
| (supports clocks |  |  |  |  |  |  | (supports clocks |  |  |  |  |  | (supports clocks |  |  |  |  |  |  |
| TX: C1,C2,...Ct1 |  |  |  |  |  |  | TX: C1,C2,...Ct2 |  |  |  |  |  | TX: C1,C2,...Ct3 |  |  |  |  |  |  |
| RX: C1,C2,...Cr1) |  |  |  |  |  |  | RX: C1,C2,...Cr2) |  |  |  |  |  | RX: C1,C2,...Cr3) |  |  |  |  |  |  |
|  |  |  |  |  |  | beacon (clock C1,rate{C1,k}) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | inform max clock of coordinator (Ct1,Cr1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | C_SEL2 ≤ min (Cr1, Ct2) |  |  |  |  |  | C_SEL3 ≤ min (Cr1, Ct3) |  |  |  |  |  |  |
|  |  |  |  |  | contentionaccess period (clock C1,rate{C1,j}) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | inform max clock of device 2 (Ct2,Cr2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | contentionaccess period(clock C1,rate{C1,m}) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | inform max clock of device 3 (Ct3,Cr3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C_SEL12 ≤ min (Cr2, Ct1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C_SEL13 ≤ min (Cr3, Ct1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | CFP: device 2 to coordinator (clock C_SEL2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | CFP: coordinator to device 2 (clock C_SEL12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | CFP: device 3 to coordinator (clock C_SEL3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | CFP: coordinator to device3 (clock C_SEL13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 142—Clock-rate selection for star topology (without explicit notification)

6.5.3 Clock-rate selection for multicast topology

6.5.3.1 Explicit notification

Figure 143 and Figure 144 show the clock-rate selection for multicast topologies assuming bi-directional communication.

| Copyright © 20xx IEEE. All rights reserved. | 253 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  |  |  |  | clock-rate selection for multicast topology |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | coordinator |  |  |  |  | device 2 |  |  |  |  |  | device 3 |  |  |  |
| (supports clocks |  |  |  |  | (supports clocks |  |  |  |  |  | (supports clocks |  |  |  |  |
| TX: C1,C2,...Ct1 |  |  |  |  | TX: C1,C2,...Ct2 |  |  |  |  |  | TX: C1,C2,...Ct3 |  |  |  |  |
| RX: C1,C2,...Cr1) |  |  |  |  | RX: C1,C2,...Cr2) |  |  |  |  |  | RX: C1,C2,...Cr3) |  |  |  |  |
|  |  |  |  | beacon (clock C1, rate{C1,j}) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | inform max clock of coordinator (Ct1,Cr1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | C_SEL2 ≤ min (Cr1, Ct2) |  |  |  |  |  | C_SEL3 ≤ min (Cr1, Ct3) |  |  |  |  |

contention access period(clock C 1, rate{C1,j}) inform max clock of device 2 (Ct2,Cr2)

contention access period(clock C 1, rate{C1,m}) inform max clock of device 3 (Ct3,Cr3)

C_SEL1 ≤ min (Cr2, Cr3, Ct1)

C_SEL12 ≤ min (Cr2, Ct1)

C_SEL13 ≤ min (Cr3, Ct1)

CFP: inform clock change to C_SEL1 (clock C_SEL12)

CFP: inform clock change to C_SEL1 (clock C_SEL13) multicast message (clock C_SEL1)

Figure 143—Clock-rate selection for multicast (assuming bi-directional communication)

254	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.5.3.2 Without explicit notification

|  |  |  |  | clock-rate selection for multicast topology |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | coordinator |  |  |  |  | device 2 |  |  |  |  |  | device 3 |  |  |  |
| (supports clocks |  |  |  |  | (supports clocks |  |  |  |  |  | (supports clocks |  |  |  |  |
| TX: C1,C2,...Ct1 |  |  |  |  | TX: C1,C2,...Ct2 |  |  |  |  |  | TX: C1,C2,...Ct3 |  |  |  |  |
| RX: C1,C2,...Cr1) |  |  |  |  | RX: C1,C2,...Cr2) |  |  |  |  |  | RX: C1,C2,...Cr3) |  |  |  |  |
|  |  |  |  | beacon (clock C1, rate{C1,j}) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | inform max clock of coordinator (Ct1,Cr1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | C_SEL2 ≤ min (Cr1, Ct2) |  |  |  |  |  | C_SEL3 ≤ min (Cr1, Ct3) |  |  |  |  |

contention access period(clock C 1, rate{C1,j}) inform max clock of device 2 (Ct2,Cr2)

contention access period(clock C1, rate{C1,m}) inform max clock of device 3 (Ct3,Cr3)

C_SEL1 ≤ min (Cr2, Cr3, Ct1)

multicast message (clock C_SEL1)

Figure 144—Clock-rate selection for multicast (bi-directional communication and no explicit notification)

6.6 Mes s age s equence charts illus trating MAC-PHY interaction

This subclause illustrates the main tasks specified in this standard. Each task is described by use of a message sequence chart to illustrate the chronological order, rather than the exact timing, of the primitives required for each task.

The primitives necessary for the coordinator to start a new OWPAN are shown in Figure 145. The first action the next higher layer takes after resetting the MAC sublayer is to initiate a scan to search for other OWPANs in the area. An active scan is required. The steps for performing an active scan are shown in Figure 149.

Once a new OWPAN is established, the coordinator is ready to accept requests from other devices to join the OWPAN. Figure 146 shows the primitives issued by a device requesting association, while Figure 147 illustrates the steps taken by a coordinator allowing association. In the process of joining a OWPAN, the device requesting association will perform either a passive or an active scan to determine which OWPANs in the area are allowing association; Figure 148 and Figure 149 detail the primitives necessary to complete a passive scan and an active scan, respectively.

| Copyright © 20xx IEEE. All rights reserved. | 255 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | The primitives necessary for transmitting and receiving a single data frame are shown next. The actions |  |
| --- | --- | --- |
| 2 | taken by the originator of the frame are shown in Figure 150, while the actions taken by the recipient are |  |
| 3 | shown in Figure 151. |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 13 |  |  |
| 14 |  |  |
| 15 |  |  |
| 16 |  |  |
| 17 |  |  |
| 18 |  |  |
| 19 |  |  |
| 20 |  |  |
| 21 |  |  |
| 22 |  |  |
| 23 |  |  |
| 24 |  |  |
| 25 |  |  |
| 26 |  |  |
| 27 |  |  |
| 28 |  |  |
| 29 |  |  |
| 30 |  |  |
| 31 |  |  |
| 32 |  |  |
| 33 |  |  |
| 34 |  |  |
| 35 |  |  |
| 36 |  |  |
| 37 |  |  |
| 38 |  |  |
| 39 |  |  |
| 40 |  |  |
| 41 |  |  |
| 42 |  |  |
| 43 |  |  |
| 44 |  |  |
| 45 |  |  |
| 46 |  |  |
| 47 |  |  |
| 48 |  |  |
| 49 |  |  |
| 50 |  |  |
| 51 |  |  |
| 52 |  |  |
| 53 |  |  |
| 54 |  |  |
|  | 256 | Copyright © 20xx IEEE. All rights reserved. |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

.

Figure 145—OWPAN start message sequence chart—coordinator

| Copyright © 20xx IEEE. All rights reserved. | 257 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 146—Association message sequence chart—device

258	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 147—Association message sequence chart—coordinator

| Copyright © 20xx IEEE. All rights reserved. | 259 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 148—Passive scan message sequence chart

260	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 149—Active scan message sequence chart

| Copyright © 20xx IEEE. All rights reserved. | 261 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 150—Data-transmission message sequence chart—originator

262	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 151—Data-transmission message sequence chart—recipient

7. Security s uite s pecifications

7.1 Overview

The MAC sublayer is responsible for providing security services on specified incoming and outgoing frames when requested to do so by the higher layers. This standard supports the following security services (as defined in 4.6 for definitions):

—  Data confidentiality

| Copyright © 20xx IEEE. All rights reserved. | 263 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

—  Data authenticity

—  Replay protection

The information determining how to provide the security is found in the security-related PIB (as defined in Table 99 in 7.5.1).

7.2 Functional des cription

- A device may optionally implement security. A device that does not implement security shall not provide a

- mechanism for the MAC sublayer to perform any cryptographic transformation on incoming and outgoing

- frames nor require any PIB attributes associated with security. A device that implements security shall

- provide a mechanism for the MAC sublayer to provide cryptographic transformations on incoming and

| 14 | outgoing frames using information in the PIB attributes associated with security when the |
| --- | --- |
| 15 | macSecurityEnabled attribute is set to TRUE. |
| 16 |  |

- If the MAC sublayer is required to transmit a frame or receives an incoming frame, the MAC sublayer shall

- process the frame as specified in 7.2.1 and 7.2.3, respectively.

19

20	7.2.1 Outgoing frame security procedure

21

- The inputs to this procedure are the frame to be secured and the SecurityLevel, KeyIdMode, KeySource, and

- KeyIndex parameters from the originating primitive or automatic request PIB attributes. The outputs from

- this procedure are the status of the procedure and, if this status is SUCCESS, the secured frame. If outgoing

- frame security procedure is not successful, the frame is discarded

| 26 | The outgoing frame security procedure involves the following steps as applicable: |  |  |
| --- | --- | --- | --- |
| 27 |  |  |  |
| 28 | a) | If the Security Enabled subfield of the frame control field of the frame to be secured is set to zero, |  |
| 29 |  |  |  |
| 30 |  | the procedure shall set the security level to zero. |  |
| 31 | b) | If the Security Enabled subfield of the frame control field of the frame to be secured is set to one, the |  |
| 32 |  | procedure shall set the security level to the SecurityLevel parameter. If the resulting security level is |  |
| 33 |  | zero, the procedure shall return with a status of UNSUPPORTED_SECURITY. |  |
| 34 | c) | If the macSecurityEnabled attribute is set to FALSE and the security level is not equal to zero, the |  |
| 35 |  | procedure shall return with a status of UNSUPPORTED_SECURITY. |  |
| 36 | d) | The procedure shall determine whether the frame to be secured satisfies the constraint on the |  |
| 37 |  | maximum length of MAC frames, as follows: |  |
| 38 |  | 1) The procedure shall set the length M, in octets, of the Authentication field to zero if the security |  |
| 39 |  | level is equal to zero and shall determine this value from the security level and Table 97 |  |
| 40 |  | otherwise. |  |
| 41 |  | 2) The procedure shall determine the length AuxLen, in octets, of the auxiliary security header |  |
| 42 |  | (see 7.4) using KeyIdMode and the security level. |  |
| 43 |  | 3) The procedure shall determine the data expansion as AuxLen + M. |  |
| 44 |  | 4) The procedure shall check whether the length of the frame to be secured, including data |  |
| 45 |  | expansion and FCS, is less than or equal to aMaxPHYFrameSize. If this check fails, the |  |
| 46 |  | procedure shall return with a status of FRAME_TOO_LONG. |  |
| 47 | e) | If the security level is zero, the procedure shall set the secured frame to be the frame to be secured |  |
| 48 |  | and return with the secured frame and a status of SUCCESS. |  |
| 49 | f) | The procedure shall set the frame counter to the macFrameCounter attribute. If the frame counter |  |
| 50 |  | has the value 0xffffffff, the procedure shall return with a status of COUNTER_ERROR and all keys |  |
| 51 |  | associated with the device shall be reinitialized and updated as discussed in 7.5.5. |  |
| 52 | g) | The procedure shall obtain the key using the outgoing frame key retrieval procedure as described in |  |
| 53 |  | 7.2.2. If that procedure fails, the procedure shall return with a status of UNAVAILABLE_KEY. |  |
| 54 | h) | The procedure shall insert the auxiliary security header into the frame, with fields set as follows: |  |
|  | 264 | Copyright © 20xx IEEE. All rights reserved. |  |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  | 1) The Security Level subfield of the Security Control field shall be set to the security level. |  | 1 |  |
| --- | --- | --- | --- | --- |
|  | 2) The Key Identifier Mode subfield of the Security Control field shall be set to the KeyIdMode |  | 2 |  |
|  | parameter. |  | 3 |  |
|  | 3) The Frame Counter field shall be set to the frame counter. |  | 4 |  |
|  | 4) If the KeyIdMode parameter is set to a value not equal to zero, the Key Source and Key Index |  | 5 |  |
|  | subfields of the Key Identifier field shall be set to the KeySource and KeyIndex parameters, |  | 6 |  |
|  | respectively. |  | 7 |  |
| i) | The procedure shall then use aExtendedAddress, the frame counter, the security level, and the key to |  | 8 |  |
|  | produce the secured frame according to the transformation process known as CCM* [or the |  | 9 |  |
|  | extension of CCM, which is the combined counter with CBC-MAC (i.e., cipher block chaining |  | 10 |  |
|  | message authentication code) mode of operation] that is described in the security operations (see |  | 11 |  |
|  | 7.3.4). |  | 12 |  |
|  | 1) If the SecurityLevel parameter specifies the use of encryption (as defined in Table 97 in |  | 13 |  |
|  | 7.4.2.1), the encryption operation shall be applied only to the actual payload field within the |  | 14 |  |
|  | MSDU, i.e., the Beacon Payload field (see 5.2.2.1.10), Command Payload field (see 5.2.2.4.3), |  | 15 |  |
|  | or Data Payload field (see 5.2.2.2.2), depending on the frame type. The corresponding payload |  | 16 |  |
|  | field is passed to the CCM* transformation process described in 7.3.4 as the unsecured payload |  | 17 |  |
|  | (as defined in Table 94 in 7.3.4.2). The resulting encrypted payload shall substitute the original |  | 18 |  |
|  | payload. |  | 19 |  |
|  | 2) The remaining fields in the MSDU part of the frame shall be passed to the CCM* |  | 20 |  |
|  | transformation process described in 7.3.4 as the nonpayload fields (see Table 94). |  | 21 |  |
|  | 3) The ordering and exact manner of performing the encryption and integrity operations and the |  | 22 |  |
|  | placement of the resulting encrypted data or integrity code within the MSDU field shall be as |  | 23 |  |
|  | defined in 7.3.4. |  | 24 |  |
| j) | The procedure shall increment the frame counter by one and set the macFrameCounter attribute to |  | 25 |  |
|  | the resulting value. |  | 26 |  |
| k) The procedure shall return with the secured frame and a status of SUCCESS. |  |  | 27 |  |
| 7.2.2 Outgoing frame key retrieval procedure |  |  | 28 |  |
|  |  |  | 29 |  |
| The inputs to this procedure are the frame to be secured and the KeyIdMode, KeySource, and KeyIndex |  |  | 30 |  |
|  |  |  | 31 |  |
| parameters from the originating primitive. The outputs from this procedure are a passed or failed status and, |  |  | 32 |  |
| if passed, a key. |  |  | 33 |  |
| The outgoing frame key retrieval procedure involves the following steps as applicable: |  |  | 34 |  |
|  |  |  | 35 |  |
| a) If the KeyIdMode parameter is set to 0x00 (implicit key identification), the procedure |  | shall | 36 |  |
|  |  |  | 37 |  |
|  | determine the key lookup data and key lookup size as follows: |  | 38 |  |
|  | 1) If the Destination Addressing Mode subfield of the frame control field of the frame is set to |  | 39 |  |
|  | 0x00 and the macOWPANCoordShortAddress attribute is set to a value in the range 0x0000– |  | 40 |  |
|  | 0xfffd (i.e., the short address is used), the key lookup data shall be set to the 2-octet Source |  | 41 |  |
|  | OWPAN Identifier field of the frame right-concatenated (see B.2.1. of IEEE Std 802.15.4- |  | 42 |  |
|  | 2006) with the 2-octet macOWPANCoordShortAddress attribute right-concatenated with the |  | 43 |  |
|  | single octet 0x00. The key lookup size shall be set to five. |  | 44 |  |
|  | 2) If the Destination Addressing Mode subfield of the frame control field of the frame is set to |  | 45 |  |
|  | 0x00 and the macOWPANCoordShortAddress attribute is set to 0xfffe (i.e., the extended |  | 46 |  |
|  | address is used), the key lookup data shall be set to the 8-octet |  | 47 |  |
|  | macOWPANCoordExtendedAddress attribute right-concatenated (see B.2.1 of IEEE Std |  | 48 |  |
|  | 802.15.4-2006) with the single octet 0x00. The key lookup size shall be set to nine. |  | 49 |  |
|  | 3) If the Destination Addressing Mode subfield of the frame control field of the frame is set to |  | 50 |  |
|  | 0x02, the key lookup data shall be set to the 2-octet Destination OWPAN Identifier field of the |  | 51 |  |
|  | frame right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the 2-octet Destination |  | 52 |  |
|  | Address field of the frame right-concatenated with the single octet 0x00. The key lookup size |  | 53 |  |
|  | shall be set to five. |  | 54 |  |
|  | Copyright © 20xx IEEE. All rights reserved. | 265 |  |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

  - If the Destination Addressing Mode subfield of the frame control field of the frame is set to 0x03, the key lookup data shall be set to the 8-octet Destination Address field of the frame right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the single octet 0x00. The key lookup size shall be set to nine.

- If the KeyIdMode parameter is set to a value not equal to 0x00 (explicit key identification), the procedure shall determine the key lookup data and key lookup size as follows:

  - If the KeyIdMode parameter is set to 0x01, the key lookup data shall be set to the 8-octet macDefaultKeySource attribute right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the single octet KeyIndex parameter. The key lookup size shall be set to nine.

  - If the KeyIdMode parameter is set to 0x02, the key lookup data shall be set to the 4-octet KeySource parameter right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the single octet KeyIndex parameter. The key lookup size shall be set to five.

  - If the KeyIdMode parameter is set to 0x03, the key lookup data shall be set to the 8-octet KeySource parameter right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the single octet KeyIndex parameter. The key lookup size shall be set to nine.

- The procedure shall obtain the KeyDescriptor by passing the key lookup data and the key lookup size to the KeyDescriptor lookup procedure as described in 7.2.5. If that procedure returns with a failed status, this procedure shall also return with a failed status.

- d)   The MAC sublayer shall set the key to the Key element of the KeyDescriptor.

- e)   The procedure shall return with a passed status, having obtained the key identifier and the key.

- NOTE—For broadcast frames, the outgoing frame key retrieval procedure will result in a failed status if implicit key

- identification is used. Hence, explicit key identification should be used for broadcast frames.

| 25 | 7.2.3 Incoming frame security procedure |  |  |
| --- | --- | --- | --- |
| 26 |  |  |  |
| 27 | The input to this procedure is the frame to be unsecured. The outputs from this procedure are the unsecured |  |  |
| 28 |  |  |  |
|  | frame, the security level, the key identifier mode, the key source, the key index, and the status of the |  |  |
| 29 |  |  |  |
|  | procedure. All outputs of this procedure are assumed to be invalid unless and until explicitly set in this |  |  |
| 30 |  |  |  |
|  | procedure. It is assumed that the PIB attributes associating KeyDescriptors in macKeyTable with a single, |  |  |
| 31 |  |  |  |
|  | unique device or a number of devices will have been established by the next higher layer. The incoming |  |  |
| 32 |  |  |  |
|  | frame security procedure involves the following steps: |  |  |
| 33 |  |  |  |
| 34 | a) If the Security Enabled field of the frame control field of the frame to be unsecured is set to zero, the |  |  |
| 35 |  |  |  |
|  | procedure shall set the security level to zero. |  |  |
| 36 |  |  |  |
|  | b) If the Security Enabled field of the Frame Control field of the frame to be unsecured is set to one, the |  |  |
| 37 |  |  |  |
|  | procedure shall set the unsecured frame to be the frame to be unsecured and return with a status of |  |  |
| 38 |  |  |  |
|  | UNSUPPORTED_LEGACY. |  |  |
| 39 |  |  |  |
|  | c) If the Security Enabled field of the Frame Control field of the frame to be unsecured is set to one, the |  |  |
| 40 |  |  |  |
|  | procedure shall set the security level and the key identifier mode to the corresponding fields of the |  |  |
| 41 |  |  |  |
|  | Security Control field of the auxiliary security header of the frame to be unsecured, and the key |  |  |
| 42 |  |  |  |
|  | source and key index to the corresponding fields of the Key Identifier field of the auxiliary security |  |  |
| 43 |  |  |  |
|  | header of the frame to be unsecured, if present. If the resulting security level is zero, the procedure |  |  |
| 44 |  |  |  |
|  | shall | set the unsecured frame to be the frame to be unsecured and return with a status of |  |
| 45 |  |  |  |
|  | UNSUPPORTED_SECURITY. |  |  |
| 46 |  |  |  |
|  | d) If the macSecurityEnabled attribute is set to FALSE, the procedure shall set the unsecured frame to |  |  |
| 47 |  |  |  |
|  | be the frame to be unsecured and return with a status of SUCCESS if the security level is equal to |  |  |
| 48 |  |  |  |
|  | zero and with a status of UNSUPPORTED_SECURITY otherwise. |  |  |
| 49 |  |  |  |
|  | e) The procedure shall determine whether the frame to be unsecured meets the minimum security level |  |  |
| 50 |  |  |  |
|  | by passing the security level, the frame type, and, depending on whether the frame is a MAC |  |  |
| 51 |  |  |  |
|  | command frame, the first octet of the MSDU (i.e., command frame identifier for a MAC command |  |  |
| 52 |  |  |  |
|  | frame) to the incoming security level checking procedure as described in 7.2.8. If that procedure |  |  |
| 53 |  |  |  |
| 54 |  |  |  |
|  | 266 | Copyright © 20xx IEEE. All rights reserved. |  |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  | fails, the procedure shall set the unsecured frame to be the frame to be unsecured and return with a |  | 1 |  |
| --- | --- | --- | --- | --- |
|  | status of IMPROPER_SECURITY_LEVEL. |  | 2 |  |
| f) | If the security level is set to zero, the procedure shall set the unsecured frame to be the frame to be |  | 3 |  |
|  | unsecured and return with a status of SUCCESS. |  | 4 |  |
| g) | The procedure shall obtain the KeyDescriptor, DeviceDescriptor, and KeyDeviceDescriptor using |  | 5 |  |
|  | the incoming frame security material retrieval procedure described in 7.2.4. If that procedure fails, |  | 6 |  |
|  | the procedure shall set the unsecured frame to be the frame to be unsecured and return with a status |  | 7 |  |
|  | of UNAVAILABLE_KEY. |  | 8 |  |
| h) | The procedure shall determine whether the frame to be unsecured conforms to the key usage policy |  | 9 |  |
|  | by passing the KeyDescriptor, the frame type, and, depending on whether the frame is a MAC |  | 10 |  |
|  | command frame, the first octet of the MSDU (i.e., command frame identifier for a MAC command |  | 11 |  |
|  | frame) to the incoming key usage policy checking procedure as described in 7.2.9. If that procedure |  | 12 |  |
|  | fails, the procedure shall set the unsecured frame to be the frame to be unsecured and return with a |  | 13 |  |
|  | status of IMPROPER_KEY_TYPE. |  | 14 |  |
| i) | If the Exempt element of the DeviceDescriptor is set to FALSE and if the incoming security level |  | 15 |  |
|  | checking procedure of step e) above had as output the “conditionally passed” status, the procedure |  | 16 |  |
|  | shall set the unsecured frame to be the frame to be unsecured and return with a status of |  | 17 |  |
|  | IMPROPER_SECURITY_LEVEL. |  | 18 |  |
| j) | The procedure shall set the frame counter to the Frame Counter field of the auxiliary security header |  | 19 |  |
|  | of the frame to be unsecured. If the frame counter has the value 0xffffffff, the procedure shall set the |  | 20 |  |
|  | unsecured frame to be the frame to be unsecured and return with a status of COUNTER_ERROR. |  | 21 |  |
| k) | The procedure shall determine whether the frame counter is greater than or equal to the |  | 22 |  |
|  | FrameCounter element of the DeviceDescriptor. If this check fails, the procedure shall set the |  | 23 |  |
|  | unsecured frame to be the frame to be unsecured and return with a status of COUNTER_ERROR. |  | 24 |  |
| l) | The procedure shall then use the ExtAddress element of the DeviceDescriptor, the frame counter, the |  | 25 |  |
|  | security level, and the Key element of the KeyDescriptor to produce the unsecured frame according |  | 26 |  |
|  | to the CCM* inverse transformation process described in the security operations (see 7.3.5). |  | 27 |  |
|  | 1) If the security level specifies the use of encryption (as defined in Table 97 in 7.4.2.1), the |  | 28 |  |
|  | decryption operation shall be applied only to the actual payload field within the MSDU, i.e., the |  | 29 |  |
|  | Beacon Payload field (see 5.2.2.1.10), Command Payload field (see 5.2.2.4.3), or Data Payload |  | 30 |  |
|  | field (see 5.2.2.2.2), depending on the frame type. The corresponding payload field shall be |  | 31 |  |
|  | passed to the CCM* inverse transformation process described in 7.3.5 as the secure payload. |  | 32 |  |
|  | 2) The remaining fields in the MSDU part of the frame shall be passed to the CCM* inverse |  | 33 |  |
|  | transformation process described in 7.3.5 as the nonpayload fields. |  | 34 |  |
| m) | If the CCM* inverse transformation process fails, the procedure shall set the unsecured frame to be |  | 35 |  |
|  | the frame to be unsecured and return with a status of SECURITY_ERROR. |  | 36 |  |
| n) | The procedure shall increment the frame counter by one and set the FrameCounter element of the |  | 37 |  |
|  | DeviceDescriptor to the resulting value. |  | 38 |  |
| o) | If the FrameCounter element is equal to 0xffffffff, the procedure shall set the Blacklisted element of |  | 39 |  |
|  | the KeyDeviceDescriptor. |  | 40 |  |
| p) | The procedure shall return with the unsecured frame and a status of SUCCESS. |  | 41 |  |
| 7.2.4 Incoming frame security material retrieval procedure |  |  | 42 |  |
|  |  |  | 43 |  |
| The input to this procedure is the frame to be unsecured. The outputs from this procedure are a passed or |  |  | 44 |  |
|  |  |  | 45 |  |
| failed status and, if passed, a KeyDescriptor, a DeviceDescriptor, and a KeyDeviceDescriptor. |  |  | 46 |  |
| The incoming frame security material retrieval procedure involves the following steps as applicable: |  |  | 47 |  |
|  |  |  | 48 |  |
| a) | If the Key Identifier Mode subfield of the Security Control field of the auxiliary security header of |  | 49 |  |
|  |  |  | 50 |  |
|  | the frame is set to 0x00 (implicit key identification), the procedure shall determine the key lookup |  | 51 |  |
|  | data and the key lookup size as follows: |  | 52 |  |
|  | 1) If the source address mode of the frame control field of the frame is set to 0x00 and the |  | 53 |  |
|  | macOWPANCoordShortAddress attribute is set to a value in the range 0x0000–0xfffd (i.e., the |  | 54 |  |
|  | Copyright © 20xx IEEE. All rights reserved. | 267 |  |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

short address is used), the key lookup data shall be set to the 2-octet Destination OWPAN Identifier field of the frame right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the 2-octet macOWPANCoordShortAddress attribute right-concatenated with the single octet 0x00. The key lookup size shall be set to five.

  - If the source address mode of the frame control field of the frame is set to 0x00 and the macOWPANCoordShortAddress attribute is set to 0xfffe (i.e., the extended address is used), the key lookup data shall be set to the 8-octet macOWPANCoordExtendedAddress attribute right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the single octet 0x00. The key lookup size shall be set to nine.

  - If the source address mode of the frame control field of the frame is set to 0x02, the key lookup data shall be set to the 2-octet Source OWPAN Identifier field of the frame, right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the 2-octet Source Address field of the frame right-concatenated with the single octet 0x00. The key lookup size shall be set to five.

  - If the source address mode of the frame control field of the frame is set to 0x03, the key lookup data shall be set to the 8-octet Source Address field of the frame right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the single octet 0x00. The key lookup size shall be set to nine.

- If the Key Identifier Mode subfield of the Security Control field of the auxiliary security header of the frame is set to a value not equal to 0x00 (explicit key identification), the procedure shall determine the key lookup data and key lookup size as follows:

  - If the key identifier mode is set to 0x01, the key lookup data shall be set to the 8-octet macDefaultKeySource attribute right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the 1-octet Key Index subfield of the Key Identifier field of the auxiliary security header. The key lookup size shall be set to nine.

  - If the key identifier mode is set to 0x02, the key lookup data shall be set to the right-concatenation (see B.2.1 of IEEE Std 802.15.4-2006) of the 4-octet Key Source subfield and the 1-octet Key Index subfield of the Key Identifier field of the auxiliary security header. The key lookup size shall be set to five.

  - If the key identifier mode is set to 0x03, the key lookup data shall be set to the right-concatenation (see B.2.1 of IEEE Std 802.15.4-2006) of the 8-octet Key Source subfield and the 1-octet Key Index subfield of the Key Identifier field of the auxiliary security header. The key lookup size shall be set to nine.

- The procedure shall obtain the KeyDescriptor by passing the key lookup data and the key lookup size to the KeyDescriptor lookup procedure as described in 7.2.5. If that procedure returns with a failed status, the procedure shall also return with a failed status.

- The procedure shall determine the device lookup data and the device lookup size as follows:

  - If the source address mode of the frame control field of the frame is set to 0x00 and the macOWPANCoordShortAddress attribute is set to a value in the range 0x0000–0xfffd (i.e., the short address is used), the device lookup data shall be set to the 2-octet Destination OWPAN Identifier field of the frame right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the 2-octet macOWPANCoordShortAddress attribute. The device lookup size shall be set to four.

  - If the source address mode of the frame control field of the frame is set to 0x00 and the macOWPANCoordShortAddress attribute is set to 0xfffe (i.e., the extended address is used), the device lookup data shall be set to the 8-octet macOWPANCoordExtendedAddress attribute. The device lookup size shall be set to eight.

  - If the source address mode of the frame control field of the frame is set to 0x02, the device lookup data shall be set to the 2-octet Source OWPAN Identifier field of the frame, right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the 2-octet Source Address field of the frame. The device lookup size shall be set to four.

  - If the source address mode of the frame control field of the frame is set to 0x03, the device lookup data shall be set to the 8-octet Source Address field of the frame. The device lookup size shall be set to eight.

- The procedure shall obtain the DeviceDescriptor and the KeyDeviceDescriptor by passing the KeyDescriptor, the device lookup data, and the device lookup size to the blacklist checking

268	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  | procedure as described in 7.2.6. If that procedure returns with a failed status, the procedure shall also |  | 1 |  |
| --- | --- | --- | --- | --- |
|  | return with a failed status. |  | 2 |  |
| f) | The procedure shall return with a passed status having obtained the KeyDescriptor, the |  | 3 |  |
|  | DeviceDescriptor, and the KeyDeviceDescriptor. |  | 4 |  |
| 7.2.5 Key descriptor lookup procedure |  |  | 5 |  |
|  |  |  | 6 |  |
| The inputs to this procedure are the key lookup data and the key lookup size. The outputs from this |  |  | 7 |  |
|  |  |  | 8 |  |
| procedure are a passed or failed status and, if passed, a KeyDescriptor. |  |  | 9 |  |
| The KeyDescriptor lookup procedure involves the following steps as applicable: |  |  | 10 |  |
|  |  |  | 11 |  |
| a) For each KeyDescriptor in the macKeyTable attribute and for each KeyIdLookupDescriptor in the |  |  | 12 |  |
|  |  |  | 13 |  |
|  | KeyIdLookupList of the KeyDescriptor, the procedure shall check whether the LookupDataSize |  | 14 |  |
|  | element of the KeyIdLookupDescriptor indicates the same integer value, shown in Table 105, as the |  | 15 |  |
|  | key lookup size and whether the LookupData element of the KeyIdLookupDescriptor is equal to the |  | 16 |  |
|  | key lookup data. If both checks pass (i.e., there is a match), the procedure shall return with this |  | 17 |  |
|  | (matching) KeyDescriptor and a passed status. |  | 18 |  |
| b) The procedure shall return with a failed status. |  |  | 19 |  |
| 7.2.6 Blacklist checking procedure |  |  | 20 |  |
|  |  |  | 21 |  |
| The inputs to this procedure are the KeyDescriptor, the device lookup data, and the device lookup size. The |  |  | 22 |  |
|  |  |  | 23 |  |
| outputs from this procedure are a passed or failed status and, if passed, a DeviceDescriptor and a |  |  | 24 |  |
| KeyDeviceDescriptor. |  |  | 25 |  |
| The blacklist checking procedure involves the following steps as applicable: |  |  | 26 |  |
|  |  |  | 27 |  |
| a) For each KeyDeviceDescriptor in the KeyDeviceList of the KeyDescriptor: |  |  | 28 |  |
|  |  |  | 29 |  |
|  | 1) The procedure shall obtain the DeviceDescriptor using the DeviceDescriptorHandle element of |  | 30 |  |
|  | the KeyDeviceDescriptor. |  | 31 |  |
|  | 2) If the UniqueDevice element of the KeyDeviceDescriptor is set to TRUE, the procedure shall |  | 32 |  |
|  | return with the DeviceDescriptor, the KeyDeviceDescriptor, and a passed status if the |  | 33 |  |
|  | BlackListed element of the KeyDeviceDescriptor is set to FALSE, or the procedure shall return |  | 34 |  |
|  | with a failed status if this Blacklisted element is set to TRUE. |  | 35 |  |
|  | 3) If the UniqueDevice element of the KeyDeviceDescriptor is set to FALSE, the procedure shall |  | 36 |  |
|  | execute the DeviceDescriptor lookup procedure as described in 7.2.7, with the device lookup |  | 37 |  |
|  | data and the device lookup size as inputs. If the corresponding output of that procedure is a |  | 38 |  |
|  | passed status, the procedure shall return with the DeviceDescriptor, the KeyDevice-Descriptor, |  | 39 |  |
|  | and a passed status if the Blacklisted element of the KeyDeviceDescriptor is set to FALSE, or the |  | 40 |  |
|  | procedure shall return with a failed status if this Blacklisted element is set to TRUE. |  | 41 |  |
| b) The procedure shall return with a failed status. |  |  | 42 |  |
|  |  |  | 43 |  |
| 7.2.7 Device descriptor lookup procedure |  |  | 44 |  |
|  |  |  | 45 |  |
| The inputs to this procedure are the DeviceDescriptor, the device lookup data, and the device lookup size. |  |  | 46 |  |
| The output from this procedure is a passed or failed status. |  |  | 47 |  |
|  |  |  | 48 |  |
| The DeviceDescriptor lookup procedure involves the following steps as applicable: |  |  | 49 |  |
|  |  |  | 50 |  |
| a) If the device lookup size is four and the device lookup data is equal to the OWPAN ID element of |  |  | 51 |  |
|  | the DeviceDescriptor right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the |  | 52 |  |
|  | ShortAddress element of the Device-Descriptor, this procedure shall return with a passed status. |  | 53 |  |
|  |  |  | 54 |  |
|  | Copyright © 20xx IEEE. All rights reserved. | 269 |  |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

|  | IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC |  | IEEE Draft P802.15.13/D0 |  |
| --- | --- | --- | --- | --- |
| 1 | b) | If the device lookup size is eight and the device lookup data is equal to the ExtAddress element of |  |  |
| 2 |  | the DeviceDescriptor, this procedure shall return with a passed status. |  |  |
| 3 | c) | The procedure shall return with a failed status. |  |  |
| 4 | 7.2.8 Incoming security level checking procedure |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |

- The inputs to this procedure are the incoming security level, the frame type and the command frame

- identifier. The output from this procedure is a passed, failed, or “conditionally passed” status.

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

The incoming security level checking procedure involves the following steps as applicable:

- It is recommended to use MIC for all secure messages as defined in Table 97. For each SecurityLevelDescriptor in the macSecurityLevelTable attribute:

  - If the frame type is not equal to 0x03 and the frame type is equal to the FrameType element of the SecurityLevelDescriptor, the procedure shall compare the incoming security level (as SEC1) with the SecurityMinimum element of the SecurityLevelDescriptor (as SEC2) according to the algorithm described in 7.4.2.1. If this comparison fails (i.e., evaluates to FALSE), the procedure shall return with a “conditionally passed” status if the DeviceOverrideSecurityMinimum element of the SecurityLevelDescriptor is set to TRUE and the security level is set to zero and with a failed status otherwise.

  - If the frame type is equal to 0x03, the frame type is equal to the FrameType element of the SecurityLevelDescriptor, and the command frame identifier is equal to the CommandFrame-Identifier element of the SecurityLevelDescriptor, the procedure shall compare the incoming security level (as SEC1) with the SecurityMinimum element of the SecurityLevelDescriptor (as SEC2) according to the algorithm described in 7.4.2.1. If this comparison fails (i.e., evaluates to FALSE), the procedure shall return with a “conditionally passed” status if the DeviceOverrideSecurityMinimum element of the SecurityLevelDescriptor is set to TRUE and the security level is set to zero and with a failed status otherwise.

- The procedure shall return with a passed status.

7.2.9 Incoming key usage policy checking procedure

The inputs to this procedure are the KeyDescriptor, the frame type, and the command frame identifier. The output from this procedure is a passed or failed status.

The incoming key usage policy checking procedure involves the following steps as applicable:

  - For each KeyUsageDescriptor in the KeyUsageList of the KeyDescriptor:

    - If the frame type is not equal to 0x03 and the frame type is equal to the FrameType element of the KeyUsageDescriptor, the procedure shall return with a passed status.

    - If the frame type is equal to 0x03, the frame type is equal to the FrameType element of the KeyUsageDescriptor, and the command frame identifier is equal to the CommandFrame-Identifier element of the KeyUsageDescriptor, the procedure shall return with a passed status.

  - The procedure shall return with a failed status.

- Security operations

- This subclause describes the parameters for the CCM* security operations, as specified in Annex A of IEEE

- Std 802.15.4-2006.

51

52

53

54

270	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

7.3.1 Integer and octet representation

The integer and octet representation conventions specified in Annex A of IEEE Std 802.15.4-2006 are used throughout 7.3.

7.3.2 CCM* nonce

The CCM* nonce is a 13- octet string and is used for the advanced encryption standard (AES)-CCM* mode of operation (see B.2.2 of IEEE Std 802.15.4-2006). The nonce shall be formatted as shown in Figure 152, with the left most field in the figure defining the first (and left most) octets and the right most field defining the last (and right most) octet of the nonce.

| Octets: 8 | 4 | 1 |
| --- | --- | --- |
| Source address | Frame counter | Security level |

Figure 152—CCM* nonce

The source address shall be set to the extended address aExtendedAddress of the device originating the frame, the frame counter to the value of the respective field in the auxiliary security header (see 7.4), and the security level to the security level identifier corresponding to the Security Level subfield of the Security Control field of the auxiliary security header as defined in Table 97.

The source address, frame counter, and security level shall be represented as specified in 7.3.1.

7.3.3 CCM* prerequisites

Securing a frame involves the use of the CCM* mode encryption and authentication transformation, as described in B.4.1. of IEEE Std 802.15.4-2006. Unsecuring a frame involves the use of the CCM* decryption and authentication checking process, as described in B.4.2 of IEEE Std 802.15.4-2006. The prerequisites for the CCM* forward and inverse transformations are as follows:

— The underlying block cipher shall be the AES encryption algorithm as specified in B.3.1 of IEEE Std 802.15.4-2006.

—  The bit ordering shall be as defined in 7.3.1.

—  The length in octets of the Length field L shall be 2 octets.

—  The length of the Authentication field M shall be 0 octets, 4 octets, 8 octets, or 16 octets, as required.

The length of the Authentication field M for the CCM* forward transformation and the CCM* inverse transformation is determined from Table 97, using the Security Level subfield of the Security Control field of the auxiliary security header of the frame.

7.3.4 CCM* transformation data representation

This subclause describes how the inputs and output of the CCM* forward transformation, as described in B.4.1 of IEEE Std 802.15.4-2006, are formed.

The inputs are as follows:

—  Key

—  Nonce

| Copyright © 20xx IEEE. All rights reserved. | 271 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

—  a data

—  m data

The output is c data.

7.3.4.1 Key and nonce data inputs

The Key data for the CCM* forward transformation is passed by the outgoing frame security procedure described in 7.2.1. The nonce data for the CCM* transformation is constructed as described in 7.3.2.

7.3.4.2 a data and m data

In the CCM* transformation process, the data fields shall be applied as in Table 94.

Table 94—a data and m data for all security levels

| Security level identifier | a data | m data |
| --- | --- | --- |
| 0x00 | None | None |
| 0x01 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields ¦¦ | None |
|  | Unsecured payload fields |  |
| 0x02 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields ¦¦ | None |
|  | Unsecured payload fields |  |
| 0x03 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields ¦¦ | None |
|  | Unsecured payload fields |  |
| 0x04 | None | Unsecured payload fields |
| 0x05 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields | Unsecured payload fields |
| 0x06 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields | Unsecured payload fields |
| 0x07 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields | Unsecured payload fields |

7.3.4.3 c data output

- In the CCM* transformation process, the data fields that are applied, or right-concatenated and applied,

- represent octet strings.

40

- The secured payload fields right-concatenated with the authentication tag shall substitute the unsecured

- payload field in the original unsecured frame to form the secured frame (see Table 95).

43

44

45

46

47

48

49

50

51

52

53

54

Table 95—c data for all security levels

| Security level identifier | c data |
| --- | --- |

0x00	None

0x01	MIC-32

0x02	MIC-64

0x03	MIC-128

272	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 95—c data for all security levels

| Security level identifier | c data |
| --- | --- |
| 0x04 | Secured payload fields |
| 0x05 | Secured payload fields ¦¦ MIC-32 |
| 0x06 | Secured payload fields ¦¦ MIC-64 |
| 0x07 | Secured payload fields ¦¦ MIC-128 |

7.3.5 CCM* inverse transformation data representation

This subclause describes how the inputs and output of the CCM* inverse transformation, as described in C.4.2 of IEEE Std 802.15.4-2006, are formed.

The inputs are as follows:

—  Key

—  Nonce

—  c data

—  a data

The output is m data.

7.3.5.1 Key and nonce data inputs

The key data for the CCM* inverse transformation is passed by the incoming frame security procedure described in 7.2.3. The nonce data for the CCM* transformation is constructed as described in 7.3.2.

7.3.5.2 c data and a data

In the CCM* inverse transformation process, the data fields shall be applied as in Table 96.

Table 96—c data and a data for all security levels

| Security level identifier | c data | a data |
| --- | --- | --- |
| 0x00 | None | None |
| 0x01 | MIC-32 | MHR ¦¦ Auxiliary security header |
|  |  | ¦¦ Nonpayload fields ¦¦Secured |
|  |  | payload fields |
| 0x02 | MIC-64 | MHR ¦¦ Auxiliary security header |
|  |  | ¦¦ Nonpayload fields ¦¦ Secured |
|  |  | payload fields |
| 0x03 | MIC-128 | MHR ¦¦ Auxiliary security header |
|  |  | ¦¦ Nonpayload fields ¦¦ Secured |
|  |  | payload fields |

| Copyright © 20xx IEEE. All rights reserved. | 273 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

Table 96—c data and a data for all security levels (continued)

| Security level identifier | c data | a data |
| --- | --- | --- |
| 0x04 | Secured payload fields | MHR ¦¦ Auxiliary security header |
|  |  | ¦¦ Nonpayload fields |
| 0x05 | Secured payload fields ¦¦ | MHR ¦¦ Auxiliary security header |
|  | MIC-32 | ¦¦ Nonpayload fields |
| 0x06 | Secured payload fields ¦¦ | MHR ¦¦ Auxiliary security header |
|  | MIC-64 | ¦¦ Nonpayload fields |
| 0x07 | Secured payload fields ¦¦ | MHR ¦¦ Auxiliary security header |
|  | MIC-128 | ¦¦ Nonpayload fields |

7.3.5.3 m data output

- The m data shall then substitute secured payload fields and authentication tag in the original secured frame

- to form the unsecured frame.

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

7.4 Auxiliary Security header

The Auxiliary Security Header field has a variable length and contains information required for security processing, including a Security Control field, a Frame Counter field, and a Key Identifier field. The Auxiliary Security Header field shall be present only if the Security Enabled subfield of the frame control field is set to one. The Auxiliary Security Header field shall be formatted as illustrated in Figure 153.

| Octets: 1 | 4 | 0/1/5/9 |
| --- | --- | --- |
| Security Control | Frame Counter | Key Identifier |

Figure 153—Format of the auxiliary security header

7.4.1 Integer and octet representation

- The auxiliary security header is a MAC frame field (see 5.2.1.7) and, therefore, uses the representation

- conventions specified in 5.2.

41

42	7.4.2 Security Control field

43

- The Security Control field is 1 octet in length and is used to provide information about what protection is

- applied to the frame. The Security Control field shall be formatted as shown in Figure 154.

46

47

48

49

50

51

52

53

54

| Bit: 0–2 | 3–4 | 5–7 |
| --- | --- | --- |
| Security Level | Key Identifier Mode | Reserved |

Figure 154—Security Control field format

274	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

7.4.2.1 Security Level subfield

The Security Level subfield indicates the actual frame protection that is provided. This value can be adapted on a frame-by-frame basis and allows for varying levels of data authenticity (to allow minimization of security overhead in transmitted frames where required) and for optional data confidentiality. The cryptographic protection offered by the various security levels is shown in Table 97. When nontrivial protection is required, replay protection is always provided.

Table 97—Security levels available to the MAC sublayer

|  | Security Control |  |  | Data authenticity |
| --- | --- | --- | --- | --- |
| Security level | field | Security | Data | (including length M of |
| identifier | (Figure 154) | attributes | confidentiality | authentication tag, in |
|  | b2 b1 b0 |  |  | octets) |
| 0x00 | '000' | None | OFF | NO (M = 0) |
| 0x01 | '001' | MIC-32 | OFF | YES (M = 4) |
| 0x02 | '010' | MIC-64 | OFF | YES (M = 8) |
| 0x03 | '011' | MIC-128 | OFF | YES (M = 16) |
| 0x04 | '100' | ENC | ON | NO (M = 0) |
| 0x05 | '101' | ENC-MIC-32 | ON | YES (M = 4) |
| 0x06 | '110' | ENC-MIC-64 | ON | YES (M = 8) |
| 0x07 | '111' | ENC-MIC-128 | ON | YES (M = 16) |

Security levels can be ordered according to the corresponding cryptographic protection offered. Here, a first security level SEC1 is greater than or equal to a second security level SEC2 if and only if SEC1 offers at least the protection offered by SEC2, both with respect to data confidentiality and with respect to data authenticity. The statement “SEC1 is greater than or equal to SEC2” shall be evaluated as TRUE if both of the following conditions apply:

- Bit position b2 in SEC1 is greater than or equal to bit position b2 in SEC2 (where Encryption OFF < Encryption ON).

- The integer value of bit positions b1 b0 in SEC1 is greater than or equal to the integer value of bit positions b1 b0 in SEC2 [where increasing integer values indicate increasing levels of data authenticity provided, i.e., message integrity code (MIC)-0 < MIC-32 < MIC-64 < MIC-128].

Otherwise, the statement shall be evaluated as FALSE.

For example, ENC-MIC-64  MIC-64 is TRUE because ENC-MIC-64 offers the same data authenticity protection as MIC-64, plus confidentiality. On the other hand, MIC-128  ENC-MIC- 64 is FALSE because even though MIC-128 offers stronger data authenticity than ENC-MIC-64, it offers no confidentiality.

7.4.2.2 Key Identifier Mode subfield

The Key Identifier Mode subfield indicates whether the key that is used to protect the frame can be derived implicitly or explicitly; furthermore, it is used to indicate the particular representations of the Key Identifier field (see 7.4.4) if derived explicitly. The Key Identifier Mode subfield shall be set to one of the values listed

| Copyright © 20xx IEEE. All rights reserved. | 275 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- in Table 98. The Key Identifier field of the auxiliary security header (see 7.4.4) shall be present only if this

- subfield has a value that is not equal to 0x00.

| 3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4 |  |  | Table 98—Values of the Key Identifier mode |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  | Key Identifier |  | Key Identifier field |  |
| 8 |  |  |  |  |  |  |
|  |  | Key Identifier mode | Mode subfield | Description | length |  |
| 9 |  |  | b1 b0 |  | (octets) |  |
| 10 |  |  |  |  |  |  |
|  |  | 0x00 | '00' | Key is determined implicitly from the | 0 |  |
| 11 |  |  |  |  |  |  |
|  |  |  |  | originator and recipient(s) of the frame, |  |  |
| 12 |  |  |  |  |  |  |
|  |  |  |  | as indicated in the frame header. |  |  |
| 13 |  |  |  |  |  |  |
|  |  | 0x01 | '01' | Key is determined from the 1-octet Key | 1 |  |
| 14 |  |  |  |  |  |  |
| 15 |  |  |  | Index subfield of the Key Identifier field |  |  |
|  |  |  |  | of the auxiliary security header in |  |  |
| 16 |  |  |  |  |  |  |
|  |  |  |  | conjunction with macDefaultKeySource. |  |  |
| 17 |  |  |  |  |  |  |
|  |  | 0x02 | '10' | Key is determined explicitly from the | 5 |  |
| 18 |  |  |  |  |  |  |
| 19 |  |  |  | 4-octet Key Source subfield and the |  |  |
|  |  |  |  | 1-octet Key Index subfield of the Key |  |  |
| 20 |  |  |  |  |  |  |
|  |  |  |  | Identifier field of the auxiliary security |  |  |
| 21 |  |  |  |  |  |  |
|  |  |  |  | header. |  |  |
| 22 |  |  |  |  |  |  |
|  |  | 0x03 | '11' | Key is determined explicitly from the | 9 |  |
| 23 |  |  |  |  |  |  |
|  |  |  |  | 8-octet Key Source subfield and the |  |  |
| 24 |  |  |  |  |  |  |
|  |  |  |  | 1-octet Key Index subfield of the Key |  |  |
| 25 |  |  |  |  |  |  |
|  |  |  |  | Identifier field of the auxiliary security |  |  |
| 26 |  |  |  | header. |  |  |
| 27 |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  |
| 29 | 7.4.3 Frame Counter field |  |  |  |  |  |
| 30 |  |  |  |  |  |  |

- The Frame Counter field is 4 octets in length and represents the macFrameCounter attribute of the originator

- of a protected frame. It is used to provide semantic security of the cryptographic mechanism used to protect

- a frame and to offer replay protection.

34

35	7.4.4 Key Identifier field

36

- The Key Identifier field has a variable length and identifies the key that is used for cryptographic protection

- of outgoing frames, either explicitly or in conjunction with implicitly defined side information. The Key

- Identifier field shall be present only if the Key Identifier Mode subfield of the Security Control field of the

- auxiliary security header (see 7.4.2.2) is set to a value different from 0x00. The Key Identifier field shall be

- formatted as illustrated in Figure 155.

42

43

44

45

46

47

48

49

50

51

52

53

54

| Octets: 0/4/8 | 1 |
| --- | --- |
| Key Source | Key Index |

Figure 155—Format for the Key Identifier field, if present

276	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

7.4.4.1 Key Source subfield

The Key Source subfield, when present, is either 4 octets or 8 octets in length, according to the value specified by the Key Identifier Mode subfield of the Security Control field (see 7.4.2.2), and indicates the originator of a group key.

7.4.4.2 Key Index subfield

The Key Index subfield is 1 octet in length and allows unique identification of different keys with the same originator.

It is the responsibility of each key originator to make sure that actively used keys that it issues have distinct key indices and that the key indices are all different from 0x00.

7.5 Security-related MAC PIB attributes

The security-related MAC PIB attributes contain the following:

—  Key table (macKeyTable, macKeyTableEntries)

—  Device table (macDeviceTable, macDeviceTableEntries)

—  Minimum security level table (macSecurityLevelTable, macSecurityLevelTableEntries)

—  Frame counter (macFrameCounter)

— Automatic request attributes (macAutoRequestSecurityLevel, macAutoRequestKeyIdMode, macAutoRequestKeySource, macAutoRequestKeyIndex)

—  Default key source (macDefaultKeySource)

—  coordinator address (macOWPANCoordExtendedAddress, macOWPANCoordShortAddress)

- PIB security material

The PIB security-related attributes are presented in Table 99, Table 100, Table 101, Table 102, Table 103, Table 104, and Table 105.

Table 99—Security-related MAC PIB attributes

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macKeyTable | 0x71 | List of Key- | — | A table of KeyDescriptor | (empty) |
|  |  | Descriptor |  | entries, each containing |  |
|  |  | entries (see |  | keys and related |  |
|  |  | Table 100) |  | information required for |  |
|  |  |  |  | secured communications. |  |
| macKeyTableEntries | 0x72 | Integer | Implementati | The number of entries in | 0 |
|  |  |  | on specific | macKeyTable. |  |
| macDeviceTable | 0x73 | List of | — | A table of Device- | (empty) |
|  |  | Device- |  | Descriptor entries, each |  |
|  |  | Descriptor |  | indicating a remote device |  |
|  |  | entries (see |  | with which this device |  |
|  |  | Table 104) |  | securely communicates. |  |
| macDeviceTable- | 0x74 | Integer | Implementa- | The number of entries in | 0 |
| Entries |  |  | tion specific | macDeviceTable. |  |

| Copyright © 20xx IEEE. All rights reserved. | 277 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 99—Security-related MAC PIB attributes (continued)

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macSecurity- | 0x75 | Table of | — | A table of SecurityLevel- | (empty) |
| LevelTable |  | SecurityLevel |  | Descriptor entries, each |  |
|  |  | Descriptor |  | with information about the |  |
|  |  | entries (see |  | minimum security level |  |
|  |  | Table 103) |  | expected depending on |  |
|  |  |  |  | incoming frame type and |  |
|  |  |  |  | subtype. |  |
| macSecurity- | 0x76 | Integer | Implementa- | The number of entries in | 0 |
| LevelTableEntries |  |  | tion specific | macSecurityLevelTable. |  |
| macFrameCounter | 0x77 | Integer | 0x00000000– | The outgoing frame | 0x00000000 |
|  |  |  | 0xffffffff | counter for this device. |  |
| macAutoRequest- | 0x78 | Integer | 0x00–0x07 | The security level used for | 0x06 |
| SecurityLevel |  |  |  | automatic data requests. |  |
| macAutoRequest- | 0x79 | Integer | 0x00–0x03 | The key identifier mode | 0x00 |
| KeyIdMode |  |  |  | used for automatic data |  |
|  |  |  |  | requests. This attribute is |  |
|  |  |  |  | invalid if the macAuto- |  |
|  |  |  |  | RequestSecurityLevel |  |
|  |  |  |  | attribute is set to 0x00. |  |
| macAutoRequest- | 0x7a | As specified | — | The originator of the key | All octets |
| KeySource |  | by the mac- |  | used for automatic data | 0xff |
|  |  | AutoRequest- |  | requests. This attribute is |  |
|  |  | KeyIdMode |  | invalid if the macAuto- |  |
|  |  | parameter |  | RequestKeyIdMode |  |
|  |  |  |  | element is invalid or set to |  |
|  |  |  |  | 0x00. |  |
| macAutoRequest- | 0x7b | Integer | 0x01–0xff | The index of the key used | All octets |
| KeyIndex |  |  |  | for automatic data | 0xff |
|  |  |  |  | requests. This attribute is |  |
|  |  |  |  | invalid if the macAuto- |  |
|  |  |  |  | RequestKeyIdMode |  |
|  |  |  |  | attribute is invalid or set to |  |
|  |  |  |  | 0x00. |  |
| macDefaultKey- | 0x7c | Set of 8 octets | — | The originator of the | All octets |
| Source |  |  |  | default key used for key | 0xff |
|  |  |  |  | identifier mode 0x01. |  |
| macOWPANCoord- | 0x7d | IEEE address | An extended | The 64-bit address of the | — |
| ExtendedAddress |  |  | 64-bit IEEE | coordinator. |  |
|  |  |  | address |  |  |
| macOWPANCoordS | 0x7e | Integer | 0x0000–0xffff | The 16-bit short address | 0x0000 |
| hort-Address |  |  |  | assigned to the |  |
|  |  |  |  | coordinator. A value of |  |
|  |  |  |  | 0xfffe indicates that the |  |
|  |  |  |  | coordinator is only using |  |
|  |  |  |  | its 64-bit extended address. |  |
|  |  |  |  | A value of 0xffff indicates |  |
|  |  |  |  | that this value is unknown. |  |

278	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 100—Elements of KeyDescriptor

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| KeyIdLookupList | List of KeyId- | — | A list of KeyIdLookupDescriptor entries |
|  | LookupDescriptor |  | used to identify this KeyDescriptor. |
|  | entries (see |  |  |
|  | Table 105) |  |  |
| KeyIdLookupListEntries | Integer | Implementation | The number of entries in |
|  |  | specific | KeyIdLookupList. |
| KeyDeviceList | List of KeyDevice- | — | A list of KeyDeviceDescriptor entries |
|  | Descriptor entries |  | indicating which devices are currently |
|  | (see Table 102) |  | using this key, including their blacklist |
|  |  |  | status. |
| KeyDeviceListEntries | Integer | Implementation | The number of entries in KeyDeviceList. |
|  |  | specific |  |
| KeyUsageList | List of KeyUsage- | — | A list of KeyUsageDescriptor entries |
|  | Descriptor entries |  | indicating the frame types with which this |
|  | (see Table 101) |  | key may be used. |
| KeyUsageListEntries | Integer |  | The number of entries in KeyUsageList. |
| Key | Set of 16 octets | — | The actual value of the key. |

Table 101—Elements of KeyUsageDescriptor

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| FrameType | Integer | 0x00–0x03 | As defined in 5.2.1.1.2. |
| CommandFrameIdentifier | Integer | 0x00–0x09 | As defined in Table 29. |

Table 102—Elements of KeyDeviceDescriptor

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| DeviceDescriptorHandle | Integer | Implementa- | Handle to the DeviceDescriptor |
|  |  | tion specific | corresponding to the device (see |
|  |  |  | Table 104). |
| UniqueDevice | Boolean | TRUE or | Indication of whether the device indicated |
|  |  | FALSE | by DeviceDescriptorHandle is uniquely |
|  |  |  | associated with the KeyDescriptor, i.e., it |
|  |  |  | is a link key as opposed to a group key. |
| Blacklisted | Boolean | TRUE or | Indication of whether the device indicated |
|  |  | FALSE | by DeviceDescriptorHandle previously |
|  |  |  | communicated with this key prior to the |
|  |  |  | exhaustion of the frame counter. If TRUE, |
|  |  |  | this indicates that the device shall not use |
|  |  |  | this key further because it exhausted its |
|  |  |  | use of the frame counter used with this |
|  |  |  | key. |

| Copyright © 20xx IEEE. All rights reserved. | 279 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 103—Elements of SecurityLevelDescriptor

|  | Name | Type | Range | Description |
| --- | --- | --- | --- | --- |
|  | FrameType | Integer | 0x00–0x03 | As defined in 5.2.1.1.2. |
|  | CommandFrameIdentifier | Integer | 0x00–0x09 | As defined in Table 29. |
|  | SecurityMinimum | Integer | 0x00–0x07 | The minimal required/expected security |
|  |  |  |  | level for incoming MAC frames with the |
|  |  |  |  | indicated frame type and, if present, |
|  |  |  |  | command frame type (as defined in |
|  |  |  |  | Table 97 in 7.4.2.1). |
|  | DeviceOverrideSecurity- | Boolean | TRUE or | Indication of whether originating devices |
|  | Minimum |  | FALSE | for which the Exempt flag is set may |
|  |  |  |  | override the minimum security level |
|  |  |  |  | indicated by the SecurityMinimum |
|  |  |  |  | element. If TRUE, this indicates that for |
|  |  |  |  | originating devices with Exempt status, |
|  |  |  |  | the incoming security level zero is |
|  |  |  |  | acceptable, in addition to the incoming |
|  |  |  |  | security levels meeting the minimum |
|  |  |  |  | expected security level indicated by the |
|  |  |  |  | SecurityMinimum element. |

Table 104—Elements of DeviceDescriptor

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| OWPANId | Device OWPAN | 0x0000–0xffff | The 16-bit OWPAN identifier of the device |
|  | ID |  | in this DeviceDescriptor. |
| ShortAddress | Device short | 0x0000–0xffff | The 16-bit short address of the device in |
|  | address |  | this DeviceDescriptor. A value of 0xfffe |
|  |  |  | indicates that this device is using only its |
|  |  |  | extended address. A value of 0xffff |
|  |  |  | indicates that this value is unknown. |
| ExtAddress | IEEE address | Any valid 64-bit device | The 64-bit IEEE extended address of the |
|  |  | address | device in this DeviceDescriptor. This |
|  |  |  | element is also used in unsecuring |
|  |  |  | operations on incoming frames. |
| FrameCounter | Integer | 0x00000000–0xffffffff | The incoming frame counter of the device |
|  |  |  | in this DeviceDescriptor. This value is used |
|  |  |  | to ensure sequential freshness of frames. |
| Exempt | Boolean | TRUE or FALSE | Indication of whether the device may |
|  |  |  | override the minimum security level |
|  |  |  | settings defined in Table 103. |

7.5.2 Key table

The key table holds key descriptors (keys with related key-specific information) that are required for security processing of outgoing and incoming frames. Key-specific information in the key table is identified based on information explicitly contained in the requesting primitive or in the received frame, as described in the outgoing frame key retrieval procedure (see 7.2.2) and the incoming frame security material retrieval procedure (see 7.2.4), as well as in the KeyDescriptor lookup procedure (see 7.2.5).

280	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 105—Elements of KeyIdLookupDescriptor

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| LookupData | Set of 5 or 9 octets | — | Data used to identify the key. |
| LookupDataSize | Integer | 0x00–0x01 | A value of 0x00 indicates a set |
|  |  |  | of 5 octets; a value of 0x01 |
|  |  |  | indicates a set of 9 octets. |

7.5.3 Device table

The device table holds device descriptors (device -specific addressing information and security-related information) that, when combined with key-specific information from the key table, provide all the keying material needed to secure outgoing (see 7.2.1) and unsecure incoming frames (see 7.2.3). Device-specific information in the device table is identified based on the originator of the frame, as described in the DeviceDescriptor lookup procedure (see 7.2.7), and on key-specific information, as described in the blacklist checking procedure (see 7.2.6).

7.5.4 Minimum security level table

The minimum security level table holds information regarding the minimum security level the device expects to have been applied by the originator of a frame, depending on frame type and, if it concerns a MAC command frame, the command frame identifier. Security processing of an incoming frame will fail if the frame is not adequately protected, as described in the incoming frame security procedure (see 7.2.3) and in the incoming security level checking procedure (see 7.2.8).

7.5.5 Frame counter

The 4-octet frame counter is used to provide replay protection and semantic security of the cryptographic building block used for securing outgoing frames. The frame counter is included in each secured frame and is one of the elements required for the unsecuring operation at the recipient(s). The frame counter is incremented each time an outgoing frame is secured, as described in the outgoing frame security procedure (see 7.2.1). When the frame counter reaches its maximum value of 0xffffffff, the associated keying material can no longer be used, thus requiring all keys associated with the device to be updated. This provides a mechanism for ensuring that the keying material for every frame is unique and, thereby, provides for sequential freshness.

7.5.6 Automatic request attributes

Automatic request attributes hold all the information needed to secure outgoing frames generated automatically and not as a result of a higher layer primitive, as is the case with automatic data requests.

7.5.7 Default key source

The default key source is information commonly shared between originator and recipient(s) of a secured frame, which, when combined with additional information explicitly contained in the requesting primitive or in the received frame, allows an originator or a recipient to determine the key required for securing or unsecuring this frame, respectively. This provides a mechanism for significantly reducing the overhead of security information contained in secured frames in particular use cases as shown in 7.2.2 and 7.2.4.

| Copyright © 20xx IEEE. All rights reserved. | 281 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

|  | IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC | IEEE Draft P802.15.13/D0 |  |
| --- | --- | --- | --- |
| 1 | 7.5.8 Coordinator address |  |  |
| 2 | The address of the coordinator is information commonly shared between all devices in a OWPAN, which, |  |  |
| 3 |  |  |  |
| 4 | when combined with additional information explicitly contained in the requesting primitive or in the |  |  |
| 5 | received frame, allows an originator of a frame directed to | the coordinator or a recipient of a frame |  |

- originating from the coordinator to determine the key and security-related information required for securing

- or unsecuring, respectively, this frame as shown in 7.2.2 and 7.2.4.

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

8. PHY layer s pecification

8.1 Overview

This clause specifies seven(ED NOTE: or is this going to be eight?) PHY options for IEEE Std 802.15.13.

The PHY is responsible for the following tasks:

—  Activation and deactivation of the OWC transceiver

—  WQI for received frames

—  Channel selection

—  Data transmission and reception

—  Error correction

—  Synchronization

Constants and attributes that are specified and maintained by the PHY are written in the text of this clause in italics. Constants have a general prefix of “a”, e.g., aMaxPHYFrameSize, and are listed in Table 143. Attributes have a general prefix of “phy”, e.g., phyCurrentChannel, and are listed in Table 144.

This subclause specifies requirements that are common to all of the IEEE 802.15.13 PHYs.

8.2 Operating modes

A compliant IEEE 802.15.7-20xx PHY shall implement at least one of the PHY I or PHY II mandatory modes (as defined in Clause 10 and Clause 10) given in Table 106 and Table 107. A device implementing the PHY III mode in Table 108 shall also implement PHY II mode for coexistence as summarized in 4.4.1.2. The PHY modulation modes may operate in the presence of dimming. Modulation using OOK under dimming provides constant range and variable data rate by inserting compensation time as defined in 4.4.3.1.4 while modulation using VPPM under dimming provides constant data rate and variable range by adjusting the pulse width as summarized in 4.4.3.1.6.

As shown in Table 106 through Table 110, the standard provides channel coding support for error correction. PHY I supports concatenated coding with Reed-Solomon (RS) and convolutional coding (CC) since it has been designed for outdoor use with short frames. PHY II and PHY III support only RS coding. PHY I and PHY II also support a run length limited (RLL) code to provide DC balance, clock recovery, and flicker mitigation.

In addition to modulation and coding, multiple optical rates are provided for all PHY types in order to support a broad class of optical transmitters (LEDs) for various applications. The choice of optical rate used for communication is decided by the MAC during device discovery. The MAC shall select the optical clock rate for communication during the optical clock-rate selection process as defined in 6.5. The preamble shall be sent at clock rate chosen by the TX and supported by the RX. The preamble is a time domain sequence

282	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

and does not have any modulation, channel coding, or line coding. The PHY header shall be sent at the lowest data rate for the chosen clock rate. The clock rate does not change through the frame between the preamble, header, and payload.

Table 106—PHY I operating modes

| Modulation | RLL code | Optical |  | FEC |  | Data rate |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | clock rate |  |  |  |  |  |
|  |  |  | Outer code (RS) |  | Inner code (CC) |  |  |
|  |  |  | (15,7) |  | 1/4 | 11.67 kb/s |  |
|  |  |  | (15,11) |  | 1/3 | 24.44 kb/s |  |
| OOK | Manchester | 200 kHz | (15,11) |  | 2/3 | 48.89 kb/s |  |
|  |  |  | (15,11) |  | none | 73.3 kb/s |  |
|  |  |  | none |  | none | 100 kb/s |  |
|  |  |  | (15,2) |  | none | 35.56 kb/s |  |
| VPPM | 4B6B | 400 kHz | (15,4) |  | none | 71.11 kb/s |  |
|  |  |  | (15,7) |  | none | 124.4 kb/s |  |
|  |  |  | none |  | none | 266.6 kb/s |  |

Table 107—PHY II operating modes

| Modulation | RLL code | Optical clock | FEC | Data rate |  |
| --- | --- | --- | --- | --- | --- |
|  |  | rate |  |  |  |
|  |  | 3.75 MHz | RS(64,32) | 1.25 Mb/s |  |
|  |  |  | RS(160,128) | 2 Mb/s |  |
| VPPM | 4B6B |  | RS(64,32) | 2.5 Mb/s |  |
|  |  | 7.5 MHz | RS(160,128) | 4 Mb/s |  |
|  |  |  | none | 5 Mb/s |  |

| Copyright © 20xx IEEE. All rights reserved. | 283 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 107—PHY II operating modes

| Modulation | RLL code | Optical clock | FEC | Data rate |  |
| --- | --- | --- | --- | --- | --- |
|  |  | rate |  |  |  |
|  |  | 15 MHz | RS(64,32) | 6 Mb/s |  |
|  |  |  | RS(160,128) | 9.6 Mb/s |  |
|  |  | 30 MHz | RS(64,32) | 12 Mb/s |  |
|  |  |  | RS(160,128) | 19.2 Mb/s |  |
| OOK | 8B10B | 60 MHz | RS(64,32) | 24 Mb/s |  |
|  |  |  | RS(160,128) | 38.4 Mb/s |  |
|  |  |  | RS(64,32) | 48 Mb/s |  |
|  |  | 120 MHz | RS(160,128) | 76.8 Mb/s |  |
|  |  |  | none | 96 Mb/s |  |

Table 108—PHY III operating modes

| Modulation | Optical clock rate | FEC | Data rate |  |
| --- | --- | --- | --- | --- |
| 4-CSK | 12 MHz | RS(64,32) | 12 Mb/s |  |
| 8-CSK |  | RS(64,32) | 18 Mb/s |  |
| 4-CSK |  | RS(64,32) | 24 Mb/s |  |
| 8-CSK |  | RS(64,32) | 36 Mb/s |  |
| 16-CSK | 24 MHz | RS(64,32) | 48 Mb/s |  |
| 8-CSK |  | none | 72 Mb/s |  |
| 16-CSK |  | none | 96 Mb/s |  |

|  | Table 109—PHY VII operating modes |
| --- | --- |
|  | Operating Modes (16/310r0) |
| Modulation | DCO-OFDM, eU-OFDM, RPO-OFDM |
| FEC | Convolutional Coding (131, 171) |
| Code Rates | 1/2, 2/3, 3/4 |
| MIMO | Up to 16x16 |

284	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 109—PHY VII operating modes

Operating Modes (16/310r0)

| Modulation | Bandwidth | Optical | FEC | Data Rate |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Clock Rate |  | (SISO) |  |
| BPSK |  |  | 1/2 | 0.75 - 12 Mb/s |  |
| BPSK |  |  | 3/4 | 1.375 - 18 Mb/s |  |
| 4-QAM |  |  | 1/2 | 1.5 - 24 Mb/s |  |
| 4-QAM | 5 - 40 MHz | 10 - 80 MHz | 3/4 | 2.75 - 36 Mb/s |  |
| 16-QAM |  |  | 1/2 | 3 - 48 Mb/s |  |
| 16-QAM |  |  | 3/4 | 4.5 - 72 Mb/s |  |
| 64-QAM |  |  | 2/3 | 6-96 Mb/s |  |
| 64-QAM |  |  | 3/4 | 6.75 - 108 Mb/s |  |

Table 110—PHY VIII Operating Modes

PHY Operating Modesa

|  | modulation |  | DC-biased DMT |  |  |
| --- | --- | --- | --- | --- | --- |
| subcarrier spacing |  |  | 195.3125 KHz |  |  |
| cyclic prefix (CP) |  |  | 160, 320 ns |  |  |
| symbol duration (w/o CP) |  |  | 5.12 us |  |  |
|  | FEC |  | Low-density parity-check code (LDPC) |  |  |
| information block size |  |  | 120, 540 bytes |  |  |
|  | code rates |  | 1/2, 2/3, 5/6, 16/18, 20/21 |  |  |
|  | repetitions |  | 1, 2, 3, 4, 6, 8 |  |  |
| Bandwidth |  | Optical Clock Rate | Used Carriers | Min. to Max. Data |  |
|  |  | (Factor) | IFFT size using DMT | Rateb |  |
| 10 MHz |  | 25 MS/s (1/8) | 45 (128) | 1.5 to 103 Mb/s |  |
| 25 MHz |  | 50 MS/s (1/4) | 107 (256) | 3 to 257 Mb/s |  |
| 50 MHz |  | 100 MS/s (1/2) | 215 (512) | 7 to 512 Mb/s |  |
| 100 MHz |  | 200 MS/s (1/1) | 450 (1024) | 14 to 1028 Mb/s |  |
| 200 MHz |  | 400 MS/s (2/1) | 950 (2048) | 35 to 2056 Mb/s |  |
| 500 MHz |  | 1 GS/s (5/1) | 2375 (2x2048) | 87 to 5140 Mb/s |  |
| 1 GHz |  | 2 GS/s (100/1) | 4750 (3x2048) | 145 to 10281 Mb/s |  |

| Copyright © 20xx IEEE. All rights reserved. | 285 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- a The PHY parameters are derived from the ITU-T recommendation G.9960 (12/2015). They were generically

- scaled towards lower and higher bandwidths, for achieving improved energy efficiency and higher data rate,

- respectively.

4

5

6

7

8

9

10

11

12

13

14

  - Max./Min. data rates assume that all used subcarriers are occupied. Max. rates assume 12 bits/subcarrier and code rate 20/21. Data rate is variable by means of adaptive bit loading, different FEC rates and repetitions. Min. rates assume 1 bit/subcarrier, rate ½ and 3 repetitions. Even lower data rates are possible by partial usage of subcarriers though adaptive bit loading.

- General requirements

| 18 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 19 |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  |
|  |  | (nm) |  | (nm) |  |  |  |
| 24 |  |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  |  |  |
| 38 |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  |  |

45

46	8.3.2 Optical mapping

47

- A high switching level from the PHY, applied to the light source, shall result in a high radiated intensity. A

- low switching level from the PHY, applied to the light source, shall result in a reduced radiated intensity. The

- extinction ratio, defined as the ratio of the high radiated intensity to the low radiated intensity, is at the

- discretion of the implementer.

52

53

54

286	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 8.3.3 Maximum error tolerance for multiple optical sources |  | 1 |  |
| --- | --- | --- | --- |
| If multiple optical sources are used for communication, it is recommended the optical sources have similar |  | 2 |  |
|  |  | 3 |  |
| frequency responses in order to assist communication. The digital input to all the optical sources from the |  | 4 |  |
| PHY shall be synchronized. Figure 156 shows the allowable spread at the output of the optical sources, |  | 5 |  |
| assuming a synchronized digital input. The maximum spread at the average signal intensity level during the |  | 6 |  |
| rise and fall time at the output of the optical sources shall not vary by more than 12.5% of the clock period. |  | 7 |  |
|  |  | 8 |  |
|  |  | 9 |  |
|  |  | 10 |  |
|  |  | 11 |  |
|  |  | 12 |  |
|  |  | 13 |  |
|  |  | 14 |  |
|  |  | 15 |  |
|  |  | 16 |  |
|  |  | 17 |  |
|  |  | 18 |  |
|  |  | 19 |  |
|  |  | 20 |  |
|  |  | 21 |  |
|  |  | 22 |  |
|  |  | 23 |  |
|  |  | 24 |  |
|  |  | 25 |  |
|  |  | 26 |  |
|  |  | 27 |  |
|  |  | 28 |  |
|  |  | 29 |  |
|  |  | 30 |  |
|  |  | 31 |  |
|  |  | 32 |  |
|  |  | 33 |  |
|  |  | 34 |  |
|  |  | 35 |  |
|  |  | 36 |  |
|  |  | 37 |  |
|  |  | 38 |  |
| Figure 156—Maximum allowable spread at the output of optical sources |  | 39 |  |
|  |  | 40 |  |
|  |  | 41 |  |
|  |  | 42 |  |
|  |  | 43 |  |
|  |  | 44 |  |
|  |  | 45 |  |
|  |  | 46 |  |
|  |  | 47 |  |
|  |  | 48 |  |
|  |  | 49 |  |
|  |  | 50 |  |
|  |  | 51 |  |
|  |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 287 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

|  | IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC | IEEE Draft P802.15.13/D0 |  |
| --- | --- | --- | --- |
| 1 | 8.3.4 Minimum LIFS, SIFS, and RIFS periods |  |  |
| 2 | An interframe spacing (IFS) is used to provide spacing between adjacent frames. The minimum spacing |  |  |
| 3 |  |  |  |
| 4 | between frames is dependent on the MAC mode of operation. The | standard provides three types of |  |

- interframe spacing: long (LIFS), short (SIFS), and reduced (RIFS). For peer-to-peer and star topologies, the

- SIFS, LIFS, and RIFS period is based on the currently negotiated optical clock rate by the MAC before

- starting data communication. Once the optical clock rate is selected, the SIFS, LIFS, and RIFS period is

8	fixed to the values shown in Table 112. The clock-rate negotiation for a peer-to-peer and star topology is

- provided in 6.5. For a star topology, the beacon and CAP periods are defined at the lowest optical clock rate

- to ensure fair access to the medium. For a broadcast topology, the IFS is defined based on the optical clock

- rate chosen for broadcasting data to other devices. The minimum LIFS, SIFS, and RIFS periods for each of

- the PHYs are shown in Table 112. A detailed description, use, and illustration of LIFS, SIFS, and RIFS is

- shown in Figure 23.

| 14 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 |  |  | Table 112—Minimum LIFS, SIFS, and RIFS periods |  |  |  |  |
| 16 |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |
| 18 |  | PHY | macMinLIFSPeriod | macMinSIFSPeriod | macMinRIFSPeriod | Units |  |
| 19 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | optical |  |
| 20 |  | PHY I | 400 | 120 | 40 |  |  |
|  |  |  |  |  |  | clocks |  |
| 21 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | optical |  |
| 22 |  | PHY II | 400 | 120 | 40 |  |  |
| 23 |  |  |  |  |  | clocks |  |
| 24 |  | PHY III | 400 | 120 | 40 | optical |  |
| 25 |  |  |  |  |  | clocks |  |
| 26 |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |
| 28 | 8.3.5 TX-to-RX turnaround time |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |

- The TX-to-RX turnaround time shall be as shown in Table 143 and shall be measured at the air interface

- from the trailing edge of the last clock of the last transmission until the receiver is ready to begin the

- reception of the next PHY frame.

33

34

35

36

37

8.3.6 RX-to-TX turnaround time

The RX-to-TX turnaround time shall be as shown in Table 143 and shall be measured at the air interface from the trailing edge of the last clock of the received frame until the transmitter is ready to begin

- transmission of the resulting acknowledgment. Actual transmission start times are specified by the MAC

- sublayer.

| 40 |  |
| --- | --- |
| 41 | 8.3.7 Transmit data clock frequency tolerance |
| 42 |  |
| 43 | The transmitted data clock frequency tolerance shall be ±20 ppm maximum. |
| 44 |  |
| 45 | 8.3.8 Wavelength quality indicator (WQI) |
| 46 |  |
| 47 | 8.3.8.1 OOK and VPPM WQI support |
| 48 |  |

- The WQI measurement is a characterization of the strength and/or quality of a received frame. The

- measurement may be implemented using receiver energy detection (ED), a signal-to-noise ratio estimation,

- or a combination of these methods. The use of the WQI result by the network or application layers is not

- specified in this standard. The WQI measurement shall be performed for each received frame, and the result

- shall be reported to the MAC sublayer using the PD-DATA.indication, specified in 9.3.3, as an integer

- ranging from 0x00 to 0xff. The minimum and maximum WQI values (0x00 and 0xff) should be associated

288	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| with the lowest and highest quality IEEE 802.15.7 signals detectable by the receiver, and WQI values in |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- |
| between should be uniformly distributed between these two limits. At least seven unique values of WQI |  |  |  | 2 |  |
| shall be used. WQI value shall indicate the band plan ID, as given by the value in the PHY header of the |  |  |  | 3 |  |
| received frame. A single WQI value set consists of band plan ID and corresponding WQI value as defined in |  |  |  | 4 |  |
| Table 44. |  |  |  | 5 |  |
|  |  |  |  | 6 |  |
|  |  |  |  | 7 |  |
|  |  |  |  | 8 |  |
|  |  |  |  | 9 |  |
|  |  |  |  | 10 |  |
|  |  |  |  | 11 |  |
|  |  |  |  | 12 |  |
|  |  |  |  | 13 |  |
|  |  |  |  | 14 |  |
|  |  |  |  | 15 |  |
|  |  |  |  | 16 |  |
| 8.3.9 Clear channel assessment (CCA) |  |  |  | 17 |  |
|  |  |  |  | 18 |  |
| The IEEE 802.15.13 PHY may provide the capability to perform CCA according to at least one of the |  |  |  | 19 |  |
|  |  |  |  | 20 |  |
| following three methods: |  |  |  | 21 |  |
| a) CCA Mode 1: Energy above threshold. CCA may report a busy medium upon detecting any energy |  |  |  | 22 |  |
|  |  |  |  | 23 |  |
|  |  | above the energy detect threshold. |  | 24 |  |
| b) | CCA Mode 2: Carrier sense only. CCA may report a busy medium only upon the detection of a |  |  | 25 |  |
|  |  | signal with the modulation characteristics of IEEE Std 802.15.13. This signal may be above or |  | 26 |  |
|  |  | below the energy detect threshold. |  | 27 |  |
| c) | CCA Mode 3: Carrier sense with energy above threshold. CCA may report a busy medium only |  |  | 28 |  |
|  |  | upon the detection of a signal with the modulation characteristics of IEEE Std 802.15.13 with energy |  | 29 |  |
|  |  | above the energy detect threshold. See 4.3 for conceptual guidance. |  | 30 |  |
| For any of the CCA modes, if the PLME-CCA.request primitive, specified in 9.2.1, is received by the PHY |  |  |  | 31 |  |
|  |  |  |  | 32 |  |
| during reception of a PPDU, CCA may report a busy medium. PPDU reception is considered to be in |  |  |  | 33 |  |
| progress following detection of the preamble, and it remains in progress until the number of octets specified |  |  |  | 34 |  |
| by the decoded PHR has been received. |  |  |  | 35 |  |
| A busy channel may be indicated by the PLME-CCA.confirm primitive with a status of BUSY as specified |  |  |  | 36 |  |
|  |  |  |  | 37 |  |
| in 9.2.2. |  |  |  | 38 |  |
| A clear channel may be indicated by the PLME-CCA.confirm primitive with a status of IDLE as specified in |  |  |  | 39 |  |
|  |  |  |  | 40 |  |
| 9.2.2. |  |  |  | 41 |  |
| The PHY PIB attribute phyCCAMode may indicate the appropriate operation mode as specified in |  |  |  | 42 |  |
|  |  |  |  | 43 |  |
| Table 143. |  |  |  | 44 |  |
|  |  |  |  | 45 |  |
| 8.4 Data modes |  |  |  | 46 |  |
|  |  |  |  | 47 |  |
| The PHY shall support the following normal data transmission modes as shown in Figure 157: |  |  |  | 48 |  |
|  |  |  |  | 49 |  |
| a) |  | Single mode |  | 50 |  |
|  |  |  |  | 51 |  |
| b) |  | Packed mode |  |  |  |
|  |  |  |  | 52 |  |
| c) |  | Burst mode |  |  |  |
|  |  |  |  | 53 |  |
|  |  |  |  | 54 |  |
|  |  | Copyright © 20xx IEEE. All rights reserved. | 289 |  |  |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- In addition, there is a fourth mode for data transfer called “dimmed OOK” mode, which is used for data

- transfer while dimming in conjunction with OOK.

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

Figure 157—Data modes supported by the MAC (single, packed, burst)

The single mode transfers one PPDU per frame. This may be used for very short data communication such as acknowledgments, association, beaconing, or for information broadcast mode, for example.

The packed mode contains multiple PPDUs per frame and is used to send multiple consecutive PPDUs to the same destination within the frame for high throughput. Thus, the overhead of sending multiple MAC and PHY headers to the same destination is eliminated in this mode, providing higher MAC efficiency. This can be used in most modes as the preferred means of data communication.

The burst data mode uses a reduced length PHY preamble, as defined in 8.6.1, after the first frame in the burst. In addition, the RIFS is used between frames instead of the SIFS. The shorter preamble increases the efficiency and throughput in this mode.

The dimmed OOK mode is used to support data transfer under dimming requirements, as summarized in 4.4.3.1.4.

8.5 Dimming and flicker mitigation

- Editor Note: this clause needs careful review due to a mixture of informative and normative text. The infor-

- mative text should be in clause 4.5.3.x and the normative text needs to be in clause 8.5.x.x. Please submit

- editing comments. Thanks!

47

- A compliant IEEE 802.15.13 device shall honor all dimming requests from the upper layer. The dimming

- request from the upper layers to the PHY shall be indicated using the PHY PIB attribute, phyDim as shown

- in Table 144. The PHY shall support dimming using one of the techniques specified in either 8.5.1 or 8.5.2,

- when the phyDim PHY PIB attribute is set.

52

53

54

290	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

8.5.1 Dimming during idle time

The dimming during idle time is supported to avoid flicker and is achieved by the methods described in 4.4.3.1.1.

8.5.1.1 Idle pattern and compensation time dimming

An in-band or out -of-band idle pattern whose duty cycle variation results in brightness variation may be optionally inserted between the data frames for light dimming. Note that the concept of out-of-band includes the option of using an un-modulated DC bias to maintain properly dimmed visibility. The compensation time (which means “ON” or “OFF” time of a light source) can be also inserted into either the idle pattern or into the data frame (if using the dimmed OOK mode) to reduce or increase the average brightness of a light source.

8.5.1.2 Visibility pattern dimming

The visibility pattern is an in -band idle pattern and is sent as part of the payload of the CVD frame as defined in 5.2.2.6. A set of 11 base low resolution patterns with 10% step size shall be used for dimming using visibility patterns. Any set of 11 base low resolution visibility patterns of any length can be used as long as there is no conflict between the visibility pattern and a valid RLL code. A set of 11 patterns are provided in Table 113 as an example for 8B10B code. The low resolution patterns shall be used to develop high resolution visibility patterns by averaging them across time to generate the required high resolution pattern. For example, if visibility patterns are available at 10% resolution, then a 25% visibility pattern can be attained for example, by alternately sending a 20% visibility pattern followed by a 30% visibility pattern. This method guarantees all visibility patterns will retain the same properties as the base low resolution visibility patterns. The high resolution visibility pattern shall be provided by using the low resolution patterns using the algorithm specified in Figure 158. The visibility patterns are repeated to satisfy the frame length as mentioned in the PHY header.

Table 113—Example of visibility patterns for 8B10B code

| Visibility pattern | Percentage visibility |
| --- | --- |
| 11111 11111 | 100% |
| 11110 11111 | 90% |
| 11110 11110 | 80% |
| 11101 11100 | 70% |
| 11001 11100 | 60% |
| 10001 11100 | 50% |
| 00001 11100 | 40% |
| 00001 11000 | 30% |
| 00001 10000 | 20% |
| 00001 00000 | 10% |
| 00000 00000 | 0% |

| Copyright © 20xx IEEE. All rights reserved. | 291 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

Let the following values be defined as follows:

—   Visibility patterns: V0, V1, … , VK

— Desired visibility = dv (expressed as a percentage value) e.g., for a 25.3% visibil-ity, dv = 25.3

Desired precision = p, p ≤ 0, p is an integer (expressed as a logarithm value) e.g., for 0.01%, precision, p = –2

Then, to achieve visibility dv:

—   repeat Vsel1pat reppat1 times, and

—   repeat Vsel2pat reppat2 times.

Figure 158—Algorithm for achieving 0.1% dimming resolution with visibility patterns

8.5.2 Dimming during data transmission time

- The dimming technologies on data transmission time depend on the PHY modulation schemes and are

- designed to avoid flicker. As stated in 8.5, all devices shall honor dimming requests but a device shall not be

- required to support communication for any dimming request. In this case a device may issue a disassociation

- notification command (see 5.4.3) with the reason given in Table 32. Due to non-linear human eye response

- to light, dimming levels as low as 0.1% shall be supported (square law phenomenon).

37

38	8.5.2.1 CSK-mode dimming

39

- The CSK-mode dimming is described in 4.4.3.1.3. In CSK, total average power of multiple light sources is

- constant. For dimming control, the instantaneous power per light source is changed in order to adjust the

- average intensity to the required level. CSK keeps the center color of the color constellation with required

- intensity. A color stabilization scheme for illuminators is also provided in 8.6.

44

45	8.5.2.2 OOK-mode dimming

46

- The OOK-mode dimming is described in 4.4.3.1.4. The OOK-mode dimming is supported by using the

- dimmed OOK bit field set in the PHY header as explained in Table 117 in 8.6.2. An arbitrary dimming level

- accuracy can be achieved by the combined use of the compensation length (described in 8.6.4.2), and optical

- mapping and extinction ratio (described in 8.3.2). If any requested dimming results in unsatisfactory

- performance (e.g., flicker generation or color shifting) while trying to maintain compliance to this standard,

- then the device shall disassociate from the network. The dimming method used by an unassociated device is

- out-of-scope of this standard.

54

292	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

8.5.2.3 VPPM-mode dimming

VPPM-mode dimming is described in 4.4.3.1.6. VPPM modulation, as implemented here (see 10.6), supports a dimming-level resolution of 10%. To support a dimming resolution of 0.1%, as prescribed in 5.4.15, the VPPM PHY shall use the algorithm provided below.

The algorithm relies on the following symbols: VS0, VS1, VS2, … VS10. VS0 corresponds to the light source being turned off ( macDim = 0) and VS10 corresponds to the light source fully being turned on (macDim = 1000). VS1 to VS9 are the VPPM symbols for d = 0.1 to 0.9 (see 10.6).

- Choose the dimming level macDim (see Table 91).

- First, determine the type of the corresponding symbols, viz. k1  macDim /100

and k2  macDim /100 , where  stands for rounding to the next lower integer and  for rounding to the next higher integer.

- Next, calculate the number of how often each symbol is to be sent: rep_2 = macDim – 100  k1 and rep_1 = 100 – rep_2.

- Then, to achieve the desired dimming level macDim:

—Sequentially assign VSk1 rep_1 times, and then, —assign VSk2 rep_2 times.

—If the number of VPPM data symbols, to be sent, is not modulo 100, then add VPPM idle-pattern symbols so that the number of VPPM symbols to be sent becomes multiples of 100. The con-figurations of VPPM data and idle-pattern symbols are shown in Figure 159.

|  |  |  |  |  | data transmission |  |  |  |  |  |  |  |  |  |  |  |  |  | none data |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | VPPM | VPPM |  | VPPM |  |  |  | VPPM |  |  |  |  |  | VPPM |  | VPPM |  |  | VPPM |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | idle |  |  |  |  |  |  |  |  |  |  |
|  |  | data | data |  |  | data |  |  | data |  |  |  |  |  | data |  |  |  |  | idle pattern |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | pattern |  |  |  |  |  |  |  |  |  |  |
|  |  | VSk1 | VSk2 |  |  | VSk1 |  |  | VSk2 |  |  |  |  |  | VSk1 |  | VSk1 |  |  | VSk2 |  |  |  |  |  |  |  |
|  |  | fractionally dimmed |  |  |  | fractionally dimmed |  |  |  |  |  |  |  |  |  |  | fractionally dimmed |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | none |  |  |  |
|  |  |  |  |  |  |  | data transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | data |  |  |  |
|  |  | VPPM | VPPM |  | VPPM |  |  |  | VPPM |  |  |  |  |  |  | VPPM |  |  |  | VPPM |  |  | VPPM |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | idle |  |  |  |
|  |  | data | data |  |  | data |  |  | data |  |  |  |  |  |  | data |  |  |  | data |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | pattern |  |  |  |  |  |
|  |  | VSk1 | VSk2 |  |  | VSk1 |  |  | VSk2 |  |  |  |  |  |  |  | VSk1 |  |  | VSk2 |  |  |  | VSk2 |  |  |  |

fractionally dimmed   fractionally dimmed   fractionally dimmed

Figure 159—Sequential proportional cycling between two duty symbols to achieve fractional dimming with 0.1% accuracy in VPPM-mode

The upper panel shows padding with VSk1 and then VSk2 idle patterns. The lower panel shows padding with VSk2 idle-patterns.

Note that during data transmission time, only VPPM symbols between VS1 and VS9 can carry data information, as shown in Table 188. This is because VS0 (light full off) and VS10 (light full on) cannot carry

| Copyright © 20xx IEEE. All rights reserved. | 293 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- data information because there are no transitions during these two symbols. Therefore, when a macDim

- value less than 100 is required, data information is carried only by VS1 symbols. Similarly, data information

- is carried only by VS9 symbols when a macDim value greater than 900 is required. All dimming requests

- must be honored even if data transmission is not possible. It is recommended that the receiver changes its

- matched filter in step with the change in the transmitter-symbol shape in order to enable optimum detection.

- By default, a 50% duty cycle shall be used for VPPM. If dimming is supported using VPPM modulation, a

- dimming notification command shall be sent by the MAC. Both the TX and RX shall use the above

- algorithm for VPPM dimming. The transmitter shall honor all dimming requests from the upper layer. It is

- recommended that the transmitter uses the receiver’s capability information as provided in 5.4.26.1.1 for

- VPPM dimming support. This information is obtained during the device discovery process described in

- 5.1.3.4.

13

14	8.5.2.4 PHY VII dimming

15

- Two dimming methods are envisioned in this PHY specification. The reverse polarity optical OFDM method

- is described in subclause 6.3.8 (see previous section of this document). The second dimming method is ana-

- log dimming which comprises adjusting the bias level of the analog signal modulating the light source, i.e.,

- adjusting the operating point of the emitter. The level is set using the service primitives in subclause 6.7

- which interface the PHY layer and the MAC layer.

21

22	8.5.2.4.1 Reversed Polarity Optical OFDM for PHY Layer Dimming

23

- Reverse polarity optical OFDM (RPO-OFDM) is defined as an optional feature in this specification to facil-

- itate dimming capabilities. This modulation scheme is expected to work in conjunction with eU-OFDM, but

- it can also be realized using DCO-OFDM. The eU-OFDM specification should be used for the generation of

- a unipolar signal, which would be indicated with the 'eU' bit defined in subclause 6.2.3.2. The current sub-

- clause provides the means for generating a RPO-OFDM signal using the DCO-OFDM waveform or the eU-

- OFDM waveform generated using the eU-OFDM specification. Note that RPO-OFDM modulation does not

- prevent the use of SC-OFDMA precoding and/or adaptive bit loading and/or MIMO encoding.

31

- The RPO-OFDM modulation incorporates dimming while maintaining the average power per time-domain

- OFDM symbol and eliminating energy-intensive and adaptive DC component that carries no additional

- information. Accordingly, a constant signal-to-noise ratio (SNR) for a wide dimming range is achieved, the

- full active operational range of the device is utilized and high energy efficiency is realized. Here, the time-

- domain samples polarity of individual OFDM symbols is properly set to generate an OFDM waveform that

- has similar characteristics of a pulse-width modulation (PWM) signal in controlling the dimming percent-

- age. Such OFDM waveform also has two periods equivalent to the "on-time" and "off-time" periods of a

- PWM signal. Over an equivalent PWM period, the average forward current through the device is equivalent

- to the target dimming percentage. Assuming a 1Amp maximum forward current through the LED, the RPO-

- OFDM signal for two different dimming ratios of 20% and 70% duty cycle are shown in Fig. 6.3.8.1.

42

43

44

45

46

47

48

49

50

51

52

53

54

294	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 160—RPO-OFDM signal

8.5.3 Flicker mitigation

Flicker mitigation can be divided into intra-frame mitigation and interframe flicker mitigation as described in 4.4.3.2.

Intra-frame flicker mitigation refers to mitigation flicker within the transmission of a data frame. Intra-frame flicker in OOK is avoided by the use of the dimmed OOK mode as described in 8.5.2.2, and RLL coding as described in 8.2. VPPM inherently does not cause any interframe flicker and also uses a RLL code. Interframe flicker is avoided in CSK by ensuring constant average power across multiple light sources along with scrambling and the high optical clock rates (MHz) at which this modulation is used.

Interframe flicker mitigation applies to both data transmission (RX mode) and idle periods. While idling, visibility patterns or idle patterns as described in 8.5.1 may be used to ensure light emission by the OWC transmitters have the same average brightness over adjacent MFTPs as during data transmission. These patterns can be modulated in-band or out-of-band as in 8.5.1.1.

When the dimmer setting is changed above the MAC sublayer, the MAC and PHY layers adjust the data transmission and idle time transmission to adjust to the new dimmer settings. A summary of the different mitigation techniques for interframe and intra-frame flicker is provided in Table 114.

8.6 PPDU format

For convenience, the PPDU frame structure is presented so that the left most field as written in this standard shall be transmitted or received first. All multiple octet fields shall be transmitted or received least significant octet first and each octet shall be transmitted or received least significant bit (LSB) first. The

| Copyright © 20xx IEEE. All rights reserved. | 295 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

|  | IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC |  |  |  | IEEE Draft P802.15.13/D0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | Table 114—Flicker mitigation for various modulation modes |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  | Data transmission | Idle or RX periods |  |  |
| 4 |  | Flicker mitigation |  |  |  |  |  |
|  |  |  |  | (Intra-frame flicker) | (Interframe flicker) |  |  |
| 5 |  |  |  |  |  |  |  |
| 6 |  | OOK modulation |  | Dimmed OOK mode, |  |  |  |
| 7 |  |  |  | RLL code |  |  |  |
| 8 |  |  |  |  |  |  |  |
|  |  |  |  | VPPM guarantees no intra-frame flicker, |  |  |  |
| 9 |  | VPPM modulation |  |  | Idle/visibility patterns |  |  |
|  |  |  |  | RLL code |  |  |  |
| 10 |  |  |  |  |  |  |  |
| 11 |  | CSK modulation |  | Constant average power across multiple light sources, |  |  |  |
| 12 |  |  |  | scrambler, high optical clock rates (MHz) |  |  |  |
| 13 |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |

- same transmission order should apply to data fields transferred between the PHY layer and MAC sublayer.

- The PPDU frame structure shall be formatted as illustrated in Figure 161.

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

| Preamble | PHY | HCS | Optional | PSDU |  |
| --- | --- | --- | --- | --- | --- |
| (see 8.6.1) | header |  | fields | (see 8.6.5) |  |
|  |  | (see 8.6.3) |  |  |  |
|  | (see 8.6.2) |  | (see 8.6.4) |  |  |
| SHR |  | PHR |  | PHY payload |  |

Figure 161—Format of the PPDU

8.6.1 Preamble field

The preamble field is used by the transceiver to obtain optical clock synchronization with an incoming message. The standard defines one fast locking pattern (FLP) followed by choice of four topology

- dependent patterns (TDPs) for the purposes of distinguishing different PHY topologies. The MAC shall

- select the optical clock rate for communication during the clock-rate selection process as defined in 6.5. The

- preamble shall be sent at a clock rate chosen by the TX and supported by the RX. The preamble is a time

- domain sequence and does not have any channel coding or line coding.

36

- The preamble first starts with a FLP of at least 64 alternate ones and zeros. The FLP is fixed to start as a

- “1010…” pattern i.e., it ends with a ‘0’. This maximum transition sequence is used to lock the CDR circuit.

- The fast locking pattern length shall not exceed the maximum as shown in Figure 162. After the fast locking

- pattern, four repetitions of one of four TDPs (defined in Figure 163) shall be sent. The TDP shall be 15 bits

- in length and the TDP shall be inverted every other repetition to provide DC balance.

42

43

44

45

46

47

48

49

50

51

52

53

54

64 to 16,384 bits60 bits

| FLP (101010 …) | TDP | ~TDP | TDP | ~TDP |
| --- | --- | --- | --- | --- |

Figure 162—Preamble transmission

296	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

P1 : 1 1 1 1 0 1 0 1 1 0 0 1 0 0 0

P2 : 0 0 1 0 1 1 1 0 1 1 1 1 1 1 0

P3 : 1 0 0 1 1 0 0 0 0 0 1 0 0 1 1

P4 : 0 1 0 0 0 0 1 1 0 1 0 0 1 0 1

Figure 163—TDPs for various topologies

The preambles shall be transmitted using an OOK modulation. The preamble field for single data mode and packed data mode shall be formatted as illustrated in Figure 162. For PHY III, all the three light sources shall transmit the same preamble pattern simultaneously in the supported frequency bands within the error tolerance specified in 8.3.3.

For the burst mode transmission, the FLP shall be included only for the first frame. Subsequent frames shall not include the FLP in the burst mode since the receiver is already synchronized to the transmitter. This reduces the preamble length by at least half and provides higher throughput at the MAC layer. The preamble field for burst data mode shall be formatted as illustrated in Figure 164.

Figure 164—Burst preamble transmission

The TDP used for a specific a topology is defined in Table 115. The topologies are given in 4.2.

Table 115—TDP assignments for various topologies

| TDP | Topology |
| --- | --- |
| P1 | Topology independent (visibility) |
| P2 | Peer-to-peer |
| P3 | Star |
| P4 | Broadcast |

The same preamble sequences shall be used for all PHY types. The number of repetitions of the FLP can be extended by the MAC during idle time or for different operating modes for better synchronization or to provide visibility or image array receiver based device discovery.

8.6.2 PHY header

The PHY header, as shown in Table 116, shall be transmitted with an OOK modulation. For PHY III, all light sources shall transmit the same header contents simultaneously within the error tolerance specified in 8.3.3 and the band plan ID field shall be set to be the band plan ID of the lowest wavelength. The MAC shall

| Copyright © 20xx IEEE. All rights reserved. | 297 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- select the optical clock rate for communication during the clock-rate selection process, as defined in 6.5. The

- PHY header shall be sent at the lowest data rate for the chosen optical clock rate. The clock rate does not

- change throughout the frame between the preamble, header, and payload. If the dimmed OOK extension bit

- is set in the PHY header for dimming support, additional fields are transmitted after the PHY header as

- shown in Table 117.

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Table 116—PHY header

| PHY header fields | Bit-width | Explanation on usage |  |
| --- | --- | --- | --- |
| Burst mode | 1 | Reduce preamble and IFS |  |
| Channel number | 3 | Band plan ID |  |
| MCS ID | 6 | Provide information about PHY |  |
|  |  | type and data rate |  |
| PSDU length | 16 | Length up to |  |
|  |  | aMaxPHYFrameSize |  |
|  |  | Information on compensation |  |
| Dimmed OOK extension | 1 | time, resync, and length of sub- |  |
|  |  | frame |  |
| Reserved fields | 5 | Future use |  |

Table 117—Dimmed OOK extension

| Extension fields | Bit-width | Explanation on usage |  |
| --- | --- | --- | --- |
| Compensation length | 10 | Compensation length in optical |  |
|  |  | clocks |  |
| Resync length | 4 | Number of resync optical clocks |  |
| Subframe length | 10 | Length of subframe in optical |  |
|  |  | clocks |  |
| OFCS | 8 | Optional field check sequence |  |

8.6.2.1 Burst mode

The burst mode bit indicates that the next frame following the current frame is part of the burst mode. Refer to 5.2.2.2 for more detailed information.

8.6.2.2 Channel number

The channel number indicates the code used from Table 111. The channel number field for PHY III shall be the band plan ID of the lowest wavelength. Refer to 8.3.1 for more detailed information.

298	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

8.6.2.3 MCS ID

The modulation and coding scheme (MCS) ID shall be indicated in the PHY header based on Table 118.

Table 118—MCS ID

|  | MCS indication | PHY | Data rate | Unit |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 000000 |  | 11.67 |  |  |
| 1 | 000001 |  | 24.44 |  |  |
| 2 | 000010 |  | 48.89 |  |  |
| 3 | 000011 |  | 73.3 |  |  |
| 4 | 000100 | I | 100 | kb/s |  |
| 5 | 000101 |  | 35.56 |  |  |
| 6 | 000110 |  | 71.11 |  |  |
| 7 | 000111 |  | 124.4 |  |  |
| 8 | 001000 |  | 266.6 |  |  |
| 16 | 010000 |  | 1.25 |  |  |
| 17 | 010001 |  | 2 |  |  |
| 18 | 010010 |  | 2.5 |  |  |
| 19 | 010011 |  | 4 |  |  |
| 20 | 010100 |  | 5 |  |  |
| 21 | 010101 |  | 6 |  |  |
| 22 | 010110 | II | 9.6 | Mb/s |  |
| 23 | 010111 |  | 12 |  |  |
| 24 | 011000 |  | 19.2 |  |  |
| 25 | 011001 |  | 24 |  |  |
| 26 | 011010 |  | 38.4 |  |  |
| 27 | 011011 |  | 48 |  |  |
| 28 | 011100 |  | 76.8 |  |  |
| 29 | 011101 |  | 96 |  |  |

| Copyright © 20xx IEEE. All rights reserved. | 299 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

Table 118—MCS ID (continued)

|  | MCS indication | PHY | Data rate | Unit |
| --- | --- | --- | --- | --- |
| 32 | 100000 |  | 12 |  |
| 33 | 100001 |  | 18 |  |
| 34 | 100010 |  | 24 |  |
| 35 | 100011 | III | 36 | Mb/s |
| 36 | 100100 |  | 48 |  |
| 37 | 100101 |  | 72 |  |
| 38 | 100110 |  | 96 |  |
|  | others |  | reserved |  |

8.6.2.4 Length of PSDU field

The PSDU length field specifies the total number of octets contained in the PSDU. It is a value between 0 and aMaxPHYFrameSize as shown in 9.5.1.

8.6.2.5 Dimmed OOK extension

The dimmed OOK bit shall be set to one when supporting dimming while using OOK modulation. The dimmed OOK bit shall be set when the MAC PIB attribute, macUseDimmedOOKmode, as defined in Table 91, indicates the dimmed OOK mode usage. The dimmed OOK extension bit indicates that more optional fields are present at the end of the header. These fields are described in 8.6.4.2, 8.6.4.3, 8.6.4.4, and 8.6.4.5.

8.6.3 Header check sequence (HCS)

The PHY header shall be protected with a 2 octet CRC-16 HCS. A schematic of the CRC processing used for HCS calculation is shown in Annex C. The HCS bits shall be processed in the transmit order. The registers shall be initialized to all ones.

8.6.4 Optional fields

| Tail bits |  | Compensation | Resync | Subframe | OFCS |  | Channel |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (see 8.6.4.1) |  | length | length | length | (see 8.6.4.5) |  | estimation |
|  |  | (see 8.6.4.2) | (see 8.6.4.3) | (see 8.6.4.4) |  |  | sequence |
|  |  |  |  |  |  |  | (see 8.6.4.6) |
| (a) |  |  | (b) |  |  |  | (c) |

Figure 165—PPDU option fields

- The optional fields shall be formatted as shown in Figure 165. The optional fields in Figure 165 (a) shall be

- transmitted only when PHY I is used with an optical clock of 200 kHz based on the MCS ID chosen in the

- PHR. The optional fields in Figure 165 (b) shall be transmitted after the tail bits only if the dimmed OOK bit

- is set in the PHR. The optional field in Figure 165 (c) shall be transmitted only if PHY III is selected based

300	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| on the MCS ID chosen in the PHR. The dimmed OOK mode shall not be used with PHY III. i.e., the |  | 1 |  |
| --- | --- | --- | --- |
| optional fields (b) and (c) shall never be used simultaneously. Optional fields (a) and (c) shall also never be |  | 2 |  |
| transmitted simultaneously since they correspond to different PHY types. |  | 3 |  |
| 8.6.4.1 Tail bits |  | 4 |  |
|  |  | 5 |  |
| Six tail bits of zeros shall be added after the HCS when PHY I is used with an optical clock rate of 200 kHz. |  | 6 |  |
|  |  | 7 |  |
| 8.6.4.2 Compensation length |  | 8 |  |
|  |  | 9 |  |
| The compensation length has a 10-bit value, which indicates the number of compensation symbols at the |  | 10 |  |
|  |  | 11 |  |
| optical clock rate. The values of these compensation symbols are user defined. When used, this field shall be |  | 12 |  |
| set to a value between 0 to 1023. |  | 13 |  |
| 8.6.4.3 Resync length |  | 14 |  |
|  |  | 15 |  |
| The resync length has a 4-bit value, which indicates the number of resync symbols at the optical clock rate. |  | 16 |  |
|  |  | 17 |  |
| The resync pattern used is the same as the FLP. When used, this field shall be set to a value from 0 to 15, |  | 18 |  |
| with a default value of 15. |  | 19 |  |
| 8.6.4.4 Subframe length and generation |  | 20 |  |
|  |  | 21 |  |
| The subframe length has a 10-bit value, which indicates the number of uncoded data bits in the subframe. |  | 22 |  |
|  |  | 23 |  |
| When used, this field shall be set to a value of 0 to 1023. The subframes shall be generated at the transmitter |  | 24 |  |
| after the FCS has be determined and the FEC has been applied. The FEC and FCS shall not include the |  | 25 |  |
| compensation symbols and the resync symbols. All subframes shall have the same length except for the last |  | 26 |  |
| subframe, which may be truncated to meet the frame length. |  | 27 |  |
| 8.6.4.5 Optional field check sequence generation |  | 28 |  |
|  |  | 29 |  |
| The PPDU optional field check sequence (OFCS) value is calculated across the compensation length, resync |  | 30 |  |
|  |  | 31 |  |
| length and subframe length fields (as shown in Figure 165a) and inserted into the OFCS field. |  | 32 |  |
| The OFCS field shall be an 8-bit sequence (ITU-T I.432.1). It shall be the remainder of the division (modulo |  | 33 |  |
|  |  | 34 |  |
| 2) by the generator polynomial x8 + x2 + x + 1 of the product x8 multiplied by the content of the header |  | 35 |  |
| excluding the OFCS field. |  | 36 |  |
|  |  | 37 |  |
| The initial content of the register of the device computing the remainder of the division is preset to all ones |  | 38 |  |
| and is then modified by division of the header, excluding the OFCS field, by the generator polynomial. The |  | 39 |  |
| resulting remainder is the 8-bit OFCS. |  | 40 |  |
| 8.6.4.6 Channel estimation sequence |  | 41 |  |
|  |  | 42 |  |
|  |  | 43 |  |
| The channel estimation sequences are three optional 8-bit sequences and are used only for PHY III |  | 44 |  |
| operation. The information about PHY III is obtained after decoding the PHY header. The channel |  | 45 |  |
| estimation sequence details are discussed in 12.9. |  | 46 |  |
| 8.6.5 PSDU field |  | 47 |  |
|  |  | 48 |  |
|  |  | 49 |  |
| The PSDU field has a variable length and carries the data of the PHY frame. The FCS is appended if the |  | 50 |  |
| PSDU has a non-zero byte payload. Six tail bits of zeros are attached to end of the PSDU, if PHY I is used |  | 51 |  |
| with data rates of 11.67 kb/s, 24.44 kb/s, or 48.89 kb/s. The structure of the PSDU field is as shown in |  | 52 |  |
| Figure 166. |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 301 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 166—PSDU field structure

The complete PPDU format for the PHY is shown in Figure 167.

Figure 167—PPDU structure

8.7 PPDU Format

editor note: Clause 8.7 is proposed by for OFDM based PHYs. Clause 8.6 is original 15.7-2011. These two clauses should be merged

8.7.1 PPDU format for low bandwidth PHY

The fields of the presented PHY frame are specified in terms of the functionality they are expected to pro-vide. The different parts of the PHY frame structure in Figure 168. are envisioned to occupy different

302	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

OFDM frames in order to achieve a modular PHY frame design, which can be augmented easily based on the functionality which the specific PHY implementation supports.

Figure 168—PHY frame structure

Figure 169—Timing parameters for the PHY control information (specification for 20 MHz)

8.7.1.1 Preamble

The function of the preamble is to enable the PHY to identify the existence of a packet and its beginning. The preamble sequence can also be used for signal detection and automatic gain control (AGC). It is speci-fied as the following 160-sample time domain sequence (having the duration equivalent of two OFDM frames). The sequence has been normalized by the factor 1/sqrt(66) in order to ensure an average energy level of 1 in the information signal. Four possible preamble sequences have been defined, where different nodes communicating to each other can adopt different sequences. The sequences have very low cross-cor-relation to each other, which eliminates the possibility for false packet detection between interfering nodes and enabling interference estimation, which in turn, enables interference management techniques at the higher MAC layer.

S1160 = {0 0 2 -7 -8 4 9 15 9 -4 -1 -9 -9 4 9 15 9 -4 -2 -10 -8 4 10 14 8 -4 -6 5 6 -4 -10 -15 -8 4 6 -5 -6 4 9 15 9 -4 -1 -9 -9 4 8 15 9 -4 -1 -9 -9 4 9 14 8 -5 -6 4 6 -4 -9 -15 -9 3 1 9 8 -4 -9 -15 -9 3 1 9 8 -4 -9 -15 -9 4 2 9 8

| Copyright © 20xx IEEE. All rights reserved. | 303 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- -5 -8 -15 -9 4 1 9 8 -4 -10 -14 -8 4 6 -5 -6 4 8 15 9 -4 -2 -10 -9 4 10 14 8 -4 -7 5 6 -4 -10 -15 -8 4 6 -5 -6 4 10

- 14 8 -4 -6 4 6 -5 -9 -15 -9 3 1 9 8 -5 -9 -14 -9 4 6 -5 -6 4 10 14 8 -5 -6 5 6 -4 -10 -15}/sqrt(66)

3

- S2160 = {0 0 -2 7 7 -4 -10 -15 -9 4 6 -5 -6 4 10 14 8 -4 -6 4 6 -4 -9 -15 -9 4 1 9 8 -5 -10 -15 -8 5 6 -5 -6 4 10

- 14 8 -4 -7 4 6 -4 -10 -15 -8 4 6 -5 -6 4 8 15 9 -3 -2 -9 -9 4 10 14 8 -5 -6 5 6 -4 -9 -15 -9 4 1 9 8 -5 -9 -15 -9 4

- 1 9 8 -5 -8 -15 -9 3 1 9 8 -4 -9 -15 -9 3 2 9 9 -4 -10 -14 -8 4 6 -5 -6 4 9 15 9 -4 -2 -9 -8 4 9 15 9 -4 -2 -10 -8 5

- 9 15 8 -4 -7 5 6 -4 -10 -15 -8 4 6 -5 -6 4 9 15 9 -3 -2 -9 -9 4 9 15 9 -4 -2 -10 -8 4 9 15}/sqrt(66)

8

- S3160 = {0 0 -2 7 7 -5 -9 -15 -10 4 1 9 8 -5 -9 -15 -9 4 1 8 8 -4 -8 -15 -9 4 1 9 8 -4 -10 -14 -8 4 6 -5 -6 4 9 14

- 8 -5 -6 5 6 -4 -10 -15 -9 4 6 -5 -7 4 10 15 8 -5 -7 5 6 -4 -10 -15 -8 4 6 -5 -7 4 8 15 10 -4 -1 -9 -8 4 8 15 9 -3 -

- 1 -9 -9 4 9 15 9 -3 -2 -9 -8 4 9 14 8 -4 -6 4 6 -4 -9 -15 -9 3 1 9 8 -5 -9 -15 -8 4 6 -4 -6 4 9 15 10 -4 -1 -10 -8 5

- 10 14 8 -4 -6 4 6 -5 -10 -15 -8 4 6 -5 -7 4 9 15 9 -4 -1 -9 -8 4 10 14 8 -5 -6 5 6 -4 -10 -15}/sqrt(66)

13

- S4160 = {0 0 -2 7 7 -4 -9 -15 -8 4 6 -5 -6 4 9 15 9 -4 -2 -10 -9 4 10 15 8 -4 -6 4 6 -4 -10 -14 -8 4 6 -5 -6 4 9 15

- 9 -4 -2 -10 -9 4 9 14 8 -4 -6 4 6 -5 -9 -15 -9 4 1 9 8 -4 -9 -14 -8 4 6 -5 -6 4 9 15 9 -3 -2 -9 -8 4 9 15 9 -4 -2 -9

- -9 4 9 15 9 -4 -2 -9 -9 4 9 14 8 -5 -6 4 6 -4 -10 -14 -8 4 6 -5 -6 4 10 14 8 -4 -7 5 6 -4 -10 -15 -9 4 6 -5 -6 4 9

- 14 7 -4 -6 5 6 -4 -9 -15 -9 3 1 9 8 -5 -8 -15 -9 4 1 9 8 -5 -9 -15 -9 3 1 9 8 -5 -10 -15}/sqrt(66)

18

- The time-domain cross-correlation of the specified preamble delivers an easily-identifiable sharp peak value

- which is expected to be used for packet detection and synchronization.

21

22	8.7.1.2 Channel Estimation

23

- A sequence of two identical OFDM training symbols is used to estimate the channel impulse response. The

- sequence can also be used for additional fine-timing synchronization. The channel estimation sequence con-

- tains the following values modulated on the subcarriers of two identical OFDM frames (index 0 corresponds

- to the DC subcarrier modulation value):

28

- E0 to 63 = {0 0 0 -1 -1 1 -1 1 -1 -1 1 1 1 -1 -1 -1 1 -1 1 1 1 1 1 -1 -1 1 1 -1 1 0 0 0 0 0 0 0 1 -1 1 1 -1 -1 1 1 1

- 1 1 -1 1 -1 -1 -1 1 1 1 -1 -1 1 -1 1 -1 -1 0 0}

31

- The sequence is Hermitian symmetric in order to satisfy the requirement for a real time-domain signal after

- the IDFT and has very good auto-correlation properties. In Figure 169, the channel estimation OFDM sym-

- bol is transmitted twice in two identical copies of the time-equivalent signal of the frequency-domain modu-

- lation sequence E0 to 63. The time-domain signal is obtained after an IDFT operation on E0 to 63. The guard

- interval GI2 is a cyclic extension of this same time-domain signal and has a duration of 32 samples (twice

- the length of the typical cyclic extension for this PHY mode specification).

38

39	8.7.1.3 Header

40

- The Header contains all information necessary for demodulating the subsequent frame payload. It is always

- encoded in 1/2 FEC rate BPSK modulation using DCO-OFDM. The header is logically split into three sec-

- tions, which are encoded in separate OFDM frames (each part may be encoded in more than one OFDM

- frame) in order to achieve a modular PHY frame design.

| 45 |  |  |
| --- | --- | --- |
| 46 | The two components of the header in the current PHY take the following form. |  |
| 47 | 8.7.1.3.1 Basic Header |  |
| 48 |  |  |
| 49 |  |  |

- The basic header contains the minimum information required for demodulating the subsequent payload. In

- the low-bandwidth PHY mode, this header includes information such as the constellation size, the FEC rate

- and the payload size. The Basic Header also indicates whether an Advanced Modulation Header (containing

- further information regarding the modulation format) is used. If no advanced modulation header is used,

- then the DATA portion of the PHY frame is expected to appear after the basic header. The basic header con-

304	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

tains 24 bits and fits within 2 OFDM frames of the current PHY mode. The field contains the information and is structured as depicted in Figure 170.

Figure 170—Structure of the Basic Header

The individual fields of the basic header are described as follows.

8.7.1.3.2 RATE

The RATE field consists of three bits and indicates the QAM constellation size and the FEC rate (achieved with the use of a convolutional encoder and puncturing) used for the subsequent payload. The values speci-fied in Table 119 are valid for the RATE field.

Table 119— Valid RATE values

|  |  |  | Data rate | Data rate | Data rate | Data rate | Data rate |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R0-R2 | Modulation | FEC rate | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) |  |
|  |  |  | (40 MHz) | (20 MHz) | (15 MHz) | (10 MHz) | (5 MHz) |  |
| 110 | BPSK | 1/2 | 12 | 6 | 4.5 | 3 | 1.5 |  |
| 111 |  | 3/4 | 18 | 9 | 6.75 | 4.5 | 2.25 |  |
| 010 | 4-QAM | 1/2 | 24 | 12 | 9 | 6 | 3 |  |
| 011 |  | 3/4 | 36 | 18 | 13.5 | 9 | 4.5 |  |
| 100 | 16-QAM | 1/2 | 48 | 24 | 18 | 12 | 6 |  |
| 111 |  | 3/4 | 72 | 36 | 27 | 18 | 9 |  |
| 010 | 64-QAM | 2/3 | 96 | 48 | 36 | 24 | 12 |  |
| 011 |  | 3/4 | 108 | 54 | 40.5 | 27 | 13.5 |  |

8.7.1.3.3 Reserved bits (R)

One bit is reserved for introducing additional transmission rates in future modifications of the standard.

8.7.1.3.4 LENGTH

An 11-bit field which indicates the size of the payload in octets. Hence, the size of the payload is between 0 and 2048 octets.

| Copyright © 20xx IEEE. All rights reserved. | 305 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 8.7.1.3.5 Advanced Modulation Header Bit (A) |  |
| --- | --- | --- |
| 2 | One bit indicating whether an Advanced Modulation Header is included in the next OFDM frame: |  |
| 3 |  |  |
| 4 | 1  The next OFDM frame is an Advanced Modulation Header. |  |
| 5 |  |  |
| 6 | 0  The next OFDM frame is part of the DATA portion (payload) of the PHY frame. |  |
| 7 |  |  |
| 8 | 8.7.1.3.6 High-reliability MAC Header Bit (M) |  |
| 9 |  |  |
| 10 |  |  |

- One bit indicating whether a high-reliability MAC header encoding is used. If the advanced modulation

- header is not enabled, then the next OFDM frame encodes the high-reliability MAC header described in sub-

- clause 6.2.5.1.

14

- 1  The OFDM frame following the advanced modulation header is part of the high-reliability MAC

- header encoding.

17

- 0  The OFDM frame following the advanced modulation header is part of the conventional DATA por-

- tion (payload) of the PHY frame encoding using the RATE specified in subclause

| 20 | 8.7.1.3.7 Parity Check Bit (P) |  |
| --- | --- | --- |
| 21 |  |  |
| 22 | An even parity check bit for the information in bits 0 - 16. |  |
| 23 |  |  |
| 24 | 8.7.1.3.8 SIGNAL TAIL |  |
| 25 |  |  |
| 26 |  |  |

- Six bits set to zero complete the Basic Header. These bits re-set the state machine of the convolutional

- encoder used in the current PHY mode.

29

30	8.7.1.3.9 Advanced Modulation Header

31

- The advanced modulation header is encoded in separate OFDM frames from the Basic Header, so that the

- overall structure is modular, i.e., when the advanced modulation header is not required, it can be easily

- excluded. Like the basic header, the advanced modulation header is encoded using 1/2 FEC rate BPSK. The

- advanced modulation header is an optional field which contains the information necessary for demodulating

- the subsequent waveform. It contains information necessary to identify if adaptive bit and energy allocation

- is used. It also contains commands for the PHY necessary for estimating the channel quality indicators

- (CQIs), necessary for enabling adaptive bit and energy loading during the real-time operation of the system.

- The format is presented in Figure 171.

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Figure 171—Advanced Modulation Header format

306	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 8.7.1.3.10 Adaptive Bit Loading Use (B) |  | 1 |  |
| --- | --- | --- | --- |
| One bit indicating whether adaptive bit loading is used to encode the subsequent payload: |  | 2 |  |
|  |  | 3 |  |
| 1  Adaptive bit loading is used. In this case, the subsequent payload is encoded with the loading scheme |  | 4 |  |
|  |  | 5 |  |
| which the receiving station has estimated and suggested to the transmitter using the feedback channels and |  | 6 |  |
| negotiation procedures in the MAC layer. |  | 7 |  |
| 0  Adaptive bit loading is not used. In this case, the subsequent payload is encoded with the default load- |  | 8 |  |
|  |  | 9 |  |
| ing scheme and the rate specified in the Basic Header. |  | 10 |  |
| The low-bandwidth PHY mode is not required to support adaptive bit loading. If a PHY which does not sup- |  | 11 |  |
|  |  | 12 |  |
| port bit loading encounters a packet which has this bit set to 1, it will ignore the packet, as it will no be able |  | 13 |  |
| to demodulate it. |  | 14 |  |
| 8.7.1.3.11 Estimate CQIs (C) |  | 15 |  |
|  |  | 16 |  |
| One bit indicating whether the CQIs should be calculated in the PHY for the current transmission frame. The |  | 17 |  |
|  |  | 18 |  |
| channel estimation symbols preceding the PHY header are used for the estimation of the CQIs if the MIMO |  | 19 |  |
| mode is not enabled. If the MIMO mode is enabled, the MIMO reference symbols described in 6.2.4. |  | 20 |  |
| 1  the CQIs should be estimated |  | 21 |  |
|  |  | 22 |  |
| 0  the CQIs should not be estimated |  | 23 |  |
|  |  | 24 |  |
| Upon estimation of the CQIs, the PHY conveys the results to the MAC using a predefined PHY service |  | 25 |  |
|  |  | 26 |  |
| primitive. The calculation of the bit and power allocation scheme as well as the necessary exchange for |  | 27 |  |
| updating the bit and power allocation scheme at the transmitter and receiver are handled at the MAC layer. |  | 28 |  |
| 8.7.1.3.12 Use of eU-OFDM (eU) |  | 29 |  |
|  |  | 30 |  |
| Indicates whether the PHY DATA (payload) field is encoded using eU-OFDM. |  | 31 |  |
|  |  | 32 |  |
| 1  the DATA (payload) is encoded using eU-OFDM. |  | 33 |  |
|  |  | 34 |  |
| 0  the DATA (payload) is not encoded using eU-OFDM. |  | 35 |  |
|  |  | 36 |  |
| Note that the use of this alternative waveform can be negotiated in advance using control/management |  | 37 |  |
|  |  | 38 |  |
| frames. Furthermore, the use of eU-OFDM encoding does not prohibit the use of any additional advanced |  | 39 |  |
| waveforms such as SC-FDMA or RPO-OFDM. |  | 40 |  |
| 8.7.1.3.13 Number of Streams in eU-OFDM (STR) |  | 41 |  |
|  |  | 42 |  |
| Two bits indicating the number of U-OFDM streams superimposed in the signal encoding procedure. The |  | 43 |  |
|  |  | 44 |  |
| valid values for this field are: |  | 45 |  |
| 8.7.1.3.14 Use of Single-carrier FDMA |  | 46 |  |
|  |  | 47 |  |
| Indicates whether the PHY DATA (payload) field is encoded using SC-FDMA. |  | 48 |  |
|  |  | 49 |  |
| 1  the DATA (payload) is encoded using SC-FDMA. |  | 50 |  |
|  |  | 51 |  |
| 0  the DATA (payload) is not encoded using SC-FDMA. |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 307 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 120— Valid STR values

| STR | Number of Streams |
| --- | --- |
| 00 | 1 |
| 10 | 2 |
| 01 | 3 |
| 11 | 4 |

Note that the use of this alternative waveform can be negotiated in advance using control/management frames. Furthermore, the use of SC-FDMA encoding does not prohibit the use of any additional advanced waveforms such as eU-OFDM or RPO-OFDM.

8.7.1.3.15 SC-FDMA pre-coder DFT Size (DFT)

A number specifiying the DFT size used in the SC-FDMA pre-coding procedure. As only 24 subcarriers are modulated with unique data, the DFT size is a number between 1 and 24.

8.7.1.3.16 Number of Reverse Polarity Optical OFDM frames (NRPO)

Indicates the number of OFDM data frames which are encoded using RPO -OFDM as described in subclause 6.3.8. By convention, the LSB of this number is transmitted first. A value of NRPO='000000' indicates that RPO-OFDM is not used.

Note that the use of this alternative waveform can be negotiated in advance using control/management frames. Furthermore, the use of RPO-OFDM encoding does not prohibit the use of any additional advanced waveforms such as eU-OFDM or SC-FDMA.

8.7.1.3.17 Signal scaling factor in Reverse Polarity Optical OFDM frames (GRPO)

Indicates the factor by which the OFDM data signal is scaled as described in subclause 6.3.8. If NRPO='000000', this field is not relevant for the signal transmission.

8.7.1.3.18 Relaying Operation Enabled (RE)

1  Relaying should be performed for the current PHY frame.

0  Relaying should not be performed for the current PHY frame.

8.7.1.3.19 Relaying Operation Type (RT)

Two bits which specify the type of relaying operation that should be performed. The valid values for this field are specified as:

8.7.1.3.20 MIMO Enable Bit (M)

One bit indicating whether a MIMO mode is enabled for the current PHY frame:

1  MIMO mode is enabled and the subsequent payload is encoded using the MIMO scheme already negotiated by the transmitter and the receiver.

308	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 121— Valid RT values

| RT | Relaying and |  |
| --- | --- | --- |
|  | Duplexing Mode |  |
| 00 | FD-AF |  |
| 01 | FD-DF |  |
| 10 | HD-AF |  |
| 11 | HD-DF |  |

0  MIMO mode is not enabled and the subsequent payload is encoded using the SISO scheme specified by the parameters in the basic header and the advanced modulation header.

8.7.1.3.21 MIMO Reference Symbols Format (RS)

One bit which indicates the format of the reference symbols used for CQI estimation.

1  MIMO Reference Symbols Format I is used.

0  MIMO Reference Symbols Format II is used.

The two MIMO reference symbols formats are specified in 6.2.4.

8.7.1.3.22 6.2.3.2.13. Parity Check Bit (P)

An even parity check bit for the information in bits 0 - 16.

8.7.1.3.23 SIGNAL TAIL

Six bits set to zero complete the advanced modulation header. These bits re-set the state machine of the con-volutional encoder used in the current PHY mode.

8.7.1.3.24 MIMO Reference Symbols (RSs)

Figure 172—High-reliability MAC header structure

The MIMO reference symbols constitute N MIMO OFDM frames (or the time-frame equivalent of NMIMO OFDM frames), where NMIMO = number of MIMO channels. Depending on the reference symbol format in

use, the bit RS is set to '0' for reference symbols format I and set to '1' for reference symbols format II. The symbol formats are described as follows.

| Copyright © 20xx IEEE. All rights reserved. | 309 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 8.7.1.4 Data |
| --- | --- |
| 2 |  |

- The Data field contains the MPDU transferred from the higher layer. The size of this field is specified in the

- basic PHY header. The MPDU is encoded according to the modulation format specified in the PHY header.

- The Data field contains a Service field, a MAC header portion of the MPDU encoded in the most robust

- modulation format, as well as the remaining portion of the MPDU complemented with a TAIL field and pad-

- ding bits, which extend the length of the field to an integer number of OFDM frames.

8

9	8.7.1.4.1 High-reliability MAC Header

10

- Robust transmission of the polling and acknowledgement information ensures avoiding a lot of unnecessary

- retransmissions. Furthermore, when this information is encoded separately from the rest of the payload,

- errors in the payload (especially for long payloads) which cause the packet to be discarded (and retransmit-

- ted) do not influence the polling and acknowledgement mechanism. As a result, the MAC header (the part of

- the header required for polling and acknowledgments) is encoded in a special high-reliability header using

- the lowest data-rate (most robust) modulation format 1/2 FEC rate BPSK separately from the data payload.

- The high-reliability MAC header has the structure described in Fig. 6.2.5.2.1.

18

19	8.7.1.4.2 Protocol Version

20

- The first two bits of the MAC frame indicating the MAC protocol version. If an incompatible MAC frame

- version is encountered in this frame, the PHY will drop the rest of the packet and signal an incompatible

- MAC protocol version to the MAC layer management entity.

24

25	8.7.1.4.3 Polled station number

26

- In the downlink transmission, this field specified the number of the station which is being polled by the

- access point. On the uplink, these bits are left unspecified.

| 29 | 8.7.1.4.4 Packet sequence number |  |
| --- | --- | --- |
| 30 |  |  |
| 31 | A twelve-bit number which specifies the sequence number of the packet that is being acknowledged. |  |
| 32 |  |  |
| 33 | 8.7.1.4.5 Acknowledgement (ACK) |  |
| 34 |  |  |
| 35 |  |  |

- A two-bit field which enables the AP and the stations to acknowledge packet reception as well as beacon

- reception. Bit 30 is set to '1', when a transmitting node (station or access point) is acknowledging the recep-

- tion of the packet with the sequence number specified by the field described in 6.2.5.1.3. Bit 31 is set to '1',

- when a station is acknowledging the reception of a beacon frame, indicating it is still active and connected to

- the AP.

| 41 | 8.7.1.4.6 Reserved bits (R) |  |
| --- | --- | --- |
| 42 |  |  |
| 43 | Nine bits (at positions 32-40) are reserved for future use. Hence, they have not been specified. |  |
| 44 |  |  |
| 45 | 8.7.1.4.7 Parity check bit (P) |  |
| 46 |  |  |
| 47 | An even parity check bit for the information in bits 0 - 40. |  |
| 48 |  |  |
| 49 | 8.7.1.4.8 SIGNAL TAIL |  |
| 50 |  |  |
| 51 |  |  |

- Six bits set to zero complete the high-reliability MAC header. These bits re-set the state machine of the con-

- volutional encoder used in the current PHY mode.

54

310	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

8.7.1.4.9 Service Field

The Service field has 16 bits, which shall be denoted as bits 0-15. The bit 0 shall be transmitted first in time. The bits from 0 -6 of the SERVICE field, which are transmitted first, are set to zeros and are used esti-mate the initial state of the transmitter scrambler and to synchronize the descrambler in the receiver. The remaining 9 bits (7-15) of the SERVICE field shall be reserved for future use. All reserved bits shall be set to 0. Refer to Figure 173.

Figure 173—SERVICE field bit assignment

8.7.1.4.10 General Payload Data

The general payload is encoded according to the modulation format specified in the PHY header. This field contains the MPDU transferred from the higher layer except for the parts of the MAC Header already encoded in the high-reliability MAC header as described in 6.2.5.2. followed by a tail field of six zero bits necessary to reset the convolutional encoder and a four-byte frame check sequence. In case the DATA field does not fit within an exact number of OFDM frames, padding bits are introduced.

8.7.1.4.11 Tail Field

The PPDU TAIL field shall be six zero bits, which are required to return the convolutional encoder to the zero state. This procedure improves the error probability of the convolutional decoder, which relies on future bits when decoding and which may not be available past the end of the message. The PLCP tail bit field shall be produced by replacing six scrambled zero bits following the message end with six nonscrambled zero bits.

8.7.1.4.12 Padding Bits

The number of bits in the DATA field shall be a multiple of N CBPS, the number of coded bits in an OFDM symbol (24, 48, 96, or 144 bits). To achieve this, the length of the message is extended so that it becomes a

multiple of NDBPS, the number of data bits per OFDM symbol. At least 6 bits are appended to the message, in order to accommodate the TAIL bits, as described in 3.5.2. The number of OFDM symbols, NSYM; the

number of bits in the DATA field, NDATA; and the number of pad bits, NPAD, are computed from the length of the PSDU (LENGTH) as follows:

| NSYM  Ceiling16  8  LENGTH  6 NDBPS  | (6) |
| --- | --- |
| NDATA  NSYM  NDBPS | (7) |
| NPAD  NDATA  16  8  LENGTH  6 | (8) |

| Copyright © 20xx IEEE. All rights reserved. | 311 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

Figure 174—Data Scrambler / Descrambler

The function Ceiling (.) is a function that returns the smallest integer value greater than or equal to its argu-ment value. The appended bits ("pad bits") are set to zeros and are subsequently scrambled with the rest of the bits in the DATA field.

8.7.1.4.13 Data scrambler and descrambler

The DATA field, composed of SERVICE, PSDU, tail, and pad parts, shall be scrambled with a length -127 frame-synchronous scrambler. The octets of the PSDU are placed in the transmit serial bit stream, bit 0 first and bit 7 last. The frame synchronous scrambler uses the generator polynomial S(x) as follows, and is illus-trated in Figure 174:

| S x   x7  x4  1 | (9) |  |
| --- | --- | --- |

- The 127-bit sequence generated repeatedly by the scrambler shall be (leftmost used first), 00001110

- 11110010 11001001 00000010 00100110 00101110 10110110 00001100 11010100 11100111 10110100

- 00101010 11111010 01010001 10111000 1111111, when the all ones initial state is used. The same scram-

- bler is used to scramble transmit data and to descramble receive data. When transmitting, the initial state of

- the scrambler will be set to a pseudo-random nonzero state. The seven LSBs of the SERVICE field will be

- set to all zeros prior to scrambling to enable estimation of the initial state of the scrambler in the receiver.?

| 35 | 8.7.2 PPDU Format for high bandwidth PHY |
| --- | --- |
| 36 |  |
| 37 | 8.7.2.1 Frame structure |
| 38 |  |

- The data signal, denoted as payload in Figure 175, is transmitted together with additional signals in a com-

- pound frame, so that the payload can be decoded frame-wise. The preamble is followed by a PHY header.

- The preamble allows coarse time synchronization in its first two parts, followed by a reference sequence

- used for fine timing as well as channel estimation, what enables the receiver to decode the header informa-

- tion. After the header, eventually there are additional channel estimation (ACE) symbols, basically needed

- for the support of multiple-input multiple-output (MIMO), before the payload is then transmitted. In case of

- using MIMO in 802.15.13, all transmitters transmit the same preamble and header information, while only

- the payload makes explicit use of MIMO. The ACE symbols contain an orthogonal sequence of symbols

- (same symbols but with alternate signs) in order to estimate the MIMO channel.

48

49

50

51

52

53

54

312	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Preamble |  | Channel |  |  | Header | Additional channel | Payload |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | estimation symbol |  |  |  | 1, 2 or 4 symbols | estimation symbols |  |  |

Figure 175—P2P PHY frame, taken over from G.hn specification

8.7.2.2 Frame types

By using the same frame structure, the PHY can transport different frame types. An overview is provided in Table 122.

editor’s note: frame types in 802.15.7 and 802.15.4 are on MAC level. Different types of MAC frame types are specified in 5.4. Please consider how to harmonize.

Table 122—PHY frame types

| Frame | Frame |  |  |  | Duration | Ext. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | type | Description | Header | Payload | indicator | Header |  |
| type |  |  |  |  |  |  |  |
|  | (FT) |  |  |  | (DRI( | indicator |  |
|  |  | A frame carrying the medium |  |  |  |  |  |
| MAP/ |  | access plan (MAP) or relayed | Yes | Yes | 1 | 0 |  |
|  | 0000 | MAP (RMAP) where the |  |  |  |  |  |
| RMAP |  |  |  |  |  |  |  |
|  |  | payload contains a MAC Pro- |  |  |  |  |  |
|  |  | tocol Data Unit (MPDU) |  |  |  |  |  |
|  |  | Data and management frame, |  |  |  |  |  |
| MSG | 0001 | carrying user data or manage- | Yes | Yes | 1 | 0 |  |
|  |  | ment data or both; the pay- |  |  |  |  |  |
|  |  | load contains an MPDU |  |  |  |  |  |
|  |  | Acknowledge (ACK) control |  |  |  |  |  |
| ACK | 0010 | frame, the relevant ARQ data | Yes | No | 1 | 0 or 1 |  |
|  |  | is communicated in the |  |  |  |  |  |
|  |  | header |  |  |  |  |  |
|  |  | A request-to-send control |  |  |  |  |  |
| RTS | 0011 | frame; the relevant data is | Yes | No | 1 | 0 |  |
|  |  | communicated in the header |  |  |  |  |  |
|  |  | A clear-to-send control |  |  |  |  |  |
| CTS | 0100 | frame; the relevant data is | Yes | No | 1 | 0 |  |
|  |  | communicated in the header |  |  |  |  |  |
| CTMG | 0101 | Sort control frame, carrying a | Yes | No | 1 | 0 or 1 |  |
|  |  | short control message |  |  |  |  |  |
| PROBE | 0110 | A frame carrying probe sym- | Yes | Yes | 1 | 0 |  |
|  |  | bols in its payload |  |  |  |  |  |

| Copyright © 20xx IEEE. All rights reserved. | 313 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  |  | Table 122—PHY frame types |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 |  | Frame | Frame |  |  |  |  | Duration | Ext. |  |
| 4 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | type | Description | Header |  | Payload | indicator | Header |  |
| 5 |  | type |  |  |  |  |  |  |  |  |
|  |  |  | (FT) |  |  |  |  | (DRI( | indicator |  |
| 6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | An ACK retransmission |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |
| 8 |  | ACKRQ | 0111 | request frame; the relevant | Yes |  | No | 0 | 0 |  |
|  |  |  |  | data is communicated in the |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | header |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | A bidirectional MSG frame, |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |  |
| 12 |  | BMSG | 1000 | it contains data and manage- | Yes |  | Yes | 1 | 0 or 1 |  |
|  |  |  |  | ment frames in the payload |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | and ACK |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | A bidirectional ACK frame, |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |
| 16 |  | BACK | 1001 | contains ACK and data and | Yes |  | Yes | 1 | 0 or 1 |  |
|  |  |  |  | management frames in the |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | payload |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | An acknowledge frame for a |  |  |  |  |  |  |
| 19 |  | ACTMG | 1010 |  | Yes |  | No | 0 | 0 |  |
| 20 |  |  |  | control message (CTMG) |  |  |  |  |  |  |
|  |  |  |  | frame |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |  |  |  |
| 22 |  | Reserved | 1011 to | Reserved frame types |  |  |  |  |  |  |
| 23 |  |  | 1110 |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Frame Type Extension, |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |  |  |  |
|  |  | FTE | 1111 | pointer to a set of additional | Yes |  | No | 0 or 1 | 0 or 1 |  |
| 26 |  |  |  | frame types |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |  |  |  |
| 28 | 8.7.2.3 Preamble |  |  |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |  |  |  |

- The preamble is prepended to a PHY frame and is intended for frame detection/synchronization, initial

- channel estimation and OFDM symbol alignment. It may be used for recovery of the following header seg-

- ment of the PHY frame.

34

- The preamble is divided into three sections; each consists of a repetition of Nn OFDM symbols Sn, where the

- subscript n=1,2,3 denotes the section. Each Sn is output by the IFFT. The inputs of the IFFT, i.e. complex

- valued symbols, are generated by feeding into the constellation scrambler described in 2.2.2.7.2 a constant

- series of 1s. The three sections are windowed and combined additively as outlines in Figure 176.

39

40

41

42

43

44

45	Figure 176—Preamble structure

46

47

- In the default case, every kn-th subcarrier is used, starting from the subcarrier index i=0. For n=3, the used

- subcarrier spacing kn and the LSFR seed differs from the cases in which n=1,2 as described in Table 123.

50

51

52

53

54

314	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 123— Default properties of preamble sub-sequence

| property | Section 1 | Section 2 | Section 3 |
| --- | --- | --- | --- |
| OFDM symbol | S1 | S1=-S1 | S3 |
| Nn | 10 | 4 | 2.5 |
| kn | 4 | 4 | 1 |
| LFSR seed | 16E616 | - | 1105 |

8.7.2.4 Channel estimation

The final OFDM symbol in the preamble is used for channel estimation. It is useful to detect the header information and when using single-input single-output (SISO) transmission, it can also be used for the detection of data. In case MIMO is used additional channel estimation (ACE) symbols are used for channel estimation. These additional symbols are sent after the header, as part of the data block in the frame.

8.7.2.5 PHY frame header

The PHY-frame header always fits into an integer number of OFDM symbols and is transmitted using a sin-gle, predefined set of modulation and coding parameters. i.e. 1 bit/symbol with code rate ½. The core part of the PHY-frame header is 168 bits long. It is composed of a common part and a variable part. The fields of the PHY-frame header are defined in the following table.

Table 124—PHY-frame header definition

| Field | Octet | Bits |  | Description |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Common Port |  |  |
| FT | 0 | [3:0] |  | Frame type |  |
| DOD |  | [7:4] |  | Domain ID |  |
| SID | 1 | [7:0] |  | DEVICE_ID of the source node |  |
| DID | 2 | [7:0] |  | DEVICE_ID, MULTICAST_ID or BROADCAST_ID |  |
|  |  |  |  | of the destination node(s) |  |
| MI |  | [0] |  | Multicast indication identifying whether the DID is a |  |
|  |  |  |  | unicast or multicast destination |  |
| DRI |  | [1] |  | Duration indication identifying whether FTSF starts |  |
|  |  |  |  | with a 16-bit duration field |  |
|  | 3 |  |  |  |  |
| EHI |  | [2] |  | Extended header indication |  |
| HSI |  | [3] |  | Header segmentation indication |  |
| Reserved |  | [7:4] |  | Reserved (zero if not used) |  |

| Copyright © 20xx IEEE. All rights reserved. | 315 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 124—PHY-frame header definition

| Field | Octet | Bits |  | Description |
| --- | --- | --- | --- | --- |
|  |  |  | Variable Port |  |
| FTSF | 4 to 18 | [119:0] |  | Frame-type specific field |
|  |  |  | Common Port |  |
| HCS | 19+20 | [15:0] |  | Header check sequence |

The common part contains fields that are common for all PHY-frame types. The variable frame-type specific field (FTSF) part contains fields according to the PHY-frame type. The content of the core part is protected by the 16-bit header check sequence (HCS).

8.7.2.5.1 Common part fields of the PHY-frame header

The frame type (FT) field is a 4-bit field that indicates the type of PHY frame.

The domain ID (DOD) field contains the domain ID to which the source and destination devices of the PHY frame belong. It is represented as a 4-bit unsigned integer with valid values in the range from 0 to 15. Value 0 is a special value reserved for inter-domain communication.

The destination ID (DID) field identifies the destination node(s) of the PHY frame. It is represented as an 8-bit unsigned integer with valid values in the range from 0 to 250.

If the multicast indication (MI) bit is set to zero, the DID field shall contain the DEVICE_ID of the destina-tion node (for unicast transmission). If the MI bit is set to one, the DID field shall contain a MULTICAST-_ID or BROADCAST_ID of the destination nodes.

If the duration indication (DRI) bit is set to zero, the PHY frame shall not contain any payload (i.e., contains only preamble and PHY-frame header). If the DRI bit is set to one, the FTSF shall start with a duration field. The duration field contains the duration of a single PHY frame or PHY frame sequence. It shall be repre-sented as a 16-bit unsigned integer with valid values in steps of 0.25 us. It shall be the smallest integer larger than or equal to the actual duration.

If the extended header indication (EHI) field is set to zero, the PHY-frame header shall contain 168 informa-tion bits. If it is set to one, the PHY frame header shall contain 2×168 information bits. The EHI field shall be set according to the frame type as shown in Table 124.

The header check sequence (HCS) field is intended for PHY-frame header verification. It is a 16-bit cyclic redundancy check (CRC) and shall be computed over all the fields of the PHY-frame header in the order they are transmitted, starting with the LSB of the first field of the PHY frame header (FT) and ending with the MSB of the last field of the FTSF. The HCS is computed using the following generator polynomial of degree 16

G(x) = x16 + x12 + x5 + 1

The value of the HCS is the remainder after the contents (treated as a polynomial where the first input bit is associated with the highest degree, X168-17, where 168 is the header length in bits, and the last input bit is associated with X0 of the calculation field is multiplied by x16 and then divided by G(x). The HCS field is transmitted starting with the coefficient of the highest order term.

316	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 8.7.2.5.2 Variable part fields of the PHY-frame header |  | 1 |  |
| --- | --- | --- | --- |
| The ITU-T recommendation G.9960-2015 contains a very detailed description of the variable frame-type |  | 2 |  |
|  |  | 3 |  |
| specific field (FTSF) in the PHY-frame header. Here, only the main parameters in each frame type are |  | 4 |  |
| explained. |  | 5 |  |
| The medium access plan (MAP) and relayed MAP (RMAP) PHY frames are used to schedule transmission |  | 6 |  |
|  |  | 7 |  |
| opportunities among the devices in a direct or relayed transmission. The FTSF of MAP and RMAP frames |  | 8 |  |
| contains the duration of the MAP frame, the network time reference, i.e. the start of the first OFDM symbol |  | 9 |  |
| of the preamble with 10 ns resolution. The start time of the next MAC cycle, the size of repetition blocks, |  | 10 |  |
| scrambler initialization value, the information block size of the FEC code word used for the payload, the |  | 11 |  |
| number of repetitions used for encoding the data, the concatenation factor in the FEC, the band plan which |  | 12 |  |
| contains the used bandwidth for this frame. The MAP type indicates what kind of bit allocation table is used |  | 13 |  |
| for the bitloading of the payload. Finally, the type of MAP or RMAP transmission is indicated and the num- |  | 14 |  |
| ber of hops from the master domain. |  | 15 |  |
| The data and management frame (MSG) is used to carry user data or management data or both. The parame- |  | 16 |  |
|  |  | 17 |  |
| ters in the MSG frame are the duration of the MSG frame, the information block size of the payload, code |  | 18 |  |
| rate, number of repetitions, concatenation factor, scrambler initialization, and if a master is detected or not. |  | 19 |  |
| Next there are the identifier of the bit allocation table, either the band plan (for uniform number of bits |  | 20 |  |
| loaded) or the subcarrier grouping used for the bit loading, what cyclic prefix is used, a connection identifier, |  | 21 |  |
| whether or not an acknowledgement reply is required, the number of the PHY frame in a burst frame and the |  | 22 |  |
| burst end flag. An important number for the use of MIMO is the number of additional channel estimation |  | 23 |  |
| symbols (ranging from 0 to 7). Notice also the important connection management field, by which connec- |  | 24 |  |
| tions can be initialized or released, with or without acknowledgement but always without transporting a pay- |  | 25 |  |
| load. Moreover, the MSG frame contains information about the bandwidth reservation, by which the devices |  | 26 |  |
| get informed about the length of the connection queue, requests for bidirectional transmissions and further |  | 27 |  |
| information to control the transmission of the payload. |  | 28 |  |
| The acknowledgement frame (ACK) is used to communicate the relevant ARQ data in the header. It con- |  | 29 |  |
|  |  | 30 |  |
| tains detailed information for the flow control, requests for bidirectional transmissions, a bad burst indicator, |  | 31 |  |
| means for channel estimation control, the identifier for the runtime (i.e. time-variant) bit allocation table and |  | 32 |  |
| a complex field for acknowledgement data depending on whether unicast or multicast is being used. Essen- |  | 33 |  |
| tially, this field indicates to the transmitter what segment of a frame was lost. Obviously, that list is being |  | 34 |  |
| used to control the selective repeat mechanism, which is one of the key features in G.hn to reduce the latency |  | 35 |  |
| and improve the robustness likewise. |  | 36 |  |
| The request-to-send (RTS) control frame contains specific means to establish the exchange of data. It con- |  | 37 |  |
|  |  | 38 |  |
| tains the transmission times of the RTS, CTS and MSG frames, as well as of eventually needed ACK frames |  | 39 |  |
| and their duration, the device ID of the node that should respond to the request. |  | 40 |  |
| The PROBE FTSF is composed of a common part and a variable part. The common part contains fields for |  | 41 |  |
|  |  | 42 |  |
| the duration of the probe frame and its type, containing either silent signals or channel estimation probe |  | 43 |  |
| symbols, the number of probe symbols, the used probe guard interval and an additional probe frame type |  | 44 |  |
| specific field. |  | 45 |  |
| The ACK retransmission request (ACKRQ) frame shall allow the receiver to request retransmission of spe- |  | 46 |  |
|  |  | 47 |  |
| cific receiver windows for either data or management connections or both. Moreover, there are bidirectional |  | 48 |  |
| variants of the bidirectional MSG (BMSG) and ACK (BACK) frames and an opportunity to add additional |  | 49 |  |
| frame types by using the frame type extension (FTE) and an opportunity to add an extended header which is |  | 50 |  |
| similar to the original frame types while adding further functionalities that the original frames can no longer |  | 51 |  |
| host. This information is sent in further OFDM symbols immediately after the original header. |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 317 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

9. PHY s ervice s pecifications

9.1 Overview

The PHY provides an interface between the MAC sublayer and the physical optical channel. The PHY conceptually includes a management entity called the PLME. This entity provides the layer management service interfaces through which layer management functions may be invoked. The PLME is also responsible for maintaining a database of managed objects pertaining to the PHY. This database is referred to as the PHY PAN information base (PIB).

Figure 177 depicts the components and interfaces of the PHY.

Figure 177—PHY layer service access points

The PHY provides two services, accessed through two SAPs: the PHY data service, accessed through the PHY data SAP (PD-SAP), and the PHY management service, accessed through the PLME’s SAP (PLMESAP). The optical SAP provides an interface between the PHY layer and the optical channel and is not specified in this standard. Any required light source drivers are considered to be part of the optical channel.

9.2 PHY management s ervice

- The PLME-SAP allows the transport of management commands between the MLME or the DME and the

- PLME. Table 125 lists the primitives supported by the PLME-SAP. These primitives are discussed in the

- subclauses referenced in Table 125.

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Table 125—PLME-SAP primitives

| PLME-SAP primitive | Request | Confirm |
| --- | --- | --- |
| PLME-CCA | 9.2.1 | 9.2.2 |
| PLME-GET | 9.2.3 | 9.2.4 |
| PLME-SET | 9.2.5 | 9.2.6 |
| PLME-SET-TRX-STATE | 9.2.7 | 9.2.8 |
| PLME-SWITCH | 9.2.9 | 9.2.10 |

318	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

9.2.1 PLME-CCA.request

The PLME-CCA.request primitive requests that the PLME perform a CCA as defined in 8.3.9.

The semantics of the PLME-CCA.request primitive are as follows:

PLME-CCA.request	()

There are no parameters associated with the PLME-CCA.request primitive.

9.2.1.1 When generated

The PLME-CCA.request primitive is generated by the MLME and issued to its PLME whenever the access algorithm requires an assessment of the channel.

9.2.1.2 Effect on receipt

If the receiver is enabled on receipt of the PLME-CCA.request primitive, the PLME will cause the PHY to perform a CCA.

9.2.2 PLME-CCA.confirm

The PLME-CCA.confirm primitive reports the results of a CCA.

The semantics of the PLME-CCA.confirm primitive are as follows:

PLME-CCA.confirm	(

status

)

Table 126 specifies the parameters for the PLME-CCA.confirm primitive.

Table 126—PLME-CCA.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
|  |  | TRX_OFF, |  |
| status | Enumeration | TX_ON, BUSY, | The result of the request to perform a CCA. |
|  |  | IDLE |  |

9.2.2.1 When generated

The PLME-CCA.confirm primitive is generated by the PLME and issued to its MLME in response to a PLME-CCA.request primitive. When the PHY has completed the CCA, the PLME will issue the PLME-CCA.confirm primitive with a status of either BUSY or IDLE, depending on the result of the CCA.

If the PLME- CCA.request primitive is received while the transceiver is disabled (TRX_OFF state) or if the transmitter is enabled (TX_ON state), the PLME will issue the PLME-CCA.confirm primitive with a status of TRX_OFF or TX_ON, respectively.

| Copyright © 20xx IEEE. All rights reserved. | 319 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 9.2.2.2 Effect on receipt |
| --- | --- |
| 2 |  |

- On receipt of the PLME-CCA.confirm primitive, the MLME is notified of the results of the CCA. If the

- CCA attempt was successful, the status parameter is set to either BUSY or IDLE. Otherwise, the status

- parameter will indicate the error.

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

9.2.3 PLME-GET.request

The PLME-GET.request primitive requests information about a given PHY PIB attribute.

The semantics of the PLME-GET.request primitive are as follows:

PLME-GET.request	(

PIBAttribute

)

Table 127 specifies the parameters for the PLME-GET.request primitive.

Table 127—PLME-GET.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |

| PIBAttribute | Enumeration | As defined in | The identifier of the PHY PIB attribute to get. |  |
| --- | --- | --- | --- | --- |
|  |  | Table 144 |  |  |
| 9.2.3.1 Appropriate usage |  |  |  |  |

- The PLME-GET.request primitive is generated by the MLME and issued to its PLME to obtain information

- from the PHY PIB.

32

33	9.2.3.2 Effect on receipt

34

- On receipt of the PLME-GET.request primitive, the PLME will attempt to retrieve the requested PHY PIB

- attribute from its database.

| 37 | 9.2.4 PLME-GET.confirm |  |  |
| --- | --- | --- | --- |
| 38 |  |  |  |
| 39 | The PLME-GET.confirm primitive reports the results of an information request from the PHY PIB. |  |  |
| 40 |  |  |  |
| 41 | The semantics of the PLME-GET.confirm primitive are as follows: |  |  |
| 42 |  |  |  |
| 43 | PLME-GET.confirm | ( |  |
| 44 |  |  |  |
| 45 |  | status, |  |
| 46 |  | PIBAttribute, |  |
| 47 |  | PIBAttributeValue |  |
| 48 |  | ) |  |
| 49 | Table 128 specifies the parameters for the PLME-GET.confirm primitive. |  |  |
| 50 |  |  |  |
| 51 |  |  |  |
| 52 |  |  |  |
| 53 |  |  |  |
| 54 |  |  |  |
|  | 320 | Copyright © 20xx IEEE. All rights reserved. |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 128—PLME-GET.confirm parameters

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
|  |  | SUCCESS, | The result of the request |  |
| Status | Enumeration |  | for PHY PIB attribute |  |
|  |  | UNSUPPORTED_ATTRIBUTE |  |  |
|  |  |  | information. |  |
| PIBAttribute | Enumeration | As defined in Table 144 | The identifier of the PHY |  |
|  |  |  | PIB attribute to get. |  |
| PIBAttributeValue | Various | Attribute specific | The value of the indicated |  |
|  |  |  | PHY PIB attribute to get. |  |

9.2.4.1 When generated

The PLME-GET.confirm primitive is generated by the PLME and issued to its MLME in response to a PLME-GET.request primitive. If the identifier of the PIB attribute is not found in the database, the PLME will issue the PLME-GET.confirm primitive with a status of UNSUPPORTED_ATTRIBUTE.

If the requested PHY PIB attribute is successfully retrieved, the PLME will issue the PLME-GET.confirm primitive with a status of SUCCESS.

9.2.4.2 Effect on receipt

On receipt of the PLME-GET.confirm primitive, the MLME is notified of the results of its request to read a PHY PIB attribute. If the request to read a PHY PIB attribute was successful, the status parameter is set to SUCCESS. Otherwise, the status parameter will indicate the error.

9.2.5 PLME-SET.request

The PLME-SET.request primitive attempts to set the indicated PHY PIB attribute to the given value.

The semantics of the PLME-SET.request primitive are as follows:

PLME-SET.request	(

PIBAttribute,

PIBAttributeValue

)

Table 129 specifies the parameters for the PLME-SET.request primitive.

Table 129—PLME-SET.request parameters

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
| PIBAttribute | Enumeration | As defined in | The identifier of the PIB attribute to set. |  |
|  |  | Table 144 |  |  |
|  |  | Attribute spe- |  |  |
| PIBAttributeValue | Various | cific, as defined | The value of the indicated PIB attribute to set. |  |
|  |  | in Table 144 |  |  |

| Copyright © 20xx IEEE. All rights reserved. | 321 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 9.2.5.1 When generated |
| --- | --- |
| 2 |  |

- The PLME-SET.request primitive is generated by the MLME and issued to its PLME to write the indicated

- PHY PIB attribute.

5

6	9.2.5.2 Effect on receipt

7

- On receipt of the PLME-SET.request primitive, the PLME will attempt to write the given value to the

- indicated PHY PIB attribute in its database.

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

9.2.6 PLME-SET.confirm

The PLME-SET.confirm primitive reports the results of the attempt to set a PIB attribute.

The semantics of the PLME-SET.confirm primitive are as follows:

PLME-SET.confirm	(

status,

PIBAttribute

)

Table 130 specifies the parameters for the PLME-SET.confirm primitive.

Table 130—PLME-SET.confirm parameters

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
|  |  | SUCCESS, | The status of the attempt to set |  |
| status | Enumeration | UNSUPPORTED_ATTRIBUTE, |  |  |
|  |  |  | the requested PIB attribute. |  |
|  |  | INVALID_PARAMETER |  |  |
| PIBAttribute | Enumeration | As defined in Table 144 | The identifier of the PIB |  |
|  |  |  | attribute being confirmed. |  |

9.2.6.1 When generated

- The PLME-SET.confirm primitive is generated by the PLME and issued to its MLME in response to a

- PLME-SET.request primitive.

40

- If the PIBAttribute parameter specifies an attribute that is not found in the database, as shown in Table 144,

- the PLME will issue the PLME-SET.confirm primitive with a status of UNSUPPORTED_ATTRIBUTE. If

- the PIBAttibuteValue parameter specifies a value that is out of the valid range for the given attribute, the

- PLME will issue the PLME-SET.confirm primitive with a status of INVALID_PARAMETER.

45

- If the requested PHY PIB attribute is successfully written, the PLME will issue the PLME-SET.confirm

- primitive with a status of SUCCESS.

48

49	9.2.6.2 Effect on receipt

50

- On receipt of the PLME-SET.confirm primitive, the MLME is notified of the result of its request to set the

- value of a PHY PIB attribute. If the requested value was written to the indicated PHY PIB attribute, the

- status parameter is set to SUCCESS. Otherwise, the status parameter will indicate the error.

54

322	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

9.2.7 PLME-SET-TRX-STATE.request

The PLME -SET-TRX-STATE.request primitive requests that the PHY entity change the internal operating state of the transceiver. The transceiver will have three main states as follows:

- Transceiver disabled (TRX_OFF)

- Transmitter enabled (TX_ON)

- Receiver enabled (RX_ON)

The semantics of the PLME-SET-TRX-STATE.request primitive are as follows:

PLME-SET-TRX-STATE.request	(

state

)

Table 131 specifies the parameters for the PLME-SET-TRX-STATE.request primitive.

Table 131—PLME-SET-TRX-STATE.request parameters

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
|  |  | RX_ON, TRX_OFF, | The new state in which to configure the |  |
| state | Enumeration | FORCE_TRX_OFF, |  |  |
|  |  |  | transceiver. |  |
|  |  | TX_ON |  |  |

9.2.7.1 When generated

The PLME -SET-TRX- STATE.request primitive is generated by the MLME and issued to its PLME when the current operational state of the receiver needs to be changed.

9.2.7.2 Effect on receipt

On receipt of the PLME-SET-TRX-STATE.request primitive, the PLME will cause the PHY to attempt to change to the requested state.

9.2.8 PLME-SET-TRX-STATE.confirm

The PLME-SET- TRX-STATE.confirm primitive reports the result of a request to change the internal operating state of the transceiver.

The semantics of the PLME-SET-TRX-STATE.confirm primitive are as follows:

PLME-SET-TRX-STATE.confirm	(

status

)

Table 132 specifies the parameters for the PLME-SET-TRX-STATE.confirm primitive.

9.2.8.1 When generated

The PLME-SET-TRX-STATE.confirm primitive is generated by the PLME and issued to its MLME after attempting to change the internal operating state of the transceiver.

| Copyright © 20xx IEEE. All rights reserved. | 323 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

Table 132—PLME-SET-TRX-STATE.confirm parameters

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
|  |  | SUCCESS, RX_ON, | The result of the request to change the state |  |
| status | Enumeration | TRX_OFF, TX_ON, |  |  |
|  |  |  | of the transceiver. |  |
|  |  | BUSY_RX, BUSY_TX |  |  |

9.2.8.2 Effect on receipt

- On receipt of the PLME-SET-TRX-STATE.confirm primitive, the MLME is notified of the result of its

- request to change the internal operating state of the transceiver.

14

- If the state change is accepted, the PHY will issue the PLME-SET-TRX-STATE.confirm primitive with a

- status of SUCCESS. If this primitive requests a state that the transceiver is already configured, the PHY will

- issue the PLME-SET-TRX-STATE.confirm primitive with a status indicating the current state, i.e., RX_ON,

- TRX_OFF, or TX_ON. If this primitive is issued with RX_ON or TRX_OFF argument and the PHY is busy

- transmitting a PPDU, the PHY will issue the PLME-SET-TRXSTATE.confirm primitive with a status

- BUSY_TX and defer the state change until the end of transmission. If this primitive is issued with TX_ON

- or TRX_OFF argument and the PHY is in RX_ON state and has already received a valid preamble, the PHY

- will issue the PLME-SET-TRX-STATE.confirm primitive with a status BUSY_RX and defer the state

- change until the end of reception of the PPDU. If this primitive is issued with FORCE_TRX_OFF, the PHY

- will cause the PHY to go the TRX_OFF state irrespective of the state the PHY is in.

25

26	9.2.9 PLME-SWITCH.request

27

- The PLME-SWITCH.request primitive request is used by the DME to request that the PHY entity select the

- switch to enable the appropriate cells in the SW-BIT-MAP. The semantics of the PLME-SWITCH.request

- primitive are as follows:

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

PLME-SWITCH.request	(

SW-BIT-MAP,

DIR

)

Table 133 specifies the parameters for the PLME-SET-TRX-STATE.request primitive.

Table 133—PLME-SWITCH.request parameters

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
|  |  |  | One bit for each optical source or photode- |  |
|  |  |  | tector and is dependent on the direction. |  |
|  |  |  | Setting the kth bit to a “1” brings the corre- |  |
| SW-BIT-MAP | Vector of | Boolean | sponding optical source or photodetector |  |
|  | ‘n’  ‘m’entries |  | into the cell group. ‘n’ is the number of cells |  |
|  |  |  | and ‘m’ is the number of distinct data |  |
|  |  |  | streams from the PHY. The value of ‘m’ is |  |
|  |  |  | three for PHY III. |  |
| DIR |  | Boolean | ‘0’ is for TX and ‘1’ is for ‘RX’ |  |

324	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

9.2.9.1 When generated

The PLME-SWITCH.request primitive is generated by the DME and issued to its PLME when the current cell selection needs to be changed.

9.2.9.2 Effect on receipt

On receipt of the PLME-SWITCH.request primitive, the PLME will cause the PHY to attempt to change to the cell.

9.2.10 PLME-SWITCH.confirm

The PLME-SWITCH.confirm primitive reports the result of a request to change the currently operating cell.

The semantics of the PLME-SWITCH.confirm primitive are as follows:

PLME-SWITCH.confirm	(

status

)

Table 134 specifies the parameters for the PLME-SWITCH.confirm primitive.

Table 134—PLME-SWITCH.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS | The result of the request to change the cell. |

9.2.10.1 When generated

The PLME-SWITCH.confirm primitive is generated by the PLME and issued to its DME after attempting to change the cell.

9.2.10.2 Effect on receipt

On receipt of the PLME-SWITCH.confirm primitive, the DME is notified of the result of its request to change the currently operating cell.

If the PHY switch is able to select the new cell, the PHY will issue the PLME-SWITCH.confirm primitive with a status of SUCCESS.

9.3 PHY data s ervice

The PD-SAP supports the transport of MPDUs between a local MAC sublayer and a local PHY layer entity. Table 135 lists the primitives supported by the PD-SAP.

9.3.1 PD-DATA.request

The PD-DATA.request primitive requests the transfer of data from the MAC sublayer to form a PSDU at the local PHY entity.

| Copyright © 20xx IEEE. All rights reserved. | 325 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

Table 135—PD-SAP primitives

| PD-SAP primitive | Request | Confirm | Indication |
| --- | --- | --- | --- |
| PD-DATA | 9.3.1 | 9.3.2 | 9.3.3 |

The semantics of the PD-DATA.request primitive are as follows:

PD-DATA.request	(

psduLength,

psdu,

bandplanID

)

Table 136 specifies the parameters for the PD-DATA.request primitive.

Table 136—PD-DATA.request parameters

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
| psduLength | Unsigned | 0–aMaxPHYFrameSize | The number of octets in the PSDU to be |  |
|  | Integer |  | transmitted by the PHY entity. |  |
| psdu | Set of octets |  | The set of octets forming the PSDU to |  |
|  |  |  | be transmitted by the PHY entity. |  |
| bandplanID | Unsigned | 0–6 | Color band channel of PSDU. |  |
|  | Integer |  |  |  |

9.3.1.1 When generated

- The PD-DATA.request primitive is generated by a local MAC sublayer entity and issued to its PHY entity to

- request the transmission of an PPDU.

36

37	9.3.1.2 Effect on receipt

38

- The receipt of the PD-DATA.request primitive by the PHY entity will cause the transmission of the supplied

- PPDU.

41

42	9.3.2 PD-DATA.confirm

43

- The PD-DATA.confirm primitive confirms the end of the transmission of data from a local MAC sublayer

- entity.

46

47

48

49

50

51

52

53

54

The semantics of the PD-DATA.confirm primitive are as follows:

PD-DATA.confirm	(

status

)

Table 137 specifies the parameters for the PD-DATA.confirm primitive.

326	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 137—PD-DATA.confirm parameters

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
|  |  | SUCCESS, | The result of the request to |  |
| status | Enumeration | RX_ON, TRX- |  |  |
|  |  |  | transmit a frame. |  |
|  |  | _OFF |  |  |

9.3.2.1 When generated

The PD-DATA.confirm primitive is generated by the PHY entity and issued to its MAC sublayer entity in response to a PD-DATA.request primitive. Provided the transmitter is enabled (TX_ON state), the PHY will first construct a PPDU containing the supplied PSDU, and then transmit the PPDU. When the PHY entity has completed the transmission, it will issue the PD-DATA.confirm primitive with a status of SUCCESS.

If the PD- DATA.request primitive is received while the receiver is enabled (RX_ON state) or if the transceiver is disabled (TRX_OFF state), the PHY entity will issue the PD-DATA.confirm primitive with a status of RX_ON or TRX_OFF, respectively.

9.3.2.2 Effect on receipt

On receipt of the PD-DATA.confirm primitive, the MAC sublayer entity is notified of the result of its request to transmit. If the transmission attempt was successful, the status parameter is set to SUCCESS. Otherwise, the status parameter will indicate the error.

9.3.3 PD-DATA.indication

The PD-DATA.indication primitive indicates the transfer of data from the PHY to the local MAC sublayer entity.

The semantics of the PD-DATA.indication primitive are as follows:

PD-DATA.indication	(

psduLength,

psdu,

ppduLinkQuality

)

Table 138 specifies the parameters for the PD-DATA.indication primitive.

9.3.3.1 When generated

The PD -DATA.indication primitive is generated by the PHY entity and issued to its MAC sublayer entity to transfer a received PSDU. This primitive will not be generated if the received psduLength field is zero or greater than aMaxPHYFrameSize.

9.3.3.2 Effect on receipt

On receipt of the PD-DATA.indication primitive, the MAC sublayer is notified of the arrival of data across the PHY data service.

| Copyright © 20xx IEEE. All rights reserved. | 327 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  |  | Table 138—PD-DATA.indication parameters |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  |  |  |  |  |  |  |
| 3 |  | Name |  | Type | Valid | Description |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  | Unsigned |  | The number of octets contained in |  |
| 6 |  | psduLength |  |  | 0–aMaxPHYFrameSize | the PSDU received by the PHY |  |
|  |  |  |  | Integer |  |  |  |
| 7 |  |  |  |  |  | entity. |  |
| 8 |  |  |  |  |  |  |  |
|  |  | psdu |  | Set of octets | — | The set of octets forming the |  |
| 9 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | PSDU received by the PHY entity. |  |
| 10 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Wavelength quality indication |  |
| 11 |  |  |  |  |  |  |  |
|  |  | ppduLinkQuality |  | Unsigned | 0x00–0xff | (WQI) value measured during |  |
| 12 |  |  |  |  |  |  |  |
|  |  |  |  | Integer |  | reception of the PPDU as defined |  |
| 13 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | in 5.4.26.2. |  |
| 14 |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |
| 16 | 9.3.3.3 PHY Data Service |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |

- editor note: contents here should be merged into Clause 9.3. Consider merging first. Please PureLIFI pro-

- vide instructions on how to merge. See comment 223 of https://mentor.ieee.org/802.15/dcn/17/15-17-0297-

- 02-0013-comment-against-7m-d1.xlsx

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

9.3.3.3.1 PHY Data Service Primitives

Table 139 presents the PHY data service primitives.

Table 139—PHY data service primitives.

| Primitive | Request | Indicate | Confirm |
| --- | --- | --- | --- |
| PHY-DATA | X | X | X |
| PHY-MAC-HEADER | X | X | X |

Table 140 presents the PHY data service primitive parameters.

Table 140—PHY data service primitive parameters.

| Parameter | Associated Primitive | Value |  |
| --- | --- | --- | --- |
| DATA | PHY-DATA.request | One Octet taking values in the interval |  |
|  | PHY-DATA.indicate | X'00' – X'FF' |  |
|  | PHY-MAC-HEADER.request | Six octets containing the information |  |
| MAC_HEADER | PHY-MAC-HEADER.indicate | for the high-reliability MAC header |  |
|  |  | described in subclause 6.2.5.1. |  |

328	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 9.3.3.3.2 Detailed PHY Data Service Specification |  | 1 |  |
| --- | --- | --- | --- |
| 6.7.3.2.1 PHY-DATA.request |  | 2 |  |
|  |  | 3 |  |
| 6.7.3.2.1.1 Function |  | 4 |  |
|  |  | 5 |  |
| This primitive defines the transfer of an octet of data from the MAC sublayer to the local PHY entity. |  | 6 |  |
|  |  | 7 |  |
| 6.7.3.2.1.2 Semantics of the service primitive |  | 8 |  |
|  |  | 9 |  |
| The primitive provides the following parameters: |  | 10 |  |
|  |  | 11 |  |
| PHY-DATA.request(DATA) |  | 12 |  |
|  |  | 13 |  |
| The DATA parameter is an octet of value X'00' through X'FF'. |  | 14 |  |
|  |  | 15 |  |
| 6.7.3.2.1.3 When generated |  | 16 |  |
|  |  | 17 |  |
| This primitive is generated by the MAC sublayer to transfer an octet of data to the PHY entity. This |  | 18 |  |
|  |  | 19 |  |
| primitive can only be issued following a transmit initialization response (PHY-TXSTART.confirm) from the |  | 20 |  |
|  |  | 21 |  |
| PHY. |  | 22 |  |
| 6.7.3.2.1.4 Effect of receipt |  | 23 |  |
|  |  | 24 |  |
| The receipt of this primitive by the PHY entity causes the PHY to transmit an octet of data. When the PHY |  | 25 |  |
|  |  | 26 |  |
| entity receives the octet, it will issue a PHY-DATA.confirm to the MAC sublayer. |  | 27 |  |
| 6.7.3.2.2 PHY-DATA.indicate |  | 28 |  |
|  |  | 29 |  |
| 6.7.3.2.2.1 Function |  | 30 |  |
|  |  | 31 |  |
| This primitive indicates the transfer of data from the PHY to the local MAC entity. |  | 32 |  |
|  |  | 33 |  |
| 6.7.3.2.2.2 Semantics of the service primitive |  | 34 |  |
|  |  | 35 |  |
| The primitive provides the following parameters: |  | 36 |  |
|  |  | 37 |  |
| PHY-DATA.indicate (DATA) |  | 38 |  |
|  |  | 39 |  |
| The DATA parameter is an octet of value X'00' through X'FF'. |  | 40 |  |
|  |  | 41 |  |
| 6.7.3.2.2.3 When generated |  | 42 |  |
|  |  | 43 |  |
| The PHY-DATA.indication is generated by a receiving PHY entity to transfer the received octet of data to |  | 44 |  |
|  |  | 45 |  |
| the local MAC entity. |  | 46 |  |
| 6.7.3.2.2.4 Effect of receipt |  | 47 |  |
|  |  | 48 |  |
| The effect of receipt of this primitive by the MAC is unspecified. |  | 49 |  |
|  |  | 50 |  |
| 6.7.3.2.3 PHY-DATA.confirm |  | 51 |  |
|  |  | 52 |  |
| 6.7.3.2.3.1 Function |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 329 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- This primitive is issued by the PHY to the local MAC entity to confirm the transfer of data from the MAC

- entity to the PHY.

| 3 | 6.7.3.2.3.2 Semantics of the service primitive |  |
| --- | --- | --- |
| 4 |  |  |
| 5 | The semantics of the primitive are as follows: |  |
| 6 |  |  |
| 7 | PHY-DATA.confirm |  |
| 8 |  |  |
| 9 | This primitive has no parameters. |  |
| 10 |  |  |
| 11 | 6.7.3.2.3.3 When generated |  |
| 12 |  |  |
| 13 |  |  |

- This primitive will be issued by the PHY to the MAC entity when the PHY has completed the transfer of

- data from the MAC entity. The PHY will issue this primitive in response to every PHY-DATA.request prim-

- itive issued by the MAC sublayer.

| 17 | 6.7.3.2.3.4 Effect of receipt |  |
| --- | --- | --- |
| 18 |  |  |
| 19 | The receipt of this primitive by the MAC will cause the MAC to start the next MAC entity request. |  |
| 20 |  |  |
| 21 | 6.7.3.2.4 PHY-MAC-HEADER.request |  |
| 22 |  |  |
| 23 | 6.7.3.2.4.1 Function |  |
| 24 |  |  |
| 25 |  |  |

- This primitive defines the transfer of the high-reliability MAC header information from the MAC sublayer

- to the local PHY entity.

| 28 | 6.7.3.2.4.2 Semantics of the service primitive |  |
| --- | --- | --- |
| 29 |  |  |
| 30 | The primitive provides the following parameters: |  |
| 31 |  |  |
| 32 | PHY-MAC-HEADER.request(MAC_HEADER) |  |
| 33 |  |  |
| 34 | The MAC_HEADER parameter is six octets containing the information described in subclause 6.2.5.1. |  |
| 35 |  |  |
| 36 | 6.7.3.2.4.3 When generated |  |
| 37 |  |  |
| 38 | This primitive is generated by the MAC sublayer to transfer an octet of data to the PHY entity. This |  |
| 39 |  |  |
| 40 |  |  |

- primitive can only be issued following a transmit initialization response (PHY-TXSTART.confirm) from the

- PHY.

43

44	6.7.3.2.4.4 Effect of receipt

45

- The receipt of this primitive by the PHY entity causes the PHY to transmit the high-reliability MAC header

- data. When the PHY entity receives these six octets, it will issue a PHY-MAC-HEADER.confirm to the

- MAC sublayer.

| 49 | 6.7.3.2.5 PHY-MAC-HEADER.indicate |  |  |
| --- | --- | --- | --- |
| 50 |  |  |  |
| 51 | 6.7.3.2.5.1 Function |  |  |
| 52 |  |  |  |
| 53 |  |  |  |
| 54 |  |  |  |
|  | 330 | Copyright © 20xx IEEE. All rights reserved. |  |

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| This primitive indicates the transfer of the high-reliability MAC header information from the PHY to the |  | 1 |  |
| --- | --- | --- | --- |
| local MAC entity. |  | 2 |  |
| 6.7.3.2.5.2 Semantics of the service primitive |  | 3 |  |
|  |  | 4 |  |
| The primitive provides the following parameters: |  | 5 |  |
|  |  | 6 |  |
| PHY-MAC-HEADER.indicate (MAC_HEADER) |  | 7 |  |
|  |  | 8 |  |
| The MAC_HEADER parameter is six octets containing the information described in subclause 6.2.5.1. |  | 9 |  |
|  |  | 10 |  |
| 6.7.3.2.5.3 When generated |  | 11 |  |
|  |  | 12 |  |
| The PHY-MAC-HEADER.indication is generated by a receiving PHY entity to transfer the received six |  | 13 |  |
|  |  | 14 |  |
| octets of the high-reliability MAC header to the local MAC entity. |  | 15 |  |
| 6.7.3.2.5.4 Effect of receipt |  | 16 |  |
|  |  | 17 |  |
| The effect of receipt of this primitive by the MAC is unspecified. |  | 18 |  |
|  |  | 19 |  |
| 6.7.3.2.6 PHY-MAC-HEADER.confirm |  | 20 |  |
|  |  | 21 |  |
| 6.7.3.2.6.1 Function |  | 22 |  |
|  |  | 23 |  |
| This primitive is issued by the PHY to the local MAC entity to confirm the transfer of the high-reliability |  | 24 |  |
|  |  | 25 |  |
| MAC header information from the MAC entity to the PHY. |  | 26 |  |
| 6.7.3.2.6.2 Semantics of the service primitive |  | 27 |  |
|  |  | 28 |  |
| The semantics of the primitive are as follows: |  | 29 |  |
|  |  | 30 |  |
| PHY-MAC-HEADER.confirm |  | 31 |  |
|  |  | 32 |  |
| This primitive has no parameters. |  | 33 |  |
|  |  | 34 |  |
| 6.7.3.2.6.3 When generated |  | 35 |  |
|  |  | 36 |  |
| This primitive will be issued by the PHY to the MAC entity when the PHY has completed the transfer of the |  | 37 |  |
|  |  | 38 |  |
| high-reliability MAC header information from the MAC entity. The PHY will issue this primitive in |  | 39 |  |
| response to every PHY-MAC-HEADER.request primitive issued by the MAC sublayer. |  | 40 |  |
| 6.7.3.2.6.4 Effect of receipt |  | 41 |  |
|  |  | 42 |  |
| The receipt of this primitive by the MAC will cause the MAC to start the next MAC entity request. |  | 43 |  |
|  |  | 44 |  |
|  |  | 45 |  |
| 9.4 PHY enumeration des cription |  | 46 |  |
|  |  | 47 |  |
| Table 141 shows a description of the PHY enumeration values defined in the PHY specification. |  | 48 |  |
|  |  | 49 |  |
|  |  | 50 |  |
| 9.5 PHY cons tants and PIB attributes |  | 51 |  |
|  |  | 52 |  |
| This subclause specifies the constants and attributes required by the PHY. |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 331 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | Table 141—PHY enumeration description |  |  |  |
| --- | --- | --- | --- | --- |
| 2 |  |  |  |  |
| 3 | Enumeration | Value | Description |  |
| 4 |  |  |  |  |
| 5 | BUSY | 0x00 | The CCA attempt has detected a busy channel. |  |
| 6 |  |  |  |  |
|  | BUSY_RX | 0x01 | The transceiver is asked to change its state while receiving. |  |
| 7 |  |  |  |  |
| 8 | BUSY_TX | 0x02 | The transceiver is asked to change its state while |  |
| 9 |  |  | transmitting. |  |
| 10 |  |  |  |  |
|  | FORCE_TRX_OFF | 0x03 | The transceiver is to be switched off. |  |
| 11 |  |  |  |  |
| 12 | IDLE | 0x04 | The CCA attempt has detected an idle channel. |  |
| 13 |  |  |  |  |
|  | INVALID_PARAMETER | 0x05 | A SET/GET request was issued with a parameter in the |  |
| 14 |  |  |  |  |
|  |  |  | primitive that is out of the valid range. |  |
| 15 |  |  |  |  |
|  |  |  | The transceiver is in, or is to be configured into, the receiver |  |
| 16 | RX_ON | 0x06 |  |  |
| 17 |  |  | enabled state. |  |
| 18 | SUCCESS | 0x07 | The request completed successfully. |  |
| 19 |  |  |  |  |
|  |  |  | The transceiver is in, or is to be configured into, the |  |
| 20 | TRX_OFF | 0x08 |  |  |
|  |  |  | transceiver disabled state. |  |
| 21 |  |  |  |  |
| 22 | TX_ON | 0x09 | The transceiver is in, or is to be configured into, the |  |
| 23 |  |  | transmitter enabled state. |  |
| 24 |  |  |  |  |
|  | UNSUPPORTED_ATTRIBUTE | 0x0a | A SET/GET request was issued with the identifier of an |  |
| 25 |  |  |  |  |
|  |  |  | attribute that is not supported. |  |
| 26 |  |  |  |  |
| 27 |  |  |  |  |

- PHY PIB Attributes The PHY PIB comprises the attributes required to manage the PHY sublayer of a

- device. The attributes contained in the IEEE802.15.13-20xx PHY PIB are presented in Table 144 - PHY PIB

- Attributes.

31

- The additional PHY PIB attributes added for Offset Variable Pulse Width Modulation for Smart Device

- Flash Light PHY is presented the Table 142.

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Table 142—IEEE802.15.7r1 PHY PIB Attributes Additions

| Attribute | Identifier | Type | Range | Description |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | This attribute specifies the |  |
|  |  |  |  | application specific PHY |  |
|  |  |  |  | mode. |  |
| phySMFlashLIGHTAp- |  |  |  | 0 : Normal Data (Media |  |
|  | 0x10 | Unsigned | 0~255 | Content, Information |  |
| plicationSpecificMode |  |  |  |  |  |
|  |  |  |  | Content based on the |  |
|  |  |  |  | Application used for) |  |
|  |  |  |  | 1 : ID Data |  |
|  |  |  |  | 2 : Authentication Data |  |

9.5.1 PHY constants

The constants that define the characteristics of the PHY are presented in Table 143. These constants are hardware dependent and shall not be changed during operation.

332	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The constants that define the characteristics of the PHY are presented in Table 143. These constants are hardware dependent and shall not be changed during operation. As mentioned in Section 2.2, we choose the preamble frequency aPreambleFrequency because of the following two main reasons:

- Preamble Frequency must be reliably detected by commodity cameras (which have different read-our durations ranging from 20 to 30 microseconds).

- Preamble Frequency can be easily generated by mainstream low-cost MCUs, which normally operates with clock rates of 8 MHz or 16 MHz.

Table 143—PHY constants

| Constant | Description | Value |  |
| --- | --- | --- | --- |
| aMaxPHYFrameSize | The maximum PSDU size (in octets) the PHY | 1023 for PHY I, 65535 |  |
|  | shall be able to receive. | for PHY II, III |  |
| aTurnaroundTime-TX-RX | TX-to-RX maximum turnaround time (as defined | zero optical clock cycles |  |
|  | in 8.3.5) |  |  |
|  |  | PHY I: ≤ 240 optical |  |
| aTurnaroundTime-RX-TX | RX-to-TX maximum turnaround time (as defined | clock cycles, |  |
|  | in 8.3.6) | PHY II, III: ≤ 5120 opti- |  |
|  |  | cal clock cycles |  |
| aPreambleFrequency | The preamble field frequency. This should be uni- | 2232 Hz = 16 MHz/ |  |
| (aPF) | versal across different hardware. | (1024 x 7) |  |
| aFrequencyLabelingRatio | This indicates the frequency ratio of the frequency | 1.5 |  |
| (aFLR) | labeling field (aPF×aFLR). |  |  |
| aFSKPreamble1 | The first preamble frequency used for OCC | 200Hz |  |
|  | modes, including FSK and OOK modulations. |  |  |
|  | The number of LEDs (light sources) at Tx in case |  |  |
| aOccDefaultSpatialResolu- | of spatial modulation. |  |  |
|  | The default spatial resolution is applied for S2- | 2 light sources |  |
| tion |  |  |  |
|  | PSK, S8-PSK, and HS-PSK. |  |  |

All the PHY PIB are labeled as frequency ratio, using aPreambleFrequency as the baseline. If one would like to extend the PIB attribute table, do not overlap the ratio with aFrequencyLabelingRatio, since it will seriously interfere the interpretation of all subsequent data symbols.

9.5.2 PHY PIB attributes

The PHY PIB comprises the attributes required to manage the PHY of a device. Each of these attributes can be read or written using the PLME-GET.request and PLME-SET.request primitives, respectively. The attributes contained in the PHY PIB are presented in Table 144.

The PHY PIB comprises the attributes required to manage the PHY of a device. Each of these attributes can be assigned to the receiver through the optional field during the synchronization frame. The attributes contained in the PHY PIB are presented in Table 6.

All the PHY PIB are labelled as frequency ratio, using aPreambleFrequency as the baseline. If one would like to extend the PIB attribute table, do not overlap the ratio with aFrequencyLabelingRatio, since it will seriously interfere with the interpretation of all subsequent data symbols.

| Copyright © 20xx IEEE. All rights reserved. | 333 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- The phyOccApplicationSpecificMode is used to specify the PHY mode for a given geolocation such as the

- APP associated with a particular store, The information is provided by an out-of-band channel (i.e. WiFi)

- and is provided by the particular store prior usage (i.e. by downloading the stores OCC APP).

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Table 144—PHY PIB attributes

| Attribute | Identifier | Type | Range | Description |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | The wavelength used for all |  |
| phyCurrentChannel | 0x00 | Integer | 0–6 | following transmissions and |  |
|  |  |  |  | receptions (as defined in |  |
|  |  |  |  | 8.3.1). |  |
|  |  |  |  | b0=CCA mode 1 |  |
|  |  |  |  | b1=CCA mode 2 |  |
| phyCCAMode | 0x01 | Octet | enumerated | b2=CCA mode 3 |  |
|  |  |  |  | b3–b7=reserved |  |
|  |  |  |  | The CCA modes are defined |  |
|  |  |  |  | in 8.3.9. |  |
| phyDim |  |  |  | 0 is 0% or no visibility and |  |
|  | 0x02 | Integer | 0–1000 | 1000 is 100% visibility (full |  |
|  |  |  |  | brightness). |  |
|  |  |  |  | Indicates the PSDU carries |  |
|  |  |  |  | the frequencies that will be |  |
|  |  |  |  | used for the data frame later |  |
|  |  |  |  | on. |  |
| phyFrequencyLabeling* |  |  | Frequency | * This PIB attribute is explic- |  |
|  |  |  | ration = 1.5 |  |  |
|  |  |  |  | itly defined for future extensi- |  |
|  |  |  |  | bility. For now, only |  |
|  |  |  |  | aFrequencyLabelingRatio can |  |
|  |  |  |  | control the frequency ratio of |  |
|  |  |  |  | the Frequency Labeling field. |  |
|  |  |  |  | Indicates the the PSDU car- |  |
| phyUseSplitterSymbol |  |  | Frequency | ries the flag to toggle whether |  |
|  |  |  | ratio = 0.83 | the device is going to use SSs |  |
|  |  |  |  | or not. |  |
|  |  |  |  | Indicates the PSDU carries |  |
| phySplitterFrequency |  |  | Frequency | the splitter frequency. If the |  |
|  |  |  |  | SS is already in used, it will |  |
|  |  |  | ratio = 0.71 |  |  |
|  |  |  |  | use the original phySplitter- |  |
|  |  |  |  | Frequency until next cycle. |  |
|  |  |  |  | Indicates the PSDU carries |  |
| phySplitterDuration |  |  | Frequency | the duration of the SS. This is |  |
|  |  |  |  | represented as a ratio of sym- |  |
|  |  |  | ratio = 0.62 |  |  |
|  |  |  |  | bol duration to splitter dura- |  |
|  |  |  |  | tion in integer. |  |

334	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 144—PHY PIB attributes (continued)

| Attribute | Identifier | Type | Range | Description |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Indicates the PSDU carries |  |
|  |  |  |  | the duration of a data symbol |  |
|  |  |  |  | in the PSDU. This is repre- |  |
|  |  |  |  | sented as a ratio of the symbol |  |
|  |  |  |  | duration to 1/30 second in the |  |
|  |  |  |  | base 2 log scale. For example, |  |
| phySymbolDuration |  |  | Frequency | if the symbol duration is 1/ |  |
|  |  |  |  | 120 second, then the PSDU |  |
|  |  |  | ratio = 0.55 |  |  |
|  |  |  |  | would contain an integer -2. If |  |
|  |  |  |  | the symbol duration is 1/15 |  |
|  |  |  |  | second, then the PSDU would |  |
|  |  |  |  | contain an integer 2. Note that |  |
|  |  |  |  | this does not affect the dura- |  |
|  |  |  |  | tion of the preamble field and |  |
|  |  |  |  | the optional field. |  |
|  |  |  |  | This attribute is set to a one to |  |
| phyUseExtendedMode |  |  |  | indicate that an extended pre- |  |
|  | 0x03 | Integer | 0–1 | amble or visibility pattern is |  |
|  |  |  |  | to be used. Otherwise, it is set |  |
|  |  |  |  | to zero. |  |
|  |  |  | The row index | A table with three columns |  |
| phyColorFunction |  | 256 by 3 | ranges from 0 | per row. The first row is the |  |
|  | 0x04 | matrix of | to 255 and the | index, the second and the |  |
|  |  | integer | elements range | third columns define the |  |
|  |  |  | from 0 to 255. | color. |  |
|  |  |  |  | The frequency of blinking |  |
|  |  |  |  | notification: |  |
|  |  |  |  | 0: 0.25 Hz |  |
|  |  |  |  | 1: 0.5 Hz |  |
|  |  |  |  | 2: 0.75 Hz |  |
| phyBlinkingNotification- |  |  |  | 3: 1 Hz |  |
|  | 0x05 | Integer | 0–10 | 4: 1.25 Hz |  |
| Frequency |  |  |  |  |  |
|  |  |  |  | 5: 1.5 Hz |  |
|  |  |  |  | 6: 1.75 Hz |  |
|  |  |  |  | 7: 2 Hz |  |
|  |  |  |  | 8: 2.25 Hz |  |
|  |  |  |  | 9: 2.5 Hz |  |
|  |  |  |  | 10: 2.75 Hz |  |
|  |  |  |  | This attribute specifies the |  |
| phyOccApplicationSpe- |  |  |  | application specific PHY |  |
|  | 0x06 | Integer | 0-N | mode as shown in Table 181. |  |
| cificMode |  |  |  | E.g. Kookmin OCC Applica- |  |
|  |  |  |  | tion specific PHY modes are |  |
|  |  |  |  | shown in table 126. |  |
| phyOccProposerID |  |  |  | This attributes specifies the |  |
|  | 3 bit | Integer | 0-7 | OCC proposers as shown in |  |
|  |  |  |  | table 127. |  |

| Copyright © 20xx IEEE. All rights reserved. | 335 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

Table 144—PHY PIB attributes (continued)

| Attribute | Identifier | Type | Range | Description |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | This attribute specifies the |  |
|  |  |  |  | application specific PHY |  |
|  |  |  |  | mode. |  |
| phyINVApplicationSpe- |  |  |  | 0 : Normal Data (Media Con- |  |
|  | 0x10 | Unsigned | 0~255 | tent, Information Content |  |
| cifiMode |  |  |  |  |  |
|  |  |  |  | based on the Application used |  |
|  |  |  |  | for) |  |
|  |  |  |  | 1 : ID Data |  |
|  |  |  |  | 2 : Authentication Data |  |
|  |  |  |  | The optical clock rate is |  |
|  |  |  |  | applied for S2-PSK, S8-PSK, |  |
| phyOccOpticalClock- |  |  |  | and the S2-PSK of HS-PSK |  |
|  |  | integer | 5-20 | modulation. |  |
| Rate |  |  |  | 0: 5 Hz |  |
|  |  |  |  | 1: 10 Hz |  |
|  |  |  |  | 2: 15 Hz |  |
|  |  |  |  | 3: 20 Hz |  |
|  |  |  |  | The symbol rate is applied for |  |
|  |  |  |  | FSK, 2D-sequential color |  |
| phyOccSymbolRate |  | integer | 5-10 | code, invisible code modula- |  |
|  |  |  |  | tion: |  |
|  |  |  |  | 0: 5 Hz |  |
|  |  |  |  | 1: 10 Hz |  |
|  |  |  |  | The sub-packet rate is applied |  |
|  |  |  |  | for OOK modulation, indicat- |  |
| phyOccSubPacketRate |  |  |  | ing the number of data sub- |  |
|  |  | integer | 60-120 | packets (DS) repeated per sec- |  |
|  |  |  |  | ond. |  |
|  |  |  |  | 0: 60 DS/sec |  |
|  |  |  |  | 1: 120 DS/sec |  |
|  |  |  |  | This attribute specifies the |  |
|  |  |  |  | application specific PHY |  |
|  |  |  |  | mode. |  |
| phySMFlashLIGHTAp- |  |  |  | 0 : Normal Data (Media Con- |  |
|  | 0x10 | Unsigned | 0~255 | tent, Information Content |  |
| plicationSpecificMode |  |  |  |  |  |
|  |  |  |  | based on the Application used |  |
|  |  |  |  | for) |  |
|  |  |  |  | 1 : ID Data |  |
|  |  |  |  | 2 : Authentication Data |  |
|  |  |  |  | This attribute specifies the |  |
|  |  |  |  | application specific PHY |  |
|  |  |  |  | mode. |  |
| phyVTASCApplication- |  |  |  | 0 : Normal Data (Media Con- |  |
|  | 0x10 | Unsigned | 0~255 | tent, Information Content |  |
| SpecificMode |  |  |  |  |  |
|  |  |  |  | based on the Application used |  |
|  |  |  |  | for) |  |
|  |  |  |  | 1 : ID Data |  |
|  |  |  |  | 2 : Authentication Data |  |

10. PHY II s pecifications

- PHY II is targeted towards applications requiring high data rates, as shown in Table 107. For PHY II, the

- PHY header shall be sent at one of the following data rates: 1.25 Mb/s, 2.5 Mb/s, 6 Mb/s, 12 Mb/s, 24 Mb/s,

336	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

or 48 Mb/s, depending on the selected optical clock rate. Support for 1.25 Mb/s at an optical clock of 3.75 MHz is mandatory.

10.1 Reference modulator diagram

A reference implementation is in Figure 178. The PHR and PSDU parts of the frame are subject to the FEC for error protection. The PHR is encoded using parameters corresponding to the lowest data rate for the currently negotiated clock rate.

Figure 178—Reference modulator diagram for PHY II

10.2 Forward error correction encoder

A systematic Reed- Solomon code operating on GF(256) shall be used for PHY II to correct errors and increase the system reliability. The Reed-Solomon code may be shortened for the last block if it does not meet the block size requirements, as specified for PHY I in 10.2. No zero padding is required for the RS code.

The Reed-Solomon code is defined over GF(256) with a primitive polynomial x8+x4+x3+x2+1. The generator for the RS(160, 128) code and the RS(64, 32) code is given by

g(x) =

| x32+α11x31+α8x30+α109x29+α194x28+α254x27+α173x26+α11x25+α75x24+α218x23+α148x23+α14 |  |
| --- | --- |
| 9x21+α44x20+α0x19+α137x18+α104x17+α43x16+α137x15+α203x14+α99x13+α176x12+a59x11+α91 |  |
| x10+α194x9+α84x8+α53x7+α248x6+α107x5+α80x4+α28x3+α215x2+α251x+α18 | (10) |

where α is a primitive element in GF(256).

For the VPPM modes using 4B6B encoding, the RS code word (d1,..., d8) from the GF(256) RS code is broken into 2 nibbles (d1,.., d4) and (d5,.., d8). These nibbles are sent LSB first to the 4B6B encoder as shown in Figure 179.

Figure 179—GF(256) RS encoder usage with 4B6B encoder

| Copyright © 20xx IEEE. All rights reserved. | 337 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1	10.3 Run-length limiting encoder

2

- All PHY II VPPM modes shall use 4B6B encoding as defined in 10.5.1. All OOK PHY II modes shall use

- 8B10B encoding as specified in ANSI/INCITS 373.

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

10.4 Data mapping for VPPM

All PHY II VPPM modes shall use data mapping as defined in 10.6.

11. PHY VII Low-bandwidth PHY

11.1 Adaptive OFDM Concept

OFDM in the current specification enables a highly adaptive modular implementation, which supports both a high-efficiency PHY mode designed to enable optimal utilization of the low-bandwidth resources (up to 40 MHz of single-sided bandwidth) as well as a low -complexity PHY mode designed to enable high energy efficiency in mobile applications. Furthermore, the specification enables the adaptive and modular introduc-tion of energy-efficient waveforms such as enhanced unipolar OFDM (eU-OFDM), performance enhancing waveforms such as single-carrier FDMA (SC-FDMA) and waveforms designed for improved illumination control such as reverse polarity optical OFDM (RPO-OFDM). In addition, the specification supports the application of adaptive bit and energy loading techniques as well as multiple-input multiple-output (MIMO) techniques which can leverage the additional communication capacity of multiple light sources as well as the additional communication capacity introduced by the utilization of different optical wavelengths and light polarization for communication. The PHY specification supports relaying mechanisms for the cases when dedicated relay terminals are available. A DC-biased optical OFDM (DCO-OFDM) waveform is the basis and the foundation of all concepts, which are specified as optional features and can be applied in a non-conflicting manner.

11.2 Waveform

11.2.1 Adaptive OFDM signal generation

- The adaptive OFDM concept adopted in the current PHY specification allows for adaptive adjustment of the

- waveform in terms of the carrier mapping as well as in terms of the application of sophisticated signal pro-

- cessing and dimming techniques such as single-carrier pre-coding, unipolar modulation for improved trans-

- mission rates and efficiency and reverse polarity OFDM for dimming applications. The specification

- supports the application of adaptive bit and energy loading techniques as well as adaptive multiple-input

- multiple-output (MIMO) techniques which can leverage the additional communication capacity of multiple

- light sources as well as the additional communication capacity introduced by the utilization of different opti-

- cal wavelengths and light polarization for communication. The individual features of the adaptive signal

- generation approach are described as follows.

45

46	11.2.2 Carrier mapping

47

- The Low-bandwidth PHY mode is required to support fixed carrier mapping. All subcarriers are designated

- for information transfer by a single communication node in an allocated time interval. No special carrier

- designation for multiple access techniques exists, and no special carriers are designated for control informa-

- tion. Depending on whether the current PHY implementation supports adaptive bit allocation for the carrier

- mapping procedure (an optional feature), the mapping can be described as follows.

53

54

338	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

11.2.2.1 PHY does not support adaptive bit and power allocation

The QAM modulation symbols to which the binary information has been mapped are divided into groups of 24 symbols, where the original order at the output of the QAM modulation encoder is preserved (the bit-stream mapping to QAM symbols is described in subclause 6.3.5). Each ordered group of 24 symbols is mapped to the ordered set of subcarrier indices [3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 22, 23, 24, 25, 26, 27, 28], where indices 7 and 21 are assigned to fixed pilot values of 1 and -1, respectively. Zeros are assigned to subcarrier indices [0,1,2,29,30,31,32,33,34,62,63]. The Hermitian symmetric sequence of the 26 symbols assigned to indices [3-28] is assigned to subcarrier indices [35-61].

11.2.2.2 PHY supports adaptive bit and power allocation

Figure 180—IDFT realization by means of an IFFT algorithm.

If the PHY mode supports adaptive bit loading, the carrier mapping is determined by means of a bit alloca-tion table negotiated and specified at the MAC layer protocol through an exchange of control and manage-ment frames.

11.2.3 IDFT

The IDFT size is fixed in the Low-bandwidth PHY mode to enable lower implementation complexity. The modulation of the different frequency-domain subcarriers is achieved through an IDFT operation, described as follows:

| 63 | j2k |  |  |
| --- | --- | --- | --- |
|  | ----------- |  |  |
|  S  ke | 64 | (11) |  |

k = 0

where S [k] is the symbol mapped to subcarrier index k, as described in 6.3.2. Conventionally, the inverse discrete Fourier transform, as shown in Equation (6.3.3.1), is implemented with an IFFT algorithm. The DFT/IDFT size in the current PHY mode is fixed to 64. The DC subcarrier (subcarrier index 0 in the IFFT operation) and the 180-degree subcarrier (subcarrier index 32 in the IFFT operation) are set to 0. The infor-mation and pilot symbols with coefficients 1 to 26 are mapped to IFFT inputs 3 to 28, while the Hermitian symmetry symbols are mapped onto IFFT inputs 35 to 61. The high-frequency subcarriers, at inputs 29 to 34, form a guard interval and are set to zero. The low-frequency subcarriers, at inputs 0-2 and 62-63, are set to zero in order to avoid possible low-frequency distortion in the system due to baseline wandering and

| Copyright © 20xx IEEE. All rights reserved. | 339 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- background light interference. The mapping is illustrated in Figure 180. After performing an IFFT, the out-

- put is cyclically extended to the desired length as described in 17.2.4.

3

4	11.2.4 Cyclic prefix

5

- The cyclic prefix size is fixed in the Low-bandwidth PHY mode to enable lower implementation complexity.

- The cyclic prefix/guard interval (GI) is a cyclic extension of the IDFT result in each OFDM frame, where

- the cyclic prefix is formed by the last 16 samples of the IDFT result, appended in front of the 64-sample

- IDFT result. In the current PHY mode, the cyclic prefix size is not variable and cannot be changed.

10

11	11.2.5 Subcarrier modulation mapping

12

- The OFDM subcarriers shall be modulated by using BPSK, QPSK, 16-QAM, or 64-QAM, depending on the

- RATE requested. The encoded and interleaved binary serial input data shall be divided into groups of NBPSC

- (1, 2, 4, or 6) bits and converted into complex numbers representing BPSK, QPSK, 16-QAM, or 64-QAM

- constellation points. The conversion shall be performed according to Gray-coded constellation mappings,

- illustrated in Figure 10, with the input bit, b0, being the earliest in the stream. The output values, d, are

- formed by multiplying the resulting  (I+jQ)  value by a normalization factor KMOD, as described in

- Equation (12).

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

| d =  I + jQ KMOD | (12) |
| --- | --- |

The normalization factor, KMOD, depends on the base modulation mode, as prescribed in Table 145. Note that the modulation type can be different from the start to the end of the transmission, as the signal changes from SIGNAL to DATA, as shown in Figure 512. The purpose of the normalization factor is to achieve the same average power for all mappings. In practical implementations, an approximate value of the normaliza-tion factor can be used, as long as the device conforms with the modulation accuracy requirements.

Table 145—Modulation-dependent normalization factor KMOD

| Modulation | KMOD |
| --- | --- |
| BPSK | 1 |
| QPSK | ------ |
|  | 1 |
|  | 2 |
| 16-QAM | --------- |
|  | 1 |
|  | 10 |
| 64-QAM | --------- |
|  | 1 |
|  | 42 |

For BPSK, b0 determines the I value, as illustrated in Table 146. For QPSK, b0 determines the I value and b1 determines the Q value, as illustrated in Table 147. For 16-QAM, b0b1 determines the I value and b2b3 deter-mines the Q value, as illustrated in Table 148. For 64-QAM, b0b1b2 determines the I value and b3b4b5 deter-mines the Q value, as illustrated in Table 149.

340	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 146—BPSK encoding table

| Input bit (b0) | I-out | Q-out |
| --- | --- | --- |
| 0 | -1 | 0 |
| 1 | 1 | 0 |

Table 147—QPSK encoding table

| Input bit (b0) | I-out |  | Input bit (b1) | Q-out |
| --- | --- | --- | --- | --- |
| 0 | -1 |  | 0 | -1 |
| 1 | 1 |  | 1 | 1 |

Table 148—16-QAM encoding table

| Input bit (b0b1) | I-out |  | Input bit (b0b1) | Q-out |
| --- | --- | --- | --- | --- |
| 00 | -3 |  | 00 | -3 |
| 01 | -1 |  | 01 | -1 |
| 11 | 1 |  | 11 | 1 |
| 10 | 3 |  | 10 | 3 |

Table 149—64-QAM encoding table

| Input bit (b0b1b2) | I-out |  | Input bit (b3b4b5) | Q-out |
| --- | --- | --- | --- | --- |
| 000 | -7 |  | 000 | -7 |
| 001 | -5 |  | 001 | -5 |
| 011 | -3 |  | 011 | -3 |
| 010 | -1 |  | 010 | -1 |

| Copyright © 20xx IEEE. All rights reserved. | 341 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 149—64-QAM encoding table

| Input bit (b0b1b2) | I-out |  | Input bit (b3b4b5) | Q-out |
| --- | --- | --- | --- | --- |
| 110 | 1 |  | 110 | 1 |
| 111 | 3 |  | 111 | 3 |
| 101 | 5 |  | 101 | 5 |
| 100 | 7 |  | 100 | 7 |

11.2.6 Single-carrier modulation

Optional pre-coding before the OFDM modulator can be used to reduce the probability of clipping and enhance power efficiency. As explained in subclause 6.3.2, the current specification supports loading of 24 subcarriers with unique information. Hence, N<24 M-QAM symbols are passed through an N-point DFT before mapped to subcarriers [3, 4, 5, 6, 8, …, 20, 22, …, 28] as illustrated in Fig. 6.3.3.1. The DFT is com-puted as:

| N – 1 | j2k |  |  |
| --- | --- | --- | --- |
|  | –----------- |  |  |
|  S  ke | N | (13) |  |

k = 0

and the output result is mapped onto the different subcarriers in the order [12, 13, 14, …, 23, 0, 1, 2, …, 11] → [3,..,6,8,…,20,22,…,28]. If N<24, then the result of the N-point DFT is mapped to the middle of the 24 data subcarriers using the order ['0', '0', …, '0', floor(N/2)+1, floor(N/2)+2, …, N-1 0, 1, 2, …, floor(N/2), '0', …, '0', '0'] →[3,..,6,8,…,20,22,…,28], where '0' corresponds to a zero value placed on that respective sub-carrier since there are less values than subcarriers. When N<24, the spectral efficiency is reduced. If N<24, the output of the N-point DFT is replicated floor(24/N) times before mapped to the 24 data -carrying subcar-riers. Any subcarriers not loaded with information are set to '0'. For example, if N =9, then the output of the 10-point DFT is organized in the order ['0', '0', '0', 0, 1, 2, 3, 4, 5, 6, 7, 8, 0, 1, 2, 3, 4, 5, 6, 7, 8, '0', '0', '0']. If N=6, then the output of the 6-point DFT is organized in the order [0, 1, 2, 3, 4, 5, 0, 1, 2, 3, 4, 5, 0, 1, 2, 3, 4, 5, 0, 1, 2, 3, 4, 5]. If N=5, then the output of the 5-point DFT is organized in the order ['0', '0', 0, 1, 2, 3, 4, 0, 1, 2, 3, 4, 0, 1, 2, 3, 4, 0, 1, 2, 3, 4, '0', '0']. In all implementations, the DC value (value with index 0) of the DFT-precoding output should be mapped to the 13th data subcarrier (subcarrier 16 of the IDFT presented in Figure 180). Figure 181 illustrates the concept, where for the presented examples in this subclause, A=[0, 1, 2, …, 11] B=[12, 13, 14, …, 23], A=[0, 1, 2, 3, 4] B=[5, 6, 7, 8], A=[0, 1, 2] B=[3, 4, 5], and A=[0, 1, 2] B=[3, 4], respectively. If more than one copy of the N-DFT output is mapped onto the OFDM subcarriers, a pulse shaping filter such as a root-raised cosine filter can be used to shape the signal spectrum centred around subcarrier 16. The filter should not be applied to the pilot subcarriers (at positions 7 and 21 of the 64-point IDFT operation described in 6.3.2.1 and 6.3.3) as this might lead to significant attenuation of the pilot values, which are necessary for phase correction at the receiver. The use of the RRC filter is indicated with the value SC bits specified in subclause 6.2.3.2.5.

342	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 181—SC-OFDMA subcarrier mapping

11.2.7 Unipolar modulation

The real time-domain OFDM signal, generated at the PHY layer, is used to modulate the light emitting device (a light emitting diode (LED) or a laser diode (LD)), which serves as the transmitter front -end. The modulation is conducted only within the active operational range of the device. In this range, the electrical signal and the light output signal can only be positive at all times. The conventional approach for modulating the LED active range with an OFDM signal shall be to set a positive operating point, around which the bipo-lar OFDM signal can be realized. Figure 182(a) illustrates this principle. The positive bias can be introduced as part of the analog front-end (in the case of AC -coupled LED drivers) or as part of the information signal (in case of DC-coupled drivers). This approach is known as DC-biased optical OFDM (DCO-OFDM).

| Copyright © 20xx IEEE. All rights reserved. | 343 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

(a) DC-biased optical OFDM (DCO-OFDM).

(b) Enhanced Unipolar OFDM (eU-OFDM).

Figure 182—LED active range modulation

- An optional alternative modulation approach termed enhanced unipolar OFDM (eU-OFDM) is introduced in

- this subclause. It constitutes a digital processing algorithm, which can turn the bipolar OFDM signal into a

- strictly unipolar information signal without the addition of an energy intensive DC component that carries

- no additional information.

36

- The transmitter signals to the receiver the new transmission PHY mode using the eU and STR bits in the

- advanced modulation PHY header. For compliance purposes, the PLCP preamble and the PHY headers are

- encoded in a DCO-OFDM fashion as described in 3.3 - 3.5. Following the four BPSK OFDM symbols con-

- taining the PHY header, as well as the NMIMO reference symbols when applicable, the data field is encoded

- in an eU-OFDM fashion (see Fig. 6.3.7.1(b)). The eU-OFDM algorithm works as follows.

42

- For a single eU-OFDM stream, specified by STR = '00', two consecutive copies of every OFDM symbol are

- generated. The polarity of the samples in the second copy is inverted, and finally, all negative samples in the

- resulting time-domain signal are set to zero. The resulting positive signal can be used to modulate the trans-

- mitter. The concept is illustrated in Figure 183.

47

- For two eU-OFDM streams, indicated by STR = '10', every three OFDM symbols are grouped into an eU-

- OFDM block, where the first two symbols are assigned to data stream 1 (St1) and the remaining one symbol

- is assigned to data stream 2 (St2). The two symbols in St1 are modulated using the algorithm described for

- STR='00' and presented in Figure 183. The symbol in St2 is modulated in a similar manner, but instead of

- two copies, four consecutive copies are created for the OFDM symbol in St2, where the first two copies are

- kept unchanged, while the polarity of the samples in the next two copies is inverted. Following this proce-

- dure, all negative samples in both St1 and St2 are removed, and the two signals are summed. Any time-

344	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

domain oversampling and pulse shaping should be done after the removal of the negative samples. If done before the negative samples are removed, the oversampling and pulse shaping should also be performed at the receiver side during the signal re- modulation process required for the data recovery as explained in the RX algorithm. The resulting positive signal can be used to modulate the transmitter. The concept is pre-sented in Figure 184.

Figure 183—Unipolar OFDM generation (one stream)

For three eU-OFDM streams, indicated by STR = '01'', every seven OFDM symbols are grouped into an eU-OFDM block, where the first four symbols are assigned to data stream 1 (St1), the next 2 symbols are assigned to data stream 2 (St2) and the last symbol is assigned to data stream 3 (St3). The four symbols in St1 and the two streams in St2 are modulated using the algorithm described for STR='10' and presented in Figure 184. The symbol in St3 is modulated in a similar manner, however, eight consecutive copies of that symbol are generated, where the first four copies are left unchanged, while the polarity of the samples in the following four copies is reversed. Following this procedure, all negative samples in St1, St2 and St3 are removed. Any time-domain oversampling and pulse shaping should be done after the removal of the nega-tive samples. If done before the negative samples are removed, the oversampling and pulse shaping should also be performed at the receiver side during the signal re -modulation process required for the data recovery as explained in the RX algorithm. The signals in the three streams are summed and the resulting positive sig-nal can be used to modulate the transmitter. The concept is presented in Figure 185.

| Copyright © 20xx IEEE. All rights reserved. | 345 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 184—Unipolar OFDM generation (two streams)

346	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 185—Unipolar OFDM generation (three streams)

For four eU-OFDM streams, indicated by STR = '11', every fifteen OFDM symbols are grouped into an eU-OFDM block, where the first eight symbols are assigned to data stream 1 (St1), the next four symbols are assigned to data stream 2 (St2), the following two symbols are assigned to data stream 3 (St3) and the last symbol is assigned to data stream 4 (St4). In the first stream, two consecutive copies of every OFDM symbol are transmitted, where the second copy is multiplied by -1 (the signs of all samples are inverted in the time-domain) as described in the cases for STR = '00', STR = '10' and STR = '01'. In the second stream, four con-secutive copies of every OFDM symbol are transmitted, where the first two copies of the symbol are trans-

| Copyright © 20xx IEEE. All rights reserved. | 347 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- mitted in their original format, while the signs of the time-domain samples of the third and the fourth copy

- are inverted, i.e., the samples are multiplied by -1 as described in the cases for STR='10' and STR = '01'. In

- the third stream, eight consecutive copies of every OFDM symbol are transmitted, where the first four cop-

- ies are conveyed in their original format, while the signs of the time-domain samples of the fifth, sixth, sev-

- enth and eighth copy are inverted, i.e., the samples are multiplied by -1 as described in the case for STR =

- '01'. In the fourth stream, sixteen consecutive copies of every OFDM symbol are transmitted, where the first

- eight copies are conveyed in their original format, while the signs of the time-domain samples of the ninth,

- tenth, eleventh and twelfth, thirteenth, fourteenth, fifteenth and sixteenth copy are inverted, i.e., the samples

- are multiplied by -1 At this point, all negative samples in the four streams are removed and the signals from

- the three streams are added together. Any oversampling and pulse shaping should be done after the removal

- of the negative samples. If done before the negative samples are removed, the oversampling and pulse shap-

- ing should also be performed at the receiver side during the signal re-modulation process required for the

- data recovery as explained in the RX algorithm. The resulting positive signal can be used to modulate the

- transmitter.

15

- An algorithm for demodulating the received eU-OFDM data is recommended as follows. In order to demod-

- ulate the data encoded in the first stream (St1), the samples in every even OFDM symbol interval are sub-

- tracted from the samples in every odd OFDM symbol interval. For example, the samples in the second

- interval are subtracted from the samples in the first interval, the samples in the fourth interval are subtracted

- from the samples in the third interval, etc. Note that due to the structure of the other data streams (St2 up to

- St4), the subtraction operation completely removes interference from these streams. After the subtraction

- operation, the resulting data at St1 can be demodulated using the conventional OFDM demodulator algo-

- rithm. After the data at St1 is demodulated, the stream is re-modulated again and subtracted from the overall

- received signal. If time-domain oversampling and pulse shaping has been conducted in the time-domain

- before the negative samples have been removed in the signal generation procedure, then the same procedure

- (including the subsequent matched filtering at the receiver) should be applied to the re-modulated signal

- before it is subtracted from the overall received stream. Following the subtraction of the re-modulated St1

- signal, all symbol copies at St2 which were originally identical (the symbols in question are the symbols that

- have identical polarity) should be summed. For example, in Fig. 6.3.7.3, the first and the second symbols

- should be summed, the third and the fourth symbols should be summed, the fifth and the sixth symbols

- should be summed, etc. The resulting signal at St2, then, has the same format as the signal at St1. Hence, it

- can be demodulated after each even symbol is subtracted from each odd symbol. Upon demodulation, the

- resulting bits can be re-modulated and the resulting waveform can be subtracted from the overall received

- waveform as in the case for St1. The procedure is re-iterated for the remaining streams up to St4 (depending

- on the configuration) until all data bits are recovered.

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

348	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 186—RPO-OFDM signal

11.2.8 Dimming

Reverse polarity optical OFDM (RPO-OFDM) is defined as an optional feature in this specification to facil-itate dimming capabilities. This modulation scheme is expected to work in conjunction with eU-OFDM, but it can also be realized using DCO-OFDM. The eU-OFDM specification should be used for the generation of a unipolar signal, which would be indicated with the 'eU' bit defined in subclause 6.2.3.2. The current sub-clause provides the means for generating a RPO-OFDM signal using the DCO-OFDM waveform or the eU-OFDM waveform generated using the eU- OFDM specification. Note that RPO-OFDM modulation does not prevent the use of SC-OFDMA precoding and/or adaptive bit loading and/or MIMO encoding.

The RPO-OFDM modulation incorporates dimming while maintaining the average power per time-domain OFDM symbol and eliminating energy-intensive and adaptive DC component that carries no additional information. Accordingly, a constant signal-to-noise ratio (SNR) for a wide dimming range is achieved, the full active operational range of the device is utilized and high energy efficiency is realized. Here, the time-domain samples polarity of individual OFDM symbols is properly set to generate an OFDM waveform that has similar characteristics of a pulse-width modulation (PWM) signal in controlling the dimming percent-age. Such OFDM waveform also has two periods equivalent to the "on-time" and "off-time" periods of a PWM signal. Over an equivalent PWM period, the average forward current through the device is equivalent to the target dimming percentage. Assuming a 1W maximum optical power, the RPO-OFDM signal for two different dimming ratios of 70% and 20% duty-cycles (D) are shown in Fig. 6.3.8.1.

| Copyright © 20xx IEEE. All rights reserved. | 349 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

(a) Enhanced Unipolar OFDM (eU-OFDM).

(b) Reverse Polarity Optical OFDM (RPO-OFDM).

Figure 187—RPO-OFDM modulation signal

The PHY header information and MIMO reference symbols are transmitted using the default DCO-OFDM format. The OFDM symbols encoded in a DCO-OFDM/SC-OFDMA/eU-OFDM format are modified as fol-

lows. The first max(min(N OFDM,127) - NRPO, 0) symbols are transmitted as GRPOSOFDM/32 + Smin, where SOFDM is the time-domain DCO- OFDM/SC -OFDMA/eU-OFDM signal, Smin is the minimum possible sig-nal bias level and NOFDM is the number of OFDM data symbols remaining until the end of the packet. The

following min(NOFDM, N RPO) symbols are transmitted as Smax - GRPOSOFDM/32, where Smax is the maxi-mum possible signal bias level. The scaling factor GRPO enables the data-carrying part of the signal to be

scaled independently from the part of the signal which contains the control information in the PHY headers, which enables more robust transmission of the control information. The time-domain modulation signal generated using this approach is illustrated in Fig. 6.3.8.1. The value of NRPO is specified in subclause 6.2.3.2.7 and the value of GRPO is specified in subclause 6.2.3.2.8.

350	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 188—MIMO reference symbols format I

11.3 MIMO

11.3.1 Modified frame structure

In MIMO mode, the PHY layer frame structure is modified with the addition of the MIMO reference sym-bols (see Figure 188) described in the following subclause 6.4.2.

| Copyright © 20xx IEEE. All rights reserved. | 351 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

11.3.2 Reference symbols for MIMO channel estimation

Figure 189—MIMO reference symbols format II

11.3.2.1 MIMO Reference Symbols Format I

The MIMO reference symbols consist of NMIMO OFDM frame intervals. For each MIMO transmitter, only one OFDM frame interval is set to the desired channel estimation sequence (CES). All other intervals are set to zero. At the same time, the CES transmission intervals never coincide for any two transmitters. Hence, the MIMO reference symbols for the different transmitters are orthogonal to each other. The format is presented in Fig. 6.4.2.1.1.

11.3.2.2 MIMO Reference Symbols Format II

The MIMO reference symbols consist of NMIMO OFDM frame intervals. Every frame interval is set to the desired channel estimation sequence (CES). In addition, the CESs for each transmitter are modified by adjusting the polarity of the individual CES sequences according to a pre-determined set of Walsh sequences, where a value of '1' in the Walsh sequence corresponds to an unmodified CES sequence while a value of ' -1' corresponds to a CES sequence with reverse polarity. The format is presented in Fig. 6.4.2.2.1. The CES sequences in white are left unmodified, while the CES sequences in gray are multiplied by -1.

The Walsh sequences for a MIMO configuration with two transmitters (NMIMO = 2) correspond to the rows of the matrix W2MIMO:

| 1 | 1 | (14) |  |
| --- | --- | --- | --- |
| 1 | –1 |  |  |

The Walsh sequences for four transmitters (NMIMO = 4) correspond to the rows of the matrix W4MIMO:

352	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 1 | 1 | 1 |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | –1 | 1 | –1 | (15) |  |
| 1 | 1 | –1 | –1 |  |  |
| 1 | –1 | –1 | 1 |  |  |

The Walsh sequences for four transmitters (NMIMO = 8) correspond to the rows of the matrix W8MIMO:

|  | 1 | 1 | 1 | 1 |  | 1 | 1 | 1 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 –1 1 |  |  | –1 1 –1 1 |  |  |  |  | –1 |  |  |  |
|  | 1 | 1 | –1 –1 1 1 –1 –1 |  |  |  |  |  |  |  |  |  |
|  | 1 –1 –1 1 1 –1 –1 1 |  |  |  |  |  |  |  |  | (16) |  |  |
|  | 1 | 1 | 1 |  | 1 –1 –1 –1 –1 |  |  |  |  |  |  |  |
|  | 1 –1 1 |  |  | –1 –1 1 –1 1 |  |  |  |  |  |  |  |  |
|  | 1 | 1 | –1 –1 –1 –1 1 |  |  |  |  |  | 1 |  |  |  |
|  | 1 –1 –1 1 –1 1 1 |  |  |  |  |  |  |  | –1 |  |  |  |
| As a general rule, |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2k |  |  |  |  | 2k – 1 |  | 2k – 1 |  |  |  |  |
|  |  |  |  |  | WMIMO |  |  | WMIMO |  |  | (17) |  |
| WMIMO = |  |  |  |  |  | 2k – 1 |  | 2k – |  | 1 |  |  |
|  |  |  |  |  | WMIMO |  |  | –WMIMO |  |  |  |  |

x4  x3  x2  x1

x4  x3  x2  x1

Figure 190—Spatial repetition coding

| Copyright © 20xx IEEE. All rights reserved. | 353 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  |  | x7 |  | x5 | x3 | x1 |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | x8 | x6 | x4 | x2 |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  | Figure 191—Spatial multiplexing |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11.3.3 MIMO transmission modes |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |

- The system supports up to MMIMO=8 transmitters and NMIMO=8 receivers. MIMO operation mode refers to

- the combination of selected transmission type and configuration. MIMO configuration refers to the active

- number of transmitters and receivers. Based on the communication channel conditions, a MIMO system

- with MMIMO transmitters and NMIMO receivers (denoted as MMIMO × NMIMO MIMO) may operate using

- only Mactive < MMIMO transmitters and Mactive < NMIMO receivers. For example, an 8×8 MIMO system may

- operate as a 4×4 MIMO system keeping only a subset of their transmitters and receivers active. Supported

- transmission types shall be repetition coding and spatial multiplexing. The transmission type for the individ-

- ual transmitter elements and the subcarrier loading shall be negotiated at the MAC layer. The receiver shall

- monitor the MIMO operation modes using the MIMO reference symbols and shall signal the mode as well

- as the subcarrier loading using control frames in the MAC layer.

27

28	11.3.3.1 Spatial Repetition Coding:

29

- In the repetition coding (RC), the same information is transmitted from more than one transmit elements.

- The concept is illustrated in Figure 191 for a 2×2 MIMO system.

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

354	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 192—MIMO mode selection algorithm

11.3.3.2 Spatial Multiplexing:

In the spatial multiplexing (SMUX) more than one transmit element sends independent information. The concept is illustrated in Figure 191 for a 2×2 MIMO system.

The MIMO mode selection and adaptive bit loading routine is expected to follow the algorithm described in Figure 192. In the figure, the constraint metric denotes quality of service requirements that must be satisfied in a given application, for example the level of maximum bit error rate. The performance metric denotes a quality of service metric which the system attempts to optimize, such as the Phy_ SAP data rate. Valid data rates for spatial multiplexing MIMO communication with different number of transmitted streams are shown in Table 150 to Table 154.

11.4 Channel coding

The PPDU shall be encoded with a convolutional encoder of coding rate R = 1/2, 2/3, or 3/4, corresponding to the desired data rate. The convolutional encoder shall use the industry-standard generator polynomials, g0 = 1338 and g1 = 1718, of rate R = 1/2, as shown in Figure 193. The bit denoted as "A" shall be output from the encoder before the bit denoted as "B". The summation operation presented in Figure 193 is a modulo-2 summation, i.e., an XOR operation. Higher rates shall be derived from this encoding mechanism by employ-ing "puncturing." Puncturing is a procedure for omitting some of the encoded bits in the transmitter (thus reducing the number of transmitted bits and increasing the coding rate) and inserting a dummy "zero" metric into the convolutional decoder on the receive side in place of the omitted bits. The puncturing patterns are illustrated in Figure 194. Decoding by the Viterbi algorithm is recommended.

| Copyright © 20xx IEEE. All rights reserved. | 355 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

Table 150—Valid rate values for MIMO Spatial Multiplexing for 5 MHz Bandwidth

|  |  | Data | Data | Data | Data | Data | Data | Data | Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | FEC | rate | rate | rate | rate | rate | rate | rate | rate |  |
|  |  | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) |  |
|  | rate |  |  |  |  |  |  |  |  |  |
|  |  | 1 Tx | 2 Tx | 3 Tx | 4 Tx | 5 Tx | 6 Tx | 7 Tx | 8 Tx |  |
| BPSK | 1/2 | 1.5 | 3 | 4.5 | 6 | 7.5 | 9 | 10.5 | 12 |  |
|  | 3/4 | 2.25 | 4.5 | 6.75 | 9 | 11.25 | 13.5 | 15.75 | 18 |  |
| 4-QAM | 1/2 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |  |
|  | 3/4 | 4.5 | 9 | 13.5 | 18 | 22.5 | 27 | 31.5 | 36 |  |
| 16-QAM | 1/2 | 6 | 12 | 18 | 24 | 30 | 36 | 42 | 48 |  |
|  | 3/4 | 9 | 18 | 27 | 36 | 45 | 54 | 63 | 72 |  |
| 64-QAM | 2/3 | 12 | 24 | 36 | 48 | 60 | 72 | 84 | 96 |  |
|  | 3/4 | 13.5 | 27 | 40.5 | 54 | 67.5 | 81 | 94.5 | 108 |  |

Table 151—Valid rate values for MIMO Spatial Multiplexing for 10 MHz Bandwidth

|  |  | Data | Data | Data | Data | Data | Data | Data | Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FEC | rate | rate | rate | rate | rate | rate | rate | rate |  |
| Modulation |  | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) |  |
|  | rate |  |  |  |  |  |  |  |  |  |
|  |  | 1 Tx | 2 Tx | 3 Tx | 4 Tx | 5 Tx | 6 Tx | 7 Tx | 8 Tx |  |
| BPSK | 1/2 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |  |
|  | 3/4 | 4.5 | 9 | 13.5 | 18 | 22.5 | 27 | 31.5 | 36 |  |
| 4-QAM | 1/2 | 6 | 12 | 18 | 24 | 30 | 36 | 42 | 48 |  |
|  | 3/4 | 9 | 18 | 27 | 36 | 45 | 54 | 63 | 72 |  |
| 16-QAM | 1/2 | 12 | 24 | 36 | 48 | 60 | 72 | 84 | 96 |  |
|  | 3/4 | 18 | 36 | 54 | 72 | 90 | 108 | 126 | 144 |  |
| 64-QAM | 2/3 | 24 | 48 | 72 | 96 | 120 | 144 | 168 | 192 |  |
|  | 3/4 | 27 | 54 | 81 | 108 | 135 | 162 | 189 | 216 |  |

11.4.1 Channel coding for the header

- The PHY headers (both the basic header and the advanced modulation header) shallbe encoded using 1/2

- rate FEC coding (no puncturing is used). The encoded bits shall also be mapped to a BPSK symbol constel-

- lation.

48

49	11.4.2 Channel coding for the data

50

- The DATA field, composed of SERVICE, PSDU, tail, and pad parts, shallbe encoded using the rate specified

- by the MAC layer protocol.

53

54

356	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 152—Valid rate values for MIMO Spatial Multiplexing for 15 MHz Bandwidth

|  |  | Data | Data | Data | Data | Data | Data | Data | Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | FEC | rate | rate | rate | rate | rate | rate | rate | rate |  |
|  |  | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) |  |
|  | rate |  |  |  |  |  |  |  |  |  |
|  |  | 1 Tx | 2 Tx | 3 Tx | 4 Tx | 5 Tx | 6 Tx | 7 Tx | 8 Tx |  |
| BPSK | 1/2 | 4.5 | 9 | 13.5 | 18 | 22.5 | 27 | 31.5 | 36 |  |
|  | 3/4 | 6.75 | 13.5 | 20.25 | 27 | 33.75 | 40.5 | 47.25 | 54 |  |
| 4-QAM | 1/2 | 9 | 18 | 27 | 36 | 45 | 54 | 63 | 72 |  |
|  | 3/4 | 13.5 | 27 | 40.5 | 54 | 67.5 | 81 | 94.5 | 108 |  |
| 16-QAM | 1/2 | 18 | 36 | 54 | 72 | 90 | 108 | 126 | 144 |  |
|  | 3/4 | 27 | 54 | 81 | 108 | 135 | 162 | 189 | 216 |  |
| 64-QAM | 2/3 | 36 | 72 | 96 | 144 | 180 | 216 | 252 | 288 |  |
|  | 3/4 | 40.5 | 81 | 121.5 | 162 | 202.5 | 243 | 283.5 | 324 |  |

Table 153—Valid rate values for MIMO Spatial Multiplexing for 20 MHz Bandwidth

|  |  | Data | Data | Data | Data | Data | Data | Data | Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FEC | rate | rate | rate | rate | rate | rate | rate | rate |  |
| Modulation |  | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) |  |
|  | rate |  |  |  |  |  |  |  |  |  |
|  |  | 1 Tx | 2 Tx | 3 Tx | 4 Tx | 5 Tx | 6 Tx | 7 Tx | 8 Tx |  |
| BPSK | 1/2 | 6 | 12 | 18 | 24 | 30 | 36 | 42 | 48 |  |
|  | 3/4 | 9 | 18 | 27 | 36 | 45 | 54 | 63 | 72 |  |
| 4-QAM | 1/2 | 12 | 24 | 36 | 48 | 60 | 72 | 84 | 96 |  |
|  | 3/4 | 18 | 36 | 54 | 72 | 90 | 108 | 126 | 144 |  |
| 16-QAM | 1/2 | 24 | 48 | 72 | 96 | 120 | 144 | 168 | 192 |  |
|  | 3/4 | 36 | 72 | 96 | 144 | 180 | 216 | 252 | 288 |  |
| 64-QAM | 2/3 | 48 | 96 | 144 | 192 | 240 | 288 | 336 | 384 |  |
|  | 3/4 | 54 | 108 | 162 | 216 | 270 | 324 | 378 | 432 |  |

| Copyright © 20xx IEEE. All rights reserved. | 357 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 154—Valid rate values for MIMO Spatial Multiplexing for 40 MHz Bandwidth

|  |  | Data | Data | Data | Data | Data | Data | Data | Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | FEC | rate | rate | rate | rate | rate | rate | rate | rate |  |
|  |  | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) | (Mb/s) |  |
|  | rate |  |  |  |  |  |  |  |  |  |
|  |  | 1 Tx | 2 Tx | 3 Tx | 4 Tx | 5 Tx | 6 Tx | 7 Tx | 8 Tx |  |
| BPSK | 1/2 | 12 | 24 | 36 | 48 | 60 | 72 | 84 | 96 |  |
|  | 3/4 | 18 | 36 | 54 | 72 | 90 | 108 | 126 | 144 |  |
| 4-QAM | 1/2 | 24 | 48 | 72 | 96 | 120 | 144 | 168 | 192 |  |
|  | 3/4 | 36 | 72 | 108 | 144 | 180 | 216 | 252 | 288 |  |
| 16-QAM | 1/2 | 48 | 96 | 144 | 192 | 240 | 288 | 336 | 384 |  |
|  | 3/4 | 72 | 144 | 216 | 288 | 360 | 432 | 504 | 576 |  |
| 64-QAM | 2/3 | 96 | 192 | 288 | 384 | 480 | 576 | 672 | 768 |  |
|  | 3/4 | 108 | 216 | 324 | 432 | 540 | 648 | 756 | 864 |  |

11.4.3 Data interleaving

Figure 193—Convolution encoder (133,171)

All encoded data bits shall be interleaved by a block interleaver with a block size corresponding to the num-

ber of bits in two OFDM symbols, 2NCBPS. The interleaver is defined by a two-step permutation. The first permutation ensures that adjacent coded bits are mapped onto nonadjacent subcarriers. The second ensures

that adjacent coded bits are mapped alternately onto less and more significant bits of the constellation and, thereby, long runs of low reliability (LSB) bits are avoided. The index of the coded bit before the first per - mutation shall be denoted by k; i shall be the index after the first and before the second permutation; and j shall be the index after the second permutation, just prior to modulation mapping.

The first permutation is defined by the rule:

358	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

| IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | IEEE Draft P802.15.13/D0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i = |  | 2NCBPS |  |  |  |  |  |  |  |  |  |  |  |  | k |  | k | = | 0, 1, , 2N |  |  | – 1 |  |  |  |  |  |  | (18) | 1 |  |
|  |  | ----------------- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  k mod 16 + floor |  |  |  |  |  |  |  |  |  | ----- , |  |  |  |  |  | CBPS |  |  |  |  |  |  |  |  | 2 |  |
|  |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |
| The function floor (.) denotes the largest integer not exceeding the parameter. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |
| The second permutation is defined by the rule |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |
|  |  |  |  |  | i |  |  |  i + |  |  |  |  |  | – floor |  |  |  |  | i |  |  |  |  |  |  |  |  |  |  | 8 |  |
| j = | s  floor |  |  |  |  |  |  |  |  | 2N |  |  |  |  |  |  | 16 |  |  |  | mod s, i = 0, 1, , 2N |  |  |  |  |  |  | – 1 | (19) | 9 |  |
|  |  |  |  |  |  - + |  |  |  |  |  | CBPS |  |  |  |  |  |  |  | --------------------- |  |  |  |  |  |  |  | CBPS |  |  |  |  |
|  |  |  |  |  |  s |  |  |  |  |  |  |  |  |  |  |  |  |  | 2NCBPS |  |  |  |  |  |  |  |  |  |  | 10 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11 |  |
| The value of s is determined by the number of coded bits per subcarrier, NBPSC, according to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 13 |  |
|  |  |  |  NBPSC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 14 |  |
| s = | max |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (20) | 15 |  |
|  |  |  |  | ------------------, 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16 |  |
| The deinterleaver, which performs the inverse relation, is also defined by two permutations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 17 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 18 |  |
| Here the index of the original received bit before the first permutation shall be denoted by j; i shall be the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 19 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |
| index after the first and before the second permutation; and k shall be the index after the second permutation, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 21 |  |
| just prior to delivering the coded bits to the convolutional (Viterbi) decoder. The first permutation is defined |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 22 |  |
| by the rule |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 23 |  |
|  |  |  |  |  | j |  |  |  |  |  |  |  |  |  |  | j |  |  |  |  |  |  |  |  |  |  |  |  |  | 24 |  |
| i = | s  floor |  |  |  |  |  |  |  | j + | floor |  |  | 16  |  |  |  |  |  | mod s, | j | = 0, 1, , 2N |  |  |  |  | – 1 |  |  | (21) | 25 |  |
|  |  |  |  |  | - | + |  |  |  |  |  |  |  |  | --------------------- |  |  |  |  |  |  |  |  | CBPS |  |  |  |  |  |  |  |
|  |  |  |  |  |  s |  |  |  |  |  |  |  |  |  | 2NCBPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 26 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 27 |  |
| where s is defined in Equation (20). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 29 |  |
| This permutation is the inverse of the permutation described in Equation (19). The second permutation is |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 31 |  |
| defined by the rule: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 32 |  |
| k = | 16  i –  2N |  |  |  |  |  |  |  |  | – 1floor |  |  |  |  |  |  | j |  |  | 0, 1, , 2N |  |  |  |  | – 1 |  |  |  | (22) | 33 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 34 |  |
|  |  |  |  |  |  |  | CBPS |  |  |  |  |  |  |  | 16  --------------------- , i = |  |  |  |  |  |  |  | CBPS |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2NCBPS |  |  |  |  |  |  |  |  |  |  |  |  |  | 35 |  |
| This permutation is the inverse of the permutation described in Equation (5). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 36 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 37 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 38 |  |
| 11.5 Relaying |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 39 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  |
| 11.5.1 Amplify and forward |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 41 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 42 |  |
| The PPDU shallbe buffered at the receiving node and upon successful demodulation of the advanced modu- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 43 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 44 |  |
| lation header and identification of the amplify and forward specification described in subclauses 6.2.3.2.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 45 |  |
| and 6.2.3.2.9, the PPDU shallbe retransmitted without any further processing at the PHY layer. In FD relay- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 46 |  |
| ing mode, the relay STA shall simultaneously receive the information on the downlink and shall transmit it |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 47 |  |
| on the uplink. In HD relaying mode, the relay STA shall store the data packets in order to retransmit them to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 48 |  |
| the STA during its designated transmission period. The relay STA can erase the stored packet after it |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 49 |  |
| receives the ACK. The method is proprietary. In both configurations, the PHY header shallbe demodulated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 |  |
| before a decision for relaying can be made. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 51 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 52 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 53 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 54 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Copyright © 20xx IEEE. All rights reserved. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 359 |  |  |
|  |  |  |  |  |  |  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 11.5.2 Decode and forward |
| --- | --- |
| 2 |  |

- The PPDU shallbe completely demodulated and upon successful identification of the decoded and forward

- specification described in subclauses 6.2.3.2.8 and 6.2.3.2.9, the PPDU shallbe re-modulated and re-trans-

- mitted. The relay STA can erase the stored packet after it receives the ACK. The method is proprietary.

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

360	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 194—Puncturing (bit stealing) algorithm

| Copyright © 20xx IEEE. All rights reserved. | 361 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 11.6 PHY Service Specifications |
| --- | --- |
| 2 |  |
| 3 | editor note: Clause 11.6 should be merged into Clause 9.2. Suggest Nikola give instructions on how to |

- merge. See comment 232 of https://mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-

- 7m-d1.xlsx

6

7	11.6.1 Overview

8

- The PHY services provided to the IEEE 802.15.13 WLAN MAC are described in this clause. These services

- are described in an abstract way and do not imply any particular implementation or exposed interface.

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

11.6.2 PHY Management Service

11.6.2.1 PHY Management Service Primitives

Table 155 presents the PHY management service primitives.

Table 155—PHY management service

| Primitive | Request | Indicate | Confirm |
| --- | --- | --- | --- |
| PHY-TXSTART | X |  | X |
| PHY-TXEND | X |  | X |
| PHY-RXSTART |  | X |  |
| PHY-RXEND |  | X |  |
| PHY-SET-LOADING | X |  | X |
| PHY-PROVIDE-CSI |  | X |  |

Table 156 presents the parameters used by the management service primitives.

Table 156—PHY management service primitive parameters

| Parameter | Associated Primitive | Value |
| --- | --- | --- |
| TXVECTOR | PHY-TXSTART.request | A set of parameters |
| RXVECTOR | PHY-RXSTART.indicate | A set of parameters |
| RXERROR | PHY-RXEND.indicate | NoError, FormatViolation, Unsupported Rate |
| BIT_LOADING_VECTOR | PHY-SET-LOADING.request | A set of parameters |
| CSI_VECTOR | PHY-PROVIDE-CSI.request | A set of parameters |

Table 157 presents the scalar parameters associated with the vector parameters in Table 156.

362	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 157—PHY management service primitive parameter vector descriptions

| Parameter | Associated Vector | Value |  |
| --- | --- | --- | --- |
| DATARATE | TXVECTOR | A 3-bit value setting the data rate according to |  |
|  | RXVECTOR | the specification in Table 6.2.3.1.1.1 |  |
|  | TXVECTOR | The length of the data to be transmitted in |  |
| LENGTH |  | octets as indicated by the LENGTH parameter |  |
|  | RXVECTOR |  |  |
|  |  | described in subclause 6.2.3.1.3 |  |
|  |  | Indicates whether adaptive bit and energy |  |
| BIT_LOADING | TXVECTOR | loading should be used at the PHY to encode |  |
|  |  | the data. The possible values are ‘YES’ or |  |
|  |  | ‘NO’. |  |
|  |  | Indicates whether the channel state informa- |  |
| CHANNEL_ESTIMATION | TXVECTOR | tion should be estimated along with the current |  |
|  |  | transmission. |  |
|  |  | Indicates whether eU-OFDM encoding should |  |
|  |  | be used with the current transmission. The pos- |  |
| EU_OFDM | TXVECTOR | sible values of this parameter are ‘YES’ or |  |
|  |  | ‘NO’ |  |
|  |  | Indicates the number of eU-OFDM streams |  |
| EU_OFDM_STREAMS | TXVECTOR | that should be used for the encoding in the cur- |  |
|  |  | rent transmission. |  |
|  |  | Indicates whether SC-FDMA precoding |  |
| SC_FDMA | TXVECTOR | should be used at the PHY layer for the current |  |
|  |  | transmission. |  |
| DFT_SIZE | TXVECTOR | The size of the DFT that should be used in the |  |
|  |  | SC-FDMA precoding. |  |
|  |  | The number of OFDM frames transmitted with |  |
| RPO_FRAMES | TXVECTOR | reversed polarity. Possible values are integers |  |
|  |  | between 0 and 127. |  |
| RPO_GAIN | TXVECTOR | A gain factor for the RPO-OFDM data trans- |  |
|  |  | mission. Possible values are between 0 and 63. |  |
|  |  | Indicates whether relaying should be used for |  |
| RELAYING | TXVECTOR | the current transmission. Possible values are |  |
|  |  | 'YES' or 'NO'. |  |
|  |  | The type of relaying operation that should be |  |
| RELAYING_TYPE | TXVECTOR | used. Possible values are 'FD-AF', 'HD-AF', |  |
|  |  | 'FD-DF' or 'HD-DF' as specified in Table |  |
|  |  | 6.2.3.2.10. |  |
|  | TXVECTOR | Indicates whether MIMO is used in the current |  |
| MIMO_ENABLED | BIT_LOADING_VECTOR | transmission. Possible values are 'YES' or |  |
|  | CSI_VECTOR | 'NO'. |  |
|  |  | The type of reference symbols used for chan- |  |
| REFERENCE_SYMBOLS | TXVECTOR | nel estimation in the MIMO transmission. Pos- |  |
|  |  | sible values are 'Type I' or 'Type II'. |  |

| Copyright © 20xx IEEE. All rights reserved. | 363 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

Table 157—PHY management service primitive parameter vector descriptions

| Parameter | Associated Vector | Value |  |
| --- | --- | --- | --- |
| MIMO_CHANNELS | BIT_LOADING_VECTOR | The number of MIMO Channels. Possible val- |  |
|  |  | ues are between 0 and 16. |  |
|  |  | A set of 24 values, corresponding to the bit |  |
|  |  | loading on each of the 24 information-bearing |  |
|  |  | subcarriers. Possible values are any one of the |  |
|  |  | constellation sizes presented in subclause |  |
| BIT_LOADING | BIT_LOADING_VECTOR | 6.3.5. If MIMO_ENABLED is equal to 'YES', |  |
|  |  | then this parameter is a vector containting |  |
|  |  | MIMO_CHANNELS vectors each with 24 |  |
|  |  | values corresponding to the bit loading on each |  |
|  |  | of the 24 subcarriers for the respective MIMO |  |
|  |  | channel out of MIMO_CHANNELS channels. |  |
|  |  | A set of 24 values, corresponding to the scal- |  |
|  |  | ing of the energy on each of the 24 informa- |  |
|  |  | tion-bearing subcarriers. A value of 1 |  |
|  |  | corresponds to the energy being scaled by 1 or, |  |
|  |  | i.e., unchnaged. If MIMO_ENABLED is equal |  |
| ENERGY_LOADING | BIT_LOADING_VECTOR | to 'YES', then this parameter is a vector con- |  |
|  |  | tainting MIMO_CHANNELS vectors each |  |
|  |  | with 24 values corresponding to the energy |  |
|  |  | scaling on each of the 24 subcarriers for the |  |
|  |  | respective MIMO channel out of |  |
|  |  | MIMO_CHANNELS channels. |  |
|  |  | A set of 24 values, corresponding to the chan- |  |
|  |  | nel state information on each of the 24 infor- |  |
|  |  | mation-bearing subcarriers. If |  |
|  |  | MIMO_ENABLED is equal to 'YES', then this |  |
| CSI | CSI_VECTOR | parameter is a vector containting |  |
|  |  | MIMO_CHANNELS vectors each with 24 |  |
|  |  | values corresponding to the energy scaling on |  |
|  |  | each of the 24 subcarriers for the respective |  |
|  |  | MIMO channel out of MIMO_CHANNELS |  |
|  |  | channels. |  |

11.6.2.2 Detailed PHY Management Service Specification

11.6.2.2.1 PHY-TXSTART.request

6.7.2.2.1.1 Function

- This primitive is a request by the MAC sublayer to the local PHY entity to start the transmission of an

- MPDU.

| 45 | 6.7.2.2.1.2 Semantics of the service primitive |  |
| --- | --- | --- |
| 46 |  |  |
| 47 | The primitive provides the following parameters: |  |
| 48 |  |  |
| 49 | PHY-TXSTART.request(TXVECTOR) |  |
| 50 |  |  |
| 51 |  |  |

- The TXVECTOR represents a list of parameters that the MAC sublayer provides to the local PHY entity in

- order to transmit an MPDU. This vector contains PHY management parameters. The required PHY param-

- eters are listed in Table 6.7.2.1.3.

364	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 6.7.2.2.1.3 When generated |  | 1 |  |
| --- | --- | --- | --- |
| This primitive will be issued by the MAC sublayer to the PHY entity when the MAC sublayer needs to begin |  | 2 |  |
|  |  | 3 |  |
| the transmission of an MPDU. |  | 4 |  |
| 6.7.2.2.1.4 Effect of receipt |  | 5 |  |
|  |  | 6 |  |
| The effect of receipt of this primitive by the PHY entity will be to start the transmission process. |  | 7 |  |
|  |  | 8 |  |
| 11.6.2.2.2 PHY-TXSTART.confirm |  | 9 |  |
|  |  | 10 |  |
| 6.7.2.2.2.1 Function |  | 11 |  |
|  |  | 12 |  |
| This primitive is issued by the PHY to the local MAC entity to confirm the start of a transmission. The PHY |  | 13 |  |
|  |  | 14 |  |
| will issue this primitive in response to every PHY-TXSTART.request primitive issued by the MAC sublayer. |  | 15 |  |
| 6.7.2.2.2.2 Semantics of the service primitive |  | 16 |  |
|  |  | 17 |  |
| The semantics of the primitive are as follows: |  | 18 |  |
|  |  | 19 |  |
| PHY-TXSTART.confirm |  | 20 |  |
|  |  | 21 |  |
| This primitive has no parameters. |  | 22 |  |
|  |  | 23 |  |
| 6.7.2.2.2.3 When generated |  | 24 |  |
|  |  | 25 |  |
| This primitive will be issued by the PHY to the MAC entity when the PHY has received a PHY- |  | 26 |  |
|  |  | 27 |  |
| TXSTART.request from the MAC entity and is ready to begin accepting outgoing data octets from the MAC. |  | 28 |  |
| 6.7.2.2.2.4 Effect of receipt |  | 29 |  |
|  |  | 30 |  |
| The receipt of this primitive by the MAC entity will cause the MAC to start the transfer of data octets. |  | 31 |  |
|  |  | 32 |  |
| 11.6.2.2.3 PHY-TXEND.request |  | 33 |  |
|  |  | 34 |  |
| 6.7.2.2.3.1 Function |  | 35 |  |
|  |  | 36 |  |
| This primitive is a request by the MAC sublayer to the local PHY entity that the current transmission of the |  | 37 |  |
|  |  | 38 |  |
| MPDU be completed. |  | 39 |  |
| 6.7.2.2.3.2 Semantics of the service primitive |  | 40 |  |
|  |  | 41 |  |
| The semantics of the primitive are as follows: |  | 42 |  |
|  |  | 43 |  |
| PHY-TXEND.request |  | 44 |  |
|  |  | 45 |  |
| This primitive has no parameters. |  | 46 |  |
|  |  | 47 |  |
| 6.7.2.2.3.3 When generated |  | 48 |  |
|  |  | 49 |  |
| This primitive will be generated when the MAC sublayer has received the last PHY-DATA.confirm from the |  | 50 |  |
|  |  | 51 |  |
| local PHY entity for the MPDU currently being transferred. |  | 52 |  |
| 6.7.2.2.3.4 Effect of receipt |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 365 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- The effect of receipt of this primitive by the local PHY entity will be to stop the transmission process upon

- transmission of the last octet transferred from the MAC layer.

| 3 | 11.6.2.2.4 PHY-TXEND.confirm |  |
| --- | --- | --- |
| 4 |  |  |
| 5 | 6.7.2.2.4.1 Function |  |
| 6 |  |  |
| 7 |  |  |

- This primitive is issued by the PHY to the local MAC entity to confirm the completion of a transmission.

- The PHY issues this primitive in response to every PHY-TXEND.request primitive issued by the MAC sub-

- layer.

| 11 | 6.7.2.2.4.2 Semantics of the service primitive |  |
| --- | --- | --- |
| 12 |  |  |
| 13 | The semantics of the primitive are as follows: |  |
| 14 |  |  |
| 15 | PHY-TXEND.confirm |  |
| 16 |  |  |
| 17 | This primitive has no parameters. |  |
| 18 |  |  |
| 19 | 6.7.2.2.4.3 When generated |  |
| 20 |  |  |
| 21 |  |  |

- This primitive will be issued by the PHY to the MAC entity when the PHY has received a PHY-

- TXEND.request immediately after transmitting the end of the last bit of the last data octet indicating that the

- symbol containing the last data octet has been transferred.

25

26	6.7.2.2.4.4 Effect of receipt

27

- The receipt of this primitive by the MAC entity provides a possible time reference for a contention backoff

- protocol.

| 30 | 11.6.2.2.5 PHY-RXSTART.indicate |  |
| --- | --- | --- |
| 31 |  |  |
| 32 | 6.7.2.2.5.1 Function |  |
| 33 |  |  |
| 34 |  |  |

- This primitive is an indication by the PHY to the local MAC entity that the PHY layer has received a valid

- packet synchronization sequence and PHY header.

| 37 | 6.7.2.2.5.2 Semantics of the service primitive |  |
| --- | --- | --- |
| 38 |  |  |
| 39 | The primitive provides the following parameter: |  |
| 40 |  |  |
| 41 | PHY-RXSTART.indicate (RXVECTOR) |  |
| 42 |  |  |
| 43 |  |  |

- The RXVECTOR represents a list of parameters that the PHY provides the local MAC entity upon receipt of

- a valid PHY header. This vector may contain both MAC and MAC management parameters. The required

- parameters are listed in Table 6.7.2.1.3.

47

48	6.7.2.2.5.3 When generated

49

- This primitive is generated by the local PHY entity to the MAC sublayer when the PHY has successfully

- validated the PHY header at the start of a new PPDU.

52

53	6.7.2.2.5.4 Effect of receipt

54

366	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| The effect of receipt of this primitive by the MAC is unspecified. |  | 1 |  |
| --- | --- | --- | --- |
| 11.6.2.2.6 PHY-RXEND.indicate |  | 2 |  |
|  |  | 3 |  |
| 6.7.2.2.6.1 Function |  | 4 |  |
|  |  | 5 |  |
| This primitive is an indication by the PHY to the local MAC entity that the MPDU currently being received |  | 6 |  |
|  |  | 7 |  |
| is complete. |  | 8 |  |
| 6.7.2.2.6.2 Semantics of the service primitive |  | 9 |  |
|  |  | 10 |  |
| The primitive provides the following parameter: |  | 11 |  |
|  |  | 12 |  |
| PHY-RXEND.indicate (RXERROR) |  | 13 |  |
|  |  | 14 |  |
| The RXERROR parameter can convey one or more of the following values: NoError, | FormatViolation, or | 15 |  |
|  |  | 16 |  |
| UnsupportedRate. A number of error conditions may occur after the PHY layer protocol has detected what |  | 17 |  |
| appears to be a valid preamble and header. The following describes the parameter returned for each of those |  | 18 |  |
| error conditions. |  | 19 |  |
| - NoError. This value is used to indicate that no error occurred during the receive process at the |  | 20 |  |
|  |  | 21 |  |
| PHY layer. |  | 22 |  |
| - FormatViolation. This value is used to indicate that the format of the received PPDU was in error. |  | 23 |  |
|  |  | 24 |  |
| - UnsupportedRate. This value is used to indicate that during the reception of the incoming PPDU, |  | 25 |  |
|  |  | 26 |  |
| a nonsupported date rate was detected. |  | 27 |  |
| 6.7.2.2.6.3 When generated |  | 28 |  |
|  |  | 29 |  |
| This primitive is generated by the PHY for the local MAC entity to indicate that the receive state machine |  | 30 |  |
|  |  | 31 |  |
| has completed a reception with or without errors. In the case of an RXERROR value of NoError, the MAC |  | 32 |  |
| can use the PHY-RXEND.indicate as reference for channel access timing. |  | 33 |  |
| 6.7.2.2.6.4 Effect of receipt |  | 34 |  |
|  |  | 35 |  |
| The effect of receipt of this primitive is for the MAC to begin inter-frame space processing if applicable. |  | 36 |  |
|  |  | 37 |  |
| 11.6.2.2.7 PHY-SET-LOADING.request |  | 38 |  |
|  |  | 39 |  |
| 6.7.2.2.7.1 Function |  | 40 |  |
|  |  | 41 |  |
| This primitive is a request by the MAC sublayer to the local PHY entity to update the bit loading scheme |  | 42 |  |
|  |  | 43 |  |
| used for adaptive bit and energy loading. |  | 44 |  |
| 6.7.2.2.7.2 Semantics of the service primitive |  | 45 |  |
|  |  | 46 |  |
| The primitive provides the following parameters: |  | 47 |  |
|  |  | 48 |  |
| PHY-SET-LOADING.request(BIT_LOADING_VECTOR) |  | 49 |  |
|  |  | 50 |  |
| The BIT_LOADING_VECTOR represents a list of parameters that the MAC sublayer provides to the local |  | 51 |  |
|  |  | 52 |  |
| PHY entity in order to set the energy and the constellation size on each data-bearing subcarrier in a SISO or |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 367 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- MIMO configuration. This vector contains PHY management parameters. The required PHY parameters

- are listed in Table 6.7.2.1.3.

3

4	6.7.2.2.7.3 When generated

5

- This primitive will be issued by the MAC sublayer to the PHY entity when the MAC sublayer needs to

- update the bit and energy allocation scheme.

8

9	6.7.2.2.7.4 Effect of receipt

10

- The effect of receipt of this primitive by the PHY entity will be to update the bit and energy allocation

- scheme accordingly.

| 13 | 11.6.2.2.8 PHY-SET-LOADING.confirm |  |
| --- | --- | --- |
| 14 |  |  |
| 15 | 6.7.2.2.8.1 Function |  |
| 16 |  |  |
| 17 |  |  |

- This primitive is issued by the PHY to the local MAC entity to confirm the update of the bit and energy allo-

- cation scheme.

| 20 | 6.7.2.2.8.2 Semantics of the service primitive |  |
| --- | --- | --- |
| 21 |  |  |
| 22 | The semantics of the primitive are as follows: |  |
| 23 |  |  |
| 24 | PHY-SET-LOADING.confirm |  |
| 25 |  |  |
| 26 | This primitive has no parameters. |  |
| 27 |  |  |
| 28 | 6.7.2.2.8.3 When generated |  |
| 29 |  |  |
| 30 |  |  |

- This primitive will be issued by the PHY to the MAC entity when the PHY has updated the bit and energy

- allocation scheme following the PHY-SET-LOADING.request by the MAC.

| 33 | 6.7.2.2.8.4 Effect of receipt |  |
| --- | --- | --- |
| 34 |  |  |
| 35 | The receipt of this primitive by the MAC will cause the MAC to start the next MAC entity request. |  |
| 36 |  |  |
| 37 | 11.6.2.2.9 PHY-PROVIDE-CSI.indicate |  |
| 38 |  |  |
| 39 | 6.7.2.2.9.1 Function |  |
| 40 |  |  |
| 41 |  |  |

- This primitive is an indication by the PHY to the local MAC entity that the PHY layer has received a request

- to estimate the channel state information and has successfully completed the task.

| 44 | 6.7.2.2.9.2 Semantics of the service primitive |  |
| --- | --- | --- |
| 45 |  |  |
| 46 | The primitive provides the following parameter: |  |
| 47 |  |  |
| 48 | PHY-PROVIDE-CSI.indicate (CSI_VECTOR) |  |
| 49 |  |  |
| 50 |  |  |

- The CSI_VECTOR represents a list of parameters that the PHY provides the local MAC entity upon receipt

- of a valid PHY header and successful estimation of the CSI. The required parameters are listed in Table

- 6.7.2.1.3.

54

368	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.7.2.2.9.3 When generated

This primitive is generated by the local PHY entity to the MAC sublayer when the PHY has successfully validated the PHY header at the start of a new PPDU and estimated the CSI.

6.7.2.2.9.4 Effect of receipt

The effect of receipt of this primitive by the MAC is unspecified.

12. PHY VIII High-bandwidth PHY

12.1 Adaptive OFDM

The idea of the adaptive OFDM physical layer is shown in Figure 195. At the transmitter (Tx), the input data are transported via orthogonal subcarriers. Each data symbol carrying one or more bits is mapped onto a constellation point, according to a variable modulation format for each subcarrier. A Hermetian symmetry operation is then performed to create a real-valued waveform. An OFDM symbol is generated by feeding symbols in the frequency domain into the inverse fast Fourier transform (IFFT) followed by the insertion of a cyclic prefix (CP). The output of the OFDM signal may then be clipped in the digital domain and passed through the digital-to-analog converter (DAC) and low-pass filter (LPF). If any pulse shaping is employed, then it is recommended to perform clipping after the pulse shaping operation. A bias is normally added to ensure a unipolar all positive signal before it is used for intensity modulation of the optical source (i.e., light emitting diode (LED) or a laser diode (LD)).

Figure 195—Overview of the standalone link (one link direction)

| Copyright © 20xx IEEE. All rights reserved. | 369 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- Following conversion from optical to electrical signal and signal detection, the inverse operations are per-

- formed at the receiver (Rx), where a frequency-domain equalizer (FDE) is used to reconstruct the received

- constellation points on each sub-carrier, after passing them through the OWC channel. The desired mapping

- of information bits onto the sub-carriers is sent by the receiver to the transmitter over the reverse link. In the

5	example shown in Figure 195, the metrics reporting is carrying a so-called noise-enhancement vector. A

- power- and bit-loading algorithm determines the power and modulation formats for the data transport on

- each used subcarrier. The loading algorithm typically maximizes the throughput assuming a fixed power

- budget while satisfying a predefined bit error rate (BER) before forward error correction (FEC).

| 9 | 12.1.1 Adaptive OFDM Waveform |  |
| --- | --- | --- |
| 10 |  |  |
| 11 | The PHY uses the adaptive OFDM waveform in both link directions with following extensions: |  |
| 12 |  |  |
| 13 | - The waveform is always non-negative and real-valued. |  |
| 14 |  |  |
| 15 | - A bias is added and clipping (if needed) is implemented in the digital domain. |  |
| 16 |  |  |
| 17 | - Optional single-carrier or unipolar modulation to improve power efficiency and support dimming. |  |
| 18 |  |  |
| 19 |  |  |

- OFDM signal generation is shown in Figure 196. A block of 2N data symbols is transmitted. An optional

- pre-coding is used for the generation of singlecarrier or unipolar modulation schemes based on OFDM to

- improve power efficiency and support dimming. The signal is then passed through a carrier mapping unit,

- used for precoding and Hermitian symmetry. Next, the IFFT is performed, the CP is added and controlled

- clipping is performed in the digital domain.

| 25 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 26 |  |  |  |  |  |  |
| 27 | Precoding | Carrier | IFFT | CP | controlled |  |
| 28 |  |  |  |  |  |  |
|  | (optional) | Mapping |  | insertion | clipping |  |
| 29 |  |  |  |  |  |  |
| 30 |  | Figure 196—OFDM signal generation |  |  |  |  |
| 31 |  |  |  |  |  |  |
| 32 | 12.1.1.1 Carrier mapping |  |  |  |  |  |
| 33 |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  |

- Carrier mapping is performed as illustrated in Figure 197. Note that the subcarrier x0 may be used to add a

- constant bias signal to the output signal. In order to create a real-valued waveform, only half of the subcarri-

- ers are used, while conjugate symmetry is enforced as

| 38 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 39 | * |  |  |  |
| 40 | x2N – i = xi | , i = 1, 2, | , N – 1 | (23) |
| 41 |  |  |  |  |

- where the star indicates complex conjugation. The resulting discrete multi-tone (DMT) signal is real-valued,

- even if symbols xi are complex.

| 44 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45 |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 46 |  |  | x |  |  | conjugate |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |
| 47 |  |  | x10 |  |  |  |  |  |  |  |  | x | 1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |
| 48 | s/p |  | x N-1 |  |  | symmetry |  |  |  |  |  | xN-1 |  |  |  |  |  |  | IFFT |  | p/s |  |
| 49 |  |  |  |  |  | x2N-i = xi* |  |  |  |  |  | xN |  |  |  |  |  |  |  |  |  |  |
| 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | i=1,2,...N-1 |  |  |  |  |  | x2N2N-1 |  |  |  |  |  |  |  |  |  |  |
| 51 |  |  | Figure 197—Carrier mapping for standalone link |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 54 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 370 |  |  |  |  | Copyright © 20xx IEEE. All rights reserved. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

12.1.1.2 IFFT

The time-domain signal X(k) is given by

|  |  | 2N – 1 |  |  | ik |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 |  |  |  | j2------ |  |  |
| X  k = |  |  | x | e | 2N | (24) |  |
|  | ------ |  |  |  |  |  |  |
|  | 2N |  | i |  |  |  |  |

i = 0

where i denotes the sample index, xi the complex-valued baseband signals in the frequency domain and 2N the block size of the IFFT.

12.1.1.3 Cyclic prefix

At the output of the IFFT, in the serial block of 2N samples, the last CP samples are copied as a sub- block being repeated and appended at the beginning of the block of samples, see Figure 198. By adding the CP at the transmitter, and removing it at the receiver, the multipath channel matrix can be transformed from Toeplitz- shape into a circulant shape, which allows the use of IFFT at the transmitter and FFT at the receiver to obtain a diagonal channel in the frequency domain, so that a simple single-tap frequency-domain equal-izer (FDE) can be used.

| last CP |  |  |  |  |  | last CP |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| samples | 2N samples |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | samples |  |  |  |  |  |  |  |  |  |  |  |  |  |
| repeated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 198—Cyclic prefix insertion

12.1.2 Single-carrier modulation

Optional pre-coding before the OFDM modulator can be used to reduce the probability of clipping and enhance power efficiency while sacrificing no or minor spectral efficiency [1, 2].

For single-carrier (SC) transmission, "outer" pre-coding, together with an "inner" OFDM transmitter is used to emulate SC transmission inside the OFDM concept. The novel schemes require little more advanced sig-nal processing, and the same minor increase of sophistication can be expected at the receiver, i.e. decoding is straightforward. The schemes are shown in principle in Figure 199. Details can be found in [1, 2].

| Copyright © 20xx IEEE. All rights reserved. | 371 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | A. |  | M data |  |  |  | M- |  | Map M |  |  |  |  |  | CS | 2N- | CP |  |
| 3 |  |  | symbols |  |  | DFT |  |  | to 2N |  |  |  |  |  |  | IDFT |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | B. |  | M data |  |  |  | M- |  | Map M | repeat | frequency- |  |  |  | CS | 2N- | CP |  |
| 8 |  |  | symbols |  |  | DFT |  |  | to 2N | signals | domain filter |  |  |  |  | IDFT |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | C. |  | M data |  |  |  | M- |  | Map M | repeat | frequency- | 2N- | MSK | 2N- | CS | 2N- | CP |  |
|  |  |  | symbols |  |  | DFT |  |  | to 2N | signals | domain filter | IDFT | modulator | DFT |  | IDFT |  |  |
| 13 | Figure 199—Single-carrier modulation schemes used to improve the power efficiency of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | the OFDM transmitter. Top: Pure DFT precoding uses a roll-off factor = 0. Center: A root- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | raised-cosine filter can be added in the frequency domain, to realize > 0. Bottom: A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | Gaussian filter can be used in the same way and a minimum-shift keying (MSK) modulation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | can be added in the time domain. In this way, the classical GMSK waveform can be realized |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  |  | inside an OFDM system. |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 | 12.1.2.1 DFT-pre-coded OFDM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 | The simplest SC transmitter is shown in Figure 199 row A. First, the symbol sequence is passed through the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N-DFT and then mapped directly onto the desired frequency sub-band after using a cyclic shift (CS) so that |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | the DC signal is in the center. Finally, the precoded sequence is passed through the M-IDFT and the cyclic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | prefix (CP) is added. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | As shown in [2], this procedure yields a SC signal having a roll-off factor α=0. The rectangular filtering |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | causes "ringing" in the time domain which increases the peak-to-average power ratio (PAPR). As this is a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | special case of the RRC-filtered SC transmission, details are described in the next subsection. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 | 12.1.2.2 RRC-filtered single-carrier modulation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34 | In the middle row in Figure 199, an additional root-raised-cosine filter is introduced in the frequency domain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | where α≥0. In order to realize filtering in the frequency domain, oversampling is emulated by repeating the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | DFT output block in the frequency domain. Afterwards, the root-raised cosine (RRC) filter is applied in the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | frequency domain. The sequence is then mapped directly onto the desired frequency sub-band using a cyclic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | shift (CS) so that the DC signal is in the center. In the following, these steps are described in detail. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 | A data sequence a(n) of length M is used where n = 1, 2, …, M. The sequence is up-sampled by factor F as |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | follows |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 43 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 44 | b k = |  |  |  | a n, if k=FN |  |  |  |  |  |  |  |  |  |  |  | (25) |  |
| 45 |  |  |  |  |  |  | 0, else |  |  |  |  |  |  |  |  |  |  |  |
| 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 47 | with | F | = | 2N | – | 0.5 |  | , where k = 1, 2, …, FM and 2N is the number of samples in the final waveform w/o the |  |  |  |  |  |  |  |  |  |  |
| 48 |  |  |  | M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ----- |  |  |  |  | is used here to indicate that z is rounded to the nearest integer less than or equal to z. |  |  |  |  |  |  |  |  |  |
| 49 | CP. The notation |  |  |  |  |  |  | z |  |  |  |  |  |  |  |  |  |  |
|  | Note that F-times up-sampling followed by M-DFT is equivalent to M-DFT and subsequent spectral repeti- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | tion, provided that the ratio 2N/M is an integer. The proof is given in [B20]. Accordingly, up-sampling and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 51 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2N-DFT can be replaced by M-DFT and repeating the output signal in the frequency domain. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 54 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

372	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Next step is a flexible frequency-domain filter. It is implemented so that bandwidth can be easily changed as a function of the block size M. Therefore, a vector is defined with running index s = [-M, …, M] the bell-shape part of the filter is computed as

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | s | i |  |  |  |  |  | – 1 – M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ---- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G | l | = | 0.5 |  |  |  |  | 1 + cos |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (26) |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ------------------------------------------ |  |  |  |  |  |  |  |  |  | M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where l = 1, 2, …2M+1. The filter is transparent in the range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a = |  |  | M + 1 – |  |  |  |  |  |  |  |  |  1 – M |  |  |  |  |  |  |  |  |  |  |  | , , M + 1 + |  |  |  |  |  |  |  |  |  |  1 – M |  |  |  |  |  | (27) |  |
|  |  |  |  |  |  |  |  |  |  |  | ----------------------2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ---------------------- |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |
| There are two regions where the filter attenuates totally. They are given by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b = |  |  | 1, , |  |  |  |  | M + 1 – |  |  |  |  |  |  |  |  1 + M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (28) |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | ---------------------- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  1 + M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| c = |  |  | M + 1 |  |  |  |  | + |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | , , 2M + 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (29) |  |
|  |  |  |  |  |  |  |  |  |  |  | ---------------------- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Gl is now set as Ga = 1, Gb = 0 and Gc = 0 in the respective regions indicated by vectors a, b and c . Note that up-conversion is equivalent to performing sequentially 2N-DFT of the time-domain sequence, a cyclic shift by Ncenter and 2N-IDFT of the shifted signal, as shown in [2].

In row B. Figure 199, the synthesis of filtered QAM is summarized in the frequency domain. First the data symbol sequence is passed through the M-DFT and the output is repeated in the frequency domain. Next, the signal is filtered in the frequency domain and the cyclic shift is applied to up-convert the signal to the desired center sub-carrier Ncenter. Finally, the signal is passed through the 2N-IDFT and the cyclic prefix is added [2].

Note that in the SC transmitter, carrier mapping has been modified compared to LTE. In this way, wave-forms become comparable to time-domain single-carrier signal, see [2]. The new mapping is sketched in Figure 200.

| Copyright © 20xx IEEE. All rights reserved. | 373 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

|  | IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC |  |  |  |  |  |  |  | IEEE Draft P802.15.13/D0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | DC |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  | A |  | B | DFT output signal |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |
| 7 | 1 | ... | M/2 | ... M |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  | A |  |  |  |  |  |  |  | B |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |
| 11 | 1 ... |  | M/2 |  |  |  |  |  | 2N- M/2 +1...2N |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | A |  | B |  |  |  |  | A | B |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  | cyclic shift (CS) |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  | A | B | A |  | B |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  | Figure 200—Generation of filtered single-carrier signals |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |  |  |  |  |
| 22 | The direct current (DC) sub-carrier of the M-DFT output vector (having index 1) is first mapped onto the |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  |  |  |  |  |
|  | DC sub-carrier of the 2N-IDFT. The two blocks |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |  |  |  |  |
| 26 | A = | 2, , | M | and | B = | M | + 1, | , M |  | (30) |  |
| 27 |  |  | ---- |  |  | ---- |  |  |  |  |  |
|  |  |  | 2 |  |  | 2 |  |  |  |  |  |
| 28 |  |  |  |  |  |  |  |  |  |  |  |
| 29 | are then mapped onto the first and last sub-carriers, see Figure 200. Periodic replica are added in the fre- |  |  |  |  |  |  |  |  |  |  |
| 30 | quency domain to emulate up-sampling. Finally, the frequency-domain filter is applied and the cyclic shift is |  |  |  |  |  |  |  |  |  |  |
| 31 | used to modulate the signal onto the center subcarrier. |  |  |  |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  |  |  |  |  |  |
| 33 | 12.1.2.3 Gaussian minimum shift keying |  |  |  |  |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  |  |  |  |  |  |
| 35 | GMSK is known for zero PAPR in radio links. Although this goal is not reached in OWC links, due to the |  |  |  |  |  |  |  |  |  |  |
| 36 | real-valued waveform, GMSK offers ultra- robust signaling in case of very low SINR. First, the classical |  |  |  |  |  |  |  |  |  |  |
| 37 | time-domain GMSK single-carrier transmitter is reviewed. The serial data symbol sequence a(n) is up-sam- |  |  |  |  |  |  |  |  |  |  |
| 38 | pled as in subsection 2.2.2.5.2 yielding b(k). After applying a Gaussian filter in the time domain, the filtered |  |  |  |  |  |  |  |  |  |  |
| 39 | signal c(k) is obtained. The classical Gaussian filter is approximated in the time domain using a finite |  |  |  |  |  |  |  |  |  |  |
| 40 | impulse response (FIR) with some memory. Next, c(k) is passed into a minimum shift keying (MSK) modu- |  |  |  |  |  |  |  |  |  |  |
| 41 | lator where it is first accumulated yielding the phase |  |  |  |  |  |  |  |  |  |  |
| 42 |  |  |  |  |  |  |  |  |  |  |  |
| 43 |  k | =  k – 1 |  |  |  |  |  |  |  | (31) |  |
| 44 |  |  |  | + ------c k – 1 |  |  |  |  |  |  |  |
| 45 |  |  |  | 2F |  |  |  |  |  |  |  |
| 46 | and then inserted into the complex amplitude |  |  |  |  |  |  |  |  |  |  |
| 47 |  |  |  |  |  |  |  |  |  |  |  |
| 48 | x k | = I + jQ = cos k + j sin k |  |  |  |  |  |  |  | (32) |  |
| 49 |  |  |  |  |  |  |  |  |  |  |  |

- Note that in-phase signal I and the quadrature signal Q in are fed by the same phase but at a shift of 90°

- yielding single side band (SSB) modulation when up-converting the sequence to the desired center fre-

- quency. This is often performed using an analog IQ modulator. The same SSB up-conversion can be reached

- by means of digital signal processing. Therefore, the complex-valued GMSK baseband signal is multiplied

374	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

sample-by-sample with a digitally synthesized complex-valued oscillation due to single OFDM sub-carrier, being the center frequency of the desired GMSK-modulated signal. Finally, a window of length M is applied in the time domain.

The equivalent processing for GMSK using OFDM is summarized on row C. in Figure 199. As before, the data sequence a(n) is fed into the M-DFT and up-sampling is emulated by repeating the output signal in the frequency domain. Next, a Gaussian filter is applied in the frequency domain. A vector is created with run-ning index s = [-R, …, R] where R≤M, the filter is computed as

|  | 2 | 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| – |  | sn |  |  |  | 1 |  |  |
|  |  |  |  |  | ln 2 |  |  |  |
| Gn = e |  |  | where |  = | ------------2 | MBT------------ | (33) |  |

where n = 1, 2, ..., 2R +1 and BT is the bandwidth-time product. BT=0.3 is recommended. GMSK is a non-linear SSB phase modulation. Thus, the two functions of accumulating the signal and generating the in-phase and quadrature signals are better realized in the time domain. The main idea is to insert the GMSK modulator after frequency -domain filtering, but in the time domain. Using M-IDFT of the filtered data sequence, c(k) is obtained. Next, c(k) is normalized to unit peak amplitude and feed it into the time-domain MSK modulator described above. Then up-conversion is applied. It can be equivalently implemented as shown in row C in Figure 199 or in the time-domain as described above. Finally, the CP is added.

GMSK causes adjacent channel interference since SSB phase modulation is a non-linear process. Even if the GMSK modulator input is confined in the frequency domain, four-wave mixing between in-band sub-carri-ers creates out-of-band interference. Such interference can be cut using an optional post-modulation filter in the frequency domain attenuating totally outside the range s=[-R, ...,R] and correct the power, accordingly.

12.1.2.4 From complex- to real-valued single-carrier transmission

The above waveforms yield complex-valued sequences. Same as in the adaptive OFDM approach, the com-plex-valued waveform covers only the first N subcarriers and then Hermetian symmetry is needed to gener-ate a real-valued waveform. Conjugate symmetry is obtained as

x2N – i = xi ,	i = 1, 2, , N – 1	(34)

The resulting discrete multi-tone (DMT) signal is real-valued, even if symbols xn are complex.

12.1.3 Unipolar modulation

To modulate the intensity of light, the signal needs to be real-valued and non-negative. For adaptive OFDM, besides the Hermitean symmetry operation described in subsection 3.2.2.1, a DC bias has to be added, which results in a reduced power efficiency.

12.1.3.1 ACO-OFDM

One way to overcome this disadvantage is asymmetrically-clipped optical (ACO) OFDM as a first unipolar modulation scheme to improve the power efficiency [F-5].

For ACO-OFDM, the time domain signal is made unipolar by simply clipping the negative part at the zero level, which does not need a large DC bias. That is

| xclipped k | = |  | x k if |  x k 0 | (35) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | else |  |  |
|  |  |  |  |  |  |  |

| Copyright © 20xx IEEE. All rights reserved. | 375 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- where x (k) is the bipolar real-valued OFDM signal after the IFFT and xclipped(k) the unipolar signal after

- clipping.

3

- If only the odd subcarriers are modulated by signals, the effect of clipping is that inter-carrier interference

- (ICI) is created that falls only on the even subcarriers. The effect of clipping on the odd subcarriers is simply

- a multiplication of these components by a constant 0.5. I.e., clipping does not result in inter-carrier interfer-

- ence (ICI) on the odd subcarriers, if the even subcarriers remain vacant. The diagram of ACO-OFDM mod-

- ulation is shown in Figure 201.

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Figure 201—ACO-OFDM modulation

12.1.3.2 eU-OFDM

A real-valued OFDM signal, generated at the PHY layer, is used to modulate the light. The modulation is conducted only within the active operational range of the device. In this range, the electrical signal and the light output signal cannot be negative at all times. The conventional approach is to set a positive operating point, around which the bipolar OFDM signal can be realized. Figure 202 (left) illustrates this principle. The positive bias can be introduced as part of the analog front-end (in the case of AC-coupled LED drivers) or as part of the information signal (in case of DC-coupled drivers). This approach is known as DC-biased optical OFDM (DCO-OFDM).

Enhanced unipolar OFDM (eU-OFDM) is an optional alternative modulation scheme. It turns the bipolar OFDM signal into a strictly unipolar information signal without the addition of an energy intensive DC com-ponent that carries no additional information, see Figure 202 (right).

Figure 202—Left: Frame start using DC-biased optical OFDM (DCO-OFDM). Right: Same using Enhanced Unipolar OFDM (eU-OFDM)

12.1.3.3 Enhanced unipolar OFDM signal generation

The eU-OFDM signals are generated in layers, indicated by the variable STR.

For one stream, i.e. STR = '00', two consecutive copies of every OFDM symbol are created. The polarity of the samples in the second copy is inverted, and finally, all negative samples in the resulting time-domain double-symbol are set to zero. Any time-domain oversampling and pulse shaping is done after the removal

376	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

of negative samples. The resulting positive signal is used to modulate the transmitter. The concept is illus-trated in Figure 203.

Figure 203—Unipolar OFDM signal generation (one stream).

For two streams, STR = '10', every three OFDM symbols are grouped into one eU-OFDM block, where the first two symbols are assigned to stream 1 (St1) and the remaining single symbol is assigned to stream 2 (St2).

The first two symbols in St1 are modulated using the algorithm described for STR='00' and shown in Figure 203. The single symbol in St2 is modulated in a similar manner, but instead of two copies, four con-secutive copies are created for the OFDM symbol in St2, where the first two copies are kept unchanged, while the polarity of the samples in the next two copies is inverted.

Following this procedure, all negative samples in both St1 and St2 are removed, and the two signals are summed up. Any time -domain oversampling and pulse shaping is done after the removal of the negative samples. The resulting positive signal can be used to modulate the transmitter.

For three streams, i.e. STR = '01', every seven OFDM symbols are grouped into an eU-OFDM block, where the first four symbols are assigned to data stream 1 (St1), the next 2 symbols are assigned to data stream 2 (St2) and the last symbol is assigned to data stream 3 (St3).

The four symbols in St1 and the two symbols in St2 are modulated using the algorithm described for two streams, see 4.2.4.2.1.2. The symbol in St3 is modulated in a similar manner, however, eight consecutive copies of that symbol are generated, where the first four copies are left unchanged, while the polarity of the samples in the following four copies is reversed.

| Copyright © 20xx IEEE. All rights reserved. | 377 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- Following this procedure, all negative samples in St1, St2 and St3 are removed. Any time-domain oversam-

- pling and pulse shaping is done after the removal of the negative samples. The signals in the three streams

- are summed up and the resulting positive signal can be used to modulate the transmitter.

4

- For four streams, i.e. STR = '11', every fifteen OFDM symbols are grouped into an eU-OFDM block, where

- the first eight symbols are assigned to data stream 1 (St1), the next four symbols are assigned to data stream

- 2 (St2), the following two symbols are assigned to data stream 3 (St3) and the last symbol to data stream 4

- (St4).

9

- In the first stream, two consecutive copies of every OFDM symbol are transmitted, where the second copy is

- multiplied by -1. In the second stream, four consecutive copies of every OFDM symbol are transmitted,

- where the first two copies are transmitted in their original format, while the signs of the third and the fourth

- copy are multiplied by -1. In the third stream, eight consecutive copies of every OFDM symbol are transmit-

- ted, where the first four copies are conveyed in their original format, while the signs of last four copies are

- multiplied by -1. In the fourth stream, sixteen consecutive copies of the single OFDM symbol are transmit-

- ted, where the first eight copies are used in their original format, while the last eight copies are are multiplied

- by -1.

18

- Finally, all negative samples in the four streams are removed and the signals from the four streams are

- summed up. Any oversampling and pulse shaping is done after the removal of the negative samples. The

- resulting positive signal can be used to modulate the transmitter.

22

23	12.1.4 Multiple-input multiple-output

24

- The use of several multiple-input multiple-output (MIMO) schemes is foreseen in order to support diversity

- and spatial multiplexing, which can both be realized both as non-imaging and imaging MIMO [F-6, F-7].

- The performance of wavelength-division multiplexing (WDM) and wavelength-shift keying (WSK) trans-

- mission can also be improved by using MIMO transmission schemes.

29

30	12.1.4.1 Signal model

31

- For two transmitters and two receivers, the signal model for non-imaging MIMO transmission on each sub-

- carrier can be expressed as

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

| y = | y1 | = Hx + n = | H11 | H12 |  | x1 | + | n1 | (36) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | y2 |  | H21 | H22 |  | x2 |  | n2 |  |

where, in general, bold upper case letters describe matrices and bold lower case letters describe vectors. The received signals are denoted as yi where i=1…n Rx and nRx is the number of receivers. The transmitted sig-nals are described as xj where j=1…nTx and nTx is the number of transmitters. The channel matrix elements Hij represent the channel gain from the jth transmitter to the ith receiver.

12.1.4.2 Non-imaging MIMO

In case of non-imaging MIMO, see Figure 204, light from each of all transmitters (usually having a wide beam) is received by all receivers (usually having wide field -of-views, FOV) , but channels may have differ-ent path loss and impulse response as shown in Figure 6. In case of non-imaging MIMO, the channel matrix H is usually fully occupied and the elements Hij may depend on the subcarrier index n because of the fre-quency response of optoelectronic components and the superposition of LOS and NLOS propagation effects.

378	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure 204—Non-imaging MIMO transmission

12.1.4.3 Imaging MIMO

Imaging MIMO uses imaging optics between the transmitters and receivers, which are usually arranged in arrays. Because of the lens, the LOS component is often dominant while the diffuse reflections are reduced due to the reduced FOV at the receiver. Ideally, the transmitter array is imaged 1:1 onto the receiver array. In this case, the channel crosstalk can be neglected, and the channel matrix H has a diagonal shape, i.e. can be simplified and becomes diagonal, i.e.

|  |  |  | Hii | if i=1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hij | = |  |  |  | (37) |  |
|  |  |  | 0 | else |  |  |
|  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |

Figure 205—Imaging MIMO transmission

12.1.4.4 Polarization-division multiplexing

Lights emitting from incoherent LEDs include all polarization directions that can be decomposed as two orthogonal bases, e.g. of horizontal (h) and vertical (v) polarization. For LED-based polarization-division multiplexing (PDM), a linear h-polarizer is inserted that can only allow h-polarized components passing through at the first transmitter (TX1); meanwhile, a linear v-polarizer is used at TX2. Note that many laser diodes yield linearly polarized light so that the transmitter-sided polarizers may not be needed when using lasers. After passing through Tx- sides polarizers, the polarized light is mixed up after free-space propaga-tion. At the receiver (RX), two polarizers are then implemented to filter out the unwanted polarized lights, thus obtaining the transmitting signals.

| Copyright © 20xx IEEE. All rights reserved. | 379 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 |  |  |
| --- | --- | --- |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 13 |  |  |
| 14 |  |  |
| 15 |  |  |
| 16 |  |  |
| 17 | Figure 206—Polarization-division multiplexing |  |
| 18 |  |  |
| 19 | 12.1.4.5 Wavelength-division multiplexing |  |
| 20 |  |  |
| 21 | t.b.d. |  |
| 22 |  |  |
| 23 | 12.1.4.6 Link setup for MIMO |  |
| 24 |  |  |
| 25 |  |  |

- Initial link setup and header transmission detection are performed in the single-input single-output (SISO)

- mode, in order to improve the reliability of transmission. Transmission of the preamble and the header is

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

done using all transmitters, while detection can be improved by using maximum ratio combining (MRC) based on individual estimates of the superimposed channels from all transmitters at each receiver hi  njTx1 Hij . The number of used transmitters is included in the header.

12.1.4.7 Additional channel estimation symbols for MIMO

Additional channel estimation (ACE) symbols are sent in the beginning of the data field in the PHY frame. They are defined in the frequency domain and over multiple OFDM symbols. Each ACE symbol contains the same sequence of bits on all subcarriers that is only passed through the constellation scrambler. For MIMO, the sequence {sn} contains all 1s, but is multiplied as a whole with a sign taken out of an orthogonal sequence. For 1 transmitter (Tx), the channel estimation symbol in the preamble is used

1 Tx:  [{sn}]

For 2 Txs, the first symbol and one ACE signal are sent as

2 Txs:

Tx1 [{sn} {sn}],

Tx2 [{sn} {-sn}].

380	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| where the first symbol is always contained in the header and only one additional symbol is sent for ACE. |  | 1 |  |
| --- | --- | --- | --- |
| The scheme in G.9963 obviously makes use of the Hadamard sequences. It can be easily extended to 4 and 8 |  | 2 |  |
| transmitters |  | 3 |  |
| 4 Txs: |  | 4 |  |
|  |  | 5 |  |
| Tx1 [{sn} {sn} {sn} {sn}] |  | 6 |  |
|  |  | 7 |  |
|  |  | 8 |  |
| Tx2 [{sn} {-sn} {sn} {-sn}], |  | 9 |  |
|  |  | 10 |  |
| Tx3 [{sn} {sn} {-sn} {-sn}], |  | 11 |  |
|  |  | 12 |  |
| Tx4 [{sn} {-sn} {-sn} {sn}]. |  | 13 |  |
|  |  | 14 |  |
| 8 Txs: |  | 15 |  |
|  |  | 16 |  |
| Tx1 [{sn} {sn} {sn} {sn}{sn} {sn} {sn} {sn}] |  | 17 |  |
|  |  | 18 |  |
| Tx2 [{sn} {-sn} {sn} {-sn}{sn} {-sn} {sn} {-sn}], |  | 19 |  |
|  |  | 20 |  |
| Tx3 [{sn} {sn} {-sn} {-sn}{sn} {sn} {-sn} {-sn}], |  | 21 |  |
|  |  | 22 |  |
| Tx4 [{sn} {-sn} {-sn} {sn}{sn} {-sn} {-sn} {sn}]. |  | 23 |  |
|  |  | 24 |  |
| Tx5 [{sn} {sn} {sn} {sn}{-sn} {-sn} {-sn} {-sn}] |  | 25 |  |
|  |  | 26 |  |
| Tx6 [{sn} {-sn} {sn} {-sn}{-sn} {sn} {-sn} {sn}], |  | 27 |  |
|  |  | 28 |  |
| Tx7 [{sn} {sn} {-sn} {-sn}{-sn} {-sn} {sn} {sn}], |  | 29 |  |
|  |  | 30 |  |
| Tx8 [{sn} {-sn} {-sn} {sn}{-sn} {sn} {sn} {-sn}]. |  | 31 |  |
| 12.1.4.8 Adaptive MIMO transmission |  | 32 |  |
|  |  | 33 |  |
|  |  | 34 |  |
| In the following, several transmission modes will be described that can be used to operate a MIMO link. The |  | 35 |  |
| main objective is to enable a dynamic trade-off between spatial diversity and spatial multiplexing, so that the |  | 36 |  |
| best number of streams is always selected to maximize the throughput and to operate the link reliably. It is |  | 37 |  |
| assumed that the MIMO link will be operated adaptively in a bidirectional closed-loop manner and that |  | 38 |  |
| MIMO metrics reports regarding the forward link are provided over the reverse link. |  | 39 |  |
| 12.1.4.8.1 Optimal MIMO transmission |  | 40 |  |
|  |  | 41 |  |
|  |  | 42 |  |
| Ideally, with having full channel state information conveyed from the receiver to the transmitter, the MIMO |  | 43 |  |
| transmission can be described as follows. The transmission on each subcarrier is most conveniently formu- |  | 44 |  |
| lated in the frequency domain as |  | 45 |  |
|  |  | 46 |  |
| yn  Hn xn  ν n | (38) | 47 |  |
|  |  | 48 |  |
| where the nTx  1 vector xn contains the signals transmitted from all transmitters at the OFDM sub-carrier |  | 49 |  |
|  |  | 50 |  |
| with index n. The nRx1 vectors yn and vn contain the received signals and the noise, respectively. The inte- |  | 51 |  |
| gers nTx and nRx denote the numbers of transmitters and receivers, respectively. |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 381 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- The nRx nTx matrix Hn denotes the channel matrix for sub-carrier n with the channel coefficients between

- each transmitter and each receiver. It is related to the time-domain channel impulse response matrices Hl as

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

| L1 |  | nl  |  |
| --- | --- | --- | --- |
| Hn  Hl exp j2 |  |  | (39) |
| l0 |  | N  |  |

where L denotes the number of resolved multi-paths. In the optimal way, based on full channel state infor-mation (CSI) at the transmitter and at the receiver, the channel capacity is approached asymptotically by per-forming a singular value decomposition (SVD) of Hn on each sub-carrier,

| Hn  Un Dn VnH | (40) |
| --- | --- |

which gives the matrices Vn and Un containing the Eigenvectors of the channel matrix in the transmit and receive spaces, respectively.

The diagonal matrix Dn which contains i = 1…min(nTx, nRx) singular values in , referred to as the amplitude gains of the spatial Eigenmodes. The superscript H denotes the conjugate transpose of a matrix. In the infor-mation theory, the capacity is asymptotically approached for infinite N by a joint water-filling across all spa-tial Eigenmodes i and all sub-carriers n. Unlike in the information theory, in practise we employ discrete instead of continuous modulation alphabets. A joint bit-loading and power allocation algorithm is therefore used with individual modulation on each Eigenmode and each sub-carrier, according to the current channel state, so that important optimization criteria (throughput, fairness, stability of queues) can be fulfilled.

The transmitted signal vector xn=Vndn is obtained from the data vector dn and the spatially multiplexed data

| ˆ |  | D | 1 | H | y n | . The noise in each stream is boosted differ- |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| signals are reconstructed at the receiver as d n |  |  |  |  U n |  |  |  |
| ently, according to the singular value for each stream. |  |  |  |  |  |  |  |

Depending on the availability of the CSI, there are modifications. When CSI is available only at the receiver, no pre-processing is applied. Assuming additionally linear detection which requires a simple matrix-vector multiplication, the transmitted signals on each sub-carrier may be reconstructed using the minimum mean-square error detector given by the formula

| ˆ |  | H   2 | 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | I | H | yn |  |  |
| xn |  | Hn Hn |  | Hn |  | (41) |  |

where I and ² are the (nTx×nTx) identity matrix and noise variance at one receiver, respectively.

Figure 207—Adaptive MIMO transmission

382	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 12.1.4.8.2 Practical implementation |  | 1 |  |
| --- | --- | --- | --- |
| Following the optimal MIMO transmission scheme, spatial processing is introduced both at the transmitter |  | 2 |  |
|  |  | 3 |  |
| and at the receiver, see Figure 207. Moreover, a variable number of streams is used because in some cases, |  | 4 |  |
| depending on the time-variant wireless channel, a higher capacity is reached with a reduced number of |  | 5 |  |
| streams. For instance, if an LED is directed away from the receiver, it is hardly useful for transmission and |  | 6 |  |
| may be switched off. |  | 7 |  |
| User data are de-multiplexed yielding Q parallel data streams, where Q is an integer ranging from 1 to nTx. |  | 8 |  |
|  |  | 9 |  |
| The data in each stream are transported using an individually selectable modulation scheme on each subcar- |  | 10 |  |
| rier, in order to maximize the throughput, which is also denoted as per-stream rate control. All active streams |  | 11 |  |
| are then passed through a spatial scheme processing unit, in which channel knowledge, obtained over the |  | 12 |  |
| reverse link is used to identify the best spatial pre-processing of all streams transported in parallel. It is clear |  | 13 |  |
| that the MIMO channel rank can vary over time and also as a function of the subcarrier index n. Accord- |  | 14 |  |
| ingly, and as a natural extension of the adaptive OFDM approach, the selection of the best MIMO transmis- |  | 15 |  |
| sion mode is done for each subcarrier or for a group of subcarriers. Several simplifications of the spatial |  | 16 |  |
| processing are introduced now in order to reduce complexity. |  | 17 |  |
| 12.1.4.8.3 MIMO transmission modes |  | 18 |  |
|  |  | 19 |  |
|  |  | 20 |  |
| Spatial repetition code: One important simplification is that only one stream is transmitted and |  | 21 |  |
| received over all LEDs and PDs, respectively. This mode is useful, e.g., in order to create an omnidirectional |  | 22 |  |
| transmitter characteristics. This can be reached using the precoding vector vn = (1 1 1 1 …1)T. |  | 23 |  |
|  |  | 24 |  |
| Transmitter selection: In order to save energy, modulation may be switched off for some LEDs, |  | 25 |  |
| which results in zeros at the respective positions in the precoding vector vn. |  | 26 |  |
|  |  | 27 |  |
| Receiver selection: As only one stream is transmitted using multiple LEDs, maximum ratio com- |  | 28 |  |
| bining (MRC) is optimal. However, it requires an ADC at each receiver as well as multiple FFTs. Often, few |  | 29 |  |
| links in the MIMO channel have free LOS and a reduced path loss, accordingly, and for all modulation fre- |  | 30 |  |
| quencies. Hence, the channel matrix is "sparse". For reduced complexity, it may be sufficient to select the |  | 31 |  |
| strongest received signals and to combine them using equal gain combining (EGC). This can be realized |  | 32 |  |
| already in the analog domain so that fewer ADC are sufficient. |  | 33 |  |
|  |  | 34 |  |
| Combined transmitter and receiver selection: There can be a combination of transmitter and |  | 35 |  |
| receiver selection. |  | 36 |  |
|  |  | 37 |  |
| Transmitter and receiver selection for multiple streams: The above two schemes can even be com- |  | 38 |  |
| bined with multi-stream transmission as long as the number of streams Q is equal or smaller than the mini- |  | 39 |  |
| mum of the numbers of active transmitters and receivers. At the receiver side, the residual cross-talk is then |  | 40 |  |
| reduced by multi-stream processing. |  | 41 |  |
|  |  | 42 |  |
| WDM transmission: For WDM, because different colors are used, normally the number of streams |  | 43 |  |
| is the same as the number of transmitter ports. In this case, multiple streams are transmitted in parallel and |  | 44 |  |
| the precoding matrix on all subcarriers is given by Vn = 1n. Because color separation behind the receiver fil- |  | 45 |  |
| ters may be imperfect, MIMO reference symbols can be transmitted, and MIMO channel estimation and pro- |  | 46 |  |
| cessing can be performed in order to reduce the residual cross-talk and to increase the spectral efficiency. |  | 47 |  |
| WSK transmission: For WSK transmission, e.g. in case of an RGBY LED, the precoding vector vn |  | 48 |  |
|  |  | 49 |  |
| = (aR aG aB aY)T is used. If MIMO reference symbols are transmitted, imperfect color calibration at the |  | 50 |  |
| transmitter, which could also be falsified by reflecting surfaces, can be compensated by MIMO processing at |  | 51 |  |
| the receiver side. |  | 52 |  |
|  |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 383 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 12.1.5 Coordinated wireless network |
| --- | --- |
| 2 |  |

- The idea of the coordinated wireless network (COW) is to deploy multiple APs so that continuous coverage

- is reached for mobile optical wireless user devices in a desired service area. There may be overlapping cov-

- erage areas, where horizontal handover from one AP to another AP is needed and non-overlapping areas,

- where vertical handover to another wireless technology is needed. In the following, the overlapping case is

- described.

8

9	12.1.5.1 Link setup for COW transmission

10

- Same as in the MIMO mode, initial link setup and header transmission detection are performed in the single-

- input single-output (SISO) mode, in order to improve the reliability of transmission. Transmission of the

- preamble and the header is done over all transmitters of all APs. Channel estimation symbols used by differ-

- ent APs are made orthogonal to each other. Header detection can be improved by using maximum ratio com-

- bining (MRC) based on individual estimates of the superimposed channels from all transmitters at each

- receiver h  nTx  H  . A list of APs in the jointly served area is included in the header, together with the

- ij1    ij

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

assigned comb shift, see 4.2.6.2. As described below, the list can include spatial reuse of comb shifts for multiple APs.

| LED-specific | frequency |  |
| --- | --- | --- |
| reference |  |  |
| sequence | reuse 1/7 |  |
| { |  |  |

| index |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| carrier |  |  |  |  |  | for noise |  |
|  |  |  |  |  |  | estimation |  |

symbol index

Figure 208—Additional channel estimation symbols used in COW.

In Figure 208 the center figure shows additional channel estimation symbols used in the coordinated wire-less network over multiple ACE symbols, each LED at a given access point transmits another sequence. Subcarriers marked with the same color are assigned to the same access point. Subcarrier combs can be reused after a certain distance. The assignment of ACE symbols can be defined and changed dynamically by the network management, e.g. if a new AP is added to the network.

12.1.5.2 ACE symbols for COW transmission

- In order to identify different APs in the COW topology and to maintain the possibility to use MIMO at each

- AP, channel estimation symbols for different APs are made orthogonal in the frequency domain. The general

384	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| idea is that, during the entire channel estimation block, each AP is assigned another comb of subcarriers |  | 1 |  |
| --- | --- | --- | --- |
| (marked with the same specific color in Figure 208), from a set of orthogonal combs [11]. |  | 2 |  |
| Using a comb of subcarriers only, instead of all subcarriers, is possible because only as many subcarriers are |  | 3 |  |
|  |  | 4 |  |
| needed for channel estimation as there are taps in the CP, in order to identify all multi-paths. Advanced and |  | 5 |  |
| computationally efficient algorithms are available in the literature in order to accurately interpolate the chan- |  | 6 |  |
| nel frequency response based on such subcarrier combs [12]. Note that the same comb can be reused by APs |  | 7 |  |
| after a certain distance, at which the signal is sufficiently attenuated. |  | 8 |  |
| The comb scheme can be extended to MIMO. Based on the comb symbols used for COW transmission, the |  | 9 |  |
|  |  | 10 |  |
| same rules are applied as for single-cell MIMO, see 4.2.5.7. |  | 11 |  |
| 12.1.5.3 Cell- and user-specific ACE |  | 12 |  |
|  |  | 13 |  |
| Note that ACE signals are sent twice in the COW topology. In the first period, also denoted as cell-specific |  | 14 |  |
|  |  | 15 |  |
| AC (CS-ACE), the ACRE sequence is sent directly from all transmitters, so that the physical channel matrix |  | 16 |  |
| H is estimated on each subcarrier. This information is useful for joint transmitter optimization, after the AP |  | 17 |  |
| received the estimated CSI via feedback from its UDs. |  | 18 |  |
| In the second period, also denoted as user-specific ACE (US-ACE), the ACE sequence is passed though the |  | 19 |  |
|  |  | 20 |  |
| transmitter optimization, before being transmitted. Note that the joint transmitter optimization depends on |  | 21 |  |
| the channel of other UDs attached to other APs as well. Only by using US-ACE, the UD can estimate the |  | 22 |  |
| modified effective channel matrix Heffective and adapt its receiver processing, accordingly. |  | 23 |  |
| 12.1.5.4 Adaptive COW transmission |  | 24 |  |
|  |  | 25 |  |
| editor note: suggest Volker to provide text in the comments resolution against D0. See comment 241 of |  | 26 |  |
|  |  | 27 |  |
| https://mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx |  | 28 |  |
| t.b.d. |  | 29 |  |
|  |  | 30 |  |
| 12.1.6 Relaying |  | 31 |  |
|  |  | 32 |  |
| Relaying operation supports two modes: 1) HD/DF: half duplex (HD) with decode and forward (DF) relay- |  | 33 |  |
|  |  | 34 |  |
| ing mode and 2) FD/AF: full duplex (FD) with amplify and forward (AF) relaying mode sand full duplex |  | 35 |  |
| (DF) duplexing mode. |  | 36 |  |
| 12.1.6.1 Full duplex (FD) Mode |  | 37 |  |
|  |  | 38 |  |
| In FD relaying mode, the REL simultaneously receives and transmits both in uplink and downlink support- |  | 39 |  |
|  |  | 40 |  |
| ing the AF mode. |  | 41 |  |
| 12.1.6.2 Half duplex (HD) relaying |  | 42 |  |
|  |  | 43 |  |
| In HD relaying mode, the relay REL shall decode and store the data packets in order to retransmit them to |  | 44 |  |
|  |  | 45 |  |
| the REL in its transmission period supporting the DF mode. The REL can erase the stored packet after it |  | 46 |  |
| receives the ACK. |  | 47 |  |
| 12.1.7 Payload data |  | 48 |  |
|  |  | 49 |  |
| In front of the OFDM modulator (and an eventual preprocessing to generate single-carrier and unipolar sig- |  | 50 |  |
|  |  | 51 |  |
| nals), payload data are scrambled and then fed into the forward error correction (FEC). Next they are |  | 52 |  |
| mapped onto constellation points. Finally there is an additional constellation scrambling operation. |  | 53 |  |
|  |  | 54 |  |
| Copyright © 20xx IEEE. All rights reserved. | 385 |  |  |
| This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 12.1.8 Data scrambling |
| --- | --- |
| 2 |  |

- For data scrambling, a linear feedback shift register (LSFR) is used. It is defined by the number of registers

- NLFSR. The position of feedback taps is commonly described via a generator polynomial and the initializa-

- tion of the registers s1 to sNLFSR.

| 6 | In Figure 209 the LFSR used for data scrambling is shown. It implements the generator polynomial |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | g(x)=x23+x18+1 and its register range from s |  |  |  | 1 | to s . |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  | 23 |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Data in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | LSB |  |  |  |  |  |  |  |  |  |  |  |  | MSB |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Data out |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | s1 | s2 | s3 | s4 |  | s5 | s6 | ... | s18 | s19 | s20 | s21 | s22 | s23 |  |
| 18 |  |  | Figure 209—LFSR for data scrambling |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- All data starting from the first bit of the PHY-frame header (PFH) and ending by the last bit of the payload

- shall be scrambled with a pseudorandom sequence generated by this LFSR.

23

24	The LFSR generator is initialized at the first bit of the header with the initialization vector equal to

- 0x2AAAAA in hexadecimal notation or 0010 1010 1010 1010 1010 1010 in binary notation where the LSB

- corresponds to C1. This initialization is used for the scrambling of the header data. The first bit to be scram-

- bled shall be XOR'ed with the first bit generated by the LFSR after initialization (i.e., C18 XOR C23 of the

- initialization vector).

29

- The special value for SI (scrambler initialization) in the PHY-frame header of 016 in hexadecimal notation

- indicates that the scrambler is not re-initialized between the header and the payload. An initialization of the

- SI field to values other than the special value is optional, as described in the G.hn specification.

33

34	12.1.8.1 Forward-error correction

35

- The channel coding for the header uses low-density parity-check codes (LDPC) according to the G.hn stan-

- dard, see [4] (editor note: Volker will provide the reference. See comment 247 of https://mentor.ieee.org/

- 802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx). A short LDPC code of K=168 data bits

- with code rate ½ is used. The resulting block is modulated with QPSK with repetitions in frequency. If band-

- width is low and SNR shall be increased, the header symbol can be repeated in time.

41

42

43

44

45

46

47

48

49

50	Figure 210—The low-density parity check code from the G.hn standard is used

51

52

53

54

386	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The channel coding for the data uses LDPC according to the G.hn standard, see [4]. The LDPC code may be selected between two information block sizes of 4320 or 920 bits bits and 5 code rates of 1/2, 2/3, 5/6, 16/18 or 20/21.

The channel coding for MIMO stream-interleaved encoding is used, consistent with the G.hn standard. Stream-wise decoding is not considered, as it would increase both, complexity and latency. To be further detailed.

t.b.d., see G.9960-2015

editor note: suggest Volker to provide text in the comments resolution against D0. See comment 243 of https://mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx

12.1.8.2 Bit-to-Symbol Mapper

t.b.d., see G.9960-2015

editor note: suggest Volker to provide text in the comments resolution against D0. See comment 244 of https://mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx

12.1.8.3 Constellation Scrambling

In Figure 211 the LFSR used for the constellation scrambler is shown. It implements the generator polyno-mial g(x)=x13+x12+x11+x8+1 and its registers range from s1 to s13 being the least significant bit (LSB) and most significant bits (MSB), respectively.

|  |  | LSB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MSB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | s1 | s2 | s3 |  | s4 | s5 | s6 | s7 | s8 |  |  |  | s9 | s10 | s11 |  |  |  | s12 |  |  |  | s13 |  |  |
|  |  |  |  |  | Figure 211—LFSR for constellation scrambling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The phase of the complex symbols generated by the bit-to-symbol mapper is shifted before being fed into the IFFT. The phase shifts are determined by the pseudo-random bit-sequence generated by the LSFR depicted in Figure 211. The two LSBs s1 and s2 determine the shift as given by Table 158. For a generated phase shift φi, the LSFR is shifted by 2i. Due to the constellation scrambling, the phase of an originally gen-erated complex symbol x0i is shifted by φi resulting in xi, the actual input of the IFFT:

| 0 | exp j i |  |  |
| --- | --- | --- | --- |
| xi = xi |  | (42) |  |

For the header, ACE and payload, the shift of the i-th subcarrier is φi and therefore the LSFR is shifted by 2i for each subcarrier. For these cases, the LSFR is initialized to the seed of 1FFFF16 in hexadecimal notation for each OFDM symbol, where the LSB of the seed corresponds to s1.

| Copyright © 20xx IEEE. All rights reserved. | 387 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 158—Phase shift values for LFSR states s1 and s2

| Register states of the LFSR |  | Phase shift |
| --- | --- | --- |
| s2 | s1 |  |
| 0 | 0 | 0 |
| 0 | 1 | π/2 |
| 1 | 0 | π |
| 1 | 1 | 3π/4 |

388	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Annex A

(informative)

Bibliography

A.1 General

Bibliographical references are resources that provide additional or helpful material but do not need to be understood or used to implement this standard. Reference to these resources is made for informational use only.

[B1] Berman, S. M., Greenhouse, D. S., Bailey, I. L., Clear, R. and Raasch, T. W., “Human electroretinogram responses to video displays, fluorescent lighting and other high frequency sources,” Optometry and Vision Science 68:645–662, 1991.

[B2] CIE (1932) Commission internationale de l'Eclairage proceedings, 1931. Cambridge University Press, Cambridge.

[B3] Colman, R. S., Frankel, F., Ritvo, E., and Freeman, B., “The effects of fluorescent and incandescent illumination upon repetitive behaviors in autistic children,” J. Autism Childhood Schz. 6:157–162, 1976.

[B4] Harding, G. F. A. and Jeavons, P., “Photosensitive Epilepsy,” Mac Keith Press, 1994.

[B5] IEEE 802.15 document 15-10 -0151-01-0007, “High-power high-bandwidth linear driving circuit for OWC applications,” Baumgartner, Robert, et. al., https://mentor.ieee.org/802.15/dcn/10/15-10-0151-01-0007-high-power-high-bandwidth-linear-driving-circuit-for-OWC-applications.pdf.

[B6] IEEE Std 802.11TM-2007, Information technology—Telecommunications and information exchange between systems—Local and metropolitan area networks—Specific requirements—Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications.

[B7] IEEE Std 802.15.3TM-2003, IEEE Standard for Information technology—Telecommunications and information exchange between systems—Local and metropolitan area networks—Specific requirements— Part 15.3: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for High Rate Wireless Personal Area Networks (OWPANs).

[B8] ISO/IEC 7498-1:1994, Information technology—Open Systems Interconnection—Basic Reference Model: The Basic Model.9

[B9] Kang, T. G., et. al., “White Paper: IEEE 802.15.7 OWC Regulations,” July 2010, IEEE802.15 contribution 15-10-0615-01-0007, https://mentor.ieee.org/802.15/dcn/10/15-10-0615-01-0007-white-paper-ieee-802-15-7-OWC-regulations.pdf.

[B10] Küller, R. and Laike, T., “The impact of flicker from fluorescent lighting on well-being, performance, and physiological arousal,” Ergonomics 41(4): 433–447, 1998.

[B11] Lindner, H. and Kropf, S., “Asthenopic complaints associated with fluorescent lamp illumination (FLI): The role of individual disposition,” Lighting Res. Technol. Vol. 25:59–69, 1993.

9ISO publications are available from the ISO Central Secretariat, Case Postale 56, 1 rue de Varembé, CH-1211, Genève 20, Switzerland/Suisse (http://www.iso.ch/). ISO publications are also available in the United States from the Sales Department, American National Standards Institute, 25 West 43rd Street, 4th Floor, New York, NY 10036, USA (http://www.ansi.org/).

| Copyright © 20xx IEEE. All rights reserved. | 389 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

[B12] Schubert, E. Fred, “Light Emitting Diodes,” Cambridge University Press, 2003.

[B13] Stone, P. T., “Fluorescent lighting and health,” Lighting Res. Technol. 24:55–61, 1990.

[B14] Walewski, J. W., “Color stabilization for CSK by use of visibility frames,” January 2011, IEEE802.15 contribution 15-1-0262-05-0007, https://mentor.ieee.org/802.15/dcn/11/15-10-0262-05-0007-color-stabilization-for-csk-by-use-of-visibility-frames.pdf.

[B15] Wilkins, A. J., I. Nimmo-Smith, A. Slater, and L. Bedocs, “Fluorescent lighting headaches and eye-strain,” Lighting Res. Technol. Vol. 21:11–18, 1989.

[B16] Wright, P. S., “An overview of harmonic and flicker emission standards and their associated measurements,” Power Eng. J. 15(2): 87–93, 2001.

- [B17] Yokoi, A., et. al., “More description about CSK constellation,” March 2011, IEEE 802.15 contribution

- 15-11-0247-00-0007,   https://mentor.ieee.org/802.15/dcn/11/15-11-0247-00-0007-csk-constellation-in-all-

- color-band-combinations.pdf.

17

- [B18] H. Elgala and T.D.C. Little, "Reverse polarity optical OFDM (RPO-OFDM): dimming compatible

- OFDM for gigabit OWC links", Optics Express, vol. 21, issue 20, pp. 24288 - 24299, 2013.

20

- [B19] V. Jungnickel, T. Hindelang, T. Haustein, W. Zirwas, SC-FDMA Waveform Design, Performance,

- Power Dynamics and Evolution to MIMO, Proc. IEEE Portable, March 2007.

| 23 | [B20] V. Jungnickel, L. Grobe, Localized SC-FDMA with Constant Envelope, Proc. Int. Symp. Personal, |  |  |  |
| --- | --- | --- | --- | --- |
| 24 |  |  |  |  |
|  | Indoor and Mobile Radio Systems (PIMRC), IEEE, London, UK, Sept. 2013, pp. 24-29. |  |  |  |
| 25 |  |  |  |  |
| 26 | [B21] Xingxing Huang, Siyuan Chen, Zhixin Wang, Jianyang Shi, Yiguang Wang, Jiangnan Xiao and Nan |  |  |  |
| 27 |  |  |  |  |
|  | Chi, "2.0-Gb/s visible light link based on adaptive bit allocation OFDM of a single phosphorescent white |  |  |  |
| 28 |  |  |  |  |
|  | LED," IEEE Photonics Journal 7(5), October 2015. |  |  |  |
| 29 |  |  |  |  |
| 30 | [B22] H. Elgala and T.D.C. Little, "Reverse polarity optical OFDM (RPO-OFDM): dimming compatible |  |  |  |
| 31 |  |  |  |  |
|  | OFDM for gigabit OWC links", Optics Express, vol. 21, issue 20, pp. 24288 - 24299, 2013. |  |  |  |
| 32 |  |  |  |  |
| 33 |  |  |  |  |
| 34 | A.2 Regulatory documents |  |  |  |
| 35 |  |  |  |  |
| 36 | [B23] ANSI C82.77-2002, American National Standard for Harmonic Emission Limits—Related Power |  |  |  |
| 37 |  |  |  |  |
|  | Quality Requirements for Lighting Equipment.10 |  |  |  |
| 38 |  |  |  |  |
| 39 | [B24] ANSI C82.SSL1 Operational Characteristics and Electrical Safety of SSL Power Supplies and |  |  |  |
| 40 |  |  |  |  |
|  | Drivers. |  |  |  |
| 41 |  |  |  |  |
| 42 | [B25] ANSI Z136.1, American National Standard for Safe Use of Lasers (2007). |  |  |  |
| 43 |  |  |  |  |
| 44 | [B26] ANSI Z136.6, American National Standard for Safe Use of Lasers Outdoors (2005). |  |  |  |
| 45 |  |  |  |  |
| 46 | [B27] ARIB STD-T66, Second Generation Low Power Data Communication System/Wireless LAN System |  |  |  |
| 47 |  |  |  |  |
|  | 1999.12.14 (H11.12.14) Version 1.0.11 |  |  |  |
| 48 |  |  |  |  |
| 49 |  |  |  |  |
| 50 |  |  |  |  |
| 51 |  |  |  |  |
| 52 |  |  |  |  |
|  | 10ANSI C82 publications are available from NEMA publications are available from the National Electrical Manufacturers Association, |  |  |  |

- 1300 N. 17th St., Ste. 1847, Rosslyn, VA 22209, USA.

- 11ARIB publications are available from the Association of Radio Industries and Businesses (http://www.arib.or.jp).

390	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| [B28] |  | BS EN 29241-3:1993, Ergonomic requirements for office work with visual display terminals (VDTs). |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Visual display requirements. |  |  |  |  | 2 |  |
| [B29] |  | EN61000-3-3: Electronic compatibility (EMC) Part 3: Limits—Section 3: Limitation of voltage |  |  | 3 |  |
|  |  |  |  |  | 4 |  |
| fluctuations and flicker in low voltage supply systems for equipment with rated current 16 A and smaller. |  |  |  |  | 5 |  |
| International Electrotechnical Commission, 1994. |  |  |  |  | 6 |  |
| [B30] |  | ERC Recommendation 70-03, Relating to the Use of Short Range Devices (SRDs), April 2002.12 |  |  | 7 |  |
|  |  |  |  |  | 8 |  |
|  |  |  |  |  | 9 |  |
| [B31] IEC 1000-3-3 (1994), Part 3: Limits—Section 3: Limitation of voltage fluctuations and flicker in low- |  |  |  |  | 10 |  |
| voltage supply systems for equipment with rated current 16 A. |  |  |  |  | 11 |  |
|  |  |  |  |  | 12 |  |
| [B32] |  | IEC 1000-3-5: Electromagnetic compatibility. Part 3: Limits—Section 5: Limitation of voltage |  |  | 13 |  |
| fluctuations and flicker in low-voltage power supply systems for equipment with rated current greater than |  |  |  |  | 14 |  |
| 16 A. 1994. |  |  |  |  | 15 |  |
|  |  |  |  |  | 16 |  |
| [B33] |  | IEC 60050-112 ed1.0 (2010-01) TC/SC 1—International Electrotechnical Vocabulary—Part 112: |  |  | 17 |  |
| Quantities and units. |  |  |  |  | 18 |  |
|  |  |  |  |  | 19 |  |
| [B34] IEC 60050-300 ed1.0 (2001-07), International Electrotechnical Vocabulary—Electrical and electronic |  |  |  |  | 20 |  |
| measurements and measuring instruments—Part 311: General terms relating to measurements—Part 312: |  |  |  |  | 21 |  |
| General terms relating to electrical measurements—Part 313: Types of electrical measuring instruments— |  |  |  |  | 22 |  |
| Part 314: Specific terms according to the type of instrument. |  |  |  |  | 23 |  |
|  |  |  |  |  | 24 |  |
| [B35] |  | IEC 60050-845 ed1.0 (1987-12), International Electrotechnical Vocabulary, Lighting. |  |  | 25 |  |
|  |  |  |  |  | 26 |  |
| [B36] |  | IEC 60825-1 ed2.0 (2007-03), Safety of laser products—Part 1: Equipment classification and |  |  | 27 |  |
| requirements. |  |  |  |  | 28 |  |
|  |  |  |  |  | 29 |  |
| [B37] |  | IEC 60825-12 ed1.0 (2004-02), Safety of laser products—Part 12: Safety of free space optical |  |  | 30 |  |
| communication systems used for transmission of information. |  |  |  |  | 31 |  |
|  |  |  |  |  | 32 |  |
| [B38] |  | IEC 61347-1 ed2.0 (2007-01), Lamp control gear—Part 1: General and safety requirements. |  |  | 33 |  |
|  |  |  |  |  | 34 |  |
| [B39] IEC 61347-2-13 ed1.0 (2006-05), Lamp control gear—Part 2-13: Particular requirements for d.c. or |  |  |  |  | 35 |  |
| a.c. supplied electronic control gear for LED modules |  |  |  |  | 36 |  |
|  |  |  |  |  | 37 |  |
| [B40] |  | IEC 62384-am1 ed1.0 (2009-07) Amendment 1—DC or AC supplied electronic control gear for LED |  |  | 38 |  |
| modules—Performance requirements. |  |  |  |  | 39 |  |
|  |  |  |  |  | 40 |  |
| [B41] |  | IEC 62471 ed1.0 (2006-07), Photobiological safety of lamps and lamp systems. |  |  | 41 |  |
|  |  |  |  |  | 42 |  |
| [B42] |  | IEC/TR 62471-2 ed1.0 (2009-08), Photobiological safety of lamps and lamp systems—Part 2: |  |  | 43 |  |
| Guidance on manufacturing requirements relating to non-laser optical radiation safety. |  |  |  |  | 44 |  |
| [B43] |  | IEC/TS 61000-3-5 Ed. 2.0 b Cor.2:2010 Corrigendum 2—Electromagnetic compatibility (EMC)— |  |  | 45 |  |
|  |  |  |  |  | 46 |  |
| Part 3-5: Limits—Limitation of voltage fluctuations and flicker in low-voltage power supply systems for |  |  |  |  | 47 |  |
| equipment with rated current greater than 75 A. |  |  |  |  | 48 |  |
| [B44] |  | ANSI/IESNA RP-16-05, Nomenclature and Definitions for Illuminating Engineering.13 |  |  | 49 |  |
|  |  |  |  |  | 50 |  |
|  |  |  |  |  | 51 |  |
|  |  |  |  |  | 52 |  |
| 12ERC publications are available from Global Engineering Documents, 15 Inverness Way East, Englewood, Colorado 80112, USA |  |  |  |  | 53 |  |
| (http://global.ihs.com/). |  |  |  |  | 54 |  |
|  |  |  | Copyright © 20xx IEEE. All rights reserved. | 391 |  |  |
|  |  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | [B45] ANSI/IESNA RP-27.1-05, Photobiological Safety for Lamps and Lamp Systems—General |
| --- | --- |
| 2 | Requirements. |
| 3 |  |

- [B46] IESNA LM-79-08, IES Approved Method for the Electrical and Photometric Measurements of Solid-

- State Lighting Products.

| 6 | [B47] IESNA LM-80-08, IES Approved Method: Measuring Lumen Maintenance of LED Light Sources. |  |  |  |
| --- | --- | --- | --- | --- |
| 7 |  |  |  |  |
| 8 | [B48] IESNA TM-16-05, Technical Memorandum on Light Emitting Diode (LED) Sources and Systems. |  |  |  |
| 9 |  |  |  |  |
| 10 | [B49] Recommendation ITU-R RA.1630, Technical and operational characteristics of ground-based |  |  |  |
| 11 |  |  |  |  |
|  | astronomy systems for use in sharing studies with active services between 10 THz and 1000 THz. |  |  |  |
| 12 |  |  |  |  |
| 13 | [B50] Recommendation ITU-R RS.1744, Technical and operational characteristics of ground-based |  |  |  |
| 14 | meteorological aids systems operating in the frequency range 272–750 THz.14 |  |  |  |
| 15 |  |  |  |  |
| 16 |  |  |  |  |
| 17 |  |  |  |  |
| 18 |  |  |  |  |
| 19 |  |  |  |  |
| 20 |  |  |  |  |
| 21 |  |  |  |  |
| 22 |  |  |  |  |
| 23 |  |  |  |  |
| 24 |  |  |  |  |
| 25 |  |  |  |  |
| 26 |  |  |  |  |
| 27 |  |  |  |  |
| 28 |  |  |  |  |
| 29 |  |  |  |  |
| 30 |  |  |  |  |
| 31 |  |  |  |  |
| 32 |  |  |  |  |
| 33 |  |  |  |  |
| 34 |  |  |  |  |
| 35 |  |  |  |  |
| 36 |  |  |  |  |
| 37 |  |  |  |  |
| 38 |  |  |  |  |
| 39 |  |  |  |  |
| 40 |  |  |  |  |
| 41 |  |  |  |  |
| 42 |  |  |  |  |
| 43 |  |  |  |  |
| 44 |  |  |  |  |
| 45 |  |  |  |  |
| 46 |  |  |  |  |
| 47 |  |  |  |  |
| 48 |  |  |  |  |
| 49 |  |  |  |  |
| 50 |  |  |  |  |
| 51 |  |  |  |  |
|  | 13 |  |  |  |

- American National Standards Institute, 25 West 43rd Street, 4th Floor, New York, NY 10036, USA (http://www.ansi.org/).

- 14This recommendation replaces ITU-R SA.1744. ITU-T publications are available from the International Telecommunications Union,

- Place des Nations, CH-1211, Geneva 20, Switzerland/Suisse (http://www.itu.int/).IESNAistheIlluminatingEngineeringSocietyStandardsfromANSI.IESNApublications are available from the Sales Department,

392	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Annex B

(normative)

Service-specific convergence sublayer (SSCS)

B.1 IEEE 802.2 convergence s ublayer

The IEEE 802.2 convergence sublayer exists above the IEEE 802.15.7 MCPS. This sublayer provides an interface between an instance of an IEEE 802.2 LLC sublayer and the IEEE 802.15.7 MCPS.

B.1.1 MA-UNITDATA.reques t

The MA-UNITDATA.request primitive requests the transfer of a LLC protocol data unit (LPDU) (i.e., MSDU) from a local IEEE 802.2 Type 1 LLC sublayer entity to a single peer IEEE 802.2 Type 1 LLC sublayer entity or multiple peer IEEE 802.2 Type 1 LLC sublayer entities in the case of a group address.

The semantics of the MA-UNITDATA.request primitive is as follows:

MA-UNITDATA.request	(

SrcAddr,

DstAddr,

RoutingInformation,

data,

priority,

ServiceClass

)

Table B.1 specifies the parameters for the MA-UNITDATA.request primitive.

Table B.1—MA-UNITDATA.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SrcAddr | IEEE | Any valid | The individual IEEE address of the entity from which the |
|  | address | IEEE address | MSDU is being transferred. |
| DstAddr | IEEE | Any valid | The individual IEEE address of the entity to which the |
|  | address | IEEE address | MSDU is being transferred. |
| RoutingInformation | — | null | This parameter is not used by the MAC sublayer and shall be |
|  |  |  | specified as a null value. |
| data | Set of | — | The set of octets forming the MSDU to be transmitted by the |
|  | octets |  | MAC sublayer entity. |
| priority | — | null | This parameter is not used by the MAC sublayer and shall be |
|  |  |  | specified as a null value. |
| ServiceClass | — | null | This parameter is not used by the MAC sublayer and shall be |
|  |  |  | specified as a null value. |

| Copyright © 20xx IEEE. All rights reserved. | 393 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | B.1.1.1 Appropriate usage |
| --- | --- |
| 2 |  |

- The MA-UNITDATA.request primitive is generated by a local IEEE 802.2 Type 1 LLC sublayer entity when

- an LPDU (MSDU) is to be transferred to a peer IEEE 802.2 Type 1 LLC sublayer entity or entities.

5

6	B.1.1.2 Effect on receipt

7

- On receipt of the MA-UNITDATA.request primitive, the MAC sublayer entity shall begin the transmission

- of the supplied MSDU.

10

- The MAC sublayer first builds an MPDU to transmit from the supplied arguments. The MPDU shall be

- transmitted using the unslotted CSMA-CA algorithm in the contention period of the frame and without

- requesting a handshake.

14

- If the unslotted CSMA-CA algorithm indicates a busy channel, the MAC sublayer shall issue the MA-

- UNITDATA-STATUS.indication primitive with a status of CHANNEL_ACCESS_FAILURE. If the MPDU

- was successfully transmitted, the MAC sublayer shall issue the MA-UNITDATA-STATUS.indication

- primitive with a status of SUCCESS.

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

B.1.2 MA-UNITDATA.indication

The MA -UNITDATA.indication primitive indicates the transfer of an LPDU (i.e., MSDU) from the MAC sublayer to the local IEEE 802.2 Type 1 LLC sublayer entity.

The semantics of the MA-UNITDATA.indication primitive is as follows:

MA-UNITDATA.indication	(

SrcAddr,

DstAddr,

RoutingInformation,

data,

ReceptionStatus,

priority,

ServiceClass

)

Table B.2 specifies the parameters for the MA-UNITDATA.indication primitive.

Table B.2—MA-UNITDATA.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SrcAddr | IEEE | Any valid | The individual IEEE address of the entity from which |
|  | address | IEEE address | the MSDU has been received. |
| DstAddr | IEEE | Any valid | The individual IEEE address of the entity to which the |
|  | address | IEEE address | MSDU is being transferred. |
| RoutingInformation | — | null | This parameter is not used by the MAC sublayer and |
|  |  |  | shall be specified as a null value. |

394	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table B.2—MA-UNITDATA.indication parameters (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| data | Set of octets | — | The set of octets forming the MSDU received by the |
|  |  |  | MAC sublayer entity. |
| ReceptionStatus | — | null | This parameter is not used by the MAC sublayer and |
|  |  |  | shall be specified as a null value. |
| priority | — | null | This parameter is not used by the MAC sublayer and |
|  |  |  | shall be specified as a null value. |
| ServiceClass | — | null | This parameter is not used by the MAC sublayer and |
|  |  |  | shall be specified as a null value. |

B.1.2.1 When generated

On receipt of a data frame at the local MAC sublayer entity, the FCS field is checked. If it is valid, the MAC sublayer shall issue the MA-UNITDATA.indication primitive to the IEEE 802.2 Type 1 LLC sublayer entity, indicating the arrival of a MSDU. If the FCS is not valid, the frame shall be discarded, and the IEEE 802.2 Type 1 LLC sublayer entity shall not be informed.

B.1.2.2 Appropriate usage

The appropriate usage of the MA-UNITDATA.indication primitive by the IEEE 802.2 Type 1 LLC sublayer entity is not specified in this standard.

B.1.3 MA-UNITDATA-STATUS.indication

The MA -UNITDATA-STATUS.indication primitive reports the results of a request to transfer a LPDU (MSDU) from a local IEEE 802.2 Type 1 LLC sublayer entity to a single peer IEEE 802.2 Type 1 LLC sublayer entity or to multiple peer IEEE 802.2 Type 1 LLC sublayer entities.

The semantics of the MA-UNITDATA-STATUS.indication primitive is as follows:

MA-UNITDATA-STATUS.indication ( SrcAddr, DstAddr, status,

ProvPriority, ProvServiceClass

)

Table B.3 specifies the parameters for the MA-UNITDATA-STATUS.indication primitive.

B.1.3.1 When generated

The MA-UNITDATA-STATUS.indication primitive is generated by the MAC sublayer entity in response to an MA-UNITDATA.request primitive issued by the IEEE 802.2 Type 1 LLC sublayer.

| Copyright © 20xx IEEE. All rights reserved. | 395 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table B.3—MA-UNITDATA-STATUS.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SrcAddr | IEEE | Any valid IEEE address | The individual IEEE address of the |
|  | address |  | entity from which the MSDU has been |
|  |  |  | transferred. |
| DstAddr | IEEE | Any valid IEEE address | The individual IEEE address of the |
|  | address |  | entity to which the MSDU has been |
|  |  |  | transferred. |
| status | Enumeration | SUCCESS, TRANSMIS- | The status of the last MSDU |
|  |  | SION_PENDING, NO_BEACON, | transmission. |
|  |  | or CHANNEL_ACCESS_FAIL- |  |
|  |  | URE |  |
| ProvPriority | — | null | This parameter is not used by the MAC |
|  |  |  | sublayer and shall be specified as a |
|  |  |  | null value. |
| ProvServiceClass | — | null | This parameter is not used by the MAC |
|  |  |  | sublayer and shall be specified as a |
|  |  |  | null value. |

B.1.3.2 Appropriate usage

The receipt of the MA-UNITDATA-STATUS.indication primitive by the IEEE 802.2 Type 1 LLC sublayer entity signals the completion of the current data transmission.

396	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Annex C

(normative)

Cyclic redundancy check

The CRC field is 2 octets in length. The CRC shall be calculated using the following standard generator polynomial of degree 16:

| G16 x = x16 + x12 + x5 + 1 | (C.1) |
| --- | --- |

The CRC shall be calculated for transmission using the following algorithm:

— Let M  x = b0xk – 1 + b1xk – 2 +  + bk – 2x + bk – 1 be the polynomial representing the sequence of bits for which the checksum is to be computed.

—  Multiply M(x) by x16, giving the polynomial x16  M  x .

— Divide x16  M  x modulo 2 by the generator polynomial, G16(x), to obtain the remainder polynomial, R  x = r0x15 + r1x14 +  + r14x + r15.

—  The CRC field is given by the coefficients of the remainder polynomial, R(x).

Here, binary polynomials are represented as bit strings, in highest polynomial degree first order.

A typical implementation is depicted in Figure C.1.

CRC-16 Generator Polynomial: G(x) = x16 + x12 + x5 + 1

Input

Data Field (LSB first)

- Initialize the remainder register (r0 through r15) to all ones.

- Shift the data into the divider in the order of transmission (LSB).

- After the last bit of the data field is shifted into the divider,

the remainder register contains the CRC.

4. The CRC is appended to the data so that r0 is transmitted first.

Figure C.1—Typical CRC implementation

| Copyright © 20xx IEEE. All rights reserved. | 397 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Annex D

(normative)

Channel assignment

Table D.1 shows the bit patterns that are used to assign multiple channels for OWC.

Table legend: X=not used and O=used

Table D.1—Multiple channel assignment table

| Bit |  | Band 1 | Band 2 | Band 3 | Band 4 | Band 5 | Band 6 | Band 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0000000 |  |  |  | No multiple channel mode |  |  |  |  |
| 0000001 | X |  | X | X | X | X | X | O |
| 0000010 | X |  | X | X | X | X | O | X |
| 0000011 | X |  | X | X | X | X | O | O |
| 0000100 | X |  | X | X | X | O | X | X |
| 0000101 | X |  | X | X | X | O | X | O |
| 0000110 | X |  | X | X | X | O | O | X |
| 0000111 | X |  | X | X | X | O | O | O |
| 0001000 | X |  | X | X | O | X | X | X |
| 0001001 | X |  | X | X | O | X | X | O |
| 0001010 | X |  | X | X | O | X | O | X |
| 0001011 | X |  | X | X | O | X | O | O |
| 0001100 | X |  | X | X | O | O | X | X |
| 0001101 | X |  | X | X | O | O | X | O |
| 0001110 | X |  | X | X | O | O | O | X |
| 0001111 | X |  | X | X | O | O | O | O |
| 0010000 | X |  | X | O | X | X | X | X |
| 0010001 | X |  | X | O | X | X | X | O |
| 0010010 | X |  | X | O | X | X | O | X |
| 0010011 | X |  | X | O | X | X | O | O |
| 0010100 | X |  | X | O | X | O | X | X |
| 0010101 | X |  | X | O | X | O | X | O |
| 0010110 | X |  | X | O | X | O | O | X |
| 0010111 | X |  | X | O | X | O | O | O |
| 0011000 | X |  | X | O | O | X | X | X |

398	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table D.1—Multiple channel assignment table (continued)

| Bit |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  | Band 5 |  | Band 6 | Band 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0011001 | X |  | X |  | O |  | O |  | X |  | X |  | O |
| 0011010 | X |  | X |  | O |  | O |  | X |  | O |  | X |
| 0011011 | X |  | X |  | O |  | O |  | X |  | O |  | O |
| 0011100 | X |  | X |  | O |  | O |  | O |  | X |  | X |
| 0011101 | X |  | X |  | O |  | O |  | O |  | X |  | O |
| 0011110 | X |  | X |  | O |  | O |  | O |  | O |  | X |
| 0011111 | X |  | X |  | O |  | O |  | O |  | O |  | O |
| 0100000 | X |  | O |  | X |  | X |  | X |  | X |  | X |
| 0100001 | X |  | O |  | X |  | X |  | X |  | X |  | O |
| 0100010 | X |  | O |  | X |  | X |  | X |  | O |  | X |
| 0100011 | X |  | O |  | X |  | X |  | X |  | O |  | O |
| 0100100 | X |  | O |  | X |  | X |  | O |  | X |  | X |
| 0100101 | X |  | O |  | X |  | X |  | O |  | X |  | O |
| 0100110 | X |  | O |  | X |  | X |  | O |  | O |  | X |
| 0100111 | X |  | O |  | X |  | X |  | O |  | O |  | O |
| 0101000 | X |  | O |  | X |  | O |  | X |  | X |  | X |
| 0101001 | X |  | O |  | X |  | O |  | X |  | X |  | O |
| 0101010 | X |  | O |  | X |  | O |  | X |  | O |  | X |
| 0101011 | X |  | O |  | X |  | O |  | X |  | O |  | O |
| 0101100 | X |  | O |  | X |  | O |  | O |  | X |  | X |
| 0101101 | X |  | O |  | X |  | O |  | O |  | X |  | O |
| 0101110 | X |  | O |  | X |  | O |  | O |  | X |  | O |
| 0101111 | X |  | O |  | X |  | O |  | O |  | O |  | X |
| 0110000 | X |  | O |  | O |  | X |  | X |  | X |  | X |
| 0110001 | X |  | O |  | O |  | X |  | X |  | X |  | O |
| 0110010 | X |  | O |  | O |  | X |  | X |  | O |  | X |
| 0110011 | X |  | O |  | O |  | X |  | X |  | O |  | O |
| 0110100 | X |  | O |  | O |  | X |  | O |  | X |  | X |
| 0110101 | X |  | O |  | O |  | X |  | O |  | X |  | O |
| 0110110 | X |  | O |  | O |  | X |  | O |  | O |  | X |
| 0110111 | X |  | O |  | O |  | X |  | O |  | O |  | O |
| 0111000 | X |  | O |  | O |  | O |  | X |  | X |  | X |
| 0111001 | X |  | O |  | O |  | O |  | X |  | X |  | X |

| Copyright © 20xx IEEE. All rights reserved. | 399 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table D.1—Multiple channel assignment table (continued)

| Bit |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  | Band 5 |  | Band 6 | Band 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0111010 | X |  | O |  | O |  | O |  | X |  | X |  | X |
| 0111011 | X |  | O |  | O |  | O |  | X |  | X |  | X |
| 0111100 | X |  | O |  | O |  | O |  | O |  | X |  | X |
| 0111101 | X |  | O |  | O |  | O |  | O |  | X |  | X |
| 0111110 | X |  | O |  | O |  | O |  | O |  | O |  | X |
| 0111111 | X |  | O |  | O |  | O |  | O |  | O |  | O |
| 1000000 | O |  | X |  | X |  | X |  | X |  | X |  | X |
| 1000001 | O |  | X |  | X |  | X |  | X |  | X |  | O |
| 1000010 | O |  | X |  | X |  | X |  | X |  | O |  | X |
| 1000011 | O |  | X |  | X |  | X |  | X |  | O |  | O |
| 1000100 | O |  | X |  | X |  | X |  | O |  | X |  | X |
| 1000101 | O |  | X |  | X |  | X |  | O |  | X |  | O |
| 1000110 | O |  | X |  | X |  | X |  | O |  | O |  | X |
| 1000111 | O |  | X |  | X |  | X |  | O |  | O |  | O |
| 1001000 | O |  | X |  | X |  | O |  | X |  | X |  | X |
| 1001001 | O |  | X |  | X |  | O |  | X |  | X |  | O |
| 1001010 | O |  | X |  | X |  | O |  | X |  | O |  | X |
| 1001011 | O |  | X |  | X |  | O |  | X |  | O |  | O |
| 1001100 | O |  | X |  | X |  | O |  | O |  | X |  | X |
| 1001101 | O |  | X |  | X |  | O |  | O |  | X |  | O |
| 1001110 | O |  | X |  | X |  | O |  | O |  | O |  | X |
| 1001111 | O |  | X |  | X |  | O |  | O |  | O |  | O |
| 1010000 | O |  | X |  | O |  | X |  | X |  | X |  | X |
| 1010001 | O |  | X |  | O |  | X |  | X |  | X |  | O |
| 1010010 | O |  | X |  | O |  | X |  | X |  | O |  | X |
| 1010011 | O |  | X |  | O |  | X |  | X |  | O |  | O |
| 1010100 | O |  | X |  | O |  | X |  | O |  | X |  | X |
| 1010101 | O |  | X |  | O |  | X |  | O |  | X |  | O |
| 1010110 | O |  | X |  | O |  | X |  | O |  | O |  | X |
| 1010111 | O |  | X |  | O |  | X |  | O |  | O |  | O |
| 1011000 | O |  | X |  | O |  | O |  | X |  | X |  | X |
| 1011001 | O |  | X |  | O |  | O |  | X |  | X |  | O |
| 1011010 | O |  | X |  | O |  | O |  | X |  | O |  | X |

400	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table D.1—Multiple channel assignment table (continued)

| Bit |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  | Band 5 |  | Band 6 | Band 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1011011 | O |  | X |  | O |  | O |  | X |  | O |  | O |
| 1011100 | O |  | X |  | O |  | O |  | O |  | X |  | X |
| 1011101 | O |  | X |  | O |  | O |  | O |  | X |  | O |
| 1011110 | O |  | X |  | O |  | O |  | O |  | O |  | X |
| 1011111 | O |  | X |  | O |  | O |  | O |  | O |  | O |
| 1100000 | O |  | O |  | X |  | X |  | X |  | X |  | X |
| 1100001 | O |  | O |  | X |  | X |  | X |  | X |  | O |
| 1100010 | O |  | O |  | X |  | X |  | X |  | O |  | X |
| 1100011 | O |  | O |  | X |  | X |  | X |  | O |  | O |
| 1100100 | O |  | O |  | X |  | X |  | O |  | X |  | X |
| 1100101 | O |  | O |  | X |  | X |  | O |  | X |  | O |
| 1100110 | O |  | O |  | X |  | X |  | O |  | O |  | X |
| 1100111 | O |  | O |  | X |  | X |  | O |  | O |  | O |
| 1101000 | O |  | O |  | X |  | O |  | X |  | X |  | X |
| 1101001 | O |  | O |  | X |  | O |  | X |  | X |  | O |
| 1101010 | O |  | O |  | X |  | O |  | X |  | O |  | X |
| 1101011 | O |  | O |  | X |  | O |  | X |  | O |  | O |
| 1101100 | O |  | O |  | X |  | O |  | O |  | X |  | X |
| 1101101 | O |  | O |  | X |  | O |  | O |  | X |  | O |
| 1101110 | O |  | O |  | X |  | O |  | O |  | O |  | X |
| 1101111 | O |  | O |  | X |  | O |  | O |  | O |  | O |
| 1110000 | O |  | O |  | O |  | X |  | X |  | X |  | X |
| 1110001 | O |  | O |  | O |  | X |  | X |  | X |  | O |
| 1110010 | O |  | O |  | O |  | X |  | X |  | O |  | X |
| 1110011 | O |  | O |  | O |  | X |  | X |  | O |  | O |
| 1110100 | O |  | O |  | O |  | X |  | O |  | X |  | X |
| 11101001 | O |  | O |  | O |  | X |  | O |  | X |  | O |
| 1110110 | O |  | O |  | O |  | X |  | O |  | O |  | X |
| 1110111 | O |  | O |  | O |  | X |  | O |  | O |  | O |
| 1111000 | O |  | O |  | O |  | O |  | X |  | X |  | X |
| 1111001 | O |  | O |  | O |  | O |  | X |  | X |  | O |
| 1111010 | O |  | O |  | O |  | O |  | X |  | O |  | X |
| 1111011 | O |  | O |  | O |  | O |  | X |  | O |  | O |

| Copyright © 20xx IEEE. All rights reserved. | 401 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table D.1—Multiple channel assignment table (continued)

| Bit |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  | Band 5 |  | Band 6 | Band 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1111100 | O |  | O |  | O |  | O |  | O |  | X |  | X |
| 1111101 | O |  | O |  | O |  | O |  | O |  | X |  | O |
| 1111110 | O |  | O |  | O |  | O |  | O |  | O |  | X |
| 1111111 | O |  | O |  | O |  | O |  | O |  | O |  | O |

An example is shown in Figure D.1 where it is assumed that red, green and blue are available at the optical sources. If a certain optical source uses HP1 (00001) and another optical source in the adjacent cell uses HP2 (00011), then hopping pattern application in the adjacent cell is that HP1 operates R in first frame or time slot, B in second frame or time slot, G in third frame or time slot, but HP2 is operating at G in first frame or time slot, G and R in second frame or time slot, R and B in third frame or time slot. This mechanism can avoid interference between optical sources. Also the hopping pattern application is not limited to one frame or one time slot. A hopping pattern across multiple frames or time slots is fine.

Table D.2 shows a hopping pattern example applicable to OWC. If coordinator assign pattern '00001' to a device by using H_pattern, then the device’s frame or time slot moves according to the hopping pattern. Also one hopping pattern or multiple hopping patterns can be assigned to one user.

Figure D.1—Hopping pattern assignment

402	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table D.2—Example of hopping pattern assignment

| Pattern | 00001 | 00011 | 00101 |
| --- | --- | --- | --- |
| Frame/time slot | HP1 | HP2 | HP3 |
| 1 | R | G | B |
| 2 | B | G/R | B |
| 3 | G | R/B | G |
| 4 | G/R | B | G/R |
| 5 | G/R | R | G/B |
| 6 | R/B | G | R/B |
| 7 | G | B | R |
| 8 | B | R | G |
| 9 | R | G/B | R |

| Copyright © 20xx IEEE. All rights reserved. | 403 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Annex E

(informative)

Considerations for OWC using LED displays

E.1 Introduction—Dynamic dis plays vs . addres s ed dis plays

This annex discusses two types of LED involved displays: dynamic displays and addressed displays. The fundamental different between the two has to do with the amount of time the pixel is illuminated. In dynamic displays the pixels on a line are illuminated once every frame for T_ frame/N_line seconds where N_line is the number of lines controlled by a line scan controller and T_frame is the time to sweep N_line once; that is, the pixels on a line are operated dynamically so that they are only on for a fraction of the total frame time (hence the name “dynamic”). In the addressed display, the pixel is illuminated for the duration of the frame and is readdressed once per frame for possible state change.

E.2 Dynamic dis plays

E.2.1 Operation mechanis m

In general, a dynamic display consists of a host controller, a line scan controller, a display data buffer, and LED matrix, as shown in Figure E.1. The line scan controller selects a line for display, and the display data buffer transmits state information, such as the on/off state or the color selection, to each LED pixel on the selected line. The line scan controller determines the active time of each line, where the active time indicates whether the LED pixels on the selected line are switched ON or OFF as per the display data buffer for the active time of the selected line. Therefore, OWC using a dynamic display is tightly coupled with the active time.

Figure E.1—General architecture of LED signboard operated by the dynamic display mechanism

404	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

| IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC |  |  | IEEE Draft P802.15.13/D0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It is well known that most of PC monitors and TV display images use 60 displayed-frames per second. In the |  |  |  |  | 1 |  |
| case of PC monitors or TVs, the total displayed-frame number (i.e., different display information) is actually |  |  |  |  | 2 |  |
| 30 frames per second because each displayed-frame is transmitted twice. The display mechanism of a |  |  |  |  | 3 |  |
| dynamic display is similar to a PC monitor or TV. Assuming that a dynamic display consists of 16x16 lines |  |  |  |  | 4 |  |
| and displays images or text through 60 displayed-frames per second, with 16 lines per displayed-frame, the |  |  |  |  | 5 |  |
| active time slot period for OWC assigned to each line is 1.042 ms per displayed-frame, as shown in |  |  |  |  | 6 |  |
| Figure E.2. Therefore, an active time slot can transmit 130 byte at 1 Mbit/s, as shown in Figure E.3. |  |  |  |  | 7 |  |
|  |  |  |  |  | 8 |  |
|  |  |  |  |  | 9 |  |
|  |  | 1 display frame time = 1/60 sec |  |  | 10 |  |
|  |  |  |  |  | 11 |  |
|  |  |  |  |  | 12 |  |
| line 1 |  |  |  |  | 13 |  |
|  |  |  |  |  | 14 |  |
|  |  |  |  |  | 15 |  |
| line 2 |  |  |  |  | 16 |  |
|  |  |  |  |  | 17 |  |
| • |  | 1.042ms | • |  | 18 |  |
|  |  |  |  |  | 19 |  |
| • |  |  | • |  |  |  |
| • |  |  | • |  | 20 |  |
| line 16 |  |  |  |  | 21 |  |
|  |  |  |  |  | 22 |  |
|  |  |  |  |  | 23 |  |
|  |  | active time per display -frame per line = (1/60)/16 sec |  |  | 24 |  |
|  |  |  |  |  | 25 |  |
|  |  | Figure E.2—Operation mechanism of dynamic display |  |  | 26 |  |
|  |  |  |  |  | 27 |  |
|  |  |  |  |  | 28 |  |
|  |  |  |  |  | 29 |  |
|  |  |  |  |  | 30 |  |
| line 1 | slot 1 |  | slot 1 |  | 31 |  |
|  |  |  |  |  | 32 |  |
|  |  |  |  |  | 33 |  |
| line 2 |  | slot 2 | slot 2 |  | 34 |  |
|  |  |  |  |  | 35 |  |
|  |  |  |  |  | 36 |  |
| • |  |  |  |  | 37 |  |
| • | header | data | 130 bytes |  | 38 |  |
| • |  |  |  |  | 39 |  |
| • |  |  |  |  |  |  |
| • |  |  | • |  | 40 |  |
|  |  |  | • |  | 41 |  |
|  |  |  | • |  | 42 |  |
| line 16 |  |  | slot 16 |  | 43 |  |
|  |  |  |  |  | 44 |  |
|  |  |  |  |  | 45 |  |
|  | Figure E.3—An example of OWC data transmission on a dynamic display |  |  |  | 46 |  |
|  |  |  |  |  | 47 |  |
|  |  |  |  |  | 48 |  |
| E.2.2 Reduced brightnes s mitigation on OWC dynamic dis plays |  |  |  |  | 49 |  |
|  |  |  |  |  | 50 |  |
| The OWC dynamic display may be less bright than the non-OWC dynamic display because of the OWC |  |  |  |  | 51 |  |
|  |  |  |  |  | 52 |  |
| modulation during the active time periods. Therefore, it is important to minimize the reduction of the |  |  |  |  |  |  |
|  |  |  |  |  | 53 |  |
| average brightness of the dynamic display during the shortest time period that the human can distinguish. |  |  |  |  |  |  |
|  |  |  |  |  | 54 |  |
|  |  | Copyright © 20xx IEEE. All rights reserved. |  | 405 |  |  |
|  | This is an unapproved IEEE Standards Draft, subject to change. |  |  |  |  |  |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1	This is done to keep the display performing adequately for its intended function. Figure E.4 shows an

- example of an operational mechanism to mitigate the average brightness reduction per display frame time

- that can arise for a dynamic display having 16x16 lines operating as shown in Figure E.2 (i.e., the OWC data

- is carried during the active time of each line). The average brightness per display frame time in Figure E.4 is

- twice as large as that in Figure E.2 because the active time is twice as long.

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

|  | 1 frame time = 1/60 sec |  |  |  |
| --- | --- | --- | --- | --- |
| line 1 | slot 1 |  |  | slot 1 |
| line 2 |  | slot 2 | • • • | slot 2 |
|  | header | data |  | 260 bytes |
|  |  |  | • |  |
|  |  |  | • |  |
|  |  |  | • |  |
| line 8 |  |  | slot 8 |  |
| line 9 | slot 1 |  |  | slot 1 |
| line 10 |  | slot 2 | • • • | slot 2 |
|  | header | data |  | 260 bytes |
|  |  |  | • |  |
|  |  |  | • |  |
|  |  |  | • |  |
| line 16 |  |  | slot 8 |  |

active time per display -frame per line = (1/60)/8 sec

Figure E.4—An example for the mitigation of the average brightness reduction on the OWC dynamic display

E.2.3 OWC application us ing dynamic dis plays

A OWC enabled dynamic display can be used in the broadcast topology. The OWC broadcast topology in this standard consists of mainly the beacon and the downlink, as shown in Figure 20. Therefore, the OWC broadcast topology using a dynamic display can be constructed by the assignment of the active time slots and the use of GTS field in the beacon frame. Figure E.5 shows the OWC broadcast topology construction using the dynamic display. The active time slot #1 is assigned to the beacon and the active time slots from #2 to #8 are assigned to the downlink in Figure E.5. The GTS fields of the beacon frame can be used to indicate the GTS number, GTS length, and GTS direction for the broadcast topology. Multiple GTS slots can also be used depending upon the desired service level, the subscriber's grade, and the QoS policy.

406	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  | CFP |  |
| --- | --- | --- |
| line 1 | beacon | beacon |
| line 2 | GTS #1 | GTS #1 |
|  | GTS #2 | GTS #2 |
| • |  |  |
| • |  |  |
| • |  |  |
| • |  | • |
| • |  | • |
|  |  | • |
|  |  | GTS #7 |
| line 9 | beacon | beacon |
| line 10 | GTS #1 | GTS #1 |
| • |  |  |
| • |  | • |
| • |  | • |
|  |  | • |
| line 16 |  | GTS #7 |

Figure E.5—OWC broadcast topology construction using the dynamic display

E.3 Addres s ed dis plays

In an addressed display a particular pixel is addressed (i.e., refreshed) once every frame; that is, once addressed the pixel maintains its current state until readdressed during the next frame. The role of the LED can be either to provide pixel back lighting, as in the case of an LCD (liquid crystal display), or the pixel can be formed directly from a LED device as in the case of LED signage (which would typically be implemented as a pixel constructed from a compound RGB LED). Generally the addressed pixels in a frame are serially updated and there is no need for a retrace blanking interval as in the old days of cathode ray tubes.

E.3.1 LCD dis play us ing LED backlighting modulation

In LED backlighting, there are numerous LEDs that provide illuminance for all LCD pixels, while the intensity of a particular pixel is determined by the transmittance of the pixel LCD. This means the data modulation of the LED backlighting is going to radiate from all the pixels. The radiation is a density (mW/ unit area) and the best performance occurs when the sensor views the whole screen (i.e., ingests the most power). Viewing the whole screen also provides intensity averaging over all the pixels, which is advantageous since in the some scenes various pixels might go dark. Given enough area averaging, there no particular relationship between the data rate and the frame refresh rate.

E.3.2 LED pixel modulation

One has the option of either modulating individual pixels or groups of pixels. In other words, one can generate at least two display sections that can transmit different data. One could even go further and create intermediate sections that transmit an aggregate of the data transmitted in the adjacent sections. For this to

| Copyright © 20xx IEEE. All rights reserved. | 407 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- work one needs a detector capable of spatially resolving such sections. Partitioning of the display is done by

- using the cell mechanism and the PHY SAP, which were introduced for mobility support (see 5.1.13). For

3	example, the cell partitioning in Figure 45 can be interpreted as a 2x2 display with four sections if the

- transmitters are operated in the broadcast mode. Also, wavelength division multiplexing of RGB pixels can

- be enabled with this mechanism, so that each color carries a different data stream. Averaging over an area of

- the sign allows some insensitivity to scene-to-scene pixel intensity variation.

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

408	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Annex F

(informative)

Receiver performance variation on multi-color channels

Many applications using colored light sources can be considered in OWC. For example, Figure F.1 describes the scenario that a OWC receiver receives some information from a traffic signal light sources with color “A” and color “B”. Figure F.2 describes that an user with a OWC receiver can get the audio information from a color “A” lamp, the video information from a color “B” lamp, or the navigation information from a color “C” lamp. Figure F.3 shows that the multiplexing technologies such as Wavelength Division Multiplexing (WDM) can be applied to OWC applications using colorful light sources.

OWC services, using multiple color channels according to the OWC band plan, should be attained by only one OWC receiver. It is undesirable if a OWC receiver exhibits better receiver performance only on, for example, the color “A” channel but it does not exhibit the same performance on the color “B” or color “C” channel as on the color “A” channel. Therefore, a uniform performance on each color channel may be desired.

There are two main factors influencing the performance variation of a multi-color OWC receiver. One is the conversion relations between the radiometric and photometric units. The other is the photo sensitivity characteristics of a photo-detector, such as a Si photo-detector that depends on the wavelength variation, assuming such photo-detectors will be used as a receiver in OWC.

First, suppose that there are two light sources or OWC transmitters with red color and green color respectively and a OWC receiver to perceive the variation of received powers under the multiple color channels which originates from the conversion relations between the radiometric and photometric units. Also suppose that each color light radiates from two light sources at the same divergence angle, and they radiate with the same luminous flux, (lumens), so that the human eye senses the same brightness when simultaneously viewing the two light sources respectively at the same distance. However, when each color light radiates with the same luminous flux then the radiation of the two light sources in radiometric power (Watts) are each different, which is the origin of the CIE sensitivity curves. The CIE sensitivity curves indicate what the human eye senses, and it turns out that the green light is perceived as being brighter than red light when the radiometric radiation powers of the two light sources are equal. Therefore, the radiometric received powers of a receiver are different on red and green channels, respectively, even though the divergence angles and luminous fluxes of two light sources are equal, the same receiver is used, and the distances between the receiver and the light source are equal.

| Copyright © 20xx IEEE. All rights reserved. | 409 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure F.1—OWC application using traffic signal light sources

Figure F.2—OWC application using colored light sources

410	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Figure F.3—OWC application using WDM technology

Of course, the OWC light source power can be also defined in radiometric unit, Watt. However, it is more reasonable that the light source power is defined in photometric unit because the light sources will be used not only for a OWC transmitter but also for illumination or visual display related to human eye.

Table F.1 describes the receiver input powers, calculated in Watts, from the assumption of 1 lumen on each of the seven color bands (given in Table 111). The assumption that the lights have only monochromatic component (shown as example wavelength in Table F.1) on each color band is also used for simple calculations. V() is the human eye sensitivity function, which indicates CIE sensitivity curves (Schubert [B12]).

Table F.1—Calculated color channel power at receiver

| Wavelength band |  |  | Spectral width | Example | V() at | Receiver input |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | example |  |  |
|  | (nm) |  | (nm) | wavelength (nm) |  | power (Watts) @ 1m |  |
|  |  |  |  |  | wavelength |  |  |
| 380 |  | 478 | 98 | 430 | 0.0273 | 0.0536 |  |
| 478 |  | 540 | 62 | 510 | 0.5030 | 0.0029 |  |
| 540 |  | 588 | 48 | 565 | 0.9788 | 0.0015 |  |
| 588 |  | 633 | 45 | 610 | 0.5030 | 0.0029 |  |
| 633 |  | 679 | 46 | 655 | 0.0817 | 0.0179 |  |
| 679 |  | 726 | 47 | 700 | 0.0041 | 0.3571 |  |
| 726 |  | 780 | 54 | 750 | 0.0001 | 14.6413 |  |

| Copyright © 20xx IEEE. All rights reserved. | 411 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

- The second factor causing the performance variation of a OWC receiver across multiple color channels is

- the photo sensitivity characteristics of optical receivers, such as Si photo-detectors, which is wavelength

3	dependent. Figure F.4 shows the photo sensitivity characteristics of a Si photo-detector according to the

- wavelength variation. It has been known that the photo sensitivity value of Si photo-detector is higher on

- longer wavelength than on shorter wavelength in the visible band as shown in Figure F.4. Figure F.4 shows

- that a Si photo-detector produce more electrical current on red color channel than on green or blue color

- channel even though the radiometric received powers on each color channel are equal.

8

- Table F.2 shows the photo-detector output current obtained from both the wavelength dependence of photo

- sensitivity shown in Figure F.4 and the conversion relations between the radiometric and photometric units

| 11 | described in Table F.1. The photo-detector output currents in Table F.2 were calculated only at 430 nm, |  |
| --- | --- | --- |
| 12 | 510 nm, and 655 nm among the example wavelengths in seven color bands, as shown in Table F.1, for |  |
| 13 | convenience. |  |
| 14 | Table F.2 indicates that a OWC receiver with Si photo-detector performs differently on multiple color |  |
| 15 |  |  |

- channels even though the radiometric received powers are equal on each color channel. Therefore, two main

- factors, the unit conversion and the photo sensitivity of a photo-detector depending on wavelength, need to

- be sufficiently considered in order that the performance of a OWC receiver can be maintained uniformly on

- multiple color channels.

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

|  | 0.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.7 |  |  |  |  |  |  |  |  |  |
| (A/W) | 0.6 |  |  |  |  |  |  |  |  |  |
|  | 0.5 | (0.46) |  | Red |  |  |  |  |  |  |
| sensitivity |  |  |  |  |  |  |  |  |  |  |
|  | 0.3 | (0.3) | Green |  |  |  |  |  |  |  |
|  | 0.4 |  |  |  |  |  |  |  |  |  |
| photo | 0.2 | (0.21) |  |  |  |  |  |  |  |  |
|  |  |  |  | Blue |  |  |  |  |  |  |
|  | 0.1 |  |  |  |  |  |  |  |  |  |
|  | 0.0 |  |  | 430 | 510 | 655 |  |  |  |  |
|  | 300 |  | 400 | 500 | 600 | 700 | 800 | 900 | 1000 |  |

wavelength (nm)

Figure F.4—Typical Si photo-detector wavelength sensitivity

Table F.2—Photo-detector current from Figure F.4 with conditions of Table F.1

| Example wavelength | Receiver input power | Photo sensitivity (A/W) | Photo-detector output |  |
| --- | --- | --- | --- | --- |
| (nm) | (Watt) @ 1 m |  | current (mA) @ 1 lm |  |
| 430 | 0.0536 | 0.21 | 11.26 |  |
| 510 | 0.0029 | 0.30 | 0.87 |  |
| 655 | 0.0179 | 0.46 | 8.23 |  |

412	Copyright © 20xx IEEE. All rights reserved.

This is an unapproved IEEE Standards Draft, subject to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Annex G

(informative)

Notes on the receiver algorithm

In order to demodulate the data encoded in the first stream (St1), the samples in every even OFDM symbol interval are subtracted from the samples in every odd OFDM symbol interval. For example, the samples in the second symbol are subtracted from the samples in the first symbol, the samples in the fourth symbol are subtracted from the samples in the third symbol, etc. Note that due to the structure of the other data streams (St2 up to St4), the subtraction operation completely removes interference from these streams.

After the subtraction operation, the resulting data at St1 can be demodulated using the conventional OFDM demodulator algorithm. After the data in St1 are demodulated, the stream is re-modulated again and sub-tracted from the overall received signal.

Following the subtraction of the re-modulated St1 signal, all frame copies at St2 which were originally iden-tical (the symbols in question are the symbols that have identical polarity) should be summed up. For exam-ple, the first and the second symbols should be summed, the third and the fourth symbols should be summed, the fifth and the sixth symbols frames should be summed, etc. The resulting signal at St2, then, has the same format as the signal at St1. Hence, it can be demodulated after each even symbol is subtracted from each odd frame. Upon demodulation, the resulting bits can be re-modulated and the resulting waveform can be sub-tracted from the overall received waveform as in the case for St1. The procedure is re-iterated for the remaining streams up to St4 (depending on the configuration) until all data bits are recovered.

| Copyright © 20xx IEEE. All rights reserved. | 413 |
| --- | --- |

This is an unapproved IEEE Standards Draft, subject to change.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | Annex H |  |
| --- | --- | --- |
| 2 |  |  |
| 3 | (informative) |  |
| 4 |  |  |
| 5 | Color function support |  |
| 6 |  |  |
| 7 |  |  |

- Color function is a function that provides information, such as device status and channel quality, to the

- human eye via color. In IEEE Std 802.15.7TM-2011, color function is specified. The CVD frame, using

- various colors, can be used to display various statuses of a device. The colors mapped for each status of the

- devices are based on the phyColorFunction (see IEEE Std 802.15.7TM-2011). The colors chosen for different

- statuses are left to the discretion of the implementer. Multiple statuses may choose the same color,

| 13 | depending on the number of colors supported by the device. The use of color function through the CVD |  |  |
| --- | --- | --- | --- |
| 14 |  |  |  |
|  | frame has the potential to change the color of the emitted light. |  |  |
| 15 |  |  |  |
| 16 |  |  |  |
| 17 |  |  |  |
| 18 |  |  |  |
| 19 |  |  |  |
| 20 |  |  |  |
| 21 |  |  |  |
| 22 |  |  |  |
| 23 |  |  |  |
| 24 |  |  |  |
| 25 |  |  |  |
| 26 |  |  |  |
| 27 |  |  |  |
| 28 |  |  |  |
| 29 |  |  |  |
| 30 |  |  |  |
| 31 |  |  |  |
| 32 |  |  |  |
| 33 |  |  |  |
| 34 |  |  |  |
| 35 |  |  |  |
| 36 |  |  |  |
| 37 |  |  |  |
| 38 |  |  |  |
| 39 |  |  |  |
| 40 |  |  |  |
| 41 |  |  |  |
| 42 |  |  |  |
| 43 |  |  |  |
| 44 |  |  |  |
| 45 |  |  |  |
| 46 |  |  |  |
| 47 |  |  |  |
| 48 |  |  |  |
| 49 |  |  |  |
| 50 |  |  |  |
| 51 |  |  |  |
| 52 |  |  |  |
| 53 |  |  |  |
| 54 |  |  |  |
|  | 414 | Copyright © 20xx IEEE. All rights reserved. |  |
|  |  | This is an unapproved IEEE Standards Draft, subject to change. |  |