IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG 802.15 minutes for July, 2017 Plenary meeting of IG LPWA |  |
| Date Submitted | [July 13th, 2017] |  |
| Source | [Charlie Perkins] [Futurewei] | Voice: [+1-408-330-4586] Fax: [ ] E-mail: [charliep@computer.org] |
| Re: | [TG 802.15 IG LPWA meeting minutes] |  |
| Abstract | TG 802.15 IG LPWA minutes from the Berlin IEEE 802 Wireless Plenary Meeting |  |
| Purpose | [Record the transactions of the TG 802.15 IG LPWA Plenary meeting during July, 2017] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Minutes for IEEE TG 802.15 IG LPWA Berlin Plenary Meeting

10-13 July, 2017

Chair – Jörg Robert

Objectives (all meetings in room Lyon):

- Monday PM1: Agenda / Status Review / Liaison with ETSI LN

- Tuesday PM1: ETSI LTN / Review of discussion about IG LPWA at IETF 98 / Contributions

- Wednesday PM1 : Contributions: Evaluations for Modulations, FEC

- Thursday PM1: Contributions: Evaluations / IG Report / AoB / Next Steps

Monday PM1 (July 10) – Agenda / Status Review / BATS /Liaison with ETSI LTN

- Open

- Agenda

  - Open

  - IEEE-SA Stds. Board Bylaws on Patents in Std's. & Guidelines

  - Approval of the Agenda (DCN 15-17-0380-00-lpwa)

  - Approval of Daejeon Minutes (DCN 15-17-0229-00-lpwa)

  - Update on Literature List

  - Outcome of Last IG LPWA Telco

  - BATS (DCN 15-17-0383-00-lpwa)

  - ETSI LTN

  - IETF 98 summary

  - Recess

- IEEE-SA Stds. Board Bylaws on Patents in Std's. & Guidelines

  - No relevant IPR noted

- Approval of the Agenda (DCN 15-17-0380-00-lpwa)

  - Approved without objection

- Summary from January interim meeting

  - Minutes are available (DCN 15-17-0229-00-lpwa)

  - Approved without objection

- Update on Literature List (DCN 15-16-0749-01-lpwa)

- 7 contributions planned during plenary meeting

- Contributions to literature list welcome (DCN 15-16-0749-01-lpwa)

- Outcome of IG LPWA telephone conferences from Feb 22

  - Minutes are available (DCN 15-17-0347-00-lpwa)

  - No discussion / objections

- BATS

  - 2g bat transmitter affixed to 15 bats: must be limited to 10% of animal’s weight

  - Wake-up receivers

  - Inner base stations for localization, LPWAN for telemetry

  - Forney Interleaver Reed-Solomon, Convolutional: Final bitrate = 4 bits/second

  - Error rate of 10% with no coding at 0db noise, with coding 10% at -4 dB

  - Max pathloss of L=150dB

  - Graph for predicted media loss for different antenna configurations

  - 2 TB disk collects data and last for a week

  - 2016 Field Trial: communication works

  - Shows received signals must stronger than bat signals – strong interference

  - Later this year will repeat experiment in Panama, but 950 MHz instead

- ETSI LTN (“low throughput networks”)

  - See presentation in (DCN 15-17-0373-00-lpwa)

  - Access to ETSI LTN documents is only granted to ETSI members

  - SIGFOX, Sony, Fraunhofer, Telensa

  - No “beauty contest” between thr profiles

  - Discussion about whether ETSI publications can attain the force of law

  - IG LPWA Liaison to ETSI LTN

  - MoU between IEEE and ETSI exists, can extend it to IG LPWA

  - Review of the ETSI document, which is quite extensive

Recess at 3:25pm

Minutes for LP-WA Tuesday (July 11 PM1)

Meeting called to order at 1:35.  Attendance: 26

Agenda for PM1 session:

- Open

- Contributions / IG Report

- Recess

Discussion about ETSI LTN

- TS 103 357 has the 4 different proposals

- Spectrum regulation in TR 103 526, which is NOT a document from LTN

- Document will be published by ERM TG28

- TS 103 435 v1.1.1 has answers for questions from Monday in document in 15-17-0204-00-lpwa

Pat Kinney: IEEE Std 802.15.4 - low power wide area network (15-17-0248-00-lpwa)

- LECIM == 802.15.4k was intended to be LPWAN, considering to change the name

- Fragmentation done at PHY layer; description about details

  - If done at the MAC layer or above, would be 20 bytes or more overhead

- Fragment acknowledgement, policy variations

- Priority channel access

- TRLE – repeater by using PAN

- Use cases: pipelines, soil monitoring, water leak, inventory control, bridge stress monitor

Joerg Robert: Suitabiity analysis of IEEE Std 802.15.4 (15-17-0346-00-lpwa)

- Existing IEEE 802.15.4 technologies fulfill many LPWAN requirements

- PHY treats each fragment as a separate PSDU

- Compression techniques are used to minimize overhead

- Reassembly is achieved on a “high PHY” sublayer (not MAC layer)

- Low rate O.K. for rare messages

- Fragmentation makes it worse than competitors (15-16-0486-00)

- But much better than 6lowpan fragmentation, which requires MAC header per fragment

- Adding FEC at a lower PHY fragmentation is beneficial (Long long discussion about results.)

Joerg Robert: Packet Splitting for Improved Robustness (15-17-0344-00-lpwa)

- Interference in License-Exempt Bands

Recess at 3:05pm

Minutes for LP-WA Wednesday (July 12 PM1)

Meeting called to order at 1:35

Attendance: 25

Agenda for PM1 session:

- Open

- Contributions / IG Report

- Recess

Continued Discussion about Packet Splitting for Improved Robustness from University of Erlangen-Nuernberg (DCN 15-17-0355-00)

- More complex receiver

- On-air data rate 100bit/s

Discussion about IETF LPWAN WG [Pascal Thubert from Cisco Systems] (DCN 15-17-0417-01)

- Overview document describing four technologies is about to be finished (includes previous LoRa version due to timeline)

- What is still needed? Where are gaps in the existing technologies?

- (flooding the network with a lot of messages. Bit Map split up in Groups and Bitmap)

- SCHC: Whole header is compressed by using a book of all possible messages of a device. The dictionary will have to be provided by the manufacture.

- Small devices will not work on the uncompressed packets and do not need to know the dictionary. A short message may only need e.g. 3 bytes in compressed form.

- Not yet designed how the IP address is assigned to a device.

- Without return path, is there a need for more redundancy to compensate for losses of fragments?

- Jörg will upload Rev. 2 with typo corrections.

Suitability Evaluation of Modulation Schemes (DCN 15-17-0374-00-lpwa)

- Meaning of entries in the tables

  - Qualitative Evaluation:

    - Red: “Don’t do it”

    - Yellow: “Possible but maybe not optimal”

    - Green: “Good”

  - Active Interfering Users are from the same systems, interference model covers interference from other systems.

- OFDM (Orthogonal Frequency Division Multiplexing) splits the data into multiple parallel sub-carriers

  - Discussion if evaluation is too simple because the performance depends on chosen parameters/implementation. E.g. higher data rate with lesser on-time for better power performance.

- CDMA (Code Division Multiple Access) spreads the data using orthogonal sequences

  - Bad interference situation with a lot of users because power control is needed. Same situation with large cell areas.

- DSSS (Direct Sequence Spread Spectrum) spreads the data using defined sequences (similar to CDMA without orthogonal codes)

  - Comparable to CDMA

- FCSS (Frequency Chirp Spread Spectrum) spreads the data using frequency chirps (typically frequency ramp)

  - Comparable to DSSS but not very wideband, localization worse.

- FHSS (Frequency Hopping Spread Spectrum) spreads narrow-band signals onto different frequencies using frequency hopping

  - Table represents the transmitter. Receiver is more power consuming (FEC) and cannot run on CR2025 battery.

- Use of narrow-band modulation (e.g. FSK, MSK, ...) with non-coherent decoding

  - Without frequency hopping very sensitive to interference (small bandwidth, long transmission time). Cell radius restricted.

  - Localization difficult small band.

  - For low power transmission time very long, conflict with FCC.

  - High velocity, channel changes too quickly for demodulation

- Use of narrow-band modulation (e.g. FSK, MSK, ...) with coherent decoding

  - Comparable to the non-coherent ones. But channel estimation is required.

Suitability Evaluation of FEC Schemes (DCN 15-17-0375-00-lpwa)

- Qualitative Evaluation:

  - Red: “Don’t do it”

  - Yellow: “Possible but maybe not optimal”

  - Green: “Good”

- No FEC:  Simple transmission of the data without forward error-correcting codes

  - Discussion: loss of coding gain no problem if the link margin is high enough

  - Majority of SC transmissions in IEEE 802.15.4 do not use FEC in order to decrease collision risk.

- Reed Solomon / BCH

  - Efficient for high rate codes but not low rates.

- Convolutional Code: Encode the data using convolutional encoder, decoding typically based on Viterbi algorithm

- Turbo Codes are based on concatenated convolutional codes with iterative decoding

  - For short data lengths the iterative part of the codes does not improve the quality very much.

- Low Density Parity Check (LDPC) Code are based on block codes with spare code matrices

- Polar Codes: New code class for short codes that is currently discussed for 5G for protecting signaling information

Recess at 3:25pm

Minutes for LP-WA Thursday (July 13 PM1)

Agenda for Thursday PM1 session:

- Open

- Contributions / IG Report

- Review of Time Line

- AoB

- Adjourn

Open Session at 1:37pm

Discussion about performance of coding schemes; TMO paper from 1998  <Jörg Robert>

- Imperfectness (Fig. 13) measured in dB-SNR

- Pointer will be provided in literature contribution for our reference, but not on mentor

- Question about IPR for the various coding schemes

Suitability Evaluation of Connectivity  (DCN 15-17-0376-01-lpwa) Jörg Robert

- Gateway / Transparent [Transmission of the data without IP]

- IP w/o Header Compression

- IP w/ Header Compression [Static Context Header Compression (SCHC)]

  - Note: higher level protocols work unchanged because header is restored

  - Discussion whether SCHC suitable for multi-hop

  - Discussion about IP header compression

Suitability Evaluation of Network Topologies  (DCN 15-17-0379-00-lpwa) Jörg Robert

- Star (single hop)

- Extended Star (single hop)

- Device to Device

- Base-Station Assisted Network (single hop) <Jussi Haapola>

- Mesh

Suitability Evaluation of MAC Schemes (DCN 15-17-0378-00-lpwa) Jörg Robert

- ALOHA

- Slotted ALOHA

- CSMA

- CSMA/CA

  - Actually there are several different kinds of CSMA, CSMA/CA

- Fully Synchronized Network

Suitability Evaluation of Encryption Schemes (DCN 15-17-0377-00-lpwa) Jörg Robert

- Symmetric Key: Transmitter and receiver know the same key which has to be kept secret

- Public Key: Decryption with private key that is only known to receiver

- AES-128/256: Symmetric key

- RSA: Public Key System based on prime numbers as trap-door function

- Elliptic Curves: Public Key System based on algebraic structure of elliptic curves

- Over the Air Key Update

Next Steps

- Copy discussed items to report document

- Agree on 802.15.4k suitability

    - Indicate use-cases (defined in 16/770r5) that are currently not supported and are highly suitable for LPWAN

- Finalize IG report

- Agree on recommendations to IEEE

Timeline review

- March 2017 Plenary (Vancouver)

  - Fixed evaluation criteria

  - Presentation of contributions with focus technology options for LPWA

- May 2017 Daejeon

- July 2017 Plenary (Berlin)

  - Presentation of contributions with focus technology options for LPWA

  - Final discussion on IG report

August 2017 Telephone Conference

- Proposed date: 3rd August (Thursday), 17:00 CEST, 8:00AM PDT

- Discussion on IG report draft

Adjourned at 3:22pm